3GPP TSG-RAN2 Meeting #118-e								R2-220xxxx

# Electronic, 9th – 20th May, 2022

Source: 		Email discussion Rapporteur (Huawei, HiSilicon)

Title: 	Summary for [Post118-e][604] 38.321 Positioning CR (Huawei)

Agenda item:	6.6.2

Document for: 	Discussion and Decision

General

This document contains the list of comments made during the review of the MAC CR for Positioning

Contacts

| Name | Company | Email address |
| --- | --- | --- |
| Xiaolong Li | Xiaomi | lixiaolong1@xiaomi.com |
| Jianxiang Li | CATT | lijianxiang@catt.cn |

Comments

| Companies | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Xiaomi | We are not clear why UE needs to cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE, in our understanding, the UE shall not cancel it. | Suggest to remove the sentence marked with yellow. The MAC entity shall, 1>if Positioning Measurement Gap Activation/Deactivation Request MAC CE has been triggered, and not cancelled: 2> if indication from upper layer has been received that the triggered Positioning Measurement Gap Activation/Deactiation Request MAC CE should be cancelled: 3> cancel the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE. 2> if UL-SCH resources are available for a new transmission and these UL-SCH resources can accommodate the Positioning Measurement Gap Activation/Deactivation Request MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Positioning Measurement Gap Activation/Deactivation Request MAC CE according to the upper layer's request; 3> cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE. 2> else: 3> trigger a Scheduling Request for Positioning Measurement Gap Activation/Deactivation Request MAC CE. |  |
| Xiaomi | The reference clause should be 5.26.2. | The MAC entity shall, if the TA of the configured Positioning SRS is valid according to clause 5.26.2: - transmit Positioning Periodic SRS or Semi-Persistent SRS defined in TS 38.214 [7]. |  |
| CATT | 6.1.3.42 Does N in the numEntry description mean the N in Oct N in Figure 6.1.3.42-1: PPW Activation/Deactivation Command MAC CE? If so, the description of numEntry should be updated as N, not N-1. If not, please use another number, such as M without confusion with the existing N in this MAC CE. | - numEntry: This field indicates the number of entries N in the MAC CE. 00 indicates that N equals to 2; 01 indicates that N equals to 3 and so on. The length of the field is 2 bits; |  |