3GPP TSG-RAN WG2 #119bis-e       										       R2-22xxxxx

Online, 10 – 19 Oct, 2022

Agenda Item:	xxx

Source: 	Huawei

Title: 	Report of [POST119-e][313][NES] Details of solutions (Huawei)

Document for:	Discussion and decision

# Introduction

During RAN2 #119-e, the NES techniques were discussed online and categorized as follows:

Solution groups:

- Adaption of MIB/SSB/SIB

-  partial/simplified SSB

2	Increase of SSB/SIB periodicity

3	On demand SSB/SIB1 (FFS if there are enhancements for other SIBs)

- FFS for on-demand MIB

4	Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

5	Handover/Fast PCell change for NES

- CHO or new configuration

- group HO

6	Resource adaptation (frequency and time domain)

- Including PRACH, SRS, PUSCH, PUCCH resources and periodicities

- cell DTX/DRX

- measurement

- reference signal type and configuration of reference signal pattern for connected mode

- BWP adaptation

7	Any Cell activation/re-activation or UE wake up request signal (connected/idle)

8	Paging enhancements (includes paging-less solutions)

9	Cell selection/reselection (ie. cell prioritization also including legacy UEs)

Things to study

- Study group configuration and signalling for transitions for different solutions

- pre-configuration and L1/L2 signaling to trigger change of configuration

2	Identify/capture RAN2 impact to legacy for the different solutions

3	Awareness of the NES states at the UE side for the different solutions

4	Aim to minimize DL signalling for NES

5	Consider UE complexity and energy consumption

6	UE assistance information for the specific network energy technique, it’s benefits and impact to UE/NW

This email discussion aims to understand the solutions better (e.g. general analysis of NES gain, complexity, RAN2 impact), and attempt prioritization of the solutions.

- [POST119-e][313][NES] Details of solutions (Huawei)

- Capture more RAN2 details/impact/benefit on the solution groups and additional things to study

- Attempt prioritization of solutions

Deadline: Long

The discussion is divided into two phases:

Phase 1: Companies are invited to check whether the solutions are captured correctly (especially proponent companies of the solution).

Note: The solutions we listed here are more stage2-like, as the main intention is to have a complete list of the directions that would be discussed in RAN2. Companies can first assess whether these stage-2 like solutions have captured the major principle from the inputs, it is not likely that all the stage 3 details can be discussed within this email thread.

(Deadline: Wednesday 2022-09-14 0800 UTC)

Phase 2: Companies are invited to provide views on each solution.

(Deadline for companies’ inputs: Thursday 2022-09-22 0800 UTC

Deadline for rapporteur’s summary: Monday 2022-09-26 0800 UTC)

# Discussion

## Solution 1 & 4: Adaptation of MIB/SSB/SIB & Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

Solution statement:

1)	SSB/SIB-less:

| Introduction | Some NES Cells do not transmit SSB/SIB, UE receives SSB/SIB from a different cell (e.g. anchor cell). |
| --- | --- |
| Scenario | Multi-carrier; UEs in all states (Connected/Idle/Inactive) |
| NES gain | Reduced time domain symbols for SSB/SIB-less NES cell. Possibly increased power consumption for anchor cell when the anchor cell broadcasts system information for other NES cells. |
| Impact to legacy UEs | No impact, legacy UEs can access from anchor cell |
| UE assistance needed | No |
| RAN2 impact | UE capability, extended SIB for anchor cell, cell selection/reselection, RACH, etc |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

2)  Partial/simplified SSB, MIB-less:

| Introduction | Simplified SSB without MIB (MIB is transmitted on anchor cell), or a newly defined RS. |
| --- | --- |
| Scenario | Multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols |
| Impact to legacy UEs | No impact, legacy UEs can access from the anchor cell. |
| UE assistance needed | No |
| RAN2 impact | Initial access, measurement, etc |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

In case some technique related to solution 1 & 4 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 2: Increase of SSB/SIB1 periodicity

Solution statement:

| Introduction | Longer SSB/SIB1 periodicity |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols |
| Impact to legacy UEs | Longer access delay |
| UE assistance needed | UE preferred SSB configuration |
| RAN2 impact | SSB/SIB1 periodicity is more of network implementation if no bigger periodicity is introduced. Other impacts depend on the solution details, e.g, to introduce a second periodicity (NES-specific periodicity) |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

In case some technique related to solution 2 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 3 & 7: On-demand SSB/SIB1 & Any Cell activation/re-activation or UE wake up request signal (connected/idle)

Solution statement:

On-demand SSB/SIB1, triggered by WUS

| Introduction | Cells in NES state only transmit discovery signals (DRS), UE uses wake-up signals (WUS) to trigger the transmission of SSB/SIB1 (FFS enhancements to other SIBs and MIB) |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols |
| Impact to legacy UEs | Legacy UEs can only access the cell after it is wakened. |
| UE assistance needed | No (if wake-up signal is not considered as “assistance information”) |
| RAN2 impact | Informing UEs of the DRS/WUS configuration; Procedures related to DRS (measurement, cell selection/reselection etc); Procedures related to WUS (triggering condition for sending WUS etc). |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

In case some technique related to solution 3 & 7 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 5: Handover/Fast PCell change for NES

Solution statement:

- Group HO/CHO

| Introduction | Pre-configure the candidate target cell(s) to the UEs, and trigger the HO/CHO with group-common signalling. |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in connected state |
| NES gain | Reduced HO commands |
| Impact to legacy UEs | Not applicable to legacy UEs |
| UE assistance needed | No |
| RAN2 impact | Details of the group-common HO/CHO signalling, details for pre-configuration of candidate cells, etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

Group configuration update

For group configuration update, the solution is quite similar to the resource adaptation (Solution 6), which also discusses “triggering a configuration change using group-common signalling”. So no duplicated discussion will happen here.

In case some technique related to solution 5 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 6: Resource adaptation (frequency and time domain)

Solution statement:

- Resource adaptation (e.g. 2-step approach, combined with NW state transition)

| Introduction | Pre-configure the UEs using dedicated signalling, and trigger a configuration change or UE behaviour change using group-common signalling (the triggering is based on NW state transition between NES state and non-NES state). |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in connected state |
| NES gain | Reduced signalling and faster NW state transition. |
| Impact to legacy UEs | Not applicable to legacy UEs |
| UE assistance needed | No |
| RAN2 impact | Details of pre-configuration signalling and group common signalling, etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

- Cell DTX/DRX

| Introduction | Configure DRX in a UE-group or cell-specific manner, so that DTX at the gNB can be applied and aligned. |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in connected state |
| NES gain | Reduce the always-on transmission/monitoring |
| Impact to legacy UEs | Current spec already allows DTX/DRX, where legacy UEs can only be configured by per-UE signalling. |
| UE assistance needed | Traffic characteristic/status etc. More information can be provided by proponents. |
| RAN2 impact | Current spec already allows configuring same DRX configuration for UEs. FFS on additional enhancements. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

- On-demand measurement, adaptation of RS

| Introduction | UE is allowed to notify network its preference on reference signal pattern for measurements via assistance information. The Network can use MAC CE to activate / deactivate a different pattern of reference signals for measurements |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in Connected state |
| NES gain | NW can apply sparse reference signal transmission unless additional RS is requested by the UE |
| Impact to legacy UEs | Legacy UEs cannot request on-demand RS for measurements, so they may not get sufficient RS samples to satisfy the RAN4 measurement requirement when the network decides sparse reference signal transmission. |
| UE assistance needed | Preferred RS pattern |
| RAN2 impact | UE indication, and NW activation/deactivation of RS patterns etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

- BWP adaptation

| Introduction | UEs can be configured with a cell-NES specific BWP, and use group common signalling to switch UEs to the cell-NES specific BWP |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in Connected state |
| NES gain | Reduced frequency-domain resources, and NW can use the same smaller transmission bandwidth to serve all UEs |
| Impact to legacy UEs | Legacy UEs don’t support the group signalling, but relying on the existing UE-dedicated BWP switching or timer. |
| UE assistance needed | To be provided by proponents |
| RAN2 impact | Group-common BWP switching signalling. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

In case some technique related to solution 6 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 8: Paging enhancements (includes paging-less solutions)

Solution statement:

Paging reception from a non-camped cell

| Introduction | UE camps on a cell (for Idle/Inactive), or is served by a cell (for Connected), but receives paging on another cell |
| --- | --- |
| Scenario | Multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols for the carrier where UE camps or is served |
| Impact to legacy UEs | No impact, the legacy UE can receive paging from the anchor cell |
| UE assistance needed | No |
| RAN2 impact | Indicating paging configuration to the UE, etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

In case some technique related to solution 8 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 9: Cell selection/reselection (i.e. cell prioritization also including legacy UEs)

Solution statement:

| Introduction | NES cells can be de-prioritized during cell selection/reselection, optionally, UE is made aware of cell state (NES or non-NES). |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in Idle/Inactive |
| NES gain | Reduced time domain symbols if the cell is in NES state |
| Impact to legacy UEs | - In case legacy mechanism (frequency priority, or adding frequency/cell-specific offsets) is used, there is no impact on legacy UEs - In case cell state (NES, or non-NES, or other states) is introduced, legacy UEs are not aware. |
| UE assistance needed | No |
| RAN2 impact | Cell selection/reselection enhancement etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |

In case some technique related to solution 9 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

# Conclusion

To be completed

# References

- R2-2208703, Report from UP, Small data, URLLC/IIoT, RACH indication, NWES and UAV, Session chair (InterDigital)