3GPP TSG-RAN WG2 #119bis-e       										       R2-22xxxxx

Online, 10 – 19 Oct, 2022

Agenda Item:	xxx

Source: 	Huawei

Title: 	Report of [POST119-e][313][NES] Details of solutions (Huawei)

Document for:	Discussion and decision

# Introduction

During RAN2 #119-e, the NES techniques were discussed online and categorized as follows:

Solution groups:

- Adaption of MIB/SSB/SIB

-  partial/simplified SSB

2	Increase of SSB/SIB periodicity

3	On demand SSB/SIB1 (FFS if there are enhancements for other SIBs)

- FFS for on-demand MIB

4	Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

5	Handover/Fast PCell change for NES

- CHO or new configuration

- group HO

6	Resource adaptation (frequency and time domain)

- Including PRACH, SRS, PUSCH, PUCCH resources and periodicities

- cell DTX/DRX

- measurement

- reference signal type and configuration of reference signal pattern for connected mode

- BWP adaptation

7	Any Cell activation/re-activation or UE wake up request signal (connected/idle)

8	Paging enhancements (includes paging-less solutions)

9	Cell selection/reselection (ie. cell prioritization also including legacy UEs)

Things to study

- Study group configuration and signalling for transitions for different solutions

- pre-configuration and L1/L2 signaling to trigger change of configuration

2	Identify/capture RAN2 impact to legacy for the different solutions

3	Awareness of the NES states at the UE side for the different solutions

4	Aim to minimize DL signalling for NES

5	Consider UE complexity and energy consumption

6	UE assistance information for the specific network energy technique, it’s benefits and impact to UE/NW

This email discussion aims to understand the solutions better (e.g. general analysis of NES gain, complexity, RAN2 impact), and attempt prioritization of the solutions.

- [POST119-e][313][NES] Details of solutions (Huawei)

- Capture more RAN2 details/impact/benefit on the solution groups and additional things to study

- Attempt prioritization of solutions

Deadline: Long

The discussion is divided into two phases:

Phase 1: Companies are invited to check whether the solutions are captured correctly (especially proponent companies of the solution).

Note: The solutions we listed here are more stage2-like, as the main intention is to have a complete list of the directions that would be discussed in RAN2. Companies can first assess whether these stage-2 like solutions have captured the major principle from the inputs, it is not likely that all the stage 3 details can be discussed within this email thread.

(Deadline: Wednesday 2022-09-14 0800 UTC)

Phase 2: Companies are invited to provide views on each solution.

(Deadline for companies’ inputs: Thursday 2022-09-22 0800 UTC

Deadline for rapporteur’s summary: Monday 2022-09-26 0800 UTC)

# Discussion

## Solution 1 & 4: Adaptation of MIB/SSB/SIB & Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

Solution statement:

1)	SSB/SIB-less:

| Introduction | Some NES Cells do not transmit SSB/SIB, UE receives SSB/SIB from a different cell (e.g. anchor cell). |
| --- | --- |
| Scenario | Multi-carrier; UEs in all states (Connected/Idle/Inactive) |
| NES gain | Reduced time domain symbols for SSB/SIB-less NES cell. Possibly increased power consumption for anchor cell when the anchor cell broadcasts system information for other NES cells. |
| Impact to legacy UEs | No impact, legacy UEs can access from anchor cell |
| UE assistance needed | No |
| RAN2 impact | UE capability, extended SIB for anchor cell, cell selection/reselection, RACH, etc |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | First, we have clarification questions for proponent: 1. whether the anchor cell is always in inter-frequency or can be intra-frequency (compared with cell for data transmission). Note that RF retuning gap may be needed to be specified if inter-frequency to receive SSB/SIB 2. why "connected UE" needs to support this feature? Note that in legacy system, connected UE typically acquires updated SIB via dedicated signaling. 3. whether anchor cell can only perform initial access or it can also perform data transmission? Note that if it can't perform data transmission, it will have impact on legacy UE. Then, we have below comments/suggestions: 1. clarify definition of "anchor cell" and make it clear whether it is intra-frequency or inter-frequency, and impact to legacy UE depending on clarification of our question 3) 2. For "RAN2 impact": a) what is UE capability impact? Note that if it is IDLE/INACTIVE UE, we don't have capability signalling b) what is "extended SIB" is not clear. suggest to change to "SSB/SIB mechanism for anchor cell" c) what is RACH impact is not clear. Does it mean UE sends preamble in anchor cell and followed steps in other cell, or the UE performs cell search/sync in anchor cell and RACH in other cell? d) do we allow paging in SSB/SIB-less cell or only in anchor cell? d) how to acquire timing for SSB-less cell is missed. e) It seems RRC connection procedure needs some spec changes because this solution basically requires camped cell is different from cell performing data transmission (i.e. RRC connected cell). |
| Fraunhofer | For us it is not clear why Solution 4 is conflated with Solution 1. In our understanding Solutions 1, 2 and 3 are all ways to reduce always-on signals. Reducing such always-on signals could be done either: - In a non-backward compatible way on a single carrier, or - In a multi-carrier scenario, one carrier (anchor) is kept backward compatible whereas other carriers can be relieved of having too much always-on signals. This is exactly what Solution 4 is about in our opinion. Therefore, Solution 4 should be described separately, as a technique which enhances solutions 1, 2 or 3 for the multi-carrier case, making it more practical because legacy supported can be defined. With that separate description it will be clear that “No impact, legacy UEs can access from anchor cell” refers to solution 4, not to 1 |
| CATT | Some NES cells may transmit SSB but not transmit SIB, so maybe the description could be reworded to "SSB and/or SIB" |
| Ericsson | - The current terminology “reduced time domain symbols” may be a bit ambiguous because it may be misinterpreted as the reduction of the symbol length. We suggest changing it to “reduction of symbols in time domain”. - For the RAN2 impact, we understand the UE capability aspect may anyway be applicable to all of the solutions listed here and it is a discussion subject we usually take later once the feature is specified. Therefore, we suggest removing “UE capability” from “RAN2 impact” for now. - We agree with Apple that it would be good to clarify whether the solution is meant for an inter or intra frequency case, or for both. |

2)  Partial/simplified SSB, MIB-less:

| Introduction | Simplified SSB without MIB (MIB is transmitted on anchor cell), or a newly defined RS. |
| --- | --- |
| Scenario | Multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols |
| Impact to legacy UEs | No impact, legacy UEs can access from the anchor cell. |
| UE assistance needed | No |
| RAN2 impact | Initial access, measurement, etc |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | We are confused by this solution: 1. Does "Simplified SSB without MIB" means MIB and SSB are sent in different cell? Then, we don't think legacy UE can live with it. 2. Some info of MIB is sent implicitly sent in SSB (e.g. 4bit LSB of SFN, some SSB index are implicitly sent via DMRS sequence of SSB). Then, "MIB" here means complete MIB with these implicitly sent bits or same as legacy MIB? 3. We are not sure "a newly defined RS" is to replace simplified SSB or MIB? For RAN2 impacts, we have similar comments of solution 1). The current description is not clear. |
| Fraunhofer | As we described on previous answer, “No impact, legacy UEs can access from the anchor cell.”, refers to solution 4 and not to 1 . To understand impact and the proposal we should also describe which functionalities a “partial/simplified SSB” supports. Initial access? RRM? RLM/BFD? Or are different combinations at the discussion table? |
| Ericsson | - The current terminology “reduced time domain symbols” may be a bit ambiguous because it may be misinterpreted as the reduction of the symbol length. We suggest changing it to “reduction of symbols in time domain”. - Since MIB is transmitted on the anchor cell, the same reasoning as in “SSB/SIB-less” case applies for NES gain, i.e., we could add under NES gain “Possibly increased power consumption for anchor cell when MIB is transmitted on the anchor cell only.”. |

In case some technique related to solution 1 & 4 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 2: Increase of SSB/SIB1 periodicity

Solution statement:

| Introduction | Longer SSB/SIB1 periodicity |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols |
| Impact to legacy UEs | Longer access delay |
| UE assistance needed | UE preferred SSB configuration |
| RAN2 impact | SSB/SIB1 periodicity is more of network implementation if no bigger periodicity is introduced. Other impacts depend on the solution details, e.g, to introduce a second periodicity (NES-specific periodicity) |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | 1. why "connected UE" needs to support this feature? Note in legacy system, connected UE typically acquires updated SIB via dedicated signaling. 2. For "Impacts to legacy UEs", we think besides "longer access delay", some other impacts are missed: a) Some legacy UE may fail in access because it is specified that the UE can assume 20ms SSB periodicity in initial access (Clause 4 of TS 38.213). Then based on it, some UE implementation always performs SSB combining per 20ms. These UEs may fail in initial access. b) Current RAN4 measurement requirement (also including RLM/BFR) depends on SSB periodicity. If longer SSB periodicity is used, legacy UE may not satisfy the corresponding requirements. For RAN2 impacts: 1) we think impact to IDLE and CONNECTED measurement is missed due to longer periodicity of SSB as some RRM requirement in RAN4 depends on SSB periodicity 2) we think impact to cell selection / reselection is missed resulted from measurement impacts. 3) we think some other RAN4 requirement impacts need further check (e.g. legacy RAN4 requirements of measurement in DRX and Scell activation/deactivation depend on SSB periodicity) |
| Fraunhofer | We think that “NES gain” as “Reduced time domain symbols” does not describe accurately the gains of this method. The main advantage of this increased periodicity is to enable the gNBs to reach deeper sleep states, which should provide energy savings far beyond just “Reduced time domain symbols”. From a signalling perspective, increasing the SSB periodicity is already supported in a legacy way (up to 160ms). However, SSB periods above 20ms does not come without impact. The most crucial, also stated by Apple, is the assumption of 20ms periodicity for initial access leading to delayed or even failed initial access. In addition to that, we have to analyze: 1) RRM impact, 2) RLM/BFD impact, 3) UE power consumption impact. Mitigating such impacts may be crucial to make Solution 2 practical. (see e.g. listed proposals below) |
| Ericsson | - The current terminology “reduced time domain symbols” may be a bit ambiguous because it may be misinterpreted as the reduction of the symbol length. We suggest changing it to “reduction of symbols in time domain”. - The current wording “UE assistance needed” seems to hint that what we are evaluating is whether UE assistance from the UE is required for the solution to work. It is not required for this case. Therefore, maybe a more appropriate wording is to use “UE assistance information candidates” instead of “UE assistance needed”. Note that this comment is applicable to all solutions. |

In case some technique related to solution 2 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

| Fraunhofer | Pre-paging dense SSB (combined with increased SSB period) - R2-2207960 |
| --- | --- |
| Introduction | SSB burst period is increased, but just prior to paging denser synchronization is sent (e.g. multiple SSBs per burst on the same beam) |
| Scenario | Single-carrier, multi-carrier; UEs in all states |
| NES gain | Increased period can be used more often, without affecting paging performance or UE power consumption |
| Impact to legacy UEs | No legacy support |
| UE assistance needed | None |
| RAN2 impact | SSB timing and information |

| Fraunhofer | Cell Classification - R2-2207960 |
| --- | --- |
| Introduction | The UEs classify different cells regarding RRM relevancy (preferably in a way aligned with RRM relaxation) |
| Scenario | UEs in connected mode |
| NES gain | The network can precisely know which cells are most relevant to the UE mobility. Therefore, the network can switch more cells to NES mode. |
| Impact to legacy UEs | No legacy support |
| UE assistance needed | Cell classification (RRM relevancy) |
| RAN2 impact | Signaling |

## Solution 3 & 7: On-demand SSB/SIB1 & Any Cell activation/re-activation or UE wake up request signal (connected/idle)

Solution statement:

On-demand SSB/SIB1, triggered by WUS

| Introduction | Cells in NES state only transmit discovery signals (DRS), UE uses wake-up signals (WUS) to trigger the transmission of SSB/SIB1 (FFS enhancements to other SIBs and MIB) |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols |
| Impact to legacy UEs | Legacy UEs can only access the cell after it is wakened. |
| UE assistance needed | No (if wake-up signal is not considered as “assistance information”) |
| RAN2 impact | Informing UEs of the DRS/WUS configuration; Procedures related to DRS (measurement, cell selection/reselection etc); Procedures related to WUS (triggering condition for sending WUS etc). |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | 1. For scenario of "multi-carrier", it is not clear how it works. Does it mean WUS can trigger cross-carrier activation? or DRS can indicate cross-carrier discovery? or SSB and/or SIB1 can be sent in cross-carrier way? 2. For impacts to legacy UE, it should at least include measurement, cell selection/reselection, RACH (as RA occasion is associated to selected SSB). For RAN2 impacts: 1. how to acquire timing is missed 2. impact to RACH is missed, as RA occasion is associated to selected SSB 3. impact to paging is missed: whether paging can be sent when SSB is not sent? 4. for "procedures related to WUS", we think UE MAC behaviour to send WUS and UE MAC behaviour after sending WUS (e.g. monitoring gNB response similar to RACH), and retransmission behaviour are missed. |
| Fraunhofer | We think that “NES gain” as “Reduced time domain symbols” does not describe accurately the gains of this method (Solution 3). The main advantage of on-demand SSB is to enable the gNBs to reach deeper sleep states, which should provide energy savings far beyond just “Reduced time domain symbols”. It is clear that Solution 7 is essential to implement Solution 3. Nonetheless, Solution 7 also can be used with other methods. For example, some of the proposals in RAN1 and RAN2 involve sending the wake-up signal to change SSB periodicity (related to solution 2). Therefore, Solution 7 also warrants a separate description (independent of Solution 3) |
| CATT (R2-2207511) | Autonomous cell activation/deactivation triggering in the Connected UE based on conditions unambiguously known at both the UE and gNB. Examples are the implicit secondary cell activation in a CA duplication configuration, when duplication is activated, or PUCCH SCell/sSCell autonomous activation/deactivation based on UCI mapping onto it. This saves the explicit activation/deactivation commands which both consume gNB power and involve some delay in the cell activation/deactivation. |
| Ericsson | - The current terminology “reduced time domain symbols” may be a bit ambiguous because it may be misinterpreted as the reduction of the symbol length. We suggest changing it to “reduction of symbols in time domain”. - Regarding the “Impact on legacy UEs”, we think that one could add the following as well. If legacy UEs are camping on this cell without gNB knowledge, there will be impact when the gNB turns the cell off. It will sort itself out eventually, e.g., UE will interpret it as lost coverage and go to the other cell. However, if this happens too often there will be impact. |

In case some technique related to solution 3 & 7 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| CATT (R2-2207511) | On-demand SIB acquisition from another UE via PC5-S connection |
| --- | --- |
| Introduction | Idle/Inactive UE reselecting a cell first checks (via PC5-S connection, leveraging sidelink relay discovery) if other UEs have the needed SIBs, and acquire those from such UEs. |
| Scenario | Any configuration (single cell, CA, DC). Idle/Inactive states. |
| NES gain | Reduced transmission of on-demand SIBs by the network, which can be numerous considering high-density cells where the majority of UEs moving in and out are in idle/inactive. And in practice, a UE reselecting a cell will typically request the missing SIBs to that cell, although many other UEs in proximity likely already have acquired such SIBs. |
| Impact to legacy UEs | None |
| UE assistance needed | No |
| RAN2 impact | Sidelink relay discovery procedure is upgraded to signal the stored SIBs of relay UE in sidelink discovery message, and the remote UE can select relay UE which has stored its interested SIBs to establish the PC5-S connection. Note: 1) Legacy sidelink relay procedure already supports transferring SIBs to another UE 2) Relay UE does not need to acquire the requested SIBs from the gNB if it already has them 3) Requesting UE does not need to first acquire SIB12 to check if gNB supports SL relay, since gNB is not involved in the operation. 4) This solution requires no additional RS and has no RAN1 impact. |

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 5: Handover/Fast PCell change for NES

Solution statement:

- Group HO/CHO

| Introduction | Pre-configure the candidate target cell(s) to the UEs, and trigger the HO/CHO with group-common signalling. |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in connected state |
| NES gain | Reduced HO commands |
| Impact to legacy UEs | Not applicable to legacy UEs |
| UE assistance needed | No |
| RAN2 impact | Details of the group-common HO/CHO signalling, details for pre-configuration of candidate cells, etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | We are not sure the intention of "pre-configuration of candidate cells". Is it for CHO only or traditional HO as well? At least for traditional HO, we don't understand why pre-configuration is needed as only one target cell is selected in traditional HO. For CHO, why the legacy candidate cell configuration (i.e. included in CHO command) can't be reused? For RAN2 impacts: 1. we think UE behaviour upon reception of group-common signaling is missed (e.g. how to handle T304 timer, whether/how to send response group-common signaling) 2. we think how UE updates security upon reception of group-common signaling has RAN2 impact and should be list. |

Group configuration update

For group configuration update, the solution is quite similar to the resource adaptation (Solution 6), which also discusses “triggering a configuration change using group-common signalling”. So no duplicated discussion will happen here.

In case some technique related to solution 5 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 6: Resource adaptation (frequency and time domain)

Solution statement:

- Resource adaptation (e.g. 2-step approach, combined with NW state transition)

| Introduction | Pre-configure the UEs using dedicated signalling, and trigger a configuration change or UE behaviour change using group-common signalling (the triggering is based on NW state transition between NES state and non-NES state). |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in connected state |
| NES gain | Reduced signalling and faster NW state transition. |
| Impact to legacy UEs | Not applicable to legacy UEs |
| UE assistance needed | No |
| RAN2 impact | Details of pre-configuration signalling and group common signalling, etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | 1. what is "pre-configure the UEs" is not clear. In our understanding, it means preconfigure UE group(s) and related to group-common configuration. Please proponent confirm. And we suggest to make it clear if proponent can confirm. 2. "Impact to legacy UEs", we think it has legacy impacts because NW NES state are adapted which will impact legacy UE performs (e.g. if SSB periodicity is changed via NW NES state changing, legacy UE is not aware of it but it will have impacts on at least RRM/RLM performance of the legacy UE.) For RAN2 impacts: 1. UE behaviour upon reception of the group common signling is missed. 2. We suggest to add "legacy UE impacts due to not aware of NW NES state change". |
| Fraunhofer | What is being pre-configured? We agree with Apple that this is not clear. |
| Ericsson | - It is not clear what is meant by “1) Resource adaptation (e.g. 2-step approach, combined with NW state transition)”. Is the proposed solution applicable to both frequency and time domain resources? It may be good to clarify this in the description in the table. |

- Cell DTX/DRX

| Introduction | Configure DRX in a UE-group or cell-specific manner, so that DTX at the gNB can be applied and aligned. |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in connected state |
| NES gain | Reduce the always-on transmission/monitoring |
| Impact to legacy UEs | Current spec already allows DTX/DRX, where legacy UEs can only be configured by per-UE signalling. |
| UE assistance needed | Traffic characteristic/status etc. More information can be provided by proponents. |
| RAN2 impact | Current spec already allows configuring same DRX configuration for UEs. FFS on additional enhancements. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | 1. For scenario, we don't think Cell DRX/DTX is applied to UE in connected state. In current RAN1 discussion, there are multiple proposals on UE in IDLE/INACTIVE state. Actually, if the DRX/DTX pattern is cell specific, it will impact IDLE / INACTIVE UE. So, we suggest to change to "UEs in all states" 2. For UE assistance needed", we can add "preferred DRX/DTX pattern" similar to existing preferred UE CDRX pattern. For RAN2 impacts: 1. As we clarified in 1), we suggest to add "whether/how to apply to IDLE/INACTIVE UEs" 2. We suggest to add "UE behaviour during Cell DRX/DTX ON / OFF duration" 3. We suggest to add "NW expected behaviour / NES state during Cell DRX/DTX OFF duration" (although we don't usually specify NW behaviour, we should at least clarify what NW is expected to do during DRX/DTX OFF duration. Otherwise, it is difficulty to specify clear UE behaviour). |
| Ericsson | - It is not clear at the moment whether DTX/DRX would be at cell level or not, as also detailed in the introduction here. Therefore, in the name of the solution in bold we could say “Network DTX/DRX” instead of “Cell DTX/DRX”. - We understand the current specifications allow DRX but not necessarily DTX since the UE can send on the UL even if it’s supposed to be off according to DRX settings. Hence, it could be update to “Current spec already allows DRX”. -Related to the previous comment, we suggest having “Configuration of DTX/DRX pattern for UEs” instead of “Current spec already allows configuring same DRX configuration for UEs.” -The current wording (“UE assistance needed”) seems to hint that what we are evaluating is whether UE assistance from the UE is required for the solution to work. It is not required for this case. Therefore, maybe a more appropriate wording is to use “UE assistance information candidates” instead of “UE assistance needed”. Note that this comment is applicable to all solutions. - We suggest adding the “short-term” in front of “traffic characteristics/status” information in order to avoid the confusion with the average long-term traffic characteristics. -We would also like to point out that RAN1 is discussion DTRX for both idle and connected UEs, and hence we agree with Apple’s comment to replace “UEs in connected states” by “UEs in all states”. |

- On-demand measurement, adaptation of RS

| Introduction | UE is allowed to notify network its preference on reference signal pattern for measurements via assistance information. The Network can use MAC CE to activate / deactivate a different pattern of reference signals for measurements |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in Connected state |
| NES gain | NW can apply sparse reference signal transmission unless additional RS is requested by the UE |
| Impact to legacy UEs | Legacy UEs cannot request on-demand RS for measurements, so they may not get sufficient RS samples to satisfy the RAN4 measurement requirement when the network decides sparse reference signal transmission. |
| UE assistance needed | Preferred RS pattern |
| RAN2 impact | UE indication, and NW activation/deactivation of RS patterns etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Ericsson | - We understand “The Network can use MAC CE to activate / deactivate a different pattern of reference signals for measurements” is not essential for this solution and can be discussed at a later stage. Therefore, this aspect can be removed from the “introduction” description and from RAN2 impact (“NW activation/deactivation of RS patterns”). - The current wording “UE assistance needed” seems to hint that what we are evaluating is whether UE assistance from the UE is required for the solution to work. It is not required for this case. Therefore, maybe a more appropriate wording is to use “UE assistance information candidates” instead of “UE assistance needed”. Note that this comment is applicable to all solutions. |

- BWP adaptation

| Introduction | UEs can be configured with a cell-NES specific BWP, and use group common signalling to switch UEs to the cell-NES specific BWP |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in Connected state |
| NES gain | Reduced frequency-domain resources, and NW can use the same smaller transmission bandwidth to serve all UEs |
| Impact to legacy UEs | Legacy UEs don’t support the group signalling, but relying on the existing UE-dedicated BWP switching or timer. |
| UE assistance needed | To be provided by proponents |
| RAN2 impact | Group-common BWP switching signalling. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | We have two questions for clarification for proponents: 1. For scenario, if cell specific BWP, we are not whether it is also applied to IDLE/INACTIVE UE (i.e. the cell specific BWP is configured in SIB) 2. whether the cell specific BWP is always one of UE dedicated configured BWP, or it can be different. If it can be different, we may have extra RAN2 impacts. |
| CATT | About the RAN2 impact: maybe it is a RAN1 signalling/impact? |
| Ericsson | - To state “cell-NES specific BWP” seems to imply that a new BWP definition is required for this solution, which is not the case. We suggest changing it to “BWP that allows NES”. - Regarding NES gain, note that simply changing BWP configuration to a shorter BW is already possible. This solution is actually aiming at defining a group signaling for that legacy behaviour. Therefore, the new gain in this case is the same as listed for group signaling previously, i.e., “Reduced signalling” rather than “Reduced frequency-domain resources, and NW can use the same smaller transmission bandwidth to serve all UEs”. - The current wording (“UE assistance needed”) seems to hint that what we are evaluating is whether UE assistance from the UE is required for the solution to work. It is not required for this case. Therefore, maybe a more appropriate wording is to use “UE assistance information candidates” instead of “UE assistance needed”. Note that this comment is applicable to all solutions. |

In case some technique related to solution 6 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 8: Paging enhancements (includes paging-less solutions)

Solution statement:

Paging reception from a non-camped cell

| Introduction | UE camps on a cell (for Idle/Inactive), or is served by a cell (for Connected), but receives paging on another cell |
| --- | --- |
| Scenario | Multi-carrier; UEs in all states |
| NES gain | Reduced time domain symbols for the carrier where UE camps or is served |
| Impact to legacy UEs | No impact, the legacy UE can receive paging from the anchor cell |
| UE assistance needed | No |
| RAN2 impact | Indicating paging configuration to the UE, etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | 1. whether the anchor cell is always in inter-frequency or can be intra-frequency (compared with cell for data transmission). Note RF retuning timing pattern may be needed to be specified if inter-frequency to receive paging 2. For scenario, it is stated that this solution is only applied to multi-carrier. Maybe proponent can confirm whether it is correct. 3. Suggest to add definition of "anchor cell" For RAN2 impacts: 1. As we clarify, if anchor cell can be in different frequency, maybe we need to specify RF retuning gap for the UE to leave to anchor cell to receive paging. 2. We are not sure the turning off paging decision is made in gNB (i.e. camping cell receives paging message from AMF but it is allowed to not send paging) or CN (i.e., AMF only sends paging message to anchor cells). If it is made in CN, we will have SA2/CT1 impacts. |
| Ericsson | - The current terminology “reduced time domain symbols” may be a bit ambiguous because it may be misinterpreted as the reduction of the symbol length. We suggest changing it to “reduction of symbols in time domain”. - It is not clear here what is meant by “paging”. For example, is Public Warning Indication (Short message ETWS/CMAS) counted as paging? There could be different interpretations here if not clarified. |

In case some technique related to solution 8 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

## Solution 9: Cell selection/reselection (i.e. cell prioritization also including legacy UEs)

Solution statement:

| Introduction | NES cells can be de-prioritized during cell selection/reselection, optionally, UE is made aware of cell state (NES or non-NES). |
| --- | --- |
| Scenario | Single-carrier, multi-carrier; UEs in Idle/Inactive |
| NES gain | Reduced time domain symbols if the cell is in NES state |
| Impact to legacy UEs | - In case legacy mechanism (frequency priority, or adding frequency/cell-specific offsets) is used, there is no impact on legacy UEs - In case cell state (NES, or non-NES, or other states) is introduced, legacy UEs are not aware. |
| UE assistance needed | No |
| RAN2 impact | Cell selection/reselection enhancement etc. |

For the above technique, please comment if you think there is some major principle missing:

| Company | Comments |
| --- | --- |
| Apple | 1. For "introduction", we are confused by current description: a) It is mentioned "aware of cell state is optional": we are not sure how cell selection/reselection can be enhanced if UE is not aware of cell state. In our understanding, only when NES capable UE can identify NES cell, the cell selection/reselection enhancement is possible. b) It is mentioned "NES cells can be de-prioritized". However, we believe correct logic is that NES cell is prioritized for NES capable UE at least in cell reselection. For cell selection, maybe NES cell should be de-prioritized for NES capable UE for KPI of initial access latency consideration. But it can be further studied in RAN2. So, we suggest to change this part as below: NES cells can be prioritized or de-prioritized for NES capable UEs during cell selection/reselection, optionally, after NES capable UE is made aware of cell state (NES or non-NES). 2. For "NES gain": a) what is " Reduced time domain symbols " is not clear. We think it should be "Reduce time domain SSB symbols". b) we think main gain of cell reselection enhancement is to ensure legacy UE performance is not impacted. 3) For "Impact to legacy UEs", we think it has legacy impacts if NW NES state is not aware by legacy UE (e.g. if SSB periodicity is changed via NW NES state changing, legacy UE is not aware of it but it will have impacts on at least RRM/RLM performance of the legacy UE.) |
| Ericsson | - The current terminology “reduced time domain symbols” may be a bit ambiguous because it may be misinterpreted as the reduction of the symbol length. We suggest changing it to “reduction of symbols in time domain”. |

In case some technique related to solution 9 is submitted to RAN2 #119-e but not presented above, it can be listed here:

| #Proponent Company | #Technique |
| --- | --- |
| Introduction |  |
| Scenario |  |
| NES gain |  |
| Impact to legacy UEs |  |
| UE assistance needed |  |
| RAN2 impact |  |

# Conclusion

To be completed

# References

- R2-2208703, Report from UP, Small data, URLLC/IIoT, RACH indication, NWES and UAV, Session chair (InterDigital)