3GPP TSG-RAN WG2 Meeting #119bis-e	R2-22xxxxx

E-meeting, October 10 – 19, 2022

Agenda Item:	6.1.4

Source: 	OPPO

Title:  	Report of [Post119bis-e][606][MBS-R17] MAC CR review (OPPO)

Document for:	Discussion and decision

# Introduction

This document is to kick off the following email discussion:

- [Post119bis-e][606][MBS-R17] MAC CR review (OPPO)

Scope: Finalize the MAC CR according to the agreements from the meeting

Outcome: In-principle agreed MAC CR

Deadline: Oct 21st 1000 UTC (agreeable CR available)

Contact Information

| Company | (name) Email |
| --- | --- |
| OPPO | Shukun Wang (wangshukun@oppo.com) |

# Discussion

Based on the agreement below, there is no consensus on how to capture the agreement during offline discussion #603.

- Conditions “or when unicast DRX is configured” and “if multicast DRX is not configured (FFS: and unicast DRX is configured)” are added in subclause 5.7b, to start and stop –PTM timers. The FFS and the exact wording (e.g. a note vs a procedural change) to be resolved during MAC CR review.

- Conditions “or when multicast DRX is configured” and “if DRX is not configured (FFS: and multicast DRX is configured)” are added in subclause 5.7, to start and stop unicast timers. The FFS and the exact wording (e.g. a note vs a procedural change) to be resolved during MAC CR review.

Companies are requested to provide their comments on how to capture the agreement in MAC CR

Option 1: The CR is as the changes in current draft MAC CR on the server.

Option 2: The changes as below.

## 5.7b Discontinuous Reception (DRX) for MBS Multicast *****Text omitted***** When multicast DRX is configured for a G-RNTI or G-CS-RNTI or when unicast DRX is configured, the MAC entity shall for this G-RNTI or G-CS-RNTI: 1> if a MAC PDU is received in a configured downlink multicast assignment: 2> if HARQ feedback is enabled: 3> start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 2> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 1> if a drx-HARQ-RTT-TimerDL-PTM expires: 2> if the data of the corresponding HARQ process was not successfully decoded: 3> start the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL-PTM. 1> if a DRX Command MAC CE with DCI scrambled with a G-RNTI is received: 2> stop drx-onDurationTimerPTM of the DRX for this G-RNTI; 2> stop drx-InactivityTimerPTM of the DRX for this G-RNTI. 1> if [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM: 2> start drx-onDurationTimerPTM after drx-SlotOffsetPTM from the beginning of the subframe. 1> if the MAC entity is in Active Time for this G-RNTI or G-CS-RNTI; or 1> if multicast DRX is not configured and unicast DRX is configured: 2> monitor the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL multicast transmission: 3> if HARQ feedback is enabled: 4> start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 4> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. *****Text omitted*****

## 5.7 Discontinuous Reception (DRX) *****Text omitted***** When DRX is configured or when multicast DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment for unicast: 2> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 3> if the corresponding HARQ process is configured with HARQ feedback enabled: 4> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 4> start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 2> else: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. NOTE 1a: Void. NOTE 1b: Void. 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process; 2> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process. *****Text omitted***** 1> if a DRX group is in Active Time; or 1> if DRX is not configured and multicast DRX is configured: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 4> if the corresponding HARQ process is configured with HARQ feedback enabled: 5> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 5> start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> else: 4> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. NOTE 3: When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback. 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported; 3> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; *****Text omitted*****

Option 3: others.

Q1: Which option do companies prefer?

| Company | Option 1/2/others | Comments |
| --- | --- | --- |
| OPPO | Option 2 | The text is clear and easy to read. |

# Conclusions

Based on the discussion above, we propose: