3GPP TSG-RAN WG2#120	R2-221xxxx

Toulouse, France, 14th - 18th Nov., 2022

Title:	Reply LS on UL scenario of reactive RAN feedback for burst sending time adjustment

Response to:	R2-2211038/S2-2209879 LS on UL scenario of reactive RAN feedback for burst sending time adjustment

Release:	Rel-18

Work Item:	FS_5TRS_URLLC

Source:	RAN2

To:	SA2

Cc:	RAN3

Contact Person:

Name: Tao Cai

E-mail Address: tao.cai@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: NA

- 1. Overall Description:

RAN2 would like to thank SA2 for sharing the conclusion of KI#6 “Adapting downstream scheduling based on RAN feedback for low latency communication”, and asking the question on feasibility to extend the adaptation mechanism also to the UL case based on UE feedback to RAN using RRC signalling.

From RAN2 perspective, RAN2 thinks that it is possible to extend the adaptation mechanism also to the UL case based on UE feedback to RAN using UL signalling.

As the usefulness of the solution is up to SA2 to determine, RAN2 would like to provide some inputs about the feasibility of the solution to assist the evaluation:

- Considering that gNB will attempt to adjust the scheduling to reduce the time-offset between the data burst arrival and the transmission resources as much as possible, the adjustment by the application layer to change the data generation periodicity (to reduce the current offset) may interfere with the approach the gNB applies and that may result in overall longer latency.

- For some cases, UE implementation based adjustment solution could be considered, e.g., the UE can internally inform the Application running on the UE with the time of the grant occasion or time difference between packet reception and the grant occasion.

Besides, RAN2 understands that the usefulness and efficiency of this solution will depend on how dynamically the UE reported information changes as well as the delay requirement for providing such information.

Question:

- In this adaptation mechanism applied to the UL, how dynamically would the UE reported information change, and what would be the delay requirement for providing such information?

- 2. Actions:

To: SA2

ACTION: RAN2 respectfully asks SA2 to take the above information into account in the relevant work and provide answer to the question.

- 3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #121     			27 Feb  – 03 Mar  2023			                	Athens

TSG-RAN WG2 Meeting #121bis-e     			17 April – 26 April 2023							Online