3GPP TSG-RAN WG2 Meeting #121bis-e	R2-22xxxxx

e-Meeting, 17th April – 26th April 2023

Agenda Item:	8.11.2

Source:	ZTE

Title:	Report of [Post121][606][eMBS] Service continuity and notifications (ZTE)

Document for:	Discussion, Decision

# 1	Introduction

This document is the report of the following email discussion,

[Post121][606][eMBS] Service continuity and notifications (ZTE)

Scope: Based on the companies' contributions discuss:

    - Service continuity (frequency/cell prioritization, neighbor cell list etc.)

    - Notifications for session activation, deactivation etc. (e.g. group paging or MCCH change notification, "special" UEs handling etc.)

Outcome: Report

Deadline: 5th Apr. 23:59 UTC.

Please provide your comments before 5th Apr. 23:59 UTC.

The summary for discussion is organized as above scope per chair guide.

Service continuity in section 3. Other than the frequency/cell prioritization, neighbor cell list, this part also includes the analysis to some scenarios as well, to cover several outstanding UE behaviour/flow for service continuity.

Notification mechanism in section 4. This part includes whether and how to notify UE upon events like session state change, data availability and "special" UE handling.

Issues not covered, if found, please kindly add them to the list in section 5.

# 2 Contact information

Participants are encouraged to leave their contact information in the following table.

| Company | Contact info (name, email address) |
| --- | --- |
| NEC | Rao Shi, shi_rao@nec.cn |
| Ericsson | martin.van.der.zee@ericsson.com |
| Samsung | Vinay Kumar Shrivastava, shrivastava@samsung.com |
| MediaTek | Xiaonan Zhang(xiaonan.zhang@mediatek.com) |
| CATT | zhourui@catt.cn |
| Qualcomm | Umesh Phuyal (uphuyal@qti.qualcomm.com) |
| LGE | sangwon7.kim@lge.com |
| Nokia | Jarkko.t.koskela@nokia.com |
| TD Tech, Chengdu TD Tech | limei.wei@td-tech.com |
| Huawei, HiSilicon | xubin10@huawei.com |
| vivo | yitao.mo@vivo.com |
| Kyocera | Masato Fujishiro (masato.fujishiro.fj@kyocera.jp) |
| Lenovo | Mingzeng Dai, daimz4@lenovo.com |
| Sharp | Fangying Xiao (Fangying.xiao@cn.sharp-world.com) |
| Intel | Yujian Zhang (yujian.zhang@intel.com) |
| Xiaomi | Xiaofei Liu (liuxiaofei@xiaomi.com) |
| ITRI | Jung Mao (moumou3@itri.org.tw) |
| ZTE | QI Tao (qi.tao3@zte.com.cn) |
| Canon Research Centre France | Yacine EL KOLLI, yacine.elkolli@crf.canon.fr |
| CMCC | Xiaoman Liu (liuxiaoman@chinamobile.com ) |
| Spreadtrum | Lifeng.han@unisoc.com |
| Apple | Fangli XU (fangli_xu@apple.com) |

# 3 Service continuity

Agreements made so far that's related to service continuity.

RAN2#119-e:

- Multicast service continuity after cell reselection in RRC_INACTIVE state (i.e. without resuming RRC connection) will be supported (if the configuration of the new cell is available for the UE). FFS whether there are cases where the UE needs to resume the connection. FFS RAN3 impacts due to inter-gNB mobility. - Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration. - We will have a mixed approach and we start with the following: - 1. When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - 2. MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. - 3. We assume that the UE can only receive multicast service after it joined the session. - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

RAN2#120:

- We will have a mixed approach and we start with the following: 1. When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). 2. MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. 3. We assume that the UE can only receive multicast service after it joined the session. FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

RAN2#121:

- Serving cell will not provide the PTM configuration of neighbour cells from other gNBs. - FFS whether the network can provide PTM configuration for intra-gNB cells.

Service continuity in this section will be covering the issues on frequency/cell prioritization, neighbor cell list etc. Before that, we aim to confirm the scenarios brought up by companies. It should be noted that Access categories and RRC resume cause value were also mentioned in companies contributions. However it is suggested to firstly have a common understanding on the essential service continuity scenarios and related solutions first (e.g., when will UE trigger RRC connection resumption).

## 3.1 Scenarios

The following service continuity scenarios/solutions were proposed by companies for UE in RRC_INACTIVE [2, 4, 9, 11, 14-17, 19].

1. Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection. This is consistent with RAN2#121 agreement that new SIB/MCCH for multicast reception in RRC_INACITVE, and also earlier agreement that "MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB".

2. When a UE enters to a cell for which PTM configuration is not available in multicast MCCH, the UE may return to RRC_CONNECTED state for an active multicast session. This is also a natural continuation of RAN2#119-e agreement that "if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration."

3. UE is able to trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold. Companies are concerned that in RRC_INACTIVE the reception quality might not meet the QoS requirement and network is not aware. Therefore it is reasonable for UE to resume to RRC_CONNECTED to have the QoS requirement met. This is also related to previous RAN2 FFS on "if there are other cases when UE triggers resume."

### Q1: Companies are invited to provide their views on the following,

1. Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

2. When a UE enters to a cell for which PTM configuration is not available in multicast MCCH, the UE may return to RRC_CONNECTED state for an active multicast session.

3. UE is able to trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold.

| Company | List the supported bullet (acceptable/unacceptable) | Comments if any |
| --- | --- | --- |
| NEC | 1. acceptable 2. comment 3. acceptable | For 1 PTM configuration of neighbor cell provided by dedicated RRC is not needed if MCCH is introduced, also even though UE acquires the PTM configuration of neighbor cell, the valid is not guaranteed, so that UE anyway need to read MCCH for a new PTM configuration after cell reselection. For 2 The intention is no problem, but we are wondering whether UE can initiate RRCResumeRequest without entering RRC_CONNECTED to acquire the PTM configuration in this case. For 3 Agree. As Mcast scheduling is common for multiple UEs, NW can not ensure a RRC_INACTIVE UE’s channel state since there is no CSI feedback from such kind of UEs. UE should be responsible for its Mcast scheduling. |
| Ericsson | 1 and 2: comment 3: acceptable/addition | 1 and 2: In case mobility is based on cell re-selection there are issues to be resolved: - Congestion happens dynamically, on a cell level, and cannot be pre-planned - It is not efficient to enable multicast reception in RRC_INACTIVE in all cells of the gNB, when there is only congestion in one cell. And it is cumbersome to update the set of cells dynamically (e.g. update NCL info in the active set, and add SIB/MCCH in the new cells). - The Rel-17 broadcast frequency prioritization is not suitable to handle this dynamic congestion scenario. And in case there is congestion, we assume that the gNB will only release UEs to RRC_INACTIVE when there is no capacity left in RRC_CONNECTED on all carriers. In this Rel-18 use case there is a not a single “MBS frequency”. 3: The gNB may release stationary and UEs in good coverage with preference, but still the UE may roam into bad coverage, in which case it should resume. In addition to the QoS issue, we also think that mobility should be based on HO, i.e. the UE resumes when the quality is above the threshold and the UE initiates cell re-selection. |
| Samsung | 1. support 2. support 3. comment | For 2: We understand if the PTM configuration is not available in multicast MCCH, the cell is likely not supporting multicast (or specific session) in RRC_INACTIVE. It is therefore natural continuation as indicated by Rapporteur to resume RRC connection and move to RRC_CONNECTED For 3: We think the UEs which may face reception quality issue are more likely be the cell-edge UEs. For these UEs, cell reselection may be more relevant in order to continue multicast reception. Moreover, UE by its own may not know the congestion situation at network which may have, in the first place, caused the UE to be in RRC_INACTIVE. We may need to consider the extreme situation if all cell-edge UEs may attempt to move to RRC_CONNECTED due to threshold! To address the issue related to congestion raised by Ericsson, we think we should consider a cell level indication to UEs about congestion status that may help the mobility and state transition procedure. |
| MediaTek | Support scenario 1 ,3 Comment for 2 | Both scenarios 1,2,3 are valid. Scenario 1 and 2 are for mobility case, scenario 1 should be the baseline, and scenario 2 behavior should be supplementary when PTM configuration is not available from MCCH of the re-selected cell. However, based on previous discussion, the UE may not need to completely switch back to CONNECTED state to obtain the PTM configuration (e.g., by RRCResume-RRCRelease), so we think RAN2#119-e agreement is better for scenario 2: Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration. Scenario 3 is another issue when the reception quality is low in INACTIVE state, and it is natural that UE should switch back to CONNECTED for better QoS. Meanwhile, some cases of service continuity are not listed, e.g., when network update the configuration. In this case, UE may obtain PTM configuration from MCCH but no need to acquire from the new SIB. In fact, we prefer to discuss the following cases of service continuity separately to make it clearer. E.g., - Mobility - When network changes the configuration - Except for the two above (the reception quality is below a configured threshold) |
| CATT | 1:acceptable 2:comments 3:comments | 1: Since we have agreed to introduce MCCH for inactive multicast, it is straightforward to reuse the similar principle as R17 MBS broadcast. It is not necessary to provide the PTM configuration of neighboring cells to UE in source cell. 2: The question has dependency on the solution for session deactivation notification (as in Q9).In Q9, there is one option “Option 1. PTM config availability in MCCH.”, with this option, how can UE determine whether the session is active if PTM configuration is not available in multicast MCCH? 3: It is not essential but a optimization, can be discussed later |
| Qualcomm | Generally ok | We think the general intention of these bullets is fine. Specifics depend on conclusion of other discussion points below. |
| LGE | 1, 2 and 3: acceptable | 1: RAN2 already agreed serving cell will not provide the PTM configuration of neighbour cells from other gNBs, so UE should acquire new SIB and multicast MCCH to get PTM configuration after cell reselection in this case. However, it is FFS whether the network can provide PTM configuration for intra-gNB cells. If UE can get the PTM configuration in advance, the UE doesn’t need to acquire new SIB and MCCH. The FFS should be discussed in RAN3. 2: ‘may’ can be removed. There is no other way to keep receiving the multicast. 3: It is essential to guarantee the QoS requirements of the multicast. |
| Nokia | 1, 2, 3 (but to be handled separately) and 4 (missing scenario) | Regarding 1: This should be the baseline behavior. Additionally, we think both Multicast MCCH configuration (i.e., SIB) and PTM configuration of intra-gNB cells could be provided to the UE when released to RRC_INACTIVE. In that scenario, UE can immediately try to decode the multicast transmission when reselecting an intra-gNB cell, and in parallel receive periodic MCCH to see if that was updated etc.. Regarding 2: Yes – In order to avoid UE returning to RRC_CONNECTED state when the session is not active, UE should be informed of the session activation status. Regarding 3: We support to have something like this, but this does not seem to be service continuity issue though so maybe it should be discussed separately. We think we are missing a scenario from service continuity i.e., UE is receiving multicast in RRC_CONNECTED in the serving cell. A handover is triggered to target cell which provides the service in RRC_INACTIVE state. One should be able to send UE directly to RRC_INACTIVE during handover in order to avoid delay of first completing handover and then releasing UE to INACTIVE state. |
| TD Tech, Chengdu TD Tech | 1: Acceptable 2: Acceptable with comments 3: Acceptable | For 2, if it’s difficult to exchange the availability of a multicast session in RRC_INACTIVE state for inter-gNB case dynamically (for example, for a multicast session in RRC_INACITVE state in the source cell, the availability of the multicast session in a neighbor cell is not known to the source cell for inter-gNB case), UE enters into RRC_CONNECTED for the mobility for inter-gNB case. |
| Huawei, HiSilicon | 1. Support 2. Support with comments 3. Support | For 2, we tend to agree with NEC and MTK that the UE may not have to go into RRC_CONNECTED to get the PTM configuration. If the NW can provide multicast for RRC_INACTIVE, the NW should be able to use RRC release to configure the PTM configuration (already supported by signaling). There is no need to set a restriction that the UE must go into RRC_CONNECTED as the NW may still have to release the UE after that. 2. When a UE enters to a cell for which PTM configuration is not available in multicast MCCH, the UE should resume to get the PTM configuration for an active multicast session, e.g., by entering RRC_CONNECTED or via RRCRelease without entering RRC_CONNECTED. |
| vivo | Okay for 1 Comments for 2 and 3 | For option 2, the new cell that not providing the PTM configuration, in fact, may not support multicast service. Thus, even though the UE resumes the RRC connection, it is likely that service continuity cannot be guaranteed. Instead, we think should have been that the UE may request the unicast reception (i.e. triggering RRC resumption) before moving to that cell not providing the multicast PTM configuration for RRC INACTIVE state based on the NCL info. Then the NW by implementation does the service continuity (e.g. via separate multicast delivery or handover the UE to a cell that supports multicast reception in CONNECTED or INACTIVE). For option 3, we think BFD/RLM is not supported. In this sense, it is hard for UE to determine whether the reception quality is good or not. If this option is the majority view, we think the definition of “reception quality” should be clarified. |
| Kyocera | 1, 2 and 3: Acceptable | Regarding 1 and 2, we support these scenarios in general. However, we think it means MCCH can provide the initial PTM configuration. RAN2 agreed that “When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration.”, so we understand that RRC Release provides an initial PTM configuration to the UE, then MCCH is used if the PTM configuration needs to be updated. In case of cell reselection, the target cell may be also considered as the cell which needs to provide the initial PTM configuration. So, we wonder if RRC Release should be used in this case. So, we tend to agree with Huawei that RAN2 should consider the two-step PTM reconfiguration, i.e., RRC Release in response of RRC Resume Request. Regarding 3, we support this scenario. We think the threshold should be configured by the gNB, e.g., via RRC Release. It’s FFS what criteria is used, e.g., RSRP, BLER, etc. |
| Lenovo | 1. OK 2. OK with comments 3. OK | 2. we tend to agree with NEC, MTK and HW’s comments that the UE is not necessary to enter RRC_CONNECTED for PTM configuration. the network can directly provide the updated PTM configuration in the RRCRelease message without entering RRC_CONECTED state. |
| Sharp | 1: support 2 and 3: comments | For 2, as we already agreed that RRCRelease message can be used to carry the PTM configuration for multicast in RRC_INACTIVE. So UE just resume the RRC connection for new PTM configuration but whether UE needs to return to RRC_CONNECTED state is up to gNB. For 3, it is an optimization and can be discussed later |
| Intel | Support 1 and 2 Comments for 3 | For 1: this is aligned with previous RAN2 agreement: “MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB”. For 2: this is aligned with previous RAN2 agreement: “Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.” For 3: this is an optimization. For multicast service which can be received in RRC_INACTIVE, network needs to dimension suitable coverage to avoid RRC state transition which cause additional congestion. |
| Xiaomi | 1: Acceptable 2: Acceptable with comments 3: Comments | For 2, we share the same view with MTK that there is no need to restrict UE to enter the RRC CONNECTED state, we can stick on the previous agreement. For 3, we agree with Samsung that it is more relevant to the cell reselection, but as an optimization, it is not essential and can be discussed later. |
| ITRI | Support 1, 2 Comments for 2 and 3 | For 1, this is aligned with RAN2 agreement. For 2, UE just resume the RRC connection for new PTM configuration, and network could decide whether to return UE to RRC_CONNECTED state. For 3, we think this is a optimization, can be discussed later. |
| ZTE | OK with 1, 2 and 3 | Some refinements based on above suggestion needs to be considered. |
| Canon | 1 is acceptable with comment 2 is acceptable with comment, No strong view on 3 | Option 1 is acceptable if the content of MCCH is protected (e.g by scrambling) Option 2: PTM configuration may be available as pre-configuration (see Q5) |
| CMCC | - Acceptable - Comment Acceptable | For 1, we agreed that the neighbor cells’ PTM configuration is not provided to UEs and MCCH is introduced, it’s natural for UE to reuse the R17 broadcast way to acquire the PTM configurations. For 2, in our point of view, the previous agreements including two cases: the first is the session UE wants to receive in RRC_INACTIVE is not available while the gNB support MBS and UE can trigger resume procedure to acquire the PTM configuration for RRC_INACTIVE reception, while the other case is that the gNB is non-MBS supporting node, in this case, there’s no MCCH, therefore, UE resumes to RRC_CONNECTED for multicast data reception. For 3, it’s essential to ensure UE’s reception quality. |
| Spreadtrum | 1. Support 2. Support with comments 3. Support | For 2, we think the UE can avoid entering the RRC connected state only for the PTM configuration acquisition, e.g. SDT-like procedure can be used. For 3, we agree with CMCC that it’s essential for reception performance in UE. The details of threshold need to be clarified and the threshold should be configured by the gNB. For the mobility scenarios, we agree with Nokia. We missed the scenario: during the handover procedure, if the target gNB decides that UE will receive the Multicast session in RRC_INACTIVE in target cell, then the UE can be switched to RRC_INACTIVE state before or during the handover procedure. |
| Apple | Support 1,2 and 3 | For 1, It should be the baseline based on the assumption that UE cannot acquire the PTM configuration of neigbhor cell in advance. According to the agreements last meeting (see below), it seems still open if network can provide the neigbhor cell’s PTM configuration for intra-gNB case. - Serving cell will not provide the PTM configuration of neighbour cells from other gNBs. - FFS whether the network can provide PTM configuration for intra-gNB cells. For 2, We share other companies’view that UE may aquire the PTM configuration via RRCResume procedure but not enter RRC-CONNECTED state. So maybe the scenario 2 can be updated as below: 2. When a UE enters to a cell for which PTM configuration is not available in multicast MCCH, the UE may initiate RRC Resume procedure return to RRC_CONNECTED state for an active multicast session. In addition, if the the newly selected cell doesnot enable the INACTIVE multicast feature (e.g. not provide the new SIB and MCCH), UE should also initiate the RRC resume procedure for the multicast service continuity purpose, no matter of the multicast session is activated or not. |

## 3.2 Frequency/cell prioritization

Frequency based cell reselection is the very basic mechanism to enable service continuity for broadcast, that was defined in LTE and later adopted in MBS broadcast in Rel-17. Since UE is also able to receive MBS multicast in RRC_INACTIVE, companies suggest that such mechanism can be applied as well [4, 6, 14, 16, 18]. That is, it is beneficial if UE can be guided to frequencies or cells on which the multicast service may also be provided (for UE in RRC_INACTIVE).

There are concerns though: the way how multicast service for RRC_INACTIVE UEs is deployed shall be different from broadcast service, i.e., no per frequency deployment. Per frequency deployment may be usual for broadcast which is intended for larger area broadcasting, while multicast reception for UEs in RRC_INACTIVE may be across only a few cells where congestion is happening. Therefore, there is doubt whether the such mechanism can be reused [3]. Moreover, whether current USD are ready to be reused needs to be confirmed by other working group [14], and whether a new SIBx may be needed as the SIB21 equivalent, are undetermined.

### Q2: Whether a frequency prioritization/de-prioritization mechanism is needed for cell reselection for multicast reception in RRC_INACTIVE?

| Company | Yes or No | Comment if any |
| --- | --- | --- |
| NEC | Yes | As long as cell reselection mechanism is performed, frequency priority is anyway helpful. |
| Ericsson | See comment | The UE will not be happy if it is released to RRC_INACTIVE on F1 because there is congestion, but on F2 and F3 there is still connected mode capacity. We are not sure what a suitable solution is, but we think that the Rel-17 broadcast way is not suitable. Frequency prioritization, at least in Rel-17 broadcast, is used to start receiving a session, i.e. not to continue reception. What use case do we try to solve, i.e. during congestion the UE is released to a frequency, where the session will not be activated? |
| Samsung | Yes | It is preferable to have frequency based prioritization as well as de-prioritization (e.g. dispersing UEs from loaded frequency/cell) |
| MediaTek | Yes |  |
| CATT | Yes | We think the general principle is we need a solution to enable UE receiving multicast in INACTIVE to reselect to a multicast cell as much as possible. Even though multicast service is not deployed in a static large area as broadcast, but it is very likely the multicast service are only provide on cells of certain frequency at certain place, so multicast frequency prioritization may be not perfect but still makes sense to enable UE to reselect to a multicast cell as much as possible. |
| Qualcomm | Yes | We understand this question is about per frequency (and not per cell covered in Q3) |
| LGE | Yes | Though broadcast is not based on MBSFN, the frequency prioritization help UE move to a cell providing the same broadcast session. It would be also useful for multicast continuity in RRC_INACTIVE. |
| Nokia | Yes | Based on previous RAN2 agreement, a Rel-18 UE in RRC_INACTIVE state should be able to move without going to connected state among the cells in the RNA. RAN2 should define a mechanism to guide the UE to a frequency/cell that provides the multicast service for service continuity purposes, similar to Rel-17 broadcast frequency prioritization feature. Otherwise, the UE may end up camping in a cell that does not provide the multicast service, e.g., out of the service area. In our view, concerns are not relevant: There is no such requirement for broadcast to be deployed per frequency either.The goal is to be able to guide the RRC_INACTIVE/IDLE UEs to a cell where the session is provided rather than a cell that does not. Similar achievement is needed for multicast for UEs in RRC_INACTIVE. Note that [3] considers only public safety scenarios, which is not the only objective of WID. Rather than USD, it is about service announcement, if FSAI based mechanism is to be reused.(which is the easiest and leanest approach in our view). It could easily be introduced via SA2 – we would just include FSAI multicast information into SIB. |
| TD Tech, Chengdu TD Tech | Yes | A frequency prioritization/de-prioritization mechanism is optional for multicast sessions in RRC_INACTIVE state. In other words, a new SIB like SIB21 can be introduced for multicast reception in RRC_INACTIVE state. If the multicast sessions in the current cell are not provided on a same frequency, no SIB21-like SIB is used in the current cell. |
| Huawei, HiSilicon | Yes with comments | The frequency prioritization mechanism is beneficial for multicast reception in RRC_INACTVIE at least for the scenario where a multicast service is provided on the same frequency. But different from broadcast, there is no pre-planned area for multicast in RRC_INACTIVE, thus it is not possible to use FSAI based solution as MBS broadcast. We think dedicated frequency priority can be used as baseline for frequency prioritization if needed. |
| vivo | No | We assume the multicast case is different from the broadcast case. In R17, broadcast service(s) is usually deployed per frequency based, e.g. same broadcast service on the intra-frequency cells. Thus, in MBS USD and FSAI, there is some frequency information to help UE for cell reselection and frequency prioritization for MBS broadcast service continuity. However, for multicast service(s), there has never been a feature based on the same frequency deployment from R17, and also no specific USD and SIB for frequency information. As the Rel-18 multicast reception is inherited from Rel-17 multicast, we assume it is useless to introduce the frequency-based cell reselection mechanism for multicast reception in the RRC_INACTIVE state. |
| Kyocera | Yes | We think the existing frequency prioritization mechanism can be the baseline. We tend to share some companies’ views that this is not a perfect solution, so in case there is a critical issue in this solution, we’re open for further discussion. |
| Lenovo | Yes | We are open to discuss whether FSAI based solution is reused or not. |
| Sharp | Yes | Considering that multicast may be still active for UE in RRC_INACTIVE state, it is benefit to for UE to reselect to a cell/frequency providing the multicast service. |
| Intel | Yes | We think frequency prioritization mechanism (similar to what is defined for broadcast) is helpful for multicast reception in RRC_INACTIVE. |
| Xiaomi | Yes |  |
| ITRI | Yes |  |
| ZTE | Yes | how can network provide such frequency prioritization can be FFS. |
| Canon | No strong view |  |
| CMCC | Yes | We think it’s useful to have a frequency prioritization/de-prioritization mechanism to ensure UE to reselect a cell supporting MBS and ensure the service continuity without entering RRC_CONNECTED mode. |
| Spreadtrum | Yes | The R17 frequency prioritization mechanism can be as baseline. |
| Apple | Yes | Frequency prioritization is benefical for the multicast service continuity during INACTIVE mobility. |

A related issue is if multicast reception for UEs in RRC_CONNECTED might not be enough based on frequency prioritization/de-prioritization, whether other per cell based prioritization should be defined?

### Q3: Whether a mechanism should be defined to help UE to choose the right cell to camp on, i.e., per cell based prioritization in cell re-selection, to continue the multicast reception in RRC_INACTIVE?

| Company | Yes or No | Comment if any |
| --- | --- | --- |
| NEC | No | We prefer simply reusing frequency priority mechanism. Otherwise more specification impact is needed. Also legacy neighbor cell list (NCL) in the MCCH can provide the cell information as assistance information and it is enough. |
| Ericsson | No | The right cell to camp on is the strongest/best ranked cell on the frequency. And the UE does not prefer to select a cell where it can receive multicast in RRC_INACTIVE, but RRC_CONNECTED is preferred. |
| Samsung | No | We prefer frequency prioritization based mechanism. Agree with NEC that NCL can help UE about cell level information. |
| MediaTek | No | Shared the same view with NEC |
| CATT | No | See answer to Q2.and we understand there should be only on prioritization solution, so Q3 and Q2 are exclusive. |
| Qualcomm | No | Agree with companies’ comments above. Per frequency prioritization is useful, but per cell mechanism can be complicated. Existing Qoffset can be used, no overoptimization is needed (to keep it simple). |
| LGE | No |  |
| Nokia | No | UE should camp on best cell on frequency as has been done always in NR (and LTE). Otherwise once UE starts connection there will be unwanted interference to cell that is “closer” to the UE. |
| TD Tech, Chengdu TD Tech | No |  |
| Huawei, HiSilicon | No | Frequency prioritization mechanism is sufficient. |
| vivo | No | As usual, the UE should re-select the cell with the highest quality for link robustness. Besides, we assume the current redirectedCarrierInfo via RRC Release can be reused for Rel-18. Having all these, no further optimization is needed. |
| Kyocera | No | We agree with companies that the per-cell based prioritization makes the things complicated. |
| Lenovo | No | We tend to agree that frequency prioritization can be reused. |
| Sharp | No | Agree with comments from other companies that cell based prioritization is not needed. |
| Intel | No |  |
| Xiaomi |  | We agree that the best cell principle should be followed for cell reselection. As for the cell level cell reselection optimization for multicast reception in RRC_INACTIVE state, we share the same view with Ericsson that the existing Qoffset can be considered, no further optimization is needed. |
| ITRI | No |  |
| ZTE | see comments | but as QC mentioned, offset per cell can be re-used. |
| Canon | Yes | Providing load information to the UE so as to avoid selecting a cell that is overloaded |
| CMCC | No |  |
| Spreadtrum | No |  |
| Apple | No |  |

## 3.2 Neighbour cell list

MCCH in legacy system functions for both PTM configuration provisioning and service availability in cells inside a neighbouring cell list (NCL). Different from the frequency prioritization in above section, based on NCL UE can be aware of the service availability in one specific neighbouring cell, and then UE can decide whether to apply unicast bearer in the target cell.

It is agreed that a new MCCH (with new SIB) is to be defined, to support multicast reception in RRC_INACTIVE in RAN2#121. For multicast reception in RRC_INACTIVE, similar mechanism can be adopted, suggested by companies [3, 15, 19], e.g., UE resumes RRC connection immediately if service is not available in the re-selected cell if known by UE beforehand. In such case, there is no need for UE to monitor MCCH in the re-selected cell to be aware of service availability. There are concerns from [17] that the benefits to reduce the latency is quite limited though.

So the next question is whether to apply the similar NCL mechanism that was defined for broadcast, to multicast in Rel-18 as well.

### Q4: Whether the neighbor cell list mechanism for multicast reception in RRC_INACTIVE is needed, e.g., UE resumes RRC connection immediately if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell?

Please also note NCL here is not used for cell re-selection which is handled in Q3.

| Company | Yes or no | Comment if any |
| --- | --- | --- |
| NEC | No strong view | The intention of Cell priority (Q3) is to determine which cell UE need to re-select; The intention of NCL is that, after cell reselection, UE immediately initiate RRC connection resume. But still can be considered as an assistance information facilitating Cell reselection. We are open for NCL as this is a legacy mechanism can be reused. |
| Ericsson | No | It is complex for the NW to configure and the benefit is very low. Extreme congestion, where there are only mission critical UEs in connected mode, and some need to be released to RRC_INACTIVE to avoid service denial, are expected rare cases, and we do not see a need for this optimization. PS: there can also be cases where the session is provided on the neighbour cell, but the PTM configuration is different. This discussion is related to the discussion in which area the PTM configuration is valid. |
| Samsung | Yes | It is straight-forward to re-use the NCL approach of broadcast also for multicast reception in RRC_INACTIVE. As asked in the question, this is only about the multicast session availability on the neighboring cells and UE can resume RRC connection at earliest if session is not available on re-selected cell for seamless service continuity i.e., avoiding interruption due to time taken for reading multicast MCCH in the re-selected cell. |
| MediaTek | Yes, but | The signaling design for multicast NCL may be a little different since the multicast service are not open to all UEs (only authorized UEs). We can further discuss whether multicast NCL information can be included in multicast MCCH at service level only. |
| CATT | Yes | It is straightforward to reuse the mechanism in R17 broadcast as the similar motivation exists. But it is worth to mention that NCL is optional and if NCL is not present, UE still needs to read new multicast SIB/MCCH of target cell to determine whether need to resume the RRC connection. |
| Qualcomm | Yes | It is beneficial for the UE if it is possible to know whether service is available in the neighbor cell without having to attempt to read MCCH. |
| LGE |  | The NCL would be useful for multicast reception also, but such UE behavior doesn’t need to be specified and can be up to UE implementation. |
| Nokia | No | There is no mandate as such in the current specifications where the UE skips reading MCCH in the re-selected cell. In fact, the UE must read the SIB and MCCH in the re-selected cell. However, the NCL mechanism for broadcast is an optimization and there is also an open issue in Rel-17 about whether the UE requests unicast service in the source or target cell which needs to be resolved first. Our view is the UE can request the unicast transmission in the source cell before going into a neighbor cell as this minimizes the service interruption. |
| TD Tech, Chengdu TD Tech | Yes with comments | We think the NCL mechanism shall be enhanced to further indicate whether or not a same PTM configuration is applied in a neighbor cell. |
| Huawei, HiSilicon | Yes | Agree with Samsung and QC. A similar mechanism can be introduced for multicast in RRC_INACTVE for service continuity, which is supported for broadcast already. It is even more useful in multicast considering that the multicast UE is always required by AS layer itself to return RRC_CONNECTED in case PTM configuration is available (for MBS broadcast the UE may only trigger unicast in case it is required by application layer). |
| vivo | Yes | The network can optionally provide multicast NCL in Rel-18. |
| Kyocera | Yes | We think the NCL is useful for the UE, and it’s already introduced for MBS broadcast in Rel-17 (and even from LTE SC-PTM). It’s an optional information from the NW point of view, so we don’t see any reason not to have this mechanism for MBS broadcast in Rel-18. |
| Lenovo | Yes with comments | In Rel-17, the MBS-NeighbourCellList IE is introduced for indicating a list of neighbour cells where ongoing MBS sessions provided via broadcast MRB in the current cell are also provided. This allows the UE, e.g. to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. Different with broadcast, the multicast transmission status in a neighbour cell may include: - multicast session is not provided by the neighbour cell either by PTM transmission or PTP transmission, e.g., the neighbour cell is out of multicast area; - the multicast session is supported by the neighbour cell but PTM transmission has not been started e.g. it is the first UE moves to the cell for the multicast reception or only PTP transmission is used for a small group of UEs. For the first case, same with broadcast, the UE may need to request unicast reception of the multicast service before moving to the neighbour cell. For the second case, since the PTM configuration is not provided by MCCH, the UE may need prepare to resume to RRC_CONNECTED state when cell reselection to the neighbour cell. To support above both cases, NCL needs to be enhanced for support multicast reception in RRC_INACTIVE. |
| Sharp | Yes | the mechanism defined for broadcast in R17 can be reused. |
| Intel | Yes | It is beneficial to reuse NCL mechanism for multicast to minimize service interruption time. |
| Xiaomi | Yes |  |
| ITRI | Yes |  |
| ZTE | No strong view. | but we want to echo the concern from companies that, - (from Ericsson) the benefit may be marginal. the CP delay happens when UE tries to get PTM Config in MCCH first, and it depends on how SIB/MCCH is configured. The benefits are not as much as expected, maybe hundreds of milliseconds. - (from Nokia) that there were indeed different understanding in how to utilize NCL in broadcast. the legacy mechanism is vague and the motivation is not clear. therefore it is OK not to have such mechanism. |
| Canon | No strong view |  |
| CMCC | Yes | NCL is useful to reduce service interruption, and we can reuse the R17 broadcast way. |
| Spreadtrum | Yes | We think the NCL mechanism in R17 can be reused. |
| Apple | Yes for the first part before e.g. | Agree to have NCL as R17 broadcast design. But about the UE operation when UE reselects a cell in NCL, it’s no need for UE to always initiate the RRC resume procedure; UE can still receive multicast INACTIVE in this cell if the cell support the multicast in INACTIVE state. |

# 4 Notification mechanism

Agreements made so far (since we are going with a mixed solution, specific agreements/assumption on option1/2 only will not be considered).

RAN2#119-e:

- In Rel-18, multicast reception for UEs in INACTIVE supports at least the following scenarios, with the assumption that the UE already has a valid PTM configuration: - Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception. - Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session FFS for state changes, e.g. due to service being not provided in INACTIVE anymore etc.

RAN2#119bis-e:

For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2. Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS). As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification). If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways). Rel-17 mechanism (NAS-based indication) is applicable for multicast session release. FFS if any enhancement is needed. FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options) 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). - 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS).

RAN2#120:

- We will have a mixed approach and we start with the following: 1. When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). 2. MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. 3. We assume that the UE can only receive multicast service after it joined the session. FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

In Rel-17, one UE in non RRC_CONNENCTED state may be notified when the session is activated/released, or when multicast data is available to be transmitted. Upon such notification (i.e., group paging), UE resumes to RRC_CONNECTED state to receive the multicast data.

While in Rel-18, UE is also able to receive multicast data in RRC_INACTIVE, e.g., in case of large number of UEs and potential high level of network congestion. Therefore a preferred UE behaviour may be UE to start or stop monitoring multicast transmission accordingly without RRC connection resumption, if UE is already in RRC_INACITVE while being capable and allowed to receive multicast data in RRC_INACTIVE. There are possible session state change or its implications that may need to be notified to UE:

Session activation. Previous RAN2 discussion confirmed UE may be notified upon multicast session activation and continues to stay in RRC_INACTIVE for multicast data reception. Detailed notification mechanism is FFS;

Session deactivation. It was also agreed that UE may be notified when the multicast session is deactivated;

Session release. While Rel-17 mechanism (NAS-based indication) is applicable for multicast session release, it is FFS if any enhancement is needed.

Based on [25, 34], UE might not be aware of the difference between "session activation" and "data transmission resumed", or the difference between "session deactivation" and "temporary no data". In the following text, they are suggested to be handled together for simplicity. However whether this is a feasible way shall still be open, if there are different views.

Companies' inputs to RAN2#121 showed a trend or interest that upon such notifications from network a UE that is already in RRC_INACTIVE, it should stay in RRC_INACTIVE and start/stop monitor multicast service (or G-RNTI) accordingly. Below table shows the categorized events (e.g., session state change, data availability, among others), and the potential UE behaviour (e.g., RRC state transitioning, G-RNTI monitoring) based on companies' contribution. The design aims for one purpose: to enable UE to stay in RRC_INACTIVE, if possible. Consequently, mechanism shall be clear on how to transition UE to RRC_CONNECTED when network thinks it is needed, if legacy mechanism is enhanced.

Please note this table full of question mark is only for "Example UE behaviour upon various events including session state change" with confirmed and not confirmed issues, i.e., the table is not the final proposal, but only an input to the discussion itself.

Table 1. Example UE behaviour upon various events including session state change.

| Section | Events, e.g., multicast session states, or data transmission states change | UE behaviour |  |
| --- | --- | --- | --- |
|  |  | UE's RRC state | whether UE needs to monitor G-RNTI |
| 4.1 | Session activation (or data transmission resumed) | UE stays in RRC_INACTIVE (confirmed) | yes (confirmed) |
| 4.2 | Session deactivation (or temporary no data) | UE stays in RRC_INACTIVE? | no? |
| 4.3 | Session release | UE stays in RRC_INACTIVE? | no? |
| 4.4 | Network intends to resume UE's RRC connection, e.g., service being not provided in INACTIVE anymore. | UE resumes to RRC_CONNECTED | follow RRC_CONNECTED configuration |

Various notification mechanism were proposed by companies (with candidate solutions like, group paging, MCCH, or PTM configuration availability), on how to notify UE, as the continuation of discussion from RAN2#119bis-e in which a few options were proposed.

The discussion is organized in the following way that in corresponding sections (4.1 to 4.3),

1. The scenarios are to be confirmed first, e.g., should UE be kept in RRC_INACTIVE upon session release? Apparently companies have different views.

2. Notification mechanism is to be discussed for each supported scenario in later part of that section. It should be well known that the solutions from section 4.1 to 4.4 are actually coupled, e.g., in some of the proposed solutions group paging is enhanced such that RRC state/whether to monitor G-RNTI/session state are explicitly indicated. Such mechanisms work for more than one events, e.g., for both session activation/deactivation, etc.

Current discussion are organized to discuss them separately for clearer understanding although the solutions may overlap.

## 4.1 Session activation or data transmission resumed

According to the agreements in RAN2#119bis-e meeting, Rel-18 UE in RRC_INACTIVE UE can be informed when session is activated. And as a baseline, group paging can be used to inform Rel-18 UE about the session activation. Moreover, UE can receive the multicast session in RRC_INACTIVE upon session activation, and the detailed solution is FFS, with a few options on the table.

- Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS). - As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification). - FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options) 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS).

The first coming question is about the "special UE" handling from the latest SA2 progress. It is understood that 5GC's UE level MBS assistance information suggests that such UE is preferred to be handled differently. It might be a leading UE in a group, therefore the UE might need special treatment in network scheduling with less control/user plane latency, e.g., by keeping the UE in RRC_CONNECTED. However, such UE may be released to RRC_INACTIVE too, e.g., for an deactivated multicast session [35]. How to resume only such "special" UE while keeping other "normal" UEs in RRC_INACTIVE, is the issue companies are trying to resolve.

Companies suggest that UE needs a pre-configuration, e.g., a valid PTM configuration in dedicated RRC signaling [36] or an indication that allows UE [24] to stay in RRC_INACTIVE, and start multicast data reception upon session activation/data transmission resumed. Such PTM configuration is only for UE that is suitable to receive multicast data in RRC_INACTIVE, but not the ones indicated by 5GC's UE level MBS assistance information that reception in RRC_CONNECTED is suggested [36]. Therefore, this "special" UEs without such configuration will always resume to RRC_CONNECTED.

Meanwhile, [24, 35] suggests that UEs, e.g., the ones preferred by 5GC to be served in RRC_CONNECTED for a multicast session, can be configured when UEs are released to RRC_INACTIVE state, e.g., to always come to RRC_CONNECTED state in case of session activation.

Others think it might not be needed, as if the gNB wants to select a subset of UEs to perform the RRC state change, it can use some POs for the group paging. Furthermore, the gNB can also use the legacy paging to trigger the RRC state transition for a specific UE. No enhancement is needed for Rel-17 group paging [32].

Therefore we have the question as below.

### Q5: Rel-18 UE can stay in RRC_INACTIVE and start multicast data reception upon session activation/data transmission resumed only if there is a pre-configuration (e.g., valid PTM configuration or an indication that allows one UE to receive multicast data in RRC_INACTIVE.), such that the special UE indicated by 5GC, without such pre-configuration, always go to RRC_CONNECTED?

| Company | Yes or no | Comment if any |
| --- | --- | --- |
| NEC | Comment (YES only for PTM config) | This is involved with UE specific Mcast reception requirement, we think there is no need for an explicit indication that allows one UE to receive Mcast data in RRC_INACTIVE. The presence/absence of PTM configuration in RRCRelease can already implicitly indicate whether UE can receive Mcast in RRC_INACTIVE or not. |
| Ericsson | Yes only for PTM config and session activation | We do not think that a “special” UE would be released to RRC_INACTIVE when the session is de-activated to save power. The UE may experience delay to comeback, and should not be released. RAN2 has not agree that the UE will be notified when there is temporarily no data, and when there is new data again. We also think that the gNB should not be required to do so, i.e. this can be left to gNB implementation. |
| Samsung | Yes (only for PTM config) | Agree with NEC, presence/absence of PTM config in RRC Release with suspendConfig is sufficient indication for UE to continue the relevant multicast session in RRC_INACTIVE. In general, the session may be in activated or deactivated state while the UE is released to RRC_INACTIVE. We understand the main reason for the gNB is to alleviate congestion by reducing the number of RRC_CONNECTED UEs and UEs may be released irrespective of multicast session state. |
| MediaTek | Yes (only for PTM config) | Agree with Samsung and NEC. Meanwhile we want to clarify the valid PTM configuration is obtained via RRCRelease message (not via SIB-MCCH) |
| CATT | No | For valid PTM configuration(Ii.e. the one preconfigured via dedidated RRC signalling), according to the previous agreement it can be optionally provided via dedicated RRC signaling, but it is not mandatorily provided. For the indication that allows one UE to receive multicast data in RRC_INACTIVE, if it is about 5GC UE level MBS assistance information(i.e.UE is preferred to be kept in the RRC Connected state) ,that seems a upper layer information and optional and only be available for some UEs |
| Qualcomm | Yes, if (1) UE has already joined the session; and (2) there is valid PTM config; and (3) network doesn’t indicate the UE to go back to CONNECTED (or indicate to receive multicast in INACTIVE) | All 3 conditions need to satisfy (i.e., it is AND, not OR) |
| LGE | YES | When RRC connection is suspended, NW should indicate whether the UE is allowed to receive the multicast in RRC_INACTIVE or not. Though UE can acquire PTM configuration via MCCH, if NW doesn’t allow the UE to receive the multicast in RRC_INACTIVE, the UE should resume RRC connection upon session activation. UPDATE: Yes, such special UE would not be configured to receive the multicast in RRC_INACTIVE when suspending RRC connection, and the UE will resume RRC connection upon receiving the group paging as in R17. |
| Nokia | No | We do not really see motivation to require pre-configuration for UE to stay in RRC_INACTIVE. In the discussion above, PTM configuration is mentioned as an example pre-configuration information, but it is obvious that a UE must have valid PTM configuration for multicast reception in RRC_INACTIVE but we must have some other criteria in addition to having a valid PTM configuration for UE to decide whether it can stay in RRC_INACTIVE or moves to RRC_CONNECTED. Something with explicit network control. Also, whether to keep the UE in RRC_INACTIVE or not is a “cell-based” decision. A UE in RRC_INACTIVE state may reselect to another cell and that cell may decide differently than the source cell. Therefore, we should have explicit information (e.g. in paging message) to determine whether a UE stays in RRC_INACTIVE or comes back to RRC_CONNECTED. For some special UEs, such pre-configuration could make sense, as this is not a cell-level decision but common throughout the network for the UE. However, such pre-configuration should be something other than a valid PTM configuration, e.g., explicit signalling that UE always comes back to RRC_CONNECTED. Such a UE should always move to CONNECTED to receive multicast. [UPDATE Nokia_v2]: If UE is not indicated to move to RRC_CONNECTED (e.g. in paging) then it is OK for UE to stay in RRC_INACTIVE – of course UE needs to have PTM configuration to receive the multicast e.g. via RRCRelease or MCCH. For the special UE handling – Eithere they never released to INACTIVE but if that is allowed then we need some mechanism (maybe just a rule) that such UE will always move to CONNECTED to receive multicast. We wonder if we never allow special UE to be released to INACTIVE if that is really up to UE interest due to power saving. |
| TD Tech, Chengdu TD Tech |  | We think the availability of a preconfiguration is just one case. There are other cases. For example, a preconfiguration and an UE ID list are used together to decide which UEs can receive in RRC_INACTIVE state. |
| Huawei, HiSilicon | Yes (only for PTM config) | It is a simple way to indicate whether the UE is required to enter RRC_CONNECTED for multicast reception which is aligned with SA2 and RAN3 conclusion on the per UE MBS assistance information. Regarding the indication, it is a stage3 issue. Probably there is no need for an extra indication other than the valid PTM configuration (at least a list of TMGIs for inactive reception is required, the exact PTM configuration may be optional) for this purpose. Besides, the indication mechanism cannot always work as the UE may not be able to receive the multicast in RRC_INACTIVE without resume after mobility, even if the indication says so. |
| vivo | Yes | We fail to see the motivation behind this question. Based on the achieved agreement, we think the UE anyway has to be indicated with PTM configuration before doing the multicast reception in INACTIVE. Otherwise, how can the UE receive the multicast data in RRC INACTIVE state? In Rel-18, multicast reception for UEs in INACTIVE supports at least the following scenarios, with the assumption that the UE already has a valid PTM configuration: - - Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception. - - Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session |
| Kyocera | - | We share Vivo and other companies that the UE needs to have a valid PTM configuration if it stays in INACTIVE and receive the multicast session of interest. We agree with Q5 as a scenario, but it’s not all. For example, the NW may make more UEs to transition back to Connected, if the congestion has been gone when the multicast session is activated. So, we think more flexible mechanism for selective paging is needed. |
| Lenovo | Yes (only for PTM config) | The presence/absence of PTM configuration in RRCRelease is sufficient to indicate whether UE can receive mulitcast in RRC_INACTIVE or not. We do not think any additional indication for ‘special UE’ is needed. For example, the ‘special UE’ should not be released to RRC_INACTIVE state. |
| Sharp | Yes | We slightly prefer to rely on whether valid PTM configuration is provided in Rerelease message or not. |
| Intel | Yes (only for PTM configuration) | As long as UE has valid PTM configuration (via MCCH or dedicated RRC signalling), UE can receive multicast in RRC_INACTIVE. The PTM configuration itself is an implicit indication therefore explicit indication is not needed. |
| Xiaomi | Yes | When UE is released to RRC_INACTIVE state, anyway gNB shall inform UE whether or which multicast session reception can be supported in RRC_INACTIVE state so that UE can decide how to receive the multicast session upon session activation or data transition resumed. |
| ITRI | Yes | We also share the same view with vivo. |
| ZTE | Yes (an indication) | we want to highlight two things that might be ignored by companies - the question is about the "special UE" that is preferred to be in RRC_CONNECTED. this question is related to Q6 in which the group paging or MCCH is for all interested UE, such special UE if released can not resume RRC connection, which may be problematic. - as CATT commented, the PTM config from RRC Release is only optional. Network might not be offering such config at all. Therefore such indication may be needed. |
| Canon | Yes | To avoid a massive MBS configuration at MBS session activation, it is proposed to pre-configure the UEs before the MBS session activation. |
| CMCC |  | Valid PTM configuration is essential for RRC_INACTIVE reception for UE, but whether it is pre-configured is up to the network as agreed in last RAN2 meeting. And we think the indication that allows one UE to receive multicast data in RRC_INACTIVE is optional. |
| Spreadtrum | Yes | The gNB can decide whether one UE which has valid PTM configuration needs to come back RRC connected state based on its policy, e.g. based on the cell load. So we think an indication is needed. |
| Apple | Yes (only for PTM config part) | Having available PTM configuration is the prerequisite for INACTIVE UE to receive multicast in INACTIVE state when the session is activated. UE may acquire the PTM configuration via RRCRelease message or via MCCH. But if INACTIVE UE has the available PTM configuration, it’s still possible for NW to page UE back to CONNECTED state for multicast reception. Therefore, NW should explicitly indicate the expected RRC state for multicast reception to UE when the multicast session is activated. |

On the "FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated", several options were proposed as summarized below:

Option 1. PTM config availability (e.g., the configuration was previously provided to UE via dedicated RRC signaling or via MCCH) to the UE [39]. It works with the benefits of less paging signaling overhead, however this option may require UE to consistently monitor MCCH and also cause frequent MCCH change with higher UE power consumption [25]. It may also ask for a different treatment between Rel-17 and Rel-18 UEs, e.g., Rel-18 UE to ignore group paging, which may result in potential compatibility issue [25].

Option 2. Group paging [24, 25, 28, 30, 32, 35-38, 40-44]. This is the most supported option, as RAN2 already agreed that group paging is the baseline for session activation, and it features less spec impact and shorter latency, as companies stated. One [38] suggest that if UE has pre-configured information of multicast reception state, it follows the indication in group paging but ignore the pre-configured one. And there are variations on whether and what enhancement is needed.

  - Enhanced group paging by adding different indication per session in group paging, e.g., when the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not [25, 30-32, 35, 36, 38, 40, 43]. This option enables gNB to control the inactive reception dynamically, and UE does not need to always monitor MCCH.

  - While some others suggest adding session state explicitly in group paging. [33, 44]

  - [37] suggests that in the group paging it is explicitly indicating UE whether to monitor G-RNTI, i.e., multicast data transmission.

  - While some suggest legacy group paging without enhancement could be used to indicate session activation, as long as UE is with valid pre-configuration [29] or UE can also start acquiring the PTM config upon Rel-17 group paging [26]. There are also concerns [24, 43]suggesting that a pre-configuration can not deal with the varying network condition with dynamic parameters, e.g., cell load, audience size and congestion level at the gNB, if a pre-configuration is required. However this is not always needed as in [26], the PTM config can be indicated in MCCH. This sub-option features the least spec impact without further enhancing the paging design.

Option 3. Indication in MCCH. MCCH is already agreed for PTM configuration update and mobility, it would be good to use MCCH for informing session activation [34], e..g, together with activation indication in the MCCH. However, relying on MCCH may lead to following issues: 1) UE has to always monitor MCCH [25], and 2) higher control plane latency. There are also different MCCH based solution though: notification of start/stop monitoring is added to the multicast MCCH for better reliability in case UE might miss the group paging since UE stays in RRC_INACTIVE [37].

Others, if needed.

### Q6: How to notify Rel-18 UE to stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon session activation/data transmission resumed?

Option 1. PTM config availability.

Option 2. Group paging. Please also indicate whether and what enhancement is needed.

Option 3. Enhanced MCCH. Please also indicate whether and what enhancement is needed.

Option 4. Explicit indication in RRC release with suspend config (the UE specific configuration doesn’t need to be changed when multicast is activated).

Others. Please elaborate the details in comment.

| Company | Which option(s) | Comment if any |
| --- | --- | --- |
| NEC | Option 2 | RRC_INACTIVE UE anyway need to monitor paging, also group paging is already a per-TMGI (session) configuration. When Mcast session activation, reusing group paging to indicate which RRC state UE will receive Mcast session is more straightforward, an example like each TMGI in the PagingGroupList can be corresponded to one paging cause – e.g., session in connected, session in inactive. Compared with MCCH, the indication about e.g., session in connected, session in inactive anyway need to be designed in MCCH or paging, but MCCH method cause UE continuously monitoring MCCH which bring more power consumption than paging. |
| Ericsson | Option 3 Option 2 when MCCH is not configured | The MCCH is optionally, when the PTM configuration does not change, and mobility is based on HO. In such case group paging is needed to indicate the session state change. But when the MCCH is configured, it does not make sense to use group paging and impact legacy UEs, i.e. the MCCH can be used. |
| Samsung | Option 2 | Paging is always read by UEs. Group paging for activation is existing Rel-17 mechanism and it is natural and quite simpler to extend this also for Rel-18 multicast reception. Enhancement required could be to indicate UE to stay in RRC_INACTIVE state, as Rel-17 Group paging implies transiting to RRC_CONNECTED by default. Adding another mechanism based on MCCH reading for this is not only redundant but is also complex and power inefficient. |
| MediaTek | Option 2 (without enhancement) | We think group paging can be used for session activate without enhancement. If we agree in Q5 that UE can stay in RRC INACTIVE and receive multicast when session activate only if it has valid configuration, the simplest and most compatible way is to reused group paging so that the UEs which have PTM configuration can receive multicast in RRC INACTIVE while Rel-17 UE/Rel-18 UE which does not have PTM configuration will resume RRC Connection. The variety of e.g., cell load or congestion level may not be an issue since the load of a cell is mainly determined by the UE in RRC CONNECTED state. Pre-configuring UEs is to make them not resume to RRC CONNECTED, which will not aggravate the cell load, and even the opposite. |
| CATT | Option 2 | It is straightforward to further enhance based on group paging as group paging is already used for session activation notification. And it enables gNB to control the inactive reception dynamically. And if MCCH-like solution is used, it will cause a lot of extra MCCH changes and increase the UE power consumption. And for the detailed solution, UE needs to be indicated whether it can receive the multicast session in RRC_INACTIVE or not as dynamic control is one of the major reason for choosing group paging solution. |
| Qualcomm | Option 2 | A single flag per TMGI should be enough in Rel-18 group paging to indicate whether all the UEs receiving the service with a given TMGI should stay in RRC_INACTIVE and continue to receive the service, or all the UEs receiving the service with a given TMGI should move to RRC_CONNECTED. However, to move a selected subset of UEs, the specific UEs need to be notified, which can be done using legacy paging. UE-specific paging (i.e. PagingRecordList) can be (re)used to move specific UE(s) to RRC_CONNECTED. This overrides the per-TMGI flag in the group paging for the specific UE(s). Specifically, regarding Option 1, there is additional issue on top of what moderator listed above with using PTM config availability as an implicit indication. Considering the discussion about temporary data stop/deactivation (in later sections) – this would imply that in deactivation case the PTM config must be made non-available. That is not a good approach. |
| LGE | Option 4 | RAN2 agreed the network can choose which UEs receive in RRC INACTIVE and which in RRC Connected and can move UEs between the states for Multicast service reception. It should be configured via dedicated signalling per UE whether UE can receive multicast in RRC_INACTIVE, and the UE specific configuration doesn’t need to be changed when multicast is activated. Since the PTM configuration can be provided via MCCH, all R18 UEs can acquire the TPM configuration. Even though UE acquires PTM configuration via MCCH, if the UE is not allowed to receive the multicast in RRC_INACTIVE, the UE should resume RRC connection upon session activation. |
| Nokia | 2, 3 | Regarding 1- Please see our answer above. PTM configuration in dedicated signalling is only for the current cell, but not the neighboring gNBs. It is FFS whether it could also comprise intra-gNB neighbor cells.As indicated in previous question the decision to move/provide service in INACTIVE is cell level decision and UE may reselect to another cell and that cell may decide differently than the source cell. Such one should be providing the information in the new cell to the UE e.g. via paging message whether UE should receive the service in INACTIVE or CONNECTED. A cell can provide the PTM configuration in MCCH already before multicast session activation. However, this does not mean that the service is active. It can only help UE to immediately receive multicast once receiving the group-paging. Regarding 3- a UE in RRC_INACTIVE state may be out of the service area of the multicast session initially, but then may go into a cell within the service area. This UE may have missed the group paging performed by the gNB (as above). Unless we would like to have some periodic group paging at the cell, we need an indication in SIB/MCCH that the session is active/deactive. As stated above, a valid PTM configuration alone cannot determine whether the session is active/deactive. As the UE would read SIB initially, it would be better if we provide the session activation status ALSO in the new SIB to be defined for multicast. Otherwise, the UE would need to wait longer to see if the session is active or deactive until it has received MCCH information. Note that if the status is only provided in SIB, but not MCCH, “deactivation” may become problematic. |
| TD Tech, Chengdu TD Tech | Option 2 | The information includes: (1) an indicator for multicast reception in RRC_INACTIVE state (2) UE ID list if needed (3) updated PTM configuration if needed |
| Huawei, HiSilicon | Option 2 | To us, the UE doesn’t need to monitor MCCH during session deactivation, which is beneficial for UE power saving. In this case, the MCCH based solution will lead to the following problems: 1) Upon reception of paging, the UE has to additionally acquire the MCCH before it decides whether to resume, leading to extra delay. 2) Even though the UEs receive paging in different POs, they have to wait for the same MCCH transmission occasion to check whether to resume. This might cause RACH congestion due to many UEs resuming simultaneously, if the PTM configuration is absent in MCCH for the concerned service. |
| vivo | Option 2 | Generally, there will be both Rel-17 and Rel-18 UEs receiving the multicast service. So, the Rel-17 group paging anyway will be there for session activation. Then, it seems a spontaneous logic to reuse it in Rel-18. Specifically, upon the reception of Rel-17 group paging, the Rel-18 UE with valid PTM configuration can start to monitor the GC-PDCCH. Based on the above understanding, Option 1 is not power efficient as the time duration between deactivation and reactivation can be a bit long. Option 3 seems redundant. |
| Kyocera | Option 2 | As companies commented above, the UE anyway needs to monitor paging and the UE does not need to acquire MCCH before session activation, so we think Option 2 is the efficient solution. Regarding the enhancement, we think the selective paging (i.e., to page a subset of UEs) should be ensured as RAN2 agreed. We also think the Rel-17 behaviour was that all the UE goes back to Connected when TMGI of interest was provided in the group paging. To avoid these, at least the Rel-18 group paging needs to a “flag” to indicate the UE is allowed to stay in INACTIVE, which is associated with each TMGI (or each UE-ID). |
| Lenovo | Option 2 | Option 1: Since we have already agreed a mixed solution, in which the UE will update the PTM configuration by MCCH e.g. after mobility or PTM configuration update. When the PTM configuration is updated by MCCH, the configuration is not valid anymore. Then Option 1 can not be work well in the mixed solution e.g. in case of mobility to another cell or PTM configuration update by MCCH. Option 2: In Rel-17, when receiving group paging, all UEs that are interested in the multicast session shall enter RRC_CONECTED state for the multicast reception. From backwards compatibility’s point view, the Rel-18 UE that are interested in the multicast session shall enter RRC_CONECTED state for the multicast reception when receiving the legacy group paging. In order to avoid all related UEs returning into RRC_CONNECTED state in Rel-18, the group paging needs to be enhanced anyway. Option 3: using MCCH for MC session activation has following drawback: - MCCH is usually common for all UEs. It needs additional efforts for informing partial of UEs or a specific UE to stay in RRC_INACTIVE state for MC session activation. - Using MCCH may cause RACH congestion due to many UEs acquires MCCH and may returns to RRC_CONNECTES state simultaneously. Therefore, the option 2 is preferred for informing MC session activation. |
| Sharp | Option 2 | When congestion is alleviate, gNB may switch all UEs to RRC_CONNECTED including the UE has valid PTM configuration. So, in addition to the session activation indication, a new indication in the group paging is needed to indicate which multicast session should be received in RRC_CONNECTED for all UEs joined the session. For example, |
| Intel | Option 1 | Option 1 has the benefit of less paging signalling overhead. Regarding the concerns on monitoring MCCH, Option 1 does not require additional monitoring of MCCH since MCCH is only acquired upon receiving change notification on PDCCH when MCCH content is changed. There is also no issue for back compatibility since Rel-17 UEs can only receive multicast in RRC_CONNECTED, while Rel-18 UEs only transits to RRC_CONNECTED when PTM configuration for the multicast service is not available. |
| Xiaomi | Option 2 | In R18, for UE who is configured with PTM configuration for multicast reception in RRC_INACTIVE state, when the multicast session is activated, UE can stay in RRC_INACTIVE and receive the multicast session. However, in some cases, from the NW perspective, when the multicast session is activated, for the UE configured with the multicast reception in INACTIVE state, NW may reconsider the UE RRC state for the multicast reception based on the current cell load. So, to provide the flexible network control, when the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not. |
| ZTE | Option 2 | the details on whether and how to enhance group paging can be next step. there might be a few alternatives: - if legacy paging, upon such paging, UE can be triggered to monitor MCCH and UE decides to go RRC_CONNECTED or stays in RRC_INACTIVE based on the availability of the PTM config of the interested multicast services. - if enhanced group paging, UE monitor G-RNTI or not/UE resumes RRC connection or not, can be pending on specific group paging design. |
| Canon | Option 2 | With enhancement of group paging to indicate session activation cause |
| CMCC | Option 2 | Group paging is agreed to notify the session activation, therefore, it’s natural to enhance group paging in indicate the reception RRC states. |
| Spreadtrum | Option 2 | A reception state indication can be contained in group paging message for a subgroup of UEs. |
| Apple | Option 2 | Network should indicate the expected RRC state per TMGI in the group paging for multicast activation notification. |

An immediate question is, upon session activation or data transmission resumed, if PTM configuration is not available to UE (e.g., either UE is not pre-configured, or there is no corresponding PTM config available in MCCH), does UE resume RRC connection? (this may be a mis-configuration from network or intended by network, but it would be good to clarify UE behaviour).

### Q7: Upon session activation/data transmission resumed, if PTM configuration is not available to UE, does UE need to resume RRC connection?

| Company | Yes or no | Comment if any |
| --- | --- | --- |
| NEC | YES but | As long as a UE is interested one Mcast session and this session is activated, UE can acquire the PTM configuration by RRCResumeRequest if no PTM configuration is available, but more detail like RRC resume without entering RRC_CONNECTED which is more efficient can be further considered. |
| Ericsson | Yes | If the UE is no longer interested in the multicast session, it needs to leave the session and notify the NW about this, i.e. this is Rel-17 behavior. |
| Samsung | Yes | If there is no PTM configuration available to UE, that implies the cell does not support the session in RRC_INACTIVE (with no config in MCCH) but the session is still available in RRC_CONNECTED (as group paging indicates activation). This is same as Rel-17 behaviour. |
| MediaTek | Yes, but | This behavior may be similar to the answer in Q1. I.e., UE may not need to fully resume to RRC CONNECTED state, but to obtain the PTM configuration by RRCResume—RRCRelease way (as we agreed) and then receive multicast service in RRC INACTIVE. This can help to alleviate the system load in congestion scenario. |
| CATT | Yes | But The Question is not clear. The intention should for multicast session that is allowed to be received in INACTIVE, but cannot read it from the question itself. If the session can only be received in CONNECTED state, of course UE in INACTIVE cannot get PTM configuration and need to resume RRC connection. |
| Qualcomm | Yes, see comment | The question should also include the case if UE has not joined the multicast session. Group paging can be used to indicate activation/data transmission resumed. Upon such indication, UEs in RRC_INACTIVE that have not joined the multicast session but interested to receive it need to move to RRC_CONNECTED (i.e. similar to Rel-17). |
| LGE |  | The presence or absence of the PTM configuration should not be used as a condition to decide whether to resume RRC connection, because all R18 UE can acquire the PTM configuration via MCCH. When multicast is activated, UE only needs to consider whether it is configured to receive the multicast in RRC_INACTIVE by network or not. |
| Nokia | Depends | If the UE receives group-paging without new indication telling UE to stay in RRC_INACTIVE, the UE would go to RRC_CONNECTED. If the UE receives group-paging with new indication telling UE to stay in RRC_INACTIVE, it would stay in RRC_INACTIVE. We do not believe that the gNB would send such indication and not include PTM configuration in MCCH. No need to specify such behavior. It can be left to UE implementation to reconnect. |
| TD Tech, Chengdu TD Tech | Yes |  |
| Huawei, HiSilicon | Yes with comments | Similar with our comments on Q1: UE should resume but not necessarily enter RRC_CONNECTED as long as it can get valid PTM configuration. |
| vivo | Yes with comments | If a valid configuration is not provided for the Rel-18 capable UE, then it should be regarded as a Rel-17 multicast capable UE. Basically, we think the Rel-18 capable UE shall also support Rel-17 multicast. |
| Kyocera | Yes | Assuming the group paging is used for session activation notification (in Q6), we don’t think there needs to modify Rel-17 UE behaviour. |
| Lenovo | Yes |  |
| Sharp | Yes | UE should resume the RRC Connection, but whether go to RRC_CONNECTED state is determined by gNB for that RRCRelease message can be used to deliver the PTM configuration. |
| Intel | Yes |  |
| Xiaomi | Yes |  |
| ITRI | Yes |  |
| ZTE | Yes |  |
| Canon | No | The UE may send a RRC Resume Request message and the network may respond with a RRC Release with suspendConfig message to maintain the UE in RRC_INACTIVE state |
| CMCC | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |

## 4.2 Session deactivation or temporary no data

According to the agreements in RAN2#119bis-e meeting, Rel-18 UE in RRC_INACTIVE UE may be informed when session is deactivated. This sounds reasonable to avoid unnecessary monitoring of the multicast transmission [35, 40]. How to notify and the behavior of RRC_INACTIVE UEs are FFS.

If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways).

Similarly, to avoid worsening network congestion, companies prefer UE to stay in RRC_INACTIVE upon session deactivation or temporary no data. Again, it is suggested to confirm upon session deactivation/temporary no data, a UE who is already in RRC_INACTIVE, can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

### Q8: For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session deactivation/temporary no data?

| Company | Yes or no | Comment if any |
| --- | --- | --- |
| NEC | Yes | It is for power saving, there is no need for UE to always monitor G-RNTI when session is (temporarily) deactivated in RRC_INACTIVE. |
| Ericsson | Yes for deactivation only | RAN2 did not agree that the gNB has to notify the UE in RRC_INACTIVE when there is temporary not data. However this can be left to gNB implementation. |
| Samsung | Yes | For power efficiency, UE need not monitor corresponding G-RNTI upon session deactivation, while in RRC_INACTIVE. There is also no reason to change RRC state when session is deactivated, so UE should continue in RRC_INACTIVE, until network explicitly asks UE to move to RRC_CONNECTED. Agree with Ericsson there may not be notification from gNB for temporary no data. However, we think UE needs to consider some limit to data inactivity. There is an existing mechanism for data inactivity for multicast reception in RRC_CONNECTED. UE transits to RRC_IDLE if multicast data is not received for a defined time (dataInactivityTimer). Same situation is relevant for multicast reception in RRC_INACTIVE. UE may not receive multicast data for long (e.g. due to data inactivity or session release not informed to RRC_INACTIVE UE etc.) and it may unnecessary monitor for multicast channel and consume power. Therefore, extending data inactivity mechanism of RRC_CONNECTED also to RRC_INACTIVE seems reasonable. |
| MediaTek | Yes | It is beneficial for both UE power saving and system load. |
| CATT | Yes | It is straightforward as we have agreed that the UE may be notified when the multicast session is deactivated, or such notification makes no sense. |
| Qualcomm | Yes |  |
| LGE | Yes |  |
| Nokia | Yes | MCCH can include a flag to indicate session deactivation. After seeing such flag, the UE can stop decoding G-RNTI until UE receives activation indication. |
| TD Tech, Chengdu TD Tech | Yes |  |
| Huawei, HiSilicon | Yes with comments | The UE doesn’t need to monitor MCCH-RNTI either during session deactivation, which is beneficial for UE power saving. There is no benefit monitoring the MCCH-RNTI during session deactivation. |
| vivo | Yes | Based on the agreement (even though “may” is used), we assume the UE can be notified of stopping multicast reception in RRC INACTIVE state. |
| Kyocera | Yes | It’s straightforward and beneficial for UE power saving. |
| Lenovo | Yes | It is beneficial from power saving point of view. |
| Sharp | Yes | It is beneficial for UE power saving which is the purpose of notifying UE about the session deactivation. |
| Intel | Yes |  |
| Xiaomi | Yes |  |
| ITRI | Yes |  |
| ZTE | Yes | it fits the general principle to minimize UE-network interaction. |
| Canon | Yes | At MBS deactivation. The UE may stop receiving multicast data, however, the session could be reactivated and later multicast data can be received. |
| CMCC | Yes | It helps to UE power saving. |
| Spreadtrum | Yes | It is beneficial for UE power saving. |
| Apple | Yes |  |

Views are even more diverse on how to notify UE to stay in RRC_INACTIVE and stop monitoring G-RNTI upon session deactivation deactivated/temporary no data.

Option 1. PTM config availability in MCCH. No explicit indication is needed, but notify UE via the removal of PTM configuration of the multicast service [39]. MCCH can be used to notify the UEs in RRC_INACTIVE about the multicast session similar as broadcast [36, 30] as MCCH is agreed to be used to notify PTM config change.

Option 2. Group paging [25, 26, 31, 37, 43, 44]. An indication in Paging message to indicate UE to start/stop monitoring G-RNTI upon session deactivation or temporarily no data [25, 37, 43]. While there are some others suggest adding session state explicitly in group paging [26, 31, 44]. It may also be good to have a unified solution for both session activation and deactivation to avoid the complexity [32]. [35] proposes to add a new group of identity indicating which multicast session is deactivated.

Option 3. Enhanced MCCH [28, 30, 37]. Notification of start/stop monitoring is added to the multicast MCCH for better reliability, i.e., UE might miss the group paging while network is not aware [37].

Option 4. A new MAC CE [26, 40]. Upon the reception of the deactivation MAC CE, the UE suspends the corresponding PTM configuration (e.g. stop GC-PDCCH monitoring). Companies' concern is, group paging might not be a suitable way to carry deactivation notification since there may be impacts on non-MBS or non-interested UEs to increase their probability and power consumption of checking with irrelevant paging messages [26]. The MAC CE method, compared to MCCH, features less delay, less impact on other UEs and more power-saving gain.

Option 5. DCI. It is stated that compared with MAC CE, DCI indication have lower latency and less overhead, which has been used for the broadcast MCCH change notification. Considering the resource efficiency and latency, DCI indication may be a good way for the notification of session deactivation for UEs receiving multicast in RRC INACTIVE [29]. After reception of such indication in DCI, UE stops monitoring G-RNTI and stays in RRC_INACTIVE when receiving the notification for session deactivation.

Others. Please elaborate in comments.

### Q9: How to notify Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session deactivation/temporary no data?

- Option 1. PTM config availability in MCCH.

- Option 2. Group paging. Please also indicate what enhancement is needed.

- Option 3. Enhanced MCCH. Please also indicate what enhancement is needed.

- Option 4. MAC CE. (MAC CE multiplexed with data? Please elaborate.)

- Option 5. DCI. (DCI associated to the PDCCH addressed to MCCH-RNTI? Please elaborate.)

- Others. Please elaborate the details in comment.

| Company | Which option(s) | Comment if any |
| --- | --- | --- |
| NEC | Option 2 | To our understanding, UE anyway need to monitor paging and group paging is already a per-TMGI(session) configuration. One simple solution can solve all the problems is that adding additional indication to each TMGI in the pagingGroupList, example like, usging paging cause (session activate in Connected / Inactive, session deactivate) to correspond to each TMGI. For option 3, note that we agree that MCCH can carry updated PTM configuration, but for notification, it is not efficient as UE will always (continuously) monitor MCCH which cause more power consumption than paging. In a word, enhanced group paging can notify UE of any session state with less spec impact. |
| Ericsson | Option 3 Option 2 when MCCH is not configured | PS: we were not sure why activation and deactivation is discussed separately, and why companies have different solutions for both cases. The MCCH is optionally, when the PTM configuration does not change, and mobility is based on HO. In such case group paging is needed to indicate the session state change. But when the MCCH is configured, it does not make sense to use group paging and impact legacy UEs, i.e. the MCCH can be used. |
| Samsung | Option 2 | Agree with NEC. Group Paging mechanism is already available for per-TMGI group notification and can be enhanced to also indicate the cause (e.g. session deactivation). On contrary, newer approaches for enhanced MCCH and MAC CE are too complex, involves large standards impact and also redundant. We prefer not to complicate with making 2 different approaches for a single purpose. |
| MediaTek | Option 3 | The MCCH can be enhanced to carry the information for session deactivation. UE may not need to always monitor MCCH, but notified by DCI that something is changed. Some options are also relate to the other email discussion which discussing the DCI format for multicast in INACTIVE. |
| CATT | Option 2 | We agree with the point from Ericsson that activation and deactivation should not be discussed separately. Group paging is already used for session activation notification, so it is natural to also use it for session deactivation notification. furthermore, as we mentioned in Q6, if MCCH-like solution is used, it will cause a lot of extra MCCH changes and increase the UE power consumption. |
| Qualcomm | Option 3 | While both group paging and MCCH-based options are possible to indicate deactivation of multicast session while UE is in RRC_INACTIVE. Since RAN2 previously agreed that MCCH is used to indicate PTM configuration change while the UE is in RRC_INACTIVE, similarly, deactivation of the session can be indicated by MCCH. |
| LGE | Option 2 | We also prefer to use the same message to notify the session activation and de-activation. |
| Nokia | 3 | Regarding 1-. A cell can provide the PTM configuration in MCCH already before multicast session activation. However, this does not mean that the service is active/deactive. It can only help UE to immediately receive multicast once receiving the group-paging. Regarding 2- No need to enhance the paging to indicate deactivation. The UE anyway reads MCCH, and deactivation is not as urgent as activation, so no paging is needed and MCCH can be relied on. Regarding 3- It is simple to include the deactivation status of the multicast session on MCCH. In addition, for a UE that comes newly to a cell in RRC_INACTIVE, it would be good to optionally be able to include deactivation status ALSO in SIB. Regarding 4/5- In our view, such options does not have any benefits compared to 3. Also, it is not clear yet which DCI is to be used for scheduling INACTIVE UEs. It is also not clear why there would be power saving gain compared to MCCH and why delay is critical for deactivation. |
| TD Tech, Chengdu TD Tech | Option 3 |  |
| Huawei, HiSilicon | Option 3 | Agree with QC and Nokia. The UE will anyway monitor MCCH-RNTI during an active session so the deactivation can be indicated via MCCH, which is similar to the R17 mechanism when the MBS broadcast stops. For option 2, to avoid the impacts to the legacy R17 UEs, a new TMGI list to deactivate MBS services has to be introduced in the paging message. This will further increase the load of paging which can be avoided by option3. Besides, the UE may miss this notification in paging message in some cases such as during cell reselection and will not know the session has been deactivated. |
| vivo | Option 4 | For Option 1/2/3/, after the reception of the last packet, the UE may still monitor the GC-PDCCH as the deactivation notification can only be sent in the next modification period/next available PO, incurring unnecessary power consumption. In this sense, we think the LTE MAC-CE based solution can be reused. Option 5 definitely brings RAN1 impacts, which should be avoided as NO TU is allocated for RAN1. |
| Kyocera | Option 4 | Regarding Options 1, 2 and 3, we agree with Vivo that there is a large time-lag between the last MTCH data transmission and the notification transmission. Also, Option 1 has RAN1 impact, which is not in the WID. In addition, SC-PTM Stop Indication MAC CE was specified in LTE SC-PTM, so Option 4 is the well-known solution. |
| Lenovo | Option 3 | Option 2: When receives the group paging with the indication, the rel-17 UE cannot understand the indication and will perform the legacy group paging i.e., returning into RRC_CONNECTED state. It will cause the Rel-17 returning RRC_CONNECTED state unnecessarily for MC session deactivation. Option 4&5: The UE may miss the MAC CE during cell reselection. If the UE misses the MAC CE, the UE shall treat the MC session as ‘activation’ and continues monitoring the MCCH and MTCH even the MC session is deactivated. Option 3: It causes more frequent MCCH change and increases UE power consumption, since the UE needs to acquire the MCCH for the MC session state changes for any ongoing multicast session. Considering the MC session deactivation is triggered not frequently, Option 3 is more acceptable than other two solutions, in which the additional MCCH change is not so frequently and thus the UE power consumption is acceptable |
| Sharp | Option 2 or option 4 | For option 2, a new indication is added in the paging message. For option 4, MAC CE multiplexed w/o data are fine. |
| Intel | Option 1 | For Rel-18 UE receiving multicast in RRC_INACTIVE, there is no need to explicitly indicate in AS layer regarding session deactivation. It can be up to gNB implementation e.g. to stop providing the relevant configuration of the multicast session in MCCH. |
| Xiaomi | Option 2 | The session state needs to be included in paging message so that UE can get aware of the paging cause and no need to enter connected state upon receiving group paging message according to legacy procedure. |
| ITRI | Option 3 | Including the deactivation status of the multicast session on MCCH is a simple way to indicate UE whether to stop monitoring. |
| ZTE | Option 1 / 2 | See our answer in Q6. - if session activation is done by legacy group paging, then PTM config availability in MCCH will work, e.g., UE stops monitoring G-RNTI and stays in RRC_INACTIVE. - if done by enhanced paging, it could work for both session activation and deactivation. |
| Canon | No strong view |  |
| CMCC | Option 3 | Both group paging and MCCH can be used for session deactivation notification. Since UE need to monitor MCCH when the session activated (notified by group paging) for potential MCCH change notification, therefore, it’s better to enhance MCCH for this purpose. |
| Spreadtrum | Option 2 | We prefer to use same message for multicast session activation and deactivation. |
| Apple | Option 2 | We prefer the same message for both activation and deactivation purpose. For option 3, we may need first check whether the MCCH scheme is always enabled in NW and UE side. If it’s possible for either UE or NW not to support it, then the solution 3 should be excluded. |

## 4.3 Session release

According to the agreements in RAN2#119bis-e meeting, Rel-17 mechanism is applicable for multicast session release, but whether any enhancement is needed is FFS.

Rel-17 mechanism (NAS-based indication) is applicable for multicast session release. FFS if any enhancement is needed.

In Rel-17, upon session release, Rel-17 UE in RRC_INACTIVE or RRC_IDLE shall be paged back to RRC_CONNECTED, e.g., to release PTM config, and finish the NAS procedure.

It was proposed that, to avoid aggravating the congestion of the cell, UE can also stay in the RRC_INACTIVE state and stop monitoring corresponding G-RNTI upon session release [28, 31, 43, 44]. Companies further suggest that, UE might not be required to transit to RRC_CONNECTED immediately, e.g. when a group paging with session release is received. For instance, if UE is already receiving another multicast session in RRC_INACTIVE or network is too congested; forcing all UEs at once to transit to RRC_CONNECTED and to immediately respond to group paging with session release is not desirable.

There are also proposal that suggesting session release in current SA2 procedure requires UE to resume RRC connection, therefore no enhancement is needed [32, 34, 39]. Others think the notification of session deactivation could work for session release, e.g., to stop monitoring the data transmission. UE just stops the G-RNTI monitoring, the rest is network implementation, e.g., network ignores the group paging intentionally when needed, or network delays the NAS message delivery until UE re-connects to RRC_CONNECTED [37, 40].

### Q10: For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release?

Note: whether there will be NAS layer interaction issues, is one of the possible concern, e.g., what happens if NAS message can not reach one UE, this might be out of RAN2 expertise but worth some attention.

| Company | Yes or no | Comment if any |
| --- | --- | --- |
| NEC | No strong view | In legacy, when session release happen, NW will use legacy unicast paging to explicitly release UE when UE enter RRC_CONNECTED, e.g., release MRB confg… However considering congestion situation, UE may not back to RRC_CONNECTED and just directly release the session, which is a good intention. But as rapporteur’s note, we are not sure whether this could impact NAS layer… |
| Ericsson | No | The NW is not required to inform the UE when the session is release via group paging, but it can inform the UE later when the is in RRC_CONNECTED, see 23.247 section 7.2.2: Alternatively, for UEs without activated UP, the SMF does not trigger message to the AMF, instead the SMF marks that the UE is to be informed of the MBS Session release. In this case, the SMF initiates PDU Session Modification to inform the UE of the MBS Session release at next UP activation of the associated PDU Session, if needed. |
| Samsung | Yes | Objectives should be two-fold for multicast session release: - All the UEs should not transition to RRC_CONNECTED at once due to session release - UEs should not monitor multicast channel further for the session which is already released. Therefore, we think UE can be notified about session release in group paging and UE may stay in RRC_INACTIVE with no need to indefinitely monitor for a released session and can complete the NAS signaling when it reconnects to RRC_CONNECTED. |
| MediaTek | No strong view | The intention is good, but it seems the benefit is small compared with its complexity, since session release may not happen very often. We may need a LS to check whether it is acceptable if we want to support this feature. |
| CATT | No | R17 group paging is sufficient to move UE to CONNECTED for session release,which was already supported in R17 MBS. And we agree with Ericsson that NW can chose to inform UE later if congestion exists. |
| Qualcomm | Yes | The question is not very clear. Agree with Ericsson’s comment but that would mean ‘yes’ to the question: one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release until the SMF initiates PDU session modification to inform the UE of the MBS session release (but not indefinitely). |
| LGE |  | Same view as Ericsson. |
| Nokia | No | If the UE is indicated that the session is deactive, the UE can immediately stop monitoring G-RNTI. Note that Rel-17 mechanism allows indication of session deactivation to the gNB by the core network, once release is triggered. Rel-17 specifications also allow the network/gNB to not page the UEs to ALWAYS come to CONNECTED as soon as release is triggered in the core network. Rather, network can wait that the UE comes back to RRC_CONNECTED for other purposes, and then the session would be released for the UE. We do not think that there are any drawbacks of this operation, which does not require any enhancements. |
| TD Tech, Chengdu TD Tech | Yes |  |
| Huawei, HiSilicon | No | We think Rel-17 mechanism should be reused: tither the NW invoke the UEs by group paging and send session release to the UE; or the group paging is not sent and the UE still monitors the G-RNTI in RRC_INACTIVE. There shouldn’t be a case where UE stay in RRC_INACTIVE and stops the G-RNTI monitoring. |
| vivo | No for enhancements | As per the agreement, the NW can page the UEs via paging message and then transit it into RRC CONNECTED, releasing the PTM configuration, same as the Rel-17 mechanism. |
| Kyocera | No | We think Rel-17 mechanism still works as Huawei and other companies pointed out. |
| Lenovo | No | We think a good NW implementation is to trigger the UE back to RRC_CONNECTED state and release the MRB. |
| Sharp | No | The issue has already been resolved in R17 and we do not think RAN level enhancement is needed. |
| Intel | See comments | This is related to NAS procedure. For session release, according to TS 23.247 clause 7.2.2.3, gNB transports NAS message PDU SESSION MODIFICATION COMMAND to UE. UE is paged by gNB and and responds with NAS message PDU SESSION MODIFICATION COMPLETE. If SDT is configured, UE can receive / transmit NAS messages in RRC_INACTIVE state; otherwise, UE needs to transit to RRC_CONNECTED to receive / transmit NAS messages. In summary, we don’t think enhancements in RAN for session release is needed. |
| Xiaomi | No | When the multicast session is release, in order to avoid the mismatch between the UE and CN, UE who is configured to receive the multicast session in RRC_INACTIVE state needs to be switched to RRC_CONNECTED state to have signaling exchange between UE and CN. If we want to support the case in Q10, as rapporteur pointed out, NAS layer interaction issues needs to be checked first with other WG. |
| ITRI | No | We share the same view with Intel that the enhancement may not be needed. |
| ZTE | Yes | Beneficial for easing network congestion. |
| Canon | No strong view |  |
| CMCC | Yes |  |
| Spreadtrum | No | We think Rel-17 mechanism can be reused and no enhancement in RAN is needed. |
| Apple | No | Same understanding as Intel that for session release NW will intiaite the NAS procedure and initaites the UE specific paging to trigger UE back to CONNECTED state. |

Therefore it is suggested that to explicitly indicating UE the session state (e.g., session released) in group paging, to align the session state between 5GC and UE without resumption of UE's RRC connection [28, 31, 44, 43].

This can also be achieved by explicitly indicate whether to monitor G-RNTI upon session release, i.e., UE shall be notified later on NAS message (PDU session modification on session release) [37].

### Q11: How to notify Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release?

Option 1. No enhancement needed.

Option 2. Indicating UE the multicast session state through group paging.

Option 3. Indicating UE to stop monitoring G-RNTI.

| Company | Which option(s) | Comment if any |
| --- | --- | --- |
| NEC | Opt-1/2 |  |
| Ericsson | Option 1 | There is no need to notify the UE about session state (activation, deactivation, temporary data, temporary no data, session release), but just “start/stop” monitoring for a TMGI. |
| Samsung | Option 2 | Group paging with indication for session release can avoid massive transition of all the UEs to RRC_CONNECTED at once and also avoids unnecessary monitoring for a released session in RRC_INACTIVE |
| MediaTek | Option 1 |  |
| CATT | Option 1 |  |
| Qualcomm | Option 1 | Similar comment as Ericsson, there is no need to notify the UE about session state (activation, deactivation, temporary data, temporary no data, session release) as long as when to start / stop monitoring for a G-RNTI is clearly indicated to the UE. |
| LGE | Option 1 |  |
| Nokia | 1 | Please see our reply above. No need to make any enhancements if deactivation is communicated to the UE. |
| TD Tech, Chengdu TD Tech | Option 1 |  |
| Huawei, HiSilicon | Option 1 | See comments above. |
| vivo | Option 1 | The NW can first indicate the session deactivation for stopping GC-PDCCH monitoring. Then the NW pages the UE into CONNECTED or triggering MT-SDT for NAS-level release. No further enhancement is needed as no performance gain is foreseen. |
| Kyocera | Option 1 |  |
| Lenovo | Option 1 | Legacy group paging informing UE entering RRC_CONNECTED is used for multicast session release. |
| Sharp | Option 1 |  |
| Intel | Option 1 | As in our reply to Q10, this is related to NAS procedure. gNB could use legacy paging without enhancement. |
| Xiaomi | Option 2 | To avoid the impacts on R17 UEs causing unnecessary reconnection to network, session state indication is needed so as the R17 UEs can ignore such group paging message based on this. |
| ITRI | Option 1 |  |
| ZTE | Option 2 | indicating UE the session status, e.g., session release, to UE through group paging could be straightforward. |
| CMCC | Option 1 |  |
| Spreadtrum | Option 1 |  |
| Apple | Option 1 |  |

## 4.4 Network resumes UE's RRC connection

It could happen that network wants to resume UE's RRC connection, e.g, upon turning off the multicast reception for UE in RRC_INACTIVE when network congestion is over/eased or reception UE number is lower, network may enable Rel-17 delivery mode instead. Depending on the design details in above section, there are different solutions:

Legacy group paging (or a group paging without the Rel-18 enhancement) [24, 25, 31, 32].

Enhancement to group paging to explicitly resume UE's RRC connection.

Enhanced MCCH. Other companies [26, 28] also consider to enhance MCCH to indicate preferred UE state per session. Based on such explicit UE RRC state, UE resumes to RRC_CONNECTED if needed. The Multicast MCCH includes a new indicator for state transition per service level. If there is a transition indicator included in the MCCH message for UE’s interested & joined service, the UE immediately transfers from RRC_INACTIVE to RRC_CONNECTED state. While [32] think the MCCH based notification has a too high granularity, it can not notify only a subset of the UEs to resume RRC connection.

### Q12: How to indicate RRC_INACTIVE UE to transition to RRC_CONNECTED state.

- Option 1: Group paging with no enhancement.

- Option 2: Enhanced group paging to indicate preferred UE RRC state.

- Option 3: Enhanced MCCH to indicate preferred UE RRC state.

- Option 4: Legacy UE-specific paging.

- Option 5: Enhanced group paging to indicate transition to RRC_CONNECTED though the UE is configured to receive multicast in RRC_INACTIVE.

- Others. Please elaborate in comments.

| Company | Which option(s) | Comment if any |
| --- | --- | --- |
| NEC | Opt-2 | Same comment as Q6/9. To our understanding, UE anyway need to monitor paging and group paging is already a per-TMGI(session) configuration. One simple solution can solve all the problems is that adding additional indication to each TMGI in the pagingGroupList, example like, using paging cause (session activate in Connected / Inactive, session deactivate, session release and so on) to correspond to each TMGI. In a word, enhanced group paging can notify UE of any session state with less spec impact. |
| Ericsson | Option 1 | We do not understand what “preferred UE RRC state” means: preferred RRC state from NW or UE perspective? If the NW tells the UE to comeback this is then up to UE implementation whether to comply, i.e. NW pages for nothing? We do not see a need for that. |
| Samsung | Option 1 | Rel-17 Group paging is sufficient to transition UE to RRC_CONNECTED state by default i.e. Group paging with no enhancement is an obvious approach. Only to keep the UE in RRC_INACTIVE, enhancement for Group Paging is needed as commented in Q6 |
| MediaTek | Option 1 and option4 | This question can be discussed in different cases: Case 1: Network intends to indicate all UEs to switch to RRC CONNECTED state. This may be used when the congestion alleviate and the network decide to switch off the multicast in INACTIVE. In this case, the group paging can be reused with no enhancement. Case 2: Network select a subset of UEs to switch to RRC CONNECTED state. In this case, there is no benefit for enhancing group paging compared with unicast paging. Also it may introduce extra UE power consumption since other UEs need to check for unnecessary information. |
| CATT | Option 1 and Option 4 | It is workable to reuse R17 group paging. The expected behavior of R18 UE when it is paged for transiting to CONNECTED to continue the multicast reception is same as that for R17 UE, so the R17 group paging can be reused without enhancement. And if gNB only wants to address a subset of all the Ues,it can chose to send the group paging message on subset of the available POs. Besides, legacy individual paging can also be used to move certain multicast receiving UE(s) from RRC_INACTIVE to RRC_CONNECTED. |
| Qualcomm | Option 4 | See Q6 also. Group paging can indicate whether ALL the UEs should stay in INACTIVE or move ALL the UEs to CONNECTED. Legacy paging can be used to move selected UEs to CONNECTED (overriding the group paging indication). |
| LGE | Option 5 | The multicast list in the existing group paging only can indicate the session is activated. When the existing group paging is received, R18 UE decides whether to initiate RRC Resume to receive the multicast in RRC_CONN or stay RRC_INACTIVE based on the UE-specific network configuration. Therefore, network cannot indicate UE which is allowed to receive the multicast in RRC_INACTIVE to transition to RRC_CONN using the existing multicast list in the group paging. |
| Nokia | Option 1 | Option 2 simply would mean if no other flag is transmitted within Rel-17 group paging, UE would come back to RRC_CONNECTED; and if enhancements are done (as proposed for activation) to keep UEs in RRC_INACTIVE, the UE would not come back. Regarding Option 3- No need to enhance MCCH to indicate the state. If the UE sees that the session is active (via activation flag in MCCH that we propose above) and if the UE cannot find PTM configuration in MCCH, the UE would directly understand that it should reconnect. |
| TD Tech, Chengdu TD Tech | Option 3 |  |
| Huawei, HiSilicon | Option 1 and Option 4 | Legacy group paging can be reused to invoke the UEs from RRC_INACTIVE to RRC_CONNECTED. If the NW wants to invoke only some of the UEs, the group paging can be sent on a subset of POs or unicast paging can be used. Or in another way, the NW can use UAC mechanism to prevent too many UEs from going to RRC_CONNECTED. |
| vivo | Option 3 with comments Option 4 | For option 3, we share a similar view with Ericsson that the definition of “preferred RRC state” is not clear. Our understanding is that an multicast MCCH message (e.g. via indication) can be used to send an amount of Rel-18 UEs back to the CONNECTED state. For option 1, anyway, enhancement for either session activation or state change is needed. we should try to introduce any negative impacts on legacy UEs. The same logic is also applicable for option 2. Option 4 has already been supported based on the current spec. We fail to see a reason to exclude it. |
| Kyocera | Option 1 and Option 4 | We wonder if Option 4 also includes Option 1 since Option 1 is also legacy paging although there is the difference whether to use TMGI or UE-ID. |
| Lenovo | Option 1&4 |  |
| Sharp | Option 1 and Option 4 | see the comments in Q6. |
| Intel | Option 1 |  |
| Xiaomi | Option 1 and 4 | If the expected UE behavior is only to resume the RRC connection, it is sufficient to reuse the legacy group paging and legacy UE-specific paging with no enhancement. |
| ITRI | Option 1, 3, and 4 |  |
| ZTE | Option 1 | two possible solutions: - legacy group paging + PTM config not available. - in case group paging itself it to be enhanced, a legacy group paging will do the job to resume UE. |
| Canon | No strong view |  |
| CMCC | Option1 and Option 4 | With Rel-17 group paging, UE would come back to RRC_CONNECTED, and as discussed in other question, Rel-17 group paging can be enhanced to indicate that UE stays in RRC_INACTIVE, therefore, without enhancements, Rel-17 paging can be used to indicate UE to RRC_CONNECTED. Besides, unicast paging is workable too. |
| Spreadtrum | Option1 and Option 4 | Rel-17 group paging can be used for a subgroup of UEs while the legacy UE-specific paging is used for one specific UE. |
| Apple | Option 1 and Option 4 |  |

# 5 Issues not covered

### Q13: For any issues not covered in current questions, please kindly add it up here.

| Company | Comment if any |
| --- | --- |
| NEC | To consider the MCCH monitoring issue: The cost of this mechanism is that UE needs to continuously monitor MCCH during RRC_INACTIVE for possible updated PTM configuration (or notification if agreed) even if there is no any updated PTM configuration, which increase UE power consumption. [rapp]: thank you Rao for the comments. this is not in the scope of current email discussion. but I assume we can always consider this in company contribution, if needed. |
| Ericsson | @NEC: in case PTM reconfiguration happens only seldom, the NW could decide to only configure the MCCH in SIB when a PTM change is pending, and release the MCCH configuration in SIB when the reconfiguration is completed. [NEC] This is also an optional solution (i.e., SIB-controlled MCCH transmission), however we are concerning: 1. If we are putting session notification (e.g., activation/deactivation) into MCCH, I am afraid the frequency of MCCH monitoring may not be a “seldom”; 2. If PTM config of neighbor cell is provided in MCCH instead of dedicated RRC, a UE who re-select to a new cell should acquire PTM config by MCCH. In this case, SIB-controlled MCCH transmission could not work well; 3. The legacy Rel-17 MCCH could include multiple sessions which means any one of PTM reconfiguration means MCCN change notification, currently I am not sure whether the “false alarm” issue boost the frequency of MCCH monitoring. |
| Samsung | As also stated in comment for Q8: There is an existing mechanism for data inactivity for multicast reception in RRC_CONNECTED. UE transits to RRC_IDLE if multicast data is not received for a defined time (dataInactivityTimer). Same situation is relevant for multicast reception in RRC_INACTIVE. UE may not receive multicast data for long (e.g. due to data inactivity or session release not informed to RRC_INACTIVE UE etc.) and it may unnecessary monitor for multicast channel and consume power. Therefore, extending data inactivity mechanism of RRC_CONNECTED also to RRC_INACTIVE seems reasonable. [rapp]: thank you Vinay for bringing this up. Yes I am aware. the dataInactivitityTimer could be another issue we need to consider in the future, maybe under UP or MAC, but not in the scope of current discussion. |
| MediaTek | We may need to discuss the notification when network update the configuration and check whether to reuse R17 mechanism (MCCH change notification) [Qualcomm] Similar view. [rapp]: Thank you Xiaonan and Umesh. For now notification for PTM update is not in the scope, please check the content of the email discussion in the introduction part. I thought about adding this but dropped it in the final version. but I assume we can always consider this in company contribution. |
| Nokia | Counting: Rel-17 multicast delivery mode provides significant benefits to the UE compared to reception of multicast in RRC_INACTIVE state, as it offers HARQ feedback that can feed MCS selection and enable retransmissions. If the reception of a multicast session by the UEs in RRC_INACTIVE is enabled, the gNB should transmit the data in all beams, as the gNB would not be aware under which beam the UE is located, which may decrease spectral efficiency compared to Rel-17 mechanism where the gNB is aware of the location of the UE (and not transmit data multiple times in different beams). In our view, for deciding on whether to provide a multicast session to UEs in RRC_INACTIVE state, a gNB that uses delivery of multicast session to the UEs in RRC_INACTIVE state should estimate periodically the number of UEs in RRC_INACTIVE state in the cell that are receiving a multicast session, if the multicast session is active. Therefore, some sort of mechanism is required to evaluate the number of UEs receiving a multicast session in RRC_INACTIVE state in a cell. Q1: Scenario 4 mentioned in the Q1 in our comment [rapp]: thank you Jarkko. Two issues, - counting is also being discussed in RAN3 on network awareness of UE's distribution. This is a relatively separate topic, I think we can have a separate discussion on this. - In section 3.1 I tried to focus service continuity in RRC_INACTIVE and any possible RRC resumption cases. I assume we can always bring this up, i.e., mobility in RRC_CONNECTED in company contribution. |
| TD Tech, Chengdu TD Tech | The following two issues are related to service continuity. - MCCH configuration can be area specific. In other words, the new SIB for multicast MCCH can be area specific just as an existing SIB can be area specific. - The PTM configuration of a multicast session can be applied to the cells of a same gNB. In other words, the sane PTM configuration can be applied to intra-gNB case. [rapp]: thank you Limei. We tried in last RAN2 meeting and did not achieve any consensus in Area specific PTM, therefore I think we'd better not repeat the discussion here. |
| Huawei, HiSilicon | We should discuss whether the UE needs to monitor the MCCH-RNTI besides G-RNTI during the session deactivation. [rapp]: thank you Xubin for bringing this up. I think this is related to the notification mechanism and UE behaviour upon different notification, e.g., after session deactivation, whether UE need to monitor MCCH. In different solutions there are corresponding different UE behaviour. We can review this during the discussion of the solutions in section 4. |
| Kyocera | We agree to the issue raised by MediaTek, i.e., the notification for PTM configuration update. We share Nokia’s view, i.e., on Counting. In addition, we think RAN2 should discuss and ensure the service continuity during RRC state transition, i.e., from Connected to INACTIVE, and from INACTIVE to Connected, although it might be Stage-3 details. [rapp] dear Masato, please check my feedback above to MTK/Nokia. As for your cases you brought up, I do think they are important and need to be considered. It depends on how the PTM config are provisioned and part of them is being discussed in this email and also the UP email. |
| Lenovo | We may need to discuss whether a new access category and RRC cause are needed in case that the RRC Resume procedure is triggered by multicast reception. [rapp]: dear Mingzeng, please check in the beginning of section 3, it was intended to come back to the issues you mentioned in later stage and to focus for now: "It should be noted that Access categories and RRC resume cause value were also mentioned in companies contributions. However it is suggested to firstly have a common understanding on the essential service continuity scenarios and related solutions first" |
| Apple | We may need to clarify whether the MCCH scheme is mandatory for the inactive multicast reception feature, i.e. whether it’s possible for UE not to support it and for gNB not to enable it. |

# 6 Conclusions

TBD

# 7 Reference

## # PTM config and mobility

- R2-2300286	Discuss on PTM configuration for multicast in RRC INACTIVE	MediaTek inc.	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301036	PTM configuration for multicast reception in RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-18

- R2-2300242	Initial Considerations on Mixed Approach	vivo Mobile Com. (Chongqing)	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301586	PTM configuration and mobility aspects on multicast reception in RRC INACTIVE 	Kyocera 	discussion	Rel-18

- R2-2300335	PTM configuration and mobility aspects for multicast reception in RRC_INACTIVE	Qualcomm Incorporated	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300178	Discussions on PTM Configuration and Mobility	CATT, CBN	discussion	NR_MBS_enh-Core

- R2-2300100	Discussion on multicast reception in RRC_INACTIVE state	OPPO	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300243	Discussion on Mixed Approach from PHY Aspect	vivo Mobile Com. (Chongqing)	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300283	Analysis of MCCH for sending PTM configuration	TD Tech, Chengdu TD Tech	discussion	Rel-18

- R2-2300525	Discussion on PTM configuration aspects and mobility	Samsung R&D Institute India	discussion	Rel-18

- R2-2300666	Discussion on PTM configuration and Mobility	Spreadtrum Communications	discussion	Rel-18

- R2-2300672	Discussion on PTM configuration and mobility 	NEC Corporation	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300735	PTM Configuration and Mobility for INACTIVE Multicast Reception	Apple	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300876	PTM configuration aspects and mobility	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300947	PTM configuration and mobility for multicast reception in RRC_INACTIVE	Lenovo	discussion	Rel-18

- R2-2301162	PTM configuration and mobility for multicast reception in RRC_INACTIVE	Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301206	PTM configuration aspects and mobility	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301235	Discussion on PTM configuration and mobility	CMCC	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301559	PTM configuration for multicast reception in RRC_INACTIVE	Intel Corporation	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301672	Multicast in RRC_INACTIVE	Sharp	discussion

- R2-2301691	Considerations on the PTM configuration and mobility for multicast reception in RRC_INACTVE state	Beijing Xiaomi Software Tech	discussion	Rel-18

- R2-2301843	PTM Configuration delivery for multicast reception in RRC_INACTIVE	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh

- R2-2301070	Ensuring desired level of reliability for an MBS session in RRC_INACTIVE	InterDigital Inc.	discussion	Rel-18	NR_MBS_enh-Core

## # notification

- R2-2300877	Notifications and RRC state transitions	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300179	Discussion on Notifications and RRC state transitions	CATT, CBN	discussion	NR_MBS_enh-Core

- R2-2300244	Discussion on (De)Activation and State Transition	vivo Mobile Com. (Chongqing)	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300252	HARQ operation during RRC state transitions for multicast reception	NEC	discussion	NR_MBS_enh-Core

- R2-2300284	Common signalling for multicast reception in RRC_INACTIVE state	TD Tech, Chengdu TD Tech	discussion	Rel-18

- R2-2300287	Notification and state transition for multicast in RRC INACTIVE	MediaTek inc.	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300336	Notifications and RRC state transitions multicast reception in RRC_INACTIVE	Qualcomm Incorporated	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300526	Discussion on Notification and RRC state transitions	Samsung R&D Institute India	discussion	Rel-18

- R2-2300667	Discussion on Notification and RRC state transition	Spreadtrum Communications	discussion	Rel-18

- R2-2300736	Group Notification and RRC State Transition for Multicast Reception	Apple	discussion	Rel-18	NR_MBS_enh-Core

- R2-2300948	Notification and State Transmission for Multicast Reception in RRC_INACTIVE	Lenovo	discussion	Rel-18

- R2-2301037	Multicast activation deactivation notification and RRC state transitions	LG Electronics Inc.	discussion	Rel-18

- R2-2301163	Notification and RRC state transition for multicast reception in RRC_INACTIVE	Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301205	Notifications and RRC state transitions	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301236	Discussion on notification for RRC_INACTIVE multicast reception Ues	CMCC	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301560	Notification and RRC state transition for multicast reception in RRC_INACTIVE	Intel Corporation	discussion	Rel-18	NR_MBS_enh-Core

- R2-2301587	Notification and RRC state transition aspects on multicast reception in RRC INACTIVE 	Kyocera 	discussion	Rel-18	R2-2212521

- R2-2301594	Session state change for UEs receiving Multicast in RRC_INACTIVE state	TCL Communication Ltd.	discussion

- R2-2301674	Group Paging and Multicast session received in RRC_INACTIVE	Sharp	discussion

- R2-2301692	Considerations on the notification and RRC transitions for the multicast reception in RRC_INACTIVE state	Beijing Xiaomi Software Tech	discussion	Rel-18

- R2-2301844	Multicast session status change notification	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh