3GPP TSG RAN meeting #99										RP-22xxxx

Rotterdam, Netherlands, March 20-23, 2023

Status Report to TSG

Agenda item:			9.3.2.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | NR_DualTxRx_MUSIM |  |  |  |  |
| Unique ID | 941001 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223492 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 12/2023 | Performance part: 06/2024 | Testing part: - |  |
| Overall Completion level | Study Item: - | Core part: Overall: 30% | Performance Part: - Overall: 0% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Xiaodong Yang |
|  | Company | vivo |
|  | Email | Yangxiaodong5g@vivo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? |  |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2 121

Temporary capability restriction

- A2a: When the UE is in Connected mode in two NR networks, it is up to the UE implementation to select which NW to perform signaling for temporary UE capability restrictions.

- A2b: When the UE is in Connected mode in NR NW A and moving from Idle/Inactive to connected mode in NR NW B, the signaling for temporary UE capability restrictions can happen on NW A. FFS how to handle if UE is moving from IDLE/INACTIVE in NW A and is in CONNECTED with NW B.

- A2c: When the UE is in Connected mode in both networks and one is E-UTRAN, the signaling for temporary UE capability restrictions happens on the NR network.

- A3: The UE will request a temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. FFS whether the UE can indicate if it is already connecting with reduced capabilities during connection set-up/resume.

- A4: RAN2 to discuss whether prohibit timer is needed for the signaling of temporary UE capability restrictions This can wait until after progress is made on the signaling framework.

- A1: UAI can be used for the signaling of temporary UE capability changes for dual-active MUSIM. FFS if we have additional signalling (depends on e.g. SCell/SCG deactivation usability for MUSIM)

- A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted:

•	Transmission and reception capabilities (e.g. MIMO layers)

•	Measurement capabilities (e.g. gaps)

•	Supported bandwidth

•	Supported band-combinations (FFS whether this is CA or DC or both)

- FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion).

- FFS whether UE reports some or all of the above or whether we can do something simpler

- A6: For dual-active MUSIM, UE signaling will support the request for release (and reversal) of SCells and SCG. The signaling details (e.g. implicit or explicit request of each SCell or SCG) is FFS. FFS if we support deactivation (based on discussion in which case it can be used). It is up to network how to react to UE request.

- RAN2 does not intend to create new procedures for e.g. SCell/SCG deactivation for MUSIM purposes in Rel-18. Existing procedures can be used based on NW choice.

- B4: RAN2 considers the only RAN3 impact may be to support the UE request of SCG/SCell release via SRB3 (if supported) for MUSIM purpose (e.g. cause value).  If this can be done via inter-node messages, RAN2 expects no RAN3 impacts.

- RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details.

R17 MUSIM Gap coordination

- The UE is only allowed to provide MUSIM assistance information for Rel-17 MUSIM gap preference to NR MN and NR MN configures the UE with Re-17 MUSIM gap(s). This requires no specification impacts.

- Use inter-node messages to convey Rel-17 MUSIM gap configuration from MN to SN in NW A when UE is in NR-DC.

2.2.2	Remaining Open issues

1 Continue to discuss the scenarios and mechanism to indicate preference on temporary UE capability restriction in NW A when the UE needs transmission or reception for MUSIM purpose.

2 MUSIM Gap coordination and priority handling

2.3	RAN3

2.3.1	Agreements

    - Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 106

Issue 1-1-2: Considerations on one-shot RRM mobility procedures

- Agreements:

  - No need to consider the collision between SMTC and MUSIM gaps for RRC Re-establishment, RRC Connection Release with Redirection

  - FFS on collision between SMTC and MUSIM gaps for handover and Scell activation

Issue 2-1-1: On introduction of priority for MUSIM gaps

- Agreements:

  - P1: The priority level of MUSIM shall be configured to be comparable to priority level of other MGs

  - MUSIM gap and Type-2 gap cannot be configured with the same priority

- Agreements

  - The priority level of MUSIM gaps should be configured/allocated by NW A

Issue 2-1-2: Priority/usage indication on MUSIM gaps from UE side

- Agreements:

  - Down-select from option 1 and option 2

    - Option 1: UE can indicate its preferred priority per each MUSIM gap

    - Option 2: UE indicates the MUSIM gap with the highest priority level and the MGRP should be larger or equal to a threshold

- Agreements

  - UE can optionally indicate its preferred priority for all or a subset MUSIM gaps

  - It is up to NW A on how to use this information

Issue 2-3-1:  Solutions for collision between MUSIM gap and Type-2 MG

- Agreements:

  - Gap sharing will not be considered for the collision between MUSIM gaps and Type-2 gaps.

Issue 4-1-1: Whether to define network B requirements

- Agreements

  - Define NW B measurement/cell reselection requirements in IDLE mode only

    - Prioritize the scenario where

      - MUSIM gap is not colliding with other NW A gaps and not dropped

      - NW A configures MUSIM gaps requested by UE

    - FFS whether and how to define test cases for these requirements

2.4.2	Remaining Open issues

1. Collisions between gaps and priority rules

2. On network A requirements

3. On network B requirements

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	RAN Plenary Intervention

5.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2 #121

R2-2300902	Work planning of R18 MUSIM	vivo	Work Plan	Rel-18

R2-2300098	Scenarios Clarification for R18 MUSIM	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2300099	Initial Consideration on Temporary UE Capability Restriction	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2300435	Scenarios and requirements for capability restriction request for Rel-18 MUSIM	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2300436	Signalling to indicate temporary capability reduction for Rel-18 MUSIM	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2300496	Applicable scenarios for R18 MUSIM	Huawei, HiSilicon	discussion

R2-2300498	Solutions for MUSIM capability restriction removal of restriction	Huawei, HiSilicon	discussion

R2-2300753	Discussion on Temporary Capability Restriction for DualRx/DualTx MUSIM UEs	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2300773	Report of [Post119bis-e][212][MUSIM] Rel-18 MUSIM solutions 	Qualcomm Incorporated, vivo	report

R2-2300816	Discussion on temporary UE capability restriction for MUSIM	MediaTek Inc.	discussion

R2-2300855	RAN3 impact of temporary UE capability switching for MUSIM	China Telecom	discussion	Rel-18	NR_DualTxRx_MUSIM

R2-2300903	Discussion on temporary capability restriction for Rel-18 Multi-SIM	vivo	discussion	Rel-18

R2-2300922	Baseline signaling procedure for primary scenarios of Dual TX/RX MUSIM operation	Nokia, Nokia Shanghai Bell	discussion	Rel-18

R2-2300923	Analysis on additional capability coordination scenarios for Dual TX/RX MUSIM operation	Nokia, Nokia Shanghai Bell	discussion	Rel-18

R2-2300969	Consideration on dual Tx/Rx Multi-SIM	Lenovo	discussion	Rel-18

R2-2301116	Capability sharing issue for SRS Tx switching capability	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	R2-2210060

R2-2301117	Discussion on the band conflicts for MUSIM	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301173	UE Capability restriction for Dual-Active MUSIM	China Telecommunications	discussion	Rel-18

R2-2301428	UE Capability Update for Dual-Active MUSIM	Qualcomm Incorporated	discussion

R2-2301448	Overall Dual-RX/TX MUSIM Solution	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301449	Discussion on MUSIM gaps for a Dual-RX/Dual-TX UE	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301543	Discussion on Dual Tx/Rx Multi-SIM	ASUSTeK	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301673	Capability Restriction for eMUSIM	Sharp	discussion

R2-2301709	Consideration on the Temporary  Capability Restriction	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301742	General Solution for Rel-18 MUSIM	LG Electronics	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301743	LS on SCG Deactivation while Multi-SIM Operation	LG Electronics	LS out	Rel-18	NR_DualTxRx_MUSIM-Core	To:RAN4

=> Withdrawn

R2-2301881	Discussion on possible solutions for dual Rx/Tx MUSIM devices	DENSO CORPORATION	discussion	NR_DualTxRx_MUSIM-Core

R2-2300517	MUSIM Band Conflict Issue Handling	Samsung R&D Institute India	discussion	Rel-18

R2-2300754	Discussion on Signaling solutions for Band Conflict Mitigation for DualRx/Dual Tx MUSIM UEs	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2300904	Discussion on MUSIM band conflict handling	vivo	discussion	Rel-18

R2-2301446	Coordination of MUSIM gaps for NR-DC	Qualcomm Incorporated	discussion

R2-2301710	Consideration on the Scheduling Gap for the MR-DC	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301744	Further Considerations for Rel-18 MUSIM	LG Electronics	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2301778	Further discussion on MN-SN MUSIM gaps coordination 	Samsung Electronics Austria	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2302008	Report of [Post121][202][MUSIM] LS to RAN4 on Rel-18 MUSIM impacts (vivo) 	vivo	report

R2-2302007	LS to RAN4 on Rel-18 MUSIM impacts	RAN2	LS out	Rel-18	NR_DualTxRx_MUSIM-Core	To:RAN4

RAN4 #106

R4-2300060 Discussion on priority rule of MUSIM 	Charter Communications, Inc	Discussion

R4-2300233 On R18 MUSIM - collisions between gaps and priority rules	Apple	Discussion

R4-2300234 LS on MUSIM gap priority	Apple	Approval

R4-2300235 On R18 MUSIM - network A requirements	Apple	Discussion

R4-2300236 On R18 MUSIM - network B requirements	Apple	Discussion

R4-2300874 Discussion on collisions between gaps and priority rules for MUSIM gaps	CMCC	Discussion

R4-2300875 Discussion on open issues for MUSIM gaps	CMCC	Discussion

R4-2300876 Discussion on requirements for MUSIM gaps	CMCC	Discussion

R4-2300902 Discussion on collisions between gaps and priority rules for Rel-17 MUSIM gaps	Xiaomi	Discussion

R4-2300903 Discussion on network A requirements for Rel-17 MUSIM gaps	Xiaomi	Discussion

R4-2300904 Discussion on network B requirements for Rel-17 MUSIM gaps	Xiaomi	Discussion

R4-2300905 Discussion on other issues for Rel-17 MUSIM gaps	Xiaomi	Discussion

R4-2301284 Considerations on general aspects for MUSIM gaps	vivo	Discussion

R4-2301285 Considerations on collisions between gaps and priority rules for MUSIM gaps	vivo	Discussion

R4-2301286 Considerations on network A requirements for RRM requirements of MUSIM gaps	vivo	Discussion

R4-2301287 Considerations on network B requirements for RRM requirements of MUSIM gaps	vivo	Discussion

R4-2301288 Consideration on other issues for RRM requirements for R17 MUSIM gaps	vivo	Discussion

R4-2301289 LS on priority for MUSIM gaps	vivo	Approval

R4-2301384 Discussion on collision between gaps and priority rules	ZTE Corporation	Approval

R4-2301385 Discussion on neywork A requirements	ZTE Corporation	Approval

R4-2301386 Discussion on network Brequirements	ZTE Corporation	Approval

R4-2301643 Discussion on collision between gap and priority rules	OPPO	Approval

R4-2301644 Discussion on network A requirements	OPPO	Approval

R4-2301645 Discussion on network B requirements	OPPO	Approval

R4-2301646 Discussion on other aspects for MUSIM gaps	OPPO	Approval

R4-2301977 Discussion on collision handling for MUSIM gaps	Huawei, HiSilicon	Approval

R4-2301978 Discussion on NW-A requirements with MUSIM gaps	Huawei, HiSilicon	Discussion

R4-2301979 Discussion on NW-B requirements with MUSIM gaps	Huawei, HiSilicon	Discussion

R4-2301980 Discussion on other issues related to MUSIM gaps	Huawei, HiSilicon	Discussion

R4-2302113 Discussions on general issues in MUSIM gaps	Ericsson	Discussion

R4-2302114 Discussions on collision between MUSIM gaps	Ericsson	Discussion

R4-2302115 Discussions on NW-A’s requirement in MUSIM gaps	Ericsson	Discussion

R4-2302116 Discussions on NW-B’s requirement in MUSIM gaps	Ericsson	Discussion

R4-2302117 Discussions on other issues in MUSIM gaps	Ericsson	Discussion

R4-2302334 On requirements for Rel-17 MUSIM gaps	Qualcomm Incorporated	Approval

R4-2302356 Discussion on the general aspects of MUSIM gaps	MediaTek inc.	Discussion

R4-2302357 Discussion on RRM requirements for MUSIM gaps collision handling	MediaTek inc.	Discussion

R4-2302358 Discussion on NW A RRM requirements for MUSIM	MediaTek inc.	Discussion

R4-2302359 Discussion on NW B RRM requirements for MUSIM	MediaTek inc.	Discussion

R4-2302602 Discussion on Collisions between gaps and priority rules	Nokia, Nokia Shanghai Bell	Discussion

R4-2302603 Discussion on network A requirements	Nokia, Nokia Shanghai Bell	Discussion

R4-2302604 Discussion on network B requirements	Nokia, Nokia Shanghai Bell	Discussion

R4-2302605 Discussion on other aspects of MUSIM	Nokia, Nokia Shanghai Bell	Discussion

R4-2302782	Topic summary for [106][224] NR_DualTxRx_MUSIM Moderator (vivo) Information

R4-2303242	Ad-hoc minutes for NR Dual Tx/Rx Multi-SIM vivo Approval

R4-2303248	WF on NR Dual Tx/Rx Multi-SIM vivo Approval

R4-2303310	WF on NR Dual Tx/Rx Multi-SIM vivo Approval

R4-2301289	LS on priority for MUSIM gaps vivo Approval

R4-2303249	LS on priority for MUSIM gaps vivo Approval