3GPP TSG-RAN WG2 Meeting #123	R2-2308336

# Toulouse, France, 21– 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Num | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XR Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Qualcomm (Rapporteurs) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New mechanisms have been agreed for the support of XR services in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An overview of the new mechanisms for the support of XR services is added: - Description of XR Awareness based on TS 23.501 - Power Saving Enhancements - Capacity Enhancements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A stage 2 overview of the agreed mechanisms for the support of XR services in NR is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 16.15 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.321 ... CR ... TS 38.322 ... CR ... TS 38.331 ... CR ... TS 38.306 ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[9]	3GPP TS 37.324: " E-UTRA and NR; Service Data Protocol (SDAP) specification".

[10]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[15]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[16]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[17]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[20]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer"

[21]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[22]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[23]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[24]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[25]	Void.

[26]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[27]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[28]	3GPP TS 24.501: "NR; Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[30]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[31]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[32]	3GPP TS 38.470: "NG-RAN; F1 application protocol (F1AP) ".

[33]	3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[34]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[35]	3GPP TS 38.101-2: "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[36]	3GPP TS 38.101-3: "User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[37]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[38]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[39]	3GPP TS 22.104 "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[41]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[42]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[43]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[44]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[45]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[46]	3GPP TS 26.346 "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[47]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[48]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[49]	3GPP TS 28.541: "5G Network Resource Model (NRM)".

[50]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[51]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984".

[52]	3GPP TS 38.211: "NR; Physical channels and modulation".

[53]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS)".

[54]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[AA]	3GPP TR 38.835: "NR; Study on XR enhancements for NR".

Next Modified Subclause

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

AR	Augmented Reality

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFR	Common Frequency Resource

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ePWS	enhancements of Public Warning System

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDB	Packet Delay Budget

PDC	Propagation Delay Compensation

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PER	Packet Error Rate

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PRS	Positioning Reference Signal

PS-RNTI	Power Saving RNTI

PSDB	PDU-Set Delay Budget

PSER	PDU-Set Error Rate

PSI	PDU-Set Importance

PSIHI	PDU-Set Integrated Handling Information

PSS	Primary Synchronisation Signal

PTM	Point to Multipoint

PTP	Point to Point

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QMC	QoE Measurement Collection

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RLM	Radio Link Monitoring

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

SMTC	SS/PBCH block Measurement Timing Configuration

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRAP	Sidelink Relay Adaptation Protocol

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SSSG	Search Space Set Group

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TB	Transport Block

TCE	Trace Collection Entity

TNL	Transport Network Layer

TPC	Transmit Power Control

TRP	Transmit/Receive Point

TRS	Tracking Reference Signal

U2N	UE-to-Network

UCI	Uplink Control Information

UDC	Uplink Data Compression

UE-Slice-MBR	UE Slice Maximum Bit Rate

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

VR	Virtual Reality

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

XR	Extended Reality

Next Modified Subclause

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

Boundary IAB-node: as defined in TS 38.401 [4].

Broadcast MRB: A radio bearer configured for MBS broadcast delivery.

CAG Cell: a PLMN cell broadcasting at least one Closed Access Group identity.

CAG Member Cell: for a UE, a CAG cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN, and for that PLMN, a CAG identifier belonging to the Allowed CAG list of the UE for that PLMN.

CAG-only cell: a CAG cell that is only available for normal service for CAG UEs.

Cell-Defining SSB: an SSB with an RMSI associated.

Child node: IAB-DU's and IAB-donor-DU's next hop neighbour node; the child node is also an IAB-node.

Conditional Handover (CHO): a handover procedure that is executed only when execution condition(s) are met.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

DAPS Handover: a handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB.

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time, as defined in TS 23.501 [3].

Direct Path: a type of UE-to-Network transmission path, where data is transmitted between a UE and the network without sidelink relaying.

Downstream: direction toward child node or UE in IAB-topology.

Early Data Forwarding: data forwarding that is initiated before the UE executes the handover.

Earth-centered, earth-fixed: a global geodetic reference system for the Earth intended for practical applications of mapping, charting, geopositioning and navigation, as specified in NIMA TR 8350.2 [51].

Feeder link: wireless link between the NTN Gateway and the NTN payload.

Geosynchronous Orbit: earth-centered orbit at approximately 35786 kilometres above Earth's surface and synchronised with Earth's rotation. A geostationary orbit is a non-inclined geosynchronous orbit, i.e. in the Earth's equator plane.

Group ID for Network Selection: an identifier used during SNPN selection to enhance the likelihood of selecting a preferred SNPN that supports a Default Credentials Server or a Credentials Holder, as specified in TS 23.501 [3].

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

High Altitude Platform Station: airborne vehicle embarking the NTN payload placed at an altitude between 8 and 50 km.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

IAB-DU: gNB-DU functionality supported by the IAB-node to terminate the NR access interface to UEs and next-hop IAB-nodes, and to terminate the F1 protocol to the gNB-CU functionality, as defined in TS 38.401 [4], on the IAB-donor.

IAB-MT: IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. IAB-MT function used in 38-series of 3GPP Specifications corresponds to IAB-UE function defined in TS 23.501 [3].

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes. The IAB-node does not support backhauling via LTE.

IAB topology: the unison of all IAB-nodes and IAB-donor-DUs whose F1 and/or RRC connections are terminated at the same IAB-donor-CU.

Indirect Path: a type of UE-to-Network transmission path, where data is forwarded via a U2N Relay UE between a U2N Remote UE and the network.

Inter-donor partial migration: migration of an IAB-MT to a parent node underneath a different IAB-donor-CU while the collocated IAB-DU and its descendant IAB-node(s), if any, are terminated at the initial IAB-donor-CU. The procedure renders the said IAB-node as a boundary IAB-node.

Intra-system Handover: handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: handover that involves a CN change (EPC or 5GC).

Late Data Forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

Mapped Cell ID: in NTN, it corresponds to a fixed geographical area.

MBS Radio Bearer: A radio bearer configured for MBS delivery.

MSG1: preamble transmission of the random access procedure for 4-step random access (RA) type.

MSG3: first scheduled transmission of the random access procedure.

MSGA: preamble and payload transmissions of the random access procedure for 2-step RA type.

MSGB: response to MSGA in the 2-step random access procedure. MSGB may consist of response(s) for contention resolution, fallback indication(s), and backoff indication.

Multicast/Broadcast Service: A point-to-multipoint service as defined in TS 23.247 [45].

Multicast MRB: A radio bearer configured for MBS multicast delivery.

Multi-hop backhauling: using a chain of NR backhaul links between an IAB-node and an IAB-donor.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Non-CAG Cell: a PLMN cell which does not broadcast any Closed Access Group identity.

Non-Geosynchronous orbit: earth-centered orbit with an orbital period that does not match Earth's rotation on its axis. This includes Low and Medium Earth Orbit (LEO and MEO). LEO operates at altitudes between 300 km and 1500 km and MEO at altitudes between 7000 km and 25000 km, approximately.

Non-terrestrial network: an NG-RAN consisting of gNBs, which provide non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway.

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X communication as defined in TS 23.287 [40] and the ProSe communication (including ProSe non-Relay and UE-to-Network Relay communication) as defined in TS 23.304 [48], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery and ProSe UE-to-Network Relay discovery for Proximity based Services as defined in TS 23.304 [48] between two or more nearby UEs, using NR technology but not traversing any network node.

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

NTN payload: a network node, embarked on board a satellite or high altitude platform station, providing connectivity functions, between the service link and the feeder link. In the current version of this specification, the NTN payload is a TNL node.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Parent node: IAB-MT's next hop neighbour node; the parent node can be IAB-node or IAB-donor-DU

PC5 Relay RLC channel: an RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network Relay.

PDU Set: one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s) or video slice(s) etc for XR Services), as defined in TS 23.501 [3].

PLMN Cell: a cell of the PLMN.

RedCap UE: a UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [11].

Relay discovery: AS functionality enabling 5G ProSe UE-to-Network Relay Discovery as defined in TS 23.304 [48], using NR technology but not traversing any network node.

Satellite: a space-borne vehicle orbiting the Earth embarking the NTN payload.

Service link: wireless link between the NTN payload and UE.

SNPN Access Mode: mode of operation whereby a UE only accesses SNPNs.

SNPN-only cell: a cell that is only available for normal service for SNPN subscribers.

SNPN Identity: the identity of Stand-alone NPN defined by the pair (PLMN ID, NID).

Transmit/Receive Point: part of the gNB transmitting and receiving radio signals to/from UE according to physical layer properties and parameters inherent to that element.

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE.

Upstream: direction toward parent node in IAB-topology.

Uu Relay RLC channel: an RLC channel between L2 U2N Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network Relay.

V2X sidelink communication: AS functionality enabling V2X communication as defined in TS 23.285 [41], between nearby UEs, using E-UTRA technology but not traversing any network node.

Xn: network interface between NG-RAN nodes.

Next Modified Subclause (new)

## 16.X	eXtended Reality Services

### 16.X.1	General

This subclause describes the functionalities for the support of eXtended Reality (XR) services that require high data rate and low latency communications. An overview of XR services is available in TR 38.835 [AA], while the service requirements are documented in TS 22.261 [19].

### 16.X.2	Awareness

XR-Awareness relies on QoS flows, PDU Sets, Data Bursts and traffic assistance information (see TS 23.501 [3]). To enable PDU Set based QoS handling, PDU Set QoS Parameters may be provided by the SMF to the gNB as part of the QoS profile of the QoS flow (at least one of them shall be provided):

-	PDU Set Delay Budget (PSDB): as defined in TS 23.501 [3], upper bound for the duration between the reception time of the first PDU (at the UPF for DL, at the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE in DL, at the UPF in UL). A QoS Flow is associated with only one PSDB, and when available, it applies to both DL and UL and supersedes the PDB of the QoS flow.

NOTE 1:		the AN PSDB is derived by subtracting the CN PDB from the PSDB.

-	PDU Set Error Rate (PSER): as defined in TS 23.501 [3], upper bound for a rate of non-congestion related PDU Set losses between RAN and the UE. A QoS Flow is associated with only one PSER, and when available, it applies to both DL and UL and supersedes the PER of the QoS flow.

NOTE 2:	In this release, a PDU set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

-	PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer, as defined in TS 23.501 [3].

NOTE 3:	The PDU Set QoS parameters are common for all PDU Sets within a QoS flow.

In addition, the UPF can identify PDUs that belong to PDU Sets, and may determine the following PDU Set Information which it sends to the gNB in the GTP-U header:

-	PDU Set Sequence Number;

-	Indication of End PDU of the PDU Set;

-	PDU Sequence Number within a PDU Set;

-	PDU Set Size in bytes;

-	PDU Set Importance (PSI), which identifies the relative importance of a PDU Set compared to other PDU Sets within the same QoS Flow.

Traffic assistance information may also be provided by 5GC to the gNB:

-	Via TSCAI (for both GBR and non-GBR QoS flows):

-	UL and/or DL Periodicity;

-	N6 Jitter Information (i.e. between UPF and Data Network) associated with the DL Periodicity.

-	Indication of End of Data Burst in the GTP-U header of the last PDU in downlink.

In the uplink, the UE needs to be able to identify PDU Sets and Data Bursts dynamically, including PSI. How this is done is left up to UE implementation.

### 16.X.3	Power Saving

#### 16.X.3.1	Physical Layer Enhancements

NOTE:	place holder for RAN1 to capture physical layer enhancements.

#### 16.X.3.2	Layer 2 Enhancements

Most XR frame rates (15, 30, 45, 60, 72, 90 and 120fps) correspond to periodicities that are not an integer (66.66, 33.33, 22.22, 16.66, 13.88, 11.11 and 8.33ms respectively).  The gNB may configure DRX cycle expressed in rational numbers so that DRX matches those periodicities, e.g. for the traffic with a periodicity of 60 fps, the network may configure the UE with a DRX cycle of 50/3 ms.

### 16.X.4	Capacity

#### 16.X.4.1	Physical Layer Enhancements

The following enhancements for configured grant-based PUSCH transmission are introduced:

-     Support of multiple CG PUSCH transmission occasions within a single period of a CG configuration;

#### -     Indication of unused CG PUSCH occasion(s) of a CG configuration with Uplink Control Information  multiplexed in CG PUSCH transmission of the CG configuration.

#### 16.X.4.2	Layer 2 Enhancements

##### 16.X.4.2.1	Assistance Information

In order to enhance the scheduling of uplink resources for XR, the following improvements are introduced:

-	One additional BS table to reduce the quantisation errors in BSR reporting (e.g. for high bit rates):

-	The code points of this new table follow a linear distribution;

Editor's Notes: can consider piecewise linearity when discussing how the BSR table values are defined.

-	The gNB configures the BS table(s) that an LCG is eligible to use, and when there is more than one, the UE selects the table (criteria FFS).

-	Delay knowledge of buffered data in a separate MAC CE, consisting of remaining time, and distinguishing how much data volume is buffered for which delay per LCG.

-	Additional BSR triggering conditions to allow timely availability of buffer status information can be investigated further.

-	Reporting of uplink assistance information (jitter range and burst arrival time associated with UL data burst periodicity) per QoS flow by the UE via UE Assistance Information.

##### 16.X.4.2.2	Discard

When the PSIHI is set for a QoS flow, as soon as one PDU of a PDU set is known to be lost, the remaining PDUs of that PDU Set can be considered as no longer needed by the application and may be subject to discard operation at the transmitter to free up radio resources.

NOTE:	It cannot always be assumed that the remaining PDUs are not useful and can safely be discarded. Also, in case of Forward Error Correction (FEC), active discarding of PDUs when assuming that a large enough number of packets have already been transmitted for FEC to recover without the remaining PDUs is not recommended as it might trigger an increase of FEC packets.

In uplink, the UE may be configured with PDU Set based discard operation for a specific DRB. When configured, the UE discards all packets in a PDU set when one PDU belonging to this PDU set is discarded, e.g. based on discard timer expiry.

In case of congestion, the PSI may be used for PDU set discarding. In uplink, dedicated signalling is used to trigger discard mechanism based on PSI.

NOTE:	In addition to considering the PSI within a QoS Flow for PDU set discarding, the gNB could also consider the relative PSI across QoS Flows of the same Priority Level.

Editor's Notes: once the exact mechanism is agreed, the above statement will be revised.

##### 16.X.4.2.3	Configured Grant

The following enhancements for configured grant-based transmission are recommended:

-	Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration;

-	Dynamic indication of unused CG PUSCH occasion(s) based on a new UCI by the UE.

Editor's Notes: once the exact mechanisms are agreed, the above statements will be revised.

End of Changes

List of Agreements

Editor's Notes: temporary Annex to capture all WI agreements.

#### RAN2#121

-	Companies should evaluate the RAN2 specification impacts and any other RAN2 aspects of their proposals for XR DRX.

-	Companies should evaluate the (high-level) impacts to RAN1/4 specification from their proposals for XR DRX.

-	Companies should try to coordinate with each other offline and bring joint proposals to next meeting. RAN2 aims to exclude proposals with least support in the next meeting.

-	Companies should evaluate the RAN2 specification impacts and any other RAN2 aspects of their proposals for SFN wrap-around.

-	Same as for DRX solutions, companies should try to coordinate with each other offline and bring joint proposals to next meeting. RAN2 aims to exclude proposals with least support in the next meeting.

-	Whether the issue of retransmission-less CG for UL pose transmission is addressed in the WI needs to be discussed in RAN.

-	New BSR tables are fixed (=specified) or semi-static (RRC-based).

-	FFS how many BSR tables are defined.

#### RAN2#121bis-e

-	RAN2 can discuss if XR traffic is only about GBR or can also be non-GBR (this may require RAN3 views)

-	UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

-	FFS on whether EoDB signalling is needed.

-	RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration.

-	To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

-	Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR UEs.

-	As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

-	Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

-	At least linear distribution is used for generating code points in new BSR table(s).  FFS whether exponential distribution can be considered too.  FFS if piecewise linear distribution is supported.

-	New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

-	Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

-	As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size.

-	Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

-	Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

-	PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

-	There is support to adopt NTN solution for the retransmission-less CG. If adopted, RAN2 aims to only consider option1 or option 2:

-	Option1: Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG.

-	Option2: Adapt the NTN solution by disabling the HARQ RTT timer per HARQ processes for both CG and DG.

-	FFS how the solution ensures consistent HARQ operation.

#### RAN2#122

-	UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

-	Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

-	UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

-	RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

-	Do not use PIN delay budget request for jitter reporting for XR services.

-	Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN.

-	On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

-	Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications.

-	Not use broadcast signalling for counter and reference SFN in XR

-	UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

-	When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

-	Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

-	We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

-	PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

-	Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

-	FFS how/whether to minimize additional UL signalling after this indication.

-	FFS if the NW indication is a one-shot or also subsequent packets.

-	For retransmission less CG enhancement in XR, adapt the NTN solution by disabling the HARQ RTT timer per CG configuration. Specifically, the following modifications shall be introduced:

-	A new RRC parameter for disabling drx-HARQ-RTT-TimerUL for a CG configuration;

-	Changes in the procedural text of DRX operations for CG in the MAC specification;

-	A new UE capability for supporting disabling drx-HARQ-RTT-TimerUL for a CG configuration.

#### RAN2#123

-	UE reports Burst Arrival time and Jitter associated with the UL data burst periodicity in uplink using UAI.

-	UE reports UL data burst periodicity in uplink using UAI.

-	All UAI fields for XR are optional fields in RRC. FFS how to handle persistency of signalled information (e.g. UE reports BAT first, then jitter).

-	Consider exact jitter range later on (e.g. via email discussion)

-	UE can also report there is no jitter (e.g. for pose)

-	Reuse UAI framework, e.g. network can configure when UE is allowed to report UAI. Exact triggering upon being configured and change of UAI is up to UE implementation. Network can configure prohibit timer for the reporting.

-	The maximum value of the counter (NSFN) is 2^16 = 65536.

-	Network sets DRX reference SFN (drx-ReferenceSFN) to either 0 or 512, in the same way as in Rel-16 IIoT.

-	Use the following option (option A): both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to the DRX formula. NSFN is initialized to 0.

-	Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG.

-	When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time.

-	RAN2 aims to define a single MAC CE for the DSR reporting (including the buffer status). FFS if this extends BSR MAC CE or is a new MAC CE.

-	Many companies think single value per LCG is sufficient. Some companies think scheduler needs more information.

-	Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS.

-	Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS.

-	PDCP discard timer for PDU sets supports cases where PDUs of a PDU Set arrive at different instances of time.

-	Companies should bring detailed Stage-3 proposals, preferably co-signed by several supporters, to the next meeting, at which time RAN2 aims to decide on which solution to use.

-	Send LS to RAN1 (MTK) informing them of the error in the formula and tell RAN2 will capture the HARQ process formula in RAN2 specifications. Ask RAN1 whether the validity is going to be defined in RAN1 specifications. Can ask clarifications how the validity works if there are ambiguities for RAN2 specifications.

-	UE capabilities will be discussed in the next meeting(s) based on company inputs. Companies are encouraged to provide also Stage-3 details of their proposals, e.g. draftCRs on the capabilities to allow better comparison of the proposals.

-	Interested companies bringing documents to this AI should contact specification rapporteur to consolidate their proposals offline.