3GPP TSG RAN WG2 #123			R2-230xxxx

Toulouse, France, 21st – 26th August, 2022

Title:	LS on SL positioning MAC agreements

Response to:

Release:	Release 18

Work Item:	NR_pos_enh2

Source:	Huawei, HiSilicon (to be RAN2)

To:	RAN1, SA2

Cc:

Contact Person:

Name:	Yinghao Guo

E-mail Address:	yinghaoguo@huawei.com

Attachments: -

1. Overall Description:

During RAN2#123, the following agreements have been achieved for the SL-PRS priority

Define 8 priority levels for SL-PRS priority, same as the number of priority levels for SL-SCH. RAN2 understand that the SL-PRS priority levels are mapped from the sidelink positioning/ranging QoS

The following agreements have been achieved for resource allocation for SL-PRS

The SL-PRS priority can be provided by the UE’s own high layer when it triggers the SL-PRS transmission. When aperiodic/one-shot SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, design a new MAC CE for the UE to send to the gNB for SL-PRS resource request. At least when periodic SL-PRS transmission is triggered for UE configured with Scheme 1 SL-PRS resource allocation, at least for the case when LMF is not involved in giving the grant, the UE sends an RRC message to the gNB for providing the assistance information for CG configuration.

The following agreements have been achieved for CBR measurements

Support CBR measurement on both shared and dedicated resource pool for SL-PRS transmission.

2. Actions:

To RAN1/SA2:

ACTION: 	RAN2 respectfully asks RAN1/SA2 to take the agreements in SL-PRS priority in the future work and provide feedbacks on relevant agreements

To RAN1:

ACTION: 	RAN2 would like to ask RAN1 whether the following cases are possible

- Higher layer provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer’s signalling

- Lower layer signalling provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer signalling

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN WG2 Meeting R2-123-bis		Xiamen			2023-10-09	2023-10-13

TSG-RAN WG2 Meeting R2-124			Chicago			2023-11-13	2023-11-17