3GPP TSG-RAN WG2 Meeting #123	R2-230xxxx

# Toulouse, France,  August 21 – 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC running CR for Rel-18 multi-path |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Multi-path is supported in Rel-18, the corresponding description and siganalling are to be captured in RRC specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TBD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-path can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TBD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AM MRB: An MRB associated with at least an AM RLC bearer for PTP transmission.

BH RLC channel: An RLC channel between two nodes, which is used to transport backhaul packets.

Broadcast MRB: A radio bearer configured for MBS broadcast delivery.

CEIL: Mathematical function used to 'round up' i.e. to the nearest integer having a higher or equal value.

DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources.

Dedicated signalling: Signalling sent on DCCH logical channel between the network and a single UE.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signalling. In the dormant BWP, the UE stops monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured. For each serving cell other than the SpCell or PUCCH SCell, the network may configure one BWP as a dormant BWP.

Field: The individual contents of an information element are referred to as fields.

FLOOR: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Frequency Selection Area ID: An identity used for broadcast MBS session to guide the frequency selection of the UE as defined in TS 23.247 [67].

Global cell identity: An identity to uniquely identifying an NR cell. It is consisted of cellIdentity and plmn-Identity of the first PLMN-Identity in plmn-IdentityList in SIB1.

Information element: A structural element containing single or multiple fields is referred as information element.

MBS Radio Bearer: A radio bearer that is configured for MBS delivery.

Multicast/Broadcast Service: A point-to-multipoint service as defined in TS 23.247 [67].

Multicast MRB: A radio bearer configured for MBS multicast delivery.

MUSIM gap: Period that the UE may use to perform MUSIM operations.

Multi-path: Mode of operation of a remote UE in RRC_CONNECTED configured with a direct path on which the UE connects to network using NR Uu, and an indirect path on which the UE connects to network via another relay UE using L2 U2N relay operation or non-3GPP connectivity.

NCSG: Network controlled small gap as defined in TS 38.133 [14].

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

N3C indirect path: In multi-path, the indirect path on which the remote UE connects to network via another relay UE using non-3GPP connectivity.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [55], and ProSe Communication (including ProSe UE-to-Network Relay and non-Relay communication) as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery and ProSe UE-to-Network Relay discovery for Proximity based Services as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

PNI-NPN identity: an identifier of a PNI-NPN comprising of a PLMN ID and a CAG -ID combination.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

PC5 Relay RLC channel: An RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network relay.

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.

PUCCH SCell: An SCell configured with PUCCH by PUCCH-Config.

PUSCH-Less SCell: An SCell configured without PUSCH.

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [26].

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

Small Data Transmission: A procedure used for transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state (i.e. without the UE transitioning to RRC_CONNECTED state).

SNPN identity: an identifier of an SNPN comprising of a PLMN ID and an NID combination.

SL indirect path: In Multi-path, the indirect path on which the remote UE connects to network via another relay UE using L2 U2N relay operation.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

Split DRB: In MR-DC or MP, a DRB that supports transmission via MCG (i.e. direct path in MP) and SCG/indirect path in MP, as well as duplication of PDCP PDUs as defined in TS 37.340 [41].

Split SRB: In MR-DC or MP, an SRB that supports transmission via MCG (i.e. direct path in MP) and SCG/indirect path in MP, as well as duplication of RRC PDUs as defined in TS 37.340 [41].

SSB Frequency: Frequency referring to the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block.

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: A UE that communicates with the network via a U2N Relay UE.

Uu Relay RLC channel: An RLC channel between L2 U2N Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network relay.

UE Inactive AS Context: UE Inactive AS Context is stored when the connection is suspended and restored when the connection is resumed. It includes information as defined in clause 5.3.8.3.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [56], between nearby UEs, using E-UTRA technology but not traversing any network node.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BAP	Backhaul Adaptation Protocol

BCD	Binary Coded Decimal

BFD	Beam Failure Detection

BH	Backhaul

BLER	Block Error Rate

BWP	Bandwidth Part

CA	Carrier Aggregation

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CAPC	Channel Access Priority Class

CBR	Channel Busy Ratio

CCCH	Common Control Channel

CFR	Common Frequency Resources

CG	Cell Group

CHO	Conditional Handover

CLI	Cross Link Interference

CMAS	Commercial Mobile Alert Service

CP	Control Plane

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

C-RNTI	Cell RNTI

CSI	Channel State Information

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DFN	Direct Frame Number

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

DL-SCH	Downlink Shared Channel

DM-RS	Demodulation Reference Signal

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered Inertial

EN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to EPC

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRA/EPC	E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

G-CS-RNTI	Group Configured Scheduling RNTI

GERAN	GSM/EDGE Radio Access Network

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

G-RNTI	Group RNTI

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

HRNN	Human Readable Network Name

HSDN	High Speed Dedicated Network

H-SFN	Hyper SFN

IAB	Integrated Access and Backhaul

IAB-DU	IAB-node DU

IAB-MT	IAB Mobile Termination

IDC	In-Device Coexistence

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

LBT	Listen Before Talk

LEO	Low Earth Orbit

MAC	Medium Access Control

MBS	Multicast/Broadcast Service

MBS FSAI	MBS Frequency Selection Area Identity

MCCH	MBS Control Channel

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MIB	Master Information Block

MP	Multi-path

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MR-DC	Multi-Radio Dual Connectivity

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MUSIM	Multi-Universal Subscriber Identity Module

N3C	Non-3GPP Connection

N/A	Not Applicable

NE-DC	NR E-UTRA Dual Connectivity

(NG)EN-DC	E-UTRA NR Dual Connectivity (covering E-UTRA connected to EPC or 5GC)

NGEN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to 5GC

NID	Network Identifier

NPN	Non-Public Network

NR-DC	NR-NR Dual Connectivity

NR/5GC	NR connected to 5GC

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

posSIB	Positioning SIB

PPW	PRS Processing Window

PRS	Positioning Reference Signal

PSCell	Primary SCG Cell

PTM	Point to Multipoint

PTP	Point to Point

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RLM	Radio Link Monitoring

RMTC	RSSI Measurement Timing Configuration

RNA	RAN-based Notification Area

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

RS	Reference Signal

SBAS	Satellite Based Augmentation System

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SD-RSRP	Sidelink Discovery RSRP

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SLSS	Sidelink Synchronisation Signal

SNPN	Stand-alone Non-Public Network

SpCell	Special Cell

SRAP	Sidelink Relay Adaptation Protocol

SRB	Signalling Radio Bearer

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDD	Time Division Duplex

TEG	Timing Error Group

TM	Transparent Mode

TMGI	Temporary Mobile Group Identity

U2N	UE-to-Network

UDC	Uplink Data Compression

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

VR	Virtual Reality

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

NEXT CHANGE

### 4.2.2	Signalling radio bearers

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

-	SRB0 is for RRC messages using the CCCH logical channel;

-	SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel;

-	SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel. SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation;

-	SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel;

-	SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation.

In downlink, piggybacking of NAS messages is used only for one dependant (i.e. with joint success/failure) procedure: bearer establishment/modification/release. In uplink piggybacking of NAS message is used only for transferring the initial NAS message during connection setup and connection resume.

NOTE 1:	The NAS messages transferred via SRB2 are also contained in RRC messages, which however do not include any RRC protocol control information.

Once AS security is activated, all RRC messages on SRB1, SRB2, SRB3 and SRB4, including those containing NAS messages, are integrity protected and ciphered by PDCP. NAS independently applies integrity protection and ciphering to the NAS messages, see TS 24.501 [23].

Split SRB is supported for all the MR-DC options as well as MP in both SRB1 and SRB2 (split SRB is not supported for SRB0, SRB3 and SRB4).

For operation with shared spectrum channel access in FR1, SRB0, SRB1 and SRB3 are assigned with the highest priority Channel Access Priority Class (CAPC), (i.e. CAPC = 1) while CAPC for SRB2 is configurable.

NEXT CHANGE

## 4.4	Functions

The RRC protocol includes the following main functions:

-	Broadcast of system information:

-	Including NAS common information;

-	Information applicable for UEs in RRC_IDLE and RRC_INACTIVE (e.g. cell (re-)selection parameters, neighbouring cell information) and information (also) applicable for UEs in RRC_CONNECTED (e.g. common channel configuration information);

-	Including ETWS notification, CMAS notification;

-	Including positioning assistance data.

-	RRC connection control:

-	Paging;

-	Establishment/modification/suspension/resumption/release of RRC connection, including e.g. assignment/modification of UE identity (C-RNTI, fullI-RNTI, etc.), establishment/modification/suspension/resumption/release of SRBs (except for SRB0);

-	Access barring;

-	Initial AS security activation, i.e. initial configuration of AS integrity protection (SRBs, DRBs) and AS ciphering (SRBs, DRBs);

-	RRC connection mobility including e.g. intra-frequency and inter-frequency handover, path switch from a PCell to a target L2 U2N Relay UE or from a L2 U2N Relay UE to a target PCell, associated AS security handling, i.e. key/algorithm change, specification of RRC context information transferred between network nodes;

-	Establishment/modification/suspension/resumption/release of RBs carrying user data (DRBs/MRBs);

-	Radio configuration control including e.g. assignment/modification of ARQ configuration, HARQ configuration, DRX configuration;

-	In case of DC, cell management including e.g. change of PSCell, addition/modification/release of SCG cell(s);

-	In case of CA, cell management including e.g. addition/modification/release of SCell(s);

-	In case of MP, path management including e.g. indirect path addition/modification/release;

-	QoS control including assignment/ modification of semi-persistent scheduling (SPS) configuration and configured grant configuration for DL and UL respectively, assignment/ modification of parameters for UL rate control in the UE, i.e. allocation of a priority and a prioritised bit rate (PBR) for each RB of UE and logical channel of IAB-MT.

-	Recovery from radio link failure.

-	Inter-RAT mobility including e.g. AS security activation, transfer of RRC context information;

-	Measurement configuration and reporting:

-	Establishment/modification/release of measurement configuration (e.g. intra-frequency, inter-frequency and inter- RAT measurements);

-	Setup and release of measurement gaps;

-	Measurement reporting.

-	Configuration of BAP entity and BH RLC channels for the support of IAB-node.

-	Configuration of SRAP entity and Uu/PC5 Relay RLC channels for the support of L2 U2N relay.

-	Other functions including e.g. generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information.

-	Support of self-configuration and self-optimisation.

-	Support of measurement logging and reporting for network performance optimisation, as specified in TS 37.320 [61];

-	Support of transfer of application layer measurement configuration and reporting.

NEXT CHANGE

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the establishment of BH RLC Channels for IAB is performed only when AS security has been activated;

-	the establishment of Uu Relay RLC channels and PC5 Relay RLC channels (other than SL-RLC0 and SL-RLC1) for L2 U2N Relay UE is performed only when AS security has been activated, and the establishment of PC5 Relay RLC channels for L2 U2N Remote UE (other than SL-RLC0 and SL-RLC1) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one RLC bearer or BH RLC channel is setup in SCG;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB or multicast MRB or, for IAB, SRB2, are setup and not suspended;

-	the conditionalReconfiguration for CPC is included only when at least one RLC bearer is setup in SCG;

-	the conditionalReconfiguration for CHO or CPA is included only when AS security has been activated, and SRB2 with at least one DRB or multicast MRB or, for IAB, SRB2, are setup and not suspended;

-	the addition and change of indirect path for MP is performed only when AS security has been activated.

NEXT CHANGE

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

1>	if the RRCReconfiguration includes the daps-SourceRelease:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR):

2>	if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover):

3>	re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message);

1>	else:

2>	if the RRCReconfiguration includes the fullConfig:

3>	perform the full configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform AS security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

NOTE 0:	If this RRCReconfiguration is associated to the MCG and includes reconfigurationWithSync in spCellConfig and dedicatedSIB1-Delivery, the UE initiates (if needed) the request to acquire required SIBs, according to clause 5.2.2.3.5, only after the random access procedure towards the target SpCell is completed.

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the dedicatedPosSysInfoDelivery:

2>	perform the action upon reception of the contained posSIB(s), as specified in clause 5.2.2.4.16;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.5.9;

1>	if the RRCReconfiguration message includes the bap-Config:

2>	perform the BAP configuration procedure as specified in 5.3.5.12;

1>	if the RRCReconfiguration message includes the iab-IP-AddressConfigurationList:

2>	if iab-IP-AddressToReleaseList is included:

3>	perform release of IP address as specified in 5.3.5.12a.1.1;

2>	if iab-IP-AddressToAddModList is included:

3>	perform IAB IP address addition/update as specified in 5.3.5.12a.1.2;

1>	if the RRCReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.13;

1>	if the RRCReconfiguration message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR:

2>	if needForGapNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA:

2>	if needForGapNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	if the RRCReconfiguration message includes the onDemandSIB-Request:

2>	if onDemandSIB-Request is set to setup:

3>	consider itself to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

2>	else:

3>	consider itself not to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

NOTE 0a:	If the sl-ConfigDedicatedNR was received embedded within an E-UTRA RRCConnectionReconfiguration message, the UE does not build an NR RRCReconfigurationComplete message for the received sl-ConfigDedicatedNR.

1>	if the RRCReconfiguration message includes the sl-L2RelayUE-Config:

2>	perform the L2 U2N Relay UE configuration procedure as specified in 5.3.5.15;

1>	if the RRCReconfiguration message includes the sl-L2RemoteUE-Config:

2>	perform the L2 U2N Remote UE configuration procedure as specified in 5.3.5.16;

1>	if the RRCReconfiguration message includes the dedicatedPagingDelivery:

2>	perform the Paging message reception procedure as specified in 5.3.2.3;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedEUTRA-Info:

2>	perform related procedures for V2X sidelink communication in accordance with TS 36.331 [10], clause 5.3.10 and clause 5.5.2;

1>	if the RRCReconfiguration message includes the ul-GapFR2-Config:

2>	perform the FR2 UL gap configuration procedure as specified in 5.3.5.13c;

1>	if the RRCReconfiguration message includes the musim-GapConfig:

2>	perform the MUSIM gap configuration procedure as specified in 5.3.5.9a;

1>	if the RRCReconfiguration message includes the appLayerMeasConfig:

2>	perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

1>	if the RRCReconfiguration message includes the ue-TxTEG-RequestUL-TDOA-Config:

2>	if ue-TxTEG-RequestUL-TDOA-Config is set to setup:

3>	perform the UE positioning assistance information procedure as specified in 5.7.14;

2>	else:

3>	release the configuration of UE positioning assistance information;

1>	if the RRCReconfiguration message includes the sl-IndirectPathAddChange:

2>	perform the SL indirect path specific configuration procedure as specified in 5.3.5.xx.1.2;

1>	if the RRCReconfiguration message includes the n3c-IndirectPathConfigRemote:

2>	perform configuration procedure for the remote UE part of N3C indirect path as specified in 5.3.5.xx.2.2;

1>	if the RRCReconfiguration message includes the n3c-IndirectPathConfigRelay:

2>	perform the configuration procedure for the relay UE part of N3C indirect path as specified in 5.3.5.xx.2.3;

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

NOTE 0b:	The UE does not expect that the reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier is received in both masterCellGroup and in secondaryCellGroup. Network only configures at most one of reportUplinkTxDirectCurrent, reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier in one RRC message.

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

3>	if the RRCReconfiguration message is applied due to conditional reconfiguration execution and the RRCReconfiguration message does not include the reconfigurationWithSync in the masterCellGroup:

4>	include in the selectedCondRRCReconfig the condReconfigId for the selected cell of conditional reconfiguration execution;

2>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG:

3>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include the logMeasAvailable in the RRCReconfigurationComplete message;

4>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableBT in the RRCReconfigurationComplete message;

4>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableWLAN in the RRCReconfigurationComplete message;

3>	if the sigLoggedMeasType in VarLogMeasReport is included:

4>	if T330 timer is running and the logged measurements configuration is for NR:

5>	set sigLogMeasConfigAvailable to true in the RRCReconfigurationComplete message;

4>	else:

5>	if the UE has logged measurements available for NR:

6>	set sigLogMeasConfigAvailable to false in the RRCReconfigurationComplete message;

3>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

4>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

4>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE was configured with successHO-Config when connected to the source PCell; and

3>	if the applied RRCReconfiguration is not due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

4>	perform the actions for the successful handover report determination as specified in clause 5.7.10.6, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the MCG;

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report:

4>	include successHO-InfoAvailable in the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was received via SRB1, but not within mrdc-SecondaryCellGroup or E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume:

3>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapsConfigNR; or

4>	if the NeedForGapsInfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapsInfoNR and set the contents as follows:

6>	include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNR is configured:

7>	for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band;

6>	else:

7>	include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR; or

4>	if the needForGapNCSG-InfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoNR and set the contents as follows:

6>	include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNCSG-NR is configured:

7>	for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band;

6>	else:

7>	include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA; or

4>	if the needForGapNCSG-InfoEUTRA information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoEUTRA and set the contents as follows:

6>	if requestedTargetBandFilterNCSG-EUTRA is configured, for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band; otherwise, include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or

2>	if the RRCReconfiguration message was received via E-UTRA RRC message RRCConnectionReconfiguration within MobilityFromNRCommand (handover from NR standalone to (NG)EN-DC);

3>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SecondaryCellGroupConfig specified in TS 36.331 [10]:

4>	submit the RRCReconfigurationComplete message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], clause 5.6.2a.

3>	else if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

4>	submit the RRCReconfigurationComplete message via E-UTRA embedded in E-UTRA RRC message RRCConnectionResumeComplete as specified in TS 36.331 [10], clause 5.3.3.4a;

3>	else:

4>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4/5.4.2.3;

3>	if the scg-State is not included in the E-UTRA message (RRCConnectionReconfiguration or RRCConnectionResume) containing the RRCReconfiguration message:

4>	perform SCG activation as specified in 5.3.5.13a;

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else if the SCG was deactivated before the reception of the E-UTRA RRC message containing the RRCReconfiguration message:

5>	if bfd-and-RLM was not configured to true before the reception of the E-UTRA RRCConnectionReconfiguration or RRCConnectionResume message containing the RRCReconfiguration message or if lower layers indicate that a Random Access procedure is needed for SCG activation:

6>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

5>	else the procedure ends;

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

2>	if the RRCReconfiguration message was received within nr-SecondaryCellGroupConfig in RRCConnectionReconfiguration message received via SRB3 within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4;

3>	if the scg-State is not included in the RRCConnectionReconfiguration:

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

NOTE 1:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2>	else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE 2:	In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1 or within DLInformationTransferMRDC via SRB3, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3 but not within DLInformationTransferMRDC, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SCG within mrdc-SecondaryCellGroup:

3>	submit the RRCReconfigurationComplete message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

2>	if the scg-State is not included in the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message:

3>	perform SCG activation as specified in 5.3.5.13a;

3>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

4>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

3>	else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message:

4>	if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or

4>	if lower layers indicate that a Random Access procedure is needed for SCG activation:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else the procedure ends;

2>	else

3>	perform SCG deactivation as specified in 5.3.5.13b;

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>	if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

4>	if the scg-State is not included in the RRCReconfiguration message containing the RRCReconfiguration message:

5>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

6>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

5>	else:

6>	the procedure ends;

4>	else:

5>	perform SCG deactivation as specified in 5.3.5.13b;

5>	the procedure ends;

3>	else:

4>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

5>	if the RRCReconfiguration includes the scg-State:

6>	perform SCG deactivation as specified in 5.3.5.13b;

4>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	else:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>	else (RRCReconfiguration was received via SRB1):

2>	if the UE is in NR-DC and;

2>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

3>	if the RRCReconfiguration includes the scg-State:

4>	perform SCG deactivation as specified in 5.3.5.13b;

3>	else:

4>	perform SCG activation without SN message as specified in 5.3.5.13b1;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if ta-Report is configured with value enabled and the UE supports TA reporting:

4>	indicate TA report initiation to lower layers;

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2, SRB4, DRBs, multicast MRB, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, that are suspended;

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1>	if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE):

2>	stop timer T304 for that cell group if running;

2>	if sl-PathSwitchConfig was included in reconfigurationWithSync:

3>	stop timer T420;

3>	release all radio resources, including release of the RLC entities and the MAC configuration at the source side;

3>	reset MAC used in the source cell;

NOTE 2b:	PDCP and SDAP configured by the source prior to the path switch that are reconfigured and re-used by target when delta signalling is used, are not released as part of this procedure.

2>	stop timer T310 for source SpCell if running;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	for each DRB configured as DAPS bearer, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if T350 is running:

4>	stop timer T350;

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG; or

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPA or CPC was configured:

3>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

3>	remove all the entries within VarConditionalReconfiguration as specified in TS 36.331 [10], clause 5.3.5.9.6, if any;

3>	for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	for the associated reportConfigId:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) or the leave without response timer for the MUSIM associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

3>	if SIB12 is provided by the target PCell, and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication/discovery related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the UE is capable of NR sidelink communication/discovery and SIB12 is provided by the target PCell, and the UE has initiated transmission of a SidelinkUEInformationNR message since it was configured to do so in accordance with 5.8.3.2:

4>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

2>	if reconfigurationWithSync was included in masterCellGroup:

3>	if configured with application layer measurements and if application layer measurement report container has been received from upper layers for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers:

4>	re-submit the MeasurementReportAppLayer message or all segments of the MeasurementReportAppLayer message to lower layers for transmission via SRB4;

2>	if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21:

3>	if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message:

4>	initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 4: The UE sets the content of UEAssistanceInformation according to latest configuration (i.e. the configuration after applying the RRCReconfiguration message) and latest UE preference. The UE may include more than the concerned UE assistance information within the UEAssistanceInformation according to 5.7.4.2. Therefore, the content of UEAssistanceInformation message might not be the same as the content of the previous UEAssistanceInformation message.

NEXT CHANGE

##### 5.3.5.5.12	Uu Relay RLC channel release

The L2 U2N Relay UE or relay UE on N3C indirect path shall:

1>	for each Uu-RelayRLC-ChannelID value included in the uu-RelayRLC-ChannelToReleaseList that is part of the current configuration within the same cell group (LCH release):

2>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.13	Uu Relay RLC channel addition/modification

For each Uu-RelayRLC-ChannelConfig received in the uu-RelayRLC-ChannelToAddModList the L2 U2N Relay UE or relay UE on N3C indirect path shall:

1>	if the current configuration contains a Uu Relay RLC channel with the same uu-RelayRLC-ChannelID within the same cell group:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

1>	else (a logical channel with the given uu-RelayRLC-ChannelID was not configured before within the same cell group):

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig.

NEXT CHANGE

#### 5.3.5.xx	MP configuration

5.3.5.xx.0	Introduction

In case of MP, a remote UE is configured with one direct path (i.e. MCG) and one indirect path. The remote UE connects to network on the direct path using NR Uu, and connects to network via a relay UE on indirect path using L2 U2N relay operation or using non-3GPP connection.

On indirect path, for the hop between the remote UE and the relay UE, when the remote UE connects to a L2 U2N Relay UE, the network provides the configuration of SL indirect path to the remote UE and the L2 U2N Relay UE as specified in 5.3.5.xx.1; when the remote UE connects to a relay UE using non-3GPP connection, the network provides the configuration of N3C indirect path to the remote UE and the relay UE as specified in 5.3.5.xx.2.

##### 5.3.5.xx.1	Configuration of SL indirect path

###### 5.3.5.xx.1.1 General

To configure SL indirect path, the network provides:

-	sidelink dedicated configuration as specified in 5.3.5.14, L2 U2N Remote UE configuration as specified in 5.3.5.16, and SL indirect path specific configuration as specified in 5.3.5.xx.1.2, to the L2 U2N Remote UE;

-	sidelink dedicated configuration as specified in 5.3.5.14, L2 U2N Relay UE configuration as specified in 5.3.5.15, as well as Uu Relay RLC channel as specified in 5.3.5.5.12 and 5.3.5.5.13, to the L2 U2N Relay UE.

###### 5.3.5.xx.1.2 SL indirect path specific configuration

The L2 U2N Remote UE shall:

1>	if sl-IndirectPathAddChange is set to setup:

2>	consider the relay UE indicated by the sl-IndirectPathRelayUE-Identity to be the (target) L2 U2N Relay UE on SL indirect path and indicate to upper layer (to trigger the PC5 unicast link establishment) with the L2 U2N Relay UE;

2>	start timer [T4xx] for the corresponding L2 U2N Relay UE with the timer value set to [T4xx];

2>	indicate to upper layer (to trigger the PC5 unicast link release) with the source L2 U2N Relay UE if any (i.e. SL indirect path change);

1>	else if sl-IndirectPathAddChange is set to release:

2>	consider the SL indirect path is released and release the corresponding configurations;

2>	indicate to upper layer (to trigger the PC5 unicast link release) with the L2 U2N Relay UE.

##### 5.3.5.xx.2	Configuration of N3C indirect path

###### 5.3.5.xx.2.1 General

To configure N3C indirect path, the network provides:

-	remote UE part of non-3GPP indirect path configuration including relay UE identification as specified in 5.3.5.xx.2.2, to the remote UE;

-	relay UE part of non-3GPP indirect path configuration including bearer mapping configurations as specified in 5.3. 5.xx.2.3, as well as Uu Relay RLC channel as specified in 5.3.5.5.12 and 5.3.5.5.13, to the relay UE.

NOTE: The data transmission/reception between the remote UE and the relay UE via the non-3GPP connection is left to UE implementation.

###### 5.3.5.xx.2.2 Remote UE part of N3C indirect path configuration

The remote UE shall:

1>	if n3c-IndirectPathConfigRemote is set to setup:

2>	consider the non-3GPP connection with the relay UE indicated by the n3c-RelayIdentification to be used for the indirect path;

1>	else if n3c-IndirectPathConfigRemote is set to release:

2>	consider the indirect path is not to be used and release the corresponding configuration.

###### 5.3.5.xx.2.3 Relay UE part of N3C indirect path configuration

The relay UE shall:

1>	if n3c-IndirectPathConfigRelay is set to setup:

2>	if n3c-MappingToReleaseList is included:

3>	perform release of N3C bearer mapping configuration as specified in 5.3.5.xx.2.4.1;

2>	if n3c-MappingToAddModList is included:

3>	perform N3C bearer mapping addition/modification as specified in 5.3.5.xx.2.4.2;

1>	else if n3c-IndirectPathConfigRelay is set to release:

2>	consider the indirect path of the remote UE is released and release the corresponding configuration.

###### 5.3.5.xx.2.4 Bearer mapping management on N3C indirect path

5.3.5.xx.2.4.1 Bearer mapping release

The UE shall:

1>	if the release is triggered by reception of the n3c-MappingToReleaseList:

2>	for each SL-RemoteUE-RB-Identity value included in n3c-MappingToReleaseList:

3>	release the corresponding n3c-EgressRLC-ChannelUu and n3c-RemoteUE-RB-Identity;

5.3.5.xx.2.4.2 Bearer mapping addition and modification

The UE shall:

1>	for each n3c-RemoteUE-RB-Identity value included in the n3c-MappingToAddModList that is not part of the current UE configuration:

2>	associate the remote UE’s RB indicated in n3c-RemoteUE-RB-Identity to the Uu RLC channel indicated in n3c-EgressRLC-ChannelUu;

1>	for the n3c-RemoteUE-RB-Identity value included in the n3c-MappingToAddModList that is part of the current UE configuration:

2>	update the associated Uu RLC channel to the one indicated in n3c-EgressRLC-ChannelUu.

NEXT CHANGE

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while the SCG is deactivated, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with clause 5.3.5.8.2; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with clause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.11.3 in NE-DC; or

1>	upon reconfiguration with sync failure of the SCG while MCG transmission is suspended in accordance with clause 5.3.5.8.3; or

1>	upon SCG change failure while MCG transmission is suspended in accordance with TS 36.331 [10] clause 5.3.5.7a; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with clause 5.3.5.8.2 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.5.5 in NE-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG is suspended; or

1>	upon T316 expiry, in accordance with clause 5.7.3b.5; or

1>	upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.3; or

1>	upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.10; or

1>	upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED while T301 is not running;

1>	if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with clause 5.3.10 while the transmission of indirect path is suspended as specified in 5.xyy; or

1>	if MP is configured, upon detecting sidelink radio link failure of SL indirect path by L2 U2N Remote UE, in accordance with clause 5.8.9.3, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon reception of NotificationMessageSidelink including indicationType in accordance with clause 5.8.9.10, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

Editor’s Note: FFS how to handle relayUE-HO.

1>	if MP is configured, upon detecting the failure of N3C indirect path by remote UE in accordance with clause 5.xxx, while MCG transmission (i.e. direct path) is suspended.

NOTE 0:	It is up to UE implementation whether to initiate the procedure while T346g is running.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	if UE is not configured with attemptCondReconfig:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

2>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

2>	release musim-LeaveAssistanceConfig, if configured;

2>	release ul-GapFR2-PreferenceConfig, if configured;

2>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

2>	release propDelayDiffReportConfig, if configured;

2>	release rrm-MeasRelaxationReportingConfig, if configured;

2>	release maxBW-PreferenceConfigFR2-2, if configured;

2>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

2>	release minSchedulingOffsetPreferenceConfigExt, if configured;

1>	release successHO-Config, if configured;

1>	if any DAPS bearer is configured:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	release sl-L2RelayUE-Config, if configured;

1>	release sl-L2RemoteUE-Config, if configured;

1>	release the SRAP entity, if configured;

1>	if MP via L2 U2N Relay UE is configured:

2>	release sl-IndirectPathAddChange;

2> indicate upper layers to trigger PC5 unicast link release of the SL indirect path;

1>	if MP via non-3GPP connection is configured:

2>	release n3c-IndirectPathConfigRemote;

2> consider the non-3GPP connection is not used;

1>	if the UE is acting as a relay UE on N3C indirect path:

2>	release n3c-IndirectPathConfigRelay;

2> consider the non-3GPP connection is not used;

1>	if the UE is acting as L2 U2N Remote UE and MP via L2 U2N Relay UE is not configured:

2>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

3>	indicate upper layers to trigger PC5 unicast link release;

3>	perform either cell selection in accordance with the cell selection process as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else (i.e., maintain the PC5 RRC connection):

3>	consider the connected L2 U2N Relay UE as suitable and perform actions as specified in clause 5.3.7.3a;

NOTE 1:	It is up to Remote UE implementation whether to release or keep the current PC5 unicast link.

1> else (i.e. not acting as L2 U2N Remote UE or acting L2 U2N Remote UE on SL indirect path when MP is configured):

2>	if the UE is capable of L2 U2N Remote UE:

3>	perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20].

NOTE 2:	For L2 U2N Remote UE, if both a suitable cell and a suitable relay are available, the UE can select either one based on its implementation.

NEXT CHANGE

#### 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	if timer T316 is running;

2>	stop timer T316;

2>	clear the information included in VarRLF-Report, if any;

1>	stop timer T350, if running;

1>	stop timer T346g, if running;

1>	if the AS security is not activated:

2>	ignore any field included in RRCRelease message except waitTime;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

2>	if voiceFallbackIndication is included:

3>	consider the RRC connection release was for EPS fallback for IMS voice (see TS 23.502 [43]);

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

NOTE 1a:	The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in NR or other RATs unless specified otherwise.

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	if the measIdleConfig contains validityAreaList:

4>	store the received validityAreaList in VarMeasIdleConfig;

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer (except those associated with broadcast MRBs) that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

2>	if srs-PosRRC-Inactive is configured:

3>	apply the configuration and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer;

NOTE 1b:	The Network should provide full configuration to UE for SRS for Positioning in RRC_INACTIVE.

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

2>	for each measId of the MCG measConfig and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if the UE is acting as L2 U2N Remote UE and is not configured with MP:

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	indicate upper layers to trigger PC5 unicast link release;

3>	else (i.e., maintain the PC5 RRC connection):

4>	establish or re-establish (e.g. via release and add) SL RLC entity for SRB1;

2>	else:

3>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

5>	replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

NOTE 1c:	suspendConfig is not stored as part of UE Inactive AS Context, except for the fields explicitly specified.

3>	store any previously or subsequently received application layer measurement report containers for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication/discovery related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	release Uu Relay RLC channel(s), if configured;

2>	release PC5 Relay RLC channel(s), if configured;

2>	release the SRAP entity, if configured;

2>	if MP via L2 U2N Relay UE is configured:

3>	release sl-IndirectPathAddChange;

3> indicate upper layers to trigger PC5 unicast link release of the SL indirect path;

2>	if MP via non-3GPP connection is configured:

3>	release n3c-IndirectPathConfigRemote;

3> consider the non-3GPP connection is not used;

2>	if the UE is acting as a relay UE on N3C indirect path:

3>	release n3c-IndirectPathConfigRelay;

3> consider the non-3GPP connection is not used;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_INACTIVE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

NOTE 3:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 4:	It is left to UE implementation whether to stop T430, if running, when going to RRC_INACTIVE.

NEXT CHANGE

### 5.7.3b	MCG failure information

#### 5.7.3b.1	General

Figure 5.7.3b.1-1: MCG failure information

The purpose of this procedure is to inform the network about an MCG failure the UE has experienced i.e. MCG radio link failure. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB or multicast MRB setup or, for IAB, SRB2, may initiate the fast MCG link recovery procedure in order to continue the RRC connection without re-establishment.

#### 5.7.3b.2	Initiation

In case of MR-DC, a UE configured with split SRB1 or SRB3 initiates the procedure to report MCG failures when neither MCG nor SCG transmission is suspended, the SCG is not deactivated, t316 is configured, and when the following condition is met.

In case of MP, a remote UE configured with split SRB1 initiates the procedure to report direct path failures when neither direct path nor indirect path transmission is suspended, t316 is configured, and when the following condition is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10.3, while T316 is not running.

Upon initiating the procedure, the UE shall:

1>	stop timer T310 for the PCell, if running;

1>	stop timer T312 for the PCell, if running;

1>	suspend MCG transmission for all SRBs, DRBs, multicast MRBs, except SRB0, and, if any, BH RLC channels;

1>	reset MCG MAC;

1>	stop conditional reconfiguration evaluation for CHO, if configured;

1>	stop conditional reconfiguration evaluation for CPC, if configured;

1>	initiate transmission of the MCGFailureInformation message in accordance with 5.7.3b.4.

NOTE:	The handling of any outstanding UL RRC messages during the initiation of the fast MCG link recovery is left to UE implementation.

NEXT CHANGE

##### 5.8.9.10.4	Actions related to reception of NotificationMessageSidelink message

Upon receiving the NotificationMessageSidelink, the U2N Remote UE shall:

1>	if the indicationType is included:

2>	if the UE is L2 U2N Remote UE in RRC_CONNECTED:

3> if MP is configured and MCG transmission (i.e. direct path) is not suspended as specified in clause 5.7.3b;

4> initiate the indirect path failure information procedure as specified in 5.xyy to report indirect path failure;

3>	else if T301 is not running:

4> initiate the RRC connection re-establishment procedure as specified in 5.3.7;

2>	else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE):

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	indicate upper layers to trigger PC5 unicast link release;

3>	else (i.e., maintain the PC5 RRC connection):

4>	if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection:

5>	consider cell re-selection occurs;

NOTE 1:	For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link.

NOTE 2:	The L2 U2N Remote UE may ignore the NotificationMessageSidelink if it does not release the PC5 unicast link in source side yet during an indirect-to-direct path switch, i.e. T304 is running.

NEXT CHANGE

NEXT CHANGE

### 6.2.2	Message definitions

<Omit unrelated part>

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                                                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs                                           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=         SEQUENCE {

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

sk-Counter                               SK-Counter                                                            OPTIONAL,   -- Need N

nonCriticalExtension                     RRCReconfiguration-v1610-IEs                                          OPTIONAL

}

RRCReconfiguration-v1610-IEs ::=        SEQUENCE {

otherConfig-v1610                       OtherConfig-v1610                                                    OPTIONAL, -- Need M

bap-Config-r16                          SetupRelease { BAP-Config-r16 }                                      OPTIONAL, -- Need M

iab-IP-AddressConfigurationList-r16     IAB-IP-AddressConfigurationList-r16                                  OPTIONAL, -- Need M

conditionalReconfiguration-r16          ConditionalReconfiguration-r16                                       OPTIONAL, -- Need M

daps-SourceRelease-r16                  ENUMERATED{true}                                                     OPTIONAL, -- Need N

t316-r16                                SetupRelease {T316-r16}                                              OPTIONAL, -- Need M

needForGapsConfigNR-r16                 SetupRelease {NeedForGapsConfigNR-r16}                               OPTIONAL, -- Need M

onDemandSIB-Request-r16                 SetupRelease { OnDemandSIB-Request-r16 }                             OPTIONAL, -- Need M

dedicatedPosSysInfoDelivery-r16         OCTET STRING (CONTAINING PosSystemInformation-r16-IEs)               OPTIONAL, -- Need N

sl-ConfigDedicatedNR-r16                SetupRelease {SL-ConfigDedicatedNR-r16}                              OPTIONAL, -- Need M

sl-ConfigDedicatedEUTRA-Info-r16        SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16}                      OPTIONAL, -- Need M

targetCellSMTC-SCG-r16                  SSB-MTC                                                              OPTIONAL, -- Need S

nonCriticalExtension                    RRCReconfiguration-v1700-IEs                                         OPTIONAL

}

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

otherConfig-v1700                       OtherConfig-v1700                                              OPTIONAL, -- Need M

sl-L2RelayUE-Config-r17                 SetupRelease { SL-L2RelayUE-Config-r17 }                       OPTIONAL, -- Need M

sl-L2RemoteUE-Config-r17                SetupRelease { SL-L2RemoteUE-Config-r17 }                      OPTIONAL, -- Need M

dedicatedPagingDelivery-r17             OCTET STRING (CONTAINING Paging)                               OPTIONAL, -- Cond PagingRelay

needForGapNCSG-ConfigNR-r17             SetupRelease {NeedForGapNCSG-ConfigNR-r17}                     OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17          SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                  OPTIONAL, -- Need M

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

ul-GapFR2-Config-r17                    SetupRelease { UL-GapFR2-Config-r17 }                          OPTIONAL, -- Need M

scg-State-r17                           ENUMERATED { deactivated }                                     OPTIONAL, -- Need N

appLayerMeasConfig-r17                  AppLayerMeasConfig-r17                                         OPTIONAL, -- Need M

ue-TxTEG-RequestUL-TDOA-Config-r17      SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17}              OPTIONAL,  -- Need M

nonCriticalExtension                    RRCReconfiguration-v18xx-IEs                                                    OPTIONAL

}

RRCReconfiguration-v18xx-IEs ::=        SEQUENCE {

sl-IndirectPathAddChange-r18                  SetupRelease { SL-IndirectPathAddChange-r18 }                           OPTIONAL,  -- Need M

n3c-IndirectPathConfigRemote-r18           SetupRelease { N3C-IndirectPathConfigRemote-r18 }                    OPTIONAL,  -- Need M

n3c-IndirectPathConfigRelay-r18            SetupRelease { N3C-IndirectPathConfigRelay-r18 }                     OPTIONAL,  -- Need M

nonCriticalExtension                       SEQUENCE {}                                                    OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

BAP-Config-r16 ::=                      SEQUENCE {

bap-Address-r16                         BIT STRING (SIZE (10))                                    OPTIONAL, -- Need M

defaultUL-BAP-RoutingID-r16             BAP-RoutingID-r16                                         OPTIONAL, -- Need M

defaultUL-BH-RLC-Channel-r16            BH-RLC-ChannelID-r16                                      OPTIONAL, -- Need M

flowControlFeedbackType-r16             ENUMERATED {perBH-RLC-Channel, perRoutingID, both}        OPTIONAL, -- Need R

...

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

OnDemandSIB-Request-r16 ::=                  SEQUENCE {

onDemandSIB-RequestProhibitTimer-r16         ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30}

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms1000, ms1500, ms2000}

IAB-IP-AddressConfigurationList-r16 ::= SEQUENCE {

iab-IP-AddressToAddModList-r16      SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressConfiguration-r16 OPTIONAL, -- Need N

iab-IP-AddressToReleaseList-r16     SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressIndex-r16         OPTIONAL, -- Need N

...

}

IAB-IP-AddressConfiguration-r16 ::=     SEQUENCE {

iab-IP-AddressIndex-r16                 IAB-IP-AddressIndex-r16,

iab-IP-Address-r16                      IAB-IP-Address-r16                                                OPTIONAL,  -- Need M

iab-IP-Usage-r16                        IAB-IP-Usage-r16                                                  OPTIONAL,  -- Need M

iab-donor-DU-BAP-Address-r16            BIT STRING (SIZE(10))                                             OPTIONAL,  -- Need M

...

}

SL-ConfigDedicatedEUTRA-Info-r16 ::=            SEQUENCE {

sl-ConfigDedicatedEUTRA-r16                    OCTET STRING                                              OPTIONAL,  -- Need M

sl-TimeOffsetEUTRA-List-r16                    SEQUENCE (SIZE (8)) OF SL-TimeOffsetEUTRA-r16             OPTIONAL    -- Need M

}

SL-TimeOffsetEUTRA-r16 ::=        ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1, ms1dot25, ms1dot5, ms1dot75,

ms2, ms2dot5, ms3, ms4, ms5, ms6, ms8, ms10, ms20}

UE-TxTEG-RequestUL-TDOA-Config-r17 ::=  CHOICE {

oneShot-r17                             NULL,

periodicReporting-r17                   ENUMERATED { ms160, ms320, ms1280, ms2560, ms61440, ms81920, ms368640, ms737280 }

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

appLayerMeasConfig This field is used to configure application layer measurements. This field is absent when the UE is configured to operate with shared spectrum channel access or if sl-L2RemoteUE-Config-r17 is configured or not released.

bap-Config This field is used to configure the BAP entity for IAB nodes.

bap-Address Indicates the BAP address of an IAB-node. The BAP address of an IAB-node cannot be changed once configured for the cell group to the BAP entity.

conditionalReconfiguration Configuration of candidate target SpCell(s) and execution condition(s) for conditional handover, conditional PSCell addition or conditional PSCell change. The field is absent if any DAPS bearer is configured or if the masterCellGroup includes ReconfigurationWithSync or if the sl-L2RemoteUE-Config or sl-L2RelayUE-Config is configured. For conditional PSCell change, the field is absent if the secondaryCellGroup includes ReconfigurationWithSync. The RRCReconfiguration message contained in DLInformationTransferMRDC cannot contain the field conditionalReconfiguration for conditional PSCell change or for conditional PSCell addition.

daps-SourceRelease Indicates to UE that the source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released.

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedPagingDelivery This field is used to transfer Paging message for the associated L2 U2N Remote UE to the L2 U2N Relay UE in RRC_CONNECTED.

dedicatedPosSysInfoDelivery This field is used to transfer SIBPos to the UE in RRC_CONNECTED.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE (including L2 U2N Remote UE). The field has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8, SIB19, SIB21 to the UE with an active BWP with no common search space configured or the L2 U2N Remote UE in RRC_CONNECTED. For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is also used to transfer the SIBs requested on-demand.

defaultUL-BAP-RoutingID This field is used for IAB-node to configure the default uplink Routing ID, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BAP-RoutingID can be (re-)configured when IAB-node IP address for F1-C related traffic changes. This field is mandatory only for IAB-node bootstrapping.

defaultUL-BH-RLC-Channel This field is used for IAB-nodes to configure the default uplink BH RLC channel, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BH-RLC-Channel can be (re-)configured when IAB-node IP address for F1-C related traffic changes, and the new IP address is anchored at a different IAB-donor-DU. This field is mandatory for IAB-node bootstrapping. If the IAB-MT is operating in EN-DC, the default uplink BH RLC channel is referring to an RLC channel on the SCG; Otherwise, it is referring to an RLC channel either on the MCG or on the SCG depending on whether the MN or the SN configures this field.

flowControlFeedbackType This field is only used for IAB-node that support hop-by-hop flow control to configure the type of flow control feedback. Value perBH-RLC-Channel indicates that the IAB-node shall provide flow control feedback per BH RLC channel, value perRoutingID indicates that the IAB-node shall provide flow control feedback per routing ID, and value both indicates that the IAB-node shall provide flow control feedback both per BH RLC channel and per routing ID.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

iab-IP-Address This field is used to provide the IP address information for IAB-node.

iab-IP-AddressIndex This field is used to identify a configuration of an IP address.

iab-IP-AddressToAddModList List of IP addresses allocated for IAB-node to be added and modified.

iab-IP-AddressToReleaseList List of IP address allocated for IAB-node to be released.

iab-IP-Usage This field is used to indicate the usage of the assigned IP address. If this field is not configured, the assigned IP address is used for all traffic.

iab-donor-DU-BAP-Address This field is used to indicate the BAP address of the IAB-donor-DU where the IP address is anchored.

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. Value false indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

mrdc-ReleaseAndAdd This field indicates that the current SCG configuration is released and a new SCG is added at the same time.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup, otherConfig, conditionalReconfiguration, measConfig, bap-Config and IAB-IP-AddressConfigurationList. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

musim-GapConfig Indicates the MUSIM gap configuration and controls setup/release of MUSIM gaps. In this version of the specification, the network does not configure MUSIM gap together with concurrent measurement gap or preconfigured measurement gap for positioning.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

onDemandSIB-Request If the field is present, the UE is allowed to request SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5.

onDemandSIB-RequestProhibitTimer Prohibit timer for requesting SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxBW-PreferenceConfigFR2-2, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, maxMIMO-LayerPreferenceConfigFR2-2, minSchedulingOffsetPreferenceConfig, minSchedulingOffsetPreferenceConfigExt, rlm-RelaxationReportingConfig, bfd-RelaxationReportingConfig, btNameList, wlanNameList, sensorNameList and obtainCommonLocation can be included.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs, multicast MRBs) including SDAP/PDCP. In (NG)EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3. SRB4 should not be configured if sl-L2RemoteUE-Config-r17 is configured or not released.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

scg-State Indicates that the SCG is in deactivated state. This field is not used - in an RRCReconfiguration message received: - within mrdc-SecondaryCellGroup, or - in an E-UTRA RRCConnectionReconfiguration message, or - in an E-UTRA RRCConnectionResume message or - in an RRCReconfiguration message received via SRB3, except if the RRCReconfiguration message is included in DLInformationTransferMRDC. The field is absent if CPA or CPC is configured for the UE, or if the RRCReconfiguration message is contained in CondRRCReconfig.

sl-L2RelayUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Relay UE. The field is absent if conditionalReconfiguration is configured for CHO.

sl-L2RemoteUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Remote UE. The field is absent if conditionalReconfiguration is configured for CHO, or if appLayerMeasConfig or SRB4 is configured/not released.

secondaryCellGroup Configuration of secondary cell group ((NG)EN-DC or NR-DC).

sk-Counter A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication/discovery.

sl-ConfigDedicatedEUTRA-Info This field includes the E-UTRA RRCConnectionReconfiguration as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRCConnectionReconfiguration can only includes sidelink related fields for V2X sidelink communication, i.e. sl-V2X-ConfigDedicated, sl-V2X-SPS-Config, measConfig and/or otherConfig.

sl-TimeOffsetEUTRA This field indicates the possible time offset to (de)activation of V2X sidelink transmission after receiving DCI format 3_1 used for scheduling V2X sidelink communication. Value ms0dpt75 corresponds to 0.75ms, ms1 corresponds to 1ms and so on. The network includes this field only when sl-ConfigDedicatedEUTRA is configured.

targetCellSMTC-SCG The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. When UE receives this field, UE applies the configuration based on the timing reference of NR PCell for PSCell addition and PSCell change for the case of no reconfiguration with sync of MCG, and UE applies the configuration based on the timing reference of target NR PCell for the case of reconfiguration with sync of MCG. If both this field and the smtc in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

t316 Indicates the value for timer T316 as described in clause 7.1. Value ms50 corresponds to 50 ms, value ms100 corresponds to 100 ms and so on. This field can be configured only if the UE is configured with split SRB1 or SRB3.

ue-TxTEG-RequestUL-TDOA-Config Configures the periodicity of UE reporting for the association between Tx TEG and SRS Positioning resources. When configured with oneShot UE reports the association only one time. When configured with periodicReporting UE reports the association periodically and the periodicReporting indicates the periodicity. Value ms160 corresponds to 160ms, value ms320 corresponds to 320ms and so on.

ul-GapFR2-Config Indicates the FR2 UL gap configuration to UE. In EN-DC and NGEN-DC, the SN decides and configures the FR2 UL gap pattern. In NE-DC, the MN decides and configures the FR2 UL gap pattern. In NR-DC without FR2-FR2 band combination, the network entity which is configured with FR2 serving cell(s) decides and configures the FR2 UL gap pattern.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is absent in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the AS security algorithms associated to the master key. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |
| SCG | The field is mandatory present in: - an RRCReconfiguration message contained in an RRCResume message (or in an RRCConnectionResume message, see TS 36.331 [10]), - an RRCReconfiguration message contained in an RRCConnectionReconfiguration message, see TS 36.331 [10], which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). The field is optional present, Need M, in: - an RRCReconfiguration message transmitted on SRB3, - an RRCReconfiguration message contained in another RRCReconfiguration message (or in an RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1 - an RRCReconfiguration message contained in another RRCReconfiguration message which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation) Otherwise, the field is absent |
| PagingRelay | For L2 U2N Relay UE, the field is optionally present, Need N. Otherwise, it is absent. |

NEXT CHANGE

### 6.3.2	Radio resource control information elements

<Omit unrelated part>

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

]],

[[

f1c-TransferPathNRDC-r17                   ENUMERATED {mcg, scg, both}                                             OPTIONAL,   -- Need M

uplinkTxSwitching-2T-Mode-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Cond 2Tx

uplinkTxSwitching-DualUL-TxState-r17       ENUMERATED {oneT, twoT}                                                 OPTIONAL,   -- Cond 2Tx

uu-RelayRLC-ChannelToAddModList-r17        SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17

OPTIONAL,   -- Need N

uu-RelayRLC-ChannelToReleaseList-r17       SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelID-r17

OPTIONAL,   -- Need N

simultaneousU-TCI-UpdateList1-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList2-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList3-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList4-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

rlc-BearerToReleaseListExt-r17             SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentityExt-r17           OPTIONAL,   -- Need N

iab-ResourceConfigToAddModList-r17  SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfig-r17   OPTIONAL, -- Need N

iab-ResourceConfigToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfigID-r17 OPTIONAL  -- Need N

]],

[[

reportUplinkTxDirectCurrentMoreCarrier-r17 ReportUplinkTxDirectCurrentMoreCarrier-r17                            OPTIONAL  -- Need N

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...,

[[

lowMobilityEvaluationConnected-r17  SEQUENCE {

s-SearchDeltaP-Connected-r17        ENUMERATED {dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1},

t-SearchDeltaP-Connected-r17        ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                               OPTIONAL,   -- Need R

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

deactivatedSCG-Config-r17           SetupRelease { DeactivatedSCG-Config-r17 }                  OPTIONAL    -- Cond SCG-Opt

]]

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]]

}

DAPS-UplinkPowerConfig-r16 ::=      SEQUENCE {

p-DAPS-Source-r16                   P-Max,

p-DAPS-Target-r16                   P-Max,

uplinkPowerSharingDAPS-Mode-r16     ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

DeactivatedSCG-Config-r17 ::=       SEQUENCE {

bfd-and-RLM-r17                     BOOLEAN,

...

}

GoodServingCellEvaluation-r17 ::=       SEQUENCE {

offset-r17                              ENUMERATED {db2, db4, db6, db8}                         OPTIONAL   -- Need S

}

SL-PathSwitchConfig-r17 ::=         SEQUENCE {

targetRelayUE-Identity-r17          SL-SourceIdentity-r17,

t420-r17                            ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

...

}

IAB-ResourceConfig-r17 ::=          SEQUENCE {

iab-ResourceConfigID-r17            IAB-ResourceConfigID-r17,

slotList-r17                        SEQUENCE (SIZE (1..5120)) OF INTEGER (0..5119)                           OPTIONAL,    -- Need M

periodicitySlotList-r17             ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160}     OPTIONAL,    -- Need M

slotListSubcarrierSpacing-r17       SubcarrierSpacing                                                        OPTIONAL,    -- Need M

...

}

IAB-ResourceConfigID-r17 ::=        INTEGER(0..maxNrofIABResourceConfig-1-r17)

ReportUplinkTxDirectCurrentMoreCarrier-r17 ::= SEQUENCE (SIZE(1.. maxSimultaneousBands)) OF IntraBandCC-CombinationReqList-r17

IntraBandCC-CombinationReqList-r17::=   SEQUENCE {

servCellIndexList-r17                   SEQUENCE (SIZE(1.. maxNrofServingCells)) OF ServCellIndex,

cc-CombinationList-r17                  SEQUENCE (SIZE(1.. maxNrofReqComDC-Location-r17)) OF IntraBandCC-Combination-r17

}

IntraBandCC-Combination-r17::=      SEQUENCE (SIZE(1.. maxNrofServingCells)) OF CC-State-r17

CC-State-r17::=                     SEQUENCE {

dlCarrier-r17                       CarrierState-r17                             OPTIONAL, -- Need N

ulCarrier-r17                       CarrierState-r17                             OPTIONAL  -- Need N

}

CarrierState-r17::=                 CHOICE {

deActivated-r17                     NULL,

activeBWP-r17                       INTEGER (0..maxNrofBWPs)

}

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

CC-State field descriptions

dlCarrier Indicates DL carrier activation state for this carrier and the related active BWP Index, if activated.

ulCarrier Indicates UL carrier activation state for this carrier and the related active BWP Index, if activated.

CellGroupConfig field descriptions

bap-Address BAP address of the parent node in cell group.

bh-RLC-ChannelToAddModList Configuration of the backhaul RLC entities and the corresponding MAC Logical Channels to be added and modified.

bh-RLC-ChannelToReleaseList List of the backhaul RLC entities and the corresponding MAC Logical Channels to be released.

f1c-TransferPath The F1-C transfer path that an EN-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with lte, IAB-MT can only use LTE leg for F1-C transfer. If IAB-MT is configured with nr, IAB-MT can only use NR leg for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select an LTE leg or a NR leg for F1-C transfer. If the field is not configured, the IAB node uses the NR leg as the default one.

f1c-TransferPathNRDC The F1-C transfer path that an NR-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with mcg, IAB-MT can only use the MCG for F1-C transfer. If IAB-MT is configured with scg, IAB-MT can only use the SCG for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select the MCG or the SCG for F1-C transfer.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

npn-IdentityInfoList This field is used to transfer npn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to SNPN Identity. If this field is absent, the UE uses the npn-IdentityInfoList in SIB1 of the PCell.

plmn-IdentityInfoList This field is used to transfer plmn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to PLMN Identity. If this field is absent, the UE uses the plmn-IdentityInfoList in SIB1 of the PCell.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

reportUplinkTxDirectCurrentMoreCarrier Enables reporting of uplink Direct Current location information when the UE is configured with intra-band CA. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. The UE only reports the uplink Direct Current location information that are related to the indicated cc-CombinationList. The network does not include carriers which locate in DL only spectrum described in TS 38.101-2 [39], clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3 in FR2 in the IntraBandCC-CombinationReqList. I.e. DL-only carrier in FR2 frequency spectrum is not used to calculate the default DC location.

reportUplinkTxDirectCurrentTwoCarrier Enables reporting of uplink Direct Current location information when the UE is configured with uplink intra-band CA with two carriers. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

rlc-BearerToReleaseListExt List of the RLC entities and the corresponding MAC Logical Channels to be released for multicast MRBs.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellSIB20 This field is used to transfer SIB20 of the SCell in order to allow the UE for MBS broadcast reception on SCell. The network configures this field only for a single SCell at a time.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration. If the field is included for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or (NG)EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and (NG)EN-DC case where UE supports dynamic UL Tx switching.

uplinkTxSwitchingPowerBoosting Indicates whether the UE is allowed to enable 3dB boosting on the maximum output power for transmission on carrier2 under the operation state in which 2-port transmission can be supported on carrier2 for inter-band UL CA case with dynamic UL Tx switching as defined in TS 38.101-1 [15]. Network can only configure this field for dynamic UL Tx switching in inter-band UL CA case with power Class 3 as defined in TS 38.101-1 [15].

uplinkTxSwitching-2T-Mode Indicates 2Tx-2Tx switching mode is configured for inter-band UL CA or SUL, in which the switching gap duration for a triggered uplink switching (as specified in TS 38.214 [19]) is equal to the switching time capability value reported for the switching mode. If this field is absent and uplinkTxSwitching is configured, it is interpreted that 1Tx-2Tx UL Tx switching is configured as specified in TS 38.214 [19]. In this case, there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured.

uplinkTxSwitching-DualUL-TxState Indicates the state of Tx chains if the state of Tx chains after the UL Tx switching is not unique (as specified in TS 38.214 [19]) in case of 2Tx-2Tx switching is configured and uplinkTxSwitchingOption is set to dualUL. Value oneT indicates 1Tx is assumed to be supported on the carriers on each band, value twoT indicates 2Tx is assumed to be supported on that carrier.

uu-RelayRLC-ChannelToAddModList List of the Uu RLC entities and the corresponding MAC Logical Channels to be added or modified.

uu-RelayRLC-ChannelToReleaseList List of the Uu RLC entities and the corresponding MAC Logical Channels to be released.

DeactivatedSCG-Config field descriptions

bfd-and-RLM If the field is set to true, the UE shall perform RLM and BFD on the PSCell when the SCG is deactivated and the network ensures that beamFailure-r17 is not configured in the radioLinkMonitoringConfig of the DL BWP of the PSCell in which the UE performs BFD. If set to false, the UE is not required to perform RLM and BFD on the PSCell when the SCG is deactivated.

DAPS-UplinkPowerConfig field descriptions

p-DAPS-Source The maximum total transmit power to be used by the UE in the source cell group during DAPS handover.

p-DAPS-Target The maximum total transmit power to be used by the UE in the target cell group during DAPS handover.

uplinkPowerSharingDAPS-Mode Indicates the uplink power sharing mode that the UE uses in DAPS handover (see TS 38.213 [13]).

GoodServingCellEvaluation field descriptions

offset The parameter "X" (dB) for the good serving cell quality criterion in RRC_CONNECTED, for a cell operating in FR1 and FR2, respectively. If this field is absent, the UE applies the (default) value of 0 dB for "X".

IAB-ResourceConfig field descriptions

iab-ResourceConfigID This ID is used to indicate the specific resource configuration addressed by the MAC CEs specified in TS 38.321 [3].

periodicitySlotList Indicates the periodicity in ms of the list of slot indexes indicated in slotList.

slotList Indicates the list of slot indexes to which the information indicated in the specific MAC CE applies to, as specified in TS 38.321 [3]. The values of the entries in the slotList are strictly less than the value of the periodicitySlotList.

slotListSubcarrierSpacing Subcarrier spacing used as reference for the slotList configuration. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120 or 480 kHz

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon or sets to the same periodicity as ssb-Periodicity-r17 in nonCellDefiningSSB-r17 if the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17 for RedCap. For case of NR PCell change, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. For a RedCap UE, if the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17, this field corresponds to the NCD-SSB indicated by nonCellDefiningSSB-r17, otherwise, this field corresponds to the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL.

ReportUplinkTxDirectCurrentMoreCarrier field descriptions

IntraBandCC-Combination Indicates the state of the carriers and BWPs indexes of the carriers in a CC combination, each carrier in this combination corresponds to an entry in servCellIndexList with same order. This IE shall have the same size as servCellIndexList.

IntraBandCC-CombinationReqList Indicates the list of the requested carriers/BWPs combinations for an intra-band CA component.

servCellIndexList indicates the list of cell index for an intra-band CA component.

SCellConfig field descriptions

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in an SCell in RRC_CONNECTED. This field is always configured when the network enables BFD relaxation for the UE in this SCell. This field is absent if failureDetectionSetN is present for the SCell.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated while this SCell is deactivated. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the legacy DRX group shall belong to another Frequency Range. If drx-ConfigSecondaryGroup is configured, the field is optionally present. The network always includes the field if the field was previously configured for this SCell and the SCell remains in the secondary DRX group. Removal of an individual SCell from the secondary DRX group is supported by using an SCell release and addition. Otherwise, if drx-ConfigSecondaryGroup is not configured, the field is absent and the UE shall release the field. The UE shall also release the field if drx-ConfigSecondaryGroup is released without including sCellToAddModList.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SpCellConfig field descriptions

deactivatedSCG-Config Configuration applicable when the SCG is deactivated. The network always configures this field before or when indicating that the SCG is deactivated in an RRCReconfiguration, RRCResume, E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume message.

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables BFD relaxation for the UE in this SpCell. This field is absent if failureDetectionSetN is present for the SpCell.

goodServingCellEvaluationRLM Indicates the criterion for a UE to detect the good serving cell quality for RLM relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables RLM relaxation for the UE in this SpCell.

lowMobilityEvaluationConnected Indicates the criterion for a UE to detect low mobility in RRC_CONNECTED in an SpCell. The s-SearchDeltaP-Connected is the parameter "SSearchDeltaP-connected". Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on. The t-SearchDeltaP-Connected is the parameter "TSearchDeltaP-Connected". Value s5 means 5 seconds, value s10 means 10 seconds and so on. Low mobility criterion is configured in NR PCell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC.

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

| Conditional Presence | Explanation |
| --- | --- |
| 2Tx | The field is optionally present, Need R, if uplinkTxSwitching is configured; otherwise it is absent, Need R. |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DirectToIndirect-PathSwitch | The field is mandatory present for the L2 U2N remote UE at path switch to the target L2 U2N Relay UE. It is absent otherwise. |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the SCell belongs to and configured with preConfigInd. It is absent, Need R, otherwise. |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup: - at change of AS security key derived from KgNB, - in an RRCReconfiguration message contained in a DLInformationTransferMRDC message, - path switch of L2 U2N remote UE to the target PCell, - path switch of L2 U2N remote UE to the target L2 U2N Relay UE, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell, - change of AS security key derived from S-KgNB in NR-DC while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, - MN handover in (NG)EN-DC. Otherwise, it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages and is absent in the masterCellGroup in RRCReconfiguration messages if source configuration is not released during DAPS handover. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N: - in the masterCellGroup at - SCell addition, - reconfiguration with sync, - resume of an RRC connection. - in the secondaryCellGroup, when the SCG is not indicated as deactivated at: - SCG activation while the SCG was previously deactivated, - SCell addition, - reconfiguration with sync. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |
| SCellSIB20-Opt | This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise. |
| SCG-Opt | The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise. |

NOTE:	In case of change of AS security key derived from S-KgNB/S-KeNB, if reconfigurationWithSync is not included in the masterCellGroup, the network releases all existing MCG RLC bearers associated with a radio bearer with keyToUse set to secondary. In case of change of AS security key derived from KgNB/KeNB, if reconfigurationWithSync is not included in the secondaryCellGroup, the network releases all existing SCG RLC bearers associated with a radio bearer with keyToUse set to primary.

NEXT CHANGE

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling, MBS multicast and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup1

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

headerCompression       CHOICE {

notUsed                 NULL,

rohc                    SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0001           BOOLEAN,

profile0x0002           BOOLEAN,

profile0x0003           BOOLEAN,

profile0x0004           BOOLEAN,

profile0x0006           BOOLEAN,

profile0x0101           BOOLEAN,

profile0x0102           BOOLEAN,

profile0x0103           BOOLEAN,

profile0x0104           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

uplinkOnlyROHC          SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0006           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

...

},

integrityProtection     ENUMERATED { enabled }                                          OPTIONAL,   -- Cond ConnectedTo5GC1

statusReportRequired    ENUMERATED { true }                                             OPTIONAL,   -- Cond Rlc-AM-UM

outOfOrderDelivery      ENUMERATED { true }                                             OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                 OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                      OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                           OPTIONAL,   -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                     OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond MoreThanOneRLC

t-Reordering                ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40,

ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250,

ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24,

spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14,

spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03,

spare02, spare01 }                                          OPTIONAL, -- Need S

...,

[[

cipheringDisabled       ENUMERATED {true}                                                   OPTIONAL    -- Cond ConnectedTo5GC

]],

[[

discardTimerExt-r16     SetupRelease { DiscardTimerExt-r16 }                                OPTIONAL,    -- Cond DRB2

moreThanTwoRLC-DRB-r16  SEQUENCE {

splitSecondaryPath-r16  LogicalChannelIdentity                                          OPTIONAL,   -- Cond SplitBearer2

duplicationState-r16    SEQUENCE (SIZE (3)) OF BOOLEAN                                  OPTIONAL    -- Need S

}                                                                                           OPTIONAL,   -- Cond MoreThanTwoRLC-DRB

ethernetHeaderCompression-r16  SetupRelease { EthernetHeaderCompression-r16 }               OPTIONAL    -- Need M

]],

[[

survivalTimeStateSupport-r17   ENUMERATED {true}                                            OPTIONAL,   -- Cond Drb-Duplication

uplinkDataCompression-r17      SetupRelease { UplinkDataCompression-r17 }                   OPTIONAL,   -- Cond Rlc-AM

discardTimerExt2-r17           SetupRelease { DiscardTimerExt2-r17 }                        OPTIONAL,   -- Need M

initialRX-DELIV-r17            BIT STRING (SIZE (32))                                       OPTIONAL    -- Cond MRB-Initialization

]],

[[

primaryPathOnIndirectPath-r18         ENUMERATED {true}                                            OPTIONAL   -- Cond SplitBearerMP

]]

}

EthernetHeaderCompression-r16 ::=  SEQUENCE {

ehc-Common-r16                     SEQUENCE {

ehc-CID-Length-r16                 ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink-r16               SEQUENCE {

drb-ContinueEHC-DL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL,   -- Need M

ehc-Uplink-r16                 SEQUENCE {

maxCID-EHC-UL-r16              INTEGER (1..32767),

drb-ContinueEHC-UL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL    -- Need M

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

DiscardTimerExt2-r17 ::= ENUMERATED {ms2000, spare3, spare2, spare1}

UplinkDataCompression-r17 ::= CHOICE {

newSetup                      SEQUENCE {

bufferSize-r17                ENUMERATED {kbyte2, kbyte4, kbyte8, spare1},

dictionary-r17                ENUMERATED {sip-SDP, operator}                            OPTIONAL    -- Need N

},

drb-ContinueUDC           NULL

}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms and so on. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

discardTimerExt Value in ms of discardTimer specified in TS 38.323 [5]. Value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1ms and so on. If this field is present, the field discardTimer is ignored and discardTimerExt is used instead.

discardTimerExt2 Value in ms of discardTimerExt specified in TS 38.323 [5]. Value ms2000 corresponds to 2000 ms. If this field is present, the field discardTimer and discardTimerExt are ignored and discardTimerExt2 is used instead.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer. This field can be configured for both DRB and multicast MRB.

duplicationState This field indicates the uplink PDCP duplication state for the associated RLC entities at the time of receiving this IE. If set to true, the PDCP duplication state is activated for the associated RLC entity. The index for the indication is determined by ascending order of logical channel ID of all RLC entities other than the primary RLC entity indicated by primaryPath in the order of MCG and SCG, as in clause 6.1.3.32 of TS 38.321 [3]. If the number of associated RLC entities other than the primary RLC entity is two, UE ignores the value in the largest index of this field. If the field is absent, the PDCP duplication states are deactivated for all associated RLC entities.

ethernetHeaderCompression This fields configures Ethernet Header Compression. This field can only be configured for a bi-directional DRB or a bi-directional multicast MRB. The network reconfigures ethernetHeaderCompression only upon reconfiguration involving PDCP re-establishment and with neither drb-ContinueEHC-DL nor drb-ContinueEHC-UL configured. Network only configures this field when uplinkDataCompression is not configured.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC and EHC can be both configured simultaneously for a DRB or a multicast MRB. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment or involving PDCP entity reconfiguration to configure DAPS bearer(s), and without any drb-ContinueROHC. Network configures headerCompression to notUsed when outOfOrderDelivery is configured. Network only configures this field when uplinkDataCompression is not configured.

initialRX-DELIV Indicates the initial value of RX_DELIV during PDCP window initialization for multicast MRB as specified in TS 38.323 [5].

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

moreThanOneRLC This field configures UL data transmission when more than one RLC entity is associated with the PDCP entity. This field is not present if the bearer is configured as DAPS bearer.

moreThanTwoRLC-DRB This field configures UL data transmission when more than two RLC entities are associated with the PDCP entity for DRBs.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. For PDCP entity with more than two associated RLC entities for UL transmission, this field is always present. If the field moreThanTwoRLC-DRB is present, the value of this field is ignored and the state of the duplication is indicated by duplicationState. For PDCP entity with more than two associated RLC entities, only NR RLC bearer is supported.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs, except for the split SRB2 of the IAB-MT, and, when the SCG is deactivated, for DRBs. In MR-DC, the NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW always indicates logicalChannel if CA based PDCP duplication is configured in the cell group indicated by cellGroup of this field. In MP, when the primay path is set to indirect path, the field cellGroup and logicalChannel are absent, and the field primaryPathOnIndirectPath is set to true.

primaryPathOnIndirectPath Indicates that the primary RLC entity is on indirect path for DRB, when MP is configured and more than one RLC entity is associated with the PDCP entity.

splitSecondaryPath Indicates the LCID of the split secondary RLC entity as specified in TS 38.323 [5] for fallback to split bearer operation when UL data transmission with more than two RLC entities is associated with the PDCP entity. This RLC entity belongs to a cell group that is different from the cell group indicated by cellGroup in the field primaryPath.

statusReportRequired For AM DRBs, AM MRBs and DAPS UM DRBs, indicates whether the DRB or the multicast MRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For DAPS AM DRBs, it also indicates whether the DRB is configured to send a second PDCP status report in the uplink, as specified in TS 38.323 [5].

survivalTimeStateSupport Indicates whether the DRB associated with this PDCP entity has survival time state support. If this field is configured to be true, all associated RLC entities are activated for PDCP duplication upon reception of a retransmission grant addressed to CS-RNTI, as specified in TS 38.321 [3].

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms20 corresponds to 20 ms, value ms40 corresponds to 40 ms, and so on. When the field is absent the UE applies the value infinity. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bytes, value b100 corresponds to 100 bytes, value b200 corresponds to 200 bytes, and so on. The network sets this field to infinity for UEs not supporting splitDRB-withUL-Both-MCG-SCG and when the SCG is deactivated. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied.

uplinkDataCompression Indicates the UDC configuration that the UE shall apply. Network does not configure uplinkDataCompression for a DRB, if headerCompression or ethernetHeaderCompression is already configured or outOfOrderDelivery or DAPS is configured for the DRB. The maximum number of DRBs where uplinkDataCompression can be applied is two. The network reconfigures uplinkDataCompression only upon reconfiguration involving PDCP re-establishment. If the field is set to drb-ContinueUDC, the PDCP entity continues the uplink data compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is set to drb-ContinueUDC only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

EthernetHeaderCompression field descriptions

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

ehc-CID-Length Indicates the length of the CID field for EHC packet. The value bits7 indicates the length is 7 bits, and the value bits15 indicates the length is 15 bits. Once the field ethernetHeaderCompression-r16 is configured for a DRB or a multicast MRB, the value of the field ehc-CID-Length for this DRB or multicast MRB is not reconfigured to a different value.

ehc-Common Indicates the configurations that apply for both downlink and uplink.

ehc-Downlink Indicates the configurations that apply for only downlink. If the field is configured, then Ethernet header compression is configured for downlink. Otherwise, it is not configured for downlink.

ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplnik. Otherwise, it is not configured for uplink.

maxCID-EHC-UL Indicates the value of the MAX_CID_EHC_UL parameter as specified in TS 38.323 [5]. The total value of MAX_CID_EHC_UL across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE.

UplinkDataCompression field descriptions

bufferSize This field indicates the buffer size applied for UDC as specified in TS 38.323 [5]. Value kbyte2 means 2048 bytes, kbyte4 means 4096 bytes and so on.

dictionary This field indicates which pre-defined dictionary is used for UDC as specified in TS 38.323 [5]. The value sip-SDP means that UE shall prefill the buffer with standard dictionary for SIP and SDP defined in TS 38.323 [5], and the value operator means that UE shall prefill the buffer with operator-defined dictionary.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB/multicast MRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M. |
| DRB2 | This field is optionally present in case of DRB, need M. Otherwise, it is absent for SRBs and MRBs. |
| Drb-Duplication | For SRBs, this field is absent. For DRBs, this field is absent if duplication is not configured. Otherwise, this field is optional, need R. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than one associated logical channel and upon RRC reconfiguration with the association of additional logical channels to the PDCP entity. The field is also mandatory present in case the field moreThanTwoRLC-DRB is included in PDCP-Config. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent. Need R. |
| MoreThanTwoRLC-DRB | For SRBs, this field is absent. For DRBs, this field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than two associated logical channels and upon RRC reconfiguration with the association of one or more additional logical channel(s) to the PDCP entity so that the PDCP entity has more than two associated logical channels. Upon RRC reconfiguration when a PDCP entity is associated with more than two logical channels, this field is optionally present, Need M. Otherwise, the field is absent, Need R. |
| Rlc-AM | For RLC AM, the field is optionally present, need M. Otherwise, the field is absent. |
| Rlc-AM-UM | In case of DRB, for RLC UM (if the UE supports DAPS handover) or RLC AM, the field is optionally present, need R. In case of multicast MRB, if multicast MRB is associated with at least one RLC AM entity, the field is optionally present, need R. Otherwise, the field is absent. |
| Setup | The field is mandatory present in case of SRB or DRB setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is absent for SRBs. Otherwise, the field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. |
| SplitBearer2 | The field is mandatory present, in case of a split bearer. Otherwise the field is absent. |
| SplitBearerMP | The field is absent for SRBs. Otherwise, the field is optional present, need M, when MP is configured and the radio bearer are configured with more than one associated RLC mapped to different paths. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| ConnectedTo5GC1 | The field is optionally present, need R, if the UE is connected to NR/5GC or if the UE supports user plane integrity protection when connected to E-UTRA/EPC (as specified in TS 33.401 [30]). Otherwise the field is absent. |
| Setup1 | This field is mandatory present in case of SRB and DRB setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| MRB-Initialization | This field is mandatory present in case of multicast MRB setup. In case of PDCP re-establishment for multicast MRB, this field is optionally present, Need N. Otherwise, this field is absent, Need N. |

NEXT CHANGE

#### –	Uu-RelayRLC-ChannelConfig

The IE Uu-RelayRLC-ChannelConfig is used to configure an RLC entity, a corresponding logical channel in MAC for Uu Relay RLC channel between L2 U2N Relay UE and network, or between a relay UE and network on N3C indirect path in case of MP.

Uu-RelayRLC-ChannelConfig information element

-- ASN1START

-- TAG-UURELAYRLCCHANNELCONFIG-START

Uu-RelayRLC-ChannelConfig-r17::= SEQUENCE {

uu-LogicalChannelIdentity-r17    LogicalChannelIdentity                    OPTIONAL,   -- Cond RelayLCH-SetupOnly

uu-RelayRLC-ChannelID-r17        Uu-RelayRLC-ChannelID-r17,

reestablishRLC-r17               ENUMERATED {true}                         OPTIONAL,   -- Need N

rlc-Config-r17                   RLC-Config                                OPTIONAL,   -- Cond RelayLCH-Setup

mac-LogicalChannelConfig-r17     LogicalChannelConfig                      OPTIONAL,   -- Cond RelayLCH-Setup

...

}

-- TAG-UURELAYRLCCHANNELCONFIG-STOP

-- ASN1STOP

Uu-RelayRLC-ChannelConfig field descriptions

uu-LogicalChannelIdentity Indicates the logical channel id for Uu Relay RLC channel of the L2 U2N Relay UE.

uu-RelayRLC-ChannelID Indicates the Uu Relay RLC channel in the link between L2 U2N Relay UE and network.

reestablishRLC Indicates that RLC should be re-established.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters.

| Conditional Presence | Explanation |
| --- | --- |
| RelayLCH-Setup | This field is mandatory present upon creation of a new logical channel for a Uu Relay RLC channel. It is optionally present, Need M, otherwise. |
| RelayLCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a Uu Relay RLC channel. It is absent otherwise. |

NEXT CHANGE

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling, multicast MRBs and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIOBEARERCONFIG-START

RadioBearerConfig ::=                   SEQUENCE {

srb-ToAddModList                        SRB-ToAddModList                                        OPTIONAL,   -- Cond HO-Conn

srb3-ToRelease                          ENUMERATED{true}                                        OPTIONAL,   -- Need N

drb-ToAddModList                        DRB-ToAddModList                                        OPTIONAL,   -- Cond HO-toNR

drb-ToReleaseList                       DRB-ToReleaseList                                       OPTIONAL,   -- Need N

securityConfig                          SecurityConfig                                          OPTIONAL,   -- Need M

...,

[[

mrb-ToAddModList-r17                    MRB-ToAddModList-r17                                    OPTIONAL,   -- Need N

mrb-ToReleaseList-r17                   MRB-ToReleaseList-r17                                   OPTIONAL,   -- Need N

srb4-ToAddMod-r17                       SRB-ToAddMod                                            OPTIONAL,   -- Need N

srb4-ToRelease-r17                      ENUMERATED{true}                                        OPTIONAL    -- Need N

]]

}

SRB-ToAddModList ::=                    SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=                        SEQUENCE {

srb-Identity                            SRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

discardOnPDCP                           ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

[[

srb-Identity-v1700                      SRB-Identity-v1700                                      OPTIONAL    -- Need M

]]

}

DRB-ToAddModList ::=                    SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),

sdap-Config                             SDAP-Config

}                                                                                               OPTIONAL,   -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

[[

daps-Config-r16                         ENUMERATED{true}                                        OPTIONAL    -- Cond DAPS

]]

}

DRB-ToReleaseList ::=                   SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=                      SEQUENCE {

securityAlgorithmConfig                 SecurityAlgorithmConfig                                 OPTIONAL,   -- Cond RBTermChange1

keyToUse                                ENUMERATED{master, secondary}                           OPTIONAL,   -- Cond RBTermChange

...

}

MRB-ToAddModList-r17 ::=                SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-ToAddMod-r17

MRB-ToAddMod-r17 ::=                    SEQUENCE {

mbs-SessionId-r17                       TMGI-r17                                                OPTIONAL,   -- Cond MRBSetup

mrb-Identity-r17                        MRB-Identity-r17,

mrb-IdentityNew-r17                     MRB-Identity-r17                                        OPTIONAL,   -- Need N

reestablishPDCP-r17                     ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP-r17                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config-r17                         PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

MRB-ToReleaseList-r17 ::=               SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-Identity-r17

-- TAG-RADIOBEARERCONFIG-STOP

-- ASN1STOP

DRB-ToAddMod and MRB-ToAddMod field descriptions

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

daps-Config Indicates that the bearer is configured as DAPS bearer.

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts/indirect path of the configuration.

eps-BearerIdentity The EPS bearer ID determines the EPS bearer.

mbs-SessionId Indicates which multicast MBS session the bearer is associated with.

mrb-Identity Identification of the multicast MRB.

mrb-IdentityNew New identity of the multicast MRB when mrb-Identity needs to be changed, e.g. as a result of a handover.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured. Network doesn't include this field for DRB if the bearer is configured as DAPS bearer.

recoverPDCP Indicates that PDCP should perform recovery according to TS 38.323 [5]. Network doesn't include this field if the bearer is configured as DAPS bearer.

sdap-Config The SDAP configuration determines how to map QoS flows to DRBs when NR or E-UTRA connects to the 5GC and presence/absence of UL/DL SDAP headers.

RadioBearerConfig field descriptions

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included after AS security has been activated, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig. The field is not included when configuring SRB1 before AS security is activated.

srb3-ToRelease Release SRB3. SRB3 release can only be done over SRB1 and only at SCG release and reconfiguration with sync.

SecurityConfig field descriptions

keyToUse Indicates if the bearers configured with the list in this IE RadioBearerConfig are using the master key or the secondary key for deriving ciphering and/or integrity protection keys. For MR-DC, network should not configure SRB1 and SRB2 with secondary key and SRB3 with the master key. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

SRB-ToAddMod field descriptions

discardOnPDCP Indicates that PDCP should discard stored SDU and PDU according to TS 38.323 [5].

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to reconfiguration with sync, for SRB2 when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR, the network does not set this field to true. For LTE SRBs using NR PDCP, it could be for handover, RRC connection reestablishment or resume. Network doesn't include this field if any DAPS bearer is configured.

srb-Identity, srb-Identity-v1700 Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only. Value 4 is applicable for SRB4 only. If srb-Identity-v1700 is received for an SRB, the UE shall ignore srb-Identity (i.e. without suffix) for this SRB.

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of: - set up of signalling and data radio bearer, - change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| RBTermChange1 | The field is mandatory present in case of: - set up of signalling and data radio bearer, - change of termination point for the radio bearer between MN and SN, - handover from E-UTRA/EPC or E-UTRA/5GC to NR, - handover from NR or E-UTRA/EPC to E-UTRA/5GC if the UE supports NGEN-DC. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding DRB/multicast MRB is being setup or corresponding DRB/multicast MRB is reconfigured with NR PDCP or corresponding SRB associated with two RLC entities is being setup or if the number of RLC bearers associated with the DRB/multicast MRB or SRB is changed. The field is optionally present, Need S, if the corresponding SRB associated with one RLC entity is being setup or corresponding SRB is reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |
| HO-Conn | The field is mandatory present - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured, - or in case of RRCSetup. Otherwise the field is optionally present, need N. Upon RRCSetup, only SRB1 can be present. |
| HO-toNR | If mrb-ToAddModList is not included, the field is mandatory present - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured. In case of RRCSetup, the field is absent; otherwise the field is optionally present, need N. |
| DAPS | The field is optionally present, need N, in case masterCellGroup includes ReconfigurationWithSync, SCell(s) and SCG are not configured, multi-DCI/single-DCI based multi-TRP are not configured in any DL BWP, supplementaryUplink is not configured, ethernetHeaderCompression is not configured for the DRB, conditionalReconfiguration is not configured, and NR sidelink and V2X sidelink are not configured. Otherwise the field is absent. |
| MRBSetup | The field is mandatory present if the corresponding multicast MRB is being setup; otherwise the field is optionally present, need M. |

NEXT CHANGE

#### –	N3C-IndirectPathConfigRelay

The IE N3C-IndirectPathConfigRelay indicates the N3C indirect path related configuration used by relay UE.

Non3GPP-IndirectPathConfigRelay information element

-- ASN1START

-- TAG-N3C-INDIRECTPATHCONFIGRELAY-START

N3C-IndirectPathConfigRelay-r18 ::=           SEQUENCE {

n3c-MappingToReleaseList-r18             SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17     OPTIONAL, -- Need N

n3c-MappingToAddModList-r18                SEQUENCE (SIZE (1..maxLC-ID)) OF N3C-MappingToAddMod-r17       OPTIONAL, -- Need N

...

}

n3c-MappingToAddMod-r18 ::=            SEQUENCE {

n3c-RemoteUE-RB-Identity-r18             SL-RemoteUE-RB-Identity-r17,

n3c-EgressRLC-ChannelUu-r18              Uu-RelayRLC-ChannelID-r17                                        OPTIONAL, -- Need M

...

}

-- TAG-N3C-INDIRECTPATHCONFIGRELAY-STOP

-- ASN1STOP

N3C-IndirectPathConfigRelay field descriptions

n3c-MappingToAddModList Indicates the list of mappings between the bearer identity of the remote UE and the egress RLC channel to be added or modified. The egress RLC channel is associated to only one remote UE’s end-to-end bearer.

n3c-MappingToReleaseList Indicates the list of mappings between the bearer identity of the remote UE and the egress RLC channel to be released.

#### –	N3C-IndirectPathConfigRemote

The IE N3C-IndirectPathConfigRemote indicates the N3C indirect path related configuration used by remote UE.

N3C-IndirectPathConfigRemote information element

-- ASN1START

-- TAG-N3C-INDIRECTPATHCONFIGREMOTE-START

N3C-IndirectPathConfigRemote-r18 ::=  SEQUENCE {

n3c-RelayIdentification-r18          SEQUENCE {

n3c-CellGlobalId-r18                 SEQUENCE {

n3c-PLMN-Id-18                        PLMN-Identity,

n3c-CellIdentity-r18                  CellIdentity

},

n3c-C-RNTI-r18                       RNTI-Value

Editor’s Note: FFS whether/how IDLE/INACTIVE relay case

}

OPTIONAL, -- Cond N3CIndirectPathAddition

...

}

-- TAG-N3C-INDIRECTPATHCONFIGREMOTE-STOP

-- ASN1STOP

N3C-IndirectPathConfigRemote field descriptions

n3c-RelayIdentification Indicates the NCGI and C-RNTI of relay UE on N3C indirect path.

| Conditional Presence | Explanation |
| --- | --- |
| N3CIndirectPathAddition | The field is mandatory present in case of N3C indirect path addition. Otherwise, it is absent. |

NEXT CHANGE

#### –	SL-IndirectPathAddChange

The IE SL-IndirectPathAddChange specifies the configuration information of SL indirect path for SL indirect path addition/change.

SL-IndirectPathAddChange information element

-- ASN1START

-- TAG-SL-INDIRECTPATHADDCHANGE-START

SL-IndirectPathAddChange-r18 ::=              SEQUENCE {

sl-IndirectPathRelayUE-Identity-r18          SL-SourceIdentity-r17,

sl-IndirectPathCellIdentity-r18              CellIdentity,

t4xx-r18                                     ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

...

}

-- TAG-SL-INDIRECTPATHADDCHANGE-STOP

-- ASN1STOP

SL-IndirectPathAddChange field descriptions

sl-IndirectPathRelayUEIdentity Indicates the L2 source ID of the L2 U2N Relay UE of SL indirect path.

sl-IndirectPathCellIdentity Identify the serving cell of the indicated L2 U2N Relay UE.

T4xx Indicates the timer value of T4xx to be used during indirect path addition or change.

NEXT CHANGE