3GPP TSG-RAN WG2 Meeting #123bis	R2-230xxxx

Xiamen, China, 9 – 13 October 2023

Agenda item:	7.5.1

Source: 	Huawei, HiSilicon

Title: 	Open issues for RRC CR of XR enhancements WI

Document for:	Discussion and Decision

# 1. Introduction

This document aims to facilitate the discussion on open issues related to RRC CR for XR enhancements, as per the following e-mail discussion:

- [POST123bis][023][XR] 38.331 Running CR (Huawei)

Scope:

- Review running CR

- Identify open issues

- Get inputs for subset of open issues (focus more detailed open issues that would help with CR finalisation.

Deadline: long

The comments on the changes implemented in the updated RRC CR can be provided by the companies directly in the CR using comments. In this document, companies are requested to provide their input for some of the open issues, mainly related to detailed stage-3 signalling. Please provide your contact details in the table below.

| Company | Delegate’s e-mail and name |
| --- | --- |

# 2. Input for the RRC open issues

Following implementation of the latest agreements, many of the editor’s notes could be removed from the RRC CR. Some small changes were also made by the CR rapporteur, even though they were not agreed directly. For these, the companies are requested to provide their comments directly in the CR document. The remaining ones are discussed in the following subsections.

## 2.1	UAI triggering

There is a following EN in the running RRC CR:

“Editor’s note: The UE may not have UL traffic information available immediately after being configured to provide it by the gNB. RAN2 may need to discuss whether this should be handled somehow, e.g. specifying that the UE may delay the first transmission of this information until the relevant information is available at the UE.”

A number of contributions suggested to add a NOTE in specifications clarifying that the information should be provided by the UE only when it is available. There were also suggestions that UE could report that no satisfactory value of a certain parameter is available. The rapporteur believes that a NOTE would be sufficient, aiming to clarify that the UE is not required to report immediately, but may delay sending the information until it is available. The following note is then proposed:

“NOTE: The UE should only initiate transmission of the UEAssistanceInformation message to provide UL traffic information once the UE gathers sufficient information, i.e. it is not required to do so immediately after being configured to provide UL traffic information.”

Question 1: Do you agree to add the following note in TS 38.331? Do you have any comment/suggestion on the wording of the note?

NOTE: The UE should only initiate transmission of the UEAssistanceInformation message to provide UL traffic information once the UE gathers sufficient information, i.e. it is not required to do so immediately after being configured to provide UL traffic information.

| Company | Comments |
| --- | --- |

## 2.2	Supported frame rates and DRX periodicities

According to TR 38.835 [2]: “Most XR frame rates (15, 30, 45, 60, 72, 90 and 120fps) correspond to periodicities which are not an integer (66.66, 33.33, 22.22, 16.66, 13.88, 11.11 and 8.33ms respectively).”

When expressing these periodicities with rational numbers, they would correspond to:

| Frame rate [fps] | Periodicity in decimal number [ms] | Periodicity in rational number [ms] |
| --- | --- | --- |
| 15 | 66.6(6) | 200/3 |
| 30 | 33.3(3) | 100/3 |
| 45 | 22.2(2) | 200/9 |
| 60 | 16.6(6) | 50/3 |
| 72 | 13.8(8) | 125/9 |
| 90 | 11.1(1) | 100/9 |
| 120 | 8.3(3) | 25/3 |

For both short and long DRX, cycles corresponding to the above frame rates were added in the running RRC CR. After this, there were already some long DRX cycles which were multiple of short DRX cycles, but some had to be added specifically:

- For ms125over9, double and triple were added, i.e. ms250over9 and ms125over3

- For ms200over9, double was added, i.e. ms400over3 (only one was added, since larger value may not be able to meet XR traffic’s PSDB requirement)

The resulting DRX configuration is as follows in the updated RRC CR:

DRX-ConfigExt2-v18xx ::= SEQUENCE { drx-NonIntegerLongCycleStartOffset-r18 CHOICE { ms25over3 INTEGER(0..8), ms100over9 INTEGER(0..11), ms125over9 INTEGER(0..13), ms50over3 INTEGER(0..16), ms200over9 INTEGER(0..22), ms250over9 INTEGER(0..27), ms100over3 INTEGER(0..33), ms125over3 INTEGER(0..41), ms200over3 INTEGER(0..66), ms400over3 INTEGER(0..133) }, shortDRX-r18 SEQUENCE { drx-NonIntegerShortCycle-r18 ENUMERATED {ms25over3, ms100over9, ms125over9, ms50over3, ms200over9, ms100over3, ms200over3, spare1 }, drx-ShortCycleTimer-r18 INTEGER (1..16) } OPTIONAL, -- Need R drx-TimeReferenceSFN-r18 ENUMERATED {sfn512} OPTIONAL -- Need S }

Question 2: Companies are requested to provide comments on the current values of short and long DRX cycles, e.g. are any additional values needed etc.

| Company | Comments |
| --- | --- |

## 2.3	Jitter and periodicity signalling

Jitter

With respect to jitter signalling, RAN2 needs to decide on:

- Range of the signalling values, e.g. [-4, 4], [-8, 8]

- Granularity of the signalling, e.g. 0.5ms, 1ms.

- Whether the jitter is symmetrical, i.e. lower and upper limit have the same value but opposite sign, or lower limit and upper limit can be different and should be reported separately.

For the rapporteur to prepare the signalling accordingly, companies are then requested to provide their views on the aspects mentioned above.

Question 3: Please provide your view on jitter signalling in the table below.

| Company name | Value range, e.g. [-4, 4], [-8, 8] | Granularity, e.g. 0.5ms, 1ms etc. | Symmetrical or asymmetrical | Rationale/comments |
| --- | --- | --- | --- | --- |

Periodicity

Similarly, traffic periodicity signalling needs to be decided. Proposals that were mentioned included:

- Reuse of TSCAI signalling. Periodicity in TSCAI is defined as follows in TS 38.413:

#### 9.3.1.132 Periodicity This IE indicates the Periodicity of the TSC QoS flow as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Periodicity; M; INTEGER (0..640000, …); Periodicity expressed in units of 1 us..

- Follow the definition of non-integer DRX cycles. Short DRX cycles would be most appropriate since they only have values corresponding to the XR frame rates, as clarified in section 2.2:

drx-NonIntegerShortCycle-r18 ENUMERATED {ms25over3, ms100over9, ms125over9, ms50over3, ms200over9, ms100over3, ms200over3, spare1 },

- Reuse CG-like signalling, which is currently defined in TS 38.331 as follows:

periodicity ENUMERATED {sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14, sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14, sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14, sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12, sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12, sym1280x12, sym2560x12 }

In rapporteur’s view, it seems straightforward to reuse the agreed DRX-like signalling for signalling of non-integer periodicities of traffic. However, there is also pose/control traffic which can have integer periodicity. Also, periodicity as specified with TSCAI currently allows to express the periodicity with a very high accuracy which would allow to easily signal a value of e.g. 33.33 ms which seems to be a sufficient approximation of the real periodicity. Considering the above, the rapporteur would like to propose to focus on two options:

- Option 1: Signal the periodicity with integer value for both integer periodicities and non-integer periodicities (in which case the value closest to the real periodicity value is signalled by the UE, e.g. 33,33ms is signalled for 100/3 ms)

- Option 2: Signal the periodicity differently (choice structure) depending on whether it is integer or non-integer value:

  - Integer periodicities are signalled with integer value

  - Non-integer periodicities are signalled in the same way as signalled for non-integer DRX cycles, e.g. ms25over3, ms100over9 etc.

Companies are also requested to provide their view on the signalling range and granularity, e.g. current TSCAI supports periodicities in the range of 0-640 ms signalled with a granularity of 1us.

Question 4: Companies are requested to provide their views in the table below.

| Company name | Preferred option | Signalling range | Granularity | Other comments |
| --- | --- | --- | --- | --- |

## 2.4	Confirmation of other parameter values

The current running CR contains editor’s notes capturing the need to confirm the values of several parameters:

- Remaining time threshold

remainingTimeThreshold-r18 ENUMERATED {ms5, ms10, ms15, ms20, ms25, ms30, ms50, spare}

- PSI discard timer

PSI-DiscardTimer-r18 ::= ENUMERATED {ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms300, ms500 }

- UAI prohibit timer for UL Traffic Information

ul-TrafficInfoProhibitTimer-r18 ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1}

Question 5: Companies are requested to provide their comments on the proposed values of the parameters mentioned above, e.g. are any more values needed, are some values inappropriate etc.

- Remaining time threshold

| Company | Comments |
| --- | --- |

- PSI discard timer

| Company | Comments |
| --- | --- |

- UAI prohibit timer for UL Traffic Information

| Company | Comments |
| --- | --- |

## 2.5	Other issues

If companies see any other issues to discuss/solve for the XR RRC CR, companies are requested to raise them here.

| Company | Issue description |
| --- | --- |

# 3. Summary

TBD