3GPP TSG-RAN WG2 Meeting #123bis	R2-230xxxx

Xiamen, China, 9th  – 13th Oct. 2023

Source:	vivo

Title:	Summary of discussions on open issues in RLC running CR

Agenda Item:	7.5.1

Document for:	Discussion and Decision

# Introduction

This document aims to facilitate the discussion on open issues related to RLC CR for XR enhancements, as per the following e-mail discussion:

- [POST123bis][027][XR] 38.322 Running CR (Vivo)

Scope:

- Review running CR

- Identify open issues

- Get inputs for subset of open issues (focus on more detailed open issues that would help with CR finalisation).

Deadline: long

The comments on the changes implemented in the updated RLC CR can be provided by the companies directly in the CR using comments. In this document, companies are requested to provide their input for some of the open issues, mainly related to detailed stage-3 aspects.

- Contact information

Please provide your contact information in the below table:

| Company | Name and email address |  |
| --- | --- | --- |
| vivo | Chenli (chenli5g@vivo.com) |  |
| Ericsson | Richard (Richard.tano@ericsson.com) |  |
| Xiaomi | Liyanhua1@xiaomi.com |  |
| Futurewei | Yunsong Yang (yyang1@futurewei.com) |  |
| Huawei, HiSilicon | Li Qiang (qiangli3@huawei.com) |  |
| LG Electronics | SeungJune Yi (seungjune.yi@lge.com) |  |
| Samsung | Weiping Sun (wp.son@samsung.com) |  |
| Intel | Marta Martinez Tarradell (marta.m.tarradell@intel.com) |  |

# Input for the RLC open issues

  - How to determine delay critical data in RLC

Delay critical RLC SDU/PDU set is determined based on the remaining PDCP discardTimer value is less than a threshold. But discardTimer is running in PDCP layer, how to determine this delay critical data in RLC should be dteremined.

One option is similar as legacy (e.g. When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU), i.e. based on the indication from PDCP layer.

The corresponding TP is as below:

Delay-critical RLC SDU: the RLC SDU for which the remaining discardTimer value is less than a [threshold] , according to the indication from upper layer (e.g. PDCP).

Another option is left to UE implementation to have such cross layer indication.

- Companies are invited to provide your views on how to determine delay critical data in RLC.

- Option 1: based on the indication from PDCP layer.

- Option 2: up to UE implementation.

- Option 3: others, please specify

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Ericsson | 1 | Reasonable to mimic legacy procedure with PDCP indication. |
| Xiaomi | 1 | Simiar as legacy way in discarding in current spec: When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report or LWA status report, the UE shall discard the PDCP SDU along with the corresponding PDCP PDU. If the corresponding PDCP PDU has already been submitted to lower layers, the discard is indicated to lower layers. |
| Futurewei | 1 | Further more, we think delay-critical data should be determined by the PDCP entity and indicated to the RLC entity. Therefore, as far as RLC is concerned, the definition of Delay-critical RLC SDU should be as simple as the following: Delay-critical RLC SDU: the RLC SDU indicated by the associated PDCP entity as being delay-critical data, as defined in TS 38.323 [4]. This helps to simplify the RLC spec (by putting the burden on the PDCP spec), considering a shortened timer value for discarding may be used on PDUs with a PSI value of less importance under network congestion. |
| Huawei, HiSilicon | 1 | Should follow up the this way for cross-layer interaction, simply specify that UE determine the delay critical data based on the indication from PDCP, but no need to specify details. |
| LGE | 1 | RLC does not need to know the remaining time of PDCP discardTimer. RLC considers the RLC SDU as the delay-critical RLC SDU when the PDCP indicates it. |
| Samsung | 2 | It is clear that discardTimer is managed per PDCP SDU, and hence, it is more likely to be accessible at PDCP layer. While, there could be more than one solutions on how RLC can be aware of the remaining discardTimer value of a certain UL data in RLC layer. It is not desirable to impose certain restrictions on the interactions between RLC and PDCP internally, in order to embrace various options for different UE implementations. |
| Intel | 1 | Share same view as Ericsson |

Summary:

(to be added after the discussion)

  - How to handle the case with discardTimer expires

Delay critical RLC SDU/PDU set is determined based on the remaining PDCP discardTimer value is less than a threshold. Rapporteur thinks the data with discardTimer expired [indicated by PDCP] is also included in the case that discardTimer value is less than a threshold, as the current remaining discardTimer is zero. Regarding whether need to capture (if needed, how to capture) this case explicitly, rapporteur would like to check companies’ views.

The possible TP is as below (if needed):

Delay-critical RLC SDU: the RLC SDU for which the remaining discardTimer value is less than a [threshold] (including discardTimer expires), [according to the indication from upper layer (e.g. PDCP)].

- Companies are invited to provide your views on whether agree the data with discardTimer expired [indicated by PDCP] is also included in the case that discardTimer value is less than a threshold.

- Yes

- No

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Ericsson | Yes | But clearly only data that will be transmitted (not discarded) should be included since that is the data that will use grant resources. |
| Xiaomi | No | It is for data volum calculation? Then No. in Current spec, the discarded packets in PDCP will not be count into BSR reporting. |
| Futurewei |  | Agree that discarded SDUs should not be counted for the data colume to be reported. |
| Huawei, HiSilicon | Yes | This is a case that some SDUs are still stored in RLC buffer but the corresponding SDUs in PDCP have already be discarded. Those packets will occupy the UL grant, so should also be considered as delay critical data volume. |
| LGE | Yes | Agree with Huawei. In addition, RLC Control PDU shall also be considered in delay critical data volume. |
| Samsung | No | The UL data with discardTimer expired is discarded by PDCP/RLC, and hence, should not be considered in DSR triggering/reporting. That is, the UL data with discardTimer expired should be excluded from the case that remaining discardTimer value is less than a threshold. |
| Intel | See response | It seems unclear in which case this discussion point 2) applies. From companies’ responses, it seems there are two possible understanding of the target scenario (as this does not seem fully clear from the discussion point): - If the scenario is related to the DSR, we share the view explained by others that this should be considered, i.e. Yes. - If the scenario is related to the expiry of the discardTimer, we share the view explained by others that this should not be considered, i.e. No |

- If Yes, companies are invited to provide your views on whether need to capture this case explicitly

- Yes, and where agree the above TP

- No

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Ericsson | Yes | Clarify that it is only data that will not be discarded (RLC data that has not started transmission). |
| Xiaomi | Yes | We need to caputure in PDCP that when the remaining discardTimer value is less than a threshold for a PDCP SDU and the corresponding PDCP Data PDU has already been submitted to lower layers, the indication is delivered from PDCP to RLC. For RLC, we need to capure that for the purpose of MAC delay status reporting, when get indicated from upper layer (e.g. PDCP) that a particular RLC SDU whose remaining discardTimer value is less than a [threshold], the transmitting RLC entity shall consider the corresponending packets as delay-critical RLC data volume. |
| Futurewei |  | We think the further-revised definition, as below, is sufficient: Delay-critical RLC SDU: the RLC SDU indicated by the associated PDCP entity as being delay-critical data, as defined in TS 38.323 [4], but not discarded yet. |
| Huawei, HiSilicon | Yes | Agree with Ericsson, should clarify that only for those data that will not be discarded according to clause 5.4 of 322 |
| LGE | Comment | We think following definition is sufficient. Delay-critical RLC SDU: the RLC SDU for which delay-critical indication is provided by PDCP and not discarded by RLC. |
| Samsung | No | There is no need to capture it as a separate case. Note that legacy PDCP/RLC specifications do not specify it as a separate case in data volume calculation. |

  - Whether consider PDU(s) pending for RLC AM retx

During the data volume calculation for DSR, rapporteur thinks the PDU(s) pending for RLC AM retransmission should be also included as legacy data volume calculation for BSR.

The corresponding TP is:

For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume: - if pdu-SetDiscard is configured: - RLC SDUs and RLC SDU segments belonging to the delay-critical PDU Set that have not yet been included in an RLC data PDU; - RLC data PDUs that contain the RLC SDUs or RLC SDU segments belonging to the delay-critical PDU Set and are pending for initial transmission; - RLC data PDUs that contain the RLC SDUs or RLC SDU segments belonging to the delay-critical PDU Set and are pending for retransmission (RLC AM). - else: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs that contain the delay-critical RLC SDUs or RLC SDU segments, and are pending for initial transmission; - RLC data PDUs that contain the delay-critical RLC SDUs or RLC SDU segments, and are pending for retransmission (RLC AM).

- Companies are invited to provide your views on whether agree the RLC data the PDU(s) pending for RLC AM retransmission should be also included in the data volume calculation for DSR.

- Yes

- No

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Ericsson | Yes | Data will take up grant space. |
| Xiaoi | Yes | Same as BSR reporting. |
| Futurewei | Yes | Except that we think the determination of delay-critical data based on whether pdu-SetDiscard is configured or not should be done by the PDCP entity, not by the RLC entity. The RLC entity just follow the indication from the PDCP entity. So, we can delete the text from “if” to “else:” |
| Huawei, HiSilicon | Yes | As commented to Q3, we should consider those data that will not be discarded according to clause 5.4 of 322, not only for RLC AM retransmission, but also include RLC UM segmentation. |
| LGE | Yes |  |
| Samsung | Yes | Same view as Ericsson and Xiaomi. |
| Intel | Yes |  |

Summary:

(to be added after the discussion)

  - In case PDU set discard is not configured

In RAN2#123bis meeting, it was agreed that

- The data volume calculation to be reported in the DSR will consider the at size of the full remaining PDUs in the PDU set (if any PDU within the PDU set is with remaining time below the threshold), if the PDU set discard is configured. FFS what to report for the case of not PDU set discard configured

It is FFS what to report for the case of not PDU set discard configured. This issue is common with PDCP open issue.

Rapporteur thinks the data volume calculation to be reported in the DSR will consider the size of any buffered SDU/PDU with remaining time below the threshold, if the PDU set discard is not configured.

The corresponding TP is as below:

- if pdu-SetDiscard is configured: - xxxx - else: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs that contain the delay-critical RLC SDUs or RLC SDU segments, and are pending for initial transmission; - RLC data PDUs that contain the delay-critical RLC SDUs or RLC SDU segments, and are pending for retransmission (RLC AM).

- Companies are invited to provide your views on whether agree the data volume calculation to be reported in the DSR will consider the size of any buffered SDU/PDU with remaining time below the threshold, if the PDU set discard is not configured.

- Yes

- No

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Ericsson | Yes | All data below threshold should be reported. If multiple reporting values is allowed then there will be granularity in the reporting, otherwise all SDUs will be clumped together into one value. |
| Xiaomi | - | It is ture. But, RLC do not need to know whether this flag is configured or not. PDCP will keep this association of Pakets and Packets Set. RLC only sees Pakcets. |
| Futurewei | - | As we commented before and agreeing with Xiaomi here, the determination of delay-critical data based on whether pdu-SetDiscard is configured or not should be implemented in the PDCP spec, not in the RLC spec. |
| Huawei, HiSilicon | No | We think in this release, if the PDU set discarding is not configured, the remaining time reporting/DSR no need to be processed. Otherwise the current specification will be impacted a lot, it need to design two parallel PDCP discard timer indication ways, two parallel remaining time calculation principles (for PDU set configured the minimal remaining time is the remaining time of those member PDUs, while for PDU set not configured case each PDU should consider its own remaining time, and situation may be more complicated as SA2 clarified that a XR QoS flow may be mixed with PDU sets and signle PDUs), etc. |
| LGE | Comments | RLC does not need to know the remaining time of PDCP discardTimer. In addition, RLC does not need to know whether pdu-SetDiscard is configured or not. RLC can act only based on the indication from PDCP. |
| Samsung | Yes | All buffered RLC SDU/SDU segment/PDU with remaining time below the threshold should be reported. |
| Intel | See comment | It is unclear of the use case behind of reporting DSR when PDU Set Discard is not configured. If this information is deemed helpful form gNB PoV, it could be enabled as explained but we prefer not to enable a feature unless its usability is clear. |

Summary:

(to be added after the discussion)

  - Others

If companies have any other issues to discuss/solve for the XR RLC CR, companies are requested to raise them here.

| Company’s name | Comments, if any |
| --- | --- |
| Huawei, HiSilicon | We also need to consider how to deal with Control PDU |
| LGE | RLC does not need to know: - Remaining time of PDCP discardTimer - Configuration of pdu-SetDiscard - PDU Set - Delay-critical PDU Set RLC needs to know: - Whether the RLC SDU has remaining time less than the threshold (new indication from PDCP) |

# Conclusion

This contribution is the report of email discussion: xxxx with the following proposals: