3GPP TSG-RAN2 #124	R2-23xxxxx

Chicago, USA, Nov 13-17, 2023

Agenda Item:	7.2.1

Source: 	Xiaomi

Title:  	Open issue list for Rel-18 positioning capability

Document for:	Discussion and Decision

# Introduction

RAN1 has sent RAN1 feature list in R1-2310635, RAN2 also agreed some RAN2 related capabilities:

Agreements:

Introduce the UE capability on supporting positioning mode(i.e. UE based, UE assisted) per positioning method in SLPP.

Introduce the UE capability on supporting periodical reporting per positioning method in SLPP.

Introduce the UE capability on supporting lower value of response time (e.g. 10ms) per positioning method in SLPP.

Introduce a single UE capability on supporting RAT-dependent positioning integrity for DL-TDOA and DL-AoD respectively.  Additional finer-grained capabilities are not excluded if a need is found.

Introduce the UE capability on UE supporting preconfigured SRS, in both RRC and LPP.

Don’t introduce the UE capability on supporting activation indication/request of the pre-configuration SRS preconfigured SRS, it can be a component for the capability on supporting pre-configured SRS.

Working assumption:

Don’t introduce the UE capability on supporting alignment of PRS to fixed (e)DRX.

In this contribution, we provide the open issues relating to RAN1 and RAN2 UE feature list.

# Discussion

Open issue 1: the applicable SL positioning methods of each RAN1 UE feature

In RAN1 UE feature list, it is unclear about the applicable SL positioning methods of some features, the following lists the assumpted positioning method for each RAN1 feature, which requries RAN2 confirmation:

| COMMON for all SL-PRS related positioning method | 41-1-1 | Common SL PRS Processing Capability in a SL BWP |
| --- | --- | --- |
|  | 41-1-1 | Common SL PRS Processing Capability in a SL BWP |
|  | 41-1-2 | Receiving SL-PRS in a shared resource pool |
|  | 41-1-3 | Receiving SL-PRS in a dedicated resource pool |
|  | 41-1-4a | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool |
|  | 41-1-4b | Transmitting SL-PRS scheme 1 in a dedicated resource pool |
|  | 41-1-4c | Transmitting SL-PRS mode 2 in a dedicated resource pool |
|  | 41-1-5 | SL-PRS congestion control in a dedicated resource pool |
|  | 41-1-7e | SL PRS measurement for SL PRS-RSRP |
|  | 41-1-7f | SL PRS measurement for SL PRS-RSRPP |
|  | 41-1-8 | Support of random selection in a dedicated resource pool |
|  | 41-1-10 | Support of full sensing in a dedicated resource pool |
|  | 41-1-11 | TDM-based multiplexing of SL-PRS [transmission/reception] from different UEs in the same slot in dedicated resource pool |
|  | 41-1-12 | Comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool |
|  | 41-1-13 | Reporting the additional paths for SL positioning |
|  | 41-1-14 | LoS/NLoS indicator for SL positioning [per measurement] |
|  | 41-1-17 | Open loop SL pathloss based power control and SL RSRP report [for dedicated resource pool] |
|  | 41-1-19 | ARP location provision for sidelink as assistance data |
|  | 41-1-19a | Report of Rx ARP-ID with SL positioning measurements |
| SL-RTT | 41-1-7c | SL PRS measurement for UE Rx – Tx time difference without Tx time stamp |
|  | 41-1-7d | SL PRS measurement for UE Rx – Tx time difference with Tx time stamp |
| SL-TDOA | 41-1-7a | SL PRS measurement for SL-RSTD |
| SL-TOA | 41-1-7b | SL PRS measurement for SL RTOA |
| SL-AOA | 41-1-7g | SL PRS measurement for SL AoA |

Besides, there is the following editor note in 38.355:

- 35 Editor's note	FFS on whether any positioning method specific capability IEs should be grouped by positioning method.

RAN2 also needs to address the above editor note.

Open issue 2: whether periodical reporting capability is indicated per positioning mode per positioning method.

RAN2 has agreed to indicate periodical reporting capability per positioning method. However, for Uu DL-TDOA/DL-AOD positioning,

periodical reporting capability is indicated per positioning mode per positioning method. RAN2 needs to further consider whether periodical reporting capability is indicated per positioning mode per positioning method.

Open issue 3: whether [10ms] granularity response time capability is indicated per positioning mode per positioning method.

RAN2 has agreed to indicate 10ms granularity response time capability per positioning method. However, for Uu DL-TDOA/DL-AOD positioning,

10ms granularity response time capability is indicated per positioning mode per positioning method. RAN2 needs to further consider whether 10ms granularity response time is indicated per positioning mode per positioning method.

Open issue 4: for UE-only mode, if server UE calculates the position, whether positioning mode is referred as UE-based, UE-assisted, or standalone mode.

The definition of UE-based, UE-assisted and standalone in TS38.305 is as below:

As used in this document, the suffixes "-based" and "-assisted" refer respectively to the node that is responsible for making the positioning calculation (and which may also provide measurements) and a node that provides measurements (but which does not make the positioning calculation). Thus, an operation in which measurements are provided by the UE to the LMF to be used in the computation of a position estimate is described as "UE-assisted" (and could also be called "LMF-based"), while one in which the UE computes its own position is described as "UE-based".

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

According to the above definition, for UE-only mode, if server UE calculates the position, the positioning mode is more like standalone mode instead of UE-based mode, since in this case it is not the UE itself calculates its own location.

Open issue 5: whether capability for scheduled location time function (if supported) is needed. If so, whether it is indicated per positioning mode per positioning method.

In last RAN2 meeting, the capability on scheduled location time is discussed but no conclusion is made due to no consensus on whether scheduled location time functionality is supported or not. If supported, RAN2 needs to discuss whether scheduled location time capability is needed or not. If needed, whether it is indicated per positioning mode per positioning method.

Open issue 6: whether to introduce the UE capability on UE roles (anchor UE, server UE, target UE, anchor UE with location) in SLPP. If so, which roles are considered.

In last RAN2 meeting, RAN2 discussed on whether to introduce the UE capability on UE roles (anchor UE, server UE) in SLPP. No conclusion was made. The major concern is this information may be carried together with candidate anchor UE list outside UE capabilities. But this requires that discovery procedure is performed in advance.

Open issue 7: the details of reqeust capability.

According to [Post123bis][412][POS] TS 38.355 (Intel), the details of request capability message will be discussed in capability discussion. According to the current draft TS 38.355:

- – RequestCapabilities -- ASN1START -- TAG-REQUESTCAPABILITIES-START RequestCapabilities ::= SEQUENCE { criticalExtensions CHOICE { requestCapabilities RequestCapabilities-IEs, criticalExtensionsFuture SEQUENCE {} } } RequestCapabilities-IEs ::= SEQUENCE { commonIEsRequestCapabilities OCTET STRING OPTIONAL, -- Containing CommonIEsRequestCapabilities commonSL-PRS-MethodsIEsRequestCapabilities OCTET STRING OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsRequestCapabilities sl-AoA-RequestCapabilities OCTET STRING OPTIONAL, -- Containing SL-AoA-RequestCapabilities sl-RTT-RequestCapabilities OCTET STRING OPTIONAL, -- Containing SL-RTT-RequestCapabilities sl-TDOA-RequestCapabilities OCTET STRING OPTIONAL, -- Containing SL-TDOA-RequestCapabilities sl-TOA-RequestCapabilities OCTET STRING OPTIONAL, -- Containing SL-TOA-RequestCapabilities lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } -- TAG-REQUESTCAPABILITIES-STOP -- ASN1STOP

RAN2 needs to decide the contents of each IEs in the request capabilities. Currently, rapporteur has not identified any information that needs to be contained in any of the IEs in the request capabilities. Companies are encouraged to provide contribution if any information is identified necessary.

Open issue 8: whether capability related to server UE should be defined.

For example, whether we should introduce supported positioning methods for server UE capability. Or any other server UE functionalities should be introduced.

Open issue 9: How to define the UE capability on 41-2-4.

RAN1 provided the UE feature of 41-2-4, we are not clear whether a new positioning method of UE based carrier phase positioning is introduced or not, so it should be further discussed and then define the UE capability.

| 41. NR_pos_enh2 | 41-2-4 | UE-based Carrier Phase Positioning | 1. Support of carrier phase measurement for UE-based positioning 2. Support of Assistance data for UE-based Carrier Phase Positioning | FFS | No | N.A. | UE-based Carrier Phase Positioning is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Open issue 10: Whether Additional finer-grained capabilities for RAT dependent positioning integrity is needed or not.

Open issue 11: Whether the UE capability on supporting alignment of PRS to fixed (e)DRX is needed or not.

Open issue 12: Whether the UE capability on supporting SRS with validity area request by RRC message is needed or not.

In addition, according to the discussion in last RAN2 meeting, whether to support scheduled location time and triggered event will not be discussed in capability discussion. Only capability signalling will be discussed in capability discussion when they are determined to be supported.