3GPP TSG-RAN2 #124	R2-23xxxxx

Chicage, Nov 13-17, 2023

Agenda Item:	7.2.2

Source: 	Xiaomi

Title:  	TP for SLPP and RRC capability signalling for SL positioning

Document for:	Discussion and Decision

# Introduction

RAN1 has sent RAN1 feature list in R1-2310635, although a lot of unresolved part marked with yellow are still there, we can start our initial stage-3 implementation of RAN1 feature list. For those RAN1 feature list marked as yellow, if it is implementable, it will be implementated, based on some assumption, and marked as yellow.

RAN2 also agreed some RAN2 related capabilities:

Agreements:

Introduce the UE capability on supporting positioning mode(i.e. UE based, UE assisted) per positioning method in SLPP.

Introduce the UE capability on supporting periodical reporting per positioning method in SLPP.

Introduce the UE capability on supporting lower value of response time (e.g. 10ms) per positioning method in SLPP.

In this contribution, we provide the TP for 38.355 and 38.331 for UE capabilities based on RAN1 and RAN2 feature list.

# Discussion

## Assumption and Clarification

Assumption on the corresponding positioning method for each RAN1 features:

| COMMON for all SL-PRS related positioning method | 41-1-1 | Common SL PRS Processing Capability in a SL BWP |
| --- | --- | --- |
|  | 41-1-1 | Common SL PRS Processing Capability in a SL BWP |
|  | 41-1-2 | Receiving SL-PRS in a shared resource pool |
|  | 41-1-3 | Receiving SL-PRS in a dedicated resource pool |
|  | 41-1-4a | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool |
|  | 41-1-4b | Transmitting SL-PRS scheme 1 in a dedicated resource pool |
|  | 41-1-4c | Transmitting SL-PRS mode 2 in a dedicated resource pool |
|  | 41-1-5 | SL-PRS congestion control in a dedicated resource pool |
|  | 41-1-7e | SL PRS measurement for SL PRS-RSRP |
|  | 41-1-7f | SL PRS measurement for SL PRS-RSRPP |
|  | 41-1-8 | Support of random selection in a dedicated resource pool |
|  | 41-1-10 | Support of full sensing in a dedicated resource pool |
|  | 41-1-11 | TDM-based multiplexing of SL-PRS [transmission/reception] from different UEs in the same slot in dedicated resource pool |
|  | 41-1-12 | Comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool |
|  | 41-1-13 | Reporting the additional paths for SL positioning |
|  | 41-1-14 | LoS/NLoS indicator for SL positioning [per measurement] |
|  | 41-1-17 | Open loop SL pathloss based power control and SL RSRP report [for dedicated resource pool] |
|  | 41-1-19 | ARP location provision for sidelink as assistance data |
|  | 41-1-19a | Report of Rx ARP-ID with SL positioning measurements |
| SL-RTT | 41-1-7c | SL PRS measurement for UE Rx – Tx time difference without Tx time stamp |
|  | 41-1-7d | SL PRS measurement for UE Rx – Tx time difference with Tx time stamp |
| SL-TDOA | 41-1-7a | SL PRS measurement for SL-RSTD |
| SL-TOA | 41-1-7b | SL PRS measurement for SL RTOA |
| SL-AOA | 41-1-7g | SL PRS measurement for SL AoA |

Assumption on Per FS/Per Band/Per FCPC:

-	For those FFS on whether per band or not, i.e. 1/4a/4b/4c/10/11/12/19/19a there are not implemented.

Assumption on positioning mode:

-	It is unclear whether Periodical Reporting and 10ms granularity response time is per position method or  per positioning mode per positioning method. They are implemented per positioning method, open issue will be added to the open issue list.

-	It is unclear whether UE-only mode is considered as standalone mode or not. The assumption is that it is considered as UE-based or UE-assisted mode, open issue will be added to the open issue list.

Clarification:

-	Feature indexes are added in the implementation( e.g. feature name) just for easy reference, and will be removed later.

-	Prerequisite feature groups are still addressed by feature index, and will be changed to feature name after the yellow part of prerequisite feature groups are resolved.

-	For those FFS on whether in SLPP or not, i.e. 4b/11/12/13/14/19/19a, they are not implemented.

## TP for RAN1+2 feature list--TS38.355

- –	Common-SL-PRS-Methods-ProvideCapabilities

-- ASN1START

-- TAG-CommonSLPRSMethodsProvideCapabilities-START

Common-SL-PRS-Methods-ProvideCapabilities ::= SEQUENCE {

Sl-PRS-CapabilityBandList					SEQUENCE (SIZE (1..maxBands)) OF

SL-PRS-CapabilityPerBand,

...,

}

SL-PRS-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-2 Receiving SL-PRS in a shared resource pool

sl-PRS-RxInSharedResourcePool				ENUMERATED {supported}			OPTIONAL,

--R1 41-1-3 Receiving SL-PRS in a dedicated resource pool

sl-PRS-RxInDedicatedResourcePool			ENUMERATED {supported}			OPTIONAL,

--R1 41-1-7e SL PRS measurement for SL PRS-RSRP

sl-PRS-RSRP-Meas							ENUMERATED {supported}			OPTIONAL,

--R1 41-1-7f SL PRS measurement for SL PRS-RSRPP

sl-PRS-RSRPP-Meas							ENUMERATED {supported}			OPTIONAL,

--R1 41-1-17 Open loop SL pathloss based power control and SL RSRP report [for dedicated resource pool]

sl-PathlossBasedOLPC-SL-RSRP-Report			ENUMERATED {supported}			OPTIONAL,

...,

}

-- TAG-CommonSLPRSMethodsProvideCapabilities-START

-STOP

-- ASN1STOP

Common-SL-PRS-Methods-ProvideCapabilities field descriptions

sl-PRS-RxInDedicatedResourcePool-r18(41-1-3) Indicates whether UE supports receiving SL-PRS in dedicated resource pool and receiving SCI format 1B. UE supporting this feature shall also support [15-2, 15-4, 41-1-2]. [UE indicating support of FG 41-1-1 must indicate either this feature group or feature group 41-1-2 is supported or both are supported]

sl-PRS-RxInSharedResourcePool-r18(41-1-2) Indicates whether UE supports receiving SL-PRS in shared resource pool and receiving SCI format 2D. UE supporting this feature shall also support [15-1, 15-4, 41-1-1]. [UE indicating support of FG 41-1-1 must indicate either this feature group or feature group 41-1-3 is supported or both are supported]

sl-PRS-RSRP-Meas-r18(41-1-7e) Indicates whether UE supports SL PRS measurement for SL PRS-RSRP, and is comprised of the following functional components: - Support SL PRS-RSRP measurement based on SL-PRS. - Support SL PRS-RSRP measurement reporting.

sl-PRS-RSRPP-Meas-r18(41-1-7f) Indicates whether UE supports SL PRS measurement for SL PRS-RSRPP, and is comprised of the following functional components: - Support SL PRS-RSRPP measurement based on SL-PRS. - Support SL PRS-RSRPP measurement reporting.

sl-PathlossBasedOLPC-SL-RSRP-Report(41-1-17) Indicates whether UE supports Open loop SL pathloss based power control and SL RSRP report [for dedicated resource pool] for unicast transmissions.

- –	SL-TDOA-ProvideCapabilities

The IE SL-TDOA-ProvideCapabilities is used to indicate the support of SL-TDOA and to provide SL-TDOA positioning capabilities to the location server.

-- ASN1START

-- TAG-SLTDOAPROVIDECAPABILITIES-START

SL-TDOA-ProvideCapabilities ::= SEQUENCE {

sl-TDOA-CapabilityBandList					SEQUENCE (SIZE (1..maxBands)) OF

SL-TDOA-CapabilityPerBand,

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime-r17				PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...,

}

SL-TDOA-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-7a	SL PRS measurement for SL-RSTD

sl-PRS-RSTD-Meas-r18						ENUMERATED {1,2,3,4}						OPTIONAL,

...,

}

-- TAG-SLTDOAPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-TDOA-ProvideCapabilities field descriptions

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

sl-PRS-RSTD-Meas-r18(41-1-7a) Indicates whether UE supports SL PRS measurement for SL-RSTD, and is comprised of the following functional components: - Support SL RSTD measurement based on SL-PRS - Support SL RSTD measurement reporting [FFS reporting N SL RSTD measurements for different SL-PRS reception for the same pair of UEs].

sl-TDOA-Mode This field specifies the SL-TDOA mode(s) supported by the target device.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

- –	SL-TOA-ProvideCapabilities

The IE SL-TOA-ProvideCapabilities is used to indicate the support of SL-TOA and to provide SL-TOA positioning capabilities to the location server.

-- ASN1START

-- TAG-SLTOAPROVIDECAPABILITIES-START

SL-TOA-ProvideCapabilities ::= SEQUENCE {

sl-TOA-CapabilityBandList					SEQUENCE (SIZE (1..maxBands)) OF

SL-TOA-CapabilityPerBand,

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime					PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...,

}

SL-TOA-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-7b SL PRS measurement for SL RTOA

sl-RTOA-Meas-r18							ENUMERATED {1,2,3,4}						OPTIONAL,

...,

}

-- TAG-SLTOAPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-TOA-ProvideCapabilities field descriptions

sl-RTOA-Meas-r18 Indicates whether UE supports SL PRS measurement for SL-RTOA, and is comprised of the following functional components: - Support SL RTOA measurement based on SL-PRS - Support SL RTOA measurement reporting [FFS reporting N SL RTOA measurements for different SL-PRS reception for the same pair of UEs].

- –	SL-AOA-ProvideCapabilities

The IE SL-AOA-ProvideCapabilities is used to indicate the support of SL-AOA and to provide SL-AOA positioning capabilities to the location server.

-- ASN1START

-- TAG-SLAOAPROVIDECAPABILITIES-START

SL-AOA-ProvideCapabilities ::= SEQUENCE {

sl-AOA-CapabilityBandList					SEQUENCE (SIZE (1..maxBands)) OF

SL-AOA-CapabilityPerBand,

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime					PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...,

}

SL-AOA-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-7g SL PRS measurement for SL RTOA

sl-AOA-Meas-r18							BIT STRING { gcs 					(0),

lcsWithTranslation 	(1),

lcsWithoutTranslation 	(2)}	(SIZE (1..8))	OPTIONAL,

...,

}

-- TAG-SLPRSAOAPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-AOA-ProvideCapabilities field descriptions

sl-AOA-Meas-r18 Indicates whether UE supports SL PRS measurement for SL-AOA, and is comprised of the following functional components: - Support SL AOA measurement based on SL-PRS - Support SL AOA measurement reporting types.

The value indicates the supported SL AOA measurement reporting types. The left most bit in the bitmap corresponds to GCS, the next bit in the bitmap corresponds to LCS with translation, the right most bit in the bitmap corresponds to LCS without translation. A bit in the bitmap is set to 1 if the corresponding type is supported by the UE.

- –	SL-RTT-ProvideCapabilities

The IE SL-RTT-ProvideCapabilities is used to indicate the support of SL-RTT and to provide SL-RTT positioning capabilities to the location server.

-- ASN1START

-- TAG-SLRTTPROVIDECAPABILITIES-START

SL-RTT-ProvideCapabilities ::= SEQUENCE {

sl-RTT-CapabilityBandList					SEQUENCE (SIZE (1..maxBands)) OF

SL-RTT-CapabilityPerBand,

positioningModes							PositioningModes,

ten-ms-unit-ResponseTime					PositioningModes						OPTIONAL,

periodicalReporting						PositioningModes						OPTIONAL,

...,

}

SL-RTT-CapabilityPerBand ::= SEQUENCE {

--R1 41-1-7c SL PRS measurement for UE Rx – Tx time difference without Tx time stamp

Sl-UE-Rx-TxTimeDiffMeasWithoutTxTimeStamp-r18	ENUMERATED {1,2,3,4}				OPTIONAL,

--R1 41-1-7d SL PRS measurement for UE Rx – Tx time difference with Tx time stamp

Sl-UE-Rx-TxTimeDiffMeasWithTxTimeStamp-r18		SEQUENCE {

m-r18									ENUMERATED {1,2,3,4}

n-r18									ENUMERATED {1,2,3,4}

}																			OPTIONAL,

...,

}

-- TAG-SLRTTPROVIDECAPABILITIES-STOP

-- ASN1STOP

SL-RTT-ProvideCapabilities field descriptions

Sl-UE-Rx-TxTimeDiffMeasWithoutTxTimeStamp-r18(41-1-7c) Indicates whether UE supports SL PRS measurement for UE Rx – Tx time difference without Tx time stamp, and is comprised of the following functional components: - Support UE Rx – Tx time difference measurement based on SL PRS. - Support UE Rx – Tx time difference measurement reporting without Tx time stamp. [FFS reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs]

Sl-UE-Rx-TxTimeDiffMeasWithTxTimeStamp-r18(41-1-7d) Indicates whether UE supports SL PRS measurement for UE Rx – Tx time difference with Tx time stamp, and is comprised of the following functional components: - Support UE Rx – Tx time difference measurement based on SL PRS. - Support UE Rx – Tx time difference measurement reporting with Tx time stamp. - Support reporting M Rx-Tx measurements for the same SL-PRS transmission (or reception) and different SL-PRS reception (or transmission) for the same pair of UEs. [FFS reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs]

- –	PositioningModes

The IE PositioningModes is used to indicate several positioning modes using a bit map.

-- ASN1START

PositioningModes ::= SEQUENCE {

posModes		BIT STRING {	ue-based		(0),

ue-assisted	(1) } (SIZE (1..8)),

...

}

-- ASN1STOP

PositioningModes field descriptions

posModes This field specifies the positioning mode(s). This is represented by a bit string, with a one-value at the bit position means the particular positioning mode is addressed; a zero-value means not addressed.

## TP for RAN1 feature list-TS38.331

R1 features: 41-1-2/3/5/8/17:

BandSidelink-r16 ::=  SEQUENCE {

freqBandSidelink-r16                          FreqBandIndicatorNR,

--R1 41-1-2 Receiving SL-PRS in a shared resource pool

sl-PRS-RxInSharedResourcePool-r18		 			ENUMERATED {supported}			OPTIONAL,													--R1 41-1-3 Receiving SL-PRS in a dedicated resource pool

sl-PRS-RxInDedicatedResourcePool-r18				ENUMERATED {supported}				OPTIONAL,

--R1 41-1-5 SL-PRS congestion control in a dedicated resource pool

sl-PRS-CongestionCtrlInDedicatedResourcePool-r18		ENUMERATED {time1, time2}			OPTIONAL,

--R1 41-1-8 Support of random selection in a dedicated resource pool

sl-PRS-TxRandomSelection-r18							ENUMERATED {supported}			OPTIONAL,

--R1 41-1-17 Open loop SL pathloss based power control and SL RSRP report [for dedicated resource pool]

sl-PathlossBasedOLPC-SL-RSRP-Report-r18			ENUMERATED {supported}			OPTIONAL,

--15-1

sl-Reception-r16                              SEQUENCE {

harq-RxProcessSidelink-r16                    ENUMERATED {n16, n24, n32, n48, n64},

pscch-RxSidelink-r16                          ENUMERATED {value1, value2},

scs-CP-PatternRxSidelink-r16                  CHOICE {

fr1-r16                                       SEQUENCE {

scs-15kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r16                                       SEQUENCE {

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r16                                BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-RxSidelink-r16                     ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-2

sl-TransmissionMode1-r16                      SEQUENCE {

harq-TxProcessModeOneSidelink-r16             ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeOne-r16           CHOICE {

fr1-r16                                       SEQUENCE {

scs-15kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r16                                       SEQUENCE {

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r16                                BIT STRING (SIZE (16))                OPTIONAL

}

},

extendedCP-TxSidelink-r16                     ENUMERATED {supported}                        OPTIONAL,

harq-ReportOnPUCCH-r16                        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-4

sync-Sidelink-r16                             SEQUENCE {

gNB-Sync-r16                                  ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNB-ENB-r16     ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNSS-r16        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-10

sl-Tx-256QAM-r16                              ENUMERATED {supported}                            OPTIONAL,

--15-11

psfch-FormatZeroSidelink-r16                  SEQUENCE {

psfch-RxNumber                                ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64},

psfch-TxNumber                                ENUMERATED {n4, n8, n16}

}                                                                                               OPTIONAL,

--15-12

lowSE-64QAM-MCS-TableSidelink-r16             ENUMERATED {supported}                            OPTIONAL,

--15-15

enb-sync-Sidelink-r16                         ENUMERATED {supported}                            OPTIONAL,

...,

[[

--15-3

sl-TransmissionMode2-r16                      SEQUENCE {

harq-TxProcessModeTwoSidelink-r16              ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r16           ENUMERATED {supported}                        OPTIONAL,

dl-openLoopPC-Sidelink-r16                    ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-5

congestionControlSidelink-r16                 SEQUENCE {

cbr-ReportSidelink-r16                        ENUMERATED {supported}                        OPTIONAL,

cbr-CR-TimeLimitSidelink-r16                  ENUMERATED {time1, time2}

}                                                                                               OPTIONAL,

--15-22

fewerSymbolSlotSidelink-r16                   ENUMERATED {supported}                            OPTIONAL,

--15-23

sl-openLoopPC-RSRP-ReportSidelink-r16         ENUMERATED {supported}                            OPTIONAL,

--13-1

sl-Rx-256QAM-r16                              ENUMERATED {supported}                            OPTIONAL

]],

[[

ue-PowerClassSidelink-r16                         ENUMERATED {pc2, pc3, spare6, spare5, spare4, spare3, spare2, spare1}

OPTIONAL

]],

[[

--32-4a

sl-TransmissionMode2-RandomResourceSelection-r17 SEQUENCE {

harq-TxProcessModeTwoSidelink-r17             ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17               CHOICE {

fr1-r17                                           SEQUENCE {

scs-15kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-30kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-60kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL

},

fr2-r17                                           SEQUENCE {

scs-60kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-120kHz-r17                                    BIT STRING (SIZE (16))            OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-Mode2Random-r17                    ENUMERATED {supported}                        OPTIONAL,

dl-openLoopPC-Sidelink-r17                    ENUMERATED {supported}                            OPTIONAL

}                                                                                               OPTIONAL,

--32-4b

sync-Sidelink-v1710                           SEQUENCE {

sync-GNSS-r17                                 ENUMERATED {supported}                        OPTIONAL,

gNB-Sync-r17                                  ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNB-ENB-r17     ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNSS-r17        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--32-4c

enb-sync-Sidelink-v1710                       ENUMERATED {supported}                            OPTIONAL,

--32-5a-2

rx-IUC-Scheme1-PreferredMode2Sidelink-r17     ENUMERATED {supported}                            OPTIONAL,

--32-5a-3

rx-IUC-Scheme1-NonPreferredMode2Sidelink-r17  ENUMERATED {supported}                            OPTIONAL,

--32-5b-2

rx-IUC-Scheme2-Mode2Sidelink-r17              ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-6-1

rx-IUC-Scheme1-SCI-r17                        ENUMERATED {supported}                            OPTIONAL,

--32-6-2

rx-IUC-Scheme1-SCI-ExplicitReq-r17            ENUMERATED {supported}                            OPTIONAL

]]

}

# Annex RAN2+RAN3 UE feature list

- 7	Release 18 UE feature list

- 7.2	Layer-2 and Layer-3 features

- 7.2.x	NR_pos_enh2

Table 7.2.x-1: Layer-2 and Layer-3 feature list for NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.355 [x] | Parent IE in TS 38.355 [x] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | x-1 | Periodical Reporting | Indicates whether the UE supports periodical Reporting for SL-AoA, SL-RTT, SL-TDOA or SL-TOA. |  | periodicalReporting | NR-Multi-RTT-ProvideCapabilities-r16 or NR-DL-TDOA-ProvideCapabilities-r16 or NR-ECID-ProvideCapabilities-r16 or NR-DL-AoD-ProvideCapabilities-r16 LPP | N/A | N/A |  | Optional with capability signalling |
|  | x-2 | 10 ms granularity for response time | Indicates whether the 'ten-milli-seconds' response time unit is supported by the target device. |  | ten-ms-unit-ResponseTime | NR-ECID-ProvideCapabilities or NR-DL-TDOA-ProvideCapabilities or NR-DL-AoD-ProvideCapabilities or NR-Multi-RTT-ProvideCapabilities | N/A | N/A |  | Optional with capability signalling |
|  | x-3 | Positioning Modes | Indicates what positoining mode the UE supports for SL-AoA, SL-RTT, SL-TDOA or SL-TOA. The positioning mode includes ue-based, and ue-assisted. |  | positioningModes | SL-AOA-ProvideCapabilities, or SL-RTT-ProvideCapabilities, or SL-TDOA-ProvideCapabilities, or SL-TOA-ProvideCapabilities | N/A | N/A |  | Optional with capability signalling |