3GPP TSG-RAN2 Meeting # 123b	R2-231xxxx

# Xiamen, China, October 9 - 13

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | draftCR | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR of TS 38.351 for NR sidelink relay enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-9 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel18 features for NR sidelink relay enhancements are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel18 features for NR sidelink relay enhancements are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel18 features for NR sidelink relay enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TBD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Egress RLC channel: a RLC channel on which a packet is transmitted by a U2N Relay UE, a U2N Remote UE or a network node.

Egress link: a radio link on which a packet is transmitted by a U2N Relay UE, a U2N Remote UE or a network node.

Ingress RLC channel: a RLC channel on which a packet is received from a U2N Relay UE, a U2N Remote UE or a network node.

Ingress link: a radio link on which a packet is received from a U2N Relay UE, a U2N Remote UE or a network node.

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE.

U2U Relay UE: a UE that provides functionality to support connectivity for U2U Remote UE(s).

U2U Remote UE: a UE that communicates with other UE via a U2U Relay UE.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

SRAP	Sidelink Relay Adaptation Protocol

U2N	UE-to-Network

U2U	UE-to-UE

Next Change

4.2.2	SRAP entities

Figure 4.2.2-1 represents one possible structure for the SRAP sublayer. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

Figure 4.2.2-1: SRAP structure overview

On the U2N Relay UE, the SRAP sublayer contains one SRAP entity at Uu interface and a separate collocated SRAP entity at the PC5 interface. On the U2N Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface. On the U2N Remote UE, U2U Relay UE and U2U Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface.

Editor’s Notes: FFS on the detailed SRAP entity description for U2U Remote UE and U2U Relay UE.

Each SRAP entity has a transmitting part and a receiving part. Across the PC5 interface in the U2N case, the transmitting part of the SRAP entity at the U2N Remote UE has a corresponding receiving part of an SRAP entity at the U2N Relay UE, and vice versa. Across the Uu interface, the transmitting part of the SRAP entity at the U2N Relay UE has a corresponding receiving part of an SRAP entity at the gNB, and vice versa.

Across the PC5 interface in the U2U case, the transmitting part of the SRAP entity at the U2U Remote UE has a corresponding receiving part of an SRAP entity at the U2U Relay UE, and vice versa.

Figure 4.2.2-2 and Figure 4.2.2-3 represents the functional view of the SRAP entity in U2N Relay case for the SRAP sublayer at PC5 interface and at Uu interface respectively. Figure 4.2.2-4 represents the functional view of the SRAP entity in U2U Relay case for the SRAP sublayer at PC5 interface.

Figure 4.2.2-2: Example of functional view of SRAP sublayer at PC5 interface in U2N Relay case

Figure 4.2.2-3: Example of functional view of SRAP sublayer at Uu interface

In the example of Figure 4.2.2-2 and Figure 4.2.2-3, at relay UE:

-	For data packet not corresponding to SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the receiving part on the SRAP entity of PC5 interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of Uu interface. As an alternative, the receiving part may deliver SRAP SDUs to the transmitting part on the collocated SRAP entity. When passing SRAP SDUs, the receiving part removes the SRAP header and the transmitting part of the relay UE adds the SRAP header with the same SRAP header content as carried on the SRAP Data PDU header prior to removal. Passing SRAP SDUs in this manner is therefore functionally equivalent to passing SRAP Data PDUs, in implementation. The following specification therefore refers to the passing of SRAP data packets.

-	For UL data packet corresponding to SRB0, the receiving part on the SRAP entity of PC5 interface delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of Uu interface, and the transmitting part on the SRAP entity of Uu interface adds the SRAP header in accordance with clause 5.3.3.

-	For DL data packet corresponding to SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the transmitting part on the SRAP entity of PC5 interface removes the SRAP header in accordance with clause 5.2.2. As an alternative for handling DL data packet corresponding to SRB0 not shown in Figure 4.2.2-2 or Figure 4.2.2-3, the receiving part on the SRAP entity of Uu interface removes the SRAP header and delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of PC5 interface.

Figure 4.2.2-4: Example of functional view of SRAP sublayer at PC5 interface in U2U Relay UE case

In the example of Figure 4.2.2-4, at U2U relay UE:

-	For U2U data packet, the receiving part on the SRAP entity of PC5 interface between the U2U Relay UE and the U2U Remote UE delivers SRAP Data PDUs to the transmitting part on the SRAP entity of PC5 interface between the U2U Relay UE and the peer U2U Remote UE.  As an alternative, the receiving part may deliver SRAP SDUs to the transmitting part on the SRAP entity. When passing SRAP SDUs, the receiving part removes the SRAP header and the transmitting part of the relay UE adds the SRAP header with the same SRAP header content as carried on the SRAP Data PDU header prior to removal. Passing SRAP SDUs in this manner is therefore functionally equivalent to passing SRAP Data PDUs, in implementation. The following specification therefore refers to the passing of SRAP data packets.

Next Change

4.5	Configurations

The configuration of the SRAP entity for U2N Remote UE includes:

-	Mapping from a radio bearer identified by BEARER ID field to egress PC5 Relay RLC channel via RRC;

-	The local identity via RRC.

The configuration of the SRAP entity for U2N Relay UE includes:

-	The local identity for each U2N Remote UE via RRC;

-	Mapping from a radio bearer identified by UE ID field and BEARER ID field to egress Uu Relay RLC channel for each U2N Remote UE via RRC;

-	Mapping from a radio bearer identified by UE ID field and BEARER ID field to egress PC5 Relay RLC channel for each U2N Remote UE via RRC.

The configuration of the SRAP entity for U2U Remote UE includes:

-	Mapping from a radio bearer identified by BEARER ID field to egress PC5 Relay RLC channel;

-	The local identities.

The configuration of the SRAP entity for U2U Relay UE includes:

-	Mapping from a radio bearer identified by BEARER ID field to egress PC5 Relay RLC channel for each U2U Remote UE pair and each direction of data transfer;

Editor’s Notes: FFS on the detailed configurations of the SRAP entity for U2U Remote UE and U2U Relay UE (e.g., how to identify the radio bearer mapping configuration).

Next Change

5.x	U2U SL Data transfer

5.x.1	Transmitting operation of U2U Remote UE

The transmitting part of the SRAP entity on the PC5 interface of U2U Remote UE can receive SRAP SDU from upper layer and constructs SRAP Data PDU.

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.x.1.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.x.1.2;

-	Submit this SRAP Data PDU to the determined egress PC5 Relay RLC channel.

5.x.1.1	UE ID field and BEARER ID field determination

For an U2U SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID field corresponding to [XXX], configured as specified in TS 38.331 [3];

-	Determine the BEARER ID field for SL-SRBs as the fixed value (i.e., set 0/1/2/3 for SL-SRB0/1/2/3 respectively).

Editor’s Notes: The BEARER ID determination for DRBs will be added after we conclude on “FFS how to derive 5-bits value BEARER ID from SLRB configuration index”.

5.x.1.2	Egress RLC channel determination

Editor’s Notes: FFS on the detailed Egress RLC channel determination at U2U Remote UE.

5.x.2	Receiving operation of U2U Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the PC5 interface between the U2U Relay UE and the U2U Remote UE shall:

-	deliver the SRAP data packet to the transmitting part of the SRAP entity on the PC5 interface between the U2U Relay UE and the peer U2U Remote UE.

5.x.3	Transmitting operation of U2U Relay UE

The transmitting part of the SRAP entity of U2U Relay UE on the PC5 interface between the U2U Relay UE and the peer U2U Remote UE can receive SRAP data packets from the receiving part of the SRAP entity of the same U2U Relay UE on the PC5 interface between the U2U Relay UE and the U2U Remote UE, and constructs SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity of the U2U Relay UE has an SRAP Data PDU to transmit on the PC5 interface between U2U Relay UE and the peer U2U Remote UE, the transmitting part of the SRAP entity of U2U Relay UE shall:

-	Determine the egress link in accordance with clause 5.x.3.1;

-	Determine the egress RLC channel in accordance with clause 5.x.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

5.x.3.1	Egress link determination

For a U2U SRAP Data PDU to be transmitted, SRAP entity shall:

-	Determine the egress link on PC5 interface towards the peer U2U remote based on the UE ID pair in the U2U SRAP Data PDU.

Editor’s Notes: FFS on the detailed Egress link determination at U2U Relay UE.

5.x.3.2	Egress RLC channel determination

Editor’s Notes: FFS on the detailed Egress RLC channel determination at U2U Relay UE.

5.x.4	Receiving operation of U2U Remote UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	Remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer entity corresponding to the BEARER ID and UE ID fields of this SRAP Data PDU.

Next Change

5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	discard the received SRAP Data PDU.

When any of the U2N Remote UE, the U2N Relay UE, the U2U Remote UE or the U2U Relay UE receives a SRAP PDU with invalid or reserved values, the SRAP entity shall:

-	discard the received SRAP PDU.

Editor’s Notes: FFS on the other error handling for U2U Remote UE and U2U Relay UE.

Next Change

6.2.2	Data PDU

Figure 6.2.2-1 shows the format of the U2N SRAP Data PDU with SRAP header being configured. This SRAP Data PDU format is applicable to U2N SRAP SDU except those for SRB0 delivered over PC5 interface.

Figure 6.2.2-1: U2N SRAP Data PDU format with SRAP header

Figure 6.2.2-2 shows the format of the U2N SRAP Data PDU consisting only of a data field without any SRAP header. This SRAP Data PDU format is applicable to U2N SRAP SDU for SRB0 delivered over PC5 interface.

Figure 6.2.2-2: U2N SRAP Data PDU format without SRAP header

Figure 6.2.2-3 shows the format of the U2U SRAP Data PDU. This SRAP Data PDU format is applicable to U2U SRAP SDU delivered over PC5 interface.

Figure 6.2.2-3: U2U SRAP Data PDU format with SRAP header

Next Change

6.3	Parameters

6.3.1	General

If not otherwise mentioned in the definition of each field the bits in the parameters shall be interpreted as follows: the left most bit is the first and most significant and the right most bit is the last and least significant bit.

Unless otherwise mentioned, integers are encoded in standard binary encoding for unsigned integers. In all cases the bits appear ordered from MSB to LSB when read in the PDU.

6.3.2	UE ID

Length: 8 bits.

In U2N Relay case, this field carries local identity of U2N Remote UE. In U2U Relay case, two such fields are present, one for local identity of each U2U Remote UE.

6.3.3	BEARER ID

Length: 5 bits.

In U2N Relay case, this field carries information to identify Uu radio bearer for U2N Remote UE. For SRBs, the value is set to SRB Identity (which is configured by RRC parameter srb-Identity). For DRBs, the value is set to DRB Identity (which is configured by RRC parameter drb-Identity) minus 1.

In U2U Relay case, this field carries information to identify PC5 radio bearer for U2U Remote UE. For SL-SRBs, the value is set to 0/1/2/3 for SL-SRB 0/1/2/3 respectively.

Editor’s Notes: FFS how to derive 5-bits value BEARER ID from SLRB configuration index.

End of Change

  - Annex: Open issue list

|  | Open Issue | CR-Rapporteur Comment |
| --- | --- | --- |
| 1 | To solve ENs EN at clause 4.5: Editor’s Notes: FFS on the detailed configurations of the SRAP entity for U2U Remote UE and U2U Relay UE (e.g., how to identify the radio bearer mapping configuration). EN at clause 5.x.1.2: Editor’s Notes: FFS on the detailed Egress RLC channel determination at U2U Remote UE. EN at clause 5.x.3.2: Editor’s Notes: FFS on the detailed Egress RLC channel determination at U2U Relay UE. EN at clause 5.x.3.1: Editor’s Notes: FFS on the detailed Egress link determination at U2U Relay UE. | Depends on the 331 Running CR discussion on how the SRAP configuration is derived for the 2 hops |
| 2 | To solve ENs EN at clause 5.X.1.1: Editor’s Notes: The BEARER ID determination for DRBs will be added after we conclude on “FFS how to derive 5-bits value BEARER ID from SLRB configuration index”. EN at clause 6.3.3: Editor’s Notes: FFS how to derive 5-bits value BEARER ID from SLRB configuration index. | Rapp understanding according to legacy SL mechanism, it can be 5 bits LSB of the 9 bits SLRB configuration index. |
| 3 | EN in clause 5.4: Editor’s Notes: FFS on the other error handling for U2U Remote UE and U2U Relay UE. | The other error handling includes that UE discard the SRAP PDU if the UE ID or BEARER ID field contains in the received SRAP PDU does not match with the SRAP configuration, or if the SRAP PDU within invalid or reserved value. |
| 4 | EN in clause 4.2.2: Editor’s Notes: FFS on the detailed SRAP entity description for U2U Remote UE and U2U Relay UE. | As commented by company, for the SRAP entity modelling at U2U Relay UE, only one SRAP entity is needed, so the current running CR (in clause 4.2.2) is reworded to address this comment. If no objection, the EN can be removed. |