3GPP TSG-RAN WG2 Meeting #123bis	R2-2310XXX

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.11.1

Source:	CMCC

Title:	38.300 running CR open issues for eMBS

Document for:	Discussion

Introduction

This document captures the outcome of the stage 2 Running CR open issues in the following email discussion:

- [Post123bis][610][eMBS] 38.300 CR update and open issues (CMCC)

Scope: Running CR update and open issues

Intended outcome:

- Endorsed running CR

- List of open issues for TS 38.300 (separate document)

Deadline: Long

Please provide your comments before Oct. 26th  1000 UTC

Contact Points

Rapporteur encourages the participating delegates to provide their contact information in this table.

| Company | Name | Email Address |
| --- | --- | --- |

Discussion

- Open issue 1: FFS that the above description of PTM configuration(s) delivery will be revised according to future conclusions.

In the previous discussion, RAN2 agreed to use a mixed approach to provide PTM configuration, and furtherly, it was agreed to indicate whether one multicast service can be received in RRC_INACTIVE and “the stop monitoring of G-RNTI”. Besides, in last RAN2 meeting, there’s agreements on the rules of MCCH’s present, then we think the FFS can be removed and description to capture the latest agreements are as following:

Proposal 1: Remove the Editor’s note for PTM configuration description in section 16.10.5.2

Do you agree with Proposal 1?

| Company | Yes/No | Comments |
| --- | --- | --- |

- Open issue 2: Whether the UE can get the initial PTM configuration via multicast MCCH.

Since RAN2 agreed that if network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration, then it’s possible that UE may have no PTM configuration when it is released to RRC_INACTIVE state. In last RAN2 meeting, it was agreed that if “the stop of G-RNTI monitoring” for a session  is indicated in RRCRelease message and the PTM configuration of the corresponding multicast session is not included in same message, UE reads multicast MCCH(if present) upon receiving group paging that indicates to allow the multicast reception in RRC_INACTIVE.  Therefore, the related editor note can be removed.

Proposal 2: Remove the Editor’s note for initial PTM configuration acquisition in section 16.10.5.2

Do you agree with Proposal 2?

| Company | Yes/No | Comments |
| --- | --- | --- |

- Open issue 3: FFS how the UE is indicated about cells being synchronized (i.e. what information the network needs to provide to the UE).

In last meeting, agreements on how to indicate about cells being synchronized were made, then the editor’s note can be removed.

Proposal 3: Remove the Editor’s note for cells’ synchronized indication in section 16.10.5.3.X.

Do you agree with Proposal 3?

| Company | Yes/No | Comments |
| --- | --- | --- |

- Open issue 4: Whether we need something more, e.g. frequency priorities in MCCH or a solution based on FSAI.

On frequency prioritization, RAN2 agreed to used dedicated frequencies in RRCRelease message and legacy.

Dedicated frequencies in RRCRelease can be used by the NW, as legacy

And it should be noticed that the dedicated frequency information may become invalid due to mobility or related timer expiring. From this point of view, additional mechanism can be considered, such as frequency priorities in MCCH or a solution based on FSAI. If additional mechanism is considered, extra efforts is needed. However, the time of the WI is limited, considering the potential workload and effect to SA2/SA6 (for FSAI based solution),we suggest that no extra frequency prioritization mechanism is introduced and the editor’s note can be removed.

Option 1: No extra frequency prioritization mechanism is introduced and the editor’s note can be removed;

Option 2a: Frequency priorities in MCCH;

Option 2b: FSAI based solution for frequency prioritization.

Proposal 4: No extra frequency prioritization mechanism is introduced and the editor’s note can be removed

Do you agree with Proposal 4?

| Company | Yes/No | Comments |
| --- | --- | --- |

- Open issue 5: Whether/how we need to address ping-pong issue.

Based on the discussion in last meeting, most companies thought there’s no ping-pong issue, and RAN2 agreed to introduce no new measurements and measurement requirements and no TTT. Thus, the editor’s note can be removed.

Proposal 5: Remove the Editor’s note for frequency prioritization enhancement in section 16.10.5.3.X.

Do you agree with Proposal 5?

| Company | Yes/No | Comments |
| --- | --- | --- |

- Open issue 6: Whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise).

For multicast CFR in RRC_INACTIVE design, RAN2 agreed that Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. Since in Rel-17, the simultaneous reception of multicast and broadcast is also supported, and no there’s no restriction to the CFR configuration of multicast and broadcast, therefore, we think the same principle can be reused. Besides, even one CFR is completely contained the other one, the configuration of the two CFR may be different, the gain for the UE is not clear, therefore, we prefer to remove the editor’s note without capturing such restrictions.

Proposal 6: Remove the Editor’s note for frequency prioritization enhancement in section 16.10.5.7.

Do you agree with Proposal 6 and the corresponding TP?

| Company | Yes/No | Comments |
| --- | --- | --- |

- Other open issues

Are there any additional open issues for discussion?

| Company | Open issue |
| --- | --- |

Conclusion

Based on the discussion above, it is proposed as following:

TBD