3GPP TSG-RAN WG2 Meeting #123bis	R2-231xxxx

2023 Xiamen, China, October 9th – 13th, 2023

Agenda Item: 	7.11.2.2

Source:	Apple

Title:	MAC open issues for eMBS (Apple)

Document for:		Discussion and Decision

# Introduction

This document captures the outcome of the MAC open issues discussion in the following email discussion:

- [Post123bis][612][eMBS] 38.321 CR update and open issues (Apple)

Scope: Running CR update and open issues

Intended outcome:

- Endorsed running CR

- List of open issues for TS 38.321 (separate document)

Deadline: Long

Please provide your comments before October 27th 0100 UTC.

# Contact Points

Rapporteur encourages the participating delegates to provide their contact information in this table.

| Company | Name | Email Address |
| --- | --- | --- |

# Discussion

### Open issue 1: Whether DRX Command MAC CE is applicable for inactive multicast DRX operation?

In R17 multicast DRX operation in RRC_CONNECTED state, network can use DRX Command MAC CE to request a specific MBS multicast session to leave DRX active time, and network will stop the scheduling data for this MBS multicast session.

For R18 inactive multicast DRX operation, since the R17 multicast DRX operation is reused for it and the purpose of DRX command MAC CE is still applicable for the inactive multicast DRX operation, network can also send the DRX Command MAC CE to inform RRC_INACTIVE UE out of DRX active time for a specific multicast session, which is good for UE power.

Proposal 1: DRX Command MAC CE is applicable for inactive multicast DRX operation.

With the proposal 3, we can remove the Editor Note 1. TP is as below.

5.7b Discontinuous Reception (DRX) for MBS Multicast …… 1> if a DRX Command MAC CE indicated by PDCCH addressed to a G-RNTI or G-CS-RNTI, or by a configured downlink multicast assignment is received: 2> stop drx-onDurationTimerPTM of the DRX for this G-RNTI or G-CS-RNTI; 2> stop drx-InactivityTimerPTM of the DRX for this G-RNTI or G-CS-RNTI.

##### Question 1: Do you agree with proposal 1 and the TP?

| Company | Proposal 1? | TP? | Comments |
| --- | --- | --- | --- |

### Open issue 2: FFS on the value of RNTI for multicast MCCH

In previous RAN2 discussion [1], majority view was to introduce the fix value for the multicast MCCH RNTI, and some companies thought multicast MCCH may be scheduled by G-RNTI at that time.

Rapp summary in [AT121bis-e][604] [1] On proposal a (i.e. fix value for multicast MCCH RNTI), 2 companies doesnot agree with it. On proposal b (i.e. exclude using G-RNTI for MCCH scheduling), 2 companies think we should consider to use G-RNTI for MCCH scheduling; but other companies cannot understand how it can work. Considering the company’s concerns, rapporteur suggests not to agree these proposals, and invites companies to provide the contribution to next meeting.

Since later RAN2 agreed to introduce multicast MCCH RNTI for the multicast MCCH scheduling, we think it should be agreeable to introduce a new fix value for the multicast MCCH RNTI.

Proposal 2: Introduce a new fix RNTI value for multicast-MCCH-RNTI.

Following is the corresponding TP:

Table 7.1-1: RNTI values. Value (hexa-decimal); RNTI. 0000; N/A. 0001–FFF2; RA-RNTI, MSGB-RNTI, Temporary C-RNTI, C-RNTI, CI-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, PS-RNTI, SL-RNTI, SLCS-RNTI SL Semi-Persistent Scheduling V-RNTI, AI-RNTI, G-RNTI, G-CS-RNTI, and CG-SDT-CS-RNTI. FFF3–FFFA; Reserved. FFFB; Multicast-MCCH-RNTI. FFFC; PEI-RNTI. FFFD; MCCH-RNTI. FFFE; P-RNTI. FFFF; SI-RNTI. Editor Note: FFS on the value of the multicast-MCCH-RNTI.

##### Question 2: Do you agree with proposal 2 and the TP?

| Company | Proposal 2? | TP? | Comments |
| --- | --- | --- | --- |

### Open issue 3: FFS on the value of the LCID for multicast MCCH

In previous RAN2 discussion [1], majority view was to introduce the new fix LCID value for the multicast MCCH, and some companies thought it’s unnecessary to introduce the new LCID for MCCH if RNTI value can be used to distinguish the multicast MCCH and broadcast MCCH.

Rapp summary in [AT121bis-e][604] [1] Proposal 10 In R2-2303420 (for agreement, 15/17): Introduce a new LCID in Table 6.2.1-1 for multicast MCCH. According to company’s input (including comments part) - On P10, 4 companies donot support it and they think the new LCID may not needed.

With the new RNTI value introduced for multicast-MCCH-RNTI, since it can be used to distinguish multicast MCCH and broadcast MCCH, it seems no issue to use the same LCID for both multicast MCCH and broadcast MCCH.

Proposal 3: The same LCID value is used for multicast MCCH and broadcast MCCH.

Following is the corresponding TP:

Table 6.2.1-1c Values of LCID for MBS broadcast and multicast on DL-SCH Codepoint/Index; LCID values. 0; Broadcast MCCH and multicast MCCH. 1–32; Identity of the logical channel of broadcast MTCH. 33–63; Reserved.

##### Question 3: Do you agree with proposal 3 and the TP?

| Company | Proposal 3? | TP? | Comments |
| --- | --- | --- | --- |

### Open issue 4: About the Editor Note in section 5.3.1 (DL Assignment reception)

Since RAN1 has confirmed in [2] that it is feasible to reuse the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE as the multicast MTCH in RRC_CONNECTED, then we assume Editor Note 1 in section 5.3.1 can be removed.

Editor Note 1: whether to restrict the multicast MTCH in this section in RRC_CONNECTED state is FFS.

Proposal 4: Remove the Editor Note 1 in section 5.3.1.

### 5.3.1 DL Assignment reception

##### Question 4: Do you agree with proposal 4 and the TP?

| Company | Proposal 4? | TP? | Comments |
| --- | --- | --- | --- |

### Other open issues?

##### Question 5: Please provide MAC open issues which is not listed above.

| Company | Open issues? |
| --- | --- |

# Conclusion

According to the analysis in section 2, we propose that:

.

# Reference

[1] R2-2304521 Report of [AT121bis-e][604][eMBS] UP issues for Multicast in RRC Inactive (Apple)

[2] R2-2309425	  Reply LS on multicast reception in RRC_INACTIVE (R1-2308612; contact: Apple)	RAN1	LS in	Rel-18	NR_MBS_enh-Core	To:RAN2