3GPP TSG-RAN WG2 Meeting #124	R2-231xxxx

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item: 	7.11.2.2

Source:	Apple

Title:	MAC open issues for eMBS (Apple)

Document for:		Discussion and Decision

# Introduction

This document captures the outcome of the MAC open issues discussion in the following email discussion:

- [Post123bis][612][eMBS] 38.321 CR update and open issues (Apple)

Scope: Running CR update and open issues

Intended outcome:

- Endorsed running CR

- List of open issues for TS 38.321 (separate document)

Deadline: Long

Please provide your comments before October 27th 0100 UTC.

# Contact Points

Rapporteur encourages the participating delegates to provide their contact information in this table.

| Company | Name | Email Address |
| --- | --- | --- |
| Lenovo | Mingzeng Dai | daimz4@lenovo.com |
| Huawei, HiSilicon | Xubin | xubin10@huawei.com |
| CATT | Zhou Rui | zhourui@catt.cn |
| vivo | Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Ericsson | Henrik Enbuske | Henrik.enbuske@ericsson.com |
| LGE | Seong Kim | sj117.kim@lge.com |
| Nokia | Subin Narayanan | Subin.narayanan@nokia.com |
| Qualcomm | Umesh Phuyal | uphuyal@qti.qualcomm.com |
| Samsung | Sangkyu Baek | sangkyu.baek@samsung.com |

# Discussion

### Open issue 1: Whether DRX Command MAC CE is applicable for inactive multicast DRX operation?

In R17 multicast DRX operation in RRC_CONNECTED state, network can use DRX Command MAC CE to request a specific MBS multicast session to leave DRX active time, and network will stop the scheduling data for this MBS multicast session.

For R18 inactive multicast DRX operation, since the R17 multicast DRX operation is reused for it and the purpose of DRX command MAC CE is still applicable for the inactive multicast DRX operation, network can also send the DRX Command MAC CE to inform RRC_INACTIVE UE out of DRX active time for a specific multicast session, which is good for UE power.

Proposal 1: DRX Command MAC CE is applicable for inactive multicast DRX operation.

With the proposal 3, we can remove the Editor Note 1. TP is as below.

5.7b Discontinuous Reception (DRX) for MBS Multicast …… 1> if a DRX Command MAC CE indicated by PDCCH addressed to a G-RNTI or G-CS-RNTI, or by a configured downlink multicast assignment is received: 2> stop drx-onDurationTimerPTM of the DRX for this G-RNTI or G-CS-RNTI; 2> stop drx-InactivityTimerPTM of the DRX for this G-RNTI or G-CS-RNTI.

##### Question 1: Do you agree with proposal 1 and the TP?

| Company | Proposal 1? | TP? | Comments |
| --- | --- | --- | --- |
| Lenovo | Yes | Yes |  |
| Huawei, HiSilicon | Yes | Yes |  |
| CATT | Yes | Yes |  |
| vivo | Yes | Yes | The existing beahvior can be reused without limitation. |
| Ericsson | Yes | Yes |  |
| LGE | Yes | Yes |  |
| Nokia | Yes | Yes |  |
| Qualcomm | See comment | Needs update | The condition 1> says G-RNTI, G-CS-RNTI or SPS, but RAN2 agreed SPS is not supported for multicast reception in RRC_INACTIVE. So, simply deleting Ed Note does not seem enough. |
| Samsung | Yes | Update | Agree with QC. SPS-related texts should be taken out |

[Summary]

All companies support the proposal 1 and the TP. And one company commented that we may need to further clarify that SPS is not supported for multicast reception in RRC_INACTIVE somewhere. Since there is no G-CS-RNTI configuration in PTM configuration used in RRC_INACTIVE, and the clarification has been captured in section 16.10.5.4 of 38.300 running CR for eMBS (as below). Rapporteur assume current description in MAC spec should be fine.

“SPS is not supported for MBS multicast session data reception for UEs in RRC_INACTIVE state.”

Proposal 1: DRX Command MAC CE is applicable for inactive multicast DRX operation.

Proposal 1a: Remove EN1 in section 5.7b of running MAC CR for eMBS.

### Open issue 2: FFS on the value of RNTI for multicast MCCH

In previous RAN2 discussion [1], majority view was to introduce the fix value for the multicast MCCH RNTI, and some companies thought multicast MCCH may be scheduled by G-RNTI at that time.

Rapp summary in [AT121bis-e][604] [1] On proposal a (i.e. fix value for multicast MCCH RNTI), 2 companies doesnot agree with it. On proposal b (i.e. exclude using G-RNTI for MCCH scheduling), 2 companies think we should consider to use G-RNTI for MCCH scheduling; but other companies cannot understand how it can work. Considering the company’s concerns, rapporteur suggests not to agree these proposals, and invites companies to provide the contribution to next meeting.

Since later RAN2 agreed to introduce multicast MCCH RNTI for the multicast MCCH scheduling, we think it should be agreeable to introduce a new fix value for the multicast MCCH RNTI.

Proposal 2: Introduce a new fix RNTI value for multicast-MCCH-RNTI.

Following is the corresponding TP:

Table 7.1-1: RNTI values. Value (hexa-decimal); RNTI. 0000; N/A. 0001–FFF2; RA-RNTI, MSGB-RNTI, Temporary C-RNTI, C-RNTI, CI-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, PS-RNTI, SL-RNTI, SLCS-RNTI SL Semi-Persistent Scheduling V-RNTI, AI-RNTI, G-RNTI, G-CS-RNTI, and CG-SDT-CS-RNTI. FFF3–FFFA; Reserved. FFFB; Multicast-MCCH-RNTI. FFFC; PEI-RNTI. FFFD; MCCH-RNTI. FFFE; P-RNTI. FFFF; SI-RNTI. Editor Note: FFS on the value of the multicast-MCCH-RNTI.

##### Question 2: Do you agree with proposal 2 and the TP?

| Company | Proposal 2? | TP? | Comments |
| --- | --- | --- | --- |
| Lenovo | Yes | Yes |  |
| Huawei, HiSilicon | See comments | See comments | We think it is more flexible to make the Multicast-MCCH-RNTI configurable. In case multicast is not provided for RRC_INACTIVE in a cell (also we agreed in some case MCCH can be absent), it doesn’t makes much sense if the value is still occupied. |
| CATT | Yes | Yes |  |
| vivo | Yes | Yes with comments | Making Multicast-MCCH-RNTI configurable seems too complicated. When a UE reselects to another cell, it has to read SIB1 for the configured RNTI, making the TBS of SIB1 unnecessarily larger. The benefit is unknown considering the space of RNTI is big enough. Using fixed value is simpler. Additionally, we suggest naming it as Multicast MCCH-RNTI (“-” is not needed between Multicast and MCCH). |
| Ericsson | Yes | Yes | No strong opinion, fixed value is straightforward and simple though. |
| LGE | Yes | Yes |  |
| Nokia | Yes | Yes |  |
| Qualcomm | Yes | Yes | Agree with vivo’s comment to remove hyphen. Also agree with Ericsson and vivo on configurability. |
| Samsung | Yes | Yes | Agree with vivo |

[Summary]

For the proposal 2 (i.e. fix value), All companies except one support it. And one company would like to make it configurable. As majority view is in favor of the simple fixed value approach, Rapporteur suggests to agree proposal 2.

For the TP, two companies propose to change the term to “multicast MCCH-RNTI”. Since the term "multicast-MCCH-RNTI" is used in other running CRs for eMBS, rapporteur suggests keeping the term consistent here.

Proposal 2: Introduce a new fix RNTI value for multicast-MCCH-RNTI.

Proposal 2a: Agree the following TP of Table 7.1-1 (RNTI values) in MAC running CR.

Table 7.1-1: RNTI values. Value (hexa-decimal); RNTI. 0000; N/A. 0001–FFF2; RA-RNTI, MSGB-RNTI, Temporary C-RNTI, C-RNTI, CI-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, PS-RNTI, SL-RNTI, SLCS-RNTI SL Semi-Persistent Scheduling V-RNTI, AI-RNTI, G-RNTI, G-CS-RNTI, and CG-SDT-CS-RNTI. FFF3–FFFA; Reserved. FFFB; Multicast-MCCH-RNTI. FFFC; PEI-RNTI. FFFD; MCCH-RNTI. FFFE; P-RNTI. FFFF; SI-RNTI. Editor Note: FFS on the value of the multicast-MCCH-RNTI.

### Open issue 3: FFS on the value of the LCID for multicast MCCH

In previous RAN2 discussion [1], majority view was to introduce the new fix LCID value for the multicast MCCH, and some companies thought it’s unnecessary to introduce the new LCID for MCCH if RNTI value can be used to distinguish the multicast MCCH and broadcast MCCH.

Rapp summary in [AT121bis-e][604] [1] Proposal 10 In R2-2303420 (for agreement, 15/17): Introduce a new LCID in Table 6.2.1-1 for multicast MCCH. According to company’s input (including comments part) - On P10, 4 companies donot support it and they think the new LCID may not needed.

With the new RNTI value introduced for multicast-MCCH-RNTI, since it can be used to distinguish multicast MCCH and broadcast MCCH, it seems no issue to use the same LCID for both multicast MCCH and broadcast MCCH.

Proposal 3: The same LCID value is used for multicast MCCH and broadcast MCCH.

Following is the corresponding TP:

Table 6.2.1-1c Values of LCID for MBS broadcast and multicast on DL-SCH Codepoint/Index; LCID values. 0; Broadcast MCCH and multicast MCCH. 1–32; Identity of the logical channel of broadcast MTCH. 33–63; Reserved.

##### Question 3: Do you agree with proposal 3 and the TP?

| Company | Proposal 3? | TP? | Comments |
| --- | --- | --- | --- |
| Lenovo | Yes | Yes | No strong requirements to introduce a separate LCID due to RNTI is different with broadcast. |
| Huawei, HiSilicon | Yes | See comments | Maybe “Broadcast MCCH or multicast MCCH”? Using “and” may cause the confusion that broadcast and multicast share the same logical channel. |
| CATT | Yes | Yes |  |
| vivo | Yes | Yes |  |
| Ericsson | Yes | Yes, comment | Agree w HW that using or seems to be a better choice. |
| LGE | Yes | Yes, comment | Agree with Huawei. |
| Nokia | Please see the comments |  | The TP is not ok. This table does not apply to multicast MTCH and thus the table header is not correct. To my understanding LCID for multicast MCCH should be provided in Table 6.2.1-1 or table header for Table 6.2.1-1c should be changed to indicate it is for multicast MCCH. |
| Samsung | Yes | Yes |  |

[Summary]

All companies support proposal 3. And for the TP, the comments on the wording and the title are acceptable and adopted.

Proposal 3: The same LCID value is used for multicast MCCH and broadcast MCCH.

Proposal 3a: Agree the following TP of Table 6.2.1-1c in MAC running CR.

Table 6.2.1-1c Values of LCID for MBS broadcast and multicast MCCH on DL-SCH Codepoint/Index; LCID values. 0; Broadcast MCCH or multicast MCCH. 1–32; Identity of the logical channel of broadcast MTCH. 33–63; Reserved.

### Open issue 4: About the Editor Note in section 5.3.1 (DL Assignment reception)

Since RAN1 has confirmed in [2] that it is feasible to reuse the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE as the multicast MTCH in RRC_CONNECTED, then we assume Editor Note 1 in section 5.3.1 can be removed.

Editor Note 1: whether to restrict the multicast MTCH in this section in RRC_CONNECTED state is FFS.

Proposal 4: Remove the Editor Note 1 in section 5.3.1.

### 5.3.1 DL Assignment reception

##### Question 4: Do you agree with proposal 4 and the TP?

| Company | Proposal 4? | TP? | Comments |
| --- | --- | --- | --- |
| Lenovo | Yes | Yes |  |
| Huawei, HiSilicon | Yes | Yes |  |
| CATT | Yes | Yes |  |
| vivo | Yes | Yes |  |
| Ericsson | Yes | Yes |  |
| Nokia | Yes | Yes |  |
| Qualcomm | No | No | RAN2 agreed SPS is not supported for multicast reception in RRC_INACTIVE. This section includes SPS aspects also. |
| Samsung | Yes | Yes | For SPS, we think other sections capture it. |

[Summary]

All companies except one support the proposal 4 and the TP. And one company commented that we may need to further clarify that SPS is not supported for multicast reception in RRC_INACTIVE. Since there is no G-CS-RNTI configuration in PTM configuration used in RRC_INACTIVE, and the clarification has been captured in section 16.10.5.4 of 38.300 running CR for eMBS (as below). Rapporteur assumes current description without the EN1 in this section of MAC spec should be fine.

“SPS is not supported for MBS multicast session data reception for UEs in RRC_INACTIVE state.”

Based the further discussion on RAN2 reflector, companies are fine to reflect the restrictionin section 5.8.1a.

Proposal 4: Remove the Editor Note 1 in section 5.3.1 of MAC running CR.

Proposal 4a: Agree to capture the following TP of section 5.8.1a in MAC running CR.

- 5.8.1a Downlink for Multicast To be used in RRC_CONNECTED, MBS Semi-Persistent Scheduling (SPS) is configured by RRC on one Serving Cell per BWP. Multiple assignments can be active simultaneously in the same BWP. For the DL MBS SPS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating SPS activation or deactivation.

### Other open issues?

##### Question 5: Please provide MAC open issues which is not listed above.

| Company | Open issues? |
| --- | --- |
| Qualcomm | May be not a ‘MAC’ issue as such, but since this was discussed as part of email discussions handled by the MAC CR rapporteur, the following is still open FFS and need to be concluded: Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise). We have previously provided a Tdoc R2-2310476 where we explained that iff multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, the CFR cases agreed to be supported already ensure that both the CFRs fully contain CORESET#0. And if multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, to enable UE to monitor a single (the larger) CFR and receive both services without BWP switch, one CFR needs to be completely contained within the other. Consequently we proposed: Proposal 1. When Multicast CFR for RRC_INACTIVE and broadcast CFR are configured differently, one of the two CFRs is fully contained (or overlapping) with the other CFR. Proposal 2. If multicast CFR for RRC_INACTIVE is not configured, the default is same as CORESET#0. [Rapp] This issue is discussed under the email discussion#610, and it is listed as the open issue #6. [Post123bis][610][eMBS] 38.300 CR update and open issues (CMCC) ### Open issue 6: Whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise). For multicast CFR in RRC_INACTIVE design, RAN2 agreed that Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. Since in Rel-17, the simultaneous reception of multicast and broadcast is also supported, and no there’s no restriction to the CFR configuration of multicast and broadcast, therefore, we think the same principle can be reused. Besides, even one CFR is completely contained the other one, the configuration of the two CFR may be different, the gain for the UE is not clear, therefore, we prefer to remove the editor’s note without capturing such restrictions. Proposal 6: Remove the Editor’s note for CFR in section 16.10.5.7. |

# Conclusion

The proposals captured are the following:

< Open issue 1> Whether DRX Command MAC CE is applicable for inactive multicast DRX operation?

Proposal 1: DRX Command MAC CE is applicable for inactive multicast DRX operation.

Proposal 1a: Remove EN1 in section 5.7b of running MAC CR for eMBS.

<Open issue 2> FFS on the value of RNTI for multicast MCCH

Proposal 2: Introduce a new fix RNTI value for multicast-MCCH-RNTI.

Proposal 2a: Agree the following TP of Table 7.1-1 (RNTI values) in MAC running CR.

Table 7.1-1: RNTI values. Value (hexa-decimal); RNTI. 0000; N/A. 0001–FFF2; RA-RNTI, MSGB-RNTI, Temporary C-RNTI, C-RNTI, CI-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, PS-RNTI, SL-RNTI, SLCS-RNTI SL Semi-Persistent Scheduling V-RNTI, AI-RNTI, G-RNTI, G-CS-RNTI, and CG-SDT-CS-RNTI. FFF3–FFFA; Reserved. FFFB; Multicast-MCCH-RNTI. FFFC; PEI-RNTI. FFFD; MCCH-RNTI. FFFE; P-RNTI. FFFF; SI-RNTI. Editor Note: FFS on the value of the multicast-MCCH-RNTI.

<Open issue 3> FFS on the value of the LCID for multicast MCCH

Proposal 3: The same LCID value is used for multicast MCCH and broadcast MCCH.

Proposal 3a: Agree the following TP of Table 6.2.1-1c in MAC running CR.

Table 6.2.1-1c Values of LCID for MBS broadcast and multicast MCCH on DL-SCH Codepoint/Index; LCID values. 0; Broadcast MCCH or multicast MCCH. 1–32; Identity of the logical channel of broadcast MTCH. 33–63; Reserved.

<Open issue 4> About the Editor Note in section 5.3.1 (DL Assignment reception)

Proposal 4: Remove the Editor Note 1 in section 5.3.1 of MAC running CR.

Proposal 4a: Agree to capture the following TP of section 5.8.1a in MAC running CR.

- 5.8.1a Downlink for Multicast To be used in RRC_CONNECTED, MBS Semi-Persistent Scheduling (SPS) is configured by RRC on one Serving Cell per BWP. Multiple assignments can be active simultaneously in the same BWP. For the DL MBS SPS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating SPS activation or deactivation.

# Reference

[1] R2-2304521 Report of [AT121bis-e][604][eMBS] UP issues for Multicast in RRC Inactive (Apple)

[2] R2-2309425	  Reply LS on multicast reception in RRC_INACTIVE (R1-2308612; contact: Apple)	RAN1	LS in	Rel-18	NR_MBS_enh-Core	To:RAN2