3GPP TSG-RAN WG2 Meeting #124							   R2-231xxxx

Chicago, USA, 13 – 17 November, 2023

Agenda Item:	7.13.1

Source:		CATT

Title:			[Post123bis][658][R18 SON/MDT] Running UE capabilities CR of SON/MDT (CATT)

Document for:	Information

## 1	Introduction

This is to collect the open issues of UE capabilities that need to be addressed to complete the Rel-18 NR SON/MDT WI by November.

- [Post123bis][658][R18 SON/MDT] Running UE capabilities CR of SON/MDT (CATT)

Scope: Constructing basic CR and continue running

Output: running CR for technical endorse

Deadline: Two weeks

Please provide your feedback before 12:00 UTC, Friday October 27th.

Table 0: Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CATT | Haocheng Wang | wanghaocheng@catt.cn |

## 2 Open issue list for UE capabilities

Table 1: Open issue list for UE capabilities

| Number and use case | Open issues | Involved agreements |
| --- | --- | --- |
| (1) CPAC | Whether and how to define a MRO CPAC related UE capability? | RAN2#123: 3 For CPAC MRO, UE logs the below information in SCGFailureInformation: the type of the first triggered CPAC event if multiple events are configured the time duration between the two triggered CPAC events if multiple events are configured |
| (2) NPN | What is the usage of logged MDT related NPN capability? a) Inclusion of NPN ID in logged MDT procedures, upon request from the network (wording in agreement); b) SNPN checking (for SNPN only); c) To assist the area scope configuration (for both SNPN and PNI-NPN). | RAN2#123bis[1]: 2 Include the 3 cases of cell based/TAI based/SNPN list based SNPN related area scopes in the logged MDT configuration and a critical extension (i.e. AreaConfiguration-r18) can be considered in R18. FFS how to optimize the signalling structure to avoid much overhead. 5: Introduce A new UE capability bit (optional with signalling) for NPN in logged MDT. This bit indicates whether the UE supports the inclusion of NPN ID in logged MDT procedures, upon request from the network. |
|  | Whether to set separate SNPN and PNI-NPN capabilities for logged MDT? | RAN2#119bis: 1 SNPN ID (e.g.,NID ID) checking is needed before sending the availability indication for corresponding SON and MDT report. The details can be discussed case by case. FFS PNI-NPN ID checking. 2 Include the NPN ID into SON/MDT report, whether SNPN ID or PNI-NPN ID related info should be included can be discussed per use case. RAN2#120: 2. PNI-NPN (CAG) ID checking is NOT performed before sending the logged MDT availability indication related to a PNI-NPN network. 3. Details of the checking of NPN IDs (e.g., Proposal 1 of R2-2211354) are FFS. RAN2#122: - Include CAG ID(s) in the logged MDT area configuration. RAN2#123: 1. Include SNPN ID (list) in the logged MDT area configuration following RAN3 agreement to align with the future NPN evolution. 2. No new UE variables will be introduced for PNI-NPNs. 4. Assuming ESNPN is supported, include a list of SNPN IDs in the logged MDT report. |
|  | Whether to restrict the UE capability of NPN for RLF report to SNPN only? | RAN2#120: 1. PNI-NPN (CAG) ID checking is NOT performed before sending the RLF/HOF report availability indication related to a PNI-NPN network. 4 Introduce SPNP ID (e.g., NID) to RLF/HOF report. Details of how to introduce it are FFS. RAN2#123: 3. UE performs SNPN ID checking before transmitting the information for corresponding SON and MDT reports, upon the network requests for it. |
|  | Whether and how to define the UE capabilities for CEF/RA report/MHI/OOC for NPN. | RAN2#123bis: => Consider MHI, CEF and RA report enhancements for NPN networks in Rel-18. Similar conclusions should be reached rapidly and repetitive discussions should be avoided. => Consider to introduce enhancements for OOC analysis involving NPN network. |
| (3) Other? |  |  |

### 2.1	MRO CPAC

For issue (1) of MRO CPAC, it is agreed that the UE logs the below information in SCGFailureInformation:

the type of the first triggered CPAC event if multiple events are configured

the time duration between the two triggered CPAC events if multiple events are configured

Since this is only impact on the SCGFailureInformation message, Rapp to introduce an optional UE capability without signalling for MRO CPAC.

Q1: For MRO CPAC, do companies agree to define an optional UE capability without signalling to indicate if the UE supports the delivery of the CPAC related parameters for MRO in SCGFailureInformation message?

| Company name | Y/N | Comments (if any) |
| --- | --- | --- |
| CATT | Yes | There is only additional information in the SCGFailureInformation message for MRO CPAC, so Optional UE capability without signalling is appropriate. |

Rapp’s summary:

Proposal:

### 2.2	SON/MDT enhancement on NPN

For issue (2) of SON/MDT enhancement on NPN, it is agreed to introduce a new UE capability bit (optional with signalling) for NPN in logged MDT. This bit indicates whether the UE supports “the inclusion of NPN ID in logged MDT procedures, upon request from the network”.

| loggedMDT-NPN-r18 Indicates whether the UE supports the inclusion of NPN ID in logged MDT procedures, upon request from the network. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Rapp thinks 2 issues should be further discussed for this capability:

a) What is the usage of logged MDT related NPN capability?

- Wording in the agreement: Inclusion of NPN ID in logged MDT procedures, upon request from the network;

- For SNPN ID checking;

- To assist the area scope configuration in the logged MDT configuration for SNPN and PNI-NPN.

b) Whether to set separate SNPN and PNI-NPN capabilities for logged MDT?

For a), it is FFS in last meeting whether the NPN ID should be reported to the NW in the logged MDT report. And the NPN ID can be used for NW checking before (for available) and after (for report) the request from the network. So here the wording should not be “UE supports the inclusion of NPN ID in logged MDT procedures, upon request from the network” which is ambiguous and inaccurate.

And besides the inclusion of NPN ID in logged MDT procedures (in UE variable) for NPN ID checking, Rapp thinks the network should send the logged MDT configuration based on this capability. And together with b), since we have agreed that “Include CAG ID(s) in the logged MDT area configuration.” and “Include the 3 cases of cell based/TAI based/SNPN list based SNPN related area scopes in the logged MDT configuration”, whether it is allowed to configure the SNPN specific or PNI-NPN related area scope should be based on the UE support of SNPN and PNI-NPN for logged MDT capability respectively.

So based on the reasons listed above, Rapp suggests changing the wording and dividing this capability bit into two. Please check if the following change is acceptable:

| loggedMDT-PNI-NPN-r18 Indicates whether the UE supports Logged MDT for PNI-NPN(s). | UE | No | No | No |
| --- | --- | --- | --- | --- |
| loggedMDT-SNPN-r18 Indicates whether the UE supports Logged MDT for SNPN(s). | UE | No | No | No |

Q2: For SON/MDT enhancement on NPN, do companies agree to:

- Change the wording of the UE capabilities for NPN in logged MDT as above?

- Introduce separate capability bits for SNPN and PNI-NPN?

| Company name | Y/N | Comments (if any) |
| --- | --- | --- |
| CATT | Yes for both a) and b) | - The wording should be improved since the checking is performed before and after the request from the network. And the capability is also necessary for the NPN specific area scope configuration. We can simply indicate that whether the UE supports Logged MDT for PNI-NPN(s)/SNPN(s). - Separate capabilities should be defined. |

Rapp’s summary:

Proposal:

For issue (2) of SON/MDT enhancement on NPN, it is agreed to use the SNPN ID for checking and introduce SNPN ID (e.g., NID) into RLF/HOF report. PNI-NPN (CAG) ID checking is NOT performed, and CAG ID will not be included in RLF/HOF report either. Therefore the procedure of RLF/HOF report for PNI-NPN is totally same as the normal PLMN.

Rapp thinks this capability can be restricted to SNPN network only. Please check if the following change is acceptable:

RLF Report for NPN It is optional for UE to support the checking and the delivery of the SNPN ID in the RLF-report.

Q3: For SON/MDT enhancement on NPN, do companies agree to restrict the UE capability of NPN for RLF report to SNPN only?

| Company name | Y/N | Comments (if any) |
| --- | --- | --- |
| CATT | Yes | It is agreed that the CAG related information will not be used for checking or for reporting. This capability can be restricted to SNPN network only. |

Rapp’s summary:

Proposal:

Also for issue (2) of SON/MDT enhancement on NPN, it is agreed that:

=>	Consider MHI, CEF and RA report enhancements for NPN networks in Rel-18. Similar conclusions should be reached rapidly and repetitive discussions should be avoided.

=>	Consider to introduce enhancements for OOC analysis involving NPN network.

Since for use cases of CEF/RA report/MHI/OOC have no further agreements, Rapp thinks the related capabilities can wait to be added until there are conclusions.

Rapp’s summary:

Proposal:

### 2.3	Others

Q4: Do companies have any further open issue? If so, please provide feedback.

| Company | Comments |
| --- | --- |

Rapp’s summary:

Proposal:

## 3	Conclusions

[To be added]

## 4	Reference

[1]	R2-2311276 Report from SON/MDT session