| Company | Clause | Comment | Rapp Response |
| --- | --- | --- | --- |
| Xiaomi | 5.8.9.1.2 | 1> set the entry included in the sl-CarrierToAddModList, : 2> set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB; Bullet 2 is RB modification not carrier modification |  |
| Xiaomi | 5.8.9.1a.6.1 | 1> for groupcast and broadcast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerToAddModListSizeExt in sl-ConfigDedicatedNR for a sl-ServedRadioBearer, and if the SL-TxProfile of at least one associated QoS flow for the sl-ServedRadioBearer indicates backwardsCompatible and UE decides to use PDCP duplication; or According to the agreement, this case only applies to RRC_IDLE/INACTIVE. If at least one QoS flow having Tx profile with value set to backwards compatible is mapped to the radio bearer, legacy carrier is used for transmission for this radio bearer, for RRC_IDLE/RRC_INACTIVE/OOC case. For RRC_CONNECTED, NW should ensure all the Qos flows for the same RB indicates the same Tx profile. Also “use the legacy carrier” is not reflected. |  |
| Xiaomi | 5.8.9.1b.1 | Suggest to have separate section for release condition and release operation. Regarding release condition; 1> for unicast, if the sidelink carrier release was triggered due to the reception of the RRCReconfigurationSidelink message; or - for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR ,upper layer or due to change of UE capabilities of either UE; or 1> for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the maximum number of consecutive HARQ DTX for peer UE has been reached For operation： 2> for each sl-Carrier-Id value included in the sl-CarrierToReleaseList: 3> if the current UE configuration includes a sidelink carrier with value sl-Carrier-Id: 4> release the sidelink carrier for reception or transmission |  |
| Xiaomi | 5.8.9.1b.2 | Similar as above. Condition for add/modify - for unicast, if the sidelink carrier release was triggered due to the reception of the RRCReconfigurationSidelink message; or - for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR ,upper layer or due to change of UE capabilities of either UE; Regarding add/modify operation: 1> for unicast, if the sidelink carrier addition was triggered due to the reception of the RRCReconfigurationSidelink message; 2> for each sl-Carrier-Id value included in the sl-CarrierToAddModList that is not part of the current UE configuration (sidelink carrier addition): 3> add the sidelink carrier, corresponding to the sl-Carrier-Id, in accordance with the sl-AbsoluteFrequencyPointA for reception; - for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR ,upper layer or due to change of UE capabilities of either UE; - add the sidelink carrier taking into account at least carrier(s) mapped to the sidelink QoS flow(s) configured by the upper layer, carriers configured in sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR, and carrier(s) supported by both UEs |  |
| Xiaomi | 6.3.5 | sl-FreqInfoToAddModListExt-v18xy SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigExt-v18xy should be maxNrofFreqSL-1-r18? |  |
| Xiaomi | 6.3.5 | SL-SCCH-CarrierSetConfig-r18 ::= SEQUENCE { sl-destinationList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16, allowedCarrierFreqSet1-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16), allowedCarrierFreqSet2-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16) } Why the CC set is configured per DST? Should be per LCH configuration |  |
| Xiaomi | 6.3.5 | ue-toUE-COT-SharingED-Threshold-r18 INTEGER (-85..-52) should be To |  |
| Xiaomi | 6.3.5 | sl-threshCBR-FreqKeeping-r15 should be r18 |  |
| Xiaomi | 6.6.2 | RRCReconfigurationSidelink-v18xy-IEs ::= SEQUENCE { sl-CarrierToAddModList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierConfig-r18 OPTIONAL, -- Need N sl-CarrierToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierId-r18 OPTIONAL, -- Need N sl-RLC-BearerToAddModList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfig-r18 OPTIONAL, -- Need N sl-RLC-BearerToReleaseList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfigIndex-r18 OPTIONAL -- Need N } SL-RLC-BearerConfig-r18 ::= CHOICE { srb SEQUENCE { sl-SRB-IdentityWithDuplication INTEGER (1..3), sL-RLC-BearerConfigIndex-r16 SL-RLC-BearerConfigIndex-r18, ... }, drb SEQUENCE { slrb-PC5-ConfigIndex-r18 SLRB-PC5-ConfigIndex-r16, sL-RLC-BearerConfigIndex-r18 SL-RLC-BearerConfigIndex-r18, sl-RLC-ConfigPC5-r18 SL-RLC-ConfigPC5-r16 OPTIONAL, -- Need M sl-MAC-LogicalChannelConfigPC5-r18 SL-LogicalChannelConfigPC5-r16 OPTIONAL, -- Need M ... Should be v18xy? |  |
| Xiaomi | 6.6.2 | drb SEQUENCE { slrb-PC5-ConfigIndex-r18 SLRB-PC5-ConfigIndex-r16, sL-RLC-BearerConfigIndex-r18 SL-RLC-BearerConfigIndex-r18, sl-RLC-ConfigPC5-r18 SL-RLC-ConfigPC5-r16 OPTIONAL, -- Need M sl-MAC-LogicalChannelConfigPC5-r18 SL-LogicalChannelConfigPC5-r16 OPTIONAL, -- Need M ... } According to 331, the value ragne of additional LCID is 1-32 while according to running MAC spec, the LCID for addditoinal bearer is 21-36, which is not aligned. |  |
|  | 6.6.2 | SL-CarrierId-r18 ::= INTEGER (0..maxNrofFreqSL-1-r18) Should be 1.. maxNrofFreqSL-1-r18? |  |
| ZTE |  | WID should be “NR_SL_enh2-Core” |  |
| ZTE | 6.2.2 | What is “aligned with sl-QoS-flowID in SL QoS info”? sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, aligned with the sl-QoS-FlowIdentity in SL-QoS-Info. |  |
| ZTE | 6.2.2 | FD of following IE is missing SL-CarrierFailure-r18 ::= SEQUENCE { sl-DestinationIdentity-r18 SL-DestinationIdentity-r16, sl-CarrierFailure-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16) } |  |
| ZTE | 6.3.1 | Why does not SIB12 include “SL-SCCH-CarrierSetConfig”? |  |
| ZTE | 6.3.2 | The FD of “sl-frequency” is missing. The value of this IE is integer, so is it ID of frequency within SL frequency list configured to UE? MeasObjectNR-SL-v18xy ::= SEQUENCE { sl-Frequency INTEGER (1..maxNrofFreqSL-r16), tx-PoolMeasToRemoveList-r16 Tx-PoolMeasList-r16 OPTIONAL, -- Need N tx-PoolMeasToAddModList-r16 Tx-PoolMeasList-r16 OPTIONAL -- Need N } |  |
| ZTE | 6.3.5 | 1. we think the intention of network configure SCCH allowed carrier is to mimic DRB case, i.e. per logical channel, seems current signaling design is per logical channel type. Suggest to add “SRB ID” within SCCH allowed carrier list 2. following description is not enough for two allowed SRB carrier list. Current description does not clarify why are two allowed carrier lists configured. Seems current wording mimic LTE V2X’s signaling design. In LTE V2X, we also have following description in LTE MAC spec. We think following description is also necessary for NR V2X. If duplication is activated as specified in TS 36.323 [4], the MAC entity shall map different sidelink logical channels which correspond to the same PDCP entity onto different carriers in accordance with clause 5.14.1.5, or onto different carriers of different carrier sets (if configured in allowedCarrierFreqList for the corresponding destination). For a given sidelink logical channel, it is up to UE implementation which carrier set to select among the carrier sets configured in allowedCarrierFreqList (if configured) for the corresponding destination. Suggest to adopt following description allowedCarrierFreqSet1, allowedCarrierFreqSet2 Indicates the set of carrier frequencies applicable for the transmission of the MAC SDUs from the sidelink SRB logical channels whose associated destination is included in sl-destinationList. If present, network ensures allowedCarrierFreqSet1 and allowedCarrierFreqSet2 do not include the same carrier frequency. For a given sidelink SRB logical channel, it is up to UE implementation which carrier set to select among the carrier sets. If duplication is activated as specified in TS 38.323 [*], the MAC entity shall map different sidelink logical channels of SL RLC bearer which correspond to the same PDCP entity onto different carrier sets. 3. the meaning of allowedCarrierFreqSet1 is not clear. The value of entry in this list is an integer, is it ID of frequency within SL frequency list configured to UE. |  |
| ZTE | 6.3.5 | Is it appropriate to use such long IE name in ASN.1? harq-ACKFeedbackRatioforContentionWindowAdjustmentGC-Option2-r18 INTEGER (10..100) OPTIONAL, -- Need M |  |
| ZTE | 6.3.5 | Following agreement is missing. 2: For SL-DRB the CAPC value is (pre)configurable per-DRB as in NR-U. |  |
| ZTE | 6.3.5 | Following FD should be removed, since corresponding IE is not present. sl-AbsoluteFrequencySSB Indicates the frequency location of sidelink SSB. The transmission bandwidth for sidelink SSB is within the bandwidth of this sidelink BWP. |  |
| ZTE | 9.2 | The new added “MAC configuration” for duplication shares same name with legacy MAC configuration for SRB. Suggested to use “MAC configuration associated to additional RLC configuration” |  |
| ZTE | 9.x | Do we need to move R17 tx profile to the new added clause 9.X? |  |