| Company | Clause | Comment | Rapp Response |
| --- | --- | --- | --- |
| Xiaomi | 5.8.9.1.2 | 1> set the entry included in the sl-CarrierToAddModList, : 2> set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB; Bullet 2 is RB modification not carrier modification | Thanks for catching it, will be corrected in the next iteration |
| Xiaomi | 5.8.9.1a.6.1 | 1> for groupcast and broadcast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerToAddModListSizeExt in sl-ConfigDedicatedNR for a sl-ServedRadioBearer, and if the SL-TxProfile of at least one associated QoS flow for the sl-ServedRadioBearer indicates backwardsCompatible and UE decides to use PDCP duplication; or According to the agreement, this case only applies to RRC_IDLE/INACTIVE. If at least one QoS flow having Tx profile with value set to backwards compatible is mapped to the radio bearer, legacy carrier is used for transmission for this radio bearer, for RRC_IDLE/RRC_INACTIVE/OOC case. For RRC_CONNECTED, NW should ensure all the Qos flows for the same RB indicates the same Tx profile. Also “use the legacy carrier” is not reflected. | For the 1st issue, although I got your point that there might be no case of mixed Tx profile for CONNECTED case, but still UE has to diff between backwards-(in)compatible cases, and then the only delta part is about “at least”? that seems a bit difficult to further differentiate. Let’s hear more view before change. For the 2nd issue, it will be corrected in the next iteration |
| Xiaomi | 5.8.9.1b.1 | Suggest to have separate section for release condition and release operation. Regarding release condition; 1> for unicast, if the sidelink carrier release was triggered due to the reception of the RRCReconfigurationSidelink message; or - for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR ,upper layer or due to change of UE capabilities of either UE; or 1> for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the maximum number of consecutive HARQ DTX for peer UE has been reached For operation： 2> for each sl-Carrier-Id value included in the sl-CarrierToReleaseList: 3> if the current UE configuration includes a sidelink carrier with value sl-Carrier-Id: 4> release the sidelink carrier for reception or transmission | The reason I use the current style (instead of separate condition/action as u pointed out) is that I feel it is a bit hard to outline each condition for add/mod/release (e.g., I am not super sure about the yellow part) separately. But we are open, let’s see if any similar comment, before change. |
| Xiaomi | 5.8.9.1b.2 | Similar as above. Condition for add/modify - for unicast, if the sidelink carrier release was triggered due to the reception of the RRCReconfigurationSidelink message; or - for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR ,upper layer or due to change of UE capabilities of either UE; Regarding add/modify operation: 1> for unicast, if the sidelink carrier addition was triggered due to the reception of the RRCReconfigurationSidelink message; 2> for each sl-Carrier-Id value included in the sl-CarrierToAddModList that is not part of the current UE configuration (sidelink carrier addition): 3> add the sidelink carrier, corresponding to the sl-Carrier-Id, in accordance with the sl-AbsoluteFrequencyPointA for reception; - for unicast, after receiving the RRCReconfigurationCompleteSidelink message if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR ,upper layer or due to change of UE capabilities of either UE; - add the sidelink carrier taking into account at least carrier(s) mapped to the sidelink QoS flow(s) configured by the upper layer, carriers configured in sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR, and carrier(s) supported by both UEs | The reason I use the current style (instead of separate condition/action as u pointed out) is that I feel it is a bit hard to outline each condition for add/mod/release (e.g., I am not super sure about the yellow part) separately. But we are open, let’s see if any similar comment, before change. |
| Xiaomi | 6.3.5 | sl-FreqInfoToAddModListExt-v18xy SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigExt-v18xy should be maxNrofFreqSL-1-r18? | The intention was not to define a new constant, but reuse the old one which equals to 8 |
| Xiaomi | 6.3.5 | SL-SCCH-CarrierSetConfig-r18 ::= SEQUENCE { sl-destinationList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16, allowedCarrierFreqSet1-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16), allowedCarrierFreqSet2-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16) } Why the CC set is configured per DST? Should be per LCH configuration | It is to follow the legacy LTE design, and the set1/2 are used for the two LCH respectively, so it is per-LCH as agreed. |
| Xiaomi | 6.3.5 | ue-toUE-COT-SharingED-Threshold-r18 INTEGER (-85..-52) should be To | True, will be corrected |
| Xiaomi | 6.3.5 | sl-threshCBR-FreqKeeping-r15 should be r18 | True, will be corrected |
| Xiaomi | 6.6.2 | RRCReconfigurationSidelink-v18xy-IEs ::= SEQUENCE { sl-CarrierToAddModList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierConfig-r18 OPTIONAL, -- Need N sl-CarrierToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-CarrierId-r18 OPTIONAL, -- Need N sl-RLC-BearerToAddModList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfig-r18 OPTIONAL, -- Need N sl-RLC-BearerToReleaseList-r18 SEQUENCE (SIZE(1..maxNrofSLRB-r16)) OF SL-RLC-BearerConfigIndex-r18 OPTIONAL -- Need N } SL-RLC-BearerConfig-r18 ::= CHOICE { srb SEQUENCE { sl-SRB-IdentityWithDuplication INTEGER (1..3), sL-RLC-BearerConfigIndex-r16 SL-RLC-BearerConfigIndex-r18, ... }, drb SEQUENCE { slrb-PC5-ConfigIndex-r18 SLRB-PC5-ConfigIndex-r16, sL-RLC-BearerConfigIndex-r18 SL-RLC-BearerConfigIndex-r18, sl-RLC-ConfigPC5-r18 SL-RLC-ConfigPC5-r16 OPTIONAL, -- Need M sl-MAC-LogicalChannelConfigPC5-r18 SL-LogicalChannelConfigPC5-r16 OPTIONAL, -- Need M ... Should be v18xy? | In PC5-RRC module, there was no r16 IE defined, so I start from r18 |
| Xiaomi | 6.6.2 | drb SEQUENCE { slrb-PC5-ConfigIndex-r18 SLRB-PC5-ConfigIndex-r16, sL-RLC-BearerConfigIndex-r18 SL-RLC-BearerConfigIndex-r18, sl-RLC-ConfigPC5-r18 SL-RLC-ConfigPC5-r16 OPTIONAL, -- Need M sl-MAC-LogicalChannelConfigPC5-r18 SL-LogicalChannelConfigPC5-r16 OPTIONAL, -- Need M ... } According to 331, the value ragne of additional LCID is 1-32 while according to running MAC spec, the LCID for addditoinal bearer is 21-36, which is not aligned. | True, will be corrected |
|  | 6.6.2 | SL-CarrierId-r18 ::= INTEGER (0..maxNrofFreqSL-1-r18) Should be 1.. maxNrofFreqSL-1-r18? | I thought about this, and not quite sure if we should exclude 0, i.e., the legacy carrier, so having this included for now. but open to hear more view. |
| ZTE |  | WID should be “NR_SL_enh2-Core” | True, to be corrected |
| ZTE | 6.2.2 | What is “aligned with sl-QoS-flowID in SL QoS info”? sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, aligned with the sl-QoS-FlowIdentity in SL-QoS-Info. | Thanks for reminding, seems no need to include the flow-ID, as long as it aligns with the old flow-list. |
| ZTE | 6.2.2 | FD of following IE is missing SL-CarrierFailure-r18 ::= SEQUENCE { sl-DestinationIdentity-r18 SL-DestinationIdentity-r16, sl-CarrierFailure-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16) } | True, will be corrected in the next iteration |
| ZTE | 6.3.1 | Why does not SIB12 include “SL-SCCH-CarrierSetConfig”? | Based on 123bis conclusion 4. For SCCH, at least for RRC_IDLE/RRC_INACTIVE/OOC cases, leave the decision of per-LCH carrier set for PDCP duplication to Tx UE implementation So no need for such configuration in SIB? Sorry if any missing point |
| ZTE | 6.3.2 | The FD of “sl-frequency” is missing. The value of this IE is integer, so is it ID of frequency within SL frequency list configured to UE? MeasObjectNR-SL-v18xy ::= SEQUENCE { sl-Frequency INTEGER (1..maxNrofFreqSL-r16), tx-PoolMeasToRemoveList-r16 Tx-PoolMeasList-r16 OPTIONAL, -- Need N tx-PoolMeasToAddModList-r16 Tx-PoolMeasList-r16 OPTIONAL -- Need N } | True, will correct it in the next iteration (yes to your second question) |
| ZTE | 6.3.5 | 1. we think the intention of network configure SCCH allowed carrier is to mimic DRB case, i.e. per logical channel, seems current signaling design is per logical channel type. Suggest to add “SRB ID” within SCCH allowed carrier list 2. following description is not enough for two allowed SRB carrier list. Current description does not clarify why are two allowed carrier lists configured. Seems current wording mimic LTE V2X’s signaling design. In LTE V2X, we also have following description in LTE MAC spec. We think following description is also necessary for NR V2X. If duplication is activated as specified in TS 36.323 [4], the MAC entity shall map different sidelink logical channels which correspond to the same PDCP entity onto different carriers in accordance with clause 5.14.1.5, or onto different carriers of different carrier sets (if configured in allowedCarrierFreqList for the corresponding destination). For a given sidelink logical channel, it is up to UE implementation which carrier set to select among the carrier sets configured in allowedCarrierFreqList (if configured) for the corresponding destination. Suggest to adopt following description allowedCarrierFreqSet1, allowedCarrierFreqSet2 Indicates the set of carrier frequencies applicable for the transmission of the MAC SDUs from the sidelink SRB logical channels whose associated destination is included in sl-destinationList. If present, network ensures allowedCarrierFreqSet1 and allowedCarrierFreqSet2 do not include the same carrier frequency. For a given sidelink SRB logical channel, it is up to UE implementation which carrier set to select among the carrier sets. If duplication is activated as specified in TS 38.323 [*], the MAC entity shall map different sidelink logical channels of SL RLC bearer which correspond to the same PDCP entity onto different carrier sets. 3. the meaning of allowedCarrierFreqSet1 is not clear. The value of entry in this list is an integer, is it ID of frequency within SL frequency list configured to UE. | True, will correct that in the next iteration. Indeed it was copied from LTE, but if the suggested addition was captured in MAC, why not now for NR also capture in MAC? Sure, definition of the integer are added into FD. |
| ZTE | 6.3.5 | Is it appropriate to use such long IE name in ASN.1? harq-ACKFeedbackRatioforContentionWindowAdjustmentGC-Option2-r18 INTEGER (10..100) OPTIONAL, -- Need M | It is copied from rrc parameter list from R1, but also I got the same feeling.. I will use abbr of CW to shorten it. |
| ZTE | 6.3.5 | Following agreement is missing. 2: For SL-DRB the CAPC value is (pre)configurable per-DRB as in NR-U. | Oh, sorry for missing that, thanks for catching it. |
| ZTE | 6.3.5 | Following FD should be removed, since corresponding IE is not present. sl-AbsoluteFrequencySSB Indicates the frequency location of sidelink SSB. The transmission bandwidth for sidelink SSB is within the bandwidth of this sidelink BWP. | True, will remove. |
| ZTE | 9.2 | The new added “MAC configuration” for duplication shares same name with legacy MAC configuration for SRB. Suggested to use “MAC configuration associated to additional RLC configuration” | OK, will add |
| ZTE | 9.x | Do we need to move R17 tx profile to the new added clause 9.X? | Tend to avoid change to the legacy part |
| Huawei, HiSilicon | 6.3.2 | Within IE MeasObjectNR-SL, the FD for field sl-Frequency is needed, as this field is new and appears for the first time. The FD can reuse the change on similar SUI fields, " The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on." | True, as replied to ZTE. |
| Huawei, HiSilicon | 6.3.2 | Within IE MeasResultsSL: though RAN2 has no explicit agreement on the frequency dimension (i.e. carrier index), it is straightforward such frequency dimension (i.e. carrier index) is needed for measurement results. | Yet I thought the meas-ID which is included in MR, and associated to a MO, will already play the role for that? |
| Ericsson | 5.8.3.1, 5.8.3.2 | The texts refer to “sidelink carrier failure” | Better to add “SL CA”, since sidelink carrier failure is only valid for SL CA. |
| Ericsson | 5.8.9.1.2 | 1> set the entry included in the sl-CarrierToReleaseList corresponding to the sidelink carrier(s) for which MAC entity indicates that the maximum number of consecutive HARQ DTX for a specific destination has been reached; | The bullet 1> needs to be improved to state that the “the maximum number of consecutive HARQ DTX for a specific destination and a carrier” |
| Ericsson | 6.3.5 | 1. SL-SCCH-CarrierSetConfig-r18 ::= SEQUENCE { sl-DestinationList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16, sl-SRB-Identity SEQUENCE (SIZE (1..3)) OF SRB-Identity, allowedCarrierFreqSet1-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16), allowedCarrierFreqSet2-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16) } 2. Sl-SRB-Identity This field indicates the list of sidelink SRB identities that the allowedCarrierFreqSet1 and allowedCarrierFreqSet2 apply. 3. ue-ToUE-COT-SharingED-Threshold Indicates the energy detection threshold that is to be used by a UE to initiate a channel occupancy to be shared to other UE(s), and another UE that shares the initiated channel occupancy shall use this configured parameter for accessing the channel(s) as specified in TS 37.213 [48], clause 4.5.5 for sidelink channel access. Unit in dBm. Value -85 corresponds to -85 dBm, value -84 corresponds to -84 dBm, and so on (i.e. in steps of 1dBm). | - the two fields allowedCarrierFreqSet1 and allowedCarrierFreqSet2 need to be renamed as “sl-” - the field name Sl-SRB-Identity need to be lower case. - For the highlighted text, can it be reformulated that “the COT initiating UE shall use this configured parameter for accessing the channel” to simplify the text. No need to repeat the same text for the UE and the other UE. |
| Qualcomm | 5.2.2.4.13 And other sections | “2> if sl-FreqInfoList/sl-FreqInfoListSizeExt is included in SIB12-IEs: 3> if configured to receive NR sidelink communication: 4> use the resource pool(s) indicated by sl-RxPool for NR sidelink communication reception, as specified in 5.8.7; 3> if configured to transmit NR sidelink communication: 4> use the resource pool(s) indicated by sl-TxPoolSelectedNormal, or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8; 4> perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.5.3.1; - use the synchronization configuration parameters for NR sidelink communication on frequencies included in sl-FreqInfoList/sl-FreqInfoListSizeExt, as specified in 5.8.5; …” | Based on the text with sl-FreqInfoListSizeExt in this CR, seems it contains extra frequencies. If so, would sl-FreqInfoListSizeExt be more clear? |
| Qualcomm | 5.8.3.3 | 5> if sl-FreqInfoListSizeExt is included in SIB12-IEs: 6> set sl-QoS-InfoList to include the frequency(ies), and Tx Profile mapped to the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink communication transmission; | Not sure if frequencies should be included in sl-QoS-InfoList for backward compatible reason (sl-QoS-InfoList-r16 ). Both frequencies and Tx Profiles can be handled the same way, i.e., the frequency(ies) and Tx Profile(s) mapped to the sidelink QoS flow(s) which is included in sl-QoS-InfoList, i.e., QoS profile(s) of the sidelink QoS flow(s) of the associated destination. |
| Qualcomm | 6.2.2 | #### – SidelinkUEInformationNR The SidelinkUEinformationNR message is used for the indication of NR sidelink UE information to the network. | Not sure if we need to have r18 for sl-QoS-Infolist here. |
|  | 6.2.2 | sl-FreqInfoList, sl-FreqInfoListSizeExt #### This field indicates the NR sidelink communication/discovery configuration on some carrier frequency (ies). In this release, only one entry can be configured in the list. | Suggest having some description of the sl-FreqInfoListSizeExt |