3GPP TSG RAN WG2 Meeting #124	R2-23xxxxx

Chicago, USA, 13th – 17th Nov., 2023

Title:	[DRAFT] LS on MAC agreements for SL positioning

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	Huawei, HiSilicon [to be RAN2]

To:	RAN1

Contact Person:

Name:	Yinghao Guo

E-mail Address:	yinghaoguo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

During RAN2#124, the following agreements have been achieved in RAN2 related to SL positioning.

- Uplink transmission can be considered as prioritized when uplink cannot be transmitted together with sidelink and none of the V2X sidelink communications or NR sidelink communications or sidelink PRS transmissions are prioritized. - The prioritization between SR triggered by UL-SCH and SL-PRS shall follow the same principle as that between UL-SCH and SL-SCH, i.e. based on configured UL/SL prioritization thresholds. - The prioritization between SR triggered by SL-SCH and SL-PRS shall be based on direct comparison between the SL priority for SL-PRS and the SL logical channel that triggered the SR. - Reuse the legacy threshold for SL communications for SL-PRS prioritization. - Implement support for retransmission on dedicated resource pool. - When there are both SL-PRS and SL-SCH data pending for transmission at resource selection, the resource selection should be within the smaller one of the SL-PRS delay budget of the pending SL-PRSs and PDB of the logical channels.

In addition, RAN2 would like to ask RAN1 the following questions:

- On the maximum number of parallel SL-PRS transmission

  - What is the maximum total number of parallel SL-PRS transmission on SL-PRS shared/dedicated resource pool for a certain pair of Tx-Rx UE?

  - For a certain pair of Tx-Rx UE, what is the maximum number of parallel SL-PRS transmission supported on a SL-PRS shared resource pool and dedicated SL-PRS resource pool, respectively?

- When SL-PRS is transmitted on a SL-PRS shared resource pool where PSFCH is configured, if the associated PSSCH transmission is positively acknowledged, should the UE continue to perform SL-PRS retransmission?

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the above information into account.

3. Date of Next TSG RAN WG2 Meetings:

TSG RAN WG2 Meeting #125	26th Feb – 01st March 2024	Athens, GR

TSG RAN WG2 Meeting #125-bis	15th Apr – 19th Apr 2024	China (TBC)