| Company | Clause (if comment on CR) or RIL number (if comment on RIL list) | Suggested Change / Comment | Rapp Response |
| --- | --- | --- | --- |
| Xiaomi | 5.8.9.1a.6.2 | The above change should be withdrawn. | Failed to recall the reason for this change.. let’s withdrawn it. Thanks for catching it. |
| ZTE | 6.3.5 | The name of “sl-RLC-BearerToAddModListSizeExt” and “sl-RLC-BearerToReleaseListSizeExt” are so strange. From my view, I think it should be “sl-RLC-BearerToAddModListExt”, i.e. “size” should be removed from the IE name, like “sl-FreqInfoToAddModListExt”. |  |
| Nokia | 2 | [xx] 3GPP TS 24.588: "Technical Specification Group Core Network and Terminals; Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3". Missing space before “Stage 3” |  |
| Nokia | 5.8.6.2 | 4> for each freqneucy which is used for NR sidelink communication/discovery, if the UE detects one or more SLSSIDs for which the PSBCH-RSRP exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and for which the UE received the corresponding MasterInformationBlockSidelink message (candidate SyncRef UEs), or if the UE detects GNSS that is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14], or if the UE detects a cell, select a synchronization reference according to the priority group order as defined in 5.8.6.2a: We think it may be easier to read if the “for each” is independent from the “if the UE detects...” and this is then level 5 “freqneucy” should be changed to “frequency” |  |
| Nokia | 5.8.6.2 | 2> else if the concerned frequency(ies) are included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12, and sl-SyncPriority for concerned frequency(ies) are not configured or are set to gnss, and GNSS is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]; or if the concerned frequency(ies) are included in SL-PreconfigurationNR, and sl-SyncPriority in SidelinkPreconfigNR is set to gnss and GNSS is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]: For readability, we would suggest the “; or if the concerned frequencies” would be separate bullet. But, we also understand this is the legacy way in the spec |  |
|  |  | 5> select the synchronisation reference source(s) o according to the following priority group order as defined in 5.8.6.2a; Missing deletion of “o” in “n each concerned” |  |
| Nokia | 5.8.6.2b | 1> perform a full search (i.e. covering all subframes and all possible SLSSIDs) on each indicated frequency to detect candidate SLSS, in accordance with TS 38.133 [14] Missing “;” |  |
| Nokia | 5.8.9.1a.5.1 | For NR sidelink communication, additional sidelink RLC bearer release is initiated in the following cases: 1> for sidelink DRB, the release conditions are met as in clause 5.8.9.1a.1.1 for the associated sidelink DRB; or 1> for sidelink SRB, the release conditions are met as in clause 5.8.9.1a.3 for the associated sidelink SRB; or 1> for sidelink DRB, if SL-RLC-BearerConfigIndex (if any) of the sidelink DRB is included in sl-RLC-BearerToReleaseList in RRCReconfigurationSidelink; 1> for sidelink DRB, if SL-RLC-BearerConfigIndex (if any) of the additional Sidelink RLC Bearer is included in sl-RLC-BearerToReleaseListSizeExt in sl-ConfigDedicatedNR; or Missing “or” in the third bullet, excess “or” in fourth bullet |  |
| Nokia | 5.8.9.1b.1.1 | For NR sidelink communication, sidelink carrier release is initiated in the following cases: 1> for unicast, if sl-Carrier-Id of the sidelink carrier is received in sl-CarrierToReleaseList in the RRCReconfigurationSidelink; or 1> for unicast, if a sidelink carrier failure has been indicated by MAC layer; or 1> for unicast, if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or upper layer; or 1> for unicast, if the sidelink SRB(s), DRB(s) or additional sidelink RLC bearer(s), which was associated with the sidelink carrier(s), are released according to clause 5.8.9.1a.3.1, 5.8.9.1a.1.1 or 5.8.9.1a.5.1; Would suggest to have all conditions under a single “for unicast” bullet level as this would mean easier reading in case other cast types exists. Similar comment for 5.8.9.1b.1.2, 5.8.9.1b.2.1, and 5.8.9.1b.2.2 |  |
| Nokia | 6.3.4 | sl-SyncPriority This field indicates synchronization priority order, as specified in clause 5.8.6. The same value is configured in sl-SyncPriority across all carrier frequencies configured for UEs performing NR sidelink communication on multiple carrier frequencies.. We think a better wording may be “sl-SyncPriority is configured with the same value across all carrier frequencies...” Also “sl-SyncPriority” should be in cursive |  |
| Nokia | 6.3.4 | ue-ToUE-COT-SharingED-Threshold Indicates the energy detection threshold that is a UE uses to initiate a channel occupancy with other UE(s), and the other UE(s) that shares the initiated channel occupancy shall use this configured parameter for accessing the channel(s) as specified in TS 37.213 [48], clause 4.5.5 for sidelink channel access. Unit in dBm. Value -85 corresponds to -85 dBm, value -84 corresponds to -84 dBm, and so on (i.e. in steps of 1dBm).. Excessive “is” after “Indicates the energy detection threshold that is” |  |
| Nokia | 6.3.4 | harq-ACKFeedbackRatioforCW-AdjustmentGC-Option2 Indicates the ratio threshold for contention window adjustment for SL groupcast option 2 as specified in TS 37.213 [48], clause 4.5.4. Unit is percentage.. We believe that there is a missing hyphen between ACK and Feedback |  |