- Issue 1: DRX and measurement gaps during rach-less LTM switch [Technical Issue]

  - Confirm the intention that DRX should neither delay the completion of the LTM reconfiguration nor incur addition explicit RRC reconfigurations.

  - Postpone the discussion on the application of measurement gap and DRX configuration may be applied, during RACH-less LTM cell switch.

Issue 2: cg-LTM-RetransmissionTimer [Technical/Implementation Issue]

  - Proposal 7:	Agree on the intention from P1/2 in R2-2400880/R2-2401085 (details to be reviewed via post email).

The agreement is postponed to be captured in the next meeting, since there is the parallel post email discussion on [POST125][028][RACH-less] CR to 38.321, which is doing modification to the cg-LTM-RetransmissionTimer at the same time.

Issue 3: LTM MAC CE with Msg1 repetition for eRedCap in CovEnh WI [Implementation Issue]

The parallel email discussion “[POST125][805][CE_enh] Updated MAC CR” is capturing the “eRedCap+Msg1 repetition---merge the TP from R2-2401774”

We may need to do the similar change for the case of “CFRA indicated by LTM MAC CE with Msg1 repetition” to eRedCap, in the next meeting.

Issue 4: Need of “UL TCI state ID” field in LTM cell switch MAC CE.

Companies are encouraged to check if this “UL TCI state ID” field is really needed.

Since there is UL measurement for the source cell to select the UL TCI state.

Please only bring contribution if you see no need of this field. Otherwise, we don’t need to discuss this at all.