3GPP TSG-RAN2 Meeting #125bis	draft R2-2403956

# Changsha, China, April 15-19, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4693 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR Support for UAV (Uncrewed Aerial Vehicles) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV-Core |  |  |  |  |  | Date: |  |  | 2024-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Resolution fo remaining ASN.1 RILs based on outcome of [POST125][008] need to be captured. For more details, see Report of [POST125][008][UAV] Draft TP for simulMultiTriggerSingleMeasReport in R2-2402503. RAN2 received LS from RAN4 in R4-2403830/R2-2402137 and discussed the LS and the related RILs N135 and J061. RAN2#125bis agreed to use TP in R2-2403725 section 2.1 as a baseline and do further updates as needed. Additionally, RAN2#125bis agreed: When a measId is removed from VarMeasReportList, the associated periodical reporting timer should be stopped if it is running. Furthermore, since freqBandIndicatorNR-Aerial-r18 in NR-MultiBandInfoAerial-r18 reuses the existing conditional presence definition (Cond OptULNotSIB2) it needs to be clarified that the field is mandatory only when the parent IE is included in SIB4 or frequencyInfoDL-SIB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Resolution of following RILs are included: [Z077], [V823], [V824], [W015], [N135], [J061], including the additional agreement from RAN2#125bis. - In field description of nr-NS-PmaxListAerial, add reference to RAN4 specification TS 38.101-1 clause 6.2K. - Clarify in Cond OptULNotSIB2 that the field is mandatory only when the parent IE is included in SIB4 or frequencyInfoDL-SIB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for NR support for UAV remains ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.4.2, 5.5.2.4.3, 5.2.2.4.5, 5.5.4.1, 5.5.5.1, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the access is for NTN:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

3>	if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	if the UE is a fixed VSAT UE and the cellBarredFixedVSAT in the acquired SIB1 is set to barred or the cellBarredFixedVSAT is not included in the acquired SIB1, or

3>	if the UE is a mobile VSAT UE and the cellBarredMobileVSAT in the acquired SIB1 is set to barred or the cellBarredMobileVSAT is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the access is for ATG:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running; and

2>	if the cellBarredATG in the acquired SIB1 is set to barred or the cellBarredATG is not included in the acquired SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselectionRedCap is set to allowed, upon which the procedure ends;

2> else:

3>	if the cellBarredRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 1 Rx branch; or

3>	if the cellBarredRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 2 Rx branches; or

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE is a 2Rx XR UE and is in RRC_IDLE or in RRC_INACTIVE, or if the 2Rx XR UE is in RRC_CONNECTED while T311 is running:

2>	if the cellBarred2RxXR is present in the acquired SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if the intraFreqReselection2RxXR is present in the acquired SIB1:

4>	perform barring based on intraFreqReselection2RxXR as specified in TS 38.304 [20] upon which the procedure ends;

3>	else:

4>	perform barring as if intraFreqReselection2RxXR is set to allowed upon which the procedure ends;

1>	if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running:

2>	if cellBarred in the acquired MIB is set to barred:

3>	if cellBarredNES is absent in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE is an eRedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the eRedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselection-eRedCap is not present in SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselection-eRedCap is set to allowed upon which the procedure ends;

2>	else:

3>	if the cellBarred-eRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 1 Rx branch; or

3>	if the cellBarred-eRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 2 Rx branches; or

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselection-eRedCap as specified in TS 38.304 [20] upon which the procedure ends;

1>	if the cellAccessRelatedInfo contains an entry of a selected SNPN or PLMN and in case of PLMN the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN;

1>	else if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

1>	if the UE in RRC_INACTIVE is configured for feature(s) that it does not support in current serving cell:

2>	not use the corresponding configuration in current serving cell;

NOTE 0:	The requirement above applies only to UE that indicates different support of UE capabilities for TN and NTN.

1>	if in RRC_CONNECTED while T311 is not running:

2>	disregard the frequencyBandList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers, if included;

2>	forward the trackingAreaList to upper layers, if included;

2>	forward the received posSIB-MappingInfo to upper layers, if included;

2>	apply the configuration included in the servingCellConfigCommon;

2>	if the UE has a stored valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

3>	use the stored version of the required SIB or posSIB;

2>	else:

3>	acquire the required SIB or posSIB requested by upper layer as defined in clause 5.2.2.3.5;

NOTE 1:	Void.

1>	else:

2>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList or frequencyBandListAerial for downlink for TDD, or one or more of the frequency bands indicated in the frequencyBandList or frequencyBandListAerial for uplink for FDD, and they are not downlink only bands, and

2>	if the UE is IAB-MT or supports at least one additionalSpectrumEmission in the nr-NS-PmaxList or nr-NS-PmaxListAerial for a supported band in the downlink for TDD, or a supported band in uplink for FDD, and

2>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which

-	is smaller than or equal to the carrierBandwidth (indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP or, for (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured, and

2>	if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which

-	is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured), and which

-	is wider than or equal to the bandwidth of the initial downlink BWP or, for (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured, and

2>	if frequencyShift7p5khz is present and the UE supports corresponding 7.5kHz frequency shift on this band; or frequencyShift7p5khz is not present, and

2>	if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on this band:

3>	if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

3>	else if UE is IAB-MT but not a mobile IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN nor SNPN of the equivalent SNPN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3>	else if UE is NCR-MT and if ncr-Support is not provided:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3>	else if UE is a mobile IAB-MT and if mobileIAB-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN nor SNPN of the equivalent SNPN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3>	else:

4>	apply a supported uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UEs, RedCap-specific initial uplink BWP, if configured, and which

-	is wider than or equal to the bandwidth of the initial BWP for the uplink or, for a (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured;

4>	apply a supported downlink channel bandwidth with a maximum transmission bandwidth which

- is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UEs, RedCap-specific initial downlink BWP, if configured, and which

- is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured;

4>	if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which SIB1 includes nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial:

5>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which SIB1 includes nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial;

4>	else if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which SIB1 does not include nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxList within frequencyBandList for the same NR frequency band number:

5>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which SIB1 does not include nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxList within frequencyBandList for the same NR frequency band number;

4>	else:

5>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

4>	if the UE is aerial UE and SIB1 includes nr-NS-PmaxListAerial for the selected frequency band within frequencyBandListAerial in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD but the UE capable of nr-NS-PmaxListAerial does not support any of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial for the selected frequency band:

5>	consider the cell as barred in accordance with TS 38.304 [20];

5>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs, or intraFreqReselection2RxXR for 2Rx XR UEs is set to notAllowed, upon which the procedure ends;

4>	forward the cellIdentity to upper layers;

4>	forward the trackingAreaCode to upper layers;

4>	forward the trackingAreaList to upper layers, if included;

4>	forward the received posSIB-MappingInfo to upper layers, if included;

4>	forward the PLMN identity or SNPN identity or PNI-NPN identity to upper layers;

4>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

5>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

6>	initiate an RNA update as specified in 5.3.13.8;

5>	if configured to receive MBS multicast in RRC_INACTIVE and not indicated to stop monitoring G-RNTI for at least one MBS multicast session:

6>	if SIB24 is not scheduled in SIB1 in the new cell after cell selection or cell reselection:

7>	initiate an RRC connection resume procedure for multicast reception as specified in 5.3.13.1d;

4>	forward the ims-EmergencySupport to upper layers, if present;

4>	forward the eCallOverIMS-Support to upper layers, if present;

4>	forward the UAC-AccessCategory1-SelectionAssistanceInfo or UAC-AC1-SelectAssistInfo for the selected PLMN/SNPN to upper layers, if present and set to a, b or c;

4>	if the UE is in SNPN access mode:

5>	forward the imsEmergencySupportForSNPN indicators with the corresponding SNPN identities to upper layers, if present;

4>	apply the configuration included in the servingCellConfigCommon;

4>	apply the specified PCCH configuration defined in 9.1.1.3;

4>	if the UE has a stored valid version of a SIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

5>	use the stored version of the required SIB;

4>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s), in accordance with clause 5.2.2.1:

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

6>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in clause 5.2.2.3.3;

4>	if the UE has a stored valid version of a posSIB, in accordance with clause 5.2.2.2.1, of one or several required posSIB(s), in accordance with clause 5.2.2.1:

5>	use the stored version of the required posSIB;

4> if the UE has not stored a valid version of a posSIB, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) in accordance with clause 5.2.2.1:

5>	for the SI message(s) that, according to the posSI-SchedulingInfo, contain at least one requested posSIB and for which posSI-BroadcastStatus is set to broadcasting:

6>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the posSI-SchedulingInfo, contain at least one requested posSIB for which posSI-BroadcastStatus is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in clause 5.2.2.3.3a;

4>	if the UE is aerial UE and, for the selected frequency band, it supports at least one additionalSpectrumEmission value indicated by nr-NS-PmaxListAerial within frequencyBandListAerial in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD:

5>	apply the first listed additionalSpectrumEmission which it supports among the values indicated by nr-NS-PmaxListAerial for the selected frequency band within frequencyBandListAerial in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD;

4>	else:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD;

4>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList or nr-NS-PmaxListAerial:

5>	apply the additionalPmax for UL;

4>	else:

5>	apply the p-Max in uplinkConfigCommon for UL;

4>	if supplementaryUplink is present in servingCellConfigCommon; and

4>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and

4>	if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported supplementary uplink band; and

4>	if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and

4>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP of the SUL:

5>	consider supplementary uplink as configured in the serving cell;

5>	select the first frequency band in the frequencyBandList for the supplementaryUplink which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

5>	apply a supported supplementary uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial BWP of the SUL;

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList for the supplementaryUplink;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList for the supplementaryUplink:

6>	apply the additionalPmax in supplementaryUplink for SUL;

5>	else:

6>	apply the p-Max in supplementaryUplink for SUL;

NOTE 2:	For an out of coverage L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE receiving SIB1 from its connected L2 U2N Relay UE, it is up to Remote UE implementation whether to consider and apply the following parameters: frequencyBandList, carrierBandwidth, frequencyShift7p5khz, frequency band, channel bandwidth, the configuration included in the servingCellConfigCommon, the specified PCCH configuration, additionalSpectrumEmission, additionalPmax, and p-Max.

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs, or intraFreqReselection2RxXR for 2Rx XR UEs is set to notAllowed;

##### 5.2.2.4.3	Actions upon reception of SIB2

Upon receiving SIB2, the UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if, for the entry in frequencyBandList with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within the frequencyBandList; or

2>	if, for the entry in frequencyBandListAerial with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission value indicated by the nr-Ns-PmaxListAerial within the frequencyBandListAerial:

3>	if the UE is aerial UE and it supports at least one additionalSpectrumEmission value indicated by nr-NS-PmaxListAerial:

4>	apply the first listed additionalSpectrumEmission which it supports among the values indicated by nr-NS-PmaxListAerial ;

3>	else:

4>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList or nr-NS-PmaxListAerial:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

3>	if the UE selects a frequency band (from the procedure in clause 5.2.2.4.2) for the supplementary uplink:

4>	if, for the entry in frequencyBandListSUL with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within the frequencyBandListSUL:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandListSUL;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

4>	else:

5>	apply the p-Max.

2>	else:

3>	apply the p-Max;

##### 5.2.2.4.4	Actions upon reception of SIB3

No UE requirements related to the contents of this SIB3 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

##### 5.2.2.4.5	Actions upon reception of SIB4

Upon receiving SIB4 the UE shall:

1>	if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	for each entry in the interFreqCarrierFreqList:

3>	if the UE is neither a RedCap nor an eRedCap UE nor 2Rx XR UE; or

3>	if the UE is a RedCap UE and the interFreqCarrierFreqList-v1700 is absent; or

3>	if the UE is an eRedCap UE and the interFreqCarrierFreqList-v1800 is absent; or

3>	if the UE is a 2Rx XR UE and the interFreqCarrierFreqList-v1800 is absent; or

3>	if the UE is a RedCap UE and redCapAccessAllowed is present in interFreqCarrierFreqList-v1700; or

3>	if the UE is an eRedCap UE and eRedCapAccessAllowed is present in interFreqCarrierFreqList-v1800; or

3>	if the UE is a 2Rx XR UE and accessAllowed2RxXR is present in interFreqCarrierFreqList-v1800:

4>	select the first frequency band in the frequencyBandList (or for aerial UE frequencyBandListAerial), and frequencyBandListSUL, if present, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList (or for aerial UE as indicated by nr-NS-PmaxListAerial), if present:

4>	if, the frequency band selected by the UE in frequencyBandList or frequencyBandListAerial to represent a non-serving NR carrier frequency is not a downlink only band:

5>	if, for the selected frequency band, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within the frequencyBandList; or

5>	if, for the selected frequency band, the UE supports at least one additionalSpectrumEmission value indicated by the nr-NS-PmaxListAerial within the frequencyBandListAerial:

6>	if the UE is aerial UE and it supports at least one additionalSpectrumEmission value indicated by nr-NS-PmaxListAerial:

7>	apply the first listed additionalSpectrumEmission which it supports among the values indicated by nr-NS-PmaxListAerial;

6>	else:

7>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList;

6>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList or nr-NS-PmaxListAerial:

7>	apply the additionalPmax;

6>	else:

7>	apply the p-Max;

6>	if frequencyBandListSUL is present in SIB4 and, for the frequency band selected in frequencyBandListSUL, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within FrequencyBandListSUL:

7>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandListSUL;

7>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList:

8>	apply the additionalPmax;

7>	else:

8>	apply the p-Max;

6>	else:

7>	apply the p-Max;

5>	else:

6>	apply the p-Max;

1>	if in RRC_IDLE or RRC_INACTIVE, and T331 is running:

2>	perform the actions as specified in 5.7.8.1a;

Next Change

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the corresponding reportConfig includes measRSSI-ReportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	if the eventA3 or eventA5 or eventA3H1 or eventA3H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig:

5>	if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

4>	if the eventX2 is configured in the corresponding reportConfig:

5>	consider only the serving L2 U2N Relay UE to be applicable;

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1, eventA2, eventD1, eventD2, eventX2, eventH1 or eventH2:

5>	if useAllowedCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if eventB1 or eventB2 is configured in the corresponding reportConfig:

5>	consider a serving cell, if any, on the associated E-UTRA frequency as neighbour cell;

4>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModListEUTRAN defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns UTRA-FDD:

4>	if eventB1-UTRA-FDD or eventB2-UTRA-FDD is configured in the corresponding reportConfig; or

4>	if corresponding reportConfig includes reportType set to periodical:

5>	consider a neighbouring cell on the associated frequency to be applicable when the concerned cell is included in the cellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns L2 U2N Relay UE:

4>	if eventY1-Relay or eventY2-Relay or eventZ1-Relay is configured in the corresponding reportConfig; or

4>	if corresponding reportConfig includes reportType set to periodical:

5>	consider any L2 U2N Relay UE fulfilling upper layer criteria detected on the associated frequency to be applicable for this measId;

2>	else if the corresponding reportConfig includes a reportType set to reportCGI:

3>	consider the cell detected on the associated measObject which has a physical cell identity matching the value of the cellForWhichToReportCGI included in the corresponding reportConfig within the VarMeasConfig to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the reportSFTD-Meas is set to true:

5>	consider the NR PSCell to be applicable;

4>	else if the reportSFTD-NeighMeas is included:

5>	if cellsForWhichToReportSFTD is configured in the corresponding reportConfig:

6>	consider any NR neighbouring cell detected on the associated measObjectNR which has a physical cell identity that is included in the cellsForWhichToReportSFTD to be applicable;

5>	else:

6>	consider up to 3 strongest NR neighbouring cells detected based on parameters in the associated measObjectNR to be applicable when the concerned cells are not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if the reportSFTD-Meas is set to true:

5>	consider the E-UTRA PSCell to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to cli-Periodical or cli-EventTriggered:

3>	consider all CLI measurement resources included in the corresponding measObject to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to rxTxPeriodical:

3>	consider all Rx-Tx time difference measurement resources included in the corresponding measObject to be applicable;

2>	if the corresponding reportConfig concerns the reporting for NR sidelink communication/discovery (i.e. reportConfigNR-SL):

3>	consider the transmission resource pools indicated by the tx-PoolMeasToAddModList defined within the VarMeasConfig for this measId to be applicable;

2>	if the reportType is set to eventTriggered, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered, and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig:

3>	if the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	if the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	else:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

4>	if the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

5>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	if the corresponding reportConfig does not include numberOfTriggeringCells; or

4>	if the corresponding reportConfig includes numberOfTriggeringCells and a measurement report was previously sent to the network for at least one of the concerned cell(s):

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable L2 U2N Relay UEs for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first L2 U2N Relay UE triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable L2 U2N Relay UEs not included in the relaysTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent L2 U2N Relay UE triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the L2 U2N Relay UEs included in the relaysTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the relaysTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools not included in the poolsTriggeredList for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent transmission resource pool triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more applicable transmission resource pools included in the poolsTriggeredList defined within the VarMeasReportList for this measId for all measurements taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the poolsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the leaving condition applicable for this event is fulfilled for the associated VarMeasReport within the VarMeasReportList for this measId during timeToTrigger defined within the VarMeasConfig for this event:

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove the measurement reporting entry within the VarMeasReportList for this measId;

3>	stop the periodical reporting timer for this measId, if running;

NOTE 1:	Void.

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	if the corresponding reportConfig includes measRSSI-ReportConfig:

4>	initiate the measurement reporting procedure as specified in 5.5.5 immediately when RSSI sample values are reported by the physical layer after the first L1 measurement duration;

3>	else if the corresponding reportConfig includes the ul-DelayValueConfig:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided from lower layers of the associated DRB identity;

3>	else if the corresponding reportConfig includes the ul-ExcessDelayConfig:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided from lower layers of the associated DRB identity(ies) according to the configured threshold per DRB identity(ies);

3>	else if the reportAmount exceeds 1:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell or for the serving L2 U2N Relay UE (if the UE is a L2 U2N Remote UE);

3>	else (i.e. the reportAmount is equal to 1):

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and for the strongest cell among the applicable cells, or for the NR SpCell and for the strongest L2 U2N Relay UEs among the applicable L2 U2N Relay UEs; or initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the serving L2 U2N Relay UE and for the strongest cell among the applicable cells, or for the serving L2 U2N Relay UE and for the strongest L2 U2N Relay UEs among the applicable L2 U2N Relay UEs (if the UE is a L2 U2N Remote UE);

2>	if, in case the corresponding reportConfig concerns the reporting for NR sidelink communication/discovery, reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and CBR measurement results become available;

2>	if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable CLI measurement resources for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first CLI measurement resource triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more CLI measurement resources not included in the cli-TriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent CLI measurement resource triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to cli-EventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the CLI measurement resources included in the cli-TriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cli-TriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to cli-Periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for at least one CLI measurement resource;

2>	if reportType is set to rxTxPeriodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	if the corresponding reportConfig includes a reportType is set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the drx-SFTD-NeighMeas is included:

5>	if the quantity to be reported becomes available for each requested pair of PCell and NR cell:

6>	stop timer T322;

6>	initiate the measurement reporting procedure, as specified in 5.5.5;

4>	else

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for each requested pair of PCell and NR cell or the maximal measurement reporting delay as specified in TS 38.133 [14];

3>	else if the corresponding measObject concerns E-UTRA:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the pair of PCell and E-UTRA PSCell or the maximal measurement reporting delay as specified in TS 38.133 [14];

2>	if reportType is set to reportCGI:

3>	if the UE acquired the SIB1 or SystemInformationBlockType1 for the requested cell; or

3>	if the UE detects that the requested NR cell is not transmitting SIB1 (see TS 38.213 [13], clause 13):

4>	stop timer T321;

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

4>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon the expiry of T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	upon the expiry of T322 for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

If AS security has been activated successfully and if SCell activation(s) indication is received from lower layer, the UE shall:

1>	if reportType is set to reportOnActivation for any measId included in the measIdList within VarMeasConfig:

2>	if the activated SCell(s) fulfills the measurement requirement as specified in TS 38.133 [14]:

3>	include a measurement reporting entry within the VarMeasReportList for this measId:

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5.

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1>	for this measurement, consider the NR serving cell corresponding to the associated measObjectNR associated with this event.

Inequality A1-1 (Entering condition)

Ms – Hys > Thresh

Inequality A1-2 (Leaving condition)

Ms + Hys < Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell indicated by the measObjectNR associated to this event.

NOTE:	If the SCell indicated by the measObjectNR associated to this event is not detectable, then the UE should consider for the value of Ms the lowest value of the value range of the measurement quantity as the SCell measurement.

Inequality A2-1 (Entering condition)

Ms + Hys < Thresh

Inequality A2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than SpCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE 1:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Inequality A3-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

NOTE 2:	The definition of Event A3 also applies to CondEvent A3.

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled.

Inequality A4-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality A4-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell or the measurement result of serving PSCell (i.e., in case it is configured as candidate PSCell for CondEvent A4 evaluation) for CHO with candidate SCG(s) case, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the measurement object specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

NOTE:	The definition of Event A4 also applies to CondEvent A4.

#### 5.5.4.6	Event A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE 1:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality A5-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality A5-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality A5-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

NOTE 2:	The definition of Event A5 also applies to CondEvent A5.

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1>	for this measurement, consider the (secondary) cell corresponding to the measObjectNR associated to this event to be the serving cell.

NOTE:	The reference signal(s) of the neighbour(s) and the reference signal(s) of the SCell are both indicated in the associated measObjectNR.

Inequality A6-1 (Entering condition)

Mn + Ocn – Hys > Ms + Ocs + Off

Inequality A6-2 (Leaving condition)

Mn + Ocn + Hys < Ms + Ocs + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within the associated measObjectNR), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within the associated measObjectNR, or cellIndividualOffset as defined within reportConfigNR), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined within reportConfigNR for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

#### 5.5.4.8	Event B1 (Inter RAT neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition B1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B1-2, as specified below, is fulfilled.

Inequality B1-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality B1-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the neighbour inter-RAT cell, utra-FDD-Q-OffsetRange as defined within the measObjectUTRA-FDD corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell, or cellIndividualOffset as defined within reportConfigInterRAT), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. b1-ThresholdEUTRA as defined within reportConfigInterRAT for this event, b1-ThresholdUTRA-FDD as defined for UTRA-FDD within reportConfigInterRAT for this event).

Mn is expressed in dBm or in dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.9	Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition B2-1 and condition B2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B2-3 or condition B2-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality B2-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality B2-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality B2-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality B2-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the inter-RAT neighbour cell, utra-FDD-Q-OffsetRange as defined within the measObjectUTRA-FDD corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell, or cellIndividualOffset as defined within reportConfigInterRAT), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. b2-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. b2-Threshold2EUTRA as defined within reportConfigInterRAT for this event, b2-Threshold2UTRA-FDD as defined for UTRA-FDD within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mn is expressed in dBm or dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.10	Event I1 (Interference becomes higher than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition I1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition I1-2, as specified below, is fulfilled.

Inequality I1-1 (Entering condition)

Mi – Hys > Thresh

Inequality I1-2 (Leaving condition)

Mi+ Hys < Thresh

The variables in the formula are defined as follows:

Mi is the measurement result of the interference, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. i1-Threshold as defined within reportConfigNR for this event).

Mi, Thresh are expressed in dBm.

Hys is expressed in dB.

#### 5.5.4.11	Event C1 (The NR sidelink channel busy ratio is above a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition C1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition C1-2, as specified below, is fulfilled;

Inequality C1-1 (Entering condition)

Inequality C1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of channel busy ratio of the transmission resource pool, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR-SL for this event).

Thresh is the threshold parameter for this event (i.e. c1-Threshold as defined within reportConfigNR-SL for this event).

Ms is expressed in decimal from 0 to 1 in steps of 0.01.

Hys is expressed is in the same unit as Ms.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.12	Event C2 (The NR sidelink channel busy ratio is below a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition C2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition C2-2, as specified below, is fulfilled;

Inequality C2-1 (Entering condition)

Inequality C2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of channel busy ratio of the transmission resource pool, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR-SL for this event).

Thresh is the threshold parameter for this event (i.e. c2-Threshold as defined within reportConfigNR-SL for this event).

Ms is expressed in decimal from 0 to 1 in steps of 0.01.

Hys is expressed is in the same unit as Ms.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.13	Void

#### 5.5.4.14	Void

#### 5.5.4.15	Event D1 (Distance between UE and referenceLocation1 is above threshold1 and distance between UE and referenceLocation2 is below threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition D1-1 and condition D1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition D1-3 or condition D1-4, i.e. at least one of the two, as specified below, are fulfilled;

Inequality D1-1 (Entering condition 1)

Ml1-Hys>Thresh1

Inequality D1-2 (Entering condition 2)

Ml2+Hys<Thresh2

Inequality D1-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D1-4 (Leaving condition 2)

Ml2-Hys>Thresh2

The variables in the formula are defined as follows:

Ml1 is the distance between UE and a reference location for this event (i.e. referenceLocation1 as defined within reportConfigNR for this event), not taking into account any offsets.

Ml2 is the distance between UE and a reference location for this event (i.e. referenceLocation2 as defined within reportConfigNR for this event), not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresisLocation as defined within reportConfigNR for this event).

Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1, from a reference location configured with parameter referenceLocation1 within reportConfigNR for this event.

Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2, from a reference location configured with parameter referenceLocation2 within reportConfigNR for this event.

Ml1 is expressed in meters.

Ml2 is expressed in the same unit as Ml1.

Hys is expressed in the same unit as Ml1.

Thresh1 is expressed in the same unit as Ml1.

Thresh2 is expressed in the same unit as Ml1.

NOTE:	The definition of Event D1 also applies to CondEvent D1.

#### 5.5.4.15a	Event D2 (Distance between UE and serving cell moving reference location is above threshold1 and distance between UE and a moving reference location is below threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition D2-1 and condition D2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition D2-3 or condition D2-4, i.e. at least one of the two, as specified below, are fulfilled;

Inequality D2-1 (Entering condition 1)

Ml1-Hys>Thresh1

Inequality D2-2 (Entering condition 2)

Ml2+Hys<Thresh2

Inequality D2-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D2-4 (Leaving condition 2)

Ml2-Hys>Thresh2

The variables in the formula are defined as follows:

Ml1 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on movingReferenceLocation and the corresponding epoch time and satellite ephemeris for the serving cell broadcast in SIB19.

Ml2 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on referenceLocation2 as defined within reportConfigNR for this event and the corresponding epoch time and satellite ephemeris for the candidate cell.

Hys is the hysteresis parameter for this event (i.e. hysteresisLocation as defined within reportConfigNR for this event).

Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1 in reportConfigNR for this event, from a moving reference location derived based on the parameter movingReferenceLocation within SIB19.

Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2, from a moving reference location derived based on the parameter referenceLocation2 within reportConfigNR for this event.

Ml1 is expressed in meters.

Ml2 is expressed in the same unit as Ml1.

Hys is expressed in the same unit as Ml1.

Thresh1 is expressed in the same unit as Ml1.

Thresh2 is expressed in the same unit as Ml1.

NOTE:	The definition of Event D2 also applies to CondEvent D2.

#### 5.5.4.16	CondEvent T1 (Time measured at UE is within a duration from threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition T1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition T1-2, as specified below, is fulfilled;

Inequality T1-1 (Entering condition)

Inequality T1-2 (Leaving condition)

The variables in the formula are defined as follows:

Mt is the time measured at UE.

Thresh1 is the threshold parameter for this event (i.e. t1-Threshold as defined within reportConfigNR for this event).

Duration is the duration parameter for this event (i.e. duration as defined within reportConfigNR for this event).

Mt is expressed in ms.

Thresh1 is expressed in the same unit as Mt.

Duration is expressed in the same unit as Mt.

#### 5.5.4.17	Event X1 (Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition X1-1 and condition X1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition X1-3 or condition X1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality X1-1 (Entering condition 1)

Mr + Hys < Thresh1

Inequality X1-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality X1-3 (Leaving condition 1)

Mr – Hys > Thresh1

Inequality X1-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Mn is the measurement result of the NR cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the NR cell (i.e. offsetMO as defined within measObjectNR corresponding to the NR cell).

Ocn is the cell specific offset of the NR cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the NR cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the cell.

Hys is the hysteresis parameter for this event.

Thresh1 is the threshold parameter for this event (i.e. x1-Threshold1-Relay/eventX1-SD-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. x1-Threshold2 as defined within reportConfigNR for this event).

Mr is expressed in dBm.

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mr.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.18	Event X2 (Serving L2 U2N Relay UE becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition X2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition X2-2, as specified below, is fulfilled;

Inequality X2-1 (Entering condition)

Mr + Hys < Thresh

Inequality X2-2 (Leaving condition)

Mr – Hys > Thresh

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. x2-Threshold-Relay/eventX2-SD-Threshold as defined within reportConfigNR for this event).

Mr is expressed in dBm.

Hys are expressed in dB.

Thresh is expressed in the same unit as Mr.

#### 5.5.4.19	Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition Y1-1 and condition Y1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Y1-3 or condition Y1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality Y1-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality Y1-2 (Entering condition 2)

Mr– Hys > Thresh2

Inequality Y1-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality Y1-4 (Leaving condition 2)

Mr + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mr is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. y1-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. y1-Threshold2-Relay as defined within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mr is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mr.

#### 5.5.4.20	Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition Y2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Y2-2, as specified below, is fulfilled;

Inequality Y2-1 (Entering condition)

Mr– Hys > Thresh

Inequality Y2-2 (Leaving condition)

Mr + Hys < Thresh

The variables in the formula are defined as follows:

Mr is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. y2-Threshold-Relay as defined within reportConfigInterRAT for this event).

Mr is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh is expressed in the same unit as Mr.

#### 5.5.4.20b	Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition Z1-1 and condition Z1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Z1-3 or condition Z1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality Z1-1 (Entering condition 1)

Mr + Hys < Thresh1

Inequality Z1-2 (Entering condition 2)

Mn – Hys > Thresh2

Inequality Z1-3 (Leaving condition 1)

Mr – Hys > Thresh1

Inequality Z1-4 (Leaving condition 2)

Mn + Hys < Thresh2

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Mn is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. sl-rsrp in z1-Threshold1-Relay as defined within reportConfigInterRAT if the UE measures SL-RSRP, or sd-rsrp in z1-Threshold1-Relay as defined within reportConfigInterRAT if the UE measures SD-RSRP for this event).

Thresh2 is the threshold parameter for this event (i.e. z1-Threshold2-Relay as defined within reportConfigInterRAT for this event).

Mr is expressed in dBm or dB, depending on the measurement quantity of serving L2 U2N Relay UE.

Mn is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mr.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.21	Event H1 (The Aerial UE altitude becomes higher than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H1-2, as specified below, is fulfilled;

Inequality H1-1 (Entering condition)

Ms – Hys > Thresh

Inequality H1-2 (Leaving condition)

Ms + Hys < Thresh

The variables in the formula are defined as follows:

Ms is the Aerial UE altitude relative to the sea level.

Hys is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Ms, Hys, Thresh are expressed in meters.

#### 5.5.4.22	Event H2 (The Aerial UE altitude becomes lower than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H2-2, as specified below, is fulfilled;

Inequality H2-1 (Entering condition)

Ms + Hys < Thresh

Inequality H2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the Aerial UE altitude relative to the sea level.

Hys is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Ms, Hys, Thresh are expressed in meters.

#### 5.5.4.23	Event A3H1 (Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes higher than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A3H1-1 and condition A3H1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3H1-3 or condition A3H1-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE 1:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3H1-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off

Inequality A3H1-2 (Entering condition 2)

Ms – Hys2 > Thresh

Inequality A3H1-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off

Inequality A3H1-4 (Leaving condition 2)

Ms + Hys2 < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys1 is the hysteresis parameter for this event (i.e. a3-Hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1, Ofp, Ocp, Off are expressed in dB.

Ms, Hys2, Thresh are expressed in meters.

#### 5.5.4.24	Event A3H2 (Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes lower than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A3H2-1 and condition A3H2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3H2-3 or condition A3H2-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3H2-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off

Inequality A3H2-2 (Entering condition 2)

Ms + Hys2 < Thresh

Inequality A3H2-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off

Inequality A3H1-4 (Leaving condition 2)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys1 is the hysteresis parameter for this event (i.e. a3-Hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1, Ofp, Ocp, Off are expressed in dB.

Ms, Hys2, Thresh are expressed in meters.

#### 5.5.4.25	Event A4H1 (Neighbour becomes better than threshold1 and the Aerial UE altitude becomes higher than a threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A4H1-1 and condition A4H1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4H1-3 or condition A4H1-4, i.e. at least one of the two, as specified below, is fulfilled.

Inequality A4H1-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Thresh1

Inequality A4H1-2 (Entering condition 2)

Ms – Hys2 > Thresh2

Inequality A4H1-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Thresh1

Inequality A4H1-4 (Leaving condition 2)

Ms + Hys2 < Thresh2

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the measurement object specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a4-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mn.

Ms, Hys2, Thresh2 are expressed in meters.

#### 5.5.4.26	Event A4H2 (Neighbour becomes better than threshold1 and the Aerial UE altitude becomes lower than a threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A4H2-1 and condition A4H2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4H2-3 or condition A4H2-4, i.e. at least one of the two, as specified below, is fulfilled.

Inequality A4H2-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Thresh1

Inequality A4H2-2 (Entering condition 2)

Ms + Hys2 < Thresh2

Inequality A4H2-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Thresh1

Inequality A4H2-4 (Leaving condition 2)

Ms – Hys2 > Thresh2

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the measurement object specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a4-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mn.

Ms, Hys2, Thresh2 are expressed in meters.

#### 5.5.4.27	Event A5H1 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes higher than a threshold3)

The UE shall:

1>	consider the entering condition for this event to be satisfied when all of condition A5H1-1 and condition A5H1-2 and condition A5H1-3, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5H1-4 or condition A5H1-5 or condition A5H1-6, i.e. at least one of the three, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5H1-1 (Entering condition 1)

Mp + Hys1 < Thresh1

Inequality A5H1-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys1 > Thresh2

Inequality A5H1-3 (Entering condition 3)

Ms – Hys2 > Thresh3

Inequality A5H1-4 (Leaving condition 1)

Mp – Hys1 > Thresh1

Inequality A5H1-5 (Leaving condition 2)

Mn + Ofn + Ocn + Hys1 < Thresh2

Inequality A5H1-6 (Leaving condition 3)

Ms + Hys2 < Thresh3

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a5-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh3 is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

Ms, Hys2, Thresh3 are expressed in meters.

#### 5.5.4.28	Event A5H2 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes lower than a threshold3)

The UE shall:

1>	consider the entering condition for this event to be satisfied when all of condition A5H2-1 and condition A5H2-2 and condition A5H2-3, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5H2-4 or condition A5H2-5 or condition A5H2-6, i.e. at least one of the three, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5H2-1 (Entering condition 1)

Mp + Hys1 < Thresh1

Inequality A5H2-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys1 > Thresh2

Inequality A5H2-3 (Entering condition 3)

Ms + Hys2 < Thresh3

Inequality A5H2-4 (Leaving condition 1)

Mp – Hys1 > Thresh1

Inequality A5H2-5 (Leaving condition 2)

Mn + Ofn + Ocn + Hys1 < Thresh2

Inequality A5H2-6 (Leaving condition 3)

Ms – Hys2 > Thresh3

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a5-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh3 is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

Ms, Hys2, Thresh3 are expressed in meters.

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

The UE shall, for each entry in the VarMeasReportList:

1>	if the eventH1 or eventH2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

1>	else if the eventA3H1 or eventA3H2 or eventA4H1 or eventA4H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList associated with the same measObjectNR for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	for each serving cell configured with servingCellMO:

2>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

1>	set the servCellId within measResultServingMOList to include each NR serving cell that is configured with servingCellMO, if any;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

2>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2>	for each measObjectId referenced in the measIdList which is also referenced with servingCellMO, other than the measObjectId corresponding with the measId that triggered the measurement reporting:

3>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

4>	set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each best non-serving cell included in the measurement report:

6>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2, or eventA3H1, or eventA3H2, or eventA4H1, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventA3H1, or eventA3H2, or eventA4H1, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NR-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListNR-SCG to include for each NR SCG serving cell that is configured with servingCellMO, if any, the following:

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

5>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

4>	else:

5>	if SSB based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

5>	else if CSI-RS based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

4>	if results for the serving cell derived based on SSB are included:

5>	include the ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR of the serving cell;

4>	if results for the serving cell derived based on CSI-RS are included:

5>	include the refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the MeasObjectNR of the serving cell;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

6>	set the measResultNeighCellListNR within measResultServFreqListNR-SCG to include one entry with the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR, where availability is considered according to the measurement configuration associated with the SCG;

7>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

8>	for each best non-serving cell included in the measurement report:

9>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

1>	if the measRSSI-ReportConfig is configured within the corresponding reportConfig for this measId:

2>	set the rssi-Result to the linear average of sample value(s) provided by lower layers in the reportInterval;

2>	set the channelOccupancy to the rounded percentage of sample values which are beyond the channelOccupancyThreshold within all the sample values in the reportInterval;

1>	if the UE is acting as L2 U2N Remote UE:

2>	set the sl-MeasResultServingRelay in accordance with the following:

3>	set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the serving L2 U2N Relay UE;

3>	set the sl-RelayUE-Identity to include the Source L2 ID of the serving L2 U2N Relay;

3>	if the measurement of serving L2 U2N Relay UE is based on SL-RSRP:

4>	set the sl-MeasResult to include the SL-RSRP of the serving L2 U2N Relay UE;

4>	set the sl-MeasQuantity to sl-rsrp, if supported by the UE;

3>	else:

4>	set the sl-MeasResult to include the SD-RSRP of the serving L2 U2N Relay UE;

4>	set the sl-MeasQuantity to sd-rsrp, if supported by the UE;

NOTE 1:	In case of no data transmission from L2 U2N Relay UE to L2 U2N Remote UE, it is left to UE implementation whether to use SL-RSRP or SD-RSRP when setting the sl-MeasResultServingRelay of the serving L2 U2N Relay UE.

1>	if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	if the measurement report concerns the candidate L2 U2N Relay UE:

4>	set the sl-MeasResultsCandRelay in measResultNeighCells to include the best candidate L2 U2N Relay UEs up to maxNrofRelayMeas in accordance with the following:

5>	if the reportType is set to eventTriggered:

6>	include the L2 U2N Relay UEs included in the relaysTriggeredList as defined within the VarMeasReportList for this measId;

5>	else:

6>	include the applicable L2 U2N Relay UEs for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	for each L2 U2N Relay UE that is included in the sl-MeasResultsCandRelay:

6>	set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the concerned L2 U2N Relay UE;

6>	set the sl-RelayUE-Identity to include the Source L2 ID of the concerned L2 U2N Relay UE;

6>	set the sl-MeasResult to include the SD-RSRP of the concerned L2 U2N Relay UE;

5>	for each included L2 U2N Relay UE, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

6>	set the sl-MeasResult to include the quantity(ies) indicated in the reportQuantityRelay within the concerned reportConfigRelay in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best L2 U2N Relay UE is included first;

6>	if the UE supports multipathRemoteUE-PC5L2 and idle/inactive relay UE reporting, and if the sl-RelayIndicationMP is contained in the discovery message received from the concerned L2 U2N Relay UE:

7>	set the sl-RelayIndicationMP in the sl-MeasResultsCandRelay;

3>	else:

4>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

5>	if the reportType is set to eventTriggered and eventId is not set to eventD1 or eventD2 or eventH1 or eventH2:

6>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

5>	else:

6>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	for each cell that is included in the measResultNeighCells, include the physCellId;

5>	if the reportType is set to eventTriggered or periodical:

6>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

7>	if the measObject associated with this measId concerns NR:

8>	if rsType in the associated reportConfig is set to ssb:

9>	set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

9>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

8>	else if rsType in the associated reportConfig is set to csi-rs:

9>	set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

9>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

7>	if the measObject associated with this measId concerns E-UTRA:

8>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

7>	if the measObject associated with this measId concerns UTRA-FDD and if ReportConfigInterRAT includes the reportQuantityUTRA-FDD:

8>	set the measResult to include the quantity(ies) indicated in the reportQuantityUTRA-FDD within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

2>	else:

3>	if the cell indicated by cellForWhichToReportCGI is an NR cell:

4>	if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), trackingAreaList (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList;

5>	include frequencyBandList if available;

5>	for each PLMN-IdentityInfo in plmn-IdentityInfoList:

6>	if the gNB-ID-Length is broadcast:

7>	include gNB-ID-Length;

4>	if nr-CGI-Reporting-NPN is supported by the UE and npn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the npn-IdentityInfoList including npn-IdentityList, trackingAreaCode, ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the npn-IdentityInfoList;

5>	for each NPN-IdentityInfo in NPN-IdentityInfoList:

6>	if the gNB-ID-Length is broadcast:

7>	include gNB-ID-Length;

5>	include cellReservedForOtherUse if available;

4>	else if MIB indicates the SIB1 is not broadcast:

5>	include the noSIB1 including the ssb-SubcarrierOffset and pdcch-ConfigSIB1 obtained from MIB of the concerned cell;

3>	if the cell indicated by cellForWhichToReportCGI is an E-UTRA cell:

4>	if all mandatory fields of the cgi-Info-EPC for the concerned cell have been obtained:

5>	include in the cgi-Info-EPC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to EPC;

4>	if the UE is E-UTRA/5GC capable and all mandatory fields of the cgi-Info-5GC for the concerned cell have been obtained:

5>	include in the cgi-Info-5GC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to 5GC;

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>	include the freqBandIndicator;

5>	if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

5>	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;

1>	if the corresponding measObject concerns NR:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigNR for this measId:

3>	set the measResultSFTD-NR in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrp-Result to the RSRP of the NR PSCell derived based on SSB;

2>	else if the reportSFTD-NeighMeas is included within the corresponding reportConfigNR for this measId:

3>	for each applicable cell which measurement results are available, include an entry in the measResultCellListSFTD-NR and set the contents as follows:

4>	set physCellId to the physical cell identity of the concerned NR neighbour cell.

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true:

5>	set rsrp-Result to the RSRP of the concerned cell derived based on SSB;

1>	else if the corresponding measObject concerns E-UTRA:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigInterRAT for this measId:

3>	set the measResultSFTD-EUTRA in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrpResult-EUTRA to the RSRP of the EUTRA PSCell;

1>	if average uplink PDCP delay values are available:

2>	set the ul-PDCP-DelayValueResultList to include the corresponding average uplink PDCP delay values;

1>	if PDCP excess delay measurements are available:

2>	set the ul-PDCP-ExcessDelayResultList to include the corresponding PDCP excess delay measurements;

1>	if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows:

2>	include the locationTimestamp;

2>	include the locationCoordinate, if available;

2>	include the velocityEstimate, if available;

2>	include the locationError, if available;

2>	include the locationSource, if available;

2>	if available, include the gnss-TOD-msec,

1>	if the coarseLocationRequest is set to true in the corresponding reportConfig for this measId:

2>	include coarseLocationInfo, if available;

1>	if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the wlan-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultWLAN, in order of decreasing RSSI for WLAN APs;

1>	if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the BT-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultBT, in order of decreasing RSSI for Bluetooth beacons;

1>	if the includeSensor-Meas is configured in the corresponding reportConfig for this measId, set the sensor-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the sensor-MeasurementInformation;

2>	if available, include the sensor-MotionInformation;

1>	if the includeAltitudeUE is set to true in the corresponding reportConfig for this measId:

2>	set the altitudeUE to include the altitude of the UE;

1>	if there is at least one applicable transmission resource pool for NR sidelink communication/discovery (for measResultsSL):

2>	set the measResultsListSL to include the CBR measurement results in accordance with the following:

3>	if the reportType is set to eventTriggered:

4>	include the transmission resource pools included in the poolsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable transmission resource pools for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

3>	if the corresponding measObject concerns NR sidelink communication/discovery, then for each transmission resource pool to be reported:

4>	set the sl-poolReportIdentity to the identity of this transmission resource pool;

4>	set the sl-CBR-ResultsNR to the CBR measurement results on PSSCH and PSCCH of this transmission resource pool provided by lower layers, if available;

NOTE 1:	Void.

1>	if there is at least one applicable CLI measurement resource to report:

2>	if the reportType is set to cli-EventTriggered or cli-Periodical:

3>	set the measResultCLI to include the most interfering SRS resources or most interfering CLI-RSSI resources up to maxReportCLI in accordance with the following:

4>	if the reportType is set to cli-EventTriggered:

5>	if trigger quantity is set to srs-RSRP i.e. i1-Threshold is set to srs-RSRP:

6>	include the SRS resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

5>	if trigger quantity is set to cli-RSSI i.e. i1-Threshold is set to cli-RSSI:

6>	include the CLI-RSSI resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

4>	else:

5>	if reportQuantityCLI is set to srs-rsrp:

6>	include the applicable SRS resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	else:

6>	include the applicable CLI-RSSI resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	for each SRS resource that is included in the measResultCLI:

5>	include the srs-ResourceId;

5>	set srs-RSRP-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering SRS resource is included first;

4>	for each CLI-RSSI resource that is included in the measResultCLI:

5>	include the rssi-ResourceId;

5>	set cli-RSSI-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering CLI-RSSI resource is included first;

1>	if there is at least one applicable UE Rx-Tx time difference measurement to report:

2>	set measResultRxTxTimeDiff to the latest measurement result;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical or cli-Periodical or rxTxPeriodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1>	if the measurement reporting was configured by a sl-ConfigDedicatedNR received within the RRCConnectionReconfiguration:

2>	submit the MeasurementReport message to lower layers for transmission via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified TS 36.331 [10], clause 5.6.28;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured and the SCG is not deactivated:

3>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2>	else:

3>	submit the MeasurementReport message via E-UTRA embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the measurement configuration that triggered this measurement report is associated with the SCG:

3>	if SRB3 is configured and the SCG is not deactivated:

4>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

3>	else:

4>	submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the MeasurementReport message via SRB1 to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

#### 5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered or reportOnScellActivation:

2>	consider the trigger quantity as the sorting quantity if available, otherwise RSRP as sorting quantity if available, otherwise RSRQ as sorting quantity if available, otherwise SINR as sorting quantity;

1>	if reportType is set to periodical:

2>	if a single reporting quantity is set to true in reportQuantityRS-Indexes;

3>	consider the configured single quantity as the sorting quantity;

2>	else:

3>	if rsrp is set to true;

4>	consider RSRP as the sorting quantity;

3>	else:

4>	consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting quantity and if absThreshSS-BlocksConsolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

3>	if includeBeamMeasurements is set to true, include the SS/PBCH based measurement results for the quantities in reportQuantityRS-Indexes for each SS/PBCH block index;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and, if absThreshCSI-RS-Consolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation;

3>	if includeBeamMeasurements is set to true, include the CSI-RS based measurement results for the quantities in reportQuantityRS-Indexes for each CSI-RS index.

#### 5.5.5.3	Sorting of cell measurement results

The UE shall determine the sorting quantity according to parameters of the reportConfig associated with the measId that triggered the reporting:

1>	if the reportType is set to eventTriggered:

2>	for an NR cell, consider the quantity used in the aN-Threshold (for eventA1, eventA2, eventA4, eventA4H1 and eventA4H2) or in the a5-Threshold2 (for eventA5, eventA5H1 and eventA5H2) or in the aN-Offset (for eventA3, eventA3H1, eventA3H2 and eventA6) or in the x1-Threshold2 (for eventX1) as the sorting quantity;

2>	for an E-UTRA cell, consider the quantity used in the bN-ThresholdEUTRA as the sorting quantity;

2>	for an UTRA-FDD cell, consider the quantity used in the bN-ThresholdUTRA-FDD as the sorting quantity;

2>	for a candidate L2 U2N Relay UE, consider the y1-Threshold2-Relay (for eventY1-Relay) or y2-Threshold-Relay (for eventY2-Relay) or z1-Threshold2-Relay (for eventZ1) as the sorting quantity;

1>	if the reportType is set to periodical:

2>	determine the sorting quantity according to reportQuantityCell for an NR cell, and according to reportQuantity for an E-UTRA cell, as below:

3>	if a single quantity is set to true:

4>	consider this quantity as the sorting quantity;

3>	else:

4>	if rsrp is set to true;

5>	consider RSRP as the sorting quantity;

4>	else:

5>	consider RSRQ as the sorting quantity;

2>	determine the sorting quantity according to reportQuantityUTRA-FDD for UTRA-FDD cell, as below:

3>	if a single quantity is set to true:

4>	consider this quantity as the sorting quantity;

3>	else:

4>	consider RSCP as the sorting quantity.

2>	for a candidate L2 U2N Relay UE, consider the reportQuantityRelay as the sorting quantity;

Next Change

### 6.3.2	Radio resource control information elements

#### –	AdditionalSpectrumEmission

The IE AdditionalSpectrumEmission is used to indicate emission requirements to be fulfilled by the UE (see TS 38.101-1 [15], clause 6.2.3/6.2A.3, TS 38.101-2 [39], clause 6.2.3/6.2A.3, and TS 38.101-5 [75], clause 6.2.3). If an extension is signalled using the extended value range (as defined by the IE AdditionalSpectrumEmission-v1760), the corresponding original field, using the value range as defined by the IE AdditionalSpectrumEmission (without suffix) shall be set to value 7.

AdditionalSpectrumEmission information element

-- ASN1START

-- TAG-ADDITIONALSPECTRUMEMISSION-START

AdditionalSpectrumEmission ::=              INTEGER (0..7)

AdditionalSpectrumEmission-v1760 ::=        INTEGER (8..39)

AdditionalSpectrumEmission-r18 ::=          INTEGER (0..39)

-- TAG-ADDITIONALSPECTRUMEMISSION-STOP

-- ASN1STOP

#### –	AdvancedReceiver-MU-MIMO

The IE AdvancedReceiver-MU-MIMO is used to provide a set of assistance information for R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions.

AdvancedReceiver-MU-MIMO information element

-- ASN1START

-- TAG-ADVANCEDRECEIVER-MU-MIMO-START

AdvancedReceiver-MU-MIMO-r18 ::=      SEQUENCE {

precodingAndResourceAllocation-r18    BOOLEAN                                                OPTIONAL,    -- Need M

dmrsPowerBoosting-r18                 BOOLEAN                                                OPTIONAL,    -- Need M

pdsch-TimeDomainAllocation-r18        BOOLEAN                                                OPTIONAL,    -- Need M

mcs-Table-r18                         ENUMERATED {qam1024, qam256, qam64, spare1}            OPTIONAL,    -- Need R

advReceiver-MU-MIMO-DCI-1-1-r18       ENUMERATED {enabled}                                   OPTIONAL,    -- Need R

...

}

-- TAG-ADVANCEDRECEIVER-MU-MIMO-STOP

-- ASN1STOP

Editor's note: whether the IE type BOOLEAN within AdvancedReceiver-MU-MIMO-r18 needs to be changed to ENUMERATED {true} depends on further RAN4 clarification.

Editor's note: whether dmrsPowerBoosting-r18 needs to be removed depends on further RAN4 clarification.

AdvancedReceiver-MU-MIMO field descriptions

advReceiver-MU-MIMO-DCI-1-1 Configure the presence of the co-scheduled UE information field in DCI format 1_1 (see TS 38.212 [17], clause 7.3.1.2.2).

dmrsPowerBoosting If the field is set to true, the UE can assume the DMRS power boosting configurations (i.e., Number of DMRS CDM groups without data, see TS 38.214 [19], table 4.1-1) of all the co-scheduled UE(s), which has the same DM-RS sequence as the target UE, is the same as the target UE. If the field is set to false, the UE can not assume the DMRS power boosting configurations (i.e., Number of DMRS CDM groups without data, see TS 38.214 [19], table 4.1-1) of all the co-scheduled UE(s), which has the same DM-RS sequence as the target UE, is the same as the target UE.

mcs-Table Indicates the MCS table with the highest modulation order among all MCS tables configured to the co-scheduled UE(s), which has the same DMRS sequence as the target UE.

pdsch-TimeDomainAllocation If the field is set to true, the UE can assume the time domain resource assignment for PDSCH symbols of all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, is same as the target UE. If the field is set to false, the UE can not assume the time domain resource assignment for PDSCH symbols of all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, is same as the target UE.

precodingAndResourceAllocation If the field is set to true, the UE can assume the precoding and resource allocation of the co-scheduled UE are the same in the PRG-level grid configured to the target UE when PRG=2 or 4, when the target UE and any co-scheduled UEs are in different CDM groups and with the same DMRS sequence. If the field is set to false, the UE can not assume the precoding or resource allocation of the co-scheduled UE is the same in the PRG-level grid configured to the target UE when PRG=2 or 4, when the target UE and any co-scheduled UEs are in different CDM groups and with the same DMRS sequence.

NOTE:	the same DMRS sequence represents the same root DMRS sequence r(n), see TS 38.211 [16], clause 7.4.1.1.1.

#### –	Aerial-Config

The IE Aerial-Config provides configuration parameters for aerial UE.

Aerial-Config information element

-- ASN1START

-- TAG-AERIAL-CONFIG-START

Aerial-Config-r18 ::= SEQUENCE {

flightPathUpdateThrConfig-r18     SEQUENCE {

flightPathUpdateDistanceThr-r18   SetupRelease { FlightPathUpdateDistanceThr-r18 }        OPTIONAL, -- Need M

flightPathUpdateTimeThr-r18       SetupRelease { FlightPathUpdateTimeThr-r18 }            OPTIONAL  -- Need M

}                                                                                             OPTIONAL, -- Need M

...

}

FlightPathUpdateDistanceThr-r18 ::=   INTEGER (0..1023)

FlightPathUpdateTimeThr-r18 ::=       INTEGER (0..16383)

-- TAG-AERIAL-CONFIG-STOP

-- ASN1STOP

Aerial-Config field descriptions

flightPathUpdateDistanceThr Distance threshold for triggering flight path update indication. Actual value is 5 x field value in meters.

flightPathUpdateTimeThr Time threshold for triggering flight path update indication. Value in seconds.

#### –	Alpha

The IE Alpha defines possible values of a the pathloss compensation coefficient for uplink power control. Value alpha0 corresponds to the value 0, Value alpha04 corresponds to the value 0.4, Value alpha05 corresponds to the value 0.5 and so on. Value alpha1 corresponds to value 1. See also clause 7.1 of TS 38.213 [13].

-- ASN1START

-- TAG-ALPHA-START

Alpha ::=                       ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1}

-- TAG-ALPHA-STOP

-- ASN1STOP

#### –	Altitude

The IE Altitude is used to indicate altitude relative to sea level. The actual value is the field value in meters.

Altitude information element

-- ASN1START

-- TAG-ALTITUDE-START

Altitude-r18 ::=              INTEGER (minAltitude-r18..maxAltitude-r18)

-- TAG-ALTITUDE-STOP

-- ASN1STOP

#### –	AMF-Identifier

The IE AMF-Identifier (AMFI) comprises of an AMF Region ID, an AMF Set ID and an AMF Pointer as specified in TS 23.003 [21], clause 2.10.1.

AMF-Identifier information element

-- ASN1START

-- TAG-AMF-IDENTIFIER-START

AMF-Identifier ::=                      BIT STRING (SIZE (24))

-- TAG-AMF-IDENTIFIER-STOP

-- ASN1STOP

#### –	ARFCN-ValueEUTRA

The IE ARFCN-ValueEUTRA is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) E-UTRA carrier frequency, as defined in TS 36.101 [22].

ARFCN-ValueEUTRA information element

-- ASN1START

-- TAG-ARFCN-VALUEEUTRA-START

ARFCN-ValueEUTRA ::=                INTEGER (0..maxEARFCN)

-- TAG-ARFCN-VALUEEUTRA-STOP

-- ASN1STOP

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-1 [15], TS 38.101-2 [39] and TS 38.101-5 [75], clause 5.4.2.

-- ASN1START

-- TAG-ARFCN-VALUENR-START

ARFCN-ValueNR ::=               INTEGER (0..maxNARFCN)

-- TAG-ARFCN-VALUENR-STOP

-- ASN1STOP

#### –	ARFCN-ValueUTRA-FDD

The IE ARFCN-ValueUTRA-FDD is used to indicate the ARFCN applicable for a downlink (Nd, FDD) UTRA-FDD carrier frequency, as defined in TS 25.331 [45].

ARFCN-ValueUTRA-FDD information element

-- ASN1START

-- TAG-ARFCN-ValueUTRA-FDD-START

ARFCN-ValueUTRA-FDD-r16 ::=                INTEGER (0..16383)

-- TAG-ARFCN-ValueUTRA-FDD-STOP

-- ASN1STOP

#### –	ATG-Config

The IE ATG-Config provides parameters needed for the UE to access NR via ATG access.

ATG-Config information element

-- ASN1START

-- TAG-ATG-CONFIG-START

ATG-Config-r18 ::=          SEQUENCE {

atg-gNB-Location-r18         ReferenceLocation-r17                                 OPTIONAL,    -- Need R

heightgNB-r18                INTEGER (-16384..16383)                               OPTIONAL,    -- Need R

cellSpecificKoffset-r18      INTEGER(1..3)                                         OPTIONAL,    -- Need R

ta-ReportATG-r18             ENUMERATED {enabled}                                  OPTIONAL     -- Need R

}

-- TAG-ATG-CONFIG-STOP

-- ASN1STOP

ATG-Config field descriptions

atg-gNB-Location Indicates the BS location for ATG access.

cellSpecificKoffset Scheduling offset used for the timing relationships that are modified for ATG. Unit in slot.

heightgNB Indicates the height of the BS for ATG access relative to the sea level. Unit in meter.

ta-ReportATG When this field is included in SIB22, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

#### –	AvailabilityCombinationsPerCell

The IE AvailabilityCombinationsPerCell is used to configure the AvailabilityCombinations applicable for a cell of the IAB DU (see TS 38.213 [13], clause 14). Note that the IE AvailabilityCombinationsPerCellIndex can only be configured up to 511.

AvailabilityCombinationsPerCell information element

-- ASN1START

-- TAG-AVAILABILITYCOMBINATIONSPERCELL-START

AvailabilityCombinationsPerCell-r16 ::=     SEQUENCE {

availabilityCombinationsPerCellIndex-r16     AvailabilityCombinationsPerCellIndex-r16,

iab-DU-CellIdentity-r16                      CellIdentity,

positionInDCI-AI-r16                         INTEGER(0..maxAI-DCI-PayloadSize-1-r16)                              OPTIONAL, -- Need M

availabilityCombinations-r16                 SEQUENCE (SIZE (1..maxNrofAvailabilityCombinationsPerSet-r16)) OF AvailabilityCombination-r16,

...,

[[

availabilityCombinationsRB-Groups-r17   SEQUENCE (SIZE (1..maxNrofAvailabilityCombinationsPerSet-r16)) OF AvailabilityCombinationRB-Groups-r17    OPTIONAL -- Need M

]],

[[

positionInDCI-AI-RBGroups-v1720              INTEGER(0..maxAI-DCI-PayloadSize-1-r16)                              OPTIONAL -- Need M

]]

}

AvailabilityCombinationsPerCellIndex-r16 ::= INTEGER(0..maxNrofDUCells-r16)

AvailabilityCombination-r16 ::=         SEQUENCE {

availabilityCombinationId-r16           AvailabilityCombinationId-r16,

resourceAvailability-r16                SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7)

}

AvailabilityCombinationId-r16 ::=       INTEGER (0..maxNrofAvailabilityCombinationsPerSet-1-r16)

AvailabilityCombinationRB-Groups-r17 ::= SEQUENCE {

availabilityCombinationId-r17    AvailabilityCombinationId-r16,

rb-SetGroups-r17                  SEQUENCE (SIZE (1..maxNrofRB-SetGroups-r17)) OF RB-SetGroup-r17                           OPTIONAL, -- Need R

resourceAvailability-r17         SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7)    OPTIONAL -- Need R

}

RB-SetGroup-r17 ::=       SEQUENCE {

resourceAvailability-r17  SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7) OPTIONAL, -- Need R

rb-Sets-r17                SEQUENCE (SIZE (1..maxNrofRB-Sets-r17)) OF INTEGER (0..7)                             OPTIONAL  -- Need R

}

-- TAG-AVAILABILITYCOMBINATIONSPERCELL-STOP

-- ASN1STOP

AvailabilityCombination field descriptions

availabilityCombinationId This ID is used in the DCI Format 2_5 payload to dynamically select this AvailabilityCombination, see TS 38.213 [13], clause 14.

resourceAvailability Indicates the resource availability of soft symbols for a set of consecutive slots in the time domain. The meaning of this field is described in TS 38.213 [13], Table 14.3. If included in RB-SetGroup within AvailabilityCombinationRB-Groups-r17, it indicates the availability of soft resources for an RB set group. If included in AvailabilityCombinationRB-Groups-r17 when the rb-SetGroups is not configured, it indicates the availability of soft resources in one or multiple slots for all RB sets of a DU cell.

AvailabilityCombinationsPerCell field descriptions

iab-DU-CellIdentity The ID of the IAB-DU cell for which the availabilityCombinations are applicable.

positionInDCI-AI The (starting) position (bit) of the availabilityCombinationId for the indicated IAB-DU cell (iab-DU-CellIdentity) within the DCI payload. If positionInDCI-AI-RBGroups is not configured, it applies to the availabilityCombinationId included in availabilityCombinations and in availabilityCombinationsRB-Groups. If positionInDCI-AI-RBGroups is configured, it applies to the availabilityCombinationId included in availabilityCombinations.

positionInDCI-AI-RBGroups The (starting) position (bit) of the availabilityCombinationId associated to the availabilityCombinationsRB-Groups for the indicated IAB-DU cell (iab-DU-CellIdentity) within the DCI payload.

AvailabilityCombinationRB-Groups field descriptions

rb-SetGroups Indicates the RB set groups configured for the availability combination. Each group includes consecutive RB sets.

rb-Sets Indicates the one or more RB set indexes associated to one or more RB sets configured for one RB set group.

#### –	AvailabilityIndicator

The IE AvailabilityIndicator is used to configure monitoring a PDCCH for Availability Indicators (AI).

AvailabilityIndicator information element

-- ASN1START

-- TAG-AVAILABILITYINDICATOR-START

AvailabilityIndicator-r16 ::=    SEQUENCE {

ai-RNTI-r16                      AI-RNTI-r16,

dci-PayloadSizeAI-r16            INTEGER (1..maxAI-DCI-PayloadSize-r16),

availableCombToAddModList-r16    SEQUENCE (SIZE(1..maxNrofDUCells-r16)) OF AvailabilityCombinationsPerCell-r16          OPTIONAL, -- Need N

availableCombToReleaseList-r16   SEQUENCE (SIZE(1..maxNrofDUCells-r16)) OF AvailabilityCombinationsPerCellIndex-r16     OPTIONAL, -- Need N

...

}

AI-RNTI-r16 ::=                      RNTI-Value

-- TAG-AVAILABILITYINDICATOR-STOP

-- ASN1STOP

AvailabilityIndicator field descriptions

ai-RNTI Used by an IAB-MT for detection of DCI format 2_5 indicating AvailabilityCombinationId for an IAB-DU's cells.

availableCombToAddModList A list of availabilityCombinations to add for the IAB-DU's cells. (see TS 38.213 [13], clause 14).

availableCombToReleaseList A list of availabilityCombinations to release for the IAB-DU's cells. (see TS 38.213 [13], clause 14).

dci-PayloadSizeAI Total length of the DCI payload scrambled with ai-RNTI (see TS 38.213 [13]).

#### –	BAP-RoutingID

The IE BAP-RoutingID is used for IAB-node to configure the BAP Routing ID.

BAP-RoutingID information element

-- ASN1START

-- TAG-BAPROUTINGID-START

BAP-RoutingID-r16::=        SEQUENCE{

bap-Address-r16              BIT STRING (SIZE (10)),

bap-PathId-r16               BIT STRING (SIZE (10))

}

-- TAG-BAPROUTINGID-STOP

-- ASN1STOP

BAP-RoutingID field descriptions

bap-Address The ID of a destination IAB-node or IAB-donor-DU used in the BAP header.

bap-PathId The ID of a path used in the BAP header.

#### –	BeamFailureRecoveryConfig

The IE BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYCONFIG-START

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                                                          OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                                                        OPTIONAL, -- Need M

rsrp-ThresholdSSB                   RSRP-Range                                                                OPTIONAL, -- Need M

candidateBeamRSList                 SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR   OPTIONAL, -- Need M

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two,

four, eight, sixteen}                                      OPTIONAL, -- Need M

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                           OPTIONAL, -- Need M

recoverySearchSpaceId               SearchSpaceId                                                             OPTIONAL, -- Need R

ra-Prioritization                   RA-Prioritization                                                         OPTIONAL, -- Need R

beamFailureRecoveryTimer            ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}            OPTIONAL, -- Need M

...,

[[

msg1-SubcarrierSpacing              SubcarrierSpacing                                                         OPTIONAL  -- Need M

]],

[[

ra-PrioritizationTwoStep-r16        RA-Prioritization                                                         OPTIONAL, -- Need R

candidateBeamRSListExt-v1610        SetupRelease{ CandidateBeamRSListExt-r16 }                                OPTIONAL  -- Need M

]],

[[

spCell-BFR-CBRA-r16                 ENUMERATED {true}                                                         OPTIONAL  -- Need R

]]

}

PRACH-ResourceDedicatedBFR ::=      CHOICE {

ssb                                 BFR-SSB-Resource,

csi-RS                              BFR-CSIRS-Resource

}

BFR-SSB-Resource ::=                SEQUENCE {

ssb                                 SSB-Index,

ra-PreambleIndex                    INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=              SEQUENCE {

csi-RS                              NZP-CSI-RS-ResourceId,

ra-OccasionList                     SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)   OPTIONAL,   -- Need R

ra-PreambleIndex                    INTEGER (0..63)                                                                 OPTIONAL,   -- Need R

...

}

CandidateBeamRSListExt-r16::=       SEQUENCE (SIZE(1.. maxNrofCandidateBeamsExt-r16)) OF PRACH-ResourceDedicatedBFR

-- TAG-BEAMFAILURERECOVERYCONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryConfig field descriptions

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

candidateBeamRSList, candidateBeamRSListExt-v1610 Set of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters. This set includes all elements of candidateBeamRSList (without suffix) and all elements of candidateBeamRSListExt-v1610. The UE maintains candidateBeamRSList and candidateBeamRSListExt-v1610 separately: Receiving candidateBeamRSListExt-v1610 set to release releases only the entries that were configured by candidateBeamRSListExt-v1610, and receiving candidateBeamRSListExt-v1610 set to setup replaces only the entries that were configured by candidateBeamRSListExt-v1610 with the newly signalled entries. The network configures these reference signals to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided.

msg1-SubcarrierSpacing Subcarrier spacing for contention free beam failure recovery (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free random access to recover from beam failure (see TS 38.213 [13], clause 6).

ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access procedure for BFR (see TS 38.321 [3], clause 5.1.1).

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources.

rach-ConfigBFR Configuration of random access parameters for BFR.

recoverySearchSpaceId Search space to use for BFR RAR. The network configures this search space to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided. The CORESET associated with the recovery search space cannot be associated with another search space. Network always configures the UE with a value for this field when contention free random access resources for BFR are configured.

rootSequenceIndex-BFR PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1) for beam failure recovery.

spCell-BFR-CBRA Indicates that UE is configured to send MAC CE for SpCell BFR as specified in TS38.321 [3].

ssb-perRACH-Occasion Number of SSBs per RACH occasion for CF-BFR, see TS 38.213 [13], clause 8.1.

BFR-CSIRS-Resource field descriptions

csi-RS The ID of a NZP-CSI-RS-Resource configured in the CSI-MeasConfig of this serving cell. This reference signal determines a candidate beam for beam failure recovery (BFR).

ra-OccasionList RA occasions that the UE shall use when performing BFR upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots. If the field is absent the UE uses the RA occasion associated with the SSB that is QCLed with this CSI-RS.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS. If the field is absent, the UE uses the preamble index associated with the SSB that is QCLed with this CSI-RS.

BFR-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing BFR upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell. It determines a candidate beam for beam failure recovery (BFR).

#### –	BeamFailureRecoveryRSConfig

The IE BeamFailureRecoveryRSConfig is used to configure the UE with candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.17.

BeamFailureRecoveryRSConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYRSCONFIG-START

BeamFailureRecoveryRSConfig-r16 ::= SEQUENCE {

rsrp-ThresholdBFR-r16               RSRP-Range                                                               OPTIONAL, -- Need M

candidateBeamRS-List-r16            SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16     OPTIONAL, -- Need M

...,

[[

candidateBeamRS-List2-r17            SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16     OPTIONAL  -- Need R

]]

}

-- TAG-BEAMFAILURERECOVERYRSCONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryRSConfig field descriptions

candidateBeamRS-List A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery. The network always configures this parameter in every instance of this IE.

candidateBeamRS-List2 A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery.

rsrp-ThresholdBFR L1-RSRP threshold used for determining whether a candidate beam may be included by the UE in MAC CE for BFR (see TS 38.321 [3] and TS 38.213 [13], clause 6). The network always configures this parameter in every instance of this IE.

#### –	BetaOffsets

The IE BetaOffsets is used to configure beta-offset values, see TS 38.213 [13], clause 9.3.

BetaOffsets information element

-- ASN1START

-- TAG-BETAOFFSETS-START

BetaOffsets ::=                     SEQUENCE {

betaOffsetACK-Index1                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetACK-Index2                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetACK-Index3                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2          INTEGER(0..31)                                                          OPTIONAL  -- Need S

}

-- TAG-BETAOFFSETS-STOP

-- ASN1STOP

BetaOffsets field descriptions

betaOffsetACK-Index1 Up to 2 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index2 Up to 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index3 Above 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetCSI-Part1-Index1 Up to 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part1-Index2 Above 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index1 Up to 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index2 Above 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

#### –	BetaOffsetsCrossPri

The IE BetaOffsetsCrossPri is used to configure beta-offset values for cross-priority HARQ-ACK multiplexing on PUSCH.

BetaOffsetsCrossPri information element

-- ASN1START

-- TAG-BETAOFFSETSCROSSPRI-START

BetaOffsetsCrossPri-r17 ::= SEQUENCE (SIZE(3)) OF INTEGER(0..31)

-- TAG-BETAOFFSETSCROSSPRI-STOP

-- ASN1STOP

#### –	BH-LogicalChannelIdentity

The IE BH-LogicalChannelIdentity is used to identify a logical channel between an IAB-node and its parent IAB-node or IAB-donor-DU.

BH-LogicalChannelIdentity information element

-- ASN1START

-- TAG-BHLOGICALCHANNELIDENTITY-START

BH-LogicalChannelIdentity-r16 ::=    CHOICE {

bh-LogicalChannelIdentity-r16        LogicalChannelIdentity,

bh-LogicalChannelIdentityExt-r16     BH-LogicalChannelIdentity-Ext-r16

}

-- TAG-BHLOGICALCHANNELIDENTITY-STOP

-- ASN1STOP

BH-LogicalChannelIdentity field descriptions

bh-LogicalChannelIdentity ID used for the MAC logical channel.

bh-LogicalChannelIdentityExt ID used for the MAC logical channel.

#### –	BH-LogicalChannelIdentity-Ext

The IE BH-LogicalChannelIdentity-Ext is used to identify a logical channel between an IAB-node and its parent node.

BH-LogicalChannelIdentity-Ext information element

-- ASN1START

-- TAG-BHLOGICALCHANNELIDENTITYEXT-START

BH-LogicalChannelIdentity-Ext-r16 ::=   INTEGER (320.. maxLC-ID-Iab-r16)

-- TAG-BHLOGICALCHANNELIDENTITYEXT-STOP

-- ASN1STOP

#### –	BH-RLC-ChannelConfig

The IE BH-RLC-ChannelConfig is used to configure an RLC entity, a corresponding logical channel in MAC for BH RLC channel between IAB-node and its parent node.

BH-RLC-ChannelConfig information element

-- ASN1START

-- TAG-BHRLCCHANNELCONFIG-START

BH-RLC-ChannelConfig-r16::=      SEQUENCE {

bh-LogicalChannelIdentity-r16    BH-LogicalChannelIdentity-r16     OPTIONAL,   -- Cond LCH-SetupOnly

bh-RLC-ChannelID-r16             BH-RLC-ChannelID-r16,

reestablishRLC-r16               ENUMERATED {true}                 OPTIONAL,   -- Need N

rlc-Config-r16                   RLC-Config                        OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig-r16     LogicalChannelConfig              OPTIONAL,   -- Cond LCH-Setup

...

}

-- TAG-BHRLCCHANNELCONFIG-STOP

-- ASN1STOP

BH-RLC-ChannelConfig field descriptions

bh-LogicalChannelIdentity Indicates the logical channel id for BH RLC channel of the IAB-node.

bh-RLC-ChannelID Indicates the BH RLC channel in the link between IAB-MT of the IAB-node and IAB-DU of the parent IAB-node or IAB-donor-DU.

reestablishRLC Indicates that RLC should be re-established.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present upon creation of a new logical channel for a BH RLC channel. It is optionally present, Need M, otherwise. |
| LCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a BH RLC channel. It is absent, Need M otherwise. |

#### –	BH-RLC-ChannelID

The IE BH-RLC-ChannelID is used to identify a BH RLC channel in the link between IAB-MT of the IAB-node and IAB-DU of the parent IAB-node or IAB-donor-DU.

BH-RLC-ChannelID information element

-- ASN1START

-- TAG-BHRLCCHANNELID-START

BH-RLC-ChannelID-r16 ::=    BIT STRING (SIZE (16))

-- TAG-BHRLCCHANNELID-STOP

-- ASN1STOP

#### –	BSR-Config

The IE BSR-Config is used to configure buffer status reporting.

BSR-Config information element

-- ASN1START

-- TAG-BSR-CONFIG-START

BSR-Config ::=                      SEQUENCE {

periodicBSR-Timer                   ENUMERATED { sf1, sf5, sf10, sf16, sf20, sf32, sf40, sf64,

sf80, sf128, sf160, sf320, sf640, sf1280, sf2560, infinity },

retxBSR-Timer                       ENUMERATED { sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, sf2560,

sf5120, sf10240, spare5, spare4, spare3, spare2, spare1},

logicalChannelSR-DelayTimer         ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}                OPTIONAL, -- Need R

...

}

-- TAG-BSR-CONFIG-STOP

-- ASN1STOP

BSR-Config field descriptions

logicalChannelSR-DelayTimer Value in number of subframes. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on.

periodicBSR-Timer Value in number of subframes. Value sf1 corresponds to 1 subframe, value sf5 corresponds to 5 subframes and so on.

retxBSR-Timer Value in number of subframes. Value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes and so on.

#### –	BWP

The IE BWP is used to configure generic parameters of a bandwidth part as defined in TS 38.211 [16], clause 4.5, and TS 38.213 [13], clause 12.

For each serving cell the network configures at least an initial downlink bandwidth part and one (if the serving cell is configured with an uplink) or two (if using supplementary uplink (SUL)) initial uplink bandwidth parts. Furthermore, the network may configure additional uplink and downlink bandwidth parts for a serving cell.

The uplink and downlink bandwidth part configurations are divided into common and dedicated parameters.

BWP information element

-- ASN1START

-- TAG-BWP-START

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }                                                 OPTIONAL    -- Need R

}

-- TAG-BWP-STOP

-- ASN1STOP

BWP field descriptions

cyclicPrefix Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. Normal CP is supported for all subcarrier spacings and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing. (see TS 38.211 [16], clause 4.2). Except for SUL, the network ensures the same cyclic prefix length is used in active DL BWP and active UL BWP within a serving cell.

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL / FrequencyInfoUL-SIB / FrequencyInfoDL-SIB within ServingCellConfigCommon / ServingCellConfigCommonSIB) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz For the initial DL BWP and operation in licensed spectrum this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell. Except for SUL, the network ensures the same subcarrier spacing is used in active DL BWP and active UL BWP within a serving cell. For the initial DL BWP and operation with shared spectrum channel access, the value of this field corresponds to the subcarrier spacing of the SSB associated to the initial DL BWP.

#### –	BWP-Downlink

The IE BWP-Downlink is used to configure an additional downlink bandwidth part (not for the initial BWP).

BWP-Downlink information element

-- ASN1START

-- TAG-BWP-DOWNLINK-START

BWP-Downlink ::=                    SEQUENCE {

bwp-Id                              BWP-Id,

bwp-Common                          BWP-DownlinkCommon                                         OPTIONAL,   -- Cond SetupOtherBWP

bwp-Dedicated                       BWP-DownlinkDedicated                                      OPTIONAL,   -- Cond SetupOtherBWP

...

}

-- TAG-BWP-DOWNLINK-STOP

-- ASN1STOP

BWP-Downlink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The network configures the BWPs with consecutive IDs from 1. The Network does not include the value 0, since value 0 is reserved for the initial BWP.

| Conditional Presence | Explanation |
| --- | --- |
| SetupOtherBWP | The field is mandatory present upon configuration of a new DL BWP. The field is optionally present, Need M, otherwise. |

#### –	BWP-DownlinkCommon

The IE BWP-DownlinkCommon is used to configure the common parameters of a downlink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-DownlinkCommon information element

-- ASN1START

-- TAG-BWP-DOWNLINKCOMMON-START

BWP-DownlinkCommon ::=              SEQUENCE {

genericParameters                   BWP,

pdcch-ConfigCommon                  SetupRelease { PDCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pdsch-ConfigCommon                  SetupRelease { PDSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...

}

-- TAG-BWP-DOWNLINKCOMMON-STOP

-- ASN1STOP

BWP-DownlinkCommon field descriptions

pdcch-ConfigCommon Cell specific parameters for the PDCCH of this BWP. This field is absent for a dormant BWP.

pdsch-ConfigCommon Cell specific parameters for the PDSCH of this BWP.

#### –	BWP-DownlinkDedicated

The IE BWP-DownlinkDedicated is used to configure the dedicated (UE specific) parameters of a downlink BWP.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                                     OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                                       OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }                        OPTIONAL,   -- Need M

...,

[[

sps-ConfigToAddModList-r16          SPS-ConfigToAddModList-r16                                        OPTIONAL,   -- Need N

sps-ConfigToReleaseList-r16         SPS-ConfigToReleaseList-r16                                       OPTIONAL,   -- Need N

sps-ConfigDeactivationStateList-r16 SPS-ConfigDeactivationStateList-r16                               OPTIONAL,   -- Need R

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoveryRSConfig-r16}                    OPTIONAL,   -- Cond SCellOnly

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }                                     OPTIONAL    -- Need M

]],

[[

preConfGapStatus-r17                BIT STRING (SIZE (maxNrofGapId-r17))                              OPTIONAL,   -- Cond PreConfigMG

beamFailureRecoverySpCellConfig-r17 SetupRelease { BeamFailureRecoveryRSConfig-r16}                   OPTIONAL,   -- Cond SpCellOnly

harq-FeedbackEnablingforSPSactive-r17 BOOLEAN                                                         OPTIONAL,   -- Need R

cfr-ConfigMulticast-r17             SetupRelease { CFR-ConfigMulticast-r17 }                          OPTIONAL,   -- Need M

dl-PPW-PreConfigToAddModList-r17    DL-PPW-PreConfigToAddModList-r17                                  OPTIONAL,   -- Need N

dl-PPW-PreConfigToReleaseList-r17   DL-PPW-PreConfigToReleaseList-r17                                 OPTIONAL,   -- Need N

nonCellDefiningSSB-r17              NonCellDefiningSSB-r17                                            OPTIONAL,   -- Need R

servingCellMO-r17                   MeasObjectId                                                  OPTIONAL -- Cond MeasObject-NCD-SSB

]],

[[

tci-inDCI-18                        SetupRelease {TCI-inDCI-r18}                                      OPTIONAL    -- Need M

]]

}

SPS-ConfigToAddModList-r16 ::=          SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config

SPS-ConfigToReleaseList-r16 ::=         SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationState-r16 ::=     SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationStateList-r16 ::= SEQUENCE (SIZE (1..maxNrofSPS-DeactivationState)) OF SPS-ConfigDeactivationState-r16

DL-PPW-PreConfigToAddModList-r17 ::=    SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-PreConfig-r17

DL-PPW-PreConfigToReleaseList-r17 ::=   SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-ID-r17

TCI-inDCI-r18 ::=                   SEQUENCE {

tci-SelectionPresentInDCI-r18       ENUMERATED { enabled }                                            OPTIONAL,   -- Need R

applyIndicatedTCI-StateDCI-1-0-r18  ENUMERATED {first, second, both}                                  OPTIONAL    -- Need R

}

-- TAG-BWP-DOWNLINKDEDICATED-STOP

-- ASN1STOP

BWP-DownlinkDedicated field descriptions

applyIndicatedTCI-StateDCI-1-0 This field indicates, for PDSCH reception scheduled or activated by DCI format 1_0, if UE applies the first, the second or both "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5. Only when the UE is configured with PDSCH-CJT(cjt-Scheme-PDSCH in IE ServingCellConfig) and the UE supports two joint TCI states for PDSCH-CJT or the UE is configured with PDSCH-SFN(sfnSchemePDSCH in IE ServingCellConfig), the RRC configuration can indicate both indicated joint/DL TCI states are applied. This field is absent if more than one value for the field coresetPoolIndex is configured in controlResourceSet for the same DL BWP.

beamFailureRecoverySCellConfig Configuration of candidate RS for beam failure recovery on SCells.

beamFailureRecoverySpCellConfig Configuration of candidate RS for beam failure recovery on the SpCell. This field can only be configured when beamFailure-r17 is configured in RadioLinkMonitoringConfig.

cfr-ConfigMulticast UE specific common frequency resource configuration for MBS multicast for one dedicated BWP. This field can be configured within at most one serving cell.

dl-PPW-PreConfigToAddModList Indicates a list of DL-PRS processing window configurations to be added or modified for the dedicated DL BWP.

dl-PPW-PreConfigToReleaseList Indicates a list of DL-PRS processing window configurations to be released for the dedicated DL BWP.

harq-FeedbackEnablingforSPSactive If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled for the HARQ process corresponding to the first SPS PDSCH after activation. Otherwise, UE follows configuration of HARQ feedback enabled/disabled for the HARQ process corresponding to the first SPS PDSCH after activation.

nonCellDefiningSSB If configured, the UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM, BFD, beam management). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitly to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE. In the MIB associated with this NCD-SSB, the systemFrameNumber field indicates the frame boundary and frame number of the NCD-SSB. The subCarrierSpacingCommon and dmrs-TypeA-Position field in the MIBs associated with CD-SSB and NCD-SSB in the same cell are configured with the same values, respectively.

pdcch-Config UE specific PDCCH configuration for one BWP.

pdsch-Config UE specific PDSCH configuration for one BWP.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated upon the switch to this BWP. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and nonCellDefiningSSB in BWP-DownlinkDedicated of the associated downlink BWP: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB in the nonCellDefiningSSB. If the field is present in a downlink BWP and the BWP is activated, the UE uses this measurement object for serving cell measurements (e.g., including those used in measurement report triggering events), otherwise, the UE uses the servingCellMO in ServingCellConfig IE.

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList. Network does not configure SPS in one BWP using this field and sps-ConfigMulticastToAddModList-r17 simultaneously.

sps-ConfigDeactivationStateList Indicates a list of the deactivation states in which each state can be mapped to a single or multiple SPS configurations to be deactivated, see clause 10.2 in TS 38.213 [13]. If a state is mapped to multiple SPS configurations, each of these SPS configurations is configured with the same harq-CodebookID.

sps-ConfigToAddModList Indicates a list of one or more DL SPS configurations to be added or modified in one BWP. Except for reconfiguration with sync, the NW does not reconfigure a SPS configuration when it is active (see TS 38.321 [3]).

sps-ConfigToReleaseList Indicates a list of one or more DL SPS configurations to be released. The NW may release a SPS configuration at any time.

radioLinkMonitoringConfig UE specific configuration of radio link monitoring for detecting cell- and beam radio link failure occasions. The maximum number of failure detection resources should be limited up to 8 for both cell and beam radio link failure detection. For SCells, only periodic 1-port CSI-RS can be configured in IE RadioLinkMonitoringConfig.

sl-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving the SL grants (via SL-RNTI or SL-CS-RNTI) for NR sidelink communication/discovery.

sl-V2X-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving SL grants (i.e. sidelink SPS) for V2X sidelink communication.

tci-SelectionPresentInDCI Indicates if a [TCI selection field] is present or absent in DCI format 1_1 and DCI format 1_2 for a DL BWP, see TS 38.214 [19] clause 5.1.5 and TS 38.212 [17] clauses 7.3.1.2.2 and 7.3.1.2.3.

| Conditional Presence | Explanation |
| --- | --- |
| MeasObject-NCD-SSB | This field is optionally present, Need S, if nonCellDefiningSSB is configured in this DL BWP. It is absent otherwise. |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the BWP belongs to and configured with preConfigInd. It is absent, Need R, otherwise. |
| ScellOnly | The field is optionally present, Need M, in the BWP-DownlinkDedicated of an Scell. It is absent otherwise. |
| SpCellOnly | The field is optionally present, Need M, in the BWP-DownlinkDedicated of an Spcell. It is absent otherwise. |

#### –	BWP-Id

The IE BWP-Id is used to refer to Bandwidth Parts (BWP). The initial BWP (including RedCap-specific initial BWP, if configured) is referred to by BWP-Id 0. The other BWPs are referred to by BWP-Id 1 to maxNrofBWPs.

BWP-Id information element

-- ASN1START

-- TAG-BWP-ID-START

BWP-Id ::=                          INTEGER (0..maxNrofBWPs)

-- TAG-BWP-ID-STOP

-- ASN1STOP

#### –	BWP-Uplink

The IE BWP-Uplink is used to configure an additional uplink bandwidth part (not for the initial BWP).

BWP-Uplink information element

-- ASN1START

-- TAG-BWP-UPLINK-START

BWP-Uplink ::=                      SEQUENCE {

bwp-Id                              BWP-Id,

bwp-Common                          BWP-UplinkCommon                                            OPTIONAL,   -- Cond SetupOtherBWP

bwp-Dedicated                       BWP-UplinkDedicated                                         OPTIONAL,   -- Cond SetupOtherBWP

...

}

-- TAG-BWP-UPLINK-STOP

-- ASN1STOP

BWP-Uplink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The network configures the BWPs with consecutive IDs from 1. The Network does not include the value 0, since value 0 is reserved for the initial BWP.

| Conditional Presence | Explanation |
| --- | --- |
| SetupOtherBWP | The field is mandatory present upon configuration of a new UL BWP. The field is optionally present, Need M, otherwise. |

#### –	BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell, excluding additionalRACH-perPCI-ToAddModList and additionalRACH-perPCI-ToReleaseList, are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF  RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

additionalRACH-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF RACH-ConfigTwoTAIndex-r18

OPTIONAL,  -- Need N

rsrp-ThresholdMsg1-RepetitionNum2-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum4-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum8-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

preambleTransMax-Msg1-Repetition-r18     ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}      OPTIONAL   -- Cond Msg1Rep1

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

BWP-UplinkCommon field descriptions

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 32 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

additionalRACH-perPCI-ToAddModList List of RACH configurations for the additional PCIs. The RACH configuration for an additional PCI is applied for Random Access procedure initiated by PDCCH order towards to the additional PCI, as specified in TS 38.321 clause 5.1.1b. This list includes the same number of elements like additionalPCI-ToAddModList for this serving cell and the n-th element of this list is for the PCI in the n-th element of additionalPCI-ToAddModList. This configuration may be different for different UEs.

enableRA-PrioritizationForSlicing Indicates whether or not the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep should override the ra-PrioritizationForAccessIdentity. The field is applicable only when the UE is configured by upper layers with both NSAG and Access Identity 1 or 2. If value TRUE is configured, the UE should only apply the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep. If value FALSE is configured, the UE should only apply ra-PrioritizationForAccessIdentity. If the field is absent, whether to use ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep or ra-PrioritizationForAccessIdentity is up to UE implementation.

mcs-Msg3-Repetitions Configuration of eight candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. Only the first 4 configured or default MCS indexes are used for PUSCH transmission scheduled by RAR UL grant. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {0, 1, 2, 3, 4, 5, 6, 7} (see TS 38.214 [19], clause 6.1.4).

msgA-ConfigCommon Configuration of the cell specific PRACH and PUSCH resource parameters for transmission of MsgA in 2-step random access type procedure. The NW can configure msgA-ConfigCommon only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP or DL BWPs associated with nonCellDefiningSSB or, for (e)RedCap UEs, the RedCap-specific initial downlink BWP.

numberOfMsg3-RepetitionsList The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {n1, n2, n3, n4} (see TS 38.214 [19], clause 6.1.2.1).

preambleTransMax-Msg1-Repetition Max number of transmissions of MSG1 repetitions number (2, 4 and 8) performed before switching to higher repetition number (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when more than one repetition numbers are configured in shared RO. If the field is absent, switching from lower repetition number to higher repetition number is not allowed.

pucch-ConfigCommon Cell specific parameters for the PUCCH of this BWP.

pusch-ConfigCommon Cell specific parameters for the PUSCH of this BWP.

rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP or DL BWPs associated with nonCellDefiningSSB or, for (e)RedCap UEs, the RedCap-specific initial downlink BWP. The network configures rach-ConfigCommon (without suffix) and/or rach-ConfigCommon-r17, whenever it configures contention free random access (for reconfiguration with sync or for beam failure recovery), the UE then applies the corresponding configuration depending on the RACH resource set selected upon RACH initialization, as specified in TS 38.321 [3]. For RedCap-specific initial uplink BWP, rach-ConfigCommon is always configured when msgA-ConfigCommon is configured in this BWP.

rach-ConfigCommonIAB Configuration of cell specific random access parameters for the IAB-MT. The IAB specific IAB RACH configuration is used by IAB-MT, if configured.

rsrp-ThresholdMsg1-RepetitionNum2, rsrp-ThresholdMsg1-RepetitionNum4, rsrp-ThresholdMsg1-RepetitionNum8 Threshold used by the UE for determining whether to select resources indicating Msg1 repetition number 2, 4 or 8 in this BWP, as specified in TS 38.321 [3]. The value applies to all the BWPs and all RACH configurations. For a given MSG1 repetition number, this corresponding field is mandatory if both set(s) of Random Access resources with MSG1 repetition indication associated with this MSG1 repetition number and set(s) of Random Access resources without MSG1 repetition indication are configured in the BWP, or if the set(s) of Random Access resources with MSG1 repetition indication associated with this MSG1 repetition number and set(s) of Random Access resources with MSG1 repetition indication associated with a lower repetition number are configured in the BWP. It is absent otherwise.

rsrp-ThresholdMsg3 Threshold used by the UE for determining whether to select resources indicating Msg3 repetition in this BWP, as specified in TS 38.321 [3]. The field is mandatory if both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication are configured in the BWP. It is absent otherwise.

useInterlacePUCCH-PUSCH If the field is present, the UE uses uplink frequency domain resource allocation Type 2 for cell-specific PUSCH, e.g., PUSCH scheduled by RAR UL grant (see TS 38.213 [13] clause 8.3 and TS 38.214 [19], clause 6.1.2.2) and uses interlaced PUCCH Format 0 and 1 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Msg1Rep1 | This field is optionally present, Need R, if the set(s) of Random Access resources with MSG1 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise. |
| Msg3Rep | This field is optionally present, Need S, if the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise. |
| RA-PrioSliceAI | The field is optionally present in SIB1, Need R, if both parameters ra-PrioritizationForAccessIdentity and the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep are present in SIB1. It is absent otherwise. |
| SpCellOnly2 | The field is optionally present, Need M, in the BWP-UplinkCommon of an SpCell. It is absent otherwise. |
| 2TA-Only | The field is optionally present, Need N in the BWP-UplinkCommon if additionalPCI-ToAddModList is present in spCellConfigDedicated or sCellConfigDedicated and it has the same number of entries as the additionalPCI-ToAddModList. It is absent otherwise. |

#### –	BWP-UplinkDedicated

The IE BWP-UplinkDedicated is used to configure the dedicated (UE specific) parameters of an uplink BWP.

BWP-UplinkDedicated information element

-- ASN1START

-- TAG-BWP-UPLINKDEDICATED-START

BWP-UplinkDedicated ::=             SEQUENCE {

pucch-Config                        SetupRelease { PUCCH-Config }                                           OPTIONAL,   -- Need M

pusch-Config                        SetupRelease { PUSCH-Config }                                           OPTIONAL,   -- Need M

configuredGrantConfig               SetupRelease { ConfiguredGrantConfig }                                  OPTIONAL,   -- Need M

srs-Config                          SetupRelease { SRS-Config }                                             OPTIONAL,   -- Need M

beamFailureRecoveryConfig           SetupRelease { BeamFailureRecoveryConfig }                              OPTIONAL,   -- Cond SpCellOnly

...,

[[

sl-PUCCH-Config-r16                 SetupRelease { PUCCH-Config }                                           OPTIONAL,   -- Need M

cp-ExtensionC2-r16                  INTEGER (1..28)                                                         OPTIONAL,   -- Need R

cp-ExtensionC3-r16                  INTEGER (1..28)                                                         OPTIONAL,   -- Need R

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pucch-ConfigurationList-r16         SetupRelease { PUCCH-ConfigurationList-r16 }                            OPTIONAL,   -- Need M

lbt-FailureRecoveryConfig-r16       SetupRelease { LBT-FailureRecoveryConfig-r16 }                          OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r16                 ConfiguredGrantConfigToAddModList-r16                 OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r16                ConfiguredGrantConfigToReleaseList-r16                OPTIONAL,   -- Need N

configuredGrantConfigType2DeactivationStateList-r16   ConfiguredGrantConfigType2DeactivationStateList-r16   OPTIONAL    -- Need R

]],

[[

ul-TCI-StateList-r17                CHOICE {

explicitlist                        SEQUENCE {

ul-TCI-ToAddModList-r17             SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF TCI-UL-State-r17          OPTIONAL,   -- Need N

ul-TCI-ToReleaseList-r17            SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF TCI-UL-StateId-r17        OPTIONAL    -- Need N

},

unifiedTCI-StateRef-r17         ServingCellAndBWP-Id-r17

}                                                                                                           OPTIONAL,  -- Need R

ul-powerControl-r17                Uplink-powerControlId-r17                                                OPTIONAL,  -- Cond NoTCI-PC

pucch-ConfigurationListMulticast1-r17  SetupRelease { PUCCH-ConfigurationList-r16 }                         OPTIONAL,  -- Need M

pucch-ConfigurationListMulticast2-r17  SetupRelease { PUCCH-ConfigurationList-r16 }                         OPTIONAL   -- Need M

]],

[[

pucch-ConfigMulticast1-r17          SetupRelease { PUCCH-Config }                                           OPTIONAL,  -- Need M

pucch-ConfigMulticast2-r17          SetupRelease { PUCCH-Config }                                           OPTIONAL   -- Need M

]],

[[

pathlossReferenceRSToAddModList-r17     SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseList-r17    SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL  -- Need N

]]

}

ConfiguredGrantConfigToAddModList-r16    ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig

ConfiguredGrantConfigToReleaseList-r16   ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfigIndex-r16

ConfiguredGrantConfigType2DeactivationState-r16 ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfigIndex-r16

ConfiguredGrantConfigType2DeactivationStateList-r16  ::=

SEQUENCE (SIZE (1..maxNrofCG-Type2DeactivationState)) OF ConfiguredGrantConfigType2DeactivationState-r16

-- TAG-BWP-UPLINKDEDICATED-STOP

-- ASN1STOP

BWP-UplinkDedicated field descriptions

beamFailureRecoveryConfig Configuration of beam failure recovery. If supplementaryUplink is present, the field is present only in one of the uplink carriers, either UL or SUL.

configuredGrantConfig A Configured-Grant of type1 or type2. It may be configured for UL or SUL but in case of type1 not for both at a time. Except for reconfiguration with sync, the NW does not reconfigure configuredGrantConfig when there is an active configured uplink grant Type 2 (see TS 38.321 [3]). However, the NW may release the configuredGrantConfig at any time. Network can only configure configured grant in one BWP using either this field or configuredGrantConfigToAddModList.

configuredGrantConfigToAddModList Indicates a list of one or more configured grant configurations to be added or modified for one BWP. Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration when it is active (see TS 38.321 [3]). The network configures multiple CG configurations for one BWP with either all configurations or no configuration configured with cg-RetransmissionTimer-r16.

configuredGrantConfigToReleaseList Indicates a list of one or more UL Configured Grant configurations to be released. The NW may release a configured grant configuration at any time.

configuredGrantConfigType2DeactivationStateList Indicates a list of the deactivation states in which each state can be mapped to a single or multiple Configured Grant type 2 configurations to be deactivated when the corresponding deactivation DCI is received, see clause 7.3.1 in TS 38.212 [17] and clause 10.2 in TS 38.213 [13].

cp-ExtensionC2, cp-ExtensionC3 Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. For 30 kHz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60 kHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3.

lbt-FailureRecoveryConfig Configures parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

pathlossReferenceRSToAddModList A list of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for path loss estimation for PUSCH, PUCCH and SRS for unified TCI state operation. If unifiedTCI-StateType is not configured for the serving cell, no element in this list is configured.

pucch-Config PUCCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL, the network configures PUCCH only on the BWPs of one of the uplinks (normal UL or SUL). The network configures PUCCH-Config at least on non-initial BWP(s) for SpCell and on all BWP(s) for PUCCH SCell. If supported by the UE, the network may configure at most one additional SCell of a cell group with PUCCH-Config (i.e. PUCCH SCell). If PUCCH cell switching is supported by the UE, the network may configure two TDD serving cells with PUCCH-Config within each PUCCH group. For supporting PUCCH cell switching in the PUCCH group with the SpCell, the TDD SpCell and one TDD SCell shall have PUCCH-Config on their normal UL. For supporting PUCCH cell switching in the PUCCH group with only SCells, two TDD SCells shall have PUCCH-Config on their normal UL. In (NG)EN-DC and NE-DC, the NW configures at most one serving cell per frequency range with PUCCH. In (NG)EN-DC and NE-DC, if two PUCCH groups are configured, the serving cells of the NR PUCCH group in FR2 use the same numerology. For NR-DC, the maximum number of PUCCH groups in each cell group is one, and only the same numerology is supported for the cell group with carriers only in FR2. The NW may configure PUCCH for a BWP when setting up the BWP. The network may also add/remove the pucch-Config in an RRCReconfiguration with reconfigurationWithSync (for SpCell or PUCCH SCell) or with SCell release and add (for PUCCH SCell) to move the PUCCH between the UL and SUL carrier of one serving cell. In other cases, only modifications of a previously configured pucch-Config are allowed. If one (S)UL BWP of a serving cell is configured with PUCCH, all other (S)UL BWPs must be configured with PUCCH, too.

pucch-ConfigurationList PUCCH configurations for two simultaneously constructed HARQ-ACK codebooks (see TS 38.213 [13], clause 9.1). Different PUCCH Resource IDs are configured in different PUCCH-Config within the pucch-ConfigurationList if configured.

pucch-ConfigurationListMulticast1 PUCCH configurations for two simultaneously constructed HARQ-ACK codebooks for MBS multicast (see TS 38.213, clause 9).

pucch-ConfigurationListMulticast2 PUCCH configurations for two simultaneously constructed NACK-only feedback for MBS multicast (see TS 38.213, clause 9).

pusch-Config PUSCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL and if it has a PUSCH-Config for both UL and SUL, an UL/SUL indicator field in DCI indicates which of the two to use. See TS 38.212 [17], clause 7.3.1.

pucch-ConfigMulticast1 PUCCH configuration for the HARQ-ACK codebook for MBS multicast when multicast feedback is not configured with a priority value (see TS 38.213 [13], clause 9). If the field is not configured, pucch-Config applies.

pucch-ConfigMulticast2 PUCCH configuration for the NACK-only feedback for MBS multicast when multicast feedback is not configured with a priority value (see TS 38.213 [13], clause 9). If the field is not configured, pucch-Config applies.

sl-PUCCH-Config Indicates the UE specific PUCCH configurations used for the HARQ-ACK feedback reporting for NR sidelink communication.

srs-Config Uplink sounding reference signal configuration.

ul-powerControl Configures power control parameters for PUCCH, PUSCH and SRS when UE is configured with unifiedTCI-StateType for this serving cell. For each serving cell, ul-powerControl is either configured in all BWP-UplinkDedicated or it is not configured in any BWP-UplinkDedicated. When unifiedTCI-StateRef in the BWP-UplinkDedicated or in the PDSCH-Config if the unifiedTCI-StateType is set to joint, of a serving cell refers to another serving cell, ul-powerControl is either configured in all BWP-UplinkDedicated of these two serving cells or it is not configured in any BWP-UplinkDedicated of these two serving cells.

ul-TCI-StateList Indicates the applicable UL TCI states for PUCCH, PUSCH and SRS.

ul-TCI-ToAddModList Indicates a list of UL TCI states.

unifiedTCI-StateRef Provides the serving cell and UL BWP where UL TCI states applicable to this UL BWP are defined. The value of unifiedTCI-StateType of current serving cell is the same in the serving cell indicated by unifiedTCI-StateRef.

useInterlacePUCCH-PUSCH If the field is present, the UE uses uplink frequency domain resource allocation Type 2 for PUSCH (see TS 38.213 [13], clause 8.3 and TS 38.214 [19], clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for PUCCH (see TS 38.213 [13], clause 9.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| NoTCI-PC | The field is optionally present, Need R, if unifiedTCI-StateType is configured for this serving cell and ul-powerControl is not configured for any UL TCI state or joint TCI state of this serving cell. Otherwise it is absent, Need R |
| SpCellOnly | The field is optionally present, Need M, in the BWP-UplinkDedicated of an SpCell. It is absent otherwise. |

NOTE 1:	In case of RRCReconfiguration with reconfigurationWithSync, the UE performs a MAC reset, which involves releasing the PUCCH-CSI/SRS/SR configuration in accordance with clause 5.3.12 and TS 38.321 [3], clauses 5.12 and 5.2. Hence, for these parts of the dedicated radio resource configuration, delta signalling is not supported in the message when reconfigurationWithSync is included.

#### –	CandidateBeamRS

The IE CandidateBeamRS inlcudes candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.17.

CandidateBeamRS information element

-- ASN1START

-- TAG-CANDIDATEBEAMRS-START

CandidateBeamRS-r16 ::=                SEQUENCE {

candidateBeamConfig-r16                CHOICE {

ssb-r16                                SSB-Index,

csi-RS-r16                             NZP-CSI-RS-ResourceId

},

servingCellId                          ServCellIndex                                                            OPTIONAL  -- Need R

}

-- TAG-CANDIDATEBEAMRS-STOP

-- ASN1STOP

CandidateBeamRS field descriptions

candidateBeamConfig Indicates the resource (i.e. SSB or CSI-RS) defining this beam resource.

servingCellId If the field is absent, the RS belongs to the serving cell in which BeamFailureRecoveryRSConfig is configured.

#### –	CandidateTCI-State

The IE CandidateTCI-State defines a TCI states configuration which associate one or more reference signal with a corresponding quasi-colocation (QCL) type.

CandidateTCI-State information element

-- ASN1START

-- TAG-CANDIDATETCI-STATE-START

CandidateTCI-State-r18 ::=           SEQUENCE {

tci-StateId-r18                      TCI-StateId,

qcl-Type1-r18                        LTM-QCL-Info-r18,

qcl-Type2-r18                        LTM-QCL-Info-r18                                                    OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r18           PathlossReferenceRS-Id-r17                                          OPTIONAL,   -- Need R

...

}

LTM-QCL-Info-r18 ::=                 SEQUENCE {

referenceSignal-r18                  CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

qcl-Type-r18                         ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-CANDIDATETCI-STATE-STOP

-- ASN1STOP

CandidateTCI-State field descriptions

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the LTM candidate that includes this CandidateTCI-State.

qcl-Type1, qcl-Type2 QCL information for the TCI state.

tci-StateId The ID number of the TCI state.

#### –	CandidateTCI-UL-State

The IE CandidateTCI-UL-State defines an uplink TCI states configuration.

CandidateTCI-UL-State information element

-- ASN1START

-- TAG-CANDIDATETCI-UL-STATE-START

CandidateTCI-UL-State-r18 ::=           SEQUENCE {

tci-UL-StateId-r18                      TCI-UL-StateId-r17,

referenceSignal-r18                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

},

pathlossReferenceRS-Id-r18              PathlossReferenceRS-Id-r17                                   OPTIONAL,   -- Need R

...

}

-- TAG-CANDIDATETCI-UL-STATE-STOP

-- ASN1STOP

CandidateTCI-UL-State field descriptions

csi-RS-Index Indicates an NZP-CSI-RS-Resource of the LTM candidate that includes this CandidateTCI-UL-State.

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the LTM candidate that includes this CandidateTCI-UL-State.

referenceSignal Reference signal with which spatial relation information is provided.

ssb-Index The index of a SSB/PBCH block as indicated in ltm-SSB-Config of the LTM candidate that includes this CandidateTCI-UL-State.

tci-UL-StateID The ID number of the uplink TCI state.

#### –	CellAccessRelatedInfo

The IE CellAccessRelatedInfo indicates cell access related information for this cell.

CellAccessRelatedInfo information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFO-START

CellAccessRelatedInfo   ::=         SEQUENCE {

plmn-IdentityInfoList               PLMN-IdentityInfoList,

cellReservedForOtherUse             ENUMERATED {true}             OPTIONAL,   -- Need R

...,

[[

cellReservedForFutureUse-r16        ENUMERATED {true}             OPTIONAL,   -- Need R

npn-IdentityInfoList-r16            NPN-IdentityInfoList-r16      OPTIONAL    -- Need R

]],

[[

snpn-AccessInfoList-r17             SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17    OPTIONAL    -- Need R

]]

}

SNPN-AccessInfo-r17 ::=         SEQUENCE {

extCH-Supported-r17                 ENUMERATED {true}             OPTIONAL,   -- Need R

extCH-WithoutConfigAllowed-r17      ENUMERATED {true}             OPTIONAL,   -- Need R

onboardingEnabled-r17               ENUMERATED {true}             OPTIONAL,   -- Need R

imsEmergencySupportForSNPN-r17      ENUMERATED {true}             OPTIONAL    -- Need R

}

-- TAG-CELLACCESSRELATEDINFO-STOP

-- ASN1STOP

CellAccessRelatedInfo field descriptions

cellReservedForFutureUse Indicates whether the cell is reserved, as defined in 38.304 [20] for future use. The field is applicable to all PLMNs and NPNs. This field is ignored by IAB-MT and NCR-MT.

cellReservedForOtherUse Indicates whether the cell is reserved, as defined in 38.304 [20]. The field is applicable to all PLMNs. This field is ignored by IAB-MT and NCR-MT for cell barring determination, but still considered by NPN capable IAB-MT and NPN capable NCR-MT for determination of an NPN-only cell.

npn-IdentityInfoList The npn-IdentityInfoList is used to configure a set of NPN-IdentityInfo elements. Each of those elements contains a list of one or more NPN Identities and additional information associated with those NPNs. The total number of PLMNs (identified by a PLMN identity in plmn -IdentityList), PNI-NPNs (identified by a PLMN identity and a CAG-ID), and SNPNs (identified by a PLMN identity and a NID) together in the PLMN-IdentityInfoList and NPN-IdentityInfoList does not exceed 12, except for the NPN-only cells. A PNI-NPN and SNPN can be included only once, and in only one entry of the NPN-IdentityInfoList. In case of NPN-only cells the PLMN-IdentityList contains a single element that does not count to the limit of 12 and the cellIdentity of the first entry of the PLMN-IdentityInfoList is set to the same value as the cellIdentity-r16 of the first entry of the NPN-IdentityInfoList. The NPN index is defined as B+c1+c2+…+c(n-1)+d1+d2+…+d(m-1)+e(i) for the NPN identity included in the n-th entry of NPN-IdentityInfoList and in the m-th entry of npn-Identitylist within that NPN-IdentityInfoList entry, and the i-th entry of its corresponding NPN-Identity, where - B is the index used for the last PLMN in the PLMN-IdentittyInfoList; in NPN-only cells B is considered 0; - c(j) is the number of NPN index values used in the j-th NPN-IdentityInfoList entry; - d(k) is the number of NPN index values used in the k-th npn-IdentityList entry within the n-th NPN-IdentityInfoList entry; - e(i) is - i if the n-th entry of NPN-IdentityInfoList entry is for SNPN(s); - 1 if the n-th entry of NPN-IdentityInfoList entry is for PNI-NPN(s).

plmn-IdentityInfoList The plmn-IdentityInfoList is used to configure a set of PLMN-IdentityInfo elements. Each of those elements contains a list of one or more PLMN Identities and additional information associated with those PLMNs. A PLMN-identity can be included only once, and in only one entry of the PLMN-IdentityInfoList. The PLMN index is defined as b1+b2+…+b(n-1)+i for the PLMN included at the n-th entry of PLMN-IdentityInfoList and the i-th entry of its corresponding PLMN-IdentityInfo, where b(j) is the number of PLMN-Identity entries in each PLMN-IdentityInfo, respectively.

snpn-AccessInfoList This list provides access related information for each SNPN in npn-IdentityInfoList, see TS 23.501 [32]. The n-th entry of the list contains the access related information of the n-th SNPN in npn-IdentityInfoList.

SNPN-AccessInfo field descriptions

extCH-Supported Indicates whether the SNPN supports access using credentials from a Credentials Holder as specified in TS 23.501 [32].

extCH-WithoutConfigAllowed Indicates whether the SNPN allows registration attempts with credentials from a Credentials Holder from UEs that are not explicitly configured to select the SNPN as specified in TS 23.501 [32].

imsEmergencySupportForSNPN Indicates whether the SNPN supports IMS emergency bearer services for UEs in limited service mode in the cell. If absent, IMS emergency call is not supported by the SNPN in the cell for UEs in limited service mode.

onboardingEnabled Indicates whether the onboarding SNPN allows registration for onboarding in the cell as specified in TS 23.501 [32].

#### –	CellAccessRelatedInfo-EUTRA-5GC

The IE CellAccessRelatedInfo-EUTRA-5GC indicates cell access related information for an LTE cell connected to 5GC.

CellAccessRelatedInfo-EUTRA-5GC information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFOEUTRA-5GC-START

CellAccessRelatedInfo-EUTRA-5GC  ::=    SEQUENCE {

plmn-IdentityList-eutra-5gc             PLMN-IdentityList-EUTRA-5GC,

trackingAreaCode-eutra-5gc              TrackingAreaCode,

ranac-5gc                               RAN-AreaCode                                OPTIONAL,

cellIdentity-eutra-5gc                  CellIdentity-EUTRA-5GC

}

PLMN-IdentityList-EUTRA-5GC::=          SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity-EUTRA-5GC

PLMN-Identity-EUTRA-5GC ::=             CHOICE {

plmn-Identity-EUTRA-5GC                 PLMN-Identity,

plmn-index                              INTEGER (1..maxPLMN)

}

CellIdentity-EUTRA-5GC ::=              CHOICE {

cellIdentity-EUTRA                      BIT STRING (SIZE (28)),

cellId-index                            INTEGER (1..maxPLMN)

}

-- TAG-CELLACCESSRELATEDINFOEUTRA-5GC-STOP

-- ASN1STOP

#### –	CellAccessRelatedInfo-EUTRA-EPC

The IE CellAccessRelatedInfo-EUTRA-EPC indicates cell access related information for an LTE cell connected to EPC.

CellAccessRelatedInfo-EUTRA-EPC information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFOEUTRA-EPC-START

CellAccessRelatedInfo-EUTRA-EPC  ::=    SEQUENCE {

plmn-IdentityList-eutra-epc             PLMN-IdentityList-EUTRA-EPC,

trackingAreaCode-eutra-epc              BIT STRING (SIZE (16)),

cellIdentity-eutra-epc                  BIT STRING (SIZE (28))

}

PLMN-IdentityList-EUTRA-EPC::=          SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity

-- TAG-CELLACCESSRELATEDINFOEUTRA-EPC-STOP

-- ASN1STOP

#### –	CellDTXDRX-Config

The IE CellDTXDRX-Config is used to configure cell DTX/DRX related parameters.

CellDTXDRX-Config information element

-- ASN1START

-- TAG-CELLDTXDRX-CONFIG-START

CellDTXDRX-Config-r18 ::=              SEQUENCE {

cellDTXDRX-onDurationTimer-r18         CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

cellDTXDRX-CycleStartOffset-r18        CHOICE {

ms10                                   INTEGER(0..9),

ms20                                   INTEGER(0..19),

ms32                                   INTEGER(0..31),

ms40                                   INTEGER(0..39),

ms60                                   INTEGER(0..59),

ms64                                   INTEGER(0..63),

ms70                                   INTEGER(0..69),

ms80                                   INTEGER(0..79),

ms128                                  INTEGER(0..127),

ms160                                  INTEGER(0..159),

ms256                                  INTEGER(0..255),

ms320                                  INTEGER(0..319),

ms512                                  INTEGER(0..511),

ms640                                  INTEGER(0..639),

ms1024                                 INTEGER(0..1023),

ms1280                                 INTEGER(0..1279),

ms2048                                 INTEGER(0..2047),

ms2560                                 INTEGER(0..2559),

ms5120                                 INTEGER(0..5119),

ms10240                                INTEGER(0..10239)

},

cellDTXDRX-SlotOffset-r18                  INTEGER (0..31),

cellDTXDRXconfigType-r18                   ENUMERATED {dtx, drx, dtxdrx},

cellDTXDRXactivationStatus-r18             ENUMERATED {activated, deactivated}           OPTIONAL   -- Need N

}

-- TAG-CELLDTXDRX-CONFIG-STOP

-- ASN1STOP

CellDTXDRX-Config field descriptions

cellDTXDRX-CycleStartOffset cellDTXDRX-Cycle in ms and cellDTXDRX-StartOffset in multiples of 1 ms. The configured cellDTXDRX-Cycle is an integer multiple of configured drx-longCycle or vice versa.

cellDTXDRX-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

cellDTXDRX-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

cellDTXDRXactivationStatus Initial activation status of cell DTX/DRX indicating whether the UE shall activate the configuration according to the received parameters. This field is only used upon setup of a cell DTX/DRX configuration.

cellDTXDRXconfigType Indicates whether the configuration is for cell DTX only, cell DRX only, or joint cell DTX/DRX configuration.

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

]],

[[

f1c-TransferPathNRDC-r17                   ENUMERATED {mcg, scg, both}                                             OPTIONAL,   -- Need M

uplinkTxSwitching-2T-Mode-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Cond 2Tx

uplinkTxSwitching-DualUL-TxState-r17       ENUMERATED {oneT, twoT}                                                 OPTIONAL,   -- Cond 2Tx

uu-RelayRLC-ChannelToAddModList-r17        SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17

OPTIONAL,   -- Need N

uu-RelayRLC-ChannelToReleaseList-r17       SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelID-r17

OPTIONAL,   -- Need N

simultaneousU-TCI-UpdateList1-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList2-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList3-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList4-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

rlc-BearerToReleaseListExt-r17             SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentityExt-r17           OPTIONAL,   -- Need N

iab-ResourceConfigToAddModList-r17  SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfig-r17   OPTIONAL, -- Need N

iab-ResourceConfigToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfigID-r17 OPTIONAL  -- Need N

]],

[[

reportUplinkTxDirectCurrentMoreCarrier-r17 ReportUplinkTxDirectCurrentMoreCarrier-r17                            OPTIONAL  -- Need N

]],

[[

prioSCellPRACH-OverSP-PeriodicSRS-r17      ENUMERATED {enabled}                                                  OPTIONAL  -- Need R

]],

[[

ncr-FwdConfig-r18                          SetupRelease { NCR-FwdConfig-r18 }                                 OPTIONAL,  -- Cond NCR

autonomousDenialParameters-r18             SetupRelease {AutonomousDenialParameters-r18}                      OPTIONAL,   -- Need M

nonCollocatedTypeMRDC-r18                  ENUMERATED { true }                                                OPTIONAL,   -- Need R

nonCollocatedTypeNR-CA-r18                 ENUMERATED { true }                                                OPTIONAL,   -- Need R

uplinkTxSwitchingMoreBands-r18             SetupRelease { UplinkTxSwitchingMoreBands-r18 }                    OPTIONAL    -- Need M

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...,

[[

lowMobilityEvaluationConnected-r17  SEQUENCE {

s-SearchDeltaP-Connected-r17        ENUMERATED {dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1},

t-SearchDeltaP-Connected-r17        ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                               OPTIONAL,   -- Need R

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

deactivatedSCG-Config-r17           SetupRelease { DeactivatedSCG-Config-r17 }                  OPTIONAL    -- Cond SCG-Opt

]]

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]],

[[

rach-LessHO-r18                 RACH-LessHO-r18                                                 OPTIONAL,   -- Need N

sl-IndirectPathMaintain-r18     ENUMERATED{true}                                                OPTIONAL     -- Cond MP

]]

}

DAPS-UplinkPowerConfig-r16 ::=      SEQUENCE {

p-DAPS-Source-r16                   P-Max,

p-DAPS-Target-r16                   P-Max,

uplinkPowerSharingDAPS-Mode-r16     ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

DeactivatedSCG-Config-r17 ::=       SEQUENCE {

bfd-and-RLM-r17                     BOOLEAN,

...

}

GoodServingCellEvaluation-r17 ::=       SEQUENCE {

offset-r17                              ENUMERATED {db2, db4, db6, db8}                         OPTIONAL   -- Need S

}

SL-PathSwitchConfig-r17 ::=         SEQUENCE {

targetRelayUE-Identity-r17          SL-SourceIdentity-r17,

t420-r17                            ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

...

}

IAB-ResourceConfig-r17 ::=          SEQUENCE {

iab-ResourceConfigID-r17            IAB-ResourceConfigID-r17,

slotList-r17                        SEQUENCE (SIZE (1..5120)) OF INTEGER (0..5119)                           OPTIONAL,    -- Need M

periodicitySlotList-r17             ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160}     OPTIONAL,    -- Need M

slotListSubcarrierSpacing-r17       SubcarrierSpacing                                                        OPTIONAL,    -- Need M

...

}

IAB-ResourceConfigID-r17 ::=        INTEGER(0..maxNrofIABResourceConfig-1-r17)

ReportUplinkTxDirectCurrentMoreCarrier-r17 ::= SEQUENCE (SIZE(1.. maxSimultaneousBands)) OF IntraBandCC-CombinationReqList-r17

IntraBandCC-CombinationReqList-r17::=   SEQUENCE {

servCellIndexList-r17                   SEQUENCE (SIZE(1.. maxNrofServingCells)) OF ServCellIndex,

cc-CombinationList-r17                  SEQUENCE (SIZE(1.. maxNrofReqComDC-Location-r17)) OF IntraBandCC-Combination-r17

}

IntraBandCC-Combination-r17::=      SEQUENCE (SIZE(1.. maxNrofServingCells)) OF CC-State-r17

CC-State-r17::=                     SEQUENCE {

dlCarrier-r17                       CarrierState-r17                             OPTIONAL, -- Need N

ulCarrier-r17                       CarrierState-r17                             OPTIONAL  -- Need N

}

CarrierState-r17::=                 CHOICE {

deActivated-r17                     NULL,

activeBWP-r17                       INTEGER (0..maxNrofBWPs)

}

AutonomousDenialParameters-r18 ::=  SEQUENCE {

autonomousDenialSlots-r18           ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1},

autonomousDenialValidity-r18        ENUMERATED {n200, n500, n1000, n2000}

}

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

}

UplinkTxSwitchingMoreBands-r18::=              SEQUENCE {

uplinkTxSwitchingBandList-r18                  SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FreqBandIndicatorNR OPTIONAL,  -- Need M

uplinkTxSwitchingBandPairList-r18              UplinkTxSwitchingBandPairList-r18                OPTIONAL,   -- Need M

uplinkTxSwitchingAssociatedBandDualUL-List-r18 UplinkTxSwitchingAssociatedBandDualUL-List-r18   OPTIONAL,   -- Need M

...

}

UplinkTxSwitchingBandPairList-r18::=      SEQUENCE (SIZE (1.. maxULTxSwitchingBandPairs)) OF UplinkTxSwitchingBandPairConfig-r18

UplinkTxSwitchingBandPairConfig-r18::=    SEQUENCE {

bandInfoUL1-r18                           UplinkTxSwitchingBandIndex-r18,

bandInfoUL2-r18                           UplinkTxSwitchingBandIndex-r18,

switchingOptionConfigForBandPair-r18      ENUMERATED {switchedUL, dualUL},

switching2T-Mode-r18                      ENUMERATED {enabled}                                             OPTIONAL,   -- Need S

switchingPeriodConfigForBandPair-r18      ENUMERATED {n35us, n140us}                                       OPTIONAL,   -- Need S

...

}

UplinkTxSwitchingAssociatedBandDualUL-List-r18::= SEQUENCE (SIZE (0..maxSimultaneousBands)) OF UplinkTxSwitchingAssociatedBandDualUL-r18

UplinkTxSwitchingAssociatedBandDualUL-r18::=  SEQUENCE {

transmitBand-r18                              UplinkTxSwitchingBandIndex-r18,

associatedBand-r18                            UplinkTxSwitchingBandIndex-r18

}

UplinkTxSwitchingBandIndex-r18::=  INTEGER (1..maxSimultaneousBands)

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

AutonomousDenialParamters field descriptions

autonomousDenialSlots Indicates the maximum number of the UL slots for which the UE is allowed to deny any UL transmission. Value n2 corresponds to 2 slots, value n5 to 5 slots and so on.

autonomousDenialValidity Indicates the validity period over which the UL autonomous denial slots shall be counted. Value n200 corresponds to 200 slots, value n500 corresponds to 500 slots and so on.

CC-State field descriptions

dlCarrier Indicates DL carrier activation state for this carrier and the related active BWP Index, if activated.

ulCarrier Indicates UL carrier activation state for this carrier and the related active BWP Index, if activated.

CellGroupConfig field descriptions

bap-Address BAP address of the parent node in cell group.

bh-RLC-ChannelToAddModList Configuration of the backhaul RLC entities and the corresponding MAC Logical Channels to be added and modified.

bh-RLC-ChannelToReleaseList List of the backhaul RLC entities and the corresponding MAC Logical Channels to be released.

f1c-TransferPath The F1-C transfer path that an EN-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with lte, IAB-MT can only use LTE leg for F1-C transfer. If IAB-MT is configured with nr, IAB-MT can only use NR leg for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select an LTE leg or a NR leg for F1-C transfer. If the field is not configured, the IAB node uses the NR leg as the default one.

f1c-TransferPathNRDC The F1-C transfer path that an NR-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with mcg, IAB-MT can only use the MCG for F1-C transfer. If IAB-MT is configured with scg, IAB-MT can only use the SCG for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select the MCG or the SCG for F1-C transfer.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

ncr-FwdConfig Configuration of side control information for the NCR-Fwd access link.

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

npn-IdentityInfoList This field is used to transfer npn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to SNPN Identity. If this field and plmn-IdentityInfoList are both absent, the UE uses the npn-IdentityInfoList in SIB1 of the PCell.

plmn-IdentityInfoList This field is used to transfer plmn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to PLMN Identity. If this field and npn-IdentityInfoList are both absent, the UE uses the plmn-IdentityInfoList in SIB1 of the PCell.

prioSCellPRACH-OverSP-PeriodicSRS When configured, the UE applies UL power control prioritization by prioritizing PRACH transmission on SCell over semi-persistent and/or periodic SRS transmission as defined in clause 7.5 of TS 38.213 [13].

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

reportUplinkTxDirectCurrentMoreCarrier Enables reporting of uplink Direct Current location information when the UE is configured with intra-band CA. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. The UE only reports the uplink Direct Current location information that are related to the indicated cc-CombinationList. The network does not include carriers which locate in DL only spectrum described in TS 38.101-2 [39], clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3 in FR2 in the IntraBandCC-CombinationReqList. I.e. DL-only carrier in FR2 frequency spectrum is not used to calculate the default DC location.

reportUplinkTxDirectCurrentTwoCarrier Enables reporting of uplink Direct Current location information when the UE is configured with uplink intra-band CA with two carriers. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

rlc-BearerToReleaseListExt List of the RLC entities and the corresponding MAC Logical Channels to be released for multicast MRBs.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellSIB20 This field is used to transfer SIB20 of the SCell in order to allow the UE for MBS broadcast reception on SCell. The network configures this field only for a single SCell at a time.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in these lists.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or (NG)EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and (NG)EN-DC case where UE supports dynamic UL Tx switching.

uplinkTxSwitchingPowerBoosting Indicates whether the UE is allowed to enable 3dB boosting on the maximum output power for transmission on carrier2 under the operation state in which 2-port transmission can be supported on carrier2 for inter-band UL CA case with dynamic UL Tx switching as defined in TS 38.101-1 [15]. Network can only configure this field for dynamic UL Tx switching in inter-band UL CA case with power Class 3 as defined in TS 38.101-1 [15].

uplinkTxSwitching-2T-Mode Indicates 2Tx-2Tx switching mode is configured for inter-band UL CA or SUL, in which the switching gap duration for a triggered uplink switching (as specified in TS 38.214 [19]) is equal to the switching time capability value reported for the switching mode. If this field is absent and uplinkTxSwitching is configured, it is interpreted that 1Tx-2Tx UL Tx switching is configured as specified in TS 38.214 [19]. In this case, there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured.

uplinkTxSwitching-DualUL-TxState Indicates the state of Tx chains if the state of Tx chains after the UL Tx switching is not unique (as specified in TS 38.214 [19]) in case of 2Tx-2Tx switching is configured and uplinkTxSwitchingOption is set to dualUL. Value oneT indicates 1Tx is assumed to be supported on the carriers on each band, value twoT indicates 2Tx is assumed to be supported on that carrier. This field applies for all band pairs if uplinkTxSwitchingMoreBands is configured.

uplinkTxSwitchingMoreBands Indicates UL band list, band pair list and other configurations for ULTx switching.

uu-RelayRLC-ChannelToAddModList List of the Uu RLC entities and the corresponding MAC Logical Channels to be added or modified.

uu-RelayRLC-ChannelToReleaseList List of the Uu RLC entities and the corresponding MAC Logical Channels to be released.

DeactivatedSCG-Config field descriptions

bfd-and-RLM If the field is set to true, the UE shall perform RLM and BFD on the PSCell when the SCG is deactivated and the network ensures that beamFailure-r17 is not configured in the radioLinkMonitoringConfig of the DL BWP of the PSCell in which the UE performs BFD. If set to false, the UE is not required to perform RLM and BFD on the PSCell when the SCG is deactivated.

DAPS-UplinkPowerConfig field descriptions

p-DAPS-Source The maximum total transmit power to be used by the UE in the source cell group during DAPS handover.

p-DAPS-Target The maximum total transmit power to be used by the UE in the target cell group during DAPS handover.

uplinkPowerSharingDAPS-Mode Indicates the uplink power sharing mode that the UE uses in DAPS handover (see TS 38.213 [13]).

GoodServingCellEvaluation field descriptions

offset The parameter "X" (dB) for the good serving cell quality criterion in RRC_CONNECTED, for a cell operating in FR1 and FR2, respectively. If this field is absent, the UE applies the (default) value of 0 dB for "X".

IAB-ResourceConfig field descriptions

iab-ResourceConfigID This ID is used to indicate the specific resource configuration addressed by the MAC CEs specified in TS 38.321 [3].

periodicitySlotList Indicates the periodicity in ms of the list of slot indexes indicated in slotList.

slotList Indicates the list of slot indexes to which the information indicated in the specific MAC CE applies to, as specified in TS 38.321 [3]. The values of the entries in the slotList are strictly less than the value of the periodicitySlotList.

slotListSubcarrierSpacing Subcarrier spacing used as reference for the slotList configuration. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120 or 480 kHz

RACH-LessHO field descriptions

ssb-Index This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission, see TS 38.321 [3]. This field is present when dynamic grant is used for initial uplink transmission in RACH-less handover in NTN.

targetNTA This field refers to the timing adjustment, see TS 38.213 [13] and TS 38.321 [3], indicating the NTA value which the UE shall use for the target PTAG of handover. The value zero corresponds to NTA=0, while the value source corresponds to the NTA value of the source serving cell. Only value source is configured by the network in case source cell is a mobile IAB cell.

tci-StateID This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission. This field is present in case this cell is a mobile IAB cell.

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

sl-IndirectPathMaintain Indicates that the L2 U2N Remote UE keeps the PC5 connection with its connected L2 U2N Relay UE.

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon or sets to the same periodicity as ssb-Periodicity-r17 in nonCellDefiningSSB-r17 if the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17. For case of NR PCell change, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. If the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17, this field corresponds to the NCD-SSB indicated by nonCellDefiningSSB-r17, otherwise, this field corresponds to the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL.

ReportUplinkTxDirectCurrentMoreCarrier field descriptions

IntraBandCC-Combination Indicates the state of the carriers and BWPs indexes of the carriers in a CC combination, each carrier in this combination corresponds to an entry in servCellIndexList with same order. This IE shall have the same size as servCellIndexList.

IntraBandCC-CombinationReqList Indicates the list of the requested carriers/BWPs combinations for an intra-band CA component.

servCellIndexList indicates the list of cell index for an intra-band CA component.

SCellConfig field descriptions

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in an SCell in RRC_CONNECTED. This field is always configured when the network enables BFD relaxation for the UE in this SCell. This field is absent if failureDetectionSetN is present for the SCell.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated while this SCell is deactivated. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration. If the field is included for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the default DRX group shall belong to another Frequency Range. If drx-ConfigSecondaryGroup is configured, the field is optionally present. The network always includes the field if the field was previously configured for this SCell and the SCell remains in the secondary DRX group. Removal of an individual SCell from the secondary DRX group is supported by using an SCell release and addition. Otherwise, if drx-ConfigSecondaryGroup is not configured, the field is absent and the UE shall release the field. The UE shall also release the field if drx-ConfigSecondaryGroup is released without including sCellToAddModList.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent and absoluteFrequencySSB is included, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. If the SCell is an SSB-less SCell (i.e., the IE absoluteFrequencySSB in ServingCellConfigCommon is absent), this field is absent.

SpCellConfig field descriptions

deactivatedSCG-Config Configuration applicable when the SCG is deactivated. The network always configures this field before or when indicating that the SCG is deactivated in an RRCReconfiguration, RRCResume, E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume message.

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables BFD relaxation for the UE in this SpCell. This field is absent if failureDetectionSetN is present for the SpCell.

goodServingCellEvaluationRLM Indicates the criterion for a UE to detect the good serving cell quality for RLM relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables RLM relaxation for the UE in this SpCell.

lowMobilityEvaluationConnected Indicates the criterion for a UE to detect low mobility in RRC_CONNECTED in an SpCell. The s-SearchDeltaP-Connected is the parameter "SSearchDeltaP-connected". Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on. The t-SearchDeltaP-Connected is the parameter "TSearchDeltaP-Connected". Value s5 means 5 seconds, value s10 means 10 seconds and so on. Low mobility criterion is configured in NR PCell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC.

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

SL-PathSwitchConfig field descriptions

targetRelayUE-Identity Indicates the L2 source ID of the target L2 U2N Relay UE during path switch.

t420 Indicates the timer value of T420 to be used during path switch.

UplinkTxSwitchingMoreBands field descriptions

uplinkTxSwitchingBandList Indicates the NR frequency band number of the UL bands for UL Tx switching. If the UE needs to determine location of switching period as specified in TS 38.101-1 [15], the UE considers that the bands are listed in decreasing order of priority, i.e. the first/leftmost entry corresponds to the band with the highest priority, the next entry corresponds to the band with the second highest priority, and so on. The last entry corresponds the band with the lowest priority.

uplinkTxSwitchingBandPairList Indicates the band pairs involved in UL Tx switching, as well as the per band pair configurations.

uplinkTxSwitchingAssociatedBandDualUL-List Indicates the associated band for the transmitting band indicated by transmitBand which the transmitting carrier(s) is on as specified in TS 38.214 [19], clause 6.1.6. The network ensures that each band pair of a transmitting band and an associated band supports the dualUL switching option.

UplinkTxSwitchingBandIndex The value n indicates the band included at the n-th entry of uplinkTxSwitchingBandList.

UplinkTxSwitchingBandPairConfig field descriptions

bandInfoUL1, bandInfoUL2 Indicates the band index for a band pair. The value n indicates the band included at the n-th entry of uplinkTxSwitchingBandList.

switching2T-Mode Indicates 2Tx-2Tx switching mode is configured to the band pair. If this field is absent when uplink Tx switching is configured, it is interpreted that 1Tx-2Tx/1Tx-1Tx UL Tx switching is configured as specified in TS 38.214 [19].

switchingOptionConfigForBandPair Indicates the switching option for the band pair as specified in TS 38.214 [19], clause 6.1.6.

switchingPeriodConfigForBandPair Indicates the value of switching period for the band pair as specified in TS 38.214 [19], clause 6.1.6. Value n35us represents 35 us, n140us represents 140us. If the field is absent, 210 us is applied.

| Conditional Presence | Explanation |
| --- | --- |
| 2Tx | The field is optionally present, Need R, if uplinkTxSwitching is configured; otherwise it is absent, Need R. |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DirectToIndirect-PathSwitch | The field is mandatory present for the L2 U2N remote UE at path switch to the target L2 U2N Relay UE (including direct to indirect path switch and indirect to indirect path switch). It is absent otherwise. Note: the target L2 U2N Relay UE should not be the same as serving L2 U2N Relay UE for inter-gNB indirect to indirect path switch. |
| MP | This field is optionally present, Need N, if a L2 U2N remote UE is configured to perform MP direct path addition during indirect-to-direct path swith procedure, or to perform MP direct path release during direct-to-indirect path switch procedure. It is absent otherwise. |
| NCR | The field is optionally present, Need M, for NCR-MT. It is absent otherwise. |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the SCell belongs to and configured with preConfigInd. It is absent, Need R, otherwise. |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup: - at change of AS security key derived from KgNB, - in an RRCReconfiguration message contained in a DLInformationTransferMRDC message, - path switch of L2 U2N remote UE to the target PCell, - path switch of L2 U2N remote UE to the target L2 U2N Relay UE, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell, - change of AS security key derived from S-KgNB in NR-DC while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, - MN handover in (NG)EN-DC. Otherwise, it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages and is absent in the masterCellGroup in RRCReconfiguration messages if source configuration is not released during DAPS handover. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N: - in the masterCellGroup at - SCell addition, - reconfiguration with sync, - resume of an RRC connection. - in the secondaryCellGroup, when the SCG is not indicated as deactivated at: - SCG activation while the SCG was previously deactivated, - SCell addition, - reconfiguration with sync. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |
| SCellSIB20-Opt | This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise. |
| SCG-Opt | The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise. |

NOTE:	In case of change of AS security key derived from S-KgNB/S-KeNB, if reconfigurationWithSync is not included in the masterCellGroup, the network releases all existing MCG RLC bearers associated with a radio bearer with keyToUse set to secondary. In case of change of AS security key derived from KgNB/KeNB, if reconfigurationWithSync is not included in the secondaryCellGroup, the network releases all existing SCG RLC bearers associated with a radio bearer with keyToUse set to primary.

#### –	CellGroupId

The IE CellGroupId is used to identify a cell group. Value 0 identifies the master cell group. Other values identify secondary cell groups. In this version of the specification only values 0 and 1 are supported.

CellGroupId information element

-- ASN1START

-- TAG-CELLGROUPID-START

CellGroupId ::=                             INTEGER (0.. maxSecondaryCellGroups)

-- TAG-CELLGROUPID-STOP

-- ASN1STOP

#### –	CellIdentity

The IE CellIdentity is used to unambiguously identify a cell within a PLMN/SNPN.

CellIdentity information element

-- ASN1START

-- TAG-CELLIDENTITY-START

CellIdentity ::=                         BIT STRING (SIZE (36))

-- TAG-CELLIDENTITY-STOP

-- ASN1STOP

#### –	CellReselectionPriority

The IE CellReselectionPriority concerns the absolute priority of the concerned carrier frequency, as used by the cell reselection procedure. Corresponds to parameter "priority" in TS 38.304 [20]. Value 0 means lowest priority. The UE behaviour for the case the field is absent, if applicable, is specified in TS 38.304 [20].

CellReselectionPriority information element

-- ASN1START

-- TAG-CELLRESELECTIONPRIORITY-START

CellReselectionPriority ::=             INTEGER (0..7)

-- TAG-CELLRESELECTIONPRIORITY-STOP

-- ASN1STOP

#### –	CellReselectionSubPriority

The IE CellReselectionSubPriority indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency for E-UTRA and NR. Value oDot2 corresponds to 0.2, value oDot4 corresponds to 0.4 and so on.

CellReselectionSubPriority information element

-- ASN1START

-- TAG-CELLRESELECTIONSUBPRIORITY-START

CellReselectionSubPriority ::=          ENUMERATED {oDot2, oDot4, oDot6, oDot8}

-- TAG-CELLRESELECTIONSUBPRIORITY-STOP

-- ASN1STOP

#### –	CFR-ConfigMulticast

The IE CFR-ConfigMulticast indicates UE specific common frequency resource configuration for multicast for one dedicated BWP.

CFR-ConfigMulticast information element

-- ASN1START

-- TAG-CFR-CONFIGMULTICAST-START

CFR-ConfigMulticast-r17::= SEQUENCE {

locationAndBandwidthMulticast-r17              INTEGER (0..37949)                       OPTIONAL,    -- Need S

pdcch-ConfigMulticast-r17                      PDCCH-Config                             OPTIONAL,    -- Need M

pdsch-ConfigMulticast-r17                      PDSCH-Config                             OPTIONAL,    -- Need M

sps-ConfigMulticastToAddModList-r17            SPS-ConfigMulticastToAddModList-r17      OPTIONAL,    -- Need N

sps-ConfigMulticastToReleaseList-r17           SPS-ConfigMulticastToReleaseList-r17     OPTIONAL     -- Need N

}

SPS-ConfigMulticastToAddModList-r17 ::=	SEQUENCE (SIZE (1..8)) OF SPS-Config

SPS-ConfigMulticastToReleaseList-r17 ::= SEQUENCE (SIZE (1..8)) OF SPS-ConfigIndex-r16

-- TAG-CFR-CONFIGMULTICAST-STOP

-- ASN1STOP

CFR-ConfigMulticast field descriptions

locationAndBandwidthMulticast Frequency domain location and bandwidth for MBS multicast. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting N^size_BWP=275. The first PRB is a PRB determined by subcarrierSpacing of the associated BWP and offsetToCarrier corresponding to this subcarrier spacing. If not configured, the UE applies the value of locationAndBandwidth of the DL BWP in which the cfr-ConfigMulticast is configured.

pdcch-ConfigMulticast UE specific group-common PDCCH configuration for MBS multicast for one CFR.

pdsch-ConfigMulticast UE specific group-common PDSCH configuration for MBS multicast for one CFR.

sps-ConfigMulticastToAddModList Indicates a list of one or more DL SPS configurations for MBS multicast.

sps-ConfigMulticastToReleaseList Indicates a list of one or more DL SPS configurations to be released. The NW may release a SPS configuration at any time.

#### –	CGI-InfoEUTRA

The IE CGI-InfoEUTRA indicates EUTRA cell access related information, which is reported by the UE as part of E-UTRA report CGI procedure.

CGI-InfoEUTRA information element

-- ASN1START

-- TAG-CGI-INFOEUTRA-START

CGI-InfoEUTRA ::=                        SEQUENCE {

cgi-info-EPC                            SEQUENCE {

cgi-info-EPC-legacy                 CellAccessRelatedInfo-EUTRA-EPC,

cgi-info-EPC-list                   SEQUENCE (SIZE (1..maxPLMN)) OF CellAccessRelatedInfo-EUTRA-EPC             OPTIONAL

}                                                                                                                       OPTIONAL,

cgi-info-5GC                            SEQUENCE (SIZE (1..maxPLMN)) OF CellAccessRelatedInfo-EUTRA-5GC             OPTIONAL,

freqBandIndicator                       FreqBandIndicatorEUTRA,

multiBandInfoList                       MultiBandInfoListEUTRA                                                      OPTIONAL,

freqBandIndicatorPriority               ENUMERATED {true}                                                           OPTIONAL

}

-- TAG-CGI-INFOEUTRA-STOP

-- ASN1STOP

#### –	CGI-InfoEUTRALogging

The IE CGI-InfoEUTRALogging indicates EUTRA cell related information, which is reported by the UE as part of RLF reporting procedure.

CGI-InfoEUTRALogging information element

-- ASN1START

-- TAG-CGI-INFOEUTRALOGGING-START

CGI-InfoEUTRALogging ::=         SEQUENCE {

plmn-Identity-eutra-5gc          PLMN-Identity                                          OPTIONAL,

trackingAreaCode-eutra-5gc       TrackingAreaCode                                       OPTIONAL,

cellIdentity-eutra-5gc           BIT STRING (SIZE (28))                                 OPTIONAL,

plmn-Identity-eutra-epc          PLMN-Identity                                          OPTIONAL,

trackingAreaCode-eutra-epc       BIT STRING (SIZE (16))                                 OPTIONAL,

cellIdentity-eutra-epc           BIT STRING (SIZE (28))                                 OPTIONAL

}

-- TAG-CGI-INFOEUTRALOGGING-STOP

-- ASN1STOP

CGI-InfoEUTRALogging field descriptions

cellIdentity-eutra-epc, cellIdentity-eutra-5GC Unambiguously identify a cell within the context of the PLMN. It belongs the first PLMN entry of plmn-IdentityList (when connected to EPC) or of plmn-IdentityList-r15 (when connected to 5GC) in SystemInformationBlockType1.

plmn-Identity-eutra-epc, plmn-Identity-eutra-5GC Identifies the PLMN of the cell for the reported cellIdentity: the first PLMN entry of plmn-IdentityList (when connected to EPC) or of plmn-IdentityList-r15 (when connected to 5GC) in SystemInformationBlockType1 that contained the reported cellIdentity.

trackingAreaCode-eutra-epc, trackingAreaCode-eutra-5gc Indicates Tracking Area Code to which the cell indicated by cellIdentity-eutra-epc, cellIdentity-eutra-5GC belongs.

#### –	CGI-InfoNR

The IE CGI-InfoNR indicates cell access related information, which is reported by the UE as part of report CGI procedure.

CGI-InfoNR information element

-- ASN1START

-- TAG-CGI-INFO-NR-START

CGI-InfoNR ::=                    SEQUENCE {

plmn-IdentityInfoList               PLMN-IdentityInfoList               OPTIONAL,

frequencyBandList                   MultiFrequencyBandListNR            OPTIONAL,

noSIB1                              SEQUENCE {

ssb-SubcarrierOffset                INTEGER (0..15),

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1

}                                                                       OPTIONAL,

...,

[[

npn-IdentityInfoList-r16            NPN-IdentityInfoList-r16            OPTIONAL

]],

[[

cellReservedForOtherUse-r16         ENUMERATED {true}                   OPTIONAL

]]

}

-- TAG-CGI-INFO-NR-STOP

-- ASN1STOP

CGI-InfoNR field descriptions

noSIB1 Contains ssb-SubcarrierOffset and pdcch-ConfigSIB1 fields acquired by the UE from MIB of the cell for which report CGI procedure was requested by the network in case SIB1 was not broadcast by the cell.

cellReservedForOtherUse Contains cellReservedForOtherUse field acquired by the UE that supports nr-CGI-Reporting-NPN from SIB1 of the cell for which report CGI procedure was requested by the network.

#### –	CGI-Info-Logging

The IE CGI-Info-Logging indicates the NR Cell Global Identifier (NCGI) for logging purposes (e.g. RLF report), the globally unique identity, and the TAC information of a cell in NR.

CGI-Info-Logging information element

-- ASN1START

-- TAG-CGI-INFO-LOGGING-START

CGI-Info-Logging-r16 ::=     SEQUENCE {

plmn-Identity-r16                    PLMN-Identity,

cellIdentity-r16                     CellIdentity,

trackingAreaCode-r16                 TrackingAreaCode               OPTIONAL

}

-- TAG-CGI-INFO-LOGGING-STOP

-- ASN1STOP

CGI-Info-Logging field descriptions

cellIdentity Unambiguously identify a cell within the context of the PLMN. It belongs the first PLMN-IdentityInfo IE of PLMN-IdentityInfoList in SIB1.

plmn-Identity Identifies the PLMN of the cell for the reported cellIdentity: the first PLMN entry of plmn-IdentityList (in SIB1) in the instance of PLMN-IdentityInfoList that contained the reported cellIdentity.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs.

#### –	CLI-RSSI-Range

The IE CLI-RSSI-Range specifies the value range used in CLI-RSSI measurements and thresholds. The integer value for CLI-RSSI measurements is according to Table 10.1.22.2.2-1 in TS 38.133 [14].

CLI-RSSI-Range information element

-- ASN1START

-- TAG-CLI-RSSI-RANGE-START

CLI-RSSI-Range-r16 ::=                      INTEGER(0..76)

-- TAG-CLI-RSSI-RANGE-STOP

-- ASN1STOP

#### –	ClockQualityMetrics

The IE ClockQualityMetrics is used to configure RAN timing synchronisation status information as specified in TS 38.473 [36]

ClockQualityMetrics information element

-- ASN1START

-- TAG-CLOCKQUALITYMETRICS-START

ClockQualityMetrics-r18 ::= SEQUENCE {

synchronisationState-r18        ENUMERATED {locked, holdover, freerun, spare1}                        OPTIONAL,   -- Need N

tracebilityToUTC-r18            BOOLEAN                                                               OPTIONAL,   -- Need N

tracebilityToGNSS-r18           BOOLEAN                                                               OPTIONAL,   -- Need N

clockFrequencyStability-r18     BIT STRING (SIZE(16))                                                 OPTIONAL,   -- Need N

clockAccuracy-r18               CHOICE {

value INTEGER (1..40000000),

index INTEGER (32..47)

}                                                                                                     OPTIONAL,   -- Need N

parentTimeSource-r18            ENUMERATED {syncE, pTP, gNSS,atomicClock, terrestialRadio,

serialTimeCode, nTP, handset, other, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}           OPTIONAL,   -- Need N

...

}

-- TAG-CLOCKQUALITYMETRICS-STOP

-- ASN1STOP

#### –	CodebookConfig

The IE CodebookConfig is used to configure codebooks of Type-I and Type-II (see TS 38.214 [19], clause 5.2.2.2)

CodebookConfig information element

-- ASN1START

-- TAG-CODEBOOKCONFIG-START

CodebookConfig ::=                                  SEQUENCE {

codebookType                                        CHOICE {

type1                                               SEQUENCE {

subType                                             CHOICE {

typeI-SinglePanel                                   SEQUENCE {

nrOfAntennaPorts                                    CHOICE {

two                                                 SEQUENCE {

twoTX-CodebookSubsetRestriction                     BIT STRING (SIZE (6))

},

moreThanTwo                                         SEQUENCE {

n1-n2                                               CHOICE {

two-one-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction              BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction              BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction            BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction            BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction           BIT STRING (SIZE (64))

},

typeI-SinglePanel-codebookSubsetRestriction-i2      BIT STRING (SIZE (16))        OPTIONAL    -- Need R

}

},

typeI-SinglePanel-ri-Restriction                    BIT STRING (SIZE (8))

},

typeI-MultiPanel                                    SEQUENCE {

ng-n1-n2                                                CHOICE {

two-two-one-TypeI-MultiPanel-Restriction                BIT STRING (SIZE (8)),

two-four-one-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (16)),

four-two-one-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (8)),

two-two-two-TypeI-MultiPanel-Restriction                BIT STRING (SIZE (64)),

two-eight-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (32)),

four-four-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (16)),

two-four-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (128)),

four-two-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (64))

},

ri-Restriction                          BIT STRING (SIZE (4))

}

},

codebookMode                                        INTEGER (1..2)

},

type2                                   SEQUENCE {

subType                                 CHOICE {

typeII                                  SEQUENCE {

n1-n2-codebookSubsetRestriction         CHOICE {

two-one                                 BIT STRING (SIZE (16)),

two-two                                 BIT STRING (SIZE (43)),

four-one                                BIT STRING (SIZE (32)),

three-two                               BIT STRING (SIZE (59)),

six-one                                 BIT STRING (SIZE (48)),

four-two                                BIT STRING (SIZE (75)),

eight-one                               BIT STRING (SIZE (64)),

four-three                              BIT STRING (SIZE (107)),

six-two                                 BIT STRING (SIZE (107)),

twelve-one                              BIT STRING (SIZE (96)),

four-four                               BIT STRING (SIZE (139)),

eight-two                               BIT STRING (SIZE (139)),

sixteen-one                             BIT STRING (SIZE (128))

},

typeII-RI-Restriction                   BIT STRING (SIZE (2))

},

typeII-PortSelection                    SEQUENCE {

portSelectionSamplingSize               ENUMERATED {n1, n2, n3, n4}                   OPTIONAL,       -- Need R

typeII-PortSelectionRI-Restriction      BIT STRING (SIZE (2))

}

},

phaseAlphabetSize                       ENUMERATED {n4, n8},

subbandAmplitude                        BOOLEAN,

numberOfBeams                           ENUMERATED {two, three, four}

}

}

}

CodebookConfig-r16  ::=                SEQUENCE  {

codebookType                           CHOICE {

type2                                  SEQUENCE {

subType                                CHOICE {

typeII-r16                             SEQUENCE  {

n1-n2-codebookSubsetRestriction-r16    CHOICE {

two-one                                BIT STRING (SIZE (16)),

two-two                                BIT STRING (SIZE (43)),

four-one                               BIT STRING (SIZE (32)),

three-two                              BIT STRING (SIZE (59)),

six-one                                BIT STRING (SIZE (48)),

four-two                               BIT STRING (SIZE (75)),

eight-one                              BIT STRING (SIZE (64)),

four-three                             BIT STRING (SIZE (107)),

six-two                                BIT STRING (SIZE (107)),

twelve-one                             BIT STRING (SIZE (96)),

four-four                              BIT STRING (SIZE (139)),

eight-two                              BIT STRING (SIZE (139)),

sixteen-one                            BIT STRING (SIZE (128))

},

typeII-RI-Restriction-r16              BIT STRING (SIZE(4))

},

typeII-PortSelection-r16  SEQUENCE {

portSelectionSamplingSize-r16          ENUMERATED {n1, n2, n3, n4},

typeII-PortSelectionRI-Restriction-r16 BIT STRING (SIZE (4))

}

},

numberOfPMI-SubbandsPerCQI-Subband-r16 INTEGER (1..2),

paramCombination-r16                   INTEGER (1..8)

}

}

}

CodebookConfig-r17  ::=               SEQUENCE  {

codebookType                          CHOICE   {

type1                                 SEQUENCE  {

typeI-SinglePanel-Group1-r17          SEQUENCE {

nrOfAntennaPorts                      CHOICE {

two                                   SEQUENCE {

twoTX-CodebookSubsetRestriction1-r17  BIT STRING (SIZE (6))

},

moreThanTwo                            SEQUENCE {

n1-n2                                        CHOICE {

two-one-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction1-r17      BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction1-r17      BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction1-r17    BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction1-r17    BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction1-r17   BIT STRING (SIZE (64))

}

}

}

}                                                                                       OPTIONAL,  -- Need R

typeI-SinglePanel-Group2-r17           SEQUENCE {

nrOfAntennaPorts                       CHOICE {

two                                    SEQUENCE {

twoTX-CodebookSubsetRestriction2-r17   BIT STRING (SIZE (6))

},

moreThanTwo                            SEQUENCE {

n1-n2                                        CHOICE {

two-one-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction2-r17      BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction2-r17      BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction2-r17    BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction2-r17    BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction2-r17   BIT STRING (SIZE (64))

}

}

}

}                                                                                       OPTIONAL,  -- Need R

typeI-SinglePanel-ri-RestrictionSTRP-r17                    BIT STRING (SIZE (8))       OPTIONAL,  -- Need R

typeI-SinglePanel-ri-RestrictionSDM-r17                     BIT STRING (SIZE (4))       OPTIONAL   -- Need R

},

type2                                 SEQUENCE {

typeII-PortSelection-r17              SEQUENCE {

paramCombination-r17                   INTEGER (1..8),

valueOfN-r17                           ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r17 INTEGER(1..2)                                OPTIONAL,  -- Need R

typeII-PortSelectionRI-Restriction-r17 BIT STRING (SIZE (4))

}

}

}

}

CodebookConfig-v1730  ::=             SEQUENCE  {

codebookType                          CHOICE   {

type1                                 SEQUENCE  {

codebookMode                          INTEGER (1..2)                                    OPTIONAL   -- Need R

}

}

}

CodebookConfig-r18  ::=                   SEQUENCE {

codebookType                              CHOICE {

type2                                     CHOICE {

typeII-CJT-r18                            SEQUENCE {

n1-n2-codebookSubsetRestrictionList-r18   SEQUENCE (SIZE (1..4)) OF CBSR-r18,

paramCombination-CJT-r18                  INTEGER (1..7),

paramCombination-CJT-L-r18                SEQUENCE (SIZE (1..4)) OF INTEGER (1..5),

restrictedCMR-Selection-r18               ENUMERATED {enable},

valueOfO3-r18                             ENUMERATED {n1, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

typeII-RI-Restriction-r18                 BIT STRING (SIZE (4)),

codebookMode-r18                          INTEGER (1..2)

},

typeII-CJT-PortSelection-r18              SEQUENCE {

paramCombination-CJT-PS-r18               INTEGER (1..5),

paramCombination-CJT-PS-alpha-r18         SEQUENCE (SIZE (1..4)) OF INTEGER (1..8),

restrictedCMR-Selection-r18               ENUMERATED {enable},

valueOfO3-r18                             ENUMERATED {n1, n4}                          OPTIONAL,  -- Need R

valueOfN-CJT-r18                          ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

typeII-PortSelectionRI-Restriction-r18    BIT STRING (SIZE (4)),

codebookMode-r18                          INTEGER (1..2)

},

typeII-Doppler-r18                        SEQUENCE {

n1-n2-codebookSubsetRestriction-r18       CBSR-r18,

paramCombination-Doppler-r18              INTEGER (1..9),

td-dd-config-r18                          TD-DD-Config-r18,

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

predictionDelay-r18                       ENUMERATED {m0,n0,n1,n2 },

typeII-RI-Restriction-r18                 BIT STRING (SIZE (4))

},

typeII-DopplerPortSelection-r18           SEQUENCE {

paramCombinationDoppler-PS-r18            INTEGER (1..8),

td-dd-config-r18                          TD-DD-Config-r18,

valueOfN-Doppler-r18                      ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

predictionDelay-r18                       ENUMERATED {m0,n0,n1,n2 },

typeII-PortSelectionRI-Restriction-r18    BIT STRING (SIZE (4))

}

}

}

}

CBSR-r18 ::=    CHOICE {

two-one                                BIT STRING (SIZE (8)),

two-two                                BIT STRING (SIZE (27)),

four-one                               BIT STRING (SIZE (16)),

three-two                              BIT STRING (SIZE (35)),

six-one                                BIT STRING (SIZE (24)),

four-two                               BIT STRING (SIZE (43)),

eight-one                              BIT STRING (SIZE (32)),

four-three                             BIT STRING (SIZE (59)),

six-two                                BIT STRING (SIZE (59)),

twelve-one                             BIT STRING (SIZE (48)),

four-four                              BIT STRING (SIZE (75)),

eight-two                              BIT STRING (SIZE (75)),

sixteen-one                            BIT STRING (SIZE (64))

}

TD-DD-Config-r18 ::=        SEQUENCE {

vectorLengthDD-r18           ENUMERATED {n1,n2,n4,n8},

unitDurationDD-r18           ENUMERATED {m1,m2}                                    OPTIONAL,  -- Need R

aperiodicResourceOffset-r18  INTEGER(1..2)                                         OPTIONAL,  -- Need R

tdCQI-r18                    ENUMERATED {n11,n12,n2, spare1}                              OPTIONAL   -- Need R

}

-- TAG-CODEBOOKCONFIG-STOP

-- ASN1STOP

CodebookConfig field descriptions

codebookMode CodebookMode as specified in TS 38.214 [19], clause 5.2.2.2 8 and 5.2.2.9.

codebookType CodebookType including possibly sub-types and the corresponding parameters for each (see TS 38.214 [19], clause 5.2.2.2).

n1-n2-codebookSubsetRestriction, n1-n2-codebookSubsetRestrictionList Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.3). Number of bits for codebook subset restriction is CEIL(log2(nchoosek(O1*O2,4)))+8*n1*n2 where nchoosek(a,b) = a!/(b!(a-b)!). The number of elements in n1-n2-codebookSubSetRestrictionList is up to the number of elements of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet(S) indicated by nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig in which the CodebookConfig is included. Each element in n1-n2-codebookSubsetRestrictionList has same n1-n2 bitstring.

n1-n2 Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.1).

ng-n1-n2 Codebook subset restriction for Type I Multi-panel codebook (see TS 38.214 [19], clause 5.2.2.2.2).

numberOfBeams Number of beams, L, used for linear combination.

numberOfPMI-SubbandsPerCQI-Subband Field indicates how PMI subbands are defined per CQI subband according to TS 38.214 [19], clause 5.2.2.2.5, 5.2.2.2.8 and 5.2.2.9.

paramCombination, paramCombination-CJT-r18, paramCombination-CJT-L-r18, paramCombination-CJT-PS-r18, paramCombination-CJT-PS-alpha, paramCombinationDoppler-r18, paramCombinationDoppler-PS-r18 Field describes supported parameter combination (M, , ) as specified in TS 38.214 [19] Clause 5.2.2. For fields paramCombination-CJT-L-r18 or paramCombination-CJT-PS-alpha the number of elements is 1, 2 or 4, and the values are configured according to the number of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet elements from Tables 5.2.2.2.8-1 and 5.2.2.2.9-1 in TS 38.214 [19], respectively. The allowed configuration combinations for paramCombination-CJT-r18 and paramCombination-CJT-L-r18 are given in Table 5.2.2.2.8-3 in TS 38.214 [19]. The allowed configuration combinations for paramCombination-CJT-PS-r18 and paramCombination-CJT-PS-alpha-r18 are given in Table 5.2.2.2.9-3 in TS 38.214 [19].

phaseAlphabetSize The size of the PSK alphabet, QPSK or 8-PSK.

portSelectionSamplingSize The size of the port selection codebook (parameter d), see TS 38.214 [19] clause 5.2.2.2.6.

predictionDelay Prediction delay for Doppler and Doppler port selection codebooks see TS 38.214 [19], Clause 5.2.1.4. The first value m0 means that the first slot for which the CSI corresponds to is the slot where the CSI reference resource is located at. For the other three candidate values (n0, n1, n2), then the first slot for which the CSI corresponds to is given by l= n+delta, where delta can take on values of 0, 1, 2 and n the slot in which CSI is reported.

restrictedCMR-Selection Enabling CSI-RS resource (CMR) restriction, (see TS 38.214 [19], clause 5.2.2.2.8)

ri-Restriction Restriction for RI for TypeI-MultiPanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.2).

subbandAmplitude If subband amplitude reporting is activated (true).

twoTX-CodebookSubsetRestriction Codebook subset restriction for 2TX codebook (see TS 38.214 [19] clause 5.2.2.2.1).

typeI-SinglePanel-codebookSubsetRestriction-i2 i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI (see TS 38.214 [19] clause 5.2.2.2.1).

typeI-SinglePanel-ri-Restriction Restriction for RI for TypeI-SinglePanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.1).

typeI-SinglePanel-Group1, typeI-SinglePanel-Group2 Configures codebooks for CSI calculation when UE is configured with two CMR Groups with CMRGroupingAndPairing in the NZP-CSI-RS-ResourceSet associated with the CSI-ReportConfig. Network configures the same number of ports for both codebooks.

typeI-SinglePanel-ri-RestrictionSDM, typeI-SinglePanel-ri-RestrictionSTRP Restriction for RI for N Resource Pairs when two CMR Groups are configured with CMRGroupingAndPairing in the NZP-CSI-RS-ResourceSet associated with the CSI-ReportConfig (see TS 38.214 [19], clause 5.2.1.4.2).

typeII-PortSelectionRI-Restriction Restriction for RI for TypeII-PortSelection-RI-Restriction (see TS 38.214 [19], clauses 5.2.2.2.4, 5.2.2.2.6 and 5.2.2.2.7).

typeII-RI-Restriction Restriction for RI for TypeII-RI-Restriction (see TS 38.214 [19], clauses 5.2.2.2.3 and 5.2.2.2.5).

valueOfN Field provides the value of parameter N as specified in TS 38.214 [19], clause 5.2.2.2.7. The field is present only when M=2 set by paramCombination, see TS 38.214 [19].

valueOfO3 O3: oversampling factor for frequency-domain basis selection offset Only applicable when CodebookMode = 'Mode1'.

valueOfN-CJT Field describes the size of the window of FD basis for Rel-18 Type II CJT PS codebook. For M=2, N=2,4 (see TS 38.214 [19], clause 5.2.2.2.9)

valueOfN-Doppler Field describes the size of the window of FD basis for Rel-18 Type II Doppler PS codebook. For M=2, N=2,4 (see TS 38.214 [19], clause 5.2.2.2.9)

TD-DD-Config field descriptions

aperiodicResourceOffset Offset m between two consecutive aperiodic CSI-RS resources. See TS 38.214 [19], clause 5.2.1.4.

tdCQI Number of TD CQIs (X) in each SB and ref slot(s) + ref W2(s). 1-1: 1 TD CQI, ref = 1st slot and 1st W2 1-2: 1 TD CQI, ref = 1st and last slots, and 1st and last W2 2: 2 TC CQIs (see TS 38.214 [19], clause 5.2.1.4).

unitDurationDD DD/TD unit duration (in slots), for AP-CSI-RS only. Note: For P/SP-CSI-RS, d is set to CSI-RS periodicity (see TS 38.214 [19], clause 5.2.1.4). If the value of aperiodicResourceOffset is '1', network configures value 'm1' for this field.

vectorLengthDD Doppler-/time-domain (DD/TD) basis vector length; See TS 38.214 [19], clause 5.2.1.4.

#### –	CommonLocationInfo

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo information element

-- ASN1START

-- TAG-COMMONLOCATIONINFO-START

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

-- TAG-COMMONLOCATIONINFO-STOP

-- ASN1STOP

CommonLocationInfo field descriptions

gnss-TOD-msec Parameter type gnss-TOD-msec defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationTimeStamp Parameter type DisplacementTimeStamp defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationError Parameter LocationError defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter LocationSource defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

velocityEstimate Parameter type Velocity defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

#### –	CondReconfigId

The IE CondReconfigId is used to identify a CHO, CPA, CPC, subsequent CPAC, CHO with target SCG, or CHO with candidate SCG(s) configuration.

CondReconfigId information element

-- ASN1START

-- TAG-CONDRECONFIGID-START

CondReconfigId-r16 ::=                    INTEGER (1.. maxNrofCondCells-r16)

-- TAG-CONDRECONFIGID-STOP

-- ASN1STOP

#### –	CondReconfigToAddModList

The IE CondReconfigToAddModList concerns a list of conditional reconfigurations to add or modify, with for each entry the condReconfigId and the associated fields.

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondPSCell-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigCHO-WithSCG

subsequentCondReconfig-r18       SubsequentCondReconfig-r18                            OPTIONAL,    -- Need M

securityCellSetId-r18            SecurityCellSetId-r18                                 OPTIONAL,    -- Need M

scpac-ConfigComplete-r18         ENUMERATED {true}                                     OPTIONAL     -- Cond CPAC

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

SubsequentCondReconfig-r18 ::=   SEQUENCE {

condExecutionCondToReleaseList-r18   CondExecutionCondToReleaseList-r18                OPTIONAL,   -- Need N

condExecutionCondToAddModList-r18    CondExecutionCondToAddModList-r18                 OPTIONAL,   -- Need N

...

}

CondExecutionCondToAddModList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondExecutionCondToAddMod-r18

CondExecutionCondToAddMod-r18 ::=    SEQUENCE {

subsequentCondReconfigId-r18         CondReconfigId-r16,

subsequentCondExecutionCond-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

subsequentCondExecutionCondSCG-r18   OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL,    -- Need M

...

}

CondExecutionCondToReleaseList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for CHO, CPA, intra-SN CPC without MN involvement, or MN initiated inter-SN CPC. When configuring 2 triggering events (Meas Ids) for a candidate cell, the network ensures that both refer to the same measObject. The network configures at most one from condEventD1, condEventD2 or condEventT1 for the same candidate cell. For CPA and for MN-initiated inter-SN CPC, the network only indicates MeasId(s) associated with condEventA4. For intra-SN CPC, the network only indicates MeasId(s) associated with condEventA3 or condEventA5.

condExecutionCondPSCell The execution condition that needs to be fulfilled for the associated PSCell in order to trigger the execution of a conditional reconfiguration for CHO with candidate SCG(s). The Meas Ids refer to the measConfig associated with the MCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. The network only indicates MeasId(s) associated with condEventA4.

condExecutionCondSCG Contains execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for SN initiated inter-SN CPC. The Meas Ids refer to the measConfig associated with the SCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. For each condReconfigId, the network always configures either condExecutionCond or condExecutionCondSCG (not both). The network only indicates MeasId(s) associated with condEventA3 or condEventA5.

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config.

scpac-ConfigComplete This field indicates whether the configuration contained in condRRCReconfig for subsequent CPAC is a complete configuration.

subsequentCondReconfig Contains the execution conditions that need to be fulfilled in order to trigger the execution of a subsequent CPAC. If the field is configured, the configuration of candidate PSCells for subsequent CPAC is supported. The subsequent execution condition is used for conditional reconfiguration evaluation for other candidate cells when the RRCReconfiguration message contained in condRRCReconfig has been applied.

CondExecutionCondToAddMod field descriptions

subsequentCondExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for SN initiated intra-SN subsequent CPAC without MN involvement. When configuring 2 triggering events (Meas Ids) for a candidate cell, the network ensures that both refer to the same measObject. The network only indicates MeasId(s) associated with condEventA3 or condEventA5.

subsequentCondExecutionCondSCG Contains execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for SN initiated inter-SN subsequent CPAC, SN initiated intra-SN subsequent CPAC with MN involvement, or MN initiated inter-SN subsequent CPAC. The Meas Ids refer to the measConfig associated with the SCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. The network only indicates MeasId(s) associated with condEventA3 or condEventA5.

| Conditional Presence | Explanation |
| --- | --- |
| condReconfigAdd | The field is mandatory present when a condReconfigId is being added. Otherwise the field is optional, need M. |
| condReconfigCHO-WithSCG | This field is optional present, need M, if the RRCReconfiguration message contained in corresponding condRRCReconfig includes the nr-SCG and condExecutionCond is configured. Otherwise, it is absent. |
| CPAC | The field is optionally present, need M, when the conditional reconfiguration includes at least one candidate PSCell supporting subsequent CPAC. Otherwise, the field is absent, need R. |

#### –	ConditionalReconfiguration

The IE ConditionalReconfiguration is used to add, modify and release the configuration of conditional reconfiguration.

ConditionalReconfiguration information element

-- ASN1START

-- TAG-CONDITIONALRECONFIGURATION-START

ConditionalReconfiguration-r16 ::=   SEQUENCE {

attemptCondReconfig-r16              ENUMERATED {true}              OPTIONAL,   -- Cond CHO

condReconfigToRemoveList-r16         CondReconfigToRemoveList-r16   OPTIONAL,   -- Need N

condReconfigToAddModList-r16         CondReconfigToAddModList-r16   OPTIONAL,   -- Need N

...,

[[

scpac-ReferenceConfiguration-r18     SetupRelease {ReferenceConfiguration-r18}          OPTIONAL,   -- Need M

servingSecurityCellSetId-r18         SecurityCellSetId-r18                              OPTIONAL,   -- Cond condInitialSCPAC

sk-CounterConfiguration-r18          SK-CounterConfiguration-r18                        OPTIONAL    -- Cond condInitialSCPAC

]]

}

CondReconfigToRemoveList-r16 ::=     SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

SK-CounterConfiguration-r18      ::= SEQUENCE {

sk-CounterConfigToReleaseList-r18    SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SecurityCellSetId-r18    OPTIONAL,    -- Need N

sk-CounterConfigToAddModList-r18     SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfig-r18     OPTIONAL     -- Need N

}

SK-CounterConfig-r18 ::=             SEQUENCE {

securityCellSetId-r18                SecurityCellSetId-r18,

sk-CounterList-r18                   SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

SecurityCellSetId-r18 ::= INTEGER (1.. maxSecurityCellSet-r18)

-- TAG-CONDITIONALRECONFIGURATION-STOP

-- ASN1STOP

ConditionalReconfiguration field descriptions

attemptCondReconfig If present, the UE shall perform conditional reconfiguration if selected cell is a target candidate cell and it is the first cell selection after failure as described in clause 5.3.7.3.

condReconfigToAddModList List of the configuration of candidate SpCells to be added or modified for CHO, CPA or CPC.

condReconfigToRemoveList List of the configuration of candidate SpCells to be removed.

scpac-ReferenceConfiguration Includes the reference configuration for the candidate supporting subsequent CPAC.

securityCellSetId This field is used to determine whether UE should perform security update when conditional reconfiguration containing subsequentCondReconfig is executed.

servingSecurityCellSetId This field identifies the security cell set for serving PSCell.

sk-counterConfiguration Includes a list of sk-Counter from which the UE should select the sk-counter used to derive S-KgNB for inter-SN subsequent CPAC. If this field is configured, the network shall not configure the field sk-Counter within the RRCReconfiguration message for conditional reconfiguration execution for subsequent CPAC.

| Conditional Presence | Explanation |
| --- | --- |
| CHO | The field is optional present, Need R, if the UE is configured with at least a candidate SpCell for CHO. Otherwise the field is not present. |
| condInitialSCPAC | The field is mandatory present upon the initial conditional reconfiguration, generated by the MN, which includes at least one inter-SN candidate PSCell supporting subsequent CPAC. The field is absent for any conditional reconfiguration generated by the SN. Otherwise, the field is optional, need M. |

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2). Multiple Configured Grant configurations may be configured in one BWP of a serving cell.

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                INTEGER (1..64)                                                         OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant           SEQUENCE {

timeDomainOffset                    INTEGER (0..5119),

timeDomainAllocation                INTEGER (0..15),

frequencyDomainAllocation           BIT STRING (SIZE(18)),

antennaPort                         INTEGER (0..31),

dmrs-SeqInitialization              INTEGER (0..1)                                                         OPTIONAL,   -- Need R

precodingAndNumberOfLayers          INTEGER (0..63),

srs-ResourceIndicator               INTEGER (0..15)                                                        OPTIONAL,   -- Need R

mcsAndTBS                           INTEGER (0..31),

frequencyHoppingOffset              INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                          OPTIONAL,   -- Need R

pathlossReferenceIndex              INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...,

[[

pusch-RepTypeIndicator-r16          ENUMERATED {pusch-RepTypeA,pusch-RepTypeB}                             OPTIONAL,   -- Need M

frequencyHoppingPUSCH-RepTypeB-r16  ENUMERATED {interRepetition, interSlot}                                OPTIONAL,   -- Cond RepTypeB

timeReferenceSFN-r16                ENUMERATED {sfn512}                                                    OPTIONAL    -- Need S

]],

[[

pathlossReferenceIndex2-r17         INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)                       OPTIONAL,   -- Need R

srs-ResourceIndicator2-r17          INTEGER (0..15)                                                        OPTIONAL,   -- Need R

precodingAndNumberOfLayers2-r17     INTEGER (0..63)                                                        OPTIONAL,   -- Need R

timeDomainAllocation-v1710          INTEGER (16..63)                                                       OPTIONAL,   -- Need M

timeDomainOffset-r17                INTEGER (0..40959)                                                     OPTIONAL,   -- Need R

cg-SDT-Configuration-r17            CG-SDT-Configuration-r17                                               OPTIONAL    -- Need M

]],

[[

srs-ResourceSetId-r18               SRS-ResourceSetId                                                      OPTIONAL,   -- Need R

cg-LTM-Configuration-r18            CG-RRC-Configuration-r18                                               OPTIONAL, -- Cond LTM

cg-SDT-PeriodicityExt-r18          ENUMERATED {

sym1x14x1280, sym2x14x1280, sym4x14x1280 , sym8x14x1280, sym16x14x1280,

sym32x14x1280, sym48x14x1280, sym64x14x1280, sym96x14x1280, sym128x14x1280,

sym192x14x1280, sym240x14x1280, sym256x14x1280, sym384x14x1280, sym472x14x1280,

sym480x14x1280, sym512x14x1280, sym768x14x1280, sym944x14x1280, sym960x14x1280,

sym1408x14x1280, sym1536x14x1280, sym1888x14x1280, sym1920x14x1280,

sym2816x14x1280, sym3072x14x1280, sym3776x14x1280, sym5632x14x1280,

sym6144x14x1280, sym7552x14x1280, sym7680x14x1280, sym11264x14x1280,

sym15104x14x1280, sym15360x14x1280, sym22528x14x1280, sym30208x14x1280,

sym45056x14x1280, sym60416x14x1280, sym90112x14x1280, sym180224x14x1280,

sym4x12x1280, sym8x12x1280, sym16x12x1280, sym32x12x1280, sym192x12x1280,

sym384x12x1280, sym960x12x1280, sym1888x12x1280, sym3776x12x1280,

sym5632x12x1280, sym11264x12x1280, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1

}                                                                      OPTIONAL,   -- Need R

timeReferenceHyperSFN-r18           INTEGER (0..1023)                                                      OPTIONAL,   -- Need R

cg-RRC-Configuration-r18            CG-RRC-Configuration-r18                                       OPTIONAL, -- Cond RACH-LessHO

applyIndicatedTCI-State-r18         ENUMERATED {first, second, both}                                       OPTIONAL    -- Need R

]]

}                                                                                                              OPTIONAL,   -- Need R

...,

[[

cg-RetransmissionTimer-r16          INTEGER (1..64)                                                            OPTIONAL,   -- Need R

cg-minDFI-Delay-r16                 ENUMERATED

{sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14,

sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14,sym15x14, sym16x14

}                                                       OPTIONAL,   -- Need R

cg-nrofPUSCH-InSlot-r16             INTEGER (1..7)                                                  OPTIONAL,   -- Need R

cg-nrofSlots-r16                    INTEGER (1..40)                                                 OPTIONAL,   -- Need R

cg-StartingOffsets-r16              CG-StartingOffsets-r16                                          OPTIONAL,   -- Need R

cg-UCI-Multiplexing-r16             ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

cg-COT-SharingOffset-r16            INTEGER (1..39)                                                 OPTIONAL,   -- Need R

betaOffsetCG-UCI-r16                INTEGER (0..31)                                                 OPTIONAL,   -- Need R

cg-COT-SharingList-r16              SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16                 OPTIONAL,   -- Need R

harq-ProcID-Offset-r16              INTEGER (0..15)                                                 OPTIONAL,   -- Need M

harq-ProcID-Offset2-r16             INTEGER (0..15)                                                 OPTIONAL,   -- Need M

configuredGrantConfigIndex-r16      ConfiguredGrantConfigIndex-r16                                  OPTIONAL,   -- Cond CG-List

configuredGrantConfigIndexMAC-r16   ConfiguredGrantConfigIndexMAC-r16                               OPTIONAL,   -- Cond CG-IndexMAC

periodicityExt-r16                  INTEGER (1..5120)                                               OPTIONAL,   -- Need R

startingFromRV0-r16                 ENUMERATED {on, off}                                            OPTIONAL,   -- Need R

phy-PriorityIndex-r16               ENUMERATED {p0, p1}                                             OPTIONAL,   -- Need R

autonomousTx-r16                    ENUMERATED {enabled}                                OPTIONAL    -- Cond LCH-BasedPrioritization

]],

[[

cg-betaOffsetsCrossPri0-r17         SetupRelease { BetaOffsetsCrossPriSelCG-r17 }                   OPTIONAL,   -- Need M

cg-betaOffsetsCrossPri1-r17         SetupRelease { BetaOffsetsCrossPriSelCG-r17 }                   OPTIONAL,   -- Need M

mappingPattern-r17                  ENUMERATED {cyclicMapping, sequentialMapping}                   OPTIONAL,   -- Cond SRSsets

sequenceOffsetForRV-r17             INTEGER (0..3)                                                  OPTIONAL,   -- Need R

p0-PUSCH-Alpha2-r17                 P0-PUSCH-AlphaSetId                                             OPTIONAL,   -- Need R

powerControlLoopToUse2-r17          ENUMERATED {n0, n1}                                             OPTIONAL,   -- Need R

cg-COT-SharingList-r17              SEQUENCE (SIZE (1..50722)) OF CG-COT-Sharing-r17                OPTIONAL,   -- Need R

periodicityExt-r17                  INTEGER (1..40960)                                              OPTIONAL,   -- Need R

repK-v1710                          ENUMERATED {n12, n16, n24, n32}                                 OPTIONAL,   -- Need R

nrofHARQ-Processes-v1700            INTEGER(17..32)                                                 OPTIONAL,   -- Need M

harq-ProcID-Offset2-v1700           INTEGER (16..31)                                                OPTIONAL,   -- Need R

configuredGrantTimer-v1700          INTEGER(33..288)                                                OPTIONAL,   -- Need R

cg-minDFI-Delay-v1710               INTEGER (238..3584)                                             OPTIONAL    -- Need R

]],

[[

harq-ProcID-Offset-v1730            INTEGER (16..31)                                                OPTIONAL,   -- Need R

cg-nrofSlots-r17                    INTEGER (1..320)                                                OPTIONAL    -- Need R

]],

[[

disableCG-RetransmissionMonitoring-r18  ENUMERATED {true}                                           OPTIONAL,   -- Need R

nrofSlotsInCG-Period-r18            INTEGER (2..32)                                                 OPTIONAL,   -- Need R

uto-UCI-Config-r18                      SEQUENCE {

nrofBitsInUTO-UCI-r18               INTEGER (3..8),

betaOffsetUTO-UCI-r18               INTEGER (0..31),

...

}                                                                                                   OPTIONAL    -- Need R

]]

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

CG-COT-Sharing-r16 ::= CHOICE {

noCOT-Sharing-r16                   NULL,

cot-Sharing-r16                     SEQUENCE {

duration-r16                       INTEGER (1..39),

offset-r16                         INTEGER (1..39),

channelAccessPriority-r16          INTEGER (1..4)

}

}

CG-COT-Sharing-r17 ::=  CHOICE {

noCOT-Sharing-r17                   NULL,

cot-Sharing-r17                     SEQUENCE {

duration-r17                       INTEGER (1..319),

offset-r17                         INTEGER (1..319)

}

}

CG-StartingOffsets-r16 ::= SEQUENCE {

cg-StartingFullBW-InsideCOT-r16         SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingFullBW-OutsideCOT-r16        SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingPartialBW-InsideCOT-r16      INTEGER (0..6)                                       OPTIONAL,   -- Need R

cg-StartingPartialBW-OutsideCOT-r16     INTEGER (0..6)                                       OPTIONAL    -- Need R

}

BetaOffsetsCrossPriSelCG-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (1..4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17      BetaOffsetsCrossPri-r17

}

CG-SDT-Configuration-r17 ::= SEQUENCE {

cg-SDT-RetransmissionTimer   INTEGER (1..64)                                                 OPTIONAL,   -- Need R

sdt-SSB-Subset-r17       CHOICE {

shortBitmap-r17          BIT STRING (SIZE (4)),

mediumBitmap-r17         BIT STRING (SIZE (8)),

longBitmap-r17           BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

sdt-SSB-PerCG-PUSCH-r17   ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

sdt-P0-PUSCH-r17         INTEGER (-16..15)                                                   OPTIONAL, -- Need M

sdt-Alpha-r17            ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M

sdt-DMRS-Ports-r17       CHOICE {

dmrsType1-r17            BIT STRING (SIZE (8)),

dmrsType2-r17            BIT STRING (SIZE (12))

}                                                                                            OPTIONAL,  -- Need M

sdt-NrofDMRS-Sequences-r17  INTEGER (1..2)                                                   OPTIONAL   -- Need M

}

CG-RRC-Configuration-r18 ::=   SEQUENCE {

cg-RRC-RetransmissionTimer-r18 INTEGER (1..64)                                               OPTIONAL,   -- Need R

cg-RRC-RSRP-ThresholdSSB-r18   RSRP-Range                                                    OPTIONAL,   -- Need R

rrc-SSB-Subset-r18             CHOICE {

shortBitmap-r18                BIT STRING (SIZE (4)),

mediumBitmap-r18               BIT STRING (SIZE (8)),

longBitmap-r18                 BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

rrc-SSB-PerCG-PUSCH-r18        ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

rrc-P0-PUSCH-r18               INTEGER (-16..15)                                             OPTIONAL,   -- Need M

rrc-Alpha-r18                  ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M

rrc-DMRS-Ports-r18             CHOICE {

dmrsType1-r18                  BIT STRING (SIZE (8)),

dmrsType2-r18                  BIT STRING (SIZE (12))

}                                                                                             OPTIONAL,  -- Need M

rrc-NrofDMRS-Sequences-r18  INTEGER (1..2)                                                    OPTIONAL,  -- Need M

...

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

ConfiguredGrantConfig field descriptions

antennaPort Indicates the antenna port(s) to be used for this configuration, and the maximum bitwidth is 5. See TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1. The UE ignores this field in case of CG-SDT.

applyIndicatedTCI-State This field indicates, for PUSCH transmission(s) corresponding a Type1-CG configuration, if UE applies the first, the second or both "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5. If more than one value for the field coresetPoolIndex is configured in IE controlResourceSet for the BWP, the value 'first' corresponds to the "indicated" joint/UL TCI states specific to coresetPoolIndex value 0 and the value 'second' correspond to the coresetPoolIndex value 1, respectively. In this case, network does not configure the value 'both'.

autonomousTx If this field is present, the Configured Grant configuration is configured with autonomous transmission, see TS 38.321 [3].

betaOffsetCG-UCI Beta offset for CG-UCI in CG-PUSCH, see TS 38.213 [13], clause 9.3

betaOffsetUTO-UCI Beta offset value for UTO-UCI multiplexing on CG PUSCH, see TS 38.213 [13], clause 9.3.

cg-betaOffsetsCrossPri0, cg-betaOffsetsCrossPri1 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK in CG-PUSCH with different priorities. The field cg-betaOffsetsCrossPri0 indicates multiplexing LP HARQ-ACK in HP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p1. The field cg-betaOffsetsCrossPri1 indicates multiplexing HP HARQ-ACK in LP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p0.

cg-COT-SharingList Indicates a table for COT sharing combinations (see 37.213 [48], clause 4.1.3). One row of the table can be set to noCOT-Sharing to indicate that there is no channel occupancy sharing. If the cg-RetransmissionTimer-r16 is configured and the UE operates as an initiating device in semi-static channel access mode (see TS 37.213 [48], clause 4.3), then cg-COT-SharingList-r16 is configured.

cg-COT-SharingOffset Indicates the offset from the end of the slot where the COT sharing indication in UCI is enabled where the offset in symbols is equal to 14*n, where n is the signaled value for cg-COT-SharingOffset. Applicable when ul-toDL-COT-SharingED-Threshold-r16 is not configured (see 37.213 [48], clause 4.1.3).

cg-DMRS-Configuration DMRS configuration (see TS 38.214 [19], clause 6.1.2.3).

cg-minDFI-Delay Indicates the minimum duration (in unit of symbols) from the ending symbol of the PUSCH to the starting symbol of the PDCCH containing the downlink feedback indication (DFI) carrying HARQ-ACK for this PUSCH. The HARQ-ACK received before this minimum duration is not considered as valid for this PUSCH (see TS 38.213 [13], clause 10.5). The following minimum duration values are supported, depending on the configured subcarrier spacing [symbols]: 15 kHz: 7, m*14, where m = {1, 2, 3, 4} 30 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8} 60 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} 120 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32} 480 kHz: m*14, where m = {2, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100, 104, 108, 112, 116, 120, 124, 128} 960 kHz: m*14, where m = {4, 8, 16, 24, 32, 40, 48, 56, 64, 72, 80, 88, 96, 104, 112, 120, 128, 136, 144, 152, 160, 168, 176, 184, 192, 200, 208, 216, 224, 232, 240, 248, 256}

cg-nrofPUSCH-InSlot Indicates the number of consecutive PUSCH configured to CG within a slot where the SLIV indicating the first PUSCH and additional PUSCH appended with the same length (see TS 38.214 [19], clause 6.1.2.3). The network can only configure this field if cg-RetransmissionTimer is configured.

cg-nrofSlots Indicates the number of allocated slots in a configured grant periodicity following the time instance of configured grant offset (see TS 38.214 [19], clause 6.1.2.3). cg-nrofSlots-r17 is only applicable for operation with shared spectrum channel access in FR2-2. When cg-nrofSlots-r17 is configured, the UE shall ignore cg-nrofSlots-r16. The network can only configure this field if cg-RetransmissionTimer is configured.

cg-RetransmissionTimer Indicates the initial value of the configured retransmission timer (see TS 38.321 [3]) in multiples of periodicity. The value of cg-RetransmissionTimer is always less than or equal to the value of configuredGrantTimer. This field is always configured together with harq-ProcID-Offset. This field is not configured for operation in licensed spectrum or simultaneously with harq-ProcID-Offset2. The network does not configure this field for CG-SDT.

cg-SDT-PeriodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 (see TS 38.321 [3], clause 5.8.2) for extended CG-SDT periodicities. If this field is present, the fields periodicity and periodicityExt are ignored. The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: n*14*1280, where n={1, 2, 4, 8, 48, 96, 240, 472, 944, 1408, 2816} 30 kHz: n*14*1280, where n={2, 4, 8, 16, 96, 192, 480, 944, 1888, 2816, 5632} 60 kHz with normal CP n*14*1280, where n={4, 8, 16, 32, 192, 384, 960, 1888, 3776, 5632,11264} 60 kHz with ECP: n*12*1280, where n={4, 8, 16, 32, 192, 384, 960, 1888, 3776, 5632,11264} 120 kHz: n*14*1280, where n={8, 16, 32, 64, 384, 768, 1920, 3776, 7552, 11264, 22528} 480 kHz: n*14*1280, where n={32, 64, 128, 256, 1536, 3072, 7680, 15104, 30208, 45056, 90112} 960 kHz: n*14*1280, where n={64, 128, 256, 512, 3072, 6144, 15360, 30208, 60416, 90112, 180224}

cg-StartingOffsets This field is not applicable for a UE which is allowed to operate as an initiating device in semi-static channel access mode, i.e., not applicable for a UE configured with UE FFP parameters (e.g. period, offset) regardless whether the UE would initiate its own COT or would share gNB's COT.

cg-UCI-Multiplexing If present, this field indicates that in the case of PUCCH overlapping with CG-PUSCH(s) including CG-UCI within a PUCCH group, HARQ-ACK is multiplexed on the CG-PUSCH including CG-UCI (see TS 38.213 [13], clause 9).

configuredGrantConfigIndex Indicates the index of the Configured Grant configurations within the BWP.

configuredGrantConfigIndexMAC Indicates the index of the Configured Grant configurations within the MAC entity.

disableCG-RetransmissionMonitoring Indicates that the UE shall disable waking-up to monitor possible grants for retransmissions corresponding to this ConfiguredGrantConfig when DRX is configured. When this field is configured, the UE does not start the drx-HARQ-RTT-TimerUL for PUSCH transmissions using configured uplink grants corresponding to this ConfiguredGrantConfig. See TS 38.321 [3], clause 5.7.

configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321 [3]) in multiples of periodicity. When cg-RetransmissonTimer is configured, if HARQ processes are shared among different configured grants on the same BWP, configuredGrantTimer * periodicity is set to the same value for the configurations that share HARQ processes on this BWP. The value of the extension configuredGrantTimer is 2 times the configured value.

dmrs-SeqInitialization The network configures this field if transformPrecoder is disabled or when the value of sdt-NrofDMRS-Sequences is set to 1. Otherwise, the field is absent.

frequencyDomainAllocation Indicates the frequency domain resource allocation, see TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1).

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured. The field frequencyHopping applies to configured grant for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3.1).

frequencyHoppingOffset Frequency hopping offset used when frequency hopping is enabled (see TS 38.214 [19], clause 6.1.2 and clause 6.3).

frequencyHoppingPUSCH-RepTypeB Indicates the frequency hopping scheme for Type 1 CG when pusch-RepTypeIndicator is set to 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.1). The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, the frequency hopping is not enabled for Type 1 CG.

harq-ProcID-Offset For operation with shared spectrum channel access configured with cg-RetransmissionTimer-r16, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)]. harq-ProcID-Offset-v1730 is only applicable for operation with shared spectrum channel access in FR2-2. If the field harq-ProcID-Offset-v1730 is present, the UE shall ignore the harq-ProcID-Offset-r16. The network does not configure this field for CG-SDT.

harq-ProcID-Offset2 Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.4.1. This field is not configured together with cg-RetransmissionTimer-r16. If the field harq-ProcID-Offset2-v1700 is present, the UE shall ignore the harq-ProcID-Offset2-r16.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' for PUSCH transmission with a Type 1 configured grant and/or a Type 2 configured grant as described in clause 6.1.2.3 of TS 38.214 [19]

mcs-Table Indicates the MCS table the UE shall use for PUSCH without transform precoding. If the field is absent the UE applies the value qam64.

mcs-TableTransformPrecoder Indicates the MCS table the UE shall use for PUSCH with transform precoding. If the field is absent the UE applies the value qam64.

mcsAndTBS The modulation order, target code rate and TB size (see TS 38.214 [19], clause 6.1.2). The NW does not configure the values 28~31 in this version of the specification.

nrofBitsInUTO-UCI Indicates the number of bits in the UTO-UCI bitmap (see TS 38.212 [17], clause 6.2.7, 6.3.2, TS 38.213 [13], clause 9.3.1, TS 38.214 [19], clause 5.2.3). When this field is configured, UTO-UCI is enabled for the UE.

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1. If the UE is configured with nrofHARQ-Processes-v1700, the UE shall ignore nrofHARQ-Processes (without suffix).

nrofSlotsInCG-Period Number of consecutive slots for CG PUSCH transmission occasions in a period of a single CG PUSCH configuration, see TS 38.214 [19], clause 6.1. The network does not configure this field for operation on shared spectrum.

pathlossReferenceIndex Indicates the reference signal index used as PUSCH pathloss reference (see TS 38.213 [13], clause 7.1.1). In case of CG-SDT, the UE does not use this field.

pathlossReferenceIndex2 Indicates the reference signal used as PUSCH pathloss reference for the second SRS resource set. When this field is present, pathlossReferenceIndex indicates the reference signal used as PUSCH pathloss reference for the first SRS resource set

p0-PUSCH-Alpha Index of the P0-PUSCH-AlphaSet to be used for this configuration.

p0-PUSCH-Alpha2 Index of the P0-PUSCH-AlphaSet to be used for second SRS resource set. If this field is present, the p0-PUSCH-Alpha provides index for the P0-PUSCH-AlphaSet to be used for first SRS resource set.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} 480 and 960 kHz: n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} In case of SDT, the network does not configure periodicity values less than 5ms.

periodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). If this field is present, the UE shall ignore field periodicity (without suffix). Network does not configure periodicityExt-r17 together with periodicityExt-r16. The following periodicites are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 640. 30 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP: periodicityExt*14, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: periodicityExt*12, where periodicityExt has a value between 1 and 2560. 120 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 5120. 480 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 20480. 960 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 40960. In case of SDT, the network does not configure periodicity values less than 5ms.

phy-PriorityIndex Indicates the PHY priority of CG PUSCH at least for PHY-layer collision handling. Value p0 indicates low priority and value p1 indicates high priority. The network does not configure this for CG-SDT.

powerControlLoopToUse Closed control loop to apply (see TS 38.213 [13], clause 7.1.1).

powerControlLoopToUse2 Closed control loop to apply to second SRS resource set (see TS 38.213 [13], clause 7.1.1). If this field is present, the powerControlLoopToUse applies to the first SRS resource set.

precodingAndNumberOfLayers Indicates the precoding and number of layers (see TS 38.212 [17], clause 7.3.1.1.2, and TS 38.214 [19], clause 6.1.2.3). In case of CG-SDT, network sets this field to 1.

precodingAndNumberOfLayers2 Indicates the precoding and number of layers for the second SRS resource set. When this field is present, precodingAndNumberOfLayers indicated the precoding and number of layers for the first SRS resource set.

pusch-RepTypeIndicator Indicates whether UE follows the behavior for PUSCH repetition type A or the behavior for PUSCH repetition type B for each Type 1 configured grant configuration. The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B' (see TS 38.214 [19], clause 6.1.2.3). The value pusch-RepTypeB is not configured simultaneously with cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16. The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation with shared spectrum channel access.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent. Note: rbg-Size is used when the transformPrecoder parameter is disabled.

repK-RV The redundancy version (RV) sequence to use. See TS 38.214 [19], clause 6.1.2. The network configures this field if repetitions are used, i.e., if repK is set to n2, n4 or n8. This field is not configured when cg-RetransmissionTimer is configured. Otherwise, the field is absent.

repK Number of repetitions K, see TS 38.214 [19]. If the field repK-v1710 is present, the UE shall ignore the repK (without suffix).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1. For Type 1 UL data transmission without grant, resourceAllocation should be resourceAllocationType0 or resourceAllocationType1.

rrc-ConfiguredUplinkGrant Configuration for "configured grant" transmission with fully RRC-configured UL grant (Type1). If this field is absent the UE uses UL grant configured by DCI addressed to CS-RNTI (Type2).

sequenceOffsetForRV Configures the RV offset for the starting RV for the first repetition (first actual repetition in PUSCH repetition Type B) towards the second 'SRS resource set' for PUSCH configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

srs-ResourceSetId Indicates the associated SRS resource set for PUSCH+PUSCH simultaneous uplink transmsision for CG-type 1 PUSCH.

srs-ResourceIndicator Indicates the SRS resource to be used. The network does not configure this for CG-SDT.

srs-ResourceIndicator2 Indicates the SRS resource to be used for the second SRS resource set. When this field is present, the srs-ResourceIndicator is used for the first SRS resource set.

startingFromRV0 This field is used to determine the initial transmission occasion of a transport block for a given RV sequence, see TS 38.214 [19], clause 6.1.2.3.1. The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation.

timeDomainAllocation, timeDomainAllocation-v1710 Indicates a combination of start symbol and length and PUSCH mapping type, see TS 38.214 [19], clause 6.1.2 and TS 38.212 [17], clause 7.3.1. If the field timeDomainAllocation-v1710 is present, the UE shall ignore timeDomainAllocation field (without suffix).

timeDomainOffset Offset related to the reference SFN indicated by timeReferenceSFN, see TS 38.321 [3], clause 5.8.2. timeDomainOffset-r17 is only applicable to 480 kHz and 960 kHz. If timeDomainOffset-r17 is present, the UE shall ignore timeDomainOffset (without suffix).

timeReferenceHyperSFN Indicates H-SFN used for determination of the offset of a resource in time domain. The UE uses the closest H-SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2.

timeReferenceSFN Indicates SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2. If the field timeReferenceSFN is not present, the reference SFN is 0.

transformPrecoder Enables or disables transform precoding for type1 and type2. If the field is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList), see TS 38.214 [19], clause 6.1.3.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic. The network does not configure this for CG-SDT.

CG-COT-Sharing field descriptions

channelAccessPriority Indicates the Channel Access Priority Class that the gNB can assume when sharing the UE initiated COT (see 37.213 [48], clause 4.1.3).

duration Indicates the number of DL transmission slots within UE initiated COT (see 37.213 [48], clause 4.1.3).

offset Indicates the number of DL transmission slots from the end of the slot where CG-UCI is detected after which COT sharing can be used (see 37.213 [48], clause 4.1.3).

CG-StartingOffsets field descriptions

cg-StartingFullBW-InsideCOT A set of configured grant PUSCH transmission starting offsets (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingFullBW-OutsideCOT A set of configured grant PUSCH transmission starting offset indices (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-InsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-OutsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

CG-SDT-Configuration and CG-RRC-Configuration field descriptions

cg-RRC-RSRP-ThresholdSSB An RSRP threshold configured for SSB selection for the CG as specified in TS 38.321 [3]. This field is absent in case CG-RRC-Configuration IE is received as part of an LTM-Candidate IE.

cg-SDT-RetransmissionTimer, cg-RRC-RetransmissionTimer Indicates the initial value of the configured grant retransmission timer used for the initial transmission of CG with CCCH (for CG-SDT) or DCCH message (see TS 38.321 [3]) in multiples of periodicity.

sdt-DMRS-Ports, rrc-DMRS-Ports Indicates the set of DMRS ports for SSB to PUSCH mapping (see TS 38.213 [13]). The first (left-most / most significant) bit corresponds to DMRS port 0, the second most significant bit corresponds to DMRS port 1, and so on. A bit set to 1 indicates that this DMRS port is used for mapping. In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-NrofDMRS-Sequences, rrc-NrofDMRS-Sequences Indicates the number of DMRS sequences for SSB to PUSCH mapping (see TS 38.213 [13]). In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-SSB-Subset, rrc-SSB-Subset Indicates SSB subset for SSB to CG PUSCH mapping within one CG configuration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not included in the SSB subset for SSB to CG PUSCH mapping while value 1 indicates that the corresponding SS/PBCH block is included in SSB subset for SSB to CG PUSCH mapping. If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs. In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-SSB-PerCG-PUSCH, rrc-SSB-PerCG-PUSCH The number of SSBs per CG PUSCH (see TS 38.213 [13]). Value one corresponds to 1 SSBs per CG PUSCH, value two corresponds to 2 SSBs per CG PUSCH and so on. In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-P0-PUSCH, rrc-P0-PUSCH Indicates P0 value for PUSCH in steps of 1dB (see TS 38.213 [13]). When this field is configured, the UE ignores the p0-PUSCH-Alpha.

sdt-Alpha, rrc-Alpha Indicates alpha value for PUSCH. alpha0 indicates value 0 is used, alpha04 indicates value 4 is used and so on (see TS 38.213 [13]). When this field is configured, the UE ignores the p0-PUSCH-Alpha.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-BasedPrioritization | This field is optionally present, Need R, if lch-BasedPrioritization is configured in the MAC entity. It is absent otherwise. |
| RACH-LessHO | The field is optionally present, Need N, if rach-LessHO is present in reconfigurationWithSync. It is absent otherwise. |
| RepTypeB | The field is optionally present if pusch-RepTypeIndicator is set to pusch-RepTypeB, Need S, and absent otherwise. |
| CG-List | The field is mandatory present when included in configuredGrantConfigToAddModList-r16, otherwise the field is absent. |
| CG-IndexMAC | The field is mandatory present if at least one configured grant is configured by configuredGrantConfigToAddModList-r16 in any BWP of this MAC entity, otherwise it is optionally present, need R. |
| LTM | The field is optionally present, Need R, if the UE is configured with at least an LTM candidate configuration. Otherwise, the field is absent. |
| SRSsets | This field is mandatory present when UE is configured with two SRS sets configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage codebook or non-codebook. Otherwise it is absent, Need R |

#### –	ConfiguredGrantConfigIndex

The IE ConfiguredGrantConfigIndex is used to indicate the index of one of multiple UL Configured Grant configurations in one BWP.

ConfiguredGrantConfigIndex information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIGINDEX-START

ConfiguredGrantConfigIndex-r16 ::= INTEGER (0.. maxNrofConfiguredGrantConfig-1-r16)

-- TAG-CONFIGUREDGRANTCONFIGINDEX-STOP

-- ASN1STOP

#### –	ConfiguredGrantConfigIndexMAC

The IE ConfiguredGrantConfigIndexMAC is used to indicate the unique Configured Grant configurations index per MAC entity.

ConfiguredGrantConfigIndexMAC information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIGINDEXMAC-START

ConfiguredGrantConfigIndexMAC-r16 ::= INTEGER (0.. maxNrofConfiguredGrantConfigMAC-1-r16)

-- TAG-CONFIGUREDGRANTCONFIGINDEXMAC-STOP

-- ASN1STOP

#### –	ConnEstFailureControl

The IE ConnEstFailureControl is used to configure parameters for connection establishment failure control.

ConnEstFailureControl information element

-- ASN1START

-- TAG-CONNESTFAILURECONTROL-START

ConnEstFailureControl ::=   SEQUENCE {

connEstFailCount                    ENUMERATED {n1, n2, n3, n4},

connEstFailOffsetValidity           ENUMERATED {s30, s60, s120, s240, s300, s420, s600, s900},

connEstFailOffset                   INTEGER (0..15)                                                         OPTIONAL    -- Need S

}

-- TAG-CONNESTFAILURECONTROL-STOP

-- ASN1STOP

ConnEstFailureControl field descriptions

connEstFailCount Number of times that the UE detects T300 expiry on the same cell before applying connEstFailOffset.

connEstFailOffset Parameter "Qoffsettemp" in TS 38.304 [20]. If the field is absent, the value of infinity shall be used for "Qoffsettemp".

connEstFailOffsetValidity Amount of time that the UE applies connEstFailOffset before removing the offset from evaluation of the cell. Value s30 corresponds to 30 seconds, value s60 corresponds to 60 seconds, and so on.

#### –	ControlResourceSet

The IE ControlResourceSet is used to configure a time/frequency control resource set (CORESET) in which to search for downlink control information (see TS 38.213 [13], clause 10.1). For the UE not supporting multipleCORESET in FR1, in order to receive MBS multicast in CFR within the UE's active BWP, if a CORESET is not configured within the PDCCH-ConfigMulticast, the CORESET other than CORESET#0 configured within the UE's active BWP for scheduling unicast can be used for scheduling MBS multicast, and the CORESET is expected to be included completely within the CFR and the parameters configured in the CORESET are expected to be supported by the UE for MBS multicast.

ControlResourceSet information element

-- ASN1START

-- TAG-CONTROLRESOURCESET-START

ControlResourceSet ::=              SEQUENCE {

controlResourceSetId                ControlResourceSetId,

frequencyDomainResources            BIT STRING (SIZE (45)),

duration                            INTEGER (1..maxCoReSetDuration),

cce-REG-MappingType                 CHOICE {

interleaved                         SEQUENCE {

reg-BundleSize                      ENUMERATED {n2, n3, n6},

interleaverSize                     ENUMERATED {n2, n3, n6},

shiftIndex                          INTEGER(0..maxNrofPhysicalResourceBlocks-1)       OPTIONAL -- Need S

},

nonInterleaved                      NULL

},

precoderGranularity                 ENUMERATED {sameAsREG-bundle, allContiguousRBs},

tci-StatesPDCCH-ToAddList           SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP

tci-StatesPDCCH-ToReleaseList       SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP

tci-PresentInDCI                        ENUMERATED {enabled}                                  OPTIONAL, -- Need S

pdcch-DMRS-ScramblingID                 INTEGER (0..65535)                                    OPTIONAL, -- Need S

...,

[[

rb-Offset-r16                       INTEGER (0..5)                                            OPTIONAL, -- Need S

tci-PresentDCI-1-2-r16              INTEGER (1..3)                                            OPTIONAL, -- Need S

coresetPoolIndex-r16                INTEGER (0..1)                                            OPTIONAL, -- Need S

controlResourceSetId-v1610          ControlResourceSetId-v1610                                OPTIONAL  -- Need S

]],

[[

followUnifiedTCI-State-r17          ENUMERATED {enabled}                                      OPTIONAL  -- Need R

]],

[[

applyIndicatedTCI-State-r18         ENUMERATED {first, second, both,  none}                   OPTIONAL  -- Cond FollowUTCI

]]

}

-- TAG-CONTROLRESOURCESET-STOP

-- ASN1STOP

ControlResourceSet field descriptions

applyIndicatedTCI-State This field indicates, for PDCCH reception on this CORESET, if UE applies the first, the second, both or none "indicated" DL only TCI or joint TCI as specified in TS 38.213 [13], clause 10.1.

cce-REG-MappingType Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG) (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

controlResourceSetId Identifies the instance of the ControlResourceSet IE. Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon (controlResourceSetZero) and is hence not used here in the ControlResourceSet IE. Other values identify CORESETs configured by dedicated signalling or in SIB1 or SIB20. The controlResourceSetId is unique among the BWPs of a serving cell. If the field controlResourceSetId-v1610 is present, the UE shall ignore the controlResourceSetId field (without suffix).

coresetPoolIndex The index of the CORESET pool for this CORESET as specified in TS 38.213 [13] (clauses 9 and 10) and TS 38.214 [19] (clauses 5.1 and 6.1). If the field is absent, the UE applies the value 0.

duration Contiguous time duration of the CORESET in number of symbols (see TS 38.211 [16], clause 7.3.2.2).

followUnifiedTCI-State When set to enabled, for PDCCH reception on this CORESET, the UE applies the "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5.

frequencyDomainResources Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from the first RB group in the BWP or MBS CFR where the CORESET is configured. When at least one search space is configured with freqMonitorLocation-r16, only the first NRBG,set0size bits are valid (see TS 38.213 [13], clause 10.1). The first (left-most / most significant) bit corresponds to the first RB group in the BWP or MBS CFR where the CORESET is configured, and so on. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the bandwidth part within which the CORESET is configured are set to zero (see TS 38.211 [16], clause 7.3.2.2).

interleaverSize Interleaver-size (see TS 38.211 [16], clause 7.3.2.2).

pdcch-DMRS-ScramblingID PDCCH DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.3.1). When the field is absent the UE applies the value of the physCellId configured for this serving cell.

precoderGranularity Precoder granularity in frequency domain (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1).

reg-BundleSize Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles (see TS 38.211 [16], clause 7.3.2.2).

shiftIndex When the field is absent the UE applies the value of the physCellIdconfigured for this serving cell (see TS 38.211 [16], clause 7.3.2.2).

tci-PresentInDCI This field indicates if TCI field is present or absent in DCI format 1_1 and DCI format 4_2. When the field is absent the UE considers the TCI to be absent/disabled. In case of cross carrier scheduling, the network sets this field to enabled for the ControlResourceSet used for cross carrier scheduling in DCI format 1_1 in the scheduling cell if enableDefaultBeamForCCS is not configured (see TS 38.214 [19], clause 5.1.5).

tci-PresentDCI-1-2 Configures the number of bits for "Transmission configuration indicator" in DCI format 1_2. When the field is absent the UE applies the value of 0 bit for the "Transmission configuration indicator" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.5). In case of cross carrier scheduling, the network configures this field for the ControlResourceSet used for cross carrier scheduling in DCI format 1_2 in the scheduling cell if enableDefaultBeamForCCS is not configured (see TS 38.214 [19], clause 5.1.5).

tci-StatesPDCCH-ToAddList A subset of the TCI states defined in pdsch-Config, either with tci-StatesToAddModList or dl-OrJointTCI-StateList, included in the BWP-DownlinkDedicated corresponding to the serving cell and to the DL BWP to which the ControlResourceSet belong to. They are used for providing QCL relationships between the DL RS(s) in one RS Set (TCI-State) and the PDCCH DMRS ports (see TS 38.213 [13], clause 6.). The network configures at most maxNrofTCI-StatesPDCCH entries. The QCL relationships defined herein do not apply to MBS broadcast.

| Conditional Presence | Explanation |
| --- | --- |
| FollowUTCI | The field is absent if the field followUnifiedTCI-State is present or if more than one value for the field coresetPoolIndex is configured in controlResourceSet for the same bandwidthpart. Otherwise, it is optionally present, Need R. |
| NotSIB-initialBWP | The field is absent in SIB1/SIB20 and in the PDCCH-ConfigCommon of the initial BWP in ServingCellConfigCommon, if SIB1/SIB20 is broadcasted. Otherwise, it is optionally present, Need N. |

#### –	ControlResourceSetId

The ControlResourceSetId IE concerns a short identity, used to identify a control resource set within a serving cell. The ControlResourceSetId = 0 identifies the ControlResourceSet#0 configured via PBCH (MIB) and in controlResourceSetZero (ServingCellConfigCommon). The ID space is used across the BWPs and MBS CFRs of a Serving Cell.

ControlResourceSetId information element

-- ASN1START

-- TAG-CONTROLRESOURCESETID-START

ControlResourceSetId ::=                INTEGER (0..maxNrofControlResourceSets-1)

ControlResourceSetId-r16 ::=            INTEGER (0..maxNrofControlResourceSets-1-r16)

ControlResourceSetId-v1610 ::=          INTEGER (maxNrofControlResourceSets..maxNrofControlResourceSets-1-r16)

-- TAG-CONTROLRESOURCESETID-STOP

-- ASN1STOP

#### –	ControlResourceSetZero

The IE ControlResourceSetZero is used to configure CORESET#0 of the initial BWP (see TS 38.213 [13], clause 13).

ControlResourceSetZero information element

-- ASN1START

-- TAG-CONTROLRESOURCESETZERO-START

ControlResourceSetZero ::=                  INTEGER (0..15)

-- TAG-CONTROLRESOURCESETZERO-STOP

-- ASN1STOP

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CROSSCARRIERSCHEDULINGCONFIG-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...,

[[

carrierIndicatorSize-r16            SEQUENCE {

carrierIndicatorSizeDCI-1-2-r16        INTEGER (0..3),

carrierIndicatorSizeDCI-0-2-r16        INTEGER (0..3)

}                                                                                       OPTIONAL,  -- Cond CIF-PRESENCE

enableDefaultBeamForCCS-r16         ENUMERATED {enabled}                                OPTIONAL  -- Need S

]],

[[

ccs-BlindDetectionSplit-r17         ENUMERATED {oneSeventh, threeFourteenth, twoSeventh, threeSeventh,

oneHalf, fourSeventh, fiveSeventh, spare1}      OPTIONAL  -- Need R

]]

}

-- TAG-CROSSCARRIERSCHEDULINGCONFIG-STOP

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

carrierIndicatorSizeDCI-0-2, carrierIndicatorSizeDCI-1-2 Configures the number of bits for the field of carrier indicator in PDCCH DCI format 0_2/1_2. The field carrierIndicatorSizeDCI-0-2 refers to DCI format 0_2 and the field carrierIndicatorSizeDCI-1-2 refers to DCI format 1_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1).

ccs-BlindDetectionSplit Indicates the share of blind detection candidates and non-overlapping CCEs for PDCCH monitoring on an SpCell and an SCell when cross-carrier scheduling is configured from the SCell for the SpCell (see TS 38.213 [13], clause 10.1.1). The network only configures this field when it sets the field other for an SpCell, i.e., when it configures cross-carrier scheduling of the SpCell by a PDCCH on an Scell.

cif-Presence The field is used to indicate whether carrier indicator field is present (value true) or not (value false) in PDCCH DCI formats, see TS 38.213 [13]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [13]. If configured for an SpCell, the non-fallback DCI formats on the SpCell include same number of CIF bits as the corresponding non-fallback DCI formats on the scheduling cell, and the CIF bits are considered reserved.

enableDefaultBeamForCCS This field indicates whether default beam selection for cross-carrier scheduled PDSCH or aperiodic CSI-RS is enabled, see TS 38.214 [19]. If not present, the default beam selection behaviour is not applied, i.e. Rel-15 behaviour is applied. This field can only be configured in the cross-scheduled SCell or SpCell.

other Parameters for cross-carrier scheduling. If configured for an SpCell, the SpCell can be scheduled by the PDCCH on another SCell as well as by the PDCCH on the SpCell. If configured for an SCell, the SCell is scheduled by a PDDCH on another cell.

own Parameters for self-scheduling, i.e., a serving cell is scheduled by its own PDCCH.

schedulingCellId If configured for an SpCell, this field indicates which SCell, in addition to the SpCell, signals the downlink allocations and uplink grants, if applicable, for the concerned SpCell. If configured for an Scell, this field indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell. In case the UE is configured with two PUCCH groups, the scheduling cell and the scheduled cell are within the same PUCCH group. If drx-ConfigSecondaryGroup is configured in the MAC-CellGroupConfig associated with this serving cell, the scheduling cell and the scheduled cell belong to the same Frequency Range. In addition, the serving cell with an aperiodic CSI trigger and the PUSCH resource scheduled for the report are on the same carrier and serving cell, but the cell for which CSI is reported may belong to the same or a different Frequency Range. The network should not trigger a CSI request for a serving cell in the other Frequency Range when that serving cell is outside Active Time.

| Conditional Presence | Explanation |
| --- | --- |
| CIF-PRESENCE | The field is mandatory present if the cif-Presence is set to true. The field is absent otherwise. |

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of aperiodic trigger states. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). Upon reception of the value associated with a trigger state, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state.

CSI-AperiodicTriggerStateList information element

-- ASN1START

-- TAG-CSI-APERIODICTRIGGERSTATELIST-START

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...,

[[

ap-CSI-MultiplexingMode-r17         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]],

[[

ltm-AssociatedReportConfigInfo-r18  LTM-CSI-ReportConfigId-r18                                    OPTIONAL  -- Need R

]]

}

CSI-AssociatedReportConfigInfo ::=  SEQUENCE {

reportConfigId                      CSI-ReportConfigId,

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference     INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)               OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesForInterference INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)          OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

...,

[[

resourcesForChannel2-r17        CHOICE {

nzp-CSI-RS2-r17                 SEQUENCE {

resourceSet2-r17                INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info2-r17                   SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL   -- Cond Aperiodic

},

csi-SSB-ResourceSet2-r17        INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)

}                                                                                             OPTIONAL,  -- Need R

csi-SSB-ResourceSetExt          INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)          OPTIONAL   -- Need R

]],

[[

resourcesForChannelTDCP-r18     SEQUENCE {

resourceSet2TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

resourceSet3TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)              OPTIONAL  -- Need R

}                                                                                                     OPTIONAL,  -- Cond TDCP

applyIndicatedTCI-State-r18     CHOICE {

perSet-r18                      ENUMERATED {first, second},

perResource-r18                 SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF ENUMERATED {first, second}

}                                                                                                     OPTIONAL,  -- Need R

applyIndicatedTCI-State2-r18    CHOICE {

perSet-r18                     ENUMERATED {first, second},

perResource-r18                SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF  ENUMERATED {first, second}

}                                                                                                     OPTIONAL, -- Cond SecondCSICMR

csi-ReportSubConfigTriggerList-r18  CSI-ReportSubConfigTriggerList-r18                                OPTIONAL   -- Need R

]]

}

-- TAG-CSI-APERIODICTRIGGERSTATELIST-STOP

-- ASN1STOP

CSI-AperiodicTriggerState field descriptions

ltm-AssociatedReportConfigInfo This field configures the aperiodic CSI reports of LTM candidate cells. If ltm-associatedReportConfigInfo is configured the UE shall ignore the field associatedReportConfigInfoList.

CSI-AssociatedReportConfigInfo field descriptions

ap-CSI-MultiplexingMode Indicates if the behavior of transmitting aperiodic CSI on the first PUSCH repetitions corresponding to two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' is enabled or not.

applyIndicatedTCI-State, applyIndicatedTCI-State2 This field indicates, for an aperiodic CSI-RS resource set (perSet) or for CSI-RS resource (perResource), if UE applies the first or the second "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5. If more than one value for the field coresetPoolIndex is configured in the DL BWP used to trigger the CSI report, the value 'first' corresponds to the "indicated" joint/DL TCI states specific to coresetPoolIndex value 0 and the value 'second' correspond to the value 1, respectively. The applyIndicatedTCI-State is for ResourcesForChannel, and applyIndicatedTCI-State2 is for ResourcesForChannels2.

csi-IM-ResourcesForInterference CSI-IM-ResourceSet for interference measurement. Entry number in csi-IM-ResourceSetList in the CSI-ResourceConfig indicated by csi-IM-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on). The indicated CSI-IM-ResourceSet should have exactly the same number of resources like the NZP-CSI-RS-ResourceSet indicated in resourceSet within nzp-CSI-RS.

csi-ReportSubConfigTriggerList A list of sub-configuration ID(s) of N sub-configurations out of L configured sub-configurations within a CSI-ReportConfig associated with a triggering state for aperiodic CSI reporting on PUSCH.

csi-SSB-ResourceSet, csi-SSB-ResourceSet2 CSI-SSB-ResourceSet for channel measurements. Entry number in csi-SSB-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

nzp-CSI-RS-ResourcesForInterference NZP-CSI-RS-ResourceSet for interference measurement. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by nzp-CSI-RS-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

qcl-info, qcl-info2 List of references to TCI-States for providing the QCL source and QCL type for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources of the NZP-CSI-RS-ResourceSet indicated by resourceSet within nzp-CSI-RS. Each TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resourcesForChannelMeasurement (in the CSI-ReportConfig indicated by reportConfigId above) belong to. First entry in qcl-info corresponds to first entry in nzp-CSI-RS-Resources of that NZP-CSI-RS-ResourceSet, second entry in qcl-info corresponds to second entry in nzp-CSI-RS-Resources, and so on (see TS 38.214 [19], clause 5.2.1.5.1). When this field is absent for aperiodic CSI RS, and applyIndicatedTCI-State or applyIndicatedTCI-State2 is not configured, the UE shall use QCL information included in the "indicated" DL only/Joint TCI state as specified in TS 38.214 [19].

reportConfigId The reportConfigId of one of the CSI-ReportConfigToAddMod configured in CSI-MeasConfig

resourcesForChannel2 Configures reference signals for channel measurement corresponding to the second resource set for L1-RSRP measurement as configured in IE CSI-ResourceConfig when nrofReportedGroups-r17 is configured in IE CSI-ReportConfig. If this is present, network configures csi-SSB-ResourceSetExt instead of csi-SSB-ResourceSet and the UE ignores csi-SSB-ResourceSet in resourcesForChannel, and the resourcesForChannel configures the reference signals for channel measurement corresponding to the first resource set for L1-RSRP measurement (see TS 38.214 [19], clause 5.2.1.4).

resourcesForChannelTDCP Configures reference signals for channel measurement corresponding to the second resource set and third resource set for TDCP reporting. All CSI resources of these two resource sets always share the same QCL-info with the resource sets indicated by resourcesForChannel as spcified in TS 38.214 [19].

resourceSet NZP-CSI-RS-ResourceSet for channel measurements. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

| Conditional Presence | Explanation |
| --- | --- |
| Aperiodic | The field is mandatory present if the NZP-CSI-RS-Resources in the associated resourceSet have the resourceType aperiodic and unifiedTCI-StateType is not configured. The field is optionally present, Need R, if the NZP-CSI-RS-Resources in the associated resourceSet have the resourceType aperiodic and unifiedTCI-StateType is configured. The field is absent otherwise. |
| CSI-IM-ForInterference | This field is mandatory present if the CSI-ReportConfig identified by reportConfigId is configured with csi-IM-ResourcesForInterference; otherwise it is absent. |
| NZP-CSI-RS-ForInterference | This field is mandatory present if the CSI-ReportConfig identified by reportConfigId is configured with nzp-CSI-RS-ResourcesForInterference; otherwise it is absent. |
| SecondCSICMR | This field is optionally present, Need R, if resourcesForChannel2 is configured with aperiodic CSI-RS. It is absent otherwise. |
| TDCP | This field is absent if resourcesForChannel2 is configured. It is optionally present, Need R, otherwise. |

#### –	CSI-FrequencyOccupation

The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource).

CSI-FrequencyOccupation information element

-- ASN1START

-- TAG-CSI-FREQUENCYOCCUPATION-START

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

-- TAG-CSI-FREQUENCYOCCUPATION-STOP

-- ASN1STOP

CSI-FrequencyOccupation field descriptions

nrofRBs Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP.

startingRB PRB where this CSI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. Only multiples of 4 are allowed (0, 4, ...)

#### –	CSI-IM-Resource

The IE CSI-IM-Resource is used to configure one CSI Interference Management (IM) resource.

CSI-IM-Resource information element

-- ASN1START

-- TAG-CSI-IM-RESOURCE-START

CSI-IM-Resource ::=                 SEQUENCE {

csi-IM-ResourceId                   CSI-IM-ResourceId,

csi-IM-ResourceElementPattern           CHOICE {

pattern0                                SEQUENCE {

subcarrierLocation-p0                   ENUMERATED { s0, s2, s4, s6, s8, s10 },

symbolLocation-p0                       INTEGER (0..12)

},

pattern1                                SEQUENCE {

subcarrierLocation-p1                   ENUMERATED { s0, s4, s8 },

symbolLocation-p1                       INTEGER (0..13)

}

}                                                                                   OPTIONAL,   -- Need M

freqBand                            CSI-FrequencyOccupation                         OPTIONAL,   -- Need M

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

...

}

-- TAG-CSI-IM-RESOURCE-STOP

-- ASN1STOP

CSI-IM-Resource field descriptions

csi-IM-ResourceElementPattern The resource element pattern (Pattern0 (2,2) or Pattern1 (4,1)) with corresponding parameters (see TS 38.214 [19], clause 5.2.2.4)

freqBand Frequency-occupancy of CSI-IM (see TS 38.214 [19], clause 5.2.2.4)

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent CSI-IM. Network always configures the UE with a value for this field for periodic and semi-persistent CSI-IM-Resources (as indicated in CSI-ResourceConfig). A change of configuration between periodic or semi-persistent and aperiodic for a CSI-IM-Resource is not supported without a release and add.

subcarrierLocation-p0 OFDM subcarrier occupancy of the CSI-IM resource for Pattern0 (see TS 38.214 [19], clause 5.2.2.4)

subcarrierLocation-p1 OFDM subcarrier occupancy of the CSI-IM resource for Pattern1 (see TS 38.214 [19], clause 5.2.2.4)

symbolLocation-p0 OFDM symbol location of the CSI-IM resource for Pattern0 (see TS 38.214 [19], clause 5.2.2.4)

symbolLocation-p1 OFDM symbol location of the CSI-IM resource for Pattern1 (see TS 38.214 [19], clause 5.2.2.4)

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent CSI-IM-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	CSI-IM-ResourceId

The IE CSI-IM-ResourceId is used to identify one CSI-IM-Resource.

CSI-IM-ResourceId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCEID-START

CSI-IM-ResourceId ::=               INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-IM-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSet

The IE CSI-IM-ResourceSet is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters.

CSI-IM-ResourceSet information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESET-START

CSI-IM-ResourceSet ::=              SEQUENCE {

csi-IM-ResourceSetId                CSI-IM-ResourceSetId,

csi-IM-Resources                    SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId,

...

}

-- TAG-CSI-IM-RESOURCESET-STOP

-- ASN1STOP

CSI-IM-ResourceSet field descriptions

csi-IM-Resources CSI-IM-Resources associated with this CSI-IM-ResourceSet (see TS 38.214 [19], clause 5.2).

#### –	CSI-IM-ResourceSetId

The IE CSI-IM-ResourceSetId is used to identify CSI-IM-ResourceSets.

CSI-IM-ResourceSetId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETID-START

CSI-IM-ResourceSetId ::=            INTEGER (0..maxNrofCSI-IM-ResourceSets-1)

-- TAG-CSI-IM-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]]

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

CSI-MeasConfig field descriptions

aperiodicTriggerStateList Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement (see TS 38.214 [19], clause 5.2.1).

csi-IM-ResourceSetToAddModList Pool of CSI-IM-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

csi-IM-ResourceToAddModList Pool of CSI-IM-Resource which can be referred to from CSI-IM-ResourceSet.

csi-ReportConfigToAddModList Configured CSI report settings as specified in TS 38.214 [19] clause 5.2.1.1.

csi-ResourceConfigToAddModList Configured CSI resource settings as specified in TS 38.214 [19] clause 5.2.1.2.

csi-SSB-ResourceSetToAddModList Pool of CSI-SSB-ResourceSet which can be referred to from CSI-ResourceConfig.

ltm-CSI-ReportConfigToAddModList Configured CSI report settings for LTM as specified in TS 38.214 [19].

nzp-CSI-RS-ResourceSetToAddModList Pool of NZP-CSI-RS-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

nzp-CSI-RS-ResourceToAddModList Pool of NZP-CSI-RS-Resource which can be referred to from NZP-CSI-RS-ResourceSet.

reportTriggerSize, reportTriggerSizeDCI-0-2 Size of CSI request field in DCI (bits) (see TS 38.214 [19], clause 5.2.1.5.1). The field reportTriggerSize applies to DCI format 0_1 and the field reportTriggerSizeDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 5.2.1.5.1).

scellActivationRS-ConfigToAddModList Configured RS for fast SCell activation as specified in TS 38.214 [19] clause 5.2.1.5.3.

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

cqi-FormatIndicator                     ENUMERATED { widebandCQI, subbandCQI }                          OPTIONAL,   -- Need R

pmi-FormatIndicator                     ENUMERATED { widebandPMI, subbandPMI }                          OPTIONAL,   -- Need R

csi-ReportingBand                       CHOICE {

subbands3                               BIT STRING(SIZE(3)),

subbands4                               BIT STRING(SIZE(4)),

subbands5                               BIT STRING(SIZE(5)),

subbands6                               BIT STRING(SIZE(6)),

subbands7                               BIT STRING(SIZE(7)),

subbands8                               BIT STRING(SIZE(8)),

subbands9                               BIT STRING(SIZE(9)),

subbands10                              BIT STRING(SIZE(10)),

subbands11                              BIT STRING(SIZE(11)),

subbands12                              BIT STRING(SIZE(12)),

subbands13                              BIT STRING(SIZE(13)),

subbands14                              BIT STRING(SIZE(14)),

subbands15                              BIT STRING(SIZE(15)),

subbands16                              BIT STRING(SIZE(16)),

subbands17                              BIT STRING(SIZE(17)),

subbands18                              BIT STRING(SIZE(18)),

...,

subbands19-v1530                        BIT STRING(SIZE(19))

}   OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

timeRestrictionForChannelMeasurements           ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements      ENUMERATED {configured, notConfigured},

codebookConfig                                  CodebookConfig                                              OPTIONAL,   -- Need R

dummy                                           ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

groupBasedBeamReporting                     CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4}                                 OPTIONAL    -- Need S

}

},

cqi-Table                   ENUMERATED {table1, table2, table3, table4-r17}                                     OPTIONAL,   -- Need R

subbandSize                 ENUMERATED {value1, value2},

non-PMI-PortIndication      SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks OPTIONAL,   -- Need R

...,

[[

semiPersistentOnPUSCH-v1530         SEQUENCE {

reportSlotConfig-v1530              ENUMERATED {sl4, sl8, sl16}

}                                                                                                           OPTIONAL    -- Need R

]],

[[

semiPersistentOnPUSCH-v1610         SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

aperiodic-v1610                     SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

reportQuantity-r16                  CHOICE {

cri-SINR-r16                         NULL,

ssb-Index-SINR-r16                   NULL

}                                                                                                           OPTIONAL,   -- Need R

codebookConfig-r16                          CodebookConfig-r16                                              OPTIONAL    -- Need R

]],

[[

cqi-BitsPerSubband-r17              ENUMERATED {bits4}                                                      OPTIONAL,   -- Need R

groupBasedBeamReporting-v1710       SEQUENCE {

nrofReportedGroups-r17              ENUMERATED {n1, n2, n3, n4}

}                                                                                                           OPTIONAL,   -- Need R

codebookConfig-r17                  CodebookConfig-r17                                                      OPTIONAL,   -- Need R

sharedCMR-r17                       ENUMERATED {enable}                                                     OPTIONAL,   -- Need R

csi-ReportMode-r17                  ENUMERATED {mode1, mode2}                                               OPTIONAL,   -- Need R

numberOfSingleTRP-CSI-Mode1-r17     ENUMERATED {n0, n1, n2}                                                 OPTIONAL,   -- Need R

reportQuantity-r17                  CHOICE {

cri-RSRP-Index-r17                  NULL,

ssb-Index-RSRP-Index-r17            NULL,

cri-SINR-Index-r17                  NULL,

ssb-Index-SINR-Index-r17            NULL

}                                                                                                           OPTIONAL    -- Need R

]],

[[

semiPersistentOnPUSCH-v1720         SEQUENCE {

reportSlotOffsetList-r17            SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-2-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-1-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL    -- Need R

}                                                                                                           OPTIONAL,   -- Need R

aperiodic-v1720                     SEQUENCE {

reportSlotOffsetList-r17            SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-2-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-1-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL    -- Need R

}                                                                                                           OPTIONAL    -- Need R

]],

[[

codebookConfig-v1730                CodebookConfig-v1730                                                    OPTIONAL    -- Need R

]],

[[

groupBasedBeamReporting-v1800       SEQUENCE {

reportingMode-r18                   ENUMERATED {jointULDL, onlyUL}

}                                                                                                           OPTIONAL,   -- Need R

reportQuantity-r18                  TDCP-r18                                                                OPTIONAL,   -- Need R

codebookConfig-r18                  CodebookConfig-r18                                                      OPTIONAL,   -- Need R

csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18

OPTIONAL,   -- Need N

csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18

OPTIONAL    -- Need N

]]

}

CSI-ReportPeriodicityAndOffset ::=  CHOICE {

slots4                              INTEGER(0..3),

slots5                              INTEGER(0..4),

slots8                              INTEGER(0..7),

slots10                             INTEGER(0..9),

slots16                             INTEGER(0..15),

slots20                             INTEGER(0..19),

slots40                             INTEGER(0..39),

slots80                             INTEGER(0..79),

slots160                            INTEGER(0..159),

slots320                            INTEGER(0..319)

}

PortIndexFor8Ranks ::=              CHOICE {

portIndex8                          SEQUENCE{

rank1-8                             PortIndex8                                                      OPTIONAL,   -- Need R

rank2-8                             SEQUENCE(SIZE(2)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank3-8                             SEQUENCE(SIZE(3)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank4-8                             SEQUENCE(SIZE(4)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank5-8                             SEQUENCE(SIZE(5)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank6-8                             SEQUENCE(SIZE(6)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank7-8                             SEQUENCE(SIZE(7)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank8-8                             SEQUENCE(SIZE(8)) OF PortIndex8                                 OPTIONAL    -- Need R

},

portIndex4                          SEQUENCE{

rank1-4                             PortIndex4                                                      OPTIONAL,   -- Need R

rank2-4                             SEQUENCE(SIZE(2)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank3-4                             SEQUENCE(SIZE(3)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank4-4                             SEQUENCE(SIZE(4)) OF PortIndex4                                 OPTIONAL    -- Need R

},

portIndex2                          SEQUENCE{

rank1-2                             PortIndex2                                                      OPTIONAL,   -- Need R

rank2-2                             SEQUENCE(SIZE(2)) OF PortIndex2                                 OPTIONAL    -- Need R

},

portIndex1                          NULL

}

PortIndex8::=                       INTEGER (0..7)

PortIndex4::=                       INTEGER (0..3)

PortIndex2::=                       INTEGER (0..1)

TDCP-r18 ::=                        SEQUENCE {

delayDSetofLengthY-r18              SEQUENCE (SIZE (1.. maxNrofdelayD-r18)) OF DelayD,

phaseReporting-r18                  ENUMERATED {enable}                                                 OPTIONAL    -- Need R

}

DelayD ::=                          ENUMERATED { symb4, slot1, slot2, slot3, slot4, slot5, slot6, slot10 }

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

CSI-ReportConfig field descriptions

carrier Indicates in which serving cell the CSI-ResourceConfig indicated below are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

codebookConfig Codebook configuration for Type-1 or Type-2 including codebook subset restriction. Network can only configure one of codebookConfig, codebookConfig-r16 or codebookConfig-r17 or codebookConfig-r18 to a UE. The network includes codebookConfig-v1730 only if codebookConfig-r17 is configured.

cqi-BitsPerSubband This field can only be present if cqi-FormatIndicator is set to subbandCQI. If the field is configured with bits4, the UE uses 4-bit sub-band CQI. If the field is not present and cqi-FormatIndicator is set to subbandCQI, the UE uses 2-bit sub-band differential CQI.

cqi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI (see TS 38.214 [19], clause 5.2.1.4).

cqi-Table Which CQI table to use for CQI calculation (see TS 38.214 [19], clause 5.2.2.1). For an (e)RedCap UE, CQI table 2 is only supported if the UE indicates support of 256QAM for PDSCH.

csi-IM-ResourcesForInterference CSI IM resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only CSI-IM resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband in order of frequency position in the BWP. The right-most bit in the bit string represents the lowest subband with the lowest frequency position in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise (see TS 38.214 [19], clause 5.2.1.4). NOTE: In TS 38.212 [17] clause 6.3.1.1.2 and TS 38.214 [19] clause 5.2.1.4, only subbands to be reported are numbered, e.g. subband #0 is the subband corresponding to the right-most bit set to 1.

csi-ReportMode Configures the CSI report modes Mode1 or Mode 2 (see TS 38.214 [19], clause 5.2.1.4.2)

csi-ReportSubConfigToAddModList List of CSI-ReportSubConfiguration(s) in a CSI report configuration to add or modify. No simultaneous configuration of portSubsetIndicator and a list of nzp-CSI-RS-resources in a same CSI report sub-configuration. The number of elements in a list is at least 2.

csi-ReportSubConfigToReleaseList List of CSI-ReportSubConfiguration(s) in a CSI report configuration to release.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

groupBasedBeamReporting Turning on/off group beam based reporting (see TS 38.214 [19], clause 5.2.1.4). If groupBasedBeamReporting (without suffix) is set to disabled, groupBasedBeamReporting-v1710 and groupBasedBeamReporting-v1800 is absent.

non-PMI-PortIndication Port indication for RI/CQI calculation. For each CSI-RS resource in the linked ResourceConfig for channel measurement, a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback (see TS 38.214 [19], clause 5.2.1.4.2). The first entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the CSI-ResourceConfig whose CSI-ResourceConfigId is indicated in a CSI-MeasId together with the above CSI-ReportConfigId; the second entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the second entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig, and so on until the NZP-CSI-RS-Resource indicated by the last entry in nzp-CSI-RS-Resources in the in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig. Then the next entry corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the second entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig and so on.

nrofReportedGroups Number of reported resource groups per CSI-report. Value n1 means one resource group, n2 means 2 resource groups, and so on. If nrofReportedGroups is configured, the UE ignores groupBasedBeamReporting (without suffix).

nrofReportedRS The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is either 2 or 4 depending on UE capability. (see TS 38.214 [19], clause 5.2.1.4) When the field is absent the UE applies the value 1.

numberOfSingleTRP-CSI-Mode1 Configures the number of reported X CSIs when csi-ReportMode is set to 'Mode 1' as described in TS 38.214 [19], clause 5.2.1.4.2. The field is present only if csi-ReportMode configures Mode 1.

nzp-CSI-RS-ResourcesForInterference NZP CSI RS resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

p0alpha Index of the p0-alpha set determining the power control for this CSI report transmission (see TS 38.214 [19], clause 6.2.1.2).

pdsch-BundleSizeForCSI PRB bundling size to assume for CQI calculation when reportQuantity is CRI/RI/i1/CQI. If the field is absent, the UE assumes that no PRB bundling is applied (see TS 38.214 [19], clause 5.2.1.4.2).

pmi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see TS 38.214 [19], clause 5.2.1.4).

pucch-CSI-ResourceList Indicates which PUCCH resource to use for reporting on PUCCH.

reportConfigType Time domain behavior of reporting configuration.

reportFreqConfiguration Reporting configuration in the frequency domain. (see TS 38.214 [19], clause 5.2.1.4).

reportQuantity The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1. If the field reportQuantity-r16, reportQuantity-r17 or reportQuantity-r18 is present, UE shall ignore reportQuantity (without suffix). Network does not configure reportQuantity-r17 or reportQuantity-r18 together with reportQuantity-r16.

reportingMode Configures the UE with reporting mode for group based reporting.(see TS 38.214 [19] clause 5.2.1.4).

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4). If the field reportSlotConfig-v1530 is present, the UE shall ignore the value provided in reportSlotConfig (without suffix).

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on. The first report is transmitted in slot n+Y, second report in n+Y+P, where P is the configured periodicity. Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on (see TS 38.214 [19], clause 6.1.2.1). The field reportSlotOffsetListDCI-0-1 applies to DCI format 0_1 and the field reportSlotOffsetListDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2.1). The fields reportSlotOffsetList-r17, reportSlotOffsetListDCI-0-1-r17 and reportSlotOffsetListDCI-0-2-r17 are only applicable for SCS 480 kHz and 960 kHz and if they are configured, the UE shall ignore the fields reportSlotOffsetList (without suffix), reportSlotOffsetListDCI-0-1 (without suffix) and reportSlotOffsetListDCI-0-2 (without suffix) for SCS 480 kHz and 960 kHz.

resourcesForChannelMeasurement Resources for channel measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources. This CSI-ReportConfig is associated with the DL BWP indicated by bwp-Id in that CSI-ResourceConfig.

sharedCMR Enables sharing of channel measurement resources between different CSI measurement hypotheses when (1) csi-ReportMode is set to 'Mode1' and numberOfSingleTRP-CSI-Mode1 is set to 1 or 2; or (2) csi-ReportMode is set to 'Mode2' (see TS 38.214 [19], clause 5.2.1.4.2).

subbandSize Indicates one out of two possible BWP-dependent values for the subband size as indicated in TS 38.214 [19], table 5.2.1.4-2 . If csi-ReportingBand is absent, the UE shall ignore this field.

timeRestrictionForChannelMeasurements Time domain measurement restriction for the channel (signal) measurements (see TS 38.214 [19], clause 5.2.1.1).

timeRestrictionForInterferenceMeasurements Time domain measurement restriction for interference measurements (see TS 38.214 [19], clause 5.2.1.1).

PortIndexFor8Ranks field descriptions

portIndex8 Port-Index configuration for up to rank 8. If present, the network configures port indexes for at least one of the ranks.

portIndex4 Port-Index configuration for up to rank 4. If present, the network configures port indexes for at least one of the ranks.

portIndex2 Port-Index configuration for up to rank 2. If present, the network configures port indexes for at least one of the ranks.

portIndex1 Port-Index configuration for rank 1.

TDCP field descriptions

delayDSetofLengthY Configures a set of Y delay values for TDCP reporting, see reference TS 38.214 clause 5.2.1.4. The symb4 denotes 4 symbols, the slot1 denotes 1 slot, the slot2 denotes 2 slots and so on. The value slot10 is applicable only to SCS >=30kHz. The parameter Y, see reference TS38.214 clause 5.2.1.4, is given by the length of the set of D values.

phaseReporting Configures the UE for phase reporting for TDCP reporting see reference TS 38.214 clause 5.2.1.4

#### –	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::=              INTEGER (0..maxNrofCSI-ReportConfigurations-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	CSI-ReportSubConfig

The IE CSI-ReportSubConfig is used to configure parameters in one sub-configuration within a CSI report configuration.

CSI-ReportSubConfig information element

-- ASN1START

-- TAG-CSI-REPORTSUBCONFIG-START

CSI-ReportSubConfig-r18 ::=             SEQUENCE {

reportSubConfigId-r18                   CSI-ReportSubConfigId-r18,

reportSubConfigParams                   CHOICE {

a1-parameters                           SEQUENCE {

codebookSubConfig-r18                   CodebookConfig                                       OPTIONAL,   -- Need R

portSubsetIndicator-r18                 CHOICE {

p2                                      BIT STRING (SIZE (2)),

p4                                      BIT STRING (SIZE (4)),

p8                                      BIT STRING (SIZE (8)),

p12                                     BIT STRING (SIZE (12)),

p16                                     BIT STRING (SIZE (16)),

p24                                     BIT STRING (SIZE (24)),

p32                                     BIT STRING (SIZE (32))

}                                                                                            OPTIONAL,   -- Need R

non-PMI-PortIndication-r18              PortIndexFor8Ranks                                   OPTIONAL   -- Need R

},

a2-parameters                           SEQUENCE {

nzp-CSI-RS-ResourceList-r18             SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18

}

}                                                                                                    OPTIONAL,   -- Need R

powerOffset-r18                         INTEGER(0..23)                                               OPTIONAL    -- Need R

}

NZP-CSI-RS-ResourceIndex-r18 ::= INTEGER (0..maxNrofNZP-CSI-RS-ResourcesPerSet-1-r18)

-- TAG-CSI-REPORTSUBCONFIG-STOP

-- ASN1STOP

CSI-ReportSubConfig field descriptions

codebookSubConfig Applicable codebook parameters for the ports indicated by portSubsetIndicator. Applicable value ranges for codebook subset restriction, rank restriction, N1, N2, and Ng and twoTX-CodebookSubsetRestriction follow existing specification according to the codebookConfig configured within the CSI-ReportConfig, and apply for the number of ports determined by portSubsetIndicator (see TS 38.214 [19], clause 5.2.1.4.1). In this field, the network always sets the codebookType to type1.

non-PMI-PortIndication Port indication for RI/CQI calculation applicable only for non-PMI feedback. The field shall be configured only if the portSubsetIndicator-r18 is configured. A port indication for each rank R, indicating which R ports out of P ports to use within the sub-configuration. P corresponds to the number of bits with value 1 in the bitmap portSubsetIndicator-r18.

nzp-CSI-RS-ResourceList List of NZP CSI RS resources for the sub-configuration that is a (sub)set of NZP CSI-RS resource(s) of the CSI-RS resource set for channel measurement associated with the sub-configuration in the CSI report configuration. Value 0 refers to the first NZP CSI RS resource of the CSI-RS resource set, value 1 refers to the second NZP CSI RS resource of the CSI-RS resource set, and so on.

port-SubsetIndicator Indicates the number of ports of the NZP CSI-RS resources indicated in nzp-CSI-RS-ResourceList and the (sub)set of CSI-RS antenna ports used for CSI calculation of the sub-configuration. In the bit string, each bit corresponds to an antenna port. When a bit is set to 1, the corresponding port is enabled for CSI calculation corresponding to the sub-configuration. When the bit is set to zero, the corresponding port is not enabled for CSI calcualton corresponding to the sub-configuration.

powerOffset When powerControlOffset is configured in a NZP CSI-RS resources indicated by nzp-CSI-RS-Resources, the power offset of PDSCH RE to NZP CSI-RS RE is equal to powerControlOffset - powerOffset.

#### –	CSI-ReportSubConfigId

The IE CSI-ReportSubConfigId is used to indicate the index of one CSI-ReportSubConfig within a CSI report configuration.

CSI-ReportSubConfigId information element

-- ASN1START

-- TAG-CSI-REPORTSUBCONFIGID-START

CSI-ReportSubConfigId-r18 ::=              INTEGER (0..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-1-r18)

-- TAG-CSI-REPORTSUBCONFIGID-STOP

-- ASN1STOP

#### –	CSI-ReportSubConfigTriggerList

The IE CSI-ReportSubConfigTriggerList is used to configure a list of sub-configuration ID(s) of N sub-configurations out of L configured sub-configurations within a CSI-ReportConfig associated with a triggering state for semi-persistent CSI reporting on PUSCH and aperiodic CSI reporting.

CSI-ReportSubConfigTriggerList information element

-- ASN1START

-- TAG-CSI-REPORTSUBCONFIGTRIGGERLIST-START

CSI-ReportSubConfigTriggerList-r18 ::= SEQUENCE (SIZE(1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18

-- TAG-CSI-REPORTSUBCONFIGTRIGGERLIST-STOP

-- ASN1STOP

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

CSI-ResourceConfig field descriptions

bwp-Id The DL BWP which the CSI-RS associated with this CSI-ResourceConfig are located in (see TS 38.214 [19], clause 5.2.1.2.

csi-IM-ResourceSetList List of references to CSI-IM resources used for CSI measurement and reporting in a CSI-RS resource set. Contains up to maxNrofCSI-IM-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).

csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig.

csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt List of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2). The csi-SSB-ResourceSetListExt provides additional references and can only be configured if csi-SSB-ResourceSetList is configured and groupBasedBeamReporting-v1710 is configured in the CSI-ReportConfig that indicates this CSI-ResourceConfig as resourcesForChannelMeasurement. If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourceForChannelMeasurement, the network configures 2 resource sets, which may be two CSI SSB resource sets (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

nzp-CSI-RS-ResourceSetList List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set. If resourceType is set to 'aperiodic', the network configures up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets. If resourceType is is set to 'periodic' or 'semiPersistent' and groupBasedBeamReporting-v1710 is not configured in IE CSI-ReportConfig, the network configures 1 resource set. If resourceType is set to 'periodic' or 'semiPersistent' and groupBasedBeamReporting-v1710 is configured, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1. If resourceType is set to 'periodic' and reportQuantity is set to 'TDCP', the network configures up to 3 resource sets, see TS 38.214 [19] clause 5.2.1.2.

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2). It does not apply to resources provided in the csi-SSB-ResourceSetList.

#### –	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::=            INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	CSI-ResourcePeriodicityAndOffset

The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, value slots5 corresponds to 5 slots, and so on.

CSI-ResourcePeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START

CSI-ResourcePeriodicityAndOffset ::=    CHOICE {

slots4                                  INTEGER (0..3),

slots5                                  INTEGER (0..4),

slots8                                  INTEGER (0..7),

slots10                                 INTEGER (0..9),

slots16                                 INTEGER (0..15),

slots20                                 INTEGER (0..19),

slots32                                 INTEGER (0..31),

slots40                                 INTEGER (0..39),

slots64                                 INTEGER (0..63),

slots80                                 INTEGER (0..79),

slots160                                INTEGER (0..159),

slots320                                INTEGER (0..319),

slots640                                INTEGER (0..639)

}

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	CSI-RS-ResourceConfigMobility

The IE CSI-RS-ResourceConfigMobility is used to configure CSI-RS based RRM measurements.

CSI-RS-ResourceConfigMobility information element

-- ASN1START

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-START

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

associatedSSB                       SEQUENCE {

ssb-Index                           SSB-Index,

isQuasiColocated                    BOOLEAN

}                                                                                                           OPTIONAL, -- Need R

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12))

},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

sequenceGenerationConfig            INTEGER (0..1023),

...,

[[

slotConfig-r17                      CHOICE {

ms4                                 INTEGER (0..255),

ms5                                 INTEGER (0..319),

ms10                                INTEGER (0..639),

ms20                                INTEGER (0..1279),

ms40                                INTEGER (0..2559)

}                                                                                                           OPTIONAL  -- Need R

]]

}

CSI-RS-Index ::=                    INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-STOP

-- ASN1STOP

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per measObjectNR depends on the configuration of associatedSSB and the support of increasedNumberofCSIRSPerMO capability (see TS 38.214 [19], clause 5.1.6.1.3).

density Frequency domain density for the 1-port CSI-RS for L3 mobility. See TS 38.211 [16], clause 7.4.1.

nrofPRBs Allowed size of the measurement BW in PRBs. See TS 38.211 [16], clause 7.4.1.

startPRB Starting PRB index of the measurement bandwidth. See TS 38.211 [16], clause 7.4.1.

CSI-RS-ResourceConfigMobility field descriptions

csi-RS-CellList-Mobility List of cells for CSI-RS based RRM measurements.

refServCellIndex Indicates the serving cell providing the timing reference for CSI-RS resources without associatedSSB. The field may be present only if there is at least one CSI-RS resource configured without associatedSSB. If this field is absent, the UE shall use the timing of the PCell for measurements on the CSI-RS resources without associatedSSB. The CSI-RS resources and the serving cell indicated by refServCellIndex for timing reference should be located in the same band.

subcarrierSpacing Subcarrier spacing of CSI-RS. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

CSI-RS-Resource-Mobility field descriptions

associatedSSB If this field is present, the UE may base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the cell indicated by the cellId in the CSI-RS-CellMobility. In this case, the UE is not required to monitor that CSI-RS resource if the UE cannot detect the SS/PBCH block indicated by this associatedSSB and cellId. If this field is absent, the UE shall base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the serving cell indicated by refServCellIndex. In this case, the UE is required to measure the CSI-RS resource even if SS/PBCH block(s) with cellId in the CSI-RS-CellMobility are not detected. CSI-RS resources with and without associatedSSB may be configured in accordance with the rules in TS 38.214 [19], clause 5.1.6.1.3.

csi-RS-Index CSI-RS resource index associated to the CSI-RS resource to be measured (and used for reporting).

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS, see TS 38.211 [16], clause 7.4.1.5.3.

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211 [16], clause 7.4.1.5.3 including table 7.4.1.5.2-1. The number of bits that may be set to one depend on the chosen row in that table.

isQuasiColocated Indicates that the CSI-RS resource is quasi co-located with the associated SS/PBCH block, see TS 38.214 [19], clause 5.1.6.1.3.

sequenceGenerationConfig Scrambling ID for CSI-RS (see TS 38.211 [16], clause 7.4.1.5.2).

slotConfig Indicates the CSI-RS periodicity (in milliseconds) and for each periodicity the offset (in number of slots). When subcarrierSpacing is set to kHz15, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 3/4/9/19/39 slots. When subcarrierSpacing is set to kHz30, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 7/9/19/39/79 slots. When subcarrierSpacing is set to kHz60, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 15/19/39/79/159 slots. When subcarrierSpacing is set kHz120, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 31/39/79/159/319 slots. When subcarrierSpacing is set to kHz480, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 127/159/319/639/1279 slots. When subcarrierSpacing is set to kHz960, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 255/319/639/1279/2559 slots. If slotConfig-r17 is present, UE shall ignore the slotConfig (without suffix).

#### –	CSI-RS-ResourceMapping

The IE CSI-RS-ResourceMapping is used to configure the resource element mapping of a CSI-RS resource in time- and frequency domain.

CSI-RS-ResourceMapping information element

-- ASN1START

-- TAG-CSI-RS-RESOURCEMAPPING-START

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

-- TAG-CSI-RS-RESOURCEMAPPING-STOP

-- ASN1STOP

CSI-RS-ResourceMapping field descriptions

cdm-Type CDM type (see TS 38.214 [19], clause 5.2.2.3.1).

density Density of CSI-RS resource measured in RE/port/PRB (see TS 38.211 [16], clause 7.4.1.5.3). Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1, values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32, value 1 (one) is allowed for X=4, 8, 12. For density = 1/2, includes 1-bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS.

firstOFDMSymbolInTimeDomain2 Time domain allocation within a physical resource block. See TS 38.211 [16], clause 7.4.1.5.3.

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. See TS 38.211 [16], clause 7.4.1.5.3.

freqBand Wideband or partial band CSI-RS, (see TS 38.214 [19], clause 5.2.2.3.1).

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211 [16], clause 7.4.1.5.3. The applicable row number in table 7.4.1.5.3-1 is determined by the frequencyDomainAllocation for rows 1, 2 and 4, and for other rows by matching the values in the column Ports, Density and CDMtype in table 7.4.1.5.3-1 with the values of nrofPorts, cdm-Type and density below and, when more than one row has the 3 values matching, by selecting the row where the column (k bar, l bar) in table 7.4.1.5.3-1 has indexes for k ranging from 0 to 2*n-1 where n is the number of bits set to 1 in frequencyDomainAllocation.

nrofPorts Number of ports (see TS 38.214 [19], clause 5.2.2.3.1).

#### –	CSI-SemiPersistentOnPUSCH-TriggerStateList

The CSI-SemiPersistentOnPUSCH-TriggerStateList IE is used to configure the UE with list of trigger states for semi-persistent reporting of channel state information on L1. See also TS 38.214 [19], clause 5.2.

CSI-SemiPersistentOnPUSCH-TriggerStateList information element

-- ASN1START

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-START

CSI-SemiPersistentOnPUSCH-TriggerStateList ::= SEQUENCE(SIZE (1..maxNrOfSemiPersistentPUSCH-Triggers)) OF CSI-SemiPersistentOnPUSCH-TriggerState

CSI-SemiPersistentOnPUSCH-TriggerState ::=     SEQUENCE {

associatedReportConfigInfo                     CSI-ReportConfigId,

...,

[[

sp-CSI-MultiplexingMode-r17                    ENUMERATED {enabled}                                           OPTIONAL   -- Need R

]],

[[

csi-ReportSubConfigTriggerList-r18             CSI-ReportSubConfigTriggerList-r18                             OPTIONAL,  -- Need R

ltm-AssociatedReportConfigInfo-r18             LTM-CSI-ReportConfigId-r18                                     OPTIONAL   -- Need R

]]

}

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-STOP

-- ASN1STOP

CSI-SemiPersistentOnPUSCH-TriggerStateList field descriptions

csi-ReportSubConfigTriggerList A list of sub-configuration ID(s) of N sub-configurations out of L configured sub-configurations within a CSI-ReportConfig associated with a triggering state for semi-persistent CSI reporting on PUSCH.

ltm-AssociatedReportConfigInfo This field configures semi-persistent CSI reports on PUSCH of LTM candidate cells. If ltm-associatedReportConfigInfo is configured the UE shall ignore the field associatedReportConfigInfo.

sp-CSI-MultiplexingMode Indicates if the behavior of transmitting SP-CSI on the first PUSCH repetitions corresponding to two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' is enabled or not.

#### –	CSI-SSB-ResourceSet

The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon and ServingCellConfig.

CSI-SSB-ResourceSet information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESET-START

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17        SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF ServingAdditionalPCIIndex-r17  OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

-- TAG-CSI-SSB-RESOURCESET-STOP

-- ASN1STOP

CSI-SSB-ResourceSet field descriptions

servingAdditionalPCIList Indicates the physical cell IDs (PCI) of the SSBs in the csi-SSB-ResourceList. If present, the list has the same number of entries as csi-SSB-ResourceList. The first entry of the list indicates the value of the PCI for the first entry of csi-SSB-ResourceList, the second entry of this list indicates the value of the PCI for the second entry of csi-SSB-ResourceList, and so on. For each entry, the following applies: - If the value is zero, the PCI is the PCI of the serving cell in which this CSI-SSB-ResourceSet is defined; - otherwise, the value is additionalPCIIndex-r17 of an SSB-MTC-AdditionalPCI-r17 configured using the additionalPCI-ToAddModList-r17 in ServingCellConfig, and the PCI is the additionalPCI-r17 in this SSB-MTC-AdditionalPCI-r17.

#### –	CSI-SSB-ResourceSetId

The IE CSI-SSB-ResourceSetId is used to identify one SS/PBCH block resource set.

CSI-SSB-ResourceId information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESETID-START

CSI-SSB-ResourceSetId ::=           INTEGER (0..maxNrofCSI-SSB-ResourceSets-1)

-- TAG-CSI-SSB-RESOURCESETID-STOP

-- ASN1STOP

#### –	DedicatedNAS-Message

The IE DedicatedNAS-Message is used to transfer UE specific NAS layer information between the 5GC CN and the UE. The RRC layer is transparent for this information.

DedicatedNAS-Message information element

-- ASN1START

-- TAG-DEDICATED-NAS-MESSAGE-START

DedicatedNAS-Message ::=        OCTET STRING

-- TAG-DEDICATED-NAS-MESSAGE-STOP

-- ASN1STOP

#### –	DL-PPW-PreConfig

The IE DL-PPW-PreConfig provides configuration for a measurement window where a UE is expected to measure the DL PRS, if it is inside the active DL BWP and with the same numerology as the active DL BWP. Based upon the indication received in the configuration, the UE identifies whether the DL PRS priority is higher than that of the other DL signals or channels and accordingly determines, for example, the UE is expected to measure the DL PRS and is not expected to receive other DL signals and channels.

DL-PPW-PreConfig information element

-- ASN1START

-- TAG-DL-PPW-PRECONFIG-START

DL-PPW-PreConfig-r17 ::= SEQUENCE {

dl-PPW-ID-r17            DL-PPW-ID-r17,

dl-PPW-PeriodicityAndStartSlot-r17     DL-PPW-PeriodicityAndStartSlot-r17,

length-r17                               INTEGER (1..160),

type-r17                                 ENUMERATED {type1A, type1B, type2}                            OPTIONAL, -- Cond MultiType

priority-r17                             ENUMERATED {st1, st2, st3}                                    OPTIONAL  -- Cond MultiState

}

DL-PPW-ID-r17 ::= INTEGER  (0..maxNrofPPW-ID-1-r17)

DL-PPW-PeriodicityAndStartSlot-r17 ::= CHOICE {

scs15                                    CHOICE {

n4                         INTEGER (0..3),

n5                         INTEGER (0..4),

n8                         INTEGER (0..7),

n10                        INTEGER (0..9),

n16                        INTEGER (0..15),

n20                        INTEGER (0..19),

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n160                       INTEGER (0..159),

n320                       INTEGER (0..319),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

...

},

scs30                                    CHOICE {

n8                         INTEGER (0..7),

n10                        INTEGER (0..9),

n16                        INTEGER (0..15),

n20                        INTEGER (0..19),

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n128                       INTEGER (0..127),

n160                       INTEGER (0..159),

n320                       INTEGER (0..319),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

n20480                     INTEGER (0..20479),

...

},

scs60                                    CHOICE {

n16                        INTEGER (0..15),

n20                        INTEGER (0..19),

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n128                       INTEGER (0..127),

n160                       INTEGER (0..159),

n256                       INTEGER (0..255),

n320                       INTEGER (0..319),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

n20480                     INTEGER (0..20479),

n40960                     INTEGER (0..40959),

...

},

scs120                                   CHOICE {

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n128                       INTEGER (0..127),

n160                       INTEGER (0..159),

n256                       INTEGER (0..255),

n320                       INTEGER (0..319),

n512                       INTEGER (0..511),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

n20480                     INTEGER (0..20479),

n40960                     INTEGER (0..40959),

n81920                     INTEGER (0..81919),

...

},

...

}

-- TAG-DL-PPW-PRECONFIG-STOP

-- ASN1STOP

DL-PPW-PreConfig field descriptions

dl-PPW-ID Indicates the pre-configured ID for DL-PRS processing window configuration.

dl-PPW-PeriodicityAndStartSlot Indicates the periodicity in slots and the offset of the starting slot with respect to SFN #0 slot #0 of the serving cell where the DL-PRS processing window is configured.

length Indicates the length of DL-PRS processing window in slots. Value 1 indicates length of one slot, value 2 indicates length of two slots and so on.

priority Indicates the priority between PDCCH/PDSCH/CSI-RS and PRS as specified in TS 38.214 [19].

type Indicates the DL-PRS processing window type as specified in TS 38.214 [19].

| Conditional Presence | Explanation |
| --- | --- |
| MultiType | The field is mandatory present when the UE reports its capability on supporting multiple processing types, otherwise it is absent. |
| MultiState | The field is mandatory present when the UE reports its capability on supporting option 1 or option 2 for the configured type, otherwise it is absent. |

#### –	DMRS-BundlingPUCCH-Config

The IE DMRS-BundlingPUCCH-Config-r17 is used to configure DMRS bundling for PUCCH.

DMRS-BundlingPUCCH-Config information element

-- ASN1START

-- TAG-DMRS-BUNDLINGPUCCH-CONFIG-START

DMRS-BundlingPUCCH-Config-r17 ::=         SEQUENCE {

pucch-DMRS-Bundling-r17                   ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pucch-TimeDomainWindowLength-r17          INTEGER (2..8)                                                  OPTIONAL,   -- Need S

pucch-WindowRestart-r17                   ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pucch-FrequencyHoppingInterval-r17        ENUMERATED {s2, s4, s5, s10}                                    OPTIONAL,   -- Need S

...

}

-- TAG-DMRS-BUNDLINGPUCCH-CONFIG-STOP

-- ASN1STOP

DMRS-BundlingPUCCH-Config field descriptions

pucch-DMRS-Bundling Indicates whether DMRS bundling and time domain window for PUCCH are jointly enabled.

pucch-FrequencyHoppingInterval Configures the number of consecutive slots for the UE to perform inter-slot frequency hopping with inter-slot bundling for PUCCH. When both inter-frequency hopping and DMRS bundling are enabled for PUCCH repetitions, the UE is expected to be configured with at least one pucch-FrequencyHoppingInterval-r17 and pucch-TimeDomainWindowLength-r17. When DMRS bundling for PUCCH is enabled by pucch-DMRS-Bundling-r17, PUCCH frequency hopping interval is only determined by the configuration of PUCCH hopping interval if PUCCH hopping interval is configured. If the field is absent, the number of consecutive slots for the UE to perform inter-slot PUCCH frequency hopping is indicated by pucch-TimeDomainWindowLength-r17.

pucch-TimeDomainWindowLength Configures the length of a nominal time domain window in slots for DMRS bundling for PUCCH. The value shall not exceed the maximum duration for DMRS bundling for PUCCH as specified in TS 38.306 [26]. If this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the time duration for the transmission of all PUCCH repetitions and the maximum duration for DMRS bundling for PUCCH as specified in TS 38.306 [26].

pucch-WindowRestart Indicates whether UE bundles PUCCH DMRS remaining in a nominal time domain window after event(s) triggered by DCI or MAC CE that violate power consistency and phase continuity requirements is enabled (see TS 38.214 [19], clause 6.1.7). NOTE: Events, which are triggered by DCI or MAC CE, but do not require UE capability to resume maintaining power consistency and/or phase continuity as specified in clause 6.1.7 of TS 38.214 [19], are excluded.

#### –	DMRS-BundlingPUSCH-Config

The IE DMRS-BundlingPUSCH-Config-r17 is used to configure DMRS bundling for PUSCH.

DMRS-BundlingPUSCH-Config information element

-- ASN1START

-- TAG-DMRS-BUNDLINGPUSCH-CONFIG-START

DMRS-BundlingPUSCH-Config-r17 ::=          SEQUENCE {

pusch-DMRS-Bundling-r17                    ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pusch-TimeDomainWindowLength-r17           INTEGER (2..32)                                                 OPTIONAL,   -- Need S

pusch-WindowRestart-r17                    ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pusch-FrequencyHoppingInterval-r17         ENUMERATED {s2, s4, s5, s6, s8, s10, s12, s14, s16, s20}        OPTIONAL,   -- Need S

...

}

-- TAG-DMRS-BUNDLINGPUSCH-CONFIG-STOP

-- ASN1STOP

DMRS-BundlingPUSCH-Config field descriptions

pusch-DMRS-Bundling Indicates whether DMRS bundling and time domain window for PUSCH are jointly enabled.

pusch-FrequencyHoppingInterval Configures the number of consecutive slots for the UE to perform inter-slot frequency hopping with inter-slot bundling for PUSCH. When both inter-frequency hopping and DMRS bundling are enabled for PUSCH repetitions, the UE is expected to be configured with at least one pusch-FrequencyHoppingInterval-r17 and pusch-TimeDomainWindowLength-r17. This parameter is shared for both DG-PUSCH and CG-PUSCH. When DMRS bundling for PUSCH is enabled by pusch-DMRS-Bundling-r17, PUSCH frequency hopping interval is only determined by the configuration of PUSCH hopping interval if PUSCH hopping interval is configured. If the field is absent, the number of consecutive slots for the UE to perform inter-slot PUSCH frequency hopping is indicated by pusch-TimeDomainWindowLength-r17. Note: For unpaired spectrum, the UE is not expected to be configured the value of s6, s8, s12, s14 and s16.

pusch-TimeDomainWindowLength Configures the length of a nominal time domain window in number of consecutive slots for DMRS bundling for PUSCH. The value shall not exceed the maximum duration for DMRS bundling for PUSCH as specified in TS 38.306 [26]. For PUSCH repetition type A/B, if this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the time duration for the transmission of all PUSCH repetitions and the maximum duration for DMRS bundling for PUSCH as specified in TS 38.306 [26]. For TBoMS, if this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the duration of TBoMS transmission (including repetition of TBoMS) and the maximum duration for DMRS bundling for PUSCH as specified in TS 38.306 [26].

pusch-WindowRestart Indicates whether UE bundles PUSCH DMRS remaining in a nominal time domain window after event(s) triggered by DCI or MAC CE that violate power consistency and phase continuity requirements is enabled (see TS 38.214 [19], clause 6.1.7). NOTE: Events, which are triggered by DCI or MAC CE, but do not require UE capability to resume maintaining power consistency and/or phase continuity as specified in clause 6.1.7 of TS 38.214 [19], are excluded.

#### –	DMRS-DownlinkConfig

The IE DMRS-DownlinkConfig is used to configure downlink demodulation reference signals for PDSCH.

DMRS-DownlinkConfig information element

-- ASN1START

-- TAG-DMRS-DOWNLINKCONFIG-START

DMRS-DownlinkConfig ::=             SEQUENCE {

dmrs-Type                           ENUMERATED {type2}                                                      OPTIONAL,   -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}                                           OPTIONAL,   -- Need S

maxLength                           ENUMERATED {len2}                                                       OPTIONAL,   -- Need S

scramblingID0                       INTEGER (0..65535)                                                      OPTIONAL,   -- Need S

scramblingID1                       INTEGER (0..65535)                                                      OPTIONAL,   -- Need S

phaseTrackingRS                     SetupRelease { PTRS-DownlinkConfig  }                                   OPTIONAL,   -- Need M

...,

[[

dmrs-Downlink-r16               ENUMERATED {enabled}                                                        OPTIONAL    -- Need R

]],

[[

dmrs-TypeEnh-r18                ENUMERATED {enabled}                                                        OPTIONAL    -- Need R

]]

}

-- TAG-DMRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

DMRS-DownlinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in DL, see Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 in TS 38.211 [16]. If the field is absent, the UE applies the value pos2. See also clause 7.4.1.1.2 for additional constraints on how the network may set this field depending on the setting of other fields.

dmrs-Downlink This field indicates whether low PAPR DMRS is used, as specified in TS38.211 [16], clause 7.4.1.1.1.

dmrs-Type Selection of the DMRS type to be used for DL (see TS 38.211 [16], clause 7.4.1.1.1). If the field is absent, the UE uses DMRS type 1.

dmrs-TypeEnh Enables the use of enhanced DMRS type1 or enhanced DMRS type2 with field dmrs-Type (see TS 38.211 [16], clause 7.4.1.1.1). If the field is present and dmrs-Type is absent, the UE uses DMRS eType 1. If the field is present and dmrs-Type is present, the UE uses DMRS eType 2.

maxLength The maximum number of OFDM symbols for DL front loaded DMRS. len1 corresponds to value 1. len2 corresponds to value 2. If the field is absent, the UE applies value len1. If set to len2, the UE determines the actual number of DM-RS symbols by the associated DCI. (see TS 38.211 [16], clause 7.4.1.1.2).

phaseTrackingRS Configures downlink PTRS. If the field is not configured, the UE assumes that downlink PTRS are absent. See TS 38.214 [19] clause 5.1.6.3.

scramblingID0 DL DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.1.1). When the field is absent the UE applies the value physCellId configured for this serving cell.

scramblingID1 DL DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.1.1). When the field is absent the UE applies the value physCellId configured for this serving cell.

#### –	DMRS-UplinkConfig

The IE DMRS-UplinkConfig is used to configure uplink demodulation reference signals for PUSCH.

DMRS-UplinkConfig information element

-- ASN1START

-- TAG-DMRS-UPLINKCONFIG-START

DMRS-UplinkConfig ::=               SEQUENCE {

dmrs-Type                           ENUMERATED {type2}                                                  OPTIONAL,   -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}                                       OPTIONAL,   -- Need S

phaseTrackingRS                     SetupRelease { PTRS-UplinkConfig }                                  OPTIONAL,   -- Need M

maxLength                           ENUMERATED {len2}                                                   OPTIONAL,   -- Need S

transformPrecodingDisabled          SEQUENCE {

scramblingID0                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

scramblingID1                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

...,

[[

dmrs-Uplink-r16                     ENUMERATED {enabled}                                            OPTIONAL    -- Need R

]]

}                                                                                                       OPTIONAL,   -- Need R

transformPrecodingEnabled           SEQUENCE {

nPUSCH-Identity                     INTEGER(0..1007)                                                OPTIONAL,   -- Need S

sequenceGroupHopping                ENUMERATED {disabled}                                           OPTIONAL,   -- Need S

sequenceHopping                     ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

...,

[[

dmrs-UplinkTransformPrecoding-r16   SetupRelease {DMRS-UplinkTransformPrecoding-r16}                OPTIONAL    -- Need M

]]

}                                                                                                       OPTIONAL,   -- Need R

...,

[[

dmrs-TypeEnh-r18                    ENUMERATED {enabled}                                                OPTIONAL    -- Need R

]]

}

DMRS-UplinkTransformPrecoding-r16  ::=  SEQUENCE {

pi2BPSK-ScramblingID0                   INTEGER(0..65535)                                               OPTIONAL,   -- Need S

pi2BPSK-ScramblingID1                   INTEGER(0..65535)                                               OPTIONAL    -- Need S

}

-- TAG-DMRS-UPLINKCONFIG-STOP

-- ASN1STOP

DMRS-UplinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in UL (see TS 38.211 [16], clause 6.4.1.1.3). If the field is absent, the UE applies the value pos2. See also clause 6.4.1.1.3 for additional constraints on how the network may set this field depending on the setting of other fields.

dmrs-Type Selection of the DMRS type to be used for UL (see TS 38.211 [16], clause 6.4.1.1.3) If the field is absent, the UE uses DMRS type 1.

dmrs-TypeEnh Enables the use of enhanced DMRS type1 or enhanced DMRS type2 with field dmrs-Type (see TS 38.211 [16], clause 6.4.1.1.3). If the field is present and dmrs-Type is absent, the UE uses DMRS eType 1. If the field is present and dmrs-Type is present, the UE uses DMRS eType 2

dmrs-Uplink This field indicates whether low PAPR DMRS is used, as specified in TS38.211 [16], clause 6.4.1.1.1.1.

dmrs-UplinkTransformPrecoding This field indicates whether low PAPR DMRS is used for PUSCH with pi/2 BPSK modulation, as specified in TS38.211 [16], clause 6.4.1.1.1.2. The network configures this field only if tp-pi2BPSK is configured in PUSCH-Config.

maxLength The maximum number of OFDM symbols for UL front loaded DMRS. len1 corresponds to value 1. len2 corresponds to value 2. If the field is absent, the UE applies value len1. If set to len2, the UE determines the actual number of DM-RS symbols by the associated DCI. (see TS 38.211 [16], clause 6.4.1.1.3).

nPUSCH-Identity Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS. If the value is absent or released, the UE uses the value Physical cell ID (physCellId). See TS 38.211 [16].

phaseTrackingRS Configures uplink PTRS (see TS 38.211 [16]).

pi2BPSK-ScramblingID0, pi2BPSK-ScramblingID1 UL DMRS scrambling initialization for pi/2 BPSK DMRS for PUSCH (see TS 38.211 [16], Clause 6.4.1.1.2). When the field is absent the UE applies the value Physical cell ID (physCellId) of the serving cell.

scramblingID0 UL DMRS scrambling initialization for CP-OFDM (see TS 38.211 [16], clause 6.4.1.1.1.1). When the field is absent the UE applies the value Physical cell ID (physCellId).

scramblingID1 UL DMRS scrambling initialization for CP-OFDM. (see TS 38.211 [16], clause 6.4.1.1.1.1). When the field is absent the UE applies the value Physical cell ID (physCellId).

sequenceGroupHopping For DMRS transmission with transform precoder the NW may configure group hopping by the cell-specific parameter groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon. In this case, the NW may include this UE specific field to disable group hopping for PUSCH transmission except for Msg3, i.e., to override the configuration in PUSCH-ConfigCommon (see TS 38.211 [16]). If the field is absent, the UE uses the same hopping mode as for Msg3.

sequenceHopping Determines if sequence hopping is enabled for DMRS transmission with transform precoder for PUSCH transmission other than Msg3 (sequence hopping is always disabled for Msg3). If the field is absent, the UE uses the same hopping mode as for msg3. The network does not configure simultaneous group hopping and sequence hopping. See TS 38.211 [16], clause 6.4.1.1.1.2.

transformPrecodingDisabled DMRS related parameters for Cyclic Prefix OFDM.

transformPrecodingEnabled DMRS related parameters for DFT-s-OFDM (Transform Precoding).

#### –	DownlinkConfigCommon

The IE DownlinkConfigCommon provides common downlink parameters of a cell.

DownlinkConfigCommon information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMON-START

DownlinkConfigCommon ::=        SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL                                 OPTIONAL,   -- Cond InterFreqHOAndServCellAdd

initialDownlinkBWP              BWP-DownlinkCommon                              OPTIONAL,   -- Cond ServCellAdd

...,

[[

initialDownlinkBWP-RedCap-r17   BWP-DownlinkCommon                              OPTIONAL    -- Need R

]]

}

-- TAG-DOWNLINKCONFIGCOMMON-STOP

-- ASN1STOP

DownlinkConfigCommon field descriptions

frequencyInfoDL Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a serving cell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.

initialDownlinkBWP-RedCap If present, (e)RedCap UEs use this DL BWP instead of initialDownlinkBWP. If absent, (e)RedCap UEs use initialDownlinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover, and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell) and upon handover from E-UTRA to NR. It is optionally present, Need M otherwise. |

#### –	DownlinkConfigCommonSIB

The IE DownlinkConfigCommonSIB provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMONSIB-START

DownlinkConfigCommonSIB ::=     SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL-SIB,

initialDownlinkBWP              BWP-DownlinkCommon,

bcch-Config                     BCCH-Config,

pcch-Config                     PCCH-Config,

...,

[[

pei-Config-r17                  PEI-Config-r17                         OPTIONAL,     -- Need R

initialDownlinkBWP-RedCap-r17   BWP-DownlinkCommon                     OPTIONAL      -- Need R

]],

[[

frequencyInfoDL-v1800           FrequencyInfoDL-SIB-v1800              OPTIONAL      -- Need R

]]

}

DownlinkConfigCommonSIB-v1760 ::=   SEQUENCE {

frequencyInfoDL-v1760               FrequencyInfoDL-SIB-v1760

}

BCCH-Config ::=                 SEQUENCE {

modificationPeriodCoeff         ENUMERATED {n2, n4, n8, n16},

...

}

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16        INTEGER (2..4)               OPTIONAL  -- Cond SharedSpectrum2

]],

[[

ranPagingInIdlePO-r17                              ENUMERATED {true}            OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

]]

}

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

SubgroupConfig-r17 ::=     SEQUENCE {

subgroupsNumPerPO-r17      INTEGER (1.. maxNrofPagingSubgroups-r17),

subgroupsNumForUEID-r17    INTEGER (1.. maxNrofPagingSubgroups-r17)                                        OPTIONAL,  -- Need S

...

}

-- TAG-DOWNLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

DownlinkConfigCommonSIB field descriptions

bcch-Config The modification period related configuration.

frequencyInfoDL-SIB Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a PCell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain. The UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment.

initialDownlinkBWP-RedCap If present, (e)RedCap UEs use this DL BWP instead of initialDownlinkBWP. If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. Otherwise, i.e., if the locationAndBandwidth of this BWP does not contain the entire CORESET#0, the UE uses this BWP for receiving DL messages during initial access (Msg2, MsgB, Msg4) and after initial access. If absent, (e)RedCap UEs use initialDownlinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

lastUsedCellOnly When present, the field indicates that the UE monitors PEI only if the latest received RRCRelease without noLastCellUpdate is from this cell. A PEI-capable UE stores its last used cell information.

nrofPDCCH-MonitoringOccasionPerSSB-InPO The number of PDCCH monitoring occasions corresponding to an SSB within a Paging Occasion, see TS 38.304 [20], clause 7.1.

pcch-Config The paging related configuration.

pei-Config The PEI related configuration.

subgroupConfig The paging subgroup related configuration.

BCCH-Config field descriptions

modificationPeriodCoeff Actual modification period, expressed in number of radio frames m = modificationPeriodCoeff * defaultPagingCycle, see clause 5.2.2.2.2. n2 corresponds to value 2, n4 corresponds to value 4, and so on.

PCCH-Config field descriptions

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

firstPDCCH-MonitoringOccasionOfPO Points out the first PDCCH monitoring occasion for paging of each PO of the PF, see TS 38.304 [20].

nAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 160 ms, N can be set to oneSixteenthT If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT}

ns Number of paging occasions per paging frame.

ranPagingInIdlePO Indicates that the network supports to send RAN paging in PO that corresponds to the i_s as determined by UE in RRC_IDLE state, see TS38.304 [20].

PEI-Config field descriptions

payloadSizeDCI-2-7 Payload size of PEI DCI, i.e., DCI format 2_7. The size is no larger than the payload size of paging DCI which has maximum of 41 bits and 43 bits for licensed and unlicensed spectrums, respectively.

pei-FrameOffset Offset, in number of frames from the start of a reference frame for PEI-O to the start of a first paging frame of the paging frames associated with the PEI-O, see TS 38.213 [13], clause 10.4A.

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

SubgroupConfig field descriptions

subgroupsNumPerPO Total number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from physical-layer signaling. The field represents the sum of CN-assigned and UEID-based subgroups supported by the network. When PEI-Config is configured, there is always at least one subgroup (UEID-based subgroup or CN-assigned subgroup) configured.

subgroupsNumForUEID Number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from physical-layer signaling, for UEID-based subgrouping method. When present, the field is set to an integer smaller than or equal to subgroupsNumPerPO. subgroupsNumPerPO equals to subgroupsNumForUEID when the network does not configure CN-assigned subgrouping. When pei-Config is configured, the field is absent when the network only configures CN-assigned subgrouping. Both this field and subgroupsNumPerPO are equal to 1 when the network does not configure subgrouping. When pei-Config is configured, if the field is absent, the UE uses subgrouping according to TS 38.304 [20], clause 7.3.0.

| Conditional Presence | Explanation |
| --- | --- |
| SharedSpectrum2 | The field is optional present, Need R, if this cell operates with shared spectrum channel access. Otherwise, it is absent, Need R. |

#### –	DownlinkPreemption

The IE DownlinkPreemption is used to configure the UE to monitor PDCCH for the INT-RNTI (interruption).

DownlinkPreemption information element

-- ASN1START

-- TAG-DOWNLINKPREEMPTION-START

DownlinkPreemption ::=              SEQUENCE {

int-RNTI                            RNTI-Value,

timeFrequencySet                    ENUMERATED {set0, set1},

dci-PayloadSize                     INTEGER (0..maxINT-DCI-PayloadSize),

int-ConfigurationPerServingCell     SEQUENCE (SIZE (1..maxNrofServingCells)) OF INT-ConfigurationPerServingCell,

...

}

INT-ConfigurationPerServingCell ::= SEQUENCE {

servingCellId                       ServCellIndex,

positionInDCI                       INTEGER (0..maxINT-DCI-PayloadSize-1)

}

-- TAG-DOWNLINKPREEMPTION-STOP

-- ASN1STOP

DownlinkPreemption field descriptions

dci-PayloadSize Total length of the DCI payload scrambled with INT-RNTI (see TS 38.213 [13], clause 11.2).

int-ConfigurationPerServingCell Indicates (per serving cell) the position of the 14 bit INT values inside the DCI payload (see TS 38.213 [13], clause 11.2).

int-RNTI RNTI used for indication pre-emption in DL (see TS 38.213 [13], clause 10).

timeFrequencySet Set selection for DL-preemption indication (see TS 38.213 [13], clause 11.2) The set determines how the UE interprets the DL preemption DCI payload.

INT-ConfigurationPerServingCell field descriptions

positionInDCI Starting position (in number of bit) of the 14 bit INT value applicable for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.2). Must be multiples of 14 (bit).

#### –	DRB-Identity

The IE DRB-Identity is used to identify a DRB used by a UE.

DRB-Identity information element

-- ASN1START

-- TAG-DRB-IDENTITY-START

DRB-Identity ::=                    INTEGER (1..32)

-- TAG-DRB-IDENTITY-STOP

-- ASN1STOP

#### –	DRX-Config

The IE DRX-Config is used to configure DRX related parameters.

DRX-Config information element

-- ASN1START

-- TAG-DRX-CONFIG-START

DRX-Config ::=                      SEQUENCE {

drx-onDurationTimer                 CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                 ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL                INTEGER (0..56),

drx-HARQ-RTT-TimerUL                INTEGER (0..56),

drx-RetransmissionTimerDL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

}

DRX-ConfigExt-v1700 ::=                 SEQUENCE {

drx-HARQ-RTT-TimerDL-r17                INTEGER (0..448),

drx-HARQ-RTT-TimerUL-r17                INTEGER (0..448)

}

DRX-ConfigExt2-v1800 ::=                SEQUENCE {

drx-NonIntegerLongCycleStartOffset-r18  CHOICE {

ms1001over240                           INTEGER(0..3),

ms25over6                               INTEGER(0..3),

ms25over3                               INTEGER(0..7),

ms1001over120                           INTEGER(0..7),

ms100over9                              INTEGER(0..10),

ms25over2                               INTEGER(0..11),

ms40over3                               INTEGER(0..12),

ms125over9                              INTEGER(0..12),

ms50over3                               INTEGER(0..15),

ms1001over60                            INTEGER(0..15),

ms125over6                              INTEGER(0..19),

ms200over9                              INTEGER(0..21),

ms250over9                              INTEGER(0..26),

ms100over3                              INTEGER(0..32),

ms1001over30                            INTEGER(0..32),

ms75over2                               INTEGER(0..36),

ms125over3                              INTEGER(0..40),

ms1001over24                            INTEGER(0..40),

ms200over3                              INTEGER(0..65),

ms1001over15                            INTEGER(0..65),

ms250over3                              INTEGER(0..82),

ms1001over12                            INTEGER(0..82),

ms400over3                              INTEGER(0..132),

...

},

shortDRX-r18                            SEQUENCE {

drx-NonIntegerShortCycle-r18            ENUMERATED {ms1001over240, ms25over6, ms25over3, ms1001over120, ms100over9, ms25over2,

ms40over3, ms125over9, ms50over3, ms1001over60, ms125over6, ms200over9,

ms100over3, ms1001over30, ms125over3, ms1001over24, ms200over3, spare15,

spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6,

spare5, spare4, spare3, spare2, spare1},

drx-ShortCycleTimer-r18                 INTEGER (1..16)

}                                                                                                            OPTIONAL,   -- Need R

drx-TimeReferenceSFN-r18                    ENUMERATED {sfn512}                                              OPTIONAL    -- Need S

}

-- TAG-DRX-CONFIG-STOP

-- ASN1STOP

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received. drx-HARQ-RTT-TimerDL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerDL (without suffix) for SCS 480 kHz and 960 kHz.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted. drx-HARQ-RTT-TimerUL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerUL (without suffix) for SCS 480 kHz and 960 kHz.

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1 ms. If drx-ShortCycle is configured, the value of drx-LongCycle shall be a multiple of the drx-ShortCycle value.

drx-NonIntegerLongCycleStartOffset drx-NonIntegerLongCycle in non-integer number of ms (e.g. ms1001over240 corresponds to 1001/240 ms, ms25over6 corresponds to 25/6 ms and so on) and drx-StartOffset in multiples of 1 ms. If drx-NonIntegerShortCycle is configured, the value of drx-NonInetegerLongCycle shall be a multiple of the drx-NonIntegerShortCycle value. If drx-NonIntegerLongCycleStartOffset-r18 is configured, the UE shall ignore drx-LongCycleStartOffset.

drx-NonIntegerShortCycle Value in non-integer number of ms, e.g. ms1001over240 corresponds to 1001/240 ms, ms25over6 corresponds to 25/6 ms and so on.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

drx-RetransmissionTimerDL Value in number of slot lengths of the BWP where the transport block was received. value sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slot lengths of the BWP where the transport block was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-ShortCycleTimer Value in multiples of drx-ShortCycle or drx-NonIntegerShortCycle (depending on which one is configured). A value of 1 corresponds to drx-ShortCycle or drx-NonIntegerShortCycle, a value of 2 corresponds to 2 * drx-ShortCycle or 2 * drx-NonIntegerShortCycle and so on.

drx-ShortCycle Value in ms. ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

drx-TimeReferenceSFN Indicates how the UE initializes the counter DRX_SFN_COUNTER, see TS 38.321 [3], clause 5.7. If the field drx-TimeReferenceSFN is not present, the reference SFN is 0.

shortDRX Configuration of a short DRX cycle. The network configures only one of shortDRX (without a suffix) or shortDRX-r18.

#### –	DRX-ConfigSecondaryGroup

The IE DRX-ConfigSecondaryGroup is used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3].

DRX-ConfigSecondaryGroup information element

-- ASN1START

-- TAG-DRX-CONFIGSECONDARYGROUP-START

DRX-ConfigSecondaryGroup-r16 ::=   SEQUENCE {

drx-onDurationTimer-r16            CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer-r16            ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

-- TAG-DRX-CONFIGSECONDARYGROUP-STOP

-- ASN1STOP

DRX-ConfigSecondaryGroup field descriptions

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0 ms, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on, as specified in TS 38.321 [3]. The network configures a drx-InactivityTimer value for the second DRX group that is smaller than the drx-InactivityTimer configured for the default DRX group in IE DRX-Config.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSeconds). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on, as specified in TS 38.321 [3]. The network configures a drx-onDurationTimer value for the second DRX group that is smaller than the drx-onDurationTimer configured for the default DRX group in IE DRX-Config.

#### –	DRX-ConfigSL

The IE DRX-ConfigSL is used to configure additional DRX parameters for the UE performing sidelink operation with resource allocation mode 1, as specified in TS 38.321 [3].

DRX-ConfigSL information element

-- ASN1START

-- TAG-DRX-CONFIGSL-START

DRX-ConfigSL-r17 ::=            SEQUENCE {

drx-HARQ-RTT-TimerSL-r17        INTEGER (0..56),

drx-RetransmissionTimerSL-r17   ENUMERATED {sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

-- TAG-DRX-CONFIGSL-STOP

-- ASN1STOP

DRX-ConfigSL field descriptions

drx-HARQ-RTT-TimerSL For sidelink configured grant Type 1, value in number of symbols of the activated DL BWP of PCell. For other cases, value in number of symbols of the BWP where the PDCCH was transmitted. Value 0 is used in case sl-PUCCH-Config is not configured and the corresponding resource pool is not configured with PSFCH.

drx-RetransmissionTimerSL For sidelink configured grant Type 1, value in number of slot lengths of the activated DL BWP of PCell. For other cases, value in number of slot lengths of the BWP where the PDCCH was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

#### –	EarlyUL-SyncConfig

The IE EarlyUL-SyncConfig is used to configure random access resources for the early UL synchronization procedure.

EarlyUL-SyncConfig information element

-- ASN1START

-- TAG-EARLYUL-SYNCCONFIG-START

EarlyUL-SyncConfig-r18 ::=         SEQUENCE {

frequencyInfoUL-r18                FrequencyInfoUL,

rach-ConfigGeneric-r18             RACH-ConfigGeneric,

bwp-GenericParameters-r18          BWP,

ssb-PerRACH-Occasion-r18           ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL, -- Need R

prach-RootSequenceIndex-r18        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                             OPTIONAL, -- Need R

ltm-PRACH-SubcarrierSpacing-r18    SubcarrierSpacing                                                          OPTIONAL, -- Need L139

n-TimingAdvanceOffset-r18          ENUMERATED { n0, n25600, n39936, spare1 }                                  OPTIONAL, -- Need R

...

}

-- TAG-EARLYUL-SYNCCONFIG-STOP

-- ASN1STOP

EarlyUL-SyncConfig field descriptions

frequencyInfoUL This field provides basic parameters of an uplink carrier for PRACH transmission on a candidate cell.

ltm-PRACH-SubcarrierSpacing Indicates subcarrier spacing of PRACH for LTM (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]).

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on a candidate cell.

rach-ConfigGeneric RACH parameters for performing a random access procedure on a candidate cell.

ssb-PerRACH-Occasion This field indicated the number of SSBs for RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |

#### –	EphemerisInfo

The IE EphemerisInfo provides satellite ephemeris. Ephemeris may be expressed either in format of position and velocity state vector in ECEF or in format of orbital parameters in ECI. Note: The ECI and ECEF coincide at epochTime, i.e., x,y,z axis in ECEF are aligned with x,y,z axis in ECI at epochTime.

EphemerisInfo information element

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

EphemerisInfo field descriptions

eccentricity Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [71]. Unit is radian. Step of 1.431 * 10-8. Actual value = field value * (1.431 * 10-8).

inclination Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

longitude Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

meanAnomaly Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

periapsis Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

semiMajorAxis Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [71]. Unit is meter. Step of 4.249 * 10-3 m. Actual value = 6500000 + field value * (4.249 * 10-3).

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

#### –	EpochTime

The IE EpochTime is used to indicate the epoch time for the NTN assistance information, and it is defined as the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The reference point for EpochTime of the serving or neighbour NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point when this field is provided in an NTN cell and the gNB when this field is provided in a TN cell.

EpochTime information element

-- ASN1START

-- TAG-EPOCHTIME-START

EpochTime-r17 ::=              SEQUENCE {

sfn-r17                        INTEGER(0..1023),

subFrameNR-r17                 INTEGER(0..9)

}

-- TAG-EPOCHTIME-STOP

-- ASN1STOP

EpochTime field descriptions

sfn For serving cell, it indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. For neighbour cell, it indicates the SFN nearest to the frame where the message indicating the epochTime is received.

#### –	EUTRA-C-RNTI

The IE EUTRA-C-RNTI identifies a UE having a RRC connection within an E-UTRA cell.

EUTRA-C-RNTI information element

-- ASN1START

-- TAG-EUTRACRNTI-START

EUTRA-C-RNTI ::=                      BIT STRING (SIZE (16))

-- TAG-EUTRACRNTI-STOP

-- ASN1STOP

#### –	FeatureCombination

The IE FeatureCombination indicates a feature or a combination of features to be associated with a set of Random Access resources (i.e. an instance of FeatureCombinationPreambles).

FeatureCombination information element

-- ASN1START

-- TAG-FEATURECOMBINATION-START

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                                    OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                                    OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                        OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                                    OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                                    OPTIONAL,  -- Need R

eRedCap-r18                ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                                    OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

-- TAG-FEATURECOMBINATION-STOP

-- ASN1STOP

FeatureCombination field descriptions

eRedCap If present, this field indicates that eRedCap is part of this feature combination. The fields redCap and eRedCap shall not be both set to true. If the UE is an eRedCap UE and there is no set of configured RA resources with eRedCap set to true among all sets of configured RA resources, the UE considers redCap to be applicable for random access procedure. This field is not configured in a set of preambles that is configured with 2-step random-access type.

msg1-Repetitions If present, this field indicates that signalling of msg1 repetition is part of this feature combination. This field is not configured in a set of preambles that is configured with 2-step random-access type.

msg3-Repetitions If present, this field indicates that signalling of msg3 repetition is part of this feature combination. This field is not configured in a set of preambles that is configured with 2-step random-access type.

nsag If present, this field indicates NSAG(s) that are part of this feature combination.

redCap If present, this field indicates that RedCap is part of this feature combination.

smallData If present, this field indicates that Small Data is part of this feature combination.

#### –	FeatureCombinationPreambles

The IE FeatureCombinationPreambles associates a set of preambles with a feature combination. For parameters which can be provided in this IE, the UE applies this field value when performing Random Access using a preamble in this featureCombinationPreambles, otherwise the UE applies the corresponding value as determined by applicable Need Code, e.g. Need S. On a specific BWP, there can be at most one set of preambles associated with a given feature combination per RA Type (i.e. 4-step RACH or 2-step RACH) per MSG1 repetition number.

FeatureCombinationPreambles information element

-- ASN1START

-- TAG-FEATURECOMBINATIONPREAMBLES-START

FeatureCombinationPreambles-r17 ::=   SEQUENCE {

featureCombination-r17                FeatureCombination-r17,

startPreambleForThisPartition-r17     INTEGER (0..63),

numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64),

ssb-SharedRO-MaskIndex-r17            INTEGER (1..15)                                           OPTIONAL, -- Need S

groupBconfigured-r17                  SEQUENCE {

ra-SizeGroupA-r17                     ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r17          ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r17        INTEGER (1..64)

}                                                                                               OPTIONAL, -- Need R

separateMsgA-PUSCH-Config-r17         MsgA-PUSCH-Config-r16                                     OPTIONAL, -- Cond MsgAConfigCommon

msgA-RSRP-Threshold-r17               RSRP-Range                                                OPTIONAL, -- Need R

rsrp-ThresholdSSB-r17                 RSRP-Range                                                OPTIONAL, -- Need R

deltaPreamble-r17                     INTEGER (-1..6)                                           OPTIONAL, -- Need R

...,

[[

msg1-RepetitionNum-r18                ENUMERATED {n2, n4, n8, spare1}                                   OPTIONAL, -- Cond Msg1Rep2

msg1-RepetitionTimeOffsetROGroup-r18  ENUMERATED {n4, n8, n16, spare1}                             OPTIONAL  -- Cond Msg1Rep3

]]

}

-- TAG-FEATURECOMBINATIONPREAMBLES-STOP

-- ASN1STOP

FeatureCombinationPreambles field descriptions

deltaPreamble Power offset between msg3 or msgA-PUSCH and RACH preamble transmission. If configured, this parameter overrides msg3-DeltaPreamble or msgA-DeltaPreamble, Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1). If msgA-DeltaPreamble is configured in separateMsgA-PUSCH-Config-r17, this field is absent. This field is set to the same value for all FeatureCombinationPreambles for MSG1 repetitions.

featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on (see TS 38.321 [3], clause 5.1.2).

msg1-RepetitionNum Indicates which MSG1-repetition number that this FeatureCombinationPreambles is associated with.

msg1-RepetitionTimeOffsetROGroup Indicates a time offset of the starting ROs between two successive RO groups for a given repetition number (2, 4 or 8) associated with this FeatureCombinationPreambles for each frequency resource index within a time period (see TS 38.213 [13]). If this field is absent, the time offset is implicitly determined (see TS 38.213 [13]). For each MSG1 repetition number, the following values are applicable. • {n16}, for RO groups for MSG1 repetition number 8 • {n8, n16}, for RO groups for MSG1 repetition number 4 • {n4, n8, n16}, for RO groups for MSG1 repetition number 2

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the concerned feature combination in the BWP. If configured, this parameter overrides msgA-RSRP-Threshold-r16. If absent, the UE applies msgA-RSRP-Threshold-r16, if configured

numberOfPreamblesPerSSB-ForThisPartition It determines how many consecutive preambles are associated to the Feature Combination starting from the starting preamble(s) per SSB.

numberOfRA-PreamblesGroupA It determines how many consecutive preambles per SSB are associated to Group A starting from the starting preamble(s). The remaining preambles associated to the Feature Combination are associated to Group B

ra-SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.2). If this feature combination preambles are associated to a RACH-ConfigCommon-twostepRA, this field correspond to ra-MsgA-SizeGroupA, otherwise it corresponds to ra-Msg3SizeGroupA.

rsrp-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]). If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, it corresponds to msgA-RSRP-ThresholdSSB, as defined in TS 38.321 [3]. If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommon, it it corresponds to rsrp-ThresholdSSB, as defined in TS 38.321 [3].

separateMsgA-PUSCH-Config If present it specifies how the 2-step RACH preambles identified by this FeatureCombinationPreambles are mapped to a PUSCH slot separate from the one defined in MsgA-ConfigCommon-r16. If the field is absent, the UE should apply the corresponding parameter in the RACH-ConfigCommonTwoStepRA of the BWP which includes the FeatureCombinationPreambles IE.

ssb-SharedRO-MaskIndex Mask index (see TS 38.321 [3]). Indicates a subset of ROs where preambles are allocated for this feature combination. If this field is configured within FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA: - in case of separate ROs are configured for 4-step and 2-step random access, this field indicates a subset of ROs configured within this RACH-ConfigCommonTwoStepRA; - in case shared ROs are used for 4-step and 2-step random access, it indicates the subset of ROs configured within RACH-ConfigCommon, which are the subset of ROs configured for 2-step random access. This field is configured when there is more than one RO per SSB. If the field is absent, all ROs configured in RACH-ConfigCommon or RACH-ConfigCommonTwoStepRA containing this FeatureCombinationPreambles are shared.

startPreambleForThisPartition It defines the first preamble associated with the Feature Combination. If the UE is provided with a number N of SSB block indexes associated with one PRACH occasion, and N<1, the first preamble in each PRACH occasion is the one having the same index as indicated by this field. If N>=1, N blocks of preambles associated with the Feature Combination are defined, each having start index + startPreambleForThisPartition, where n refers to SSB block index (see TS 38.213 [13], clause 8.1).

| Conditional Presence | Explanation |
| --- | --- |
| MsgAConfigCommon | The field is optionally present, Need S, if FeatureCombinationPreambles is included in RACH-ConfigCommonTwoStepRA. Otherwise, it is absent. If the field is absent in FeatureCombinationPreambles included in RACH-ConfigCommonTwoStepRA, the UE applies MsgA-PUSCH-Config included in the corresponding MsgA-ConfigCommon. |
| Msg1Rep2 | The field is mandatory present, Need R, if msg1-Repetitions is included in FeatureCombination for this concerned FeatureCombinationPreambles. Otherwise, it is absent. |
| Msg1Rep3 | The field is optionally present, Need S, if msg1-Repetitions is included in FeatureCombination for this concerned FeatureCombinationPreambles. Otherwise, it is absent. |

#### –	FilterCoefficient

The IE FilterCoefficient specifies the measurement filtering coefficient. Value fc0 corresponds to k = 0, fc1 corresponds to k = 1, and so on.

FilterCoefficient information element

-- ASN1START

-- TAG-FILTERCOEFFICIENT-START

FilterCoefficient ::=       ENUMERATED { fc0, fc1, fc2, fc3, fc4, fc5, fc6, fc7, fc8, fc9, fc11, fc13, fc15, fc17, fc19, spare1, ...}

-- TAG-FILTERCOEFFICIENT-STOP

-- ASN1STOP

#### –	FreqBandIndicatorNR

The IE FreqBandIndicatorNR is used to convey an NR frequency band number as defined in TS 38.101-1 [15], TS 38.101-2 [39] and TS 38.101-5 [75].

FreqBandIndicatorNR information element

-- ASN1START

-- TAG-FREQBANDINDICATORNR-START

FreqBandIndicatorNR ::=             INTEGER (1..1024)

-- TAG-FREQBANDINDICATORNR-STOP

-- ASN1STOP

#### –	FreqPriorityListDedicatedSlicing

The IE FreqPriorityListDedicatedSlicing provides dedicated cell reselection priorities for slicing in RRCRelease.

FreqPriorityListDedicatedSlicing information element

-- ASN1START

-- TAG-FREQPRIORITYLISTDEDICATEDSLICING-START

FreqPriorityListDedicatedSlicing-r17 ::= SEQUENCE (SIZE (1.. maxFreq)) OF FreqPriorityDedicatedSlicing-r17

FreqPriorityDedicatedSlicing-r17 ::=     SEQUENCE {

dl-ExplicitCarrierFreq-r17               ARFCN-ValueNR,

sliceInfoListDedicated-r17               SliceInfoListDedicated-r17                                    OPTIONAL  -- Cond Mandatory

}

SliceInfoListDedicated-r17 ::=           SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfoDedicated-r17

SliceInfoDedicated-r17 ::=               SEQUENCE {

nsag-IdentityInfo-r17                    NSAG-IdentityInfo-r17,

nsag-CellReselectionPriority-r17         CellReselectionPriority                                       OPTIONAL,  -- Need R

nsag-CellReselectionSubPriority-r17      CellReselectionSubPriority                                    OPTIONAL   -- Need R

}

-- TAG-FREQPRIORITYLISTDEDICATEDSLICING-STOP

-- ASN1STOP

FreqPriorityDedicatedSlicing field descriptions

dl-ExplicitCarrierFreq Indicates the downlink carrier frequency to which SliceInfoListDedicated is associated.

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |

#### –	FreqPriorityListSlicing

The IE FreqPriorityListSlicing indicates cell reselection priorities for slicing in SIB16.

FreqPriorityListSlicing information element

-- ASN1START

-- TAG-FREQPRIORITYLISTSLICING-START

FreqPriorityListSlicing-r17 ::= SEQUENCE (SIZE (1..maxFreqPlus1)) OF FreqPrioritySlicing-r17

FreqPrioritySlicing-r17 ::=     SEQUENCE {

dl-ImplicitCarrierFreq-r17            INTEGER (0..maxFreq),

sliceInfoList-r17                     SliceInfoList-r17                                               OPTIONAL  -- Cond Mandatory

}

SliceInfoList-r17 ::=             SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17

SliceInfo-r17 ::=                 SEQUENCE {

nsag-IdentityInfo-r17             NSAG-IdentityInfo-r17,

nsag-CellReselectionPriority-r17  CellReselectionPriority                                             OPTIONAL,  -- Need R

nsag-CellReselectionSubPriority-r17 CellReselectionSubPriority                                        OPTIONAL,  -- Need R

sliceCellListNR-r17               CHOICE {

sliceAllowedCellListNR-r17        SliceCellListNR-r17,

sliceExcludedCellListNR-r17       SliceCellListNR-r17

}                                                                                                     OPTIONAL  -- Need R

}

SliceCellListNR-r17 ::=           SEQUENCE (SIZE (1..maxCellSlice-r17)) OF PCI-Range

-- TAG-FREQPRIORITYLISTSLICING-STOP

-- ASN1STOP

FreqPriorityListSlicing field descriptions

dl-ImplicitCarrierFreq Indicates the downlink carrier frequency to which sliceInfoList is associated with. The frequency is signalled implicitly, value 0 corresponds to the serving frequency, value 1 corresponds to the first frequency indicated by the InterFreqCarrierFreqList in SIB4, and value 2 corresponds to the second frequency indicated by the InterFreqCarrierFreqList in SIB4, and so on.

SliceInfo field descriptions

sliceAllowedCellListNR List of allow-listed cells for slicing. If present, the cells listed in this list support the corresponding nsag-frequency pair, and the cells not listed in this list do not support the corresponding nsag-frequency pair, according to TS 38.304 [20], clause 5.2.4.11.

sliceCellListNR Contains either the list of allow-listed or exclude-listed cells for slicing. If absent, it implies all the cells support the corresponding nsag-frequency pair, according to 38.304 [20], clause 5.2.4.11.

sliceExcludedCellListNR List of exclude-listed cells for slicing. If present, the cells listed in this list do not support the corresponding nsag-frequency pair, and the cells not listed in this list support the corresponding nsag-frequency pair, according to TS 38.304 [20], clause 5.2.4.11.

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |

#### –	FrequencyInfoDL

The IE FrequencyInfoDL provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL information element

-- ASN1START

-- TAG-FREQUENCYINFODL-START

FrequencyInfoDL ::=                 SEQUENCE {

absoluteFrequencySSB                ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SpCellAdd

frequencyBandList                   MultiFrequencyBandListNR,

absoluteFrequencyPointA             ARFCN-ValueNR,

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...,

[[

referenceCell-r18                   ServCellIndex                                                   OPTIONAL    -- Cond SSBlessSCell

]]

}

-- TAG-FREQUENCYINFODL-STOP

-- ASN1STOP

FrequencyInfoDL field descriptions

absoluteFrequencyPointA Absolute frequency position of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the "default cell" defined in TS [xx]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting (e)RedCap, this field corresponds to the CD-SSB.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent a "default cell" is the reference cell as defined in TS 38.133 [14].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| SpCellAdd | The field is mandatory present if this FrequencyInfoDL is for SpCell. Otherwise the field is optionally present, Need S. |
| SSBlessSCell | The field is optionally present, Need S, if the inter-band SSB-less SCell is configured for this cell and absoluteFrequencySSB is absent. It is absent otherwise. |

#### –	FrequencyInfoDL-SIB

The IE FrequencyInfoDL-SIB provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL-SIB information element

-- ASN1START

-- TAG-FREQUENCYINFODL-SIB-START

FrequencyInfoDL-SIB ::=             SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB,

offsetToPointA                      INTEGER (0..2199),

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier

}

FrequencyInfoDL-SIB-v1760 ::=       SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760

}

FrequencyInfoDL-SIB-v1800 ::=       SEQUENCE {

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18

}

-- TAG-FREQUENCYINFODL-SIB-STOP

-- ASN1STOP

FrequencyInfoDL-SIB field descriptions

offsetToPointA Represents the offset to Point A as defined in TS 38.211 [16], clause 4.4.4.2.

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. If frequencyBandList-v1760 is present, it shall contain the same number of entries, listed in the same order as in frequencyBandList (without suffix).

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A (see TS 38.211 [16], clause 5.3). The network configures this for all SCSs that are used in DL BWPs in this serving cell.

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

additionalSpectrumEmission-v1760    AdditionalSpectrumEmission-v1760                        OPTIONAL    -- Need S

]],

[[

additionalSpectrumEmissionAerial-r18   AdditionalSpectrumEmission-r18                       OPTIONAL   -- Need S

]]

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If both additionalSpectrumEmission (without suffix) and additionalSpectrumEmission-v1760 are absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], tables 6.2.3.1-1A, 6.2A.3.1.1-2 and 6.2A.3.1.2-2, TS 38.101-2 [39], tables 6.2.3.1-2 and 6.2A.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured and if signalled, the same vaue in additionalSpectrumEmission-v1760 for all uplink carrier(s) of the same band with UL configured, except for additionalSpectrumEmission value corresponding to NS_55/NS_57. If NS_55/NS_57 (see TS 38.101-1 [15], table 6.2.3.1-1) is applicable for at least one uplink carrier, the network may configure either NS_55/NS_57 or NS_01 for these uplink carriers, and NS_01 for the remaining uplink carrier(s) of band n77. The additionalSpectrumEmission is applicable for all uplink carriers of the same band with UL configured.

additionalSpectrumEmissionAerial The additional spectrum emission requirements to be applied by the aerial UE on this uplink (see TS 38.101-1 [15], tables TBD, TS 38.101-2 [39], tables TBD, and TS 38.101-5 [75], table TBD). If the field is absent, the aerial UE uses value indicated by the field additionalSpectrumEmission/ additionalSpectrumEmission-v1760.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. Value in dBm. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise. |

#### –	FrequencyInfoUL-SIB

The IE FrequencyInfoUL-SIB provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL-SIB information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-SIB-START

FrequencyInfoUL-SIB ::=             SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB                            OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18                 OPTIONAL    -- Need S

]]

}

FrequencyInfoUL-SIB-v1760 ::=       SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760

}

-- TAG-FREQUENCYINFOUL-SIB-STOP

-- ASN1STOP

FrequencyInfoUL-SIB field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

frequencyBandList Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the frequencyBandList field. If frequencyBandList-v1760 is present, it shall contain the same number of entries, listed in the same order as in frequencyBandList (without suffix).

frequencyBandListAerial Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for aerial UE as defined in TS 38.101-1 [15], table TBD, TS 38.101-2 [39], table TBD, and TS 38.101-5 [75], table TBD. If the field is present, the aerial UE shall apply the first listed band which it supports in the frequencyBandListAerial field. If the field is absent, frequencyBandList applies.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A (see TS 38.211 [16], clause 5.3). The network configures this for all SCSs that are used in UL BWPs configured in this serving cell.

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL-SIB is for the paired UL for a DL (defined in a FrequencyInfoDL-SIB) or if this FrequencyInfoUL-SIB is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL-SIB is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL-SIB is for the paired UL for a DL (defined in a FrequencyInfoDL-SIB), or if this FrequencyInfoUL-SIB is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL-SIB is for a supplementary uplink (SUL). It is absent otherwise. |

#### –	GapPriority

The IE GapPriority is used to identify the priority of a gap configuration.

GapPriority information element

-- ASN1START

-- TAG-GAPPRIORITY-START

GapPriority-r17 ::=                       INTEGER (1..maxNrOfGapPri-r17)

-- TAG-GAPPRIORITY-STOP

-- ASN1STOP

#### –	HighSpeedConfig

The IE HighSpeedConfig is used to configure parameters for high speed scenarios.

HighSpeedConfig information element

-- ASN1START

-- TAG-HIGHSPEEDCONFIG-START

HighSpeedConfig-r16 ::=  SEQUENCE {

highSpeedMeasFlag-r16    ENUMERATED {true}        OPTIONAL,   -- Cond SpCellOnly

highSpeedDemodFlag-r16   ENUMERATED {true}        OPTIONAL,   -- Need R

...

}

HighSpeedConfig-v1700 ::=  SEQUENCE {

highSpeedMeasCA-Scell-r17    ENUMERATED {true}        OPTIONAL,   -- Cond SCellOnly

highSpeedMeasInterFreq-r17   ENUMERATED {true}        OPTIONAL,   -- Cond SpCellOnly2

highSpeedDemodCA-Scell-r17   ENUMERATED {true}        OPTIONAL,   -- Need R

...

}

HighSpeedConfigFR2-r17 ::=  SEQUENCE {

highSpeedMeasFlagFR2-r17                    ENUMERATED {set1, set2}                       OPTIONAL,   -- Need R

highSpeedDeploymentTypeFR2-r17              ENUMERATED {unidirectional, bidirectional}    OPTIONAL,   -- Need R

highSpeedLargeOneStepUL-TimingFR2-r17       ENUMERATED {true}                             OPTIONAL,   -- Need R

...

}

-- TAG-HIGHSPEEDCONFIG-STOP

-- ASN1STOP

HighSpeedConfig field descriptions

HighSpeedDemodCA-Scell If the field is present and UE supports demodulationEnhancementCA-r17, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SCell.

highSpeedDemodFlag If the field is present and UE supports demodulationEnhancement-r16, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SpCell.

highSpeedDeploymentTypeFR2 If the field is present, and field value is unidirectional, the UE shall assume uni-directional deployment or if field value is birectional the UE shall assume bidirectional deployment for FR2 up to 350km/h as specified in TS 38.133 [14].

highSpeedLargeOneStepUL-TimingFR2 If the field is present, large one step UE autonomous uplink transmit timing adjustment for FR2 up to 350km/h as specified in TS 38.133 [14] is enabled.

highSpeedMeasCA-Scell If the field is present and UE supports measurementEnhancementCA-r17, the UE shall apply the enhanced RRM requirements to the serving frequency of SCell for carrier aggregation to support high speed up to 500 km/h as specified in TS 38.133 [14].

highSpeedMeasFlag If the field is present and UE supports measurementEnhancement-r16, the UE shall apply the enhanced intra-NR and inter-RAT EUTRAN RRM requirements to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports intraNR-MeasurementEnhancement-r16, the UE shall apply enhanced intra-NR RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports interRAT-MeasurementEnhancement-r16, the UE shall apply enhanced inter-RAT EUTRAN RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. This parameter only applies to the serving frequency of SpCell.

highSpeedMeasFlagFR2 If the field is present and UE supports ue-PowerClass-v1700 set to pc6, the UE shall apply enhanced intra-frequency RRM requirement to the serving frequency of SpCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14]. If the field is present and the UE supports enhanced inter-frequency RRM requirements for FR2 HST in RRC_IDLE and RRC_INACTIVE, the UE shall apply enhanced inter-frequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_IDLE and RRC_INACTIVE. If the field is present for SpCell and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced inter-frequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. If the field is present for SCell(s) and and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced intra-frequency RRM requirements to the serving frequency of the corresponding SCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. The field value, set1 or set2, is applied as specified in TS38.133 [14].

highSpeedMeasInterFreq If the field is present and UE supports measurementEnhancementInterFreq-r17, the UE shall apply the enhanced RRM requirements for inter-frequency measurement in RRC_CONNECTED to support high speed up to 500 km/h as specified in TS 38.133 [14].

| Conditional Presence | Explanation |
| --- | --- |
| SCellOnly | The field is optionally present, Need R, in ServingCellConfigCommon of an SCell. It is absent otherwise. |
| SpCellOnly | The field is optionally present, Need R, in ServingCellConfigCommonSIB or in the ServingCellConfigCommon of an SpCell. It is absent otherwise. |
| SpCellOnly2 | The field is optionally present, Need R, in ServingCellConfigCommon of an SpCell. It is absent otherwise. |

#### –	Hysteresis

The IE Hysteresis is a parameter used within the entry and leave condition of an event triggered reporting condition. The actual value is field value * 0.5 dB.

Hysteresis information element

-- ASN1START

-- TAG-HYSTERESIS-START

Hysteresis ::=                      INTEGER (0..30)

-- TAG-HYSTERESIS-STOP

-- ASN1STOP

#### –	HysteresisAltitude

The IE HysteresisAltitude is a parameter used within the entry and leave condition of an altitude-based event triggered reporting condition and within altitude range based parameter configuration. The actual value is field value in meters.

HysteresisAltitude information element

-- ASN1START

-- TAG-HYSTERESISALTITUDE-START

HysteresisAltitude-r18 ::=                      INTEGER (0..64)

-- TAG-HYSTERESISALTITUDE-STOP

-- ASN1STOP

#### –	HysteresisLocation

The IE HysteresisLocation is a parameter used within entry and leave condition of a location based event triggered reporting condition. The actual value is field value * 10 meters.

HysteresisLocation information element

-- ASN1START

-- TAG-HYSTERESISLOCATION-START

HysteresisLocation-r17 ::=          INTEGER (0..32768)

-- TAG-HYSTERESISLOCATION-STOP

-- ASN1STOP

#### –	InvalidSymbolPattern

The IE InvalidSymbolPattern is used to configure one invalid symbol pattern for PUSCH transmission repetition type B applicable for both DCI format 0_1 and 0_2, see TS 38.214 [19], clause 6.1.

InvalidSymbolPattern information element

-- ASN1START

-- TAG-INVALIDSYMBOLPATTERN-START

InvalidSymbolPattern-r16 ::=     SEQUENCE {

symbols-r16                      CHOICE {

oneSlot                          BIT STRING (SIZE (14)),

twoSlots                         BIT STRING (SIZE (28))

},

periodicityAndPattern-r16        CHOICE {

n2                               BIT STRING (SIZE (2)),

n4                               BIT STRING (SIZE (4)),

n5                               BIT STRING (SIZE (5)),

n8                               BIT STRING (SIZE (8)),

n10                              BIT STRING (SIZE (10)),

n20                              BIT STRING (SIZE (20)),

n40                              BIT STRING (SIZE (40))

}                                                                OPTIONAL,   -- Need M

...

}

-- TAG-INVALIDSYMBOLPATTERN-STOP

-- ASN1STOP

InvalidSymbolPattern field descriptions

periodicityAndPattern A time domain repetition pattern at which the pattern defined by symbols recurs. This slot pattern repeats itself continuously. When the field is not configured, the UE uses the value n1 (see TS 38.214 [19], clause 6.1).

symbols A symbol level bitmap in time domain (see TS 38.214[19], clause 6.1). For oneSlot, if ECP is configured, the first 12 bits represent the symbols within the slot and the last two bits within the bitstring are ignored by the UE; Otherwise, the 14 bits represent the symbols within the slot. For twoSlots, if ECP is configured, the first 12 bits represent the symbols within the first slot and the next 12 bits represent the symbols in the second slot and the last four bits within the bit string are ignored by the UE; Otherwise, the first 14 bits represent the symbols within the first slot and the next 14 bits represent the symbols in the second slot. For the bits representing symbols in a slot, the most significant bit of the bit string represents the first symbol in the slot and the second most significant bit represents the second symbol in the slot and so on. This pattern recurs (in time domain) with the configured periodicityAndPattern.

#### –	I-RNTI-Value

The IE I-RNTI-Value is used to identify the suspended UE context of a UE in RRC_INACTIVE.

I-RNTI-Value information element

-- ASN1START

-- TAG-I-RNTI-VALUE-START

I-RNTI-Value ::=                        BIT STRING (SIZE(40))

-- TAG-I-RNTI-VALUE-STOP

-- ASN1STOP

#### –	LBT-FailureRecoveryConfig

The IE LBT-FailureRecoveryConfig-r16 is used to configure the parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

LBT-FailureRecoveryConfig information element

-- ASN1START

-- TAG-LBT-FAILURERECOVERYCONFIG-START

LBT-FailureRecoveryConfig-r16 ::=    SEQUENCE {

lbt-FailureInstanceMaxCount-r16      ENUMERATED {n4, n8, n16, n32, n64, n128},

lbt-FailureDetectionTimer-r16        ENUMERATED {ms10, ms20, ms40, ms80, ms160, ms320},

...

}

-- TAG-LBT-FAILURERECOVERYCONFIG-STOP

-- ASN1STOP

LBT-FailureRecoveryConfig field descriptions

lbt-FailureDetectionTimer Timer for consistent uplink LBT failure detection (see TS 38.321 [3]). Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

lbt-FailureInstanceMaxCount This field determines after how many LBT failure indications received from the physical layer the UE triggers uplink LBT failure recovery (see TS 38.321 [3]). Value n4 corresponds to 4, value n8 corresponds to 8, and so on.

#### –	LocationInfo

The IE LocationInfo is used to transfer available detailed location information, Bluetooth, WLAN and sensor available measurement results at the UE.

LocationInfo information element

-- ASN1START

-- TAG-LOCATIONINFO-START

LocationInfo-r16 ::=      SEQUENCE {

commonLocationInfo-r16    CommonLocationInfo-r16          OPTIONAL,

bt-LocationInfo-r16       LogMeasResultListBT-r16         OPTIONAL,

wlan-LocationInfo-r16     LogMeasResultListWLAN-r16       OPTIONAL,

sensor-LocationInfo-r16   Sensor-LocationInfo-r16         OPTIONAL,

...

}

-- TAG-LOCATIONINFO-STOP

-- ASN1STOP

#### –	LocationMeasurementInfo

The IE LocationMeasurementInfo defines the information sent by the UE to the network to assist with the configuration of measurement gaps for location related measurements.

LocationMeasurementInfo information element

-- ASN1START

-- TAG-LOCATIONMEASUREMENTINFO-START

LocationMeasurementInfo ::=     CHOICE {

eutra-RSTD                  EUTRA-RSTD-InfoList,

...,

eutra-FineTimingDetection   NULL,

nr-PRS-Measurement-r16      NR-PRS-MeasurementInfoList-r16

}

EUTRA-RSTD-InfoList ::= SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info

EUTRA-RSTD-Info ::= SEQUENCE {

carrierFreq                 ARFCN-ValueEUTRA,

measPRS-Offset              INTEGER (0..39),

...

}

NR-PRS-MeasurementInfoList-r16 ::= SEQUENCE (SIZE (1..maxFreqLayers)) OF NR-PRS-MeasurementInfo-r16

NR-PRS-MeasurementInfo-r16 ::=      SEQUENCE {

dl-PRS-PointA-r16                   ARFCN-ValueNR,

nr-MeasPRS-RepetitionAndOffset-r16  CHOICE {

ms20-r16                            INTEGER (0..19),

ms40-r16                            INTEGER (0..39),

ms80-r16                            INTEGER (0..79),

ms160-r16                           INTEGER (0..159),

...

},

nr-MeasPRS-length-r16               ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

...

}

-- TAG-LOCATIONMEASUREMENTINFO-STOP

-- ASN1STOP

LocationMeasurementInfo field descriptions

carrierFreq The EARFCN value of the carrier received from upper layers for which the UE needs to perform the inter-RAT RSTD measurements.

measPRS-Offset Indicates the requested gap offset for performing RSTD measurements towards E-UTRA. It is the smallest subframe offset from the beginning of subframe 0 of SFN=0 of the serving cell of the requested gap for measuring PRS positioning occasions in the carrier frequency carrierFreq for which the UE needs to perform the inter-RAT RSTD measurements. The PRS positioning occasion information is received from upper layers. The value of measPRS-Offset is obtained by mapping the starting subframe of the PRS positioning occasion in the measured cell onto the corresponding subframe in the serving cell and is calculated as the serving cell's number of subframes from SFN=0 mod 40. The UE shall take into account any additional time required by the UE to start PRS measurements on the other carrier when it does this mapping for determining the measPRS-Offset. NOTE: Figure 6.2.2-1 in TS 36.331[10] illustrates the measPRS-Offset field.

dl-PRS-PointA The ARFCN value of the carrier received from upper layers for which the UE needs to perform the NR DL-PRS measurements.

nr-MeasPRS-RepetitionAndOffset Indicates the gap periodicity in ms and offset in number of subframes of the requested measurement gap for performing NR DL-PRS measurements.

nr-MeasPRS-length Indicates measurement gap length in ms of the requested measurement gap for performing NR DL-PRS measurements. The measurement gap length is according to in Table 9.1.2-1 in TS 38.133 [14].

#### –	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICALCHANNELCONFIG-START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (1..16),

prioritisedBitRate                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells                 SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex

OPTIONAL,   -- Cond PDCP-CADuplication

allowedSCS-List                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                   OPTIONAL,   -- Need R

maxPUSCH-Duration                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, ms0p01-v1700, spare1}

OPTIONAL,   -- Need R

configuredGrantType1Allowed         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

logicalChannelGroup                 INTEGER (0..maxLCG-ID)                                              OPTIONAL,   -- Need R

schedulingRequestID                 SchedulingRequestId                                                 OPTIONAL,   -- Need R

logicalChannelSR-Mask               BOOLEAN,

logicalChannelSR-DelayTimerApplied  BOOLEAN,

...,

bitRateQueryProhibitTimer       ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}               OPTIONAL,    -- Need R

[[

allowedCG-List-r16                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL,   -- Need S

allowedPHY-PriorityIndex-r16        ENUMERATED {p0, p1}                                                 OPTIONAL    -- Need S

]],

[[

logicalChannelGroupIAB-Ext-r17      INTEGER (0..maxLCG-ID-IAB-r17)                                      OPTIONAL,   -- Need R

allowedHARQ-mode-r17                ENUMERATED {harqModeA, harqModeB}                                   OPTIONAL    -- Need R

]]

}                                                                                                       OPTIONAL,   -- Cond UL

...,

[[

channelAccessPriority-r16           INTEGER (1..4)                                                      OPTIONAL,   -- Need R

bitRateMultiplier-r16               ENUMERATED {x40, x70, x100, x200}                                   OPTIONAL    -- Need R

]]

}

-- TAG-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

allowedHARQ-mode Indicates the allowed HARQ mode of a HARQ process mapped to this logical channel. If the parameter is absent, there is no restriction for HARQ mode for the mapping. This field applies to SRB1, SRB2, SRB4 and DRBs.

allowedPHY-PriorityIndex This restriction applies only when the UL grant is a dynamic grant. If the field is present and the dynamic grant has a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to the dynamic grants indicating PHY-priority index equal to the values configured by this field. If the field is present and the dynamic grant does not have a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to this dynamic grant if the value of the field is p0, see TS 38.213 [13], clause 9. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any dynamic grants. Corresponds to "allowedPHY-PriorityIndex" as specified in TS 38.321 [3].

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Corresponds to 'allowedSCS-List' as specified in TS 38.321 [3]. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3].

bitRateMultiplier Bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [3]. Value x40 indicates bit rate multiplier 40, value x70 indicates bit rate multiplier 70 and so on.

bitRateQueryProhibitTimer The timer is used for bit rate recommendation query in TS 38.321 [3], in seconds. Value s0 means 0 s, s0dot4 means 0.4 s and so on.

bucketSizeDuration Value in ms. ms5 corresponds to 5 ms, value ms10 corresponds to 10 ms, and so on.

channelAccessPriority Indicates the Channel Access Priority Class (CAPC), as specified in TS 38.300 [2], to be used on uplink transmissions for operation with shared spectrum channel access in FR1. The network configures this field only for SRB2 and DRBs.

configuredGrantType1Allowed If present, or if the capability lcp-Restriction as specified in TS 38.306 [26] is not supported, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Otherwise, UL MAC SDUs from this logical channel cannot be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

logicalChannelGroup, logicalChannelGroupIAB-Ext ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to. The logicalChannelGroupIAB-Ext is only applicable to the IAB-MT. When logicalChannelGroupIAB-Ext is configured, logicalChannelGroup shall be ignored.

logicalChannelSR-Mask Controls SR triggering when a configured uplink grant of type1 or type2 is configured. true indicates that SR masking is configured for this logical channel as specified in TS 38.321 [3].

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to false if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration" in TS 38.321 [3]. The PUSCH duration is calculated based on the same length of all symbols, and the shortest length applies if the symbol lengths are different.

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. Value kBps0 corresponds to 0 kiloBytes/s, value kBps8 corresponds to 8 kiloBytes/s, value kBps16 corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| PDCP-CADuplication | The field is mandatory present if the DRB/SRB associated with this logical channel is configured with PDCP CA duplication in UL in the cell group in which this IE is included (i.e. the PDCP entity is associated with multiple RLC entities belonging to this cell group). Otherwise the field is optionally present, need R. |
| UL | The field is mandatory present for a logical channel with uplink if it serves DRB or multicast MRB. It is optionally present, Need R, for a logical channel with uplink if it serves an SRB. Otherwise it is absent. |

#### –	LogicalChannelIdentity

The IE LogicalChannelIdentity is used to identify one logical channel (LogicalChannelConfig) and the corresponding RLC bearer (RLC-BearerConfig) or BH RLC channel (BH-RLC-ChannelConfig) or Uu Relay RLC channel (Uu-RelayRLC-ChannelConfig) or PC5 Relay RLC channel (SL-RLC-ChannelConfigPC5).

LogicalChannelIdentity information element

-- ASN1START

-- TAG-LOGICALCHANNELIDENTITY-START

LogicalChannelIdentity ::=          INTEGER (1..maxLC-ID)

-- TAG-LOGICALCHANNELIDENTITY-STOP

-- ASN1STOP

#### –	LTE-NeighCellsCRS-AssistInfoList

The IE LTE-NeighCellsCRS-AssistInfoList-r17 is used to provide configuration information of neighbour LTE cells to assist the UE to perform CRS interference mitigation (CRS-IM) in scenarios with overlapping spectrum for LTE and NR.

LTE-NeighCellsCRS-AssistInfoList information element

-- ASN1START

-- TAG-LTE-NEIGHCELLSCRS-ASSISTINFOLIST-START

LTE-NeighCellsCRS-AssistInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofCRS-IM-InterfCell-r17)) OF LTE-NeighCellsCRS-AssistInfo-r17

LTE-NeighCellsCRS-AssistInfo-r17 ::=     SEQUENCE {

neighCarrierBandwidthDL-r17              ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1}   OPTIONAL,   -- Cond CRS-IM

neighCarrierFreqDL-r17                   INTEGER (0..16383)                                          OPTIONAL,   -- Need S

neighCellId-r17                          EUTRA-PhysCellId                                            OPTIONAL,   -- Need S

neighCRS-muting-r17                      ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

neighMBSFN-SubframeConfigList-r17        EUTRA-MBSFN-SubframeConfigList                              OPTIONAL,   -- Need S

neighNrofCRS-Ports-r17                   ENUMERATED {n1, n2, n4}                                     OPTIONAL,   -- Need S

neighV-Shift-r17                         ENUMERATED {n0, n1, n2, n3, n4, n5}                         OPTIONAL    -- Cond NotCellID

}

-- TAG-LTE-NEIGHCELLSCRS-ASSISTINFOLIST-STOP

-- ASN1STOP

LTE-NeighCellsCRS-AssistInfo field descriptions

neighCarrierBandwidthDL Indicates the channel bandwidth of the neighbour LTE cell in number of PRBs. If the field is absent, the UE applies the value of carrierBandwidthDL indicated in RateMatchPatternLTE-CRS for this serving cell, if configured.

neighCarrierFreqDL Indicates the downlink centre frequency of the neighbour LTE cell. If the field is absent, the UE applies the value of carrierFreqDL indicated in RateMatchPatternLTE-CRS for this serving cell, if configured.

neighCellId Indicates the physical cell ID of the neighbour LTE cell for which the other fields within the same LTE-NeighCellsCRS-AssistInfo apply. If the IE LTE-NeighCellsCRS-AssistInfoList contains multiple list entries, either this field or neighV-Shift is included in each instance. If the IE LTE-NeighCellsCRS-AssistInfoList contains multiple list entries, the entry with neighV-Shift is only used for neighbour LTE cells for which neighCellId is not provided (i.e. the entry with neighCellId takes precedence over the entry with neighV-Shift, if provided). If the IE LTE-NeighCellsCRS-AssistInfoList contains one list entry with neither this field nor neighV-Shift, the information within the entry applies to all neighbour LTE cells.

neighCRS-muting Indicates whether the CRS interference mitigation is enabled in the neighbour LTE cell, as specified in TS 36.133 [40], clause 3.6.1.1.

neighMBSFN-SubframeConfigList Indicates the MBSFN subframe configuration of the neighbour LTE cell. If RateMatchPatternLTE-CRS is configured for this serving cell and the field is absent, the UE applies the value of mbsfn-SubframeConfigList indicated in RateMatchPatternLTE-CRS for this serving cell if configured; otherwise, if the field is absent, the UE assumes MBSFN is not configured in the neighbour LTE cell.

neighNrofCRS-Ports Indicates the CRS antenna ports number of the neighbour LTE cell. If the field is absent, the UE applies the value of nrofCRS-Ports indicated in RateMatchPatternLTE-CRS for this serving cell, if configured. If RateMatchPatternLTE-CRS is not configured for this serving cell and the field is absent, the UE applies the default value n4.

neighV-Shift Indicates the shifting value v-shift of neighbour LTE cells for which the other fields within the same LTE-NeighCellsCRS-AssistInfo apply.

| Conditional Presence | Explanation |
| --- | --- |
| CRS-IM | For the serving cell with 15kHz SCS, this field is mandatory present for the UE supporting the capability of crs-IM-nonDSS-NWA-15kHzSCS-r17, but not supporting crs-IM-nonDSS-15kHzSCS-r17, if RateMatchPatternLTE-CRS is not configured for this serving cell. Otherwise it is optionally present, Need S if RateMatchPatternLTE-CRS is configured for this serving cell; Need M otherwise. For the serving cell with 30kHz SCS, this field is mandatory present for the UE supporting the capability of crs-IM-nonDSS-NWA-30kHzSCS-r17, but not supporting crs-IM-nonDSS-30kHzSCS-r17, if RateMatchPatternLTE-CRS is not configured for this serving cell. Otherwise it is optionally present, Need S if RateMatchPatternLTE-CRS is configured for this serving cell; Need M otherwise. |
| NotCellID | If the field neighCellId is present, this field shall be absent; otherwise, it is optionally present if the IE LTE-NeighCellsCRS-AssistInfoList contains one list entry, Need S, or it is mandatory present if the IE LTE-NeighCellsCRS-AssistInfoList contains multiple list entries. |

#### –	LTM-CandidateId

The IE LTM-CandidateId is used to identify an LTM candidate configuration.

LTM-CandidateId information element

-- ASN1START

-- TAG-LTM-CANDIDATEID-START

LTM-CandidateId-r18 ::=                             INTEGER (1..maxNrofLTM-Configs-r18)

-- TAG-LTM-CANDIDATEID-STOP

-- ASN1STOP

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need N

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need N

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidateId This field indicates an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the configuration contained in ltm-CandidateConfig.

ltm-ConfigComplete This field indicates whether the LTM candidate configuration within ltm-CandidateConfig is a complete configuration.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

#### –	LTM-Config

The IE LTM-Config is used to provide LTM candidate configurations.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Need M

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

LTM-Config field descriptions

attemptLTM-Switch If present, the UE shall execute an LTM cell switch if selected cell is a LTM candidate cell as described in clause 5.3.7.3.

ltm-ReferenceConfiguration This field includes an RRCReconfiguration message used to configure a reference configuration for LTM.

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |

#### –	LTM-CSI-ReportConfig

The IE LTM-CSI-ReportConfig is used to configure report on the cell in which the LTM-CSI-ReportConfig is included.

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha                                            P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP

LTM-CSI-ReportConfig field descriptions

ltm-ReportContent This field defines the content of the LTM L1 measurement report.

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4).

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH and aperiodic reporting.

LTM-ReportContent field descriptions

nrOfReportedCells This field defines how many cells are reported within a single L1 measurement report instance.

nrOfReportedRS-PerCell This field defines how many RSs per cell are reported within a single L1 measurement report instance.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an LTM candidate cell.

#### –	LTM-CSI-ReportConfigId

The IE LTM-CSI-ReportConfigId is used to identify an LTM-CSI-ReportConfig.

LTM-CSI-ReportConfigId information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIGID-START

LTM-CSI-ReportConfigId-r18 ::=            INTEGER (0..maxNrofLTM-CSI-ReportConfigurations-1-r18)

-- TAG-LTM-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

LTM-CSI-SSB-ResourceSet field descriptions

ltm-CandidateIdList This field Indicates the LTM candidate cell IDs related to the SSBs in the ltm-CSI-SSB-ResourceList. The list has the same number of entries as ltm-CSI-SSB-ResourceList. The first entry in this list shall be associated to the first entry in ltm-CSI-SSB-ResourceList, the second entry of this list shall be associated to the second entry in ltm-CSI-SSB-ResourceList, and so on.

ltm-CSI-SSB-ResourceList This field is used to indicate on SS/PBCH block resources from one or more LTM candidate cells.

#### –	LTM-CSI-ResourceConfigId

The IE LTM-CSI-ResourceConfigId is used to identify an LTM-CSI-ResourceConfig.

LTM-CSI-ResourceConfigId information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIGID-START

LTM-CSI-ResourceConfigId-r18 ::=            INTEGER (0..maxNrofLTM-CSI-ResourceConfigurations-1-r18)

-- TAG-LTM-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	LTM-TCI-Info

The IE LTM-TCI-Info is used to configure TCI related information for an LTM candidate configuration.

LTM-TCI-Info information element

-- ASN1START

-- TAG-LTM-TCI-INFO-START

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                          OPTIONAL,    -- Need R

...

}

-- TAG-LTM-TCI-INFO-STOP

-- ASN1STOP

LTM-TCI-Info field descriptions

unifiedTCI-StateType Indicates the unified TCI states type the UE is configured for this LTM candidate configuration. The value separate means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StatesToAddModList for UL TCI states. The value joint means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation.

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                            BOOLEAN                                                         OPTIONAL,   -- Need M

dataInactivityTimer                 SetupRelease { DataInactivityTimer }                            OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup-r16 }                   OPTIONAL    -- Need M

]],

[[

enhancedSkipUplinkTxDynamic-r16     ENUMERATED {true}                                               OPTIONAL,   -- Need R

enhancedSkipUplinkTxConfigured-r16  ENUMERATED {true}                                               OPTIONAL    -- Need R

]],

[[

intraCG-Prioritization-r17          ENUMERATED {enabled}                        OPTIONAL,    -- Cond LCH-PrioWithReTxTimer

drx-ConfigSL-r17                    SetupRelease { DRX-ConfigSL-r17 }           OPTIONAL,    -- Need M

drx-ConfigExt-v1700                 SetupRelease { DRX-ConfigExt-v1700 }        OPTIONAL,    -- Need M

schedulingRequestID-BFR-r17         SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestID-BFR2-r17        SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestConfig-v1700       SchedulingRequestConfig-v1700               OPTIONAL,    -- Need M

tar-Config-r17                      SetupRelease { TAR-Config-r17  }                                OPTIONAL,    -- Need M

g-RNTI-ConfigToAddModList-r17       SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17       OPTIONAL,    -- Need N

g-RNTI-ConfigToReleaseList-r17      SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17     OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToAddModList-r17    SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17    OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToReleaseList-r17   SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17  OPTIONAL,    -- Need N

allowCSI-SRS-Tx-MulticastDRX-Active-r17   BOOLEAN                                                           OPTIONAL     -- Need M

]],

[[

schedulingRequestID-PosMG-Request-r17 SchedulingRequestId                                                   OPTIONAL,    -- Need R

drx-LastTransmissionUL-r17          ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

posMG-Request-r17                   ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

drx-ConfigExt2-v1800                SetupRelease { DRX-ConfigExt2-v1800 }                                   OPTIONAL,    -- Need M

additionalBS-TableAllowed-r18       BIT STRING (SIZE (maxNrofLCGs-r18))                                     OPTIONAL,    -- Need R

dsr-ConfigToAddModList-r18          SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-DSR-Config-r18              OPTIONAL,    -- Need N

dsr-ConfigToReleaseList-r18         SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-Id-r18                      OPTIONAL,    -- Need N

tar-Config-r18                      SetupRelease { TAR-Config-r18  }                                        OPTIONAL     -- Need M

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

MBS-RNTI-SpecificConfig-r17 ::=        SEQUENCE {

mbs-RNTI-SpecificConfigId-r17          MBS-RNTI-SpecificConfigId-r17,

groupCommon-RNTI-r17                   CHOICE {

g-RNTI                                 RNTI-Value,

g-CS-RNTI                              RNTI-Value

},

drx-ConfigPTM-r17                      SetupRelease { DRX-ConfigPTM-r17 }                          OPTIONAL,   -- Need M

harq-FeedbackEnablerMulticast-r17      ENUMERATED {dci-enabler, enabled}                           OPTIONAL,   -- Need S

harq-FeedbackOptionMulticast-r17       ENUMERATED {ack-nack, nack-only}                            OPTIONAL,   -- Cond HARQFeedback

pdsch-AggregationFactor-r17            ENUMERATED {n2, n4, n8}                                     OPTIONAL    -- Cond G-RNTI

}

MBS-RNTI-SpecificConfigId-r17 ::= INTEGER (0..maxG-RNTI-1-r17)

LCG-DSR-Config-r18 ::= SEQUENCE {

lcg-Id-r18                      LCG-Id-r18,

remainingTimeThreshold-r18      INTEGER (1..64),

...

}

LCG-Id-r18 ::= INTEGER (0..maxLCG-ID)

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

additionalBS-TableAllowed Indicates whether a UE is allowed to utilize the refined buffer size levels, as specified in TS 38.321 [3], for a certain Logical Channel Group. The leftmost bit corresponds to LCG ID=0, second leftmost bit to LCG ID=1 and so on. The UE is allowed to utilize the refined buffer size levels for a Logical Channel Group only when the corresponding bit is set to 1.

allowCSI-SRS-Tx-MulticastDRX-Active Used to control the CSI/SRS transmission during MBS multicast DRX ActiveTime, see TS 38.321 [3].

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config, drx-ConfigExt, drx-ConfigExt2 Used to configure DRX as specified in TS 38.321 [3]. Network only configures drx-ConfigExt or drx-ConfigExt2 when drx-Config is configured.

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

drx-ConfigSL Used to configure additional DRX parameters for the UE performing sidelink operation with resource allocation mode 1, as specified in TS 38.321 [3]. Network only configures this field if sl-ScheduledConfig is configured and drx-Config is configured.

drx-LastTransmissionUL If this field is present, the start of the drx-HARQ-RTT-TimerUL is after the last transmission within a bundle, see TS 38.321 [3].

dsr-ConfigToAddModList List of LCG-specific DSR configurations to add or modify.

dsr-ConfigToReleaseList List of LCG-specific DSR configurations to release.

g-RNTI-ConfigToAddModList List of G-RNTI configurations to add or modify. Up to 8 G-RNTIs can be configured in total in this release based on the UE capability.

g-RNTI-ConfigToReleaseList List of G-RNTI configurations to release.

g-CS-RNTI-ConfigToAddModList List of G-CS-RNTI configurations to add or modify. Up to 8 G-CS-RNTIs can be configured in total in this release based on the UE capability.

g-CS-RNTI-ConfigToReleaseList List of G-CS-RNTI configurations to release.

intraCG-Prioritization Used to enable HARQ process ID selection based on LCH-priority for one CG as specified in TS 38.321 [3].

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3]. The network does not configure lch-BasedPrioritization with enhancedSkipUplinkTxDynamic simultaneously nor lch-BasedPrioritization with enhancedSkipUplinkTxConfigured simultaneously.

posMG-Request Indicates whether UE is configured to send UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-BFR Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet1 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet2 of the same serving cell.

schedulingRequestID-BFR2 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet2 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet1 of the same serving cell.

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

schedulingRequestID-PosMG-Request Indicates the scheduling request configuration applicable for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

skipUplinkTxDynamic, enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured If set to true, the UE skips UL transmissions as described in TS 38.321 [3]. If the UE is configured with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true, REPETITION_NUMBER (as specified in TS 38.321 [3], clause 5.4.2.1) of the corresponding PUSCH transmission of the uplink grant shall be equal to 1. The network does not configure enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true together with numberOfSlotsTBoMS-r17.

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre-emptive BSR, see TS 38.321 [3].

MBS-RNTI-SpecificConfig field descriptions

drx-ConfigPTM Used to configure DRX for PTM transmission as specified in TS 38.321 [3].

g-CS-RNTI Used to scramble the SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH for one or more MBS multicast services.

g-RNTI Used to scramble the scheduling and transmission of PTM for one or more MBS multicast services.

groupCommon-RNTI Used to configure g-RNTI or g-CS-RNTI.

harq-FeedbackEnablerMulticast Indicates whether the UE shall provide HARQ feedback for MBS multicast. Value dci-enabler means that whether the UE shall provide HARQ feedback for MBS multicast is indicated by DCI as specified in TS 38.213 [13]. Value enabled means the UE shall always provide HARQ feedback for MBS multicast. When the field is absent, the UE behavior is specified in TS 38.213 [13].

harq-FeedbackOptionMulticast Indicates the feedback mode for MBS multicast dynamically scheduled PDSCH or SPS PDSCH.

mbs-RNTI-SpecificConfigId An identifier of the RNTI specific configuration for MBS multicast.

pdsch-AggregationFactor Number of repetitions for dynamically scheduled MBS multicast data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent and groupCommon-RNTI is set to g-RNTI, the UE applies the value 1.

LCG-DSR-Config field descriptions

lcg-Id Identifier of the Logical Channel Group which the DSR configuration refers to.

remainingTimeThreshold Remaining time threshold used for triggering DSR for the Logical Channel Group, as specified in TS 38.321 [3]. Value in number of miliseconds.

| Conditional Presence | Explanation |
| --- | --- |
| G-RNTI | This field is optionally present, Need S, if groupCommon-RNTI is set to g-RNTI. The field is absent when groupCommon-RNTI is set to g-CS-RNTI. |
| HARQFeedback | The field is mandatory present when harq-FeedbackEnablerMulticast is present. It is absent otherwise. |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |
| LCH-PrioWithReTxTimer | This field is optionally present, Need R, if lch-BasedPrioritization-r16 is configured in this MAC entity and cg-RetransmissionTimer-r16 is configured for any configured grant configuration associated with this MAC entity. It is absent otherwise, Need R. |

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]],

[[

effectiveMeasWindowConfig-r18       SetupRelease {MeasWindowConfig-r18}                                 OPTIONAL    -- Need M

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

effectiveMeasWindowConfig Used to setup and release effective measurement window in NR for E-UTRA measurements.

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps. In NR-DC, the field can only be configured in the measConfig associated with MCG, and when configured, it applies to all the inter-frequency measurements configured by MN and SN.

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/release of measurement gaps.

MeasGapConfig information element

-- ASN1START

-- TAG-MEASGAPCONFIG-START

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }                                              OPTIONAL    -- Need M

]],

[[

gapToAddModList-r17           SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17                    OPTIONAL,   -- Need N

gapToReleaseList-r17          SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17                    OPTIONAL,   -- Need N

posMeasGapPreConfigToAddModList-r17      PosMeasGapPreConfigToAddModList-r17                                OPTIONAL,   -- Need N

posMeasGapPreConfigToReleaseList-r17     PosMeasGapPreConfigToReleaseList-r17                               OPTIONAL    -- Need N

]]

}

GapConfig ::=                       SEQUENCE {

gapOffset                           INTEGER (0..159),

mgl                                 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                                ENUMERATED {ms20, ms40, ms80, ms160},

mgta                                ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator                ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16           ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

mgl-r16                             ENUMERATED {ms10, ms20}                                             OPTIONAL    -- Cond PRS

]]

}

GapConfig-r17 ::=                   SEQUENCE {

measGapId-r17                       MeasGapId-r17,

gapType-r17                         ENUMERATED {perUE, perFR1, perFR2},

gapOffset-r17                       INTEGER (0..159),

mgl-r17                             ENUMERATED {ms1, ms1dot5, ms2, ms3, ms3dot5, ms4, ms5, ms5dot5, ms6, ms10, ms20},

mgrp-r17                            ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17                            ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot75},

refServCellIndicator-r17            ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL,   -- Cond NEDCorNRDC

refFR2-ServCellAsyncCA-r17          ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

preConfigInd-r17                    ENUMERATED {true}                                                   OPTIONAL,   -- Need R

ncsgInd-r17                         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

gapAssociationPRS-r17               ENUMERATED {true}                                                   OPTIONAL,   -- Need R

gapSharing-r17                      MeasGapSharingScheme                                                OPTIONAL,   -- Need R

gapPriority-r17                     GapPriority-r17                                                     OPTIONAL,   -- Need R

...

}

PosMeasGapPreConfigToAddModList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF PosGapConfig-r17

PosMeasGapPreConfigToReleaseList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF MeasPosPreConfigGapId-r17

PosGapConfig-r17 ::=                SEQUENCE {

measPosPreConfigGapId-r17           MeasPosPreConfigGapId-r17,

gapOffset-r17                       INTEGER (0..159),

mgl-r17                             ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

mgrp-r17                            ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17                            ENUMERATED {ms0, ms0dot25, ms0dot5},

gapType-r17                         ENUMERATED {perUE, perFR1, perFR2},

...

}

MeasPosPreConfigGapId-r17 ::= INTEGER (1..maxNrofPreConfigPosGapId-r17)

-- TAG-MEASGAPCONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapAssociationPRS Indicates that PRS measurement is associated with this measurement gap. The network only includes this field for one per-UE gap or for one per-FR gap. If concurrent gap (i.e. one of the gap combination as defined in Table 9.1.8-1 in TS 38.133 [14]) is configured and no gap is configured with this field, the PRS measurement is associated with the gap configured via GapConfig (without suffix), if available. If both per-UE gap and per-FR gap are configured via GapConfig and/or GapConfig-r17, the PRS measurement is always associated with the per-UE gap.

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 measurement gap). In NE-DC, gapFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap). In NR-DC, gapFR1 can only be set up in the measConfig associated with MCG. gapFR1 can not be configured together with gapUE. The applicability of the FR1 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapFR2 Indicates measurement gap configuration applies to FR2 only. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap). In NR-DC, gapFR2 can only be set up in the measConfig associated with MCG. gapFR2 cannot be configured together with gapUE. The applicability of the FR2 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range is from 0 to mgrp-1. If ncsgInd-r17 is present, this offset value refers to the starting point of VIL1 (the visible interruption length before the ML).

gapPriority Indicates the priority of this measurement gap (see TS 38.133 [14], clause 9.1.8.3). Value 1 indicates highest priority, value 2 indicates second level priority, and so on.

gapSharing Indicates the measurement gap sharing scheme that applies to this GapConfig. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapToAddModList A list of of measurement gap configuration to be added or modified. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8), the maximum number of configured measurement gap is limited by the gap combinations defined in Table 9.1.8-1 in TS 38.133 [14]. In this version of the specification, the network configures this field only in NR standalone. This field is used only for a UE that supports pre-configured measurement gap, concurrent measurement gap, or NCSG. In this version of the specification, the network does not configure concurrent measurement gap together with MUSIM gap or preconfigured measurement gap for positioning.

gapToReleaseList A list of measurement gap configuration to be released.

gapType Indicates the type of this measurement gap. Value perUE indicates that it is a per UE measurement gap, value perFR1 indicates that it is an FR1 measurement gap, and value perFR2 indicates that it is an FR2 measurement gap.

gapUE Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). In (NG)EN-DC, gapUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE measurement gap). In NE-DC, gapUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap). In NR-DC, gapUE can only be set up in the measConfig associated with MCG. If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. The applicability of the per UE measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

measGapId The ID of this measurement gap configuration.

mgl Value mgl is the measurement gap length in ms of the measurement gap. If ncsgInd-r17 is not present, the measurement gap length is according to in Table 9.1.2-1 in TS 38.133 [14]. If ncsgInd-r17 is present, this field indicates the measurement length (ML) in NCSG pattern and is configured according to Table 9.1.9.3-1 in TS 38.133 [14]. Value ms1dot5 corresponds to 1.5 ms, ms3 corresponds to 3 ms and so on. If mgl-r16 is present, UE shall ignore the mgl (without suffix). Value ms1, ms2, and ms5 can only be configured if ncsgInd is present.

mgrp If ncsgInd-r17 is not present, the mgrp field indicates the measurement gap repetition period in (ms) of the measurement gap according to Table 9.1.2-1 in TS 38.133 [14]. If ncsgInd-r17 is present, the mgrp field indicates the Visible Interruption Repetition Period (VIRP) of NCSG pattern and is configured according to Table 9.1.9.3-1 in TS 38.133 [14].

mgta Value mgta is the measurement gap timing advance in ms. The applicability of the measurement gap timing advance is according to clause 9.1.2 of TS 38.133 [14], or according to clause 9.1.9 of TS 38.133 [14] if ncsgInd is present. Value ms0 corresponds to 0 ms, ms0dot25 corresponds to 0.25 ms, ms0dot5 corresponds to 0.5 ms and ms0dot75 corresponds to 0.75 ms. For FR2, the network only configures 0 ms and 0.25 ms if ncsgInd is not present. If ncsgInd is present, the network only configures 0ms for per-UE NCSG and FR1 NCSG and only configures 0ms or 0.75ms for FR2 NCSG. Value ms0dot75 can only be configured if ncsgInd is present.

ncsgInd Indicates that the measurement gap is a NCSG as specified in 38.133 [14].

posMeasGapPreConfigToAddModList List of preconfigured measurement gap for positioning to add and/or modify. All the gaps configured are associated with the measurement of PRS for RSTD, UE-RxTx Time Difference, PRS-RSRP and PRS-RSRPP as defined in TS 38.215 [9]. In this version of the specification, the network does not configure preconfigured measurement gap for positioning together with concurrent measurement gap or MUSIM gap.

posMeasGapPreConfigToReleaseList List of preconfigured measurement gap for positioning to release.

preConfigInd Indicates whether the measurement gap is a pre-configured measurement gap.

refFR2ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

refServCellIndicator Indicates the serving cell whose SFN and subframe are used for gap calculation for this gap pattern. Value pCell corresponds to the PCell, pSCell corresponds to the PSCell, and mcg-FR2 corresponds to a serving cell on FR2 frequency in MCG.

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring and reconfiguring FR2 gap pattern to UE in: - (NG)EN-DC or NR SA with asynchronous CA involving FR2 carrier(s); - NE-DC or NR-DC with asynchronous CA involving FR2 carrier(s), if the field refServCellIndicator is set to mcg-FR2. Otherwise, it is absent, Need R. |
| NEDCorNRDC | This field is mandatory present when configuring and reconfiguring gap pattern to UE in NE-DC or NR-DC. Otherwise, it is absent, Need R. |
| PRS | This field is optionally present, Need R, when configuring gap pattern to UE for measurements of DL-PRS configured via LPP (TS 37.355 [49]). Otherwise, it is absent. |

#### –	MeasGapId

The IE MeasGapId used to identify a per UE or per FR measurement gap configuration.

MeasGapId information element

-- ASN1START

-- TAG-MEASGAPID-START

MeasGapId-r17 ::=                       INTEGER (1..maxNrofGapId-r17)

-- TAG-MEASGAPID-STOP

-- ASN1STOP

#### –	MeasGapSharingConfig

The IE MeasGapSharingConfig specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

MeasGapSharingConfig information element

-- ASN1START

-- TAG-MEASGAPSHARINGCONFIG-START

MeasGapSharingConfig ::=        SEQUENCE {

gapSharingFR2                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   -- Need M

...,

[[

gapSharingFR1                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   --Need M

gapSharingUE                    SetupRelease { MeasGapSharingScheme }       OPTIONAL    --Need M

]]

}

MeasGapSharingScheme::=         ENUMERATED {scheme00, scheme01, scheme10, scheme11}

-- TAG-MEASGAPSHARINGCONFIG-STOP

-- ASN1STOP

MeasGapSharingConfig field descriptions

gapSharingFR1 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR1. In (NG)EN-DC, gapSharingFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 gap sharing). In NE-DC, gapSharingFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap sharing). In NR-DC, gapSharingFR1 can only be set up in the measConfig associated with MCG. gapSharingFR1 can not be configured together with gapSharingUE. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingFR2 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR2. In (NG)EN-DC or NE-DC, gapSharingFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap sharing). In NR-DC, gapSharingFR2 can only be set up by MCG in the measConfig associated with MCG. gapSharingFR2 cannot be configured together with gapSharingUE. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingUE Indicates the measurement gap sharing scheme that applies to the gap set via gapUE. In (NG)EN-DC, gapSharingUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE gap sharing). In NE-DC, gapSharingUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap sharing). In NR-DC, gapSharingUE can only be set up in the measConfig associated with MCG. If gapSharingUE is configured, then neither gapSharingFR1 nor gapSharingFR2 can be configured. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

#### –	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

-- TAG-MEASID-START

MeasId ::=                          INTEGER (1..maxNrofMeasId)

-- TAG-MEASID-STOP

-- ASN1STOP

#### –	MeasIdleConfig

The IE MeasIdleConfig is used to convey information to UE about measurements requested to be done while in RRC_IDLE or RRC_INACTIVE.

MeasIdleConfig information element

-- ASN1START

-- TAG-MEASIDLECONFIG-START

MeasIdleConfigSIB-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need S

measIdleCarrierListEUTRA-r16    SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16       OPTIONAL,     -- Need S

...,

[[

measReselectionCarrierListNR-r18     SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasReselectionCarrierNR-r18    OPTIONAL,    -- Need S

measIdleValidityDuration-r18         MeasurementValidityDuration-r18                                         OPTIONAL,    -- Need S

measReselectionValidityDuration-r18  MeasurementValidityDuration-r18                                         OPTIONAL     -- Need S

]]

}

MeasIdleConfigDedicated-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need N

measIdleCarrierListEUTRA-r16    SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16       OPTIONAL,     -- Need N

measIdleDuration-r16            ENUMERATED{sec10, sec30, sec60, sec120, sec180, sec240, sec300, spare},

validityAreaList-r16            ValidityAreaList-r16                                                   OPTIONAL,     -- Need N

...,

[[

measReselectionCarrierListNR-r18     SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasReselectionCarrierNR-r18    OPTIONAL,    -- Need S

measIdleValidityDuration-r18         MeasurementValidityDuration-r18                                         OPTIONAL,    -- Need S

measReselectionValidityDuration-r18  MeasurementValidityDuration-r18                                         OPTIONAL     -- Need S

]]

}

ValidityAreaList-r16 ::= SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF ValidityArea-r16

ValidityArea-r16 ::=             SEQUENCE {

carrierFreq-r16                  ARFCN-ValueNR,

validityCellList-r16             ValidityCellList                                                     OPTIONAL   -- Need N

}

ValidityCellList ::= SEQUENCE (SIZE (1.. maxCellMeasIdle-r16)) OF PCI-Range

MeasIdleCarrierNR-r16 ::=        SEQUENCE {

carrierFreq-r16                  ARFCN-ValueNR,

ssbSubcarrierSpacing-r16         SubcarrierSpacing,

frequencyBandList                MultiFrequencyBandListNR                                             OPTIONAL,  -- Need R

measCellListNR-r16               CellListNR-r16                                                       OPTIONAL,  -- Need R

reportQuantities-r16             ENUMERATED {rsrp, rsrq, both},

qualityThreshold-r16             SEQUENCE {

idleRSRP-Threshold-NR-r16        RSRP-Range                                                           OPTIONAL,  -- Need R

idleRSRQ-Threshold-NR-r16        RSRQ-Range                                                           OPTIONAL   -- Need R

}                                                                                                     OPTIONAL,  -- Need R

ssb-MeasConfig-r16               SEQUENCE {

nrofSS-BlocksToAverage-r16          INTEGER (2..maxNrofSS-BlocksToAverage)                            OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation-r16 ThresholdNR                                                       OPTIONAL,   -- Need S

smtc-r16                            SSB-MTC                                                           OPTIONAL,   -- Need S

ssb-ToMeasure-r16                   SSB-ToMeasure                                                     OPTIONAL,   -- Need S

deriveSSB-IndexFromCell-r16         BOOLEAN,

ss-RSSI-Measurement-r16             SS-RSSI-Measurement                                               OPTIONAL    -- Need S

}                                                                                                     OPTIONAL,  -- Need S

beamMeasConfigIdle-r16           BeamMeasConfigIdle-NR-r16                                            OPTIONAL,  -- Need R

...

}

MeasIdleCarrierEUTRA-r16 ::=     SEQUENCE {

carrierFreqEUTRA-r16             ARFCN-ValueEUTRA,

allowedMeasBandwidth-r16         EUTRA-AllowedMeasBandwidth,

measCellListEUTRA-r16            CellListEUTRA-r16                                                    OPTIONAL,  -- Need R

reportQuantitiesEUTRA-r16        ENUMERATED {rsrp, rsrq, both},

qualityThresholdEUTRA-r16        SEQUENCE {

idleRSRP-Threshold-EUTRA-r16     RSRP-RangeEUTRA                                                      OPTIONAL,  -- Need R

idleRSRQ-Threshold-EUTRA-r16     RSRQ-RangeEUTRA-r16                                                  OPTIONAL   -- Need R

}                                                                                                     OPTIONAL,  -- Need S

...

}

MeasReselectionCarrierNR-r18 ::= SEQUENCE {

carrierFreq-r18                  ARFCN-ValueNR,

...

}

MeasurementValidityDuration-r18 ::=   ENUMERATED {  s5, s10, s20, s50, s100, spare3, spare2, spare1}

CellListNR-r16  ::=       SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF PCI-Range

CellListEUTRA-r16  ::=    SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF EUTRA-PhysCellIdRange

BeamMeasConfigIdle-NR-r16  ::=   SEQUENCE {

reportQuantityRS-Indexes-r16     ENUMERATED {rsrp, rsrq, both},

maxNrofRS-IndexesToReport-r16    INTEGER (1.. maxNrofIndexesToReport),

includeBeamMeasurements-r16      BOOLEAN

}

RSRQ-RangeEUTRA-r16 ::=   INTEGER (-30..46)

-- TAG-MEASIDLECONFIG-STOP

-- ASN1STOP

MeasIdleConfig field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index.

beamMeasConfigIdle Indicates the beam level measurement configuration.

carrierFreq Indicates the NR carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

carrierFreqEUTRA Indicates the E-UTRA carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR idle/inactive measurement parameters apply. The UE shall select the first listed band which it supports in the frequencyBandList field to represent the NR neighbour carrier frequency.

includeBeamMeasurements Indicates whether or not the UE shall include beam measurements in the NR idle/inactive measurement results.

maxNrofRS-IndexesToReport Max number of beam indices to include in the idle/inactive measurement result.

measCellListEUTRA Indicates the list of E-UTRA cells which the UE is requested to measure and report for idle/inactive measurements.

measCellListNR Indicates the list of NR cells which the UE is requested to measure and report for idle/inactive measurements.

measIdleCarrierListEUTRA Indicates the E-UTRA carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleCarrierListNR Indicates the NR carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleDuration Indicates the duration for performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. Value sec10 correspond to 10 seconds, value sec30 to 30 seconds and so on.

measIdleValidityDuration, measReselectionValidityDuration Indicates time values for UE to determine validity of reported idle/inactive and reselection measurements as defined in TS 38.133[14]. Value s5 correspond to 5 seconds, value s10 correspond to 10 seconds and so on.

measReselectionCarrierListNR Indicates the NR carriers for reselection measurement reporting.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation.

qualityThreshold Indicates the quality thresholds for reporting the measured cells for idle/inactive NR measurements.

qualityThresholdEUTRA Indicates the quality thresholds for reporting the measured cells for idle/inactive E-UTRA measurements.

reportQuantities Indicates which measurement quantities UE is requested to report in the idle/inactive measurement report.

reportQuantitiesEUTRA Indicates which E-UTRA measurement quantities the UE is requested to report in the idle/inactive measurement report.

reportQuantityRS-Indexes Indicates which measurement information per beam index the UE shall include in the NR idle/inactive measurement results.

smtc Indicates the measurement timing configuration for inter-frequency measurement. If this field is absent in VarMeasIdleConfig, the UE assumes that SSB periodicity is 5 ms in this frequency.

ssbSubcarrierSpacing Indicates subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent in VarMeasIdleConfig, the UE measures on all SS-blocks.

ss-RSSI-Measurement Indicates the SSB-based RSSI measurement configuration. If the field is absent in VarMeasIdleConfig, the UE behaviour is defined in TS 38.215 [89], clause 5.1.3.

validityAreaList Indicates the list of frequencies and optionally, for each frequency, a list of cells within which the UE is required to perform measurements while in RRC_IDLE and RRC_INACTIVE.

#### –	MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId.

MeasIdToAddModList information element

-- ASN1START

-- TAG-MEASIDTOADDMODLIST-START

MeasIdToAddModList ::=              SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod

MeasIdToAddMod ::=                  SEQUENCE {

measId                              MeasId,

measObjectId                        MeasObjectId,

reportConfigId                      ReportConfigId

}

-- TAG-MEASIDTOADDMODLIST-STOP

-- ASN1STOP

#### –	MeasObjectCLI

The IE MeasObjectCLI specifies information applicable for SRS-RSRP measurements and/or CLI-RSSI measurements.

MeasObjectCLI information element

-- ASN1START

-- TAG-MEASOBJECTCLI-START

MeasObjectCLI-r16 ::=                  SEQUENCE {

cli-ResourceConfig-r16               CLI-ResourceConfig-r16,

...

}

CLI-ResourceConfig-r16 ::=          SEQUENCE {

srs-ResourceConfig-r16              SetupRelease { SRS-ResourceListConfigCLI-r16 }                 OPTIONAL,   -- Need M

rssi-ResourceConfig-r16             SetupRelease { RSSI-ResourceListConfigCLI-r16 }                OPTIONAL    -- Need M

}

SRS-ResourceListConfigCLI-r16 ::=   SEQUENCE (SIZE (1.. maxNrofCLI-SRS-Resources-r16)) OF SRS-ResourceConfigCLI-r16

RSSI-ResourceListConfigCLI-r16 ::=  SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-Resources-r16)) OF RSSI-ResourceConfigCLI-r16

SRS-ResourceConfigCLI-r16 ::=       SEQUENCE {

srs-Resource-r16                    SRS-Resource,

srs-SCS-r16                         SubcarrierSpacing,

refServCellIndex-r16                ServCellIndex                                                  OPTIONAL,   -- Need S

refBWP-r16                          BWP-Id,

...

}

RSSI-ResourceConfigCLI-r16 ::=      SEQUENCE {

rssi-ResourceId-r16                 RSSI-ResourceId-r16,

rssi-SCS-r16                        SubcarrierSpacing,

startPRB-r16                        INTEGER (0..2169),

nrofPRBs-r16                        INTEGER (4..maxNrofPhysicalResourceBlocksPlus1),

startPosition-r16                   INTEGER (0..13),

nrofSymbols-r16                     INTEGER (1..14),

rssi-PeriodicityAndOffset-r16       RSSI-PeriodicityAndOffset-r16,

refServCellIndex-r16                ServCellIndex                                                  OPTIONAL,   -- Need S

...

}

RSSI-ResourceId-r16 ::=             INTEGER (0.. maxNrofCLI-RSSI-Resources-1-r16)

RSSI-PeriodicityAndOffset-r16 ::=   CHOICE {

sl10                                INTEGER(0..9),

sl20                                INTEGER(0..19),

sl40                                INTEGER(0..39),

sl80                                INTEGER(0..79),

sl160                               INTEGER(0..159),

sl320                               INTEGER(0..319),

s1640                               INTEGER(0..639),

...

}

-- TAG-MEASOBJECTCLI-STOP

-- ASN1STOP

CLI-ResourceConfig field descriptions

srs-ResourceConfig SRS resources to be used for CLI measurements.

rssi-ResourceConfig CLI-RSSI resources to be used for CLI measurements.

MeasObjectCLI field descriptions

cli-ResourceConfig SRS and/or CLI-RSSI resource configuration for CLI measurement.

SRS-ResourceConfigCLI field descriptions

refBWP DL BWP id that is used to derive the reference point of the SRS resource (see TS 38.211[16], clause 6.4.1.4.3)

refServCellIndex The index of the reference serving cell that the refBWP belongs to. If this field is absent, the reference serving cell is PCell.

srs-SCS Subcarrier spacing for SRS. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

RSSI-ResourceConfigCLI field descriptions

nrofPRBs Allowed size of the measurement BW. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 4 and the width of the active DL BWP. If the configured value is larger than the width of the active DL BWP, the UE shall assume that the actual CLI-RSSI resource bandwidth is within the active DL BWP.

nrofSymbols Within a slot that is configured for CLI-RSSI measurement (see slotConfiguration), the UE measures the RSSI from startPosition to startPosition + nrofSymbols - 1. The configured CLI-RSSI resource does not exceed the slot boundary of the reference SCS. If the SCS of configured DL BWP(s) is larger than the reference SCS, network configures startPosition and nrofSymbols such that the configured CLI-RSSI resource not to exceed the slot boundary corresponding to the configured BWP SCS. If the reference SCS is larger than SCS of configured DL BWP(s), network ensures startPosition and nrofSymbols are integer multiple of reference SCS divided by configured BWP SCS.

refServCellIndex The index of the reference serving cell. Frequency reference point of the RSSI resource is subcarrier 0 of CRB0 of the reference serving cell. If this field is absent, the reference serving cell is PCell.

rssi-PeriodicityAndOffset Periodicity and slot offset for this CLI-RSSI resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots.

rssi-SCS Reference subcarrier spacing for CLI-RSSI measurement. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz UE performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

startPosition OFDM symbol location of the CLI-RSSI resource within a slot.

startPRB Starting PRB index of the measurement bandwidth. For the case where the reference subcarrier spacing is smaller than subcarrier spacing of active DL BWP(s), network configures startPRB and nrofPRBs are as a multiple of active BW SCS divided by reference SCS.

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for E-UTRA cells.

MeasObjectEUTRA information element

-- ASN1START

-- TAG-MEASOBJECTEUTRA-START

MeasObjectEUTRA::=                          SEQUENCE {

carrierFreq                                 ARFCN-ValueEUTRA,

allowedMeasBandwidth                        EUTRA-AllowedMeasBandwidth,

cellsToRemoveListEUTRAN                     EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

cellsToAddModListEUTRAN                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-Cell         OPTIONAL,    -- Need N

excludedCellsToRemoveListEUTRAN             EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

excludedCellsToAddModListEUTRAN             SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-ExcludedCell OPTIONAL,    -- Need N

eutra-PresenceAntennaPort1                  EUTRA-PresenceAntennaPort1,

eutra-Q-OffsetRange                         EUTRA-Q-OffsetRange                                         OPTIONAL,    -- Need R

widebandRSRQ-Meas                           BOOLEAN,

...,

[[

associatedMeasGap-r17                       MeasGapId-r17                                               OPTIONAL     -- Need R

]],

[[

measSequence-r18                            MeasSequence-r18                                            OPTIONAL     -- Need R

]]

}

EUTRA-CellIndexList ::=                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-CellIndex

EUTRA-CellIndex ::=                         INTEGER (1..maxCellMeasEUTRA)

EUTRA-Cell ::=                              SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellId                                  EUTRA-PhysCellId,

cellIndividualOffset                        EUTRA-Q-OffsetRange

}

EUTRA-ExcludedCell ::=                      SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellIdRange                             EUTRA-PhysCellIdRange

}

-- TAG-MEASOBJECTEUTRA-STOP

-- ASN1STOP

EUTRAN-ExcludedCell field descriptions

cellIndexEUTRA Entry index in the cell list.

physicalCellIdRange Physical cell identity or a range of physical cell identities.

EUTRAN-Cell field descriptions

physicalCellId Physical cell identity of a cell in the cell list.

cellIndividualOffset Cell individual offset applicable to a specific cell. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

MeasObjectEUTRA field descriptions

allowedMeasBandwidth The maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 [33].

associatedMeasGap Indicates the associated measurement gap for measuring this EUTRA frequency. If this field is absent, the associated meaurment gap is the gap configured via gapFR1 or gapUE.

carrierFreq Identifies E-UTRA carrier frequency for which this configuration is valid. Network does not configure more than one MeasObjectEUTRA for the same physical frequency, regardless of the E-ARFCN used to indicate this.

cellsToAddModListEUTRAN List of cells to add/ modify in the cell list.

cellsToRemoveListEUTRAN List of cells to remove from the cell list.

eutra-PresenceAntennaPort1 When set to true, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

eutra-Q-OffsetRange Used to indicate a cell, or frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

excludedCellsToAddModListEUTRAN List of cells to add/ modify in the exclude-list of cells.

excludedCellsToRemoveListEUTRAN List of cells to remove from the exclude-list of cells.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not configured, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

widebandRSRQ-Meas If set to true, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [40]. The network may set the field to true if the measurement bandwidth indicated by allowedMeasBandwidth is 50 resource blocks or larger; otherwise the network sets this field to false.

#### –	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

-- TAG-MEASOBJECTID-START

MeasObjectId ::=                    INTEGER (1..maxNrofObjectId)

-- TAG-MEASOBJECTID-STOP

-- ASN1STOP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

excludedCellsToRemoveList           PCI-RangeIndexList                                              OPTIONAL,   -- Need N

excludedCellsToAddModList           SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

allowedCellsToRemoveList            PCI-RangeIndexList                                              OPTIONAL,   -- Need N

allowedCellsToAddModList            SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                       SSB-MTC3List-r16                                                OPTIONAL,   -- Need R

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]],

[[

associatedMeasGapSSB-r17            MeasGapId-r17                                                   OPTIONAL,   -- Need R

associatedMeasGapCSIRS-r17          MeasGapId-r17                                                   OPTIONAL,   -- Need R

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,   -- Need R

measCyclePSCell-r17                 ENUMERATED {ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, spare1}

OPTIONAL,   -- Cond SCG

cellsToAddModListExt-v1710          CellsToAddModListExt-v1710                                      OPTIONAL    -- Need N

]],

[[

associatedMeasGapSSB2-v1720         MeasGapId-r17                                               OPTIONAL, -- Cond AssociatedGapSSB

associatedMeasGapCSIRS2-v1720       MeasGapId-r17                                               OPTIONAL  -- Cond AssociatedGapCSIRS

]],

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                    OPTIONAL    -- Cond NeighbourCell

]]

}

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

[[

ssb-PositionQCL-Common-r16              SSB-PositionQCL-Relation-r16                                OPTIONAL,   -- Cond SharedSpectrum

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL    -- Need N

]],

[[

deriveSSB-IndexFromCellInter-r17    ServCellIndex                                                   OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,   -- Cond SharedSpectrum2

ssb-PositionQCL-Cells-r17           SetupRelease {SSB-PositionQCL-CellList-r17}                     OPTIONAL    -- Need M

]],

[[

cca-CellsToAddModList-r17           PCI-List                                                        OPTIONAL,   -- Need N

cca-CellsToRemoveList-r17           PCI-List                                                        OPTIONAL    -- Need N

]],

[[

ssb-ToMeasureAltitudeBasedList-r18  SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 }             OPTIONAL    -- Need M

]]

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddModListExt-v1710 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1710

CellsToAddModListExt-v1800 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1800

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

CellsToAddModExt-v1710 ::=          SEQUENCE {

ntn-PolarizationDL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL,  -- Need R

ntn-PolarizationUL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL   -- Need R

}

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18                                       OPTIONAL   -- Need R

}

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...,

[[

rmtc-Bandwidth-r17                  ENUMERATED {mhz100, mhz400, mhz800, mhz1600, mhz2000}           OPTIONAL,   -- Need R

measDurationSymbols-v1700           ENUMERATED {sym140, sym560, sym1120}                            OPTIONAL,   -- Need R

ref-SCS-CP-v1700                    ENUMERATED {kHz120, kHz480, kHz960}                             OPTIONAL,   -- Need R

tci-StateInfo-r17               SEQUENCE {

tci-StateId-r17                  TCI-StateId,

ref-ServCellId-r17               ServCellIndex                                                  OPTIONAL   -- Need R

} OPTIONAL -- Need R

]],

[[

ref-BWPId-r17                   BWP-Id                                                              OPTIONAL   -- Need R

]]

}

SSB-PositionQCL-CellsToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-CellsToAddMod-r16

SSB-PositionQCL-CellsToAddMod-r16 ::= SEQUENCE {

physCellId-r16                        PhysCellId,

ssb-PositionQCL-r16                   SSB-PositionQCL-Relation-r16

}

SSB-PositionQCL-CellList-r17 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-Cell-r17

SSB-PositionQCL-Cell-r17         ::= SEQUENCE {

physCellId-r17                        PhysCellId,

ssb-PositionQCL-r17                   SSB-PositionQCL-Relation-r17

}

SSB-ToMeasureAltitudeBasedList-r18 ::= SEQUENCE (SIZE (1..maxNrofAltitudeRanges-r18)) OF SSB-ToMeasureAltitudeBased-r18

SSB-ToMeasureAltitudeBased-r18 ::=     SEQUENCE {

altitudeRange-r18                      SEQUENCE {

altitudeMin-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeMax-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeHyst-r18                       HysteresisAltitude-r18                                   OPTIONAL   -- Need R

},

ssb-ToMeasure-r18                      SSB-ToMeasure                                                OPTIONAL   -- Need S

}

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

physCellId Physical cell identity of a cell in the cell list.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per CSI-RS resource as described in 5.5.5.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

allowedCellsToAddModList List of cells to add/modify in the allow-list of cells. It applies only to SSB resources.

allowedCellsToRemoveList List of cells to remove from the allow-list of cells.

associatedMeasGapSSB Indicates the associated measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapSSB2 Indicates the associated additional measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object for NTN deployments. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapSSB.

associatedMeasGapCSIRS Indicates the associated measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapCSIRS2 Indicates the associated additional measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object for NTN deployments. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapCSIRS. In this release of the specification, this field is not configured for NTN deployments.

cellsToAddModList List of cells to add/modify in the cell list.

cellsToRemoveList List of cells to remove from the cell list.

excludedCellsToAddModList List of cells to add/modify in the exclude-list of cells. It applies only to SSB resources.

excludedCellsToRemoveList List of cells to remove from the exclude-list of cells.

freqBandIndicatorNR The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

measCyclePSCell The parameter is used only when the PSCell is configured on the frequency indicated by the measObjectNR and the SCG is deactivated, see TS 38.133 [14]. The field may also be configured when the PSCell is not configured on that frequency. The network always configures measCyclePSCell for the measObjectNR associated with the PSCell if bfd-and-RLM is set to true and the SCG is deactivated. Value ms160 corresponds to 160 ms, value ms256 corresponds to 256 ms and so on.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not provided, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

nrofCSInrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

ntn-NeighbourCellInfo Includes satellite assistance information of an NTN neighbour cell.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL If present, this parameter indicates polarization information for uplink transmission on service link. If not present and ntn-PolarizationDL is present, UE assumes the same polarization for UL and DL.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration for SS/PBCH block and CSI-RS.

refFreqCSI-RS Point A which is used for mapping of CSI-RS to physical resources according to TS 38.211 [16] clause 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see clause 5.5.2.10).

smtc2 Secondary measurement timing configuration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value strictly shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured).

smtc3list Measurement timing configuration list for SS corresponding to IAB-MT. This is used for the IAB-node's discovery of other IAB-nodes and the IAB-Donor-DUs.

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10.

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15], or TS 38.101-5 [75]).

ssb-PositionQCL-Common Indicates the QCL relationship between SS/PBCH blocks for all measured cells as specified in TS 38.213 [13], clause 4.1.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

t312 The value of timer T312. Value ms0 represents 0 ms, ms50 represents 50 ms and so on.

ReferenceSignalConfig field descriptions

csi-rs-ResourceConfigMobility CSI-RS resources to be used for CSI-RS based RRM measurements.

ssb-ConfigMobility SSB configuration for mobility (nominal SSBs, timing configuration).

RMTC-Config field descriptions

measDurationSymbols Number of consecutive symbols for which the Physical Layer reports samples of RSSI (see TS 38.215 [9], clause 5.1.21). Value sym1 corresponds to one symbol, sym14or12 corresponds to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, and so on. If measDurationSymbols-v1700 is signalled, the UE ignores measDurationSymbols-r16.

ref-BWPId Indicates the reference BWP for the TCI state indicated in tci-StateInfo. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

ref-SCS-CP Indicates a reference subcarrier spacing and cyclic prefix to be used for RSSI measurements (see TS 38.215 [9]). Value kHz15 corresponds to 15kHz, kHz30 corresponds to 30 kHz, value kHz60-NCP corresponds to 60 kHz using normal cyclic prefix (NCP), and kHz60-ECP corresponds to 60 kHz using extended cyclic prefix (ECP). If ref-SCS-CP-v1700 is signalled, the UE ignores ref-SCS-CP-r16.

ref-ServCellId Indicates the FR2-2 reference serving cell index for the TCI state. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

rmtc-Bandwidth Indicates the bandwidth for the RSSI measurement (see TS 38. 215 [9], clause 5.1.21).

rmtc-Frequency Indicates the center frequency of the measured bandwidth for a frequency which operates with shared spectrum channel access (see TS 38. 215 [9], clause 5.1.21).

rmtc-Periodicity Indicates the RSSI measurement timing configuration (RMTC) periodicity (see TS 38.215 [9], clause 5.1.21).

rmtc-SubframeOffset Indicates the RSSI measurement timing configuration (RMTC) subframe offset for this frequency (see TS 38.215 [9], clause 5.1.21). For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDurationSymbols which shall be selected to be between 0 and the configured rmtc-Periodicity with equal probability.

tci-StateId Indicates the TCI state to be used for RSSI measurements. This field is only applicable for shared spectrum channel access in FR2-2. Network does not configure this if the UE does not have any serving cells in FR2-2 and in such a case, it is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

SSB-ConfigMobility field descriptions

cca-CellsToAddModList, cca-CellsToRemoveList Lists of cells to be added or removed from the list of neighbor cells that apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2.

deriveSSB-IndexFromCell If this field is set to true, UE assumes SFN and frame boundary alignment across cells on the same frequency carrier as specified in TS 38.133 [14]. Hence, if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, this field indicates whether the UE can utilize the timing of this serving cell to derive the index of SS block transmitted by neighbour cell. Otherwise, this field indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency.

deriveSSB-IndexFromCellInter If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field also indicates that the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all inter-frequency neighbour cells on the frequency indicated by the MeasObjectNR. When this field is included, the network should set deriveSSB-IndexFromCell to true.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. When the UE is within an altitude range indicated by altitudeRange, it ignores the ssb-ToMeasure (without suffix), and applies the corresponding ssb-ToMeasure-r18 if present, otherwise (i.e., the UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

SSB-PositionQCL-CellsToAddMod field descriptions

physCellId Physical cell identity of a cell in the cell list.

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the value signalled by ssb-PositionQCL-Common.

| Conditional Presence | Explanation |
| --- | --- |
| AssociatedGapCSIRS | This field is optionally present, Need R if associatedMeasGapCSIRS is configured, otherwise, it is absent. |
| AssociatedGapSSB | This field is optionally present, Need R if associatedMeasGapSSB is configured, otherwise, it is absent. |
| CSI-RS | This field is mandatory present if csi-rs-ResourceConfigMobility is configured, otherwise, it is absent. |
| IntraFreqConnected | This field is optionally present, Need R if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, otherwise, it is absent. |
| SCG | This field is optionallly present, Need R, in the measConfig associated with the SCG. It is absent in the measConfig associated with the MCG. |
| NeighbourCell | This field is mandatory present if this MeasObject is configured by the serving cell for a neighbour cell served by a NTN Earth moving system. Otherwise, it is absent. |
| SharedSpectrum | This field is mandatory present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| SSBorAssociatedSSB | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. Otherwise, it is absent, Need R. |

#### –	MeasObjectNR-SL

The IE MeasObjectNR-SL concerns a measurement object including a list of transmission resource pool(s) for which CBR measurement is performed for NR sidelink communication/discovery.

MeasObjectNR-SL information element

-- ASN1START

-- TAG-MEASOBJECTNR-SL-START

MeasObjectNR-SL-r16 ::=      SEQUENCE {

tx-PoolMeasToRemoveList-r16  Tx-PoolMeasList-r16                           OPTIONAL,       -- Need N

tx-PoolMeasToAddModList-r16  Tx-PoolMeasList-r16                           OPTIONAL        -- Need N

}

MeasObjectNR-SL-r18 ::=      SEQUENCE {

sl-Frequency-r18             INTEGER (1..maxNrofFreqSL-r16),

tx-PoolMeasToRemoveList-r18  Tx-PoolMeasList-r16                           OPTIONAL,       -- Need N

tx-PoolMeasToAddModList-r18  Tx-PoolMeasList-r16                           OPTIONAL        -- Need N

}

Tx-PoolMeasList-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-PoolToMeasureNR-r16)) OF SL-ResourcePoolID-r16

-- TAG-MEASOBJECTNR-SL-STOP

-- ASN1STOP

MeasObjectNR-SL field descriptions

sl-Frequency Indicates the sidelink frequency associated to this MeasObjectNR-SL. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

#### –	MeasObjectRxTxDiff

The IE MeasObjectRxTxDiff is used to configure the measurement object for UE Rx-Tx time difference measurement.

MeasObjectRxTxDiff information element

-- ASN1START

-- TAG-MEASOBJECTRXTXDIFF-START

MeasObjectRxTxDiff-r17 ::=      SEQUENCE {

dl-Ref-r17      CHOICE {

prs-Ref-r17             NULL,

csi-RS-Ref-r17          NULL,

...

}                               OPTIONAL,   -- Need R

...

}

-- TAG-MEASOBJECTRXTXDIFF-STOP

-- ASN1STOP

MeasObjectRxTxDiff field descriptions

dl-Ref configures the DL references signals to measure Rx-Tx time difference. prs-Ref-r17 indicates PRS is chosen, and csi-RS-Ref-r17 indicates that CSI-RS for tracking is chosen. Only one PRS resource set is configured by the network. Only one NZP-CSI-RS-ResourceSet can be configured with pdc-Info-r17 set to true and it is used for UE Rx-Tx time difference measurement. Only reference signals from the PCell of the MCG can be configured by the network.

#### –	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify.

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEASOBJECTTOADDMODLIST-START

MeasObjectToAddModList ::=                  SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=                      SEQUENCE {

measObjectId                                MeasObjectId,

measObject                                  CHOICE {

measObjectNR                                MeasObjectNR,

...,

measObjectEUTRA                             MeasObjectEUTRA,

measObjectUTRA-FDD-r16                      MeasObjectUTRA-FDD-r16,

measObjectNR-SL-r16                         MeasObjectNR-SL-r16,

measObjectCLI-r16                           MeasObjectCLI-r16,

measObjectRxTxDiff-r17                      MeasObjectRxTxDiff-r17,

measObjectRelay-r17                         SL-MeasObject-r16,

measObjectNR-SL-r18                         MeasObjectNR-SL-r18

}

}

-- TAG-MEASOBJECTTOADDMODLIST-STOP

-- ASN1STOP

#### –	MeasObjectUTRA-FDD

The IE MeasObjectUTRA-FDD specifies information applicable for inter-RAT UTRA-FDD neighbouring cells.

MeasObjectUTRA-FDD information element

-- ASN1START

-- TAG-MEASOBJECTUTRA-FDD-START

MeasObjectUTRA-FDD-r16 ::=                  SEQUENCE {

carrierFreq-r16                             ARFCN-ValueUTRA-FDD-r16,

utra-FDD-Q-OffsetRange-r16                  UTRA-FDD-Q-OffsetRange-r16              OPTIONAL,         -- Need R

cellsToRemoveList-r16                       UTRA-FDD-CellIndexList-r16              OPTIONAL,         -- Need N

cellsToAddModList-r16                       CellsToAddModListUTRA-FDD-r16           OPTIONAL,         -- Need N

...

}

CellsToAddModListUTRA-FDD-r16 ::=           SEQUENCE (SIZE (1..maxCellMeasUTRA-FDD-r16)) OF CellsToAddModUTRA-FDD-r16

CellsToAddModUTRA-FDD-r16 ::=               SEQUENCE {

cellIndexUTRA-FDD-r16                       UTRA-FDD-CellIndex-r16,

physCellId-r16                              PhysCellIdUTRA-FDD-r16

}

UTRA-FDD-CellIndexList-r16 ::=              SEQUENCE (SIZE (1..maxCellMeasUTRA-FDD-r16)) OF UTRA-FDD-CellIndex-r16

UTRA-FDD-CellIndex-r16 ::=                  INTEGER (1..maxCellMeasUTRA-FDD-r16)

-- TAG-MEASOBJECTUTRA-FDD-STOP

-- ASN1STOP

MeasObjectUTRA-FDD field descriptions

carrierFreq Identifies UTRA-FDD carrier frequency for which this configuration is valid. NR does not configure more than one measurement object for the same physical frequency regardless of the ARFCN used to indicate this.

cellIndexUTRA-FDD Entry index in the neighbouring cell list.

cellsToAddModList List of UTRA-FDD cells to add/modify in the neighbouring cell list.

cellsToRemoveList List of cells to remove from the neighbouring cell list.

utra-FDD-Q-OffsetRange Used to indicate a frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value is in dB.

#### –	MeasResultCellListSFTD-NR

The IE MeasResultCellListSFTD-NR consists of SFN and radio frame boundary difference between the PCell and an NR cell as specified in TS 38.215 [9] and TS 38.133 [14].

MeasResultCellListSFTD-NR information element

-- ASN1START

-- TAG-MEASRESULTCELLLISTSFTD-NR-START

MeasResultCellListSFTD-NR ::=          SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD-NR

MeasResultCellSFTD-NR ::=              SEQUENCE {

physCellId                            PhysCellId,

sfn-OffsetResult                      INTEGER (0..1023),

frameBoundaryOffsetResult             INTEGER (-30720..30719),

rsrp-Result                           RSRP-Range                      OPTIONAL

}

-- TAG-MEASRESULTCELLLISTSFTD-NR-STOP

-- ASN1STOP

MeasResultCellSFTD-NR field descriptions

sfn-OffsetResult Indicates the SFN difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

#### –	MeasResultCellListSFTD-EUTRA

The IE MeasResultCellListSFTD-EUTRA consists of SFN and radio frame boundary difference between the PCell and an E-UTRA PSCell.

MeasResultCellListSFTD-EUTRA information element

-- ASN1START

-- TAG-MEASRESULTCELLLISTSFTD-EUTRA-START

MeasResultCellListSFTD-EUTRA ::=          SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultSFTD-EUTRA

MeasResultSFTD-EUTRA ::=           SEQUENCE {

eutra-PhysCellId                    EUTRA-PhysCellId,

sfn-OffsetResult                    INTEGER (0..1023),

frameBoundaryOffsetResult           INTEGER (-30720..30719),

rsrp-Result                         RSRP-Range                      OPTIONAL

}

-- TAG-MEASRESULTCELLLISTSFTD-EUTRA-STOP

-- ASN1STOP

MeasResultSFTD-EUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed.

sfn-OffsetResult Indicates the SFN difference between the PCell and the E-UTRA cell as an integer value according to TS 38.215 [9].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the E-UTRA cell as an integer value according to TS 38.215 [9].

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, inter-RAT mobility and measured results for NR sidelink communication/discovery.

MeasResults information element

-- ASN1START

-- TAG-MEASRESULTS-START

MeasResults ::=                         SEQUENCE {

measId                                  MeasId,

measResultServingMOList                 MeasResultServMOList,

measResultNeighCells                    CHOICE {

measResultListNR                        MeasResultListNR,

...,

measResultListEUTRA                     MeasResultListEUTRA,

measResultListUTRA-FDD-r16              MeasResultListUTRA-FDD-r16,

sl-MeasResultsCandRelay-r17             OCTET STRING        -- Contains PC5 SL-MeasResultListRelay-r17

}                                                                                                                   OPTIONAL,

...,

[[

measResultServFreqListEUTRA-SCG         MeasResultServFreqListEUTRA-SCG                                             OPTIONAL,

measResultServFreqListNR-SCG            MeasResultServFreqListNR-SCG                                                OPTIONAL,

measResultSFTD-EUTRA                    MeasResultSFTD-EUTRA                                                        OPTIONAL,

measResultSFTD-NR                       MeasResultCellSFTD-NR                                                       OPTIONAL

]],

[[

measResultCellListSFTD-NR               MeasResultCellListSFTD-NR                                                   OPTIONAL

]],

[[

measResultForRSSI-r16                   MeasResultForRSSI-r16                                                       OPTIONAL,

locationInfo-r16                        LocationInfo-r16                                                            OPTIONAL,

ul-PDCP-DelayValueResultList-r16        UL-PDCP-DelayValueResultList-r16                                            OPTIONAL,

measResultsSL-r16                       MeasResultsSL-r16                                                           OPTIONAL,

measResultCLI-r16                       MeasResultCLI-r16                                                           OPTIONAL

]],

[[

measResultRxTxTimeDiff-r17              MeasResultRxTxTimeDiff-r17                                                  OPTIONAL,

sl-MeasResultServingRelay-r17           OCTET STRING                                                                OPTIONAL,

-- Contains PC5 SL-MeasResultRelay-r17

ul-PDCP-ExcessDelayResultList-r17       UL-PDCP-ExcessDelayResultList-r17                                           OPTIONAL,

coarseLocationInfo-r17                  OCTET STRING                                                                OPTIONAL

]],

[[

altitudeUE-r18                          Altitude-r18                                                                OPTIONAL

]]

}

MeasResultServMOList ::=                SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO

MeasResultServMO ::=                    SEQUENCE {

servCellId                              ServCellIndex,

measResultServingCell                   MeasResultNR,

measResultBestNeighCell                 MeasResultNR                                                                OPTIONAL,

...

}

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                  OPTIONAL

]],

[[

choCandidate-r17                        ENUMERATED {true}                                                           OPTIONAL,

choConfig-r17                           SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16                             OPTIONAL,

triggeredEvent-r17                      SEQUENCE {

timeBetweenEvents-r17                   TimeBetweenEvent-r17                                                    OPTIONAL,

firstTriggeredEvent-r17                 ENUMERATED {condFirstEvent, condSecondEvent}                            OPTIONAL

}                                                                                                               OPTIONAL

]]

}

MeasResultListEUTRA ::=                 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=                     SEQUENCE {

eutra-PhysCellId                        PhysCellId,

measResult                              MeasQuantityResultsEUTRA,

cgi-Info                                CGI-InfoEUTRA                                                               OPTIONAL,

...

}

MultiBandInfoListEUTRA ::=              SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicatorEUTRA

MeasQuantityResults ::=                 SEQUENCE {

rsrp                                    RSRP-Range                                                                  OPTIONAL,

rsrq                                    RSRQ-Range                                                                  OPTIONAL,

sinr                                    SINR-Range                                                                  OPTIONAL

}

MeasQuantityResultsEUTRA ::=            SEQUENCE {

rsrp                                    RSRP-RangeEUTRA                                                             OPTIONAL,

rsrq                                    RSRQ-RangeEUTRA                                                             OPTIONAL,

sinr                                    SINR-RangeEUTRA                                                             OPTIONAL

}

ResultsPerSSB-IndexList::=              SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::=                 SEQUENCE {

ssb-Index                               SSB-Index,

ssb-Results                             MeasQuantityResults                                                         OPTIONAL

}

ResultsPerCSI-RS-IndexList::=           SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::=              SEQUENCE {

csi-RS-Index                            CSI-RS-Index,

csi-RS-Results                          MeasQuantityResults                                                         OPTIONAL

}

MeasResultServFreqListEUTRA-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF MeasResult2EUTRA

MeasResultServFreqListNR-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResult2NR

MeasResultListUTRA-FDD-r16 ::=          SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA-FDD-r16

MeasResultUTRA-FDD-r16 ::=              SEQUENCE {

physCellId-r16                          PhysCellIdUTRA-FDD-r16,

measResult-r16                          SEQUENCE {

utra-FDD-RSCP-r16                       INTEGER (-5..91)          OPTIONAL,

utra-FDD-EcN0-r16                       INTEGER (0..49)           OPTIONAL

}

}

MeasResultForRSSI-r16 ::=        SEQUENCE {

rssi-Result-r16                  RSSI-Range-r16,

channelOccupancy-r16             INTEGER (0..100)

}

MeasResultCLI-r16 ::=            SEQUENCE {

measResultListSRS-RSRP-r16       MeasResultListSRS-RSRP-r16                                                         OPTIONAL,

measResultListCLI-RSSI-r16       MeasResultListCLI-RSSI-r16                                                         OPTIONAL

}

MeasResultListSRS-RSRP-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultSRS-RSRP-r16

MeasResultSRS-RSRP-r16 ::=       SEQUENCE {

srs-ResourceId-r16               SRS-ResourceId,

srs-RSRP-Result-r16              SRS-RSRP-Range-r16

}

MeasResultListCLI-RSSI-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultCLI-RSSI-r16

MeasResultCLI-RSSI-r16 ::=       SEQUENCE {

rssi-ResourceId-r16              RSSI-ResourceId-r16,

cli-RSSI-Result-r16              CLI-RSSI-Range-r16

}

UL-PDCP-DelayValueResultList-r16 ::= SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16

UL-PDCP-DelayValueResult-r16 ::= SEQUENCE {

drb-Id-r16                       DRB-Identity,

averageDelay-r16                 INTEGER (0..10000),

...

}

UL-PDCP-ExcessDelayResultList-r17 ::= SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-ExcessDelayResult-r17

UL-PDCP-ExcessDelayResult-r17 ::= SEQUENCE {

drb-Id-r17                        DRB-Identity,

excessDelay-r17                   INTEGER (0..31),

...

}

TimeBetweenEvent-r17 ::= INTEGER (0..1023)

-- TAG-MEASRESULTS-STOP

-- ASN1STOP

MeasResultEUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed. The UE reports a value in the range 0..503, other values are reserved.

MeasResultNR field descriptions

averageDelay Indicates average delay for the packets during the reporting period, as specified in TS 38.314 [53]. Value 0 corresponds to 0 millisecond, value 1 corresponds to 0.1 millisecond, value 2 corresponds to 0.2 millisecond, and so on.

cellResults Cell level measurement results.

choCandidate This field indicates whether the associated cell is a candidate target cell for conditional handover or conditional PSCell change or addition. This field may be included only in the SuccessHO-Report or SuccessPSCell-Report within UEInformationResponse message.

choConfig If the associated cell is a candidate target cell for conditional handover, this field indicates the conditional handover execution condition for each measId within condTriggerConfig associated to the cell. This field may be included only in the rlf-report within UEInformationResponse message.

drb-Id Indicates DRB value for which uplink PDCP delay ratio or value is provided, according to TS 38.314 [53].

firstTriggeredEvent This field is set to condFirstEvent if the execution condition associated to the first entry of choConfig was fulfilled first in time. This field is set to condSecondEvent if the execution condition associated to the second entry of choConfig was fulfilled first in time. This field may be included in rlf-report within UEInformationResponse message or in SCGFailureInformation message.

locationInfo Positioning related information and measurements.

physCellId The physical cell identity of the NR cell for which the reporting is being performed.

resultsSSB-Cell Cell level measurement results based on SS/PBCH related measurements.

resultsSSB-Indexes Beam level measurement results based on SS/PBCH related measurements.

resultsCSI-RS-Cell Cell level measurement results based on CSI-RS related measurements.

resultsCSI-RS-Indexes Beam level measurement results based on CSI-RS related measurements.

rsIndexResults Beam level measurement results.

timeBetweenEvents Indicates the time elapsed between fulfilling the conditional execution conditions included in choConfig. Value in milliseconds. The maximum value 1023 means 1023ms or longer. This field may be included in the reports associated to UEInformationResponse message, e.g., rlf-Report or in the SCGFailureInformation message.

MeasResultUTRA-FDD field descriptions

physCellId The physical cell identity of the UTRA-FDD cell for which the reporting is being performed.

utra-FDD-EcN0 According to CPICH_Ec/No in TS 25.133 [46] for FDD.

utra-FDD-RSCP According to CPICH_RSCP in TS 25.133 [46] for FDD.

MeasResults field descriptions

coarseLocationInfo This field indicates the coarse location information reported by the UE. This field is coded as the Ellipsoid-Point defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement corresponds to a granularity of approximately 2 km. It is up to UE implementation how many LSBs are set to 0 to meet the accuracy requirement

excessDelay Indicates the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs, according to the UL PDCP Excess Packet Delay per DRB mapping table, as defined in TS 38.314 [53], Table 4.3.1.e-1.

measId Identifies the measurement identity for which the reporting is being performed.

measQuantityResults The value sinr is not included when it is used for LogMeasReport-r16.

measResultCellListSFTD-NR SFTD measurement results between the PCell and the NR neighbour cell(s) in NR standalone.

measResultCLI CLI measurement results.

measResultEUTRA Measured results of an E-UTRA cell.

measResultForRSSI Includes measured RSSI result in dBm (see TS 38.215 [9]) and channelOccupancy which is the percentage of samples when the RSSI was above the configured channelOccupancyThreshold for the associated reportConfig.

measResultListEUTRA List of measured results for the maximum number of reported best cells for an E-UTRA measurement identity.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultListUTRA-FDD List of measured results for the maximum number of reported best cells for a UTRA-FDD measurement identity.

measResultNR Measured results of an NR cell.

measResultServFreqListEUTRA-SCG Measured results of the E-UTRA SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each E-UTRA SCG serving frequency.

measResultServFreqListNR-SCG Measured results of the NR SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each NR SCG serving frequency.

measResultServingMOList Measured results of measured cells with reference signals indicated in the serving cell measurement objects including measurement results of SpCell, configured SCell(s) and best neighbouring cell within measured cells with reference signals indicated in on each serving cell measurement object. If the sending of the MeasurementReport message is triggered by a measurement configured by the field sl-ConfigDedicatedForNR received within an E-UTRA RRCConnectionReconfiguration message (i.e. CBR measurements), this field is not applicable and its contents is ignored by the network.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC.

measResultSFTD-NR SFTD measurement results between the PCell and the NR PScell in NR-DC.

measResultsSL CBR measurements results for NR sidelink communication/discovery.

measResultUTRA-FDD Measured result of a UTRA-FDD cell.

sl-MeasResultsCandRelay Measurement result(s) of candiate L2 U2N relay UE(s).

sl-MeasResultServingRelay Measurement result of serving L2 U2N relay UE.

#### –	MeasResult2EUTRA

The IE MeasResult2EUTRA contains measurements on E-UTRA frequencies.

MeasResult2EUTRA information element

-- ASN1START

-- TAG-MEASRESULT2EUTRA-START

MeasResult2EUTRA ::=       SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

measResultServingCell               MeasResultEUTRA                 OPTIONAL,

measResultBestNeighCell             MeasResultEUTRA                 OPTIONAL,

...

}

-- TAG-MEASRESULT2EUTRA-STOP

-- ASN1STOP

#### –	MeasResult2NR

The IE MeasResult2NR contains measurements on NR frequencies.

MeasResult2NR information element

-- ASN1START

-- TAG-MEASRESULT2NR-START

MeasResult2NR ::=                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                           OPTIONAL,

refFreqCSI-RS                       ARFCN-ValueNR                           OPTIONAL,

measResultServingCell               MeasResultNR                            OPTIONAL,

measResultNeighCellListNR           MeasResultListNR                        OPTIONAL,

...

}

-- TAG-MEASRESULT2NR-STOP

-- ASN1STOP

#### –	MeasResultIdleEUTRA

The IE MeasResultIdleEUTRA covers the E-UTRA measurement results performed in RRC_IDLE and RRC_INACTIVE.

MeasResultIdleEUTRA information element

-- ASN1START

-- TAG-MEASRESULTIDLEEUTRA-START

MeasResultIdleEUTRA-r16 ::= SEQUENCE {

measResultsPerCarrierListIdleEUTRA-r16   SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleEUTRA-r16,

...

}

MeasResultsPerCarrierIdleEUTRA-r16 ::=  SEQUENCE {

carrierFreqEUTRA-r16                    ARFCN-ValueEUTRA,

measResultsPerCellListIdleEUTRA-r16     SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleEUTRA-r16,

...

}

MeasResultsPerCellIdleEUTRA-r16 ::=     SEQUENCE {

eutra-PhysCellId-r16                    EUTRA-PhysCellId,

measIdleResultEUTRA-r16                 SEQUENCE {

rsrp-ResultEUTRA-r16                     RSRP-RangeEUTRA                                                     OPTIONAL,

rsrq-ResultEUTRA-r16                     RSRQ-RangeEUTRA-r16                                                 OPTIONAL

},

...

}

-- TAG-MEASRESULTIDLEEUTRA-STOP

-- ASN1STOP

MeasResultIdleEUTRA field descriptions

carrierFreqEUTRA Indicates the E-UTRA carrier frequency.

eutra-PhysCellId Indicates the physical cell identity of an E-UTRA cell.

measIdleResultEUTRA Idle/inactive measurement results for an E-UTRA cell.

measResultsPerCarrierListIdleEUTRA List of idle/inactive measured results for the maximum number of reported E-UTRA carriers.

measResultsPerCellListIdleEUTRA List of idle/inactive measured results for the maximum number of reported best cells for a given E-UTRA carrier.

#### –	MeasResultIdleNR

The IE MeasResultIdleNR covers the NR measurement results performed in RRC_IDLE and RRC_INACTIVE.

MeasResultIdleNR information element

-- ASN1START

-- TAG-MEASRESULTIDLENR-START

MeasResultIdleNR-r16 ::=  SEQUENCE {

measResultServingCell-r16 SEQUENCE {

rsrp-Result-r16           RSRP-Range                                                                        OPTIONAL,

rsrq-Result-r16           RSRQ-Range                                                                        OPTIONAL,

resultsSSB-Indexes-r16    ResultsPerSSB-IndexList-r16                                                       OPTIONAL

},

measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16    OPTIONAL,

...

}

MeasResultsPerCarrierIdleNR-r16 ::=   SEQUENCE {

carrierFreq-r16                       ARFCN-ValueNR,

measResultsPerCellListIdleNR-r16      SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16,

...

}

MeasResultsPerCellIdleNR-r16 ::=  SEQUENCE {

physCellId-r16                    PhysCellId,

measIdleResultNR-r16              SEQUENCE {

rsrp-Result-r16                   RSRP-Range                                                              OPTIONAL,

rsrq-Result-r16                   RSRQ-Range                                                              OPTIONAL,

resultsSSB-Indexes-r16            ResultsPerSSB-IndexList-r16                                             OPTIONAL

},

...,

[[

validityStatus-r18                ENUMERATED {checked, spare3, spare2, spare1}                                OPTIONAL

]]

}

ResultsPerSSB-IndexList-r16 ::=   SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16

ResultsPerSSB-IndexIdle-r16 ::=   SEQUENCE {

ssb-Index-r16                     SSB-Index,

ssb-Results-r16                   SEQUENCE {

ssb-RSRP-Result-r16               RSRP-Range                                                              OPTIONAL,

ssb-RSRQ-Result-r16               RSRQ-Range                                                              OPTIONAL

}                                                                                                     OPTIONAL

}

-- TAG-MEASRESULTIDLENR-STOP

-- ASN1STOP

MeasResultIdleNR field descriptions

carrierFreq Indicates the NR carrier frequency.

measIdleResultNR Idle/inactive measurement results for an NR cell (optionally including beam level measurements).

measResultServingCell Measured results of the serving cell (i.e., PCell) from idle/inactive measurements.

measResultsPerCellListIdleNR List of idle/inactive measured results for the maximum number of reported best cells for a given NR carrier.

resultsSSB-Indexes Beam level measurement results (indexes and optionally, beam measurements).

validityStatus Indicates whether UE has checked the validity of measurement results as defined in TS 38.133 [14].

#### –	MeasResultRxTxTimeDiff

The IE MeasResultRxTxTimeDiff is used to provide Rx-Tx time difference measurement result.

MeasResultRxTxTimeDiff information element

-- ASN1START

-- TAG-MEASRESULTRXTXTIMEDIFF-START

MeasResultRxTxTimeDiff-r17 ::= SEQUENCE {

rxTxTimeDiff-ue-r17            RxTxTimeDiff-r17      OPTIONAL,

...

}

-- TAG-MEASRESULTRXTXTIMEDIFF-STOP

-- ASN1STOP

MeasResultRxTxTimeDiff field descriptions

rxTxTimeDiff-ue indicates the Rx-Tx Time difference measurement at the UE (see clause 5.1.30, TS 38.215 [9]).

#### –	MeasResultSCG-Failure

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in (NG)EN-DC and NR-DC.

MeasResultSCG-Failure information element

-- ASN1START

-- TAG-MEASRESULTSCG-FAILURE-START

MeasResultSCG-Failure ::=           SEQUENCE {

measResultPerMOList                 MeasResultList2NR,

...,

[[

locationInfo-r16                    LocationInfo-r16            OPTIONAL

]]

}

MeasResultList2NR ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-MEASRESULTSCG-FAILURE-STOP

-- ASN1STOP

#### –	MeasResultsSL

The IE MeasResultsSL covers measured results for NR sidelink communication/discovery.

MeasResultsSL information element

-- ASN1START

-- TAG-MEASRESULTSSL-START

MeasResultsSL-r16 ::=         SEQUENCE {

measResultsListSL-r16         CHOICE {

measResultNR-SL-r16           MeasResultNR-SL-r16,

...

},

...

}

MeasResultNR-SL-r16 ::=       SEQUENCE {

measResultListCBR-NR-r16      SEQUENCE (SIZE (1.. maxNrofSL-PoolToMeasureNR-r16)) OF MeasResultCBR-NR-r16,

...

}

MeasResultCBR-NR-r16 ::=      SEQUENCE {

sl-poolReportIdentity-r16     SL-ResourcePoolID-r16,

sl-CBR-ResultsNR-r16          SL-CBR-r16,

...

}

-- TAG-MEASRESULTSSL-STOP

-- ASN1STOP

MeasResultsSL field descriptions

measResultNR-SL Include the measured results for NR sidelink communication/discovery.

MeasResultNR-SL field descriptions

measResultListCBR-NR CBR measurement results for NR sidelink communication/discovery.

sl-poolReportIdentity The identity of the transmission resource pool which is corresponding to the sl-ResourcePoolID configured in a resource pool for NR sidelink communication/discovery.

#### –	MeasSequence

The IE MeasSequence is used to configure a recommended sequence for intra/inter-RAT intra/inter-frequency measurement.

MeasSequence information element

-- ASN1START

-- TAG-MEASSEQUENCE-START

MeasSequence-r18 ::=             INTEGER (1..maxMeasSequence-r18)

-- TAG-MEASSEQUENCE-STOP

-- ASN1STOP

#### –	MeasTriggerQuantityEUTRA

The IE MeasTriggerQuantityEUTRA is used to configure the trigger quantity and reporting range for E-UTRA measurements. The RSRP, RSRQ and SINR ranges correspond to RSRP-Range, RSRQ-Range and RS-SINR-Range in TS 36.331 [10], respectively.

MeasTriggerQuantityEUTRA information element

-- ASN1START

-- TAG-MEASTRIGGERQUANTITYEUTRA-START

MeasTriggerQuantityEUTRA::=                 CHOICE {

rsrp                                        RSRP-RangeEUTRA,

rsrq                                        RSRQ-RangeEUTRA,

sinr                                        SINR-RangeEUTRA

}

RSRP-RangeEUTRA ::=                 INTEGER (0..97)

RSRQ-RangeEUTRA ::=                 INTEGER (0..34)

SINR-RangeEUTRA ::=                 INTEGER (0..127)

-- TAG-MEASTRIGGERQUANTITYEUTRA-STOP

-- ASN1STOP

#### –	MeasWindowConfig

The IE MeasWindowConfig specifies the effective measurement window configuration for inter-RAT E-UTRA measurement.

MeasWindowConfig information element

-- ASN1START

-- TAG-MEASWINDOWCONFIG-START

MeasWindowConfig-r18 ::=    SEQUENCE {

windowOffsetPeriodicity     CHOICE {

periodicityMs40         INTEGER (0..39),

periodicityMs80         INTEGER (0..79),

...

},

windowDuration              ENUMERATED {ms2, ms5, ms5dot5, spare1},

...

}

-- TAG-MEASWINDOWCONFIG-STOP

-- ASN1STOP

MeasWindowConfig field descriptions

windowDuration Value windowDuration is the measurement window length in ms of effective measurement window. Value ms2 corresponds to 2 ms, value ms5 corresponds to 5 ms, and Value ms5dot5 corresponds to 5.5 ms.

windowOffsetPeriodicity Indicates the periodicity and offset of effective measurement window. The choice determines the periodicity (periodicityMs40 for 40ms and periodicityMs80 for 80 ms). The field value indicates the offset in ms.

#### –	MobilityStateParameters

The IE MobilityStateParameters contains parameters to determine UE mobility state.

MobilityStateParameters information element

-- ASN1START

-- TAG-MOBILITYSTATEPARAMETERS-START

MobilityStateParameters ::=         SEQUENCE{

t-Evaluation                        ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal                        ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium                  INTEGER (1..16),

n-CellChangeHigh                    INTEGER (1..16)

}

-- TAG-MOBILITYSTATEPARAMETERS-STOP

-- ASN1STOP

MobilityStateParameters field descriptions

n-CellChangeHigh The number of cell changes to enter high mobility state. Corresponds to NCR_H in TS 38.304 [20].

n-CellChangeMedium The number of cell changes to enter medium mobility state. Corresponds to NCR_M in TS 38.304 [20].

t-Evaluation The duration for evaluating criteria to enter mobility states. Corresponds to TCRmax in TS 38.304 [20]. Value in seconds, s30 corresponds to 30 s and so on.

t-HystNormal The additional duration for evaluating criteria to enter normal mobility state. Corresponds to TCRmaxHyst in TS 38.304 [20]. Value in seconds, value s30 corresponds to 30 seconds and so on.

#### –	MRB-Identity

The IE MRB-Identity is used to identify a multicast MRB used by a UE.

MRB-Identity information element

-- ASN1START

-- TAG-MRB-IDENTITY-START

MRB-Identity-r17 ::=                    INTEGER (1..512)

-- TAG-MRB-IDENTITY-STOP

-- ASN1STOP

#### –	MsgA-ConfigCommon

The IE MsgA-ConfigCommon is used to configure the PRACH and PUSCH resource for transmission of MsgA in 2-step random access type procedure.

-- ASN1START

-- TAG-MSGACONFIGCOMMON-START

MsgA-ConfigCommon-r16 ::=           SEQUENCE {

rach-ConfigCommonTwoStepRA-r16      RACH-ConfigCommonTwoStepRA-r16,

msgA-PUSCH-Config-r16               MsgA-PUSCH-Config-r16                                      OPTIONAL --Cond InitialBWPConfig

}

-- TAG-MSGACONFIGCOMMON-STOP

-- ASN1STOP

MsgA-ConfigCommon field descriptions

msgA-PUSCH-Config Configuration of cell-specific MsgA PUSCH parameters which the UE uses for contention-based MsgA PUSCH transmission of this BWP. If the field is not configured for the selected UL BWP, the UE shall use the MsgA PUSCH configuration of initial UL BWP.

rach-ConfigCommonTwoStepRA Configuration of cell specific random access parameters which the UE uses for contention based and contention free 2-step random access type procedure as well as for 2-step RA type contention based beam failure recovery in this BWP.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWPConfig | The field is mandatory present when MsgA-ConfigCommon is configured for the initial uplink BWP, or when MsgA-ConfigCommon is configured for a non-initial uplink BWP and MsgA-ConfigCommon is not configured for the initial uplink BWP, otherwise the field is optionally present, Need S. |

#### –	MsgA-PUSCH-Config

The IE MsgA-PUSCH-Config is used to specify the PUSCH allocation for MsgA in 2-step random access type procedure.

MsgA-PUSCH-Config information element

-- ASN1START

-- TAG-MSGA-PUSCH-CONFIG-START

MsgA-PUSCH-Config-r16 ::=                      SEQUENCE {

msgA-PUSCH-ResourceGroupA-r16                  MsgA-PUSCH-Resource-r16                                       OPTIONAL, -- Cond InitialBWPConfig

msgA-PUSCH-ResourceGroupB-r16                  MsgA-PUSCH-Resource-r16                                       OPTIONAL, -- Cond GroupBConfigured

msgA-TransformPrecoder-r16                    ENUMERATED {enabled, disabled}                                 OPTIONAL, -- Need R

msgA-DataScramblingIndex-r16                   INTEGER (0..1023)                                             OPTIONAL, -- Need S

msgA-DeltaPreamble-r16                         INTEGER (-1..6)                                               OPTIONAL  -- Need R

}

MsgA-PUSCH-Resource-r16 ::=                    SEQUENCE {

msgA-MCS-r16                                   INTEGER (0..15),

nrofSlotsMsgA-PUSCH-r16                        INTEGER (1..4),

nrofMsgA-PO-PerSlot-r16                        ENUMERATED {one, two, three, six},

msgA-PUSCH-TimeDomainOffset-r16                INTEGER (1..32),

msgA-PUSCH-TimeDomainAllocation-r16            INTEGER (1..maxNrofUL-Allocations)                            OPTIONAL, -- Need S

startSymbolAndLengthMsgA-PO-r16                INTEGER (0..127)                                              OPTIONAL, -- Need S

mappingTypeMsgA-PUSCH-r16                      ENUMERATED {typeA, typeB}                                     OPTIONAL, -- Need S

guardPeriodMsgA-PUSCH-r16                      INTEGER (0..3)                                                OPTIONAL, -- Need R

guardBandMsgA-PUSCH-r16                        INTEGER (0..1),

frequencyStartMsgA-PUSCH-r16                   INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofPRBs-PerMsgA-PO-r16                        INTEGER (1..32),

nrofMsgA-PO-FDM-r16                            ENUMERATED {one, two, four, eight},

msgA-IntraSlotFrequencyHopping-r16             ENUMERATED {enabled}                                          OPTIONAL, -- Need R

msgA-HoppingBits-r16                           BIT STRING (SIZE(2))                                          OPTIONAL, -- Cond FreqHopConfigured

msgA-DMRS-Config-r16                           MsgA-DMRS-Config-r16,

nrofDMRS-Sequences-r16                         INTEGER (1..2),

msgA-Alpha-r16                                 ENUMERATED {alpha0, alpha04, alpha05, alpha06,

alpha07, alpha08, alpha09, alpha1}                OPTIONAL, -- Need S

interlaceIndexFirstPO-MsgA-PUSCH-r16           INTEGER (1..10)                                               OPTIONAL, -- Need R

nrofInterlacesPerMsgA-PO-r16                   INTEGER (1..10)                                               OPTIONAL, -- Need R

...

}

MsgA-DMRS-Config-r16 ::=                       SEQUENCE {

msgA-DMRS-AdditionalPosition-r16               ENUMERATED {pos0, pos1, pos3}                                 OPTIONAL, -- Need S

msgA-MaxLength-r16                             ENUMERATED {len2}                                             OPTIONAL, -- Need S

msgA-PUSCH-DMRS-CDM-Group-r16                  INTEGER (0..1)                                                OPTIONAL, -- Need S

msgA-PUSCH-NrofPorts-r16                       INTEGER (0..1)                                                OPTIONAL, -- Need S

msgA-ScramblingID0-r16                         INTEGER (0..65535)                                            OPTIONAL, -- Need S

msgA-ScramblingID1-r16                         INTEGER (0..65535)                                            OPTIONAL  -- Need S

}

-- TAG-MSGA-PUSCH-CONFIG-STOP

-- ASN1STOP

MsgA-PUSCH-Config field descriptions

msgA-DataScramblingIndex Identifier used to initiate data scrambling (c_init) for msgA PUSCH. If the field is absent the UE applies the value Physical cell ID (physCellID).

msgA-DeltaPreamble Power offset of msgA PUSCH relative to the preamble received target power. Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1).

msgA-PUSCH-ResourceGroupA MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group A. If field is not configured for the selected UL BWP, the UE shall use the MsgA PUSCH configuration for group A of initial UL BWP or RedCap-specific initial UL BWP (if configured) for (e)RedCap UEs.

msgA-PUSCH-ResourceGroupB MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group B.

msgA-TransformPrecoder Enables or disables the transform precoder for MsgA transmission (see clause 6.1.3 of TS 38.214 [19]).

MsgA-PUSCH-Resource field descriptions

guardBandMsgA-PUSCH PRB-level guard band between FDMed PUSCH occasions (see TS 38.213 [13], clause 8.1A). If interlaced PUSCH is configured, value 0 is applied.

guardPeriodMsgA-PUSCH Guard period between PUSCH occasions in the unit of symbols (see TS 38.213 [13], clause 8.1A).

frequencyStartMsgA-PUSCH Offset of lowest PUSCH occasion in frequency domain with respect to PRB 0 (see TS 38.213 [13], clause 8.1A).

interlaceIndexFirstPO-MsgA-PUSCH Interlace index of the first PUSCH occasion in frequency domain if interlaced PUSCH is configured. For 30kHz SCS only the integers 1, 2, 3, 4, 5 are applicable (see TS 38.213 [13], clause 8.1A).

mappingTypeMsgA-PUSCH PUSCH mapping type A or B. If the field is absent, the UE shall use the parameter msgA-PUSCH-TimeDomainAllocation (see TS 38.213 [13], clause 8.1A).

msgA-Alpha Dedicated alpha value for MsgA PUSCH. If the field is absent, the UE shall use the value of msg3-Alpha if configured, else UE applies value 1 (see TS 38.213 [13], clause 7.1.1).

msgA-DMRS-Config DMRS configuration for msgA PUSCH (see TS 38.213 [13], clause 8.1A and TS 38.214 [19] clause 6.2.2).

msgA-HoppingBits Value of hopping bits to indicate which frequency offset to be used for second hop. See Table 8.3-1 in TS 38.213 [13].

msgA-IntraSlotFrequencyHopping Intra-slot frequency hopping per PUSCH occasion (see TS 38.213 [13], clause 8.1A).

msgA-MCS Indicates the MCS index for msgA PUSCH from the Table 6.1.4.1-1 for DFT-s-OFDM and Table 5.1.3.1-1 for CP-OFDM in TS 38.214 [19].

msgA-PUSCH-TimeDomainAllocation Indicates a combination of start symbol and length and PUSCH mapping type from the TDRA table (PUSCH-TimeDomainResourceAllocationList if provided in PUSCH-ConfigCommon, or else the default Table 6.1.2.1.1-2 in 38.214 [19] is used if pusch-TimeDomainAllocationList is not provided in PUSCH-ConfigCommon). The parameter K2 in the table is not used for msgA PUSCH. The network configures one of msgA-PUSCH-TimeDomainAllocation and startSymbolAndLengthMsgA-PO, but not both. If the field is absent, the UE shall use the value of startSymbolAndLenghtMsgA-PO.

msgA-PUSCH-TimeDomainOffset A single time offset with respect to the start of each PRACH slot (with at least one valid RO), counted as the number of slots (based on the numerology of active UL BWP). See TS 38.213 [13], clause 8.1A.

nrofDMRS-Sequences Number of DMRS sequences for MsgA PUSCH for CP-OFDM. In case of single PUSCH configuration or if the DMRS symbols of multiple configurations are not overlapped, if the DMRS resources configured in one PUSCH occasion is no larger than 8 (for len2) or 4 (for len1), then only DMRS port is configured.

nrofInterlacesPerMsgA-PO Number of consecutive interlaces per PUSCH occasion if interlaced PUSCH is configured. For 30kHz SCS only the integers 1, 2, 3, 4, 5 are applicable (see TS 38.213 [13], clause 8.1A).

nrofMsgA-PO-FDM The number of msgA PUSCH occasions FDMed in one time instance (see TS 38.213 [13], clause 8.1A).

nrofMsgA-PO-PerSlot Number of time domain PUSCH occasions in each slot. PUSCH occasions including guard period are contiguous in time domain within a slot (see TS 38.213 [13], clause 8.1A).

nrofPRBs-PerMsgA-PO Number of PRBs per PUSCH occasion (see TS 38.213 [13], clause 8.1A). An eRedcap UE ignores the PRACH and PUSCH resource parameters (for preambles group A and B, or only group B as specified in TS 38.321 [3]) for transmission of MsgA in 2-step random access type procedure if configured number of PRBs per corresponding PUSCH resources is larger than the maximum number of PRBs that can be scheduled per slot or per hop.

nrofSlotsMsgA-PUSCH Number of slots (in active UL BWP numerology) containing one or multiple PUSCH occasions, each slot has the same time domain resource allocation (see TS 38.213 [13], clause 8.1A).

startSymbolAndLengthMsgA-PO An index giving valid combinations of start symbol, length and mapping type as start and length indicator (SLIV) for the first msgA PUSCH occasion, for RRC_CONNECTED UEs in non-initial BWP as described in TS 38.214 [19] clause 6.1.2. The network configures the field so that the allocation does not cross the slot boundary. The number of occupied symbols excludes the guard period. If the field is absent, the UE shall use the value in msgA-PUSCH-TimeDomainAllocation (see TS 38.213 [13], clause 8.1A). The network configures one of msgA-PUSCH-TimeDomainAllocation and startSymbolAndLengthMsgA-PO, but not both. If the field is absent, the UE shall use the value of msgA-PUSCH-TimeDomainAllocation.

MsgA-DMRS-Config field descriptions

msgA-DMRS-AdditionalPosition Indicates the position for additional DM-RS. If the field is absent, the UE applies value pos2.

msgA-MaxLength indicates single-symbol or double-symbol DMRS. If the field is absent, the UE applies value len1.

msgA-PUSCH-DMRS-CDM-Group 1-bit indication of indices of CDM group(s). If the field is absent, then both CDM groups are used.

msgA-PUSCH-NrofPorts 0 indicates 1 port per CDM group, 1 indicates 2 ports per CDM group. If the field is absent then 4 ports per CDM group are used (see TS 38.213 [13], clause 8.1A).

msgA-ScramblingID0 UL DMRS scrambling initialization for CP-OFDM. If the field is absent the UE applies the value Physical cell ID (physCellID).

msgA-ScramblingID1 UL DMRS scrambling initialization for CP-OFDM. If the field is absent the UE applies the value Physical cell ID (physCellID).

| Conditional Presence | Explanation |
| --- | --- |
| FreqHopConfigured | This field is mandatory present when the field msgA-IntraSlotFrequencyHopping is configured. Otherwise, the field is absent. |
| GroupBConfigured | The field is mandatory present if groupB-ConfiguredTwoStepRA is configured in RACH-ConfigCommonTwoStepRA, otherwise the field is absent. |
| InitialBWPConfig | The field is mandatory present when MsgA-ConfigCommon is configured for the initial uplink BWP, or when MsgA-ConfigCommon is configured for a non-initial uplink BWP and MsgA-ConfigCommon is not configured for the initial uplink BWP, otherwise the field is optionally present, Need S. |

#### –	MultiFrequencyBandListNR

The IE MultiFrequencyBandListNR is used to configure a list of one or multiple NR frequency bands.

MultiFrequencyBandListNR information element

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-START

MultiFrequencyBandListNR ::=        SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR

-- TAG-MULTIFREQUENCYBANDLISTNR-STOP

-- ASN1STOP

#### –	MultiFrequencyBandListNR-SIB

The IE MultiFrequencyBandListNR-SIB indicates the list of frequency bands, for which cell (re-)selection parameters are common, and a list of additionalPmax and additionalSpectrumEmission.

MultiFrequencyBandListNR-SIB information element

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-SIB-START

MultiFrequencyBandListNR-SIB ::=            SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo

NR-MultiBandInfo ::=                        SEQUENCE {

freqBandIndicatorNR                         FreqBandIndicatorNR         OPTIONAL,   -- Cond OptULNotSIB2

nr-NS-PmaxList                              NR-NS-PmaxList              OPTIONAL    -- Need S

}

MultiFrequencyBandListNR-SIB-v1760 ::=      SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo-v1760

NR-MultiBandInfo-v1760 ::=                  SEQUENCE {

nr-NS-PmaxList-v1760                        NR-NS-PmaxList-v1760        OPTIONAL    -- Need S

}

MultiFrequencyBandListNR-Aerial-SIB-r18 ::= SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfoAerial-r18

NR-MultiBandInfoAerial-r18 ::=              SEQUENCE {

freqBandIndicatorNR-Aerial-r18              FreqBandIndicatorNR         OPTIONAL,   -- Cond OptULNotSIB2

nr-NS-PmaxListAerial-r18                    NR-NS-PmaxListAerial-r18    OPTIONAL    -- Need S

}

-- TAG-MULTIFREQUENCYBANDLISTNR-SIB-STOP

-- ASN1STOP

NR-MultiBandInfo field descriptions

freqBandIndicatorNR Provides an NR frequency band number as defined in TS 38.101-1 [15], TS 38.101-2 [39], table 5.2-1, and TS 38.101-5 [75], table 5.2.2-1.

freqBandIndicatorNR-Aerial Provides an NR frequency band number for aerial UE(s), as defined in TS 38.101-1 [15], TS 38.101-2 [39], table 5.2-1, and TS 38.101-5 [75], table 5.2.2-1.

nr-NS-PmaxList Provides a list of additionalPmax and additionalSpectrumEmission values. If the field is absent the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15] table 6.2.3.1-1A, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. If nr-NS-PmaxList-v1760 is present, it shall contain the same number of entries, listed in the same order as in nr-NS-PmaxList (without suffix).

nr-NS-PmaxListAerial Provides a list of additionalPmax and additionalSpectrumEmission values for aerial UE(s) as defined in TS 38.101-1 [15], clause 6.2K. If the field is absent, the value indicated by the corresponding field within frequencyBandList for the corresponding NR frequency band number applies, if present. Otherwise (i.e. the field is not present for the corresponding NR frequency band number in any of the nr-NS-PmaxList, nr-NS-PmaxList-v1760 or nr-NS-PmaxListAerial), the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15] table 6.2.3.1-1A, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

| Conditional Presence | Explanation |
| --- | --- |
| OptULNotSIB2 | The field is absent for SIB2 and is mandatory present if the parent IE is included in SIB4 or frequencyInfoDL-SIB. Otherwise, if the field is absent in frequencyInfoUL-SIB in UplinkConfigCommonSIB, the UE will use the frequency band indicated in frequencyInfoDL-SIB in DownlinkConfigCommonSIB. |

#### –	MUSIM-GapConfig

The IE MUSIM-GapConfig specifies the MUSIM gap configuration and controls setup/release of MUSIM gaps.

MUSIM-GapConfig information element

-- ASN1START

-- TAG-MUSIM-GAPCONFIG-START

MUSIM-GapConfig-r17 ::=                  SEQUENCE {

musim-GapToReleaseList-r17               SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17               OPTIONAL, -- Need N

musim-GapToAddModList-r17                SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17                 OPTIONAL, -- Need N

musim-AperiodicGap-r17                   MUSIM-GapInfo-r17                                       OPTIONAL, -- Need N

...,

[[

musim-GapPriorityToAddModListExt-v1800   SEQUENCE (SIZE (1..3)) OF GapPriority-r17               OPTIONAL, -- Need N

musim-GapKeep-r18                        ENUMERATED {true}                                       OPTIONAL  -- Need R

]]

}

MUSIM-Gap-r17 ::=                        SEQUENCE {

musim-GapId-r17                          MUSIM-GapId-r17,

musim-GapInfo-r17                        MUSIM-GapInfo-r17

}

-- TAG-MUSIM-GAPCONFIG-STOP

-- ASN1STOP

MUSIM-GapConfig field descriptions

musim-AperiodicGap Indicates the MUSIM aperiodic gap as specified in TS 38.133 [14] clause 9.1.10. If UE indicates the musim-Starting-SFN-AndSubframe when requesting aperiodic gap the network can only configure the aperiodic gap with the same start point or no aperiodic gap. If the field musim-Starting-SFN-AndSubframe is absent for aperiodic gap, network can configure any timing as the starting point for aperiodic gap or configure no aperiodic gap.

musim-GapInfo Indicates the values for musim-GapLength and musim-GapRepetitionAndOffset. When network provides periodic gap, network always signals the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred MUSIM gap configuration.

musim-GapKeep Indicates the UE is allowed to use "keep solution" for collided MUSIM periodic/aperiodic gaps. If "keep solution" is not granted, collisions between MUSIM periodic gaps are resolved based on the assigned MUSIM gap priorities as specified in TS 38.133[14].

musim-GapPriorityToAddModListExt Indicates the priority of MUSIM periodic gap(s). If the network includes musim-GapPriorityToAddModList-r18, it includes the same number of entries, and listed in the same order, as in musim-GapToAddModList-r17. For the priority of MUSIM aperiodic gap, the MUSIM aperiodic gap is always kept (not dropped) from UE perspective in case of collisions with other gaps (i.e. all gaps including MUSIM gaps, etc).

musim-GapToAddModList List of MUSIM periodic gap patterns to add or modify.

musim-GapToReleaseList List of MUSIM periodic gap patterns to release.

#### –	MUSIM-GapId

The IE MUSIM-GapId is used to identify UE periodic MUSIM gap(s) to add, modify or release.

MUSIM-GapId information element

-- ASN1START

-- TAG-MUSIM-GAPID-START

MUSIM-GapId-r17 ::=                  INTEGER (0..2)

-- TAG-MUSIM-GAPID-STOP

-- ASN1STOP

#### –	MUSIM-GapInfo

The IE MUSIM-GapInfo is used to indicate MUSIM gap parameters.

MUSIM-GapInfo information element

-- ASN1START

-- TAG-MUSIM-GAPINFO-START

MUSIM-GapInfo-r17 ::=               SEQUENCE {

musim-Starting-SFN-AndSubframe-r17  MUSIM-Starting-SFN-AndSubframe-r17             OPTIONAL, -- Cond aperiodic

musim-GapLength-r17                 ENUMERATED {ms3, ms4, ms6, ms10, ms20}         OPTIONAL, -- Cond gapSetup

musim-GapRepetitionAndOffset-r17    CHOICE {

ms20-r17                            INTEGER (0..19),

ms40-r17                            INTEGER (0..39),

ms80-r17                            INTEGER (0..79),

ms160-r17                           INTEGER (0..159),

ms320-r17                           INTEGER (0..319),

ms640-r17                           INTEGER (0..639),

ms1280-r17                          INTEGER (0..1279),

ms2560-r17                          INTEGER (0..2559),

ms5120-r17                          INTEGER (0..5119),

...

}                                                                                  OPTIONAL -- Cond periodic

}

MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE {

starting-SFN-r17                       INTEGER (0..1023),

startingSubframe-r17                   INTEGER (0..9)

}

-- TAG-MUSIM-GAPINFO-STOP

-- ASN1STOP

MUSIM-GapInfo field descriptions

musim-GapLength Indicates the length of the UE's MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for both periodic gap and aperiodic gap preference indication.

musim-GapRepetitionAndOffset Indicates the gap repetition period in ms and gap offset in number of subframes for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for the periodic MUSIM gap preference indication.

musim-Starting-SFN-AndSubframe Indicates gap starting position for the aperiodic MUSIM gap. This field is optionally present for the aperiodic MUSIM gap preference indication.

starting-SFN Indicates gap starting SFN number for the aperiodic MUSIM gap.

startingSubframe Indicates gap starting subframe number for the aperiodic MUSIM gap.

| Conditional Presence | Explanation |
| --- | --- |
| aperiodic | This field is mandatory present in case of aperiodic MUSIM gap configuration. Otherwise it is absent. |
| gapSetup | The field is mandatory present upon configuration of a new MUSIM gap. The field is optionally present, Need M, otherwise. |
| periodic | This field is mandatory present in case of periodic MUSIM gap configuration. Otherwise it is absent. |

#### –	N3C-IndirectPathConfigRelay

The IE N3C-IndirectPathConfigRelay indicates the N3C indirect path related configuration used by N3C relay UE in MP.

N3C-IndirectPathConfigRelay information element

-- ASN1START

-- TAG-N3C-INDIRECTPATHCONFIGRELAY-START

N3C-IndirectPathConfigRelay-r18 ::=    SEQUENCE {

n3c-MappingToReleaseList-r18           SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17        OPTIONAL, -- Need N

n3c-MappingToAddModList-r18            SEQUENCE (SIZE (1..maxLC-ID)) OF N3C-MappingConfig-r18              OPTIONAL, -- Need N

...

}

N3C-MappingConfig-r18 ::=              SEQUENCE {

n3c-RemoteUE-RB-Identity-r18           SL-RemoteUE-RB-Identity-r17,

n3c-RLC-ChannelUu-r18                  Uu-RelayRLC-ChannelID-r17,

...

}

-- TAG-N3C-INDIRECTPATHCONFIGRELAY-STOP

-- ASN1STOP

N3C-IndirectPathConfigRelay field descriptions

n3c-MappingToAddModList Indicates the list of mappings between the bearer identity of the N3C remote UE and the Uu RLC channel to be added or modified. The Uu RLC channel is associated to only one N3C remote UE's end-to-end bearer.

n3c-MappingToReleaseList Indicates the list of mappings between the bearer identity of the N3C remote UE and the Uu RLC channel to be released.

#### –	N3C-IndirectPathAddChange

The IE N3C-IndirectPathAddChange indicates the N3C indirect path related configuration used by N3C remote UE.

N3C-IndirectPathAddChange information element

-- ASN1START

-- TAG-N3C-INDIRECTPATHADDCHANGE-START

N3C-IndirectPathAddChange-r18 ::=  SEQUENCE {

n3c-RelayIdentification-r18        N3C-RelayUE-Info-r18,

...

}

-- TAG-N3C-INDIRECTPATHADDCHANGE-STOP

-- ASN1STOP

N3C-IndirectPathAddChange field descriptions

n3c-RelayIdentification Indicates the NCGI and C-RNTI of N3C relay UE.

#### –	N3C-RelayUE-Info

The IE N3C-RelayUE-Info includes the information of N3C relay UE.

N3C-RelayUE-Info information element

-- ASN1START

-- TAG-N3CRELAYUEINFO-START

N3C-RelayUE-Info-r18 ::=      SEQUENCE {

n3c-CellGlobalId-r18          SEQUENCE {

n3c-PLMN-Id-r18               PLMN-Identity,

n3c-CellIdentity-r18          CellIdentity

},

n3c-C-RNTI-r18                RNTI-Value

}

-- TAG-N3CRELAYUEINFO-STOP

-- ASN1STOP

#### –	NCR-AperiodicFwdConfig

The IE NCR-AperiodicFwdConfig is used to configure a list of aperiodic forwarding time resources for NCR-Fwd access link (see TS 38.212 [17], clause 7.3.1.3.9 and TS 38.213 [13], clause 20).

NCR-AperiodicFwdConfig information element

-- ASN1START

-- TAG-NCR-APERIODICFWDCONFIG-START

NCR-AperiodicFwdConfig-r18 ::= SEQUENCE {

aperiodicFwdTimeRsrcToAddModList-r18  SEQUENCE (SIZE (1..maxNrofAperiodicFwdTimeResource-r18)) OF NCR-AperiodicFwdTimeResource-r18

OPTIONAL, -- Need N

aperiodicFwdTimeRsrcToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofAperiodicFwdTimeResource-r18)) OF NCR-AperiodicFwdTimeResourceId-r18

OPTIONAL, -- Need N

referenceSCS-r18                      SubcarrierSpacing                                             OPTIONAL, -- Need M

aperiodicBeamFieldWidth-r18           INTEGER (1..6)                                                OPTIONAL, -- Need M

numberOfFields-r18                    INTEGER (1..32)                                               OPTIONAL, -- Need M

...

}

NCR-AperiodicFwdTimeResource-r18 ::= SEQUENCE {

aperiodicFwdTimeRsrcId-r18           NCR-AperiodicFwdTimeResourceId-r18,

slotOffsetAperiodic-r18              INTEGER (0..14),

symbolOffset-r18                     INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18                INTEGER (1..28),

...

}

NCR-AperiodicFwdTimeResourceId-r18 ::= INTEGER (0..maxNrofAperiodicFwdTimeResource-1-r18)

-- TAG-NCR-APERIODICFWDCONFIG-STOP

-- ASN1STOP

NCR-AperiodicFwdConfig field descriptions

aperiodicBeamFieldWidth Indicates the bitwidth of each beam index field in DCI carrying aperiodic beam indication.

aperiodicFwdTimeRsrcToAddModList List of aperiodic forwarding time resources to be added or modified.

durationInSymbols Indicates the time duration in number of symbols.

numberOfFields Indicates the number of time resource fields in DCI carrying aperiodic beam indication.

referenceSCS Indicates the reference subcarrier spacing for all the time resources in the list. Only values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable.

slotOffsetAperiodic Indicates the slot offset used to define the start slot of aperiodic time resource.

symbolOffset Indicates the symbol offset in one slot.

#### –	NCR-FwdConfig

The IE NCR-FwdConfig contains configuration related to periodic, aperiodic and semi-persistent beam indication for NCR-Fwd access link.

NCR-FwdConfig information element

-- ASN1START

-- TAG-NCR-FWDCONFIG-START

NCR-FwdConfig-r18 ::=                SEQUENCE {

periodicFwdRsrcSetToAddModList-r18   SEQUENCE (SIZE (1..maxNrofPeriodicFwdResourceSet-r18)) OF NCR-PeriodicFwdResourceSet-r18

OPTIONAL,  -- Need N

periodicFwdRsrcSetToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofPeriodicFwdResourceSet-r18)) OF NCR-PeriodicFwdResourceSetId-r18

OPTIONAL,  -- Need N

aperiodicFwdConfig-r18                      SetupRelease {NCR-AperiodicFwdConfig-r18}                   OPTIONAL,   -- Need M

semiPersistentFwdRsrcSetToAddModList-r18    SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResourceSet-r18)) OF

NCR-SemiPersistentFwdResourceSet-r18    OPTIONAL,  -- Need N

semiPersistentFwdRsrcSetToReleaseList-r18   SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResourceSet-r18)) OF

NCR-SemiPersistentFwdResourceSetId-r18  OPTIONAL,  -- Need N

...

}

-- TAG-NCR-FWDCONFIG-STOP

-- ASN1STOP

NCR-FwdConfig field descriptions

aperiodicFwdConfig Aperiodic time resource configuration for beam indication for NCR. The configuration includes a list of time domain resources that can be selected for aperiodic forwarding.

periodicFwdRsrcSetToAddModList List of periodic forwarding resource configurations to be added or modified.

periodicFwdRsrcSetToReleaseList List of periodic forwarding resource configurations to be released.

semiPersistentFwdRsrcSetToAddModList List of semi-persistent forwarding resource configurations to be added or modified.

#### –	NCR-PeriodicityAndOffset

The IE NCR-PeriodicityAndOffset is used to indicate the periodicity (in slot or ms) and offset (in slot) for periodic forwarding resource and semi-persistent forwarding resource.

NCR-PeriodicityAndOffset information element

-- ASN1START

-- TAG-NCR-PERIODICITYANDOFFSET-START

NCR-PeriodicityAndOffset-r18 ::= CHOICE {

slot                             NCR-SlotPeriodicityAndSlotOffset-r18,

ms                               NCR-MsPeriodicityAndSlotOffset-r18

}

NCR-SlotPeriodicityAndSlotOffset-r18 ::=  CHOICE {

sl1                                       NULL,

sl2                                       INTEGER(0..1),

sl4                                       INTEGER(0..3),

sl5                                       INTEGER(0..4),

sl8                                       INTEGER(0..7),

sl10                                      INTEGER(0..9),

sl16                                      INTEGER(0..15),

sl20                                      INTEGER(0..19),

sl32                                      INTEGER(0..31),

sl40                                      INTEGER(0..39),

sl64                                      INTEGER(0..63),

sl80                                      INTEGER(0..79),

sl128                                     INTEGER(0..127),

sl160                                     INTEGER(0..159),

sl256                                     INTEGER(0..255),

sl320                                     INTEGER(0..319),

sl512                                     INTEGER(0..511),

sl640                                     INTEGER(0..639),

sl1024                                    INTEGER(0..1023),

sl1280                                    INTEGER(0..1279),

sl2560                                    INTEGER(0..2559),

sl5120                                    INTEGER(0..5119),

sl10240                                   INTEGER(0..10239),

...

}

NCR-MsPeriodicityAndSlotOffset-r18 ::=    CHOICE {

ms1                                       INTEGER(0..15),

ms2                                       INTEGER(0..31),

ms4                                       INTEGER(0..63),

ms5                                       INTEGER(0..79),

ms8                                       INTEGER(0..127),

ms10                                      INTEGER(0..159),

ms16                                      INTEGER(0..255),

ms20                                      INTEGER(0..319),

ms32                                      INTEGER(0..511),

ms40                                      INTEGER(0..639),

ms64                                      INTEGER(0..1023),

ms80                                      INTEGER(0..1279),

ms128                                     INTEGER(0..2047),

ms160                                     INTEGER(0..2559),

ms256                                     INTEGER(0..4095),

ms320                                     INTEGER(0..5119),

ms512                                     INTEGER(0..8191),

ms640                                     INTEGER(0..10239),

ms1024                                    INTEGER(0..16383),

ms1280                                    INTEGER(0..20479),

ms2560                                    INTEGER(0..40959),

ms5120                                    INTEGER(0..81919),

ms10240                                   INTEGER(0..163839),

...

}

-- TAG-NCR-PERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	NCR-PeriodicFwdResourceSet

The IE NCR-PeriodicFwdResourceSet is used to configure a list of periodic forwarding resources for NCR-Fwd access link (see TS 38.213 [13], clause 20). Each periodic forwarding configuration includes a list of periodic forwarding resources, a common periodicity and a common reference SCS.

NCR-PeriodicFwdResourceSet information element

-- ASN1START

-- TAG-NCR-PERIODICFWDRESOURCESET-START

NCR-PeriodicFwdResourceSet-r18 ::=  SEQUENCE {

periodicFwdRsrcSetId-r18            NCR-PeriodicFwdResourceSetId-r18,

periodicFwdRsrcToAddModList-r18     SEQUENCE (SIZE (1..maxNrofPeriodicFwdResource-r18)) OF NCR-PeriodicFwdResource-r18

OPTIONAL,  -- Need N

periodicFwdRsrcToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofPeriodicFwdResource-r18)) OF NCR-PeriodicFwdResourceId-r18

OPTIONAL,  -- Need N

referenceSCS-r18                    SubcarrierSpacing                                                   OPTIONAL,  -- Need M

priorityFlag-r18                    ENUMERATED {true}                                                   OPTIONAL,  -- Need R

...

}

NCR-PeriodicFwdResource-r18 ::=  SEQUENCE {

periodicFwdRsrcId-r18            NCR-PeriodicFwdResourceId-r18,

beamIndex-r18                    INTEGER (0..63),

periodicTimeRsrc-r18             SEQUENCE {

periodicityAndOffset-r18     NCR-PeriodicityAndOffset-r18,

symbolOffset-r18             INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18        INTEGER (1..112)

},

...

}

NCR-PeriodicFwdResourceId-r18 ::= INTEGER (0..maxNrofPeriodicFwdResource-1-r18)

-- TAG-NCR-PERIODICFWDRESOURCESET-STOP

-- ASN1STOP

NCR-PeriodicFwdResourceSet field descriptions

beamIndex Indicates the logical beam index for NCR-Fwd access link. NCR-Fwd is assumed to be ON over the indicated time domain resource if there is beam indication.

durationInSymbols Indicates the time duration in number of symbols.

periodicFwdRsrcToAddModList List of periodic forwarding resources to be added or modified.

periodicityAndOffset Indicates the periodicity and slot offset for the periodic forwarding resource. All the periodic forwarding resources configured within the same resource set should have the same periodicity. If the periodicity is expressed in [ms], the value range of slot offset is from 0 to Periodicity[ms] * referenceSCS[kHz]/15 -1.

priorityFlag Indicates the priority for the list of periodic forwarding resources, as specified in TS 38.213 [13], clause 20.

referenceSCS Indicates the reference subcarrier spacing for all the time resources in the list. Only values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable.

symbolOffset Indicates the symbol offset in one slot.

#### –	NCR-PeriodicFwdResourceSetId

The IE NCR-PeriodicFwdResourceSetId is used to identify one NCR-PeriodicFwdResourceSet.

NCR-PeriodicFwdResourceSetId information element

-- ASN1START

-- TAG-NCR-PERIODICFWDRESOURCESETID-START

NCR-PeriodicFwdResourceSetId-r18 ::= INTEGER (0..maxNrofPeriodicFwdResourceSet-1-r18)

-- TAG-NCR-PERIODICFWDRESOURCESETID-STOP

-- ASN1STOP

#### –	NCR-SemiPersistentFwdResourceSet

The IE NCR-SemiPersistentFwdResourceSet is used to configure a list of semi-persistent forwarding resources for NCR-Fwd access link (see TS 38.213 [13], clause 20). Each semi-persistent forwarding resource configuration includes a list of semi-persistent forwarding resources, a common periodicity and a common reference SCS.

NCR-SemiPersistentFwdResourceSet information element

-- ASN1START

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESET-START

NCR-SemiPersistentFwdResourceSet-r18 ::= SEQUENCE {

semiPersistentFwdRsrcSetId-r18           NCR-SemiPersistentFwdResourceSetId-r18,

semiPersistentFwdRsrcToAddModList-r18    SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResource-r18)) OF

NCR-SemiPersistentFwdResource-r18                       OPTIONAL,   -- Need N

semiPersistentFwdRsrcToReleaseList-r18   SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResource-r18)) OF

NCR-SemiPersistentFwdResourceId-r18                     OPTIONAL,  -- Need N

referenceSCS-r18                         SubcarrierSpacing                                               OPTIONAL,  -- Need M

priorityFlag-r18                         ENUMERATED {true}                                               OPTIONAL,   -- Need R

...

}

NCR-SemiPersistentFwdResource-r18 ::= SEQUENCE {

semiPersistentFwdRsrcId-r18           NCR-SemiPersistentFwdResourceId-r18,

beamIndex-r18                         INTEGER (0..63),

semiPersistentTimeRsrc-r18            SEQUENCE {

periodicityAndOffset-r18              NCR-PeriodicityAndOffset-r18,

symbolOffset-r18                      INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18                 INTEGER (1..112)

},

...

}

NCR-SemiPersistentFwdResourceId-r18 ::= INTEGER (0..maxNrofSemiPersistentFwdResource-1-r18)

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESET-STOP

-- ASN1STOP

NCR-SemiPersistentFwdResourceSet field descriptions

beamIndex Indicates logical beam index for NCR-Fwd access link. NCR-Fwd is assumed to be ON over the indicated time domain resource if there is beam indication.

durationInSymbols Indicates the time duration in number of symbols.

periodicityAndOffset Indicates the periodicity and slot offset for the semi-persistent forwarding resource. All the semi-persistent forwarding resources configured within the same resource set should have the same periodicity. If the periodicity is expressed in [ms], the value range of slot offset is from 0 to Periodicity[ms] * referenceSCS[kHz]/15 -1.

priorityFlag Indicates the priority for the list of semi-persistent forwarding resources, as specified in TS 38.213 [13], clause 20.

referenceSCS Indicates the reference subcarrier spacing for all the time resources in the list. Only values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable.

semiPersistentFwdRsrcToAddModList List of semi-persistent forwarding resources to be added or modified.

symbolOffset Indicates the symbol offset in one slot.

#### –	NCR-SemiPersistentFwdResourceSetId

The IE NCR-SemiPersistentFwdResourceSetId is used to identify one NCR-SemiPersistentFwdResourceSet.

NCR-SemiPersistentFwdResourceSetId information element

-- ASN1START

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESETID-START

NCR-SemiPersistentFwdResourceSetId-r18 ::= INTEGER (0..maxNrofSemiPersistentFwdResourceSet-1-r18)

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESETID-STOP

-- ASN1STOP

#### –	NeedForGapsConfigNR

The IE NeedForGapsConfigNR contains configuration related to the reporting of measurement gap requirement information.

NeedForGapsConfigNR information element

-- ASN1START

-- TAG-NeedForGapsConfigNR-START

NeedForGapsConfigNR-r16 ::=        SEQUENCE {

requestedTargetBandFilterNR-r16       SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR               OPTIONAL          -- Need R

}

-- TAG-NeedForGapsConfigNR-STOP

-- ASN1STOP

NeedForGapsConfigNR field descriptions

requestedTargetBandFilterNR Indicates the target NR bands that the UE is requested to report the gap requirement information.

#### –	NeedForGapsInfoNR

The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForGapsInfoNR information element

-- ASN1START

-- TAG-NeedForGapsInfoNR-START

NeedForGapsInfoNR-r16 ::=        SEQUENCE {

intraFreq-needForGap-r16      NeedForGapsIntraFreqList-r16,

interFreq-needForGap-r16      NeedForGapsBandListNR-r16

}

NeedForGapsIntraFreqList-r16 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16

NeedForGapsBandListNR-r16 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16

NeedForGapsIntraFreq-r16  ::=                 SEQUENCE {

servCellId-r16                               ServCellIndex,

gapIndicationIntra-r16                       ENUMERATED {gap, no-gap}

}

NeedForGapsNR-r16  ::=                        SEQUENCE {

bandNR-r16                                   FreqBandIndicatorNR,

gapIndication-r16                            ENUMERATED {gap, no-gap}

}

-- TAG-NeedForGapsInfoNR-STOP

-- ASN1STOP

NeedForGapsInfoNR field descriptions

intraFreq-needForGap Indicates the measurement gap requirement information for NR intra-frequency measurement.

interFreq-needForGap Indicates the measurement gap requirement information for NR inter-frequency measurement.

NeedForGapsIntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB) do not contain the frequency domain resources of the SSB associated to the initial DL BWP (CD-SSB). Value no-gap indicates a measurement gap is not needed to measure the SSB associated to the initial DL BWP (CD-SSB) for all configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB), no matter the SSB is within the configured BWP or not. This field shall be set to 'no-gap' for the serving cell(s) belonging to the corresponding band(s) where bwpOperationMeasWithoutInterrupt-r18 is supported by the UE.

NeedForGapsNR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value no-gap indicates a measurement gap is not needed.

#### –	NeedForGapNCSG-ConfigEUTRA

The IE NeedForGapNCSG-ConfigEUTRA contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForGapNCSG-ConfigEUTRA information element

-- ASN1START

-- TAG-NeedForGapNCSG-ConfigEUTRA-START

NeedForGapNCSG-ConfigEUTRA-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-EUTRA-r17   SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA     OPTIONAL    -- Need R

}

-- TAG-NeedForGapNCSG-ConfigEUTRA-STOP

-- ASN1STOP

NeedForGapNCSG-ConfigEUTRA field descriptions

requestedTargetBandFilterNCSG-EUTRA Indicates the target E-UTRA bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForGapNCSG-ConfigNR

The IE NeedForGapNCSG-ConfigNR contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForGapNCSG-ConfigNR information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-CONFIGNR-START

NeedForGapNCSG-ConfigNR-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-NR-r17   SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR          OPTIONAL          -- Need R

}

-- TAG-NEEDFORGAPNCSG-CONFIGNR-STOP

-- ASN1STOP

NeedForGapNCSG-ConfigNR field descriptions

requestedTargetBandFilterNCSG-NR Indicates the target NR bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForGapNCSG-InfoEUTRA

The IE NeedForGapNCSG-InfoEUTRA indicates whether measurement gap or NCSG is required for the UE to perform measurements on an E-UTRA target band while NR-DC or NE-DC is not configured.

NeedForGapNCSG-InfoEUTRA information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-INFOEUTRA-START

NeedForGapNCSG-InfoEUTRA-r17 ::=   SEQUENCE {

needForNCSG-EUTRA-r17              SEQUENCE (SIZE (1..maxBandsEUTRA)) OF NeedForNCSG-EUTRA-r17

}

NeedForNCSG-EUTRA-r17  ::=         SEQUENCE {

bandEUTRA-r17                      FreqBandIndicatorEUTRA,

gapIndication-r17                  ENUMERATED {gap, ncsg, nogap-noncsg}

}

-- TAG-NEEDFORGAPNCSG-INFOEUTRA-STOP

-- ASN1STOP

NeedForGapNCSG-InfoEUTRA field descriptions

needForNCSG-EUTRA Indicates the measurement gap and NCSG requirement information for E-UTRA measurement.

NeedForNCSG-EUTRA field descriptions

bandEUTRA Indicates the E-UTRA target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform measurements on the concerned E-UTRA target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration message or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that NCSG is needed, value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed. Value nogap-noncsg also indicates interruption is not needed.

#### –	NeedForGapNCSG-InfoNR

The IE NeedForGapNCSG-InfoNR indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForGapNCSG-InfoNR information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-INFONR-START

NeedForGapNCSG-InfoNR-r17 ::=     SEQUENCE {

intraFreq-needForNCSG-r17         NeedForNCSG-IntraFreqList-r17,

interFreq-needForNCSG-r17         NeedForNCSG-BandListNR-r17

}

NeedForNCSG-IntraFreqList-r17 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForNCSG-IntraFreq-r17

NeedForNCSG-BandListNR-r17 ::=    SEQUENCE (SIZE (1..maxBands)) OF NeedForNCSG-NR-r17

NeedForNCSG-IntraFreq-r17  ::=    SEQUENCE {

servCellId-r17                    ServCellIndex,

gapIndicationIntra-r17            ENUMERATED {gap, ncsg, nogap-noncsg}

}

NeedForNCSG-NR-r17  ::=           SEQUENCE {

bandNR-r17                        FreqBandIndicatorNR,

gapIndication-r17                 ENUMERATED {gap, ncsg, nogap-noncsg}

}

-- TAG-NEEDFORGAPNCSG-INFONR-STOP

-- ASN1STOP

NeedForGapNCSG-InfoNR field descriptions

intraFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR intra-frequency measurement.

interFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR inter-frequency measurement.

NeedForNCSG-IntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap or NCSG is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB) do not contain the frequency domain resources of the SSB associated to the initial DL BWP (CD-SSB). Value ncsg indicates that a NCSG is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value nogap-noncsg indicates that neither a measurement gap nor a NCSG is needed to measure the SSB associated to the initial DL BWP (CD-SSB) for all configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB), no matter the SSB is within the configured BWP or not. This field shall be set to 'nogap-noncsg' for the serving cell cell(s) belonging to the corresponding band(s) where bwpOperationMeasWithoutInterrupt-r18 is supported by the UE. Value nogap-noncsg also indicates interruption is not needed.

NeedForNCSG-NR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that a NCSG is needed, and value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed. Value nogap-noncsg also indicates interruption is not needed.

#### –	NeedForInterruptionInfoNR

The IE NeedForInterruptionInfoNR indicates whether interruption is needed for the UE to perform SSB based measurements on an NR target band without measurement gap while NR-DC or NE-DC is not configured.

NeedForInterruptionInfoNR information element

-- ASN1START

-- TAG-NeedForInterruptionInfoNR-START

NeedForInterruptionInfoNR-r18 ::=    SEQUENCE {

intraFreq-needForInterruption-r18    NeedForInterruptionIntraFreqList-r18,

interFreq-needForInterruption-r18    NeedForInterruptionBandListNR-r18,

...

}

NeedForInterruptionIntraFreqList-r18 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForInterruptionNR-r18

NeedForInterruptionBandListNR-r18 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18

NeedForInterruptionNR-r18  ::=       SEQUENCE {

interruptionIndication-r18           ENUMERATED {no-gap-with-interruption, no-gap-no-interruption}                     OPTIONAL

}

-- TAG-NeedForInterruptionInfoNR-STOP

-- ASN1STOP

NeedForInterruptionInfoNR field descriptions

intraFreq-needForInterruption Indicates the interruption requirement information for NR intra-frequency measurement. Each entry in the list is associated to the entry in list intraFreq-needForGap-r16 with the same index. This field shall be set to no-gap-no-interruption for the serving cell(s) belonging to the corresponding band(s) where bwpOperationMeasWithoutInterrupt-r18 is supported by the UE.

interFreq-needForInterruption Indicates the interruption requirement information for NR inter-frequency measurement. Each entry in the list is associated to the entry in list interFreq-needForGap-r16 with the same index.

NeedForInterruptionNR field descriptions

interruptionIndication Indicates whether interruption is needed for the UE to perform SSB based measurements without measurement gap. Value no-gap-with-interruption indicates that interruption is needed. Value no-gap-no-interruption indicates interruption is not needed.

#### –	NextHopChainingCount

The IE NextHopChainingCount is used to update the KgNB key and corresponds to parameter NCC: See TS 33.501 [11].

NextHopChainingCount information element

-- ASN1START

-- TAG-NEXTHOPCHAININGCOUNT-START

NextHopChainingCount ::=                    INTEGER (0..7)

-- TAG-NEXTHOPCHAININGCOUNT-STOP

-- ASN1STOP

#### –	NG-5G-S-TMSI

The IE NG-5G-S-TMSI contains a 5G S-Temporary Mobile Subscription Identifier (5G-S-TMSI), a temporary UE identity provided by the 5GC which uniquely identifies the UE within the tracking area, see TS 23.003 [21].

NG-5G-S-TMSI information element

-- ASN1START

-- TAG-NG-5G-S-TMSI-START

NG-5G-S-TMSI ::=                         BIT STRING (SIZE (48))

-- TAG-NG-5G-S-TMSI-STOP

-- ASN1STOP

#### –	NonCellDefiningSSB

The IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP or a dedicated BWP that does not contain the CD-SSB.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

NonCellDefiningSSB field descriptions

absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.

ssb-Periodicity The periodicity of this NCD-SSB. The network configures only periodicities that are larger than the periodicity of serving cell's CD-SSB. If the field is absent, the UE applies the SSB periodicity of the CD-SSB (ssb-periodicityServingCell configured in ServingCellConfigCommon or ServingCellConfigCommonSIB).

ssb-TimeOffset The time offset between CD-SSB of the serving cell and this NCD-SSB. Value ms5 means the first burst of NCD-SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value ms10 means the first burst of NCD-SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero. For UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell's CD-SSB.

#### –	NPN-Identity

The IE NPN-Identity includes either a list of CAG-IDs or a list of NIDs per PLMN Identity. Further information regarding how to set the IE is specified in TS 23.003 [21].

NPN-Identity information element

-- ASN1START

-- TAG-NPN-IDENTITY-START

NPN-Identity-r16 ::=             CHOICE {

pni-npn-r16                      SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

cag-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16

},

snpn-r16                         SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

nid-List-r16                     SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

}

CAG-IdentityInfo-r16 ::=         SEQUENCE {

cag-Identity-r16                 BIT STRING (SIZE (32)),

manualCAGselectionAllowed-r16    ENUMERATED {true}                         OPTIONAL   -- Need R

}

NID-r16 ::=                      BIT STRING (SIZE (44))

-- TAG-NPN-IDENTITY-STOP

-- ASN1STOP

NPN-Identity field descriptions

cag-Identity A CAG-ID as specified in TS 23.003 [21]. The PLMN ID and a CAG ID in the NPN-Identity identifies a PNI-NPN.

cag-IdentityList The cag-IdentityList contains one or more CAG IDs. All CAG IDs associated to the same PLMN ID are listed in the same cag-IdentityList entry.

manualCAGselectionAllowed The manualCAGselectionAllowed indicates that the CAG ID can be selected manually even if it is outside the UE's allowed CAG list.

NID A NID as specified in TS 23.003 [21]. The PLMN ID and a NID in the NPN-Identity identifies a SNPN.

nid-List The nid-List contains one or more NID.

#### –	NPN-IdentityInfoList

The IE NPN-IdentityInfoList includes a list of NPN identity information.

NPN-IdentityInfoList information element

-- ASN1START

-- TAG-NPN-IDENTITYINFOLIST-START

NPN-IdentityInfoList-r16 ::=     SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r16

NPN-IdentityInfo-r16 ::=         SEQUENCE {

npn-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16,

trackingAreaCode-r16             TrackingAreaCode,

ranac-r16                        RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity-r16                 CellIdentity,

cellReservedForOperatorUse-r16   ENUMERATED {reserved, notReserved},

iab-Support-r16                  ENUMERATED {true}                                           OPTIONAL,       -- Need S

...,

[[

gNB-ID-Length-r17                INTEGER (22..32)                                            OPTIONAL        -- Need R

]],

[[

mobileIAB-Support-r18            ENUMERATED {true}                                           OPTIONAL        -- Need S

]]

}

-- TAG-NPN-IDENTITYINFOLIST-STOP

-- ASN1STOP

NPN-IdentityInfoList field descriptions

iab-Support This field combines both the support of IAB and the cell status for IAB. If the field is present, the cell supports IAB and the cell is also considered as a candidate for cell (re)selection for IAB-nodes; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell.

gNB-ID-Length Indicates the length of the gNB ID out of the 36-bit long cellIdentity. This field is always present if the mobileIAB-Support is broadcasted in a cell.

mobileIAB-Support This field indicates the support of mobile IAB and whether the cell can be considered as a candidate for cell (re)selection for mobile IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell. If the field is absent, the cell is barred for mobile IAB-node.

NPN-IdentityInfo The NPN-IdentityInfo contains one or more NPN identities and additional information associated with those NPNs. Only the same type of NPNs (either SNPNs or PNI-NPNs) can be listed in a NPN-IdentityInfo element.

npn-IdentityList The npn-IdentityList contains one or more NPN Identity elements.

trackingAreaCode Indicates the Tracking Area Code to which the cell indicated by cellIdentity field belongs.

ranac Indicates the RAN Area Code to which the cell indicated by cellIdentity field belongs.

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (for the NPN(s) identified in the npn-IdentityList) as defined in TS 38.304 [20]. This field is ignored by NPN capable IAB-MT and NPN capable NCR-MT.

#### –	NR-DL-PRS-PDC-Info

The IE NR-DL-PRS-PDC-Info defines downlink PRS configuration for PDC.

NR-DL-PRS-PDC-Info information element

-- ASN1START

-- TAG-NR-DL-PRS-PDC-INFO-START

NR-DL-PRS-PDC-Info-r17 ::=    SEQUENCE {

nr-DL-PRS-PDC-ResourceSet-r17          NR-DL-PRS-PDC-ResourceSet-r17                    OPTIONAL, -- Need R

...

}

NR-DL-PRS-PDC-ResourceSet-r17 ::=    SEQUENCE {

periodicityAndOffset-r17     NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17,

numSymbols-r17               ENUMERATED {n2, n4, n6, n12, n1-v1800, spare3, spare2, spare1},

dl-PRS-ResourceBandwidth-r17        INTEGER (1..63),

dl-PRS-StartPRB-r17                 INTEGER (0..2176),

resourceList-r17             SEQUENCE (SIZE (1..maxNrofPRS-ResourcesPerSet-r17)) OF NR-DL-PRS-Resource-r17,

repFactorAndTimeGap-r17      RepFactorAndTimeGap-r17                                    OPTIONAL,   -- Need S

...

}

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17 ::= CHOICE {

scs15-r17       CHOICE {

n4-r17                  INTEGER (0..3),

n5-r17                  INTEGER (0..4),

n8-r17                  INTEGER (0..7),

n10-r17                 INTEGER (0..9),

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n160-r17                INTEGER (0..159),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

...

},

scs30-r17       CHOICE {

n8-r17                  INTEGER (0..7),

n10-r17                 INTEGER (0..9),

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

...

},

scs60-r17       CHOICE {

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n256-r17                INTEGER (0..255),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

n40960-r17              INTEGER (0..40959),

...

},

scs120-r17      CHOICE {

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n256-r17                INTEGER (0..255),

n320-r17                INTEGER (0..319),

n512-r17                INTEGER (0..511),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

n40960-r17              INTEGER (0..40959),

n81920-r17              INTEGER (0..81919),

...

},

...

}

NR-DL-PRS-Resource-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r17            NR-DL-PRS-ResourceID-r17,

dl-PRS-SequenceID-r17               INTEGER (0..4095),

dl-PRS-CombSizeN-AndReOffset-r17    CHOICE {

n2-r17                          INTEGER (0..1),

n4-r17                          INTEGER (0..3),

n6-r17                          INTEGER (0..5),

n12-r17                         INTEGER (0..11),

...

},

dl-PRS-ResourceSlotOffset-r17       INTEGER (0..maxNrofPRS-ResourceOffsetValue-1-r17),

dl-PRS-ResourceSymbolOffset-r17     INTEGER (0..12),

dl-PRS-QCL-Info-r17                 DL-PRS-QCL-Info-r17                         OPTIONAL, -- Need N

...,

[[

dl-PRS-ResourceSymbolOffset-v1800   INTEGER (13)                                OPTIONAL  -- Need R

]]

}

DL-PRS-QCL-Info-r17 ::= CHOICE {

ssb-r17                     SEQUENCE {

ssb-Index-r17                   INTEGER (0..63),

rs-Type-r17                     ENUMERATED {typeC, typeD, typeC-plus-typeD},

...

},

dl-PRS-r17                  SEQUENCE {

qcl-DL-PRS-ResourceID-r17       NR-DL-PRS-ResourceID-r17,

...

},

...

}

NR-DL-PRS-ResourceID-r17 ::= INTEGER (0..maxNrofPRS-ResourcesPerSet-1-r17)

RepFactorAndTimeGap-r17 ::=  SEQUENCE {

repetitionFactor-r17         ENUMERATED {n2, n4, n6, n8, n16, n32, spare2, spare1},

timeGap-r17                  ENUMERATED {s1, s2, s4, s8, s16, s32, spare2, spare1}

}

-- TAG-NR-DL-PRS-PDC-INFO-STOP

-- ASN1STOP

NR-DL-PRS-PDC-ResourceSet field descriptions

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for all the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs in this resource set. All DL-PRS Resources of the DL-PRS-PDC Resource Set have the same bandwidth. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to subcarrier 0 in common resource block 0 for the DL-PRS Resource. All DL-PRS Resources of the DL-PRS-PDC Resource Set have the same value of dl-PRS-StartPRB.

numSymbols This field specifies the number of symbols per DL-PRS Resource within a slot. The UE does not expect to be configured for PDC with a PRS with numSymbols equals to n1 unless an SSB index is provided as a typeC or typeC-plus-typeD QCL source, or another PRS resource with numSymbols more than n1 is provided as QCL source.

periodicityAndOffset This field specifies the periodicity of DL-PRS allocation in slots and the slot offset with respect to SFN #0 slot #0 in the PCell where the DL-PRS-PDC Resource Set is configured (i.e., slot where the first DL-PRS Resource of DL-PRS-PDC Resource Set occurs).

repFactorAndTimeGap If this field is absent, the value for repetitionFactor is 1 (i.e., no resource repetition).

RepFactorAndTimeGap field descriptions

repetitionFactor This field specifies how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set. It is applied to all resources of the DL-PRS Resource Set. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively.

timeGap This field specifies the offset in units of slots between two repeated instances of a DL-PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL-PRS Resource Set. The time duration spanned by one DL-PRS Resource Set containing repeated DL-PRS Resources should not exceed the periodicity configured by periodicityAndOffset.

NR-DL-PRS-Resource field description

dl-PRS-ResourceSymbolOffset This field specifies the starting symbol of the DL-PRS Resource within a slot. If dl-PRS-ResourceSymbolOffset-v1800 is present, the UE shall ignore dl-PRS-ResourceSymbolOffset-r17.

#### –	NR-NS-PmaxList

The IE NR-NS-PmaxList is used to configure a list of additionalPmax and additionalSpectrumEmission, as defined in TS 38.101-1 [15], table 6.2.3.1-1A, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A for a given frequency band.

NR-NS-PmaxList information element

-- ASN1START

-- TAG-NR-NS-PMAXLIST-START

NR-NS-PmaxList ::=                      SEQUENCE (SIZE (1..maxNR-NS-Pmax)) OF NR-NS-PmaxValue

NR-NS-PmaxValue ::=                     SEQUENCE {

additionalPmax                          P-Max                               OPTIONAL,   -- Need N

additionalSpectrumEmission              AdditionalSpectrumEmission

}

NR-NS-PmaxList-v1760 ::=                SEQUENCE (SIZE (1.. maxNR-NS-Pmax)) OF NR-NS-PmaxValue-v1760

NR-NS-PmaxValue-v1760 ::=               SEQUENCE {

additionalSpectrumEmission-v1760        AdditionalSpectrumEmission-v1760    OPTIONAL   -- Need N

}

NR-NS-PmaxListAerial-r18 ::=            SEQUENCE (SIZE (1..maxNR-NS-Pmax)) OF NR-NS-PmaxValueAerial-r18

NR-NS-PmaxValueAerial-r18 ::=           SEQUENCE {

additionalPmax-r18                      P-Max                               OPTIONAL,   -- Need N

additionalSpectrumEmission-r18          AdditionalSpectrumEmission-r18

}

-- TAG-NR-NS-PMAXLIST-STOP

-- ASN1STOP

#### –	NSAG-ID

The IE NSAG-ID is used to identify an NSAG (TS 23.501 [32]) for slice-based cell reselection or slice-based random access.

NSAG-ID information element

-- ASN1START

-- TAG-NSAG-ID-START

NSAG-ID-r17 ::=                      BIT STRING (SIZE (8))

-- TAG-NSAG-ID-STOP

-- ASN1STOP

#### –	NSAG-IdentityInfo

The IE NSAG-IdentityInfo is used to identify an NSAG (TS 23.501 [32]) for slice-based cell reselection.

NSAG-IdentityInfo information element

-- ASN1START

-- TAG-NSAG-IDENTITYINFO-START

NSAG-IdentityInfo-r17 ::=            SEQUENCE {

nsag-ID-r17                          NSAG-ID-r17,

trackingAreaCode-r17                 TrackingAreaCode               OPTIONAL      -- Need R

}

-- TAG-NSAG-IDENTITYINFO-STOP

-- ASN1STOP

NSAG-IdentityInfo field descriptions

trackingAreaCode If absent, UE assumes the trackingAreaCode of the serving cell.

#### –	NTN-Config

The IE NTN-Config provides parameters needed for the UE to access NR via NTN access.

NTN-Config information element

-- ASN1START

-- TAG-NTN-CONFIG-START

NTN-Config-r17 ::=             SEQUENCE {

epochTime-r17                  EpochTime-r17                                                            OPTIONAL,  -- Need R

ntn-UlSyncValidityDuration-r17 ENUMERATED{ s5, s10, s15, s20, s25, s30, s35,

s40, s45, s50, s55, s60, s120, s180, s240, s900}              OPTIONAL,  -- Cond SIB19

cellSpecificKoffset-r17        INTEGER(1..1023)                                                         OPTIONAL,  -- Need R

kmac-r17                       INTEGER(1..512)                                                          OPTIONAL,  -- Need R

ta-Info-r17                    TA-Info-r17                                                              OPTIONAL,  -- Need R

ntn-PolarizationDL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ntn-PolarizationUL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ephemerisInfo-r17              EphemerisInfo-r17                                                        OPTIONAL,  -- Need R

ta-Report-r17                  ENUMERATED {enabled}                                                     OPTIONAL,  -- Need R

...

}

TA-Info-r17 ::=                 SEQUENCE  {

ta-Common-r17                  INTEGER(0..66485757),

ta-CommonDrift-r17             INTEGER(-257303..257303)                                                 OPTIONAL,  -- Need R

ta-CommonDriftVariant-r17      INTEGER(0..28949)                                                        OPTIONAL   -- Need R

}

-- TAG-NTN-CONFIG-STOP

-- ASN1STOP

NTN-Config field descriptions

EphemerisInfo This field provides satellite ephemeris either in format of position and velocity state vector or in format of orbital parameters. This field is excluded when determining changes in system information, i.e. changes to ephemerisInfo should neither result in system information change notifications nor in a modification of valueTag in SIB1.

epochTime If this field is absent for the serving cell, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when ntn-Config is provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig or SatSwitchWithReSync the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the target cell. For the target cell the UE considers epoch time, indicated by the SFN and sub-frame number in this field, to be the frame nearest to the frame in which the message indicating the epoch time is received. This field is excluded when determining changes in system information, i.e. changes to epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

cellSpecificKoffset Scheduling offset used for the timing relationships that are modified for NTN (see TS 38.213 [13]). The unit of the field K_offset is number of slots for a given subcarrier spacing of 15 kHz. If the field is absent UE assumes value 0.

kmac Scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. If the field is absent UE assumes value 0. The unit of kmac is number of slots for a given subcarrier spacing of 15 kHz.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL If present, this parameter indicates Polarization information for uplink service link. If not present and ntn-PolarizationDL is present, UE assumes the same polarization for UL and DL.

ntn-UlSyncValidityDuration A validity duration configured by the network for assistance information (i.e. Serving and/or neighbour satellite ephemeris and Common TA parameters) which indicates the maximum time duration (from epochTime) during which the UE can apply assistance information without having acquired new assistance information. The unit of ntn-UlSyncValidityDuration is second. Value s5 corresponds to 5 s, value s10 indicate 10 s and so on. This parameter applies to both connected and idle mode UEs. If this field is absent in ntn-Config provided via NTN-NeighCellConfig or SatSwitchWithReSync, the UE uses validity duration from the serving cell assistance information. This field is excluded when determining changes in system information, i.e. changes of ntn-UlSyncValidityDuration should neither result in system information change notifications nor in a modification of valueTag in SIB1. ntn-UlSyncValidityDuration is only updated when at least one of epochTime, ta-Info, ephemerisInfo is updated.

ta-Common Network-controlled common timing advanced value and it may include any timing offset considered necessary by the network. ta-Common with value of 0 is supported. The granularity of ta-Common is 4.072 × 10^(-3) μs. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-Common should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDrift Indicate drift rate of the common TA. The granularity of ta-CommonDrift is 0.2 × 10^(-3) μs⁄s. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDrift should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDriftVariant Indicate drift rate variation of the common TA. The granularity of ta-CommonDriftVariant is 0.2×10^(-4) μs⁄s^2. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDriftVariant should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-Report When this field is included in SIB19, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

| Conditional Presence | Explanation |
| --- | --- |
| SIB19 | The field is mandatory present for the serving cell in SIB19. The field is optionally present, Need R, otherwise. |

#### –	NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RS transmitted in the cell where the IE is included, which the UE may be configured to measure on (see TS 38.214 [19], clause 5.2.2.3.1). A change of configuration between periodic, semi-persistent or aperiodic for an NZP-CSI-RS-Resource is not supported without a release and add.

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

NZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on. The corresponding offset is also given in number of slots (see TS 38.214 [19], clause 5.2.2.3.1). Network always configures the UE with a value for this field for periodic and semi-persistent NZP-CSI-RS-Resource (as indicated in CSI-ResourceConfig).

powerControlOffset Power offset of PDSCH RE to NZP CSI-RS RE. Value in dB (see TS 38.214 [19], clauses 5.2.2.3.1 and 4.1). The UE shall ignore this field in case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE.

powerControlOffsetSS Power offset of NZP CSI-RS RE to SSS RE. Value in dB (see TS 38.214 [19], clause 5.2.2.3.1).

qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State or dl-OrJoint-TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1). In case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE, it refers to the TCI state identifier in CandidateTCI-State and is defined in ltm-DL-OrJointTCI-StateToAddModList within the LTM-Candidate IE.

resourceMapping OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource.

scramblingID Scrambling ID (see TS 38.214 [19], clause 5.2.2.3.1).

| Conditional Presence | Explanation |
| --- | --- |
| LTM | The field is optionally present in an LTM-Candidate IE. Otherwise, the field is absent. |
| Periodic | The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	NZP-CSI-RS-ResourceId

The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource.

NZP-CSI-RS-ResourceId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCEID-START

NZP-CSI-RS-ResourceId ::=           INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

-- TAG-NZP-CSI-RS-RESOURCEID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSet

The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition                          ENUMERATED { on, off }                                                  OPTIONAL,   -- Need S

aperiodicTriggeringOffset           INTEGER(0..6)                                                           OPTIONAL,   -- Need S

trs-Info                            ENUMERATED {true}                                                       OPTIONAL,   -- Need R

...,

[[

aperiodicTriggeringOffset-r16       INTEGER(0..31)                                                          OPTIONAL   -- Need S

]],

[[

pdc-Info-r17                        ENUMERATED {true}                                                       OPTIONAL,  -- Need R

cmrGroupingAndPairing-r17           CMRGroupingAndPairing-r17                                               OPTIONAL,  -- Need R

aperiodicTriggeringOffset-r17       INTEGER (0..124)                                                        OPTIONAL,  -- Need S

aperiodicTriggeringOffsetL2-r17     INTEGER(0..31)                                                          OPTIONAL   -- Need R

]],

[[

resourceType-r18                    ENUMERATED {periodic}                                                   OPTIONAL   -- Cond LTM

]]

}

CMRGroupingAndPairing-r17 ::=        SEQUENCE {

nrofResourcesGroup1-r17              INTEGER (1..7),

pair1OfNZP-CSI-RS-r17                NZP-CSI-RS-Pairing-r17                                                 OPTIONAL,  -- Need R

pair2OfNZP-CSI-RS-r17                NZP-CSI-RS-Pairing-r17                                                 OPTIONAL   -- Need R

}

NZP-CSI-RS-Pairing-r17  ::=          SEQUENCE {

nzp-CSI-RS-ResourceId1-r17           INTEGER (1..7),

nzp-CSI-RS-ResourceId2-r17           INTEGER (1..7)

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

NZP-CSI-RS-ResourceSet field descriptions

aperiodicTriggeringOffset, aperiodicTriggeringOffset-r16, aperiodicTriggeringOffset-r17 Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. For aperiodicTriggeringOffset, the value 0 corresponds to 0 slots, value 1 corresponds to 1 slot, value 2 corresponds to 2 slots, value 3 corresponds to 3 slots, value 4 corresponds to 4 slots, value 5 corresponds to 16 slots, value 6 corresponds to 24 slots. For aperiodicTriggeringOffset-r16 and aperiodicTriggeringOffset-r17, the value indicates the number of slots. aperiodicTriggeringOffset-r17 is applicable to SCS 480 kHz and 960 kHz, and only the values of integer multiples of 4 are valid, i.e. 0, 4, 8, and so on. The network configures only one of the fields. When neither field is included, the UE applies the value 0. This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

aperiodicTriggeringOffsetL2 Indicates triggering offset of aperiodic NZP CSI-RS resources used for fast activation of the SCell (see clause 5.2.1.5.3 of TS 38.214 [19]), when the NZP CSI-RS resources are activated by the MAC CE (see clause 5.9 of TS 38.321 [3]). The value indicates the number of slots. This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

cmrGroupingAndPairing Configures CMR groups and pairs. The first nrofResourcesGroup1 resources in the NZP-CSI-RS resource set belong to Group 1 and the remaining resources in the NZP-CSI-RS resource set belong to Group 2. nrofResourcesGroup1 is K1 and the number of remaining resources in the NZP-CSI-RS resource set belonging to Group 2 is K2 as specified in TS 38.214 clause 5.2.1.4.1. Maximum total number in Group 1 and Group 2 is 8 (see TS 38.214 [19], clauses 5.2.1.4.1 and 5.2.1.4.2). This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

pair1OfNZP-CSI-RS, pair2OfNZP-CSI-RS A pair of NZP CSI-RS resources. In one pair, one resource shall belong to group 1 and the other resource shall belong to group 2 (see TS 38.214 [19], clause 5.2.1.4.1).

nzp-CSI-RS-Resources NZP-CSI-RS-Resources associated with this NZP-CSI-RS resource set (see TS 38.214 [19], clause 5.2). For CSI, there are at most 8 NZP CSI RS resources per resource set. If the NZP-CSI-RS-ResourceSet is indicated in a CSI-ResourceConfig indicated by resourcesForChannelMeasurement in a CSI-ReportConfig, if the codebookType in the codebookConfig in the CSI-ReportConfig is typeII-Doppler-r18 or typeII-DopplerPortSelection-r18, there are at most 1 periodic or semi-persistent NZP CSI-RS resource and 4, 8 or 12 aperiodic NZP CSI-RS resources and if the codebookType is typeII-CJT-r18 or typeII-CJT-PortSelection-r18, there are 1, 2, 3, or 4 aperiodic, periodic or semi-persistent NZP-CSI-RS resources, see TS 38.214 5.2.1.4.

nzp-CSI-RS-ResourceId1, nzp-CSI-RS-ResourceId2 The nzp-CSI-RS-ResourceId1-r17 represents the index of the NZP CSI-RS resource in Resource Group 1, and nzp-CSI-RS-ResourceId2-r17 represents the index of the NZP CSI-RS resource in Resource Group 2.

pdc-Info Indicates that this NZP-CSI-RS-ResourceSet, if configured also with trs-Info, is used for propagation delay compensation. The field can be present only if trs-info is present. The field can be present in only one NZP-CSI-RS-ResourceSet. If network configures this field for an NZP-CSI-RS-ResourceSet, the UE measures the UE Rx-Tx time difference based on resources configured in this resource set. This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

repetition Indicates whether repetition is on/off. If the field is set to off or if the field is absent, the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter (see TS 38.214 [19], clauses 5.2.2.3.1 and 5.1.6.1.2). It can only be configured for CSI-RS resource sets which are associated with CSI-ReportConfig with report of L1 RSRP, L1 SINR or "no report". This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2).

trs-Info Indicates that the antenna port for all NZP-CSI-RS resources in the CSI-RS resource set is same. If the field is absent or released the UE applies the value false (see TS 38.214 [19], clause 5.2.2.3.1).

| Conditional Presence | Explanation |
| --- | --- |
| LTM | The field is optionally present, Need R, in an LTM-Candidate IE. Otherwise, the field is absent. |

#### –	NZP-CSI-RS-ResourceSetId

The IE NZP-CSI-RS-ResourceSetId is used to identify one NZP-CSI-RS-ResourceSet.

NZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESETID-START

NZP-CSI-RS-ResourceSetId ::=        INTEGER (0..maxNrofNZP-CSI-RS-ResourceSets-1)

-- TAG-NZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

#### –	P-Max

The IE P-Max is used to limit the UE's uplink transmission power on a carrier frequency, in TS 38.101-1 [15] and in TS 38.101-5 [75], and is used to calculate the parameter Pcompensation defined in TS 38.304 [20]. In ATG cell, actual value of P-Max = 9 + field value [dBm].

P-Max information element

-- ASN1START

-- TAG-P-MAX-START

P-Max ::=                           INTEGER (-30..33)

-- TAG-P-MAX-STOP

-- ASN1STOP

#### –	PathlossReferenceRS

The IE PathlossReferenceRS is used to configure a Reference Signal (e.g. a CSI-RS config or a SS block) to be used for path loss estimation for PUSCH, PUCCH and SRS for unified TCI state operation.

PathlossReferenceRS information element

-- ASN1START

-- TAG-PATHLOSSREFERENCERS-START

PathlossReferenceRS-r17 ::=   SEQUENCE {

pathlossReferenceRS-Id-r17          PathlossReferenceRS-Id-r17,

referenceSignal-r17                 CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

additionalPCI-r17                   AdditionalPCIIndex-r17                                         OPTIONAL  -- Cond RS-SSB

}

-- TAG-PATHLOSSREFERENCERS-STOP

-- ASN1STOP

PathlossReferenceRS field descriptions

additionalPCI Indicates the physical cell ID (PCI) of the SSB for the referenceSignal.

| Conditional Presence | Explanation |
| --- | --- |
| RS-SSB | The field is optionally present, Need R, if ssb-Index is configured for referenceSignal. Otherwise it is absent, Need R. |

#### –	PathlossReferenceRS-Id

The IE PathlossReferenceRS-Id is an ID for a reference signal (RS) configured as PUSCH, PUCCH and SRS pathloss reference RS for unified TCI state operation.

PathlossReferenceRS-Id information element

-- ASN1START

-- TAG-PATHLOSSREFERENCERS-ID-START

PathlossReferenceRS-Id-r17 ::= INTEGER (0..maxNrofPathlossReferenceRSs-1-r17)

-- TAG-PATHLOSSREFERENCERS-ID-STOP

-- ASN1STOP

#### –	PCI-ARFCN-EUTRA

The IE PCI-ARFCN-EUTRA is used to encode EUTRA PCI and ARFCN.

PCI-ARFCN-EUTRA information element

-- ASN1START

-- TAG-PCIARFCNEUTRA-START

PCI-ARFCN-EUTRA-r16 ::=             SEQUENCE {

physCellId-r16                      EUTRA-PhysCellId,

carrierFreq-r16                     ARFCN-ValueEUTRA

}

-- TAG-PCIARFCNEUTRA-STOP

-- ASN1STOP

#### –	PCI-ARFCN-NR

The IE PCI-ARFCN-NR is used to encode NR PCI and ARFCN.

PCI-ARFCN-NR information element

-- ASN1START

-- TAG-PCIARFCNNR-START

PCI-ARFCN-NR-r16 ::=                SEQUENCE {

physCellId-r16                      PhysCellId,

carrierFreq-r16                     ARFCN-ValueNR

}

-- TAG-PCIARFCNNR-STOP

-- ASN1STOP

#### –	PCI-List

The IE PCI-List concerns a list of physical cell identities, which may be used for different purposes.

PCI-List information element

-- ASN1START

-- TAG-PCI-LIST-START

PCI-List ::=                        SEQUENCE (SIZE (1..maxNrofCellMeas)) OF PhysCellId

-- TAG-PCI-LIST-STOP

-- ASN1STOP

#### –	PCI-Range

The IE PCI-Range is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of PCI-Range, the Network may configure overlapping ranges of physical cell identities.

PCI-Range information element

-- ASN1START

-- TAG-PCI-RANGE-START

PCI-Range ::=                       SEQUENCE {

start                               PhysCellId,

range                               ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, n1008,spare1}                  OPTIONAL    -- Need S

}

-- TAG-PCI-RANGE-STOP

-- ASN1STOP

PCI-Range field descriptions

range Indicates the number of physical cell identities in the range (including start). Value n4 corresponds with 4, value n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies.

start Indicates the lowest physical cell identity in the range.

#### –	PCI-RangeElement

The IE PCI-RangeElement is used to define a PCI-Range as part of a list (e.g. AddMod list).

PCI-RangeElement information element

-- ASN1START

-- TAG-PCI-RANGEELEMENT-START

PCI-RangeElement ::=                SEQUENCE {

pci-RangeIndex                      PCI-RangeIndex,

pci-Range                           PCI-Range

}

-- TAG-PCI-RANGEELEMENT-STOP

-- ASN1STOP

PCI-RangeElement field descriptions

pci-Range Physical cell identity or a range of physical cell identities.

#### –	PCI-RangeIndex

The IE PCI-RangeIndex identifies a physical cell id range, which may be used for different purposes.

PCI-RangeIndex information element

-- ASN1START

-- TAG-PCI-RANGEINDEX-START

PCI-RangeIndex ::=                  INTEGER (1..maxNrofPCI-Ranges)

-- TAG-PCI-RANGEINDEX-STOP

-- ASN1STOP

#### –	PCI-RangeIndexList

The IE PCI-RangeIndexList concerns a list of indexes of physical cell id ranges, which may be used for different purposes.

PCI-RangeIndexList information element

-- ASN1START

-- TAG-PCI-RANGEINDEXLIST-START

PCI-RangeIndexList ::=              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeIndex

-- TAG-PCI-RANGEINDEXLIST-STOP

-- ASN1STOP

#### –	PDCCH-Config

The IE PDCCH-Config is used to configure UE specific PDCCH parameters or MBS multicast PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH. If this IE is used for the scheduled SCell in case of cross carrier scheduling, the fields other than searchSpacesToAddModList and searchSpacesToReleaseList are absent. If the IE is used for a dormant BWP, the fields other than controlResourceSetToAddModList and controlResourceSetToReleaseList are absent. If this IE is used for MBS CFR, the field downlinkPreemptiom,tpc-PUSCH, tpc-SRS, uplinkCancellation, monitoringCapabilityConfig, and searchSpaceSwitchConfig are absent.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                      OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId                    OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                            OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                          OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                              OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                            OPTIONAL,   -- Need M

...,

[[

controlResourceSetToAddModListSizeExt-v1610 SEQUENCE (SIZE (1..2)) OF ControlResourceSet             OPTIONAL,   -- Need N

controlResourceSetToReleaseListSizeExt-r16 SEQUENCE (SIZE (1..5)) OF ControlResourceSetId-r16        OPTIONAL,   -- Need N

searchSpacesToAddModListExt-r16     SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16                     OPTIONAL,   -- Need N

uplinkCancellation-r16              SetupRelease { UplinkCancellation-r16 }                          OPTIONAL,   -- Need M

monitoringCapabilityConfig-r16      ENUMERATED { r15monitoringcapability,r16monitoringcapability }   OPTIONAL,   -- Need M

searchSpaceSwitchConfig-r16         SearchSpaceSwitchConfig-r16                                      OPTIONAL    -- Need R

]],

[[

searchSpacesToAddModListExt-v1700   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700                   OPTIONAL,   -- Need N

monitoringCapabilityConfig-v1710    ENUMERATED { r17monitoringcapability }                           OPTIONAL,   -- Need M

searchSpaceSwitchConfig-r17         SearchSpaceSwitchConfig-r17                                      OPTIONAL,   -- Need R

pdcch-SkippingDurationList-r17      SEQUENCE(SIZE (1..3)) OF SCS-SpecificDuration-r17                OPTIONAL    -- Need R

]],

[[

pdcch-MonitoringResumptionAfterNack-r18 ENUMERATED {true}                                            OPTIONAL,   -- Need R

searchSpacesToAddModListExt-v1800   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1800                   OPTIONAL    -- Need N

]]

}

SearchSpaceSwitchConfig-r16 ::=     SEQUENCE {

cellGroupsForSwitchList-r16         SEQUENCE(SIZE (1..4)) OF CellGroupForSwitch-r16                  OPTIONAL,   -- Need R

searchSpaceSwitchDelay-r16          INTEGER (10..52)                                                 OPTIONAL    -- Need R

}

SearchSpaceSwitchConfig-r17 ::=     SEQUENCE {

searchSpaceSwitchTimer-r17          SCS-SpecificDuration-r17                                         OPTIONAL,   -- Need R

searchSpaceSwitchDelay-r17          INTEGER (10..52)                                                 OPTIONAL    -- Need R

}

CellGroupForSwitch-r16 ::=          SEQUENCE(SIZE (1..16)) OF ServCellIndex

SCS-SpecificDuration-r17   ::=      INTEGER (1..166)

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

PDCCH-Config field descriptions

controlResourceSetToAddModList, controlResourceSetToAddModListSizeExt List of UE specifically configured Control Resource Sets (CORESETs) to be used by the UE. The network restrictions on configuration of CORESETs per DL BWP are specified in TS 38.213 [13], clause 10.1 and TS 38.306 [26]. The UE shall consider entries in controlResourceSetToAddModList and in controlResourceSetToAddModListSizeExt as a single list, i.e. an entry created using controlResourceSetToAddModList can be modified using controlResourceSetToAddModListSizeExt (or deleted using controlResourceSetToReleaseListSizeExt) and vice-versa. In case network reconfigures control resource set with the same ControlResourceSetId as used for commonControlResourceSet or commonControlResourceSetExt configured via PDCCH-ConfigCommon or via SIB20, the configuration from PDCCH-Config always takes precedence and should not be updated by the UE based on servingCellConfigCommon or based on SIB20.

controlResourceSetToReleaseList, controlResourceSetToReleaseListSizeExt List of UE specifically configured Control Resource Sets (CORESETs) to be released by the UE. This field only applies to CORESETs configured by controlResourceSetToAddModList or controlResourceSetToAddModListSizeExt and does not release the field commonControlResourceSet configured by PDCCH-ConfigCommon and commonControlResourceSetExt configured by SIB20.

downlinkPreemption Configuration of downlink preemption indications to be monitored in this cell (see TS 38.213 [13], clause 11.2).

monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability, Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapability enables the Rel-15 monitoring capability, and value r16monitoringcapability enables the Rel-16 PDCCH monitoring capability. r17monitoringcapability enables the Rel-17 PDCCH multi-slot monitoring capability. For 480 and 960 kHz SCS, only value r17monitoringcapability is applicable.

pdcch-MonitoringResumptionAfterNack This field is used to enable UE PDCCH monitoring resumption after PDCCH skipping has started, if the UE transmits a NACK due to incorrectly decoding a PDSCH scheduled by a DCI format received from the serving cell (see TS 38.213 [13], clause 10.4).

pdcch-SkippingDurationList Provides one or more values to derive the skipping duration in unit of slots, as specified in TS 38.213 [13], clause 10.4. The DCI which schedules data indicates which of the values is to be applied (see TS 38.213 [13], clause 10.4). For the 15kHz SCS, for each entry, only the first 26 values are valid and correspond to {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100}. For the 30kHz SCS, for each entry, only the first 46 values are valid and correspond to {1, 2, 3, …, 40, 60, 80, 100, 120, 160, 200}. For the 60kHz SCS, for each entry, only the first 86 values are valid and correspond to {1, 2, 3, …, 80, 120, 160, 200, 240, 320, 400}. For the 120kHz SCS, for each entry, the 166 values correspond to {1, 2, 3, …, 160, 240, 320, 400, 480, 640, 800}. For the 480kHz SCS, for each entry, the 166 values correspond to {4, 8, 12, …, 640, 960, 1280, 1600, 1920, 2560, 3200}. For the 960kHz SCS, for each entry, the 166 values correspond to {8, 16, 24, …, 1280, 1920, 2560, 3200, 3840, 5120, 6400}.

searchSpacesToAddModList, searchSpacesToAddModListExt List of UE specifically configured Search Spaces or MBS multicast Search Spaces. The network configures at most 10 Search Spaces per BWP per cell (including UE-specific and common Search Spaces). If the network includes searchSpacesToAddModListExt, it includes the same number of entries, and listed in the same order, as in searchSpacesToAddModList in each of them.

searchSpaceSwitchConfig Configuration to control the UE behavior to switch from search space group X back to search space group 0, as specified in clause 10 of TS 38.213 [13]. The network only configures either searchSpaceSwitchConfig-r16 or searchSpaceSwitchConfig-r17 for a UE.

tpc-PUCCH Enable and configure reception of group TPC commands for PUCCH.

tpc-PUSCH Enable and configure reception of group TPC commands for PUSCH.

tpc-SRS Enable and configure reception of group TPC commands for SRS.

uplinkCancellation Configuration of uplink cancellation indications to be monitored in this cell (see TS 38.213 [13], clause 11.2A).

SearchSpaceSwitchConfig field descriptions

cellGroupsForSwitchList The list of serving cells which are bundled for the search space group switching purpose (see TS 38.213 [13], clause 10.4). A serving cell can belong to only one CellGroupForSwitch. The network configures the same list for all BWPs of serving cells in the same CellGroupForSwitch.

searchSpaceSwitchDelay Indicates the value to be applied by a UE for Search Space Set Group switching; corresponds to the P value in TS 38.213 [13], clause 10.4. The network configures the same value for all BWPs of serving cells in the same CellGroupForSwitch. For 120/480/960 kHz SCS, only values 40,41, ... 52 are valid and the actual value = field value * SCS/120 kHz i.e. field value 40 corresponds to 40 with 120 kHz SCS, 160 with 480 kHz SCS and 320 with 960 kHz SCS, and so on.

searchSpaceSwitchTimer Timer (in unit of slots) to control the UE behavior to switch from search space group X back to search space group 0, as specified in clause 10 of TS 38.213 [13]. For the 15kHz SCS, only the first 26 values are valid and correspond to {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100}. For the 30kHz SCS, only the first 46 values are valid and correspond to {1, 2, 3, …, 40, 60, 80, 100, 120, 160, 200}. For the 60kHz SCS, only the first 86 values are valid and correspond to {1, 2, 3, …, 80, 120, 160, 200, 240, 320, 400}. For the 120kHz SCS, the 166 values correspond to {1, 2, 3, …, 160, 240, 320, 400, 480, 640, 800}. For the 480kHz SCS, the 166 values correspond to {4, 8, 12, …, 640, 960, 1280, 1600, 1920, 2560, 3200}. For the 960kHz SCS, the 166 values correspond to {8, 16, 24, …, 1280, 1920, 2560, 3200, 3840, 5120, 6400}.

#### –	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as in dedicated signalling.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

commonSearchSpaceList               SEQUENCE (SIZE(1..4)) OF SearchSpace                    OPTIONAL,   -- Need R

searchSpaceSIB1                     SearchSpaceId                                           OPTIONAL,   -- Need S

searchSpaceOtherSystemInformation   SearchSpaceId                                           OPTIONAL,   -- Need S

pagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

ra-SearchSpace                      SearchSpaceId                                           OPTIONAL,   -- Need S

...,

[[

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT          SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}                                                                                           OPTIONAL    -- Cond OtherBWP

]],

[[

commonSearchSpaceListExt-r16                                             SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-r16     OPTIONAL  -- Need R

]],

[[

sdt-SearchSpace-r17                 CHOICE {

newSearchSpace                      SearchSpace,

existingSearchSpace                 SearchSpaceId

}                                                                                           OPTIONAL,   -- Need R

searchSpaceMCCH-r17                 SearchSpaceId                                           OPTIONAL,   -- Need R

searchSpaceMTCH-r17                 SearchSpaceId                                           OPTIONAL,   -- Need S

commonSearchSpaceListExt2-r17       SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1700           OPTIONAL,   -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE {

sCS480KHZoneEighthT                      SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                           OPTIONAL,   -- Need R

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}                                                                                           OPTIONAL     -- Cond InitialBWP-Paging

]],

[[

followUnifiedTCI-State-v1720           ENUMERATED {enabled}                                  OPTIONAL     -- Need R

]],

[[

applyIndicatedTCI-State-r18            ENUMERATED {first, second, both, none}                OPTIONAL,   -- Cond FollowUTCI

commonSearchSpaceListExt-r18           SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1800         OPTIONAL,   -- Need R

searchSpaceMulticastMCCH-r18           SearchSpaceId                                         OPTIONAL,   -- Need R

searchSpaceMulticastMTCH-r18           SearchSpaceId                                         OPTIONAL    -- Need S

]]

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

applyIndicatedTCI-State This field indicates, for PDCCH reception in CORESET #0, if UE applies the first, the second, both or none of the "indicated" DL only TCI or joint TCI as specified in TS 38.213 [13], clause 10.1.

commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in the (e)RedCap-specific initial downlink BWP in SIB1 for (e)RedCap such that it does not have to be contained in the bandwidth of CORESET#0.

commonSearchSpaceList, commonSearchSpaceListExt, commonSearchSpaceListExt2 A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0. If the field is included, it replaces any previous list, i.e. all the entries of the list are replaced and each of the SearchSpace entries is considered to be newly created and the conditions and Need codes for setup of the entry apply. If the network includes commonSearchSpaceListExt/commonSearchSpaceListExt2, it includes the same number of entries, and listed in the same order, as in commonSearchSpaceList.

controlResourceSetZero Parameters of the common CORESET#0 which can be used in any common or UE-specific search spaces. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) controlResourceSetZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions defined in TS 38.213 [13], clause 10 are satisfied.

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

firstPDCCH-MonitoringOccasionOfPO Indicates the first PDCCH monitoring occasion of each PO of the PF on this BWP, see TS 38.304 [20]. The field sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS120KHZoneSixteenthT can be applied for SCS 480kHz, corresponding to sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS480KHZquarterT-SCS120KHZoneSixteenthT in IE DownlinkConfigCommonSIB respectively.

followUnifiedTCI-State When set to enabled, for PDCCH reception in CORESET #0, the UE applies the "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5.

pagingSearchSpace ID of the search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

pei-ConfigBWP Provides the configuration for PEI reception in this BWP. If the field is absent, the UE does not receive PEI in this BWP. For the initialDownlinkBWP-RedCap not including CD-SSB and the entire CORESET#0, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing monitors PEI in the initialDownlinkBWP that includes CORESET#0, if the initialDownlinkBWP is configured with pei-ConfigBWP.

pei-SearchSpace ID of dedicated search space for PEI. It can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0. The CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level follows Table 10.1-1 of TS38.213 [13]. SearchSpaceId = 0 can be configured for the case of SS/PBCH block and CORESET multiplexing pattern 2 or 3.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], clause 5.15 are met.

sdt-SearchSpace Common search space for CG-SDT and RA-SDT (see TS 38.213 [13]). If an existingSearchSpace is used, the network only signals the search space ID of the ra-SearchSpace.

searchSpaceMCCH ID of the search space for MCCH. If the field is absent, the UE does not receive MCCH in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0.

searchSpaceMTCH ID of the search space for MTCH of MBS broadcast. If the field is absent, the UE applies searchSpaceMCCH also for MTCH, (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0.

searchSpaceMulticastMCCH ID of the search space for multicast MCCH. If the field is absent, the UE does not receive multicast MCCH in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0.

searchSpaceMulticastMTCH ID of the search space for multicast MTCH. If the field is absent, the UE applies searchSpaceMulticastMCCH also for multicast MTCH, (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive other system information in this BWP. This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive other system information using searchSpaceOtherSystemInformation in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive SIB1 using searchSpaceSIB1 in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceZero Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0), searchSpaceZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions described in TS 38.213 [13], clause 10, are satisfied.

| Conditional Presence | Explanation |
| --- | --- |
| FollowUTCI | The field is absent if the field followUnifiedTCI-State is present. Otherwise, it is optionally present, Need R. |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon except it is the RedCap-specific initial BWP not including CD-SSB and the entire CORESET#0 in which case the field is absent, Need R; it is absent in other BWPs and when sent in system information. If SIB1 is not broadcast and there is an SSB associated to the cell, the field is optionally present, Need M, in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon (still with the same setting for all UEs). In other cases, the field is absent. |
| InitialBWP-Paging | This field is optionally present, Need R, if this BWP is the initialDownlinkBWP or initialDownlinkBWP-RedCap including CD-SSB and the entire CORESET#0, and pei-Config is configured in DownlinkConfigCommonSIB. Otherwise, this field is absent. |
| OtherBWP | This field is optionally present, Need R, if this BWP is not the initialDownlinkBWP and pagingSearchSpace is configured in this BWP. Otherwise this field is absent. |

#### –	PDCCH-ConfigSIB1

The IE PDCCH-ConfigSIB1 is used to configure CORESET#0 and search space#0.

PDCCH-ConfigSIB1 information element

-- ASN1START

-- TAG-PDCCH-CONFIGSIB1-START

PDCCH-ConfigSIB1 ::=                SEQUENCE {

controlResourceSetZero              ControlResourceSetZero,

searchSpaceZero                     SearchSpaceZero

}

-- TAG-PDCCH-CONFIGSIB1-STOP

-- ASN1STOP

PDCCH-ConfigSIB1 field descriptions

controlResourceSetZero Determines a common ControlResourceSet (CORESET) with ID #0, see TS 38.213 [13], clause 13.

searchSpaceZero Determines a common search space with ID #0, see TS 38.213 [13], clause 13.

#### –	PDCCH-ServingCellConfig

The IE PDCCH-ServingCellConfig is used to configure UE specific PDCCH parameters applicable across all bandwidth parts of a serving cell.

PDCCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDCCH-SERVINGCELLCONFIG-START

PDCCH-ServingCellConfig ::=         SEQUENCE {

slotFormatIndicator                 SetupRelease { SlotFormatIndicator }                                OPTIONAL,   -- Need M

...,

[[

availabilityIndicator-r16           SetupRelease {AvailabilityIndicator-r16}                            OPTIONAL,   -- Need M

searchSpaceSwitchTimer-r16          INTEGER (1..80)                                                     OPTIONAL    -- Need R

]],

[[

searchSpaceSwitchTimer-v1710        INTEGER (81..1280)                                                  OPTIONAL    -- Need R

]]

}

-- TAG-PDCCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDCCH-ServingCellConfig field descriptions

availabilityIndicator Use to configure monitoring a PDCCH for Availability Indicators (AI).

searchSpaceSwitchTimer The value of the timer in slots for monitoring PDCCH in the active DL BWP of the serving cell before moving to the default search space group (see TS 38.213 [13], clause 10.4). For 15 kHz SCS, {1..20} are valid. For 30 kHz SCS, {1..40} are valid. For 60kHz SCS, {1..80} are valid. For 120 kHz SCS, {1..160} are valid. For 480 kHz SCS, {1..640} are valid. For 960 kHz SCS, {1..1280} are valid. The network configures the same value for all serving cells in the same CellGroupForSwitch.

slotFormatIndicator Configuration of Slot-Format-Indicators to be monitored in the correspondingly configured PDCCHs of this serving cell.

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling, MBS multicast and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup1

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

headerCompression       CHOICE {

notUsed                 NULL,

rohc                    SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0001           BOOLEAN,

profile0x0002           BOOLEAN,

profile0x0003           BOOLEAN,

profile0x0004           BOOLEAN,

profile0x0006           BOOLEAN,

profile0x0101           BOOLEAN,

profile0x0102           BOOLEAN,

profile0x0103           BOOLEAN,

profile0x0104           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

uplinkOnlyROHC          SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0006           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

...

},

integrityProtection     ENUMERATED { enabled }                                          OPTIONAL,   -- Cond ConnectedTo5GC1

statusReportRequired    ENUMERATED { true }                                             OPTIONAL,   -- Cond Rlc-AM-UM

outOfOrderDelivery      ENUMERATED { true }                                             OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                 OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                      OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                           OPTIONAL,   -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                     OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond MoreThanOneRLC

t-Reordering                ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40,

ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250,

ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24,

spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14,

spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03,

spare02, spare01 }                                          OPTIONAL, -- Need S

...,

[[

cipheringDisabled       ENUMERATED {true}                                                   OPTIONAL    -- Cond ConnectedTo5GC

]],

[[

discardTimerExt-r16     SetupRelease { DiscardTimerExt-r16 }                                OPTIONAL,    -- Cond DRB2

moreThanTwoRLC-DRB-r16  SEQUENCE {

splitSecondaryPath-r16  LogicalChannelIdentity                                          OPTIONAL,   -- Cond SplitBearer2

duplicationState-r16    SEQUENCE (SIZE (3)) OF BOOLEAN                                  OPTIONAL    -- Need S

}                                                                                           OPTIONAL,   -- Cond MoreThanTwoRLC-DRB

ethernetHeaderCompression-r16  SetupRelease { EthernetHeaderCompression-r16 }               OPTIONAL    -- Need M

]],

[[

survivalTimeStateSupport-r17   ENUMERATED {true}                                            OPTIONAL,   -- Cond Drb-Duplication

uplinkDataCompression-r17      SetupRelease { UplinkDataCompression-r17 }                   OPTIONAL,   -- Cond Rlc-AM

discardTimerExt2-r17           SetupRelease { DiscardTimerExt2-r17 }                        OPTIONAL,   -- Need M

initialRX-DELIV-r17            BIT STRING (SIZE (32))                                       OPTIONAL    -- Cond MRB-Initialization

]],

[[

pdu-SetDiscard-r18             ENUMERATED {true}                                            OPTIONAL,   -- Need R

discardTimerForLowImportance-r18   SetupRelease { DiscardTimerForLowImportance-r18 }        OPTIONAL,   -- Cond DRB2

primaryPathOnIndirectPath-r18  ENUMERATED {true}                                            OPTIONAL    -- Cond SplitBearerMP

]]

}

EthernetHeaderCompression-r16 ::=  SEQUENCE {

ehc-Common-r16                     SEQUENCE {

ehc-CID-Length-r16                 ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink-r16               SEQUENCE {

drb-ContinueEHC-DL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL,   -- Need M

ehc-Uplink-r16                 SEQUENCE {

maxCID-EHC-UL-r16              INTEGER (1..32767),

drb-ContinueEHC-UL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL    -- Need M

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

DiscardTimerExt2-r17 ::= ENUMERATED {ms2000, spare3, spare2, spare1}

UplinkDataCompression-r17 ::= CHOICE {

newSetup                      SEQUENCE {

bufferSize-r17                ENUMERATED {kbyte2, kbyte4, kbyte8, spare1},

dictionary-r17                ENUMERATED {sip-SDP, operator}                            OPTIONAL    -- Need N

},

drb-ContinueUDC           NULL

}

DiscardTimerForLowImportance-r18 ::= ENUMERATED {ms0, ms2, ms4, ms6, ms8, ms10, ms12, ms14, ms18, ms22, ms26, ms30, ms40, ms50, ms75, ms100}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms and so on. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

discardTimerExt Value in ms of discardTimer specified in TS 38.323 [5]. Value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1ms and so on. If this field is present, the field discardTimer is ignored and discardTimerExt is used instead.

discardTimerExt2 Value in ms of discardTimerExt specified in TS 38.323 [5]. Value ms2000 corresponds to 2000 ms. If this field is present, the field discardTimer and discardTimerExt are ignored and discardTimerExt2 is used instead.

discardTimerForLowImportance Value in ms of discardTimerForLowImportance specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms2 corresponds to 2 ms and so on. The value of this timer for a PDCP entity is always configured shorter than discardTimer, discardTimerExt or discardTimerExt2, whichever is used for the PDCP entity. The presence of this field indicates that PSI-based SDU discard is configured for a DRB. The network configures at most 8 DRBs with discardTimerForLowImportance.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer. This field can be configured for both DRB and multicast MRB.

duplicationState This field indicates the uplink PDCP duplication state for the associated RLC entities at the time of receiving this IE. If set to true, the PDCP duplication state is activated for the associated RLC entity. The index for the indication is determined by ascending order of logical channel ID of all RLC entities other than the primary RLC entity indicated by primaryPath in the order of MCG and SCG, as in clause 6.1.3.32 of TS 38.321 [3]. For MP, the index for the indication is determined by ascending order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path, as in clause 6.1.3.32 of TS 38.321 [3]. If the number of associated RLC entities other than the primary RLC entity is two, UE ignores the value in the largest index of this field. If the field is absent, the PDCP duplication states are deactivated for all associated RLC entities.

ethernetHeaderCompression This fields configures Ethernet Header Compression. This field can only be configured for a bi-directional DRB or a bi-directional multicast MRB. The network reconfigures ethernetHeaderCompression only upon reconfiguration involving PDCP re-establishment and with neither drb-ContinueEHC-DL nor drb-ContinueEHC-UL configured. Network only configures this field when uplinkDataCompression is not configured.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC and EHC can be both configured simultaneously for a DRB or a multicast MRB. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment or involving PDCP entity reconfiguration to configure DAPS bearer(s), and without any drb-ContinueROHC. Network configures headerCompression to notUsed when outOfOrderDelivery is configured. Network only configures this field when uplinkDataCompression is not configured.

initialRX-DELIV Indicates the initial value of RX_DELIV during PDCP window initialization for multicast MRB as specified in TS 38.323 [5].

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

moreThanOneRLC This field configures UL data transmission when more than one RLC entity is associated with the PDCP entity. This field is not present if the bearer is configured as DAPS bearer.

moreThanTwoRLC-DRB This field configures UL data transmission when more than two RLC entities are associated with the PDCP entity for DRBs.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. For PDCP entity with more than two associated RLC entities for UL transmission, this field is always present. If the field moreThanTwoRLC-DRB is present, the value of this field is ignored and the state of the duplication is indicated by duplicationState. For PDCP entity with more than two associated RLC entities, only NR RLC bearer is supported.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdu-SetDiscard If set to true, the UE shall perform PDU Set based discarding for this PDCP entity, as specified in TS 38.323 [5]. This field is only configured for a DRB.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs, except for the split SRB2 of the IAB-MT, and except when the UE is required to set the primaryPath to refer to the SCG as specified in clause 5.7.3b.4. In this last case, if the network sends an RRCReconfiguration message (in NR-DC) or an EUTRA RRCConnectionReconfiguration message (in (NG)EN-DC) keeping SRB1 as split SRB, the network explicitly configures the primaryPath for the PDCP entity of SRB1 to refer to the MCG. In this version of the specification, only cell group ID corresponding to MCG is supported for DRBs when the SCG is deactivated. In MR-DC, the NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW always indicates logicalChannel if CA based PDCP duplication is configured in the cell group indicated by cellGroup of this field. In MP, when the primay path is set to indirect path, the field cellGroup and logicalChannel are absent, and the field primaryPathOnIndirectPath is set to true.

primaryPathOnIndirectPath Indicates that the primary RLC entity is on SL indirect path, or primary path is associated with the N3C indirect path, for DRB when MP is configured.

splitSecondaryPath Indicates the LCID of the split secondary RLC entity as specified in TS 38.323 [5] for fallback to split bearer operation when UL data transmission with more than two RLC entities is associated with the PDCP entity. This RLC entity belongs to a cell group that is different from the cell group indicated by cellGroup in the field primaryPath.

statusReportRequired For AM DRBs, AM MRBs and DAPS UM DRBs, indicates whether the DRB or the multicast MRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For DAPS AM DRBs, it also indicates whether the DRB is configured to send a second PDCP status report in the uplink, as specified in TS 38.323 [5].

survivalTimeStateSupport Indicates whether the DRB associated with this PDCP entity has survival time state support. If this field is configured to be true, all associated RLC entities are activated for PDCP duplication upon reception of a retransmission grant addressed to CS-RNTI, as specified in TS 38.321 [3].

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms20 corresponds to 20 ms, value ms40 corresponds to 40 ms, and so on. When the field is absent the UE applies the value infinity. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bytes, value b100 corresponds to 100 bytes, value b200 corresponds to 200 bytes, and so on. The network sets this field to infinity for UEs not supporting splitDRB-withUL-Both-MCG-SCG and when the SCG is deactivated. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied.

uplinkDataCompression Indicates the UDC configuration that the UE shall apply. Network does not configure uplinkDataCompression for a DRB, if headerCompression or ethernetHeaderCompression is already configured or outOfOrderDelivery or DAPS is configured for the DRB. The maximum number of DRBs where uplinkDataCompression can be applied is two. The network reconfigures uplinkDataCompression only upon reconfiguration involving PDCP re-establishment. If the field is set to drb-ContinueUDC, the PDCP entity continues the uplink data compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is set to drb-ContinueUDC only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

EthernetHeaderCompression field descriptions

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

ehc-CID-Length Indicates the length of the CID field for EHC packet. The value bits7 indicates the length is 7 bits, and the value bits15 indicates the length is 15 bits. Once the field ethernetHeaderCompression-r16 is configured for a DRB or a multicast MRB, the value of the field ehc-CID-Length for this DRB or multicast MRB is not reconfigured to a different value.

ehc-Common Indicates the configurations that apply for both downlink and uplink.

ehc-Downlink Indicates the configurations that apply for only downlink. If the field is configured, then Ethernet header compression is configured for downlink. Otherwise, it is not configured for downlink.

ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplnik. Otherwise, it is not configured for uplink.

maxCID-EHC-UL Indicates the value of the MAX_CID_EHC_UL parameter as specified in TS 38.323 [5]. The total value of MAX_CID_EHC_UL across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE.

UplinkDataCompression field descriptions

bufferSize This field indicates the buffer size applied for UDC as specified in TS 38.323 [5]. Value kbyte2 means 2048 bytes, kbyte4 means 4096 bytes and so on.

dictionary This field indicates which pre-defined dictionary is used for UDC as specified in TS 38.323 [5]. The value sip-SDP means that UE shall prefill the buffer with standard dictionary for SIP and SDP defined in TS 38.323 [5], and the value operator means that UE shall prefill the buffer with operator-defined dictionary.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB/multicast MRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M. |
| DRB2 | This field is optionally present in case of DRB, need M. Otherwise, it is absent for SRBs and MRBs. |
| Drb-Duplication | For SRBs, this field is absent. For DRBs, this field is absent if duplication is not configured. Otherwise, this field is optional, need R. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer (except for multicast MRB) with more than one associated logical channel and upon RRC reconfiguration with the association of additional logical channels to the PDCP entity. The field is also mandatory present in case the field moreThanTwoRLC-DRB is included in PDCP-Config. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent. Need R. |
| MoreThanTwoRLC-DRB | For SRBs, this field is absent. For DRBs, this field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than two associated logical channels and upon RRC reconfiguration with the association of one or more additional logical channel(s) to the PDCP entity so that the PDCP entity has more than two associated logical channels. Upon RRC reconfiguration when a PDCP entity is associated with more than two logical channels, this field is optionally present, Need M. Otherwise, the field is absent, Need R. |
| Rlc-AM | For RLC AM, the field is optionally present, need M. Otherwise, the field is absent. |
| Rlc-AM-UM | In case of DRB, for RLC UM (if the UE supports DAPS handover) or RLC AM, the field is optionally present, need R. In case of multicast MRB, if multicast MRB is associated with at least one RLC AM entity, the field is optionally present, need R. Otherwise, the field is absent. |
| Setup | The field is mandatory present in case of DRB setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is absent for SRBs. Otherwise, the field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. |
| SplitBearer2 | The field is mandatory present, in case of a split bearer. Otherwise the field is absent. |
| SplitBearerMP | The field is absent for SRBs. Otherwise, the field is optionally present, need R, when MP is configured. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| ConnectedTo5GC1 | The field is optionally present, need R, if the UE is connected to NR/5GC or if the UE supports user plane integrity protection when connected to E-UTRA/EPC (as specified in TS 33.401 [30]). Otherwise the field is absent. |
| Setup1 | This field is mandatory present in case of DRB setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| MRB-Initialization | This field is mandatory present in case of multicast MRB setup or in case UE configured with multicast reception in RRC_INACTIVE resumes the RRC connection. In case of PDCP re-establishment for multicast MRB, this field is optionally present, Need N. Otherwise, this field is absent, Need N. |

#### –	PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters. If this IE is used for MBS CFR, the following fields shall be absent: tci-StatesToAddModList, tci-StatesToReleaseList, zp-CSI-RS-ResourceToAddModList, minimumSchedulingOffsetK0, antennaPortsFieldPresenceDCI-1-2, aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2, aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, dmrs-SequenceInitializationDCI-1-2, harq-ProcessNumberSizeDCI-1-2, mcs-TableDCI-1-2, numberOfBitsForRV-DCI-1-2, pdsch-AggregationFactor, pdsch-TimeDomainAllocationListDCI-1-2, prb-BundlingTypeDCI-1-2, priorityIndicatorDCI-1-2, rateMatchPatternGroup1DCI-1-2, rateMatchPatternGroup2DCI-1-2, resourceAllocationType1GranularityDCI-1-2, vrb-ToPRB-InterleaverDCI-1-2, referenceOfSLIVDCI-1-2, resourceAllocationDCI-1-2, dataScramblingIdentityPDSCH2-r16, repetitionSchemeConfig, pdsch-ConfigDCI-1-3.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

dmrs-DownlinkForPDSCH-MappingTypeA      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

tci-StatesToAddModList                  SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State                  OPTIONAL,   -- Need N

tci-StatesToReleaseList                 SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId                OPTIONAL,   -- Need N

vrb-ToPRB-Interleaver                   ENUMERATED {n2, n4}                                                 OPTIONAL,   -- Need S

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList          SetupRelease { PDSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pdsch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

rateMatchPatternToAddModList            SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need N

rateMatchPatternToReleaseList           SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL,   -- Need N

rateMatchPatternGroup1                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rateMatchPatternGroup2                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rbg-Size                                ENUMERATED {config1, config2},

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

maxNrofCodeWordsScheduledByDCI          ENUMERATED {n1, n2}                                                 OPTIONAL,   -- Need R

prb-BundlingType                        CHOICE {

staticBundling                          SEQUENCE {

bundleSize                              ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling                     SEQUENCE {

bundleSizeSet1                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }           OPTIONAL,   -- Need S

bundleSizeSet2                      ENUMERATED { n4, wideband }                                     OPTIONAL    -- Need S

}

},

zp-CSI-RS-ResourceToAddModList                  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource

OPTIONAL,   -- Need N

zp-CSI-RS-ResourceToReleaseList                 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList    SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList   SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...,

[[

maxMIMO-Layers-r16                      SetupRelease { MaxMIMO-LayersDL-r16 }                               OPTIONAL,   -- Need M

minimumSchedulingOffsetK0-r16           SetupRelease { MinSchedulingOffsetK0-Values-r16 }                   OPTIONAL,   -- Need M

-- Start of the parameters for DCI format 1_2 introduced in V16.1.0

antennaPortsFieldPresenceDCI-1-2-r16            ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-SequenceInitializationDCI-1-2-r16          ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

harq-ProcessNumberSizeDCI-1-2-r16               INTEGER (0..4)                                              OPTIONAL,   -- Need R

mcs-TableDCI-1-2-r16                            ENUMERATED {qam256, qam64LowSE}                             OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-1-2-r16                   INTEGER (0..2)                                              OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationListDCI-1-2-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

prb-BundlingTypeDCI-1-2-r16             CHOICE {

staticBundling-r16                      SEQUENCE {

bundleSize-r16                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling-r16                     SEQUENCE {

bundleSizeSet1-r16                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }       OPTIONAL,   -- Need S

bundleSizeSet2-r16                      ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

}

}                                                                                                           OPTIONAL,   -- Need R

priorityIndicatorDCI-1-2-r16                ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

rateMatchPatternGroup1DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

rateMatchPatternGroup2DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

resourceAllocationType1GranularityDCI-1-2-r16  ENUMERATED {n2,n4,n8,n16}                                    OPTIONAL,   -- Need S

vrb-ToPRB-InterleaverDCI-1-2-r16            ENUMERATED {n2, n4}                                             OPTIONAL,   -- Need S

referenceOfSLIVDCI-1-2-r16                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

resourceAllocationDCI-1-2-r16               ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 1_2 introduced in V16.1.0

priorityIndicatorDCI-1-1-r16             ENUMERATED {enabled}                                               OPTIONAL,   -- Need S

dataScramblingIdentityPDSCH2-r16         INTEGER (0..1023)                                                  OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationList-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }        OPTIONAL,   -- Need M

repetitionSchemeConfig-r16               SetupRelease { RepetitionSchemeConfig-r16}                         OPTIONAL    -- Need M

]],

[[

repetitionSchemeConfig-v1630             SetupRelease { RepetitionSchemeConfig-v1630}                       OPTIONAL    -- Need M

]],

[[

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2-r17    ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-1-2-r17           ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2-r17     ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-RetxDCI-1-2-r17               ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pucch-sSCellDynDCI-1-2-r17                   ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

dl-OrJointTCI-StateList-r17                  CHOICE {

explicitlist                                 SEQUENCE {

dl-OrJointTCI-StateToAddModList-r17        SEQUENCE (SIZE (1..maxNrofTCI-States)) OF TCI-State

OPTIONAL,   -- Need N

dl-OrJointTCI-StateToReleaseList-r17       SEQUENCE (SIZE (1..maxNrofTCI-States)) OF TCI-StateId

OPTIONAL    -- Need N

},

unifiedTCI-StateRef-r17                  ServingCellAndBWP-Id-r17

}                                                                                                           OPTIONAL,   -- Need R

beamAppTime-r17                              ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336, spare2,

spare1}                                             OPTIONAL,   -- Need R

dummy                                        SetupRelease { Dummy-TDRA-List }                                    OPTIONAL,   -- Need M

dmrs-FD-OCC-DisabledForRank1-PDSCH-r17          ENUMERATED {true}                                           OPTIONAL,   -- Need R

minimumSchedulingOffsetK0-r17                   SetupRelease { MinSchedulingOffsetK0-Values-r17 }           OPTIONAL,   -- Need M

harq-ProcessNumberSizeDCI-1-2-v1700          INTEGER (0..5)                                                 OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-1-r17            INTEGER (5)                                                    OPTIONAL,   -- Need R

mcs-Table-r17                                ENUMERATED {qam1024}                                           OPTIONAL,   -- Need R

mcs-TableDCI-1-2-r17                         ENUMERATED {qam1024}                                           OPTIONAL,   -- Need R

xOverheadMulticast-r17                       ENUMERATED {xOh6, xOh12, xOh18}                                OPTIONAL,   -- Need S

priorityIndicatorDCI-4-2-r17                 ENUMERATED {enabled}                                           OPTIONAL,   -- Need S

sizeDCI-4-2-r17                              INTEGER (20..maxDCI-4-2-Size-r17)                              OPTIONAL    -- Need R

]],

[[

pdsch-TimeDomainAllocationListForMultiPDSCH-r17 SetupRelease { MultiPDSCH-TDRA-List-r17 }                   OPTIONAL    -- Need M

]],

[[

advancedReceiver-MU-MIMO-r18                 SetupRelease { AdvancedReceiver-MU-MIMO-r18 }                  OPTIONAL,   -- Need M

pdsch-ConfigDCI-1-3-r18                      SetupRelease { PDSCH-ConfigDCI-1-3-r18 }                       OPTIONAL    -- Need M

]]

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

MinSchedulingOffsetK0-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r16)

MinSchedulingOffsetK0-Values-r17 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r17)

MaxMIMO-LayersDL-r16 ::=                INTEGER (1..8)

PDSCH-ConfigDCI-1-3-r18 ::=                    SEQUENCE {

resourceAllocationDCI-1-3-r18                  ENUMERATED {resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

rbg-SizeDCI-1-3-r18                            ENUMERATED {config1, config2, config3, spare1}               OPTIONAL, -- Cond DCI-1-3

resourceAllocationType1GranularityDCI-1-3-r18  ENUMERATED {n2,n4,n8,n16}                                    OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-1-3-r18                  INTEGER (0..2)                                               OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-3-r18              INTEGER (0..5)                                               OPTIONAL    -- Need R

}

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Config field descriptions

advancedReceiver-MU-MIMO A set of assistance information for R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions.

antennaPortsFieldPresenceDCI-1-2 Configure the presence of "Antenna ports" field in DCI format 1_2. When the field is configured, then the "Antenna ports" field is present in DCI format 1_2. Otherwise, the field size is set to 0 for DCI format 1_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 nor dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 is configured, this field is absent.

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList, aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3 (see TS 38.214 [19], clause 5.1.4.2). The field aperiodic-ZP-CSI-RS-ResourceSetsToAddModList applies to DCI format 1_1 and the field aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.2 and TS 38.212 [17] clause 7.3.1).

beamAppTime Indicates the first slot to apply the unified TCI indicated by DCI as specified in TS 38.214 Clause 5.1.5. The value n1 means 1 symbol, n2 two symbols and so on. The first slot is at least Y symbols indicated by beamAppTime parameter after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. The same value shall be configured for all serving cells in any one of the simultaneousU-TCI-UpdateListN configured in IE CellGroupConfig based on the smallest SCS of the active BWP.

dataScramblingIdentityPDSCH, dataScramblingIdentityPDSCH2 Identifier(s) used to initialize data scrambling (c_init) for PDSCH as specified in TS 38.211 [16], clause 7.3.1.1. The dataScramblingIdentityPDSCH2 is configured if coresetPoolIndex is configured with 1 for at least one CORESET in the same BWP.

dl-OrJointTCI-StateToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports, PDCCH DMRS ports, and CSI-RS, and in case of join mode, also the PUSCH, PUCCH and SRS (see TS 38.214 [19], clause 5.1.5).

dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeA applies to DCI formats 1_1 and 1_3, and the field dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeB applies to DCI formats 1_1 and 1_3, and the field dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-FD-OCC-DisabledForRank1-PDSCH If configured, the UE may assume that the set of remaining orthogonal antenna ports, which are within the same code division multiplexing (CDM) group and have different frequency domain orthogonal cover codes (FD-OCC), are not associated with the PDSCH of another UE (see TS 38.214 [19], clause 5.1.6.2). It is applicable for PDSCH SCS of 480 and 960 kHz when rank 1 PDSCH with type-1 or type-2 DMRS is scheduled. If dmrs-TypeEnh-r18 is configured, this field is not configured.

dmrs-SequenceInitializationDCI-1_2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 1_2 If the field is absent, then the UE applies the value of 0 bit for the field "DMRS Sequence Initialization" in DCI format 1_2. If the field is present, then the UE applies the value of 1 bit as in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

harq-ProcessNumberSizeDCI-1-2 Configure the number of bits for the field "HARQ process number" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

maxMIMO-Layers Indicates the maximum number of MIMO layers to be used for PDSCH in this DL BWP. If not configured, the UE uses the maxMIMO-Layers configuration in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs, when the UE operates in this BWP. The value of maxMIMO-Layers for a DL BWP shall be smaller than or equal to the value of maxMIMO-Layers configured in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs. For MBS multicast, indicates the maximum number of MIMO layers to be used for group-common PDSCH of MBS multicast in this CFR. If not configured for CFR, the UE applies value 1. The value of maxMIMO-Layers for a CFR shall be smaller than or equal to the value of maxMIMO-Layers configured in PDSCH-ServingCellConfig IE of the serving cell to which this CFR belongs.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table Indicates which MCS table the UE shall use for PDSCH for DCI formats 1_0, 1_1 and 1_3 (see TS 38.214 [19], clause 5.1.3.1). If all fields are absent the UE applies the value 64QAM. If the field mcs-Table-r17 is present for DCI formats 1_1 and 1_3, the network does not configure the field mcs-Table (without suffix). For an (e)RedCap UE, the 256QAM MCS table for PDSCH is only supported if the UE indicates support of 256QAM for PDSCH.

mcs-TableDCI-1-2 Indicates which MCS table the UE shall use for PDSCH for DCI format 1_2 (see TS 38.214 [19], clause 5.1.3.1). If all fields are absent the UE applies the value 64QAM. If the field mcs-TableDCI-1-2-r17 is present, the network does not configure the field mcs-TableDCI-1-2-r16. For an (e)RedCap UE, the 256QAM MCS table for PDSCH is only supported if the UE indicates support of 256QAM for PDSCH.

minimumSchedulingOffsetK0 List of minimum K0 values. Minimum K0 parameter denotes minimum applicable value(s) for the TDRA table for PDSCH and for A-CSI RS triggering Offset(s) (see TS 38.214 [19], clause 5.3.1).

numberOfBitsForRV-DCI-1-2 Configures the number of bits for "Redundancy version" in the DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.1).

pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent in PDSCH-Config which is not used for MBS CFR, the UE applies the value 1.

pdsch-HARQ-ACK-EnhType3DCI-1-2 When configured, enhanced Type 3 HARQ-ACK codebook triggering by DCI format 1_2 is enabled.

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2 Enables the enhanced Type 3 codebook through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in DCI format 1_2 if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH cell group.

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 When configured, DCI format 1_2 can request the UE to report A/N for all HARQ processes and all component carriers configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-RetxDCI-1-2 When configured, DCI format 1_2 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource (see TS 38.213 [13], clause 9.1.5).

pdsch-TimeDomainAllocationList, pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH List of time-domain configurations for timing of DL assignment to DL data. The field pdsch-TimeDomainAllocationList (with or without suffix) applies to DCI format 1_0, DCI format 1_1 and DCI format 1_3 (see table 5.1.2.1.1-1 in TS 38.214 [19]), and if the field pdsch-TimeDomainAllocationListDCI-1-2 is not configured, to DCI format 1_2. If the field pdsch-TimeDomainAllocationListDCI-1-2 is configured, it applies to DCI format 1_2 (see table 5.1.2.1.1-1A in TS 38.214 [19]). The field pdsch-TimeDomainAllocationListForMultiPDSCH applies to DCI format 1_1. The network does not configure the pdsch-TimeDomainAllocationList-r16 simultaneously with the pdsch-TimeDomainAllocationList (without suffix) in the same PDSCH-Config.

prb-BundlingType, prb-BundlingTypeDCI-1-2 Indicates the PRB bundle type and bundle size(s) (see TS 38.214 [19], clause 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 [19], clause 5.1.2.3. If a bundleSize(Set) value is absent, the UE applies the value n2. The field prb-BundlingType applies to DCI formats 1_1 and 1_3, and the field prb-BundlingTypeDCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.3).

priorityIndicatorDCI-1-1, priorityIndicatorDCI-1-2, priorityIndicatorDCI-4-2 Configure the presence of "priority indicator" in DCI format 1_1/1_2/4_2. When the field is absent in the IE, then 0 bit for "priority indicator" in DCI format 1_1/1_2/4_2. The field priorityIndicatorDCI-1-1 applies to DCI format 1_1, the field priorityIndicatorDCI-1-2 applies to DCI format 1_2 and the field priorityIndicatorDCI-4-2 applies to DCI format 4_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9).

pucch-sSCellDynDCI-1-2 When configured, PUCCH cell switching based on dynamic indication in DCI format 1_2 is enabled (see TS 38.213 [13], clause 9.A).

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set. If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config for MBS CFR and PDSCH-Config for the assoicated BWP, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for MBS CFR can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for the assoicated BWP.

rateMatchPatternGroup1, rateMatchPatternGroup1DCI-1-2 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup1 applies to DCI formats 1_1 and 1_3, and the field rateMatchPatternGroup1DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternGroup2, rateMatchPatternGroup2DCI-1-2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup2 applies to DCI formats 1_1 and 1_3, and the field rateMatchPatternGroup2DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns (see TS 38.214 [19], clause 5.1.4.1). If a RateMatchPattern with the same RateMatchPatternId is configured in both MBS CFR and its associated BWP, the entire RateMatchPattern configuration, including the set of RBs/REs indicated by the patterns for the rate matching around, shall be the same and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH except PDSCH scheduled by DCI format 1_3. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).

referenceOfSLIVDCI-1-2 Enable using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV for DCI format 1_2. When the RRC parameter enables the utilization of the new reference, the new reference is applied for TDRA entries with K0=0. For other entries (if any) in the same TDRA table, the reference is slot boundary as in Rel-15. PDSCH mapping type A is not supported with the new reference. The new reference of SLIV is not configured for a serving cell configured to be scheduled by cross-carrier scheduling on a scheduling cell with different numerology (see TS 38.212 [17] clause 7.3.1 and TS 38.214 [19] clause 5.1.2.1).

repetitionSchemeConfig Configure the UE with repetition schemes. The network does not configure repetitionSchemeConfig-r16 and repetitionSchemeConfig-v1630 simultaneously to setup in the same PDSCH-Config. The network does not configure this parameter and sfnSchemePDSCH in MIMOParam-r17 simultaneously in the same serving cell.

resourceAllocation, resourceAllocationDCI-1-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 5.1.2.2). The field resourceAllocation applies to DCI format 1_1, and the field resourceAllocationDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.2.2).

resourceAllocationType1GranularityDCI-1-2 Configure the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 1_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 5.1.2.2.2).

sizeDCI-4-2 Indicates the size of DCI format 4-2 (see TS 38.213 [13], clause 10.1).

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList) (see TS 38.214 [19], clause 5.1.4.2).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see TS 38.214 [19], clause 5.1.5). If unifiedTCI-StateType is configured for the serving cell, no element in this list is configured.

unifiedTCI-StateRef Provides the serving cell and BWP where the configuration for dl-OrJointTCI-StateToAddModList-r17 are defined. When this field is present, dl-OrJointTCI-StateToAddModList and dl-OrJointTCI-StateToReleaseList are not present. The value of unifiedTCI-StateType of current serving cell is the same in the serving cell indicated by unifiedTCI-StateRef.

vrb-ToPRB-Interleaver, vrb-ToPRB-InterleaverDCI-1-2 Interleaving unit configurable between 2 and 4 PRBs (see TS 38.211 [16], clause 7.3.1.6). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

xOverheadMulticast Accounts for an overhead from CSI-RS, CORESET etc. If the field is absent, the UE applies value xOh0 (see TS 38.214 [19]).

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Each resource in this list may be referred to from only one type of resource set, i.e., aperiodic, semi-persistent or periodic (see TS 38.214 [19]).

PDSCH-ConfigDCI-1-3 field descriptions

harq-ProcessNumberSizeDCI-1-3 Configure the number of bits for the field "HARQ process number" in DCI format 1_3 (see TS 38.212 [17], clause 7.3.1).

numberOfBitsForRV-DCI-1-3 Configures the number of bits for "Redundancy version" in the DCI format 1_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.1).

rbg-SizeDCI-1-3 Selection among config 1, config 2 and config 3 for RBG size for PDSCH scheduled by DCI format 1_3. The UE ignores this field if resourceAllocationDCI-1-3 is set to resourceAllocationType1. (see TS 38.214 [19], clause 5.1.2.2.1).

resourceAllocationDCI-1-3 Configuration of resource allocation type 0 and resource allocation type 1 for DCI format 1_3 (see TS 38.214 [19], clause 5.1.2.2).

resourceAllocationType1GranularityDCI-1-3 Configure the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 1_3. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 5.1.2.2.2).

| Conditional Presence | Explanation |
| --- | --- |
| DCI-1-3 | This field is mandatory present when ScheduledCellListDCI-1-3 is configured to the serving cell. Otherwise, it is absent, Need R. |

#### –	PDSCH-ConfigCommon

The IE PDSCH-ConfigCommon is used to configure cell specific PDSCH parameters.

PDSCH-ConfigCommon information element

-- ASN1START

-- TAG-PDSCH-CONFIGCOMMON-START

PDSCH-ConfigCommon ::=                  SEQUENCE {

pdsch-TimeDomainAllocationList                  PDSCH-TimeDomainResourceAllocationList          OPTIONAL,   -- Need R

...

}

-- TAG-PDSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDSCH-ConfigCommon field descriptions

pdsch-TimeDomainAllocationList List of time-domain configurations for timing of DL assignment to DL data (see table 5.1.2.1.1-1 in TS 38.214 [19]).

#### –	PDSCH-ServingCellConfig

The IE PDSCH-ServingCellConfig is used to configure UE specific PDSCH parameters that are common across the UE's BWPs of one serving cell.

PDSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDSCH-SERVINGCELLCONFIG-START

PDSCH-ServingCellConfig ::=             SEQUENCE {

codeBlockGroupTransmission              SetupRelease { PDSCH-CodeBlockGroupTransmission }              OPTIONAL,   -- Need M

xOverhead                               ENUMERATED { xOh6, xOh12, xOh18 }                              OPTIONAL,   -- Need S

nrofHARQ-ProcessesForPDSCH              ENUMERATED {n2, n4, n6, n10, n12, n16}                         OPTIONAL,   -- Need S

pucch-Cell                              ServCellIndex                                                  OPTIONAL,   -- Cond SCellAddOnly

...,

[[

maxMIMO-Layers                          INTEGER (1..8)                                                 OPTIONAL,   -- Need M

processingType2Enabled                  BOOLEAN                                                        OPTIONAL    -- Need M

]],

[[

pdsch-CodeBlockGroupTransmissionList-r16 SetupRelease { PDSCH-CodeBlockGroupTransmissionList-r16 }     OPTIONAL    -- Need M

]],

[[

downlinkHARQ-FeedbackDisabled-r17       SetupRelease { DownlinkHARQ-FeedbackDisabled-r17 }              OPTIONAL,   -- Need M

nrofHARQ-ProcessesForPDSCH-v1700        ENUMERATED {n32}                                               OPTIONAL    -- Need R

]]

}

PDSCH-CodeBlockGroupTransmission ::=    SEQUENCE {

maxCodeBlockGroupsPerTransportBlock     ENUMERATED {n2, n4, n6, n8},

codeBlockGroupFlushIndicator            BOOLEAN,

...

}

PDSCH-CodeBlockGroupTransmissionList-r16 ::=    SEQUENCE (SIZE (1..2)) OF PDSCH-CodeBlockGroupTransmission

DownlinkHARQ-FeedbackDisabled-r17 ::= BIT STRING (SIZE (32))

-- TAG-PDSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDSCH-CodeBlockGroupTransmission field descriptions

codeBlockGroupFlushIndicator Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see TS 38.212 [17], clause 7.3.1.2.2).

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW, the maximum CBG is 4 (see TS 38.213 [13], clause 9.1.1).

PDSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see TS 38.213 [13], clause 9.1.1). Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group. The network does not configure this field if - the SCS of at least one DL BWP configured in the cell is 480 or 960 kHz - Type-1 HARQ-ACK codebook is configured and pdsch-TimeDomainAllocationListForMultiPDSCH-r17 for this serving cell contains pdsch-AllocationList with multiple entries (multiple PDSCH) - Type-2 HARQ-ACK codebook is configured and pdsch-TimeDomainAllocationListForMultiPDSCH-r17 for any cell in the same PUCCH cell group associated with this serving cell contains pdsch-AllocationList with multiple entries (multiple PDSCH)

downlinkHARQ-FeedbackDisabled Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID. The first/leftmost bit corresponds to HARQ process ID 0, the next bit to HARQ process ID 1 and so on. Bits corresponding to HARQ process IDs that are not configured shall be ignored. The bit(s) set to one identify HARQ processes with disabled DL HARQ feedback and the bit(s) set to zero identify HARQ processes with enabled DL HARQ feedback.

maxMIMO-Layers Indicates the maximum number of MIMO layers to be used for PDSCH in all BWPs of this serving cell. (see TS 38.212 [17], clause 5.4.2.1).

nrofHARQ-ProcessesForPDSCH The number of HARQ processes to be used on the PDSCH of a serving cell. Value n2 corresponds to 2 HARQ processes, value n4 to 4 HARQ processes, and so on. If both nrofHARQ-ProcessesForPDSCH and nrofHARQ-ProcessesForPDSCH-v1700 are absent, the UE uses 8 HARQ processes (see TS 38.214 [19], clause 5.1).

pdsch-CodeBlockGroupTransmissionList A list of configurations for up to two simultaneously constructed HARQ-ACK codebooks (see TS 38.213 [13], clause 9.3).

processingType2Enabled Enables configuration of advanced processing time capability 2 for PDSCH (see 38.214 [19], clause 5.3).

pucch-Cell The ID of the serving cell (of the same cell group) to use for PUCCH. If the field is absent, the UE sends the HARQ feedback on the PUCCH of the SpCell of this cell group, or on this serving cell if it is a PUCCH SCell.

xOverhead Accounts for overhead from CSI-RS, CORESET, etc. If the field is absent, the UE applies value xOh0 (see TS 38.214 [19], clause 5.1.3.2).

| Conditional Presence | Explanation |
| --- | --- |
| SCellAddOnly | It is optionally present, Need S, for (non-PUCCH) SCells when adding a new SCell. The field is absent, Need M, when reconfiguring SCells. The field is also absent for the SpCells as well as for a PUCCH SCell. |

#### –	PDSCH-TimeDomainResourceAllocationList

The IE PDSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PDSCH. The PDSCH-TimeDomainResourceAllocationList contains one or more of such PDSCH-TimeDomainResourceAllocations. The network indicates in the DL assignment which of the configured time domain allocations the UE shall apply for that DL assignment. The UE determines the bit width of the DCI field based on the number of entries in the PDSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PDSCH-TimeDomainResourceAllocationList information element

-- ASN1START

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PDSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation

PDSCH-TimeDomainResourceAllocation ::=   SEQUENCE {

k0                                      INTEGER(0..32)                                                  OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

PDSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation-r16

PDSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k0-r16                                     INTEGER(0..32)                                              OPTIONAL,   -- Need S

mappingType-r16                            ENUMERATED {typeA, typeB},

startSymbolAndLength-r16                   INTEGER (0..127),

repetitionNumber-r16                       ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16} OPTIONAL, -- Cond Formats1-0_1-1_4-0_4-1_4-2

...,

[[

k0-v1710                                INTEGER(33..128)                                               OPTIONAL    -- Need S

]],

[[

repetitionNumber-v1730                  ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}                   OPTIONAL   -- Cond Format1-2

]]

}

Dummy-TDRA-List ::= SEQUENCE (SIZE(1.. maxNrofDL-Allocations)) OF MultiPDSCH-TDRA-r17

MultiPDSCH-TDRA-List-r17 ::= SEQUENCE (SIZE(1.. maxNrofDL-AllocationsExt-r17)) OF MultiPDSCH-TDRA-r17

MultiPDSCH-TDRA-r17 ::= SEQUENCE {

pdsch-TDRA-List-r17                 SEQUENCE (SIZE(1..maxNrofMultiplePDSCHs-r17)) OF PDSCH-TimeDomainResourceAllocation-r16,

...

}

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PDSCH-TimeDomainResourceAllocation field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1). k0-v1710 is only applicable for PDSCH SCS of 480 kHz and 960 kHz. If multiple PDSCHs are configured per PDCCH, the network always configures this field. Otherwise, when the field is absent and only one PDSCH is configured per PDCCH, the UE applies the value 0.

mappingType PDSCH mapping type (see TS 38.214 [19], clause 5.3).

repetitionNumber Indicates the number of PDSCH transmission occasions for slot-based repetition scheme in IE RepetitionSchemeConfig. The parameter is used as specified in 38.214 [19].

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary (see TS 38.214 [19], clause 5.1.2.1).

MultiPDSCH-TimeDomainResourceAllocation field descriptions

pdsch-TDRA-List One or multiple PDSCHs which can be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 5.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Format1-2 | In pdsch-TimeDomainAllocationListDCI-1-2, this field is optionally present, Need R. It is absent, Need R, otherwise. |
| Formats1-0_1-1_4-0_4-1_4-2 | In pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH, and SIB20, this field is absent. Otherwise, in pdsch-TimeDomainResourceAllocationList-r16, this field is optionally present, Need R. |

#### –	PDU-SessionID

The IE PDU-SessionID identifies the PDU Session.

PDU-SessionID information element

-- ASN1START

-- TAG-PDU-SESSIONID-START

PDU-SessionID ::=   INTEGER (0..255)

-- TAG-PDU-SESSIONID-STOP

-- ASN1STOP

#### –	PHR-Config

The IE PHR-Config is used to configure parameters for power headroom reporting.

PHR-Config information element

-- ASN1START

-- TAG-PHR-CONFIG-START

PHR-Config ::=                      SEQUENCE {

phr-PeriodicTimer                   ENUMERATED {sf10, sf20, sf50, sf100, sf200,sf500, sf1000, infinity},

phr-ProhibitTimer                   ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000},

phr-Tx-PowerFactorChange            ENUMERATED {dB1, dB3, dB6, infinity},

multiplePHR                         BOOLEAN,

dummy                               BOOLEAN,

phr-Type2OtherCell                  BOOLEAN,

phr-ModeOtherCG                     ENUMERATED {real, virtual},

...,

[[

mpe-Reporting-FR2-r16               SetupRelease { MPE-Config-FR2-r16 }                     OPTIONAL     -- Need M

]],

[[

mpe-Reporting-FR2-r17               SetupRelease { MPE-Config-FR2-r17 }                     OPTIONAL,    -- Need M

twoPHRMode-r17                      ENUMERATED {enabled}                                    OPTIONAL     -- Need R

]],

[[

phr-AssumedPUSCH-Reporting-r18      ENUMERATED {enabled}                                    OPTIONAL,    -- Need R

dpc-Reporting-FR1-r18               ENUMERATED {enabled}                                    OPTIONAL     -- Need R

]]

}

MPE-Config-FR2-r16 ::=              SEQUENCE {

mpe-ProhibitTimer-r16               ENUMERATED {sf0, sf10, sf20, sf50, sf100, sf200, sf500, sf1000},

mpe-Threshold-r16                   ENUMERATED {dB3, dB6, dB9, dB12}

}

MPE-Config-FR2-r17 ::=              SEQUENCE {

mpe-ProhibitTimer-r17               ENUMERATED {sf0, sf10, sf20, sf50, sf100, sf200, sf500, sf1000},

mpe-Threshold-r17                   ENUMERATED {dB3, dB6, dB9, dB12},

numberOfN-r17                       INTEGER(1..4),

...

}

-- TAG-PHR-CONFIG-STOP

-- ASN1STOP

PHR-Config field descriptions

dpc-Reporting-FR1 Indicates if the delta power class (DPC) is reported, as specified in TS 38.321 [3].

dummy This field is not used in this version of the specification and the UE ignores the received value.

mpe-ProhibitTimer Value in number of subframes for MPE reporting, as specified in TS 38.321 [3]. Value sf10 corresponds to 10 subframes, and so on.

mpe-Reporting-FR2 Indicates whether the UE shall report MPE P-MPR in the PHR MAC control element, as specified in TS 38.321 [3].

mpe-Threshold Value of the P-MPR threshold in dB for reporting MPE P-MPR when FR2 is configured, as specified in TS 38.321 [3]. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

multiplePHR Indicates if power headroom shall be reported using the Single Entry PHR MAC control element or Multiple Entry PHR MAC control element defined in TS 38.321 [3]. True means to use Multiple Entry PHR MAC control element and False means to use the Single Entry PHR MAC control element defined in TS 38.321 [3]. The network configures this field to true for MR-DC and UL CA for NR, and to false in all other cases.

numberOfN Number of reported P-MPR values in a PHR MAC CE.

phr-AssumedPUSCH-Reporting Indicates if the PHR with an assumed PUSCH is reported, as specified in TS 38.321 [3]. The network ensures phr-AssumedPUSCH-Reporting-r18 and twoPHRMode-r17 are not configured at the same time for a UE.

phr-ModeOtherCG Indicates the mode (i.e. real or virtual) used for the PHR of the activated cells that are part of the other Cell Group (i.e. MCG or SCG), when DC is configured. If the UE is configured with only one cell group (no DC), it ignores the field.

phr-PeriodicTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. Value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes, and so on.

phr-ProhibitTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. Value sf0 corresponds to 0 subframe, value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes, and so on.

phr-Tx-PowerFactorChange Value in dB for PHR reporting as specified in TS 38.321 [3]. Value dB1 corresponds to 1 dB, dB3 corresponds to 3 dB and so on. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

phr-Type2OtherCell If set to true, the UE shall report a PHR type 2 for the SpCell of the other MAC entity. See TS 38.321 [3], clause 5.4.6. Network sets this field to false if the UE is not configured with an E-UTRA MAC entity.

twoPHRMode Indicates if the power headroom shall be reported as two PHRs (each PHR associated with a SRS resource set) is enabled or not. The network ensures phr-AssumedPUSCH-Reporting-r18 and twoPHRMode-r17 are not configured at the same time for a UE.

#### –	PhysCellId

The PhysCellId identifies the physical cell identity (PCI).

PhysCellId information element

-- ASN1START

-- TAG-PHYSCELLID-START

PhysCellId ::=                      INTEGER (0..1007)

-- TAG-PHYSCELLID-STOP

-- ASN1STOP

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]],

[[

dcp-Config-r16                      SetupRelease { DCP-Config-r16 }                                 OPTIONAL,   -- Need M

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16          ENUMERATED {semiStatic, dynamic}           OPTIONAL,   -- Cond twoPUCCHgroup

p-NR-FR2-r16                                              P-Max                                     OPTIONAL,   -- Need R

p-UE-FR2-r16                                              P-Max                                     OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR1-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR2-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

pdsch-HARQ-ACK-Codebook-r16            ENUMERATED {enhancedDynamic}                                 OPTIONAL,   -- Need R

nfi-TotalDAI-Included-r16              ENUMERATED {true}                                            OPTIONAL,   -- Need R

ul-TotalDAI-Included-r16               ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedback-r16     ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

downlinkAssignmentIndexDCI-0-2-r16     ENUMERATED { enabled }                                       OPTIONAL,   -- Need S

downlinkAssignmentIndexDCI-1-2-r16     ENUMERATED {n1, n2, n4}                                      OPTIONAL,   -- Need S

pdsch-HARQ-ACK-CodebookList-r16        SetupRelease {PDSCH-HARQ-ACK-CodebookList-r16}               OPTIONAL,   -- Need M

ackNackFeedbackMode-r16                ENUMERATED {joint, separate}                                 OPTIONAL,   -- Need R

pdcch-BlindDetectionCA-CombIndicator-r16 SetupRelease { PDCCH-BlindDetectionCA-CombIndicator-r16 }  OPTIONAL,   -- Need M

pdcch-BlindDetection2-r16                SetupRelease { PDCCH-BlindDetection2-r16 }                 OPTIONAL,   -- Need M

pdcch-BlindDetection3-r16                SetupRelease { PDCCH-BlindDetection3-r16 }                 OPTIONAL,   -- Need M

bdFactorR-r16                          ENUMERATED {n1}                                              OPTIONAL    -- Need R

]],

[[

-- start of enhanced Type3 feedback

pdsch-HARQ-ACK-EnhType3ToAddModList-r17   SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17

OPTIONAL,   -- Need N

pdsch-HARQ-ACK-EnhType3ToReleaseList-r17  SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17

OPTIONAL,    -- Need N

pdsch-HARQ-ACK-EnhType3SecondaryToAddModList-r17   SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17

OPTIONAL,    -- Need N

pdsch-HARQ-ACK-EnhType3SecondaryToReleaseList-r17  SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17

OPTIONAL,    -- Need N

pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup-r17 ENUMERATED {enabled}                        OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-EnhType3DCI-Field-r17                ENUMERATED {enabled}                            OPTIONAL,   -- Need R

-- end of enhanced Type3 feedback

-- start of triggering of HARQ-ACK re-transmission on a PUCCH resource

pdsch-HARQ-ACK-Retx-r17                   ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup-r17  ENUMERATED {enabled}                                    OPTIONAL,   -- Cond twoPUCCHgroup

-- end of triggering of HARQ-ACK re-transmission on a PUCCH resource

-- start of PUCCH Cell switching

pucch-sSCell-r17                         SCellIndex                                                    OPTIONAL,   -- Need R

pucch-sSCellSecondaryPUCCHgroup-r17      SCellIndex                                                    OPTIONAL,   -- Cond twoPUCCHgroup

pucch-sSCellDyn-r17                      ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

pucch-sSCellDynSecondaryPUCCHgroup-r17   ENUMERATED {enabled}                                       OPTIONAL,   -- Cond twoPUCCHgroup

pucch-sSCellPattern-r17                      SEQUENCE (SIZE(1..maxNrofSlots)) OF INTEGER (0..1)        OPTIONAL,   -- Need R

pucch-sSCellPatternSecondaryPUCCHgroup-r17   SEQUENCE (SIZE(1..maxNrofSlots)) OF INTEGER (0..1)        OPTIONAL,   -- Cond twoPUCCHgroup

-- end of PUCCH Cell switching

uci-MuxWithDiffPrio-r17           ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

uci-MuxWithDiffPrioSecondaryPUCCHgroup-r17     ENUMERATED {enabled}                         OPTIONAL,   -- Cond twoPUCCHgroup

simultaneousPUCCH-PUSCH-r17       ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup-r17       ENUMERATED {enabled}                  OPTIONAL,   -- Cond twoPUCCHgroup

prioLowDG-HighCG-r17              ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

prioHighDG-LowCG-r17              ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

twoQCLTypeDforPDCCHRepetition-r17 ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

multicastConfig-r17               SetupRelease { MulticastConfig-r17 }                      OPTIONAL,   -- Need M

pdcch-BlindDetectionCA-CombIndicator-r17 SetupRelease { PDCCH-BlindDetectionCA-CombIndicator-r17 }  OPTIONAL   -- Need M

]],

[[

simultaneousSR-PUSCH-diffPUCCH-Groups-r17      ENUMERATED {enabled}                         OPTIONAL    -- Cond twoPUCCHgroup

]],

[[

intraBandNC-PRACH-simulTx-r17     ENUMERATED {enabled}                                      OPTIONAL    -- Need R

]],

[[

pdcch-BlindDetection4-r17         SetupRelease { PDCCH-BlindDetection4-r17 }                OPTIONAL    -- Need M

]],

[[

simultaneousPUCCH-PUSCH-SamePriority-r17       ENUMERATED {enabled}                         OPTIONAL,   -- Need R

simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup-r17       ENUMERATED {enabled}     OPTIONAL    -- Cond twoPUCCHgroup

]],

[[

ncr-RNTI-r18                      RNTI-Value                                                OPTIONAL,   -- Cond NCR

cellDTRX-DCI-config-r18           SetupRelease { CellDTRX-DCI-config-r18 }                  OPTIONAL,   -- Need M

twoQCL-TypeD-ForMultiDCI-r18      ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableDiffPUCCH-Resource-r18                                       ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableDiffCB-Size-r18                                              ENUMERATED {enabled}     OPTIONAL    -- Need R

]]

}

PDSCH-HARQ-ACK-EnhType3-r17 ::=         SEQUENCE {

pdsch-HARQ-ACK-EnhType3Index-r17    PDSCH-HARQ-ACK-EnhType3Index-r17,

applicable-r17   CHOICE {

perCC                            SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1),

perHARQ                          SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (16))

},

pdsch-HARQ-ACK-EnhType3NDI-r17         ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3CBG-r17         ENUMERATED {true}                                            OPTIONAL,   -- Need S

...,

[[

perHARQ-Ext-r17                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (32)) OPTIONAL -- Need R

]]

}

PDSCH-HARQ-ACK-EnhType3Index-r17 ::=    INTEGER (0..maxNrofEnhType3HARQ-ACK-1-r17)

PDCCH-BlindDetection ::=                INTEGER (1..15)

DCP-Config-r16 ::=                  SEQUENCE {

ps-RNTI-r16                         RNTI-Value,

ps-Offset-r16                       INTEGER (1..120),

sizeDCI-2-6-r16                     INTEGER (1..maxDCI-2-6-Size-r16),

ps-PositionDCI-2-6-r16              INTEGER (0..maxDCI-2-6-Size-1-r16),

ps-WakeUp-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitPeriodicL1-RSRP-r16      ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitOtherPeriodicCSI-r16     ENUMERATED {true}                                               OPTIONAL    -- Need S

}

PDSCH-HARQ-ACK-CodebookList-r16 ::=     SEQUENCE (SIZE (1..2)) OF ENUMERATED {semiStatic, dynamic}

PDCCH-BlindDetectionCA-CombIndicator-r16 ::= SEQUENCE {

pdcch-BlindDetectionCA1-r16                  INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                  INTEGER (1..15)

}

PDCCH-BlindDetection2-r16 ::=                INTEGER (1..15)

PDCCH-BlindDetection3-r16 ::=                INTEGER (1..15)

PDCCH-BlindDetection4-r17 ::=                INTEGER (1..15)

MulticastConfig-r17 ::=                 SEQUENCE {

pdsch-HARQ-ACK-CodebookListMulticast-r17    SetupRelease { PDSCH-HARQ-ACK-CodebookList-r16}         OPTIONAL,   -- Need M

type1CodebookGenerationMode-r17             ENUMERATED { mode1, mode2}                              OPTIONAL    -- Need M

}

PDCCH-BlindDetectionCA-CombIndicator-r17 ::= SEQUENCE {

pdcch-BlindDetectionCA1-r17                  INTEGER (1..15)                                        OPTIONAL,   -- Need R

pdcch-BlindDetectionCA2-r17                  INTEGER (1..15)                                        OPTIONAL,   -- Need R

pdcch-BlindDetectionCA3-r17                  INTEGER (1..15)

}

CellDTRX-DCI-config-r18 ::=         SEQUENCE {

cellDTRX-RNTI-r18                   RNTI-Value,

sizeDCI-2-9-r18                     INTEGER (1..maxDCI-2-9-Size-r18)

}

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

ackNackFeedbackMode Indicates which among the joint and separate ACK/NACK feedback modes to use within a slot as specified in TS 38.213 [13] (clause 9).

bdFactorR Parameter for determining and distributing the maximum numbers of BD/CCE for mPDCCH based mPDSCH transmission as specified in TS 38.213 [13] Clause 10.1.

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

downlinkAssignmentIndexDCI-0-2 Indicates if "Downlink assignment index" is present or absent in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is absent, then 0 bit for "Downlink assignment index" in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is present, then the bitwidth of "Downlink assignment index" in DCI format 0_2 is defined in the same was as that in DCI format 0_1 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

downlinkAssignmentIndexDCI-1-2 Configures the number of bits for "Downlink assignment index" in DCI format 1_2. If the field is absent, then 0 bit is applied for "Downlink assignment index" in DCI format 1_2. Note that 1 bit and 2 bits are applied if only one serving cell is configured in the DL and pdsch-HARQ-ACK-Codebook is set to dynamic. 4 bits is applied if more than one serving cell are configured in the DL and pdsch-HARQ-ACK-Codebook is set to dynamic (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

enableDiffCB-Size This field indicates whether a different codebook size from the size determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant, is determined or not to include HARQ-ACK information associated with PDSCH reception(s) scheduled after a UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition. If enabled, a different HARQ codebook size might be determined. See clause 9 in TS 38.213 [13].

enableDiffPUCCH-Resource This field indicates whether a different PUCCH resource in time domain in a slot from the PUCCH resource indicated by the last DCI format before a UL grant in the slot, is determined or not to include HARQ-ACK information associated with PDSCH reception(s) scheduled after a UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition in the same slot. If enabled, a different PUCCH resource in time domain might be determined. See clause 9 in TS 38.213 [13].

enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant If enabled, UE multiplexes Type-1 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. See clause 9 in TS 38.213 [13]. This feature is not simultaneously enabled with PUCCH cell switching.

enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant If enabled, UE multiplexes Type-2 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. See clause 9 in TS 38.213 [13]. This feature is not simultaneously enabled with PUCCH cell switching.

enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant If enabled, UE multiplexes Type-3 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. See clause 9 in TS 38.213 [13]. This feature is not simultaneously enabled with PUCCH cell switching.

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUCCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clause 9.1.2.1). If the field harq-ACK SpatialBundlingPUCCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUCCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clause 9.1.2.1). When the field is absent, the use of spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUCCH. See TS 38.213 [13], clause 9.1.2.1. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUSCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). If the field harq-ACK SpatialBundlingPUSCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUSCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). When the field is absent, the use of spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUSCH. See TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

intraBandNC-PRACH-simulTx Enables parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA (see TS 38.213 [13], clause 8.1 and TS 38.214 [19], clause 6.2.1). This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the mcs-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

ncr-RNTI RNTI value for NCR-MT, used to scramble the PDCCHs carrying side control information (see TS 38.213 [13], clause 10.1).

nfi-TotalDAI-Included Indicates whether the NFI and total DAI fields of the non-scheduled PDSCH group is included in the non-fallback DL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

nrdc-PCmode-FR1 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 1 (FR1) (see TS 38.213 [13], clause 7.6).

nrdc-PCmode-FR2 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 2 (FR2) (see TS 38.213 [13], clause 7.6).

pdcch-BlindDetection, pdcch-BlindDetection2, pdcch-BlindDetection3, pdcch-BlindDetection4 Indicates the reference number of cells for PDCCH blind detection for the CG. Network configures the field for each CG when the UE is in NR DC and sets the value in accordance with the constraints specified in TS 38.213 [13]. The network configures pdcch-BlindDetection only if the UE is in NR-DC. The network configures pdcch-BlindDetection2 only if the UE is in NR-DC with at least one downlink cell using Rel-16 PDCCH monitoring capability. The network configures pdcch-BlindDetection3 only if the UE is in NR-DC with at least one downlink cell using Rel-15 PDCCH monitoring capability. The network configures pdcch-BlindDetection4 only if the UE is in NR-DC with at least one downlink cell using Rel-17 PDCCH monitoring capability.

pdcch-BlindDetectionCA-CombIndicator Configure one combination of pdcch-BlindDetectionCA1 (for R15) and pdcch-BlindDetectionCA2 (for R16) for UE to use for scaling PDCCH monitoring capability if the number of serving cells configured to a UE is larger than the reported capability, and if UE reports more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 as UE capability. The combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 configured by pdcch-BlindDetectionCA-CombIndicator is from the more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 reported by UE (see TS 38.213 [13], clause 10). pdcch-BlindDetectionCA-CombIndicator-r17 is used to configure one combination of pdcch-BlindDetectionCA1 (for R15), pdcch-BlindDetectionCA2 (for R16) and pdcch-BlindDetectionCA3 (for R17) for UE to use for scaling PDCCH monitoring capability if the number of serving cells configured to a UE is larger than the reported capability, and if UE reports more than one combination of pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2 and pdcch-BlindDetectionCA3 as UE capability. The combination of pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2 and pdcch-BlindDetectionCA3 configured by pdcch-BlindDetectionCA-CombIndicator-r17 is from the more than one combination of pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2 and pdcch-BlindDetectionCA3 reported by UE (see TS 38.213 [13], clause 10). pdcch-BlindDetectionCA-CombIndicator-r16 and pdcch-BlindDetectionCA-CombIndicator-r17 are not configured simultaneously.

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2). This field is only used in NR-DC. A UE does not expect to be configured with this parameter in this release of the specification.

prioLowDG-HighCG Enable PHY prioritization for the case where low-priority dynamic grant-PUSCH collides with high-priority configured grant-PUSCH on a BWP of a serving cell (see TS 38.213 [13], clause 9), when the UE has generated transport blocks for both DG-PUSCH and CG-PUSCH as described in TS 38.321 [3].

prioHighDG-LowCG Enable PHY prioritization for the case where high-priority dynamic grant PUSCH collides with low-priority configured grant PUSCH on a BWP of a serving cell (see TS 38.213 [13], clause 9), when the UE has generated transport blocks for both DG-PUSCH and CG-PUSCH as described in TS 38.321 [3].

ps-RNTI RNTI value for scrambling CRC of DCI format 2-6 used for power saving (see TS 38.213 [13], clause 10.1).

ps-Offset The start of the search-time of DCI format 2-6 with CRC scrambled by PS-RNTI relative to the start of the drx-onDurationTimer of Long DRX (see TS 38.213 [13], clause 10.3). Value in multiples of 0.125ms (milliseconds). 1 corresponds to 0.125 ms, 2 corresponds to 0.25 ms, 3 corresponds to 0.375 ms and so on.

ps-WakeUp Indicates the UE to wake-up if DCI format 2-6 is not detected outside active time (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not wake-up if DCI format 2-6 is not detected outside active time.

ps-PositionDCI-2-6 Starting position of UE wakeup and SCell dormancy indication in DCI format 2-6 (see TS 38.213 [13], clause 10.3).

ps-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start.

ps-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start.

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

p-UE-FR2 The maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR2 (configured for the cell group). A UE does not expect to be configured with this parameter in this release of the specification.

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and non-CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). If pdsch-HARQ-ACK-Codebook-r16 is signalled, UE shall ignore the pdsch-HARQ-ACK-Codebook (without suffix). For the HARQ-ACK for sidelink, if pdsch-HARQ-ACK-Codebook-r16 is signalled, the UE uses pdsch-HARQ-ACK-Codebook (without suffix) and ignores pdsch-HARQ-ACK-Codebook-r16. If the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary PUCCH group. Otherwise, this field is applied to the cell group (i.e. for all the cells within the cell group). For the HARQ-ACK for sidelink, if the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary and secondary PUCCH group and the UE ignores pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup.

pdsch-HARQ-ACK-CodebookList A list of configurations for one or two HARQ-ACK codebooks. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured). For the HARQ-ACK for sidelink, the UE uses pdsch-HARQ-ACK-Codebook and ignores pdsch-HARQ-ACK-CodebookList if this field is present.

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). It is configured for secondary PUCCH group.

pdsch-HARQ-ACK-EnhType3DCI-Field, pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup Indicates the enhanced Type 3 codebook through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in the primary PUCCH group if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH group, or in the secondary PUCCH group if the more than one enhanced Type 3 HARQ-ACK code is configured for the secondary PUCCH group, respectively.

pdsch-HARQ-ACK-EnhType3ToAddModList, pdsch-HARQ-ACK-EnhType3SecondaryToAddModList Configure the list of enhanced Type 3 HARQ-ACK codebooks for the primary PUCCH group and the secondary PUCCH group, respectively. When configured, DCI format 1_1 can request the UE to report A/N for one of the configured enhanced Type 3 HARQ-ACK codebooks in the corresponding PUCCH group (see TS 38.213 [13], clause 9.1.4). The network can configure pdsch-HARQ-ACK-EnhType3SecondaryToAddModList only if secondary PUCCH group is configured.

pdsch-HARQ-ACK-OneShotFeedback When configured, the DCI format 1_1 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-OneShotFeedbackCBG When configured, the DCI format 1_1 can request the UE to include CBG level A/N for each CC with CBG level transmission configured. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-OneShotFeedbackNDI When configured, the DCI format 1_1 can request the UE to include NDI for each A/N reported. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-Retx, pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup When configured, the DCI format 1_1 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource in the primary PUCCH group and the secondary PUCCH group, respectively (see TS 38.213 [13], clause 9.1.5).

pucch-sSCell, pucch-sSCellSecondaryPUCCHgroup indictates the alternative PUCCH cells for PUCCH cell switching in the primary and the secondary PUCCH group, respectively. For the primary PUCCH group, it is configured for cells on top of SpCell. For the secondary PUCCH group, it is configured for cell on top of the PUCCH SCell.

pucch-sSCellDyn, pucch-sSCellDynsecondaryPUCCHgroup When configured, PUCCH cell switching based on dynamic indication in DCI format 1_1 is enabled (see TS 38.213 [13], clause 9.A, clause 9.1.5), respectively for the primary PUCCH group and the secondary PUCCH group.

pucch-sSCellPattern, pucch-sSCellPatternSecondaryPUCCHgroup When configured, the UE applies the semi-static PUCCH cell switching (see TS 38.213 [13], clause 9.A) using the time domain pattern of applicable PUCCH cells indicated by this field, respectively for the primary PUCCH group and the secondary PUCCH group.

simultaneousPUCCH-PUSCH, simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup Enables simultaneous PUCCH and PUSCH transmissions with different priorities for the primary PUCCH group and the secondary PUCCH group, respectively.

simultaneousPUCCH-PUSCH-SamePriority, simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup Enables simultaneous PUCCH and PUSCH transmissions on different cells in different bands with same prioritiy for the primary PUCCH group and the secondary PUCCH group, respectively, as specified in clause 9 of TS 38.213 [13].

simultaneousSR-PUSCH-diffPUCCH-Groups Enables simultaneous SR and PUSCH transmissions in different PUCCH groups (see TS 38.321 [3], clause 5.4.1, clause 5.4.4).

sizeDCI-2-6 Size of DCI format 2-6 (see TS 38.213 [13], clause 10.3).

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig) (see TS 38.214 [19], clause 5.2.1.5.2). Network always configures the UE with a value for this field when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured.

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI (see TS 38.213 [13], clause 10.1).

twoQCL-TypeD-ForMultiDCI Indicates whether a UE is expected to identify and monitor two QCL-TypeD properties for multiple overlapping CORESETs, where the first QCL-TypeD is associated with coresetPoolIndex value 0, and the second QCL-TypeD is associated with coresetPoolIndex value 1. (See TS 38,213 [13], clause 10).

twoQCLTypeDforPDCCHRepetition Indicates whether a UE is expected UE to identify and monitor two QCL-TypeD properties for multiple overlapping CORESETs in the case of PDCCH repetition.

uci-MuxWithDiffPrio, uci-MuxWithDiffPrio-secondaryPUCCHgroup When configured, enables multiplexing a high-priority (HP) HARQ-ACK UCI and a low-priority (LP) HARQ-ACK UCI into a PUCCH or PUSCH for the primary PUCCH group and the secondary PUCCH group, respectively.

ul-TotalDAI-Included Indicates whether the total DAI fields of the additional PDSCH group is included in the non-fallback UL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see TS 38.213 [13]). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB.

MulticastConfig field descriptions

pdsch-HARQ-ACK-CodebookListMulticast A list of configurations for one or two HARQ-ACK codebooks for MBS multicast. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured).

type1CodebookGenerationMode Indicates the mode of Type-1 HARQ-ACK codebook generation, as specified in TS 38.213 [13]. Mode 1 is based on the k1 values that are in the intersection of K1 set for unicast and K1 set for multicast. Mode 2 is based on the k1 values that are in the union of K1 set for unicast and K1 set for multicast.

PDSCH-HARQ-ACK-EnhType3 field descriptions

pdsch-HARQ-ACK-EnhType3CBG When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include CBG level A/N for each CC with CBG level transmission configured of the enhanced Type 3 HARQ-ACK codebook. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC.

pdsch-HARQ-ACK-EnhType3NDI When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include NDI for each A/N reported of the enhanced Type 3 HARQ-ACK codebook.

perCC Configures enhanced Type 3 HARQ-ACK codebook using per CC configuration.

perHARQ, perHARQ-Ext Configures enhanced Type 3 HARQ-ACK codebook using per HARQ process and CC configuration. perHARQ-Ext is present only when nrofHARQ-ProcessesForPDSCH-v1700 is present in pdsch-ServingCellConfig of at least one serving cell in the PUCCH group. If perHARQ-Ext is present, the UE ignores perHARQ.

CellDTRX-DCI-config field descriptions

cellDTRX-RNTI The RNTI value for scrambling CRC of DCI format 2_9 for activating and/or deactivating Cell DTX/DRX and/or NES mode for CHO indication.

sizeDCI-2-9 The size of DCI format 2_9.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| NCR | This field is optionally present, Need M for NCR-MT. It is absent otherwise. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in (NG)EN-DC as defined in TS 38.213 [13]. It is absent otherwise. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise, Need R. |

#### –	PLMN-Identity

The IE PLMN-Identity identifies a Public Land Mobile Network. Further information regarding how to set the IE is specified in TS 23.003 [21].

PLMN-Identity information element

-- ASN1START

-- TAG-PLMN-IDENTITY-START

PLMN-Identity ::=                   SEQUENCE {

mcc                                 MCC                 OPTIONAL,                   -- Cond MCC

mnc                                 MNC

}

MCC ::=                             SEQUENCE (SIZE (3)) OF MCC-MNC-Digit

MNC ::=                             SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit

MCC-MNC-Digit ::=                   INTEGER (0..9)

-- TAG-PLMN-IDENTITY-STOP

-- ASN1STOP

PLMN-Identity field descriptions

mcc The first element contains the first MCC digit, the second element the second MCC digit and so on. If the field is absent, it takes the same value as the mcc of the immediately preceding IE PLMN-Identity. See TS 23.003 [21].

mnc The first element contains the first MNC digit, the second element the second MNC digit and so on. See TS 23.003 [21].

| Conditional Presence | Explanation |
| --- | --- |
| MCC | This field is mandatory present when PLMN-Identity is not used in a list or if it is the first entry of PLMN-Identity in a list. Otherwise it is optionally present, Need S. |

#### –	PLMN-IdentityInfoList

The IE PLMN-IdentityInfoList includes a list of PLMN identity information.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...,

[[

iab-Support-r16                     ENUMERATED {true}                                               OPTIONAL       -- Need S

]],

[[

trackingAreaList-r17                SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode             OPTIONAL,      -- Need R

gNB-ID-Length-r17                   INTEGER (22..32)                                                OPTIONAL    -- Cond eventID-TSS

]],

[[

mobileIAB-Support-r18               ENUMERATED {true}                                               OPTIONAL       -- Need S

]]

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

PLMN-IdentityInfo field descriptions

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (per PLMN), as defined in TS 38.304 [20]. This field is ignored by IAB-MT and NCR-MT.

gNB-ID-Length Indicates the length of the gNB ID out of the 36-bit long cellIdentity. This field is always present if the mobileIAB-Support is broadcasted in a cell.

iab-Support This field combines both the support of IAB and the cell status for IAB. If the field is present, the cell supports IAB and the cell is also considered as a candidate for cell (re)selection for IAB-node; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell.

mobileIAB-Support This field indicates the support of mobile IAB and whether the cell can be considered as a candidate for cell (re)selection for mobile IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell. If the field is absent, the cell is barred for mobile IAB-node.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell.

trackingAreaList List of Tracking Areas to which the cell indicated by cellIdentity field belongs. If this field is present, network does not configure trackingAreaCode. Total number of different TACs across different PLMN-IdentityInfos shall not exceed maxTAC. This field is only present in an NTN cell.

| Conditional Presence | Explanation |
| --- | --- |
| eventID-TSS | If eventID-TSS is present in SIB9, this field is mandatory present in the PLMN-IdentityInfo IE of PLMN-IdentityInfoList in SIB1. Otherwise, the field is optionally present, Need R. |

#### –	PLMN-IdentityList2

Includes a list of PLMN identities.

PLMN-IdentityList2 information element

-- ASN1START

-- TAG-PLMNIDENTITYLIST2-START

PLMN-IdentityList2-r16 ::= SEQUENCE (SIZE (1..16)) OF PLMN-Identity

-- TAG-PLMNIDENTITYLIST2-STOP

-- ASN1STOP

#### –	PRB-Id

The IE PRB-Id identifies a Physical Resource Block (PRB) position within a carrier.

PRB-Id information element

-- ASN1START

-- TAG-PRB-ID-START

PRB-Id ::=                          INTEGER (0..maxNrofPhysicalResourceBlocks-1)

-- TAG-PRB-ID-STOP

-- ASN1STOP

#### –	PTRS-DownlinkConfig

The IE PTRS-DownlinkConfig is used to configure downlink phase tracking reference signals (PTRS) (see TS 38.214 [19] clause 5.1.6.3)

PTRS-DownlinkConfig information element

-- ASN1START

-- TAG-PTRS-DOWNLINKCONFIG-START

PTRS-DownlinkConfig ::=             SEQUENCE {

frequencyDensity                    SEQUENCE (SIZE (2)) OF INTEGER (1..276)                                 OPTIONAL,   -- Need S

timeDensity                         SEQUENCE (SIZE (3)) OF INTEGER (0..29)                                  OPTIONAL,   -- Need S

epre-Ratio                          INTEGER (0..3)                                                          OPTIONAL,   -- Need S

resourceElementOffset               ENUMERATED { offset01, offset10, offset11 }                             OPTIONAL,   -- Need S

...,

[[

maxNrofPorts-r16                    ENUMERATED {n1, n2}                                                     OPTIONAL    -- Need R

]]

}

-- TAG-PTRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

PTRS-DownlinkConfig field descriptions

epre-Ratio EPRE ratio between PTRS and PDSCH. Value 0 corresponds to the codepoint "00" in table 4.1-2. Value 1 corresponds to codepoint "01", and so on. If the field is not provided, the UE applies value 0 (see TS 38.214 [19], clause 4.1).

frequencyDensity Presence and frequency density of DL PT-RS as a function of Scheduled BW. If the field is absent, the UE uses K_PT-RS = 2 (see TS 38.214 [19], clause 5.1.6.3, table 5.1.6.3-2).

maxNrofPorts The maximum number of DL PTRS ports specified in TS 38.214 [19] (clause 5.1.6.3). 2 PT-RS ports can only be configured for a DL BWP that is configured, as specified in TS 38.214 [19] clause 5.1, with a mode where a single PDSCH has association between the DM-RS ports and the TCI states as defined in TS 38.214 [19] clause 5.1.6.2.

resourceElementOffset Indicates the subcarrier offset for DL PTRS. If the field is absent, the UE applies the value offset00 (see TS 38.211 [16], clause 7.4.1.2.2).

timeDensity Presence and time density of DL PT-RS as a function of MCS. The value 29 is only applicable for MCS Table 5.1.3.1-1 (TS 38.214 [19]). If the field is absent, the UE uses L_PT-RS = 1 (see TS 38.214 [19], clause 5.1.6.3, table 5.1.6.3-1).

#### –	PTRS-UplinkConfig

The IE PTRS-UplinkConfig is used to configure uplink Phase-Tracking-Reference-Signals (PTRS).

PTRS-UplinkConfig information element

-- ASN1START

-- TAG-PTRS-UPLINKCONFIG-START

PTRS-UplinkConfig ::=                   SEQUENCE {

transformPrecoderDisabled               SEQUENCE {

frequencyDensity                    SEQUENCE (SIZE (2)) OF INTEGER (1..276)                 OPTIONAL,   -- Need S

timeDensity                         SEQUENCE (SIZE (3)) OF INTEGER (0..29)                  OPTIONAL,   -- Need S

maxNrofPorts                        ENUMERATED {n1, n2},

resourceElementOffset               ENUMERATED {offset01, offset10, offset11 }              OPTIONAL,   -- Need S

ptrs-Power                          ENUMERATED {p00, p01, p10, p11}

}                                                                                               OPTIONAL,   -- Need R

transformPrecoderEnabled                SEQUENCE {

sampleDensity                           SEQUENCE (SIZE (5)) OF INTEGER (1..276),

timeDensityTransformPrecoding           ENUMERATED {d2}                                     OPTIONAL    -- Need S

}                                                                                               OPTIONAL,   -- Need R

...,

[[

maxNrofPorts-SDM-r18                     ENUMERATED {n1, n2}                                     OPTIONAL    -- Need R

]]

}

-- TAG-PTRS-UPLINKCONFIG-STOP

-- ASN1STOP

PTRS-UplinkConfig field descriptions

frequencyDensity Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW If the field is absent, the UE uses K_PT-RS = 2 (see TS 38.214 [19], clause 6.1).

maxNrofPorts, maxNrofPorts-SDM The maximum number of UL PTRS ports for CP-OFDM (see TS 38.214 [19], clause 6.2.3.1).

ptrs-Power UL PTRS power boosting factor per PTRS port (see TS 38.214 [19], clause 6.1, table 6.2.3.1.3).

resourceElementOffset Indicates the subcarrier offset for UL PTRS for CP-OFDM. If the field is absent, the UE applies the value offset00 (see TS 38.211 [16], clause 6.4.1.2.2).

sampleDensity Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn, n=0,1,2,3,4}, that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should use depending on the scheduled BW, see TS 38.214 [19], clause 6.1, table 6.2.3.2-1.

timeDensity Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS If the field is absent, the UE uses L_PT-RS = 1 (see TS 38.214 [19], clause 6.1).

timeDensityTransformPrecoding Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM. If the field is absent, the UE applies value d1 (see TS 38.214 [19], clause 6.1).

transformPrecoderDisabled Configuration of UL PTRS without transform precoder (with CP-OFDM).

transformPrecoderEnabled Configuration of UL PTRS with transform precoder (DFT-S-OFDM).

#### –	PUCCH-Config

The IE PUCCH-Config is used to configure UE specific PUCCH parameters (per BWP).

PUCCH-Config information element

-- ASN1START

-- TAG-PUCCH-CONFIG-START

PUCCH-Config ::=                        SEQUENCE {

resourceSetToAddModList                 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet   OPTIONAL, -- Need N

resourceSetToReleaseList                SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId OPTIONAL, -- Need N

resourceToAddModList                    SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource         OPTIONAL, -- Need N

resourceToReleaseList                   SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId       OPTIONAL, -- Need N

format1                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format3                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format4                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

schedulingRequestResourceToAddModList   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig

OPTIONAL, -- Need N

schedulingRequestResourceToReleaseList  SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId

OPTIONAL, -- Need N

multi-CSI-PUCCH-ResourceList            SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId                            OPTIONAL, -- Need M

dl-DataToUL-ACK                         SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)                             OPTIONAL, -- Need M

spatialRelationInfoToAddModList         SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseList        SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

pucch-PowerControl                      PUCCH-PowerControl                                                    OPTIONAL, -- Need M

...,

[[

resourceToAddModListExt-v1610           SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceExt-v1610  OPTIONAL, -- Need N

dl-DataToUL-ACK-r16                     SetupRelease { DL-DataToUL-ACK-r16 }                                  OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-1-r16          SetupRelease { UL-AccessConfigListDCI-1-1-r16 }                       OPTIONAL, -- Need M

subslotLengthForPUCCH-r16               CHOICE {

normalCP-r16                        ENUMERATED {n2,n7},

extendedCP-r16                      ENUMERATED {n2,n6}

}                                                                                                             OPTIONAL, -- Need R

dl-DataToUL-ACK-DCI-1-2-r16             SetupRelease { DL-DataToUL-ACK-DCI-1-2-r16}                           OPTIONAL, -- Need M

numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2-r16  INTEGER (0..3)                                             OPTIONAL, -- Need R

dmrs-UplinkTransformPrecodingPUCCH-r16  ENUMERATED {enabled}                                                  OPTIONAL,  -- Cond PI2-BPSK

spatialRelationInfoToAddModListSizeExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseListSizeExt-v1610   SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

spatialRelationInfoToAddModListExt-v1610  SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfoExt-r16

OPTIONAL, -- Need N

spatialRelationInfoToReleaseListExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF

PUCCH-SpatialRelationInfoId-r16       OPTIONAL, -- Need N

resourceGroupToAddModList-r16           SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroup-r16

OPTIONAL, -- Need N

resourceGroupToReleaseList-r16          SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroupId-r16

OPTIONAL, -- Need N

sps-PUCCH-AN-List-r16                   SetupRelease { SPS-PUCCH-AN-List-r16 }                                OPTIONAL,  -- Need M

schedulingRequestResourceToAddModListExt-v1610   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfigExt-v1610

OPTIONAL -- Need N

]],

[[

format0-r17                             SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

format3Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

format4Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-2-r17          SetupRelease { UL-AccessConfigListDCI-1-2-r17 }                       OPTIONAL, -- Need M

mappingPattern-r17                      ENUMERATED {cyclicMapping, sequentialMapping}                         OPTIONAL, -- Need R

powerControlSetInfoToAddModList-r17     SEQUENCE (SIZE (1..maxNrofPowerControlSetInfos-r17)) OF PUCCH-PowerControlSetInfo-r17

OPTIONAL, -- Need N

powerControlSetInfoToReleaseList-r17    SEQUENCE (SIZE (1..maxNrofPowerControlSetInfos-r17)) OF PUCCH-PowerControlSetInfoId-r17

OPTIONAL, -- Need N

secondTPCFieldDCI-1-1-r17               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

secondTPCFieldDCI-1-2-r17               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

dl-DataToUL-ACK-r17                     SetupRelease { DL-DataToUL-ACK-r17 }                                  OPTIONAL, -- Need M

dl-DataToUL-ACK-DCI-1-2-r17             SetupRelease { DL-DataToUL-ACK-DCI-1-2-r17}                           OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-1-r17          SetupRelease { UL-AccessConfigListDCI-1-1-r17 }                       OPTIONAL, -- Need M

schedulingRequestResourceToAddModListExt-v1700 SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfigExt-v1700

OPTIONAL, -- Need N

dmrs-BundlingPUCCH-Config-r17           SetupRelease { DMRS-BundlingPUCCH-Config-r17 }                        OPTIONAL, -- Need M

dl-DataToUL-ACK-v1700                   SetupRelease { DL-DataToUL-ACK-v1700 }                                OPTIONAL, -- Need M

dl-DataToUL-ACK-MulticastDCI-Format4-1-r17 SetupRelease { DL-DataToUL-ACK-MulticastDCI-Format4-1-r17 }        OPTIONAL, -- Need M

sps-PUCCH-AN-ListMulticast-r17          SetupRelease { SPS-PUCCH-AN-List-r16 }                                OPTIONAL  -- Need M

]]

}

PUCCH-FormatConfig ::=                  SEQUENCE {

interslotFrequencyHopping               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

additionalDMRS                          ENUMERATED {true}                                                     OPTIONAL, -- Need R

maxCodeRate                             PUCCH-MaxCodeRate                                                     OPTIONAL, -- Need R

nrofSlots                               ENUMERATED {n2,n4,n8}                                                 OPTIONAL, -- Need S

pi2BPSK                                 ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

simultaneousHARQ-ACK-CSI                ENUMERATED {true}                                                     OPTIONAL  -- Need R

}

PUCCH-FormatConfigExt-r17 ::=           SEQUENCE {

maxCodeRateLP-r17                       PUCCH-MaxCodeRate                                                     OPTIONAL, -- Need R

...

}

PUCCH-MaxCodeRate ::=                   ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80}

-- A set with one or more PUCCH resources

PUCCH-ResourceSet ::=                   SEQUENCE {

pucch-ResourceSetId                     PUCCH-ResourceSetId,

resourceList                            SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId,

maxPayloadSize                          INTEGER (4..256)                                                      OPTIONAL  -- Need R

}

PUCCH-ResourceSetId ::=                 INTEGER (0..maxNrofPUCCH-ResourceSets-1)

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                        PUCCH-ResourceId,

startingPRB                             PRB-Id,

intraSlotFrequencyHopping               ENUMERATED { enabled }                                                OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                                                                OPTIONAL, -- Need R

format                                  CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

PUCCH-ResourceExt-v1610 ::=             SEQUENCE {

interlaceAllocation-r16                 SEQUENCE {

rb-SetIndex-r16                         INTEGER (0..4),

interlace0-r16                          CHOICE {

scs15                                   INTEGER (0..9),

scs30                                   INTEGER (0..4)

}

}                                                                                                             OPTIONAL,  --Need R

format-v1610                            CHOICE {

interlace1-v1610                            INTEGER (0..9),

occ-v1610                                   SEQUENCE {

occ-Length-v1610                                ENUMERATED {n2,n4}                                       OPTIONAL, -- Need M

occ-Index-v1610                                 ENUMERATED {n0,n1,n2,n3}                                 OPTIONAL  -- Need M

}

}                                                                                                            OPTIONAL,  -- Need R

...,

[[

format-v1700                         SEQUENCE {

nrofPRBs-r17                            INTEGER (1..16)

}                                                                                                            OPTIONAL,  -- Need R

pucch-RepetitionNrofSlots-r17           ENUMERATED { n1,n2,n4,n8 }                                              OPTIONAL   -- Need R

]],

[[

applyIndicatedTCI-State-r18          ENUMERATED {first, second, both}                                         OPTIONAL, -- Need R

multipanelSFN-Scheme-r18             ENUMERATED {enabled}                                                     OPTIONAL -- Need R

]]

}

PUCCH-ResourceId ::=                    INTEGER (0..maxNrofPUCCH-Resources-1)

PUCCH-format0 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format1 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10),

timeDomainOCC                                   INTEGER(0..6)

}

PUCCH-format2 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format3 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-format4 ::=                               SEQUENCE {

nrofSymbols                                     INTEGER (4..14),

occ-Length                                      ENUMERATED {n2,n4},

occ-Index                                       ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-ResourceGroup-r16 ::=                SEQUENCE {

pucch-ResourceGroupId-r16                  PUCCH-ResourceGroupId-r16,

resourcePerGroupList-r16                   SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerGroup-r16)) OF PUCCH-ResourceId

}

PUCCH-ResourceGroupId-r16 ::=              INTEGER (0..maxNrofPUCCH-ResourceGroups-1-r16)

DL-DataToUL-ACK-r16 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)

DL-DataToUL-ACK-r17 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..127)

DL-DataToUL-ACK-v1700 ::=                  SEQUENCE (SIZE (1..8)) OF INTEGER (16..31)

DL-DataToUL-ACK-DCI-1-2-r16 ::=            SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)

DL-DataToUL-ACK-DCI-1-2-r17 ::=            SEQUENCE (SIZE (1..8)) OF INTEGER (0..127)

UL-AccessConfigListDCI-1-1-r16 ::=         SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)

UL-AccessConfigListDCI-1-2-r17 ::=         SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)

UL-AccessConfigListDCI-1-1-r17 ::=         SEQUENCE (SIZE (1..3)) OF INTEGER (0..2)

DL-DataToUL-ACK-MulticastDCI-Format4-1-r17 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)

-- TAG-PUCCH-CONFIG-STOP

-- ASN1STOP

PUCCH-Config field descriptions

dl-DataToUL-ACK, dl-DataToUL-ACK-DCI-1-2 List of timing for given PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3). The dl-DataToUL-ACK-v1700 is applicable for NTN and dl-DataToUL-ACK-r17 is applicable for up to 71 GHz. If dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-v1700 is signalled, UE shall ignore the dl-DataToUL-ACK (without suffix). The value -1 corresponds to "inapplicable value" for the case where the A/N feedback timing is not explicitly included at the time of scheduling PDSCH. The fields dl-DataToUL-ACK-r17 and dl-DataToUL-ACK-DCI-1-2-r17 are only applicable for SCS of 480 kHz or 960 kHz.

dl-DataToUL-ACK-MulticastDCI-Format4-1 List of timing for given group-common PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK-MulticastDciFormat4-1 applies to DCI format 4_1 for MBS multicast (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

dmrs-BundlingPUCCH-Config Configuration of the parameters for DMRS bundling for PUCCH (see TS 38.214 [19], clause 6.1.7). DMRS bundling for PUCCH is not supported for PUCCH format 0/2. In this release, this is not applicable to FR2-2.

dmrs-UplinkTransformPrecodingPUCCH This field is used for PUCCH formats 3 and 4 according to TS 38.211, Clause 6.4.1.3.3.1.

format0 Parameters that are common for all PUCCH resources of format 0.

format1 Parameters that are common for all PUCCH resources of format 1.

format2 Parameters that are common for all PUCCH resources of format 2.

format3 Parameters that are common for all PUCCH resources of format 3.

format4 Parameters that are common for all PUCCH resources of format 4.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern for when a PUCCH resource used for repetitions of a PUCCH transmission includes first and second spatial settings for FR2, or first and second sets of power control parameters for FR1 (see TS 38.213 [13], clause 9.2.6).

numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2 Configuration of the number of bits for "PUCCH resource indicator" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

powerControlSetInfoToAddModList Configures power control sets for repetition of a PUCCH transmission in FR1. This field is not configured if ul-powerControl is configured in the BWP-UplinkDedicated in which the PUCCH-Config is included.

pucch-PowerControl Configures power control parameters PUCCH transmission. This field is not configured if unifiedTCI-StateType is configured for the serving cell.

resourceGroupToAddModList, resourceGroupToReleaseList Lists for adding and releasing groups of PUCCH resources that can be updated simultaneously for spatial relations with a MAC CE.

resourceSetToAddModList, resourceSetToReleaseList Lists for adding and releasing PUCCH resource sets (see TS 38.213 [13], clause 9.2).

resourceToAddModList, resourceToAddModListExt, resourceToReleaseList Lists for adding and releasing PUCCH resources applicable for the UL BWP and serving cell in which the PUCCH-Config is defined. The resources defined herein are referred to from other parts of the configuration to determine which resource the UE shall use for which report. If the network includes of resourceToAddModListExt, it includes the same number of entries, and listed in the same order, as in resourceToAddModList.

secondTPCFieldDCI-1-1, secondTPCFieldDCI-1-2 A second TPC field can be configured via RRC for DCI-1-1 and DCI-1-2. Each TPC field is for each closed-loop index value respectively (i.e., 1st /2nd TPC fields correspond to "closedLoopIndex" value = 0 and 1.

spatialRelationInfoToAddModList, spatialRelationInfoToAddModListSizeExt , spatialRelationInfoToAddModListExt Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321 [3], clause 5.18.8 and TS 38.213 [13], clause 9.2.2). The UE shall consider entries in spatialRelationInfoToAddModList and in spatialRelationInfoToAddModListSizeExt as a single list, i.e. an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt (or deleted using spatialRelationInfoToReleaseListSizeExt) and vice-versa. If the network includes spatialRelationInfoToAddModListExt, it includes the same number of entries, and listed in the same order, as in the concatenation of spatialRelationInfoToAddModList and of spatialRelationInfoToAddModListSizeExt. If unifiedTCI-StateType is configured for the serving cell, no element in this list is configured.

spatialRelationInfoToReleaseList, spatialRelationInfoToReleaseListSizeExt, spatialRelationInfoToReleaseListExt Lists of spatial relation configurations between a reference RS and PUCCH to be released by the UE.

sps-PUCCH-AN-List Indicates a list of PUCCH resources for DL SPS HARQ ACK. The field maxPayloadSize is absent for the first and the last SPS-PUCCH-AN in the list. If configured, this overrides n1PUCCH-AN in SPS-config.

sps-PUCCH-AN-ListMulticast The field is used to configure the list of PUCCH resources per HARQ ACK codebook for MBS multicast.

subslotLengthForPUCCH Indicates the sub-slot length for sub-slot based PUCCH feedback in number of symbols (see TS 38.213 [13], clause 9). Value n2 corresponds to 2 symbols, value n6 corresponds to 6 symbols, value n7 corresponds to 7 symbols. For normal CP, the value is either n2 or n7. For extended CP, the value is either n2 or n6.

ul-AccessConfigListDCI-1-1, ul-AccessConfigListDCI-1-2 List of the combinations of cyclic prefix extension and UL channel access type (see TS 38.212 [17], clause 7.3.1) applicable, respectively, to DCI format 1_1 and DCI format 1_2. The fields ul-AccessConfigListDCI-1-1-r16 and ul-AccessConfigListDCI-1-2-r17 are only applicable for FR1 (see TS 38.212 [17], Table 7.3.1.2.2-6). The field ul-AccessConfigListDCI-1-1-r17 indicates a list which only contains UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.2.2-6A).

PUCCH-format3 field descriptions

nrofPRBs The supported values are 1,2,3,4,5,6,8,9,10,12,15 and 16. The UE shall ignore this field when format-v1610 is configured.

PUCCH-FormatConfig, PUCCH-FormatConfigExt field descriptions

additionalDMRS If the field is present, the UE enables 2 DMRS symbols per hop of a PUCCH Format 3 or 4 if both hops are more than X symbols when FH is enabled (X=4). And it enables 4 DMRS symbols for a PUCCH Format 3 or 4 with more than 2X+1 symbols when FH is disabled (X=4). The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.2.

interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 0, 1, 2, 3 or 4 is repeated over multiple slots. For a PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. See TS 38.213 [13], clause 9.2.6.

maxCodeRate Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. See TS 38.213 [13], clause 9.2.5.

maxCodeRateLP Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. This field configures additional max code rate in the second entry of PUCCH-ConfigurationList-r16 for multiplexing low-priority (LP) HARQ-ACK and high-priority (HP) UCI in a PUCCH as described Clause 9.2.5.3 of TS 38.213 [13]. The field is absent for the first entry of PUCCH-ConfigurationList-r16.

nrofSlots Number of slots with the same PUCCH. When the field is absent the UE applies the value n1. See TS 38.213 [13], clause 9.2.6.

pi2BPSK If the field is present, the UE uses pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.5.

rb-SetIndex Indicates the RB set where PUCCH resource is allocated.

simultaneousHARQ-ACK-CSI If the field is present, the UE uses simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format 2, 3 or 4. See TS 38.213 [13], clause 9.2.5. When the field is absent the UE applies the value off. The field is not applicable for format 0 and 1.

PUCCH-Resource, PUCCH-ResourceExt field descriptions

applyIndicatedTCI-State This field indicates, for PUCCH transmission(s) corresponding to this PUCCH resource, if UE applies the first, the second or both "indicated" UL only TCI or joint TCI as specified in TS 38.213 9.2.2. For PUCCH resources belonging to a PUCCH group, network configures same value. If more than one value for the field coresetPoolIndex is configured in IE controlResourceSet for the BWP, the value 'first' corresponds to the "indicated" joint/UL TCI states specific to coresetPoolIndex value 0 and the value 'second' corresponds to the value 1, respectively. In this case, network does not configure the value 'both'.

format Selection of the PUCCH format (format 0 – 4) and format-specific parameters, see TS 38.213 [13], clause 9.2. format0 and format1 are only allowed for a resource in a first PUCCH resource set. format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. The network can only configure format-v1610 when format is set to format2 or format3. The network only configures format-v1700 when format is set to format0, format1 or format4.

interlace0 This is the only interlace of interlaced PUCCH Format 0 and 1 and the first interlace for interlaced PUCCH Format 2 and 3.

interlace1 A second interlace, in addition to interlace 0, as specified in TS 38.213 [13], clause 9.2.1. For 15kHz SCS, values {0..9} are applicable; for 30kHz SCS, values {0..4} are applicable. For 15kHz SCS, the values of interlace1 shall satisfy interlace1=mod(interlace0+X,10) where X=1, -1, or 5.

intraSlotFrequencyHopping Enabling intra-slot frequency hopping, applicable for all types of PUCCH formats. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. See TS 38.213 [13], clause 9.2.1.

nrofPRBs Indicates the number of PRBs used per PUCCH resource for the PUCCH format, see TS 38.213 [13], clause 9.2.1. This field is applicable for PUCCH format0, format1, and format4 in FR2-2. The supported values for format4 are 1,2,3,4,5,6,8,9,10,12,15 and 16.

multipanelSFN-Scheme Parameter to configure multiple panel simultaneous uplink transmission SFN scheme for PUCCH resources, see TS 38.214 [19] clause 6.1.1.

occ-Index Indicates the orthogonal cover code index (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

occ-Length Indicates the orthogonal cover code length (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

pucch-RepetitionNrofSlots Configuration of PUCCH repetition factor per PUCCH resource with associated scheduling DCI corresponding to Rel-17 dynamic PUCCH repetition. For a PUCCH resource, if both the field pucch-RepetitionNrofSlots and the field nrofSlots are present, the field nrofSlots is ignored and apply the value of pucch-RepetitionNrofSlots corresponding to Rel-17 dynamic PUCCH repetition. If this field is absent in a PUCCH resource with associated scheduling DCI, the UE applies the value of field nrofSlots.

pucch-ResourceId Identifier of the PUCCH resource.

secondHopPRB Index of first PRB after frequency hopping of PUCCH. This value is applicable for intra-slot frequency hopping (see TS 38.213 [13], clause 9.2.1) or inter-slot frequency hopping (see TS 38.213 [13], clause 9.2.6).

PUCCH-ResourceSet field descriptions

maxPayloadSize Maximum number of UCI information bits that the UE may transmit using this PUCCH resource set (see TS 38.213 [13], clause 9.2.1). In a PUCCH occurrence, the UE chooses the first of its PUCCH-ResourceSet which supports the number of bits that the UE wants to transmit. The field is absent in the first set (Set0) and in the last configured set since the UE derives the maximum number of UCI information bits as specified in TS 38.213 [13], clause 9.2.1. This field can take integer values that are multiples of 4.

resourceList PUCCH resources of format0 and format1 are only allowed in the first PUCCH resource set, i.e., in a PUCCH-ResourceSet with pucch-ResourceSetId = 0. This set may contain between 1 and 32 resources. PUCCH resources of format2, format3 and format4 are only allowed in a PUCCH-ResourceSet with pucch-ResourceSetId > 0. If present, these sets contain between 1 and 8 resources each. The UE chooses a PUCCH-Resource from this list as specified in TS 38.213 [13], clause 9.2.3. Note that this list contains only a list of resource IDs. The actual resources are configured in PUCCH-Config.

| Conditional Presence | Explanation |
| --- | --- |
| PI2-BPSK | The field is optionally present, Need R, if format3 and/or format4 are configured and pi2BPSK is configured in each of them. It is absent, Need R otherwise. |

#### –	PUCCH-ConfigCommon

The IE PUCCH-ConfigCommon is used to configure the cell specific PUCCH parameters.

PUCCH-ConfigCommon information element

-- ASN1START

-- TAG-PUCCH-CONFIGCOMMON-START

PUCCH-ConfigCommon ::=              SEQUENCE {

pucch-ResourceCommon                INTEGER (0..15)                                      OPTIONAL,   -- Cond InitialBWP-Only

pucch-GroupHopping                  ENUMERATED { neither, enable, disable },

hoppingId                           INTEGER (0..1023)                                    OPTIONAL,   -- Need R

p0-nominal                          INTEGER (-202..24)                                   OPTIONAL,   -- Need R

...,

[[

nrofPRBs                            INTEGER (1..16)                                      OPTIONAL,   -- Need R

intra-SlotFH-r17                    ENUMERATED {fromLowerEdge, fromUpperEdge}            OPTIONAL,   -- Cond InitialBWP-RedCapOnly

pucch-ResourceCommonRedCap-r17     INTEGER (0..15)                                      OPTIONAL,   -- Cond InitialBWP-RedCap

additionalPRBOffset-r17                 ENUMERATED {n2, n3, n4, n6, n8, n9, n10, n12}        OPTIONAL    -- Cond InitialBWP-RedCapOnly

]]

}

-- TAG-PUCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUCCH-ConfigCommon field descriptions

additionalPRBOffset When intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is disabled, each common PUCCH resource is mapped to a single PRB on one side of the UL BWP. This parameter determines an additional PRB offset in the PRB mapping for the PUCCH resource. If the field is not configured, the UE shall assume an additional PRB offset of zero.

hoppingId Cell-specific scrambling ID for group hopping and sequence hopping if enabled, see TS 38.211 [16], clause 6.3.2.2.

intra-SlotFH-r17 In case a separate initial UL BWP is configured for (e)RedCap UEs, the presence of this parameter indicates whether intra-slot PUCCH frequency hopping within the separate initial UL BWP in the common PUCCH resource is enabled for (e)RedCap UEs. If this field is absent, intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is enabled. If this field is present, intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is disabled and each PUCCH resource is mapped to a single PRB on one side of the UL BWP and this parameter determines whether the PRB index in the PRB mapping is counted in increasing order from the lower edge or in decreasing order from the upper edge of the UL BWP.

nrofPRBs Indicates the number of PRBs used per PUCCH resource for PUCCH format 0 and format 1 in FR2-2, see TS 38.213 [13], clause 9.2.1.

p0-nominal Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.2).

pucch-GroupHopping Configuration of group- and sequence hopping for all the PUCCH formats 0, 1, 3 and 4. Value neither implies neither group or sequence hopping is enabled. Value enable enables group hopping and disables sequence hopping. Value disable disables group hopping and enables sequence hopping (see TS 38.211 [16], clause 6.3.2.2).

pucch-ResourceCommon An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

pucch-ResourceCommonRedCap An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters for (e)RedCap UEs. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | The field is mandatory present in the PUCCH-ConfigCommon of the initial BWP (BWP#0) in SIB1. It is absent in other BWPs including the RedCap-specific initial uplink BWP, if configured. |
| InitialBWP-RedCap | The field is mandatory present in the PUCCH-ConfigCommon of the RedCap-specific initial BWP. It is optional present, Need R, in the PUCCH-ConfigCommon of the initial BWP configured by initialUplinkBWP. It is absent in other BWPs. |
| InitialBWP-RedCapOnly | The field is optional present, Need S, in the PUCCH-ConfigCommon of the RedCap-specific initial BWP. It is absent in other BWPs. |

#### –	PUCCH-ConfigurationList

The IE PUCCH-ConfigurationList is used to configure UE specific PUCCH parameters (per BWP) for two simultaneously constructed HARQ-ACK codebooks. See TS 38.213 [13], clause 9.1.

PUCCH-ConfigurationList information element

-- ASN1START

-- TAG-PUCCH-CONFIGURATIONLIST-START

PUCCH-ConfigurationList-r16  ::=     SEQUENCE (SIZE (1..2)) OF PUCCH-Config

-- TAG-PUCCH-CONFIGURATIONLIST-STOP

-- ASN1STOP

#### –	PUCCH-CSI-Resource

The IE PUCCH-CSI-Resource is used to indicate a PUCCH resource to use for reporting on PUCCH.

PUCCH-CSI-Resource information element

-- ASN1START

-- TAG-PUCCH-CSI-RESOURCE-START

PUCCH-CSI-Resource ::=              SEQUENCE {

uplinkBandwidthPartId               BWP-Id,

pucch-Resource                      PUCCH-ResourceId

}

-- TAG-PUCCH-CSI-RESOURCE-STOP

-- ASN1STOP

PUCCH-CSI-Resource field descriptions

pucch-Resource PUCCH resource for the associated uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. When two PUCCH-Config are configured within PUCCH-ConfigurationList, PUCCH-ResourceId in a PUCCH-CSI-Resource refers to a PUCCH-Resource in the PUCCH-Config used for HARQ-ACK with low priority.

#### –	PUCCH-PathlossReferenceRS-Id

The IE PUCCH-PathlossReferenceRS-Id is an ID for a reference signal (RS) configured as PUCCH pathloss reference (see TS 38.213 [13], clause 7.2).

PUCCH-PathlossReferenceRS-Id information element

-- ASN1START

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-START

PUCCH-PathlossReferenceRS-Id ::=            INTEGER (0..maxNrofPUCCH-PathlossReferenceRSs-1)

PUCCH-PathlossReferenceRS-Id-v1610 ::=      INTEGER (maxNrofPUCCH-PathlossReferenceRSs..maxNrofPUCCH-PathlossReferenceRSs-1-r16)

PUCCH-PathlossReferenceRS-Id-r17 ::=        INTEGER (0..maxNrofPUCCH-PathlossReferenceRSs-1-r17)

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-STOP

-- ASN1STOP

#### –	PUCCH-PowerControl

The IE PUCCH-PowerControl is used to configure UE-specific parameters for the power control of PUCCH.

PUCCH-PowerControl information element

-- ASN1START

-- TAG-PUCCH-POWERCONTROL-START

PUCCH-PowerControl ::=              SEQUENCE {

deltaF-PUCCH-f0                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f1                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f2                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f3                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f4                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

p0-Set                              SEQUENCE (SIZE (1..maxNrofPUCCH-P0-PerSet)) OF P0-PUCCH                 OPTIONAL, -- Need M

pathlossReferenceRSs                SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS

OPTIONAL, -- Need M

twoPUCCH-PC-AdjustmentStates        ENUMERATED {twoStates}                                                  OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSs-v1610          SetupRelease { PathlossReferenceRSs-v1610 }                             OPTIONAL -- Need M

]]

}

P0-PUCCH ::=                            SEQUENCE {

p0-PUCCH-Id                             P0-PUCCH-Id,

p0-PUCCH-Value                          INTEGER (-16..15)

}

P0-PUCCH-Id ::=                         INTEGER (1..8)

PathlossReferenceRSs-v1610 ::=          SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSsDiff-r16)) OF PUCCH-PathlossReferenceRS-r16

PUCCH-PathlossReferenceRS ::=                   SEQUENCE {

pucch-PathlossReferenceRS-Id                PUCCH-PathlossReferenceRS-Id,

referenceSignal                             CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

}

PUCCH-PathlossReferenceRS-r16 ::=                   SEQUENCE {

pucch-PathlossReferenceRS-Id-r16                    PUCCH-PathlossReferenceRS-Id-v1610,

referenceSignal-r16                                 CHOICE {

ssb-Index-r16                                       SSB-Index,

csi-RS-Index-r16                                    NZP-CSI-RS-ResourceId

}

}

PUCCH-PowerControlSetInfo-r17 ::=       SEQUENCE {

pucch-PowerControlSetInfoId-r17         PUCCH-PowerControlSetInfoId-r17,

p0-PUCCH-Id-r17                         P0-PUCCH-Id,

pucch-ClosedLoopIndex-r17               ENUMERATED { i0, i1 },

pucch-PathlossReferenceRS-Id-r17        PUCCH-PathlossReferenceRS-Id-r17

}

PUCCH-PowerControlSetInfoId-r17 ::=     INTEGER (1.. maxNrofPowerControlSetInfos-r17)

-- TAG-PUCCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUCCH field descriptions

p0-PUCCH-Value P0 value for PUCCH with 1dB step size.

PUCCH-PowerControl field descriptions

deltaF-PUCCH-f0 deltaF for PUCCH format 0 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f1 deltaF for PUCCH format 1 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f2 deltaF for PUCCH format 2 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f3 deltaF for PUCCH format 3 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f4 deltaF for PUCCH format 4 with 1dB step size (see TS 38.213 [13], clause 7.2).

p0-Set A set with dedicated P0 values for PUCCH, i.e., {P01, P02,... } (see TS 38.213 [13], clause 7.2).

pathlossReferenceRSs, pathlossReferenceRSs-v1610 A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUCCH pathloss estimation. Up to maxNrofPUCCH-PathlossReference-RSs may be configured. If the field is not configured, the UE uses the SSB as reference signal (see TS 38.213 [13], clause 7.2). The set includes Reference Signals indicated in pathlossReferenceRSs (without suffix) and in pathlossReferenceRSs-v1610. The UE maintains pathlossReferenceRSs and pathlossReferenceRSs-v1610 separately: Receiving pathlossReferenceRSs-v1610 set to release releases only the entries that were configured by pathlossReferenceRSs-v1610, and receiving pathlossReferenceRSs-v1610 set to setup replaces only the entries that were configured by pathlossReferenceRSs-v1610 with the newly signalled entries.

twoPUCCH-PC-AdjustmentStates Number of PUCCH power control adjustment states maintained by the UE (i.e., g(i)). If the field is present (n2) the UE maintains two power control states (i.e., g(i,0) and g(i,1)). If the field is absent, it maintains one power control state (i.e., g(i,0)) (see TS 38.213 [13], clause 7.2).

#### –	PUCCH-SpatialRelationInfo

The IE PUCCH-SpatialRelationInfo is used to configure the spatial setting for PUCCH transmission and the parameters for PUCCH power control, see TS 38.213, [13], clause 9.2.2.

PUCCH-SpatialRelationInfo information element

-- ASN1START

-- TAG-PUCCH-SPATIALRELATIONINFO-START

PUCCH-SpatialRelationInfo ::=           SEQUENCE {

pucch-SpatialRelationInfoId         PUCCH-SpatialRelationInfoId,

servingCellId                           ServCellIndex                                                    OPTIONAL,   -- Need S

referenceSignal                         CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId,

srs                                     PUCCH-SRS

},

pucch-PathlossReferenceRS-Id            PUCCH-PathlossReferenceRS-Id,

p0-PUCCH-Id                             P0-PUCCH-Id,

closedLoopIndex                         ENUMERATED { i0, i1 }

}

PUCCH-SpatialRelationInfoExt-r16 ::=       SEQUENCE {

pucch-SpatialRelationInfoId-v1610         PUCCH-SpatialRelationInfoId-v1610                              OPTIONAL,   -- Need S

pucch-PathlossReferenceRS-Id-v1610        PUCCH-PathlossReferenceRS-Id-v1610                             OPTIONAL,    --Need R

...

}

PUCCH-SRS ::=                       SEQUENCE {

resource                            SRS-ResourceId,

uplinkBWP                           BWP-Id

}

-- TAG-PUCCH-SPATIALRELATIONINFO-STOP

-- ASN1STOP

PUCCH-SpatialRelationInfo field descriptions

pucch-PathLossReferenceRS-Id When pucch-PathLossReferenceRS-Id-v1610 is configured, the UE shall ignore pucch-PathLossReferenceRS-Id (without suffix).

pucch-SpatialRelationInfoId When pucch-SpatialRelationInfoId-v1610 is configured, the UE shall ignore pucch-SpatialRelationInfoId (without suffix). If pucch-SpatialRelationInfoId-v1610 is absent, the UE shall use the pucch-SpatialRelationInfoId (without suffix).

servingCellId If the field is absent, the UE applies the ServCellId of the serving cell in which this PUCCH-SpatialRelationInfo is configured

#### –	PUCCH-SpatialRelationInfo-Id

The IE PUCCH-SpatialRelationInfo-Id is used to identify a PUCCH-SpatialRelationInfo

PUCCH-SpatialRelationInfo-Id information element

-- ASN1START

-- TAG-PUCCH-SPATIALRELATIONINFO-START

PUCCH-SpatialRelationInfoId ::=         INTEGER (1..maxNrofSpatialRelationInfos)

PUCCH-SpatialRelationInfoId-r16 ::=     INTEGER (1..maxNrofSpatialRelationInfos-r16)

PUCCH-SpatialRelationInfoId-v1610::=    INTEGER (maxNrofSpatialRelationInfos-plus-1..maxNrofSpatialRelationInfos-r16)

-- TAG-PUCCH-SPATIALRELATIONINFO-STOP

-- ASN1STOP

#### –	PUCCH-TPC-CommandConfig

The IE PUCCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUCCH from a group-TPC messages on DCI.

PUCCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUCCH-TPC-COMMANDCONFIG-START

PUCCH-TPC-CommandConfig ::=             SEQUENCE {

tpc-IndexPCell                          INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-OfSpcell

tpc-IndexPUCCH-SCell                    INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-ofSpCellOrPUCCH-SCell

...,

[[

tpc-IndexPUCCH-sSCell-r17                             INTEGER (1..15)                         OPTIONAL,  -- Need R

tpc-IndexPUCCH-sScellSecondaryPUCCHgroup-r17          INTEGER (1..15)                         OPTIONAL   -- Cond twoPUCCHgroup

]]

}

-- TAG-PUCCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUCCH-TPC-CommandConfig field descriptions

tpc-IndexPCell An index determining the position of the first bit of TPC command (applicable to the SpCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-SCell An index determining the position of the first bit of TPC command (applicable to the PUCCH SCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-sSCell, tpc-IndexPUCCH-sSCellSecondaryPUCCHgroup An index determining the position of the first bit of TPC command (applicable to the alternative PUCCH cell for PUCCH cell switching) inside the DCI format 2-2 payload, for the primary PUCCH group and the secondary PUCCH group respectively.

| Conditional Presence | Explanation |
| --- | --- |
| PDCCH-OfSpcell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |
| PDCCH-ofSpCellOrPUCCH-SCell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the PUCCH-SCell. The field is optionally present, need R, if the UE is configured with a PUCCH SCell in this cell group and if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise. |

#### –	PUSCH-Config

The IE PUSCH-Config is used to configure the UE specific PUSCH parameters applicable to a particular BWP.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

pusch-PowerControl                      PUSCH-PowerControl                                                  OPTIONAL,   -- Need M

frequencyHopping                        ENUMERATED {intraSlot, interSlot}                                   OPTIONAL,   -- Need S

frequencyHoppingOffsetLists             SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pusch-TimeDomainAllocationList          SetupRelease { PUSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pusch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

mcs-TableTransformPrecoder              ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

transformPrecoder                       ENUMERATED {enabled, disabled}                                      OPTIONAL,   -- Need S

codebookSubset                          ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL, -- Cond codebookBased

maxRank                                 INTEGER (1..4)                                                OPTIONAL, -- Cond codebookBased

rbg-Size                                ENUMERATED { config2}                                         OPTIONAL, -- Need S

uci-OnPUSCH                             SetupRelease { UCI-OnPUSCH}                                   OPTIONAL, -- Need M

tp-pi2BPSK                              ENUMERATED {enabled}                                          OPTIONAL, -- Need S

...,

[[

minimumSchedulingOffsetK2-r16           SetupRelease { MinSchedulingOffsetK2-Values-r16 }             OPTIONAL,  -- Need M

ul-AccessConfigListDCI-0-1-r16          SetupRelease { UL-AccessConfigListDCI-0-1-r16 }               OPTIONAL,  -- Need M

-- Start of the parameters for DCI format 0_2 introduced in V16.1.0

harq-ProcessNumberSizeDCI-0-2-r16                       INTEGER (0..4)                                OPTIONAL,   -- Need R

dmrs-SequenceInitializationDCI-0-2-r16                  ENUMERATED {enabled}                          OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-0-2-r16                           INTEGER (0..2)                                OPTIONAL,   -- Need R

antennaPortsFieldPresenceDCI-0-2-r16                    ENUMERATED {enabled}                          OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

frequencyHoppingDCI-0-2-r16                             CHOICE {

pusch-RepTypeA                                          ENUMERATED {intraSlot, interSlot},

pusch-RepTypeB                                          ENUMERATED {interRepetition, interSlot}

}                                                                                                     OPTIONAL,   -- Need S

frequencyHoppingOffsetListsDCI-0-2-r16  SetupRelease { FrequencyHoppingOffsetListsDCI-0-2-r16}        OPTIONAL,  -- Need M

codebookSubsetDCI-0-2-r16               ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL,   -- Cond codebookBased

invalidSymbolPatternIndicatorDCI-0-2-r16                ENUMERATED {enabled}                          OPTIONAL,   -- Need S

maxRankDCI-0-2-r16                                      INTEGER (1..4)                                OPTIONAL,   -- Cond codebookBased

mcs-TableDCI-0-2-r16                                    ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

mcs-TableTransformPrecoderDCI-0-2-r16                   ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

priorityIndicatorDCI-0-2-r16                            ENUMERATED {enabled}                          OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-2-r16                       ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}  OPTIONAL,  -- Need R

resourceAllocationDCI-0-2-r16                           ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

resourceAllocationType1GranularityDCI-0-2-r16           ENUMERATED { n2,n4,n8,n16 }                   OPTIONAL,   -- Need S

uci-OnPUSCH-ListDCI-0-2-r16                             SetupRelease { UCI-OnPUSCH-ListDCI-0-2-r16}   OPTIONAL,   -- Need M

pusch-TimeDomainAllocationListDCI-0-2-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 0_2 introduced in V16.1.0

-- Start of the parameters for DCI format 0_1 introduced in V16.1.0

pusch-TimeDomainAllocationListDCI-0-1-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

invalidSymbolPatternIndicatorDCI-0-1-r16          ENUMERATED {enabled}                                OPTIONAL,   -- Need S

priorityIndicatorDCI-0-1-r16                      ENUMERATED {enabled}                                OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-1-r16                 ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}        OPTIONAL,   -- Need R

frequencyHoppingDCI-0-1-r16                 ENUMERATED {interRepetition, interSlot}                   OPTIONAL,   -- Cond RepTypeB

uci-OnPUSCH-ListDCI-0-1-r16                 SetupRelease { UCI-OnPUSCH-ListDCI-0-1-r16  }             OPTIONAL,  -- Need M

-- End of the parameters for DCI format 0_1 introduced in V16.1.0

invalidSymbolPattern-r16                    InvalidSymbolPattern-r16                                  OPTIONAL,   -- Need S

pusch-PowerControl-v1610                SetupRelease {PUSCH-PowerControl-v1610}                       OPTIONAL,   -- Need M

ul-FullPowerTransmission-r16            ENUMERATED {fullpower, fullpowerMode1, fullpowerMode2}         OPTIONAL,   -- Need R

pusch-TimeDomainAllocationListForMultiPUSCH-r16  SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,  --  Need M

numberOfInvalidSymbolsForDL-UL-Switching-r16        INTEGER (1..4)                                    OPTIONAL    -- Cond RepTypeB2

]],

[[

ul-AccessConfigListDCI-0-2-r17          SetupRelease { UL-AccessConfigListDCI-0-2-r17 }               OPTIONAL,  -- Need M

betaOffsetsCrossPri0-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                   OPTIONAL,  -- Need M

betaOffsetsCrossPri1-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                   OPTIONAL,  -- Need M

betaOffsetsCrossPri0DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }            OPTIONAL,  -- Need M

betaOffsetsCrossPri1DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }            OPTIONAL,  -- Need M

mappingPattern-r17                      ENUMERATED {cyclicMapping, sequentialMapping}                 OPTIONAL,  -- Cond SRSsets

secondTPCFieldDCI-0-1-r17               ENUMERATED {enabled}                                          OPTIONAL,  -- Need R

secondTPCFieldDCI-0-2-r17               ENUMERATED {enabled}                                          OPTIONAL,  -- Need R

sequenceOffsetForRV-r17                 INTEGER (0..3)                                                OPTIONAL,  -- Need R

ul-AccessConfigListDCI-0-1-r17          SetupRelease { UL-AccessConfigListDCI-0-1-r17 }                OPTIONAL,  -- Need M

minimumSchedulingOffsetK2-r17           SetupRelease { MinSchedulingOffsetK2-Values-r17 }              OPTIONAL,  -- Need M

availableSlotCounting-r17               ENUMERATED { enabled }                                         OPTIONAL,  -- Need S

dmrs-BundlingPUSCH-Config-r17           SetupRelease { DMRS-BundlingPUSCH-Config-r17 }                 OPTIONAL,  -- Need M

harq-ProcessNumberSizeDCI-0-2-v1700     INTEGER (5)                                                    OPTIONAL,  -- Need R

harq-ProcessNumberSizeDCI-0-1-r17       INTEGER (5)                                                    OPTIONAL,  -- Need R

mpe-ResourcePoolToAddModList-r17       SEQUENCE (SIZE(1..maxMPE-Resources-r17)) OF MPE-Resource-r17    OPTIONAL,  -- Need N

mpe-ResourcePoolToReleaseList-r17      SEQUENCE (SIZE(1..maxMPE-Resources-r17)) OF MPE-ResourceId-r17  OPTIONAL   -- Need N

]],

[[

maxRank-v1810                           INTEGER (5..8)                                              OPTIONAL, -- Need R

sTx-2Panel-r18                          ENUMERATED {enabled}                                           OPTIONAL,  -- Need R

multipanelSchemeSDM-r18                 SDM-Scheme-r18                                                 OPTIONAL,  -- Need R

multipanelSchemeSFN-r18                 SFN-Scheme-r18                                                 OPTIONAL,  -- Need R

codebookTypeUL-r18                      SetupRelease { CodebookTypeUL-r18 }                            OPTIONAL,  -- Need M

applyIndicatedTCI-State-r18             ENUMERATED {first, second}                                     OPTIONAL,  -- Need R

dynamicTransformPrecoderFieldPresenceDCI-0-1-r18    ENUMERATED {enabled}                               OPTIONAL,  -- Need R

dynamicTransformPrecoderFieldPresenceDCI-0-2-r18    ENUMERATED {enabled}                               OPTIONAL,  -- Need R

pusch-ConfigDCI-0-3-r18                 SetupRelease { PUSCH-ConfigDCI-0-3-r18 }                       OPTIONAL   -- Need M

]]

}

UCI-OnPUSCH ::=                         SEQUENCE {

betaOffsets                             CHOICE {

dynamic                             SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic                          BetaOffsets

}                                                                                                 OPTIONAL, -- Need M

scaling                                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

MinSchedulingOffsetK2-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r16)

MinSchedulingOffsetK2-Values-r17 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r17)

UCI-OnPUSCH-DCI-0-2-r16 ::=             SEQUENCE {

betaOffsetsDCI-0-2-r16                  CHOICE {

dynamicDCI-0-2-r16                      CHOICE {

oneBit-r16                              SEQUENCE (SIZE (2)) OF BetaOffsets,

twoBits-r16                             SEQUENCE (SIZE (4)) OF BetaOffsets

},

semiStaticDCI-0-2-r16          BetaOffsets

}                                                                                                 OPTIONAL,   -- Need M

scalingDCI-0-2-r16                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

FrequencyHoppingOffsetListsDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

UCI-OnPUSCH-ListDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH-DCI-0-2-r16

UCI-OnPUSCH-ListDCI-0-1-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH

UL-AccessConfigListDCI-0-1-r16 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

UL-AccessConfigListDCI-0-1-r17 ::= SEQUENCE (SIZE (1..3)) OF INTEGER (0..2)

UL-AccessConfigListDCI-0-2-r17 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

BetaOffsetsCrossPriSel-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17          BetaOffsetsCrossPri-r17

}

BetaOffsetsCrossPriSelDCI-0-2-r17 ::= CHOICE {

dynamicDCI-0-2-r17      CHOICE {

oneBit-r17              SEQUENCE (SIZE (2)) OF BetaOffsetsCrossPri-r17,

twoBits-r17             SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17

},

semiStaticDCI-0-2-r17   BetaOffsetsCrossPri-r17

}

MPE-Resource-r17 ::=        SEQUENCE {

mpe-ResourceId-r17          MPE-ResourceId-r17,

cell-r17                    ServCellIndex                                                         OPTIONAL,    -- Need R

additionalPCI-r17           AdditionalPCIIndex-r17                                                OPTIONAL,    -- Need R

mpe-ReferenceSignal-r17     CHOICE {

csi-RS-Resource-r17         NZP-CSI-RS-ResourceId,

ssb-Resource-r17            SSB-Index

}

}

MPE-ResourceId-r17 ::=      INTEGER (1..maxMPE-Resources-r17)

SDM-Scheme-r18   ::=        SEQUENCE {

maxRankSDM-r18              INTEGER (1..2)                                                        OPTIONAL,    -- Need R

maxRankSDM-DCI-0-2-r18      INTEGER (1..2)                                                        OPTIONAL     -- Need R

}

SFN-Scheme-r18   ::=        SEQUENCE {

maxRankSFN-r18              INTEGER (1..2)                                                        OPTIONAL,    -- Need R

maxRankSFN-DCI-0-2-r18      INTEGER (1..2)                                                        OPTIONAL     -- Need R

}

CodebookTypeUL-r18 ::=      CHOICE {

codebook1-r18               ENUMERATED {ng1n4n1, ng1n2n2},

codebook2-r18               ENUMERATED {ng2},

codebook3-r18               ENUMERATED {ng4},

codebook4-r18               ENUMERATED {ng8}

}

PUSCH-ConfigDCI-0-3-r18 ::=                   SEQUENCE {

resourceAllocationDCI-0-3-r18                 ENUMERATED {resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

rbg-SizeDCI-0-3-r18                           ENUMERATED {config2, config3}                                  OPTIONAL,   -- Need S

resourceAllocationType1GranularityDCI-0-3-r18 ENUMERATED {n2,n4,n8,n16}                                      OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-0-3-r18                 INTEGER (0..2)                                                 OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-0-3-r18             INTEGER (0..5)                                                 OPTIONAL,   -- Need R

uci-OnPUSCH-ListDCI-0-3-r18                   SetupRelease { UCI-OnPUSCH-ListDCI-0-1-r16  }                  OPTIONAL    -- Need M

}

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

PUSCH-Config field descriptions

antennaPortsFieldPresenceDCI-0-2 Configure the presence of "Antenna ports" field in DCI format 0_2. When the field is configured, then the "Antenna ports" field is present in DCI format 0_2. Otherwise, the field size is set to 0 for DCI format 0_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 nor dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 is configured, this field is absent.

applyIndicatedTCI-State This field indicates, for a PUSCH transmission, if UE applies the first or the second "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.1. This field is absent if more than one value for the field coresetPoolIndex is configured in controlResourceSet for a DL BWP used with this UL BWP.

availableSlotCounting Indicate whether PUSCH repetitions counted on the basis of available slots is enabled. If the field is absent, PUSCH repetitions counted on the basis of available slots is disabled.

betaOffsetsCrossPri0, betaOffsetsCrossPri1, betaOffsetsCrossPri0DCI-0-2, betaOffsetsCrossPri1DCI-0-2 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK on dynamically scheduled PUSCH with different priorities, see TS 38.213 [13], clause 9.3. The field betaOffsetsCrossPrio0 indicates multiplexing low priority (LP) HARQ-ACK on dynamically scheduled high priority (HP) PUSCH. The field betaOffsetsCrossPrio1 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH. The field betaOffsetsCrossPrio0DCI-0-2 indicates multiplexing LP HARQ-ACK on dynamically scheduled HP PUSCH by DCI format 0_2. The field betaOffsetsCrossPrio1DCI-0-2 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH by DCI format 0_2.

codebookSubset, codebookSubsetDCI-0-2 Subset of PMIs addressed by TPMI, where PMIs are those supported by UEs with maximum coherence capabilities (see TS 38.214 [19], clause 6.1.1.1). The field codebookSubset applies to DCI formats 0_1 and 0_3, and the field codebookSubsetDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1).

codebookTypeUL Configures a codebook and the corresponding number of antenna port groups for codebook-based transmission of PUSCH with 8 antenna ports, see TS 38.211 [16], tables 6.3.1.5-9 to 6.3.1.5-47, and table 6.3.1.5-8 respectively). The values ng1n4n1 and ng1n2n2 correspond to codebooks with one antenna port group (Ng=1), while ng2, ng4, and ng8 correspond to codebooks with Ng=2, 4, and 8 antenna port groups, respectively.

dataScramblingIdentityPUSCH Identifier used to initialise data scrambling (c_init) for PUSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 6.3.1.1).

dmrs-BundlingPUSCH-Config Configure the parameters for DMRS bundling for PUSCH (see TS 38.214 [19], clause 6.1.7). In this release, this is not applicable to FR2-2.

dmrs-SequenceInitializationDCI-0-2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 0_2. If the field is absent, then 0 bit for the field "DMRS Sequence Initialization" in DCI format 0_2. If the field is present, then the number of bits is determined in the same way as DCI format 0_1 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeA, dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type A (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeA applies to DCI formats 0_1 and 0_3, and the field dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeB, dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type B (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeB applies to DCI formats 0_1 and 0_3, and the field dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

dynamicTransformPrecoderFieldPresenceDCI-0-1 Configure the presence of "Dynamic Transform Precoder" field in DCI format 0_1. When the field is configured, then the "Dynamic Transform Precoder" field is present in DCI format 0_1. Otherwise, the field size is set to 0 for DCI format 0_1 (See TS 38.212 [17]). The network ensures dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 and twoPHRMode-r17 cannot be configured at the same time for a UE.

dynamicTransformPrecoderFieldPresenceDCI-0-2 Configure the presence of "Dynamic Transform Precoder" field in DCI format 0_2. When the field is configured, then the "Dynamic Transform Precoder" field is present in DCI format 0_2. Otherwise, the field size is set to 0 for DCI format 0_2 (See TS 38.212 [17]). The network ensures dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 and twoPHRMode-r17 cannot be configured at the same time for a UE.

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3). The field frequencyHopping applies to DCI formats 0_0, 0_1 and 0_3 for 'pusch-RepTypeA'.

frequencyHoppingDCI-0-1 Indicates the frequency hopping scheme for DCI format 0_1 when pusch-RepTypeIndicatorDCI-0-1 is set to 'pusch-RepTypeB', The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for DCI format 0_1 for 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.1).

frequencyHoppingDCI-0-2 Indicate the frequency hopping scheme for DCI format 0_2. The value intraSlot enables 'intra-slot frequency hopping', and the value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. When pusch-RepTypeIndicatorDCI-0-2 is not set to 'pusch-RepTypeB', the frequency hopping scheme can be chosen between 'intra-slot frequency hopping and 'inter-slot frequency hopping' if enabled. When pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the frequency hopping scheme can be chosen between 'inter-repetition frequency hopping' and 'inter-slot frequency hopping' if enabled. If the field is absent, frequency hopping is not configured for DCI format 0_2 (see TS 38.214 [19], clause 6.3).

frequencyHoppingOffsetLists, frequencyHoppingOffsetListsDCI-0-2 Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 configured grant activation (see TS 38.214 [19], clause 6.3). The field frequencyHoppingOffsetLists applies to DCI formats 0_0, 0_1 and 0_3, and the field frequencyHoppingOffsetListsDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.3).

harq-ProcessNumberSizeDCI-0-2 Configure the number of bits for the field "HARQ process number" in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

invalidSymbolPattern Indicates one pattern for invalid symbols for PUSCH transmission repetition type B applicable to both DCI format 0_1 and 0_2. If InvalidSymbolPattern is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols (see TS 38.214 [19] clause 6.1).

invalidSymbolPatternIndicatorDCI-0-1, invalidSymbolPatternIndicatorDCI-0-2 Indicates the presence of an additional bit in the DCI format 0_1/0_2. If invalidSymbolPattern is absent, then both invalidSymbolPatternIndicatorDCI-0-1 and invalidSymbolPatternIndicatorDCI-0-2 are absent. The field invalidSymbolPatternIndicatorDCI-0-1 applies to the DCI format 0_1 and the field invalidSymbolPatternIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19] clause 6.1). If the field is absent, the UE behaviour is specified in TS 38.214 [19], clause 6.1.2.1.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern for when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' for PUSCH transmission and the PUSCH transmission occasions are associated with both SRS resource sets.

maxRank, maxRankDCI-0-2 Subset of PMIs addressed by TRIs from 1 to ULmaxRank (see TS 38.214 [19], clause 6.1.1.1). The field maxRank applies to DCI formats 0_1 and 0_3, and the field maxRankDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1). If network configures maxRank-v1810 UE ignores maxRank (without suffix).

mcs-Table, mcs-TableFormat0-2 Indicates which MCS table the UE shall use for PUSCH without transform precoder (see TS 38.214 [19], clause 6.1.4.1). If the field is absent the UE applies the value 64QAM. The field mcs-Table applies to DCI formats 0_0, 0_1 and 0_3, and the field mcs-TableDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

mcs-TableTransformPrecoder, mcs-TableTransformPrecoderDCI-0-2 Indicates which MCS table the UE shall use for PUSCH with transform precoding (see TS 38.214 [19], clause 6.1.4.1) If the field is absent the UE applies the value 64QAM. The field mcs-TableTransformPrecoder applies to DCI format 0_0, 0_1 and 0_3, and the field mcs-TableTransformPrecoderDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

minimumSchedulingOffsetK2 List of minimum K2 values. Minimum K2 parameter denotes minimum applicable value(s) for the Time domain resource assignment table for PUSCH (see TS 38.214 [19], clause 6.1.2.1).

mpe-ResourcePoolToAddModList List of SSB/CSI-RS resources for P-MPR reporting. Each resource is configured with serving cell index where the resource is configured for the UE. The additionalPCI is configured only if the resource is SSB. For each resource, if neither cell nor additionalPCI is present, the SSB/CSI-RS resource is from the serving cell where the PUSCH-Config is configured.

multipanelSchemeSDM Configures UE with a multiple panel simultaneous uplink transmission SDM scheme for PUSCH. Network does not configure multipanelSchemeSDM with multipanelSchemeSFN. When this paramater is configured, more than one value for the field coresetPoolIndex are configured and two SRS resource sets for codebook or noncodebook are configured.

multipanelSchemeSFN Configures UE with a multiple panel simultaneous uplink transmission SFN scheme for PUSCH. Network does not configure multipanelSchemeSFN with multipanelSchemeSDM. When this paramater is configured, more than one value for the field coresetPoolIndex are configured and two SRS resource sets for codebook or noncodebook are configured.

numberOfBitsForRV-DCI-0-2 Configures the number of bits for "Redundancy version" in the DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 6.1.2.1).

numberOfInvalidSymbolsForDL-UL-Switching Indicates the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B. If it is absent, no symbol is explicitly defined for DL-to-UL switching (see TS 38.214 [19], clause 6.1).

priorityIndicatorDCI-0-1, priorityIndicatorDCI-0-2 Configures the presence of "priority indicator" in DCI format 0_1/0_2. When the field is absent in the IE, then the UE shall apply 0 bit for "Priority indicator" in DCI format 0_1/0_2. The field priorityIndicatorDCI-0-1 applies to DCI format 0_1 and the field priorityIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17] clause 7.3.1 and TS 38.213 [13] clause 9).

pusch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 6.1.2.1). If the field is absent the UE applies the value 1.

pusch-PowerControl Configures power control parameters PUSCH transmission. This field is not configured if unifiedTCI-StateType is configured for the serving cell.

pusch-RepTypeIndicatorDCI-0-1, pusch-RepTypeIndicatorDCI-0-2 Indicates whether UE follows the behavior for "PUSCH repetition type A" or the behavior for "PUSCH repetition type B" for the PUSCH scheduled by DCI format 0_1/0_2 and for Type 2 CG associated with the activating DCI format 0_1/0_2.The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B'. The field pusch-RepTypeIndicatorDCI-0-1 applies to DCI format 0_1 and the field pusch-RepTypeIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2.1).

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1). The field pusch-TimeDomainAllocationList applies to DCI format 0_0, or DCI formats 0_1 and 0_3 when the field pusch-TimeDomainAllocationListDCI-0-1 is not configured (see TS 38.214 [19], table 6.1.2.1.1-1 and tables 6.1.2.1.1-1A and 6.1.2.1.1-1C). The network does not configure the pusch-TimeDomainAllocationList (without suffix) simultaneously with the pusch-TimeDomainAllocationListDCI-0-2-r16 or pusch-TimeDomainAllocationListDCI-0-1-r16 or pusch-TimeDomainAllocationListForMultiPUSCH-r16.

pusch-TimeDomainAllocationListDCI-0-1 Configuration of the time domain resource allocation (TDRA) table for DCI formats 0_1 and 0_3 (see TS 38.214 [19], clause 6.1, tables 6.1.2.1.1-1A and 6.1.2.1.1-1C).

pusch-TimeDomainAllocationListDCI-0-2 Configuration of the time domain resource allocation (TDRA) table for DCI format 0_2 (see TS 38.214 [19], clause 6.1.2, table 6.1.2.1.1-1B).

pusch-TimeDomainAllocationListForMultiPUSCH Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 64 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the pusch-TimeDomainAllocationListDCI-0-1-r16. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the numberOfSlotsTBoMS-r17.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH except PUSCH scheduled by DCI format 0_3. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocation, resourceAllocationDCI-0-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 6.1.2). The field resourceAllocation applies to DCI format 0_1 and the field resourceAllocationDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2).

resourceAllocationType1GranularityDCI-0-2 Configures the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 0_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 6.1.2.2.2).

secondTPCFieldDCI-0-1, secondTPCFieldDCI-0-2 A second TPC field can be configured via RRC for DCI-0-1 and DCI-0-2. Each TPC field is for each closed-loop index value respectively (i.e., 1st /2nd TPC fields correspond to "closedLoopIndex" value = 0 and 1,

sequenceOffsetForRV Configures the RV offset for the starting RV for the first repetition (first actual repetition in PUSCH repetition Type B) towards the second 'SRS resource set' for PUSCH configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

sTx-2Panel Parameter to enable PUSCH+PUSCH multiple panel simultaneous uplink transmission in multi-DCI based mTRP system. When this paramater is configured, more than one value for the field coresetPoolIndex is configured and two SRS resource sets for codebook or noncodebook are configured.

tp-pi2BPSK Enables pi/2-BPSK modulation with transform precoding if the field is present and disables it otherwise.

transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3-transformPrecoder from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList).

txConfig Whether UE uses codebook based or non-codebook based transmission (see TS 38.214 [19], clause 6.1.1). If the field is absent, the UE transmits PUSCH on one antenna port, see TS 38.214 [19], clause 6.1.1.

uci-OnPUSCH-ListDCI-0-1, uci-OnPUSCH-ListDCI-0-2 Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_1/0_2. The field uci-OnPUSCH-ListDCI-0-1 applies to DCI format 0_1 and the field uci-OnPUSCH-ListDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 List of the combinations of cyclic prefix extension, channel access priority class (CAPC), and UL channel access type (see TS 38.212 [17], clause 7.3.1) applicable for DCI format 0_1 and DCI format 0_2, respectively. The fields ul-AccessConfigListDCI-0-1-r16 and ul-AccessConfigListDCI-0-2-r17 are only applicable for FR1 (see TS 38.212 [17], Table 7.3.1.1.2-35). The field ul-AccessConfigListDCI-0-1-r17 only contains a list of UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.1.2-35A).

ul-FullPowerTransmission Configures the UE with UL full power transmission mode as specified in TS 38.213 [13]. This field is not configured if ul-powerControl is configured in the BWP-UplinkDedicated in which the PUCCH-Config is included.

PUSCH-ConfigDCI-0-3 field descriptions

harq-ProcessNumberSizeDCI-0-3 Configure the number of bits for the field "HARQ process number" in DCI format 0_3 (see TS 38.212 [17], clause 7.3.1).

numberOfBitsForRV-DCI-0-3 Configures the number of bits for "Redundancy version" in the DCI format 0_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 6.1.2.1).

rbg-SizeDCI-0-3 Selection among configuration 1, configuration 2 and configuration 3 for RBG size for PUSCH scheduled by DCI format 0_3. The UE does not apply this field if resourceAllocationDCI-0-3 is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocationDCI-0-3 Configuration of resource allocation type 0 and resource allocation type 1 for DCI format 0_3 (see TS 38.214 [19], clause 6.1.2).

resourceAllocationType1GranularityDCI-0-3 Configures the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 0_3. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 6.1.2.2.2).

uci-OnPUSCH-ListDCI-0-3 Selection between and configuration of dynamic and semi-static beta-offset for DCI format 0_3.

SDM-Scheme field descriptions

maxRankSDM, maxRankSDM-DCI-0-2 configure maximal number of MIMO layers of each panel in SDM scheme for codebook based PUSCH or for DCI 0_2 for codebook based PUSCH.

SFN-Scheme field descriptions

maxRankSFN, maxRankSFN-DCI-0-2 configure maximal number of MIMO layers of each panel in SFN scheme for codebook based PUSCH or for DCI 0_2 for codebook based PUSCH.

UCI-OnPUSCH field descriptions

betaOffsets Selection between and configuration of dynamic and semi-static beta-offset for DCI formats other than DCI format 0_2. If the field is not configured, the UE applies the value 'semiStatic' (see TS 38.213 [13], clause 9.3).

scaling Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI formats other than DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on. The value configured herein is applicable for PUSCH with configured grant (see TS 38.212 [17], clause 6.3).

UCI-OnPUSCH-DCI-0-2 field descriptions

betaOffsetsDCI-0-2 Configuration of beta-offset for DCI format 0_2. If semiStaticDCI-0-2 is chosen, the UE shall apply the value of 0 bit for the field of beta offset indicator in DCI format 0_2. If dynamicDCI-0-2 is chosen, the UE shall apply the value of 1 bit or 2 bits for the field of beta offset indicator in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

dynamicDCI-0-2 Indicates the UE applies the value 'dynamic' for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.3).

semiStaticDCI-0-2 Indicates the UE applies the value 'semiStatic' for DCI format 0_2. (see TS 38.212 [17], clause 7.3.1 and see TS 38.213 [13], clause 9.3).

scalingDCI-0-2 Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on (see TS 38.212 [17], clause 6.3).

| Conditional Presence | Explanation |
| --- | --- |
| codebookBased | The field is mandatory present if txConfig is set to codebook and absent otherwise. |
| RepTypeB | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 is set to pusch-RepTypeB. It is absent otherwise. |
| RepTypeB2 | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 or pusch-RepTypeIndicatorDCI-0-2 is set to pusch-RepTypeB. It is absent otherwise. |
| SRSsets | This field is mandatory present when UE is configured with two SRS sets in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage codebook or non-codebook. |

#### –	PUSCH-ConfigCommon

The IE PUSCH-ConfigCommon is used to configure the cell specific PUSCH parameters.

PUSCH-ConfigCommon information element

-- ASN1START

-- TAG-PUSCH-CONFIGCOMMON-START

PUSCH-ConfigCommon ::=                  SEQUENCE {

groupHoppingEnabledTransformPrecoding   ENUMERATED {enabled}                                                OPTIONAL,   -- Need R

pusch-TimeDomainAllocationList          PUSCH-TimeDomainResourceAllocationList                              OPTIONAL,   -- Need R

msg3-DeltaPreamble                      INTEGER (-1..6)                                                     OPTIONAL,   -- Need R

p0-NominalWithGrant                     INTEGER (-202..24)                                                  OPTIONAL,   -- Need R

...

}

-- TAG-PUSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUSCH-ConfigCommon field descriptions

groupHoppingEnabledTransformPrecoding For DMRS transmission with transform precoder, the NW may configure group hopping by this cell-specific parameter, see TS 38.211 [16], clause 6.4.1.1.1.2.

msg3-DeltaPreamble Power offset between msg3 and RACH preamble transmission. Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1)

p0-NominalWithGrant P0 value for PUSCH with grant (except msg3). Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.1) This field is cell specific

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1).

#### –	PUSCH-PowerControl

The IE PUSCH-PowerControl is used to configure UE specific power control parameter for PUSCH.

PUSCH-PowerControl information element

-- ASN1START

-- TAG-PUSCH-POWERCONTROL-START

PUSCH-PowerControl ::=              SEQUENCE {

tpc-Accumulation                    ENUMERATED { disabled }                                                 OPTIONAL, -- Need S

msg3-Alpha                          Alpha                                                                   OPTIONAL, -- Need S

p0-NominalWithoutGrant              INTEGER (-202..24)                                                      OPTIONAL, -- Need M

p0-AlphaSets                        SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet     OPTIONAL, -- Need M

pathlossReferenceRSToAddModList     SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseList    SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS-Id

OPTIONAL,  -- Need N

twoPUSCH-PC-AdjustmentStates        ENUMERATED {twoStates}                                                  OPTIONAL, -- Need S

deltaMCS                            ENUMERATED {enabled}                                                    OPTIONAL, -- Need S

sri-PUSCH-MappingToAddModList       SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl

OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList      SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId

OPTIONAL  -- Need N

}

P0-PUSCH-AlphaSet ::=               SEQUENCE {

p0-PUSCH-AlphaSetId                 P0-PUSCH-AlphaSetId,

p0                                  INTEGER (-16..15)                                                       OPTIONAL, -- Need S

alpha                               Alpha                                                                   OPTIONAL  -- Need S

}

P0-PUSCH-AlphaSetId ::=             INTEGER (0..maxNrofP0-PUSCH-AlphaSets-1)

PUSCH-PathlossReferenceRS ::=       SEQUENCE {

pusch-PathlossReferenceRS-Id        PUSCH-PathlossReferenceRS-Id,

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

}

}

PUSCH-PathlossReferenceRS-r16 ::=   SEQUENCE {

pusch-PathlossReferenceRS-Id-r16    PUSCH-PathlossReferenceRS-Id-v1610,

referenceSignal-r16                 CHOICE {

ssb-Index-r16                       SSB-Index,

csi-RS-Index-r16                    NZP-CSI-RS-ResourceId

}

}

DummyPathlossReferenceRS-v1710 ::= SEQUENCE {

pusch-PathlossReferenceRS-Id-r17    PUSCH-PathlossReferenceRS-Id-r17,

additionalPCI-r17                   AdditionalPCIIndex-r17                                         OPTIONAL  -- Need R

}

PUSCH-PathlossReferenceRS-Id ::=    INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)

PUSCH-PathlossReferenceRS-Id-v1610 ::= INTEGER (maxNrofPUSCH-PathlossReferenceRSs..maxNrofPUSCH-PathlossReferenceRSs-1-r16)

PUSCH-PathlossReferenceRS-Id-r17 ::= INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1-r16)

SRI-PUSCH-PowerControl ::=          SEQUENCE {

sri-PUSCH-PowerControlId            SRI-PUSCH-PowerControlId,

sri-PUSCH-PathlossReferenceRS-Id    PUSCH-PathlossReferenceRS-Id,

sri-P0-PUSCH-AlphaSetId             P0-PUSCH-AlphaSetId,

sri-PUSCH-ClosedLoopIndex           ENUMERATED { i0, i1 }

}

SRI-PUSCH-PowerControlId ::=        INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

PUSCH-PowerControl-v1610 ::=        SEQUENCE {

pathlossReferenceRSToAddModListSizeExt-v1610   SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSsDiff-r16)) OF PUSCH-PathlossReferenceRS-r16

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseListSizeExt-v1610  SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSsDiff-r16)) OF PUSCH-PathlossReferenceRS-Id-v1610

OPTIONAL, -- Need N

p0-PUSCH-SetList-r16                SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF P0-PUSCH-Set-r16      OPTIONAL, -- Need R

olpc-ParameterSet                   SEQUENCE {

olpc-ParameterSetDCI-0-1-r16        INTEGER (1..2)                                                      OPTIONAL, -- Need R

olpc-ParameterSetDCI-0-2-r16        INTEGER (1..2)                                                      OPTIONAL  -- Need R

}                                                                                                           OPTIONAL, -- Need M

...,

[[

sri-PUSCH-MappingToAddModList2-r17  SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl

OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList2-r17 SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId OPTIONAL, -- Need N

p0-PUSCH-SetList2-r17               SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF P0-PUSCH-Set-r16      OPTIONAL, -- Need R

dummy SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs-r16)) OF DummyPathlossReferenceRS-v1710          OPTIONAL  -- Need N

]]

}

P0-PUSCH-Set-r16 ::=                SEQUENCE {

p0-PUSCH-SetId-r16                  P0-PUSCH-SetId-r16,

p0-List-r16                         SEQUENCE (SIZE (1..maxNrofP0-PUSCH-Set-r16)) OF P0-PUSCH-r16            OPTIONAL, -- Need R

...

}

P0-PUSCH-SetId-r16 ::=              INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

P0-PUSCH-r16 ::=                    INTEGER (-16..15)

-- TAG-PUSCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUSCH-AlphaSet field descriptions

alpha alpha value for PUSCH with grant (except msg3) (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

p0 P0 value for PUSCH with grant (except msg3) in steps of 1dB (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 0.

P0-PUSCH-Set field descriptions

p0-List Configuration of {p0-PUSCH, p0-PUSCH} sets for PUSCH. If SRI is present in the DCI, then one p0-PUSCH can be configured in P0-PUSCH-Set. If SRI is not present in the DCI, and both olpc-ParameterSetDCI-0-1 and olpc-ParameterSetDCI-0-2 are configured to be 1 bit, then one p0-PUSCH can be configured in P0-PUSCH-Set. If SRI is not present in the DCI, and if any of olpc-ParameterSetDCI-0-1 and olpc-ParameterSetDCI-0-2 is configured to be 2 bits, then two p0-PUSCH values can be configured in P0-PUSCH-Set (see TS 38.213 [13] clause 7 and TS 38.212 [17] clause 7.3.1).

p0-PUSCH-SetId Configure the index of a p0-PUSCH-Set (see TS 38.213 [13] clause 7 and TS 38.212 [17] clause 7.3.1).

PUSCH-PowerControl field descriptions

deltaMCS Indicates whether to apply delta MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH (see TS 38.213 [13], clause 7.1).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

msg3-Alpha Dedicated alpha value for msg3 PUSCH (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

olpc-ParameterSetDCI-0-1, olpc-ParameterSetDCI-0-2 Configures the number of bits for Open-loop power control parameter set indication for DCI format 0_1/0_2 in case SRI is not configured in the DCI. 2 bits is applicable only if SRI is not present in the DCI format 0_1. The field olpc-ParameterSetDCI-0-1 applies to DCI format 0_1 and the field olpc-ParameterSetDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11).

p0-AlphaSets Configuration {p0-pusch, alpha} sets for PUSCH (except msg3 and msgA PUSCH), i.e., { {p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3/msgA PUSCH, P0-UE is set to 0 and alpha is set according to either msg3-Alpha or msgA-Alpha (see TS 38.213 [13], clause 7.1).

p0-NominalWithoutGrant P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.1).

p0-PUSCH-SetList Configure one additional P0-PUSCH-Set per SRI. If present, the one bit or 2 bits in the DCI is used to dynamically indicate among the P0 value from the existing P0-PUSCH-AlphaSet and the P0 value(s) from the P0-PUSCH-Set (See TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 17).

p0-PUSCH-SetList2 For indicating per-TRP OLPC set in DCI format 0_1/0_2 with the open-loop power control parameter set indication field, a second p0-PUSCH-SetList-r16 is used. When this field is present the p0-PUSCH-SetList-r16 corresponds to the first SRS resource set (see TS 38.213 [13]).

pathlossReferenceRSToAddModList, pathlossReferenceRSToAddModListSizeExt A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUSCH path loss estimation. The set consists of Reference Signals configured using pathLossReferenceRSToAddModList and Reference Signals configured using pathlossReferenceRSToAddModListSizeExt. Up to maxNrofPUSCH-PathlossReferenceRSs may be configured (see TS 38.213 [13], clause 7.1).

pathlossReferenceRSToReleaseList, pathlossReferenceRSToReleaseListSizeExt Lists of reference signals for PUSCH path loss estimation to be released by the UE.

sri-PUSCH-MappingToAddModList A list of SRI-PUSCH-PowerControl elements among which one is selected by the SRI field in DCI (see TS 38.213 [13], clause 7.1).

sri-PUSCH-MappingToAddModList2 A list of SRI-PUSCH-PowerControl elements for second SRS-resource set, among which one is selected by the SRI field in DCI (see TS 38.213 [13], clause 7.1). When this field is present the sri-PUSCH-MappingToAddModList corresponds to the first SRS resource set for PUSCH.

tpc-Accumulation If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation. If the field is absent, TPC accumulation is enabled (see TS 38.213 [13], clause 7.1).

twoPUSCH-PC-AdjustmentStates Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains two power control states (i.e., fc(i,0) and fc(i,1)). If the field is absent, it maintains one power control state (i.e., fc(i,0)) (see TS 38.213 [13], clause 7.1).

SRI-PUSCH-PowerControl field descriptions

sri-P0-PUSCH-AlphaSetId The ID of a P0-PUSCH-AlphaSet as configured in p0-AlphaSets in PUSCH-PowerControl.

sri-PUSCH-ClosedLoopIndex The index of the closed power control loop associated with this SRI-PUSCH-PowerControl.

sri-PUSCH-PathlossReferenceRS-Id The ID of PUSCH-PathlossReferenceRS as configured in the pathlossReferenceRSToAddModList in PUSCH-PowerControl.

sri-PUSCH-PowerControlId The ID of this SRI-PUSCH-PowerControl configuration. It is used as the codepoint (payload) in the SRI DCI field.

#### –	PUSCH-ServingCellConfig

The IE PUSCH-ServingCellConfig is used to configure UE specific PUSCH parameters that are common across the UE's BWPs of one serving cell.

PUSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PUSCH-SERVINGCELLCONFIG-START

PUSCH-ServingCellConfig ::=             SEQUENCE {

codeBlockGroupTransmission              SetupRelease { PUSCH-CodeBlockGroupTransmission }       OPTIONAL,   -- Need M

rateMatching                            ENUMERATED {limitedBufferRM}                            OPTIONAL,   -- Need S

xOverhead                               ENUMERATED {xoh6, xoh12, xoh18}                         OPTIONAL,   -- Need S

...,

[[

maxMIMO-Layers                          INTEGER (1..4)                                          OPTIONAL,   -- Need M

processingType2Enabled                  BOOLEAN                                                 OPTIONAL    -- Need M

]],

[[

maxMIMO-LayersDCI-0-2-r16               SetupRelease { MaxMIMO-LayersDCI-0-2-r16}               OPTIONAL    -- Need M

]],

[[

nrofHARQ-ProcessesForPUSCH-r17          ENUMERATED {n32}                                        OPTIONAL,   -- Need R

uplinkHARQ-mode-r17                     SetupRelease { UplinkHARQ-mode-r17}                     OPTIONAL    -- Need M

]],

[[

maxMIMO-Layers-v1810                    INTEGER (5..8)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSDM-r18                INTEGER (1..2)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSDM-DCI-0-2-r18        INTEGER (1..2)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSFN-r18                INTEGER (1..2)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSFN-DCI-0-2-r18        INTEGER (1..2)                                          OPTIONAL    -- Need R

]]

}

PUSCH-CodeBlockGroupTransmission ::=    SEQUENCE {

maxCodeBlockGroupsPerTransportBlock     ENUMERATED {n2, n4, n6, n8},

...

}

MaxMIMO-LayersDCI-0-2-r16 ::=           INTEGER (1..4)

UplinkHARQ-mode-r17 ::=                 BIT STRING (SIZE (32))

-- TAG-PUSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PUSCH-CodeBlockGroupTransmission field descriptions

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB (see TS 38.213 [13], clause 9.1).

PUSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see TS 38.214 [19], clause 5.1.5). The network does not configure this field if the SCS of at least one UL BWP configured in the cell is 480 or 960 kHz.

maxMIMO-Layers Indicates the maximum MIMO layer to be used for PUSCH in all BWPs of the corresponding UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRank to the same value. The field maxMIMO-Layers refers to DCI format 0_1. If network configures maxMIMO-Layers-v1810 the UE ignores maxMIMO-Layers (without suffix).

maxMIMO-LayersforSDM Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SDM scheme for NCB PUSCH

maxMIMO-LayersforSDM-DCI-0-2 Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SDM scheme for DCI format 0_2 for NCB PUSCH

maxMIMO-LayersforSFN Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SFN scheme for NCB PUSCH

maxMIMO-LayersforSFN-DCI-0-2 Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SFN scheme for DCI format 0_2 for NCB PUSCH

nrofHARQ-ProcessesForPUSCH The number of HARQ processes to be used on the PUSCH of a serving cell. Value n32 corresponds to 32 HARQ processes. If the field is absent, the UE uses 16 HARQ processes (see TS 38.214 [19], clause 6.1).

processingType2Enabled Enables configuration of advanced processing time capability 2 for PUSCH (see 38.214 [19], clause 6.4).

rateMatching Enables LBRM (Limited buffer rate-matching). When the field is absent the UE applies FBRM (Full buffer rate-matchingLBRM) (see TS 38.212 [17], clause 5.4.2).

xOverhead If the field is absent, the UE applies the value 'xoh0' (see TS 38.214 [19], clause 5.1.3.2).

maxMIMO-LayersDCI-0-2 Indicates the maximum MIMO layer to be used for PUSCH for DCI format 0_2 in all BWPs of the corresponding UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRankDCI-0-2 to the same value.

uplinkHARQ-mode Used to set the HARQ mode per HARQ process ID, see TS 38.321 [3]. The first/leftmost bit corresponds to HARQ process ID 0, the next bit to HARQ process ID 1 and so on. Bits corresponding to HARQ process IDs that are not configured shall be ignored. A bit set to one identifies a HARQ process with HARQmodeA and a bit set to zero identifies a HARQ process with HARQ modeB. This field applies for SRBs and DRBs.

#### –	PUSCH-TimeDomainResourceAllocationList

The IE PUSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PUSCH. PUSCH-TimeDomainResourceAllocationList contains one or more of such PUSCH-TimeDomainResourceAllocations. The network indicates in the UL grant which of the configured time domain allocations the UE shall apply for that UL grant. The UE determines the bit width of the DCI field based on the number of entries in the PUSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PUSCH-TimeDomainResourceAllocation information element

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::=  SEQUENCE {

k2                                      INTEGER(0..32)                                  OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

PUSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations-r16)) OF PUSCH-TimeDomainResourceAllocation-r16

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k2-r16                                     INTEGER(0..32)          OPTIONAL,   -- Need S

puschAllocationList-r16                    SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF PUSCH-Allocation-r16,

...

}

PUSCH-Allocation-r16 ::=  SEQUENCE {

mappingType-r16                           ENUMERATED {typeA, typeB}                     OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbolAndLength-r16                  INTEGER (0..127)                              OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbol-r16                           INTEGER (0..13)                               OPTIONAL,   -- Cond RepTypeB

length-r16                                INTEGER (1..14)                               OPTIONAL,   -- Cond RepTypeB

numberOfRepetitions-r16                   ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL,   -- Cond Format01-02

...,

[[

numberOfRepetitionsExt-r17                ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16, n20, n24, n28, n32, spare4, spare3, spare2,

spare1}                           OPTIONAL,   -- Cond Format01-02-For-TypeA

numberOfSlotsTBoMS-r17                   ENUMERATED {n1, n2, n4, n8, spare4, spare3, spare2, spare1}   OPTIONAL,   -- Need R

extendedK2-r17                            INTEGER (0..128)                              OPTIONAL    -- Cond MultiPUSCH

]]

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PUSCH-TimeDomainResourceAllocationList field descriptions

extendedK2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) configurable per PUSCH allocation. Only values {0..32} are applicable for PUSCH SCS of 120 kHz and for FR1. When the field is absent for the first PUSCH if multiple PUSCH are configured per PDCCH and k2-r16 is absent, or when the field is absent and only one PUSCH is configured per PDCCH and k2-r16 is absent, the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, the value 3 when PUSCH SCS is 120 kHz, the value 11 when PUSCH SCS is 480 kHz, and the value 21 when PUSCH SCS is 960 kHz. If multiple contiguous PUSCHs are configured per PDCCH, when the field extendedK2(n) corresponding to k2 of the PUSCH(s) in the n-th slot (n>1), or of the PUSCH(s) except the first PUSCH in the first slot (n=1), is absent, the UE applies k2 of the first PUSCH plus n-1.

k2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1). When the field is absent the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, and the value 3 when PUSCH SCS is 120 kHz. k2 is absent/ignored if extendedK2 is present.

length Indicates the length allocated for PUSCH for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1).

mappingType Mapping type (see TS 38.214 [19], clause 6.1.2.1).

numberOfRepetitions Number of repetitions for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1). When numberOfSlotsTBoMS-r17 is set to 2, 4 or 8 (i.e. TB processing over multi-slot (TBoMS) PUSCH is enabled), it indicates the number of repetitions of a single TBoMS.

numberOfRepetitionsExt Number of repetitions for DCI format 0_1/0_2 if pusch-RepTypeIndicatorDCI-0-1/pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB (see TS 38.214 [19], clause 6.1.2.1). If this field is present, the field numberOfRepeitions-r16 is ignored for PUSCH repetition Type A.

numberOfSlotsTBoMS Number of slots allocated for TB processing over multi-slot PUSCH for DCI format 0_1/0_2. If a number of repetitions K is configured by numberOfRepetitions or numberOfRepetitionsExt, the network configures numberOfSlotsTBoMS (N) and K such that N*K ≤ 32 (see TS 38.214 [19], clause 6.1.2.1). The network does not configure the numberOfSlotsTBoMS-r17 simultaneously with the pusch-TimeDomainAllocationListForMultiPUSCH-r16. The network does not configure numberOfSlotsTBoMS-r17 together with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true.

puschAllocationList The field puschAllocationList-r16 indicates one or multiple PUSCH continuous in time domain which share a common k2 (see TS 38.214 [19], clause 6.1.2.1). In this release, this field configures one or multiple PUSCH that may be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 6.1.2.1). The puschAllocationList-r16 only has one element in pusch-TimeDomainAllocationListDCI-0-1-r16 and in pusch-TimeDomainAllocationListDCI-0-2-r16.

startSymbol Indicates the index of start symbol for PUSCH for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1).

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary. (see TS 38.214 [19], clause 6.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Format01-02 | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1 and in pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present. |
| Format01-02-For-TypeA | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1, the field is optionally present if pusch-RepTypeIndicatorDCI-0-1 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is optionally present if pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. |
| NotFormat01-02-Or-TypeA | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is mandatory present. In pusch-TimeDomainAllocationListDCI-0-1, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-1 is not set to pusch-RepTypeB. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB. It is absent otherwise, Need R. |
| RepTypeB | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-1 is set to pusch-RepTypeB. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-2 is set to pusch-RepTypeB. It is absent otherwise, Need R. |
| MultiPUSCH | In case size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present for all n, if any two consecutive PUSCHs are non-contiguous. Otherwise, it is optionally present, Need S. |

#### –	PUSCH-TPC-CommandConfig

The IE PUSCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUSCH from a group-TPC messages on DCI.

PUSCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUSCH-TPC-COMMANDCONFIG-START

PUSCH-TPC-CommandConfig ::=         SEQUENCE {

tpc-Index                           INTEGER (1..15)                                                 OPTIONAL,   -- Cond SUL

tpc-IndexSUL                        INTEGER (1..15)                                                 OPTIONAL,   -- Cond SUL-Only

targetCell                          ServCellIndex                                                   OPTIONAL,   -- Need S

...

}

-- TAG-PUSCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUSCH-TPC-CommandConfig field descriptions

targetCell The serving cell to which the acquired power control commands are applicable. If the value is absent, the UE applies the TPC commands to the serving cell on which the command has been received.

tpc-Index An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

tpc-IndexSUL An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

| Conditional Presence | Explanation |
| --- | --- |
| SUL-Only | The field is optionally present, Need R, if supplementaryUplink is configured within ServingCellConfig. It is absent otherwise. |
| SUL | The field is optionally present, Need R, if supplementaryUplink is configured within ServingCellConfig. It is mandatory present otherwise. |

#### –	QFI

The IE QFI is used to indicate the QoS Flow Identifier.

QFI information element

-- ASN1START

-- TAG-QFI-START

QFI ::=                             INTEGER (0..maxQFI)

-- TAG-QFI-STOP

-- ASN1STOP

#### –	Q-OffsetRange

The IE Q-OffsetRange is used to indicate a cell, beam or measurement object specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

Q-OffsetRange information element

-- ASN1START

-- TAG-Q-OFFSETRANGE-START

Q-OffsetRange ::=                   ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-Q-OFFSETRANGE-STOP

-- ASN1STOP

#### –	Q-QualMin

The IE Q-QualMin is used to indicate for cell selection/ re-selection the required minimum received RSRQ level in the (NR) cell. Corresponds to parameter Qqualmin in TS 38.304 [20]. Actual value Qqualmin = field value [dB].

Q-QualMin information element

-- ASN1START

-- TAG-Q-QUALMIN-START

Q-QualMin ::=                       INTEGER (-43..-12)

-- TAG-Q-QUALMIN-STOP

-- ASN1STOP

#### –	Q-RxLevMin

The IE Q-RxLevMin is used to indicate for cell selection/ re-selection the required minimum received RSRP level in the (NR) cell. Corresponds to parameter Qrxlevmin in TS 38.304 [20]. Actual value Qrxlevmin = field value * 2 [dBm].

Q-RxLevMin information element

-- ASN1START

-- TAG-Q-RXLEVMIN-START

Q-RxLevMin ::=                      INTEGER (-70..-22)

-- TAG-Q-RXLEVMIN-STOP

-- ASN1STOP

#### –	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for NR and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

-- TAG-QUANTITYCONFIG-START

QuantityConfig ::=                  SEQUENCE {

quantityConfigNR-List               SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR          OPTIONAL,   -- Need M

...,

[[

quantityConfigEUTRA                 FilterConfig                                                            OPTIONAL    -- Need M

]],

[[

quantityConfigUTRA-FDD-r16          QuantityConfigUTRA-FDD-r16                                              OPTIONAL,   -- Need M

quantityConfigCLI-r16               FilterConfigCLI-r16                                                     OPTIONAL    -- Need M

]]

}

QuantityConfigNR::=                 SEQUENCE {

quantityConfigCell                  QuantityConfigRS,

quantityConfigRS-Index              QuantityConfigRS                                                        OPTIONAL    -- Need M

}

QuantityConfigRS ::=                SEQUENCE {

ssb-FilterConfig                    FilterConfig,

csi-RS-FilterConfig                 FilterConfig

}

FilterConfig ::=                    SEQUENCE {

filterCoefficientRSRP               FilterCoefficient                                       DEFAULT fc4,

filterCoefficientRSRQ               FilterCoefficient                                       DEFAULT fc4,

filterCoefficientRS-SINR            FilterCoefficient                                       DEFAULT fc4

}

FilterConfigCLI-r16 ::=             SEQUENCE {

filterCoefficientSRS-RSRP-r16       FilterCoefficient                                       DEFAULT fc4,

filterCoefficientCLI-RSSI-r16       FilterCoefficient                                       DEFAULT fc4

}

QuantityConfigUTRA-FDD-r16 ::=      SEQUENCE {

filterCoefficientRSCP-r16           FilterCoefficient                                       DEFAULT fc4,

filterCoefficientEcNO-r16           FilterCoefficient                                       DEFAULT fc4

}

-- TAG-QUANTITYCONFIG-STOP

-- ASN1STOP

QuantityConfigNR field descriptions

quantityConfigCell Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

quantityConfigRS-Index Specifies L3 filter configurations for measurement results per RS index for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

QuantityConfigRS field descriptions

csi-RS-FilterConfig CSI-RS based L3 filter configurations: Specifies L3 filter configurations for CSI-RSRP, CSI-RSRQ and CSI-SINR measurement results from the L1 filter(s), as defined in TS 38.215 [9].

ssb-FilterConfig SS Block based L3 filter configurations: Specifies L3 filter configurations for SS-RSRP, SS-RSRQ and SS-SINR measurement results from the L1 filter(s), as defined in TS 38.215 [9].

QuantityConfigUTRA-FDD field descriptions

filterCoefficientRSCP Specifies L3 filter coefficient for FDD UTRAN CPICH_RSCP measuement results from L1 filter.

filterCoefficientEcN0 Specifies L3 filter coefficient for FDD UTRAN CPICH_EcN0 measuement results from L1 filter.

#### –	RACH-ConfigCommon

The IE RACH-ConfigCommon is used to specify the cell specific random-access parameters.

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMON-START

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                       OPTIONAL,   -- Need M

groupBconfigured                    SEQUENCE {

ra-Msg3SizeGroupA                   ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB            ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA          INTEGER (1..64)

}                                                                                                       OPTIONAL,   -- Need R

ra-ContentionResolutionTimer            ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB                       RSRP-Range                                                      OPTIONAL,   -- Need R

rsrp-ThresholdSSB-SUL                   RSRP-Range                                                      OPTIONAL,   -- Cond SUL

prach-RootSequenceIndex                 CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

msg1-SubcarrierSpacing                  SubcarrierSpacing                                               OPTIONAL,   -- Cond L139

restrictedSetConfig                     ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoder                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

...,

[[

ra-PrioritizationForAccessIdentity-r16  SEQUENCE {

ra-Prioritization-r16                   RA-Prioritization,

ra-PrioritizationForAI-r16              BIT STRING (SIZE (2))

}                                                                                                       OPTIONAL,   -- Cond InitialBWP-Only

prach-RootSequenceIndex-r16             CHOICE {

l571                                    INTEGER (0..569),

l1151                                   INTEGER (0..1149)

}   OPTIONAL   -- Need R

]],

[[

ra-PrioritizationForSlicing-r17         RA-PrioritizationForSlicing-r17                          OPTIONAL,   -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17     SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL -- Cond AdditionalRACH

]]

}

-- TAG-RACH-CONFIGCOMMON-STOP

-- ASN1STOP

RACH-ConfigCommon field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 4-step RA. The network does not configure this list to have more than 32 entries.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on (see TS 38.321 [3], clause 5.1.2). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free random access (RACH-ConfigDedicated), to SI-request and to contention-based beam failure recovery (CB-BFR). But it does not apply for contention free beam failure recovery (CF-BFR) (see BeamFailureRecoveryConfig).

msg3-transformPrecoder Enables the transform precoder for Msg3 transmission according to clause 6.1.3 of TS 38.214 [19]. If the field is absent, the UE disables the transformer precoder (see TS 38.213 [13], clause 8.3).

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A. This determines implicitly the number of CB preambles per SSB available in group B. (see TS 38.321 [3], clause 5.1.1). The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1). The value range depends on whether L=839 or L=139 or L=571 or L=1151. The length of the root sequence corresponding with the index indicated in this IE should be consistent with the one indicated in prach-ConfigurationIndex in the RACH-ConfigDedicated (if configured). If prach-RootSequenceIndex-r16 is signalled, UE shall ignore the prach-RootSequenceIndex (without suffix). For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

ra-ContentionResolutionTimer The initial value for the contention resolution timer (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on.

ra-Msg3SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A (see TS 38.321 [3], clause 5.1.2). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

ra-Prioritization Parameters which apply for prioritized random access procedure on any UL BWP of SpCell for specific Access Identities (see TS 38.321 [3], clause 5.1.1a).

ra-PrioritizationForAI Indicates whether the field ra-Prioritization-r16 applies for Access Identities. The first/leftmost bit corresponds to Access Identity 1, the next bit corresponds to Access Identity 2. Value 1 indicates that the field ra-Prioritization-r16 applies otherwise the field does not apply (see TS 23.501 [32]).

ra-PrioritizationForSlicing Parameters which apply to configure prioritized CBRA 4-step random access type for slicing.

rach-ConfigGeneric RACH parameters for both regular random access and beam failure recovery.

restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

rsrp-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

rsrp-ThresholdSSB-SUL The UE selects SUL carrier to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). The value applies to all the BWPs and all RACH configurations.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEighth corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). See TS 38.213 [13].

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources defined in RACH-ConfigCommon, excluding preambles used for other purposes (e.g. for SI request). If the field is absent, all 64 preambles are available for RA. The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, i.e. it should be a multiple of the number of SSBs per RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| AdditionalRACH | The field is mandatory present if the RACH-ConfigCommon is included in an AdditionalRACH-Config. When included in initialUplinkBWP-RedCap to indicate other feature(s) than redcap and eRedCap, this field is mandatory present with at least FeatureCombinationPreambles list entries: the list entry/entries indicating only redcap or eRedCap and the other(s) indicating both redcap or eRedCap and one or multiple other feature(s) (e.g., smallData, nsag or msg3-Repetitions). Otherwise, it is optional, Need R. |
| InitialBWP-Only | This field is optionally present, Need R, if this BWP is the initial BWP of SpCell. Otherwise, the field is absent. |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |
| SUL | The field is mandatory present in rach-ConfigCommon in initialUplinkBWP if supplementaryUplink is configured in ServingCellConfigCommonSIB or if supplementaryUplinkConfig is configured in ServingCellConfigCommon; otherwise, the field is absent. This field is not configured in additionalRACH-Config. |

#### –	RACH-ConfigCommonTwoStepRA

The IE RACH-ConfigCommonTwoStepRA is used to specify cell specific 2-step random-access type parameters.

RACH-ConfigCommonTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-START

RACH-ConfigCommonTwoStepRA-r16 ::=                   SEQUENCE {

rach-ConfigGenericTwoStepRA-r16                      RACH-ConfigGenericTwoStepRA-r16,

msgA-TotalNumberOfRA-Preambles-r16                   INTEGER (1..63)                                    OPTIONAL, -- Need S

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16   CHOICE {

oneEighth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                              ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                                 INTEGER (1..16),

eight                                                INTEGER (1..8),

sixteen                                              INTEGER (1..4)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-CB-PreamblesPerSSB-PerSharedRO-r16              INTEGER (1..60)                                                OPTIONAL, -- Cond SharedRO

msgA-SSB-SharedRO-MaskIndex-r16                      INTEGER (1..15)                                                OPTIONAL, -- Need S

groupB-ConfiguredTwoStepRA-r16                       GroupB-ConfiguredTwoStepRA-r16                                 OPTIONAL, -- Need S

msgA-PRACH-RootSequenceIndex-r16                     CHOICE {

l839                                                 INTEGER (0..837),

l139                                                 INTEGER (0..137),

l571                                                 INTEGER (0..569),

l1151                                                INTEGER (0..1149)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-TransMax-r16                                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}     OPTIONAL, -- Need R

msgA-RSRP-Threshold-r16                              RSRP-Range                                                     OPTIONAL, -- Cond 2Step4Step

msgA-RSRP-ThresholdSSB-r16                           RSRP-Range                                                     OPTIONAL, -- Need R

msgA-SubcarrierSpacing-r16                           SubcarrierSpacing                                              OPTIONAL, -- Cond 2StepOnlyL139

msgA-RestrictedSetConfig-r16                         ENUMERATED {unrestrictedSet, restrictedSetTypeA,

restrictedSetTypeB}                                OPTIONAL, -- Cond 2StepOnly

ra-PrioritizationForAccessIdentityTwoStep-r16        SEQUENCE {

ra-Prioritization-r16                                RA-Prioritization,

ra-PrioritizationForAI-r16                           BIT STRING (SIZE (2))

}                                                                                                                   OPTIONAL, -- Cond InitialBWP-Only

ra-ContentionResolutionTimer-r16                     ENUMERATED {sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}     OPTIONAL, -- Cond 2StepOnly

...,

[[

ra-PrioritizationForSlicingTwoStep-r17               RA-PrioritizationForSlicing-r17              OPTIONAL, -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL  -- Cond AdditionalRACH

]]

}

GroupB-ConfiguredTwoStepRA-r16 ::=                       SEQUENCE {

ra-MsgA-SizeGroupA-r16                               ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800,

b1000, b72, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r16                         ENUMERATED {minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r16                       INTEGER (1..64)

}

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-STOP

-- ASN1STOP

RACH-ConfigCommonTwoStepRA field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 2-step RA. The network does not configure this list to have more than 16 entries.

groupB-ConfiguredTwoStepRA Preamble grouping for 2-step random access type. If the field is absent then there is only one preamble group configured and only one msgA PUSCH configuration.

msgA-CB-PreamblesPerSSB-PerSharedRO Number of contention-based preambles used for 2-step RA type from the non-CBRA 4-step type preambles associated with each SSB for RO shared with 4-step type RA. The number of preambles for 2-step RA type shall not exceed the number of preambles per SSB minus the number of contention-based preambles per SSB for 4-step type RA. The possible value range for this parameter needs to be aligned with value range for the configured SSBs per RACH occasion in ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. The field is only applicable for the case of shared ROs with 4-step type random access.

msgA-PRACH-RootSequenceIndex PRACH root sequence index. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e., not within AdditionalRACH-Config), the UE applies the value in field prach-RootSequenceIndex in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the corresponding value of prach-RootSequenceIndex in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access. For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

msgA-RestrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets for 2-step random access type preamble. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), the UE applies the value in field restrictedSetConfig in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the value of restrictedSetConfig in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access.

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

msgA-RSRP-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config) and both 2-step and 4-step are configured for the BWP, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. If the field is not configured in AdditionalRACH-Config and both 2-step and 4-step are configured in AdditionalRACH-Config, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon in the same AdditionalRACH-Config. The field is not present when RACH occasions are shared between 2-step and 4-step type random access in the BWP.

msgA-SSB-SharedRO-MaskIndex Indicates the subset of 4-step type ROs shared with 2-step random access type for each SSB. This field is configured when there is more than one RO per SSB. If the field is absent, and 4-step and 2-step has shared ROs, then all ROs are shared.

msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz. If the field is absent, the UE applies the SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]) in case of 2-step only BWP, otherwise the UE applies the same SCS as Msg1 derived from RACH-ConfigCommon. The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).

msgA-TotalNumberOfRA-Preambles Indicates the total number of preambles used for contention-based and contention-free 2-step random access type when ROs for 2-step are not shared with 4-step. If the field is absent, and 2-step and 4-step does not have shared ROs, all 64 preambles are available for 2-step random access type.

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step random access (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when 2-step and 4-step RA type are configured and switching to 4-step type RA is supported. If the field is absent, switching from 2-step RA type to 4-step RA type is not allowed.

ra-ContentionResolutionTimer The initial value for the contention resolution timer for fallback RAR in case no 4-step random access type is configured (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on. If both 2-step and 4-step random access type resources are configured on the BWP, then this field is absent. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config.

ra-Prioritization Parameters which apply for prioritized random access procedure on any UL BWP of SpCell for specific Access Identities (see TS 38.321 [3], clause 5.1.1a).

ra-PrioritizationForAI Indicates whether the field ra-Prioritization-r16 applies for Access Identities. The first/leftmost bit corresponds to Access Identity 1, the next bit corresponds to Access Identity 2. Value 1 for an Access Identity indicates that the field ra-Prioritization-r16 applies, otherwise the field does not apply.

ra-PrioritizationForSlicingTwoStep Parameters which apply to configure prioritized CBRA 2-step random access type for slicing.

rach-ConfigGenericTwoStepRA 2-step random access type parameters for both regular random access and beam failure recovery.

GroupB-ConfiguredTwoStepRA field descriptions

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see TS 38.321 [3], clause 5.1.1).

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A for idle/inactive or connected mode. The setting of the number of preambles for each group should be consistent with msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB or msgA-CB-PreamblesPerSSB-PerSharedRO if configured.

ra-MsgA-SizeGroupA Transport block size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| 2Step4Step | The field is mandatory present if both 2-step random access type and 4-step random access type are configured in the BWP, otherwise the field is not present. The field is mandatory present in msgA-ConfigCommon field in AdditionalRACH-Config if both 2-step random access type and 4-step random access type are configured for the same feature combination in the BWP. |
| 2StepOnlyL139 | The field is mandatory present if msgA-PRACH-RootSequenceIndex L=139 and no 4-step random access type is configured, otherwise the field is absent, Need S. |
| 2StepOnly | The field is mandatory present in msgA-ConfigCommon field in BWP-UplinkCommon if rach-ConfigCommon field is absent in this BWP-UplinkCommon, otherwise the field is optionally present in msgA-ConfigCommon field in BWP-UplinkCommon, Need S. The field is mandatory present in msgA-ConfigCommon field in AdditionalRACH-Config if rach-ConfigCommon field is absent in this AdditionalRACH-Config, otherwise the field is optionally present in msgA-ConfigCommon field in AdditionalRACH-Config, Need S. |
| AdditionalRACH | The field is mandatory present if the msgA-ConfigCommon is included in an AdditionalRACH-Config. When included in initialUplinkBWP-RedCap to indicate other feature(s) than redcap, this field is mandatory present with at least two FeatureCombinationPreambles list entries: one list entry indicating only redcap and the other(s) indicating both redcap and one or multiple other feature(s) (e.g. smallData, nsag or msg3-Repetitions). Otherwise, it is optional, Need R. |
| InitialBWP-Only | This field is optionally present, Need R, if this BWP is the initial BWP of SpCell. Otherwise, the field is absent. |
| SharedRO | The field is mandatory present if the 2-step random access type occasions are shared with 4-step random access type, otherwise the field is not present. |

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16    RA-Prioritization                                                       OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16                                                        OPTIONAL  -- Need S

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond Mandatory

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)                                                             OPTIONAL -- Cond Occasions

]],

[[

msg1-RepetitionNum-r18          ENUMERATED {n2, n4, n8, spare1}                                               OPTIONAL -- Cond 4StepCFRArep

]]

}

CFRA-TwoStep-r16 ::=                    SEQUENCE {

occasionsTwoStepRA-r16                  SEQUENCE {

rach-ConfigGenericTwoStepRA-r16         RACH-ConfigGenericTwoStepRA-r16,

ssb-PerRACH-OccasionTwoStepRA-r16       ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen}

}                                                                                                     OPTIONAL, -- Need S

msgA-CFRA-PUSCH-r16                     MsgA-PUSCH-Resource-r16,

msgA-TransMax-r16                       ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}    OPTIONAL, -- Need S

resourcesTwoStep-r16                    SEQUENCE {

ssb-ResourceList                        SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex                INTEGER (0..15)

},

...

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...,

[[

msgA-PUSCH-Resource-Index-r16   INTEGER (0..3071)     OPTIONAL  -- Cond 2StepCFRA

]]

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS.

CFRA field descriptions

msg1-RepetitionNum Indicates the MSG1 repetition number used for contention free 4-step random access type in TS 38.321 [3]. If this field is absent, the UE performs contention free 4-step random access without MSG1-Repetitions.

occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP.

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA. The UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within this field and use the corresponding values provided in RACH-ConfigCommon.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

totalNumberOfRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for RA. The setting should be consistent with the setting of ssb-perRACH-Occasion, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

CFRA-SSB-Resource field descriptions

msgA-PUSCH-Resource-Index Identifies the index of the PUSCH resource used for MSGA CFRA. The PUSCH resource index indicates a valid PUSCH occasion (as specified in TS 38.213 [13], clause 8.1A) and the associated DMRS resources corresponding to a PRACH slot. The PUSCH resource indexes are sequentially numbered and are mapped to valid PUSCH occasions corresponding to a PRACH slot which are ordered, first, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions; second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRSid resource index is determined first in an ascending order of a DMRS port index and then in an ascending order of a DMRS sequence index, third in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot and fourth, in increasing order of indexes for PUSCH slots. For the case of contention free 2-step random access type, if this field is absent, the UE shall use the value 0.

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

CFRA-TwoStep field descriptions

msgA-CFRA-PUSCH PUSCH resource configuration(s) for msgA CFRA.

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step type random access (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when 2-step and 4-step RA type are configured and switching to 4-step type RA is supported. If the field is absent in cfra-TwoStep, switching from 2-step RA type to 4-step RA type is not allowed.

occasionsTwoStepRA RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommonTwoStepRA in the first active UL BWP.

ra-SSB-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGenericTwoStepRA Configuration of contention free random access occasions for CFRA 2-step random access type.

ssb-PerRACH-OccasionTwoStep Number of SSBs per RACH occasion for 2-step random access type.

RACH-ConfigDedicated field descriptions

cfra Parameters for contention free random access to a given target cell. If this field and cfra-TwoStep are absent, the UE performs contention based random access.

cfra-TwoStep Parameters for contention free 2-step random access type to a given target cell. Network ensures that cfra and cfra-TwoStep are not configured at the same time. If this field and cfra are absent, the UE performs contention based random access. This field may only be present if msgA-ConfigCommon is configured on the BWP.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access type procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |
| Occasions | The field is optionally present, Need S, if the field occasions is present, otherwise it is absent. |
| 2StepCFRA | The field is optionally present for the case of 2-step RA type contention free random access, Need S, otherwise it is absent. |
| 4StepCFRArep | For non-(e)RedCap UEs, the field is optionally present, Need S, if resources is set to ssb and there is one FeatureCombinationPreambles entry indicating only msg1-Repetitions which is associated with the same Msg1 repetition number. For RedCap UEs or if RedCap is considered to be applicable for this Random Access procedure for eRedCap UEs, the field is optionally present, Need S, if resources is set to ssb and there is one FeatureCombinationPreambles entry indicating only redCap and msg1-Repetitions which is associated with the same Msg1 repetition number. For eRedCap UEs, if eRedCap is considered to be applicable for this Random Access procedure, the field is optional present, Need S, if resource is set to ssb and there is one FeatureCombinationPreambles entry indicating only eRedCap and msg1-Repetitions which is associated with the same Msg1 repetition number. Otherwise, it is absent. |

#### –	RACH-ConfigGeneric

The IE RACH-ConfigGeneric is used to specify the random-access parameters both for regular random access as well as for beam failure recovery.

RACH-ConfigGeneric information element

-- ASN1START

-- TAG-RACH-CONFIGGENERIC-START

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...,

[[

prach-ConfigurationPeriodScaling-IAB-r16    ENUMERATED {scf1,scf2,scf4,scf8,scf16,scf32,scf64}              OPTIONAL,   -- Need R

prach-ConfigurationFrameOffset-IAB-r16      INTEGER (0..63)                                                 OPTIONAL,   -- Need R

prach-ConfigurationSOffset-IAB-r16          INTEGER (0..39)                                                 OPTIONAL,   -- Need R

ra-ResponseWindow-v1610                     ENUMERATED { sl60, sl160}                                       OPTIONAL,   -- Need R

prach-ConfigurationIndex-v1610              INTEGER (256..262)                                              OPTIONAL    -- Need R

]],

[[

ra-ResponseWindow-v1700                     ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} OPTIONAL    -- Need R

]]

}

-- TAG-RACH-CONFIGGENERIC-STOP

-- ASN1STOP

RACH-ConfigGeneric field descriptions

msg1-FDM The number of PRACH transmission occasions FDMed in one time instance. (see TS 38.211 [16], clause 6.3.3.2).

msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see TS 38.211 [16], clause 6.3.3.2).

powerRampingStep Power ramping steps for PRACH (see TS 38.321 [3],5.1.3). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

prach-ConfigurationFrameOffset-IAB Frame offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. (see TS 38.211 [16], clause 6.3.3.2).

prach-ConfigurationIndex PRACH configuration index. For prach-ConfigurationIndex configured under beamFailureRecoveryConfig, the prach-ConfigurationIndex can only correspond to the short preamble format, (see TS 38.211 [16], clause 6.3.3.2). If the field prach-ConfigurationIndex-v1610 is present, the UE shall ignore the value provided in prach-ConfigurationIndex (without suffix).

prach-ConfigurationPeriodScaling-IAB Scaling factor to extend the periodicity of the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. Value scf1 corresponds to scaling factor of 1 and so on. (see TS 38.211 [16], clause 6.3.3.2).

prach-ConfigurationSOffset-IAB Subframe/Slot offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. (see TS 38.211 [16], clause 6.3.3.2).

preambleReceivedTargetPower The target power level at the network receiver side (see TS 38.213 [13], clause 7.4, TS 38.321 [3], clauses 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5). The UE shall ignore this field in case rach-ConfigGeneric is included within an EarlyUL-SyncConfig IE.

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. The UE shall ignore this field in case rach-ConfigGeneric is included within an EarlyUL-SyncConfig IE.

zeroCorrelationZoneConfig N-CS configuration, see Table 6.3.3.1-5 in TS 38.211 [16].

#### –	RACH-ConfigGenericTwoStepRA

The IE RACH-ConfigGenericTwoStepRA is used to specify the 2-step random access type parameters.

RACH-ConfigGenericTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGGENERICTWOSTEPRA-START

RACH-ConfigGenericTwoStepRA-r16 ::=     SEQUENCE {

msgA-PRACH-ConfigurationIndex-r16       INTEGER (0..262)                                                OPTIONAL, -- Cond 2StepOnly

msgA-RO-FDM-r16                         ENUMERATED {one, two, four, eight}                              OPTIONAL, -- Cond 2StepOnly

msgA-RO-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)                    OPTIONAL, -- Cond 2StepOnly

msgA-ZeroCorrelationZoneConfig-r16      INTEGER (0..15)                                                 OPTIONAL, -- Cond 2StepOnly

msgA-PreamblePowerRampingStep-r16       ENUMERATED {dB0, dB2, dB4, dB6}                                 OPTIONAL, -- Cond 2StepOnlyNoCFRA

msgA-PreambleReceivedTargetPower-r16    INTEGER (-202..-60)                                             OPTIONAL, -- Cond 2StepOnlyNoCFRA

msgB-ResponseWindow-r16                 ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80, sl160, sl320}

OPTIONAL, -- Cond NoCFRA

preambleTransMax-r16                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}  OPTIONAL, -- Cond 2StepOnlyNoCFRA

...,

[[

msgB-ResponseWindow-v1700               ENUMERATED {sl240, sl640, sl960, sl1280, sl1920, sl2560}        OPTIONAL  -- Cond NoCFRA2

]]

}

-- TAG-RACH-CONFIGGENERICTWOSTEPRA-STOP

-- ASN1STOP

RACH-ConfigGenericTwoStepRA field descriptions

msgA-PreamblePowerRampingStep Power ramping steps for msgA PRACH. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the field is absent in other cases, UE shall use the value of powerRampingStep in RACH-ConfigGeneric in the configured BWP (see TS 38.321 [3], 5.1.3). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA. The field is absent if RACH-ConfigGenericTwoStepRA is included in CFRA-TwoStep in RACH-ConfigDedicated and then the UE uses the value of msgA-PreamblePowerRampingStep in RACH-ConfigGenericTwoStepRA configured for CBRA.

msgA-PreambleReceivedTargetPower The target power level at the network receiver side (see TS 38.213 [13], clause 7.1.1 and TS 38.321 [3], clause 5.1.1). Only multiples of 2 dBm may be chosen (e.g -202, -200, -198, …). If the field is absent, UE shall use the value of preambleReceivedTargetPower in RACH-ConfigGeneric in the configured BWP. This field may only be present if no 4-step type RA is configured in the BWP. The field is absent if RACH-ConfigGenericTwoStepRA is included in CFRA-TwoStep in RACH-ConfigDedicated and then the UE uses the value of msgA-PreambleReceivedTargetPower in RACH-ConfigGenericTwoStepRA configured for CBRA.

msgA-PRACH-ConfigurationIndex Cell-specific PRACH configuration index for 2-step RA type. If the field is absent in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), the UE shall use the value of corresponding 4-step random access parameter in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the value is in the range of 256 to 262, the field prach-ConfigurationIndex-v1610 should be considered configured (see TS 38.211 [16], clause 6.3.3.2). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgA-RO-FDM The number of msgA PRACH transmission occasions Frequency-Division Multiplexed in one time instance. If the field is absent in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), UE shall use value of msg1-FDM in RACH-ConfigGeneric in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the value of msg1-FDM in RACH-ConfigCommon in the same AdditionalRACH-Config (see TS 38.211 [16], clause 6.3.3.2). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgA-RO-FrequencyStart Offset of lowest PRACH transmissions occasion in frequency domain with respect to PRB 0. If the field is absent in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), UE shall use value of msg1-FrequencyStart in RACH-ConfigGeneric in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the value of msg1-FrequencyStart in RACH-ConfigCommon in the same AdditionalRACH-Config (see TS 38.211 [16], clauses 5.3.2 and 6.3.3.2). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgA-ZeroCorrelationZoneConfig N-CS configuration for msgA preamble, see Table 6.3.3.1-5 in TS 38.211 [16]. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the field is absent in other cases, UE shall use value zeroCorrelationZoneConfig in RACH-ConfigGeneric in the configured BWP. This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgB-ResponseWindow MsgB monitoring window length in number of slots. The network configures a value lower than or equal to 40ms (see TS 38.321 [3], clause 5.1.1). The network does not configure msgB-ResponseWindow-r16 simultaneously with msgB-ResponseWindow-v1700, and if both fields are absent, the UE uses the value of msgB-ResponseWindow in RACH-ConfigGenericTwoStepRA configured for CBRA.

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5). If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the field is absent in other cases, UE shall use the value of preambleTransMax in RACH-ConfigGeneric in the configured BWP. The field is absent if RACH-ConfigGenericTwoStepRA is included in CFRA-TwoStep in RACH-ConfigDedicated and then the UE uses the value of preambleTransMax in RACH-ConfigGenericTwoStepRA configured for CBRA.

| Conditional Presence | Explanation |
| --- | --- |
| 2StepOnly | The field is mandatory present in msgA-ConfigCommon field in BWP-UplinkCommon if rach-ConfigCommon field is absent in this BWP-UplinkCommon, otherwise the field is optionally present in msgA-ConfigCommon field in BWP-UplinkCommon, Need S. The field is mandatory present in msgA-ConfigCommon in AdditionalRACH-Config if rach-ConfigCommon field is absent in this AdditionalRACH-Config, otherwise the field is optionally present in msgA-ConfigCommon field in AdditionalRACH-Config, Need S. |
| 2StepOnlyNoCFRA | The field is mandatory present if RACH-ConfigGenericTwoStepRA is included in the RACH-ConfigCommonTwoStepRA and there are no 4-step random access configurations configured in the BWP (i.e only 2-step random access type configured in the BWP), otherwise (i.e. 4-step random access configuration also exists in the BWP) the field is optionally present, Need S. When RACH-ConfigGenericTwoStepRA is included in the RACH-ConfigDedicated, this field is absent. |
| NoCFRA | The field is mandatory present if msgB-ResponseWindow-r17 is absent and RACH-ConfigGenericTwoStepRA is not included in CFRA-TwoStep in RACH-ConfigDedicated, otherwise the field is absent, Need S. |
| NoCFRA2 | The field is mandatory present if msgB-ResponseWindow-r16 is absent and RACH-ConfigGenericTwoStepRA is not included in CFRA-TwoStep in RACH-ConfigDedicated, otherwise the field is absent, Need S. |

#### –	RACH-ConfigTwoTA

The IE RACH-ConfigTwoTA is used to specify random access parameters for each additional PCI configured for the serving cell.

RACH-ConfigTwoTA information element

-- ASN1START

-- TAG-RACH-CONFIGTWOTA-START

RACH-ConfigTwoTA-r18 ::=     SEQUENCE {

rach-ConfigTwoTAIndex-r18    RACH-ConfigTwoTAIndex-r18,

rach-ConfigGeneric-r18       RACH-ConfigGeneric,

ssb-perRACH-Occasion-r18     ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}   OPTIONAL,   -- Need M

prach-RootSequenceIndex-r18  CHOICE {

l839                         INTEGER (0..837),

l139                         INTEGER (0..139)

},

msg1-SubcarrierSpacing-r18   SubcarrierSpacing                                                            OPTIONAL,   -- Cond L139

...

}

RACH-ConfigTwoTAIndex-r18  ::=  INTEGER(1.. maxNrofAdditionalPRACHConfigs-r18)

-- TAG-RACH-CONFIGTWOTA-STOP

-- ASN1STOP

RACH-ConfigTwoTA field descriptions

msg1-SubcarrierSpacing Subcarrier spacing of PRACH when prach-RootSequenceIndex has value set to l139 (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz. If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]).

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1). The value range depends on whether L=839 or L=139. For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

rach-ConfigGeneric RACH parameters for for contention free random access occasions for CFRA.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |

#### –	RA-Prioritization

The IE RA-Prioritization is used to configure prioritized random access.

RA-Prioritization information element

-- ASN1START

-- TAG-RA-PRIORITIZATION-START

RA-Prioritization ::=           SEQUENCE {

powerRampingStepHighPriority    ENUMERATED {dB0, dB2, dB4, dB6},

scalingFactorBI                 ENUMERATED {zero, dot25, dot5, dot75}                               OPTIONAL,   -- Need R

...

}

-- TAG-RA-PRIORITIZATION-STOP

-- ASN1STOP

RA-Prioritization field descriptions

powerRampingStepHighPrioritiy Power ramping step applied for prioritized random access procedure.

scalingFactorBI Scaling factor for the backoff indicator (BI) for the prioritized random access procedure. (see TS 38.321 [3], clause 5.1.4). Value zero corresponds to 0, value dot25 corresponds to 0.25 and so on.

#### –	RA-PrioritizationForSlicing

The IE RA-PrioritizationForSlicing is used to configure prioritized random access for slicing.

RA-PrioritizationForSlicing information element

-- ASN1START

-- TAG-RA-PRIORITIZATIONFORSLICING-START

RA-PrioritizationForSlicing-r17 ::=    SEQUENCE {

ra-PrioritizationSliceInfoList-r17     RA-PrioritizationSliceInfoList-r17,

...

}

RA-PrioritizationSliceInfoList-r17 ::= SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF RA-PrioritizationSliceInfo-r17

RA-PrioritizationSliceInfo-r17 ::=     SEQUENCE {

nsag-ID-List-r17                       SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF NSAG-ID-r17,

ra-Prioritization-r17                  RA-Prioritization,

...

}

-- TAG-RA-PRIORITIZATIONFORSLICING-STOP

-- ASN1STOP

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling, multicast MRBs and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIOBEARERCONFIG-START

RadioBearerConfig ::=                   SEQUENCE {

srb-ToAddModList                        SRB-ToAddModList                                        OPTIONAL,   -- Cond HO-Conn

srb3-ToRelease                          ENUMERATED{true}                                        OPTIONAL,   -- Need N

drb-ToAddModList                        DRB-ToAddModList                                        OPTIONAL,   -- Cond HO-toNR

drb-ToReleaseList                       DRB-ToReleaseList                                       OPTIONAL,   -- Need N

securityConfig                          SecurityConfig                                          OPTIONAL,   -- Need M

...,

[[

mrb-ToAddModList-r17                    MRB-ToAddModList-r17                                    OPTIONAL,   -- Need N

mrb-ToReleaseList-r17                   MRB-ToReleaseList-r17                                   OPTIONAL,   -- Need N

srb4-ToAddMod-r17                       SRB-ToAddMod                                            OPTIONAL,   -- Need N

srb4-ToRelease-r17                      ENUMERATED{true}                                        OPTIONAL    -- Need N

]],

[[

srb5-ToAddMod-r18                       SRB-ToAddMod                                            OPTIONAL,   -- Need N

srb5-ToRelease-r18                      ENUMERATED{true}                                        OPTIONAL    -- Need N

]]

}

SRB-ToAddModList ::=                    SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=                        SEQUENCE {

srb-Identity                            SRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

discardOnPDCP                           ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

[[

srb-Identity-v1700                      SRB-Identity-v1700                                      OPTIONAL    -- Need M

]],

[[

srb-Identity-v1800                      SRB-Identity-v1800                                      OPTIONAL,   -- Need M

n3c-BearerAssociated-r18                ENUMERATED{true}                                        OPTIONAL    -- Cond N3C MP

]]

}

DRB-ToAddModList ::=                    SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),

sdap-Config                             SDAP-Config

}                                                                                               OPTIONAL,   -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

[[

daps-Config-r16                         ENUMERATED{true}                                        OPTIONAL    -- Cond DAPS

]],

[[

n3c-BearerAssociated-r18                ENUMERATED{true}                                        OPTIONAL    -- Cond N3C MP

]]

}

DRB-ToReleaseList ::=                   SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=                      SEQUENCE {

securityAlgorithmConfig                 SecurityAlgorithmConfig                                 OPTIONAL,   -- Cond RBTermChange1

keyToUse                                ENUMERATED{master, secondary}                           OPTIONAL,   -- Cond RBTermChange

...

}

MRB-ToAddModList-r17 ::=                SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-ToAddMod-r17

MRB-ToAddMod-r17 ::=                    SEQUENCE {

mbs-SessionId-r17                       TMGI-r17                                                OPTIONAL,   -- Cond MRBSetup

mrb-Identity-r17                        MRB-Identity-r17,

mrb-IdentityNew-r17                     MRB-Identity-r17                                        OPTIONAL,   -- Need N

reestablishPDCP-r17                     ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP-r17                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config-r17                         PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

MRB-ToReleaseList-r17 ::=               SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-Identity-r17

-- TAG-RADIOBEARERCONFIG-STOP

-- ASN1STOP

DRB-ToAddMod and MRB-ToAddMod field descriptions

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

daps-Config Indicates that the bearer is configured as DAPS bearer.

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts/indirect path of the configuration.

eps-BearerIdentity The EPS bearer ID determines the EPS bearer.

mbs-SessionId Indicates which multicast MBS session the bearer is associated with.

mrb-Identity Identification of the multicast MRB.

mrb-IdentityNew New identity of the multicast MRB when mrb-Identity needs to be changed, e.g. as a result of a handover.

n3c-BearerAssociated Indicates that the radio bearer is associated with the N3C indirect path.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured. Network doesn't include this field for DRB if the bearer is configured as DAPS bearer or if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE. or if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE

recoverPDCP Indicates that PDCP should perform recovery according to TS 38.323 [5]. Network doesn't include this field if the bearer is configured as DAPS bearer or if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE or if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE.

sdap-Config The SDAP configuration determines how to map QoS flows to DRBs when NR or E-UTRA connects to the 5GC and presence/absence of UL/DL SDAP headers.

RadioBearerConfig field descriptions

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included after AS security has been activated, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig. The field is not included when configuring SRB1 before AS security is activated.

srb3-ToRelease Release SRB3. SRB3 release can only be done over SRB1 and only at SCG release and reconfiguration with sync.

SecurityConfig field descriptions

keyToUse Indicates if the bearers configured with the list in this IE RadioBearerConfig are using the master key or the secondary key for deriving ciphering and/or integrity protection keys. For MR-DC, network should not configure SRB1 and SRB2 with secondary key and SRB3 with the master key. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

SRB-ToAddMod field descriptions

discardOnPDCP Indicates that PDCP should discard stored SDU and PDU according to TS 38.323 [5]. For SRB3, network doesn't include this field if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to reconfiguration with sync, for SRB2 when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR, the network does not set this field to true. For LTE SRBs using NR PDCP, it could be for handover, RRC connection reestablishment or resume. Network doesn't include this field if any DAPS bearer is configured or if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE. For SRB3, network doesn't include this field if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE.

srb-Identity, srb-Identity-v1700, srb-Identity-v1800 Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only. Value 4 is applicable for SRB4 only. Value 5 is applicable for SRB5 only. If srb-Identity-v1700 or srb-Identity-v1800 is received for an SRB, the UE shall ignore srb-Identity (i.e. without suffix) for this SRB.

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of: - set up of signalling and data radio bearer, - change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| RBTermChange1 | The field is mandatory present in case of: - set up of signalling and data radio bearer, - change of termination point for the radio bearer between MN and SN, - handover from E-UTRA/EPC or E-UTRA/5GC to NR, - handover from NR or E-UTRA/EPC to E-UTRA/5GC if the UE supports NGEN-DC. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding DRB/multicast MRB is being setup or corresponding DRB/multicast MRB is reconfigured with NR PDCP or corresponding SRB associated with two RLC entities is being setup or if the number of RLC bearers associated with the DRB/multicast MRB or SRB is changed. The field is optionally present, Need S, if the corresponding SRB associated with one RLC entity is being setup or corresponding SRB is reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |
| HO-Conn | The field is mandatory present - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured, - or in case of RRCSetup. Otherwise the field is optionally present, need N. Upon RRCSetup, only SRB1 can be present. |
| HO-toNR | If mrb-ToAddModList is not included, the field is mandatory present for UEs other than NCR-MT - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured. In case of RRCSetup, the field is absent; otherwise the field is optionally present, need N. |
| DAPS | The field is optionally present, need N, in case masterCellGroup includes ReconfigurationWithSync, SCell(s) and SCG are not configured, multi-DCI/single-DCI based multi-TRP are not configured in any DL BWP, supplementaryUplink is not configured, ethernetHeaderCompression is not configured for the DRB, conditionalReconfiguration is not configured, and NR sidelink and V2X sidelink are not configured. Otherwise the field is absent. |
| MRBSetup | The field is mandatory present if the corresponding multicast MRB is being setup; otherwise the field is optionally present, need M. |
| N3C MP | The field is optionally present if the corresponding radio bearer is being setup for MP with N3C indirect path, need R. It is absent otherwise. |

#### –	RadioLinkMonitoringConfig

The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

failureDetectionResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...,

[[

beamFailure-r17                         BeamFailureDetection-r17                                              OPTIONAL  -- Need R

]]

}

BeamFailureDetection-r17 ::=        SEQUENCE {

failureDetectionSet1-r17            BeamFailureDetectionSet-r17                                               OPTIONAL, -- Need R

failureDetectionSet2-r17            BeamFailureDetectionSet-r17                                               OPTIONAL, -- Need R

additionalPCI-r17                   AdditionalPCIIndex-r17                                                    OPTIONAL  -- Need R

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id            RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

...

}

BeamFailureDetectionSet-r17  ::=    SEQUENCE {

bfdResourcesToAddModList-r17        SEQUENCE (SIZE(1..maxNrofBFDResourcePerSet-r17)) OF BeamLinkMonitoringRS-r17

OPTIONAL, -- Need N

bfdResourcesToReleaseList-r17       SEQUENCE (SIZE(1..maxNrofBFDResourcePerSet-r17)) OF BeamLinkMonitoringRS-Id-r17

OPTIONAL, -- Need N

beamFailureInstanceMaxCount-r17     ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                              OPTIONAL, -- Need R

beamFailureDetectionTimer-r17       ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}      OPTIONAL, -- Need R

...

}

BeamLinkMonitoringRS-r17 ::=        SEQUENCE {

beamLinkMonitoringRS-Id-r17         BeamLinkMonitoringRS-Id-r17,

detectionResource-r17               CHOICE {

ssb-Index                       SSB-Index,

csi-RS-Index                    NZP-CSI-RS-ResourceId

},

...

}

BeamLinkMonitoringRS-Id-r17 ::=     INTEGER (0..maxNrofFailureDetectionResources-1-r17)

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

RadioLinkMonitoringConfig field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs in the failureDetectionSet2. If candidateBeamRS-List2 is configured in IE BeamFailureRecoveryRSConfig the field indicates the physical cell IDs (PCI) of the SSBs in the candidateBeamRS-List2.

beamFailureDetectionTimer Timer for beam failure detection (see TS 38.321 [3], clause 5.17). See also the BeamFailureRecoveryConfig IE. Value in number of "Qout,LR reporting periods of Beam Failure Detection" Reference Signal (see TS 38.213 [13], clause 6). Value pbfd1 corresponds to 1 Qout,LR reporting period of Beam Failure Detection Reference Signal, value pbfd2 corresponds to 2 Qout,LR reporting periods of Beam Failure Detection Reference Signal and so on.

beamFailureInstanceMaxCount This field determines after how many beam failure events the UE triggers beam failure recovery (see TS 38.321 [3], clause 5.17). Value n1 corresponds to 1 beam failure instance, value n2 corresponds to 2 beam failure instances and so on.

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The limits of the reference signals that the network can configure are specified in TS 38.213 [13], table 5-1. The network configures at most two detectionResources per BWP for the purpose beamFailure or both. If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5. The network ensures that the UE has a suitable set of reference signals for performing cell-RLM. If failureDetectionSet1-r17 and failureDetectionSet2-r17 are present, the purpose of RadioLinkMonitoringRS in failureDetectionResourcesToAddModList only can be set to rlf.

failureDetectionSet1, failureDetectionSet2 Configures parameters for beamfailure detection towards beam failure detection resources configured in the set. If additional PCIs are configured using additionalPCI-ToAddModList for the serving cell, each RS in one set can be associated only with one PCI. Network always configures the failureDetectionSet1 and failureDetectionSet2 together. failureDetectionSetN is present if and only if candidateBeamRS-List2-r17 is configured. When a failureDetectionSetN is present, after the reconfiguration, the UE shall consider all the reference signals for this failure detection set as activated if at most maxBFD-RS-resourcesPerSetPerBWP-r17 reference signals are configured for each failure detection set, otherwise the UE shall consider all the reference signals in this failure detection set as deactivated. If bfdResourcesToAddModList-r17 in failureDetectionSetN is not present, the UE determines the RS(es) in each failureDetectionSetN as described in TS 38.213 [13], clause 6.

RadioLinkMonitoringRS field descriptions

detectionResource A reference signal that the UE shall use for radio link monitoring or beam failure detection (depending on the indicated purpose). Only periodic 1-port CSI-RS can be configured on SCell for beam failure detection purpose.

purpose Determines whether the UE shall monitor the associated reference signal for the purpose of cell- and/or beam failure detection. For SCell, network only configures the value to beamFailure.

#### –	RadioLinkMonitoringRS-Id

The IE RadioLinkMonitoringRS-Id is used to identify one RadioLinkMonitoringRS.

RadioLinkMonitoringRS-Id information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGRS-ID-START

RadioLinkMonitoringRS-Id ::=            INTEGER (0..maxNrofFailureDetectionResources-1)

-- TAG-RADIOLINKMONITORINGRS-ID-STOP

-- ASN1STOP

#### –	RAN-AreaCode

The IE RAN-AreaCode is used to identify a RAN area within the scope of a tracking area.

RAN-AreaCode information element

-- ASN1START

-- TAG-RAN-AREACODE-START

RAN-AreaCode ::=                INTEGER (0..255)

-- TAG-RAN-AREACODE-STOP

-- ASN1STOP

#### –	RateMatchPattern

The IE RateMatchPattern is used to configure one rate matching pattern for PDSCH, see TS 38.214 [19], clause 5.1.4.1.

RateMatchPattern information element

-- ASN1START

-- TAG-RATEMATCHPATTERN-START

RateMatchPattern ::=                SEQUENCE {

rateMatchPatternId                  RateMatchPatternId,

patternType                         CHOICE {

bitmaps                             SEQUENCE {

resourceBlocks                      BIT STRING (SIZE (275)),

symbolsInResourceBlock              CHOICE {

oneSlot                             BIT STRING (SIZE (14)),

twoSlots                            BIT STRING (SIZE (28))

},

periodicityAndPattern               CHOICE {

n2                                  BIT STRING (SIZE (2)),

n4                                  BIT STRING (SIZE (4)),

n5                                  BIT STRING (SIZE (5)),

n8                                  BIT STRING (SIZE (8)),

n10                                 BIT STRING (SIZE (10)),

n20                                 BIT STRING (SIZE (20)),

n40                                 BIT STRING (SIZE (40))

}                                                                                           OPTIONAL,   -- Need S

...

},

controlResourceSet                  ControlResourceSetId

},

subcarrierSpacing                   SubcarrierSpacing                                               OPTIONAL,   -- Cond CellLevel

dummy                               ENUMERATED { dynamic, semiStatic },

...,

[[

controlResourceSet-r16              ControlResourceSetId-r16                                        OPTIONAL    -- Need R

]]

}

-- TAG-RATEMATCHPATTERN-STOP

-- ASN1STOP

RateMatchPattern field descriptions

bitmaps Indicates rate matching pattern by a pair of bitmaps resourceBlocks and symbolsInResourceBlock to define the rate match pattern within one or two slots, and a third bitmap periodicityAndPattern to define the repetition pattern with which the pattern defined by the above bitmap pair occurs.

controlResourceSet This ControlResourceSet is used as a PDSCH rate matching pattern, i.e., PDSCH reception rate matches around it. In frequency domain, the resource is determined by the frequency domain resource of the CORESET with the corresponding CORESET ID. Time domain resource is determined by the parameters of the associated search space of the CORESET. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

periodicityAndPattern A time domain repetition pattern at which the pattern defined by symbolsInResourceBlock and resourceBlocks recurs. This slot pattern repeats itself continuously. Absence of this field indicates the value n1 (see TS 38.214 [19], clause 5.1.4.1).

resourceBlocks A resource block level bitmap in the frequency domain. A bit in the bitmap set to 1 indicates that the UE shall apply rate matching in the corresponding resource block in accordance with the symbolsInResourceBlock bitmap. If used as cell-level rate matching pattern, the bitmap identifies "common resource blocks (CRB)". If used for MBS broadcast CFR, the bitmap identifies "physical resource blocks" inside the MBS broadcast CFR. If used as BWP-level rate matching pattern, the bitmap identifies "physical resource blocks" inside the BWP or MBS multicast CFR. The first/ leftmost bit corresponds to resource block 0, and so on (see TS 38.214 [19], clause 5.1.4.1).

subcarrierSpacing The SubcarrierSpacing for this resource pattern. If the field is absent, the UE applies the SCS of the associated BWP. The value kHz15 corresponds to µ=0, the value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency (see TS 38.214 [19], clause 5.1.4.1): FR1: 15, 30 or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

symbolsInResourceBlock A symbol level bitmap in time domain. It indicates with a bit set to true that the UE shall rate match around the corresponding symbol. This pattern recurs (in time domain) with the configured periodicityAndPattern (see TS 38.214 [19], clause 5.1.4.1). For oneSlot, if ECP is configured, the first 12 bits represent the symbols within the slot and the last two bits within the bitstring are ignored by the UE; Otherwise, the 14 bits represent the symbols within the slot. For twoSlots, if ECP is configured, the first 12 bits represent the symbols within the first slot and the next 12 bits represent the symbols in the second slot and the last four bits within the bit string are ignored by the UE; Otherwise, the first 14 bits represent the symbols within the first slot and the next 14 bits represent the symbols in the second slot. For the bits representing symbols in a slot, the most significant bit of the bit string represents the first symbol in the slot and the second most significant bit represents the second symbol in the slot and so on.

| Conditional Presence | Explanation |
| --- | --- |
| CellLevel | The field is mandatory present if the RateMatchPattern is defined on cell level. The field is absent when the RateMatchPattern is defined on BWP level or defined for MBS broadcast CFR. If the RateMatchPattern is defined on BWP level, the UE applies the SCS of the BWP and if RateMatchPattern is defined for MBS broadcast CFR, the UE applies the SCS of the initial BWP or RedCap-specific initial BWP (if configured) for (e)RedCap UEs. |

#### –	RateMatchPatternId

The IE RateMatchPatternId identifies one RateMatchPattern (see TS 38.214 [19], clause 5.1.4.1).

RateMatchPatternId information element

-- ASN1START

-- TAG-RATEMATCHPATTERNID-START

RateMatchPatternId ::=              INTEGER (0..maxNrofRateMatchPatterns-1)

-- TAG-RATEMATCHPATTERNID-STOP

-- ASN1STOP

#### –	RateMatchPatternLTE-CRS

The IE RateMatchPatternLTE-CRS is used to configure a pattern to rate match around LTE CRS. See TS 38.214 [19], clause 5.1.4.2.

RateMatchPatternLTE-CRS information element

-- ASN1START

-- TAG-RATEMATCHPATTERNLTE-CRS-START

RateMatchPatternLTE-CRS ::=         SEQUENCE {

carrierFreqDL                       INTEGER (0..16383),

carrierBandwidthDL                  ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1},

mbsfn-SubframeConfigList            EUTRA-MBSFN-SubframeConfigList                                          OPTIONAL,   -- Need M

nrofCRS-Ports                       ENUMERATED {n1, n2, n4},

v-Shift                             ENUMERATED {n0, n1, n2, n3, n4, n5}

}

LTE-CRS-PatternList-r16 ::=         SEQUENCE (SIZE (1..maxLTE-CRS-Patterns-r16)) OF RateMatchPatternLTE-CRS

-- TAG-RATEMATCHPATTERNLTE-CRS-STOP

-- ASN1STOP

RateMatchPatternLTE-CRS field descriptions

carrierBandwidthDL BW of the LTE carrier in number of PRBs (see TS 38.214 [19], clause 5.1.4.2).

carrierFreqDL Center of the LTE carrier (see TS 38.214 [19], clause 5.1.4.2).

mbsfn-SubframeConfigList LTE MBSFN subframe configuration (see TS 38.214 [19], clause 5.1.4.2).

nrofCRS-Ports Number of LTE CRS antenna port to rate-match around (see TS 38.214 [19], clause 5.1.4.2).

v-Shift Shifting value v-shift in LTE to rate match around LTE CRS (see TS 38.214 [19], clause 5.1.4.2).

#### –	ReferenceConfiguration

The IE ReferenceConfiguration is used provide a configuration that is common, within the same cell group, to all configured non-complete candidate configurations.

ReferenceConfiguration information element

-- ASN1START

-- TAG-REFERENCECONFIGURATION-START

ReferenceConfiguration-r18 ::= OCTET STRING (CONTAINING RRCReconfiguration)

-- TAG-REFERENCECONFIGURATION-STOP

-- ASN1STOP

#### –	ReferenceLocation

The IE ReferenceLocation contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

ReferenceLocation information element

-- ASN1START

-- TAG-REFERENCELOCATION-START

ReferenceLocation-r17 ::= OCTET STRING

-- TAG-REFERENCELOCATION-STOP

-- ASN1STOP

#### –	ReferenceTimeInfo

The IE ReferenceTimeInfo contains timing information for 5G internal system clock used for, e.g., time stamping, see TS 23.501 [32], clause 5.27.1.2.

ReferenceTimeInfo information element

-- ASN1START

-- TAG-REFERENCETIMEINFO-START

ReferenceTimeInfo-r16 ::= SEQUENCE {

time-r16                            ReferenceTime-r16,

uncertainty-r16                     INTEGER (0..32767)          OPTIONAL,   -- Need S

timeInfoType-r16                    ENUMERATED {localClock}     OPTIONAL,   -- Need S

referenceSFN-r16                    INTEGER (0..1023)           OPTIONAL    -- Cond RefTime

}

ReferenceTime-r16 ::=           SEQUENCE {

refDays-r16                         INTEGER (0..72999),

refSeconds-r16                      INTEGER (0..86399),

refMilliSeconds-r16                 INTEGER (0..999),

refTenNanoSeconds-r16               INTEGER (0..99999)

}

-- TAG-REFERENCETIMEINFO-STOP

-- ASN1STOP

ReferenceTimeInfo field descriptions

referenceSFN This field indicates the reference SFN corresponding to the reference time information. If referenceTimeInfo field is received in DLInformationTransfer message, this field indicates the SFN of PCell.

time This field indicates time reference with 10ns granularity. If included in DLInformationTransfer and if UE-side TA PDC is de-activated, the indicated time may not be referenced at the network, i.e., gNB may pre-compensate for RF propagation delay. If included in DLInformationTransfer and if UE is requested to transmit UE Rx-Tx time difference measurement, the indicated time may not be referenced at the network, i.e., gNB may pre-compensate for RF propagation delay. Otherwise, the indicated time is referenced at the network, i.e., without compensating for RF propagation delay. In an NTN cell, the indicated time is referenced at the uplink time synchronization reference point (RP), i.e., UE should take into account the propagation delay between UE and RP when determining the UTC time at the UE. The indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If the referenceTimeInfo field is received in DLInformationTransfer message, the time field indicates the time at the ending boundary of the system frame indicated by referenceSFN. The UE considers this frame (indicated by referenceSFN) to be the frame which is nearest to the frame where the message is received (which can be either in the past or in the future). If the referenceTimeInfo field is received in SIB9, the time field indicates the time at the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. If referenceTimeInfo field is received in SIB9, this field is excluded when determining changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of valueTag in SIB1. NOTE: The estimated time in an NTN-cell may be less accurate than the estimated time in a TN-cell.

timeInfoType If timeInfoType is not included, the time indicates the GPS time and the origin of the time field is 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If timeInfoType is set to localClock, the origin of the time is unspecified.

uncertainty This field indicates the uncertainty of the reference time information provided by the time field. The uncertainty is 25ns multiplied by this field. If this field is absent, the uncertainty is unspecified.

| Conditional Presence | Explanation |
| --- | --- |
| RefTime | The field is mandatory present if referenceTimeInfo is included in DLInformationTransfer message; otherwise the field is absent. |

#### –	RejectWaitTime

The IE RejectWaitTime is used to provide the value in seconds for timer T302.

RejectWaitTime information element

-- ASN1START

-- TAG-REJECTWAITTIME-START

RejectWaitTime ::=                  INTEGER (1..16)

-- TAG-REJECTWAITTIME-STOP

-- ASN1STOP

#### –	RepetitionSchemeConfig

The IE RepetitionSchemeConfig is used to configure the UE with repetition schemes as specified in TS 38.214 [19] clause 5.1.

RepetitionSchemeConfig information element

-- ASN1START

-- TAG-REPETITIONSCHEMECONFIG-START

RepetitionSchemeConfig-r16 ::= CHOICE {

fdm-TDM-r16                        SetupRelease { FDM-TDM-r16 },

slotBased-r16                      SetupRelease { SlotBased-r16 }

}

RepetitionSchemeConfig-v1630 ::=   SEQUENCE {

slotBased-v1630                    SetupRelease { SlotBased-v1630 }

}

FDM-TDM-r16 ::=                SEQUENCE {

repetitionScheme-r16           ENUMERATED {fdmSchemeA, fdmSchemeB,tdmSchemeA },

startingSymbolOffsetK-r16      INTEGER (0..7)                                    OPTIONAL  -- Need R

}

SlotBased-r16 ::=              SEQUENCE {

tciMapping-r16                 ENUMERATED {cyclicMapping, sequentialMapping},

sequenceOffsetForRV-r16        INTEGER (1..3)

}

SlotBased-v1630 ::=            SEQUENCE {

tciMapping-r16                 ENUMERATED {cyclicMapping, sequentialMapping},

sequenceOffsetForRV-r16        INTEGER (0)

}

-- TAG-REPETITIONSCHEMECONFIG-STOP

-- ASN1STOP

RepetitionSchemeConfig field descriptions

fdm-TDM Configures UE with a repetition scheme among fdmSchemeA, fdmSchemeB and tdmSchemeA as specified in clause 5.1 of TS 38.214 [19]. The network does not set this field to release. Upon reception of this field in RepetitionSchemeConfig-r16, the UE shall release slotBased if previously configured in the same instance of RepetitionSchemeConfig-r16.

sequenceOffsetForRV For slot-based repetition scheme, selected RV sequence is applied to transmission occasions associated to the first TCI state. The RV sequence associated to the second TCI state is determined by a RV offset from that selected RV sequence.

slotBased Configures UE with slot-based repetition scheme. Network always configures this field when the parameter repetitionNumber is present in IE PDSCH-TimeDomainResourceAllocationList. The network does not set this field to release. Upon reception of this field in RepetitionSchemeConfig-r16, the UE shall release fdm-TDM if previously configured in the same instance of RepetitionSchemeConfig-r16.

startingSymbolOffsetK The starting symbol of the second transmission occasion has K symbol offset relative to the last symbol of the first transmission occasion. When UE is configured with tdmSchemeA, the parameter startingSymbolOffsetK is present, otherwise absent.

tciMapping Enables TCI state mapping method to PDSCH transmission occasions.

#### –	ReportConfigId

The IE ReportConfigId is used to identify a measurement reporting configuration.

ReportConfigId information element

-- ASN1START

-- TAG-REPORTCONFIGID-START

ReportConfigId ::=                          INTEGER (1..maxReportConfigId)

-- TAG-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	ReportConfigInterRAT

The IE ReportConfigInterRAT specifies criteria for triggering of an inter-RAT measurement reporting event, or an L2 U2N relay measurement reporting event. The inter-RAT measurement reporting events for E-UTRA and UTRA-FDD are labelled BN with N equal to 1, 2 and so on. The measurement reporting events for L2 U2N relay UE are labelled YN with N equal to 1, 2 and so on, and Z1.

Event B1:	Neighbour becomes better than absolute threshold;

Event B2:	PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2;

Event Y1: PCell becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

Event Y2: Candidate L2 U2N Relay UE becomes better than absolute threshold;

Event Z1: Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

ReportConfigInterRAT information element

-- ASN1START

-- TAG-REPORTCONFIGINTERRAT-START

ReportConfigInterRAT ::=                    SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfigInterRAT,

eventTriggered                              EventTriggerConfigInterRAT,

reportCGI                                   ReportCGI-EUTRA,

...,

reportSFTD                                  ReportSFTD-EUTRA

}

}

ReportCGI-EUTRA ::=                         SEQUENCE {

cellForWhichToReportCGI         EUTRA-PhysCellId,

...,

[[

useAutonomousGaps-r16           ENUMERATED {setup}                OPTIONAL     -- Need R

]]

}

ReportSFTD-EUTRA ::=                     SEQUENCE {

reportSFTD-Meas                            BOOLEAN,

reportRSRP                                 BOOLEAN,

...

}

EventTriggerConfigInterRAT ::=              SEQUENCE {

eventId                                     CHOICE {

eventB1                                     SEQUENCE {

b1-ThresholdEUTRA                           MeasTriggerQuantityEUTRA,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

...

},

eventB2                                     SEQUENCE {

b2-Threshold1                               MeasTriggerQuantity,

b2-Threshold2EUTRA                          MeasTriggerQuantityEUTRA,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

...

},

...,

[[

eventB1-UTRA-FDD-r16                         SEQUENCE {

b1-ThresholdUTRA-FDD-r16                    MeasTriggerQuantityUTRA-FDD-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger,

...

},

eventB2-UTRA-FDD-r16                         SEQUENCE {

b2-Threshold1-r16                           MeasTriggerQuantity,

b2-Threshold2UTRA-FDD-r16                   MeasTriggerQuantityUTRA-FDD-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger,

...

}

]],

[[

eventY1-Relay-r17                            SEQUENCE {

y1-Threshold1-r17                            MeasTriggerQuantity,

y1-Threshold2-Relay-r17                      SL-MeasTriggerQuantity-r16,

reportOnLeave-r17                            BOOLEAN,

hysteresis-r17                               Hysteresis,

timeToTrigger-r17                            TimeToTrigger,

...

},

eventY2-Relay-r17                            SEQUENCE {

y2-Threshold-Relay-r17                       SL-MeasTriggerQuantity-r16,

reportOnLeave-r17                            BOOLEAN,

hysteresis-r17                               Hysteresis,

timeToTrigger-r17                            TimeToTrigger,

...

}

]],

[[

eventZ1-Relay-r18                            SEQUENCE {

z1-Threshold1-Relay-r18                      SEQUENCE {

sl-RSRP-r18                                  SL-MeasTriggerQuantity-r16,

sd-RSRP-r18                                  SL-MeasTriggerQuantity-r16                OPTIONAL    -- Need S

},

z1-Threshold2-Relay-r18                      SL-MeasTriggerQuantity-r16,

reportOnLeave-r18                            BOOLEAN,

hysteresis-r18                               Hysteresis,

timeToTrigger-r18                            TimeToTrigger,

...

}

]]

},

rsType                              NR-RS-Type,

reportInterval                      ReportInterval,

reportAmount                        ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity                      MeasReportQuantity,

maxReportCells                      INTEGER (1..maxCellReport),

...,

[[

reportQuantityUTRA-FDD-r16          MeasReportQuantityUTRA-FDD-r16         OPTIONAL   -- Need R

]],

[[

includeCommonLocationInfo-r16       ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                  SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16              SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]],

[[

reportQuantityRelay-r17             SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]],

[[

cellIndividualOffsetList-r18        SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF CellIndividualOffsetList-EUTRA-r18 OPTIONAL -- Need R

]]

}

PeriodicalReportConfigInterRAT ::=              SEQUENCE {

reportInterval                                  ReportInterval,

reportAmount                                    ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity                                  MeasReportQuantity,

maxReportCells                                  INTEGER (1..maxCellReport),

...,

[[

reportQuantityUTRA-FDD-r16                      MeasReportQuantityUTRA-FDD-r16         OPTIONAL   -- Need R

]],

[[

includeCommonLocationInfo-r16       ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                  SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16              SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]],

[[

reportQuantityRelay-r17             SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]]

}

MeasTriggerQuantityUTRA-FDD-r16 ::=          CHOICE{

utra-FDD-RSCP-r16                            INTEGER (-5..91),

utra-FDD-EcN0-r16                            INTEGER (0..49)

}

MeasReportQuantityUTRA-FDD-r16 ::=        SEQUENCE {

cpich-RSCP                                BOOLEAN,

cpich-EcN0                                BOOLEAN

}

CellIndividualOffsetList-EUTRA-r18 ::=   SEQUENCE {

physCellId-r18                            EUTRA-PhysCellId,

cellIndividualOffset-r18                  EUTRA-Q-OffsetRange

}

-- TAG-REPORTCONFIGINTERRAT-STOP

-- ASN1STOP

ReportConfigInterRAT field descriptions

reportType Type of the configured measurement report. In (NG)EN-DC, and NR-DC, network does not configure report of type ReportCGI-EUTRA for SCG.

ReportCGI-EUTRA field descriptions

useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the E-UTRAN neighbour cell. When the field is included, the UE applies the corresponding value for T321.

EventTriggerConfigInterRAT field descriptions

b2-Threshold1 NR threshold to be used in inter RAT measurement report triggering condition for event B2.

bN-ThresholdEUTRA E-UTRA threshold value associated with the selected trigger quantity (RSRP, RSRQ, SINR) to be used in inter RAT measurement report triggering condition for event number bN. In the same eventB2, the network configures the same CHOICE name (rsrp, rsrq or sinr) for the MeasTriggerQuantity of the b2-Threshold1 and for the MeasTriggerQuantityEUTRA of the b2-Threshold2EUTRA.

eventId Choice of inter RAT event triggered reporting criteria.

maxReportCells Max number of non-serving cells/candidate L2 U2N Relay UEs to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantity, reportQuantityUTRA-FDD The cell measurement quantities to be included in the measurement report. If the field eventB1-UTRA-FDD or eventB2-UTRA-FDD is present, the UE shall ignore the value(s) provided in reportQuantity.

reportQuantityRelay The L2 U2N Relay UE measurement quantity to be included in measuremet report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

bN-ThresholdUTRA-FDD UTRA-FDD threshold value associated with the selected trigger quantity (RSCP, EcN0) to be used in inter RAT measurement report triggering condition for event number bN. utra-FDD-RSCP corresponds to CPICH_RSCP in TS 25.133 [46] for FDD. utra-FDD-EcN0 corresponds to CPICH_Ec/No in TS 25.133 [46] for FDD. For utra-FDD-RSCP: The actual value is field value – 115 dBm. For utra-FDD-EcN0: The actual value is (field value – 49)/2 dB.

y1-Threshold1 NR threshold to be used in measurement report triggering condition for event Y1.

y1-Threshold2-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. RSRP) to be used in measurement report triggering condition for event Y1.

y2-Threshold-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. RSRP) to be used in measurement report triggering condition for event Y2.

z1-Threshold1-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. SL-RSRP and/or SD-RSRP) to be used in measurement report triggering condition for serving Relay UE in event Z1. If the field sd-RSRP is not included, the UE considers it to be equal to sl-RSRP.

z1-Threshold2-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. SD-RSRP) to be used in measurement report triggering condition for candidate Relay UE in event Z1.

PeriodicalReportConfigInterRAT field descriptions

maxReportCells Max number of non-serving cells/candidate L2 U2N Relay UEs to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantity, reportQuantityUTRA-FDD The cell measurement quantities to be included in the measurement report. If the field reportQuantityUTRA-FDD is present, the UE shall ignore the value(s) provided in reportQuantity.

CellIndividualOffsetList-EUTRA field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific measurement event.

physCellId Physical cell identity of a E-UTRAN cell in the cell list.

#### –	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO, CPA or CPC event or of an L2 U2N relay measurement reporting event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO, CPA or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell;

Event D1:	Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 becomes shorter than configured threshold distanceThreshFromReference2;

Event D2:	Distance between UE and a moving reference location based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 for the serving cell becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation2 becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

CondEvent A4: Conditional reconfiguration candidate becomes better than absolute threshold where condEventA4 can also be used for current PSCell (i.e., in case it is configured as candidate PSCell for CondEvent A4 evaluation) for CHO with candidate SCG(s) case;

CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2;

CondEvent D1: Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent D2: Distance between UE and a moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 for the serving cell becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent T1: Time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration;

Event X1:	Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND NR Cell becomes better than another absolute threshold2;

Event X2:	Serving L2 U2N Relay UE becomes worse than absolute threshold;

For event I1, measurement reporting event is based on CLI measurement results, which can either be derived based on SRS-RSRP or CLI-RSSI.

Event I1:	Interference becomes higher than absolute threshold;

The reporting events concerning Aerial UE altitude are labelled HN with N equal to 1 and 2. Additionally, the reporting events concerning Aerial UE altitude and the neighboring cell measurements simultaneously are labelled AMHN with M equal to 3, 4, 5 and N equal to 1, 2.

Event H1:	Aerial UE altitude becomes higher than a threshold;

Event H2:	Aerial UE altitude becomes lower than a threshold;

Event A3H1:	Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes higher than a threshold;

Event A3H2:	Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes lower than a threshold;

Event A4H1:	Neighbour becomes better than threshold1 and the Aerial UE altitude becomes higher than a threshold2;

Event A4H2:	Neighbour becomes better than threshold1 and the Aerial UE altitude becomes lower than a threshold2;

Event A5H1:	SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes higher than a threshold3;

Event A5H2:	SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes lower than a threshold3.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...,

[[

useAutonomousGaps-r16            ENUMERATED {setup}                OPTIONAL   -- Need R

]]

}

ReportSFTD-NR ::=                 SEQUENCE {

reportSFTD-Meas                  BOOLEAN,

reportRSRP                       BOOLEAN,

...,

[[

reportSFTD-NeighMeas             ENUMERATED {true}                                OPTIONAL,   -- Need R

drx-SFTD-NeighMeas               ENUMERATED {true}                                OPTIONAL,   -- Need R

cellsForWhichToReportSFTD        SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId   OPTIONAL    -- Need R

]]

}

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

...,

condEventA4-r17                  SEQUENCE {

a4-Threshold-r17                 MeasTriggerQuantity,

hysteresis-r17                   Hysteresis,

timeToTrigger-r17                TimeToTrigger

},

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17           HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

condEventT1-r17                  SEQUENCE {

t1-Threshold-r17                 INTEGER (0..549755813887),

duration-r17                     INTEGER (1..6000)

},

condEventD2-r18                  SEQUENCE {

distanceThreshFromReference1-r18 INTEGER(0.. 65535),

distanceThreshFromReference2-r18 INTEGER(0.. 65535),

referenceLocation2-r18           ReferenceLocation-r17,

hysteresisLocation-r18           HysteresisLocation-r17,

timeToTrigger-r18                TimeToTrigger

}

},

rsType-r16                       NR-RS-Type,

...,

[[

nesEvent-r18                     ENUMERATED {true}                                OPTIONAL    -- Need R

]]

}

EventTriggerConfig ::=                      SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA3                                     SEQUENCE {

a3-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventA6                                     SEQUENCE {

a6-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

...,

[[

eventX1-r17                                 SEQUENCE {

x1-Threshold1-Relay-r17                     SL-MeasTriggerQuantity-r16,

x1-Threshold2-r17                           MeasTriggerQuantity,

reportOnLeave-r17                           BOOLEAN,

hysteresis-r17                              Hysteresis,

timeToTrigger-r17                           TimeToTrigger,

useAllowedCellList-r17                      BOOLEAN

},

eventX2-r17                                 SEQUENCE {

x2-Threshold-Relay-r17                      SL-MeasTriggerQuantity-r16,

reportOnLeave-r17                           BOOLEAN,

hysteresis-r17                              Hysteresis,

timeToTrigger-r17                           TimeToTrigger

},

eventD1-r17                                 SEQUENCE {

distanceThreshFromReference1-r17            INTEGER(1.. 65525),

distanceThreshFromReference2-r17            INTEGER(1.. 65525),

referenceLocation1-r17                      ReferenceLocation-r17,

referenceLocation2-r17                      ReferenceLocation-r17,

reportOnLeave-r17                           BOOLEAN,

hysteresisLocation-r17                      HysteresisLocation-r17,

timeToTrigger-r17                           TimeToTrigger

}

]],

[[

eventH1-r18                                SEQUENCE {

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

reportOnLeave-r18                           BOOLEAN,

timeToTrigger-r18                           TimeToTrigger,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventH2-r18                                SEQUENCE {

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

reportOnLeave-r18                           BOOLEAN,

timeToTrigger-r18                           TimeToTrigger,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA3H1-r18                              SEQUENCE {

a3-Offset-r18                               MeasTriggerQuantityOffset,

reportOnLeave-r18                           BOOLEAN,

a3-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA3H2-r18                              SEQUENCE {

a3-Offset-r18                               MeasTriggerQuantityOffset,

reportOnLeave-r18                           BOOLEAN,

a3-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA4H1-r18                              SEQUENCE {

a4-Threshold-r18                            MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a4-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA4H2-r18                              SEQUENCE {

a4-Threshold-r18                            MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a4-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA5H1-r18                              SEQUENCE {

a5-Threshold1-r18                           MeasTriggerQuantity,

a5-Threshold2-r18                           MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a5-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA5H2-r18                             SEQUENCE {

a5-Threshold1-r18                           MeasTriggerQuantity,

a5-Threshold2-r18                           MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a5-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventD2-r18                                 SEQUENCE {

distanceThreshFromReference1-r18            INTEGER(1.. 65535),

distanceThreshFromReference2-r18            INTEGER(1.. 65535),

referenceLocation2-r18                      ReferenceLocation-r17,

reportOnLeave-r18                           BOOLEAN,

hysteresisLocation-r18                      HysteresisLocation-r17,

timeToTrigger-r18                           TimeToTrigger

}

]]

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

useT312-r16                                 BOOLEAN                                                        OPTIONAL,   -- Need M

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]],

[[

coarseLocationRequest-r17                   ENUMERATED {true}                                              OPTIONAL,   -- Need R

reportQuantityRelay-r17                     SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]],

[[

numberOfTriggeringCells-r18                 INTEGER (2..maxCellReport)                                     OPTIONAL,   -- Need R

cellIndividualOffsetList-r18                SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellIndividualOffsetList-r18 OPTIONAL, -- Need R

eventX1-SD-Threshold1-r18                   SL-MeasTriggerQuantity-r16                                     OPTIONAL,   -- Need S

eventX2-SD-Threshold-r18                    SL-MeasTriggerQuantity-r16                                     OPTIONAL    -- Need S

-- Editor’s Note: FFS if this is proper place for Event X1 thresolds.

]]

}

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useAllowedCellList                          BOOLEAN,

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL,   -- Need M

ul-DelayValueConfig-r16                     SetupRelease { UL-DelayValueConfig-r16 }                       OPTIONAL,   -- Need M

reportAddNeighMeas-r16                      ENUMERATED {setup}                                             OPTIONAL    -- Need R

]],

[[

ul-ExcessDelayConfig-r17                    SetupRelease { UL-ExcessDelayConfig-r17 }                      OPTIONAL,   -- Need M

coarseLocationRequest-r17                   ENUMERATED {true}                                              OPTIONAL,   -- Need R

reportQuantityRelay-r17                     SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]]

}

NR-RS-Type ::=                              ENUMERATED {ssb, csi-rs}

MeasTriggerQuantity ::=                     CHOICE {

rsrp                                        RSRP-Range,

rsrq                                        RSRQ-Range,

sinr                                        SINR-Range

}

MeasTriggerQuantityOffset ::=               CHOICE {

rsrp                                        INTEGER (-30..30),

rsrq                                        INTEGER (-30..30),

sinr                                        INTEGER (-30..30)

}

MeasReportQuantity ::=                      SEQUENCE {

rsrp                                        BOOLEAN,

rsrq                                        BOOLEAN,

sinr                                        BOOLEAN

}

MeasRSSI-ReportConfig-r16 ::=               SEQUENCE {

channelOccupancyThreshold-r16               RSSI-Range-r16         OPTIONAL   -- Need R

}

CLI-EventTriggerConfig-r16 ::=              SEQUENCE {

eventId-r16                                 CHOICE {

eventI1-r16                                 SEQUENCE {

i1-Threshold-r16                            MeasTriggerQuantityCLI-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger

},

...

},

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

CLI-PeriodicalReportConfig-r16 ::=          SEQUENCE {

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCLI-r16                       MeasReportQuantityCLI-r16,

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

RxTxPeriodical-r17  ::=                     SEQUENCE {

rxTxReportInterval-r17                      RxTxReportInterval-r17                             OPTIONAL,   -- Need R

reportAmount-r17                            ENUMERATED {r1, infinity, spare6, spare5, spare4, spare3, spare2, spare1},

...

}

RxTxReportInterval-r17 ::= ENUMERATED {ms80,ms120,ms160,ms240,ms320,ms480,ms640,ms1024,ms1280,ms2048,ms2560,ms5120,spare4,spare3,spare2,spare1}

MeasTriggerQuantityCLI-r16 ::=              CHOICE {

srs-RSRP-r16                                SRS-RSRP-Range-r16,

cli-RSSI-r16                                CLI-RSSI-Range-r16

}

MeasReportQuantityCLI-r16 ::=               ENUMERATED {srs-rsrp, cli-rssi}

ReportOnScellActivation-r18 ::=             SEQUENCE {

rsType                                      NR-RS-Type,

reportQuantityRS-Indexes                    MeasReportQuantity,

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport),

includeBeamMeasurements                     BOOLEAN

}

CellIndividualOffsetList-r18 ::=    SEQUENCE {

physCellId-r18                      PhysCellId,

cellIndividualOffset-r18            Q-OffsetRangeList

}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition for cond event a3. The actual value is field value * 0.5 dB.

a4-Threshold Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a4.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration event triggered criteria.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 or a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation2 and their corresponding satellite ephemeris and epoch time. Each step represents 50m.

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

nesEvent Indicates the event is a NES-specific CHO event and the event is only considered to be satisfied if indication from lower layers is received indicating the applicability of NES-specific CHO event and the related entry condition(s) is fulfilled. This field can only be configured for condEventA3, condEventA4 or condEventA5. This field cannot be configured for CPAC.

referenceLocation1, referenceLocation2 For condEventD1, the referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell. For condEventD2, the refereceLocation2 is associated to candidate target cell.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation.

ReportConfigNR field descriptions

reportType Type of the configured measurement report. In MR-DC, network does not configure report of type reportCGI using SRB3. The condTriggerConfig is used for CHO, CPA or CPC configuration.

ReportCGI field descriptions

useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the NR neighbour cell. When the field is included, the UE applies the corresponding value for T321.

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6. The actual value is field value * 0.5 dB.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. In the same eventA5, eventA5H1, eventA5H2, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

channelOccupancyThreshold RSSI threshold which is used for channel occupancy evaluation.

coarseLocationRequest This field is used to request UE to report coarse location information.

distanceThreshFromReference1, distanceThreshFromReference2 Threshold value associated to the distance from a reference location configured with referenceLocation1 or referenceLocation2. Each step represents 50m.

eventId Choice of NR event triggered reporting criteria.

eventXN-SD-Threshold Indicates the SD-RSRP threshold value for the serving L2 U2N Relay UE in event XN (N equals 1 or 2). If this field is not included, the UE considers the SD-RSRP threshold value equals to the one indicated by x1-Threshold1-Relay/ x2-Threshold-Relay.

includeAltitudeUE This field is used to request UE to report altitude information.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

numberOfTriggeringCells Indicates the number of cells detected that are required to fulfill an event for a measurement report to be triggered. This field is applicable only for the events concerning neighbor cells, i.e. eventA3, eventA4, eventA5, eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, eventA5H2.

referenceLocation1, referenceLocation2 For eventD1, the referenceLocation1 is associated to serving cell and referenceLocation2 is associated to neighbour cell. For eventD2, the refereceLocation2 is associated to neighbour cell.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1. Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met if configured in eventD1, eventD2, eventH1, eventH2 as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

simulMultiTriggerSingleMeasReport Indicates when multiple events with the same eventID satisfy the measurement report triggering condition(s), whether to consider only the event with the smallest value between the altitude of the UE and the configured altitude threshold.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

useAllowedCellList Indicates whether only the cells included in the allow-list of the associated measObject are applicable as specified in 5.5.4.1.

useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObjectNR. If value FALSE is configured, the timer T312 is considered as disabled. Network configures value TRUE only if reportType is set to eventTriggered.

xN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event xN. If multiple thresholds are defined for event number xN, the thresholds are differentiated by M. x1-Threshold1 and x2-Threshold indicates the threshold value for the serving L2 U2N Relay UE, x1-Threshold2 indicates the threshold value for the NR Cells.

CLI-EventTriggerConfig field descriptions

i1-Threshold Threshold value associated to the selected trigger quantity (e.g. SRS-RSRP, CLI-RSSI) to be used in CLI measurement report triggering condition for event i1.

eventId Choice of CLI event triggered reporting criteria.

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a CLI measurement resource in srsTriggeredList or rssiTriggeredList, as specified in 5.5.4.1.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

CLI-PeriodicalReportConfig field descriptions

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportQuantityCLI The CLI measurement quantities to be included in the measurement report.

PeriodicalReportConfig field descriptions

coarseLocationRequest This field is used to request UE to report coarse location information.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

ul-DelayValueConfig If the field is present, the UE shall perform the actual UL PDCP Packet Average Delay measurement per DRB as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Packet Average Delay per DRB measurement as specified in TS 38.314 [53].

ul-ExcessDelayConfig If the field is present, the UE shall perform the actual UL PDCP Excess Packet Delay per DRB measurement as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Excess Packet Delay per DRB measurement as specified in TS 38.314 [53].

useAllowedCellList Indicates whether only the cells included in the allow-list of the associated measObject are applicable as specified in 5.5.4.1.

ReportSFTD-NR field descriptions

cellForWhichToReportSFTD Indicates the target NR neighbour cells for SFTD measurement between PCell and NR neighbour cells.

drx-SFTD-NeighMeas Indicates that the UE shall use available idle periods (i.e. DRX off periods) for the SFTD measurement in NR standalone. The network only includes drx-SFTD-NeighMeas field when reprtSFTD-NeighMeas is set to true.

reportSFTD-Meas Indicates whether UE is required to perform SFTD measurement between PCell and NR PSCell in NR-DC.

reportSFTD-NeighMeas Indicates whether UE is required to perform SFTD measurement between PCell and NR neighbour cells in NR standalone. The network does not include this field if reportSFTD-Meas is set to true.

reportRSRP Indicates whether UE is required to include RSRP result of NR PSCell or NR neighbour cells in SFTD measurement result, derived based on SSB. If it is set to true, the network should ensure that ssb-ConfigMobility is included in the measurement object for NR PSCell or NR neighbour cells.

RxTxPeriodical field descriptions

reportAmount This field indicates the number of UE Rx-Tx time difference measurement reports. If configured to r1, the network does not configure rxTxReportInterval and only one measurement is reported. If configured to infinity, UE periodically reports measurements according to the periodicity configured by rxTxReportInterval.

rxTxReportInterval This field indicates the measurement reporting periodicity of UE Rx-Tx time difference.

other field descriptions

MeasTriggerQuantity SINR is applicable only for CONNECTED mode events.

ReportOnScellActivation field descriptions

rsType Indicates which RS is used to provide the measurement result. Only value ssb can be set in this release.

reportQuantityRS-Indexes Indicates which measurement information per RS index is used to sort the reported measurement results and is included in the measurement report.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report.

includeBeamMeasurements Indicates whether to include the measurement result per RS index in the measurement report.

CellIndividualOffsetList field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific measurement event.

physCellId Physical cell identity of a cell in the cell list.

#### –	ReportConfigNR-SL

The IE ReportConfigNR-SL specifies criteria for triggering of a CBR measurement reporting event for NR sidelink communication/discovery. Measurement reporting events are based on CBR measurement results on the corresponding transmission resource pools. These events are labelled CN with N equal to 1 and 2.

Event C1:	CBR of NR sidelink communication/discovery is above a threshold;

Event C2:	CBR of NR sidelink communication/discovery is below a threshold;

ReportConfigNR-SL information element

-- ASN1START

-- TAG-REPORTCONFIGNR-SL-START

ReportConfigNR-SL-r16 ::=            SEQUENCE {

reportType-r16                       CHOICE {

periodical-r16                       PeriodicalReportConfigNR-SL-r16,

eventTriggered-r16                   EventTriggerConfigNR-SL-r16

}

}

EventTriggerConfigNR-SL-r16::=       SEQUENCE {

eventId-r16                          CHOICE {

eventC1                              SEQUENCE {

c1-Threshold-r16                     SL-CBR-r16,

hysteresis-r16                       Hysteresis,

timeToTrigger-r16                    TimeToTrigger

},

eventC2-r16                  SEQUENCE {

c2-Threshold-r16             SL-CBR-r16,

hysteresis-r16               Hysteresis,

timeToTrigger-r16            TimeToTrigger

},

...

},

reportInterval-r16               ReportInterval,

reportAmount-r16                 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity-r16               MeasReportQuantity-r16,

...

}

PeriodicalReportConfigNR-SL-r16 ::=  SEQUENCE {

reportInterval-r16                   ReportInterval,

reportAmount-r16                     ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity-r16                   MeasReportQuantity-r16,

...

}

MeasReportQuantity-r16 ::=           SEQUENCE {

cbr-r16                              BOOLEAN,

...

}

-- TAG-REPORTCONFIGNR-SL-STOP

-- ASN1STOP

ReportConfigNR-SL field descriptions

reportType Type of the configured CBR measurement report for NR sidelink communication/discovery.

EventTriggerConfigNR-SL field descriptions

cN-Threshold Threshold used for events C1 and C2 specified in clauses 5.5.4.11 and 5.5.4.12, respectively.

eventId Choice of NR event triggered reporting criteria.

reportAmoun Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportQuantity The sidelink measurement quantities to be included in the measurement report. In this release, this is set as the CBR measurement result.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

PeriodicalReportConfigNR-SL field descriptions

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportQuantity The sidelink measurement quantities to be included in the measurement report. In this release, this is set as the CBR measurement result.

#### –	ReportConfigToAddModList

The IE ReportConfigToAddModList concerns a list of reporting configurations to add or modify.

ReportConfigToAddModList information element

-- ASN1START

-- TAG-REPORTCONFIGTOADDMODLIST-START

ReportConfigToAddModList ::=        SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod

ReportConfigToAddMod ::=            SEQUENCE {

reportConfigId                      ReportConfigId,

reportConfig                        CHOICE {

reportConfigNR                      ReportConfigNR,

...,

reportConfigInterRAT                ReportConfigInterRAT,

reportConfigNR-SL-r16               ReportConfigNR-SL-r16

}

}

-- TAG-REPORTCONFIGTOADDMODLIST-STOP

-- ASN1STOP

#### –	ReportInterval

The IE ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1) when reportType is set to either eventTriggered, periodical, cli-EventTriggered or cli-Periodical. Value ms120 corresponds to 120 ms, value ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

-- TAG-REPORTINTERVAL-START

ReportInterval ::=                  ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

-- TAG-REPORTINTERVAL-STOP

-- ASN1STOP

#### –	ReselectionThreshold

The IE ReselectionThreshold is used to indicate an Rx level threshold for cell reselection. Actual value of threshold = field value * 2 [dB].

ReselectionThreshold information element

-- ASN1START

-- TAG-RESELECTIONTHRESHOLD-START

ReselectionThreshold ::=                INTEGER (0..31)

-- TAG-RESELECTIONTHRESHOLD-STOP

-- ASN1STOP

#### –	ReselectionThresholdQ

The IE ReselectionThresholdQ is used to indicate a quality level threshold for cell reselection. Actual value of threshold = field value [dB].

ReselectionThresholdQ information element

-- ASN1START

-- TAG-RESELECTIONTHRESHOLDQ-START

ReselectionThresholdQ ::=           INTEGER (0..31)

-- TAG-RESELECTIONTHRESHOLDQ-STOP

-- ASN1STOP

#### –	ResumeCause

The IE ResumeCause is used to indicate the resume cause in RRCResumeRequest, RRCResumeRequest1 and UEAssistanceInformation.

ResumeCause information element

-- ASN1START

-- TAG-RESUMECAUSE-START

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, mt-SDT-v1810, srs-PosConfigOrActivationReq-v1800, spare3, spare2, spare1 }

-- TAG-RESUMECAUSE-STOP

-- ASN1STOP

#### –	RLC-BearerConfig

The IE RLC-BearerConfig is used to configure an RLC entity, a corresponding logical channel in MAC and the linking to a PDCP entity (served radio bearer).

RLC-BearerConfig information element

-- ASN1START

-- TAG-RLC-BEARERCONFIG-START

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

rlc-Config                                  RLC-Config                                          OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                    LogicalChannelConfig                                OPTIONAL,   -- Cond LCH-Setup

...,

[[

rlc-Config-v1610                            RLC-Config-v1610                                    OPTIONAL    -- Need R

]],

[[

rlc-Config-v1700                            RLC-Config-v1700                                    OPTIONAL,   -- Need R

logicalChannelIdentityExt-r17               LogicalChannelIdentityExt-r17                       OPTIONAL,   -- Cond LCH-SetupModMRB

multicastRLC-BearerConfig-r17               MulticastRLC-BearerConfig-r17                       OPTIONAL,   -- Cond LCH-SetupOnlyMRB

servedRadioBearerSRB4-r17                   SRB-Identity-v1700                                  OPTIONAL    -- Need N

]]

}

MulticastRLC-BearerConfig-r17 ::=           SEQUENCE {

servedMBS-RadioBearer-r17                   MRB-Identity-r17,

isPTM-Entity-r17                            ENUMERATED {true}                                   OPTIONAL    -- Need S

}

LogicalChannelIdentityExt-r17 ::=           INTEGER (320..65855)

-- TAG-RLC-BEARERCONFIG-STOP

-- ASN1STOP

RLC-BearerConfig field descriptions

isPTM-Entity If configured, indicates that the RLC entity is used for PTM reception. When the field is absent the RLC entity is used for PTP transmission/reception.

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer. Value 4 is not configured for DRBs if SRB4 is configured.

logicalChannelIdentityExt Extended logical channel ID used commonly for the MAC logical channel and for the RLC bearer for PTM reception. If this field is configured, the UE shall ignore logicalChannelIdentity.

reestablishRLC Indicates that RLC should be re-established. Network sets this to true at least whenever the security key used for the radio bearer associated with this RLC entity changes. For SRB2, multicast MRBs and DRBs, unless full configuration is used, it is also set to true during the resumption of the RRC connection or the first reconfiguration after reestablishment. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment, the network does not set this field to true. The network does not include this field if servedRadioBearer is set to drb-Identity and the RLC-BearerConfig IE is part of an RRCReconfiguration message contained in ltm-CandidateConfig. For SRB3 and DRBs, network doesn't include this field if the RLC-BearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters. RLC mode reconfiguration can only be performed by DRB/multicast MRB release/addition or full configuration. The network may configure rlc-Config-v1610 only when rlc-Config (without suffix) is set to am.

servedMBS-RadioBearer Associates the RLC Bearer with a multicast MRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedMBS-RadioBearer.

servedRadioBearer, servedRadioBearerSRB4 Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling restrictions (moreThanOneRLC in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present upon creation of a new logical channel for a DRB or a multicast MRB or SRB4. This field is optionally present, Need S, upon creation of a new logical channel for an SRB except SRB4. It is optionally present, Need M, otherwise. |
| LCH-SetupModMRB | This field is optionally present upon creation of a new logical channel for PTM reception for a multicast MRB. If this field is included upon creation of a new logical channel for PTM reception for a multicast MRB, it shall be present when modifying this logical channel. The field is absent for logical channels configured for an SRB and a DRB. |
| LCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a DRB or an SRB (servedRadioBearer). It is absent, Need M otherwise. |
| LCH-SetupOnlyMRB | This field is mandatory present upon creation of a new logical channel for a multicast MRB and upon modification of MRB-Identity of the served MRB. It is absent, Need M otherwise. |

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs, multicast MRBs and DRBs.

RLC-Config information element

-- ASN1START

-- TAG-RLC-CONFIG-START

RLC-Config ::=                      CHOICE {

am                                  SEQUENCE {

ul-AM-RLC                           UL-AM-RLC,

dl-AM-RLC                           DL-AM-RLC

},

um-Bi-Directional                   SEQUENCE {

ul-UM-RLC                           UL-UM-RLC,

dl-UM-RLC                           DL-UM-RLC

},

um-Uni-Directional-UL               SEQUENCE {

ul-UM-RLC                           UL-UM-RLC

},

um-Uni-Directional-DL               SEQUENCE {

dl-UM-RLC                           DL-UM-RLC

},

...

}

UL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-PollRetransmit                    T-PollRetransmit,

pollPDU                             PollPDU,

pollByte                            PollByte,

maxRetxThreshold                    ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }

}

DL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly,

t-StatusProhibit                    T-StatusProhibit

}

UL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL    -- Cond Reestab

}

DL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly

}

T-PollRetransmit ::=                ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610,

ms4-v1610, spare1}

PollPDU ::=                         ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384,p20480,

p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

PollByte ::=                        ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75,

kB100, kB125, kB250, kB375, kB500, kB750, kB1000,

kB1250, kB1500, kB2000, kB3000, kB4000, kB4500,

kB5000, kB5500, kB6000, kB6500, kB7000, kB7500,

mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15,

mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity,

spare20, spare19, spare18, spare17, spare16,

spare15, spare14, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

T-Reassembly ::=                    ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, spare1}

T-StatusProhibit ::=                ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

SN-FieldLengthUM ::=                ENUMERATED {size6, size12}

SN-FieldLengthAM ::=                ENUMERATED {size12, size18}

RLC-Config-v1610 ::=                SEQUENCE {

dl-AM-RLC-v1610                     DL-AM-RLC-v1610

}

RLC-Config-v1700 ::=                SEQUENCE {

dl-AM-RLC-v1700                     DL-AM-RLC-v1700,

dl-UM-RLC-v1700                     DL-UM-RLC-v1700

}

DL-AM-RLC-v1610 ::=                 SEQUENCE {

t-StatusProhibit-v1610              T-StatusProhibit-v1610                               OPTIONAL,   -- Need R

...

}

DL-AM-RLC-v1700 ::=                 SEQUENCE {

t-ReassemblyExt-r17                 T-ReassemblyExt-r17                                  OPTIONAL    -- Need R

}

DL-UM-RLC-v1700 ::=                 SEQUENCE {

t-ReassemblyExt-r17                 T-ReassemblyExt-r17                                  OPTIONAL    -- Need R

}

T-StatusProhibit-v1610 ::=          ENUMERATED { ms1, ms2, ms3, ms4, spare4, spare3, spare2, spare1}

T-ReassemblyExt-r17 ::=             ENUMERATED {ms210, ms220, ms340, ms350, ms550, ms1100, ms1650, ms2200}

-- TAG-RLC-CONFIG-STOP

-- ASN1STOP

RLC-Config field descriptions

maxRetxThreshold Parameter for RLC AM in TS 38.322 [4]. Value t1 corresponds to 1 retransmission, value t2 corresponds to 2 retransmissions and so on.

pollByte Parameter for RLC AM in TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, value kB50 corresponds to 50 kBytes and so on. infinity corresponds to an infinite amount of kBytes.

pollPDU Parameter for RLC AM in TS 38.322 [4]. Value p4 corresponds to 4 PDUs, value p8 corresponds to 8 PDUs and so on. infinity corresponds to an infinite number of PDUs.

sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The value of sn-FieldLength of an RLC entity for the DRB/multicast MRB shall be changed only using reconfiguration with sync. The network configures only value size12 in SN-FieldLengthAM for SRB.

t-PollRetransmit Timer for RLC AM in TS 38.322 [4], in milliseconds. Value ms5 means 5 ms, value ms10 means 10 ms and so on.

t-Reassembly, t-ReassemblyExt Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-ReassemblyExt-r17 is configured, the UE shall ignore t-Reassembly (without suffix).

t-StatusProhibit Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-StatusProhibit-v1610 is present, the UE shall ignore t-StatusProhibit (without suffix).

| Conditional Presence | Explanation |
| --- | --- |
| Reestab | The field is mandatory present at RLC bearer setup. It is optionally present, need M, at RLC re-establishment. Otherwise it is absent. Need M. |

#### –	RLF-TimersAndConstants

The IE RLF-TimersAndConstants is used to configure UE specific timers and constants.

RLF-TimersAndConstants information element

-- ASN1START

-- TAG-RLF-TIMERSANDCONSTANTS-START

RLF-TimersAndConstants ::=          SEQUENCE {

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}

]]

}

-- TAG-RLF-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

RLF-TimersAndConstants field descriptions

n3xy Constants are described in clause 7.3. Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

t3xy Timers are described in clause 7.1. Value ms0 corresponds to 0 ms, value ms50 corresponds to 50 ms and so on.

#### –	RNTI-Value

The IE RNTI-Value represents a Radio Network Temporary Identity.

RNTI-Value information element

-- ASN1START

-- TAG-RNTI-VALUE-START

RNTI-Value ::=                      INTEGER (0..65535)

-- TAG-RNTI-VALUE-STOP

-- ASN1STOP

#### –	RSRP-Range

The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. For measurements, integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 156) dBm, except for the IE value 127, in which case the actual value is infinity.

RSRP-Range information element

-- ASN1START

-- TAG-RSRP-RANGE-START

RSRP-Range ::=                      INTEGER(0..127)

-- TAG-RSRP-RANGE-STOP

-- ASN1STOP

#### –	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. For measurements, integer value for RSRQ measurements is according to Table 10.1.11.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 87) / 2 dB.

RSRQ-Range information element

-- ASN1START

-- TAG-RSRQ-RANGE-START

RSRQ-Range ::=                      INTEGER(0..127)

-- TAG-RSRQ-RANGE-STOP

-- ASN1STOP

#### –	RSSI-Range

The IE RSSI-Range specifies the value range used in RSSI measurements and thresholds for NR operation with shared spectrum channel access. The integer value for RSSI measurements is according to Table 10.1.34.3-1 in TS 38.133 [14].

RSSI-Range information element

-- ASN1START

-- TAG-RSSI-RANGE-START

RSSI-Range-r16 ::=                  INTEGER(0..76)

-- TAG-RSSI-RANGE-STOP

-- ASN1STOP

#### –	RxTxTimeDiff

The IE RxTxTimeDiff contains the Rx-Tx time difference measurement at either the UE or the gNB.

RxTxTimeDiff information element

-- ASN1START

-- TAG-RXTXTIMEDIFF-START

RxTxTimeDiff-r17  ::= SEQUENCE {

result-k5-r17         INTEGER (0..61565)                 OPTIONAL, -- Need N

...

}

-- TAG-RXTXTIMEDIFF-STOP

-- ASN1STOP

RxTxTimeDiff field descriptions

result-k5 This field indicates the Rx-Tx time difference measurement, see TS 38.215 [9], clause 10.1.25.3.1 of TS 38.133 [14] for UE Rx-Tx time difference and clause 13.2.1 of TS 38.133 [14] for gNB Rx-Tx time difference.

#### –	SCellActivationRS-Config

The IE SCellActivationRS-Config is used to configure a Reference Signal for fast activation of the SCell where the IE is included (see TS 38.214 [19], clause 5.2.1.5.3. Usage of an SCellActivationRS-Config is indicated by including its scellActivationRS-Id in the Enhanced SCell activation MAC CE (see TS 38.321 [3] clause 6.1.3.55).

SCellActivationRS-Config information element

-- ASN1START

-- TAG-SCELLACTIVATIONRS-CONFIG-START

SCellActivationRS-Config-r17 ::= SEQUENCE {

scellActivationRS-Id-r17         SCellActivationRS-ConfigId-r17,

resourceSet-r17                  NZP-CSI-RS-ResourceSetId,

gapBetweenBursts-r17             INTEGER (2..31)                                                            OPTIONAL, -- Need R

qcl-Info-r17                     TCI-StateId,

...

}

-- TAG-SCELLACTIVATIONRS-CONFIG-STOP

-- ASN1STOP

SCellActivationRS-Config field descriptions

gapBetweenBursts When this field is present, there are two bursts and it indicates the gap between the two bursts in number of slots. When this field is absent, there is a single burst.

qcl-Info Reference to TCI-State for providing the QCL source and QCL type for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources of the NZP-CSI-RS-ResourceSet indicated by resourceSet (see TS 38.214 [19], clause 5.1.6.1.1.1). TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink of this serving cell indicated by firstActiveDownlinkBWP-Id in the ServingCellConfig in which this IE is included.

resourceSet nzp-CSI-ResourceSetId of the NZP-CSI-RS-ResourceSet of this serving cell used as resource configuration for one or two bursts for SCell activation. This NZP-CSI-RS-ResourceSet consists of four NZP CSI-RS resources in two consecutive slots with two NZP CSI-RS resources in each slot (see TS 38.214 [19], clause 5.1.6.1.1.1). The CSI-RS associated with this NZP-CSI-RS-ResourceSet are located in the BWP addressed by firstActiveDownlinkBWP-Id.

#### –	SCellActivationRS-ConfigId

The IE SCellActivationRS-ConfigId is used to identify one SCellActivationRS-Config.

SCellActivationRS-ConfigId information element

-- ASN1START

-- TAG-SCELLACTIVATIONRS-CONFIGID-START

SCellActivationRS-ConfigId-r17 ::=        INTEGER (1.. maxNrofSCellActRS-r17)

-- TAG-SCELLACTIVATIONRS-CONFIGID-STOP

-- ASN1STOP

#### –	SCellIndex

The IE SCellIndex concerns a short identity, used to identify an SCell. The value range is shared across the Cell Groups.

SCellIndex information element

-- ASN1START

-- TAG-SCELLINDEX-START

SCellIndex ::=                      INTEGER (1..31)

-- TAG-SCELLINDEX-STOP

-- ASN1STOP

#### –	SchedulingRequestConfig

The IE SchedulingRequestConfig is used to configure the parameters, for the dedicated scheduling request (SR) resources.

SchedulingRequestConfig information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTCONFIG-START

SchedulingRequestConfig ::=         SEQUENCE {

schedulingRequestToAddModList       SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod

OPTIONAL, -- Need N

schedulingRequestToReleaseList      SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestId

OPTIONAL  -- Need N

}

SchedulingRequestToAddMod ::=       SEQUENCE {

schedulingRequestId                 SchedulingRequestId,

sr-ProhibitTimer                    ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms32, ms64, ms128}          OPTIONAL, -- Need S

sr-TransMax                         ENUMERATED { n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

SchedulingRequestConfig-v1700 ::=       SEQUENCE {

schedulingRequestToAddModListExt-v1700  SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddModExt-v1700

OPTIONAL  -- Need N

}

SchedulingRequestToAddModExt-v1700 ::=  SEQUENCE {

sr-ProhibitTimer-v1700                  ENUMERATED { ms192, ms256, ms320, ms384, ms448, ms512, ms576, ms640, ms1082, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

OPTIONAL  -- Need R

}

-- TAG-SCHEDULINGREQUESTCONFIG-STOP

-- ASN1STOP

SchedulingRequestConfig field descriptions

schedulingRequestToAddModList, schedulingRequestToAddModListExt List of Scheduling Request configurations to add or modify. If schedulingRequestToAddModListExt is configured, it contains the same number of entries, and in the same order, as schedulingRequestToAddModList.

schedulingRequestToReleaseList List of Scheduling Request configurations to release.

SchedulingRequestToAddMod field descriptions

schedulingRequestId Used to modify a SR configuration and to indicate, in LogicalChannelConfig, the SR configuration to which a logical channel is mapped and to indicate, in SchedulingRequestresourceConfig, the SR configuration for which a scheduling request resource is used.

sr-ProhibitTimer Timer for SR transmission on PUCCH in TS 38.321 [3]. Value is in ms. Value ms1 corresponds to 1ms, value ms2 corresponds to 2ms, and so on. If sr-ProhibitTimer-v1700 is configured, UE shall ignore sr-ProhibitTimer (without suffix). If both sr-ProhibitTimer (without suffix) and sr-ProhibitTimer-v1700 are absent, the UE applies the value 0.

sr-TransMax Maximum number of SR transmissions as described in TS 38.321 [3]. Value n4 corresponds to 4, value n8 corresponds to 8, and so on.

#### –	SchedulingRequestId

The IE SchedulingRequestId is used to identify a Scheduling Request instance in the MAC layer.

SchedulingRequestId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTID-START

SchedulingRequestId ::=             INTEGER (0..7)

-- TAG-SCHEDULINGREQUESTID-STOP

-- ASN1STOP

#### –	SchedulingRequestResourceConfig

The IE SchedulingRequestResourceConfig determines physical layer resources on PUCCH where the UE may send the dedicated scheduling request (D-SR) (see TS 38.213 [13], clause 9.2.4).

SchedulingRequestResourceConfig information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-START

SchedulingRequestResourceConfig ::=     SEQUENCE {

schedulingRequestResourceId             SchedulingRequestResourceId,

schedulingRequestID                     SchedulingRequestId,

periodicityAndOffset                    CHOICE {

sym2                                    NULL,

sym6or7                                 NULL,

sl1                                     NULL,                       -- Recurs in every slot

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639)

}                                                                                                       OPTIONAL,   -- Need M

resource                                PUCCH-ResourceId                                                OPTIONAL    -- Need M

}

SchedulingRequestResourceConfigExt-v1610 ::=   SEQUENCE {

phy-PriorityIndex-r16                       ENUMERATED {p0, p1}                                         OPTIONAL,   -- Need M

...

}

SchedulingRequestResourceConfigExt-v1700 ::=    SEQUENCE {

periodicityAndOffset-r17                        CHOICE {

sl1280                                          INTEGER (0..1279),

sl2560                                          INTEGER (0..2559),

sl5120                                          INTEGER (0..5119)

}                                                                                                       OPTIONAL   -- Need M

}

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-STOP

-- ASN1STOP

SchedulingRequestResourceConfig field descriptions

periodicityAndOffset SR periodicity and offset in number of symbols or slots (see TS 38.213 [13], clause 9.2.4) The following periodicities may be configured depending on the chosen subcarrier spacing: SCS = 15 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl SCS = 30 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl, 160sl SCS = 60 kHz: 2sym, 7sym/6sym, 1sl, 2sl, 4sl, 8sl, 16sl, 20sl, 40sl, 80sl, 160sl, 320sl SCS = 120 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl SCS = 480 kHz: 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl, 1280sl, 2560sl SCS = 960 kHz: 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl, 1280sl, 2560sl, 5120sl sym6or7 corresponds to 6 symbols if extended cyclic prefix and a SCS of 60 kHz are configured, otherwise it corresponds to 7 symbols. For periodicities 2sym, 7sym and sl1 the UE assumes an offset of 0 slots. If periodicityAndOffset-r17 is present, any previously configured periodicityAndOffset (without suffix) is released, and vice versa. UE indicates support of the following SR periodicities in additionalSR-Periodicities (see TS 38.306, clause 4.2.6): SCS = 30 kHz: 5sl SCS = 120 kHz: 5sl, 10sl

phy-PriorityIndex Indicates whether this scheduling request resource is high or low priority in PHY prioritization/multiplexing handling (see TS 38.213 [13], clause 9.2.4). Value p0 indicates low priority and value p1 indicates high priority.

resource ID of the PUCCH resource in which the UE shall send the scheduling request. The actual PUCCH-Resource is configured in PUCCH-Config of the same UL BWP and serving cell as this SchedulingRequestResourceConfig. The network configures a PUCCH-Resource of PUCCH-format0 or PUCCH-format1 (other formats not supported) (see TS 38.213 [13], clause 9.2.4)

schedulingRequestID The ID of the SchedulingRequestConfig that uses this scheduling request resource.

#### –	SchedulingRequestResourceId

The IE SchedulingRequestResourceId is used to identify scheduling request resources on PUCCH.

SchedulingRequestResourceId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCEID-START

SchedulingRequestResourceId ::=     INTEGER (1..maxNrofSR-Resources)

-- TAG-SCHEDULINGREQUESTRESOURCEID-STOP

-- ASN1STOP

#### –	ScramblingId

The IE ScramblingID is used for scrambling channels and reference signals.

ScramblingId information element

-- ASN1START

-- TAG-SCRAMBLINGID-START

ScramblingId ::=                    INTEGER(0..1023)

-- TAG-SCRAMBLINGID-STOP

-- ASN1STOP

#### –	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2).

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

txDirectCurrentLocation Indicates the downlink Tx Direct Current location for the carrier. A value in the range 0..3299 indicates the subcarrier index within the carrier. The values in the value range 3301..4095 are reserved and ignored by the UE. If this field is absent for downlink within ServingCellConfigCommon and ServingCellConfigCommonSIB, the UE assumes the default value of 3300 (i.e. "Outside the carrier"). (see TS 38.211 [16], clause 4.4.2). Network does not configure this field via ServingCellConfig or for uplink carriers.

subcarrierSpacing Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

#### –	SDAP-Config

The IE SDAP-Config is used to set the configurable SDAP parameters for a data radio bearer. All configured instances of SDAP-Config with the same value of pdu-Session correspond to the same SDAP entity as specified in TS 37.324 [24].

SDAP-Config information element

-- ASN1START

-- TAG-SDAP-CONFIG-START

SDAP-Config ::=                     SEQUENCE {

pdu-Session                         PDU-SessionID,

sdap-HeaderDL                       ENUMERATED {present, absent},

sdap-HeaderUL                       ENUMERATED {present, absent},

defaultDRB                          BOOLEAN,

mappedQoS-FlowsToAdd                SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

mappedQoS-FlowsToRelease            SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

...

}

-- TAG-SDAP-CONFIG-STOP

-- ASN1STOP

SDAP-Config field descriptions

defaultDRB Indicates whether or not this is the default DRB for this PDU session. Among all configured instances of SDAP-Config with the same value of pdu-Session, this field shall be set to true in at most one instance of SDAP-Config and to false in all other instances.

mappedQoS-FlowsToAdd Indicates the list of QFIs of UL QoS flows of the PDU session to be additionally mapped to this DRB. A QFI value can be included at most once in all configured instances of SDAP-Config with the same value of pdu-Session. For QoS flow remapping, the QFI value of the remapped QoS flow is only included in mappedQoS-FlowsToAdd in sdap-Config corresponding to the new DRB and not included in mappedQoS-FlowsToRelease in sdap-Config corresponding to the old DRB.

mappedQoS-FlowsToRelease Indicates the list of QFIs of QoS flows of the PDU session to be released from existing QoS flow to DRB mapping of this DRB.

pdu-Session Identity of the PDU session whose QoS flows are mapped to the DRB.

sdap-HeaderUL Indicates whether or not a SDAP header is present for UL data on this DRB. The field cannot be changed after a DRB is established. The network sets this field to present if the field defaultDRB is set to true.

sdap-HeaderDL Indicates whether or not a SDAP header is present for DL data on this DRB. The field cannot be changed after a DRB is established.

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled SCell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent (regardless of their presence conditions). For a scheduled SpCell in the case of the cross carrier scheduling, if the search space is linked to another search space in the scheduling SCell, all the optional fields of this search space in the scheduled SpCell are absent (regardless of their presence conditions) except for nrofCandidates.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup4

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need S

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]],

[[

dci-FormatsMC                        ENUMERATED {formats0-3, formats1-3, formats0-3-And-1-3} OPTIONAL,   -- Need R

dci-FormatsNCR-r18                   ENUMERATED {formats2-8}                                OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpaceExt-r16 ::=                   SEQUENCE {

controlResourceSetId-r16                ControlResourceSetId-r16                                    OPTIONAL,   -- Cond SetupOnly2

searchSpaceType-r16                     SEQUENCE {

common-r16                              SEQUENCE {

dci-Format2-4-r16                       SEQUENCE {

nrofCandidates-CI-r16                   SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-5-r16                      SEQUENCE {

nrofCandidates-IAB-r16                  SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-6-r16                       SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL,    -- Cond Setup3

searchSpaceGroupIdList-r16                      SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)           OPTIONAL,    -- Need R

freqMonitorLocations-r16                        BIT STRING (SIZE (5))                               OPTIONAL     -- Need R

}

SearchSpaceExt-v1700 ::=            SEQUENCE {

monitoringSlotPeriodicityAndOffset-v1710 CHOICE {

sl32                                     INTEGER (0..31),

sl64                                     INTEGER (0..63),

sl128                                    INTEGER (0..127),

sl5120                                   INTEGER (0..5119),

sl10240                                  INTEGER (0..10239),

sl20480                                  INTEGER (0..20479)

}                                                                                                   OPTIONAL,   -- Cond Setup5

monitoringSlotsWithinSlotGroup-r17       CHOICE {

slotGroupLength4-r17                     BIT STRING (SIZE (4)),

slotGroupLength8-r17                     BIT STRING (SIZE (8))

}                                                                                                   OPTIONAL,   -- Need R

duration-r17                             INTEGER (4..20476)                                         OPTIONAL,   -- Need R

searchSpaceType-r17             SEQUENCE{

common-r17                      SEQUENCE {

dci-Format4-0-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-2-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-AndFormat4-2-r17  SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-7-r17               SEQUENCE {

nrofCandidates-PEI-r17          SEQUENCE {

aggregationLevel4-r17       ENUMERATED {n0, n1, n2, n3, n4}                         OPTIONAL,   -- Need R

aggregationLevel8-r17       ENUMERATED {n0, n1, n2}                                 OPTIONAL,   -- Need R

aggregationLevel16-r17      ENUMERATED {n0, n1}                                     OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL    -- Need R

}

}                                                                                                   OPTIONAL,   -- Need R

searchSpaceGroupIdList-r17          SEQUENCE (SIZE (1.. 3)) OF INTEGER (0.. maxNrofSearchSpaceGroups-1-r17)  OPTIONAL,  -- Cond DedicatedOnly

searchSpaceLinkingId-r17            INTEGER (0..maxNrofSearchSpacesLinks-1-r17)                     OPTIONAL    -- Cond DedicatedOnly

}

SearchSpaceExt-v1800 ::=            SEQUENCE {

searchSpaceType-r18                 SEQUENCE {

common-r18                          SEQUENCE {

dci-Format2-9-r18                   SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL    -- Need R

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling. The CORESETs with non-zero controlResourceSetId are configured in the same BWP as this SearchSpace except commonControlResourceSetExt which is configured by SIB20. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

dummy1, dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 according to TS 38.213 [13], clause 10.1.

dci-Format2-0 If configured, UE monitors the DCI format 2_0 according to TS 38.213 [13], clause 10.1, 11.1.1.

dci-Format2-1 If configured, UE monitors the DCI format 2_1 according to TS 38.213 [13], clause 10.1, 11.2.

dci-Format2-2 If configured, UE monitors the DCI format 2_2 according to TS 38.213 [13], clause 10.1, 11.3.

dci-Format2-3 If configured, UE monitors the DCI format 2_3 according to TS 38.213 [13], clause 10.1, 11.4

dci-Format2-4 If configured, UE monitors the DCI format 2_4 according to TS 38.213 [13], clause 11.2A.

dci-Format2-5 If configured, IAB-MT monitors the DCI format 2_5 according to TS 38.213 [13], clause 14.

dci-Format2-6 If configured, UE monitors the DCI format 2_6 according to TS 38.213 [13], clause 10.1, 10.3. DCI format 2_6 can only be configured on the SpCell.

dci-Format2-7 If configured, UE monitors the DCI format 2_7 according to TS 38.213 [13], clause 10.1, 10.4A.

dci-Format2-9 If configured, UE monitors the DCI format 2_9 according to TS 38.213 [13], clause 10.1, 11.5.

dci-Format4-0 If configured, the UE monitors the DCI format 4_0 with CRC scrambled by MCCH-RNTI/G-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-1-AndFormat4-2 If configured, the UE monitors the DCI format 4_1 and 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [11.1].

dci-Format4-1 If configured, the UE monitors the DCI format 4_1 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-2 If configured, the UE monitors the DCI format 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1). This field is not configured for operation with shared spectrum channel access in this release.

dci-FormatsMC Indicate whether the UE monitors in this USS for DCI format 0_3 or for format 1_3 or for formats 0_3 and 1_3. Separate search space sets for DCI format 0_3/1_3 and legacy DCI formats are independently configured. If this field is present, the field dci-Formats is ignored. NOTE: This parameter is used only for SearchSpace configured to the scheduling cell, while another SearchSpace configured to the reference scheduled cell (if any) configures only nrofCandidates (i.e., all other optional fields are absent) with same serachSpaceId with that for scheduling cell.

dci-Formats-MT Indicates whether the IAB-MT monitors the DCI formats 2-5 according to TS 38.213 [13], clause 14.

dci-FormatsNCR Indicates whether the NCR-MT monitors the DCI formats 2-8 according to TS 38.213 [13], clause 20.

dci-FormatsSL Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1 or for format 3-0 or for format 3-1 or for formats 3-0 and 3-1. If this field is present, the field dci-Formats is ignored and dci-FormatsSL is used.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset). For SCS 480 kHz and SCS 960 kHz, duration-r17 is used, and the configured duration is restricted to be an integer multiple of L slots and smaller than periodicity, where L is the configured length of the bitmap monitoringSlotsWithinSlotGroup-r17. If duration-r17 is absent, the UE assumes the duration in slots is equal to L. The maximum valid duration is periodicity-L. For IAB-MT, duration indicates number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_5. The IAB-MT ignores this field for DCI format 2_0 and DCI format 2_5. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

freqMonitorLocations Defines an association of the search space to multiple monitoring locations in the frequency domain and indicates whether the pattern configured in the associated CORESET is replicated to a specific RB set, see TS 38.213, clause 10.1. Each bit in the bitmap corresponds to one RB set, and the leftmost (most significant) bit corresponds to RB set 0 in the BWP. A bit set to 1 indicates that a frequency domain resource allocation replicated from the pattern configured in the associated CORESET is mapped to the RB set.

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. For SCS 15, 30, 60, and 120 kHz and if the UE is configured to monitor: - DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. - DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10). - DCI format 2_4, only the values 'sl1', 'sl2', 'sl4', 'sl5', 'sl8' and 'sl10' are applicable. For SCS 480 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40', 'sl64', and 'sl80' are applicable. - DCI format 2_1, only the values ′sl4′, ′sl8′, and ′sl16′ are applicable. - DCI format 2_4, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40' are applicable. For SCS 960 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl8', 'sl16', 'sl32', 'sl40', 'sl64', 'sl80', 'sl128', and 'sl160' are applicable. - DCI format 2_1, only the values ′sl8′, ′sl16′, and 'sl32' are applicable. - DCI format 2_4, only the values 'sl8', 'sl16', 'sl32', 'sl40', 'sl64', 'sl80' are applicable. For SCS 480 kHz and SCS 960 kHz, and the configured periodicity and offset are restricted to be an integer multiple of L slots, where L is the configured length of the bitmap provided by monitoringSlotsWithinSlotGroup-r17, i.e. for a given periodicity, the offset has a range of {0, L, 2*L, …, L*FLOOR(1/L*(periodicity-1))}. For IAB-MT, If the IAB-MT is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the IAB-MT is configured to monitor DCI format 2_0 or DCI format 2_5, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213, clause 10). If monitoringSlotPeriodicityAndOffset-v1710 is present, any previously configured monitoringSlotPeriodicityAndOffset is released, and if monitoringSlotPeriodicityAndOffset is present, any previously configured monitoringSlotPeriodicityAndOffset-v1710 is released.

monitoringSlotsWithinSlotGroup Indicates which slot(s) within a slot group are configured for multi-slot PDCCH monitoring. The first (leftmost, most significant) bit represents the first slot in the slot group, the second bit represents the second slot in the slot group, and so on. A bit set to '1' indicates that the corresponding slot is configured for multi-slot PDCCH monitoring (see TS 38.213 [13], clause 10). The number of slots for multi-slot PDCCH monitoring is configured according to clause 10 in TS 38.213 [13].

monitoringSymbolsWithinSlot The first symbol(s) for PDCCH monitoring in the slots configured for (multi-slot) PDCCH monitoring (see monitoringSlotPeriodicityAndOffset and duration). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the UE or IAB-MT. For DCI format 2_0, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10. For IAB-MT: For DCI format 2_0 or DCI format 2_5, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10.

nrofCandidates-CI The number of PDCCH candidates specifically for format 2-4 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 10.1).

nrofCandidates-PEI The number of PDCCH candidates specifically for format 2-7 for the configured aggregation level.

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1). For a search space configured with freqMonitorLocations-r16, only value ′n1′ is valid.

nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10).

searchSpaceGroupIdList-r16, searchSpaceGroupIdList-r17 List of search space group IDs which the search space is associated with. The network configures at most 2 search space groups per BWP where the group ID is either 0 or 1 if searchSpaceGroupIdList-r16 is included. The network configures at most 3 search space groups per BWP where the group ID is either 0, 1 or 2 if searchSpaceGroupIdList-r17 is included. And if searchSpaceGroupIdList-r17 is included, searchSpaceGroupIdList-r16 is ignored.

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell. In case of cross carrier scheduling, search spaces with the same searchSpaceId in scheduled cell and scheduling cell are linked to each other. The UE applies the search space for the scheduled cell only if the DL BWPs in which the linked search spaces are configured in scheduling cell and scheduled cell are both active. For an IAB-MT, the search space defines how/where to search for PDCCH candidates for an IAB-MT where each search space is associated with one ControlResearchSet and for a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpaceLinkingId This parameter is used to link two search spaces of same type in the same BWP. If two search spaces have the same SearchSpaceLinkingId UE assumes these search spaces are linked to PDCCH repetition REF. When PDCCH repetition is monitored in two linked search space (SS) sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets. The two linked SS sets have the same SS set type (USS/CSS). The two linked SS sets have the same DCI formats to monitor. For intra-slot PDCCH repetition: The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. For linking monitoring occasions across the two SS sets that exist in the same slot: The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set. The following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceMCCH, searchSpaceMTCH, peiSearchSpace, and sdt-SearchSpace. SS set configured by recoverySearchSpaceId cannot be linked to another SS set for PDCCH repetition. When a scheduled serving cell is configured to be cross-carrier scheduled by a scheduling serving cell, two PDCCH candidates (with the same AL and candidate index associated with the scheduled serving cell) are linked only if the corresponding two SS sets in the scheduling serving cell are linked and two SS sets in the scheduled serving cell with the same SS set IDs are also linked. This parameter is not applicable to search space configured with dci-FormatsSL for monitoring format 3-0 or format 3-1 or for monitoring formats 3-0 and format 3-1.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

| Conditional Presence | Explanation |
| --- | --- |
| DedicatedOnly | In PDCCH-Config, the field is optionally present, Need R. Otherwise it is absent, Need R. |
| Setup | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| Setup2 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt-r16 of the parent IE with the field searchSpaceType-r16 or searchSpaceType-r17 included. Otherwise it is optionally present, Need M. |
| Setup3 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt (without suffix) of the parent IE with the field searchSpaceType (without suffix) included. Otherwise it is optionally present, Need M. |
| Setup4 | This field is mandatory present upon creation of a new SearchSpace if monitoringSlotPeriodicityAndOffset-v1710 is not included. It is optionally present, Need M, otherwise. |
| Setup5 | This field is mandatory present upon creation of a new SearchSpace if monitoringSlotPeriodicityAndOffset (without suffix) is not included. It is optionally present, Need M, otherwise. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent, Need M, otherwise. |
| SetupOnly2 | In PDCCH-Config, the field is optionally present upon creation of a new SearchSpace and absent, Need M upon reconfiguration of an existing SearchSpace. In PDCCH-ConfigCommon, the field is absent. |

#### –	SearchSpaceId

The IE SearchSpaceId is used to identify Search Spaces. The ID space is used across the BWPs of a Serving Cell. The search space with the SearchSpaceId = 0 identifies the search space configured via PBCH (MIB) and in ServingCellConfigCommon (searchSpaceZero). The number of Search Spaces per BWP is limited to 10 including the common and UE specific Search Spaces.

SearchSpaceId information element

-- ASN1START

-- TAG-SEARCHSPACEID-START

SearchSpaceId ::=                   INTEGER (0..maxNrofSearchSpaces-1)

-- TAG-SEARCHSPACEID-STOP

-- ASN1STOP

#### –	SearchSpaceZero

The IE SearchSpaceZero is used to configure SearchSpace#0 of the initial BWP (see TS 38.213 [13], clause 13).

SearchSpaceZero information element

-- ASN1START

-- TAG-SEARCHSPACEZERO-START

SearchSpaceZero ::=                 INTEGER (0..15)

-- TAG-SEARCHSPACEZERO-STOP

-- ASN1STOP

#### –	SecurityAlgorithmConfig

The IE SecurityAlgorithmConfig is used to configure AS integrity protection algorithm and AS ciphering algorithm for SRBs and DRBs.

SecurityAlgorithmConfig information element

-- ASN1START

-- TAG-SECURITYALGORITHMCONFIG-START

SecurityAlgorithmConfig ::=         SEQUENCE {

cipheringAlgorithm                  CipheringAlgorithm,

integrityProtAlgorithm              IntegrityProtAlgorithm          OPTIONAL,   -- Need R

...

}

IntegrityProtAlgorithm ::=          ENUMERATED {

nia0, nia1, nia2, nia3, spare4, spare3,

spare2, spare1, ...}

CipheringAlgorithm ::=              ENUMERATED {

nea0, nea1, nea2, nea3, spare4, spare3,

spare2, spare1, ...}

-- TAG-SECURITYALGORITHMCONFIG-STOP

-- ASN1STOP

SecurityAlgorithmConfig field descriptions

cipheringAlgorithm Indicates the ciphering algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nea0-nea3 are identical to the LTE algorithms eea0-3. The algorithms configured for all bearers using master key shall be the same, and the algorithms configured for all bearers using secondary key, if any, shall be the same. If UE is connected to E-UTRA/EPC, this field indicates the ciphering algorithm to be used for RBs configured with NR PDCP, as specified in TS 33.501 [11].

integrityProtAlgorithm Indicates the integrity protection algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nia0-nia3 are identical to the E-UTRA algorithms eia0-3. The algorithms configured for all bearers using master key shall be the same and the algorithms configured for all bearers using secondary key, if any, shall be the same. The network does not configure nia0 except for unauthenticated emergency sessions for unauthenticated UEs in LSM (limited service mode). If UE is connected to E-UTRA/EPC, this field indicates the integrity protection algorithm to be used for SRBs configured with NR PDCP as specified in TS 33.501 [11], and DRBs configured with integrity protection as specified in TS 33.401 [30]. The network does not configure nia0 for SRB3.

#### –	SelectedPSCellForCHO-WithSCG

The IE SelectedPSCellForCHO-WithSCG is used to indicate the information of the selected target PSCell to target MN at execution of a conditional reconfiguration for CHO with candidate SCG(s).

SelectedPSCellForCHO-WithSCG information element

-- ASN1START

-- TAG-SELECTEDPSCELLFORCHO-WITHSCG-START

SelectedPSCellForCHO-WithSCG-r18 ::= SEQUENCE {

ssbFrequency-r18                     ARFCN-ValueNR,

physCellId-r18                       PhysCellId

}

-- TAG-SELECTEDPSCELLFORCHO-WITHSCG-STOP

-- ASN1STOP

#### –	SemiStaticChannelAccessConfig

The IE SemiStaticChannelAccessConfig is used to configure channel access parameters when the network is operating in semi-static channel access mode (see clause 4.3 in TS 37.213 [48].

SemiStaticChannelAccessConfig information element

-- ASN1START

-- TAG-SEMISTATICCHANNELACCESSCONFIG-START

SemiStaticChannelAccessConfig-r16 ::=    SEQUENCE {

period-r16                               ENUMERATED {ms1, ms2, ms2dot5, ms4, ms5, ms10}

}

-- TAG-SEMISTATICCHANNELACCESSCONFIG-STOP

-- ASN1STOP

SemiStaticChannelAccessConfig field descriptions

period Indicates the periodicity of the semi-static channel access mode (see TS 37.213 [48], clause 4.3). Value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, value ms2dot5 corresponds to 2.5 ms, and so on.

#### –	SemiStaticChannelAccessConfigUE

The IE SemiStaticChannelAccessConfigUE is used to configure channel access parameters for UE initiated semi-static channel access.

SemiStaticChannelAccessConfigUE information element

-- ASN1START

-- TAG-SEMISTATICCHANNELACCESSCONFIGUE-START

SemiStaticChannelAccessConfigUE-r17 ::=    SEQUENCE {

periodUE-r17                               ENUMERATED {ms1, ms2, ms2dot5, ms4, ms5, ms10, spare2, spare1},

offsetUE-r17                               INTEGER (0..559)

}

-- TAG-SEMISTATICCHANNELACCESSCONFIGUE-STOP

-- ASN1STOP

SemiStaticChannelAccessConfigUE field descriptions

periodUE Indicates the period of a channel occupancy that the UE can initiate as described in TS 37.213 [48], clause 4.3. Value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, value ms2dot5 corresponds to 2.5 ms, and so on.

offsetUE Indicates the number of symbols from the beginning of the even indexed radio frame to the start of the first period within that radio frame that the UE can initiate a channel occupancy (see TS 37.213 [48], clause 4.3), based on the smallest SCS among the configured SCSs in the serving cell. The offset duration indicated by this field is less than the period duration indicated by periodUE. The maximum value is 139, 279 and 559 for 15, 30 and 60 kHz subcarrier spacing, respectively.

#### –	Sensor-LocationInfo

The IE Sensor-LocationInfo is used by the UE to provide sensor information.

Sensor-LocationInfo information element

-- ASN1START

-- TAG-SENSORLOCATIONINFO-START

Sensor-LocationInfo-r16 ::= SEQUENCE {

sensor-MeasurementInformation-r16    OCTET STRING    OPTIONAL,

sensor-MotionInformation-r16         OCTET STRING    OPTIONAL,

...

}

-- TAG-SENSORLOCATIONINFO-STOP

-- ASN1STOP

Sensor-LocationInfo field descriptions

sensor-MeasurementInformation This field provides barometric pressure measurements as Sensor-MeasurementInformation defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

sensor-MotionInformation This field provides motion sensor measurements as Sensor-MotionInformation defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

#### –	ServingCellAndBWP-Id

The IE ServingCellAndBWP-Id is used to indicate a serving cell and an uplink or a downlink BWP.

ServingCellAndBWP-Id information element

-- ASN1START

-- TAG-SERVINGCELLANDBWP-ID-START

ServingCellAndBWP-Id-r17 ::= SEQUENCE {

servingcell-r17              ServCellIndex,

bwp-r17                      BWP-Id

}

-- TAG-SERVINGCELLANDBWP-ID-STOP

-- ASN1STOP

#### –	ServCellIndex

The IE ServCellIndex concerns a short identity, used to uniquely identify a serving cell (i.e. the PCell, the PSCell or an SCell) across the cell groups. Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

ServCellIndex information element

-- ASN1START

-- TAG-SERVCELLINDEX-START

ServCellIndex ::=                   INTEGER (0..maxNrofServingCells-1)

-- TAG-SERVCELLINDEX-STOP

-- ASN1STOP

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

[[

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                                OPTIONAL,   -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

supplementaryUplinkRelease-r16      ENUMERATED {true}                                                       OPTIONAL,   -- Need N

tdd-UL-DL-ConfigurationDedicated-IAB-MT-r16    TDD-UL-DL-ConfigDedicated-IAB-MT-r16                         OPTIONAL,   -- Cond TDD_IAB

dormantBWP-Config-r16               SetupRelease { DormantBWP-Config-r16 }                                  OPTIONAL,   -- Need M

ca-SlotOffset-r16                   CHOICE {

refSCS15kHz                         INTEGER (-2..2),

refSCS30KHz                         INTEGER (-5..5),

refSCS60KHz                         INTEGER (-10..10),

refSCS120KHz                        INTEGER (-20..20)

}                                                                                                           OPTIONAL,   -- Cond AsyncCA

dummy2                              SetupRelease { DummyJ }                                                 OPTIONAL,   -- Need M

intraCellGuardBandsDL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

intraCellGuardBandsUL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

csi-RS-ValidationWithDCI-r16        ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

lte-CRS-PatternList1-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList2-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

crs-RateMatch-PerCORESETPoolIndex-r16  ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

enableTwoDefaultTCI-States-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultTCI-StatePerCoresetPoolIndex-r16 ENUMERATED {enabled}                                          OPTIONAL,   -- Need R

enableBeamSwitchTiming-r16          ENUMERATED {true}                                                       OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType1-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType2-r16    ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

directionalCollisionHandling-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

channelAccessConfig-r16             SetupRelease { ChannelAccessConfig-r16 }                                OPTIONAL    -- Need M

]],

[[

nr-dl-PRS-PDC-Info-r17                 SetupRelease {NR-DL-PRS-PDC-Info-r17}                                OPTIONAL,   -- Need M

semiStaticChannelAccessConfigUE-r17    SetupRelease {SemiStaticChannelAccessConfigUE-r17}                   OPTIONAL,   -- Need M

mimoParam-r17                       SetupRelease {MIMOParam-r17}                                            OPTIONAL,   -- Need M

channelAccessMode2-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

timeDomainHARQ-BundlingType1-r17    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

nrofHARQ-BundlingGroups-r17         ENUMERATED {n1, n2, n4}                                                 OPTIONAL,   -- Need R

fdmed-ReceptionMulticast-r17        ENUMERATED {true}                                                       OPTIONAL,   -- Need R

moreThanOneNackOnlyMode-r17         ENUMERATED {mode2}                                                      OPTIONAL,   -- Need S

tci-ActivatedConfig-r17             TCI-ActivatedConfig-r17                                                 OPTIONAL,   -- Cond TCI_ActivatedConfig

directionalCollisionHandling-DC-r17 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

lte-NeighCellsCRS-AssistInfoList-r17  SetupRelease { LTE-NeighCellsCRS-AssistInfoList-r17 }                 OPTIONAL    -- Need M

]],

[[

lte-NeighCellsCRS-Assumptions-r17   ENUMERATED {false}                                                      OPTIONAL    -- Need R

]],

[[

crossCarrierSchedulingConfigRelease-r17 ENUMERATED {true}                                                   OPTIONAL    -- Need N

]],

[[

multiPDSCH-PerSlotType1-CB-r17      ENUMERATED {enabled, disabled}                                          OPTIONAL    -- Need R

]],

[[

lte-CRS-PatternList3-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList4-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

pdcch-CandidateReceptionWith-CRS-Overlap-r18  ENUMERATED {enabled}                                          OPTIONAL,   -- Need R

cjt-Scheme-PDSCH-r18                ENUMERATED {cjtSchemeA, cjtSchemeB}                                     OPTIONAL,   -- Need R

tag2-r18                            SEQUENCE {

tag2-Id-r18                         TAG-Id,

tag2-flag-r18                       BOOLEAN,

n-TimingAdvanceOffset2-r18          ENUMERATED { n0, n25600, n39936, spare1 }                           OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

cellDTXDRX-Config-r18               SetupRelease { CellDTXDRX-Config-r18 }                                  OPTIONAL,   -- Need M

positionInDCI-cellDTRX-r18          INTEGER (0..maxDCI-2-9-Size-1-r18)                                      OPTIONAL,   -- Need R

cellDTXDRX-L1activation-r18         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

mc-DCI-SetOfCellsToAddModList-r18   SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18    OPTIONAL,   -- Need N

mc-DCI-SetOfCellsToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF SetOfCellsId-r18         OPTIONAL    -- Need N

]]

}

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                                     OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                              OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink                          OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                                  OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                                   OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                                 OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

enablePL-RS-UpdateForPUSCH-SRS-r16  ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUSCH0-0-r16 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUCCH-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForSRS-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

uplinkTxSwitching-r16               SetupRelease { UplinkTxSwitching-r16 }                                  OPTIONAL,   -- Need M

mpr-PowerBoost-FR2-r16              ENUMERATED {true}                                                       OPTIONAL    -- Need R

]],

[[

srs-PosTx-Hopping-r18               SetupRelease { SRS-PosTx-Hopping-r18 }                                  OPTIONAL,   -- Need M

enablePL-RS-UpdateForType1CG-PUSCH-r18  ENUMERATED {enabled}                                                OPTIONAL,   -- Need R

powerBoostPi2BPSK-r18               BOOLEAN                                                                 OPTIONAL,   -- Need R

powerBoostQPSK-r18                  BOOLEAN                                                                 OPTIONAL    -- Need R

]]

}

DummyJ ::=                          SEQUENCE {

maxEnergyDetectionThreshold-r16         INTEGER(-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-20..-13),

ul-toDL-COT-SharingED-Threshold-r16     INTEGER (-85..-52)                                                  OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16         ENUMERATED {true}                                                   OPTIONAL    -- Need R

}

ChannelAccessConfig-r16 ::=         SEQUENCE {

energyDetectionConfig-r16           CHOICE {

maxEnergyDetectionThreshold-r16         INTEGER (-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-13..20)

}                                                                                                           OPTIONAL,   -- Need R

ul-toDL-COT-SharingED-Threshold-r16         INTEGER (-85..-52)                                              OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16             ENUMERATED {true}                                               OPTIONAL    -- Need R

}

IntraCellGuardBandsPerSCS-r16 ::=      SEQUENCE {

guardBandSCS-r16                       SubcarrierSpacing,

intraCellGuardBands-r16                SEQUENCE (SIZE (1..4)) OF GuardBand-r16

}

GuardBand-r16 ::=                      SEQUENCE {

startCRB-r16                          INTEGER (0..274),

nrofCRBs-r16                          INTEGER (0..15)

}

DormancyGroupID-r16 ::=         INTEGER (0..4)

DormantBWP-Config-r16::=               SEQUENCE {

dormantBWP-Id-r16                      BWP-Id                                                           OPTIONAL,   -- Need M

withinActiveTimeConfig-r16             SetupRelease { WithinActiveTimeConfig-r16 }                      OPTIONAL,   -- Need M

outsideActiveTimeConfig-r16            SetupRelease { OutsideActiveTimeConfig-r16 }                     OPTIONAL    -- Need M

}

WithinActiveTimeConfig-r16 ::=         SEQUENCE {

firstWithinActiveTimeBWP-Id-r16         BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupWithinActiveTime-r16       DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

OutsideActiveTimeConfig-r16 ::=        SEQUENCE {

firstOutsideActiveTimeBWP-Id-r16        BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupOutsideActiveTime-r16      DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

UplinkTxSwitching-r16 ::=              SEQUENCE {

uplinkTxSwitchingPeriodLocation-r16    BOOLEAN,

uplinkTxSwitchingCarrier-r16           ENUMERATED {carrier1, carrier2}

}

MIMOParam-r17 ::= SEQUENCE {

additionalPCI-ToAddModList-r17     SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF SSB-MTC-AdditionalPCI-r17  OPTIONAL,   -- Need N

additionalPCI-ToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF AdditionalPCIIndex-r17     OPTIONAL,   -- Need N

unifiedTCI-StateType-r17           ENUMERATED {separate, joint}                                         OPTIONAL,   -- Need R

uplink-PowerControlToAddModList-r17  SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControl-r17      OPTIONAL,   -- Need N

uplink-PowerControlToReleaseList-r17 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlId-r17    OPTIONAL,   -- Need N

sfnSchemePDCCH-r17                 ENUMERATED {sfnSchemeA,sfnSchemeB}                                   OPTIONAL,   -- Need R

sfnSchemePDSCH-r17                 ENUMERATED {sfnSchemeA,sfnSchemeB}                                   OPTIONAL    -- Need R

}

MC-DCI-SetOfCells-r18 ::=          SEQUENCE {

setOfCellsId-r18                  SetOfCellsId-r18,

nCI-Value-r18                     INTEGER (0..7),

scheduledCellListDCI-1-3-r18      SEQUENCE (SIZE (2..maxNrofCellsInSet-r18)) OF ServCellIndex          OPTIONAL,   -- Need R

scheduledCellListDCI-0-3-r18      SEQUENCE (SIZE (2..maxNrofCellsInSet-r18)) OF ServCellIndex          OPTIONAL,   -- Need R

scheduledCellComboListDCI-1-3-r18 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 OPTIONAL,   -- Need R

scheduledCellComboListDCI-0-3-r18 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 OPTIONAL,   -- Need R

antennaPortsDCI1-3-r18            ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI1-3

antennaPortsDCI0-3-r18            ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

tpmi-DCI0-3-r18                   ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

sri-DCI0-3-r18                    ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

priorityIndicatorDCI-1-3-r18      ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

priorityIndicatorDCI-0-3-r18      ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

dormancyDCI-1-3-r18               ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

dormancyDCI-0-3-r18               ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pdcchMonAdaptDCI-1-3-r18          ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pdcchMonAdaptDCI-0-3-r18          ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

minimumSchedulingOffsetK0DCI-1-3-r18        ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

minimumSchedulingOffsetK0DCI-0-3-r18        ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3-r18   ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

pdsch-HARQ-ACK-enhType3DCI-1-3-r18          ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3-r18  ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

pdsch-HARQ-ACK-retxDCI-1-3-r18    ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pucch-sSCellDynDCI-1-3-r18        ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

tdra-FieldIndexListDCI-1-3-r18    SEQUENCE (SIZE (1..32)) OF TDRA-FieldIndexDCI-1-3-r18                OPTIONAL,   -- Need R

tdra-FieldIndexListDCI-0-3-r18    SEQUENCE (SIZE (1..64)) OF TDRA-FieldIndexDCI-0-3-r18                OPTIONAL,   -- Need R

rateMatchListDCI-1-3-r18          SEQUENCE (SIZE (1..16)) OF RateMatchDCI-1-3-r18                      OPTIONAL,   -- Need R

zp-CSI-RSListDCI-1-3-r18          SEQUENCE (SIZE (1..8)) OF ZP-CSI-DCI-1-3-r18                         OPTIONAL,   -- Need R

tci-ListDCI-1-3-r18               SEQUENCE (SIZE (1..16)) OF TCI-DCI-1-3-r18                           OPTIONAL,   -- Need R

srs-RequestListDCI-1-3-r18        SEQUENCE (SIZE (1..16)) OF SRS-RequestCombo-r18                      OPTIONAL,   -- Need R

srs-OffsetListDCI-1-3-r18         SEQUENCE (SIZE (1..8)) OF SRS-OffsetCombo-r18                        OPTIONAL,   -- Need R

srs-RequestListDCI-0-3-r18        SEQUENCE (SIZE (1..16)) OF SRS-RequestCombo-r18                      OPTIONAL,   -- Need R

srs-OffsetListDCI-0-3-r18         SEQUENCE (SIZE (1..8)) OF SRS-OffsetCombo-r18                        OPTIONAL    -- Need R

}

SetOfCellsId-r18 ::=                   INTEGER (0..maxNrofSetsOfCells-1-r18)

ScheduledCellCombo-r18 ::=             SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER (0..maxNrofCellsInSet-1-r18)

RateMatchDCI-1-3-r18 ::=               SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (1..2))

ZP-CSI-DCI-1-3-r18 ::=                 SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (1..2))

TCI-DCI-1-3-r18 ::=                    SEQUENCE (SIZE (2.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (3))

SRS-RequestCombo-r18 ::=               SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (2..3))

SRS-OffsetCombo-r18 ::=                SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF INTEGER (0..3)

TDRA-FieldIndexDCI-1-3-r18 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofDL-Allocations-1-r18)

TDRA-FieldIndexDCI-0-3-r18 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofUL-Allocations-1-r18)

-- TAG-SERVINGCELLCONFIG-STOP

-- ASN1STOP

ChannelAccessConfig field descriptions

absenceOfAnyOtherTechnology Presence of this field indicates absence on a long term basis (e.g. by level of regulation) of any other technology sharing the carrier; absence of this field indicates the potential presence of any other technology sharing the carrier, as specified in TS 37.213 [48] clauses 4.2.1 and 4.2.3.

energyDetectionConfig Indicates whether to use the maxEnergyDetectionThreshold or the energyDetectionThresholdOffset (see TS 37.213 [48], clause 4.2.3).

energyDetectionThresholdOffset Indicates the offset to the default maximum energy detection threshold value. Unit in dB. Value -13 corresponds to -13dB, value -12 corresponds to -12dB, and so on (i.e. in steps of 1dB) as specified in TS 37.213 [48], clause 4.2.3.

maxEnergyDetectionThreshold Indicates the absolute maximum energy detection threshold value. Unit in dBm. Value -85 corresponds to -85 dBm, value -84 corresponds to -84 dBm, and so on (i.e. in steps of 1dBm) as specified in TS 37.213 [48], clause 4.2.3.

n-TimingAdvanceOffset2 The N_TA-Offset2 to be applied for all uplink transmissions on this serving cell associated to tag2. If absent, the N_TA-Offset is applied for all uplink transmissions on this serving cell associated to tag2.

ul-toDL-COT-SharingED-Threshold Maximum energy detection threshold that the UE should use to share channel occupancy with gNB for DL transmission as specified in TS 37.213 [48], clause 4.1.3 for downlink channel access and clause 4.2.3 for uplink channel access. This field is not applicable in semi-static channel access mode.

ServingCellConfig field descriptions

additionalPCI-ToAddModList List of information for the additional SSB with different PCI than the serving cell PCI. The additional SSBs with different PCIs are not used for serving cell quality derivation.

bwp-InactivityTimer The duration in ms after which the UE falls back to the default Bandwidth Part (see TS 38.321 [3], clause 5.15). When the network releases the timer configuration, the UE stops the timer without switching to the default BWP.

ca-SlotOffset Slot offset between the primary cell (PCell/PSCell) and the SCell in unaligned frame boundary with slot alignment and partial SFN alignment inter-band CA. Based on this field, the UE determines the time offset of the SCell as specified in clause 4.5 of TS 38.211 [16]. The granularity of this field is determined by the reference SCS for the slot offset (i.e. the maximum of PCell/PSCell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfigCommon or ServingCellConfigCommonSIB and this serving cell's lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfigCommon or ServingCellConfigCommonSIB). The Network configures at most single non-zero offset duration in ms (independent on SCS) among CCs in the unaligned CA configuration. If the field is absent, the UE applies the value of 0. The slot offset value can only be changed with SCell release and add.

cbg-TxDiffTBsProcessingType1, cbg-TxDiffTBsProcessingType2 Indicates whether processing types 1 and 2 based CBG based operation is enabled according to Rel-16 UE capabilities.

cellDTXDRX-Config Used to configure cell DTX/DRX for the serving cell, as specified in TS 38.321 [3]. A maximum of two cell DTX/DRX patterns can be configured per MAC entity for different serving cells. The two configured patterns are aligned, that the start and slot offset are common and the periodicity of one pattern is an integer multiple of the other. Cell DTX is configured only when connected mode DRX is configured.

cellDTXDRX-L1activation Indicates whether this serving cell has enabled L1 signaling based on DCI 2_9 for dynamic activation/deactivation of cell DTX/DRX configuration.

cjt-Scheme-PDSCH This field is used to configure CJT Tx scheme cjtSchemeA or cjtSchemeB for PDSCH reception, see TS 38.214 [19] clause 5.1.5.

channelAccessConfig List of parameters used for access procedures of operation with shared spectrum channel access (see TS 37.213 [48).

channelAccessMode2 If present, this field indicates that the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE does not apply these channel access procedures. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations. Overwrites the corresponding field in ServingCellConfigCommon or ServingCellConfigCommonSIB for this serving cell.

crossCarrierSchedulingConfig Indicates whether this serving cell is cross-carrier scheduled by another serving cell or whether it cross-carrier schedules another serving cell. If the field other is configured for an SpCell (i.e., the SpCell is cross-carrier scheduled by another serving cell), the SpCell can be additionally scheduled by the PDCCH on the SpCell.

crossCarrierSchedulingConfigRelease If this field is included, the UE shall release the cross carrier scheduling configuration configured by crossCarrierSchedulingConfig. The network may only include either crossCarrierSchedulingConfigRelease or crossCarrierSchedulingConfig at a time.

crs-RateMatch-PerCORESETPoolIndex Indicates how UE performs rate matching when both lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 are configured or when both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are configured as specified in TS 38.214 [19], clause 5.1.4.2.

csi-RS-ValidationWithDCI Indicates how the UE performs periodic and semi-persistent CSI-RS reception in a slot. The presence of this field indicates that the UE uses DCI detection to validate whether to receive CSI-RS (see TS 38.213 [13], clause 11.1).

defaultDownlinkBWP-Id The initial bandwidth part is referred to by BWP-Id = 0. ID of the downlink bandwidth part to be used upon expiry of the BWP inactivity timer. This field is UE specific. When the field is absent the UE uses the initial BWP as default BWP. (see TS 38.213 [13], clause 12 and TS 38.321 [3], clause 5.15).

directionalCollisionHandling Indicates that this serving cell is using directional collision handling between a reference and other cell(s) for half-duplex operation in TDD CA with same SCS as specified in TS 38.213 [13], clause 11.1. The half-duplex operation only applies within the same frequency range and cell group. The network only configures this field for TDD serving cells that are using the same SCS.

directionalCollisionHandling-DC For the IAB-MT, it indicates that this serving cell is using directional collision handling between a reference and other cell(s) for half-duplex operation in TDD NR-DC with same SCS within same cell group or cross different cell groups.

dormantBWP-Config The dormant BWP configuration for an SCell. This field can be configured only for a (non-PUCCH) SCell.

downlinkBWP-ToAddModList List of additional downlink bandwidth parts to be added or modified. (see TS 38.213 [13], clause 12).

downlinkBWP-ToReleaseList List of additional downlink bandwidth parts to be released. (see TS 38.213 [13], clause 12).

downlinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in DownlinkConfigCommon / DownlinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]. If the UE is an (e)RedCap UE and needs to autonomously switch to its initial downlink bandwidth part to perform a random access procedure but its current UE specific channel bandwidth does not cover the initial downlink bandwidth part, the UE autonomously changes its UE specific channel bandwidth to cover the initial downlink bandwidth part. In that case, after completion of the random access procedure, the network ensures that the UE specific channel bandwidth fully covers the UE's active downlink bandwidth part in subsequent bandwidth part switch operations.

dummy1, dummy 2 This field is not used in the specification. If received it shall be ignored by the UE.

enableBeamSwitchTiming Indicates the aperiodic CSI-RS triggering with beam switching triggering behaviour as defined in clause 5.2.1.5.1 of TS 38.214 [19].

enableDefaultTCI-StatePerCoresetPoolIndex Presence of this field indicates the UE shall follow the release 16 behavior of default TCI state per CORESETPoolindex when the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet is enabled.

enableTwoDefaultTCI-States Presence of this field indicates the UE shall follow the release 16 behavior of two default TCI states for PDSCH when at least one TCI codepoint is mapped to two TCI states is enabled

fdmed-ReceptionMulticast Indicates the Type-1 HARQ codebook generation as specified in TS 38.213 [13], clause 9.1.2.1.

firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated or to be used for RLM, BFD and measurements if included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated, upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If the field is absent for the PSCell at SCG deactivation, the UE considers the previously activated DL BWP as the BWP to be used for RLM, BFD and measurements. If the field is absent for the PSCell at SCG activation, the DL BWP to be activated is the DL BWP previously to be used for RLM, BFD and measurements. If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0. Upon reconfiguration with reconfigurationWithSync, the network sets the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value.

initialDownlinkBWP The dedicated (UE-specific) configuration for the initial downlink bandwidth-part (i.e., DL BWP#0). If any of the optional IEs are configured within this IE, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

intraCellGuardBandsDL-List, intraCellGuardBandsUL-List List of intra-cell guard bands in a serving cell for operation with shared spectrum channel access in FR1. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required.

lte-CRS-PatternList1 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround simultaneously.

lte-CRS-PatternList2 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH scheduled with a DCI detected on a CORESET with CORESETPoolIndex configured with 1. This list is configured only if CORESETPoolIndex configured with 1. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList1, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList1, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1.

lte-CRS-PatternList3 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously.

lte-CRS-PatternList4 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

lte-NeighCellsCRS-AssistInfoList A list of LTE neighbour cells configuration information which is used to assist the UE to perform CRS interference mitigation (CRS-IM) in scenarios with overlapping spectrum for LTE and NR (see TS 38.101-4 [59]). If the field is included, it replaces any previous list, i.e. all the entries of the list are replaced and each of the LTE-NeighCellsCRS-AssistInfo entries is considered to be newly created and the conditions and Need codes for setup of the entry apply.

lte-NeighCellsCRS-Assumptions If the field is not configured, the following default network configuration assumptions are valid for all LTE neighbour cells for the purpose of CRS interference mitigation (CRS-IM) in scenarios with overlapping spectrum for LTE and NR (see TS 38.101-4 [59]). - The CRS port number is the same as the one indicated in RateMatchPatternLTE-CRS if configured for the serving cell. - The CRS port number is 4 if RateMatchPatternLTE-CRS is not configured for the serving cell. - The channel bandwidth and centre frequency are the same as the ones indicated in RateMatchPatternLTE-CRS if configured for the serving cell. - The MBSFN configuration is the same as the one indicated in RateMatchPatternLTE-CRS if configured for the serving cell. If RateMatchPatternLTE-CRS is not configured for the serving cell, MBSFN subframe is not configured. - Network-based CRS interference mitigation (i.e., CRS muting), as in crs-IntfMitigConfig specified in TS 36.331 [10], is not enabled. If the field is configured (i.e. false) and LTE-NeighCellsCRS-AssistInfoList is configured, the configuration provided in LTE-NeighCellsCRS-AssistInfoList overrides the default network configuration assumptions. If the field is configured (i.e. false) and LTE-NeighCellsCRS-AssistInfoList is not configured, it is up to the UE implementation whether to apply CRS-IM operation.

mc-DCI-SetOfCellsToAddModList List of up to N (N<=4) configurations of set(s) of cells for multi-cell PDSCH/PUSCH scheduling from the serving cell, where N is reported as UE capability and up to 4 sets of cells can be configured per PUCCH group. When this field is configured to a SCell, PCell cannot be included in either ScheduledCellListDCI-1-3 or ScheduledCellListDCI-0-3.

multiPDSCH-PerSlotType1-CB Configures the UE behaviour for Type1 codebook HARQ ACK generation regarding the number of PDSCHs per slot on a serving cell as specified in TS 38.213 [13], clause 9.1.2.1. When this parameter is configured and set to disabled for a serving cell, the network does not schedule UE with more than one PDSCH in a slot on the serving cell if HARQ-ACKs of any two PDSCHs in the slot on the serving cell are supposed to be reported on one PUCCH resource in the same PUCCH slot. If two coresetPoolIndex values are configured, the number of received PDSCHs is per coresetPoolIndex value per slot for a serving cell. If the UE generates two HARQ-ACK codebooks for two priorities, the number of received PDSCHs is per priority per slot for a serving cell. If fdmed-ReceptionMulticast is configured, the number of received PDSCHs is per traffic type (unicast / multicast) per slot for a serving cell.

nr-dl-PRS-PDC-Info Configures the DL PRS for propagation delay compensation. When configured, the UE measures the UE Rx-Tx time difference based on the reference signals configured in this field.

nrofHARQ-BundlingGroups Indicates the number of HARQ bundling groups for type2 HARQ-ACK codebook.

pathlossReferenceLinking Indicates whether UE shall apply as pathloss reference either the downlink of SpCell (PCell for MCG or PSCell for SCG) or of SCell that corresponds with this uplink (see TS 38.213 [13], clause 7).

pdcch-CandidateReceptionWith-CRS-Overlap Presence of this field indicates the UE shall monitor PDCCH candidates that overlap with LTE CRS RE(s).

pdsch-ServingCellConfig PDSCH related parameters that are not BWP-specific.

positionInDCI-cellDTRX The starting bit position of an information block of DCI format 2_9 for this serving cell (see TS 38.212 [17], clause 7.3.1.3.10).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology. See TS 38.214 [19], clause 5.1.4.1. If a RateMatchPattern with the same RateMatchPatternId is configured in both ServingCellConfig/ServingCellConfigCommon and in SIB20/MCCH, the entire RateMatchPattern configuration shall be the same, including the set of RBs/REs indicated by the patterns for the rate matching around, and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

sCellDeactivationTimer SCell deactivation timer in TS 38.321 [3]. If the field is absent, the UE applies the value infinity.

sfnSchemePDCCH This parameter is used to configure single frequency network scheme for PDCCH: sfnSchemeA or sfnSchemeB as specified (see TS 38.214 [19], clause 5.1). If network includes both sfnSchemePDCCH and sfnSchemePDSCH, same value shall be configured.

sfnSchemePDSCH This parameter is used to configure single frequency network scheme for PDSCH: sfnSchemeA or sfnSchemeB as specified (see TS 38.214 [19], clause 5.1). If network includes both sfnSchemePDCCH and sfnSchemePDSCH, same value shall be configured. The network does not configure this parameter and repetitionSchemeConfig in PDSCH-Config simultaneously in the same serving cell.

semiStaticChannelAccessConfigUE When this field is configured and when channelAccessMode-r16 (see IE ServingCellConfigCommon and IE ServingCellConfigCommonSIB) is configured to semiStatic, the UE operates in semi-static channel access mode and can initiate a channel occupancy periodically (see TS 37.213 [48], Clause 4.3). The period can be configured independently from period configured in SemiStaticChannelAccessConfig-r16 if the UE indicates the corresponding capability. Otherwise, the periodicity configured by periodUE-r17 is an integer multiple of or an integer factor of the periodicity indicated by period in SemiStaticChannelAccessConfig-r16.

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon/ServingCellConfigCommonSIB of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList.

supplementaryUplink Network may configure this field only when supplementaryUplinkConfig is configured in ServingCellConfigCommon or supplementaryUplink is configured in ServingCellConfigCommonSIB.

supplementaryUplinkRelease If this field is included, the UE shall release the uplink configuration configured by supplementaryUplink. The network only includes either supplementaryUplinkRelease or supplementaryUplink at a time.

tag-Id, tag2-Id Timing Advance Group ID, as specified in TS 38.321 [3], which this cell or set of TCI-States of this cell are associated with. The tag2-Id is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex

tag2-flag If this field is set to true, the tag2-Id is associated to value 0 and tag-Id is associated to value 1 of field TI bit in RAR , fallbackRAR and in the absolute TAC MAC CE, see TS 38.321 [3]. Otherwise, the tag2-Id is associated to value 1 and tag-Id is associated to value 0 of field TI bit in RAR, fallbackRAR and in the absolute TAC MAC CE, see TS 38.321 [3].

tci-ActivatedConfig If configured for an SCell, or if configured for the PSCell when the SCG is being activated upon the reception of the containing message, the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell. If configured for the PSCell when the SCG is indicated as deactivated in the containing message: - the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD. When this field is absent for the PSCell and the SCG is being deactivated: - the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

tdd-UL-DL-ConfigurationDedicated-IAB-MT Resource configuration per IAB-MT D/U/F overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL ConfigurationCommon.

unifiedTCI-StateType Indicates the unified TCI state type the UE is configured for this serving cell. The value separate means this serving cell is configured with dl-OrJointTCI-StateList for DL TCI state and ul-TCI-ToAddModList for UL TCI state. The value joint means this serving cell is configured with dl-OrJointTCI-StateList for joint TCI state for UL and DL operation.

uplinkConfig Network may configure this field only when uplinkConfigCommon is configured in ServingCellConfigCommon or ServingCellConfigCommonSIB. Addition or release of this field can only be done upon SCell addition or release (respectively).

uplink-PowerControlToAddModList Configures UL power control parameters for PUSCH, PUCCH and SRS when field unifiedTCI-StateType is configured for this serving cell.

UplinkConfig field descriptions

carrierSwitching Includes parameters for configuration of carrier based SRS switching (see TS 38.214 [19], clause 6.2.1.3.

enableDefaultBeamPL-ForPUSCH0-0, enableDefaultBeamPL-ForPUCCH, enableDefaultBeamPL-ForSRS When the parameter is present, UE derives the spatial relation and the corresponding pathloss reference Rs as specified in 38.213, clauses 7.1.1, 7.2.1, 7.3.1 and 9.2.2. The network only configures these parameters for FR2.

enablePL-RS-UpdateForPUSCH-SRS When this parameter is present, the Rel-16 feature of MAC CE based pathloss RS updates for PUSCH/SRS is enabled. Network only configures this parameter when the UE is configured with sri-PUSCH-PowerControl. If this field is not configured, network configures at most 4 pathloss RS resources for PUSCH/PUCCH/SRS transmissions per BWP, not including pathloss RS resources for SRS transmissions for positioning. (See TS 38.213 [13], clause 7).

enablePL-RS-UpdateForType1CG-PUSCH When this parameter is present, the Rel-18 feature of MAC CE based pathloss RS updates for Type 1 CG-PUSCH is enabled. The network only configures this parameter, when the parameter enablePL-RS-UpdateForPUSCH-SRS is configured. (See TS 38.213 [13], clause 7).

firstActiveUplinkBWP-Id If configured for an SpCell, this field contains the ID of the UL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the uplink bandwidth part to be used upon activation of an SCell. The initial bandwidth part is referred to by BandiwdthPartId = 0.

initialUplinkBWP The dedicated (UE-specific) configuration for the initial uplink bandwidth-part (i.e. UL BWP#0). If any of the optional IEs are configured within this IE as part of the IE uplinkConfig, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

moreThanOneNackOnlyMode Indicates the mode of NACK-only feedback in the PUCCH transmission, as specified in TS 38.213 [13], clause 18. If multicast CFR is not configured, this field is not included. Otherwise, if the field is absent, UE uses mode 1 for multicast CFR.

mpr-PowerBoost-FR2 Indicates whether UE is allowed to boost uplink transmission power by suspending in-band emission (IBE) requirements as specified in TS 38.101-2 [39]. Network only configures this field for FR2 serving cells.

powerBoostPi2BPSK If this field is set to true, the UE determines the maximum output power for PUCCH/PUSCH transmissions that use pi/2 BPSK modulation according to TS 38.101-1 [15], clause 6.2.4. The network ensures that powerBoostPi2BPSK and powerBoostPi2BPSK-r18 are not configured at the same time for a UE.

powerBoostQPSK If this field is set to true, the UE determines the maximum output power for PUSCH transmissions that use QPSK modulation according to TS 38.101-1 [15], clause 6.2.4.

pusch-ServingCellConfig PUSCH related parameters that are not BWP-specific.

srs-PosTx-Hopping Contains configuration related to the SRS for Positioning with frequency hopping for RRC_CONNETCED state.

uplinkBWP-ToAddModList The additional bandwidth parts for uplink to be added or modified. In case of TDD uplink- and downlink BWP with the same bandwidthPartId are considered as a BWP pair and must have the same center frequency.

uplinkBWP-ToReleaseList The additional bandwidth parts for uplink to be released.

uplinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in UplinkConfigCommon / UplinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]. If the UE is an (e)RedCap UE and needs to autonomously switch to its initial uplink bandwidth part to perform a random access procedure but its current UE specific channel bandwidth does not cover the initial uplink bandwidth part, the UE autonomously changes its UE specific channel bandwidth to cover the initial uplink bandwidth part. In that case, after completion of the random access procedure, the network ensures that the UE specific channel bandwidth fully covers the UE's active uplink bandwidth part in subsequent bandwidth part switch operations.

uplinkTxSwitchingPeriodLocation Indicates whether the location of UL Tx switching period is configured in this uplink carrier in case of inter-band UL CA, SUL, or (NG)EN-DC, as specified in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of (NG)EN-DC, network always configures this field to TRUE for NR carrier (i.e. with (NG)EN-DC, the UL switching period always occurs on the NR carrier). In case of inter-band UL CA or SUL, for dynamic uplink Tx switching between 2 bands with 2 uplink carriers or 3 uplink carriers as defined in TS 38.101-1 [15], network configures this field to TRUE for the uplink carrier(s) on one band and configures this field to FALSE for the uplink carrier(s) on the other band. This field is set to the same value for the carriers on the same band.

uplinkTxSwitchingCarrier Indicates that the configured carrier is carrier1 or carrier2 for dynamic uplink Tx switching, as defined in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of (NG)EN-DC, network always configures the NR carrier as carrier 2. In case of inter-band UL CA or SUL, for dynamic uplink Tx switching between 2 bands with 2 uplink carriers or 3 uplink carriers as defined in TS 38.101-1 [15], network configures the uplink carrier(s) on one band as carrier1 and the uplink carrier(s) on the other band as carrier2. This field is set to the same value for the carriers on the same band.

DormantBWP-Config field descriptions

dormancyGroupWithinActiveTime This field contains the ID of an SCell group for Dormancy within active time, to which this SCell belongs. The use of the Dormancy within active time for SCell groups is specified in TS 38.213 [13].

dormancyGroupOutsideActiveTime This field contains the ID of an SCell group for Dormancy outside active time, to which this SCell belongs. The use of the Dormancy outside active time for SCell groups is specified in TS 38.213 [13].

dormantBWP-Id This field contains the ID of the downlink bandwidth part to be used as dormant BWP. If this field is configured, its value is different from defaultDownlinkBWP-Id, and at least one of the withinActiveTimeConfig and outsideActiveTimeConfig should be configured.

firstOutsideActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy outside active time.

firstWithinActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy within active time.

outsideActiveTimeConfig This field contains the configuration to be used for SCell dormancy outside active time, as specified in TS 38.213 [13]. The field can only be configured when the cell group the SCell belongs to is configured with dcp-Config.

withinActiveTimeConfig This field contains the configuration to be used for SCell dormancy within active time, as specified in TS 38.213 [13].

GuardBand field descriptions

startCRB Indicates the starting RB of the guard band.

nrofCRB Indicates the length of the guard band in RBs. When set to 0, zero-size guard band is used.

MC-DCI-SetOfCells field descriptions

antennaPortsDCI1-3, antennaPortsDCI0-3 Configure the indication type for antenna port(s) field in DCI format 1_3 and DCI format 0_3, respectively (see TS 38.212, clauses 7.3.1.2.4 and 7.3.1.1.4).

dormancyDCI-1-3, dormancyDCI-0-3 Configure the presence of Scell dormancy indication field in DCI format 1_3 and DCI format 0_3, respectively.

minimumSchedulingOffsetK0DCI-1-3, minimumSchedulingOffsetK0DCI-0-3 Configure the presence of minimum applicable scheduling offset indicator field in DCI format 1_3 and DCI format 0_3, respectively.

nCI-Value Configure n_CI value used for the set of cells, where unique n_CI value is configured for each set of cells.

pdcchMonAdaptDCI-1-3, pdcchMonAdaptDCI-0-3 Configure the presence of PDCCH monitoring adaptation indication field in DCI format 1_3 and DCI format 0_3, respectively.

pdsch-HARQ-ACK-enhType3DCI-1-3 Enable the enhanced Type 3 HARQ-ACK codebook triggering using DCI format 1_3.

pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3 Enables the enhanced Type 3 CB through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in DCI format 1_3 if the more than one enhanced Type HARQ-ACK codebook is configured for the primary PUCCH cell group.

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3 When configured, the DCI_format 1_3 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group.

pdsch-HARQ-ACK-retxDCI-1-3 When configured, the DCI format 1_3 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource (see TS 38.213 [13], clause 9.1.5).

priorityIndicatorDCI-1-3, priorityIndicatorDCI-0-3 Configure the presence of priority indicator field in DCI format 1_3 and DCI format 0_3, respectively (see TS 38.212 [17], clauses 7.3.1.2.4 and 7.3.1.1.4 and TS 38.213 [13] clause 9).

pucch-sSCellDynDCI-1-3 Configure the UE with PUCCH cell switching based on dynamic indication in DCI format 1_3 (see TS 38.213 [13], clause 9.A).

RateMatchDCI-1-3 Configure each row of the joint rate matching indication table for DL scheduling via DCI format 1_3, where bitmap for a cell points to a corresponding rate matching indication applicable for DCI format 1-1 (i.e., MSB and LSB of bitmap refer rateMatchPatternGroup1 and rateMatchPatternGroup2 for a cell, respectively), the order of rate matching indication bitmap in each row refers the order of cells in ScheduledCell-ListDCI-1-3, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP (i.e., first bitmap is for the first cell in ScheduledCell-ListDCI-1-X, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP and so on), the number of entries in a row of rateMatchDCI-1-3 should be the same as the number of cells, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP, included in ScheduledCell-ListDCI-1-3, and entries for co-scheduled cells in a row of rateMatchDCI-1-3 are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3.

rateMatchListDCI-1-3 Configure joint rate matching indication table for DL scheduling via DCI format 1_3.

ScheduledCellCombo Configure each row of the table for combinations of co-scheduled cells for DL scheduling via DCI format 1_3 and for UL scheduling via DCI format 0_3, where index with value INTEGER (0...3) of co-scheduled cell refers to scheduledCellListDCI-1-3 for DL and scheduledCellListDCI-0-3 for UL.

scheduledCellComboListDCI-1-3, scheduledCellComboListDCI-0-3 Configure the table for combinations of co-scheduled cells for DL scheduling via DCI format 1_3 and DCI format 0_3, respectively.

scheduledCellListDCI-1-3, scheduledCellListDCI-0-3 Configure the list of possible co-scheduled cells in the set for DL scheduling via DCI format 1_3 and DCI format 0_3 respectively, where the serving cells in the list are in ascending order of serving cell indices and are mapped to index {0, 1, 2, 3} in the set. Total number of cells within the same set of cells i.e., in scheduledCellListDCI-1-3 and scheduledCellListDCI-0-3, is up to 4. When a cell is included in either or both of scheduledCellListDCI-1-3 or scheduledCellListDCI-0-3 for one set of cells MC-DCI-SetofCells, the cell cannot be included in any of scheduledCellListDCI-1-3 or scheduledCellListDCI-0-3 for any other set of cells.

setOfCellsId Configure index of the set of cells to be indicated in DCI format 0_3/1_3.

sri-DCI0-3 Configure the indication type for SRS resource indicator field in DCI format 0_3 (See TS 38.212, clause 7.3.1.1.4).

SRS-OffsetCombo Configure each row of the joint SRS offset indicator table for DL scheduling via DCI format 1_3 and for UL scheduling via DCI format 0_3, where index for a cell points to a corresponding SRS offset indicator applicable for DCI format 1-1 and 0-1, and the order of SRS offset indicator index in each row refers the order of cells in scheduledCell-ListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3, that are configured with more than one entry in availableSlotOffsetList for at least one aperiodic SRS resource set on at least one UL BWP and so on) for DL and scheduledCellListDCI-0-3 for UL, included in scheduledCellListDCI-1-3 for srs-OffsetListDCI-1-3 and scheduledCellListDCI-0-3 for srs-OffsetListDCI-0-3, and entries for co-scheduled cells in a row of SRS-OffsetCombo are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3/0_3.

srs-OffsetListDCI-1-3, srs-OffsetListDCI-0-3 Configure joint SRS offset indicator table for DL scheduling via DCI format 1_3 and DCI format 0_3, respectively.

SRS-RequestCombo Configure each row of the joint SRS request table for DL scheduling via DCI format 1_3 and for UL scheduling via DCI format 0_3, where index for a cell points to a corresponding SRS request applicable for DCI format 1-1 and 0-1, and the order of SRS request index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3 and so on) for DL and scheduledCellListDCI-0-3 for UL. The number of entries in a row of SRS-RequestCombo should be the same as the number of cells included in scheduledCellListDCI-1-3 for srs-RequestListDCI-1-3 and scheduledCellListDCI-0-3 for srs-RequestListDCI-0-3, and entries for co-scheduled cells in a row of SRS-RequestCombo are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3/0_3.

srs-RequestListDCI-1-3, srs-RequestListDCI-0-3 Configure joint SRS request table for DL scheduling via DCI format 1_3 and DCI format 0_3, respectively.

TCI-DCI-1-3 Configure each row of the joint TCI table for DL scheduling via DCI format 1_3, where index for a cell points to a corresponding TCI applicable for DCI format 1-1, and the order of TCI index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3 that configured with tci-StatesToAddModList and so on), the number of entries in a row of TCI-DCI-1-3 should be the same as the number of cells that configured with tci-StatesToAddModList on at least one DL BWP, included in scheduledCellListDCI-1-3, and entries for co-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3.

tci-ListDCI-1-3 Configure joint TCI table for DL scheduling via DCI format 1_3

TDRA-FieldIndexDC-0-3 Configure each row of the joint TDRA field table for UL scheduling via DCI format 0_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 0-1, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-0-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 0-3, as specified in 38.212, of the first cell in scheduledCellListDCI-0-3, second TDRA index in a row is for the second smallest BWP-Id 1 that can be scheduled by the DCI format 0-3, as specified in 38.212, of the first cell and so on), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-0-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 0-3, as specified in 38.212, across cells included in scheduledCellListDCI-0-3.

TDRA-FieldIndexDCI-1-3 Configure each row of the joint TDRA field table for DL scheduling via DCI format 1_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 1-1, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-1-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in 38.212, of the first cell in scheduledCellListDCI-1-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in 38.212, of the first cell and so on ), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-1-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 1-3, as specified in 38.212, across cells included in scheduledCellListDCI-1-3.

tdra-FieldIndexListDCI-1-3, tdra-FieldIndexListDCI-0-3 Configure joint TDRA table for UL scheduling via DCI format 1_3 and DCI format 0_3, respectively.

tpmi-DCI0-3 Configure the indication type for precoding information and number of layers field in DCI format 0_3 (See TS 38.212, clause 7.3.1.1.4).

ZP-CSI-DCI-1-3 Configure each row of the joint ZP-CSI-RS trigger table for DL scheduling via DCI format 1_3, where index for a cell points to a corresponding ZP-CSI-RS trigger applicable for DCI format 1-1, and the order of ZP-CSI-RS trigger index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3, that are configured with aperiodic-ZP-CSI-RS-ResourceSetsToAddModList on at least one DL BWP and so on), the number of entries in a row of ZP-CSI-DCI-1-3 should be the same as the number of cells, that are configured with aperiodic-ZP-CSI-RS-ResourceSetsToAddModList on at least one DL BWP, included in scheduledCellListDCI-1-3, and entries for co-scheduled cells in a row of ZP-CSI-DCI-1-3 are interpreted based on the BWPs of co-scheduled cells that is determined by the BWP indicator field of DCI format 1_3.

zp-CSI-RSListDCI-1-3 Configure joint ZP-CSI-RS trigger table for DL scheduling via DCI format 1_3.

NOTE 1:	If the dedicated part of initial UL/DL BWP configuration is absent, the initial BWP can be used but with some limitations. For example, changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching.

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S. |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For (e)RedCap UEs, this field is optionally present, Need M. |
| SCellOnly | This field is optionally present, Need R, for SCells. It is absent otherwise. |
| ServingCellWithoutPUCCH | This field is optionally present, Need S, for SCells except PUCCH SCells. It is absent otherwise. |
| SyncAndCellAdd | This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for an SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M. |
| TCI_ActivatedConfig | This field is optional Need N for SCells if sCellState is configured, otherwise it is absent. This field is optional Need S for the PSCell when the SCG is indicated as deactivated or is being activated, otherwise it is absent. This field is absent for the PCell. |
| TDD | This field is optionally present, Need R, for TDD cells. It is absent otherwise. |
| TDD_IAB | For IAB-MT, this field is optionally present, Need R, for TDD cells. It is absent otherwise. |
| TypeDCI0-3 | This field is mandatory present if ScheduledCellListDCI-0-3 is configured, otherwise it is absent, Need R. |
| TypeDCI1-3 | This field is mandatory present if ScheduledCellListDCI-1-3 is configured, otherwise it is absent, Need R. |

#### –	ServingCellConfigCommon

The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMON-START

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                            OPTIONAL, -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL, -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                                       OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16          ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}                   OPTIONAL, -- Need R

ssb-PositionQCL-r16                     SSB-PositionQCL-Relation-r16                                    OPTIONAL, -- Cond SharedSpectrum

highSpeedConfig-r16                     HighSpeedConfig-r16                                             OPTIONAL  -- Need R

]],

[[

highSpeedConfig-v1700               HighSpeedConfig-v1700                                               OPTIONAL, -- Need R

channelAccessMode2-r17              ENUMERATED {enabled}                                                OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-r17      ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25}  OPTIONAL, -- Need R

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                        OPTIONAL, -- Cond SharedSpectrum2

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                              OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommon-v1700                                            OPTIONAL, -- Need R

ntn-Config-r17                      NTN-Config-r17                                                      OPTIONAL  -- Need R

]],

[[

featurePriorities-r17               SEQUENCE {

redCapPriority-r17                  FeaturePriority-r17                                             OPTIONAL, -- Need R

slicingPriority-r17                 FeaturePriority-r17                                             OPTIONAL, -- Need R

msg3-Repetitions-Priority-r17       FeaturePriority-r17                                             OPTIONAL, -- Need R

sdt-Priority-r17                    FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17            ENUMERATED {enabled}                                                OPTIONAL -- Cond SharedSpectrum2

]],

[[

featurePriorities-v1800             SEQUENCE {

msg1-Repetitions-Priority-r18       FeaturePriority-r17                                             OPTIONAL, -- Need R

eRedCapPriority-r18                 FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL, -- Need R

atg-Config-r18                      ATG-Config-r18                                                      OPTIONAL   -- Need R

]]

}

-- TAG-SERVINGCELLCONFIGCOMMON-STOP

-- ASN1STOP

ServingCellConfigCommon field descriptions

channelAccessMode If present, this field indicates which channel access procedures to apply for operation with shared spectrum channel access as defined in TS 37.213 [48]. If the field is configured as "semiStatic", the UE shall apply the channel access procedures for semi-static channel occupancy as described in clause 4.3 in TS 37.213. If the field is configured as "dynamic", the UE shall apply the channel access procedures as defined in TS 37.213, clause 4.1 and 4.2.

channelAccessMode2 If present, the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations.

dmrs-TypeA-Position Position of (first) DM-RS for downlink (see TS 38.211 [16], clause 7.4.1.1.1) and uplink (TS 38.211 [16], clause 6.4.1.1.3).

downlinkConfigCommon The common downlink configuration of the serving cell, including the frequency information configuration and the initial downlink BWP common configuration. The parameters provided herein should match the parameters configured by MIB and SIB1 (if provided) of the serving cell, with the exception of controlResourceSetZero and searchSpaceZero which can be configured in ServingCellConfigCommon even if MIB indicates that they are absent.

discoveryBurstWindowLength Indicates the window length of the discovery burst in ms (see TS 37.213 [48]). The field discoveryBurstWindowLength-r17 is applicable to SCS 480 kHz and SCS 960 kHz.

featurePriorities Indicates priorities for features, such as (e)RedCap, Slicing, SDT, MSG1-Repetitions, and MSG3-Repetitions for Coverage Enhancements. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1.

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on this serving cell if n-TimingAdvanceOffset is not configured. If tag2 is configured for this serving cell, this field is to be applied to all uplink transmissions associated to tag-id configured for this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2.

ra-ChannelAccess If present, this field indicates that the UE shall apply channel access procedures before msg1/msgA transmission for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4.5 for FR2-2.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology (see TS 38.214 [19], clause 5.1.4.1). If a RateMatchPattern with the same RateMatchPatternId is configured in both ServingCellConfig/ServingCellConfigCommon and in SIB20/MCCH, the entire RateMatchPattern configuration, including the set of RBs/REs indicated by the patterns for the rate matching around, shall be the same and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-periodicityServingCell The SSB periodicity in ms for the rate matching purpose. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssb-PositionQCL Indicates the QCL relation between SSB positions for this serving cell as specified in TS 38.213 [13], clause 4.1.

ssb-PositionsInBurst For operation in licensed spectrum, indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access, the UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst (see TS 38.213 [13], clause 4.1). If the k-th bit of ssb-PositionsInBurst is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The k-th bit is set to 0, where k > ssb-PositionQCL and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access in FR1, only mediumBitmap is used, and for FR2-2, longBitmap is used.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

supplementaryUplinkConfig The network configures this field only if uplinkConfigCommon is configured. If this field is absent, the UE shall release the supplementaryUplinkConfig and the supplementaryUplink configured in ServingCellConfig of this serving cell, if configured.

tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see TS 38.213 [13], clause 11.1.

| Conditional Presence | Explanation |
| --- | --- |
| AbsFreqSSB | The field is absent when absoluteFrequencySSB in frequencyInfoDL is absent, otherwise the field is mandatory present. |
| HOAndServCellAdd | This field is mandatory present upon SpCell change (including path switch between a serving cell and a L2 U2N Relay UE) and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent. |
| HOAndServCellWithSSB | This field is mandatory present upon SpCell change and upon serving cell (SCell with SSB or PSCell) addition. Otherwise, the field is absent. |
| SharedSpectrum | This field is mandatory present if this cell operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this cell operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

#### –	ServingCellConfigCommonSIB

The IE ServingCellConfigCommonSIB is used to configure cell specific parameters of a UE's serving cell in SIB1.

ServingCellConfigCommonSIB information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=      SEQUENCE {

downlinkConfigCommon                DownlinkConfigCommonSIB,

uplinkConfigCommon                  UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

supplementaryUplink                 UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                           OPTIONAL, -- Need S

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

ssb-PeriodicityServingCell          ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                      OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                               OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16      ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}               OPTIONAL, -- Need R

highSpeedConfig-r16                 HighSpeedConfig-r16                                         OPTIONAL  -- Need R

]],

[[

channelAccessMode2-r17              ENUMERATED {enabled}                                        OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-v1700    ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25} OPTIONAL,  -- Need R

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                      OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommonSIB-v1700                                 OPTIONAL  -- Need R

]],

[[

enhancedMeasurementLEO-r17          ENUMERATED {true}                                           OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17                ENUMERATED {enabled}                                        OPTIONAL  -- Cond SharedSpectrum2

]],

[[

downlinkConfigCommon-v1760          DownlinkConfigCommonSIB-v1760                               OPTIONAL, -- Need R

uplinkConfigCommon-v1760            UplinkConfigCommonSIB-v1760                                 OPTIONAL  -- Need R

]]

}

-- TAG-SERVINGCELLCONFIGCOMMONSIB-STOP

-- ASN1STOP

ServingCellConfigCommonSIB field descriptions

channelAccessMode If present, this field indicates which channel access procedures to apply for operation with shared spectrum channel access as defined in TS 37.213 [48]. If the field is configured as "semiStatic", the UE shall apply the channel access procedures for semi-static channel occupancy as described in clause 4.3 in TS 37.213. If the field is configured as "dynamic", the UE shall apply the channel access procedures as defined in TS 37.213, clause 4.1 and 4.2.

channelAccessMode2 If present, this field indicates that the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations.

discoveryBurstWindowLength Indicates the window length of the discovery burst in ms (see TS 37.213 [48]). The field discoveryBurstWindowLength-v1700 is applicable to SCS 480 kHz and SCS 960 kHz.

enhancedMeasurementLEO If the field is present and UE supports the enhanced cell reselection requirements for NTN LEO in RRC_IDLE/RRC_INACTIVE, the UE shall apply the enhanced cell reselection requirements for NTN LEO as specified in TS 38.133 [14], clauses 4.2C.2.3 and 4.2C.2.4.

groupPresence This field is present when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to the SS/PBCH index 0-7, the second bit corresponds to SS/PBCH block 8-15, and so on. Value 0 in the bitmap indicates that the SSBs according to inOneGroup are absent. Value 1 indicates that the SS/PBCH blocks are transmitted in accordance with inOneGroup.

inOneGroup When maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1, only the 4 leftmost bits are valid; the UE ignores the 4 rightmost bits. When maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1, all 8 bits are valid. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. When maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1, all 8 bit are valid; The first/ leftmost bit corresponds to the first SS/PBCH block index in the group (i.e., to SSB index 0, 8, and so on); the second bit corresponds to the second SS/PBCH block index in the group (i.e., to SSB index 1, 9, and so on), and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2.

ra-ChannelAccess If present, this field indicates that the UE shall apply channel access procedures before msg1/msgA transmission for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4.5 for FR2-2.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR1, only inOneGroup is used and the UE interprets this field same as mediumBitmap in ServingCellConfigCommon. The UE assumes that a bit in inOneGroup at position k > NSSBQCL is 0, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR2-2, the m-th bit in groupPresence is set to 0 for m > NSSBQCL/8, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| SharedSpectrum | This field is mandatory present if this cell operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this cell operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

#### –	ShortI-RNTI-Value

The IE ShortI-RNTI-Value is used to identify the suspended UE context of a UE in RRC_INACTIVE using fewer bits compared to I-RNTI-Value.

ShortI-RNTI-Value information element

-- ASN1START

-- TAG-SHORTI-RNTI-VALUE-START

ShortI-RNTI-Value ::=   BIT STRING (SIZE(24))

-- TAG-SHORTI-RNTI-VALUE-STOP

-- ASN1STOP

#### –	ShortMAC-I

The IE ShortMAC-I is used to identify and verify the UE at RRC connection re-establishment. The 16 least significant bits of the MAC-I calculated using the AS security configuration of the source PCell, as specified in 5.3.7.4.

ShortMAC-I information element

-- ASN1START

-- TAG-SHORTMAC-I-START

ShortMAC-I ::=                      BIT STRING (SIZE (16))

-- TAG-SHORTMAC-I-STOP

-- ASN1STOP

#### –	SINR-Range

The IE SINR-Range specifies the value range used in SINR measurements and thresholds. For measurements, integer value for SINR measurements is according to Table 10.1.16.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 46) / 2 dB.

SINR-Range information element

-- ASN1START

-- TAG-SINR-RANGE-START

SINR-Range ::=                      INTEGER(0..127)

-- TAG-SINR-RANGE-STOP

-- ASN1STOP

#### –	SI-RequestConfig

The IE SI-RequestConfig contains configuration for Msg1 based SI request without Msg1 repetition.

SI-RequestConfig information element

-- ASN1START

-- TAG-SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SI-REQUESTCONFIG-STOP

-- ASN1STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occasions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-Request configuration in number of association periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting. Otherwise: - If si-SchedulingInfo-v1700 is not present and the SI-RequestConfig is used for on-demand SI request in SI-SchedulingInfo or PosSI-SchedulingInfo, the 1st entry in the list corresponds to the first SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfig is configured in SI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which si-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfig is configured in PosSI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

SI-RequestResources field descriptions

ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SI request for SI message(s) corresponding to this SI-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.

#### –	SI-RequestConfigRepetition

The IE SI-RequestConfigRepetition contains configuration for Msg1 based SI request with Msg1 repetition.

SI-RequestConfigRepetition information element

-- ASN1START

-- TAG-SI-REQUESTCONFIGREPETITION-START

SI-RequestConfigRepetition-r18 ::=     SEQUENCE {

rach-OccasionsSI-r18                   SEQUENCE {

rach-ConfigSI-r18                      RACH-ConfigGeneric,

ssb-perRACH-Occasion-r18               ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                               OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum2-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum4-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum8-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

...

}

SI-RequestResourcesRepetition-r18 ::=  SEQUENCE {

ra-PreambleStartIndex-r18              INTEGER (0..63),

ra-ssb-OccasionMaskIndex-r18           INTEGER (0..15)                                                          OPTIONAL   -- Need R

}

-- TAG-SI-REQUESTCONFIGREPETITION-STOP

-- ASN1STOP

SI-RequestConfigRepetition field descriptions

si-RequestResourcesRepetitionNum2, si-RequestResourcesRepetitionNum4, si-RequestResourcesRepetitionNum8 Indicates the configurations for Msg1-based SI request with Msg1 repetition for repetition number 2, 4 and 8, respectively. If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting. Otherwise: - If si-SchedulingInfo-v1700 is not present and the SI-RequestConfigRepetitionNum2/SI-RequestConfigRepetitionNum4/SI-RequestConfigRepetitionNum8 is used for on-demand SI request in SI-SchedulingInfo or PosSI-SchedulingInfo, the 1st entry in the list corresponds to the first SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfigRepetitionNum2/SI-RequestConfigRepetitionNum4/SI-RequestConfigRepetitionNum8 is configured in SI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which si-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfigRepetitioNum2/SI-RequestConfigRepetitionNum4/SI-RequestConfigRepetitionNum8n is configured in PosSI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResourcesRepetitionNum2/SI-RequestConfigRepetitionNum4/SI-RequestConfigRepetitionNum8 should not result in system information change notification.

#### –	SI-SchedulingInfo

The IE SI-SchedulingInfo contains information needed for acquisition of SI messages.

SI-SchedulingInfo information element

-- ASN1START

-- TAG-SI-SCHEDULINGINFO-START

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 },

si-RequestConfig                    SI-RequestConfig                                                OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                                OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                                          OPTIONAL,  -- Need R

...

}

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

SI-SchedulingInfo-v1700 ::=         SEQUENCE {

schedulingInfoList2-r17             SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17,

dummy                              SI-RequestConfig                                                 OPTIONAL

}

SI-SchedulingInfo-v1740 ::=         SEQUENCE {

si-RequestConfigRedCap-r17          SI-RequestConfig                                                OPTIONAL   -- Cond REDCAP-MSG-1

}

SI-SchedulingInfo-v1800 ::=         SEQUENCE {

si-RequestConfigMSG1-Repetition-r18           SI-RequestConfigRepetition-r18                        OPTIONAL,  -- Cond MSG-1

si-RequestConfigRedCap-MSG1-Repetition-r18    SI-RequestConfigRepetition-r18                        OPTIONAL,  -- Cond REDCAP-MSG-1

si-RequestConfigSUL-MSG1-Repetition-r18       SI-RequestConfigRepetition-r18                        OPTIONAL   -- Cond SUL-MSG-1

}

SchedulingInfo2-r17 ::=             SEQUENCE {

si-BroadcastStatus-r17              ENUMERATED {broadcasting, notBroadcasting},

si-WindowPosition-r17               INTEGER (1..256),

si-Periodicity-r17                  ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo-r17                 SIB-Mapping-v1700

}

SIB-Mapping ::=                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo

SIB-Mapping-v1700  ::=              SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700

SIB-TypeInfo ::=                    SEQUENCE {

type                                ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610,

sibType14-v1610, spare3, spare2, spare1,... },

valueTag                            INTEGER (0..31)                                                 OPTIONAL, -- Cond SIB-TYPE

areaScope                           ENUMERATED {true}                                               OPTIONAL -- Need S

}

SIB-TypeInfo-v1700 ::=              SEQUENCE {

sibType-r17                         CHOICE {

type1-r17                           ENUMERATED {sibType15, sibType16, sibType17, sibType18, sibType19, sibType20, sibType21,

sibType22-v1800, sibType23-v1800 ,sibType24-v1800, sibType25-v1800,

spare5, spare4, spare3, spare2, spare1,...},

type2-r17                           SEQUENCE {

posSibType-r17                      ENUMERATED {posSibType1-9, posSibType1-10, posSibType2-24, posSibType2-25,

posSibType6-4, posSibType6-5, posSibType6-6, posSibType2-17a-v1770,

posSibType2-18a-v1770, posSibType2-20a-v1770, posSibType1-11-v1800,

posSibType1-12-v1800, posSibType2-26-v1800, posSibType2-27-v1800,

posSibType6-7-v1800, posSibType7-1-v1800,...,

posSibType7-2-v1800, posSibType7-3-v1800, posSibType7-4-v1800},

encrypted-r17                       ENUMERATED { true }                                     OPTIONAL, -- Need R

gnss-id-r17                         GNSS-ID-r16                                             OPTIONAL, -- Need R

sbas-id-r17                         SBAS-ID-r16                                             OPTIONAL  -- Cond GNSS-ID-SBAS

}

},

valueTag-r17                            INTEGER (0..31)                                             OPTIONAL, -- Cond NonPosSIB

areaScope-r17                           ENUMERATED {true}                                           OPTIONAL  -- Need S

}

-- TAG-SI-SCHEDULINGINFO-STOP

-- ASN1STOP

SchedulingInfo field descriptions

areaScope Indicates that a SIB is area specific. If the field is absent, the SIB is cell specific.

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. If si-SchedulingInfo-v1700 is present, the network ensures that the total number of SI messages with si-BroadcastStatus set to notBroadcasting in the list of concatenated SI messages configured by schedulingInfoList in si-SchedulingInfo and SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 does not exceed the limit of maxSI-Message when si-RequestConfig, si-RequestConfigRedCap or si-RequestConfigSUL is configured.

si-Periodicity Periodicity of the SI-message in radio frames. Value rf8 corresponds to 8 radio frames, value rf16 corresponds to 16 radio frames, and so on.

SI-SchedulingInfo field descriptions

dummy This field is not used in this specification. If received, it is ignored by the UE.

si-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigMSG1-Repetition Configuration of Msg1 repetition resources on NUL that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

si-RequestConfigRedCap Configuration of Msg1 resources for initialUplinkBWP-RedCap that the (e)RedCap UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigRedCap-MSG1-Repetition Configuration of Msg1 repetition resources for initialUplinkBWP-RedCap that the (e)RedCap UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

si-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigSUL-MSG1-Repetition Configuration of Msg1 repetition resources on SUL that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

si-WindowLength The length of the SI scheduling window. Value s5 corresponds to 5 slots, value s10 corresponds to 10 slots and so on. The network always configures si-WindowLength to be shorter than or equal to the si-Periodicity. The values s2560-v1710 and s5120-v1710 are only applicable for SCS 480 kHz.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

SchedulingInfo2 field descriptions

encrypted The presence of this field indicates that the pos-sib-type is encrypted as specified in TS 37.355 [49].

gnss-id The presence of this field indicates that the positioning SIB type is for a specific GNSS. Indicates a specific GNSS (see also TS 37.355 [49])

posSibType The posSIBs as defined in TS 37.355 [49] mapped to SI for scheduling using schedulingInfoList2.

sbas-id The presence of this field indicates that the positioning SIB type is for a specific SBAS. Indicates a specific SBAS (see also TS 37.355 [49]).

si-WindowPosition This field indicates the SI window position of the associated SI-message. The network provides si-WindowPosition in an ascending order, i.e. si-WindowPosition in the subsequent entry in schedulingInfoList2 has always value higher than in the previous entry of schedulingInfoList2. The network configures this field in a way that ensures that SI messages scheduled by schedulingInfoList and/or posSchedulingInfoList do not overlap with SI messages scheduled by schedulingInfoList2.

sib-MappingInfo Indicates which SIBs or posSIBs are contained in the SI message.

sibType The type of SIB(s) mapped to SI for scheduling using schedulingInfoList2. Value type1 indicates SIBs and value type2 indicates posSIBs.

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if gnss-id is set to sbas. It is absent otherwise. |
| MSG-1 | The field is optionally present, Need R, if si-BroadcastStatus is set to notBroadcasting for any SI-message included in schedulingInfoList or any SI-message containing type1 SIB included in schedulingInfoList2. It is absent otherwise. |
| SIB-TYPE | The field is mandatory present if the SIB type is different from SIB6, SIB7 or SIB8. For SIB6, SIB7 and SIB8 it is absent. |
| NonPosSIB | The field is mandatory present if the SIB type is type1. For type2 it is absent. |
| SUL-MSG-1 | The field is optionally present, Need R, if supplementaryUplink is configured in ServingCellConfigCommonSIB and if si-BroadcastStatus is set to notBroadcasting for any SI-message included in schedulingInfoList or any SI-message containing type1 SIB included in schedulingInfoList2. It is absent otherwise. |
| REDCAP-MSG-1 | The field is optionally present, Need R, if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if si-BroadcastStatus is set to notBroadcasting for any SI-message included in schedulingInfoList or any SI-message containing type1 SIB included in schedulingInfoList2. It is absent otherwise. |

#### –	SK-Counter

The IE SK-Counter is a counter used upon initial configuration of SN security for NR-DC and NE-DC, as well as upon refresh of S-KgNB or S-KeNB based on the current or newly derived KgNB during RRC Resume or RRC Reconfiguration, as defined in TS 33.501 [11].

-- ASN1START

-- TAG-SKCOUNTER-START

SK-Counter ::=  INTEGER (0..65535)

-- TAG-SKCOUNTER-STOP

-- ASN1STOP

#### –	SlotFormatCombinationsPerCell

The IE SlotFormatCombinationsPerCell is used to configure the SlotFormatCombinations applicable for one serving cell (see TS 38.213 [13], clause 11.1.1).

SlotFormatCombinationsPerCell information element

-- ASN1START

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-START

SlotFormatCombinationsPerCell ::=   SEQUENCE {

servingCellId                       ServCellIndex,

subcarrierSpacing                   SubcarrierSpacing,

subcarrierSpacing2                  SubcarrierSpacing                                                         OPTIONAL, -- Need R

slotFormatCombinations              SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination

OPTIONAL, -- Need M

positionInDCI                       INTEGER(0..maxSFI-DCI-PayloadSize-1)                                      OPTIONAL, -- Need M

...,

[[

enableConfiguredUL-r16              ENUMERATED {enabled}                                                      OPTIONAL  -- Need R

]]

}

SlotFormatCombination ::=           SEQUENCE {

slotFormatCombinationId             SlotFormatCombinationId,

slotFormats                         SEQUENCE (SIZE (1..maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255)

}

SlotFormatCombinationId ::=         INTEGER (0..maxNrofSlotFormatCombinationsPerSet-1)

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-STOP

-- ASN1STOP

SlotFormatCombination field descriptions

slotFormatCombinationId This ID is used in the DCI payload to dynamically select this SlotFormatCombination (see TS 38.213 [13], clause 11.1.1).

slotFormats Slot formats that occur in consecutive slots in time domain order as listed here (see TS 38.213 [13], clause 11.1.1 and TS 38.213 [13], clause 14 for IAB-MT).

SlotFormatCombinationsPerCell field descriptions

enableConfiguredUL If configured, the UE is allowed to transmit uplink signals/channels (SRS, PUCCH, CG-PUSCH) in the set of symbols of the slot when the UE does not detect a DCI format 2_0 providing a slot format for the set of symbols (see TS 38.213 [13], 11.1.1). This field is applicable only if cg-RetransmissionTimer-r16 is configured.

positionInDCI The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.1.1).

servingCellId The ID of the serving cell for which the slotFormatCombinations are applicable.

slotFormatCombinations A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see TS 38.211 [16], clause 4.3.2). The total number of slotFormats in the slotFormatCombinations list does not exceed 512.

subcarrierSpacing2 Reference subcarrier spacing for a Slot Format Combination on an FDD or SUL cell (see TS 38.213 [13], clause 11.1.1). For FDD, subcarrierSpacing (SFI-scs) is the reference SCS for DL BWP and subcarrierSpacing2 (SFI-scs2) is the reference SCS for UL BWP. For SUL, subcarrierSpacing (SFI-scs) is the reference SCS for non-SUL carrier and subcarrierSpacing2 (SFI-scs2) is the reference SCS for SUL carrier. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications.

subcarrierSpacing Reference subcarrier spacing for this Slot Format Combination. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications (see TS 38.213 [13], clause 11.1.1).

#### –	SlotFormatIndicator

The IE SlotFormatIndicator is used to configure monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI).

SlotFormatIndicator information element

-- ASN1START

-- TAG-SLOTFORMATINDICATOR-START

SlotFormatIndicator ::=     SEQUENCE {

sfi-RNTI                    RNTI-Value,

dci-PayloadSize             INTEGER (1..maxSFI-DCI-PayloadSize),

slotFormatCombToAddModList  SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell

OPTIONAL, -- Need N

slotFormatCombToReleaseList SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex                       OPTIONAL, -- Need N

...,

[[

availableRB-SetsToAddModList-r16  SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF AvailableRB-SetsPerCell-r16   OPTIONAL, -- Need N

availableRB-SetsToReleaseList-r16 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex                 OPTIONAL, -- Need N

switchTriggerToAddModList-r16     SEQUENCE (SIZE(1..4)) OF SearchSpaceSwitchTrigger-r16                                   OPTIONAL, -- Need N

switchTriggerToReleaseList-r16    SEQUENCE (SIZE(1..4)) OF ServCellIndex                                                  OPTIONAL, -- Need N

co-DurationsPerCellToAddModList-r16 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF CO-DurationsPerCell-r16     OPTIONAL, -- Need N

co-DurationsPerCellToReleaseList-r16 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex              OPTIONAL  -- Need N

]],

[[

switchTriggerToAddModListSizeExt-r16   SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroupMinus4-r16)) OF

SearchSpaceSwitchTrigger-r16  OPTIONAL, -- Need N

switchTriggerToReleaseListSizeExt-r16  SEQUENCE (SIZE(1.. maxNrofAggregatedCellsPerCellGroupMinus4-r16)) OF

ServCellIndex                 OPTIONAL  -- Need N

]],

[[

co-DurationsPerCellToAddModList-r17 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF CO-DurationsPerCell-r17 OPTIONAL  -- Need N

]]

}

CO-DurationsPerCell-r16 ::=   SEQUENCE {

servingCellId-r16             ServCellIndex,

positionInDCI-r16             INTEGER(0..maxSFI-DCI-PayloadSize-1),

subcarrierSpacing-r16         SubcarrierSpacing,

co-DurationList-r16           SEQUENCE (SIZE(1..64)) OF CO-Duration-r16

}

CO-DurationsPerCell-r17 ::=   SEQUENCE {

servingCellId-r17             ServCellIndex,

positionInDCI-r17             INTEGER(0..maxSFI-DCI-PayloadSize-1),

subcarrierSpacing-r17         SubcarrierSpacing,

co-DurationList-r17           SEQUENCE (SIZE(1..64)) OF CO-Duration-r17

}

CO-Duration-r16 ::=    INTEGER (0..1120)

CO-Duration-r17 ::=    INTEGER (0..4480)

AvailableRB-SetsPerCell-r16 ::=   SEQUENCE {

servingCellId-r16                 ServCellIndex,

positionInDCI-r16                 INTEGER(0..maxSFI-DCI-PayloadSize-1)

}

SearchSpaceSwitchTrigger-r16 ::=   SEQUENCE {

servingCellId-r16                  ServCellIndex,

positionInDCI-r16                  INTEGER(0..maxSFI-DCI-PayloadSize-1)

}

-- TAG-SLOTFORMATINDICATOR-STOP

-- ASN1STOP

SlotFormatIndicator field descriptions

availableRB-SetsToAddModList A list of AvailableRB-SetsPerCell objects (see TS 38.213 [13], clause 11.1.1).

co-DurationsPerCellToAddModList A list of CO-DurationsPerCell objects. If not configured, the UE uses the slot format indicator (SFI), if available, to determine the channel occupancy duration (see TS 38.213 [13], clause 11.1.1).

co-DurationsPerCellToReleaseList A list of CO-DurationsPerCell objects to be released. An entry created using co-DurationsPerCellToAddModList-r16 or co-DurationsPerCellToAddModList-r17 can be deleted using co-DurationsPerCellToReleaseList.

dci-PayloadSize Total length of the DCI payload scrambled with SFI-RNTI (see TS 38.213 [13], clause 11.1.1).

sfi-RNTI RNTI used for SFI on the given cell (see TS 38.213 [13], clause 11.1.1).

slotFormatCombToAddModList A list of SlotFormatCombinations for the UE's serving cells (see TS 38.213 [13], clause 11.1.1).

switchTriggerToAddModList, switchTriggerToAddModListSizeExt A list of SearchSpaceSwitchTrigger objects. Each SearchSpaceSwitchTrigger object provides position in DCI of the bit field indicating search space switching flag for a serving cell or, if cellGroupsForSwitchList is configured, group of serving cells (see TS 38.213 [13], clause 10.4). If cellGroupsForSwitchList is configured, only one of the cells belonging to the same cell group is added/modified, and the configuration applies to all cells belonging to the cellGroupsForSwitchList (see TS 38.213 [13], clause 10.4). The network configures more than 4 SearchSpaceSwitchTrigger objects only if cellGroupsForSwitchList is not configured. The UE shall consider entries in switchTriggerToAddModList and in switchTriggerToAddModListSizeExt as a single list, i.e. an entry created using switchTriggerToAddModList can be modifed using switchTriggerToAddModListSizeExt and vice-versa.

switchTriggerToReleaseModList, switchTriggerToReleaseListSizeExt A list of SearchSpaceSwitchTriggers to be released. If cellGroupsForSwitchList is configured, the SearchSpaceSwitchTrigger is released for all serving cells belonging to the same CellGroupForSwitch. The UE shall consider entries in switchTriggerToReleaseList and in switchTriggerToReleaseListSizeExt as a single list, i.e. an entry created using switchTriggerToAddModList or switchTriggerToAddModListSizeExt can be deleted using switchTriggerToReleaseList or switchTriggerToReleaseListSizeExt.

AvailableRB-SetsPerCell field descriptions

positionInDCI The (starting) position of the bits within DCI payload indicating the availability of the RB sets of a serving cell (see TS 38.213 [13], clause 11.1.1).

servingCelIId The ID of the serving cell for which the configuration is applicable.

CO-DurationsPerCell field descriptions

co-DurationList A list of Channel Occupancy duration in symbols. The maximum duration that can be configured for the following SCS: - 15 kHz: 280. - 30 kHz: 560. - 60 kHz: 1120. - 120 kHz: 560. - 480 kHz: 2240. - 960 kHz: 4480.

positionInDCI Position in DCI of the bit field indicating Channel Occupancy duration for UE's serving cells (see TS 38.213 [13], clause 11.1.1).

servingCelIId The ID of the serving cell for which the configuration is applicable.

subcarrierSpacing Reference subcarrier spacing for the list of Channel Occupancy durations (see TS 38.213 [13], clause 11.1.1). Only the following values are applicable depending on the used frequency range: FR1: 15, 30, or 60 kHz FR2-2: 120, 480, or 960 kHz

SearchSpaceSwitchTrigger field descriptions

positionInDCI The position of the bit within DCI payload containing a search space switching flag (see TS 38.213 [13], clause 11.1.1).

servingCellId The ID of the serving cell for which the configuration is applicable or the group of serving cells as indicated by CellGroupsForSwitch-r16 containing this servingCellId.

#### –	S-NSSAI

The IE S-NSSAI (Single Network Slice Selection Assistance Information) identifies a Network Slice end to end and comprises a slice/service type and a slice differentiator, see TS 23.003 [21].

S-NSSAI information element

-- ASN1START

-- TAG-S-NSSAI-START

S-NSSAI  ::=                        CHOICE{

sst                                 BIT STRING (SIZE (8)),

sst-SD                              BIT STRING (SIZE (32))

}

-- TAG-S-NSSAI-STOP

-- ASN1STOP

S-NSSAI field descriptions

sst Indicates the S-NSSAI consisting of Slice/Service Type, see TS 23.003 [21].

sst-SD Indicates the S-NSSAI consisting of Slice/Service Type and Slice Differentiator, see TS 23.003 [21].

#### –	SpeedStateScaleFactors

The IE SpeedStateScaleFactors concerns factors, to be applied when the UE is in medium or high speed state, used for scaling a mobility control related parameter.

SpeedStateScaleFactors information element

-- ASN1START

-- TAG-SPEEDSTATESCALEFACTORS-START

SpeedStateScaleFactors ::=          SEQUENCE {

sf-Medium                           ENUMERATED {oDot25, oDot5, oDot75, lDot0},

sf-High                             ENUMERATED {oDot25, oDot5, oDot75, lDot0}

}

-- TAG-SPEEDSTATESCALEFACTORS-STOP

-- ASN1STOP

SpeedStateScaleFactors field descriptions

sf-High The concerned mobility control related parameter is multiplied with this factor if the UE is in High Mobility state as defined in TS 38.304 [20]. Value oDot25 corresponds to 0.25, value oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

sf-Medium The concerned mobility control related parameter is multiplied with this factor if the UE is in Medium Mobility state as defined in TS 38.304 [20]. Value oDot25 corresponds to 0.25, value oDot5 corresponds to 0.5, value oDot75 corresponds to 0.75, and so on.

#### –	SPS-Config

The IE SPS-Config is used to configure downlink semi-persistent transmission. Multiple Downlink SPS configurations may be configured in one BWP of a serving cell.

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

SPS-Config ::=                  SEQUENCE {

periodicity                     ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

nrofHARQ-Processes              INTEGER (1..8),

n1PUCCH-AN                      PUCCH-ResourceId                                                            OPTIONAL,   -- Need M

mcs-Table                       ENUMERATED {qam64LowSE}                                                     OPTIONAL,   -- Need S

...,

[[

sps-ConfigIndex-r16             SPS-ConfigIndex-r16                                                         OPTIONAL,   -- Cond SPS-List

harq-ProcID-Offset-r16          INTEGER (0..15)                                                             OPTIONAL,   -- Need R

periodicityExt-r16              INTEGER (1..5120)                                                           OPTIONAL,   -- Need R

harq-CodebookID-r16             INTEGER (1..2)                                                              OPTIONAL,   -- Need R

pdsch-AggregationFactor-r16     ENUMERATED {n1, n2, n4, n8 }                                                OPTIONAL    -- Need S

]],

[[

sps-HARQ-Deferral-r17           INTEGER (1..32)                                                             OPTIONAL,   -- Need R

n1PUCCH-AN-PUCCHsSCell-r17      PUCCH-ResourceId                                                            OPTIONAL,   -- Need R

periodicityExt-r17              INTEGER (1..40960)                                                          OPTIONAL,   -- Need R

nrofHARQ-Processes-v1710        INTEGER(9..32)                                                              OPTIONAL,   -- Need R

harq-ProcID-Offset-v1700        INTEGER (16..31)                                                            OPTIONAL    -- Need R

]]

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

SPS-Config field descriptions

harq-CodebookID Indicates the HARQ-ACK codebook index for the corresponding HARQ-ACK codebook for SPS PDSCH and ACK for SPS PDSCH release.

harq-ProcID-Offset Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.3.1.

mcs-Table Indicates the MCS table the UE shall use for DL SPS (see TS 38.214 [19],clause 5.1.3.1. If present, the UE shall use the MCS table of low-SE 64QAM table indicated in Table 5.1.3.1-3 of TS 38.214 [19]. If this field is absent and field mcs-table in PDSCH-Config is set to 'qam256' and the activating DCI is of format 1_1, the UE applies the 256QAM table indicated in Table 5.1.3.1-2 of TS 38.214 [19]. If this field is absent and the field mcs-Table-r17 in PDSCH-Config is set to 'qam1024' and the activating DCI is format 1_1, the UE applies the 1024QAM table indicated in Table 5.1.3.1-4 of TS 38.214 [19]. Otherwise, the UE applies the non-low-SE 64QAM table indicated in Table 5.1.3.1-1 of TS 38.214 [19].

n1PUCCH-AN HARQ resource for PUCCH for DL SPS. The network configures the resource either as format0 or format1. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

n1PUCCH-AN-PUCCHsSCell HARQ resource for PUCCH on PUCCH switching SCell (sSCell) for DL SPS. The network configures the resource either as format 0 or format 1. The actual PUCCH-Resource is configured in PUCCH-Config of the PUCCH sSCell and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

nrofHARQ-Processes Number of configured HARQ processes for SPS DL (see TS 38.321 [3], clause 5.8.1). If UE is configured with nrofHARQ-Processes-v1710 UE shall ignore nrofHARQ-Processes (without suffix).

pdsch-AggregationFactor Number of repetitions for SPS PDSCH (see TS 38.214 [19], clause 5.1.2.1). When the field is absent, the UE applies the value 1 for MBS multicast data and the pdsch-AggregationFactor in pdsch-Config for other data.

periodicity Periodicity for DL SPS (see TS 38.214 [19] and TS 38.321 [3], clause 5.8.1).

periodicityExt This field is used to calculate the periodicity for DL SPS (see TS 38.214 [19] and see TS 38.321 [3], clause 5.8.1). If this field is present, the field periodicity is ignored. The following periodicities are supported depending on the configured subcarrier spacing [ms]: 15 kHz: periodicityExt, where periodicityExt has a value between 1 and 640. 30 kHz: 0.5 x periodicityExt, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP. 0.25 x periodicityExt, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: 0.25 x periodicityExt, where periodicityExt has a value between 1 and 2560. 120 kHz: 0.125 x periodicityExt, where periodicityExt has a value between 1 and 5120. 480 kHz: 0.03125 x periodicityExt, where periodicityExt has a value between 1 and 20480. 960 kHz: 0.015625 x periodicityExt, where periodicityExt has a value between 1 and 40960. periodicityExt-r17 is only applicable for SCS 480 kHz and 960 kHz.

sps-ConfigIndex Indicates the index of one of multiple SPS configurations.

sps-HARQ-Deferral Indicates the maximum number of slots or subslots the transmission of DL SPS HARQ-ACK in a slot or subslot can be deferred (see TS 38.213 [13], clause 9.2.5.4).

| Conditional Presence | Explanation |
| --- | --- |
| SPS-List | The field is mandatory present when included in sps-ConfigToAddModList-r16 or sps-ConfigMulticastToAddModList-r17, otherwise the field is absent. |

#### –	SPS-ConfigIndex

The IE SPS-ConfigIndex is used to indicate the index of one of multiple DL SPS configurations in one BWP.

SPS-ConfigIndex information element

-- ASN1START

-- TAG-SPS-CONFIGINDEX-START

SPS-ConfigIndex-r16             ::= INTEGER (0.. maxNrofSPS-Config-1-r16)

-- TAG-SPS-CONFIGINDEX-STOP

-- ASN1STOP

#### –	SPS-PUCCH-AN

The IE SPS-PUCCH-AN is used to indicate a PUCCH resource for HARQ ACK and configure the corresponding maximum payload size for the PUCCH resource.

SPS-PUCCH-AN information element

-- ASN1START

-- TAG-SPS-PUCCH-AN-START

SPS-PUCCH-AN-r16  ::=           SEQUENCE {

sps-PUCCH-AN-ResourceID-r16     PUCCH-ResourceId,

maxPayloadSize-r16              INTEGER (4..256)                     OPTIONAL    -- Need R

}

-- TAG-SPS-PUCCH-AN-STOP

-- ASN1STOP

SPS-PUCCH-AN field descriptions

maxPayloadSize Indicates the maximum payload size for the corresponding PUCCH resource ID.

sps-PUCCH-AN-ResourceID Indicates the PUCCH resource ID

#### –	SPS-PUCCH-AN-List

The IE SPS-PUCCH-AN-List is used to configure the list of PUCCH resources per HARQ ACK codebook

SPS-PUCCH-AN-List information element

-- ASN1START

-- TAG-SPS-PUCCH-AN-LIST-START

SPS-PUCCH-AN-List-r16 ::=      SEQUENCE (SIZE(1..4)) OF SPS-PUCCH-AN-r16

-- TAG-SPS-PUCCH-AN-LIST-STOP

-- ASN1STOP

#### –	SRB-Identity

The IE SRB-Identity is used to identify a Signalling Radio Bearer (SRB) used by a UE.

SRB-Identity information element

-- ASN1START

-- TAG-SRB-IDENTITY-START

SRB-Identity ::=                    INTEGER (1..3)

SRB-Identity-v1700 ::=              INTEGER (4)

SRB-Identity-v1800 ::=              INTEGER (5)

-- TAG-SRB-IDENTITY-STOP

-- ASN1STOP

#### –	SRS-CarrierSwitching

The IE SRS-CarrierSwitching is used to configure for SRS carrier switching when PUSCH is not configured and independent SRS power control from that of PUSCH.

SRS-CarrierSwitching information element

-- ASN1START

-- TAG-SRS-CARRIERSWITCHING-START

SRS-CarrierSwitching ::=            SEQUENCE {

srs-SwitchFromServCellIndex         INTEGER (0..31)                                                         OPTIONAL,   -- Need M

srs-SwitchFromCarrier               ENUMERATED {sUL, nUL},

srs-TPC-PDCCH-Group                 CHOICE {

typeA                               SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

typeB                               SRS-TPC-PDCCH-Config

}                                                                                                           OPTIONAL,   -- Need M

monitoringCells                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex               OPTIONAL,   -- Need M

...

}

SRS-TPC-PDCCH-Config ::=            SEQUENCE {

srs-CC-SetIndexlist                 SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex                                OPTIONAL    -- Need M

}

SRS-CC-SetIndex ::=                 SEQUENCE {

cc-SetIndex                         INTEGER (0..3)                                                          OPTIONAL,   -- Need M

cc-IndexInOneCC-Set                 INTEGER (0..7)                                                          OPTIONAL    -- Need M

}

-- TAG-SRS-CARRIERSWITCHING-STOP

-- ASN1STOP

SRS-CC-SetIndex field descriptions

cc-IndexInOneCC-Set Indicates the CC index in one CC set for Type A (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA.

cc-SetIndex Indicates the CC set index for Type A associated (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA. The network does not configure this field to 3 in this release of specification.

SRS-CarrierSwitching field descriptions

monitoringCells A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.3).

srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less SCell. During SRS transmission on a PUSCH-less SCell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less SCell to transmit SRS. (see TS 38.214 [19], clause 6.2.1.3).

srs-TPC-PDCCH-Group Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any.

typeA Type A trigger configuration for SRS transmission on a PUSCH-less SCell (see TS 38.213 [13], clause 11.4). In this release, the network configures at most one entry (the first entry) of typeA, and the first entry corresponds to the serving cell in which the SRS-CarrierSwitching field is configured. SRS carrier switching to SUL carrier is not supported in this version of the specification.

typeB Type B trigger configuration for SRS transmission on a PUSCH-less SCell (see TS 38.213 [13], clause 11.4).

SRS-TPC-PDCCH-Config field descriptions

srs-CC-SetIndexlist A list of pairs of [cc-SetIndex; cc-IndexInOneCC-Set] (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network does not configure this field for typeB.

#### –	SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources, a list of SRS-PosResources, a list of SRS-PosResourceSets and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources or SRS-PosResources. The network triggers the transmission of the set of SRS-Resources or SRS-PosResources using a configured aperiodicSRS-ResourceTrigger (L1 DCI). The network does not configure SRS specific power control parameters alpha (without suffix) or pathlossReferenceRS if unifiedTCI-StateType is configured for the serving cell.

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId              OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource                OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                                   OPTIONAL,   -- Need S

...,

[[

srs-RequestDCI-1-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-RequestDCI-0-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-ResourceSetToAddModListDCI-0-2-r16  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL, -- Need N

srs-ResourceSetToReleaseListDCI-0-2-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL, -- Need N

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16      SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16        OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r16        SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16            OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16         SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16  OPTIONAL  -- Need N

]],

[[

dci-TriggeringPosResourceSetLink-r18    ENUMERATED { enabled }                                                  OPTIONAL  -- Need R

]]

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]],

[[

usagePDC-r17                            ENUMERATED {true}                                              OPTIONAL, -- Need R

availableSlotOffsetList-r17             SEQUENCE (SIZE(1..4)) OF AvailableSlotOffset-r17               OPTIONAL, -- Need R

followUnifiedTCI-StateSRS-r17           ENUMERATED {enabled}                                           OPTIONAL  -- Need R

]],

[[

applyIndicatedTCI-State-r18             ENUMERATED {first, second}                                     OPTIONAL  -- Cond FollowUTCI

]]

}

AvailableSlotOffset-r17 ::=   INTEGER (0..7)

PathlossReferenceRS-Config ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

PathlossReferenceRSList-r16 ::=             SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRS-r16)) OF PathlossReferenceRS-r16

PathlossReferenceRS-r16 ::=                 SEQUENCE {

srs-PathlossReferenceRS-Id-r16              SRS-PathlossReferenceRS-Id-r16,

pathlossReferenceRS-r16                     PathlossReferenceRS-Config

}

SRS-PathlossReferenceRS-Id-r16 ::=          INTEGER (0..maxNrofSRS-PathlossReferenceRS-1-r16)

SRS-PosResourceSet-r16 ::=                  SEQUENCE {

srs-PosResourceSetId-r16                    SRS-PosResourceSetId-r16,

srs-PosResourceIdList-r16                   SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16

OPTIONAL, -- Cond Setup

resourceType-r16                            CHOICE {

aperiodic-r16                               SEQUENCE {

aperiodicSRS-ResourceTriggerList-r16        SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-1))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL, -- Need M

...

},

semi-persistent-r16                         SEQUENCE {

...

},

periodic-r16                                SEQUENCE {

...

}

},

alpha-r16                                   Alpha                                                      OPTIONAL, -- Need S

p0-r16                                      INTEGER (-202..24)                                         OPTIONAL, -- Cond Setup

pathlossReferenceRS-Pos-r16                 CHOICE {

ssb-IndexServing-r16                        SSB-Index,

ssb-Ncell-r16                               SSB-InfoNcell-r16,

dl-PRS-r16                                  DL-PRS-Info-r16

}                                                                                                      OPTIONAL, -- Need M

...

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-PosResourceSetId-r16 ::=            INTEGER (0..maxNrofSRS-PosResourceSets-1-r16)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                           OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                        OPTIONAL,   -- Need R

...,

[[

resourceMapping-r16                     SEQUENCE {

startPosition-r16                       INTEGER (0..13),

nrofSymbols-r16                         ENUMERATED {n1, n2, n4},

repetitionFactor-r16                    ENUMERATED {n1, n2, n4}

}                                                                                                      OPTIONAL    -- Need R

]],

[[

spatialRelationInfo-PDC-r17             SetupRelease { SpatialRelationInfo-PDC-r17 }                   OPTIONAL,   -- Need M

resourceMapping-r17                     SEQUENCE {

startPosition-r17                       INTEGER (0..13),

nrofSymbols-r17                         ENUMERATED {n1, n2, n4, n8, n10, n12, n14},

repetitionFactor-r17                    ENUMERATED {n1, n2, n4, n5, n6, n7, n8, n10, n12, n14}

}                                                                                                      OPTIONAL,   -- Need R

partialFreqSounding-r17                 SEQUENCE {

startRBIndexFScaling-r17                CHOICE{

startRBIndexAndFreqScalingFactor2-r17   INTEGER (0..1),

startRBIndexAndFreqScalingFactor4-r17   INTEGER (0..3)

},

enableStartRBHopping-r17                ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

transmissionComb-n8-r17                 SEQUENCE {

combOffset-n8-r17                       INTEGER (0..7),

cyclicShift-n8-r17                      INTEGER (0..5)

}                                                                                                      OPTIONAL,   -- Need R

srs-TCI-State-r17                       CHOICE {

srs-UL-TCI-State                        TCI-UL-StateId-r17,

srs-DLorJointTCI-State                  TCI-StateId

}                                                                                                      OPTIONAL    -- Need R

]],

[[

repetitionFactor-v1730                  ENUMERATED {n3}                                                OPTIONAL,   -- Need R

srs-DLorJointTCI-State-v1730            SEQUENCE {

cellAndBWP-r17                          ServingCellAndBWP-Id-r17

}                                                                                                 OPTIONAL -- Cond DLorJointTCI-SRS

]],

[[

nrofSRS-Ports-n8-r18                    ENUMERATED {ports8, ports8tdm}                                 OPTIONAL,   -- Need R

combOffsetHopping-r18                   SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n4                       BIT STRING (SIZE (4)),

transmissionComb-n8                       BIT STRING (SIZE (8))

}                                                                                                  OPTIONAL,   -- Need R

hoppingWithRepetition-r18               ENUMERATED {symbol, repetition}                            OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

cyclicShiftHopping-r18                  SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n2                       BIT STRING (SIZE (8)),

transmissionComb-n4                       BIT STRING (SIZE (12)),

transmissionComb-n8                       BIT STRING (SIZE (6))

}                                                                                                  OPTIONAL,   -- Need R

hoppingFinerGranularity-r18             ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL    -- Need R

]]

}

SRS-PosResource-r16::=                  SEQUENCE {

srs-PosResourceId-r16                   SRS-PosResourceId-r16,

transmissionComb-r16                    CHOICE {

n2-r16                                  SEQUENCE {

combOffset-n2-r16                       INTEGER (0..1),

cyclicShift-n2-r16                      INTEGER (0..7)

},

n4-r16                                  SEQUENCE {

combOffset-n4-r16                        INTEGER (0..3),

cyclicShift-n4-r16                      INTEGER (0..11)

},

n8-r16                                  SEQUENCE {

combOffset-n8-r16                       INTEGER (0..7),

cyclicShift-n8-r16                      INTEGER (0..5)

},

...

},

resourceMapping-r16                       SEQUENCE {

startPosition-r16                           INTEGER (0..13),

nrofSymbols-r16                             ENUMERATED {n1, n2, n4, n8, n12}

},

freqDomainShift-r16                       INTEGER (0..268),

freqHopping-r16                           SEQUENCE {

c-SRS-r16                                 INTEGER (0..63),

...

},

groupOrSequenceHopping-r16                ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType-r16                          CHOICE {

aperiodic-r16                             SEQUENCE {

slotOffset-r16                            INTEGER (1..32)                                      OPTIONAL,   -- Need S

...

},

semi-persistent-r16                       SEQUENCE {

periodicityAndOffset-sp-r16               SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-sp-Ext-r16           SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]]

},

periodic-r16                              SEQUENCE {

periodicityAndOffset-p-r16                SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-p-Ext-r16            SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]]

}

},

sequenceId-r16                            INTEGER (0..65535),

spatialRelationInfoPos-r16                SRS-SpatialRelationInfoPos-r16                               OPTIONAL,   -- Need R

...,

[[

srs-PosPeriodicConfigHyperSFN-Index-r18   ENUMERATED {even0, odd1}                                     OPTIONAL,   --Need R

txHoppingConfig-r18                       TxHoppingConfig-r18                                          OPTIONAL    --Need R

]]

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                                      OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-SpatialRelationInfoPos-r16 ::=      CHOICE {

servingRS-r16                           SEQUENCE {

servingCellId                           ServCellIndex                                              OPTIONAL,   -- Need S

referenceSignal-r16                     CHOICE {

ssb-IndexServing-r16                    SSB-Index,

csi-RS-IndexServing-r16                 NZP-CSI-RS-ResourceId,

srs-SpatialRelation-r16                 SEQUENCE {

resourceSelection-r16                   CHOICE {

srs-ResourceId-r16                      SRS-ResourceId,

srs-PosResourceId-r16                   SRS-PosResourceId-r16

},

uplinkBWP-r16                           BWP-Id

}

}

},

ssb-Ncell-r16                           SSB-InfoNcell-r16,

dl-PRS-r16                              DL-PRS-Info-r16

}

SSB-Configuration-r16  ::=          SEQUENCE {

ssb-Freq-r16                     ARFCN-ValueNR,

halfFrameIndex-r16                  ENUMERATED {zero, one},

ssbSubcarrierSpacing-r16            SubcarrierSpacing,

ssb-Periodicity-r16                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 }   OPTIONAL, -- Need S

sfn0-Offset-r16                     SEQUENCE {

sfn-Offset-r16                      INTEGER (0..1023),

integerSubframeOffset-r16           INTEGER (0..9)                                                 OPTIONAL  -- Need R

}                                                                                                      OPTIONAL, -- Need R

sfn-SSB-Offset-r16                  INTEGER (0..15),

ss-PBCH-BlockPower-r16              INTEGER (-60..50)                                                  OPTIONAL  -- Cond Pathloss

}

SSB-InfoNcell-r16  ::=              SEQUENCE {

physicalCellId-r16                  PhysCellId,

ssb-IndexNcell-r16                  SSB-Index                                                          OPTIONAL, -- Need S

ssb-Configuration-r16               SSB-Configuration-r16                                              OPTIONAL  -- Need S

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PosResourceId-r16 ::=               INTEGER (0..maxNrofSRS-PosResources-1-r16)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

SRS-PeriodicityAndOffset-r16 ::=        CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559),

sl5120                                  INTEGER(0..5119),

sl10240                                 INTEGER(0..10239),

sl40960                                 INTEGER(0..40959),

sl81920                                 INTEGER(0..81919),

...

}

SRS-PeriodicityAndOffsetExt-r16 ::=     CHOICE {

sl128                                   INTEGER(0..127),

sl256                                   INTEGER(0..255),

sl512                                   INTEGER(0..511),

sl20480                                 INTEGER(0..20479)

}

SpatialRelationInfo-PDC-r17 ::=   SEQUENCE {

referenceSignal                   CHOICE {

ssb-Index                         SSB-Index,

csi-RS-Index                      NZP-CSI-RS-ResourceId,

dl-PRS-PDC                        NR-DL-PRS-ResourceID-r17,

srs                               SEQUENCE {

resourceId                        SRS-ResourceId,

uplinkBWP                         BWP-Id

},

...

},

...

}

TxHoppingConfig-r18 ::=             SEQUENCE {

overlapValue-r18                    ENUMERATED {zeroRB, oneRB, twoRB, fourRB},

numberOfHops-r18                    INTEGER(2..6),

slotOffsetForRemainingHopsList-r18  SEQUENCE (SIZE (1..maxNrofHops-r18-1) ) OF SlotOffsetForRemainingHops-r18,

...

}

SlotOffsetForRemainingHops-r18 ::=  SEQUENCE {

slotOffsetRemainingHops-r18         CHOICE {

aperiodic-r18                       SEQUENCE {

slotOffset-r18                      INTEGER (1..32)                                            OPTIONAL,   -- Need S

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

semi-persistent-r18                 SEQUENCE {

periodicityAndOffset-sp-r18         SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-sp-Ext-r18     SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

periodic-r18                        SEQUENCE {

periodicityAndOffset-p-r18          SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-p-Ext-r18      SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need S

...

},

...

}

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

dci-TriggeringPosResourceSetLink Indicates whether the single DCI-triggering SRS positioning resource sets across the linked carriers is enabled or not for bandwidth aggregation.

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) (see TS 38.213 [13], clause 7.3).

SRS-Resource, SRS-PosResource field descriptions

cyclicShift-n2 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n4 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n8 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

combOffsetHopping Configures UE with comb offset hopping. The hoppingId is used to initialize pseudo random comb offset hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingWithRepetition configures time-domain hopping behavior for repetition factor R>1. The hoppingSubset indicates a set of comb offset by a bit string (see clause 6.4.1.4.3 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scohSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

cyclicShiftHopping Configures UE with cyclic shift hopping. The hoppingId is used to initialize pseudo random cyclic shift hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingFinerGranularity enables finer granular hopping, see TS 38.211 [16], clause 6.4.1.4.2. If hoppingSubset is configured, hoppingFinerGranularity is not configured. The hoppingSubset indicates a set of cyclic shift by a bit string (see clause 6.4.1.4.2 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scshSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

enableStartRBHopping When this RRC parameter is configured, start RB location hopping is enabled for partial frequency sounding in different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS as described in clause 6.4.1.4 in TS 38.211.

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS. For SRS for positioning configuration in multiple cells this field is commonly configured across cells within the validity area. c-SRS Indicates the maximum bandwidth. When TxHoppingConfig is configured the valid values for c-SRS are such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively. The same value for c-SRS is configured for all the hops when TxHoppingConfig is configured.

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2). For CLI SRS-RSRP measurement, the network always configures this parameter to 'neither'. For SRS for positioning configuration in multiple cells this field is commonly configured across cells within the validity area.

nrofSRS-Ports Number of ports. For CLI SRS-RSRP measurement, the network always configures this parameter to 'port1'.

nrofSRS-Ports-n8 Number of ports if the number of antenna ports is 8. The value 'ports8' configures UE with 8 antenna ports and the value 'ports8tdm' configures the UE with 8 antenna ports which are partitioned into 2 subsets with each subset having 4 different ports, and the subsets are mapped to different OFDM symbols, see TS 38.211 [16], clause 6.4.1.4.2. If combOffsetHopping-r18 or cyclicShiftHopping-r18 is configured, this field is not set to ports8tdm. If this field is present UE ignores the field nrofSRS-Ports.

periodicityAndOffset-p, periodicityAndOffset-p-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, sl1280 and sl2560 cannot be configured. For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz except when periodicity of 20480ms is configured. When periodicityAndOffset-p-Ext is present, periodicityAndOffset-p shall be ignored by the UE.

periodicityAndOffset-sp, periodicityAndOffset-sp-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz. When periodicityAndOffset-sp-Ext is present, periodicityAndOffset-sp shall be ignored by the UE.

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1). This parameter is not applicable to CLI SRS-RSRP measurement.

resourceMapping OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. If resourceMapping-r16 is signalled, UE shall ignore the resourceMapping (without suffix). If resourceMapping-r17 is signalled, resourceMapping-r16 is not signalled and the UE shall ignore the resourceMapping (without suffix) and only the values of nrofSymbols which are integer multiples of the configured repetitionFactor can be configured. The network can only signal repetitionFactor-v1730 if resourceMapping-r17 is signalled. When repetitionFactor-v1730 is signalled, the UE shall ignore repetitionFactor-r17. For CLI SRS-RSRP measurement, the network always configures nrofSymbols and repetitionFactor to 'n1'. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, this field is commonly configured across cells within the validity area. nrofSymbols is same for all the hops when TxHoppingConfig is configured.

resourceType Periodicity and offset for semi-persistent and periodic SRS resource, or slot offset for aperiodic SRS resource for positioning (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, only 'periodic' is applicable for resourceType. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, this field is commonly configured across cells within the validity area.

sequenceId Sequence ID used to initialize pseudo random group and sequence hopping (see TS 38.214 [19], clause 6.2.1). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, this field is commonly configured across cells within the validity area.

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS (see TS 38.214 [19], clause 6.2.1). This parameter is not applicable to CLI SRS-RSRP measurement. This field is not configured if unifiedTCI-StateType is configured for the serving cell.

spatialRelationInfo-PDC Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS-PDC (see TS 38.214 [19], clause 6.2.1). The field is present in case of resourceType=periodic and usagePDC-r17=true in the SRS-ResourceSet, otherwise the field is absent.

spatialRelationInfoPos Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS (see TS 38.214 [19], clause 6.2.1). If the IE srs-ResourceId-Ext is present, the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 63. Otherwise the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 31.

srs-PosPeriodicConfigHyperSFN-Index Indicates whether the current Hyper SFN is even or odd HSFN for SRS for Positioning transmission. If this field is not configured, the UE assumes that SRS for positioning periodictity longer than one Hyper SFN is not configured.

srs-RequestDCI-0-2 Indicate the number of bits for "SRS request" in DCI format 0_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 0_2 is applied. If the parameter srs-RequestDCI-0-2 is configured to value 1, 1 bit is used to indicate one of the first two rows of Table 7.3.1.1.2-24 in TS 38.212 [17] for triggered aperiodic SRS resource set. If the value 2 is configured, 2 bits are used to indicate one of the rows of Table 7.3.1.1.2-24 in TS 38.212 [17]. When UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication.

srs-RequestDCI-1-2 Indicate the number of bits for "SRS request" in DCI format 1_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 1_2 is applied. When the UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication (see TS 38.214 [19], clause 6.1.1.2).

srs-ResourceSetToAddModListDCI-0-2 List of SRS resource set to be added or modified for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-ResourceSetToReleaseListDCI-0-2 List of SRS resource set to be released for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-TCI-State Configuration of either a UL TCI state or a joint TCI state for the SRS resource. In case of UL TCI-State, refers to the TCI state defined in ul-TCI-StateList in the BWP-UplinkDedicated where the SRS-Config is configured. In case of joint TCI state, refers to a TCI state defined in dl-OrJointTCI-StateList in pdsch-Config of the BWP-DownlinkDedicated and serving cell indicated by cellAndBWP. This field is absent when the SRS resource is in an SRS-ResourceSet configured with followUnifiedTCI-StateSRS-r17 or applyIndicatedTCI-State, or when the field unifiedTCI-StateType is not configured to the serving cell which the SRS resource is located in.

startRBIndexAndFreqScalingFactor Configures the UE with the startRBIndex and freqScalingFactor for partial frequency sounding as described in Clause 6.4.1.4 in TS 38.211. The startRBIndexForFScaling2 gives the startRBIndex when freqScalingFactor is 2 and the startRBIndexForFScaling4 gives the startRBIndex when FreqScalingFactor is 4

transmissionComb, transmissionComb-n2, transmissionComb-n4, transmissionComb-n8 Comb value (2 or 4 or 8) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1). If network configures field transmissionComb-n8, the UE ignores transmissionComb. If srs-PosRRC-InactiveValidityAreaPreConfig or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, this field is commonly configured across cells within the validity area.

SRS-ResourceSet, SRS-PosResourceSet field descriptions

alpha alpha value for SRS power control (see TS 38.213 [13], clause 7.3). When the field is absent the UE applies the value 1. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, this field is commonly configured across cells within the validity area.

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6). When the field is not included during a reconfiguration of SRS-ResourceSet of resourceType set to aperiodic, UE maintains this value based on the Need M; that is, this list is not considered as an extension of aperiodicSRS-ResourceTrigger for purpose of applying the general rule for extended list in clause 6.1.3.

aperiodicSRS-ResourceTrigger The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6).

applyIndicatedTCI-State This field indicates, for an SRS-ResourceSet, if UE applies the first or the second "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.2.1. If more than one value for the field coresetPoolIndex is configured in IE controlResourceSet for the BWP, the value 'first' corresponds to the "indicated" joint/UL TCI states specific to coresetPoolIndex value 0 and the value 'second' correspond to the value 1, respectively. When UE is configured with two SRS resource sets with usage set to Codebook or nonCodebook, network does not configure the first set with value 'second' or second set with value 'first'.

associatedCSI-RS ID of CSI-RS resource associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause 6.1.1.2).

availableSlotOffsetList Indicates a list of up to four different available slot offset values from slot n+k to the slot where the aperiodic SRS resource set is transmitted, where slot n is the slot with the triggering DCI, and k is the slotOffset (without suffix) as described in clause 6.2.1 of TS 38.214 [19].

csi-RS ID of CSI-RS resource associated with this SRS resource set (see TS 38.214 [19], clause 6.1.1.2).

dl-PRS This field indicates a PRS configuration.

followUnifiedTCI-StateSRS When set to enabled, for SRS resource Set, the UE applies the "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5. This parameter may be configured for aperiodic SRS for BM or SRS of any time-domain behavior for codebook, non-codebook, and antenna switching.

p0 P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed (see TS 38.213 [13], clause 7.3). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, this field is commonly configured across cells within the validity area.

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRS-Pos A reference signal (e.g. a SS block or a DL-PRS config) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRSList Multiple candidate pathloss reference RS(s) for SRS power control, where one candidate RS can be mapped to SRS Resource Set via MAC CE (clause 6.1.3.27 in TS 38.321 [3]). The network can only configure this field if pathlossReferenceRS is not configured in the same SRS-ResourceSet.

resourceType Time domain behavior of SRS resource configuration, see TS 38.214 [19], clause 6.2.1. The network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS. The aperiodic SRS is not applicable for the UE in RRC_INACTIVE. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, this field is commonly configured across cells within the validity area.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE applies no offset (value 0).

srs-PowerControlAdjustmentStates Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or separate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 (see TS 38.213 [13], clause 7.3).

srs-ResourceIdList, srs-PosResourceIdList The IDs of the SRS-Resources/SRS-PosResource used in this SRS-ResourceSet/SRS-PosResourceSet. If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceIdList is commonly configured across cells within the validity area.

srs-ResourceSetId, srs-PosResourceSetId The ID of this resource set. It is unique in the context of the BWP in which the parent SRS-Config is defined. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceSetId is commonly configured across cells within the validity area.

ssb-IndexServing Indicates SSB index belonging to a serving cell where the SRS is configured.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

usage Indicates if the SRS resource set is used for beam management, codebook based or non-codebook based transmission or antenna switching. See TS 38.214 [19], clause 6.2.1. Reconfiguration between codebook based and non-codebook based transmission is not supported.

usagePDC If configured, it indicates that this SRS resource set is used for propagation delay compensation. The field can be present in only one SRS-ResourceSet.

SRS-SpatialRelationInfoPos field descriptions

csi-RS-IndexServing Indicates CSI-RS index belonging to a serving cell.

dl-PRS This field indicates a PRS configuration.

resourceSelection Indicates whether the configured SRS spatial relation resource is a SRS-Resource or SRS-PosResource.

servingCellId The serving Cell ID of the source SSB, CSI-RS, or SRS for the spatial relation of the target SRS resource. If this field is absent the SSB, the CSI-RS, or the SRS is from the same serving cell where the SRS is configured.

ssb-IndexServing Indicates SSB index belonging to a serving cell.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

SSB-InfoNCell field descriptions

physicalCellId This field specifies the physical cell ID of the neighbour cell or NCD-SSB of the serving cell for which SSB configuration is provided.

ssb-IndexNcell This field specifies the index of the SSB for a neighbour cell or NCD-SSB of the serving cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.

ssb-Configuration This field specifies the full configuration of the SSB. If this field is absent, the UE obtains the configuration for the SSB from nr-SSB-Config received as part of DL-PRS assistance data in LPP, see TS 37.355 [49], by looking up the corresponding SSB configuration using the field physicalCellId.

DL-PRS-Info field descriptions

dl-PRS-ID This field specifies the UE specific TRP ID (see TS 37.355 [49]) for which PRS configuration is provided.

dl-PRS-ResourceSetId This field specifies the PRS-ResourceSet ID of a PRS resourceSet.

dl-PRS-ResourceId This field specifies the PRS-Resource ID of a PRS resource. If this field is absent, the UE determines the dl-PRS-ResourceID based on its PRS measurement from the TRP (see TS 37.355 [49]) and DL-PRS Resource Set.

SSB-Configuration field descriptions

halfFrameIndex Indicates whether SSB is in the first half or the second half of the frame. Value zero indicates the first half and value 1 indicates the second half.

integerSubframeOffset Indicates the subframe boundary offset of the cell in which SSB is transmited.

sfn0-Offset Indiactes the time offset of the SFN0 slot 0 for the cell with respect to SFN0 slot 0 of serving cell.

sfn-Offset Specifies the SFN offset between the cell in which SSB is transmited and serving cell. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of serving cell to the beginning of the closest subsequent radio frame #0 of the cell in which SSB is transmitted.

sfn-SSB-Offset Indicates the SFN offset of the transmitted SSB relative to the start of the SSB period. Value 0 indicates that the SSB is transmitted in the first system frame, value 1 indicates that SSB is transmitted in the second system frame and so on. The network configures this field according to the field ssb-Periodicity such that the indicated system frame does not exceed the configured SSB periodicity.

ssb-Freq Indicates the frequency of the SSB.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-Periodicity Indicates the periodicity of the SSB. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

TxHoppingConfig field descriptions

numberOfHops This field specifies the number of hops. The configured values are 2,3,4,5,6.

overlapValue This field specifies the overlap during SRS transmission in terms of number of resource blocks. Value zeroRB implies 0 RB, value oneRB corresponds to 1 RB, value twoRB corresponds to 2 RBs and so on. The same value for the overlapValue is configured to all the hops.

slotOffsetForRemainingHopsList This field specifies the starting slot offset and starting symbol for the SRS resource with tx hopping for different resource types (aperiodic, semi-persistent or periodic SRS transmission). Each hop is configured with the same periodcity.

| Conditional Presence | Explanation |
| --- | --- |
| DLorJointTCI-SRS | The field is mandatory present if srs-DLorJointTCI-State is configured, otherwise it is absent Need R. |
| FollowUTCI | The field is absent if the field followUnifiedTCI-State is present. Otherwise, it is optionally present, Need R. |
| NonCodebook | This field is optionally present, Need M, in case of non-codebook based transmission, otherwise the field is absent. |
| Pathloss | The field is mandatory present if the IE SSB-InfoNcell is included in pathlossReferenceRS-Pos; otherwise it is optionally present, Need R |
| Setup | This field is mandatory present upon configuration of SRS-ResourceSet or SRS-Resource and optionally present, Need M, otherwise. |

#### –	SRS-PosTx-Hopping

The IE SRS-PosTx-Hopping specifies the frequency hopping configuration for SRS for Positioning transmission.

SRS-PosTx-Hopping information element

-- ASN1START

-- TAG- SRS-PosTx-Hopping-START

SRS-PosTx-Hopping-r18 ::=                       SEQUENCE {

srs-PosConfig-r18                               SRS-PosConfig-r17,

bwp-r18                                         BWP                                                              OPTIONAL, -- Need R

srs-PosUplinkTransmissionWindowConfig-r18       SetupRelease { SRS-PosUplinkTransmissionWindowConfig-r18 }       OPTIONAL  -- Need M

}

SRS-PosUplinkTransmissionWindowConfig-r18 ::=   SEQUENCE {

startSFN-r18                                    INTEGER(0..1023),

windowPeriodicityAndOffset-r18                  CHOICE {

periodicityAndOffset-r18                        SRS-PeriodicityAndOffset-r16,

periodicityAndOffset-Ext-r18                    SRS-PeriodicityAndOffsetExt-r16

},

duration-r18                                    ENUMERATED {s1,s2,s4,s6},

...

}

-- TAG-SRS-PosTx-Hopping-STOP

-- ASN1STOP

SRS-PosTx-Hopping field descriptions

bwp For RRC_CONNECTED state, indicates the frequency region outside of active BWP for SRS for positioning frequency hopping. For RRC_INACTIVE state indicates the BWP configuration for SRS for Positioning during the RRC_INACTIVE.

srs-PosConfig Provides the SRS Configuration to be used for frequency hopping.

srs-PosUplinkTransmissionWindowConfig UL time window for UL SRS for positioning with Tx hopping configured to be periodic with configurable starting SFN, slot and symbol number, periodicity, duration.

SRS-PosUplinkTransmissionWindowConfig field descriptions

duration Indicates the duration of the uplink SRS for positioning transmission window with frequency hopping. Value s1 indicates 1 slot, s2 indicates 2 slot and so on.

periodicityAndOffset Indicates the Periodicity and slot offset for uplink SRS for positioning transmission window occurrence with frequency hopping.

startSFN Indicates the starting SFN of the uplink SRS for positioning transmission window with frequency hopping.

#### –	SRS-PosResourceSetLinkedForAggBW

The IE SRS-PosResourceSetLinkedForAggBW provides the SRS Positioning Resource Sets that are linked for bandwidth aggregation.

SRS-PosResourceSetLinkedForAggBW information element

-- ASN1START

-- TAG- SRS-POSRESOURCESETLINKEDFORAGGBW-START

SRS-PosResourceSetLinkedForAggBW-r18	::= SEQUENCE {

srs-PosResourceSetLinked-r18             SRS-PosResourceSetId-r16,

freqInfo-r18                             ARFCN-ValueNR                                             OPTIONAL,  -- Need R

ul-bwp-ID-r18                            BWP-Id                                                    OPTIONAL,  -- Cond ConnectedMode

scs-SpecificCarrier-r18                  SCS-SpecificCarrier                                       OPTIONAL,  -- Need R

...

}

-- TAG- SRS-POSRESOURCESETLINKEDFORAGGBW-STOP

-- ASN1STOP

SRS-PosResourceSetLinkedForAggBW field descriptions

freqInfo Indicates the SRS Positioning Resource set carrier frequency that is linked for bandwidth aggregation.

scs-SpecificCarrier A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A and to be used for SRS for positioning bandwidth aggregation.

srs-PosResourceSetLinked Indicates the SRS Positioning Resource set that is linked for bandwidth aggregation.

ul-bwp-ID Indicates the SRS Positioning Resource set uplink bandwidth ID that is linked for bandwidth aggregation.

| Conditional Presence | Explanation |
| --- | --- |
| ConnectedMode | The field is mandatory present when bandwidth aggregation is performed in RRC_CONNECTED state and SRS for positioning is configured in the same carrier as data for bandwidth aggregation, otherwise it is absent Need R. |

#### –	SRS-RSRP-Range

The IE SRS-RSRP-Range specifies the value range used in SRS-RSRP measurements and thresholds. The integer value for SRS-RSRP measurements is according to Table 10.1.22.1.2-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value –140) dBm, except for the IE value 98, in which case the actual value is infinity.

SRS-RSRP-Range information element

-- ASN1START

-- TAG-SRS-RSRP-RANGE-START

SRS-RSRP-Range-r16 ::=                      INTEGER(0..98)

-- TAG-SRS-RSRP-RANGE-STOP

-- ASN1STOP

#### –	SRS-TPC-CommandConfig

The IE SRS-TPC-CommandConfig is used to configure the UE for extracting TPC commands for SRS from a group-TPC messages on DCI.

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=               SEQUENCE {

startingBitOfFormat2-3                  INTEGER (1..31)                                                     OPTIONAL,   -- Need R

fieldTypeFormat2-3                      INTEGER (0..1)                                                      OPTIONAL,   -- Need R

...,

[[

startingBitOfFormat2-3SUL               INTEGER (1..31)                                                     OPTIONAL    -- Need R

]]

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

SRS-TPC-CommandConfig field descriptions

fieldTypeFormat2-3 The type of a field within the group DCI with SRS request fields (optional), which indicates how many bits in the field are for SRS request (0 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 38.212 [17] clause 7.3.1 and , TS 38.213 [13], clause 11.4.

startingBitOfFormat2-3 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4).

startingBitOfFormat2-3SUL The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands for SUL carrier (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4).

#### –	SSB-Index

The IE SSB-Index identifies an SS-Block within an SS-Burst. See TS 38.213 [13], clause 4.1.

SSB-Index information element

-- ASN1START

-- TAG-SSB-INDEX-START

SSB-Index ::=                       INTEGER (0..maxNrofSSBs-1)

-- TAG-SSB-INDEX-STOP

-- ASN1STOP

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC-AdditionalPCI-r17 ::=       SEQUENCE {

additionalPCIIndex-r17              AdditionalPCIIndex-r17,

additionalPCI-r17                   PhysCellId,

periodicity-r17                     ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

ssb-PositionsInBurst-r17            CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

},

ss-PBCH-BlockPower-r17              INTEGER (-60..50)

}

AdditionalPCIIndex-r17  ::=  INTEGER(1..maxNrofAdditionalPCI-r17)

-- TAG-SSB-MTC-STOP

-- ASN1STOP

SSB-MTC field descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see TS 38.213 [13], clause 4.1).

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Periodicity and offset are given in number of subframes.

SSB-MTC2 field descriptions

pci-List PCIs that follow this SMTC.

SSB-MTC3 field descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see TS 38.213 [13], clause 4.1).

pci-List PCIs that follow this SMTC, used for IAB-node discovery.

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Periodicity and offset are given in number of subframes.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS block index 0, the second bit corresponds to SS block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS block is not to be measured while value 1 indicates that the corresponding SS block is to be measured (see TS 38.215 [9]). When the field is not configured the IAB-MT measures on all SS blocks. Regardless of the value of this field, SS blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

SSB-MTC4 field descriptions

pci-List PCIs that follow this SMTC.

offset Offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Offset is given in number of subframes.

SSB-MTC-AdditionalPCI field descriptions

additionalPCI PCI of the additional SSB different from serving cell PCI.

periodicity Periodicity of the SS/PBCH blocks, see 5.5.2.10. Periodicity is given in number of subframes.

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

#### –	SSB-PositionQCL-Relation

The IE SSB-PositionQCL-Relation is used to indicate the QCL relationship between SSB positions on the frequency indicated by ssbFrequency (see TS 38.213 [13], clause 4.1) for operation with shared spectrum channel access. Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

SSB-PositionQCL-Relation information element

-- ASN1START

-- TAG-SSB-POSITIONQCL-RELATION-START

SSB-PositionQCL-Relation-r16 ::=  ENUMERATED {n1,n2,n4,n8}

SSB-PositionQCL-Relation-r17 ::=  ENUMERATED {n32, n64}

-- TAG-SSB-POSITIONQCL-RELATION-STOP

-- ASN1STOP

#### –	SSB-ToMeasure

The IE SSB-ToMeasure is used to configure a pattern of SSBs. For operation with shared spectrum channel access in FR1, only mediumBitmap is used, and for FR2-2, longBitmap is used.

SSB-ToMeasure information element

-- ASN1START

-- TAG-SSB-TOMEASURE-START

SSB-ToMeasure ::=                   CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}

-- TAG-SSB-TOMEASURE-STOP

-- ASN1STOP

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR2-2, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to (k– 1) may be transmitted; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The k-th bit is set to 0, where k > ssb-PositionQCL-Common and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap. If ssb-PositionQCL is configured with a value smaller than ssb-PositionQCL-Common, only the leftmost K bits (K = ssb-PositionQCL) are applicable for the corresponding cell.

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.

#### –	SS-RSSI-Measurement

The IE SS-RSSI-Measurement is used to configure RSSI measurements based on synchronization reference signals.

SS-RSSI-Measurement information element

-- ASN1START

-- TAG-SS-RSSI-MEASUREMENT-START

SS-RSSI-Measurement ::=             SEQUENCE {

measurementSlots                    BIT STRING (SIZE (1..80)),

endSymbol                           INTEGER(0..3)

}

-- TAG-SS-RSSI-MEASUREMENT-STOP

-- ASN1STOP

SS-RSSI-Measurement field descriptions

endSymbol Within a slot that is configured for RSSI measurements (see measurementSlots) the UE measures the RSSI from symbol 0 to symbol endSymbol. This field identifies the entry in Table 5.1.3-1 in TS 38.215 [9], which determines the actual end symbol.

measurementSlots Indicates the slots in which the UE can perform RSSI measurements. The length of the BIT STRING is equal to the number of slots in the configured SMTC window (determined by the duration and by the subcarrierSpacing). The first (left-most / most significant) bit in the bitmap corresponds to the first slot in the SMTC window, the second bit in the bitmap corresponds to the second slot in the SMTC window, and so on. The UE measures in slots for which the corresponding bit in the bitmap is set to 1. In case this field is configured for a SCell with ca-SlotOffset-r16, the bits in the bitmap corresponds to the slots that are fully contained in the SMTC window.

#### –	SubcarrierSpacing

The IE SubcarrierSpacing determines the subcarrier spacing. Restrictions applicable for certain frequencies, channels or signals are clarified in the fields that use this IE.

SubcarrierSpacing information element

-- ASN1START

-- TAG-SUBCARRIERSPACING-START

SubcarrierSpacing ::=               ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, kHz480-v1700, kHz960-v1700, spare1}

-- TAG-SUBCARRIERSPACING-STOP

-- ASN1STOP

#### –	TAG-Config

The IE TAG-Config is used to configure parameters for a time-alignment group.

TAG-Config information element

-- ASN1START

-- TAG-TAG-CONFIG-START

TAG-Config ::=                      SEQUENCE {

tag-ToReleaseList                   SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG-Id                          OPTIONAL,   -- Need N

tag-ToAddModList                    SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG                             OPTIONAL    -- Need N

}

TAG ::=                             SEQUENCE {

tag-Id                              TAG-Id,

timeAlignmentTimer                  TimeAlignmentTimer,

...

}

TAG-Id ::=                          INTEGER (0..maxNrofTAGs-1)

-- TAG-TAG-CONFIG-STOP

-- ASN1STOP

TAG field descriptions

tag-Id Indicates the TAG of the SpCell or an SCell, see TS 38.321 [3]. Uniquely identifies the TAG within the scope of a Cell Group (i.e. MCG or SCG).

timeAlignmentTimer The timeAlignmentTimer for TAG with ID tag-Id, as specified in TS 38.321 [3].

#### –	TAR-Config

The IE TAR-Config is used to configure Timing Advance reporting in non-terrestrial networks and ATG network.

TAR-Config information element

-- ASN1START

-- TAG-TAR-CONFIG-START

TAR-Config-r17 ::=                      SEQUENCE {

offsetThresholdTA-r17               ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12,

ms13, ms14, ms15, spare13, spare12, spare11, spare10, spare9, spare8, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}          OPTIONAL,    -- Need R

timingAdvanceSR-r17                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

TAR-Config-r18 ::=                      SEQUENCE {

offsetThresholdTA-r18               INTEGER (1..56)                                                     OPTIONAL,    -- Need R

timingAdvanceSR-r18                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For ATG, network only configures offsetThresholdTA-r18, which is in unit of symbols.

timingAdvanceSR Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

#### –	TCI-ActivatedConfig

The IE TCI-ActivatedConfig is used to provide activated TCI states for PDSCH and/or PDCCH of the PSCell or of an SCell.

TCI-ActivatedConfig information element

-- ASN1START

-- TAG-TCI-ACTIVATEDCONFIG-START

TCI-ActivatedConfig-r17 ::= SEQUENCE {

pdcch-TCI-r17               SEQUENCE (SIZE (1..5)) OF TCI-StateId,

pdsch-TCI-r17               BIT STRING (SIZE (1..maxNrofTCI-States))

}

-- TAG-TCI-ACTIVATEDCONFIG-STOP

-- ASN1STOP

TCI-ActivatedConfig field descriptions

pdcch-TCI Indicates the TCI state for PDCCH for each configured CORESET of the DL BWP to be activated at SCell activation, to be activated for the PSCell at SCG activation and/or to be used for BFD, RLM and measurements while the SCG is deactivated. The list includes exactly as many entries as CORESETs configured in this BWP, ordered by increasing values of ControlResourceSet-Id, i.e. the first entry indicates the TCI state for the configured CORESET with the lowest ControlResourceset-Id value, the second value indicates the TCI states for the configured CORESET with the second lowest ControlResourceset-Id value, and so on.

pdsch-TCI Indicates TCI states for PDSCH reception at SCell addition/activation or of the PSCell at SCG activation. This field indicates activated TCI state(s) for this BWP ordered by increasing values of TCI-StateId, i.e. the first bit indicates the activation state of the TCI state with the lowest TCI-StateId value, the second value indicates the activation status of the TCI state with the second lowest TCI-State-Id value, and so on. A bit set to 0 indicates that the corresponding TCI state is deactivated, a bit set to 1 indicates that the TCI state is activated.

#### –	TCI-State

The IE TCI-State associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                    OPTIONAL,   -- Need R

...,

[[

additionalPCI-r17                   AdditionalPCIIndex-r17                                      OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17          PathlossReferenceRS-Id-r17                                  OPTIONAL,   -- Cond JointTCI1

ul-powerControl-r17                 Uplink-powerControlId-r17                                   OPTIONAL    -- Cond JointTCI

]],

[[

tag-Id-ptr-r18                      ENUMERATED {n0,n1}                                          OPTIONAL    -- Cond 2TA

]]

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                                               OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                                                      OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

QCL-Info field descriptions

bwp-Id The DL BWP which the RS is located in. If the field is absent, the RS is located in the DL BWP in which the TCI-State is applied by the UE.

cell The UE's serving cell in which the referenceSignal is configured. If the field is absent, the referenceSignal is configured in the serving cell in which the TCI-State is applied by the UE. The RS can be located on a serving cell other than the serving cell for which the TCI-State is applied by the UE only if the qcl-Type is configured as typeC or typeD. If the referenceSignal is set to csi-rs and unifiedTCI-StateType is configured, either both cell and bwp-Id are present or both cell and bwp-Id are absent. See TS 38.214 [19] clause 5.1.5.

referenceSignal Reference signal with which quasi-collocation information is provided as specified in TS 38.214 [19] clause 5.1.5.

qcl-Type QCL type as specified in TS 38.214 [19] clause 5.1.5.

TCI-State field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs when referenceSignal is configured as SSB for both QCL-Type1 and QCL-Type2. In case the cell is present, the additionalPCI refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell indicated by the field cell. Otherwise, it refers to a PCI value configured in a list additionalPCI-ToAddModList configured in the serving cell where the TCI-State is applied by the UE. When this field is present the cell for qcl-Type1 and qcl-Type2 is configured with same value, if present.

pathlossReferenceRS-Id The ID of the reference signal (e.g. a CSI-RS or an SS block) used for PUSCH, PUCCH and SRS path loss estimation. This field refers to an element in the list configured using pathlossReferenceRSToAddModList in the serving cell and UL BWP where the TCI State is applied by the UE.

qcl-Type1, qcl-Type2 QCL information for the TCI state as specified in TS 38.214 [19] clause 5.1.5.

tag-Id-ptr It indicates the TAG that is associated with this TCI state, value n0 means the TCI state associate with the TAG indicated by tag-Id, value n1 means this TCI state associated with the TAG indicated by tag2-Id. The tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied.

tci-StateId ID number of the TCI state.

ul-PowerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the serving cell where the dl-OrJointTCI-StateToAddModList is configured.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present for the serving cell and the serving cell is configured with unifiedTCI-StateType set to joint. It is absent, Need R, otherwise. |
| CSI-RS-Indicated | This field is mandatory present if csi-rs is included and unifiedTCI-StateType is not configured. This field is optionally present, Need R, if csi-rs is included and unifiedTCI-StateType is configured. Otherwise, it is absent, Need R. |
| JointTCI | This field is optionally present, Need R, if this serving cell is configured with unifiedTCI-StateType set to 'joint'. It is absent, Need R, otherwise. |
| JointTCI1 | This field is mandatory present, if this serving cell is configured with unifiedTCI-StateType set to 'joint'. It is absent, Need R, otherwise. |

#### –	TCI-StateId

The IE TCI-StateId is used to identify one TCI-State configuration.

TCI-StateId information element

-- ASN1START

-- TAG-TCI-STATEID-START

TCI-StateId ::=                     INTEGER (0..maxNrofTCI-States-1)

-- TAG-TCI-STATEID-STOP

-- ASN1STOP

#### –	TCI-UL-State

The IE TCI-UL-State indicates the TCI state information for UL transmission.

TCI-UL-State information element

-- ASN1START

-- TAG-TCI-UL-STATE-START

TCI-UL-State-r17 ::=             SEQUENCE {

tci-UL-StateId-r17              TCI-UL-StateId-r17,

servingCellId-r17                ServCellIndex                                         OPTIONAL,   -- Need R

bwp-Id-r17                       BWP-Id                                                OPTIONAL,   -- Cond CSI-RSorSRS-Indicated

referenceSignal-r17              CHOICE {

ssb-Index-r17                    SSB-Index,

csi-RS-Index-r17                 NZP-CSI-RS-ResourceId,

srs-r17                          SRS-ResourceId

},

additionalPCI-r17                AdditionalPCIIndex-r17                                OPTIONAL,   -- Need R

ul-powerControl-r17              Uplink-powerControlId-r17                             OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17       PathlossReferenceRS-Id-r17                            OPTIONAL,   -- Cond Mandatory

...,

[[

tag-Id-ptr-r18                   ENUMERATED {n0,n1}                                    OPTIONAL    -- Cond 2TA

]]

}

-- TAG-TCI-UL-STATE-STOP

-- ASN1STOP

TCI-UL-State field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs when referenceSignal is configured as SSB. In case the servingCellId is present, the additionalPCI refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell indicated by the field servingCellId. Otherwise, it refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell where the ul-TCI-StateList is applied by the UE.

bwp-Id The DL BWP which the CSI-RS is located in or UL BWP where the SRS is located in.

servingCellId The UE's serving cell in which the referenceSignal is configured. If the field is absent, the referenceSignal is configured in the serving cell in which the TCI-UL-State is applied by the UE.

pathlossReferenceRS-Id The ID of the reference Signal (e.g. a CSI-RS or a SS block) used for PUSCH, PUCCH and SRS path loss estimation. This field refers to an element in the list configured using pathlossReferenceRSToAddModList in the serving cell and UL BWP where the UL TCI State is applied by the UE.

tag-Id-ptr It indicates the TAG that is associated with this TCI state, value n0 means the TCI state associate with the TAG indicated by tag-Id, value n1 means this TCI state associated with the TAG indicated by tag2-Id. The tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied.

ul-powerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the serving cell where the ul-TCI-ToAddModList is configured.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present for the serving cell. It is absent, Need R, otherwise. |
| CSI-RSorSRS-Indicated | This field is mandatory present if referenceSignal is set to csi-RS-index or to srs, absent otherwise |
| Mandatory | The field is mandatory present. |

#### –	TCI-UL-StateId

The IE TCI-UL-StateId is used to identify one TCI-UL-State configuration.

TCI-UL-StateId information element

-- ASN1START

-- TAG-TCI-UL-STATEID-START

TCI-UL-StateId-r17 ::=              INTEGER (0..maxUL-TCI-1-r17)

-- TAG-TCI-UL-STATEID-STOP

-- ASN1STOP

#### –	TDD-UL-DL-ConfigCommon

The IE TDD-UL-DL-ConfigCommon determines the cell specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigCommon information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGCOMMON-START

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                                                       OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

[[

dl-UL-TransmissionPeriodicity-v1530     ENUMERATED {ms3, ms4}                                               OPTIONAL -- Need R

]]

}

-- TAG-TDD-UL-DL-CONFIGCOMMON-STOP

-- ASN1STOP

TDD-UL-DL-ConfigCommon field descriptions

referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz The network configures a not larger than any SCS of configured BWPs for the serving cell. The network or SL-PreconfigGeneral configures a not larger than the SCS of (pre-)configured SL BWP.See TS 38.213 [13], clause 11.1.

TDD-UL-DL-Pattern field descriptions

dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern, see TS 38.213 [13], clause 11.1. If the dl-UL-TransmissionPeriodicity-v1530 is signalled, UE shall ignore the dl-UL-TransmissionPeriodicity (without suffix).

nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 320.

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see TS 38.213 [13], clause 11.1).

nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 320.

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see TS 38.213 [13], clause 11.1).

#### –	TDD-UL-DL-ConfigDedicated

The IE TDD-UL-DL-ConfigDedicated determines the UE-specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigDedicated information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGDEDICATED-START

TDD-UL-DL-ConfigDedicated ::=       SEQUENCE {

slotSpecificConfigurationsToAddModList      SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig       OPTIONAL, -- Need N

slotSpecificConfigurationsToReleaseList     SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex        OPTIONAL, -- Need N

...

}

TDD-UL-DL-ConfigDedicated-IAB-MT-r16::=         SEQUENCE {

slotSpecificConfigurationsToAddModList-IAB-MT-r16  SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig-IAB-MT-r16      OPTIONAL, -- Need N

slotSpecificConfigurationsToReleaseList-IAB-MT-r16 SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex                  OPTIONAL, -- Need N

...

}

TDD-UL-DL-SlotConfig ::=            SEQUENCE {

slotIndex                           TDD-UL-DL-SlotIndex,

symbols                             CHOICE {

allDownlink                         NULL,

allUplink                           NULL,

explicit                            SEQUENCE {

nrofDownlinkSymbols                 INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL, -- Need S

nrofUplinkSymbols                   INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL  -- Need S

}

}

}

TDD-UL-DL-SlotConfig-IAB-MT-r16::=    SEQUENCE {

slotIndex-r16                           TDD-UL-DL-SlotIndex,

symbols-IAB-MT-r16                      CHOICE {

allDownlink-r16                         NULL,

allUplink-r16                           NULL,

explicit-r16                            SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)                               OPTIONAL, -- Need S

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)                               OPTIONAL  -- Need S

},

explicit-IAB-MT-r16                     SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)                               OPTIONAL, -- Need S

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)                               OPTIONAL  -- Need S

}

}

}

TDD-UL-DL-SlotIndex ::=             INTEGER (0..maxNrofSlots-1)

-- TAG-TDD-UL-DL-CONFIGDEDICATED-STOP

-- ASN1STOP

TDD-UL-DL-ConfigDedicated field descriptions

slotSpecificConfigurationsToAddModList The slotSpecificConfigurationToAddModList allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

TDD-UL-DL-ConfigDedicated-IAB-MT field descriptions

slotSpecificConfigurationsToAddModList-IAB-MT The slotSpecificConfigurationToAddModList-IAB-MT allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon with a limitation that effectively only flexible symbols can be overwritten in Rel-16.

slotSpecificConfigurationsToReleaseList-IAB-MT The slotSpecificConfigurationsToReleaseList-IAB-MT allows release of a set of slot configuration previously add with slotSpecificConfigurationToAddModList-IAB-MT.

TDD-UL-DL-SlotConfig field descriptions

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot identified by slotIndex. If the field is absent the UE assumes that there are no leading DL symbols. (see TS 38.213 [13], clause 11.1).

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot identified by slotIndex. If the field is absent the UE assumes that there are no trailing UL symbols. (see TS 38.213 [13], clause 11.1).

slotIndex Identifies a slot within a slot configuration period given in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

symbols The direction (downlink or uplink) for the symbols in this slot. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively.

TDD-UL-DL-SlotConfig-IAB-MT field descriptions

symbols-IAB-MT The symbols-IAB-MT is used to configure an IAB-MT with the SlotConfig applicable for one serving cell. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively; value explicit-IAB-MT indicates explicitly how many symbols in the beginning and end of this slot are allocated to uplink and downlink, respectively.

#### –	TrackingAreaCode

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN/SNPN, see TS 24.501 [23].

TrackingAreaCode information element

-- ASN1START

-- TAG-TRACKINGAREACODE-START

TrackingAreaCode ::= BIT STRING (SIZE (24))

-- TAG-TRACKINGAREACODE-STOP

-- ASN1STOP

#### –	T-Reselection

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA. Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselection information element

-- ASN1START

-- TAG-TRESELECTION-START

T-Reselection ::=                   INTEGER (0..7)

-- TAG-TRESELECTION-STOP

-- ASN1STOP

#### –	TimeAlignmentTimer

The IE TimeAlignmentTimer is used to configure the time alignment timer as specified in TS 38.321 [3]. The values are in ms.

TimeAlignmentTimer information element

-- ASN1START

-- TAG-TIMEALIGNMENTTIMER-START

TimeAlignmentTimer ::=              ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity}

-- TAG-TIMEALIGNMENTTIMER-STOP

-- ASN1STOP

#### –	TimeToTrigger

The IE TimeToTrigger specifies the value range used for time to trigger parameter, which concerns the time during which specific criteria for the event needs to be met in order to trigger a measurement report. Value ms0 corresponds to 0 ms and behaviour as specified in 7.1.2 applies, value ms40 corresponds to 40 ms, and so on.

TimeToTrigger information element

-- ASN1START

-- TAG-TIMETOTRIGGER-START

TimeToTrigger ::=                   ENUMERATED {

ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256,

ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560,

ms5120}

-- TAG-TIMETOTRIGGER-STOP

-- ASN1STOP

#### –	TN-AreaId

The IE TN-AreaId is used to identify a TN coverage area in an NTN system.

TN-AreaId information element

-- ASN1START

-- TAG-TN-AREAID-START

TN-AreaId-r18 ::=                  INTEGER (1..maxTN-AreaInfo-r18)

-- TAG-TN-AREAID-STOP

-- ASN1STOP

#### –	UAC-BarringInfoSetIndex

The IE UAC-BarringInfoSetIndex provides the index of the entry in uac-BarringInfoSetList. Value 1 corresponds to the first entry in uac-BarringInfoSetList, value 2 corresponds to the second entry in this list and so on. An index value referring to an entry not included in uac-BarringInfoSetList indicates no barring.

UAC-BarringInfoSetIndex information element

-- ASN1START

-- TAG-UAC-BARRINGINFOSETINDEX-START

UAC-BarringInfoSetIndex ::=                INTEGER (1..maxBarringInfoSet)

-- TAG-UAC-BARRINGINFOSETINDEX-STOP

-- ASN1STOP

#### –	UAC-BarringInfoSetList

The IE UAC-BarringInfoSetList provides a list of access control parameter sets. An access category can be configured with access parameters according to one of the sets.

UAC-BarringInfoSetList information element

-- ASN1START

-- TAG-UAC-BARRINGINFOSETLIST-START

UAC-BarringInfoSetList ::=          SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet

UAC-BarringInfoSetList-v1700 ::= SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet-v1700

UAC-BarringInfoSet ::=              SEQUENCE {

uac-BarringFactor                   ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime                     ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

uac-BarringForAccessIdentity        BIT STRING (SIZE(7))

}

UAC-BarringInfoSet-v1700 ::= SEQUENCE {

uac-BarringFactorForAI3-r17  ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40, p50, p60, p70, p75, p80, p85, p90, p95}

OPTIONAL  -- Need S

}

-- TAG-UAC-BARRINGINFOSETLIST-STOP

-- ASN1STOP

UAC-BarringInfoSetList field descriptions

uac-BarringInfoSetList List of access control parameter sets. Each access category can be configured with access parameters corresponding to a particular set by uac-barringInfoSetIndex. Association of an access category with an index that has no corresponding entry in the uac-BarringInfoSetList is valid configuration and indicates no barring.

uac-BarringForAccessIdentity Indicates whether access attempt is allowed for each Access Identity. The leftmost bit, bit 0 in the bit string corresponds to Access Identity 1, bit 1 in the bit string corresponds to Access Identity 2, bit 2 in the bit string corresponds to Access Identity 11, bit 3 in the bit string corresponds to Access Identity 12, bit 4 in the bit string corresponds to Access Identity 13, bit 5 in the bit string corresponds to Access Identity 14, and bit 6 in the bit string corresponds to Access Identity 15. Value 0 means that access attempt is allowed for the corresponding access identity.

uac-BarringFactor Represents the probability that access attempt would be allowed during access barring check.

uac-BarringFactorForAI3 Barring factor applicable for Access Identity 3. Represents the probability that access attempt would be allowed during access barring check. If absent, the UE considers the access attempt as allowed.

uac-BarringTime The average time in seconds before a new access attempt is to be performed after an access attempt was barred at access barring check for the same access category, see 5.3.14.5.

#### –	UAC-BarringPerCatList

The IE UAC-BarringPerCatList provides access control parameters for a list of access categories.

UAC-BarringPerCatList information element

-- ASN1START

-- TAG-UAC-BARRINGPERCATLIST-START

UAC-BarringPerCatList ::=           SEQUENCE (SIZE (1..maxAccessCat-1)) OF UAC-BarringPerCat

UAC-BarringPerCat ::=               SEQUENCE {

accessCategory                       INTEGER (1..maxAccessCat-1),

uac-barringInfoSetIndex              UAC-BarringInfoSetIndex

}

-- TAG-UAC-BARRINGPERCATLIST-STOP

-- ASN1STOP

UAC-BarringPerCatList field descriptions

accessCategory The Access Category according to TS 22.261 [25].

#### –	UAC-BarringPerPLMN-List

The IE UAC-BarringPerPLMN-List provides access category specific access control parameters, which are configured per PLMN/SNPN.

UAC-BarringPerPLMN-List information element

-- ASN1START

-- TAG-UAC-BARRINGPERPLMN-LIST-START

UAC-BarringPerPLMN-List ::=         SEQUENCE (SIZE (1.. maxPLMN)) OF UAC-BarringPerPLMN

UAC-BarringPerPLMN ::=              SEQUENCE {

plmn-IdentityIndex                  INTEGER (1..maxPLMN),

uac-ACBarringListType               CHOICE{

uac-ImplicitACBarringList           SEQUENCE (SIZE(maxAccessCat-1)) OF UAC-BarringInfoSetIndex,

uac-ExplicitACBarringList           UAC-BarringPerCatList

}                                                                                                     OPTIONAL     -- Need S

}

-- TAG-UAC-BARRINGPERPLMN-LIST-STOP

-- ASN1STOP

UAC-BarringPerPLMN-List field descriptions

uac-ACBarringListType Access control parameters for each access category valid only for a specific PLMN or SNPN. UE behaviour upon absence of this field is specified in clause 5.3.14.2.

plmn-IdentityIndex Index of the PLMN or SNPN across the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1.

#### –	UE-TimersAndConstants

The IE UE-TimersAndConstants contains timers and constants used by the UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstants information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTS-START

UE-TimersAndConstants ::=           SEQUENCE {

t300                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t301                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

t319                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

...

}

-- TAG-UE-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

#### –	UE-TimersAndConstantsRemoteUE

The IE UE-TimersAndConstantsRemoteUE contains timers and constants used by the L2 U2N Remote UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstantsRemoteUE information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-START

UE-TimersAndConstantsRemoteUE-r17 ::= SEQUENCE {

t300-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t301-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t319-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

...

}

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-STOP

-- ASN1STOP

UE-TimersAndConstantsRemoteUE field descriptions

t300-RemoteUE Indicates the timer value of T300 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t300 applies to L2 U2N Remote UE.

t301-RemoteUE Indicates the timer value of T301 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t301 applies to L2 U2N Remote UE.

t319-RemoteUE Indicates the timer value of T319 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t319 applies to L2 U2N Remote UE.

#### –	UL-DelayValueConfig

The IE UL-DelayValueConfig specifies the configuration of the UL PDCP Packet Delay value per DRB measurement specified in TS 38.314 [53].

UL-DelayValueConfig information element

-- ASN1START

-- TAG-ULDELAYVALUECONFIG-START

UL-DelayValueConfig-r16 ::=  SEQUENCE {

delay-DRBlist-r16            SEQUENCE (SIZE(1..maxDRB)) OF DRB-Identity

}

-- TAG-ULDELAYVALUECONFIG-STOP

-- ASN1STOP

UL-DelayValueConfig field descriptions

delay-DRBlist Indicates the DRB IDs used by UE to provide results of UL PDCP Packet Delay value per DRB measurement as specified in TS 38.314 [53].

#### –	UL-ExcessDelayConfig

The IE UL-ExcessDelayConfig IE specifies the configuration of the UL PDCP Excess Packet Delay per DRB measurement specified in TS 38.314 [53].

UL-ExcessDelayConfig information element

-- ASN1START

-- TAG-ULEXCESSDELAYCONFIG-START

UL-ExcessDelayConfig-r17 ::=  SEQUENCE {

excessDelay-DRBlist-r17       SEQUENCE (SIZE(1..maxDRB)) OF ExcessDelay-DRB-IdentityInfo-r17

}

ExcessDelay-DRB-IdentityInfo-r17 ::=  SEQUENCE {

drb-IdentityList                      SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity,

delayThreshold                        ENUMERATED {ms0dot25, ms0dot5, ms1, ms2, ms4, ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70,

ms80, ms90, ms100, ms150, ms300, ms500}

}

-- TAG-ULEXCESSDELAYCONFIG-STOP

-- ASN1STOP

UL-ExcessDelayConfig field descriptions

drb-IdentityList Indicates the DRB IDs used by UE to provide results of UL PDCP Excess Packet Delay per DRB measurement as specified in TS 38.314 [53].

delayThreshold Indicates the delay threshold for the DRB IDs indicated in DRB-IdentityList. Value ms0dot25 corresponds to 0.25ms, ms0dot5 corresponds to 0.5ms, ms1 corresponds to 1ms and so on.

#### –	UL-GapFR2-Config

The IE UL-GapFR2-Config specifies the FR2 uplink gap configuration.

UL-GapFR2-Config information element

-- ASN1START

-- TAG-UL-GAPFR2-CONFIG-START

UL-GapFR2-Config-r17 ::=      SEQUENCE {

gapOffset-r17                 INTEGER (0..159),

ugl-r17                       ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms1},

ugrp-r17                      ENUMERATED {ms5, ms20, ms40, ms160},

refFR2-ServCellAsyncCA-r17    ServCellIndex                                                      OPTIONAL -- Cond AsyncCA

}

-- TAG-UL-GAPFR2-CONFIG-STOP

-- ASN1STOP

UL-GapFR2-Config field descriptions

gapOffset Value gapOffset is the gap offset of the FR2 UL gap pattern with UGRP indicated in the field ugrp. The value range is from 0 to ugrp-1.

refFR2-ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 UL gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

ugl Value ugl is the gap length in ms of the FR2 UL gap. The FR2 UL gap length is according to Table 9.1.11-1 in TS 38.133 [14]. Value ms0dot125 corresponds to 0.125 ms, ms0dot25 corresponds to 0.25 ms and so on.

ugrp Value ugrp is the gap repetition period in (ms) of the FR2 UL gap. The FR2 UL gap repetition period is according to Table 9.1.11-1 in TS 38.133 [14].

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring FR2 UL gap pattern to UE in: - (NG)EN-DC, NR SA, NE-DC or NR-DC without FR2-FR2 band combination, with asynchronous CA involving FR2 carriers. Otherwise, it is absent. |

#### –	UplinkCancellation

The IE UplinkCancellation is used to configure the UE to monitor PDCCH for the CI-RNTI.

UplinkCancellation information element

-- ASN1START

-- TAG-UPLINKCANCELLATION-START

UplinkCancellation-r16 ::=           SEQUENCE {

ci-RNTI-r16                          RNTI-Value,

dci-PayloadSizeForCI-r16             INTEGER (0..maxCI-DCI-PayloadSize-r16),

ci-ConfigurationPerServingCell-r16   SEQUENCE (SIZE (1..maxNrofServingCells)) OF CI-ConfigurationPerServingCell-r16,

...

}

CI-ConfigurationPerServingCell-r16 ::=   SEQUENCE {

servingCellId                            ServCellIndex,

positionInDCI-r16                        INTEGER (0..maxCI-DCI-PayloadSize-1-r16),

positionInDCI-ForSUL-r16                 INTEGER (0..maxCI-DCI-PayloadSize-1-r16)                             OPTIONAL,   -- Cond SUL-Only

ci-PayloadSize-r16                       ENUMERATED {n1, n2, n4, n5, n7, n8, n10, n14, n16, n20, n28, n32, n35, n42, n56, n112},

timeFrequencyRegion-r16                  SEQUENCE {

timeDurationForCI-r16                    ENUMERATED {n2, n4, n7, n14}                                      OPTIONAL,   -- Cond SymbolPeriodicity

timeGranularityForCI-r16                 ENUMERATED {n1, n2, n4, n7, n14, n28},

frequencyRegionForCI-r16                 INTEGER (0..37949),

deltaOffset-r16                          INTEGER (0..2),

...

},

uplinkCancellationPriority-v1610         ENUMERATED {enabled}                                                  OPTIONAL    -- Need S

}

-- TAG-UPLINKCANCELLATION-STOP

-- ASN1STOP

UplinkCancellation field descriptions

ci-ConfigurationPerServingCell Indicates (per serving cell) the position of the ci-PaylaodSize bit CI values inside the DCI payload (see TS 38.213 [13], clause 11.2A).

ci-RNTI RNTI used for indication cancellation in UL (see TS 38.212 [17] clause 7.3.1 and TS 38.213 [13], clause 11.2A).

dci-PayloadSizeForCI Total length of the DCI payload scrambled with CI-RNTI (see TS 38.213 [13], clause 11.2A).

CI-ConfigurationPerServingCell field descriptions

ci-PayloadSize Configures the field size for each UL cancelation indicator of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A).

deltaOffset Configures the additional offset from the end of a PDCCH reception where the UE detects the DCI format 2_4 and the first symbol of the T_"CI" symbols, in the unit of OFDM symbols (see TS 38.213 [13], clause 11.2A).

frequencyRegionForCI Configures the reference frequency region where a detected UL CI is applicable (see TS 38.213 [13], clause 11.2A). It is defined in the same way as locationAndBandwidth.

positionInDCI Starting position (in number of bit) of the ci-PayloadSize bit CI value applicable for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.2A).

positionInDCI-ForSUL Starting position (in number of bit) of the ci-PayloadSize bit CI value applicable for SUL of this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.2A).

timeDurationForCI Configures the duration of the reference time region in symbols where a detected UL CI is applicable of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A). If the field is absent, i.e., the configured UL CI monitoring periodicity indicated by monitoringSlotPeriodicityAndOffset for DCI format 2_4 is larger than 1 slot or 1 slot with only one monitoring occasion, the UE applies the value of the configured UL CI monitoring periodicity,

timeFrequencyRegion Configures the reference time and frequency region where a detected UL CI is applicable of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A).

timeGranularityForCI Configures the number of partitions within the time region of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A).

uplinkCancellationPriority Configures uplink cancellation behavior if both UL CI and intra-UE priority indicator are configured for a given UE. If the field is present, then UL CI is only applicable to the UL transmissions indicated/configured as low priority level. If the field is absent, UL CI is applicable to UL transmission irrespective of its priority level (see TS 38.213 [13], clause 11.2A).

| Conditional Presence | Explanation |
| --- | --- |
| SUL-Only | The field is optionally present, Need R, if supplementaryUplink is configured in ServingCellConfig. It is absent otherwise. |
| SymbolPeriodicity | This field is mandatory present if the configured UL CI monitoring periodicity indicated by monitoringSlotPeriodicityAndOffset for DCI format 2_4 is 1 slot with more than one monitoring occasion, otherwise absent. |

#### –	UplinkConfigCommon

The IE UplinkConfigCommon provides common uplink parameters of a cell.

UplinkConfigCommon information element

-- ASN1START

-- TAG-UPLINKCONFIGCOMMON-START

UplinkConfigCommon ::=              SEQUENCE {

frequencyInfoUL                     FrequencyInfoUL                                 OPTIONAL,   -- Cond InterFreqHOAndServCellAdd

initialUplinkBWP                    BWP-UplinkCommon                                OPTIONAL,   -- Cond ServCellAdd

dummy                               TimeAlignmentTimer

}

UplinkConfigCommon-v1700 ::=        SEQUENCE {

initialUplinkBWP-RedCap-r17         BWP-UplinkCommon                                OPTIONAL    -- Need R

}

-- TAG-UPLINKCONFIGCOMMON-STOP

-- ASN1STOP

UplinkConfigCommon field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

initialUplinkBWP The initial uplink BWP configuration for a serving cell (see TS 38.213 [13], clause 12).

initialUplinkBWP-RedCap If present, (e)RedCap UEs use this UL BWP instead of initialUplinkBWP. If absent, (e)RedCap UEs use initialUplinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell) and upon handover from E-UTRA to NR. It is optionally present, Need M otherwise. |

#### –	UplinkConfigCommonSIB

The IE UplinkConfigCommonSIB provides common uplink parameters of a cell.

UplinkConfigCommonSIB information element

-- ASN1START

-- TAG-UPLINKCONFIGCOMMONSIB-START

UplinkConfigCommonSIB ::=               SEQUENCE {

frequencyInfoUL                         FrequencyInfoUL-SIB,

initialUplinkBWP                        BWP-UplinkCommon,

timeAlignmentTimerCommon                TimeAlignmentTimer

}

UplinkConfigCommonSIB-v1700 ::=         SEQUENCE {

initialUplinkBWP-RedCap-r17             BWP-UplinkCommon                                OPTIONAL   -- Need R

}

UplinkConfigCommonSIB-v1760 ::=         SEQUENCE {

frequencyInfoUL-v1760                   FrequencyInfoUL-SIB-v1760

}

-- TAG-UPLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

UplinkConfigCommonSIB field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

InitialUplinkBWP The initial uplink BWP configuration for a PCell (see TS 38.213 [13], clause 12).

initialUplinkBWP-RedCap If present, (e)RedCap UEs use this UL BWP instead of initialUplinkBWP. If absent, (e)RedCap UEs use initialUplinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

#### –	Uplink-PowerControl

The IE Uplink-PowerControl is used to configure UE specific power control parameter for PUSCH, PUCCH and SRS.

Uplink-PowerControl information element

-- ASN1START

-- TAG-UPLINK-POWERCONTROL-START

Uplink-powerControl-r17  ::= SEQUENCE {

ul-powercontrolId-r17        Uplink-powerControlId-r17,

p0AlphaSetforPUSCH-r17       P0AlphaSet-r17                                                               OPTIONAL, -- Need R

p0AlphaSetforPUCCH-r17       P0AlphaSet-r17                                                               OPTIONAL, -- Need R

p0AlphaSetforSRS-r17         P0AlphaSet-r17                                                               OPTIONAL  -- Need R

}

P0AlphaSet-r17 ::=           SEQUENCE {

p0-r17                       INTEGER (-16..15)                                                            OPTIONAL, -- Need R

alpha-r17                    Alpha                                                                        OPTIONAL, -- Need S

closedLoopIndex-r17          ENUMERATED { i0, i1 }

}

Uplink-powerControlId-r17 ::= INTEGER(1.. maxUL-TCI-r17)

-- TAG-UPLINK-POWERCONTROL-STOP

-- ASN1STOP

Uplink-PowerControl field descriptions

p0AlphaSetforPUSCH, p0AlphaSetforPUCCH, p0AlphaSetforSRS Configures power control parameters for PUSCH, PUCCH and SRS (see TS 38.213 [13], clause 7.2). When the field alpha is absent in p0AlphaSetforPUSCH, the UE applies the value 1 for PUSCH power control. When the field alpha is absent in p0AlphaSetforSRS, the UE applies the value 1 for SRS power control. In p0AlphaSetForPUCCH, the field alpha is absent (not used).

#### –	Uu-RelayRLC-ChannelConfig

The IE Uu-RelayRLC-ChannelConfig is used to configure an RLC entity, a corresponding logical channel in MAC for Uu Relay RLC channel between L2 U2N Relay UE and network, or between a N3C relay UE and network in case of MP.

Uu-RelayRLC-ChannelConfig information element

-- ASN1START

-- TAG-UU-RELAYRLC-CHANNELCONFIG-START

Uu-RelayRLC-ChannelConfig-r17::= SEQUENCE {

uu-LogicalChannelIdentity-r17    LogicalChannelIdentity                    OPTIONAL,   -- Cond RelayLCH-SetupOnly

uu-RelayRLC-ChannelID-r17        Uu-RelayRLC-ChannelID-r17,

reestablishRLC-r17               ENUMERATED {true}                         OPTIONAL,   -- Need N

rlc-Config-r17                   RLC-Config                                OPTIONAL,   -- Cond RelayLCH-Setup

mac-LogicalChannelConfig-r17     LogicalChannelConfig                      OPTIONAL,   -- Cond RelayLCH-Setup

...

}

-- TAG-UU-RELAYRLC-CHANNELCONFIG-STOP

-- ASN1STOP

Uu-RelayRLC-ChannelConfig field descriptions

uu-LogicalChannelIdentity Indicates the logical channel id for Uu Relay RLC channel of the L2 U2N Relay UE.

uu-RelayRLC-ChannelID Indicates the Uu Relay RLC channel in the link between L2 U2N Relay UE and network.

reestablishRLC Indicates that RLC should be re-established.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters.

| Conditional Presence | Explanation |
| --- | --- |
| RelayLCH-Setup | This field is mandatory present upon creation of a new logical channel for a Uu Relay RLC channel. It is optionally present, Need M, otherwise. |
| RelayLCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a Uu Relay RLC channel. It is absent otherwise. |

#### –	Uu-RelayRLC-ChannelID

The IE Uu-RelayRLC-ChannelID is used to identify a Uu Relay RLC channel in the link between L2 U2N Relay UE and network.

Uu-RelayRLC-ChannelID information element

-- ASN1START

-- TAG-UU-RELAYRLC-CHANNELID-START

Uu-RelayRLC-ChannelID-r17 ::= INTEGER (1..maxLC-ID)

-- TAG-UU-RELAYRLC-CHANNELID-STOP

-- ASN1STOP

#### –	UplinkTxDirectCurrentList

The IE UplinkTxDirectCurrentList indicates the Tx Direct Current locations per serving cell for each configured UL BWP in the serving cell, based on the BWP numerology and the associated carrier bandwidth.

UplinkTxDirectCurrentList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTLIST-START

UplinkTxDirectCurrentList ::=           SEQUENCE (SIZE (1..maxNrofServingCells)) OF UplinkTxDirectCurrentCell

UplinkTxDirectCurrentCell ::=           SEQUENCE {

servCellIndex                           ServCellIndex,

uplinkDirectCurrentBWP                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkTxDirectCurrentBWP,

...,

[[

uplinkDirectCurrentBWP-SUL              SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkTxDirectCurrentBWP               OPTIONAL

]]

}

UplinkTxDirectCurrentBWP ::=            SEQUENCE {

bwp-Id                                  BWP-Id,

shift7dot5kHz                           BOOLEAN,

txDirectCurrentLocation                 INTEGER (0..3301)

}

-- TAG-UPLINKTXDIRECTCURRENTLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentBWP field descriptions

bwp-Id The BWP-Id of the corresponding uplink BWP.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to true. Otherwise 7.5 kHz shift is not applied.

txDirectCurrentLocation The uplink Tx Direct Current location for the carrier. Only values in the value range of this field between 0 and 3299, which indicate the subcarrier index within the carrier corresponding to the numerology of the corresponding uplink BWP and value 3300, which indicates "Outside the carrier" and value 3301, which indicates "Undetermined position within the carrier" are used in this version of the specification.

UplinkTxDirectCurrentCell field descriptions

servCellIndex The serving cell ID of the serving cell corresponding to the uplinkDirectCurrentBWP.

uplinkDirectCurrentBWP The Tx Direct Current locations for all the uplink BWPs configured at the corresponding serving cell.

uplinkDirectCurrentBWP-SUL The Tx Direct Current locations for all the supplementary uplink BWPs configured at the corresponding serving cell.

#### –	UplinkTxDirectCurrentMoreCarrierList

The IE UplinkTxDirectCurrentMoreCarrierList indicates the Tx Direct Current locations for intra-band CA including one, two or more uplink carriers. The UE does not report the uplink Direct Current location information for SUL carrier(s).

UplinkTxDirectCurrentMoreCarrierList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTMORECARRIERLIST-START

UplinkTxDirectCurrentMoreCarrierList-r17 ::=   SEQUENCE (SIZE (1..maxNrofCC-Group-r17)) OF CC-Group-r17

CC-Group-r17 ::=           SEQUENCE {

servCellIndexLower-r17     ServCellIndex,

servCellIndexHigher-r17    ServCellIndex              OPTIONAL,

defaultDC-Location-r17     DefaultDC-Location-r17,

offsetToDefault-r17        CHOICE{

offsetValue                OffsetValue-r17,

offsetlist                 SEQUENCE (SIZE(1..maxNrofReqComDC-Location-r17)) OF OffsetValue-r17

}                                                     OPTIONAL

}

OffsetValue-r17::=         SEQUENCE {

offsetValue-r17            INTEGER (-20000.. 20000),

shift7dot5kHz-r17          BOOLEAN

}

DefaultDC-Location-r17 ::= CHOICE {

ul                         FrequencyComponent-r17,

dl                         FrequencyComponent-r17,

ulAndDL                    FrequencyComponent-r17

}

FrequencyComponent-r17 ::=  ENUMERATED {activeCarrier,configuredCarrier,activeBWP,configuredBWP}

-- TAG-UPLINKTXDIRECTCURRENTMORECARRIERLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentMoreCarrierList and CC-Group field descriptions

CC-Group The contiguous carriers sharing the same PA in an intra-band UL CA configuration. The UE shall report only one DC location for an intra-band CC combination with one active uplink carrier in case DefaultDC-Location is set to activeCarrier or activeBWP.

defaultDC-Location Indicates the default DC location derivation option. The default Tx Direct Current is located at the mathematical center of the UE bandwidth, i.e. between the lower edge of the lowest subcarrier of the lowest frequency component and the upper edge of the highest subcarrier of the highest frequency component, rounded to the subcarrier grid of the lowest SCS defined for the component carrier on which the default Direct Current is located. The lowest and highest frequency components used for derivation of mathematical center are indicated by FrequencyComponent in the associated CC-Group, where the lowest frequency component and the highest frequency component may be the same. If the mathematical center of the UE bandwidth lands on frequencies where there is no subcarrier grid defined, the subcarrier grid of the lowest SCS of the nearest lower frequency component carrier shall be extended to cover the frequency of the mathematical default Direct Current location.

offsetToDefault Indicates the DC location offset to the default DC location derived from defaultDC-Location. The lowest SCS in the CC group is used as the offset granularity. Value 0 respresents no offset. offsetValue is used in case DefaultDC-Location is set to configuredCarrier or configuredBWP. offsetlist is used in case DefaultDC-Location is set to activeCarrier or activeBWP. Each entity in this list corresponds to the entry in carriers combination in IntraBandCC-CombinationReqList of the intra-band CA component. For each CC group, the UE shall include the same number of entries, and listed in the same order as in CC-CombinationList. If DefaultDC-Location is set the activeCarrier, same offsetValue is signalled for all requested carriers combinations with same active carriers states(regardless of the active BWP index).

servCellIndexHigher Indicates the serving cell index of the highest edge of the CC-Group. If asbsent, there is only one carrier in this group indicated by servCellIndexLower.

servCellIndexLower Indicates the serving cell index of the lowest edge of the CC-Group.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to true, otherwise 7.5 kHz shift is not applied.

DefaultDC-Location field descriptions

dl Indicates that the default DC location is derived based on the DL frequencies of the frequency component.

ul Indicates that the default DC location is derived based on the UL frequencies of the frequency component.

ulAndDL Indicates that the default DC location is derived based on the edge most frequencies among any DL and UL frequency components.

#### –	UplinkTxDirectCurrentTwoCarrierList

The IE UplinkTxDirectCurrentTwoCarrierList indicates the Tx Direct Current locations when uplink intra-band CA with two carriers is configured, based on the configured carriers and BWP numerology and the associated carrier bandwidth of the carriers. The UE does not report the uplink Direct Current location information for SUL carrier(s).

UplinkTxDirectCurrentTwoCarrierList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTTWOCARRIERLIST-START

UplinkTxDirectCurrentTwoCarrierList-r16 ::=   SEQUENCE (SIZE (1..maxNrofTxDC-TwoCarrier-r16)) OF UplinkTxDirectCurrentTwoCarrier-r16

UplinkTxDirectCurrentTwoCarrier-r16 ::=       SEQUENCE {

carrierOneInfo-r16                            UplinkTxDirectCurrentCarrierInfo-r16,

carrierTwoInfo-r16                            UplinkTxDirectCurrentCarrierInfo-r16,

singlePA-TxDirectCurrent-r16                  UplinkTxDirectCurrentTwoCarrierInfo-r16,

secondPA-TxDirectCurrent-r16                  UplinkTxDirectCurrentTwoCarrierInfo-r16            OPTIONAL

}

UplinkTxDirectCurrentCarrierInfo-r16 ::=      SEQUENCE {

servCellIndex-r16                             ServCellIndex,

servCellInfo-r16                              CHOICE {

bwp-Id-r16                                    BWP-Id,

deactivatedCarrier-r16                        ENUMERATED {deactivated}

}

}

UplinkTxDirectCurrentTwoCarrierInfo-r16 ::=   SEQUENCE {

referenceCarrierIndex-r16                     ServCellIndex,

shift7dot5kHz-r16                             BOOLEAN,

txDirectCurrentLocation-r16                   INTEGER (0..3301)

}

-- TAG-UPLINKTXDIRECTCURRENTTWOCARRIERLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentTwoCarrierInfo field descriptions

referenceCarrierIndex The serving cell ID of the carrier which is to be used as the reference for interpreting the Tx Direction Current location as reported using txDirectCurrentLocation-r16. The numerology of the uplink BWP ID reported with bwp-Id-r16 for this serving cell is the numerology used for interpreting the reported subcarrier location.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to true. Otherwise 7.5 kHz shift is not applied.

txDirectCurrentLocation The uplink Tx Direct Current location for the two carrier uplink CA with the serving cells reported using carrierOneInfo-r16 and carrierTwoInfo-r16. Values in the range of this field between 0 and 3299 indicate the subcarrier index of the uplink Tx Direct Current location with the subcarrier taken from the serving cell with ID referenceCarrierIndex and the numerology of the corresponding uplink BWP reported for this serving cell. Value 3300 indicates "Outside the carrier" and value 3301 indicates "Undetermined position within the carrier".

UplinkTxDirectCurrentCarrierInfo field descriptions

bwp-Id The BWP ID of the serving cell which is part of the two carrier uplink carrier aggregation. The UE shall not report this field if the serving cell is reported as deactivated using deactivatedCarrier-r16.

deactivatedCarrier For the reported uplink Tx Direct Current location(s) corresponding to singlePA-TxDirectCurrent-r16, indicates whether the carrier is deactivated or not for this serving cell. If the carrier refers to the PCell, the UE shall not set this field to deactivated.

servCellIndex The serving cell ID of the serving cell which is part of the two carrier uplink carrier aggregation.

UplinkTxDirectCurrentTwoCarrier field descriptions

carrierOneInfo The serving cell ID and BWP ID of the first carrier of the uplink carrier aggregation for which the uplink Tx Direct Current location(s) are being reported.

carrierTwoInfo The serving cell ID and BWP ID of the second carrier of the uplink carrier aggregation for which the uplink Tx Direct Current location(s) are being reported.

singlePA-TxDirectCurrent The uplink Tx Direct Current location for the UE which support single PA for this uplink carrier aggregation. For the UEs which support dual PA for this uplink carrier aggregation, this field is for reporting the uplink Tx Direct Current location of the first PA.

secondPA-TxDirectCurrent The uplink Tx Direct Current location used by the UE with the second PA for the UEs which support dual PA for this uplink carrier aggregation. This field shall be absent for the UplinkTxDirectCurrentTwoCarrier entity where deactivatedCarrier of carrierOneInfo or carrierTwoInfo is set to deactivated.

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a Zero-Power (ZP) CSI-RS resource (see TS 38.214 [19], clause 5.1.4.2). Reconfiguration of a ZP-CSI-RS-Resource between periodic or semi-persistent and aperiodic is not supported.

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=              SEQUENCE {

zp-CSI-RS-ResourceId                ZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-ResourceId ::=            INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

ZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS (see TS 38.214 [19], clause 5.1.4.2). Network always configures the UE with a value for this field for periodic and semi-persistent ZP-CSI-RS resource (as indicated in PDSCH-Config).

resourceMapping OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot.

zp-CSI-RS-ResourceId ZP CSI-RS resource configuration ID (see TS 38.214 [19], clause 5.1.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent ZP-CSI-RS-Resources (as indicated in PDSCH-Config). The field is absent otherwise. |

#### –	ZP-CSI-RS-ResourceSet

The IE ZP-CSI-RS-ResourceSet refers to a set of ZP-CSI-RS-Resources using their ZP-CSI-RS-ResourceIds.

ZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESET-START

ZP-CSI-RS-ResourceSet ::=           SEQUENCE {

zp-CSI-RS-ResourceSetId             ZP-CSI-RS-ResourceSetId,

zp-CSI-RS-ResourceIdList            SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId,

...

}

-- TAG-ZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

ZP-CSI-RS-ResourceSet field descriptions

zp-CSI-RS-ResourceIdList The list of ZP-CSI-RS-ResourceId identifying the ZP-CSI-RS-Resource elements belonging to this set.

#### –	ZP-CSI-RS-ResourceSetId

The IE ZP-CSI-RS-ResourceSetId identifies a ZP-CSI-RS-ResourceSet.

ZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESETID-START

ZP-CSI-RS-ResourceSetId ::=                     INTEGER (0..maxNrofZP-CSI-RS-ResourceSets-1)

-- TAG-ZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP