3GPP TSG-RAN WG2 Meeting #126	R2-240xxxx

Fukuoka, Japan, May 20-24th, 2024

[POST125bis][019][Emergency Calls] Common solution (Lenovo)

Intended outcome: Discuss need for a common solution and possible solutions for a common framework

Deadline:  two weeks

Please fill in the below table:

| Contact person – Company | Email |
| --- | --- |
| Prateek – Lenovo | pmallick@lenovo.com |
| Naveen – Apple | naveen.palle@apple.com |
| Alexey Kulakov-Vodafone | Alexey.kulakov1@vodafone.com |
| Max—T-Mobile USA | Kun.lu7@t-mobile.com |
| Jussi – Nokia | Jussi-pekka.koskinen@nokia.com |
| vivo- Xiang Pan | panxiang@vivo.com |
| Eswar- ZTE | Eswar.vutukuri@zte.com.cn |
| Yulong-Huawei | shiyulong5@huawei.com |
| Qianxi – OPPO | qianxi.lu@oppo.com |
| Emre - Ericsson | emre.yavuz@ericsson.com |
| Salva – BT | Salva.diazsendra@bt.com |

Phase 1 Completed: Deadline 29th April UTC 22:00

Phase 2: Deadline for Phase 2: May 3rd, UTC 12:00

Let’s start with clarifying the aim of the common solution, as:

- Common solution for EM Calls aims only UEs which are capable of EM Calls.

- Common solution aims at enabling EM Calls for EM call capable UEs that are otherwise barred in the cell either due to MIB barring or feature specific SIB1 barring.

Based on the discussion in Phase 1, the most acceptable and working solution seems: One explicit bit in SIB1 to indicate if cell supports EM calls irrespective of the feature(s) supported in the cell. This bit will replace (e)RedCap barringExempt-eRedCap bit and therefore does not pose any further signalling load in SIB1. This information is used by a UE capable of EM Calls, barred in the cell and having no suitable cell to camp on.

Q1: Is an explicit 1-bit indication in SIB1 replacing barringExempt-eRedCap bit acceptable to your company? The 1-bit indication in SIB1 is to be used to indicate if cell supports EM calls irrespective of the feature(s) supported in the cell. It is to be used by a UE when it is considered barred in the cell and has no suitable cell to camp on, to obtain limited services.

| Company name | Yes/ No | Comments |
| --- | --- | --- |
| ZTE | Yes, with some clarifications (please see the comments) | We would like to add that the above should only be the case if the UE can access the cell normally (i.e. there is no issue with supported bandwidth, duplex mode etc) and MIB is not set to barred. So, to be clear, in this case, we still need all the text below in Red (from the endorsed R2-2402903) which is to check that the UE can access the cell “normally” although it has fewer number of Rx antennas)- the only change is the actual SIB1 bit: ----------------- When cellBarredRedCap1Rx is set to “barred” in SIB1, a RedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to “barred” and in SIB1, barringExemptIndication is set to “true” and, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to “true” and, intraFreqReselectionRedCap is present in SIB1; or ----------------- Btw, the turquoise highlighted text above seems to be missing from R2-2402903 (and 3472)?? Shouldn’t this check also be there?? i.e. for the emergency calls for 2RX/1RX to be allowed the cell should support (e)redcap in the first place, right?? Similar text as above can then be reused for eRedcap (with the same “barringExemptIndication” bit and for both 1Rx and 2Rx cases). Then, for XR we can add similar text too (e.g. as below): ------------------ proposed text for XR ----- When cellBarred2RxXR is included in SIB1, an XR UE that supports only 2Rx branches can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to “barred” and in SIB1, barringExemptIndication is set to “true”; or ----------------- Finally, for NES since the barring is based on MIB, we don’t think we need to do anything. i.e. when the MIB is set to barred and the cellBarredNES is present (note that cellBarredNES being present indicates that cell is actually not barred), the NES UE will anyway consider the cell as suitable cell. So, for this case, we don’t think anything is needed for the exemption (since the NES UE can access the cell normally for everything including EM calls). Btw, an alternative for the above is to allow UEs by default without checking the “barringExemptIndication” bit. If the intention is to go this way, it is also fine, but then this should be used for all cases. In this scenario, we still need all the above checks for each feature (to ensure EM calls can be performed in the cell), but without the check for the barringExemptIndication bit. |
| Huawei | Partially Yes, but | We still prefer and also seems more supported option B that there is no need of this barringExemptIndication bit (also mentioned above by ZTE). If clear majority prefer to introduce on bit control, we can accept to reuse/rename the barringExempt-eRedCap for 1 bit explicit control for R18 features But, this emergency call should only be used if the cell is not barred due to MIB cellbarred. We should not override the MIB cell barring function which has been there since R15 (this is also the principle for the last meeting agreed RedCap CRs.). |
| Apple | No | We have concerns with re-interpreting the Redcap bit. That redcap bit is meant to also allow R17 redcap UEs to utilize the cell for EM calls. It will create confusion and likely corner cases if a single bit will link rel-17 Redcap and all Rel-18 features. We think it is simple to just allow EMs calls for R18 and future features where they have similar cases as eRedCap and NES. AND do this without any new bit – why would we need to consider NWs which support and which do not support EM calls by the Rel-18 UEs when we already know that EM calls are possible even when the UE bars the cell (with the conditions that ZTE listed). We do not see the need for additional SIB1 bit! This will only complicate where both the NWs and UE need to have procedures with the presence and absence of this bit. No need for a SIB1 bit for eRedCap, NES and any other R18 and future features. |

Q2: Can the same 1-bit indication in SIB1 be applicable to Rel. 17 UEs, specifically for RedCap UEs (i.e., barringExempt-RedCap is not required anymore) as well to obtain limited services? And for this purpose, a magic sentence in the Rel. 17 CRs is sufficient?

| Company name | Yes/ No | Comments |
| --- | --- | --- |
| ZTE | Yes | One bit can indicate the barring exemption and we should capture that the barring exemption according to this bit is applicable only if the UE can access the cell normally please see the comment above. |
| Huawei | No | We should not bound the R18 feature and R17 feature together, which will delay the network to consider enable this feature for RedCap. R17 RedCap is already deployed. The barringExempt-RedCap is to somehow indicate whether the network has been upgraded to support this TEI feature. |
| Apple | No | Same reason as HW and also pls see our comments above. |