3GPP TSG RAN WG 2 Meeting #126	R2-24xxxxx

Fukuoka, Japan, 20th - 24th May, 2024

Source: 	Fujitsu

Title: 	Summary of [Post125bis][519][R18 Mob] Power Control Parameters after LTM cell switch (Fujitsu)

Agenda Item:	7.4.x

Document for:	Discussion and decision

1	Introduction

This document is a summary of:

[Post125bis][519][R18 Mob] Power Control Parameters after LTM cell switch (Fujitsu)

Scope: Collect RAN2 input in order to determine impacts and make decision as requested in R1 LS R1-2403683.

Intended outcome: Report

Intended three weeks

The rapporteur sets two phases of discussions.

- The first phase (Deadline Wednesday 8th May, 6:00UTC): discuss pros and cons of approaches/options proposed by RAN1 and which approach/option is preferable/acceptable.

- The second phase (Deadline Friday 10th May, 6:00UTC): decide which approach/option RAN2 adopts and discuss tentative text proposals.

Company contact persons for this discussion are invited to fill one entry in the table below:

| Company | Name | Email Address |
| --- | --- | --- |
| Fujitsu (rapporteur) | Takako Sanda | sanda.takako @ fujitsu.com |

2	Background

RAN2 receives an LS from RAN1 on the identification of the power control parameters after LTM cell switch [1]. In the LS, RAN1 points out that it is unclear how the UE would acquire the power control parameters associated with CandidateTCI-State/CandidateTCI-UL-State applied to the UL transmission after LTM cell switch till TCI-state/TCI-UL-State is indicated at the target cell. This is because power control parameters are not included under LTM-TCI-Info-r18 in the current RRC specification [2]. To solve this issue, RAN1 proposes two approaches.

<Proposed approaches from RAN1> - Approach 1 - For UL transmission after cell switch and before the serving cell TCI state is indicated, UE applies power control parameter in the ul-powerControl-r17 of the TCI-State or the TCI-UL-State, if configured, corresponding to the CandidateTCI-State or the CandidateTCI-UL-State indicated in the LTM Cell Switch Command. Otherwise, ul-powerControl-r17 configured in BWP-UplinkDedicated of the target cell is applied. - Approach 2 - Introduce necessary RRC parameters for power control under LTM configurations.

RAN1 also provides additional proposals for each approach.

<If the approach 1 is taken> It would be necessary to capture in RAN2 specifications the linkage between CandidateTCI-State/CandidateTCI-UL-State and TCI-state/TCI-UL-State for the same cell. The linkage has been agreed in RAN1#115 as follows, but RAN1 does not capture it in their specifications: Agreement UE may expect that: - For a candidate cell, the configuration of an LTM TCI state in ltm-DL-OrJointTCI-StateToAddModList-r18 and ltm-ul-TCI-ToAddModList-r18 is same as its counterpart in dl-OrJointTCI-StateList-r17 and ul-TCI-ToAddModList-r17 of the first active BWP in ServingCellConfig, at least in terms of TCI state ID, the corresponding qcl-Type1 and qcl-Type2 for the DL or joint TCI state or referenceSignal for the UL TCI state. - The LTM TCI state(s) in ltm-DL-OrJointTCI-StateToAddModList-r18 and ltm-ul-TCI-ToAddModList-r18 of a candidate cell is a subset of serving cell TCI state(s) in dl-OrJointTCI-StateList-r17 and ul-TCI-ToAddModList-r17 of the same cell.

<If the approach 2 is taken> The following parameters need to be newly added: it is noted that the final check/decision is up to RAN2. - Under LTM-TCI-Info-r18, - uplink-PowerControlToAddModList-r18 SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControl-r18 OPTIONAL, -- Need N - uplink-PowerControlToReleaseList-r18 SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF Uplink-powerControlId-r18 OPTIONAL, -- Need N - Field description: Configures UL power control parameters for PUSCH, PUCCH and SRS when field unifiedTCI-StateType is configured for this serving cell. - Under Uplink-powerControl-r18 - Uplink-powerControlId-r18 ::= INTEGER(1.. maxNrofCandidateUL-TCI-r18) - p0AlphaSetforPUSCH-r18 P0AlphaSet-r17 OPTIONAL, -- Need R - p0AlphaSetforPUCCH-r18 P0AlphaSet-r17 OPTIONAL, -- Need R - p0AlphaSetforSRS-r18 P0AlphaSet-r17 OPTIONAL -- Need R - Under CandidateTCI-State-r18, - ul-powerControl-r18 Uplink-powerControlId-r18 OPTIONAL, -- Need R - Field description: Configures power control parameters for PUCCH, PUSCH and SRS of the LTM candidate that includes this CandidateTCI-State - Under CandidateTCI-UL-State-r18, - ul-powerControl-r18 Uplink-powerControlId-r18 OPTIONAL, -- Need R - Field description: Configures power control parameters for PUCCH, PUSCH and SRS of the LTM candidate that includes this CandidateTCI-UL-State Also, RAN1 sees the necessity to define a default behaviour when this ul-powerControl-r18 under LTM-TCI-Info-r18 for a candidate cell is not configured, e.g. - UE is expected to be configured either ul-powerControl-r18 under LTM-TCI-Info-r18 in LTM-Candidate-r18 for a candidate cell or ul-powerControl-r18 under BWP-UplinkDedicated in ServingCellConfig for the candidate cell.

Based on the approaches, RAN1 asks RAN2 to take either option.

<Proposed options from RAN1> Option 1: Capture the RAN1 agreement on the linkage between TCI states for candidate cell(s) and those for target cell(s) in RAN2 specification(s) for approach 1 Option 2: Introduce the new RRC parameters above for approach 2

3	Discussion

3.1	Phase 1

In this phase, companies are invited to discuss pros and cons of approaches/options proposed by RAN1 and which approach/option is preferable/acceptable.

The rapporteur’s view is as follows:

- Approach 1/Option 1: Pros is simpler and less specification impact than approach 2/option 2. Cons is there may be some delay between receiving LTM cell switch command and identifying power control parameter compare with approach 2/option 2.

- Approach 2/Option 2: Pros is delay between receiving LTM cell switch command and identifying power control parameter may be smaller than approach 1/option 1. Cons is specification impact, which includes the determination of a default behaviour when this ul-powerControl-r18 under LTM-TCI-Info-r18 for a candidate cell is not configured, is larger than approach 1/option 1.

As a reference, images of approach 1 and approach 2 are illustrated in Figure 1 and Figure 2 in Appendix, respectively.

The rapporteur kindly asks companies to provide your opinion in the following question.

Q1: What do you think is the pros and cons of the approach 1/option 1?

| Company | Pros | Cons |
| --- | --- | --- |

Q2: What do you think is the pros and cons of the approach 2/ option 2?

| Company | Pros | Cons |
| --- | --- | --- |

Q3: Do you propose any other approaches/options to solve the issue?

| Company | Comments |
| --- | --- |

Q4: Which approach/option (including other approaches/options) do you prefer?

| Company | Approach | Comments |
| --- | --- | --- |

3.2	Phase 2

TBA

4	Conclusion

TBA

5	References

- R1-2403683, " LS on the identification of the power control parameters after LTM cell switch"

- TS 38.331 v18.1.0

Appendix

Figure 1 and Figure 2 illustrate images of approach 1 and approach 2 respectively.

<Approach 1>

For UL transmission after cell switch and before the serving cell TCI state is indicated, UE applies power control parameter in the ul-powerControl-r17 of the TCI-State or the TCI-UL-State, if configured, corresponding to the CandidateTCI-State or the CandidateTCI-UL-State indicated in the LTM Cell Switch Command. Otherwise, ul-powerControl-r17 configured in BWP-UplinkDedicated of the target cell is applied. It would be necessary to capture in RAN2 specifications the linkage between CandidateTCI-State/CandidateTCI-UL-State and TCI-state/TCI-UL-State for the same cell.

Figure 1: Approach 1

<Approach 2>

Introduce necessary RRC parameters for power control under LTM configurations. A default behaviour when this ul-powerControl-r18 under LTM-TCI-Info-r18 for a candidate cell is not configured will also need to be specified.

Figure 2: Approach 2