Companies providing input to this email discussion are requested to leave contact information below.

| Company | Delegate name | Email address |
| --- | --- | --- |
| CATT | Hao Xu | xuhao@catt.cn |
| ASUSTeK | Lider Pan | lider_pan@asus.com |

# 1	Comments on CR

| Company | Clause number | Original text in CR | Suggested modification or comments | Rapporteur response |
| --- | --- | --- | --- | --- |
| CATT | 6.2.2 | n3c-Suport This field indicates the support of N3C MP. If the field is present, the UE can perform early detection of candidate N3C relay UEs. If absent, a UE is not required to perform early detection of candidate N3C relay UEs. | n3c-Support This field indicates the support of N3C MP. If the field is present, the UE can perform early detection of candidate N3C relay UEs. If absent, a UE is not required to perform early detection of candidate N3C relay UEs. |  |
| CATT | 6.3.1 | t400-U2U-r18 ENUMERATED {ms200, ms400, ms600, ms800, ms1200, ms2000, ms3000, ms4000} OPTIONAL, -- Need M | t400-U2U-r18 ENUMERATED {ms200, ms400, ms600, ms800, ms1200, ms2000, ms3000, ms4000} OPTIONAL, -- Need R |  |
| CATT | 6.3.1 6.3.5 |  | For the new introdeced t400-U2U-r18, just wonder any procedure text’s addition is also needed? |  |
| ASUSTeK | 5.8.9.1.2 | ##### 5.8.9.1.2 Actions related to transmission of RRCReconfigurationSidelink message The UE shall set the contents of RRCReconfigurationSidelink message as follows: 1> for each sidelink DRB that is to be released, according to clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, by upper layers, or due to end-to-end sidelink release: 2> set the entry included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB; … | It seems to us that the term “end-to-end sidelink release” is not used at any other places in the RRC Spec. In our understanding, this should be “end-to-end sidelink DRB release”. |  |
| ASUSTeK | 5.8.9.1a.1.2 and 5.8.9.7.1 | 5.8.9.1a.1.2 Sidelink DRB release operations … 1> if the sidelink radio link failure is detected for a specific destination; or 1> if the sidelink DRB release is triggered by end-to-end PC5 connection failure due to per-hop PC5 link failure, in accordance with clause 5.8.9.3a: 2> release the PDCP entity, RLC entity and the logical channel of the sidelink DRB for the specific destination. … 5.8.9.7.1 PC5 Relay RLC channel release … 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB release as specified in 5.8.9.1a.1.2; or 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB modification as specified in 5.8.9.1a.2.2: 2> if the PC5 Relay RLC channel release was triggered due to per-hop PC5 link failure; or 2> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR; 3> release the RLC entity and the corresponding logical channel associated with the end-to-end DRB; … | It seems the RLC channel release due to per-hop PC5 link failure is performed twice in clauses 5.8.9.1a.1.2 and 5.8.9.7.1. In our understanding, the cause of the PC5 Relay RLC channel release in clause 5.8.9.7.1 could be due to end-to-end PC5 link failure (including T400 expiry, integrity check failure, and end-to-end PC5 connection failure due to reception of NotificationMessageSidelink, as specified in clause 5.8.9.3a) instead of per-hop PC5 link failure. Besides, as specified in clause 5.8.9.1.2, the sidelink RRC reconfiguration procedure is initiated due to end-to-end sidelink DRB release. Thus, we think the action of RLC channel release should be performed after receiving the RRCReconfigurationCompleteSidelink message. If the above understandings are correct, the following modification is suggested: 5.8.9.7.1 PC5 Relay RLC channel release … 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB release as specified in 5.8.9.1a.1.2; or 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB modification as specified in 5.8.9.1a.2.2: 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to per-hopend-to-end PC5 link failure; or 2> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR; 3> release the RLC entity and the corresponding logical channel associated with the end-to-end DRB; … |  |

# 2	Comments on RIL list

| Company | Suggested modification or comments | Rapporteur response |
| --- | --- | --- |