Companies providing input to this email discussion are requested to leave contact information below.

| Company | Delegate name | Email address |
| --- | --- | --- |
| CATT | Hao Xu | xuhao@catt.cn |
| ASUSTeK | Lider Pan | lider_pan@asus.com |
| Apple | Zhibin Wu | Zhibin_wu@apple.com |

# 1	Comments on CR

| Company | Clause number | Original text in CR | Suggested modification or comments | Rapporteur response |
| --- | --- | --- | --- | --- |
| CATT | 6.2.2 | n3c-Suport This field indicates the support of N3C MP. If the field is present, the UE can perform early detection of candidate N3C relay UEs. If absent, a UE is not required to perform early detection of candidate N3C relay UEs. | n3c-Support This field indicates the support of N3C MP. If the field is present, the UE can perform early detection of candidate N3C relay UEs. If absent, a UE is not required to perform early detection of candidate N3C relay UEs. |  |
| CATT | 6.3.1 | t400-U2U-r18 ENUMERATED {ms200, ms400, ms600, ms800, ms1200, ms2000, ms3000, ms4000} OPTIONAL, -- Need M | t400-U2U-r18 ENUMERATED {ms200, ms400, ms600, ms800, ms1200, ms2000, ms3000, ms4000} OPTIONAL, -- Need R |  |
| CATT | 6.3.1 6.3.5 |  | For the new introdeced t400-U2U-r18, just wonder any procedure text’s addition is also needed? |  |
| ASUSTeK | 5.8.9.1.2 | ##### 5.8.9.1.2 Actions related to transmission of RRCReconfigurationSidelink message The UE shall set the contents of RRCReconfigurationSidelink message as follows: 1> for each sidelink DRB that is to be released, according to clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, by upper layers, or due to end-to-end sidelink release: 2> set the entry included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB; … | It seems to us that the term “end-to-end sidelink release” is not used at any other places in the RRC Spec. In our understanding, this should be “end-to-end sidelink DRB release”. |  |
| ASUSTeK | 5.8.9.1a.1.2 and 5.8.9.7.1 | 5.8.9.1a.1.2 Sidelink DRB release operations … 1> if the sidelink radio link failure is detected for a specific destination; or 1> if the sidelink DRB release is triggered by end-to-end PC5 connection failure due to per-hop PC5 link failure, in accordance with clause 5.8.9.3a: 2> release the PDCP entity, RLC entity and the logical channel of the sidelink DRB for the specific destination. … 5.8.9.7.1 PC5 Relay RLC channel release … 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB release as specified in 5.8.9.1a.1.2; or 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB modification as specified in 5.8.9.1a.2.2: 2> if the PC5 Relay RLC channel release was triggered due to per-hop PC5 link failure; or 2> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR; 3> release the RLC entity and the corresponding logical channel associated with the end-to-end DRB; … | It seems the RLC channel release due to per-hop PC5 link failure is performed twice in clauses 5.8.9.1a.1.2 and 5.8.9.7.1. In our understanding, the cause of the PC5 Relay RLC channel release in clause 5.8.9.7.1 could be due to end-to-end PC5 link failure (including T400 expiry, integrity check failure, and end-to-end PC5 connection failure due to reception of NotificationMessageSidelink, as specified in clause 5.8.9.3a) instead of per-hop PC5 link failure. Besides, as specified in clause 5.8.9.1.2, the sidelink RRC reconfiguration procedure is initiated due to end-to-end sidelink DRB release. Thus, we think the action of RLC channel release should be performed after receiving the RRCReconfigurationCompleteSidelink message. If the above understandings are correct, the following modification is suggested: 5.8.9.7.1 PC5 Relay RLC channel release … 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB release as specified in 5.8.9.1a.1.2; or 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB modification as specified in 5.8.9.1a.2.2: 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to per-hopend-to-end PC5 link failure; or 2> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR; 3> release the RLC entity and the corresponding logical channel associated with the end-to-end DRB; … |  |
| OPPO | 5.3.7.2 | 1> release sl-L2RelayUE-Config, if configured; 1> release sl-L2RemoteUE-Config, if configured; 1> release the SRAP entity, if configured; | The U2U Relay configuration should not be released, suggested change as follows: 1> release sl-L2RelayUE-Config for U2N Relay case, if configured; 1> release sl-L2RemoteUE-Config for U2N Relay case, if configured; 1> release the SRAP entity for U2N Relay case, if configured; |  |
| OPPO | 5.8.3.3/6.2.2 SidelinkUEInformationNR | 5.8.3.3 <omit> 5> set sl-RLC-ModeIndicationistL2-U2U to include the RLC mode(s), if the associated bi-directional PC5 RLC channel(s) has been established due to the configuration by RRCReconfigurationSidelink; 6.2.2-SidelinkUEInformationNR SL-TxResourceReqL2-U2U-r18 ::= SEQUENCE { sl-DestinationIdentityL2-U2U-r18 SL-DestinationIdentity-r16 OPTIONAL, sl-TxInterestedFreqListL2-U2U-r18 SL-TxInterestedFreqList-r16, sl-TypeTxSyncListL2-U2U-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16, sl-CapabilityInformationSidelink-r18 OCTET STRING OPTIONAL, sl-U2U-InfoList-r18 SEQUENCE (SIZE (1.. maxNrofRemoteUE-r17)) OF SL-U2U-Info-r18 OPTIONAL, sl-RLC-ModeIndicationListL2-U2U-r18 SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-RLC-Mode-r18 OPTIONAL, ... } | Another alt is to reuse the legacy IE (sl-RLC-ModeIndicationList) and set the QFI by relay UE implementation, suggested change is as follows: sl-RLC-ModeIndication This field indicates the RLC mode and optionally the related QoS profiles for the sidelink radio bearer, which has not been configured by the network and is initiated by another UE in unicast. The RLC mode for one sidelink radio bearer is aligned between UE and NW by the sl-QoS-FlowIdentity. L2 U2U Relay UE set the associated sl-QoS-FlowIdentity by implementation. |  |
| OPPO | 5.8.9.1.2 | 1> for each sidelink DRB that is to be released, according to clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, by upper layers, or due to end-to-end sidelink release: | Can “by upper layers” already cover the end-to-end link release case? So we don’t need to add the “or due to end-to-end sidelink release” |  |
| OPPO | 5.8.9.1.2 | 1> for each sidelink DRB that is to be established or modified, according to clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR: 2> if the sidelink DRB is a per-hop sidelink DRB (i.e. the UE is performing non-L2 U2U relay NR sidelink communication with a peer UE): 3> if a sidelink DRB is to be established: 4> assign a new logical channel identity for the logical channel to be associated with the sidelink DRB and set sl-MAC-LogicalChannelConfigPC5 in the SLRB-Config to include the new logical channel identity; 3> set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB; 2> else if the sidelink DRB is an end-to-end sidelink DRB (i.e. the UE is acting as a L2 U2U Remote UE, and configure peer L2 U2U Remote UE with end-to-end SDAP and PDCP): 3> if the UE is in RRC_CONNECTED: 4> set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig in sl-ConfigDedicatedNR; 3> else if the UE is in RRC_IDLE/RRC_INACTIVE: 4> set the SLRB-Config included in the slrb-ConfigToAddModList, which is derived by end-to-end QoS profile, according to the sl-RadioBearerConfig in SIB12; 3> if the UE is out of coverage: 4> set the SLRB-Config included in the slrb-ConfigToAddModList, which is derived by end-to-end QoS profile, according to the sl-RadioBearerConfig in SidelinkPreconfigNR; | Since “or provide the L2 U2U Relay UE with the QoS flow to end-to-end DRB mapping” is removed, there is no need to separate E2E DRB and per-hop DRB, the only difference is for U2U Relay case, the assign logical channel identity operation is not needed, suggested change is as follows: 1> for each sidelink DRB that is to be established or modified, according to clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR: 2> if the sidelink DRB is a per-hop sidelink DRB (i.e. the UE is performing non-L2 U2U relay NR sidelink communication with a peer UE): 32> if a sidelink DRB is to be established and if the sidelink DRB is a per-hop sidelink DRB (i.e. the UE is performing non-L2 U2U relay NR sidelink communication with a peer UE): 34> assign a new logical channel identity for the logical channel to be associated with the sidelink DRB and set sl-MAC-LogicalChannelConfigPC5 in the SLRB-Config to include the new logical channel identity; 23> set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB; 2> else if the sidelink DRB is an end-to-end sidelink DRB (i.e. the UE is acting as a L2 U2U Remote UE, and configure peer L2 U2U Remote UE with end-to-end SDAP and PDCP): 3> if the UE is in RRC_CONNECTED: 4> set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig in sl-ConfigDedicatedNR; 3> else if the UE is in RRC_IDLE/RRC_INACTIVE: 4> set the SLRB-Config included in the slrb-ConfigToAddModList, which is derived by end-to-end QoS profile, according to the sl-RadioBearerConfig in SIB12; 3> if the UE is out of coverage: 4> set the SLRB-Config included in the slrb-ConfigToAddModList, which is derived by end-to-end QoS profile, according to the sl-RadioBearerConfig in SidelinkPreconfigNR; |  |
| OPPO | 5.8.9.1a.1.2 | 1> if the sidelink DRB release is triggered by end-to-end PC5 connection failure due to per-hop PC5 link failure, in accordance with clause 5.8.9.3a: | Why only mention the per-hop PC5 link failure case? |  |
| OPPO | 5.8.9.1a.2.2 | 2> for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE): 3> if the UE is in RRC_CONNECTED: 4> reconfigure the SRAP entity for the sidelink DRB, in accordance with the sl-SRAP-ConfigU2U received in sl-ConfigDedicatedNR, if included; 3> else if the UE is in RRC_IDLE or RRC_INACTIVE: 4> derive the PC5 RLC channel configuration based on per-SLRB QoS profile of this end-to-end sidelink DRB according to the configuration in SIB12; 3> else if the UE is out of coverage: 4> derive the PC5 RLC channel configuration based on per-SLRB QoS profile of this end-to-end sidelink DRB according to the configuration in SidelinkPreconfigNR; 3> if the PC5 RLC channel configuration derived by per-SLRB QoS profile of this end-to-end sidelink DRB is changed: 4> perform the PC5 Relay RLC channel release or addition/modification according to the derived PC5 RLC channel configuration as specified in 5.8.9.7.1 or 5.8.9.7.2; 4> consider the PC5 RLC channel applying the configuration derived by per-SLRB QoS profile of this end-to-end sidelink DRB as the egress PC5 relay RLC channel; 4> associate this end-to-end sidelink DRB with the PC5 RLC channel; 4> reconfigure the SRAP entity with the mapping between the end-to-end sidelink DRB and the egress PC5 relay RLC channel for the sidelink DRB. | The wording of IDLE/INACTIVE/OOC case can be simplified/aligned with CONNECTED case as follows 2> for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE): 3> if the UE is in RRC_CONNECTED: 4> reconfigure the SRAP entity for the sidelink DRB, in accordance with the sl-SRAP-ConfigU2U received in sl-ConfigDedicatedNR, if included; 3> else if the UE is in RRC_IDLE or RRC_INACTIVE: 4> reconfigure the SRAP entity for the sidelink DRB, if the derived PC5 RLC channel configuration derived by per-SLRB QoS profile of this end-to-end sidelink DRB is changed according to he configuration in SIB12;derive the PC5 RLC channel configuration based on per-SLRB QoS profile of this end-to-end sidelink DRB according to the configuration in SIB12; 3> else if the UE is out of coverage: 4> reconfigure the SRAP entity for the sidelink DRB, if the derived PC5 RLC channel configuration derived by per-SLRB QoS profile of this end-to-end sidelink DRB is changed according to he configuration in SidelinkPreconfigNR;derive the PC5 RLC channel configuration based on per-SLRB QoS profile of this end-to-end sidelink DRB according to the configuration in SidelinkPreconfigNR; 3> if the PC5 RLC channel configuration derived by per-SLRB QoS profile of this end-to-end sidelink DRB is changed: 4> perform the PC5 Relay RLC channel release or addition/modification according to the derived PC5 RLC channel configuration as specified in 5.8.9.7.1 or 5.8.9.7.2; 4> consider the PC5 RLC channel applying the configuration derived by per-SLRB QoS profile of this end-to-end sidelink DRB as the egress PC5 relay RLC channel; 4> associate this end-to-end sidelink DRB with the PC5 RLC channel; 4> reconfigure the SRAP entity with the mapping between the end-to-end sidelink DRB and the egress PC5 relay RLC channel for the sidelink DRB. |  |
| OPPO | 5.8.9.3a | 2> discard the rest of the end-to-end NR sidelink communication related radio resources and configuration for this end-to-end PC5 connection, including local ID pair in SRAP configuration; | It is not clear what “rest of” and “radio resources” mean, prefer the original wording |  |
| OPPO | 5.8.9.3b | 2> discard rest of the the end-to-end NR sidelink communication related radio resources and configuration for this end-to-end PC5 connection, including end-to-end SRB/DRB related configuration, QoS related configuration, SRAP configuration; | It is not clear what “rest of” and “radio resources” mean, prefer the original wording |  |
| OPPO | 5.8.9.5 | 3> if MCG transmission is not suspended; 4> initiate the indirect path failure information procedure as specified in 5. 7. 3c to report indirect path failure; | 3> if neither MCG transmission nor indirect path transmission is suspended; 4> initiate the indirect path failure information procedure as specified in 5. 7. 3c to report indirect path failure; |  |
| OPPO | 5.8.9.5a | 2> discard all the left radio resources and the NR sidelink communication related configuration for this end-to-end PC5-RRC connection, including local ID pair in SRAP configuration; | It is not clear what “all the left radio resources” mean, prefer the original wording |  |
| OPPO | 5.8.9.7.1 | ##### 5.8.9.7.1 PC5 Relay RLC channel release The UE shall: 1> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or 1> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR: 2> for each SL-RLC-ChannelID in sl-RLC-ChannelToReleaseList received in sl-ConfigDedicatedNR within RRCReconfiguration, or for each SL-RLC-ChannelID included in the received sl-RLC-ChannelToReleaseListPC5 that is part of the current UE sidelink configuration: 3> release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID; 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB release as specified in 5.8.9.1a.1.2; or 1> if the PC5 Relay RLC channel release was triggered by end-to-end DRB modification as specified in 5.8.9.1a.2.2: 2> if the PC5 Relay RLC channel release was triggered due to per-hop PC5 link failure; or 2> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR; 3> release the RLC entity and the corresponding logical channel associated with the end-to-end DRB; 1> if the PC5 Relay RLC channel release was triggered for a specific destination by upper layers as specified in 5.8.9.5, or due to sidelink RLF as specified in 5.8.9.3: 2> release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID of the specific destination; | The new added 3 bullets can be covered by existing condition so no need for this change: 2> if the PC5 Relay RLC channel release was triggered due to per-hop PC5 link failure; or=> can be covered by the existing third 1> condition 2> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or => can be covered by the existing first 1> condition 2> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR; => can be covered by the second 1> condition |  |
| Apple | 6.6.1 | -- ASN1START -- TAG-UEINFORMATIONREQUESTSIDELINK-START UEInformationRequestSidelink-r18 ::= SEQUENCE { rrc-TransactionIdentifier-r18 RRC-TransactionIdentifier, criticalExtensions CHOICE { ueInformationRequestSidelink-r18 UEInformationRequestSidelink-r18-IEs, criticalExtensionsFuture SEQUENCE {} } } UEInformationRequestSidelink-r18-IEs ::= SEQUENCE { sl-E2E-QoS-InfoListPC5-r18 SEQUENCE (SIZE (1.. maxNrofSL-Dest-r16)) OF SL-E2E-QoS-InfoPC5-r18 OPTIONAL, -- Need N lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } SL-E2E-QoS-InfoPC5-r18 ::= SEQUENCE { sl-DestinationIdentityRemoteUE-r18 SL-DestinationIdentity-r16, sl-E2E-SLRB-Index-r18 SLRB-PC5-ConfigIndex-r16, sl-QoS-InfoList-r18 SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16 } -- TAG-UEINFORMATIONREQUESTSIDELINK-STOP -- ASN1STOP | Several comments on the changes made here: First, the “maxNrofSL-Dest-r16” variable is only 32. It does not be able to support all end-to-end SLRB, we need to change as below: sl-E2E-QoS-InfoListPC5-r18 SEQUENCE (SIZE (1.. maxNrofSL-Dest-r16 maxNrofSLRB-r16)) OF SL-E2E-QoS-InfoPC5-r18 OPTIONAL, -- Need N Second, in the field description below, the name “sl-E2E-QoS-ConnectionListPC5 “ needs to be updated to its new changed name. Also, the description needs to explain this is a for QoS info of an end-to-end SLRB. Finally, “sl-E2E-SLRB-Index-r18“ is a new field, so the field description needs to insert a new row to explain this new field. |  |
| Apple | 5.8.3.3 | NOTE X: If UE is in RRC_CONNECTED, how to merge the split per-flow QoS on the second hop into a per-SLRB level QoS for SUI reporting is up to relay UE implementation. | For this new NOTE, we need to add “If UE is L2 U2U relay UE” part, because the NOTE is not applicable to any other UEs involved in SUI procedure. Also, better to change SUI to the full name of the message SIdleinkUEInformationNR |  |
| Apple | 5.8.9.1.2 | NOTE 3: If UE is in RRC_IDLE or in RRC_INACTIVE or out of coverage, how to merge the split per-flow QoS on the first/second hop into a per-SLRB level QoS for RLC channel configuration derivation is up to UE implementation. | For this NOTE, some changes as below to limit its applicability to L2 U2U case. NOTE 3: If a L2 U2U remote UE or L2 U2U relay UE is in RRC_IDLE or in RRC_INACTIVE or out of coverage, how to merge the split per-flow QoS on the first/second hop into a per-SLRB level QoS for RLC channel configuration derivation is up to UE implementation. |  |
| Apple | 5.8.9.11.3 | 3> perform QoS split based on the sl-QoS-InfoList for each QoS flow to decide the split PDB value for each PC5 hop; 3> set the contents of UEInformationResponseSidelink message as follows: 4> set sl-SplitQoS-InfoListPC5 to include the split PDB value for each QoS flow on the first PC5 hop between L2 U2U Relay UE and L2 U2U Remote UE; 3> submit the UEInformationResponseSidelink message to lower layers for transmission; NOTE: It is left to Relay UE implementation on how to split the PDB. | For the steps acted upon the QoS split, it is important to connect this to the procedure defined for IDLE/INACTVE and CONNECTED relay UE behaviour in other clauses correspondingly: I suggest to add more level-3 bullets as below. 3> merge the split per-flow QoS on the second hop into a per-SLRB level QoS 3> If relay UE is in IDLE/INACIVE 4> Trigger the procedure in 5.8.9.1.2 to configure the PC5 relay RLC channel(s) for the 2nd hop between the relay UE and peer remote UE , if needed; 3> If relay UE is in CONNECTED 4> Trigger the SIdelinkUEInfromationNR procedure as specified in 5.8.3.2 to report per-SLRB level QoS to the sevring gNB of the relay UE; |  |
| Apple | 5.8.9.1a.2.2 | 2> for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE): 3> if the UE is in RRC_CONNECTED: 4> associate this end-to-end sidelink DRB with the PC5 RLC channel indicated by sl-EgressRLC-ChannelPC5 included in sl-ConfigDedicatedNR, received from RRCReconfiguration; 3> else if the UE is in RRC_IDLE or RRC_INACTIVE: 4> derive the PC5 RLC channel configuration based on per-SLRB QoS profile of this end-to-end sidelink DRB according to the configuration in SIB12; 3> else if the UE is out of coverage: 4> derive the PC5 RLC channel configuration based on per-SLRB QoS profile of this end-to-end sidelink DRB according to the configuration in SidelinkPreconfigNR; 3> perform the PC5 Relay RLC channel addition/modification according to the derived PC5 RLC channel configuration as specified in 5.8.9.7.2, if needed; 3> consider the PC5 RLC channel applying the configuration derived by per-SLRB QoS profile of this end-to-end sidelink DRB as the egress PC5 relay RLC channel; 3> associate this end-to-end sidelink DRB with the PC5 RLC channel and configure the mapping between the end-to-end sidelink DRB and the egress PC5 relay RLC channel to SRAP; | First, better to align the name PC5 Relay RLC channel Second, “based on per-SLRB QoS profile of this end-to-end sidelink DRB” is very confusing, the derivation is not based on end-to-end QoS of the end-to-end DRB, but the after-split per-SLRB level QoS in each corresponding hop. So, the suggestion to modify text as below: Change all “per-SLRB QoS profile of this end-to-end sidelink DRB “ to “ per-SLRB level QoS of either 1st hop (for L2 U2U remote UE) or 2nd hop (for L2 U2U relay UE) for this end-to-end Sidelink DRB” |  |
| Apple | 6.6.1 | -- ASN1START -- TAG-UEINFORMATIONREQUESTSIDELINK-START UEInformationRequestSidelink-r18 ::= SEQUENCE { rrc-TransactionIdentifier-r18 RRC-TransactionIdentifier, criticalExtensions CHOICE { ueInformationRequestSidelink-r18 UEInformationRequestSidelink-r18-IEs, criticalExtensionsFuture SEQUENCE {} } } UEInformationRequestSidelink-r18-IEs ::= SEQUENCE { sl-E2E-QoS-InfoListPC5-r18 SEQUENCE (SIZE (1.. maxNrofSL-Dest-r16)) OF SL-E2E-QoS-InfoPC5-r18 OPTIONAL, -- Need N lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } SL-E2E-QoS-InfoPC5-r18 ::= SEQUENCE { sl-DestinationIdentityRemoteUE-r18 SL-DestinationIdentity-r16, sl-E2E-SLRB-Index-r18 SLRB-PC5-ConfigIndex-r16, sl-QoS-InfoList-r18 SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16 } -- TAG-UEINFORMATIONREQUESTSIDELINK-STOP -- ASN1STOP | Several comments on the changes made here: First, the “maxNrofSL-Dest-r16” variable is only 32. It does not be able to support all end-to-end SLRB, we need to change as below: sl-E2E-QoS-InfoListPC5-r18 SEQUENCE (SIZE (1.. maxNrofSL-Dest-r16 maxNrofSLRB-r16)) OF SL-E2E-QoS-InfoPC5-r18 OPTIONAL, -- Need N Second, in the field description below, the name “sl-E2E-QoS-ConnectionListPC5 “ needs to be updated to its new changed name. Also, the description needs to explain this is a for QoS info of an end-to-end SLRB. Finally, “sl-E2E-SLRB-Index-r18“ is a new field, so the field description needs to insert a new row to explain this new field. |  |

# 2	Comments on RIL list

| Company | Suggested modification or comments | Rapporteur response |
| --- | --- | --- |