3GPP TSG-RAN WG2 Meeting #125bis	R2-240xxxx

Changsha, China, April 15th – 19th, 2024

Agenda item:		7.2.1

Source:				CATT

Title:					[Post125bis][407][POS] Rel-18 positioning LPP CR (CATT)

Document for:	Discussion and Agreement

# 1	Introduction

This is to kick off the email discussion.

[Post125bis][407][POS] Rel-18 positioning LPP CR (CATT)

Scope: Update and check the Rel-18 positioning CR to 37.355.

Intended outcome: Endorsed CR in R2-2403818

Deadline:  Short

# 2	Discussion

Please provide your comments on the LPP running CR in the following table.

| Company | Clause No. | Comments |
| --- | --- | --- |
| Qualcomm | CommonIEsProvideCapabilities | The Table Note should use TAN style. |
|  | CommonIEsRequestLocationInformation | The Table Note should use TAN style. |
|  | nr-PeriodicityAndSlotOffsetTimeWindow-r18 | Need code is changed to 'OR", but 'ON' seems correct. |
|  | nr-SlotOffsetTimeWindow-r18 | Need code should be 'ON' 'OPTIONAL' can be better aligned on a vertical line (add some Tabs); empty line should be deleted. |
|  | nr-PeriodicityAndSlotOffsetTimeWindow field description | Add: If nr-SlotOffsetTimeWindow is present, this field should be absent. |
|  | nr-SlotOffsetTimeWindow field description | If nr-PeriodicityAndSlotOffsetTimeWindow-r18 is present, this field should be absent. delete '-r18' A cleaner solution would be to define the nr-PeriodicityAndSlotOffsetTimeWindow-r18 and nr-SlotOffsetTimeWindow-r18 as a CHOICE (i.e., only one of the two should be present) |
|  | dl-PRS-ResourceSetARP-ErrorCorrelationTime field description | "This field, if present, specifies the DL-PRS-ResourceSet ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS resource set ARP error " DL-PRS-ResourceSet should be 'DL-PRS Resource Set' resource set should be 'Resource Set' |
|  | dl-PRS-ResourceARP-ErrorCorrelationTime field description | "This field, if present, specifies the DL-PRS-Resource ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS resource ARP error " DL-PRS-Resource should be "DL-PRS Resource' resource should be 'Resource' |
|  | NR-PRU-RSCP-MeasElement-r18 | nr-PRU-RSRP-AddSampleMeasurements-r18 SEQUENCE (SIZE (1..nrNumOfSamples-1-r18)) OF INTEGER (0..61) nr-PRU-RSRP-AddSampleMeasurements-r18 SEQUENCE (SIZE (1..nrNumOfSamples-1-r18)) OF INTEGER (0..61) OPTIONAL, -- Need ON nr-PRU-FirstPathRSRP-ResultDiff-AddSampleMeasurements-r18 SEQUENCE (SIZE (1..nrNumOfSamples-1-r18 )) OF INTEGER (0..61) OPTIONAL, -- Need ON OPTIONAL, -- Need ON ASN.1 breaks across multiple lines (should be better aligned along a vertical line) |
|  | NR-DL-TDOA-RequestLocationInformation | "TPs from R2-2403190 are agreeable in principle" jointMeasurementsReq-r18 should be moved out of the BIT STRING (not a measurement) |
|  | NR-Multi-RTT-RequestLocationInformation-r16 | "TPs from R2-2403190 are agreeable in principle" jointMeasurementsReq-r18 should be moved out of the BIT STRING (not a measurement) |
| Lenovo | Coversheet | In the meeting dates the year “2024” is missing. |
|  | Coversheet | The below agreement has been already covered by the LPP capability CR, see [415]. Agreement: Move the two fields posSRS-BWA-RRC-Connected-r18 and posSRS-BWA-IndependentCA-RRC-Connected-r18 from IE SRS-CapabilityPerBand-r16 to SRS-PosResourcesPerBand-r16. |
|  | 6.5.11.5, IE NR-DL-AoD-RequestLocationInformation | The comma and the square brackets should be removed. NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE { nr-AssistanceAvailability-r16 BOOLEAN, nr-DL-AoD-ReportConfig-r16 NR-DL-AoD-ReportConfig-r16, ..., [[ multiMeasInSameReport-r17 ENUMERATED { requested } OPTIONAL -- Need ON ]], [[ ]] } |

# 3	Conclusion

Based on company feedback, the following is proposed:

# 4	References