3GPP TSG-RAN WG2 Meeting #125bis	R2-24xxxxx

Changsha, China, 15th – 19th April 2024

Agenda Item:	7.2.1

Source:	Ericsson

- Title:	 [Post125bis][408][POS] Rel-18 positioning RRC CR (Ericsson)

Document for:	Discussion, Decision

# Introduction

This is to kick off the email discussion.

[Post125bis][408][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Update and check the Rel-18 positioning CR to 38.331.

Intended outcome: Endorsed CR in R2-2403819

Deadline:  Short

# 2	RIL O800

RIL O800 raises the question: Have we made agreement that sl-RxPool and/or sl-PRS-RxPool should be only included in handover message but not other RRC dedicated message?

The description for SL Positioning mimics the SL Communication description:

Below is what is captured for SL Communication:

### 5.8.7	Sidelink communication reception

A UE capable of NR sidelink communication that is configured by upper layers to receive NR sidelink communication shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in RRCReconfiguration message or sl-FreqInfoList/sl-FreqInfoListSizeExt included in SIB12:

3>	if the UE is configured with sl-RxPool included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

And for SL Positioning

#### 5.8.18.2	NR sidelink positioning measurement

A UE capable of NR sidelink positioning that is configured by upper layers for performingSL-PRS measurement:

1>	if the conditions for NR sidelink positioning operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink positioning is included in sl-FreqInfoToAddModList in RRCReconfiguration message or sl-FreqInfoList included in SIB12 or SIB23:

3>	if the UE is configured with sl-RxPool and/or sl-PRS-RxPool included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

Companies are requested to provide their view on O800. If the current implementation is fine or if any change is needed and if yes what are the suggested changes:

Please provide your opinion/comments on the O800.

Option 1: Current CR is fine, no change needed.

Option 2: There may be other message that may be impacted, or the current changes are not correct.

| Company Name | Option 1/2 | Comments |
| --- | --- | --- |

# 3	Discussion

## 3.1	LPHAP

Please provide your comments on the LPHAP changes

| Company Name | Comments |
| --- | --- |

## 3.2	Sidelink

Please provide your comments on Sidelink changes.

| Company Name | Comments |
| --- | --- |

## 3.3	Bandwidth Aggregation

Please provide your comments on the bandwidth aggregation changes.

| Company Name | Comments |
| --- | --- |

## 3.4	REDCAP CR

Please provide your comments on the RedCap changes

| Company Name | Comments |
| --- | --- |

## 3.5	Any other comments

Please provide any other comments below.

| Company Name | Comments |
| --- | --- |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

# References