3GPP TSG-RAN WG2 #126	R2-24xxxxx

Fukuoka Japan May 20th – 24th, 2024

Agenda Item:	8.3.2.1

Source:	OPPO(Rapporteur)

Title:	Summary of [POST126][031][AIMob] Simulations (OPPO)

Document for:	Discussion, Decision

Introduction

This the summary of following post email discussion:

[POST126][031][AIMob] Simulations (Oppo)

Intended outcome: Agree to evaluation documentation and small simulation related FFS (needed to start simulation evaluation for August meeting)

Endorse Skeleton TR

Deadline:  short

| Company | Delegate | Email address |
| --- | --- | --- |
| NTT DOCOMO | Xin Wang | wangx@docomolabs-beijing.com.cn |
| Ericsson | Cecilia Eklöf | cecilia.eklof@ericsson.com |
| Samsung | Taeseop Lee | taeseop.lee@samsung.com |
| vivo | Xiang Pan | panxiang@vivo.com |
| Apple | Sasha Sirotkin | ssirotkin@apple.com |
| CATT | Tangxun | tangxun@catt.cn |
| Xiaomi | Xing Yang | Yangxing1@xiaomi.com |

Discussion

Simulation report template

It is expected simulation result on RRM measurement use case will be submitted to RAN2#127 meeting for further evaluation after the summer. In order to document simulation results reported by each company, a report template is necessary to be aligned among companies. In RAN2#126, contributions [1] and [2] proposed their understanding of how such a template can be. Table 1 lists the parameters based on agreements made so far.

| Report parameters |  | Company A | …… |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) |  |  |
|  | UE speed (30,60,90,120 Km/h) |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) |  |  |
|  | Measurement reduction rate(50%,…Note2) |  |  |
|  | Prediction window (?ms,… Note 3) |  |  |
|  | Any other parameters (Note 4) |  |  |
| Data Size (Sample number) | Training/validity |  |  |
|  | Testing |  |  |
| AI/ML model input/output | Model input (Note 5) |  |  |
|  | Model output |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) |  |  |
|  | Model complexity in a number of parameters(M) |  |  |
|  | Model complexity in model size (e.g. Mbyte) |  |  |
|  | Computational complexity [FLOPs] |  |  |
| Metrics | Average L3 cell level RSRP difference (dBm) |  |  |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) |  |  |
| ... | ... |  |  |

Table 1

Note1: Only applicable for FR1 to FR1 inter-frequency prediction. It should be N/A, if not applicable

Note2: Only applicable for intra-frequency prediction, either temporal domain case B or spatial domain. It should be N/A, if not applicable

Note3: Only applicable for intra-frequency temporal domain case A. It should be N/A, if not applicable

Note4: This could be any other parameter e.g., Inter-frequency shadow fading correction (e.g. full, partial, no), Number of configured beams, observation window(ms) etc.

Note5: Apart from input of RRM sub case 1,2,3, any other input e.g. L1 filtering for L1 beam measurement, UE location are also captured here

For prediction window, companies seem to be fine to align at least one value. And up to submitted simulation result, it is open for modification in RAN2#127 meeting. During [AT126][030][AIMob] discussion people seems to agree with rapporteur that it should be multiple times of sample period. Considering the FR1 and FR2 channel will be quite different and agreed sample period is also different, we’d better assume different prediction window for them also.

Question 1: What value(s) do you recommend for prediction window for RRM measurement use case for FR1 and FR2 respectively?

| Company | comments |
| --- | --- |
| NTT DOCOMO | We suggest using the value N*Measurement Period, where the Measurement Period is decided by the following table (Table 3). Considering the time span of the whole HO procedure, we suggest at least considering a long prediction window case to check the capability of AI/ML, e.g., N=5. |
| Ericsson | Agree with DOCOMO (e.g. max value of N=5). |
| Samsung | Considering that the RRM prediction results can be used to prepare the HO in advance, the length of prediction window needs to be aligned with the typical HO preparation time (e.g., 40 ~ 60msec) between source/target gNB. Too long prediction window may need to be considered later. Our recommendation is - FR1: 40ms or 80ms (1x or 2x sample period) - FR2: 40ms or 60ms (2x or 3x sample period) |
| vivo | For BM-Case 2 in TR 38.843, the prediction window is 80ms/160ms/320ms/640ms/800ms /others. One straightforward way is down-selecting value(s) among them. As we already agreed that measurement event prediction can be based on RRM measurement prediction result, the prediction window of RRM prediction should cover the length of TTT. Currently, one typical value of TTT is 320ms. Therefore, 320ms can be baseline for the prediction window and can be used for both FR1 and FR2 evaluation. Besides, there can be multiple prediction results within the prediction window, e.g., every 80ms. In addition, short time of stay is another KPI that is expected to be optimized with AI/ML-based mobility, whose typical value is 1s. Specifically, if the quality of the target cell is predicted to turn unacceptable after UE completes RACH to the cell, HO to the target cell should not be triggered to avoid the short time of stay or ping-pong handover. Therefore, 1s can be another optional value for the prediction window. In summary, we propose: For the prediction window, 320ms is the baseline and there can be multiple prediction results within the prediction window, e.g., every 80ms. In addition, 1s can be optional. |
| Apple | Considering this is a very preliminary number anyways, we don’t have a strong view. Having said that, we shouldn’t start from a number which is too high (and 320ms appears too high for us at this sta |
| CATT | As the input for measurement event, the RRM measurement prediction result should cover some of the samples for L3 measurement. Hence, we agree with DOCOMO that value for the prediction window is the value N*Measurement Period. |
| Xiaomi | We understand it’s related to the targeted goal. If the goal is to improve HO performance, the prediction window can be aligned with HO preparation time, which can be small, e.g. 40 or 80 ms. If the goal is to reduce measurement, the prediction window can be aligned with measurement period, e.g. N* measurement period. |

Question 2: Apart from parameters listed in Table 1, what other parameter(s) need be reported? If yes, please provide detail parameter, corresponding description and justification.

| Company | comments |
| --- | --- |
| Ericsson | Historical observation window length (for L1 measurements) for frequency and temporal prediction. It would be good the companies provide the information about the observation window length. It can be defined as the number of samples used as input to the model. |
| vivo | - applicable condition Unlike AI beam, AI mobility evaluation is not limited to the same cell, so the model can be a per-cell model (e.g., the training data is collected from UEs in the same cell) or a per-area model (e.g., the training data is collected from UEs in the whole simulation area). For the latter case, to achieve acceptable accuracy, the model size may be quite large. Therefore, the applicable condition (validity area, e.g., per cell or per area) of the model needs to be reported so that we can fairly compare model performance and model complexity. - HO parameter The handover parameters/handover strategy will have an impact on the distribution of the dataset. For instance, if the A3 Offset is set as a higher value, the UE may experience lower RSRP of the serving cell at the cell edge. To have similar distributions of RSRP, we propose the handover parameters should also be reported or we can just align a set of handover parameters (e.g., HO parameters in TR 36.839). |
| Apple | - Agree with E/// to report observation window - We acknowledge vivo’s comment on cell-specific vs. general models; if a company uses anything but a single model (e.g. multiple cell specific models), this needs to be reported - Other than the above too we don’t think anything else is needed |

Question 3: For parameters in Table 1, any further comments?

| Company | comments |
| --- | --- |
| NTT DOCOMO | For AI/ML input and output entry in Table 1, we suggest adding a note that information about the cluster-based approach, including the numbers of input and output cells and their relations, can be reported there to capture the agreements on the cluster-based approach during the last meeting. Regarding complexity, we suggest reporting the per-cell values for the cluster-based approach since the per-cell approach may require the model to run multiple times to generate the prediction for all cells concerned. For a fair comparison, the normalized value with respect to the output cell number should be reported. |
| Ericsson | Agree with DOCOMO. |
| Samsung | Agree with NTT DOCOMO |
| vivo | The current measurement reduction rate is only applicable for intra-frequency prediction. At the last meeting, FR1 to FR1 inter-frequency (frequency domain) is set as high priority for measurement reduction as well. Therefore, the definition and suggested value of measurement reduction rate for inter-frequency prediction should also be provided. |
| Apple | - How “model output” is different from “metric”? - In RAN#126 we have only defined the cluster-based approach, we have not agreed to evaluate it. Therefore, we shall not explicitly mention it. |
| CATT | Agree with DOCOMO. |

RRC parameters

Few parameters are left not agreed during [AT126][030][AIMob] discussion as following:

| L3 filtering parameter for both FR1 and FR2 | Recommended value |
| --- | --- |
| FR1 FilterCoefficient | 4 |
| FR2 FilterCoefficient(Note 6) | 4 |

Table 2

| Measurement period | Recommended value |
| --- | --- |
| FR1 to FR1 intra-frequency w.o. gap | 200ms |
| FR1 to FR1 inter-frequency with gap | 120ms |
| FR2 to FR2 intra-frequency w.o. gap | 480ms |

Table 3

| Consolidation parameter | Recommended value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1(Note 7) | -156dbm[2] |
| absThreshSS-BlocksConsolidation for FR2(Note 7) | -156dbm[2] |

Table 4

Note 6,7: These two parameters are added by rapporteur in case they could be different between FR1 and FR2

Note 7: the recommended value from [2] is just for discussion purpose.

If you have better recommendation, please provide your value(s):

| Company | Recommended values |
| --- | --- |
| NTT DOCOMO | For the measurement period of FR1-to-FR1 inter-frequency with gap (in Table 3), there is no 120ms configuration for the measurement gap repetition period (MGRP) in TS38.331. Although the measurement period does not mean the same value should be used for MGRP, we think it is beneficial for the future study (e.g., monitoring, data collection, etc) if an aligned value can be adopted. Therefore, we suggest using 160ms, which is also closer to the value we used for cases w/o MG. We are fine with other parameters. |
| Ericsson | We are fine with the proposed values. |
| Samsung | -156 dBm of absThreshSS-BlocksConsolidation is too small to measure in our view. The main scenario of this measurement and prediction is mobility-related decision e.g. handover. absThreshSS-BlocksConsolidation should be a typical value indicating the cell could be a serving cell. The exact value could be different between frequencies. But we prefer a common threshold for both FR1 and FR2. Our recommendation of absThreshSS-BlocksConsolidation is -100 dBm or similar value. We prefer to have a common measurement period if possible. 200ms can be used for both intra- and inter-frequency scenarios. For FR2, we may use the minimum value (400ms), similar to FR1. |
| vivo | From our understanding, a measurement period of 200/480ms means that we will get a L3 filtered measurement result every 200/480ms. However, the granularity seems a bit large and we think that the measurement period in the simulation should be the same as the L1 sampling period (e.g., 40ms for FR1 and 20ms for FR2). |
| Apple | Agree with Samsung to strive to have a common measurement period. No strong view otherwise. |
| CATT | Regarding the measurement period for FR1 to FR1 inter-frequency with gap, we think it should be derived based on Table 9.3.5-1 “Measurement period for inter-frequency measurements with gaps (Frequency FR1)” in 38.133, instead of Table 9.3.4-3 “Time period for time index detection (Frequency range FR1)” in 38.133 (proposed in [2]). And we suggest using 200ms for this case accordingly. |
| Xiaomi | We are fine with proposed values. |

- TR skeleton

Please provide your comments directly on TR skeleton [3] in the email discussion folder without changing original text.

Reference

R2-2404485, Simulation based evaluation of AIML aided mobility, Ericsson

R2-2404713, Discussion on simulation assumption of RRM measurement, OPPO

R2-2405693	 TR 38.744 Skeleton of AI mobility NR	OPPO	draft TR	Rel-19	38.744	0.0.1	FS_NR_AIML_Mob

R2-2405941	Summary of [POST125bis][021][AIML mobility ] Simulation assumptions and methodology	OPPO	discussion	Rel-19	FS_NR_AIML_Mob	Late