3GPP TSG RAN Meeting #104									       RP-24XXXX

Shanghai, China, June 17-20, 2024

Status Report to TSG

Agenda item:			9.2.2

| WI / SI Name | New SID: Study on solutions for Ambient IoT (Internet of Things) in NR |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: | Performance part: | Testing part: |
| Acronym | FS_Ambient_IoT_solutions |  |  |  |
| Unique ID | 1020085 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234058 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2024 (No change) | Core part: | Performance part: | Testing part: |
| Overall Completion level | Study Item: 40 % | Core part: | Performance Part: | Testing part: |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xiaodong Shen |
|  | Company | CMCC |
|  | Email | shenxiaodong@chinamobile.com |
|  | Name | Matthew Webb |
|  | Company | Huawei |
|  | Email | matthew.webb@huawei.com |
|  | Name | John Humbert |
|  | Company | T-Mobile USA |
|  | Email | John.Humbert2@T-Mobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

Agreement

The Skeleton for TR 38.769 in R1-2401795 is endorsed.

2.1.1.1	Evaluation assumptions and results

RAN1#116bis

Agreement

For R2D link in the coverage evaluation, for device 1

- Budget-Alt1 is used (note: receiver architecture is RF ED)

For D2R link in the coverage evaluation,

- Budget-Alt2 is used.

Agreement

The following scenarios are defined,

- FFS: which of these scenarios will be evaluated.

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | CW inside topology |  | - CW node inside topology 1 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different | Device 1, 2a | Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL) | Same as CW |  |
| D1T1-A2 |  |  | - CW node inside topology 1 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D |  | Same as D1T1-A1 | Same as CW |  |
| D1T1-B | CW outside topology |  | - CW node outside topology 1 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same |  | Case 1-4 (outside topology, UL) | Same as CW |  |
| D1T1-C | No CW |  | - No CW Node. | Device 2b | N/A | UL |  |
| D2T2-A1 | CW inside topology |  | - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW |  |
| D2T2-A2 |  |  | - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R |  | Same as D2T2-A1 | Same as CW |  |
| D2T2-B | CW outside topology |  | - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW |  |
| D2T2-C | No CW |  | - No CW Node. - BS communicates with R | Device 2b | N/A | FFS |  |
| Note: this table is for the case where D2R is in the same spectrum as CW2D. |  |  |  |  |  |  |  |

Agreement

For D1T1,

- InF-DH NLOS model defined in TR38.901 is used for D2R and R2D links as pathloss model in coverage evaluation.

For D2T2,

- InF-DL and InH-Office model defined in TR38.901is used as pathloss model in coverage evaluation,

  - NLOS for D2R and R2D links if InF-DL is used

  - LOS for D2R and R2D links if InH-Office is used

Agreement

The following layout is used for evaluation purpose,

- FFS: CW distribution for D1T1-B and D2T2-B

| Parameter | Assumptions for D1T1 | Assumptions for D2T2 |  |
| --- | --- | --- | --- |
| Scenario | InF-DH | InH-office | InF-DL |
| Hall size | 120x60 m | 120 x50 m | 300x150 m |
| Room height | 10 m | 3m | 10 m |
| Sectorization | None |  |  |
| BS deployment / Intermediate UE dropping | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - L=120m x W=60m; D=20m - BS height = 8 m | - L=120m x W=50m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping | - L=300m x W=150m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping |
| Device distribution | Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area | Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations | Device Height= 1.5m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations |
| Device mobility (horizontal plane only) | 3 kph | 3 kph | 3 kph |

Agreement

In the link level simulation, considering the following channel model,

- For D1T1, TDL-A channel model is used for R2D link and for D2R link for InF-DH scenario.

- For D2T2,

  - TDL-A channel model is used for R2D link and for D2R link if InF scenario is considered

  - TDL-D channel model is used for R2D link and for D2R link if InH-Office scenario is considered

- FFS delay spread for each case.

Agreement

For coverage evaluation, subject to further discussion on which scenarios to evaluate,

- In the case of CW inside topology with ’A2’ scenarios

  - The digital baseband processing of CW self-interference handling is not modelled in link level simulation (LLS). It is included in the link budget analysis by reporting the CW cancellation capability value.

- FFS: In the case of CW outside topology with ‘B’ scenarios or CW inside topology with ’A1’ scenarios

Agreement

The maximum distance targets are set separately for device 1, device 2a, device 2b, respectively

- FFS detailed values and RAN1 can further decide the target within in the range of 10m to 50m after link budget study.

- FFS whether to set different values for different scenarios

Agreement

The table below is agreed (except for the yellow part)

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A1/A2/B/C D2T2-A1/A2/B/C | D1T1-A1/A2/B/C D2T2-A1/A2/B/C |
| [0A1] | CW case | N/A | 1-1/1-2/1-4/2-2/2-3/2-4 |
| [0B] | Device 1/2a/2b | Device 1/2a/2b | Device 1/2a/2b |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: - 2(M) or 4(O) antenna elements for 0.9 GHz For Intermediate UE: - 1(M) or 2(O) | 1 |
| [1E] | Total Tx Power (dBm) | - For BS in DL spectrum for indoor - 33dBm(M), FFS: 38dBm(O), one smaller value [FFS: 23 or 26] dBm(M) - FFS: additional constraints on PSD - FFS: For UE in DL spectrum for indoor - For UL spectrum for indoor, - 23dBm (M) - FFS: 26dBm(O) Other valuesare NOT precluded subject to future discussion. | - For device 1/2a: - D2R-CWRxPower-Alt1: - Company to report CW Tx/Rx power together with CW2D distance (see [1E1]~[1E5]) - D2R-CWRxPower-Alt2: - Balanced MPL/distance (see [1E1]~[1E5], and subject to [1E3] = = [4B]) - For device 2b: - D2R-dev2bTxPower-Alt1: -10 dBm(O) - D2R-dev2bTxPower-Alt2: -20 dBm(M) Other values are NOT precluded subject to future discussion. |
| [1E1] | CW Tx power (dBm) | N/A | - 23dBm for UL spectrum, FFS 26dBm - 33dBm(M), 38dBm (O) for DL spectrum Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | - Company to report, the value equals to - UE Tx ant gain, or - BS Tx ant gain Note: only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | - For D2R-CWRxPower-Alt1: - [Company to report] - For D2R-CWRxPower-Alt2: - Calculated Note: only applicable for device 1/2a |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated Note: only applicable for device 1/2a |
| [1E5] | CW received power (dBm) | N/A | Calculated Note: only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180k(M), 360k(O), 1.08MHz(O) | UL data rate: xx bps FFS: data rate for each case |
| [1G] | Tx antenna gain (dBi) | - For BS for indoor, 6 dBi(M), 2dBi(M) - For intermediate UE, 0 dBi | - For A-IoT device, 0dBi (M), -3dBi (O) |
| [1H] | Ambient IoT backscatter loss (dB) Note: due to, e.g., - impedance mismatch - Modulation factor | N/A | - OOK: Y dB - PSK: X dB Note: Only for device 1 FFS: for device 2a |
| [1J] | FFS: Ambient IoT on-object antenna penalty | - 0.9dB or 10.4 | - 0.9dB or 10.4 |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | - 10 dB (M) - 15 dB (O) Note: Only for device 2a |
| [1N] | FFS: Cable, connector, combiner, body losses, etc. (dB) | FFS | N/A |
| [1M] | EIRP (dBm) | Calculated FFS: any limitation of the EIRP subject to future discussion | Calculated |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | FFS: relation with the transmission bandwidth used for the evaluated channel | - FFS: whether the values are single side-band or double side-band - Note: The value is used for calculating the noise power FFS: relation with the transmission bandwidth used for the evaluated channel |
| [2B1] | FFS: RF CBW (Hz) | FFS: - 10MHz - 20MHz - Other values Note: The value is used for calculating the noise power | N/A |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | FFS: Cable, connector, combiner, body losses, etc. (dB) | N/A | FFS |
| [2D] | Receiver Noise Figure (dB) | FFS: 20dB or 24dB or 30dB for Budget-Alt2 FFS: different values for device architecture | For BS as reader - 5dB For UE as reader - 7dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated | Calculated |
| [2G] | Required SNR | Reported by company | Reported by company |
| [2H] | FFS: Ambient IoT on-object antenna penalty | - 0.9dB or 10.4 | - 0.9dB or 10.4 |
| [2J] | Budget-Alt1/ Budget-Alt2 | For R2D link in the coverage evaluation, for device 1 - Budget-Alt1 is used (note: receiver architecture is RF ED) FFS: device 2 | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | For [monostatic backscatter], FFS - [140dB for BS] - [120dB for UE] For [bistatic backscatter] - Assuming CW has no impact to the receiver sensitivity loss. |
| [2K1] | Remaining CW interference (dB) | N/A | Calculated |
| [2K2] | Receiver sensitivity loss(dB) | N/A | Calculated |
| [2L] | Receiver Sensitivity (dBm) | For Budget-Alt1, - For device 1 (RF-ED), - FFS:{-30dBm ~ -36dBm} - For device 2 if RF-ED is used - FFS - For device 2 if RF-ED is not used - N/A For Budget-Alt2, - Calculated | Calculated Note: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) | TBD | TBD |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB FFS: other values are not precluded | 0 dB FFS: other values are not precluded |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| (4) MPL / distance |  |  |  |
| 4A | MPL (dB) | Calculated | Calculated |
| 4B | Distance (m) | Calculated | Calculated |

<Editor Notes: Note 1 will be updated once the table has stabilized >

Note1: calculated values in the Table XXXX are derived according to the followings,

- 1E

  - For D2R, and device 1/2(backscatter), whether this value is need (not regarded as an input variable but regarded as indirect variable), or based on backscatter activation power threshold

- 1M

  - For R2D, 1M=1E+1G

  - For D2R,

    - Device 1: 1M=1E+1G-1H-1J+1L

    - Device 2a: 1M=1E+1G-1J+1K+1L

    - Device 2b: 1M=1E+1G-1J+1L

- 2F: 2F=2E+2D+lin2dB([2B])

- 2L

  - For R2D and Budget-Alt1, [2L] = [2H]

  - For R2D and Budget-Alt2, [2L] = [2G]+[2F]

  - For D2R and Budget-Alt2, Refer to section [xxx] (Proposal [P4-3])

- 4A

  - 4A=1M+2C-2L-3A-3B+3C+[3D]

- 4B is derived from pathloss model

  - Refer to section [XXX] (Proposal [P4-3-2])

Note2: (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated.

Agreement

For coverage evaluation purpose,

- For scenarios ‘A1’ and ‘A2’,

  - The Device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss.

- For scenarios ‘B’,

  - The Device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value.

    - FFS: CW2D distance (m) value(s)

R1-2403769	[draft] LS on Ambient-IoT evaluation scenarios and assumptions	CMCC, [RAN1]

Decision: The draft LS in R1-2403769 is endorsed with the following changes:

- For the last agreement copied in the LS, remove the green highlight in the second column and delete “note 1” with its yellow highlights.

- Revise the first sentence in the LS as follows:

  - RAN1 has discussed and agreed the following aspects. RAN1 would like to clarify that parts highlighted in yellow are not yet agreed by RAN1.

- Revise the action to RAN4 as follows:

  - RAN1 respectfully asks RAN4 to take the above information into account for coexistence studies and to provide a response if needed.

Final LS is approved in R1-2403782. Note the above additional agreement reached on Friday is added in the LS compare to the endorsed draft LS in R1-2403769.

[Post-116bis-AIoT] Email discussion on Ambient IoT evaluation assumptions from April 23 until April 26 – Xiaodong (CMCC)

- focus on proposals P3.7.1-v1, P3.5.8-v2, P3.2.1-(1)-v2 and P3.5.5-v1 in section 2 of R1-2403768.

Friday decision: Focus on proposal P3.2.4-v1 in section 2 of R1-2403768 is added to the post email discussion.

Proposal#5 (V05r1)

For the R2D LLS for ED,  the following is considered as start point, report followings (as start point).

- CINR/CNR in LLS, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and/or interference (if any) power spectral density in the device ED channel bandwidth.

- signal transmission bandwidth

- ED channel bandwidth

FFS: exact definition of ED channel bandwidth for RF-ED, IF, ZIF receiver

FFS: which and how to report for R2D ZIF receiver and D2R

Proposal#2 (V05r1)

The following table of coverage evaluation assumptions in link level simulation is considered as start point.

-  Other values/options are not precluded and subject to future discussion.

Table: Coverage evaluation assumptions

| Parameters |  | Assumptions |
| --- | --- | --- |
| R2D/D2R common parameters |  |  |
| Carrier frequency |  | Refer to link budget template |
| SCS |  | 15 kHz as baseline |
| Block structure |  | Preamble + payload + CRC, to be reported by companies Blocks as agreed in 9.4.2.3, or other blocks reported by companies |
| Channel model |  | <Editor’s Note: Refer to Proposals in section 3.5.3 will be updated according to the agreements made for channel model> |
| Delay spread |  | [30, 150] ns |
| Device velocity |  | 3 km/h |
| Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| BS | Number of antenna elements | [2 or 4] 2 or 4 |
|  | Number of TXRUs | [2 or 4] 2 or 4 |
| Intermediate UE | Number of antenna elements | [1 or 2] 1 or 2 |
|  | Number of TXRUs | [1 or 2] 1 or 2 |
| Reference data rate |  | [0.1, 1, 5] kbps |
| Message size |  | - D2R: - [FFS: 16, 96, 400 bits] - R2D: - [FFS: 16, 32, 64, 400bits] |
| BLER target |  | 1%, 10% |
| Sampling frequency |  | <Editor’s Note: Refer to Proposals in section 3.5.3 will be updated according to the agreements made for channel model Sampling frequency > |
| Device 1/2a/2b |  | Options are as follows, - Device 1, RF-ED - Device 2a, RF-ED - Device 2b, RF-ED/IF-ED/ZIF <Editor’s Note: will be updated according to agreements from 9.4.1.2> |
| R2D specific parameters |  |  |
| Transmission bandwidth |  | 180 kHz as baseline |
| FFS: RF-ED bandwidth |  | [X MHz] |
| FFS: BB LPF |  | [X]-order Butterworth filter with cutoff frequency at [Y] kHz |
| Waveform |  | OOK waveform generated by OFDM modulator |
| Modulation |  | OOK Companies to report, e.g., OOK-1, OOK-4 with M chips per OFDM symbol |
| Line code |  | Companies to report, e.g., Manchester, PIE |
| FEC |  | No FEC as baseline |
| ADC bit width |  | 1-bit for device 1 4-bit for device 2 |
| Detection/decoding method for Line code |  | Companies to report |
| D2R specific parameters |  |  |
| Transmission bandwidth (w.r.t. D2R data rate) |  | 15 kHz as baseline For Device 1 and 2a, 15 kHz as baseline For Device 2b, [180] kHz as baseline [FFS: 15kHz, 180kHz] |
| Waveform (CW) |  | Companies to report waveform, e.g., unmodulated single tone, multi-tone(multiple unmodulated single tone) |
| Modulation |  | Companies to report modulation, e.g., OOK, BPSK, BFSK |
| Line code |  | Companies to report, e.g., Manchester encoding, FM0 encoding, Miller encoding, no line coding |
| FEC |  | Companies to report, e.g., CC, No FEC |
| ADC bit width |  | Companies to report, e.g., 11-bit |
| D2R receiver |  | FFS: Reader receiver, e.g., coherent receiver / non-coherent receiver |
| Other assumptions |  |  |
| Other assumptions |  | To be reported by company |
| Note: - Companies to report required SINR according to BLER target. |  |  |

RAN1#117

Agreement

In the link level simulation, coherent and non-coherent receiver can be evaluated for D2R receiver.

Agreement

For CW2D pathloss model applied to the D1T1-A1/A2/B and D2T2-A1/A2/B scenarios, using the same pathloss model defined in TR38.901 as used for R2D/D2R.

Agreement

Add Row [0D] in the link budget table as follows,

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| [0D] | Topology/Pathloss model | For D2T2: [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: [0D] InF-DH NLOS | For D2T2: [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: [0D] InF-DH NLOS |

Agreement

Update the link budget table Row [0C] as follows,

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| [0C] | Center frequency (MHz) | [0C]-Alt1: 900MHz (M), [0C]-Alt2: 2GHz (O) | [0C]-Alt1: 900MHz (M), [0C]-Alt2: 2GHz (O) |

Agreement

- For R2D link in the coverage evaluation for device 2,

  - Budget-Alt2 is used if receiver architecture is IF/ZIF ED is used

Agreement

Update the link budget table Row [1G] as follows,

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| [1G] | Tx antenna gain (dBi) | - For BS for indoor, 6 dBi(M), 2dBi(M) - For intermediate UE, 0 dBi | For A-IoT device, 0dBi (M), -3dBi (O) |

Agreement

For the link level simulation,

- An RMS delay spread of 30 ns and [150] ns is considered for TDL-A channel model.

- An RMS delay spread of 30 ns is considered for TDL-D channel model.

Agreement

For the link level simulation in coverage evaluation, {20 bits, 96 bits, 400 bits} are considered for message size.

- Note: companies to report the M value and chip length used for each message size

Agreement

For coverage evaluation,

- In the case of CW outside topology with ‘B’ scenarios or CW inside topology with ’A1’ scenarios

  - The digital baseband processing of CW interference handling is not modelled in link level simulation (LLS). It is included in the link budget analysis by reporting the CW cancellation capability value(s) ([2K] in link budget table).

  - Note1: ’A2’ scenarios have already been agreed.

  - Note2: The study of CW interference cancellation capability value(s) at D2R receiver to be discussed in 9.4.2.4 for all scenarios (and if necessary ask RAN4 about the feasibility)

  - Note3: which scenarios to be evaluated is subject to other discussion.

Agreement

- For R2D link in the coverage evaluation for device 2,

  - Budget-Alt1 is used if receiver architecture is RF ED

  - Budget-Alt2 is used if receiver architecture is IF/ZIF ED

- Note1: this does not preclude to have LLS for device 1 and 2 R2D link with RF-ED if needed.

- Note1b: For device 2 R2D link with RF-ED, Budget-Alt1 is mandatory, Budget-Alt2 is optional.

- Note2: this does not imply all M values are achievable with the sensitivity given by Budget-Alt1 for RF ED

- Note3: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB], exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

Agreement

Update the link budget table Row [3A] as follows,

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| [3A] | Shadow fading margin (dB) | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS |

Agreement

Update the ED bandwidth parameter in link level simulation table as follows:

| R2D specific parameters |  |
| --- | --- |
| ED bandwidth | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. |

Working assumption:

- For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth.

- FFS: receiver bandwidth

- On/off keying backscatter loss is not taken into account in the LLS and is included in link budget table [1H].

Agreement

For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver.

Agreement

The link budget table is updated as follows (the yellow parts are not agreed and will be discussed by email),

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A1/A2/B/C D2T2-A1/A2/B/C | D1T1-A1/A2/B/C D2T2-A1/A2/B/C |
| [0A1] | CW case | N/A | 1-1/1-2/1-4/2-2/2-3/2-4 |
| [0B] | Device 1/2a/2b | Device 1/2a/2b | Device 1/2a/2b |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Topology/Pathloss model | For D2T2: - [0D]-Alt1: InF-DL NLOS - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS | For D2T2: - [0D]-Alt1: InF-DL NLOS - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: - 2(M) or 4(O) antenna elements for 0.9 GHz For Intermediate UE: - 1(M) or 2(O) | 1 |
| [1E] | Total Tx Power (dBm) | - For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), - [1E]-R2D-Alt2: 38dBm(O), - [1E]-R2D-Alt3: 24dBm(M) - Companies to report if PSD constraints are imposed (company to report the condition for applying PSD constraints in Row [5A]: Other notes) - For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) - [1E]-R2D-Alt5:26dBm(O) | - For device 1/2a: - [1E]-D2R-Alt1: (For scenarios ‘B’) - The Device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value and other related factors. - [1E]-D2R-Alt2: (For scenarios ‘A1’ and ‘A2’) - The Device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. - For device 2b: (For scenarios ‘C’) - [1E]-D2R-Alt3: -20 dBm(M) - [1E]-D2R-Alt4: -10 dBm(O) |
| [1E1] | CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ - Report a value from the candidate values [1E]-R2D-Alt1/[1E]-R2D-Alt2/[1E]-R2D-Alt3 from [1E]-R2D if CW in DL spectrum - Report a value from the candidate values [1E]-R2D-Alt4/[1E]-R2D-Alt5 from [1E]-R2D if CW in UL spectrum. Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | - Company to report, the value equals to - UE Tx ant gain, or - BS Tx ant gain Note: only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ - D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario - D2T2-B: - 5m, - 10m, - FFS other values For scenarios ‘A1’ and ‘A2’ - Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note: only applicable for device 1/2a Note: companies to report which value(s) are evaluated. |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1E5] | CW received power (dBm) | N/A | Calculated (see note1) Note: only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) | Refer to LLS table [1a] |
| [1G] | Tx antenna gain (dBi) | - For BS for indoor, 6 dBi(M), 2dBi(M) - For intermediate UE, 0 dBi | - For A-IoT device, 0dBi |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | - OOK: 6 dB - PSK: 0 dB - FSK: Y dB It is applicable for device 1 and 2a Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss. |
| [1J] | Ambient IoT on-object antenna penalty | Not applicable | 0.9dB or 4.7dB |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | - 10 dB (M) - 15 dB (O) Note: Only for device 2a |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | - For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A - For intermediate UE, 1 dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) FFS: any limitation of the EIRP subject to future discussion | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a] [receiver bandwidth?] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver - 20dB, Device 2 - FFS other values For IF/ZIF receiver - 15dB, Device 2 | For BS as reader - 5dB For intermediate UE as reader - 7dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9dB or 4.7dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1/ Budget-Alt2 (see note1) | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. Note: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The impact of a multi-tone CW, e.g., assuming an [X] dB difference, is FFS |
| [2K1] | Remaining CW interference (dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2K2] | Receiver sensitivity loss(dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2L] | Receiver Sensitivity (dBm) | For Budget-Alt1, - For device 1 (RF-ED), for example: - {-30dBm, -36dBm, -40dBm, etc} - For device 2 (RF-ED), for example: - {-40dBm, -45dBm, etc} For Budget-Alt2, - Calculated (see note1) | Calculated (see Note 1) Note: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB FFS: other values are not precluded | 0 dB FFS: other values are not precluded |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

<Editor Notes: Note 1 will be updated once the table has stabilized >

Note1 (for email discussion): calculated values in the Table XXXX are derived according to the followings,

[1M]:

- For R2D,

  - [1M] = [1E] + [1G] - [1N] - FFS: [1J]

- For D2R

  - Device 1:

    - [1M] = [1E] + [1G] - [1H] - [1J]

  - Device 2a:

    - [1M] = [1E] + [1G] + [1K] - [1H] - [1J]

  - Device 2b:

    - [1M] = [1E] + [1G] - [1J]

[2F]:

- [2F] = [2D] + [2E] +lin2dB([2B])

[2G]

- For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth.

[2J]

- For R2D link in the coverage evaluation, for device 1

  - Budget-Alt1 is used (note: receiver architecture is RF ED)

- For R2D link in the coverage evaluation for device 2,

  - Budget-Alt1 is used if receiver architecture is RF ED

  - Budget-Alt2 is used if receiver architecture is IF/ZIF ED

- Note1a: this does not preclude to have LLS for device 1 and 2 R2D link with RF-ED if needed.

- Note1b: For device 2 R2D link with RF-ED, Budget-Alt1 is mandatory, Budget-Alt2 is optional.

- Note1c: this does not imply all M values are achievable with the sensitivity given by Budget-Alt1 for RF ED

- Note1d: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB], exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

[2K1]:

- FFS:

  - Alt1: [2K1] = [1E1] + [1E2] - [2K] or

  - Alt2: [2K1] = [1E1] + [1E2] + [2C] - [2K]

[2K2]:

- 2K2=lin2dB1+dB2lin([2K1])dB2lin([2F])

[2L]:

- For R2D and Budget-Alt2,

  - [2L] = [2G] - lin2dB([2B] / [1F]) + [2F]

  - Note 1e: the term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR.

- For D2R,

  - [2L] = [2G] + [2F] + [2K2], device 1/2a

  - [2L] = [2G] + [2F], device 2b

[4A]

- [4A]=[1M]+[2C]-[2L]-[3A]-[3B]+[3C]+[3D]

- Note 1f: For scenarios ‘A1’ and ‘A2’, The Device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. i.e.,

  - TBC: [4A] = 0.5*([1E1]+[1E2]-2*[3A]-2*[3B]-[1J]-[2L]+[2C]-[1H]) for device 1,

  - TBC: [4A] = 0.5*([1E1]+[1E2]-2*[3A]-2*[3B]-[1J]-[2L]+[2C]+[1K]) for device 2

Note2: (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated.

Proposal (for email discussion)

The link level simulation table is updated as follows,

|  | Parameters |  | Assumptions | Company result1 | Company result 2 |
| --- | --- | --- | --- | --- | --- |
|  | R2D/D2R common parameters |  |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |  |  |
| [0b] | SCS |  | 15 kHz as baseline |  |  |
| [0c] | Block structure |  | Blocks as agreed in 9.4.2.3, or other blocks reported by companies |  |  |
| [0d] | Channel model |  | <Editor’s Note: will be updated according to the agreements made for channel model> |  |  |
| [0e] | Delay spread |  | - An RMS delay spread of 30 ns and [150] ns is considered for TDL-A channel model. - An RMS delay spread of 30 ns is considered for TDL-D channel model. |  |  |
| [0f] | Device velocity |  | 3 km/h |  |  |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |  |  |
| [0h1] | BS | Number of antenna elements | 2 or 4 |  |  |
| [0h2] |  | Number of TXRUs | 2 or 4 |  |  |
| [0j1] | Intermediate UE | Number of antenna elements | 1 or 2 |  |  |
| [0j2] |  | Number of TXRUs | 1 or 2 |  |  |
| [0m] | Reference data rate |  | [0.1, 1, 5] kbps [0.1] kbps (M), [1] kbps (M), [7] kbps (O), [large value] (O) |  |  |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. - Note: companies to report the M value and chip length used for each message size |  |  |
| [0p] | BLER target |  | 1%, 10% |  |  |
| [0q] | Sampling frequency |  | <Editor’s Note: will be updated according to the agreements made for Sampling frequency > Sampling frequency is 1.92 Msps. Initial SFO (Sampling Frequency Offset) (Fe): - [0.1 ~ 1] * 10^5 ppm for device 1, reported by company - [0.1 ~ 1] * 10^4 ppm for device 2, reported by company The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. FFS: Accuracy after clock calibration at least for device 2. FFS: CFO for device 2b. Note: the values are for coverage evaluation purpose. A harmonized design approach for all devices should be considered when utilizing these values in the design. |  |  |
| [0r] | Device 1/2a/2b |  | Options are as follows, - Device 1, RF-ED - Device 2a, RF-ED - Device 2b, RF-ED/IF-ED/ZIF <Editor’s Note: will be updated according to agreements from 9.4.1.2> |  |  |
|  | R2D specific parameters |  |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz as baseline |  |  |
| [1b] | ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. |  |  |
| [1c] | BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth. Companies to report X = {3, 5}. |  |  |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |  |  |
| [1e] | Modulation |  | OOK Companies to report, e.g., OOK-1, OOK-4 with M chips per OFDM symbol |  |  |
| [1f] | Line code |  | Companies to report, e.g., Manchester, PIE |  |  |
| [1g] | FEC |  | No FEC as baseline |  |  |
| [1h] | ADC bit width |  | 1-bit for device 1 4-bit for device 2 |  |  |
| [1j] | Detection/decoding method for Line code |  | Companies to report |  |  |
|  | D2R specific parameters |  |  |  |  |
| [2a1] | Transmission bandwidth (w.r.t. D2R data rate) |  | [FFS: 15kHz, 180kHz] - [2a1]-Alt1: - DSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz (M) and Y kHz (O). - [2a1]-Alt2: - SSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for one sideband, i.e., the total transmission bandwidth for DSB is X kHz (M) and Y kHz (O). - The value of X and Y is as follows, to be down-select from alternative 1 and 2 - Alternative 1: - X = {15 (M), 180 (O)} - Y =180 - Alternative 2: - X and Y reported by companies, - the value may be related to, e.g., - Reference data rate - Coding scheme - Repetition - With or without SFS - SSB or DSB |  |  |
| [2a2] | [OOK/BPSK/BFSK chip rate] |  | Companies to report |  |  |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. - Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value. |  |  |
| [2b] | Waveform (CW) |  | Companies to report waveform, e.g., unmodulated single tone, multi-tone(multiple unmodulated single tone) |  |  |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK, BFSK |  |  |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, FM0 encoding, Miller encoding, no line coding |  |  |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC |  |  |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit |  |  |
| [2j] | D2R receiver |  | FFS: Reader receiver, e.g., coherent receiver / non-coherent receiver Companies to report, e.g., coherent receiver / non-coherent receiver |  |  |
|  | Other assumptions |  |  |  |  |
| [3a] | Other assumptions |  | To be reported by company |  |  |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |  |  |

2.1.1.2	Ambient IoT device architectures

RAN1#116bis

Agreement

Study device 2b architecture w/ RF-ED receiver with following blocks.

- Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter.

- Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks).

- Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc

- Energy storage (e.g., capacitor) stores harvested energy from energy harvester.

- Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply.

- Digital BB logic includes functional blocks like encoder, decoder, controller, etc.

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

- Clock generator provides required clock signal(s).

- Reception related blocks

  - RF BPF filter for improving selectivity.

    - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered.

  - FFS: LNA for improving signal strength and sensitivity of receiver, if present

  - RF envelope detector (RF-ED) detects envelope from RF signal.

  - BB amplifier amplifies BB signal to improve signal strength.

  - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC.

    - Depending on implementation, it may not exist.

  - Comparator or N-bit ADC

- Transmission related blocks

- Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic.

- Digital to Analog Converter (DAC) converts digital signal to analog signal.

- Low pass filter for filtering out undesired signal

- Mixer performs up converting baseband signal to RF range.

- Local oscillator (LO) for carrier frequency generation

  - FFS: PLL/FLL

- FFS: Power amplifier (PA) amplifies tx signal, if present

- Details on transmitter related blocks depends on tx waveform/modulation.

Agreement

Further study reflection amplifier for Device 2a, considering following aspects:

- Types of reflection amplifier

  - Uni-directional/one-way (for D2R)

  - Bi-directional/two-way (for both R2D and D2R)

    - FFS: switching loss (if applicable)

- One-way Amplification Gain

  - E.g. [10, 15, 25] dB

  - Considering stability, operating frequency, and power consumption characteristics

- Bandwidth

Agreement

Further study the feasibility of large frequency shift (large FS, i.e. between DL/UL spectrum of an FDD band) for device 2a considering at least following aspects.

- Power consumption characteristics

- Frequency shift range and granularity

- Image suppression or SSB backscatter for large FS

- IF carrier frequency accuracy

- Harmonics suppression

Note: the necessity (including applicable potential scenarios) of large FS can still be discussed in other agendas of the SI

Agreement

Study device 2b architecture with ZIF receiver with following blocks.

- Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter.

- Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks).

- Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc

- Energy storage (e.g., capacitor) stores harvested energy from energy harvester.

- Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply.

- Digital BB logic includes functional blocks like encoder, detector, decoder, controller, etc.

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

- Clock generator provides required clock signal(s).

- Local oscillator (LO) for generating carrier frequency for Tx and Rx

  - FFS: PLL/FLL

  - FFS: one LO or separate LOs for Tx and Rx

- Reception related blocks

  - RF BPF filter for improving selectivity.

    - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered.

  - FFS: LNA for improving signal strength and sensitivity of receiver.

  - Mixer down converts RF signal to BB stage.

    - Depending on implementation, there could be one or two mixers for Rx and Tx

  - BB amplifier amplifies BB signal

  - BB LPF can filter out undesired frequency components to improve input signal quality to comparator/ADC.

    - Depending on implementation, it may not exist.

  - Comparator or N-bit ADC

- Transmission related blocks

- Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic.

- Digital to Analog Converter (DAC) converts digital signal to analog signal.

- Low pass filter for filtering out undesired signal

- Mixer performs up converting baseband signal to RF range.

- FFS: Power amplifier (PA) amplifies tx signal, if present

- Details on transmitter related blocks depends on e.g., waveform/modulation, etc

Agreement

Study device 2b architecture with IF-ED receiver with following blocks.

- Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter.

- Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks).

- Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc

- Energy storage (e.g., capacitor) stores harvested energy from energy harvester.

- Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply.

- Digital BB logic includes functional blocks like encoder, decoder, controller, etc.

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

- Clock generator provides required clock signal(s).

- Local oscillator (LO) for generating carrier frequency for Tx, or for generating carrier frequency offset by the IF for Rx

  - FFS: PLL/FLL

  - FFS: one LO or separate LOs for Tx and Rx

- Reception related blocks

  - RF BPF filter for improving selectivity.

    - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered.

  - FFS: LNA for improving signal strength and sensitivity of receiver, if present

  - Mixer down converts RF signal to IF stage.

    - Depending on implementation, there could be one or two mixers for Rx and Tx

  - IF amplifier amplifies IF signal

  - IF filter for filtering out unwanted RF and LO signals

  - IF envelope detector (IF-ED) detects envelope from IF signal.

  - BB amplifier

    - Depending on implementation, one or both of IF amplifier and BB amplifier may exist.

  - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC.

    - Depending on implementation, it may not exist.

  - Comparator or N-bit ADC

  - Note: image rejection is required

- Transmission related blocks

- Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic.

- Digital to Analog Converter (DAC) converts digital signal to analog signal.

- Low pass filter for filtering out undesired signal

- Mixer performs up converting baseband signal to RF range.

- FFS: Power amplifier (PA) amplifies tx signal, if present

- Details on transmitter related blocks depends on e.g., waveform/modulation, etc

RAN1#117

Observation

Reflection amplifier with following characteristics could be considered for device 2a.

- Direction of amplification

  - Uni-directional reflection amplifier (baseline) can amplify backscattered signal in D2R which can improve D2R link budget.

  - Bi-directional amplifier can amplify both signal in R2D and backscatter signal in D2R at least when R2D and D2R are in the same spectrum.

  - Bi-directional amplifier has higher complexity, higher noise figure, and reduced isolation between tx and rx path.

- Amplification gain ranges from 10 to 20dB.

- Power consumption of reflection amplifier is in the range of a tens of uW to 100s of uW.

- Reflection amplifier can operate in FDD frequency bands.

- Reflection amplifier bandwidth can support 10s of MHz.

- Note: reflection amplifier can get unstable when the input power exceeds a certain value, which may be frequency-dependent.

Observation

For large frequency shift:

- Large frequency shift can be used in shifting reflected signal in tens of MHz, e.g., from FDD DL to FDD UL frequency or vice versa.

- Large frequency shift consumes 10s of uW to 100s of uW.

- Large frequency shift is not feasible for device 1.

- Large frequency shift requires a clock for IF generation which is accurate enough to avoid large guard band and interference to adjacent channels/bands.

- Large frequency shift requires image suppression and may require harmonics suppression

  - Note: details of image suppression and harmonics suppression are not discussed in RAN1

- FFS: whether large frequency shift is necessary and feasible for device 2a

Agreement

For study purpose, assume that A-IoT device has a single antenna for both communication (tx/rx) and RF energy harvesting purposes.

Agreement

The following template is used for capturing descriptions on clock/LO.

|  | Purpose | Applicable device types | Clock speed | Power consumption | [Initial clock Accuracy] | [Accuracy after clock sync] | [Clock drift] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose #1 of the clock |  |  |  |  |  |  |  |
| Purpose #N of the clock |  |  |  |  |  |  |  |
| Etc |  |  |  |  |  |  |  |

2.1.1.3	General aspects of physical layer design

RAN1#116bis

Agreement

Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS.

Agreement

Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

Agreement

Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

Agreement

The following bandwidths for D2R are defined for the purpose of the study:

- Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device.

  - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined

- Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R

  - Note: this guard band is not for coexistence with NR/LTE

- If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4.

- Bocc,D2R >= Btx,D2R

  - Possible values of each bandwidth are FFS

Agreement

For D2R, study: Manchester encoding, FM0 encoding, Miller encoding, no line coding.

- FFS: Mapping(s) from bit(s) to line-code codewords

- FFS: How to achieve small frequency shift in baseband and/or FDM(A) among devices

- Aspects to study include:

  - Spectrum shape

  - Complexity

  - Power consumption

  - BER, BLER

  - Resilience to SFO

  - If there is any relation to CFO

Agreement

A-IoT D2R study of FEC includes at least convolutional codes.

- Comparisons are encouraged to compare to the case of no FEC

- FFS details of convolutional codes, such as polynomial(s), shift-register termination, etc.

- FFS if other FEC candidates/methods will be studied.

Agreement

Study

- baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH

- baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH

- FFS: details when different CRC lengths or no CRC may be used

- FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212

Agreement

Study D2R transmission in the physical layer using repetition

- Note: Discussions regarding higher-layer repetitions are up to RAN2.

Agreement

R2D study includes subcarrier spacing of 15 kHz, from the reader perspective, for OFDM-based waveform.

- Inclusion in the study of subcarrier spacing of 30 kHz is FFS.

Agreement

For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among:

- Alt 1: Including 180 kHz, 360 kHz, and FFS other values

- Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s))

- Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s))

Agreement

For R2D CP handling for OFDM based OOK waveform:

- For potential down-selection, study among the following candidate methods

  - Method Type 1: Removal of CP at device without specified transmit-side

    - FFS: How device determines the CP location

    - FFS: Impact on feasibility of device SFO

    - FFS: relation to M, if any

  - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n.

    - FFS: Whether/how to arrange that OOK chips have equal length after CP insertion

    - FFS: relation to M, if any

    - FFS: Detail of relationship to line code codewords

    - FFS: Impact on feasibility of device SFO

  - [Other method types are not precluded]

- Study of the methods should include e.g.:

  - CP impact on R2D timing acquisition, and decoding & performance of PRDCH

  - Reader and device implementation complexities

  - Interference between R2D and NR DL/UL if in the same NR band

  - Spectrum efficiency

Agreement

Study for all devices the following for D2R baseband modulation, for potential down-selection:

- OOK

- Binary PSK

- Binary FSK

  - Strive to identify one variant of Binary FSK to study further

RAN1#117

Agreement

Study the following regarding CP location/length determination for Method Type 1:

  - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols

  - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration

- Companies are encouraged to clarify the CP removal method used and implementation aspects for the device

- Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device

- Companies should report the values of SFO, and SFO detection methods used in evaluations

Agreement

Study the following options regarding subcarrier orthogonality for Method Type 2:

- Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol)

- Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same

  - FFS: whether this alternative applies if CP length is longer than the chip duration

- Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP

- Other potential methods are not precluded

- Alt 2: Method Type 2 does not retain subcarrier orthogonality

- Proponents to bring further details to RAN1#118

- Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24.

- Companies should report the values of SFO, and SFO detection methods used in evaluations

Agreement

Define repetition types for study purposes as follows:

- Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times

- Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times

- Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times

- Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times

  - NOTE: Equivalent to extending the duration of each chip by Rchip times

Agreement

For D2R, study at least block-level and bit-level repetition type 1 and type 2.

Agreement

For R2D evaluation purposes, the R2D waveform for DFT-s-OFDM is generated as follows:

- The time domain OOK signal is the M chips of one OFDM symbol.

- A chip is represented (e.g. upsampled) by L samples

  - Companies to report L

- An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal.

  - Companies to report N’, e.g. N’=128 or equal to X

- Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D.

  - Companies report how to map and report X

- An N-points IDFT is performed to obtain the time domain signal.

  - Companies to report N, and how value was selected

Note: companies report whether/how CP samples are added.

Agreement

The study assumes the following bit to chip mapping for Manchester encoding:

  - bit 0→chips{10}, bit 1→chips{01}

- FFS: Variant of the above for CP handling

2.1.1.4	Frame structure and timing aspects

RAN1#116bis

Agreement

For R2D transmission, if OFDM-based waveform is used, the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

Agreement

To determine or derive the end of PRDCH transmission, study at least following options:

- Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH.

- Option 2: Based on R2D control information.

Agreement

For the reader to acquire the end of PDRCH transmission, study at least following options:

- Option 1: D2R postamble immediately follows the PDRCH

- Option 2: Based on control information

Agreement

For D2R transmission, study the necessity of midamble at least for the purpose of performing timing/frequency tracking or channel estimation or interference estimation, considering at least the following:

- Modulation and Coding schemes, e.g., data modulation, line/channel coding

- Receiving methods, e.g., coherent or non-coherent

- D2R transmission length/packet size

- Midamble overhead

- Timing/frequency accuracy

- Phase accuracy

Agreement

RAN1 study the R2D transmission without midamble as the baseline if Manchester encoding is used.

- FFS the necessity for the R2D transmission with midamble if PIE is used.

RAN1#117

Agreement

Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

Agreement

Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Conclusion

RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2.

Agreement

Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it:

- Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max].

  - FFS: maximum time is common or different for different A-IoT devices

  - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)

- Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min

  - FFS the maximum value(s) for TR2D

2.1.1.5	Downlink and uplink channel/signal aspects

RAN1#116bis

Agreement

For the R2D timing acquisition signal immediately preceding the transmission of a physical channel, study a preamble with at least two parts which includes a start-indicator part and a clock-acquisition part, where the start-indicator part immediately precedes the clock-acquisition part:

- Start-indicator part provides the start of the R2D transmission

  - FFS: Details of start-indicator part

- Clock-acquisition part provides at least the chip synchronization of the subsequent physical channel transmission

  - FFS: Details of clock-acquisition part, e.g. structure, encoding, length, etc.

  - FFS: Methods to determine chip duration of the subsequent physical channel transmission

  - FFS: Other functionalities

- Note: the preamble is considered not to be part of a physical channel

- FFS: other part(s) of the preamble, if any

- FFS: whether the above clock acquisition is sufficient for all devices

- FFS: how to make the preamble compact

Agreement

For D2R, a preamble preceding each PDRCH transmission is studied as the baseline at least for the D2R timing acquisition signal:

- Preamble is not part of PDRCH

- FFS: Other functionalities of the preamble

Agreement

For PRDCH generation at the reader, at least following blocks are studied as the baseline:

- CRC bits are appended if there is non-zero length CRC

  - Note: CRC details discussed in agenda item 9.4.2.1

- Line coding block

- OOK-1/OOK-4 modulation with OFDM waveform generation, including resource mapping

  - FFS details

- Note: Other blocks could be added if agreed

PRDCH generation

Agreement

For PDRCH generation at the device, at least following blocks are studied as the baseline:

- CRC bits are appended if there is non-zero length CRC

  - Note: CRC details discussed in agenda item 9.4.2.1

- Coding

  - Exact coding methods within the coding block, e.g. with/without line coding and/or FEC discussed under agenda 9.4.2.1

  - Note: If no line coding is used, there may be an additional block (e.g. square wave generator) before/after modulation block

- Modulation

- Note: Other blocks could be added if agreed

PDRCH generation

Agreement

Reference signals including at least DMRS, PTRS, CSI-RS/TRS, are not further studied for R2D.

Agreement

Reference signals including DMRS, PTRS, SRS, are not further studied for D2R

- Note: This doesn’t preclude the possibility to study preamble, midamble, postamble for different purposes, e.g. channel/interference estimation and/or proximity determination

Agreement

Proximity determination based on device side measurements is not considered.

RAN1#117

Agreement

For R2D, the only physical channel is PRDCH.

- PRDCH carries any higher-layer payload

- PRDCH carries L1 R2D control information (if defined)

- FFS details of device behaviour(s) for receiving PRDCH

Agreement

For D2R

- PDRCH carries any higher-layer payload

- PDRCH carries L1 D2R control information (if defined)

- Note: PDRCH carries the response agreed at RAN1#116

Agreement

For L1 D2R control information (if defined), the following are not considered for further study:

- CSI feedback

- Autonomous SR

- FFS: Whether any other L1 D2R control information is needed or not

Agreement

Study the following schemes for proximity determination:

- Option 1: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

    - FFS: Details on reception criteria (e.g. either successful or not) at reader and device

- Option 2: Device is determined to be near the reader based on measurements at the reader side

    - FFS: Details on measurement methods

- FFS: Whether/how transmit power of R2D and/or D2R is considered for proximity determination

Agreement

For the start-indicator part of the R2D time acquisition signal, study the two options below:

  - Option 1: ON/OFF pattern i.e. high/low voltage transmission

  - Option 2: OFF pattern, i.e. low voltage transmission

Agreement

For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration

  - FFS: Pattern design to support determination of chip duration

2.1.1.6	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device

RAN1#116bis

Agreement

For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI.

- Two unmodulated single-tones as a starting point

  - FFS: Other number of tones

  - FFS: how large gap is needed between tones

Agreement

For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI.

Agreement

Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering:

- For D2R

  - Reception performance

  - Spectrum utilization of backscattered signal corresponding to the CW waveforms

- CW interference suppression at D2R receiver

  - Including complexity and CW cancellation capability value/range (if any)

  - For scenarios ’A1’, ’A2’ and ’B’

- Relative complexity of CW generation

RAN1#117

Agreement

For the study of characteristics of CW waveforms, the following table is adopted as a template for capturing observations.

Note 1: Further row(s) can be added, if other CW waveform characteristic is agreed.

| CW waveform characteristics | Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform for backscattering | … (if any) |
| --- | --- | --- |
| D2R reception performance |  |  |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms |  |  |
| CW interference suppression at D2R receiver |  |  |
| Relative complexity of CW generation |  |  |
|  | Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node. |  |

Observation

For D2R reception performance,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [X Y] dB frequency diversity gain in a fading channel, at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

Observation

For CW interference suppression at D2R receiver,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform:

  - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction.

    - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

Agreement

For multiple unmodulated single-tone transmitted by one CW node, other number of tones (i.e. >2) is deprioritized.

- Note: other number of tones (i.e. >2) is studied only when obvious gains are provided.

Observation

For relative complexity of CW generation

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform:

  - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

2.1.2	Remaining Open issues

The following opens need to be addressed in RAN1:

- Evaluation assumptions

- Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1].

  - Clause 5.3: Applicable maximum distance target values(s)

  - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs

  - Clause 5.8: 2D distribution of devices

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4]

- Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1]

- Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology.

NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.

NOTE: strive to minimize evaluation cases in RAN1.

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848.

Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects).

- RAN1-led:

For the Ambient IoT DL and UL:

  - Frame structure, synchronization and timing, random access

  - Numerologies, bandwidths, and multiple access

  - Waveforms and modulations

  - Channel coding

  - Downlink channel/signal aspects

  - Uplink channel/signal aspects

  - Scheduling and timing relationships

  - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

For Topology 2, no difference in physical layer design from Topology 1.

2.2	RAN2

2.2.1	Agreements

2.2.1.1	Stage 2 General aspects

RAN2#125bis

Agreements

- Unless explicitly stated all agreements apply to all device types and for both topologies.

- From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

- RAN2 will support two use cases, “inventory” and “command”.  The definition, detailed wording is FFS

- Baseline procedure:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS

Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS

Step C: The device may perform the data communication with the reader as needed,: Details FFS

- We will study the support of both “inventory” and “command” in the same procedure.

- FFS if Initial Trigger Message can also include “command”.

- RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

RAN2#126

Agreements

1	As baseline, the “inventory only” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

2	As baseline, the “inventory and command” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

-	Step C: reader to device data transmission (e.g. the R2D command), and

-	Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.

Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message.  This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

3	From RAN2 point of view we will study “Command only” use case.

FFS the options on how to support it :

Initial trigger message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

2.2.1.2	Protocol and Functionality aspects

RAN2#125bis

Agreements for control plane

1	RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

2	RRM L3 measurement reporting is not supported by Ambient IoT devices.

3	RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

4	Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

5	RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

Agreements for user plane

1	SDAP is not supported for UP protocol stack.

2	PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

3	RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

4	No HARQ and RLC AM

5	FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

6	RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

RAN2#126

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

2.2.1.3	Paging

RAN2#125bis

Agreements

1.	Legacy paging message for device will not be supported.

2.	Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

3.	RAN2 assumes that the device will not support tracking/RAN area update procedure.

4.	For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

RAN2#126

Agreements

1	RAN2 will study the following cases for AIoT paging message:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

- a message containing multiple IDs of A-IoT devices.  Need to confirm the need for this use case based on SA2 discussion.

What device ID and group ID and scenarios is depending on SA2 discussion.

2	AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).  FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

3	From RAN2 perspective, we assume the device can receive as long as there is enough energy.  We will wait for RAN1 further progress on device monitoring details.

2.2.1.3	Random Access

RAN2#125bis

Agreement

1	RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

2	We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

3	Random Access is triggered by the reader

4	Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

5	Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

6	Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

7	For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

RAN2#126

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4	The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5	“Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

1	A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

2.2.2	Remaining Open issues

- RAN2-led:

  - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions.

For example:

    - Paging

    - Random access

    - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope

    - Interactions with upper layers

For functionalities not listed above, they are studied only if found essential.

2.3	RAN3

2.3.1	Agreements

2.3.1.1	RAN Architecture

RAN3#123bis

RAN3 considers both Topologies at the same time looking whether commonalities are applicable.

[Topo1] AIoT RAN node:

Corresponds to the basestation in Figure 4.2.1.1-1 in TR 38.848;

A RAN node providing AIoT radio, and connecting with an AIoT-aware CN node via the XX interface. Details of the AIoT-aware CN node is subject to SA2.

[Topo2] UE Reader:

A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848);

Providing AIoT radio, and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the basestation in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface.

For Topology 1, RAN3 starts with AIoT RAN node being aggregated.

RAN3#124

Agreements of RAN Architecture in RAN3#124 meeting are captured in the following document:

- R3-243962	[TP for TR 38.769] CB:#AIoT1_Architecture, Ericsson, Huawei

2.3.1.2	RAN-CN Interface Impact

RAN3#123bis

AIoT Paging can be used to reach one or more devices for identified AIoT services (e.g., inventory, command).

RAN3#124

Agreements of signaling and procedures for CN-RAN interface in RAN3#124 meeting are captured in the following document:

- R3-243963	Inventory and Command between AIoT CN and AIoT RAS, Huawei, CMCC, Nokia, Ericsson, ZTE, Xiaomi, Qualcomm Incorporated, Samsung, CATT, Lenovo, LG Electronics, NEC

2.3.1.3	Others

RAN3#123bis

Use cases for locating an AIoT device:

-	Find an appropriate “reader” close to the A-IoT device;

-	Find where the A-IoT device is.

Support locating the A-IoT device at “reader” granularity.

RAN3#124

Agreements of locating an Ambient IoT device in RAN3#124 meeting are captured in the following document:

- R3-243964	(TP for TR38.769) Locating Ambient-IoT device, CMCC, ZTE, CATT, Xiaomi, LGE, Huawei, Lenovo, NEC

#### 2.3.2	Remaining Open issues

- RAN3-led:

  - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable:

    - Paging

    - Device context management

    - Data transport

  - Identify RAN architecture aspects.

  - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

2.4	RAN4

2.4.1	Agreements

Agreements in RAN4#110bis meeting are captured in the following document:

- R4-2406714	WF on Ambient IoT in NR, CMCC

Agreements in RAN4#111 meeting are captured in the following documents:

- R4-2410567	WF on co-existence study for ambient IoT and NR/LTE, CMCC

- R4-2410597	WF on impacts of A-IoT on RF requirements, Huawei

- R4-2410596	TP to TR38.769 skeleton for RF part, Huawei

2.4.2	Remaining Open issues

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2400328	Ambient IoT Study Item work plan	CMCC, Huawei, T-Mobile USA

RAN1#116bis

- R1-2401970	Evaluation assumptions and results for Ambient IoT	Ericsson

- R1-2402011	Evaluation methodology and assumptions for Ambient IoT	Huawei, HiSilicon

- R1-2402040	Discussion on evaluation assumptions and results for Ambient IoT devices	FUTUREWEI

- R1-2402072	Evaluation assumptions and results for Ambient IoT	Nokia

- R1-2402105	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum Communications

- R1-2402137	Discussions on deployment scenarios and evaluation assumptions for A-IoT	Intel Corporation

- R1-2402184	Discussion on Ambient IoT evaluations	ZTE, Sanechips

- R1-2402242	Evaluation methodologies assumptions and results for Ambient IoT	vivo

- R1-2402328	Discussion on evaluation assumptions and results for A-IoT	OPPO

- R1-2402383	The evaluation methodology and preliminary results of Ambient IoT	CATT

- R1-2402466	Considerations for evaluation assuptions and results	Samsung

- R1-2402510	Discussion on evaluation assumptions and results for Ambient IoT	China Telecom

- R1-2402565	Discussion on evaluation methodology and assumptions	CMCC

- R1-2402666	Evaluation methodology and assumptions for Ambient IoT	Xiaomi

- R1-2402826	Discussion on ambient IoT evaluation framework	NEC

- R1-2402857	Evaluation assumptions for Ambient IoT	InterDigital, Inc.

- R1-2402881	Views on evaluation assumptions and link budget analysis for AIoT	Apple

- R1-2402946	On evaluation assumptions and results for A-IoT	MediaTek

- R1-2402967	Evaluation assumptions and results for Ambient IoT	Sony

- R1-2403117	Discussion on Ambient IoT evaluation	LG Electronics

- R1-2403194	Evaluation Assumptions and Results	Qualcomm Incorporated

- R1-2403244	Study on evaluation assumptions for Ambient IoT	NTT DOCOMO, INC.

- R1-2403284	Evaluation assumptions for Ambient IoT	Comba

- R1-2403397	Discussion on Evaluation assumption and preliminary results for AIoT	IIT Kanpur, Indian Institute of Technology Madras

- R1-2401971	Ambient IoT device architectures	Ericsson

- R1-2401976	Discussion on ambient IoT device architectures	TCL

- R1-2402012	Ultra low power device architectures for Ambient IoT	Huawei, HiSilicon

- R1-2402041	Discussion on Ambient IoT device architectures	FUTUREWEI

- R1-2402073	Ambient IoT device architectures	Nokia

- R1-2402106	Discussion on Ambient IoT device architectures	Spreadtrum Communications

- R1-2402185	Discussion on Ambient IoT device architectures	ZTE, Sanechips

- R1-2402243	Discussion on Ambient IoT Device architectures	vivo

- R1-2402329	Discussion on device architecture for A-IoT device	OPPO

- R1-2402384	Study of the Ambient IoT devices architecture	CATT

- R1-2402467	Considerations for Ambient-IoT device architectures	Samsung

- R1-2402511	Discussion on Ambient IoT device architectures	China Telecom

- R1-2402566	Discussion on Ambient IoT device architectures	CMCC

- R1-2402667	Discussion on ambient IoT device architectures	Xiaomi

- R1-2402725	Discussion on Ambient IoT device architectures	Honor

- R1-2402827	Device architecture requirements for ambient IoT	NEC

- R1-2402858	Device architectures for Ambient IoT	InterDigital, Inc.

- R1-2402882	Views on device architecture for AIoT	Apple

- R1-2402947	On Ambient IoT device architectures	MediaTek

- R1-2402968	Ambient IoT device architectures	Sony

- R1-2403059	Discussion on Ambient IoT device architectures	CEWiT

- R1-2403102	Discussion on the Ambient IoT device architectures	Lenovo

- R1-2403118	Discussion on Ambient IoT device architectures	LG Electronics

- R1-2403195	Ambient IoT Device Architecture	Qualcomm Incorporated

- R1-2403245	Study on device archtectures for Ambient IoT	NTT DOCOMO, INC.

- R1-2403398	Views on Architecture of Ambient IoT 	IIT Kanpur, Indian Institute of Tech Madras

- R1-2401972	General aspects of physical layer design for Ambient IoT	Ericsson

- R1-2401977	Discussion on general aspects of physical layer design for Ambient IoT	TCL

- R1-2402013	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

- R1-2402042	Discussion on physical layer design for Ambient IoT devices	FUTUREWEI

- R1-2402074	General aspects of physical layer design for Ambient IoT	Nokia

- R1-2402107	Discussion on general aspects of physical layer design for Ambient IoT	Spreadtrum Communications

- R1-2402186	Discussion on general aspects of physical layer design for Ambient IoT	ZTE, Sanechips

- R1-2402244	Discussion on General Aspects of Physical Layer Design	vivo

- R1-2402330	Discussion on general aspects of physical layer design of A-IoT communication	OPPO

- R1-2402385	Discussion on general aspects of physical layer design	CATT

- R1-2402468	Considerations on general aspects of Ambient IoT	Samsung

- R1-2402487	General aspects of physical layer design for Ambient IoT	Panasonic

- R1-2402512	Discussion on general aspects of physical layer design for Ambient IoT	China Telecom

- R1-2402547	Discussion on Physical Layer Design for Ambient-IoT	EURECOM

- R1-2402567	Discussion on general aspects of A-IoT physical layer design	CMCC

- R1-2402668	Discussion on physical layer design of Ambient IoT	Xiaomi

- R1-2402706	Considerations on Some Aspects of Physical Layer Design for Ambient IoT	Continental Automotive

- R1-2402720	Ambient IoT – General aspects of physical layer design, for uplink modulation	Wiliot Ltd.

- R1-2402736	Discussion on general aspects of physical layer design	Sharp

- R1-2402769	Discussion on general aspects of ambient IoT physical layer design	NEC

- R1-2402859	Discussion on general aspects of physical layer design for Ambient IoT	InterDigital, Inc.

- R1-2402883	Views on general physical layer design aspects for AIoT	Apple

- R1-2402948	On general aspects of physical layer design for A-IoT	MediaTek

- R1-2402969	General aspects of physical layer design for Ambient IoT	Sony

- R1-2403020	Discussion on general aspects of physical layer design	ETRI

- R1-2403060	Discussion on General aspects of physical layer design	CEWiT

- R1-2403069	Discussions on general aspects of physical layer design for Ambient IoT	Ruijie Networks Co. Ltd

- R1-2403090	Discussion on general aspects of physical layer design	Google

- R1-2403103	Discussion on the physical layer design aspects for Ambient IoT devices	Lenovo

- R1-2403119	General aspects of Ambient IoT physical layer design	LG Electronics

- R1-2403196	General aspects of physical layer design	Qualcomm Incorporated

- R1-2403246	Study on general aspects of physical layer design for Ambient IoT	NTT DOCOMO, INC.

- R1-2403281	Discussion on general aspects of physical layer design	Comba

- R1-2403309	General aspects of physical layer design for Ambient IoT	ITL

- R1-2403394	Discussion on general aspects of physical layer design for AIoT	IIT Kanpur, Indian Institute of Technology Madras

- R1-2401973	Frame structure and timing aspects for Ambient IoT	Ericsson

- R1-2402014	On frame structure and timing aspects of Ambient IoT	Huawei, HiSilicon

- R1-2402043	Discussion on Frame Structure and Timing Aspects for Ambient IoT	FUTUREWEI

- R1-2402075	Frame structure and timing aspects for Ambient IoT	Nokia

- R1-2402108	Discussion on frame structure and timing aspects for Ambient IoT	Spreadtrum Communications

- R1-2402134	Discussions on frame structure and timing aspects for A-IoT	Intel Corporation

- R1-2402154	Discussion on frame structre and timing aspects for Ambient IoT	BUPT

- Late submission

- R1-2402187	Discussion on frame structure and physical layer procedure for Ambient IoT	ZTE, Sanechips

- R1-2402245	Discussion on Frame structure, random access, scheduling and timing aspects	vivo

- R1-2402331	Discussion on frame structure and timing aspects of A-IoT	OPPO

- R1-2402386	Study of Frame structure and timing aspects for Ambient IoT	CATT

- R1-2402469	Considerations for frame structure and timing aspects	Samsung

- R1-2402513	Discussion on frame structure and timing aspects for Ambient IoT	China Telecom

- R1-2402568	Discussion on frame structure and timing aspects for A-IoT	CMCC

- R1-2402585	Discussion on physical layer procedures for ambient IoT	Lenovo

- R1-2402615	Frame structure and timing aspects of Ambient IoT	InterDigital, Inc.

- R1-2402669	Discussion on frame structre and timing aspects for Ambient IoT	Xiaomi

- R1-2402737	Discussion on frame structure and timing aspects	Sharp

- R1-2402748	Discussion on A-IoT Frame Structure and Timing Aspects	Panasonic

- R1-2402770	Discussion on frame structure and timing for ambient IoT	NEC

- R1-2402796	Discussion on frame structure and timing aspects	Fujitsu

- R1-2402884	Frame structure and timing aspects for Ambient IoT	Apple

- R1-2402949	On frame structure and timing aspects for A-IoT	MediaTek

- R1-2402970	Frame structure and timing aspects for Ambient IoT	Sony

- R1-2403021	Discussion on frame structure and timing aspects	ETRI

- R1-2403038	Discussion on frame structure and timing aspects for AIoT	TCL

- R1-2403042	Discussion on frame structure and timing aspects for Ambient IoT	Comba

- R1-2403061	Discussion on Frame structure and timing aspects	CEWiT

- R1-2403091	Discussion on frame structure and timing aspects	Google

- R1-2403120	Frame structure and timing aspects for Ambient IoT	LG Electronics

- R1-2403197	Frame structure and timing aspects	Qualcomm Incorporated

- R1-2403247	Study on frame structure and timing aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2403373	Discussion on Frame structure and timing aspects for A-IoT	China Unicom

- R1-2403393	Discussion on Frame Structure and Timing Aspects for Ambient IoT	Indian Institute of Tech (M), IIT Kanpur

- R1-2403395	Discussion on Frame structure and timing aspects for AIoT	IIT Kanpur, Indian Institute of Technology Madras

- R1-2401974	Downlink and uplink channel/signal aspects for Ambient IoT	Ericsson

- R1-2401978	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	TCL

- R1-2402015	Physical channels and signals for Ambient IoT	Huawei, HiSilicon

- R1-2402044	Discussion on D2R and R2D Channel/Signal Aspects for Ambient IoT	FUTUREWEI

- R1-2402076	R2D and D2R channel/signal aspects for Ambient IoT	Nokia

- R1-2402109	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	Spreadtrum Communications

- R1-2402135	Discussions on physical channel and signals for A-IoT	Intel Corporation

- R1-2402188	Discussion on downlink/uplink channel/signal  for Ambient IoT	ZTE, Sanechips

- R1-2402246	Discussion on  Downlink and uplink channel/signal aspects	vivo

- R1-2402332	Discussion on downlink and uplink channel/signal aspects for A-IoT	OPPO

- R1-2402387	DL and UL Physical Channels/signals design in support of Ambient IoT devices	CATT

- R1-2402470	Considerations for downlink and uplink channel/signal aspect	Samsung

- R1-2402514	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	China Telecom

- R1-2402569	Discussion on downlink and uplink channel/signal aspets	CMCC

- R1-2402586	Discussion on channel/signal aspects for ambient IoT	Lenovo

- R1-2402616	Downlink and uplink channels aspects of Ambient IoT	InterDigital, Inc.

- R1-2402670	Discussion on downlink and uplink channel_signal aspects for Ambient IoT	Xiaomi

- R1-2402705	Considerations on downlink and uplink channels/signals for A-IoT	Continental Automotive

- R1-2402738	Discussion on downlink and uplink channel/signal aspects	Sharp

- R1-2402771	Discussion on downlink and uplink channel for ambient IoT	NEC

- R1-2402797	Discussion on downlink and uplink channel/signal aspects	Fujitsu

- R1-2402856	Discussion on downlink and uplink channels and signals for A-IoT	Panasonic

- R1-2402885	Views on physical channels/signals and proximity determination for AIoT	Apple

- R1-2402950	On downlink and uplink channel/signal aspects for A-IoT	MediaTek

- R1-2402971	Downlink and uplink physical channel for Ambient IoT	Sony

- R1-2403022	Discussion on downlink and uplink channel/signal aspects for A-IoT	ETRI

- R1-2403043	Discussion on downlink and uplink channel and signal for Ambient IoT	Comba

- R1-2403092	Discussion on downlink and uplink transmission aspects	Google

- R1-2403121	Downlink and uplink channel/signal aspects for Ambient IoT	LG Electronics

- R1-2403198	Downlink and uplink channel/signal aspects	Qualcomm Incorporated

- R1-2403248	Study on downlink and uplink channel/signal aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2403374	Discussion on downlink and uplink channel aspects for A-IoT	China Unicom

- R1-2403396	Discussion on Downlink and Uplink channel/signal aspects for AIoT	IIT Kanpur, Indian Institute of Technology Madras

- R1-2401975	Waveform characteristics of carrier wave provided externally to the Ambient IoT device	Ericsson

- R1-2401979	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	TCL

- R1-2402016	On external carrier wave for backscattering based Ambient IoT device	Huawei, HiSilicon

- R1-2402045	Discussion on External Carrier Waveform Characteristics for Ambient IoT	FUTUREWEI

- R1-2402077	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Nokia

- R1-2402110	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	Spreadtrum Communications

- R1-2402136	Discussions on waveform characteristics of carrier-wave for A-IoT	Intel Corporation

- R1-2402189	Discussion on carrier wave for Ambient IoT	ZTE, Sanechips

- R1-2402247	Discussion on CW waveform and interference handling at AIoT UL receiver	vivo

- R1-2402333	Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device	OPPO

- R1-2402388	Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device	CATT

- R1-2402471	Considerations for waveform characteristics of carrier-wave	Samsung

- R1-2402515	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	China Telecom

- R1-2402570	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CMCC

- R1-2402587	Discussion on external carrier wave for ambient IoT	Lenovo

- R1-2402671	Discussion on waveform characteristics of carrier-wave	Xiaomi

- R1-2402739	Discussion on waveform characteristics of externally provided carrier-wave	Sharp

- R1-2402860	Discussion on carrier-wave for Ambient IoT	InterDigital, Inc.

- R1-2402886	Views on carrier waveform and interference handling for AIoT	Apple

- R1-2402951	On carrier-wave waveform characteristics for A-IoT	MediaTek

- R1-2402972	External carrier wave for Ambient IoT	Sony

- R1-2403002	Discussion on waveform characteristics of carrier-wave for Ambient IoT device	Panasonic

- R1-2403023	Discussion on waveform characteristics of carrier-wave provided externally to the A-IoT device	ETRI

- R1-2403062	Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CEWiT

- R1-2403093	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Google

- R1-2403094	Considerations for waveform characteristics of carrier-wave	Semtech Neuchatel SA

- R1-2403122	Considerations on carrier-wave transmission for Ambient IoT	LG Electronics

- R1-2403199	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Qualcomm Incorporated

- R1-2403249	Study on waveform characteristics of carrier-wave for Ambient IoT	NTT DOCOMO, INC.

- R1-2403399	Discussion on Carrier wave related aspects for AIoT	IIT Kanpur, Indian Institute of Technology Madras

RAN1#117

- R1-2403840	Evaluation assumptions and results for Ambient IoT	Ericsson

- R1-2403858	Discussion on evaluation assumptions and results for Ambient IoT devices	FUTUREWEI

- R1-2403885	Evaluation assumption and preliminary results for Ambient IoT	Tejas Networks Limited

- R1-2403886	Evaluation assumptions and results for Ambient IoT	Nokia

- R1-2403952	Evaluation methodology and assumptions for Ambient IoT	Huawei, HiSilicon

- R1-2404026	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum Communications

- R1-2404115	Considerations for evaluation assumptions and results	Samsung

- R1-2404177	Evaluation methodologies assumptions and results for Ambient IoT	vivo

- R1-2404284	On evaluation assumptions and link budget analysis for AIoT	Apple

- R1-2404401	The evaluation methodology and preliminary results of Ambient IoT	CATT

- R1-2404427	Discussion on evaluation assumptions and results for Ambient IoT	China Telecom

- R1-2404456	Discussion on evaluation methodology and assumptions	CMCC

- R1-2404500	Initial evaluation results for Ambient IoT	Sony

- R1-2404554	Discussion on Ambient IoT evaluations	ZTE, Sanechips

- R1-2404618	Evaluation methodology and assumptions for Ambient IoT	Xiaomi

- R1-2404793	Discussion on ambient IoT evaluation framework	NEC

- R1-2404868	Discussion on evaluation assumptions and results for A-IoT	OPPO

- R1-2404888	Discussion on Ambient IoT evaluation	LG Electronics

- R1-2404939	Discussion on the evaluation assumptions for Ambient IoT devices                	Lenovo

- R1-2404957	Evaluation assumptions for Ambient IoT	InterDigital, Inc.

- R1-2405042	Study on evaluation assumptions for Ambient IoT	NTT DOCOMO, INC.

- R1-2405076	Evaluation assumptions and results	MediaTek Inc.

- R1-2405155	Evaluation Assumptions and Results	Qualcomm Incorporated

- R1-2405214	Evaluation assumptions for Ambient IoT	Comba

- R1-2405296	Evaluation assumption and preliminary results for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2403841	Ambient IoT device architectures	Ericsson

- R1-2403859	Discussion on Rel-19 Ambient IoT device architecture	FUTUREWEI

- R1-2403880	Discussion on ambient IoT device architectures	TCL

- R1-2403887	Ambient IoT device architectures	Nokia

- R1-2403953	Ultra low power device architectures for Ambient IoT	Huawei, HiSilicon

- R1-2404027	Discussion on Ambient IoT device architectures	Spreadtrum Communications

- R1-2404116	Considerations for Ambient-IoT device architectures	Samsung

- R1-2404178	Discussion on Ambient IoT Device architectures	vivo

- R1-2404285	On device architecture for AIoT	Apple

- R1-2404321	Discussion on Ambient-IoT Device Architecture	Everactive

- Late submission

- R1-2404402	Study of the Ambient IoT devices architecture	CATT

- R1-2404428	Discussion on Ambient IoT device architectures	China Telecom

- R1-2404457	Discussion on Ambient IoT device architectures	CMCC

- R1-2404501	Ambient IoT device architectures	Sony

- R1-2404555	Discussion on Ambient IoT device architectures	ZTE, Sanechips

- R1-2404619	Discussion on ambient IoT device architectures	Xiaomi

- R1-2404794	Device architecture requirements for ambient IoT	NEC

- R1-2404869	Discussion on device architecture for A-IoT device	OPPO

- R1-2404889	Discussion on Ambient IoT device architectures	LG Electronics

- R1-2404940	Discussion on the Ambient IoT device architectures	Lenovo

- R1-2404958	Device architectures for Ambient IoT	InterDigital, Inc.

- R1-2405043	Study on device archtectures for Ambient IoT	NTT DOCOMO, INC.

- R1-2405077	Ambient IoT device architectures	MediaTek Inc.

- R1-2405156	Ambient IoT Device Architecture	Qualcomm Incorporated

- R1-2405215	Ambient IoT Device Architecture	Comba

- R1-2405297	Views on Architecture of Ambient IoT 	IIT Kanpur, Indian Institute of Tech (M)

- R1-2403842	General aspects of physical layer design for Ambient IoT	Ericsson

- R1-2403860	Discussion on physical layer design for Rel-19 Ambient IoT devices	FUTUREWEI

- R1-2403881	Discussion on general aspects of physical layer design for Ambient IoT	TCL

- R1-2403888	General aspects of physical layer design for Ambient IoT	Nokia

- R1-2403954	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

- R1-2404005	Discussion on Physical Layer Design for Ambient-IoT	EURECOM

- R1-2404028	Discussion on general aspects of physical layer design for Ambient IoT	Spreadtrum Communications

- R1-2404117	Considerations on general aspects of Ambient IoT	Samsung

- R1-2404179	Discussion on General Aspects of Physical Layer Design	vivo

- R1-2404286	On general physical layer design aspects for AIoT	Apple

- R1-2404345	On General Physical Layer Design Considerations for Ambient IoT (internet of things) Applications	Lekha Wireless Solutions

- Late submission

- R1-2404403	Discussion on general aspects of physical layer design	CATT

- R1-2404429	Discussion on general aspects of physical layer design for Ambient IoT	China Telecom

- R1-2404458	Discussion on general aspects of A-IoT physical layer design	CMCC

- R1-2404502	General aspects of physical layer design for Ambient IoT	Sony

- R1-2404556	Discussion on general aspects of physical layer design for Ambient IoT	ZTE, Sanechips

- R1-2404592	Consideration on general aspects of physical layer	Fujitsu

- R1-2404620	Discussion on physical layer design of Ambient IoT	Xiaomi

- R1-2404674	Discussion on general aspects of ambient IoT physical layer design	NEC

- R1-2404743	General aspects of physical layer design for Ambient IoT	Panasonic

- R1-2404775	Discussion on general aspects of physical layer design	ETRI

- R1-2404870	Discussion on general aspects of physical layer design of A-IoT communication	OPPO

- R1-2404890	General aspects of Ambient IoT physical layer design	LG Electronics

- R1-2404941	Discussion on the physical layer design aspects for Ambient IoT devices	Lenovo

- R1-2404959	Discussion on general aspects of physical layer design for Ambient IoT	InterDigital, Inc.

- R1-2404962	Discussion on general aspects of physical layer design	Sharp

- R1-2405044	Study on general aspects of physical layer design for Ambient IoT	NTT DOCOMO, INC.

- R1-2405078	General aspects of physical layer design	MediaTek Inc.

- R1-2405124	Discussions on general aspects of physical layer design for Ambient IoT	Ruijie Networks Co. Ltd

- R1-2405157	General aspects of physical layer design	Qualcomm Incorporated

- R1-2405216	Discussion on physical layer design for Ambient IoT	Comba

- R1-2405224	General aspects of physical layer design for Ambient IoT	ITL

- R1-2405242	Discussion on General aspects of physical layer design	CEWiT

- R1-2405269	Ambient IoT – General aspects of physical layer design, performance for uplink modulation	Wiliot Ltd.

- R1-2405298	Discussion on General aspects of physical layer design for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2403843	Frame structure and timing aspects for Ambient IoT	Ericsson

- R1-2403861	Frame Structure and Timing Aspects for Ambient IoT	FUTUREWEI

- R1-2403889	Frame structure and timing aspects for Ambient IoT	Nokia

- R1-2403955	On frame structure and timing aspects of Ambient IoT	Huawei, HiSilicon

- R1-2403966	Discussions on frame structure and timing aspects for A-IoT	Intel Corporation

- R1-2404029	Discussion on frame structure and timing aspects for Ambient IoT	Spreadtrum Communications

- R1-2404118	Considerations for frame structure and timing aspects	Samsung

- R1-2404180	Discussion on Frame structure, random access, scheduling and timing aspects	vivo

- R1-2404219	Discussion on frame structure and physical layer procedures for Ambient IoT	Lenovo

- R1-2404287	Frame structure and timing aspects for Ambient IoT	Apple

- R1-2404329	Discussion on frame structure and timing aspects for Ambient IoT	TCL

- R1-2404404	Study of Frame structure and timing aspects for Ambient IoT	CATT

- R1-2404430	Discussion on frame structure and timing aspects for Ambient IoT	China Telecom

- R1-2404459	Discussion on frame structure and timing  aspects for A-IoT	CMCC

- R1-2404503	Frame structure and timing aspects for Ambient IoT	Sony

- R1-2404519	Frame structure and timing aspects of Ambient IoT	InterDigital, Inc.

- R1-2404557	Discussion on frame structure and physical layer procedure for Ambient IoT	ZTE, Sanechips

- R1-2404593	Discussion on frame structure and timing aspects	Fujitsu

- R1-2404596	Discussion on A-IoT Frame Structure and Timing Aspects 	Panasonic

- R1-2404621	Discussion on frame structure and timing aspects for Ambient IoT	Xiaomi

- R1-2404675	Discussion on frame structure and timing for ambient IoT	NEC

- R1-2404734	Discussion on frame structre and timing aspects for Ambient IoT	BUPT

- R1-2404776	Discussion on frame structure and timing aspects	ETRI

- R1-2404798	Some Considerations on Frame Structure and Timing Aspects for A-IoT	Continental Automotive

- R1-2404803	Discussion on Frame Structure and Timing Aspects for Ambient IoT	IIT, Kharagpur

- R1-2404871	Discussion on frame structure and timing aspects of A-IoT communication	OPPO

- R1-2404891	Frame structure and timing aspects for Ambient IoT	LG Electronics

- R1-2404963	Discussion on frame structure and timing aspects	Sharp

- R1-2405045	Study on frame structure and timing aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2405079	Frame structure and timing aspects	MediaTek Inc.

- R1-2405158	Frame structure and timing aspects	Qualcomm Incorporated

- R1-2405183	Discussion on Frame structure and timing aspects for A-IoT	China Unicom

- R1-2405208	Discussion on frame structure and timing aspect	ASUSTeK

- R1-2405217	Discussion on frame structure and timing aspects for Ambient IoT	Comba

- R1-2405243	Discussion on Frame structure and timing aspects	CEWiT

- R1-2405273	Discussion on frame structure and timing aspects	Google

- R1-2403844	Downlink and uplink channel/signal aspects for Ambient IoT	Ericsson

- R1-2403862	D2R and R2D Channel/Signal Aspects for Ambient IoT	FUTUREWEI

- R1-2403882	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	TCL

- R1-2403890	R2D and D2R channel/signal aspects for Ambient IoT	Nokia

- R1-2403956	Physical channels and signals for Ambient IoT	Huawei, HiSilicon

- R1-2403967	Discussions on physical channel and signals for A-IoT	Intel Corporation

- R1-2404030	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	Spreadtrum Communications

- R1-2404119	Considerations for downlink and uplink channel/signal aspect	Samsung

- R1-2404181	Discussion on  Downlink and uplink channel/signal aspects	vivo

- R1-2404220	Discussion on channel/signal aspects for Ambient IoT	Lenovo

- R1-2404288	On physical channels/signals and proximity determination for AIoT	Apple

- R1-2404405	DL and UL Physical Channels/signals design in support of Ambient IoT devices	CATT

- R1-2404431	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	China Telecom

- R1-2404460	Discussion on downlink and uplink channel/signal aspects	CMCC

- R1-2404504	Downlink and uplink physical channel for Ambient IoT	Sony

- R1-2404520	Downlink and uplink channels aspects of Ambient IoT	InterDigital, Inc.

- R1-2404558	Discussion on channel and signal  for Ambient IoT	ZTE, Sanechips

- R1-2404576	Discussion on downlink and uplink channel/signal aspects for A-IoT	HONOR

- R1-2404594	Discussion on downlink and uplink channel/signal aspects	Fujitsu

- R1-2404622	Discussion on downlink and uplink channel_signal aspects for Ambient IoT	Xiaomi

- R1-2404676	Discussion on downlink and uplink channel for ambient IoT	NEC

- R1-2404777	Downlink and uplink channel/signal aspects for A-IoT	ETRI

- R1-2404799	Considerations on Downlink and Uplink Channels/Signals for A-IoT	Continental Automotive

- R1-2404872	Discussion on downlink and uplink channel/signal aspects for A-IoT	OPPO

- R1-2404892	Downlink and uplink channel/signal aspects for Ambient IoT	LG Electronics

- R1-2404901	Discussion on downlink and uplink channels and signals for A-IoT	Panasonic

- R1-2404937	Considerations for downlink and uplink channel/signal aspects	Semtech Neuchatel SA

- R1-2404964	Discussion on downlink and uplink channel/signal aspects	Sharp

- R1-2405046	Study on downlink and uplink channel/signal aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2405080	Downlink and uplink channel/signal aspects	MediaTek Inc.

- R1-2405159	Downlink and uplink channel/signal aspects	Qualcomm Incorporated

- R1-2405184	Discussion on downlink and uplink channel aspects for A-IoT	China Unicom

- R1-2405218	Discussion on downlink and uplink channel and signal for Ambient IoT	Comba

- R1-2405244	Discussion on Downlink and Uplink channel/signal aspects	CEWiT

- R1-2405274	Discussion on downlink and uplink transmission aspects	Google

- R1-2405300	Discussion on Downlink and uplink channel signal aspects for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2403845	Waveform characteristics of carrier wave provided externally to the Ambient IoT device	Ericsson

- R1-2403863	Discussion on External Carrier Waveform Characteristics for Rel-19 Ambient IoT devices	FUTUREWEI

- R1-2403883	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	TCL

- R1-2403891	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Nokia

- R1-2403957	On external carrier wave for backscattering based Ambient IoT device	Huawei, HiSilicon

- R1-2403968	Discussions on waveform characteristics of carrier-wave for A-IoT	Intel Corporation

- R1-2404031	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	Spreadtrum Communications

- R1-2404120	Considerations for Waveform characteristics of carrier-wave	Samsung

- R1-2404182	Discussion on CW waveform and interference handling at AIoT UL receiver	vivo

- R1-2404221	Discussion on external carrier wave for Ambient IoT	Lenovo

- R1-2404289	On carrier waveform and interference handling for AIoT	Apple

- R1-2404406	Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device	CATT

- R1-2404432	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	China Telecom

- R1-2404461	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CMCC

- R1-2404505	External carrier wave for Ambient IoT	Sony

- R1-2404559	Discussion on carrier wave for Ambient IoT	ZTE, Sanechips

- R1-2404623	Discussion on waveform characteristics of carrier-wave	Xiaomi

- R1-2404778	Waveform characteristics of carrier-wave provided externally to the A-IoT device	ETRI

- R1-2404873	Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device	OPPO

- R1-2404893	Considerations on carrier-wave transmission for Ambient IoT	LG Electronics

- R1-2404902	Discussion on waveform characteristics of carrier-wave for Ambient IoT device	Panasonic

- R1-2404938	Considerations for carrier-wave aspects	Semtech Neuchatel SA

- R1-2404960	Discussion on carrier-wave for Ambient IoT	InterDigital, Inc.

- R1-2404965	Discussion on waveform characteristics of externally provided carrier-wave	Sharp

- R1-2405006	Analyses on interference between AIoT and NR	Fujitsu

- R1-2405047	Study on waveform characteristics of carrier-wave for Ambient IoT	NTT DOCOMO, INC.

- R1-2405081	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	MediaTek Inc.

- R1-2405160	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Qualcomm Incorporated

- R1-2405185	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	China Unicom

- R1-2405219	Discussion on waveform characteristics of carrier-wave for Ambient IoT	Comba

- R1-2405245	Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CEWiT

- R1-2405275	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Google

- R1-2405301	Discussion on Carrier wave related aspects for AIoT	IIT Kanpur, Indian Institute of Tech (M)

RAN2#125bis

- R2-2402156	Discussion on general aspects for Ambient IoT	China Telecom

- R2-2402157	Discussion on user plane for Ambient IoT	China Telecom

- R2-2402158	Discussion on random access for Ambient IoT	China Telecom

- R2-2402164	Discussion on access procedure for ambient IOT	Xiaomi

- R2-2402165	Discussion on protocol stack for ambient IOT	Xiaomi

- R2-2402178	Stage 2 overall procedures, baseline assumptions on security and use cases	CATT

- R2-2402179	Discussion on the Control Plane for Ambient IoT	CATT

- R2-2402180	Discussion on Data Transmission and Protocol Stack of A-IoT	CATT

- R2-2402181	Discussion on Paging Functionality of Ambient IoT	CATT

- R2-2402182	Discussion on the Random Access for Ambient IoT	CATT

- R2-2402191	General considerations on Ambient IoT	OPPO

- R2-2402192	Discussion on user-plane aspects for Ambient IoT	OPPO

- R2-2402197	Discussion on control-plane aspects for Ambient IoT	OPPO

- R2-2402198	Discussion on paging procedure for Ambient IoT	OPPO

- R2-2402200	Discussion on random access for Ambient IoT	OPPO

- R2-2402271	Discussions on General Aspect of Ambient IoT	Fujitsu

- R2-2402272	Discussions on User Plane Protocol Stacks	Fujitsu

- R2-2402273	Discussions on paging	Fujitsu

- R2-2402274	Discussions on Random Access	Fujitsu

- R2-2402289	Control Plane for Ambient-IoT	NEC  Corporation

- R2-2402290	User Plane for Ambient-IoT	NEC

- R2-2402322	Considerations on the control plane aspects of the Ambient IOT	Beijing Xiaomi Software Tech

- R2-2402323	Considerations on the general aspects of the Ambient IOT	Beijing Xiaomi Software Tech

- R2-2402344	Discussion on control plane of Ambient IOT	Spreadtrum Communications

- R2-2402345	Discussion on user plane of Ambient IOT	Spreadtrum Communications

- R2-2402346	Discussion on random access of Ambient IOT	Spreadtrum Communications

- R2-2402349	Discussion on general aspects of A-IoT	Spreadtrum Communications

- R2-2402350	Discussion on paging functionality of A-IoT	Spreadtrum Communications

- R2-2402374	Consideration on general aspects for AIoT	ZTE Corporation, Sanechips

- R2-2402376	Consideration on CP functionality for AIoT	ZTE Corporation, Sanechips

- R2-2402377	Consideration on UP functionality for AIoT	ZTE Corporation, Sanechips

- R2-2402378	Consideration on Paging functionality for AIoT	ZTE Corporation, Sanechips

- R2-2402379	Consideration on Random Access procedure for AIoT	ZTE Corporation, Sanechips

- R2-2402392	Use Cases and Stage 2 Procedure Flow for Ambient IOT	InterDigital

- R2-2402393	Control Plane Aspects for Ambient IOT	InterDigital

- R2-2402394	User Plane Aspects for Ambient IOT	InterDigital

- R2-2402395	Paging for Ambient IOT	InterDigital

- R2-2402396	Random Access for Ambient IOT	InterDigital

- R2-2402422	General considerations for A-IoT	Intel Corporation

- R2-2402423	Required Control plane functions for A-IoT	Intel Corporation

- R2-2402424	Required User plane functions for A-IoT	Intel Corporation

- R2-2402425	Paging design for A-IoT	Intel Corporation

- R2-2402426	Random access aspects for A-IoT	Intel Corporation

- R2-2402434	Study on Paging for Ambient IoT	SHARP Corporation

- R2-2402435	Study on Random access for Ambient IoT	SHARP Corporation

- R2-2402490	General discussion on ambient IoT	vivo

- R2-2402491	Discussion on Control Plane Aspects for Ambient IoT	vivo

- R2-2402492	Discussion on User Plane Aspects for Ambient IoT	vivo

- R2-2402493	Discussion on the functionality of paging in ambient IoT	vivo

- R2-2402494	Initial Access Procedure for Ambient IoT	vivo

- R2-2402548	Discussion on random access for Ambient IoT	CMCC

- R2-2402604	Discussion on paging procedure for A-IOT	Xiaomi

- R2-2402608	Discussion on random access for Ambient IoT	ETRI

- R2-2402674	Initial Access procedure for Ambient IoT device	NEC

- R2-2402696	Overall procedure and related aspects on Ambient IoT	HONOR

- R2-2402725	Discussion on user plane for Ambient IoT	Lenovo

- R2-2402726	Discussion on paging procedure for Ambient IoT	Lenovo

- R2-2402727	Discussion on random access for Ambient IoT	Lenovo

- R2-2402786	Principles for RAN2 work on ambient IoT	MediaTek Inc.

- R2-2402793	Considerations on C-plane aspects for Ambient IoT	Lenovo

- R2-2402794	Considerations on general aspects for Ambient IoT	Lenovo

- R2-2402891	Discussion on Control plane for Ambient IoT	Apple

- R2-2402892	Discussion on User plane for Ambient IoT	Apple

- R2-2402893	Discussion on Paging for Ambient IoT	Apple

- R2-2402894	Discussion on Random Access for Ambient IoT	Apple

- R2-2402896	A-IoT device hardware capabilities	Apple

- R2-2402918	General aspects for AIoT	Samsung

- R2-2402919	Initial view on the user plane aspects of AIoT	Samsung

- R2-2402920	Initial considerations on the RACH procedure for AIoT	Samsung

- R2-2402928	Stage-2 and general aspects of Ambient IoT	Qualcomm Incorporated

- R2-2402929	Control plane aspects of Ambient IoT	Qualcomm Incorporated

- R2-2402930	Paging aspects of Ambient IoT	Qualcomm Incorporated

- R2-2402938	User plane aspects of Ambient IoT	Qualcomm Incorporated

- R2-2402939	Random access aspects of Ambient IoT	Qualcomm Incorporated

- R2-2402949	General aspects for Ambient IoT	Ericsson

- R2-2402950	UP protocol and data transmission options	Ericsson

- R2-2402951	Discussion on UL multiple access	Ericsson

- R2-2402970	Ambient-IoT General Aspects	NEC

- R2-2402971	Considerations on Ambient-IoT Paging	NEC

- R2-2402977	Discussion on ambient IoT control plane functionality	LG Electronics Inc.

- R2-2402978	Discussion on ambient IoT paging functionality	LG Electronics Inc.

- R2-2402997	General aspects of AIoT	Nokia

- R2-2403011	General considerations on A-IOT	CMCC

- R2-2403012	Discussion on User Plane of A-IoT	CMCC

- R2-2403025	Discussion on A-IoT paging	CMCC

- R2-2403026	Discussion on control plane functions and signalling for Ambient IoT	CMCC

- R2-2403027	Work plan for Ambient IoT	CMCC, Huawei, T-Mobile

- R2-2403031	Considerations on random access in AIoT	Nokia

- R2-2403055	Considerations on various aspects for Ambient IoT	Sony

- R2-2403078	Discussion on random access aspects for Ambient-IoT	Continental Automotive

- R2-2403097	General aspects, high-level procedure and security aspects for Ambient IoT	Huawei, HiSilicon, China Telecom, China Unicom, LG Electronics Inc., ZTE Corporation, Sanechips, Apple, NEC

- R2-2403098	Other control plane aspects for Ambient IoT	Huawei, HiSilicon

- R2-2403099	Data transmission and protocol stack for A-IoT	Huawei, HiSilicon

- R2-2403100	Discussion on paging-like functionality design	Huawei, HiSilicon

- R2-2403113	TP for TR 38.769 update (RAN2 sub-clause skeleton)	Huawei, CMCC, T-Mobile USA

- R2-2403114	Random access-like procedure for Ambient IoT	Huawei, HiSilicon

- R2-2403115	Discussion on paging for Ambient IoT	China Telecom

- R2-2403117	Discussion on CP aspects of Ambient-IoT	China Telecom

- R2-2403149	Consideration on paging in AIoT	Nokia

- R2-2403257	Ambient IoT Paging Method	Wiliot Ltd.

- R2-2403259	Considerations of the data transmission in AIoT	Nokia

- R2-2403260	Energy-aware design for AIoT daa transmissions	Nokia

- R2-2403348	Discussion on contention-based access	LG Electronics Inc.

- R2-2403349	Discussion on user plane protocl stack and data transmission for A-IOT	LG Electronics Inc.

- R2-2403372	Considerations on protocol architecture for ambient IoT	KT Corp.

- R2-2403405	Discussion on DL reachability for Ambient IoT	Ericsson

- R2-2403406	Overview of CP protocols for Ambient IoT	Ericsson

- R2-2403421	Initial consideration of Ambient IoT	Kyocera

- R2-2403506	Initial considerations on the paging for AIoT	Samsung

- R2-2403516	Initial views on the control plane aspects of AIoT	Samsung Electronics Czech

- R2-2403561	Design Targets for Ambient IoT	T-Mobile USA Inc.

- R2-2403563	LS on Security Requirements for Ambient IoT	T-Mobile USA Inc.

- R2-2403609	Discussion on general aspects of Ambient IoT	Futurewei

- R2-2403610	Discussion on user plane aspects for Ambient IoT	Futurewei

- R2-2403611	Discussion on Control Plane for Ambient IoT	NTT DOCOMO INC.

- R2-2403612	Discussion on paging functions for Ambient IoT	Futurewei

- R2-2403613	Discussion on random access for Ambient IoT	Futurewei

- R2-2403616	Discussion on User Plane for Ambient IoT	NTT DOCOMO INC.

- R2-2403618	Discussion on user plane functionalities in A-IOT	LG Electronics Inc.

- R2-2403620	Discussion on random access for Ambient IoT	LG Electronics Inc.

- R2-2403645	Discussion on random access for ambient IoT	Google Inc.

- R2-2403676	General considerations on Ambient IoT	Philips International B.V.

- R2-2403677	Discussion on general aspects of ambient IoT	LG Electronics Inc.

- R2-2403679	Stage 2 overall procedures, baseline assumptions on security and use cases	CATT, CEPRI

- R2-2403680	Discussion on the Control Plane for Ambient IoT	CATT, CEPRI

- R2-2403681	Discussion on Data Transmission and Protocol Stack of Ambient IoT	CATT, CEPRI

- R2-2403682	Discussion on Paging Functionality of Ambient IoT	CATT, CEPRI

- R2-2403683	Discussion on the Random Access for Ambient IoT	CATT, CEPRI

RAN2#126

- R2-2404154	Discussion on access procedure for ambient IOT	Xiaomi

- R2-2404155	Discussion on functions in protocol stack for ambient IOT	Xiaomi

- R2-2404192	Overall procedures for Inventory and Command use cases	Intel Corporation

- R2-2404193	Required functions for A-IoT	Intel Corporation

- R2-2404194	Consideration on initial trigger message	Intel Corporation

- R2-2404195	Consideration on contention based and contention free based Access	Intel Corporation

- R2-2404224	Initial Access procedure for Ambient IoT device	NEC

- R2-2404231	Discussion on stage 2 overall procedures for Ambient IoT	CATT, CEPRI

- R2-2404232	Discussion on the Functionality Aspects for Ambient IoT	CATT, CEPRI

- R2-2404233	Discussion on Paging for Ambient IoT	CATT, CEPRI

- R2-2404234	Discussion on the Random Access for Ambient IoT	CATT, CEPRI

- R2-2404243	A-IoT random access procedure	Huawei, HiSilicon

- R2-2404344	Discussions on General Aspect of Ambient IoT	Fujitsu

- R2-2404345	Discussions on AIoT Functionalities	Fujitsu

- R2-2404346	Discussions on AIoT paging	Fujitsu

- R2-2404347	Discussions on AIoT Random Access	Fujitsu

- R2-2404369	Ambient IoT device paging	TCL

- R2-2404373	Random Access for Ambient IOT	TCL

- R2-2404381	Discussion on general aspects of AIoT	Futurewei

- R2-2404382	On not supporting segmentation and reassembly in Layer 2 for AIoT	Futurewei

- R2-2404383	Discussion on AIoT paging	Futurewei

- R2-2404384	Discussion on AIoT random access	Futurewei

- R2-2404394	General discussion on ambient IoT	vivo

- R2-2404395	Discussion on functionality aspects for Ambient IoT	vivo

- R2-2404396	Discussion on AIoT Paging	vivo

- R2-2404397	Random Access Procedure for A-IoT Device	vivo

- R2-2404398	Discussion on Paging Design for Ambient IoT	China Telecom

- R2-2404498	General aspects for Ambient IoT	Ericsson

- R2-2404499	Discussion on UL multiple access	Ericsson

- R2-2404500	Ambient IoT identifiers and "paging" procedure	MediaTek Inc.

- R2-2404501	AIoT read and write operations	MediaTek Inc.

- R2-2404507	Control and user plane modelling for ambient IoT interface	MediaTek Inc.

- R2-2404508	Transfer of upper-layer AIoT information	MediaTek Inc.

- R2-2404521	Considerations on functionality aspects for Ambient IoT	Lenovo

- R2-2404533	General Aspects on Ambient IOT	InterDigital

- R2-2404534	Functions for Ambient IOT	InterDigital

- R2-2404535	Paging for Ambient IOT	InterDigital

- R2-2404536	Access Procedure for Ambient IOT	InterDigital

- R2-2404538	A-IoT Functionality	ZTE Corporation, Sanechips

- R2-2404539	Random Access procedure for A-IoT	ZTE Corporation, Sanechips

- R2-2404569	Ambient-IoT General Aspects	NEC

- R2-2404570	Ambient-IoT Paging	NEC

- R2-2404571	Discussion on the Random Access for Ambient IoT	LG Uplus

- R2-2404578	Discussion on A-IOT paging procedure	Xiaomi

- R2-2404579	Discussion on paging procedure for Ambient IoT	OPPO

- R2-2404585	Discussion on random access for Ambient IoT	OPPO

- R2-2404586	Discussion on needed functionalities for Ambient IoT communication	OPPO

- R2-2404587	Stage 2 overall procedure flow	OPPO

- R2-2404628	Outstanding stage-2 issues of Ambient IoT design	Apple

- R2-2404659	Disucssion on functional aspects for Ambient IoT	Apple

- R2-2404660	Discussion on Ambient IoT Paging	Apple

- R2-2404661	Discussion on Ambient IoT Random Access	Apple

- R2-2404689	Discussion on A-IoT paging functionality	LG Electronics Inc.

- R2-2404795	Functionality aspects for A-IoT	Ericsson

- R2-2404809	Considerations on general aspects for Ambient IoT	Lenovo

- R2-2404810	Discussion on paging procedure for Ambient IoT	Lenovo

- R2-2404811	Discussion on random access for Ambient IoT	Lenovo

- R2-2404864	Discussion on A-IoT random access procedure	ETRI

- R2-2404873	Considerations on the general aspects of the Ambient IOT	Beijing Xiaomi Software Tech

- R2-2404876	Further consideration on general aspects for AIoT	ZTE Corporation, Sanechips

- R2-2404879	Ambient-IoT Functionality Aspects	NEC

- R2-2404881	Discussion on paging-like procedure for AIoT	ZTE Corporation, Sanechips

- R2-2404891	Ambient IoT Paging Method	Wiliot Ltd.

- R2-2404903	Considerations on paging for Ambient IoT	Sony

- R2-2404904	Considerations on random access aspects for Ambient IoT	Sony

- R2-2404925	Discussion on the functionalities required for Ambient IOT	Spreadtrum Communications

- R2-2404926	Discussion on random access of Ambient IOT	Spreadtrum Communications

- R2-2404929	Discussion on general aspects of A-IoT	Spreadtrum Communications

- R2-2404930	Discussion on paging functionality of A-IoT	Spreadtrum Communications

- R2-2404956	Discussion on random access aspects for Ambient-IoT	Continental Automotive

- R2-2404981	Discussion on functionality aspects of ambient IoT	KT Corp.

- R2-2404983	Discussion on general aspects for Ambient IoT	HONOR

- R2-2405015	Further discussion on random access for Ambient IoT	CMCC

- R2-2405030	Discussion on A-IoT paging	CMCC

- R2-2405039	General considerations on A-IoT	CMCC

- R2-2405040	Discussion on protocol stack of A-IoT	CMCC

- R2-2405041	General aspects and overall procedure	Huawei, HiSilicon, Wiliot Ltd., Orange, LG Uplus, NTT DOCOMO, INC.

- R2-2405042	A-IoT functionalities	Huawei, HiSilicon

- R2-2405043	A-IoT paging functionality	Huawei, HiSilicon

- R2-2405107	Further on AIoT random access	Nokia

- R2-2405140	Random access aspects of Ambient IoT	Qualcomm Incorporated

- R2-2405194	On Paging procedure for Ambient IoT	Nokia

- R2-2405212	General aspects of Ambient IoT	Qualcomm Incorporated

- R2-2405214	Views on Functionality Aspects of Ambient IoT	Qualcomm Incorporated

- R2-2405215	Views on Paging for Ambient IoT	Qualcomm Incorporated

- R2-2405227	Discussion on DL messages for Ambient IoT UEs	Ericsson

- R2-2405233	On AIoT functionality aspect	Nokia

- R2-2405244	Further on general aspects of AIoT	Nokia

- R2-2405269	Considerations for functionality aspects	Semtech Neuchatel SA

- R2-2405270	Considerations for Random Access	Semtech Neuchatel SA

- R2-2405291	Discussion on general aspects of ambient IoT	LG Electronics Inc.

- R2-2405305	Discussion on stage-2 aspects for Ambient IoT	China Telecom

- R2-2405306	Discussion on functionalities required for A-IoT devices	China Telecom

- R2-2405307	Discussion on random access for Ambient IoT	China Telecom

- R2-2405358	Discussion on random access for ambient IoT	Google Inc.

- R2-2405383	Overall procedures for Ambient IoT	Kyocera

- R2-2405384	Functionalities for Ambient IoT	Kyocera

- R2-2405427	Discussion on Ambient IoT random access conditions	ASUSTeK

- R2-2405465	General aspects for AIoT	Samsung

- R2-2405466	Discussions on functionalities required for AIoT	Samsung

- R2-2405495	Discussion on A-IoT paging	Samsung

- R2-2405496	Discussion on A-IoT random access	Samsung

- R2-2405518	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.

- R2-2405520	Discussion on user plane aspects for Ambient IoT	LG Electronics Inc.

- R2-2405550	Discussion on random access for Ambient IoT	CEWiT

- R2-2405598	Discussion on Stage 2 aspects for Ambient IoT	NTT DOCOMO, INC.

- R2-2405603	Discussion on initial trigger message (paging-like message) on Ambient IoT	NTT DOCOMO, INC.

- R2-2405604	Discussion on random access-like procedure for Ambient IoT	NTT DOCOMO INC.

- R2-2405615	TP for TR 38.769 update and terminologies	Huawei, CMCC, T-Mobile USA

- R2-2405688	On Random Access for Ambient IoT	Philips International B.V.

- R2-2405691	Discussion on functionality aspects for Ambient IoT	Philips International B.V.

- R2-2405697	Functions for Ambient IOT	InterDigital

- R2-2405950	Report of [AT126][022][AIoT] CB on 4 step RA	Huawei

RAN3#123bis

- R3-241570	Initial views on RAN architecture for Ambient IoT	China Telecommunication

- R3-241583	Discussion on A-IoT RAN architecture	NEC

- R3-241584	Discussion on A-IoT NG RAN interface impact	NEC

- R3-241635	On Requirements and Use Cases in Ambient IoT study in 3GPP Rel-19	Ericsson

- R3-241636	On Use Cases for Ambient IoT work in 3GPP Rel-19 and related functions	Ericsson

- R3-241637	RAN-CN Protocol Aspects for Ambient IoT study in 3GPP Rel-19	Ericsson

- R3-241638	Flows and Time Lines for Ambient IoT study in 3GPP Rel-19	Ericsson

- R3-241679	TR 38.769 skeleton for Study on solutions for Ambient IoT (Internet of Things) in NR	Huawei, CMCC

- R3-241680	RAN architecture aspects for Ambient IoT	Huawei

- R3-241681	Inventory over CN-RAN interface	Huawei

- R3-241682	Data Transport and Device context management over CN-RAN interface	Huawei

- R3-241683	Location report of Ambient IoT devices	Huawei

- R3-241746	Discussion on AIoT RAN architecture	Xiaomi

- R3-241747	(TP for TR 38.769) AIoT interface impacts between RAN and CN	Xiaomi

- R3-241748	(TP for TR 38.796) AIoT device localization	Xiaomi

- R3-241807	RAN architecture considerations of ambient IoT	Lenovo

- R3-241808	Paging considerations of ambient IoT	Lenovo

- R3-241809	Device context management considerations of ambient IoT	Lenovo

- R3-241810	Data transport considerations of ambient IoT	Lenovo

- R3-241836	[TP for TR 38.769] Architecture Requirements for Supporting Ambient AIoT Devices	Nokia

- R3-241837	[TP for TR 38.769] RAN Architecture and Protocol Stack for AIoT	Nokia

- R3-241838	Paging Signalling Impacts for AIoT	Nokia

- R3-241839	[TP for TR 38.769] Paging Signalling Impacts for AIoT	Nokia

- R3-241850	Architecture aspects and Protocol stack for Ambient IoT	Qualcomm Incorporated

- R3-241851	Paging and device context management for Ambient IoT	Qualcomm Incorporated

- R3-241852	Ambient IoT positioning	Qualcomm Incorporated

- R3-241905	Discussion on NG Interface Signalling for Ambient IoT	China Telecom

- R3-241933	Consideration on A-IoT architecture aspects	CATT

- R3-241934	Discussion on Paging for A-IoT	CATT

- R3-241935	Discussion on UE context management and Data transport for A-IoT	CATT

- R3-241936	Consideration on positioning aspects for A-IoT	CATT

- R3-241957	Work Plan for Ambient IoT SI	CMCC, Huawei, T-Mobile USA

- R3-241958	Discussion on RAN Architecture for Ambient IoT	CMCC

- R3-241959	Discussion on paging for Ambient IoT	CMCC

- R3-241960	Discussion on Device Context Management and Data Transfer	CMCC

- R3-241961	Discussion on locating Ambient IoT device	CMCC

- R3-241981	Discussion on RAN architecture for Ambient IoT	LG Electronics

- R3-242038	Discussion on RAN architecture for Ambient IoT	Samsung

- R3-242039	Discussion on RAN-CN interface impact for Ambient IoT	Samsung

- R3-242087	Discussion on RAN architecture for Ambient-IOT	ZTE

- R3-242088	Discussion on NG interface impact for Ambient-IOT	ZTE

- R3-242089	Discussion on overall procedure for Ambient-IOT	ZTE

- R3-242090	Discussion on position for Ambient-IOT	ZTE

- R3-242148	TR 38.769 skeleton for Study on solutions for Ambient IoT (Internet of Things) in NR	Huawei, CMCC

- R3-242149	CB:#AIoT1_General	Huawei

- R3-242153	CB:#AIoT2_Locating	CMCC

RAN3#124

- R3-243103	TR 38.769 skeleton for Study on solutions for Ambient IoT in NR	Huawei, CMCC

- R3-243128	[TP for TR 38.769] RAN Architecture and Protocol Stack for AIoT	Nokia

- R3-243129	Paging Signalling Impacts for AIoT	Nokia

- R3-243130	[TP for TR 38.769] Paging Signalling Impacts for AIoT	Nokia

- R3-243131	[TP for TR 38.769] Data Transport and Context Management for AIoT	Nokia

- R3-243133	(TP for TR38.769) RAN architecture for Ambient-IoT	ZTE

- R3-243134	[Draft] LS on CN-RAN interface design for Ambient-IoT	ZTE

- R3-243135	(TP for TR38.769) NG interface impact for Ambient-IoT	ZTE

- R3-243136	(TP for TR38.769) Locating Ambient-IoT device	ZTE

- R3-243191	Architecture aspects and Protocol stack for Ambient IoT	Qualcomm Incorporated

- R3-243192	Paging, device context management and data transport for Ambient IoT	Qualcomm Incorporated

- R3-243193	Methods for locating an Ambient IoT device	Qualcomm Incorporated

- R3-243233	Consideration on RAN architecture aspects	Huawei

- R3-243234	Inventory over CN-RAN interface	Huawei

- R3-243235	Command and Context management over CN-RAN interface	Huawei

- R3-243236	Location report of Ambient IoT devices	Huawei

- R3-243244	RAN architecture discussion on ambient IoT	NEC

- R3-243245	RAN-CN interface impact on ambient IoT	NEC

- R3-243246	Further discussion on ambient IoT locating	NEC

- R3-243295	AIoT RAN architecture	Xiaomi

- R3-243296	AIoT interface impacts between RAN and CN	Xiaomi

- R3-243297	Locating AIoT device	Xiaomi

- R3-243400	[TP for TR 38.769] RAN architecture considerations of Ambient IoT	Lenovo

- R3-243401	[TP for TR 38.769] Paging considerations of Ambient IoT	Lenovo

- R3-243402	[TP for TR 38.769] Device context management considerations of Ambient IoT	Lenovo

- R3-243403	[TP for TR 38.769] Data transport considerations of Ambient IoT	Lenovo

- R3-243549	On functional split between RAN and CN	Ericsson

- R3-243550	Nature and Content of Information exchanged between RAN and CN for Ambient IoT	Ericsson

- R3-243551	[DRAFT] LS on AIoT - RAN3 assumptions on functional split between RAN and CN	Ericsson

- R3-243552	On locating AIoT devices	Ericsson

- R3-243609	Discussion on Paging and Context Management for Ambient IoT	China Telecom

- R3-243619	Further discussion on RAN architecture for Ambient IoT	Samsung

- R3-243620	Further discussion on RAN-CN interface impact for Ambient IoT	Samsung

- R3-243630	Discussion on AIoT RAN architecture	China Telecommunication

- R3-243674	(TP for TR 38.769) Consideration on A-IoT architecture aspects	CATT

- R3-243675	(TP for TR 38.769) On A-IoT Inventory and Command services	CATT

- R3-243676	(TP for TR 38.769) A-IoT device context management and Data transport	CATT

- R3-243677	Consideration on locating of A-IoT device	CATT

- R3-243736	Work Plan for Ambient IoT SI	CMCC, Huawei, T-Mobile USA

- R3-243737	Discussion on RAN Architecture for Ambient IoT	CMCC

- R3-243738	Discussion on paging for Ambient IoT	CMCC

- R3-243739	Discussion on Device Context Management and Data Transfer	CMCC

- R3-243740	Discussion on locating Ambient IoT device	CMCC

- R3-243769	Discussion on NG impact for Ambient IoT	LG Electronics

- R3-243807	[TP for TR 38.769] CB:#AIoT1_Architecture	Ericsson, Huawei

- R3-243808	Summary of Offline Discussion – CB: # AIoT2_CNRANinterface	Huawei

- R3-243810	CB:#AIoT3_Location	CMCC

- R3-243872	Inventory and Command between AIoT CN amd AIoT RAS	Huawei, CMCC, Nokia, Ericsson, ZTE, Xiaomi, Qualcomm Incorporated, Samsung, CATT, Lenovo, LG Electronics, NEC

- R3-243950	(TP for TR38.769) Locating Ambient-IoT device	CMCC, ZTE, CATT, Xiaomi, LGE, Huawei, Lenovo, NEC

- R3-243962	[TP for TR 38.769] CB:#AIoT1_Architecture	Ericsson, Huawei

- R3-243963	Inventory and Command between AIoT CN amd AIoT RAS	Huawei, CMCC, Nokia, Ericsson, ZTE, Xiaomi, Qualcomm Incorporated, Samsung, CATT, Lenovo, LG Electronics, NEC Electronics, NEC

- R3-243964	(TP for TR38.769) Locating Ambient-IoT device	CMCC, ZTE, CATT, Xiaomi, LGE, Huawei, Lenovo, NEC

- R3-243972	TR 38.769 skeleton for Study on solutions for Ambient IoT in NR	Huawei, CMCC

RAN4#110bis

- R4-2404459	Discussion on the general issues for AIoT CATT

- R4-2404867	A-IoT general overview  Ericsson

- R4-2405298	A-IoT TR skeleton for RF part  Huawei

- R4-2407299	On coexistence between ambient IoT and NR/LTE  Apple

- R4-2407410	AIoT deployment scenario and impact on co-existence analysis  Sony

- R4-2405379	General consideration for A-IOT OPPO

- R4-2405891	UE implementation aspects impacting work planning and study areas in RAN4  Qualcomm

- R4-2405304	A-IoT workplan  CMCC Huawei T-Mobile

- R4-2404251	Preliminary considerations on the ambient IoT device implementation and the co-existence analysis  Sony

- R4-2404355	On coexistence between ambient IoT and NR/LTE  Apple

- R4-2404438	Discussion on co-existence simulation methodology and scenarios for ambient IoT  CATT

- R4-2404549	Discussion on the coexistence study of Ambient IoT and NR/LTE  Xiaomi

- R4-2404586	Discussion on co-existence evaluation for A-IoT and NR/LTE  Spreadtrum

- R4-2404671	Discussion on the co-existence of the AIoT  vivo

- R4-2404868	Coexisting study simulation assumptions for A-IoT   Ericsson

- R4-2404985	Views on coexistence of Ambient IoT and NRLTE   Samsung

- R4-2405299	General discussion on A-IoT coexistence scenarios  Huawei

- R4-2405305	Discussion on A-IoT co-existence evaluation   CMCC

- R4-2405376	Consideration on ambient IoT coexistence with NR/LTE  China Telecom

- R4-2405392	Discussion on the Ambient IoT coexistence for NR  Qualcomm

- R4-2405620	Discussion on co-existence study for ambient IoT and NR/LTE   ZTE

- R4-2405289	Topic summary for [110bis][136] FS_Ambient_IoT_solutions  Moderator (CMCC)

- R4-2406618	WF on Ambient IoT in NR, CMCC

- R4-2406714	WF on Ambient IoT in NR, CMCC

RAN4#111

- R4-2409095	A-IoT  general overview   Ericsson

- R4-2407917	TP to TR38.769 skeleton for RF part   Huawei

- R4-2410596	TP to TR38.769 skeleton for RF part   Huawei

- R4-2407299	On coexistence between ambient IoT and NR/LTE   Apple

- R4-2407410	AIoT deployment scenario and impact on co-existence analysis   Sony

- R4-2407525	Discussion on deployment scenarios and spectrum usage for A-IoT   CATT

- R4-2407715	Discussion on deployment scenarios and spectrum usage for ambient IoT    Spreadtrum

- R4-2407918	Discussion on A-IoT deployment scenarios and spectrum usage    Huawei

- R4-2408091	Discussion on the deployment scenarios and spectrum usage for AIoT    vivo

- R4-2408219	Discussion on A-IoT deployment scenario and spectrum usage    CMCC

- R4-2408820	on deployment scenarios and spectrum usage for A-IoT    OPPO

- R4-2409094	A-IoT deployment scenario and spectrum usage    Ericsson

- R4-2409426	Discussion on deployment and spectrum usage    Qualcomm

- R4-2409573	Discussion on spectrum usage for Ambient-IoT    LGE

- R4-2409596	Discussion on deployment scenarios and spectrum usage    ZTE

- R4-2407478	Discussion on co-existence evaluation for ambient-IoT    CATT

- R4-2407716	Discussion on co-existence evaluation for ambient IoT and NR-LTE    Spreadtrum

- R4-2407821	Discussion on the coexistence study of Ambient IoT and NR    Xiaomi

- R4-2407919	A-IoT co-existence evaluations    Huawei

- R4-2408092	Preliminary co-existence evaluation for AIoT    vivo

- R4-2408218	Discussion on A-IoT co-existence evaluation    CMCC

- R4-2408236	Consideration on co-existence evaluations    China Telecom

- R4-2408819	on co-existence evaluations for A-IoT    OPPO

- R4-2409098	Coexisting study simulation assumptions     Ericsson

- R4-2409427	Discussion on Ambient IoT co-existence evaluation    Qualcomm

- R4-2409595	Discussion on Co-existence evaluations    ZTE

- R4-2407522	Discussion on RF requirements of A-IoT BS    CATT

- R4-2407822	Discussion on the RF impact of Ambient IoT BS    Xiaomi

- R4-2408093	Discussion on the RF requirement for AIoT BS    vivo

- R4-2408217	Discussion on A-IoT BS RF requirements    CMCC

- R4-2408237	Consideration on RF requirements for Ambient IoT BS    China Telecom

- R4-2409093	A-IoT BS RF overview    Ericsson

- R4-2409407	RF requirements for Ambient IoT BS    Huawei

- R4-2409597	Discussion on RF requirement of Ambient IoT BS    ZTE

- R4-2407411	Preliminary considerations on the ambient IoT device implementation and RF aspect    Sony

- R4-2407523	Discussion on RF requirements of A-IoT device    CATT

- R4-2407588	A-IoT device study and RF requirements aspects    Qualcomm

- R4-2407717	Discussion on RF requirements impact for ambient IoT device    Spreadtrum

- R4-2407823	Discussion on the RF impact of Ambient IoT device    Xiaomi

- R4-2408094	Discussion on the RF requirement for AIoT device    vivo

- R4-2408220	Discussion on A-IoT device RF requirements    CMCC

- R4-2408238	Consideration on RF requirements for Ambient IoT device    China Telecom

- R4-2408817	further discussion on the regulation and Device requirements    OPPO

- R4-2409097	A-IoT UE RF overview    Ericsson

- R4-2409598	Discussion on RF requirement of Ambient IoT device    ZTE

- R4-2409646	Discussion on RF requirements for Ambient IoT devices    Huawei

- R4-2407524	Discussion on RF requirements of A-IoT intermediate node    CATT

- R4-2407587	Intermediate node role in A-IoT system and study considerations   Qualcomm

- R4-2407718	Discussion on RF requirements impact for intermediate node (UE)    Spreadtrum

- R4-2407824	Discussion on the RF impact of intermediate UE    Xiaomi

- R4-2408095	Discussion on the RF requirement for intermediate UE    vivo

- R4-2408221	Discussion on A-IoT intermediate UE RF requirements    CMCC

- R4-2408239	Consideration on RF requirements for Intermediate Node    China Telecom

- R4-2408818	further discussion on the regulation and UE requirements    OPPO

- R4-2409096	A-IoT UE as intermediate node RF overview    Ericsson

- R4-2409599	Discussion on RF requirement of Intermediate node (UE)    ZTE

- R4-2409647	Discussion on RF requirements for intermediate UE    Huawei

- R4-2408945	Topic summary for [111][134] FS_Ambient_IoT_solutions_part1    Moderator (CMCC)

- R4-2410567	WF on co-existence study for ambient IoT and NR/LTE    CMCC

- R4-2408946	Topic summary for [111][135] FS_Ambient_IoT_solutions_part2    Moderator (Huawei)

- R4-2410597	WF on impacts of A-IoT on RF requirements    Huawei

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template