3GPP TSG-RAN WG2 Meeting #127                                                                                       R2-24xxxxx
Maastricht, Netherlands, Aug 19th – 23rd, 2024

Title:	[Draft] LS on security key update of inter-CU SCG LTM

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	CATT [To be RAN2]

To:	SA3

Cc:	RAN3

- Contact Person:	Rui Zhou

- E-mail Address:	zhourui@catt.cn

1. Overall Description:

According to the following WID objective of R19 Mob enhancement,

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling

RAN2 discussed some aspects of inter-CU LTM in DC (and will continue on the remaining aspects in the coming meetings). As part of that discussion, the security key update of inter-CU SCG LTM (i.e., the case where CU is acting as SN and MN is unchanged) is discussed with for inter-CU LTM in DC scenario the following agreements:

Inter-CU SCG LTM: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

2. Actions

To SA3 group

ACTION: 	RAN2 respectfully asks SA3 to take the above RAN2 agreements into account related to security key update of inter-CU SCG LTM as well as the agreements made for Inter-CU MCG LTM. RAN2 will continue to update as further progress is made.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #127bis		14 – 18 October 2024		                       Hefei, CN

TSG RAN WG2 Meeting #128		18 – 22 November 2024		               Orlando, US