3GPP TSG-RAN WG2 #128	R2-240xxxx

Orlando, US, November 18-22, 2024

Agenda Item:	8.13.2

Source: 	LG Electronics Inc.

Title: 	Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

Scope:

  - Discovery message initiating/forwarding condition at intermediate relay UE

  - PC5 AS conditions for discovery at the last relay UE

  - Reselection triggers for all UEs

  - (Re)selection criteria for all UEs (considering discovery models A/B and whether criteria beyond the first hop are considered)

  - Whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last relay UE via one (physically same) intermediate relay UE

Intended outcome: Report to RAN2#128

Deadline: Very long (for RAN2#128)

The purpose of the first phase of email discussion is to address the basic subjects within the scope and gather companies’ opinions on any missing parts that should be discussed. In the second phase, the Rapporteur makes summary proposals and can discuss additional subjects based on the companies’ inputs from the first phase email discussions. Also, throughout the email discussion, we will consider extensibility over 2-hops based on the WID of Rel-19 multi-hop relay.

The deadline for this email discussion will be as follows:

- Phase 1 Deadline 2024-10-21 UTC 10:00

- Phase 2 Deadline 2024-11-04 UTC 10:00

## Contact information

| Company | Email address |
| --- | --- |
| LG | seoyoung.back@lge.com |
| Ericsson | Min.w.wang@ericsson.com |
| OPPO | lengbingxue@oppo.com |
| Sharp | kawano.takuma@sharp.co.jp |
| Apple | Zhibin_wu@apple.com |

# 2. Phase 1 Discussion

## 2.1 Discovery message initiating/forwarding condition at intermediate Relay UE

(The condition to be intermediate Relay UE)

The agreement for the last Relay UE to have upper and lower Uu threshold bound was made at the last meeting as the followings:

- If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission.

However, it’s not made an agreement related to the Uu threshold condition for the intermediate Relay UE.

### [Question 2.1-1] Do you think the intermediate Relay UE has the upper and/or lower bound of the Uu threshold? Please check your opinion and you can describe the reason if you want.

| Company | Need upper bound | Need lower bound | Need both bounds | Need none of them | You can write down the reason for your answer if you want. |
| --- | --- | --- | --- | --- | --- |
| Ericsson | No | No | No | Yes | For the remote UE, the intermediate relay UE is not mandated to be in RRC_CONNECTED. In addition, the intermediate relay UE may be OOC. However, this doesn’t exclude the case where the intermediate relay UE itself may need a relay connection for itself to transmit its own data to the network. In this case, the intermediate relay UE will operate as a remote UE for its own traffic. |
| OPPO | Yes | No | No | No | For lower bound, it is not needed for the intermediate Relay since it can either in-coverage or out-of-coverage; For upper bound, it is needed since the intermediate relay should not in the very cell centre (since in that case, it should be the last relay). |
| Sharp | Yes | No | No | No | If the intermediate relay UE can act as a Last Relay UE, it should act as a Last Relay UE. And upper bound is configured for the legacy relay UE. Therefore, upper bound should be configured (but it is up to NW how to configure the threshold which value can be set to infinity). |
| Apple | No | No | No | Yes | For OOC intermediate UE, bounds are not needed. For an intermediate relay UE in coverage, the reason it does not acting as a last relay UE is that its own serving cell does not support relay. So, in that case, it will not be subject to bounds configuration either. So, in all cases, there is no need for bounds. |

(discovery message initiation at intermediate Relay UE)

The conclusion in SA2 TR 23.700-03 and the subclause 6.3.2.5.2/3 in TS 23.304, it looks like the intermediate Relay UE does not initiate a discovery announcement message broadcast in discovery model A and discovery model B.

[Conclusions on SA2 TR 23.700-03]

- When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay.

- When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

However, considering multi-hop extensibility over 2-hops, additional schemes can be required in the discovery procedure. For example, the intermediate Relay UE may be required to initiate the discovery procedure when the RLF happens or the RSRP threshold is below the configured threshold among the hops between intermediate Relay UEs or between the intermediate Relay UE and the last Relay UE. It may be helpful to fast recovery of the multi-hop relay link as the hop counts increase. However, it’s not sure whether intermediate Relay UE can initiate the discovery procedure. Also, it’s not sure whether this issue can be discussed in the RAN2, not SA2.

### [Question 2.1-2] Do you think discovery procedure should be initiated at the intermediate Relay UE considering multi-hop extensibility over 2-hops? If your answer is ‘yes’, describe how to handle this issue.

| Company | Yes/No | Describe how to handle this issue |
| --- | --- | --- |
| Ericsson | No | In Rel-19, it is sufficient to assume a baseline mechanism, i.e., it is only remote UE that triggers relay reselection. If there is RLF detected on an intermediate hop, it is enough that the RLF info is relayed to remote UE. In our view, allowing intermediate relay UE to initiate discovery, is an optimization, which can be delayed to future releases. |
| OPPO | See comments | After checking SA2 colleagues, our understanding on SA2 conclusion is intermediate relay UE should not generate discovery message for multi-hop relay without information source (i.e., the discovery message has to be generated based on the info from Last relay or remote UE). But it doesn’t restrict the discovery transmission after link establishment for link monitoring purpose. |
| Sharp | No | We think it should be discussed in SA2 if needed. And, in our view, if we assume that the intermediate relay UE also acts as a remote UE, the intermediate relay UE can initiate discovery procedure for recovery. |
| Apple | See comment | We do not understand this question. SA2 said “……based on the announcement message sent by the Intermediate Relay”. We think this means the relay UE does initiate model-A discovery message by itself although the contents are generated based on the information collected from remote UE or last relay UE via discovery process. SA2 has already agreed that the relay UE can generate discovery message transmission itself. Does the question intend to ask for the case of relay reselection instead? |

(discovery message forwarding at intermediate Relay UE)

For the discovery model A, the following agreement was made for relay UE to initiate discovery announcement message broadcast.

- For Model A discovery, the relay UE should only announce the neighbour UEs for which the SD-RSRP/SL-RSRP between the relay UE and the neighbour UE is above a configured threshold in a discovery announcement message.

In Rel-19 multi-hop U2N relay operation, SD-RSRP/SL-RSRP seems to be considered as condition for the discovery announcement message forwarding at intermediate Relay UE.

### [Question 2.1-3] Do you think the discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP is above a configured threshold?

| Company | Yes/No | Reason, if you want to describe. |
| --- | --- | --- |
| Ericsson | Yes |  |
| OPPO | See comments | We understand the channel quality between the intermediate relay and its parent relay needs to be evaluated but may not in form of discovery transmission condition. It is more like a relay selection condition, i.e., the intermediate relay selects its parent relay UE based on the RSRP, and forward the discovery announcement message from the selected parent relay UE. |
| Sharp | Yes |  |
| Apple | Yes |  |

### [Question 2.1-4] Do you have any further ideas other than threshold of SD/SL-RSRP about the AS condition for forwarding the discovery announcement message at the intermediate Relay UE? If you have anyone, you can add it freely and describe the reason.

| Company | You can describe AS conditions other than SD/SL-RSRP threshold for forwarding discovery announcement message. |
| --- | --- |

For the discovery model B, the following agreements was made at Rel-18 U2U relay operation.

- For Model B, the relay UE forwards the solicitation message only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

- For Model B, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE.

The same principle seems to be reused for the multi-hop U2N relay operation as the followings:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

- For Model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

### [Question 2.1-5] Do you think the above descriptions (in the box) about discovery message forwarding conditions are available for the multi-hop operation in discovery model B? If you have any other view, you can describe it.

| Company | Yes/No | If the answer is ‘No’, please describe your opinions. |
| --- | --- | --- |
| Ericsson | Yes | But, the second bullet needs to be updated as - For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. |
| OPPO | No for 1 Yes for 2/3 | For 1, it is not needed since same as U2N Relay case, there is no channel quality condition defined for Relay UE to determine whether to respond the solicitation message from the remote UE. |
| Sharp | Yes |  |
| Apple | Yes for 1, 3, No for 2 | I think for 2, we have to differ from U2U relay case, because the checking of RSRP threshold has to be done by adjacent UEs, which may not have remote UE involved. So, the bullet 2 has to be rephrased to reflect R19 scenario. |

## 2.2 PC5 AS conditions for discovery at the last Relay UE

In the legacy Rel-17 U2N Relay operation, the U2N Relay UE has upper and lower Uu thresholds bound to act as a U2N Relay UE. For example, the Rel-17 U2N Relay UE can initiate discovery procedure when the Uu RSRP threshold is below a configured upper bound and/or above a configured lower bound.

In multi-hop U2N Relay UE operation, the last Relay UE has a similar condition to act as the last Relay UE as the following agreement.

- If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

Meanwhile, some other companies think that PC5 AS condition can be additionally needed in the case of the last Relay UE in multi-hop. However, it’s not clear which PC5 AS condition can be specified for the last Relay UE. So, we’d like to listen to the PC5 AS condition for discovery at the last Relay UE

### [Question 2.2-1] Do you think PC5 AS condition is needed for discovery at the last Relay UE? If your answer is ‘yes’, describe the reason of the necessity in detail.

| Company | Yes/No | Describe your opinions freely |
| --- | --- | --- |
| Ericsson | No |  |
| OPPO | No | As we replied in Question 2.1-5, in single hop U2N relay, there is no PC5 AS condition defined for the relay UE to determine whether to respond the solicitation message from the remote UE. |
| Sharp | Yes | In Rel-17/18, PC5 AS condition for the last relay UE (i.e. PC5 link quality between the remote UE and the last relay UE) can be evaluated by the remote UE since it was single-hop. However, in multi-hop case, the remote UE cannot evaluate PC5 AS condition for the last relay UE. For discovery model A, PC5 AS condition is NOT needed since intermediate relay UE which directly receives discovery announce message from the last relay UE can evaluate PC5 AS condition for the last relay UE when the intermediate relay UE determines whether to forward it. However, for discovery model B, PC5 AS condition is needed since there is no UE which evaluates PC5 link quality between the intermediate relay UE and the last relay UE if the relay UE doesn’t evaluate PC5 AS condition. This is similar idea with the agreement of R18 U2U relaying as follows; “For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.” Therefore, we think PC5 AS condition is needed for Model B discovery at the last relay UE. |
| Apple | Yes | Same understanding as Sharp. |

## 2.3 (Re)selection triggers for all UEs

Based on the Rel-17 U2N Relay (re)selection triggering conditions in subclause 5.8.15.3 in TS 38.331, the Rel-19 Remote UE for multi-hop may reuse them for relay (re)selection such as the following:

[Relay (re)selection triggering conditions for Remote UE] - If the Remote UE has no serving cell; - If the Remote UE does not have a selected intermediate Relay UE; - If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected intermediate Relay UE; - If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE; - If the Remote UE receives a notification message from the (first) intermediate Relay UE caused by one of the following: h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE) h-2) if intermediate Relay UE receives RRCReconfiguration message for HO h-3) if intermediate Relay UE performs cell reselection h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE h-5) if intermediate Relay UE receives PC5-RRC connection release between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

### [Question 2.3-1] Do you think the above relay (re)selection triggering conditions (from (a) to (h) in the box) based on the legacy Rel-17 U2N can be reused for Rel-19 Remote UE? You can answer exclude and include conditions separately among them. You can add other triggering conditions with reason, if you want.

| Company | include conditions | Exclude conditions with reason | Additional conditions with reason (other than the above conditions) |
| --- | --- | --- | --- |
| Ericsson | a),b),c),d),e),g) h-1 in addition, f and h-5 may be generalized as “remote UE receives a message from intermediate relay UE indicating PC5 link release” | h-2,h-3, and h-4 the intermediate relay UE is not mandated to be in RRC_CONNECTED | - PC5 signal strength between the intermediate relay UE and the L2 last U2N Relay UE is below a (pre)configured signal strength threshold. - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by last U2N Relay UE. |
| OPPO | a) with some rewording c)-h) | a): Can be covered by “Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold ” b): We understand the intention while “has no selected intermediate relay UE” seems not the trigger condition for relay selection, but the diff between relay selection and relay reselection. | the Remote UE receives a notification message from the (first) intermediate Relay UE caused Uu RLF at the intermediate relay UE Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold Indicated by upper layer of the U2N Remote UE. |
| Sharp | A,b,c,d,e,f,g,h as baseline. And further details should be discussed later. For condition (f/g/h), “intermediate relay UE” could be changed to “first relay UE” for clarification for now. For condition (f) it can be integrated to condition (e) from AS perdpective. |  | Condition: SL/SD-RSRP of the currently selected first(intermediate) relay UE is above a configured threshold. Reason: If the PC5 link between the remote UE and the first (intermediate) relay UE is very good (i.e., they are almost in the same location), it is reasonable for the remote UE to switch to the intermediate (last) relay UE, from delay perspective. [comments] For condition (c), what is the intention of “the SL-RSRP of the currently selection intermediate Relay UE”? We think “the SL-RSRP of the currently selected intermediate Relay UE” is align with legacy and condition (d). If there is no difference, please ignore this comment. |
| Apple | All except f | For f, our understand is that this is the upper layer of intermediate relay UE, so this cannot be used as a remote UE trigger. | 1) Receiving a notification message from PC5-RRC for any path disruption occurred in one or more hops (Uu RLF, SL RLF or both…) 2) Any upper layer reasons to trigger path (re)selection |

The intermediate Relay UE may reuse similar conditions for relay (re)selection. The relay (re)selection conditions for intermediate Relay UE can be suggested like the following:

[Relay Triggering conditions for intermediate Relay UE] - If the intermediate Relay UE has no serving cell; - If the intermediate Relay UE does not have a selected last (/serving intermediate) Relay UE; - If the SL-RSRP of the currently selection last (/serving intermediate) Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE; - If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE; - If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE caused by one of the following: h-1) if last Relay UE detects Uu RLF h-2) if serving intermediate Relay UE detects PC5 RLF h-3) if last (/serving intermediate) Relay UE receives RRCReconfiguration message for HO h-4) if last (/serving intermediate) Relay UE performs cell reselection h-5) if last (/serving intermediate) Relay UE fails Uu RRC connection establishment/Resume h-6) if last (/serving intermediate) Relay UE receives Uu connection release between gNB and last (/serving intermediate) Relay UE h-7) if serving intermediate Relay UE receives PC5-RRC connection release between last (/the other serving intermediate) Relay UE and the serving intermediate Relay UE

### [Question 2.3-2] Do you think the above suggested relay (re)selection triggering conditions (from (a) to (h) in the box) based on the legacy Rel-17 U2N can be used for intermediate Relay UE? You can answer exclude and include conditions separately among them. You can add other triggering conditions with reason, if you want.

| Company | include conditions | Exclude conditions with reason | Additional conditions with reason (other than the above conditions) |
| --- | --- | --- | --- |
| Ericsson | None, in our view, intermediate relay has no need to trigger relay (re)selection, it is an optimization. |  |  |
| OPPO | a) h-1) with some rewording c)-h) | a): Can be covered by “Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold ” b): We understand the intention while “has no selected intermediate relay UE” seems not the trigger condition for relay selection, but the diff between relay selection and relay reselection. | Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold Indicated by upper layer of the U2N Remote UE. For h-1), it should be “if last(/serving intermediate) Relay UE detects Uu RLF” since intermediate relay may also detect Uu RLF. |
| Sharp | [comment] If RAN2 assumes that the intermediate relay UE also act as a remote UE, no condition is needed. We consider this assumption helps keep the specification simple and also achieves the same effect as if the intermediate relay UE were to perform relay reselection. |  |  |
| Apple | None | Intermediate relay UE shall not trigger relay reselection by itself, basically, it need just send notification message towards the remote UE about RLF. |  |

## 2.4 (Re)selection criteria for all UEs (considering discovery models A/B and whether criteria beyond the first hop are considered)

In the Rel-18 U2U Relay, the following agreements are generated.

- SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

The Rel-18 U2U relay (re)selection criteria may be reused to the Rel-19 multi-hop U2N relay (re)selection criteria.

### [Question 2.4-1] Do you think the above Rel-18 relay (re)selection criteria, from (1) to (3) in the box, can be reused for relay (re)selection criteria for the Rel-19 multi-hop?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Sharp | Yes |  |
| Apple | Yes |  |

### [Question 2.4-2] If you think that the other relay (re)selection criteria other than SD-RSRP or SL-RSRP are needed for the multi-hop relay, please write them down along with the reason.

| Company | Describe other criteria, if it exists | Reason |
| --- | --- | --- |

## 2.5 Whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last Relay UE via one (physically same) intermediate Relay UE

According to the Rel-19 WID, it’s clear that one Remote UE has one indirect path. However, it’s not clear whether the intermediate Relay UE can have one indirect path or not. Considering this issue, we can consider which topologies can be excluded or included.

In the contributions by Qualcomm (R2-2407102), several cases of the multi-hop topologies, which we have to treat, are displayed. Based on Qualcomm’s contribution, Rapporteur will display further topologies cases that we should include and exclude. All the remote UEs have only one indirect path in the provided topologies. In the figures, the red or green color means the link to support multi-hop U2N relay from Remote UE and gNB. The big blue circle means physically one Relay/Remote UE.

For each question, you can answer in terms of Rel-19 U2N relay should handle these topologies.

- Case-A

  - The red link from gNB to Remote UE(1) is for supporting multi-hp U2N relay operation.

  - The green link from gNB to Remote UE(2) is for single-hop U2N relay operation.

  - In Figure 2.5A-1, the big blue circle means the L2 ID(D) and L2 ID(D’) belong to the physically same last relay UE. And L2 ID(C) and L2 ID(C’) belong to the physically same intermediate Relay UE. However, the physically one intermediate Relay UE acts as an intermediate Relay for the Remote UE(1) and also acts as Remote UE(2) by itself. The intermediate Relay UE has one C-RNTI value because the UE is physically one UE regardless of acting as intermediate Relay UE and Remote UE simultaneously. In this case, the intermediate Relay UE has two different indirect paths via physically one last Relay UE by using different PC5 connections.

  - In Figure 2.5A-2, the big blue circle at the last Relay UE means the L2 ID(D) and L2 ID(D’) belong to the physically one UE. However, L2 ID(D) and L2 ID(D’) has connection with each physically different intermediate Relay UE and Remote UE(2). In this case, it doesn’t look like there is any problem with just the figure.

  - In Figure 2.5A-3, the big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) belong to physically the same UE. The intermediate Relay UE acts as an intermediate Relay UE for supporting Remote UE(1) and acts as Remote UE(2) by itself simultaneously. The intermediate Relay UE has two indirect connections with gNB via physically different last Relay UE(s) (i.e., via last Relay UE(1) and last Relay UE(2)). Each last Relay UE(1) and last Relay UE (2) is connected to the same gNB/cell.

(Figure 2.5A-1)

(Figure 2.5A-2)

(Figure 2.5A-3)

### [Question 2.5A-1] In Figure 2.5A-1 and 2.5A-2, do you think the last Relay UE of a multi-hop relay operation can also act as last Relay UE of a single-hop relay operation simultaneously?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes with comments | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | Yes |  |
| Apple | Yes | A relay UE cannot be limited to only support a single scenario. |

### [Question 2.5A-2] In Figure 2.5A-1, do you think the intermediate Relay UE of multi-hop relay operation can act as a Remote UE of single-hop relay operation simultaneously?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes with comments | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No if different L2 ID are used for the two roles as shown in the figure 2.5A-1 But Yes if the same ID are used for the two roles (e.g., using the red line to play the two roles simultaneously) | For L2 U2N Relay, different L2 ID means different links, allowing intermediate Relay UE of multi-hop relay operation acting as a Remote UE of single-hop relay operation simultaneously means we support multi-path with 2 indirect legs, which is out of both R18 and R19 scope. |
| Apple | Yes | There is no real technical obstacles for this. The UE can be a remote UE and a relay UE at the same time |

### [Question 2.5A-3] As Figure 2.5A-1, do you think one physical UE which acts as both intermediate Relay UE and Remote UE can have two indirect connections with different PC5 connections via physically the same last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The multi-path with 2-indirect path is out of WI scope. |
| Apple | Yes |  |

### [Question 2.5A-4] As Figure 2.5A-2, do you think physically one last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE)?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes | In the same cell, there may be both R17 remote UE and R19 remote UE, connecting to the same last relay UE. We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | Yes |  |
| Apple | Yes |  |

### [Question 2.5A-5] As Figure 2.5A-3, do you think one intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | comments | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The multi-path with 2-indirect path is out of WI scope. |
| Apple | Yes | The two last relay UE may only support one different RSC code for each of them, so this is a valid case. |

### [Question 2.5A-6] If your answer to the [Question 2.5A-3] or [Quest 2.5A-5] was 'NO', do you have any ideas on how to prevent these topologies? Because the AS layer can only see L2 ID(s), the AS layer cannot differentiate whether the different L2 ID(s) belong to the same physical UE. It seems like it should be controlled from the upper layer, but if you have any idea, you can describe it freely.

| Company | Describe how to prevent |
| --- | --- |
| OPPO | Similar issue has been discussed in R17 single-hop U2N Relay. In single-hop relay, for L2 U2N Remote UE, to prevent 2-indirect path connection, each remote UE can only have one relay UE, and according to state-2 specification, a single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. Therefore, we can make the similar restriction in MH case, i.e., each UE can only have a single parent node, and a single unicast link is established between the UE and its parent UE in L2 MH U2N Relay case. And also restriction, if needed, can be facilitated by upper layer (S2 spec). |

### [Question 2.5A-7] If your answer was ‘yes’ for one of the questions 2.5A-3, 2.5A-4, or 2.5A-5, do you have any preference on which topology should be handled priori than others for Rel-19 multi-hop? If you have any preference, you can describe it with reason.

| Company | Preference with reason |
| --- | --- |
| Ericsson | 2.5A-3 2.5A-2 2.5A-1 (follows a decreasing priority order) |
| Apple | All. No differentiation in priority. |

- Case-B

  - The red link from gNB to Remote UE(A) is for multi-hop U2N relay operation.

  - The green link from gNB to Remote UE(B) is for multi-hop U2N relay operation.

  - In Figure 2.5B-1, the big blue circle at the intermediate Relay UE means the L2 ID(B) and L2 ID(B’) belong to the physically one intermediate relay UE. Each physically different Remote UE has an indirect path via the same intermediate Relay UE and the same last Relay UE. It looks link no problem.

  - In Figure 2.5B-2, the big blue circle at the last Relay UE means the L2 ID(D) and L2 ID(D’) belong to the physically one last Relay UE. The big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically one intermediate Relay UE. Each physically different Remote UE(A) and Remote UE(B) has an indirect path via the same intermediate Relay UE and last Relay UE. However, comparing Figure 2.5B-1, the intermediate Relay UE and last Relay UE have connections via different PC5 connections to support each Remote UE.

  - In Figure 2.5B-3, the big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically one UE. The intermediate Relay UE has two different PC5 connections with physically different last Relay UE(A) and last Relay UE(B). Each last Relay UE(A) and last Relay UE (B) is connected to the same gNB/cell. The intermediate Relay UE has one C-RNTI when it is CONNECTED because the intermediate Relay UE is physically one UE. It looks like one intermediate Relay UE has two different indirect paths via physically different last Relay UE(s).

(Figure 2.5B-1)

(Figure 2.5B-2)

(Figure 2.5B-3)

### [Question 2.5B-1] As Figure 2.5B-1, do you think two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE? If your answer is ‘no’, please describe the reason.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | No | Not sure understand this scenario, but it seems that the intermediate relay UE and last relay UE use the same PC5 connection to carry two indirect paths, which may require additional design complexity which should be avoided. |
| OPPO | Yes | Compared to Figure 2.5B-2, we assume the difference here is the intermediate Relay UE and last Relay UE have a single PC5 connections to support each physically different Remote UE |
| Apple | Yes | This is a valid case. |

### [Question 2.5B-2] As Figure 2.5B-2, do you think two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically same last Relay UE? Here, the intermediate Relay UE and last Relay UE have two different PC5 connections to support each physically different Remote UE.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson |  | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The connection between intermediate relay UE and the last relay UE has to be the same link, otherwise multi-path with 2-indirect path cannot be avoided, which is out of WI scope. |
| Apple | Yes |  |

### [Question 2.5B-3] As Figure 5B-3, do you think two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s)? i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s).

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson |  | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The connection between intermediate relay UE and the last relay UE has to be the same link, otherwise multi-path with 2-indirect path cannot be avoided, which is out of WI scope. |
| Apple | Yes | This is a valid case, the intermediate relay UE be chose to use different last relay UE for different remote UE. |

### [Question 2.5B-4] If your answer of the [Question 5B-2] or [Quest 5B-3] was “NO”, do you have any idea to prevent happening these topologies? The answer to this question may be the same as the answer to [Question 5A-6]. However, if you want to describe further for the solution to prevent these topologies, you can write down.

| Company | Describe how to prevent |
| --- | --- |
| OPPO | like replied in Q2.5A-7 |

### [Question 2.5B-5] If your answer was ‘yes’ for one of the questions 2.5B-1, 2.5B-3, or 2.5B-3, do you have any preference on which topology should be handled priori than others for Rel-19 multi-hop? If you have any preference, you can describe it with reason.

| Company | Preference with reason |
| --- | --- |
| Ericsson | 2.5B-3 2.5B-2 (following decreasing priority order) |
| Apple | All. No preference of priority. |

- Case-C

  - The red and green link from gNB to Remote U are for multi-hop U2N relay operation.

  - In Figure 2.5C-1, the big blue circle at the last Relay UE means the L2 ID(D) and L2 ID(D’) belong to the physically same last relay UE. The big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically same intermediate Relay UE. And, the big blue circle at the Remote UE means the L2 ID(A) and L2 ID(A’) belong to the physically same Remote UE. Each indirect link belongs physically same last Relay UE, intermediate Relay UE, and Remote UE. However, each indirect link has its own PC5 connection (i.e., has its own L2 ID pair) to support the multi-hop relay path. Each last Relay UE, intermediate Relay UE, and Remote UE has each only one C-RNTI when they are connected.

  - In Figure 2.5C-2, each last Relay UE(A) and last Relay UE(B) is physically different last Relay UE. The big blue circle at the Remote UE means the L2 ID(A) and L2 ID(A’) belong to the physically same Remote UE. The big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically same intermediate Relay UE. Each indirect link belonging to the physically same Remote UE has connections with gNB via physically different two last Relay UE(s) and the physically same intermediate Relay UE. Each last Relay UE(A) and last Relay UE (B) is connected to the same gNB/cell. Each indirect path has its own PC5 link(s) and Uu links to support the multi-path.

(Figure 2.5C-1)

(Figure 2.5C-2)

### [Question 2.5C-1] As Figure 2.5C-1, do you think two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically same last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | No | This seems to be a multi-path scenario for remote UE, which is not in the scope |
| OPPO | No (if the L2 ID is different) | This is excluded by the conclusion for single hop U2N Relay, i.e., a single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. |
| Apple | No |  |

### [Question 2.5C-2] As Figure 2.5C-2, do you think two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | No | This seems to be a multi-path scenario for remote UE, which is not in the scope |
| OPPO | No (if the L2 ID is different) | This is excluded by the conclusion for single hop U2N Relay, i.e., a single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. |
| Apple | No |  |

### [Question 2.5C-3] If your answer of the [Question 2.5C-1] or [Question 2.5C-2] was “NO”, do you have any idea to prevent happening these topologies? The answer to this question may be the same as the answer to [Question 2.5A-6] or [Question 2.5B-4]. However, if you want to describe further for the solution to prevent these topologies, you can write down.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson |  | We can include restriction in stage 2 spec. |
| OPPO | like replied in Q2.5A-7 | like replied in Q2.5A-7 |
| Apple |  | We can ask SA2 for how to do this |

### [Question 2.5C-4] If your answer was ‘yes’ for one of the questions 2.5C-1, or 2.5B-2, do you have any preference on which topology should be handled priori than others for Rel-19 multi-hop? If you have any preference, you can describe it with reason.

| Company | Preference with reason |
| --- | --- |

- Case-D

  - Figure 2.5D-1, 2.5D-2 and 2.5D-3 are the similar to the Figure 2.5A-3, 2.5B-3 and 2.5C-2 except that each multi-hop indirect path have connect with different cells in the same gNB.

(Figure 2.5D-1)

(Figure 2.5D-2)

(Figure 2.5D-3)

### [Question 2.5D-1] For each figure, i.e., Figure 2.5D-1, 2.5D-2, 2.5D-3, do you think these three cases should be considered for Rel-19 multi-hop topologies?

| Company | For Figure 2.5D-1 (Yes/No) | For Figure 2.5D-2 (Yes/No) | For Figure 2.5D-3 (Yes/No) | Reason |
| --- | --- | --- | --- | --- |
| Ericsson | Maybe Yes | Maybe Yes | No | D-3 seems to be a multi-path scenario for remote UE, which should be avoided. |
| OPPO | No | No | No | Out of R19 WI scope. |
| Apple | Yes | Yes and Yes also for different gNB case | Maybe |  |

## 2.6 Others

### [Question 2.6] If there is further issue we should handle in this email discussion but missing, you can describe it freely.

| Company | Other issues that should be handled |
| --- | --- |
| Apple | We need also cover the case that the last relay UES are served by different gNB for case 2.5D-2. |

# 3. Phase 2 Discussion

[TDB]

# Conclusion and recommendation

In conclusion, Rapporteur recommends agreeing the following proposals:

[TDB]

# Annex A: RAN2 Agreements on Multi-hop Relay

## RAN2#127

- From RAN2 perspective, models A and B can both be supported.

- Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying.

- Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying

- Reuse existing resource pools defined for discovery message transmission and reception

- Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE

- Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18.

- If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

- If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

- FFS discovery conditions for the intermediate relay UEs.

- The following connections are assumed as a baseline to be needed:

  - From last Relay UE perspective:

    - A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

    - A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

  - From intermediate relay UE perspective (including first relay UE):

    - A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

    - An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

    - FFS what RRC states are supported for the intermediate relay UE

  - From U2N Remote UE perspective:

    - A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

    - An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

- The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18)

- The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).