3GPP TSG-RAN WG2 #128	R2-240xxxx

Orlando, US, November 18-22, 2024

Agenda Item:	8.13.2

Source: 	LG Electronics Inc.

Title: 	Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

Scope:

  - Discovery message initiating/forwarding condition at intermediate relay UE

  - PC5 AS conditions for discovery at the last relay UE

  - Reselection triggers for all UEs

  - (Re)selection criteria for all UEs (considering discovery models A/B and whether criteria beyond the first hop are considered)

  - Whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last relay UE via one (physically same) intermediate relay UE

Intended outcome: Report to RAN2#128

Deadline: Very long (for RAN2#128)

The purpose of the first phase of email discussion is to address the basic subjects within the scope and gather companies’ opinions on any missing parts that should be discussed. In the second phase, the Rapporteur makes summary proposals and can discuss additional subjects based on the companies’ inputs from the first phase email discussions. Also, throughout the email discussion, we will consider extensibility over 2-hops based on the WID of Rel-19 multi-hop relay.

The deadline for this email discussion will be as follows:

- Phase 1 Deadline 2024-10-21 UTC 10:00

- Phase 2 Deadline 2024-11-04 UTC 10:00

## Contact information

| Company | Email address |
| --- | --- |
| LG | seoyoung.back@lge.com |
| Ericsson | Min.w.wang@ericsson.com |
| OPPO | lengbingxue@oppo.com |
| Sharp | kawano.takuma@sharp.co.jp |
| Apple | Zhibin_wu@apple.com |
| CATT | xuhao@catt.cn |
| ZTE | Wang.mengzhen@zte.com.cn |
| Huawei,HiSilicon | Jagdeep Singh (jagdeep.singh6@huawei.com) |
| TCL | Zhe21.chen@tcl.com |
| Fraunhofer | julian.popp@iis.fraunhofer.de |
| Spreadtrum | Shannen.cao@unisoc.com |
| Kyocera | henry.chang@kyocera.com |
| ASUSTeK | Shawn_Hung@asus.com |
| Samsung | ww1016.wang@samsung.com |
| vivo | liangjing@vivo.com |
| InterDigital | martino.freda@interdigital.com |
| AT&T | joseph.schumacher@att.com |
| Qualcomm | jianhua@qti.qualcomm.com |
| Xiaomi | Yangxing1@xiaomi.com |

# 2. Phase 1 Discussion

## 2.1 Discovery message initiating/forwarding condition at intermediate Relay UE

(The condition to be intermediate Relay UE)

The agreement for the last Relay UE to have upper and lower Uu threshold bound was made at the last meeting as the followings:

- If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission.

However, it’s not made an agreement related to the Uu threshold condition for the intermediate Relay UE.

### [Question 2.1-1] Do you think the intermediate Relay UE has the upper and/or lower bound of the Uu threshold? Please check your opinion and you can describe the reason if you want.

| Company | Need upper bound | Need lower bound | Need both bounds | Need none of them | You can write down the reason for your answer if you want. |
| --- | --- | --- | --- | --- | --- |
| Ericsson | No | No | No | Yes | For the remote UE, the intermediate relay UE is not mandated to be in RRC_CONNECTED. In addition, the intermediate relay UE may be OOC. However, this doesn’t exclude the case where the intermediate relay UE itself may need a relay connection for itself to transmit its own data to the network. In this case, the intermediate relay UE will operate as a remote UE for its own traffic. |
| OPPO | Yes | No | No | No | For lower bound, it is not needed for the intermediate Relay since it can either in-coverage or out-of-coverage; For upper bound, it is needed since the intermediate relay should not in the very cell centre (since in that case, it should be the last relay). |
| Sharp | Yes | No | No | No | If the intermediate relay UE can act as a Last Relay UE, it should act as a Last Relay UE. And upper bound is configured for the legacy relay UE. Therefore, upper bound should be configured (but it is up to NW how to configure the threshold which value can be set to infinity). [Sharp2] In LTE U2N relay, upper bound for relay UE was introduced to avoid wasteful relay UE operation since remote UE’s position should be cell edge or OOC. So, in accordance with this principle, upper-bound is needed. For OOC/non-relay-support cell case, the upper bound is not configured via SIB/dedicated RRC signalling. In this case, UE can act as intermediate relay UE (it might be new condition). |
| Apple | No | No | No | Yes | For OOC intermediate UE, bounds are not needed. For an intermediate relay UE in coverage, the reason it does not acting as a last relay UE is that its own serving cell does not support relay. So, in that case, it will not be subject to bounds configuration either. So, in all cases, there is no need for bounds. |
| CATT | No | No | No | Yes | In RAN2#127 meeting, RAN2 concluded that the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network converage. We think there is no doubt about the OOC case. But for the IC case, the proponent of upper bound raised that the intermediate relay UE should be considered to avoid too close to the cell centre. Logically correct, but in case the last relay UE and the intermediate relay are under different network coverage/PLMN case, introduing this upper bound will make the situation more complexity. All in all, we think it is not necessary to introduce any Uu threshold for intermediate Relay UE for discovery message initiating condition. |
| ZTE | No | No | No | Yes | We think an upper bound to prevent a relay UE to act as an intermediate relay UE or to mandate the relay UE to act as a Last relay is not needed. Even if the Uu link quality is good, it is up to UE implementation (considering other factors) to whether to act as an intermediate relay or not. That is, as long as the relay UE decides to not act as a Last relay UE, it is naturally it could be an intermediate relay even it is at the centre of the cell. |
| Huawei, HiSilicon | Yes | No | No | No | It makes sense to have upper-bound for intemediate relay UE as the it should not be allowed to operate in the cente of the cell. Alternatively if the intermediate relay UE is connected to the last relay UE like a remote UE, the Uu threshold condition for the remote UE can be reused for the intermediate relay UE |
| TCL | Yes | No | No | No | Agree with SHARP |
| Fraunhofer | Opt. | No | No | Yes | The intermediate relay can make use of an upper bound, if configured. As an intermediate relay, it is not mandated to be in-coverage. If the intermediate relay becomes RRC_CONNECTED it will most likely act as a U2N relay (last relay) and will be configured with an upper bound. |
| Spreadtrum | No | No | No | Yes |  |
| Lenovo | Yes | No | No | Yes | Intermediate relay UE is not located at cell center. Therefore, it makes sense to configue an upper-bound for intemediate relay. Regarding OOC case, both threshoulds are not needed. |
| Kyocera | No | Yes | No | No | We assume if the intermediate relay UE’s RSRP is below a configured low Uu threshold it can transmit discovery, similar to remote UE. We also assume if this relay UE is above the threshold it should serve as a last relay UE which is different from requirements for intermediate relay UE. |
| Samsung | Yes | No | No | No | The intention of multi-hop SL relay is to extend coverage of the cell, especially, for the case that some of UEs are OOC. If without upper bound, it means that each UE can start the discovery procedure, even for the UEs in the center of cell. This will cause the burden at UEs side. On the other hand, it may increase the number of hops of E2E paths since the relay UEs with good signal quality may become the intermediate UEs. Thus, it is better to have an upper bound to restrict the UEs performing discovery. |
| vivo | Yes | No | No | No | Intermediate U2N relay UE can be both in-coverage or out-of-coverage, so we understand for Intermedia U2N relay UE, there is no need of any lower limit for Uu threshold condition. For upper bound, we slightly prefer to have a upper bound, because if we look at the ‘Intermediate Relay UE <-> last relay UE <-> gNB’ path, the Intermediate Relay UE is like the Rel-17 U2N remote UE, which has the upper limit for Uu threshold. Meanwhile, if the Intermediate Relay UE is too close to gNB (i.e. has a very high Uu RSRP), it is questionable why it is not serving as the last relay directly. |
| Interdigital | Yes | No | No | No | The upper bound is needed as the UE in the cell center should be the U2N instead of Intermediate Relay to minimize the number of hops to the network. |
| LG | Yes | No | No | No |  |
| AT&T | Maybe | No | No | Maybe | We tend to agree with commentors noting that a potential intermediate UE close to the cell centre may be in RRC_INACTIVE state and could potentially serve the remote UE better if instead of being an intermediate UE, they would instead perform the last UE function. It is also worth considering that discussion of “the” intermediate UE may be misleading. We envision scenarios where many intermediate candidate UEs may be distributed within a cell. We believe that an upper RSRP bound would tend to reduce the number of UEs offering intermediate service (simplifying the remote UE selection process) and would also tend to shift the distribution of candidate intermediate UEs closer to cell edge where coverage enhancement typically might be expected. Our opinion is “maybe”, because we recognize that coverage holes may also exist close to cell centre anyway, e.g., in cases where service may be needed in an enclosure or space with high penetration loss. We invite discussion on the mechanisms used to inhibit UEs from offering relay service based on RSRP or other factors. |
| Qualcomm | No | No | No | Yes |  |
| Xiaomi | No | No | No | No | Agree with ZTE |

### Summary:

18 companies provided inputs:

None of the upper and lower bound is needed: 8 companies (Ericsson, Apple, CATT, ZTE, Fraunhofer(opt: upper bound), Spreadtrum, QC, AT&T)

Upper bound needed: 11 companies (OPPO, Sharp, HW, TCL, Lenovo, Kyocera, Samsung, vivo, IDT, LG, AT&T)

It is clear that all companies believe that a lower bound Uu threshold for intermediate Relay UE is not required. So, we can make the following proposal.

Proposal 1a: A lower bound is not required for the intermediate Relay UE.

The number of companies that believe an upper bound is needed is greater than the number of companies believe an upper bound is not needed. However, it seems that the number is not significantly different. The rapporteur believes that this issue needs to be discussed further in the Phase 2 email discussion.

Proposal 1b: For the intermediate Relay UE, RAN2 needs to discuss whether upper bound is required in phase 2.

(discovery message initiation at intermediate Relay UE)

The conclusion in SA2 TR 23.700-03 and the subclause 6.3.2.5.2/3 in TS 23.304, it looks like the intermediate Relay UE does not initiate a discovery announcement message broadcast in discovery model A and discovery model B.

[Conclusions on SA2 TR 23.700-03]

- When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay.

- When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

However, considering multi-hop extensibility over 2-hops, additional schemes can be required in the discovery procedure. For example, the intermediate Relay UE may be required to initiate the discovery procedure when the RLF happens or the RSRP threshold is below the configured threshold among the hops between intermediate Relay UEs or between the intermediate Relay UE and the last Relay UE. It may be helpful to fast recovery of the multi-hop relay link as the hop counts increase. However, it’s not sure whether intermediate Relay UE can initiate the discovery procedure. Also, it’s not sure whether this issue can be discussed in the RAN2, not SA2.

### [Question 2.1-2] Do you think discovery procedure should be initiated at the intermediate Relay UE considering multi-hop extensibility over 2-hops? If your answer is ‘yes’, describe how to handle this issue.

| Company | Yes/No | Describe how to handle this issue |
| --- | --- | --- |
| Ericsson | No | In Rel-19, it is sufficient to assume a baseline mechanism, i.e., it is only remote UE that triggers relay reselection. If there is RLF detected on an intermediate hop, it is enough that the RLF info is relayed to remote UE. In our view, allowing intermediate relay UE to initiate discovery, is an optimization, which can be delayed to future releases. |
| OPPO | See comments | After checking SA2 colleagues, our understanding on SA2 conclusion is intermediate relay UE should not generate discovery message for multi-hop relay without information source (i.e., the discovery message has to be generated based on the info from Last relay or remote UE). But it doesn’t restrict the discovery transmission after link establishment for link monitoring purpose. |
| Sharp | No | We think it should be discussed in SA2 if needed. And, in our view, if we assume that the intermediate relay UE also acts as a remote UE, the intermediate relay UE can initiate discovery procedure for recovery. |
| Apple | See comment | We do not understand this question. SA2 said “……based on the announcement message sent by the Intermediate Relay”. We think this means the relay UE does initiate model-A discovery message by itself although the contents are generated based on the information collected from remote UE or last relay UE via discovery process. SA2 has already agreed that the relay UE can generate discovery message transmission itself. Does the question intend to ask for the case of relay reselection instead? |
| CATT | No | What we discudiscussed here is foucus on MH initial path discovery instead of MH path maintence. In the current agreed CR of SA2, for discovery model A, only the last relay UE will initiate the discovery procedure, and for discovery model B, only the remote UE will initiate the discovery procedure. |
| ZTE | No | Not sure the intention of this question and what conclusion could reach. Maybe we/SA2 should first discuss whether intermediate relay could/should initiate discovery procedure/perform relay reselection when the PC5 RLF happens or the PC5 RSRP is below a threshold among PC5 hops. |
| Huawei, HiSilicon | See Comments | There could be different scenarios - For the discovery procedure before the PC5 connection establishment phase, the intermediate relay UE does not initiate a discovery announcement message, it just forwards the Discovery Anouncement message as described in 6.1.2.1.1 of SA2 TR 23.700-03. - After the PC5 connection is established between the intermediate relay UE and the last Relay UE supporting multihop relay, the intermediate Relay can send the discovery anouncement message to the other remote UE so that they can use the path for multihop. - After the unicast data transmission has started or is on going there could be a situation that RLF between the hops is encounted or the RSRP threshold is below the configured threshold among the hops between intermediate Relay UEs or between the intermediate Relay UE and the last Relay UE, then the intermediate relay UE can also perform disccovery. |
| TCL | Yes |  |
| Fraunhofer | No | Agree with CATT |
| Spreadtrum | No |  |
| Lenovo | See comments | When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay. Based on the above, the intermediate relay UE transmits discovery message itself. The content of the discovery could be from the received discovery message based on the SA2 progress. The above is called as ‘forwarding’ in this document and SA2 TR. However, in 23.700, there is the following description. It seems the ‘forwarding’ is also described as ‘initiates’ in 23.700. - The 5G ProSe intermediate Relay initiates Model A discovery only when it has found a UE-to-Network Relay, which has a connection to the network. NOTE: The 5G ProSe intermediate Relay initiates Model A discovery only when it has found a UE-to-Network Relay, which has a connection to the network. The intermediate Relay may decide the maximum number of hops considering the hops to UE-to-Network Relays it has found. |
| Kyocera | No | In the case of SL-RLF between the intermediate relay UE and the last relay UE, the intermediate relay UE should inform the remote UE of the failure, then the remote UE can request the intermediate relay UE to perform discovery on behalf of the remote UE. If the remote UE can find another intermediate relay UE that has a path towards the gNB, there’s no need for the current intermediate relay UE to perform discovery. |
| Samsung | No |  |
| vivo | Yes with comments | According to the description above the question ‘For example, the intermediate Relay UE may be required to initiate the discovery procedure when the RLF happens or the RSRP threshold is below the configured threshold among the hops between intermediate Relay UEs or between the intermediate Relay UE and the last Relay UE’ We understand the intention is to ask whether the relay UE can act as a remote UE to trigger e.g. relay reselection. If that is the case, we understand there is benefits to support this scenario especially considering multiple hops may be considered. |
| Interdigital | See comment | For initial relay selection procedure, we agree that the intermediate relay should forward the discovery message only (e.g., not initiating the discovery transmission by itself). However, for other cases such as link degradation, RLF, the intermediate relay can trigger transmisison of discovery message. |
| AT&T | See comment | We tend to agree with InterDigital. It seems that different scenarios suggest different discovery methods. |
| Qualcomm | See comment | We tend to agree with InterDigital. Additionally, we only talk about discovery trigger, not generation |
| Xiaomi | No | Agree with CATT |

### Summary:

17 companies provided inputs:

Answered as intermediate Relay UE cannot initiate discovery message: 8 companies (Ericsson, Sharp, CATT, ZTE, Fraunhofer, Spreadtrum, Kyocera, Samsung)

Answered as intermediate Relay UE can initiate discovery message: 1 company (TCL)

Answered as “See comments” considers the followings: 8 companies (OPPO, Apple, HW, Lenovo, vivo, IDT, QC, AT&T)

Based on the companies’ inputs, rapporteur can find the following observations:

- The announcement message initiates at the last Relay UE (based on the SA2 spec (23.700)).

- For the intermediate Relay UE to initiate the announcement message by itself, the intermediate Relay UE should have the information for the announcement message provided by the last Relay UE.

- The solicitation message initiates only at the Remote UE (based on the SA2 spec (23.700)).

- Some companies say that the intermediate Relay UE need to trigger the discovery procedure by itself when link degradation or RLF happens. They believe that this scenario is beneficial for supporting multi-hop relay.

Based on the companies’ response, it seems clear that the intermediate Relay UE can initiate an announcement discovery message only when receiving the announcement message from the last Relay UE or, at least, when the intermediate Relay UE has the information for the announcement message from the last Relay UE.

So, we can make the following proposals.

Proposal 2: The intermediate Relay UE can initiate the announcement message upon receiving announcement message or, at least, when the intermediate Relay UE has the information for the announcement message provide by the last Relay UE. The detail can be discussed further.

(discovery message forwarding at intermediate Relay UE)

For the discovery model A, the following agreement was made for relay UE to initiate discovery announcement message broadcast.

- For Model A discovery, the relay UE should only announce the neighbour UEs for which the SD-RSRP/SL-RSRP between the relay UE and the neighbour UE is above a configured threshold in a discovery announcement message.

In Rel-19 multi-hop U2N relay operation, SD-RSRP/SL-RSRP seems to be considered as condition for the discovery announcement message forwarding at intermediate Relay UE.

### [Question 2.1-3] Do you think the discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP is above a configured threshold?

| Company | Yes/No | Reason, if you want to describe. |
| --- | --- | --- |
| Ericsson | Yes |  |
| OPPO | See comments | We understand the channel quality between the intermediate relay and its parent relay needs to be evaluated but may not in form of discovery transmission condition. It is more like a relay selection condition, i.e., the intermediate relay selects its parent relay UE based on the RSRP, and forward the discovery announcement message from the selected parent relay UE. |
| Sharp | Yes |  |
| Apple | Yes |  |
| CATT | No | For multi-hop relay discovery Model A, there is no AS channel condition should be considered for intermediate relay UE when announcing the discovery message. Since according to SA2’s agreed CR, it stated that the intermediate U2N relay had already been authenticated and established a connection to the network before serving the remote UE (for discovery or connection). That is to say, before the intermediate relay UE forwards the announcing discovery message, the PC5-S link between the intermediate relay UE and its parent relay UE had been established in advance. Considering the PC5-link is already standby, the reason to further consider PC5 quality seems not needed. |
| ZTE | Yes | For Model A discovery, from AS layer perspective, the intermediate relay UE forwards/propagates the discovery message only if the PC5 RSRP between the relay UE and the UE from which the discovery message is received is above a threshold. That is, if the PC5 link quality is not good, the intermediate relay should not forward discovery message. Otherwise, message storm may be happened and other mechanism is needed to ensure the PC5 link quality of each hop / to select a proper relay path that each hop has good PC5 link quality to avoid failure. |
| Huawei, HiSilicon | No | We have the following additional comments - SA2 never intend to reuse the U2U relay solution in their TR, including the discovery procedure then why RAN 2 needs to discuss U2U based procedures alone. Please see the comment for the rappotuer on this - The above agreement was for R18 U2U relay discovery for two hops between two remotes UEs and the same principle should not be applied to the Multi hop U2N relay scenario as the R18 U2U can only support 2 hops and if we are thinking of having extensibility of more than 2 hops it will not be possible to support it with R18 U2U mechanisms and they will have to be extended in R19 which we dont think is needed. - Moreover SA2 has concluded that Remote UE either selects the Intermediate Relay (for Model A) or selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay (Model B) as mentioned above it would be appropriate to forward the SD-RSRP or SL-RSRP information in discovery announcement message for Model A or in Relay Discovery Response message for Model B to the remote UE to select the path to reach the UE-to-Network Relay rather than dropping the message. - The other apporach is that the first relay UE already connects to the network via the last relay UE in this case, the first relay UE does not need to forward the discovery message since the first relay UE has already established the sidelink with the last relay UE. The first relay UE can perform model A or model B based on ”direct discovery” to let the remote UE find it. |
| TCL | Yes |  |
| Fraunhofer | Yes |  |
| Spreadtrum | Yes |  |
| Lenovo | No with comments. | Four cases are analyzed as follows. - Case#1: if the intermediate relay UE is in RRC connected state, this condition is no needed. - Case#2: if the intermediate relay UE is idle/inactive, it has PC5 connection towards its parent relay UE. This condition is not needed. - Case#3: the UE can meet the condition of being an intermediate relay UE. But the intermediate relay UE has selected its parent relay UE. This condition is not needed. - Case#4: The UE can meet the condition of being an intermediate relay UE. But the intermediate relay UE has not selected its parent relay UE. This condition could be needed. But in this case, the condition could be a condition of parent relay UE selection as Oppo mentioned. |
| Kyocera | Yes | We assume for Rel-19, the “neighbour UE” from the Rel-18 agreement can be mapped to the last relay UE. It would be necessary for the PC5 connection between the two relay UEs to be good enough before the intermediate relay UE forwards the discovery announcement; otherwise the path to the gNB with this intermediate relay UE cannot be properly established if it were chosen by the intermediate relay UE. We further assume that it is for the intermediate relay UE to determine if the SD-RSRP/SL-RSRP threshold is met. |
| Samsung | Yes |  |
| vivo | Yes | Our first understanding is that the Rel-18 U2U principle may be used here, and the link quality between relay UEs should be evaluated anyway. But we agree to further discuss whether/how to process this and also this may need SA2 confirmation. |
| Interdigital | Yes |  |
| LG | Yes |  |
| AT&T | Yes |  |
| Qualcomm | Yes |  |
| Xiaomi | Yes | Aligned with legacy |

### Summary:

18 companies provided inputs:

“YES”: 15 companies (Ericsson, OPPO, Sharp, Apple, ZTE, TCL, Fraunhofer, Spreadtrum, Kyocera, Samsung, vivo, IDT, LG, AT&T, QC)

“NO”: 3 companies (CATT, HW, Lenovo)

This issue seems to reflect a clear majority view. So, the following proposal is generated.

Proposal 3: The discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP is above a configured threshold.

### [Question 2.1-4] Do you have any further ideas other than threshold of SD/SL-RSRP about the AS condition for forwarding the discovery announcement message at the intermediate Relay UE? If you have anyone, you can add it freely and describe the reason.

| Company | You can describe AS conditions other than SD/SL-RSRP threshold for forwarding discovery announcement message. |
| --- | --- |
| Huawei, HiSilicon | Since SA2 has concluded (Conclusions on SA2 TR 23.700-03) that Remote UE either selects the Intermediate Relay (for Model A) or selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay (Model B) as mentioned above, it would be appropriate to forward the SD-RSRP or SL-RSRP information in discovery announcement message for Model A or in Relay Discovery Response message for Model B to the remote UE for it to select the path to reach the UE-to-Network Relay rather than dropping the message. |
| Kyocera | We assume if the two relay UEs are PC5-RRC connected the intermediate relay UE may also forward the discovery announcement. |

There is no rapporteur proposal.

For the discovery model B, the following agreements was made at Rel-18 U2U relay operation.

- For Model B, the relay UE forwards the solicitation message only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

- For Model B, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE.

The same principle seems to be reused for the multi-hop U2N relay operation as the followings:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

- For Model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

### [Question 2.1-5] Do you think the above descriptions (in the box) about discovery message forwarding conditions are available for the multi-hop operation in discovery model B? If you have any other view, you can describe it.

| Company | Yes/No | If the answer is ‘No’, please describe your opinions. |
| --- | --- | --- |
| Ericsson | Yes | But, the second bullet needs to be updated as - For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. |
| OPPO | No for 1 Yes for 2/3 | For 1, it is not needed since same as U2N Relay case, there is no channel quality condition defined for Relay UE to determine whether to respond the solicitation message from the remote UE. |
| Sharp | Yes |  |
| Apple | Yes for 1, 3, No for 2 | I think for 2, we have to differ from U2U relay case, because the checking of RSRP threshold has to be done by adjacent UEs, which may not have remote UE involved. So, the bullet 2 has to be rephrased to reflect R19 scenario. |
| CATT | Yes | For model B discovery message forwarding conditions discussion, it seems reasonable to discuss the relay discovery for multi-hop U2N relay with an analogy to Rel-18 U2U relay design for intermediate relay UE. Hence, we support it. |
| ZTE | Yes for 1 | The first bullet is needed. It is wasteful for intermediate relay to forward discovery solicitation message if the PC5 link quality towards the UE the discovery solicitation message is received is not good and may cause message storm. And other mechanism is needed to ensure the PC5 link quality of each hop / to select a proper relay path that each hop has good PC5 link quality to avoid failure. The second bullet is not a forwarding condition at intermediate relay. It is a condition for relay (re)selection. In multi-hop SL relay, since there are more hops along the path than R18 single hop U2U relay, it is safer for the intermediate relay to forward the discovery response message by checking the PC5 RSRP again. |
| Huawei, HiSilicon | No | The above agreement was for R18 U2U relay discovery and the same principle should not be applied to the Multi hop U2N relay scenario given that we have a well established U2N discovery procedures described multiple solutions in SA2 TR . Furthermore SA2 never intend to reuse the U2U relay solution, including the discovery procedure hence we think RAN 2 should not adopt U2U based dicovery mechanisms for multihop given that we have a U2N Relay discovey procedures described in the TR. See the reasons given in answer to Question 2.1-3. |
| TCL | Yes |  |
| Fraunhofer | Yes | Agree with Apple. While in a Rel-17/18 U2N case, a relay supporting to reach the destination is quite easy to discover. Introducing multi-hop for U2N connections adds U2U complexity to the U2N case. Thus, reusing U2N alone is not suitable. |
| Spreadtrum | Yes |  |
| Lenovo | No for 1. Yes for 2,3 | No for 1: see the comments for Question 2.1-3. The intermediate relay UE may have PC5 connected already towards its parent relay UE. |
| Kyocera | Yes |  |
| Samsung | Yes |  |
| vivo | Yes | We didn’t see real problems to reuse those Rel-18 U2U principle. This can be further confirmed by SA2 if needed. |
| Interdigital | Yes |  |
| LG | Yes |  |
| Interdigital | Yes |  |
| Qualcomm | Yes | For the second bullet, “the last relay UE” should be changed to “a last relay UE” |
| Xiaomi | Yes |  |

### Summary:

17 companies provided inputs:

- For the first bullet, three companies show their objections (OPPO, HW, Lenovo)

- For the second bullet, two companies show their objections (Apple, HW)

- For the third bullet, one company show their objection (HW)

Because a few companies show their objection for the model B discovery message forwarding conditions, we can discuss the following conditions as baseline for model B discovery message forwarding conditions.

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to a last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. (modified by Ericson’s comment)

- For Model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

Proposal 4: The following model B discovery forwarding condition can be baseline for further discussion:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. (modified by Ericson’s comment)

- For Model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

## 2.2 PC5 AS conditions for discovery at the last Relay UE

In the legacy Rel-17 U2N Relay operation, the U2N Relay UE has upper and lower Uu thresholds bound to act as a U2N Relay UE. For example, the Rel-17 U2N Relay UE can initiate discovery procedure when the Uu RSRP threshold is below a configured upper bound and/or above a configured lower bound.

In multi-hop U2N Relay UE operation, the last Relay UE has a similar condition to act as the last Relay UE as the following agreement.

- If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

Meanwhile, some other companies think that PC5 AS condition can be additionally needed in the case of the last Relay UE in multi-hop. However, it’s not clear which PC5 AS condition can be specified for the last Relay UE. So, we’d like to listen to the PC5 AS condition for discovery at the last Relay UE

### [Question 2.2-1] Do you think PC5 AS condition is needed for discovery at the last Relay UE? If your answer is ‘yes’, describe the reason of the necessity in detail.

| Company | Yes/No | Describe your opinions freely |
| --- | --- | --- |
| Ericsson | No |  |
| OPPO | No | As we replied in Question 2.1-5, in single hop U2N relay, there is no PC5 AS condition defined for the relay UE to determine whether to respond the solicitation message from the remote UE. |
| Sharp | Yes | In Rel-17/18, PC5 AS condition for the last relay UE (i.e. PC5 link quality between the remote UE and the last relay UE) can be evaluated by the remote UE since it was single-hop. However, in multi-hop case, the remote UE cannot evaluate PC5 AS condition for the last relay UE. For discovery model A, PC5 AS condition is NOT needed since intermediate relay UE which directly receives discovery announce message from the last relay UE can evaluate PC5 AS condition for the last relay UE when the intermediate relay UE determines whether to forward it. However, for discovery model B, PC5 AS condition is needed since there is no UE which evaluates PC5 link quality between the intermediate relay UE and the last relay UE if the relay UE doesn’t evaluate PC5 AS condition. This is similar idea with the agreement of R18 U2U relaying as follows; “For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.” Therefore, we think PC5 AS condition is needed for Model B discovery at the last relay UE. |
| Apple | Yes | Same understanding as Sharp. |
| CATT | No | It seems reasonable to discuss the relay discovery for multi-hop U2N relay with an analogy to Rel-17 U2N relay design for the last relay UE. In Rel-17 U2N relay, for relay discovery related discussion, there is no PC5 AS conditions captured yet for discovery message initiating/forwarding condition. |
| ZTE | Yes | In R17 single hop U2N relay, since there is only one hop, there is no big problem that relay UE sends discovery response message without checking PC5 link quality with the remote UE in Model B discovery, as the remote UE could evaluate the PC5 link quality toward the relay UE after receiving the response message, though actually the relay UE does not need to send the response message. However in multi-hop case, the problem is more serious without checking the PC5 link quality when the Last relay sends response message in Model B, i.e. it is wasteful to send discovery response message if the PC5 link quality is not good and may cause message storm as commented above, and other mechanism is needed to ensure the PC5 link quality of each hop / to select a proper relay path that each hop has good PC5 link quality to avoid failure. So for the Last relay UE, upon receiving a discovery solicitation message from an intermediate relay, the Last relay UE should consider the PC5 link quality towards the intermediate relay when deciding whether to response the intermediate relay. |
| Huawei, HiSilicon | No |  |
| TCL | Yes | Agree with SHARP |
| Fraunhofer | Yes | Agree with SHARP |
| Spreadtrum | No |  |
| Lenovo | No | If intermediate relay UE is one of idle/inactive/connected state, the intermediate relay UE has evaluated it already. |
| Kyocera | No | For Model A discovery additional PC5 AS condition is not needed. For Model A discovery, additional PC5 AS condition can be specified for the intermediate UE’s upon receiving the discovery announcement from the last relay UE (see response for Question 2.1-3). For Model B discovery, we assume the response to the solicitation message at the last relay UE doesn’t require the last relay UE to check PC5 AS condition before sending. |
| Samsung | Yes |  |
| vivo | Yes | Agree with Sharp that we can consider to reuse the R18 U2U principle. |
| Interdigital | Yes | Agree with Sharp. |
| LG | Yes |  |
| AT&T | Yes | Agree with Sharp |
| Qualcomm | Yes | Agree with Sharp |
| Xiaomi | No with comments | We agree there may be some different compared with legacy. But if PC5 AS condition is needed, this should be applied for all relay UEs, e.g. intermediate and relay UE. |

Summary:

18 companies provided inputs:

- YES: 11 (Sharp, Apple, ZTE, TCL, Fraunhofer, Samsung, vivo, Interdigital, LG, AT&T, QC)

- NO: 7 (Ericsson, OPPO, CATT, HW, Spreadtrum, Lenovo, Kyocera)

Companies seem to think like that:

- For the discovery announcement message initiation (in the case of model A), the last Relay UE doesn’t need to check the PC5 AS condition.

- For the discovery response message transmission (in the case of model B), the last Relay UE needs to check PC5 AS condition before sending the discovery response message to the (first) intermediate Relay UE.

- Meanwhile, in the legacy Rel-17 U2N relay, the U2N Relay UE doesn’t have any PC5 AS condition for transmitting the announcement/response discovery message.

There is no clear majority view. However, based on the companies’ response, rapporteur made the following proposals:

Proposal 5a: For the discovery announcement message initiation (in the case of model A), the last Relay UE doesn’t need to check PC5 AS condition.

Proposal 5b: In the case of discovery model B, the following two options can be further discussed

(option 1) For the discovery response message transmission (in the case of model B), the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first) intermediate Relay UE.

(option 2) As the legacy Rel-17 U2N relay, the U2N Relay UE doesn’t have any PC5 AS conditions for transmitting the announcement/response discovery message. The same principle can be applied to the last Relay UE.

## 2.3 (Re)selection triggers for all UEs

Based on the Rel-17 U2N Relay (re)selection triggering conditions in subclause 5.8.15.3 in TS 38.331, the Rel-19 Remote UE for multi-hop may reuse them for relay (re)selection such as the following:

[Relay (re)selection triggering conditions for Remote UE] - If the Remote UE has no serving cell; - If the Remote UE does not have a selected intermediate Relay UE; - If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected intermediate Relay UE; - If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE; - If the Remote UE receives a notification message from the (first) intermediate Relay UE caused by one of the following: h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE) h-2) if intermediate Relay UE receives RRCReconfiguration message for HO h-3) if intermediate Relay UE performs cell reselection h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE h-5) if intermediate Relay UE receives PC5-RRC connection release between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

### [Question 2.3-1] Do you think the above relay (re)selection triggering conditions (from (a) to (h) in the box) based on the legacy Rel-17 U2N can be reused for Rel-19 Remote UE? You can answer exclude and include conditions separately among them. You can add other triggering conditions with reason, if you want.

| Company | include conditions | Exclude conditions with reason | Additional conditions with reason (other than the above conditions) |
| --- | --- | --- | --- |
| Ericsson | a),b),c),d),e),g) h-1 in addition, f and h-5 may be generalized as “remote UE receives a message from intermediate relay UE indicating PC5 link release” | h-2,h-3, and h-4 the intermediate relay UE is not mandated to be in RRC_CONNECTED | - PC5 signal strength between the intermediate relay UE and the L2 last U2N Relay UE is below a (pre)configured signal strength threshold. - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by last U2N Relay UE. |
| OPPO | a) with some rewording c)-h) | a): Can be covered by “Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold ” b): We understand the intention while “has no selected intermediate relay UE” seems not the trigger condition for relay selection, but the diff between relay selection and relay reselection. | the Remote UE receives a notification message from the (first) intermediate Relay UE caused Uu RLF at the intermediate relay UE Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold Indicated by upper layer of the U2N Remote UE. |
| Sharp | A,b,c,d,e,f,g,h as baseline. And further details should be discussed later. For condition (f/g/h), “intermediate relay UE” could be changed to “first relay UE” for clarification for now. For condition (f) it can be integrated to condition (e) from AS perdpective. |  | Condition: SL/SD-RSRP of the currently selected first(intermediate) relay UE is above a configured threshold. Reason: If the PC5 link between the remote UE and the first (intermediate) relay UE is very good (i.e., they are almost in the same location), it is reasonable for the remote UE to switch to the intermediate (last) relay UE, from delay perspective. [comments] For condition (c), what is the intention of “the SL-RSRP of the currently selection intermediate Relay UE”? We think “the SL-RSRP of the currently selected intermediate Relay UE” is align with legacy and condition (d). If there is no difference, please ignore this comment. |
| Apple | All except f | For f, our understand is that this is the upper layer of intermediate relay UE, so this cannot be used as a remote UE trigger. | 1) Receiving a notification message from PC5-RRC for any path disruption occurred in one or more hops (Uu RLF, SL RLF or both…) 2) Any upper layer reasons to trigger path (re)selection |
| CATT | b)-h) | For a), In RAN2#127 meeting, the below had been agreed to under condition of discovery, “If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.” Have we ever accepted that one condition can simultaneously trigger both discovery and relay (re)selection? |  |
| ZTE | It is better to differentiate triggers for relay selection and re-selection. - for relay selection; c) - h) for relay re-selection; | b), same view as OPPO. | Additional triggers for relay selection: - Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold; - Upper layer of the Remote UE indicates to select (multi-hop) relay; Additional triggers for relay re-selection: - remote UE receives the notification of cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the last relay UE propagated along the multi hops; - remote UE receives the notification of PC5 RLF or PC5 RSRP degradation of an intermediate relay (not only the first intermediate relay); |
| Huawei, HiSilicon | a)/b) for relay selection c) /d)/ e)/ f)/ g)/ h) for relay (reseleciton) |  |  |
| TCL | A,b,c,d,e,f,g,h, |  |  |
| Fraunhofer | a), b), c), d), e), f), g) h-1, h-5 | h-2, h-3, h-4 | Similar to ZTE, we would like to differentiate triggers and also include triggers based on further intermediate relays. |
| Spreadtrum | All | No | Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by last U2N Relay UE. |
| Lenovo | a-h |  | In condition h, the remote UE receives one of a notification message or a PC5-S release message from the (first) intermediate Relay UE. Therefore, the PC5-S release message is missing in condition h. |
| Kyocera | a) to h) can be included as baseline. |  | The intermediate relay UE may receive notification msg from the last relay UE (e.g., Uu RLF) which should also be forwarded to the remote UE. |
| ASUSTeK | All |  | The intermediate Relay UE sends a notification message to the Remote UE if the serving cell of the intermediate Relay UE is changed due to notification message from the last (/serving intermediate) relay UE. |
| Samsung | a)- g) h) may need some clarification |  | For h), can remote UE receive the notification message from other intermediate relay UEs(except the first intermediate relay UE)? |
| vivo | a),b),c),d),e),f),g),h-5 for h-1, may be further considered whether to be applied to remote UE or intermediate Relay UE | h-2 to h-4 can be further discussed |  |
| Interdigital | We should differentiate between selection and reselection. a, b for selection. c, d, e, g, h for relay reselection. | f is the condition at intermediate relay. The condition should be associated with the remote UE. |  |
| LG | All |  |  |
| AT&T | Agree with ZTE, et.al that selection and reselection are different cases and would be more clearly discussed separately. |  |  |
| Qualcomm |  | h | The conditions in h depend on other issues, e.g. h1 is related to whether the intermediate relay will notify the RLF or intermediate relay UE can perform relay reselection. H2-h4 depends on the control plane procedure. |
| Xiaomi | All |  |  |

### Summary:

19 companies provided inputs:

For the case (a), (c), (d), (e), (g)

- YES :19 companies (Ericsson, OPPO, Sharp, Apple, CATT, ZTE, HW, TCL, Fraunhofer, Spreadtrum, Lenovo, Kyocera, ASUSTeK, Samsung, vivo, IDT, LG, AT&T, QC)

- NO: 0 company

For the case (b)

- YES :17 companies (Ericsson, Sharp, Apple, CATT, HW, TCL, Fraunhofer, Spreadtrum, Lenovo, Kyocera, ASUSTeK, Samsung, vivo, IDT, LG, AT&T, QC)

- NO: 2 companies (OPPO, ZTE)

For the case (f)

- YES :17 companies ((Ericsson, OPPO, Sharp, CATT, ZTE, HW, TCL, Fraunhofer, Spreadtrum, Lenovo, Kyocera, ASUSTeK, Samsung, vivo, LG, AT&T, QC)

- NO: 2 company (Apple, IDT)

For the case (h-1), (h-5)

- YES :18 companies (Ericsson, OPPO, Sharp, Apple, CATT, ZTE, HW, TCL, Fraunhofer, Spreadtrum, Lenovo, Kyocera, ASUSTeK, Samsung, vivo, IDT, LG, AT&T)

- NO: 1 company (QC)

For the case (h-2), (h-3), (h-4)

- YES :15 companies (OPPO, Sharp, Apple, CATT, ZTE, HW, TCL, Spreadtrum, Lenovo, Kyocera, ASUSTeK, Samsung, IDT, LG, AT&T)

- NO: 4 companies (Ericsson, Fraunhofer, vivo, QC)

Based on the majority companies’ input, the following relay (re)selection triggering conditions for Remote UE can be baseline for the multi-hop relay operation. And we can discuss further for the other relay (re)selection triggering conditions based on the companies’ inputs.

Proposal 6: The following relay (re)selection triggering conditions for Remote UE are supported for the multi-hop relay operation at least when there is only one intermediate Relay UE.

FFS: further relay (re)selection triggering conditions can be discussed at phase 2.

[Relay (re)selection triggering conditions for Remote UE] - If the Remote UE has no serving cell; - If the Remote UE does not have a selected intermediate Relay UE; - If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected intermediate Relay UE; - If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE; - If the Remote UE receives a notification message from the intermediate Relay UE caused by one of the following: h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE) h-2) if intermediate Relay UE receives RRCReconfiguration message for HO (if the intermediate Relay UE is in RRC_CONNECTED) h-3) if intermediate Relay UE performs cell reselection h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE h-5) if intermediate Relay UE receives PC5-RRC connection release between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

The intermediate Relay UE may reuse similar conditions for relay (re)selection. The relay (re)selection conditions for intermediate Relay UE can be suggested like the following:

[Relay Triggering conditions for intermediate Relay UE] - If the intermediate Relay UE has no serving cell; - If the intermediate Relay UE does not have a selected last (/serving intermediate) Relay UE; - If the SL-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE; - If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE; - If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE caused by one of the following: h-1) if last Relay UE detects Uu RLF h-2) if serving intermediate Relay UE detects PC5 RLF h-3) if last (/serving intermediate) Relay UE receives RRCReconfiguration message for HO h-4) if last (/serving intermediate) Relay UE performs cell reselection h-5) if last (/serving intermediate) Relay UE fails Uu RRC connection establishment/Resume h-6) if last (/serving intermediate) Relay UE receives Uu connection release between gNB and last (/serving intermediate) Relay UE h-7) if serving intermediate Relay UE receives PC5-RRC connection release between last (/the other serving intermediate) Relay UE and the serving intermediate Relay UE

### [Question 2.3-2] Do you think the above suggested relay (re)selection triggering conditions (from (a) to (h) in the box) based on the legacy Rel-17 U2N can be used for intermediate Relay UE? You can answer exclude and include conditions separately among them. You can add other triggering conditions with reason, if you want.

| Company | include conditions | Exclude conditions with reason | Additional conditions with reason (other than the above conditions) |
| --- | --- | --- | --- |
| Ericsson | None, in our view, intermediate relay has no need to trigger relay (re)selection, it is an optimization. |  |  |
| OPPO | a) h-1) with some rewording c)-h) | a): Can be covered by “Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold ” b): We understand the intention while “has no selected intermediate relay UE” seems not the trigger condition for relay selection, but the diff between relay selection and relay reselection. | Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold Indicated by upper layer of the U2N Remote UE. For h-1), it should be “if last(/serving intermediate) Relay UE detects Uu RLF” since intermediate relay may also detect Uu RLF. |
| Sharp | [comment] If RAN2 assumes that the intermediate relay UE also act as a remote UE, no condition is needed. We consider this assumption helps keep the specification simple and also achieves the same effect as if the intermediate relay UE were to perform relay reselection. |  |  |
| Apple | None | Intermediate relay UE shall not trigger relay reselection by itself, basically, it need just send notification message towards the remote UE about RLF. |  |
| CATT | None |  |  |
| ZTE | None | When intermediate relay triggers relay reselection, all the downstream UEs are impacted. The reselected path by the intermediate relay may be not the good path for all the downstream UEs. It may be better for each multi-hop remote UE to know the link status of the intermediate hops and to decide whether to trigger relay reselection by itself. |  |
| Huawei, HiSilicon | The behavior can be simialr to the remote UE in R17 |  |  |
| TCL | Non, agree with Ericsson |  |  |
| Fraunhofer | None | Agree with ZTE |  |
| Spreadtrum | None | Same view with sharp |  |
| Lenovo | a-h. the intermediate relay UE can be considered as Rel-17 relay UE. |  | In condition h, the intermediate relay UE receives one of a notification message or a PC5-S release message from its parent Relay UE. Therefore, the PC5-S release message is missing in condition h. |
| Kyocera | None |  | Upon detecting one of the conditions from a) – h), the remote UE should be notified by the intermediate relay UE and the remote UE may request the intermediate relay UE to perform relay reselection, if needed. |
| Samsung | At least h) |  | The intermediate relay UE can receive the notification message from its serving node. This can trigger the (re)selection. |
| vivo | c),d),g),FFS h) Consider two additional hop case, it may be easier to allow last relay UE reselection by Intermediate Relay UE especially when only the link between them deteriorates We can limit the case to only reselection. For selection, may be performed by remote UE as in SA2 discussion. |  |  |
| Interdigital | c, d, e, f g, h. It is beneficial for a Intermediate relay to trigger relay reselection if the current link is no longer good (e.g., RLF, degradation). |  |  |
| LG | None |  |  |
| AT&T | None. Agree with Ericsson et.al. |  |  |
| Qualcomm | From b to g. |  | For a, it is talking about the relay UE role change from last relay UE to intermediate relay UE? For h, it depends on control plane procedure, should wait. |
| Xiaomi | None | Agree with Apple and ZTE |  |

### Summary:

18 companies provided inputs:

- For (a)

- Yes: 3 companies (OPPO, Lenovo, IDT)

- No: 15 companies (Ericsson, Apple, CATT, ZTE, TCL, Fraunhofer, Spreadtrum, Kyocera sharp, HW, Samsung, vivo, AT&T, QC, LG)

- For (e), (f)

- Yes: 4 companies (OPPO, Lenovo, IDT, QC)

- No: 14 companies (Ericsson, Apple, CATT, ZTE, TCL, Fraunhofer, Spreadtrum, Kyocera sharp, HW, Samsung, vivo, AT&T, LG)

- For (b)

- Yes: 1 company (QC)

- No: 17 companies (Ericsson, Apple, CATT, ZTE, TCL, Fraunhofer, Spreadtrum, Kyocera sharp, HW, Samsung, vivo, IDT, AT&T, LG)

- For (c), (d), (g)

- Yes: 5 companies (OPPO, Lenovo, vivo, IDT, QC)

- No: 13 companies (Ericsson, Apple, CATT, ZTE, TCL, Fraunhofer, Spreadtrum, Kyocera sharp, HW, Samsung, AT&T, LG)

- For (h)

- Yes: 5 companies (OPPO, Lenovo, Samsung, (partly) vivo, IDT)

- No: 12 companies (Ericsson, Apple, CATT, ZTE, TCL, Fraunhofer, Spreadtrum, Kyocera sharp, HW, AT&T, LG)

** 1 company (QC): it depends on control plane procedure.

Some companies seem to think that relay (re)selection triggering conditions for the intermediate Relay UE may be required. However, according to the majority opinion, the following agreement is reached.

Proposal 7: Following relay (re)selection triggering conditions are not supported for intermediate Relay UE.

[Relay Triggering conditions for intermediate Relay UE] - If the intermediate Relay UE has no serving cell; - If the intermediate Relay UE does not have a selected last (/serving intermediate) Relay UE; - If the SL-RSRP of the currently selection last (/serving intermediate) Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE; - If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE; - If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE caused by one of the following: h-1) if last Relay UE detects Uu RLF h-2) if serving intermediate Relay UE detects PC5 RLF h-3) if last (/serving intermediate) Relay UE receives RRCReconfiguration message for HO h-4) if last (/serving intermediate) Relay UE performs cell reselection h-5) if last (/serving intermediate) Relay UE fails Uu RRC connection establishment/Resume h-6) if last (/serving intermediate) Relay UE receives Uu connection release between gNB and last (/serving intermediate) Relay UE h-7) if serving intermediate Relay UE receives PC5-RRC connection release between last (/the other serving intermediate) Relay UE and the serving intermediate Relay UE

## 2.4 (Re)selection criteria for all UEs (considering discovery models A/B and whether criteria beyond the first hop are considered)

In the Rel-18 U2U Relay, the following agreements are generated.

- SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

The Rel-18 U2U relay (re)selection criteria may be reused to the Rel-19 multi-hop U2N relay (re)selection criteria.

### [Question 2.4-1] Do you think the above Rel-18 relay (re)selection criteria, from (1) to (3) in the box, can be reused for relay (re)selection criteria for the Rel-19 multi-hop?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Sharp | Yes |  |
| Apple | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Huawei, HiSilicon | No with comments | SA2 never intend to reuse the U2U relay solutions in their TR, including the discovery procedure or reslection procedures for multihop. We can simply focus on reusing the R17 U2N solutions and principles for the U2N multi hop relays |
| TCL | Yes |  |
| Fraunhofer | Yes | The criteria can be reused as long as the overall path is also considered when re-selecting. |
| Spreadtrum | Yes |  |
| Lenovo | Yes |  |
| Kyocera | Yes |  |
| ASUSTeK | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| Interdigital | Yes |  |
| LG | Yes |  |
| AT&T | Yes |  |
| Xiaomi | Yes |  |

### Summary:

18 companies provided inputs:

- Yes: 17 companies

- No: 1 company

Proposal 8: Rel-18 relay (re)selection criteria can be reused for relay (re)selection criteria for the Rel-19 multi-hop.

- SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

### [Question 2.4-2] If you think that the other relay (re)selection criteria other than SD-RSRP or SL-RSRP are needed for the multi-hop relay, please write them down along with the reason.

| Company | Describe other criteria, if it exists | Reason |
| --- | --- | --- |
| Fraunhofer | Path quality, e.g. based on RSRP values of the hops | Selection of the first relay and intermediate relay(s) until the last relay are currently independent steps. The overall path should be considered when remote UE chooses the relay. |

Rapporteur thinks that the suggestion seems not to have clear supporters. So, we can have further discussions based on the contribution.

## 2.5 Whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last Relay UE via one (physically same) intermediate Relay UE

According to the Rel-19 WID, it’s clear that one Remote UE has one indirect path. However, it’s not clear whether the intermediate Relay UE can have one indirect path or not. Considering this issue, we can consider which topologies can be excluded or included.

In the contributions by Qualcomm (R2-2407102), several cases of the multi-hop topologies, which we have to treat, are displayed. Based on Qualcomm’s contribution, Rapporteur will display further topologies cases that we should include and exclude. All the remote UEs have only one indirect path in the provided topologies. In the figures, the red or green color means the link to support multi-hop U2N relay from Remote UE and gNB. The big blue circle means physically one Relay/Remote UE.

For each question, you can answer in terms of Rel-19 U2N relay should handle these topologies.

- Case-A

  - The red link from gNB to Remote UE(1) is for supporting multi-hp U2N relay operation.

  - The green link from gNB to Remote UE(2) is for single-hop U2N relay operation.

  - In Figure 2.5A-1, the big blue circle means the L2 ID(D) and L2 ID(D’) belong to the physically same last relay UE. And L2 ID(C) and L2 ID(C’) belong to the physically same intermediate Relay UE. However, the physically one intermediate Relay UE acts as an intermediate Relay for the Remote UE(1) and also acts as Remote UE(2) by itself. The intermediate Relay UE has one C-RNTI value because the UE is physically one UE regardless of acting as intermediate Relay UE and Remote UE simultaneously. In this case, the intermediate Relay UE has two different indirect paths via physically one last Relay UE by using different PC5 connections.

  - In Figure 2.5A-2, the big blue circle at the last Relay UE means the L2 ID(D) and L2 ID(D’) belong to the physically one UE. However, L2 ID(D) and L2 ID(D’) has connection with each physically different intermediate Relay UE and Remote UE(2). In this case, it doesn’t look like there is any problem with just the figure.

  - In Figure 2.5A-3, the big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) belong to physically the same UE. The intermediate Relay UE acts as an intermediate Relay UE for supporting Remote UE(1) and acts as Remote UE(2) by itself simultaneously. The intermediate Relay UE has two indirect connections with gNB via physically different last Relay UE(s) (i.e., via last Relay UE(1) and last Relay UE(2)). Each last Relay UE(1) and last Relay UE (2) is connected to the same gNB/cell.

(Figure 2.5A-1)

(Figure 2.5A-2)

(Figure 2.5A-3)

### [Question 2.5A-1] In Figure 2.5A-1 and 2.5A-2, do you think the last Relay UE of a multi-hop relay operation can also act as last Relay UE of a single-hop relay operation simultaneously?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes with comments | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | Yes |  |
| Apple | Yes | A relay UE cannot be limited to only support a single scenario. |
| CATT | Yes | There is no issue forseen that provent the last Relay UE of a multi-hop relay operation can also act as last Relay UE of a single-hop relay operation simultaneously. |
| ZTE | Yes | We agree that the last Relay UE of a multi-hop relay operation can also act as last relay UE of a single-hop relay operation simultaneously at least that the relay UE uses the same L2 ID. But it does not mean the case in Figure 2.5A-1 is supported. |
| Huawei, HiSilicon | Yes | Last relay UE can provide relaying service for multiple remote UE, which is similar to R17 |
| TCL | Yes |  |
| Fraunhofer | Yes | We don’t see a technical reason for not suppurting this case. |
| Sharp | yes |  |
| Spreadtrum | Yes |  |
| Lenovo | Yes |  |
| Kyocera | Yes with comments | Although this can be supported we should get clarification from SA2 whether it is intended to be supported in Rel-19. |
| ASUSTeK | Yes |  |
| Samsung | Yes | We see no difference from R17 relay if only one hop is established. |
| vivo | Yes |  |
| Interdigital | Yes |  |
| LG | Yes |  |
| AT&T | Yes with comments | Agree with Kyocera. This seems like a reasonable scenario, but it would be prudent to check this use case with SA2. |
| Qualcomm | Yes |  |
| Xiaomi | Yes |  |

### Summary:

19 companies provided inputs:

- Yes: 19 companies

- No: 0 company

Proposal 9: The last Relay UE of a multi-hop relay operation can also act as last Relay UE of a single-hop relay operation simultaneously.

### [Question 2.5A-2] In Figure 2.5A-1, do you think the intermediate Relay UE of multi-hop relay operation can act as a Remote UE of single-hop relay operation simultaneously?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes with comments | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No if different L2 ID are used for the two roles as shown in the figure 2.5A-1 But Yes if the same ID are used for the two roles (e.g., using the red line to play the two roles simultaneously) | For L2 U2N Relay, different L2 ID means different links, allowing intermediate Relay UE of multi-hop relay operation acting as a Remote UE of single-hop relay operation simultaneously means we support multi-path with 2 indirect legs, which is out of both R18 and R19 scope. |
| Apple | Yes | There is no real technical obstacles for this. The UE can be a remote UE and a relay UE at the same time |
| CATT | Yes | There is no issue forseen that provent the intermediate Relay UE of multi-hop relay operation can act as a Remote UE of single-hop relay operation simultaneously. |
| ZTE | See comments | When we discuss the two roles of a physical UE, it should be assume the same L2 ID is used for the two roles. Otherwise, different L2 IDs means different PC5 links, as OPPO commented, multi-path with 2 indirect paths is not in the WI scope. |
| Huawei, HiSilicon | Yes | The intermediate relay UE should as a remote UE. This can reduce the complexity of the multi-hop relay, since all the remote UE and intermediate relay UE connects to the cell of the last relay UE |
| TCL | Yes |  |
| Fraunhofer | Yes |  |
| Sharp | yes |  |
| Spreadtrum | Yes |  |
| Lenovo | Yes with comments | The single PC5 connection is used for intermedidate relay UE which is also acted as remote UE. |
| Kyocera | Yes | From the Rel-18 U2U relay perspective it is already possible for a relay UE to serve as a relay UE for two source remote UEs connecting to the same destination remote UE using multiple PC5 links towards the destination remote UE. The same principle for multiple relay should be applicable. |
| ASUSTeK | Yes with comments | We share the same view as OPPO in terms of multi-path with 2 indirect legs concern. Besides, according to Note 5 in subclause 6.4.3.6 of TS23.304, a single PC5 unicast link is established between a 5G ProSe Layer-2 UE-to-Network Relay and a 5G ProSe Layer-2 Remote UE, as specified in TS 38.300, for supporting PDU sessions of the 5G ProSe Layer-2 Remote UE. Furthermore, the 5G ProSe Layer-2 U2N Relay provides forwarding functionality that can relay any type of traffic over the PC5 link as specified in section 5.4.2 of TS 23.304. The above statements in TS23.304 imply one single PC5 unicast link between the Intermediate Relay UE and the Last Relay UE can support multiple PDU sessions of different Remote UEs. Therefore, if there exists a PC5 unicast link between the Intermediate Relay UE and the Last Relay UE, the Intermediate Relay UE can use the same PC5 unicast link to support a new Remote UE instead of establishing a new PC5 unicast link with the Last Relay UE. |
| Samsung | Yes | To be an intermediate node, such relay UE should be remote UE first. |
| vivo | Yes | We don’t really understand why different L2 IDs can lead to multi-path with 2 indirect paths, as the intermediate relay UE is logically two UEs based on different L2 IDs and the links also runs independently. Maybe SA2 can confirm this. |
| Interdigital | Yes |  |
| LG | Yes |  |
| AT&T | Yes | Yes. Public safety use cases envision this as a significant use case. |
| Qualcomm | Yes | The two path should be independent. |
| Xiaomi | Yes | We understand different IDs would be used for the two purposes. |

### Summary:

16 companies provided inputs:

- Yes: 16 companies

- No: 0 company

All companies seem to agree that the intermediate Relay UE of multi-hop relay operation can act as a Remote UE of single-hop relay operation simultaneously as long as having single PC5 connection.

Proposal 10: An intermediate Relay UE in multi-hop relay operation can act as a Remote UE in single-hop relay operation simultaneously as long as it has a single PC5 connection.

### [Question 2.5A-3] As Figure 2.5A-1, do you think one physical UE, which acts as both intermediate Relay UE and Remote UE, can have two indirect connections with different PC5 connections via physically the same last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The multi-path with 2-indirect path is out of WI scope. |
| Apple | Yes |  |
| CATT | No | It is a little bit confused that why two indirect connections with different PC5 connections via physically the same last Relay UE are needed? Since multi-path relay is not in the scope of Rel-19 MH Relay. |
| ZTE | No | Out of the WI scope. |
| Huawei, HiSilicon | No | In figure 2.5 A-1, there should be only one connection between intermediate relay UE and the gNB, and the remote UE can share the link since the intermediate relay UE provideds the relaying service |
| TCL | No | Agree with oppo |
| Fraunhofer | Yes | This will need some discussion to understand the impact. In general this could be supported. |
| Sharp | No |  |
| Spreadtrum | Yes |  |
| Lenovo | No | Agree with Oppo. |
| Kyocera | maybe | It depends if the physical UE’s operation as an Intermediate relay UE and a remote UE can be operated independently. |
| ASUSTeK | No | Agree with OPPO. |
| Samsung | No |  |
| vivo | Yes | Similar as comment in last question maybe this can be confirmed by SA2 |
| Interdigital | Yes | We haven’t seen any technical problem to support the case. |
| LG | No |  |
| AT&T | No | We’re open to discussion, but it isn’t clear that this objective is within the work item scope. |
| Qualcomm | Yes | For different RSC, different PC5 connections are needed. Currently, we don’t assume the intermediate Relay UE has RRC connection with the gNB on the multi-hop relay path, then it is not multi-path relay. |
| Xiaomi | Yes | We understand the situation is same as legacy that one UE may support multiple services and has multiple IDs. In this sense, we don’t need to specify anything to address this scenario. It doesn’t matter whether it’s physically one UE. We only specify the UE behavior from procedural point of view. |

### Summary:

19 companies provided inputs:

- Yes: 7 companies (Ericsson, Apple, Fraunhofer, Spreadtrum , vivo, Interdigital, Qualcomm)

- No: 11 companies (OPPO, CATT, ZTE, HW, TCL, Sharp, Lenovo, ASUSTeK, Samsung, LG, AT&T)

- Yes with a restriction: 1 company (Kyocera)

Rapporteur has a sympathy of the following OPPO’s opinion of the answer on the question [Question 2.5A-6]. It’s not a single hop relay operation, however, similar principle can be applied to the intermediate Realy UE.

“[OPPO] Similar issue has been discussed in R17 single-hop U2N Relay. In single-hop relay, for L2 U2N Remote UE, to prevent 2-indirect path connection, each remote UE can only have one relay UE, and according to state-2 specification, a single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE.

Therefore, we can make the similar restriction in MH case, i.e., each UE can only have a single parent node, and a single unicast link is established between the UE and its parent UE in L2 MH U2N Relay case.”

Rapporteur believes that the gNB cannot differentiate whether the target L2 ID(s) reported by the last Relay UE belongs the physically same intermediate Relay UE. Also, according to the companies’ response about the [Question 2.5A-5] and [Question 2.5B-2], it seems that companies do not support when the intermediate Relay UE has multiple indirect paths. So, even though the answer of this question doesn’t show majority view, rapporteur would like to suggest the following proposals:

Proposal 11a: The following topology is not considered.

- One physical UE, which acts as both an intermediate Relay UE and a Remote UE, can have two indirect connections with different PC5 connections via physically the same last Relay UE.

(Figure 2.5A-1)

Proposal 11b: When an intermediate Relay UE acts as a Remote UE simultaneously, it performs both roles of intermediate Relay UE and Remote UE via only a single unicast link between the last Relay UE and the intermediate Relay UE.

### [Question 2.5A-4] As Figure 2.5A-2, do you think physically one last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE)?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes | In the same cell, there may be both R17 remote UE and R19 remote UE, connecting to the same last relay UE. We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | Yes |  |
| Apple | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Huawei, HiSilicon | Yes | Last relay UE can provide the relaying service to multiple remote UE which can be connected via U2N relay or multi-hop relays |
| TCL | YES |  |
| Fraunhofer | Yes |  |
| Sharp | yes |  |
| Spreadtrum | Yes |  |
| Lenovo | Yes |  |
| Kyocera | Yes |  |
| ASUSTeK | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| Interdigital | Yes |  |
| LG | Yes |  |
| AT&T | Yes |  |
| Qualcomm | Yes |  |
| Xiaomi | Yes |  |

### Summary:

19 companies provided inputs:

- Yes: 19 companies

- No: 0 company

Proposal 12: The following topology is considered

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE)

(Figure 2.5A-2)

### [Question 2.5A-5] As Figure 2.5A-3, do you think one intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | comments | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The multi-path with 2-indirect path is out of WI scope. |
| Apple | Yes | The two last relay UE may only support one different RSC code for each of them, so this is a valid case. |
| CATT | No | We think it belongs to multi-path and multi-hop combination which is not in the scope of RAN2 WID. |
| ZTE | No | Out of the WI scope. |
| Huawei, HiSilicon | No | Out of the WI scope. |
| TCL | NO | As our last comment, out of scope of WI scope |
| Fraunhofer | Yes | From the use case point of view (public safety communication) this is beneficial. |
| Sharp | No |  |
| Spreadtrum | No |  |
| Lenovo | No | See above comments. |
| Kyocera | maybe | It depends if the physical UE’s operation as an Intermediate relay UE and a remote UE can be operated independently. |
| ASUSTeK | No | Agree with OPPO. |
| Samsung | No |  |
| vivo | No |  |
| Interdigital | No | Multipath is out of WI scope. |
| LG | No |  |
| AT&T | No | Again, we believe this isn’t clearly within the work item scope. |
| Qualcomm | Possible | Different last relay UE support different RSCs, For different RSC, different PC5 connections are needed. Currently, we don’t assume the intermediate Relay UE has RRC connection with the gNB on the multi-hop relay path, then it is not multi-path relay. If the intermediate Relay UE is not required to have RRC connection on multi-hop relay path, then this scenario can be supported for free, i.e. the two paths can be independent. If we want to exclude the scenario, a solution is also needed to determine this scenario. We would like to revisit whether the scenario can be supported for free after determining control plane solution. |
| Xiaomi | Yes | The two indirect connections are independent. |

### Summary:

19 companies provided inputs:

- Yes: 3 companies

- No: 14 company

- Open for further discussion: 2 companies

Proposal 13: The following topology is not considered

- One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE

(Figure 2.5A-3)

### [Question 2.5A-6] If your answer to the [Question 2.5A-3] or [Quest 2.5A-5] was 'NO', do you have any ideas on how to prevent these topologies? Because the AS layer can only see L2 ID(s), the AS layer cannot differentiate whether the different L2 ID(s) belong to the same physical UE. It seems like it should be controlled from the upper layer, but if you have any idea, you can describe it freely.

| Company | Describe how to prevent |
| --- | --- |
| OPPO | Similar issue has been discussed in R17 single-hop U2N Relay. In single-hop relay, for L2 U2N Remote UE, to prevent 2-indirect path connection, each remote UE can only have one relay UE, and according to state-2 specification, a single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. Therefore, we can make the similar restriction in MH case, i.e., each UE can only have a single parent node, and a single unicast link is established between the UE and its parent UE in L2 MH U2N Relay case. And also restriction, if needed, can be facilitated by upper layer (S2 spec). |
| CATT | We prefer to leave it to SA2. |
| ZTE | Agree with OPPO. |
| Huawei, HiSilicon | We can adopt approach 1 of the email discussion as the baseline and assume that when intermediate relay UE connects to the last relay UE as a remote UE, meanwhile it has the capability to act as a relay, it can perform the discovery procedure to let the multi-hop remote UE find it. This can guarantee the intermediate is always connected to one last relay UE, and meanwhile can act as the intermediate relay UE for multiple remote UE. |
| TCL | Each remote UE can only have one first relay UE. Also the intermediate relay UE as a remote UE can only has one last relay UE. |
| ASUSTeK | Agree with OPPO. |

There is no rapporteur’s proposal.

### [Question 2.5A-7] If your answer was ‘yes’ for one of the questions 2.5A-3, 2.5A-4, or 2.5A-5, do you have any preference on which topology should be handled priori than others for Rel-19 multi-hop? If you have any preference, you can describe it with reason.

| Company | Preference with reason |
| --- | --- |
| Ericsson | 2.5A-3 2.5A-2 2.5A-1 (follows a decreasing priority order) |
| Apple | All. No differentiation in priority. |
| Fraunhofer | 2.5A-3, 2.5A-1, 2.5A-2 |
| Kyocera | Topology 2.5A-1 should be prioritized over topologies 2.5A-2 or 2.5A-3. |

There is no rapporteur’s proposal.

- Case-B

  - The red link from gNB to Remote UE(A) is for multi-hop U2N relay operation.

  - The green link from gNB to Remote UE(B) is for multi-hop U2N relay operation.

  - In Figure 2.5B-1, the big blue circle at the intermediate Relay UE means the L2 ID(B) and L2 ID(B’) belong to the physically one intermediate relay UE. Each physically different Remote UE has an indirect path via the same intermediate Relay UE and the same last Relay UE. It looks link no problem.

  - In Figure 2.5B-2, the big blue circle at the last Relay UE means the L2 ID(D) and L2 ID(D’) belong to the physically one last Relay UE. The big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically one intermediate Relay UE. Each physically different Remote UE(A) and Remote UE(B) has an indirect path via the same intermediate Relay UE and last Relay UE. However, comparing Figure 2.5B-1, the intermediate Relay UE and last Relay UE have connections via different PC5 connections to support each Remote UE.

  - In Figure 2.5B-3, the big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically one UE. The intermediate Relay UE has two different PC5 connections with physically different last Relay UE(A) and last Relay UE(B). Each last Relay UE(A) and last Relay UE (B) is connected to the same gNB/cell. The intermediate Relay UE has one C-RNTI when it is CONNECTED because the intermediate Relay UE is physically one UE. It looks like one intermediate Relay UE has two different indirect paths via physically different last Relay UE(s).

(Figure 2.5B-1)

(Figure 2.5B-2)

(Figure 2.5B-3)

### [Question 2.5B-1] As Figure 2.5B-1, do you think two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE? If your answer is ‘no’, please describe the reason.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | Yes | We are open for discussion. |
| OPPO | Yes | Compared to Figure 2.5B-2, we assume the difference here is the intermediate Relay UE and last Relay UE have a single PC5 connections to support each physically different Remote UE |
| Apple | Yes | This is a valid case. |
| CATT | Yes | From our point of view, this is no additional design complexity to support this case. |
| ZTE | Yes | It is reasonable that one PC5 link between the intermediate relay and the last relay can serve different remote UEs. |
| Huawei,HiSilicon | Yes | Like a tree structure. |
| TCL | No | Not sure about the benefit of this scenario |
| Fraunhofer | Yes | Valid case, could depend on QoS and Application |
| Sharp | yes |  |
| Spreadtrum | No |  |
| Lenovo | Yes |  |
| Kyocera | Yes | We assume the PC5 connection means it will be mapped to one PC5-RRC connection after the PC5 connection is established. |
| ASUSTeK | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| Interdigital | Yes | This is a valid case. |
| LG | Yes |  |
| AT&T | Yes |  |
| Xiaomi | Yes |  |

### Summary:

18 companies provided inputs:

- Yes: 17 companies

- No: 3 companies (Ericsson, TCL, Spreadtrum)

Proposal 14: The following topology is considered.

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

(Figure 2.5B-1)

### [Question 2.5B-2] As Figure 2.5B-2, do you think two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically same last Relay UE? Here, the intermediate Relay UE and last Relay UE have two different PC5 connections to support each physically different Remote UE.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson |  | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The connection between intermediate relay UE and the last relay UE has to be the same link, otherwise multi-path with 2-indirect path cannot be avoided, which is out of WI scope. |
| Apple | Yes |  |
| CATT | No | It is a little bit confused that why two indirect connections with different PC5 connections via physically the same last Relay UE are needed? |
| ZTE | No | Multi-path with 2 indirect path is out of the WI scope. |
| Huawei,HiSilicon | No | Agree with OPPO |
| TCL | No | Not sure about the benefit of this scenario |
| Fraunhofer |  | Not sure about the benefits, but would be open to discuss. |
| Sharp | No |  |
| Spreadtrum | Yes |  |
| Lenovo | No | Single PC5 connection can be used for different remote UEs. |
| Kyocera | No (maybe) | We assume this topology means there will be two different PC5-RRC connections between the last relay UE and the intermediate relay UE once the links are established. |
| ASUSTeK | No | Agree with OPPO. |
| Samsung | No |  |
| vivo |  | Not sure about this 2-indirect path as the L2 IDs for intermediate relay UE are different. We can further discuss about this. |
| Interdigital | No | Agree with OPPO |
| LG | No |  |
| Qualcomm | Possible | For different RSC, different PC5 connections are needed. Currently, we don’t assume the intermediate Relay UE has RRC connection with the gNB on the multi-hop relay path, then it is not multi-path relay. If the intermediate Relay UE is not required to have RRC connection on multi-hop relay path, then this scenario can be supported for free, i.e. the two paths can be independent. If we want to exclude the scenario, a solution is also needed to determine this scenario. We would like to revisit whether the scenario can be supported for free after determining control plane solution. |
| Xiaomi | Yes |  |

### Summary:

18 companies provided inputs:

- Yes: 3 companies (Apple, Spreadtrum, Qualcomm)

- No: 12 companies

- Open for discussion: 3 (Ericsson, Fraunhofer, vivo)

As the same reasons described for the proposal 11a/b, rapporteur would like to suggest the following proposals:

Proposal 15a: The following topology scenario is not considered..

- Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically same last Relay UE. Here, the intermediate Relay UE and last Relay UE have two different PC5 connections to support each physically different Remote UE.

(Figure 2.5B-2)

Proposal 15b: When an intermediate Relay UE supports multiple Remote UEs, a single unicast line is established between last Relay UE and the intermediate Relay UE.

### [Question 2.5B-3] As Figure 5B-3, do you think two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s)? i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s).

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson |  | We are open for discussion to identify the potential design efforts to support this case. |
| OPPO | No | The connection between intermediate relay UE and the last relay UE has to be the same link, otherwise multi-path with 2-indirect path cannot be avoided, which is out of WI scope. |
| Apple | Yes | This is a valid case, the intermediate relay UE be chose to use different last relay UE for different remote UE. |
| CATT | No | We think it belongs to multi-path and multi-hop combination which is not in the scope of RAN2 WID. |
| ZTE | No | Out of the WI scope. |
| Huawei,HiSilicon | No | Agree with OPPO |
| TCL | No | Not sure about the benefit of this scenario |
| Sharp | No |  |
| Spreadtrum | No |  |
| Lenovo | No | Out of scope. |
| Kyocera |  | We assume the intermediate relay UE could potentially forward discovery messages from different last relay UE supporting different RSC values. Whether this can be supported should be confirmed by SA2. |
| ASUSTeK | No | Agree with OPPO. |
| Samsung | No |  |
| Vivo | No |  |
| Interdigital | No |  |
| LG | No |  |
| AT&T | No |  |
| Qualcomm | Possible | Different last relay UE support different RSCs, For different RSC, different PC5 connections are needed. Currently, we don’t assume the intermediate Relay UE has RRC connection with the gNB on the multi-hop relay path, then it is not multi-path relay. If the intermediate Relay UE is not required to have RRC connection on multi-hop relay path, then this scenario can be supported for free, i.e. the two paths can be independent. If we want to exclude the scenario, a solution is also needed to determine this scenario. We would like to revisit whether the scenario can be supported for free after determining control plane solution. |
| Xiaomi | Yes |  |

### Summary:

19 companies provided inputs:

- Yes: 2 companies (Apple, QC)

- No: 14 companies

- Open for discussion: 2 (Ericsson, Kyocera)

Proposal 16: The following topology scenario is not considered.

- Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s). i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s).

(Figure 2.5B-3)

### [Question 2.5B-4] If your answer of the [Question 5B-2] or [Quest 5B-3] was “NO”, do you have any idea to prevent happening these topologies? The answer to this question may be the same as the answer to [Question 5A-6]. However, if you want to describe further for the solution to prevent these topologies, you can write down.

| Company | Describe how to prevent |
| --- | --- |
| OPPO | like replied in Q2.5A-7 |
| CATT | We prefer to leave it to SA2. |
| Huawei,HiSilicon | See our comments for Q2.5A-6 |
| ASUSTeK | Agree with OPPO. |

There is no rapporteur’s proposal.

### [Question 2.5B-5] If your answer was ‘yes’ for one of the questions 2.5B-1, 2.5B-3, or 2.5B-3, do you have any preference on which topology should be handled priori than others for Rel-19 multi-hop? If you have any preference, you can describe it with reason.

| Company | Preference with reason |
| --- | --- |
| Ericsson | 2.5B-3 2.5B-2 (following decreasing priority order) |
| Apple | All. No preference of priority. |
| Fraunhofer | No preference |
| Kyocera | Topology 2.5B-1 or 2.5B-2 (if applicable) should be prioritized |

There is no rapporteur’s proposal.

- Case-C

  - The red and green link from gNB to Remote U are for multi-hop U2N relay operation.

  - In Figure 2.5C-1, the big blue circle at the last Relay UE means the L2 ID(D) and L2 ID(D’) belong to the physically same last relay UE. The big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically same intermediate Relay UE. And, the big blue circle at the Remote UE means the L2 ID(A) and L2 ID(A’) belong to the physically same Remote UE. Each indirect link belongs physically same last Relay UE, intermediate Relay UE, and Remote UE. However, each indirect link has its own PC5 connection (i.e., has its own L2 ID pair) to support the multi-hop relay path. Each last Relay UE, intermediate Relay UE, and Remote UE has each only one C-RNTI when they are connected.

  - In Figure 2.5C-2, each last Relay UE(A) and last Relay UE(B) is physically different last Relay UE. The big blue circle at the Remote UE means the L2 ID(A) and L2 ID(A’) belong to the physically same Remote UE. The big blue circle at the intermediate Relay UE means the L2 ID(C) and L2 ID(C’) (/L2 ID(B) and L2 ID(B’)) belong to the physically same intermediate Relay UE. Each indirect link belonging to the physically same Remote UE has connections with gNB via physically different two last Relay UE(s) and the physically same intermediate Relay UE. Each last Relay UE(A) and last Relay UE (B) is connected to the same gNB/cell. Each indirect path has its own PC5 link(s) and Uu links to support the multi-path.

(Figure 2.5C-1)

(Figure 2.5C-2)

### [Question 2.5C-1] As Figure 2.5C-1, do you think two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically same last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | No | This seems to be a multi-path scenario for remote UE, which is not in the scope |
| OPPO | No (if the L2 ID is different) | This is excluded by the conclusion for single hop U2N Relay, i.e., a single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. |
| Apple | No |  |
| CATT | No | We fail to see the benefit to support this scenario. |
| ZTE | No | As single hop L2 U2N relay, the remote UE can only have one PC5 unicast link with the parent relay. The following cases are just different combinations of before cases, do not need to discuss repeatedly. |
| Huawei, HiSilicon | No | A single path is enough. We dont see any benefits of having 2 paths |
| TCL | No | Not sure about the benefit of this scenario |
| Fraunhofer |  | No strong view. |
| Sharp | No |  |
| Spreadtrum | No |  |
| Lenovo | No | For a single remote UE, only one single indirect path is allowed. |
| Kyocera | No |  |
| ASUSTeK | No | Agree with OPPO. |
| Samsung | No |  |
| vivo | No |  |
| Interdigital | No |  |
| LG | No |  |
| AT&T | No |  |
| Xiaomi | NO | Multipath is not in the scope. |

### Summary:

18 companies provided inputs:

- Yes: 0 company

- No: 18 companies

Proposal 17: The following topology scenario is not considered.

- Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically same last Relay UE.

(Figure 2.5C-1)

### [Question 2.5C-2] As Figure 2.5C-2, do you think two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE?

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson | No | This seems to be a multi-path scenario for remote UE, which is not in the scope |
| OPPO | No (if the L2 ID is different) | This is excluded by the conclusion for single hop U2N Relay, i.e., a single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. |
| Apple | No |  |
| CATT | No | We think it belongs to multi-path and multi-hop combination which is not in the scope of RAN2 WID. |
| ZTE | No |  |
| Huawei,HiSilicon | No |  |
| TCL | No | Not sure about the benefit of this scenario |
| Fraunhofer | No | Unsure about the benefit. |
| Sharp | No |  |
| Spreadtrum | No |  |
| Lenovo | No |  |
| Kyocera | No |  |
| ASUSTeK | No | Agree with OPPO. |
| Samsung | No |  |
| vivo | No |  |
| Interdigital | No |  |
| LG | No |  |
| AT&T | No |  |
| Xiaomi | No | Multipath is not in scope |

### Summary:

16 companies provided inputs:

- Yes: 0 company

- No: 18 companies

Proposal 18: The following topology scenario is not considered.

- Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE.

(Figure 2.5C-2)

### [Question 2.5C-3] If your answer of the [Question 2.5C-1] or [Question 2.5C-2] was “NO”, do you have any idea to prevent happening these topologies? The answer to this question may be the same as the answer to [Question 2.5A-6] or [Question 2.5B-4]. However, if you want to describe further for the solution to prevent these topologies, you can write down.

| Company | Yes/No | Reason |
| --- | --- | --- |
| Ericsson |  | We can include restriction in stage 2 spec. |
| OPPO | like replied in Q2.5A-7 | like replied in Q2.5A-7 |
| Apple |  | We can ask SA2 for how to do this |
| CATT |  | We prefer to leave it to SA2. |
| ASUSTeK |  | Agree with OPPO. |

### [Question 2.5C-4] If your answer was ‘yes’ for one of the questions 2.5C-1, or 2.5B-2, do you have any preference on which topology should be handled priori than others for Rel-19 multi-hop? If you have any preference, you can describe it with reason.

| Company | Preference with reason |
| --- | --- |

- Case-D

  - Figure 2.5D-1, 2.5D-2 and 2.5D-3 are the similar to the Figure 2.5A-3, 2.5B-3 and 2.5C-2 except that each multi-hop indirect path have connect with different cells in the same gNB.

(Figure 2.5D-1)

(Figure 2.5D-2)

(Figure 2.5D-3)

### [Question 2.5D-1] For each figure, i.e., Figure 2.5D-1, 2.5D-2, 2.5D-3, do you think these three cases should be considered for Rel-19 multi-hop topologies?

| Company | For Figure 2.5D-1 (Yes/No) | For Figure 2.5D-2 (Yes/No) | For Figure 2.5D-3 (Yes/No) | Reason |
| --- | --- | --- | --- | --- |
| Ericsson | Maybe Yes | Maybe Yes | No | D-3 seems to be a multi-path scenario for remote UE, which should be avoided. |
| OPPO | No | No | No | Out of R19 WI scope. |
| Apple | Yes | Yes and Yes also for different gNB case | Maybe |  |
| CATT | No | No | No | Belong to optimization. |
| ZTE | No | No | No |  |
| Huawei, HiSilicon | NO | No | No |  |
| OPPO | No | No | No | Out of R19 WI scope. |
| Fraunhofer | Yes | Yes | Maybe |  |
| Sharp | No | No | No |  |
| Spreadtrum | No | No | No |  |
| Lenovo | No | No | NO |  |
| Kyocera | No | No | No |  |
| ASUSTeK | No | No | No |  |
| Samsung | No | No | No |  |
| vivo | No | No | No |  |
| Interdigital | Yes | Yes | Yes |  |
| LG | No | No | No |  |
| AT&T | No | No | No |  |
| Xiaomi | No | No | No | This scenario would result in intermediate relay UE connects to two cells simultaneously. This is not supported. |

### Summary:

18 companies provided inputs:

- For Figure 2.5D-1:

- Yes: 4 companies (Ericsson, Apple, Fraunhofer, IDT)

- No: 14 companies

- For Figure 2.5D-2:

- Yes: 4 companies (Ericsson, Apple, Fraunhofer, IDT)

- No: 14 companies

- For Figure 2.5D-3:

- Yes: 3 companies (IDT), maybe yes (Apple, Fraunhofer)

- No: 14 companies

Proposal 19: The following topology scenarios are not considered.

(2.5D-1) One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB.

(Figure 2.5D-1)

(2.5D-2) Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s) i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s). Each last Relay UE has a connection with different cells in the same gNB.

(Figure 2.5D-2)

(2.5D-3) Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB.

(Figure 2.5D-3)

## 2.6 Others

### [Question 2.6] If there is further issue we should handle in this email discussion but missing, you can describe it freely.

| Company | Other issues that should be handled |
| --- | --- |
| Apple | We need also cover the case that the last relay UES are served by different gNB for case 2.5D-2. |

# 3. Phase 2 Discussion

## 3.1 Proposals for discovery message initiating/forwarding condition at intermediate Relay UE

From the response of [Question 2.1-1], rapporteur generated the following proposals based on the companies’ response. Rapporteur wants to check the proposals can be aggregable.

### Proposal 1a: A lower bound is not required for the intermediate Relay UE.

### Proposal 1b: For the intermediate Relay UE, RAN2 needs to discuss whether upper bound is required in phase 2.

[Question 3.1-1] Do you think “Proposal 1a” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | agree |
| OPPO | Agree |
| Apple | Agree |
| Kyocera | Agree |
| Qualcomm | Agree |
| Xiaomi | Agree |
| Sharp | Agree |
| CATT | Agree |
| vivo | Agree |
| Huawei,HiSilicon | Agree |
| ZTE | Agree |
| Lenovo | Agree. |
| Spreadtrum | Agree |

For proposal 1b, it is clear that the intermediate Relay UE in OoC does not need both upper and lower bounds. However, if the intermediate Relay UE is in coverage, it is not clear whether the upper bound is necessary in this case. So, rapporteur thinks that it needs to be clarified further.

### [Question 3.1-2] Do you think the upper bound for the intermediate Relay UE is not required even if the intermediate Relay UE is in coverage?

| company | Yes/No | Reason of your answer |
| --- | --- | --- |
| Ericsson | No | Based on companies comments, for intermediate relay UE in IC, there are both scenarios/reasons where upper bound threshold maybe applicable and maybe not applicable. To simplify the design, it is sufficient to not introduce any upper threshold, rather, left to UE implementation to handle if there is a need to consider any threshold. |
| OPPO | No (upper bound is required) | In R17 single hop relay, the upper bound of Uu quality is defined for the remote UE to make sure cell centre UE (can access the network directly) doesn’t access via the relay. This principle should be followed in multi-hop case, i.e., cell centre UE should not access the network via other UE(s). If the cell centre UE acts as intermediate relay, the network coverage cannot be extended via muti-hop relay, and may even cannot reach the OOC remote UE(s), which means a waste of resource. |
| Apple | Yes (upper bound is not required) | It is not necessary to restrict the “upper bound” of intermediate relay UE. If a UE is close to the gNB, then the UE will choose to be a last relay UE instead. This can be left to UE implementation. |
| Kyocera | No | We think the upper bound is required. Intermediate relay UEs close to the center of cell will follow the requirements as a last relay UE. |
| Qualcomm | Yes (upper bound is not required) | When the UE in in-coverage acts as an intermediate Relay UE, that means the UE’s serving cell does not support multi-hop relay, and the UE cannot act as a last relay UE. Then there is no need to restrict intermediate Relay UE Uu coverage. |
| Xiaomi | Not required | Considering the intermediate UE may camp on different cell of last relay UE, we don’t see the need to use upper bound. |
| Sharp | No (upper bound is required) with comment. | In LTE U2N relay, upper bound for relay UE was introduced to avoid wasteful relay UE operation since remote UE’s position should be cell edge or OOC. So, in accordance with this principle, upper-bound is needed. However, if the intermediate relay UE always acts as a remote UE, the intermediate relay UE is already configured with upper bound. If so, the specific upper bound for an intermediate relay UE is not required. |
| CATT | Not required | For Rel-17 remote UE, the configured Uu threshold is not used to limit the remote UE not too close to its related serving cell. Instead, it is used as the triggering condition for relay discovery message. We fail to see the need to introduce this threshold for intermediate relay UE. |
| vivo | No (upper bound is required) | It is strange that we even agreed for an upper bound for last relay UE but this is not applied to intermediate relay UE. we understand the logic is the same that the intermediate relay UE should not be in the centre of the cell. |
| Huawei,HiSilicon | Not required | After further thoughts on this we think If the intermediate relay UE is connected with the last relay, the Uu RLM will be suspended, so the upper bound is not required. In other words the intermediate relay UE will first become a remote UE, stop monitor the Uu link and then take up the role of the intermediate UE hence the upper bound is not needed. |
| ZTE | Not required | Based on companies comments, the support of upper bound Uu threshold is to restrict a cell-center UE to act as an intermediate relay, that is, the intermediate relay shall not have a direct Uu link with a gNB supports MH relay. We agree with the intention but think a new upper bound for intermediate relay is not needed. Firstly, if the relay capable UE meets the conditions for the last relay, it shall be act as a last relay and can transmit discovery announcement/respond message. Then If not act as a last relay, when receiving discovery announcement/ solicitation message, it shall make sure it does/could not have a direct Uu link before acting as an intermediate relay to forward the received discovery messages; in this case, the upper bound for remote UE could be reused. That is, the conditions for last relay and remote UE are enough, any further thresholds for intermediate relay are not needed. |
| Lenovo | Is required | The upper bound can be used to restrict the cell-center UE to be intermediate relay UE. |
| Spreadtrum | Yes, not required | Not needed, intermediate relay UE does not need to initiate the discovery process and just forwards the received discovery message from last relay UE or remote UE. |

From the response of [Question 2.1-2], rapporteur generated the following proposal based on the companies’ response. Rapporteur wants to check the proposal can be aggregable.

### Proposal 2: The intermediate Relay UE can initiate the announcement message when receiving announcement message or, at least, when the intermediate Relay UE has the information for the announcement message provided by the last Relay UE. The detail can be discussed further.

[Question 3.1-3] Do you think “Proposal 2” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | Not sure RAN2 really needs to agree on anything. Is this in SA2 domain? AS layer can just do based on upper layer decision. Even in case of RLF, AS layer cannot generate/initiate discovery by AS layer itself. But whether AS layer can indicate failure to upper layer can be further discussed. |
| OPPO | We are wondering what is the “further detail discussion in RAN2”. |
| Apple | Left to SA2 to decide |
| Kyocera | In general, we agree with Proposal 2 which is aligned with SA2’s views. |
| Qualcomm | RAN2 does not need to discuss this, SA2 already captured. |
| Xiaomi | Agree |
| Sharp | Agree. Since it’s already discussed by SA2, we suggest to modify the Proposal 2 to; ### Proposal 2: RAN2 confirms that the intermediate Relay UE can initiate the announcement message when receiving announcement message or, at least, when the intermediate Relay UE has the information for the announcement message provided by the last Relay UE. The detail can be discussed in stage3, if needed. |
| CATT | Same concern as Ericsson. |
| vivo | We think this can be left to SA2. |
| Huawei,HiSilicon | In our opinion there are two cases that which needs to be considered : - The intermediate relay UE is connected to the parent relay - The intermediate relay UE is not connected to the parent relay We need to check with SA2 when the intermediate relay UE can send the announcement message. |
| ZTE | We also think this is in SA2 scope. The proposal is not needed for RAN2. |
| Lenovo | Agree. RAN2 can confirm this based on 23.700. |
| Spreadtrum | Depend on SA2 |

Based on the response of [Question 2.1-3], rapporteur generated the following proposal.

### Proposal 3: The discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP is above a configured threshold.

[Question 3.1-4] Do you think “Proposal 3” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | Agree with the proposal |
| OPPO | We want to clarify our reply that “channel quality between the intermediate relay and its parent relay needs to be evaluated but may not in form of discovery transmission condition.” doesn’t mean “YES” to the discovery forwarding condition configuration. We share the same observation with CATT that according to SA2 conclusion, intermediate relay should ensure the PC5 link with the last relay is standby, i.e., our understanding is if the channel quality is bad, the intermediate relay should reselect another last relay, but not based on the channel quality to decide whether to forward discovery message. |
| Apple | I think there is some ambiguity about the “forwarding” part in P3, which direction the forwarding operation is happening? and which hop the RSRP condition is measured? It is better to be more clear… |
| Kyocera | In addition to the configured thresholds, it could be further considered if the intermediate relay UE should be PC5 connected to the last relay UE. |
| Qualcomm | Prefer to change to: The relay UE info can be included in discovery announcement if the link quality between the intermediate Relay UE and the relay UE. |
| Xiaomi | Agree |
| Sharp | Agree with the intention. But it is better to clarify that the SD-RSRP or SL-RSRP is the SD-RSRP or SL-RSRP between the Intermediate Relay UE and the UE transmitted the discovery announcement message to the Intermediate Relay UE. |
| CATT | Thanks for echoing from OPPO on SA2 conclusion. We think what we discussed here is the condition for the discovery announcement message forwarding at intermediate Relay UE. Hence, it is not convinced that we need the current proposal according to SA2’s conclusion. With OPPO riased case, we are glad to further discuss. |
| vivo | Agree. This is reusing the R18 U2U principle. According to Apple’s comment, we could further discuss about the detailed wording. And we are ok to check with SA2 on this issue. |
| Huawei,HiSilicon | We don’t need see the motivation of having such forwarding conditions at the intermediate relay UE or to use the U2U discovery principle in general which is for U2U communication and not suited for U2N communication The intermediate relay should ensure the PC5 link with the last relay is already established and is good enough. Then there is no need for having any threshold conditions for forwarding the discovery announcement message. Additionally, forwarding the discovery announcement message from multiple last relay UEs to multiple remote UE s in the cell can lead to cross path connection where the two remote UEs may try to establish the connection through two different last relay UEs via the same intermediate relay UE which is obviously out of the scope of R19. |
| ZTE | Agree, but the wording could be updated to clarify which hop is measured. Based on the latest SA2 spec/progress, it is not clear to us whether the intermediate relay needs to have a PC5 link with the parent relay before forwarding the discovery announcement message. No matter it has the PC5 link or not, the intermediate relay should check the PC5 link quality towards the relay from which the announcement message is received before forwarding the announcement message (even if there is a PC5 link, the quality of the PC5 link may be changed, further check before forwarding is needed). |
| Lenovo | We need to clarify which node transits the discovery message forwarded by the intermediate relay UE. My understanding is remote UE’s discovery message. Regarding discovery message from the last relay UE, the intermediate relay UE has selected this last relay UE based on the selection criteria which has already considered PC5 quality. Therefore, we suggest the following modification. Proposal: The discovery announcement message from remote UE (or another child intermediate relay UE) can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP between remote UE (or another child intermediate relay UE) and the intermediate relay UE is above a configured threshold. |
| Spreadtrum | Agree |

Based on the response of [Question 2.1-5], rapporteur generated the following proposal.

### Proposal 4: The following model B discovery forwarding condition can be baseline for further discussion:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

- For Model B discovery, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. (modified by Ericson’s comment)

- For Model B, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE.

[Question 3.1-5] Do you think “Proposal 4” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | Agree with the proposal |
| OPPO | The first bullet is not needed since the quality between remote UE and intermediate relay is evaluated by the remote UE in the second bullet. |
| Apple | Agree |
| Kyocera | Agree |
| Qualcomm | For the first bullet, prefer to change to “only if the solicitation message receiving link is about a threshold”. Current wording does not reflect this. |
| Xiaomi | Agree |
| Sharp | Agree |
| CATT | Agree. Simiar as what we adopt in Rel-18 U2U relay. |
| vivo | Agree |
| Huawei, HiSilicon | Agree with Oppo that the first bullet is not needed. Furthermore Since SA2 has concluded (Conclusions on SA2 TR 23.700-03) that Remote UE either selects the Intermediate Relay (for Model A) or selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay (Model B), it would be appropriate to forward the SD-RSRP or SL-RSRP information in Relay Discovery Response message for Model B to the remote UE for it to select the path to reach the UE-to-Network Relay. |
| ZTE | Agree the first bullet. The second bullet is agreeable but it is not a discovery forwarding condition, but a condition for evaluating candidate relay UEs. Disagree with the third bullet. In multi-hop SL relay, since there are more hops along the path than R18 single hop U2U relay, it is safer for the intermediate relay to forward the discovery response message by checking the PC5 RSRP again. |
| Lenovo | Agree. |
| Spreadtrum | Agree |

## 3.2 Proposals for PC5 AS conditions for discovery at the last Relay UE

Based on the response of [Question 2.2-1], rapporteur generated the following proposals.

### Proposal 5a: For the discovery announcement message initiation (in the case of model A), the last Relay UE doesn’t need to check PC5 AS condition.

### Proposal 5b: In the case of discovery model B, the following two options can be further discussed

(option 1) For the discovery response message transmission (in the case of model B), the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first) intermediate Relay UE.

(option 2) As the legacy Rel-17 U2N relay, the U2N Relay UE doesn’t have any PC5 AS conditions for transmitting the announcement/response discovery message. The same principle can be applied to the last Relay UE.

[Question 3.2-1] Do you think “Proposal 5” is agreeable? If you disagree or think something needs to be modified, please feel free to comment. Also, for proposal 5b, if you have additional options that you want, you can suggest.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | 5a is fine, we can follow majority view for 5b. |
| OPPO | Agree 5a, for 5b, it should be option-2, since the PC5 link between the last relay and the intermediate relay should be established/reliable, otherwise, the intermediate relay would trigger relay reselection to another last relay. |
| Apple | We agree |
| Kyocera | We are fine with Proposal 5, and we prefer Option1 in Proposal 5b. |
| Qualcomm | Agree with 5a, for 5b, prefer option 1. Since in proposal 4, no AS criterion is needed for the intermediate Relay UE(s) to forward the response message to the Remote UE. |
| Xiaomi | Agree |
| Sharp | Agree with 5a, for 5b, prefer option 1 since there is no evaluation of PC5 link quality between the last relay UE and the intermediate relay UE. |
| CATT | Fine with the current proposal and agree to further discuss 5b with the listed two options. |
| vivo | Agee with 5a. For 5b, our initial thinking is that option-1 is simpler and is also aligned with R18 U2U principle, otherwise the link quality between last relay UE and Intermediate Relay UE is not considered. |
| Huawei, HiSilicon | Agree with Proposal 5a For Proposal 5b, agree that it should be option-2 as the PC5 link between the last relay and the intermediate relay should be already existing and should be good |
| ZTE | 5a is not needed. It is confusion what’s the PC5 AS condition for the last Relay UE in Model A discovery, if there is no such valid PC5 AS condition, we do not need to discuss this and the proposal is not needed. For 5b, the PC5 link quality between the last relay and the intermediate relay the solicitation message is received need be checked before response. For the wording, better to clarify what’s the PC5 AS condition and which hop is measured. |
| Lenovo | Fine with current proposal. |
| Spreadtrum | Agree |

## 3.3 Proposals for (Re)selection triggers for all UEs

Based on the response to [Question 2.3-1], rapporteur generated the following proposals. Some companies want to differentiate the triggering condition between relay selection and relay reselection. However, in the rapporteur’s understanding, the current RRC spec (TS 38.331) does not differentiate between them. So, this version does not differentiate the triggering condition for relay selection and reselection in the current state. For the FFS portion, we will discuss in the from [Question 3.3-2] to [Question 3.3-6].

### Proposal 6: The following relay (re)selection triggering conditions for Remote UE are supported for the multi-hop relay operation at least when there is only one intermediate Relay UE.

FFS: further relay (re)selection triggering conditions can be discussed at phase 2.

[Relay (re)selection triggering conditions for Remote UE] - If the Remote UE has no serving cell; - If the Remote UE does not have a selected intermediate Relay UE; - If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected intermediate Relay UE; - If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE; - If the Remote UE receives a notification message from the intermediate Relay UE caused by one of the following: h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE) h-2) if intermediate Relay UE receives RRCReconfiguration message for HO (if the intermediate Relay UE is in RRC_CONNECTED) h-3) if intermediate Relay UE performs cell reselection h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE h-5) if intermediate Relay UE receives PC5-RRC connection release between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

[Question 3.3-1] Do you think “Proposal 6” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | We don’t agree with h-2), h-3) and h-4), we think all three condition, would just introduce interruption to the remote UE, such conditions would be covered/reflected eventually by PC5 RLF. No need to specify each single exceptional condition due to intermediate relay UE. Rather discussions for RAN2 shall focus on remote UE and last relay UE, to simplify design complexity. |
| OPPO | One general suggestion is to discuss relay selection and reselection condition separately to avoid confusion. For “If the Remote UE has no serving cell;”, we should follow the same wording as in R17 U2N single hop relay, i.e., “Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold”; For “If the Remote UE does not have a selected intermediate Relay UE;”, we understand this is not a relay (re)selection trigger condition, but the difference between relay selection and reselection. |
| Apple | We do not agree n), becsue this is wrongly described as a relay UE behaviour, the correct trigger is upper layer of remote UE receives PC5-S message indicating the release of PC5 link. We do not agree h2), h3), h4), either, as similar view to Ericsson. |
| Kyocera | We agree these can be adopted as baseline. |
| Qualcomm | Do not agree h1), h2), h3), h4), h5). It should be possible the intermediate Relay can perform relay reselection when detecting the link situation, and does not need to inform the Remote UE. Otherwise, any hop failure or release or HO or cell resection, the Remote UE will trigger discovery from starting, it signalling overhead and service interruption could be very large. |
| Xiaomi | Agree |
| Sharp | Suggest to; Change to k) If the SL-RSRP of the currently selected intermediate … Remove n). (same reason with Apple) |
| CATT | Agree |
| vivo | For n), we have sympathy with Apple’s comment that this should be descripted as the remote UE’s behaviour somehow. For h1,h2,h3,h4,h5, we have doubt because we think these could be considered by intermediated relay itself, thus no need to inform remote UE. |
| Huawei, HiSilicon | Yes However we would like to clarify Conditions a)/b) are for relay selection Conditions c) /d)/ e)/ f)/ g)/ h) for relay (reselection) |
| ZTE | Same comments as in Q2.3-1 to separate relay selection and re-selection. “at least when there is only one intermediate relay UE” can be removed. |
| Lenovo | Agree with current proposal. but don’t understand why PC5-S release message from legacy R-17 U2N relay operation is not discussed in the following questions. |
| Spreadtrum | Agree |

In the [Question 2.3-1], companies suggested some other relay (re)selection triggering conditions for Remote UE. Rapporteur believes that we can further discuss the triggering conditions for the relay (re)selection of Remote UE in phase 2 based on the companies’ inputs.

### [Question 3.3-2] Do you think the following case can be the relay (re)selection triggering condition for the Remote UE?

- If direct Uu signal strength of current serving cell of the Remote UE is below a configured signal strength threshold

| Company | Yes/No | Reason for your answer |
| --- | --- | --- |
| Ericsson | Yes for relay selection, but not for reselection. |  |
| OPPO | Yes |  |
| Apple | Yes for selection only |  |
| Kyocera | Yes | We assume this only applies to selection. |
| Qualcomm | Yes |  |
| Xiaomi | Yes |  |
| Sharp | Yes | for selection |
| CATT | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes for relay selection only |  |
| ZTE | Yes | for relay selection |
| Lenovo | Yes for relay selection |  |
| Spreadtrum | Yes | for relay selection |

### [Question 3.3-3] Do you think the last Relay UE generates notification message when the Uu RSRP is decreased under a configured threshold?

| Company | Yes/No | Reason for your answer |
| --- | --- | --- |
| Ericsson | No | In R17 legacy, there is no such notification message for last U2N relay. In addition, last relay UE’s Uu radio channel quality is of course changing all the time, a temporary degradation of Uu quality shall be avoided to trigger remote UE relay reselection. For a long timer Uu quality degradation, would trigger Uu RLF eventually, which is already covered by other conditions. |
| OPPO | No |  |
| Apple | No |  |
| Kyocera | Yes | Without the notification message, the intermediate relay UE and subsequently the remote UE will not know the degradation of the Uu link. |
| Qualcomm | No |  |
| Xiaomi | No |  |
| Sharp | No |  |
| CATT | No |  |
| vivo | No | This will make the last relay not qualified anymore and can be covered by the threshold related discussions. Not needed. |
| Huawei, HiSilicon | No | We would like to stick with the legacy principles in R17 |
| ZTE | No | For the Uu link status of the last relay UE, just follow the principles in R17. |
| Lenovo | No |  |
| Spreadtrum | No |  |

### [Question 3.3-4] Do you think the intermediate Relay UE generates notification message when the PC5 RSRP is decreased under a configured threshold?

| Company | Yes/No | Reason for your answer |
| --- | --- | --- |
| Ericsson | No, | Same comments as in the above for the last U2N relay UE. |
| OPPO | No |  |
| Apple | No |  |
| Kyocera | Yes |  |
| Qualcomm | No | In this case, the intermediate Relay can reselect a relay UE |
| Xiaomi | No |  |
| Sharp | No |  |
| CATT | No |  |
| vivo | No |  |
| Huawei, HiSilicon | No |  |
| ZTE | Yes | For the intermediate relay UE, the PC5 link quality should be considered. |
| Lenovo | No |  |
| Spreadtrum | No |  |

### [Question 3.3-5] Do you think the intermediate Relay UE can generate notification message when the intermediate Relay UE receives PC5-S connection release between the intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)?

| Company | Yes/No | Reason for your answer |
| --- | --- | --- |
| Ericsson | Yes | The intermediate relay UE can directly forward the received message to the remote UE or generate by itself based on received message content. RAN2 can FFS these two options. |
| OPPO | Yes |  |
| Apple | Yes |  |
| Kyocera | Yes |  |
| Qualcomm | No | It should be possible that the intermediate relay UE to reselect a relay UE. |
| Xiaomi | Yes |  |
| Sharp | Yes | The intermediate relay UE should send notification message even if it performs relay reselection as a remote UE. |
| CATT | Yes |  |
| vivo | No with comments | Similar view as Qualcomm, we should first decide whether intermediated relay can do reselection itself. |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |

### [Question 3.3-6] Do you think the notification message generated by last Relay UE or (not first) intermediate Relay UE can be forwarded toward the Remote UE via the first intermediate Relay UE?

| Company | Yes/No | Reason for your answer |
| --- | --- | --- |
| Ericsson | Yes | How to forward such message to remote UE would be a common issue, i.e., also valid for system information forwarding and paging forwarding. RAN2 shall aim for an unified solution. |
| OPPO | See comments | We understand it is not “forwarded” notification message generated by the last relay, but the intermediate relay generates its own notification message if e.g., Uu RLF, cell change…at the intermediate relay happens which is triggered by the reception of notification message from the last relay. |
| Apple | Yes | Same view as Ericsson |
| Kyocera | Yes |  |
| Qualcomm | No | We would like to postpone this until it is clear what information should be forwarded. |
| Xiaomi | Yes |  |
| Sharp |  | Same view as OPPO |
| CATT | Yes |  |
| vivo | No |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes | Agree with the intention, but as comments above, it is not directly “forward”, but add the information received in its own PC5 notification message. |
| Lenovo | No | The first intermediate relay UE can generate its own notification message due to reception of notification message. |
| Spreadtrum | Yes |  |

Based on the response to [Question 2.3-2], rapporteur generated the following proposals.

### Proposal 7: Following relay (re)selection triggering conditions are not supported for intermediate Relay UE.

[Relay Triggering conditions for intermediate Relay UE] - If the intermediate Relay UE has no serving cell; - If the intermediate Relay UE does not have a selected last (/serving intermediate) Relay UE; - If the SL-RSRP of the currently selection last (/serving intermediate) Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE; - If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE; - If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE caused by one of the following: h-1) if last Relay UE detects Uu RLF h-2) if serving intermediate Relay UE detects PC5 RLF h-3) if last (/serving intermediate) Relay UE receives RRCReconfiguration message for HO h-4) if last (/serving intermediate) Relay UE performs cell reselection h-5) if last (/serving intermediate) Relay UE fails Uu RRC connection establishment/Resume h-6) if last (/serving intermediate) Relay UE receives Uu connection release between gNB and last (/serving intermediate) Relay UE h-7) if serving intermediate Relay UE receives PC5-RRC connection release between last (/the other serving intermediate) Relay UE and the serving intermediate Relay UE

[Question 3.3-7] Do you think “Proposal 7” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | Agree with the proposal 7. |
| OPPO | One general suggestion is to discuss relay selection and reselection condition separately to avoid confusion. For “If the Remote UE has no serving cell;”, we should follow the same wording as in R17 U2N single hop relay, i.e., “Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold”; For “If the Remote UE does not have a selected last (/serving intermediate) Relay UE;”, we understand this is not a relay (re)selection trigger condition, but the difference between relay selection and reselection. |
| Apple | Agree with P7 |
| Kyocera | Yes |
| Qualcomm | No. It should be possible to allow the intermediate Relay UE reselects a relay UE in case that the current PC5 link has problem. This will save signalling overhead and interruption time. And there is less standard efforts. Otherwise, the discovery will be re-started from the Remote UE if any hop has problem. This is very bad in multiple hops path. |
| Xiaomi | Agree |
| Sharp | Agree with P7 |
| CATT | Agree |
| vivo | No. At least for some conditions, e.g. k),i) to trigger reselection, it could be possible that the intermediate relay UE can do the reselection itself, especially considering that we need the extension to multi-hop case. If it is always the remote UE to reselect all relay UEs, it could be a waste of time and the performance may not be so good as some of the links are actually good enough to be used continually. |
| Huawei, HiSilicon | We think that at least, when the path via the itnermediate relay UE and the network is not available, the intermediate relay UE can trigger the relay (re)selection |
| ZTE | Agree with the intention but suggest to simplify the proposal. The majority view is that the intermediate relay triggering relay (re)selection is not supported, so no need to list all the details. |
| Spreadtrum | Agree |

## 3.4 Proposals for (Re)selection criteria for all UEs (considering discovery models A/B and whether criteria beyond the first hop are considered)

Based on the response to [Question 2.4-1], rapporteur generated the following proposals.

### Proposal 8: Rel-18 relay (re)selection criteria can be reused for relay (re)selection criteria for the Rel-19 multi-hop.

- SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.

- In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

- Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

[Question 3.4-1] Do you think “Proposal 8” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | Agree with the proposal |
| OPPO | Agree |
| Apple | Agree |
| Kyocera | We agree Proposal 8 can be considered as baseline. For the intermediate relay UE, if Proposal 7 is adopted, further discussion is needed as to how the intermediate relay UE can perform relay reselection. |
| Qualcomm | Agree |
| Xiaomi | Agree |
| Sharp | Agree |
| CATT | Agree |
| vivo | Agree |
| Huawei, HiSilicon | Agree |
| ZTE | Agree |
| Lenovo | Agree |
| Spreadtrum | Agree |

## 3.5 Proposals for whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last Relay UE via one (physically same) intermediate Relay UE

Based on the responses to the questions in this session, the rapporteur generated the following proposals.

### Proposal 9: The last Relay UE of a multi-hop relay operation can also act as last Relay UE of a single-hop relay operation simultaneously.

[Question 3.5-1] Do you think “Proposal 9” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | Agree with the proposal |
| OPPO | Agree |
| Apple | Agree |
| Kyocera | Yes |
| Qualcomm | Yes |
| Xiaomi | Agree |
| Sharp | Agree |
| CATT | Agree |
| vivo | Agree |
| Huawei, HiSilicon | Agree |
| ZTE | Agree, but for the wording, “last relay UE” is not used for single-hop relay. |
| Lenovo | Agree |
| Spreadtrum | Agree |

### Proposal 10: An intermediate Relay UE in multi-hop relay operation can act as a Remote UE in single-hop relay operation simultaneously as long as it has a single PC5 connection.

[Question 3.5-2] Do you think “Proposal 10” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | Don’t understand why “single PC5 connection” is a condition for this proposal. Don’t understand why two L2 IDs, would result into multiple indirect paths. Whether it is a multi-path scenarios, must for the same remote UE perspective. In this case, the intermediate relay UE needs to maintain two L2 IDs, one for relaying remote UE’s traffic purpose, one for its own traffic purpose, it is ok to support. Even for non relay scenario, a UE can have multiple destinations connected, wherein each destination is associated with different L2 IDs. |
| OPPO | Agree with the proposal with the “single PC5 connection” since otherwise it is a multi-path scenario for the intermediate relay UE. |
| Apple | We do not agree. The intermediate relay UE is not a remote UE, and we do not think there is a need to enforce the intermediate relay UE to also serve as a remote UE. If intermediate relay UE use a different Remote UE Src L2 ID, it can behave as a completely different UE, which is no need to be concerned or discussed because it will be seen as another UE to last relay UE. |
| Kyocera | We agree with Proposal 10, assuming the single PC5 connection refers the PC5-RRC connection since we assume from the SA2’s perspective the two different services associated with two L2IDs would have 2 PC5 unicast connections. |
| Qualcomm | Have same comments as Ericsson and Apple. From Intermediate Relay UE perspective, only one Uu RRC connection, but PC5 connection can be multiple. |
| Xiaomi | If different UE IDs are used for the role of relay and remote UE, it’s naturally supported by two connections. If the same ID is used, only single connection is supported. |
| Sharp | Agree as a baseline, but suggest to add “ For/Per an RSC”. |
| CATT | Agree |
| vivo | We think this is not clear for single PC5 connection, as we explained before, if two L2 IDs are used, not sure why a single PC5 connection is needed. This may relate to the Question 2.5A-6. We should further discuss this. |
| Huawei, HiSilicon | Agree with the proposal with the “single PC5 connection”. |
| ZTE | Agree with the intention, but replace “as long as it has a single PC5 connection” with “only if it has a single PC5 connection with the last relay UE”. When saying intermediate relay also acts as a remote UE, it means the same L2 ID is used for the two roles, otherwise, if two L2 IDs are used, they are been regarded as two different UEs by the peer UE. |
| Lenovo | Agree |
| Spreadtrum | Agree |

### Proposal 11a: The following topology scenario is not considered.

- One physical UE, which acts as both an intermediate Relay UE and a Remote UE, can have two indirect connections with different PC5 connections via physically the same last Relay UE.

(Figure 2.5A-1)

### Proposal 11b: When an intermediate Relay UE acts as a Remote UE simultaneously, it performs both roles of intermediate Relay UE and Remote UE via only a single unicast link between the last Relay UE and the intermediate Relay UE.

[Question 3.5-3] Do you think “Proposal 11a/b” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | P11a not agree, p11b agree. We think an intermediate Relay UE acts as a Remote UE simultaneously regardless if via one or separate PC5 connections. |
| OPPO | P11a agree; P11b, agree with the proposal with the “single unicast link” since otherwise it is a multi-path scenario for the intermediate relay UE. |
| Apple | P11a Not agree. P11b Not agree. |
| Kyocera | Yes |
| Qualcomm | Would like to revisit this scenario after we determine control plane solution. Seems this proposal assumes the intermediate Relay UE has to RRC connection on the relay path. If the intermediate relay UE is required to be connected state for the relay path, then 11a cannot be supported. Otherwise, 11a can be supported for free, that means different UEs can be treated. Additionally, if we preclude this, it also needs to define a solution to identify this scenario. |
| Xiaomi | If different UE IDs are used for the role of relay and remote UE, it’s naturally supported by two connections. If the same ID is used, only single connection is supported. |
| Sharp | Agree as a baseline, but suggest to add “ For an RSC”. |
| CATT | Agree |
| vivo | Similar comment as last question, if two L2 IDs, it is questionable about this ‘single unicast link’. We prefer not to agree with either 11a or 11b now. |
| Huawei, HiSilicon | Similar view as Oppo P11a agree; P11b, agree with the proposal with the “single unicast link” . There should be only one connection between intermediate relay UE and the gNB, and the remote UE can share the link since the intermediate relay UE provides the relaying service |
| ZTE | P11b is actually the same as P10, they can be integrated into as one proposal. Since P10/11b is agreed. P11a is not needed. One concern about L2 IDs in figure 2.5A-1, intermediate relay knows it uses two L2 IDs(C/C’) for intermediate relay operation and remote UE operation separately, but whether the last relay UE knows the two L2 IDs(C/C’) are belonging to a physically same peer UE? If not, what’s the meaning of discussing the physical UE with different L2 IDs. The “physical” is not only for the UE itself but also should be identified by the peer UE. |
| Lenovo | Agree. But same view as ZTE that p11b is same as P10.. |
| Spreadtrum | Agree |

### Proposal 14a (it was proposal 15a in phase 1): The following topology scenario is not considered.

- Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically same last Relay UE. Here, the intermediate Relay UE and last Relay UE have two different PC5 connections to support each physically different Remote UE.

(Figure 2.5B-2)

### Proposal 14b (it was proposal 15a in phase 1): When an intermediate Relay UE supports multiple Remote UEs, a single unicast line is established between last Relay UE and the intermediate Relay UE.

[Question 3.5-4] Do you think “Proposal 15a/b” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | We have no strong views for both proposals. In our view, whether it is a multi-path scenario, should be for the same remote UE perspective. in case Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically same last Relay UE, We don’t see real technical issues to support it, regardless if it is the same or different PC5 connections. Anyway, we are ok to follow the majority view. |
| OPPO | Agree |
| Apple | We do not agree P14a/P14b. We think this is completely up to Intermediate relay UE. Even in R16 NR V2X, one physical SL UE can use multiple different L2 IDs. It is unclear why we want to discuss/restrict UE implementation choices here. |
| Kyocera | Yes |
| Qualcomm | Cannot agree this proposals so far. It should be possible that different RSCs are requested by different Remote UE, so the scenario should exist. And also if the intermediate relay UE is not required to be in CONENCTED state, then this scenario can be supported for free. Would like to postpone the discussion after determining control plane solution. |
| Xiaomi | Agree with apple. We only need to discuss logical connections based on L2 ID. It doesn’t matter it’s physically same or not. |
| Sharp | Agree as a baseline, but suggest to add “ For an RSC”. |
| CATT | Agree |
| vivo | Not agree for now until it is clear about the relationship between logical connection, physical UE and multiple indirect path. |
| Huawei, HiSilicon | Agree |
| ZTE | Agree with the intention of P14b, the truth is, the intermediate relay should have only one parent relay and have only a single PC5 unicast link with its parent relay no matter in which case. P14a is not needed. And as our comments before, the concern on “physical” UE should be clarified first. |
| Lenovo | Agree |
| Spreadtrum | Agree |

### Proposal 12: The following topologies scenario are considered

(a: proposal 12 in phase 1)

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE)

(Figure 2.5A-2)

(b: proposal 14 in phase 1)

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE

(Figure 2.5B-1)

[Question 3.5-5] Do you think “Proposal 12” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | We are open to discuss both scenarios. |
| OPPO | Agree |
| Apple | P12a: Not sure why this needs to be discussed, as the single-hop scenario is a legacy scenario. And we are not sure why D’ and D has to two different L2 IDs. The last relay UE can use a single L2 ID “D” to serve both cases. P12b: Again. We do not understand why B and B’ have to be two different L2 IDs here. |
| Kyocera | Yes |
| Xiaomi | Share similar view with apple. |
| Sharp | Agree |
| CATT | Agree |
| vivo | a) should be further discussed, some issues may need discussion e.g. how last relay should indicate the number of hops in this case. b) seems OK but the L2 ID issue needs to be clarified (whether same or different) |
| Huawei, HiSilicon | Agree |
| ZTE | Case a is actually the same as P9, case b is actually the same as P14b, better to avoid duplicate discussion. |
| Lenovo | agree |
| Spreadtrum | Agree |

### Proposal 13: The following topologies scenarios are not considered

(a: proposal 13 in phase 1)

- One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE

(Figure 2.5A-3)

(b: proposal 16 in phase 1)

- Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s). i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s).

(Figure 2.5B-3)

(c: proposal 17 in phase 1)

- Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically same last Relay UE.

(Figure 2.5C-1)

(d: proposal 18 in phase 1)

- Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE.

(Figure 2.5C-2)

(e: proposal 19 in phase 1)

- (2.5D-1) One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB.

(Figure 2.5D-1)

- (2.5D-2) Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s) i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s). Each last Relay UE has a connection with different cells in the same gNB.

(Figure 2.5D-2)

- (2.5D-3) Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB.

(Figure 2.5D-3)

[Question 3.5-6] Do you think “Proposal 13” is agreeable? If you disagree or think something needs to be modified, please feel free to comment.

| company | Disagreement, suggestion or modification |
| --- | --- |
| Ericsson | We are open for discussions for scenarios where remote UEs are physically different. For scenarios where remote UEs are physically same, are not agreed, in our view, they belong to multi-path scenarios. |
| OPPO | Agree with Proposal 13 |
| Apple | P13a, we do not agree. For (Figure 2.5B-3) and (Figure 2.5D-2), we think they should be supported. Also, we do not think all remote UEs shall connected to the same gNB, as depicted above. The last relay UE may belong to different serving gNBs in those scenarios. In general, we support the cross-path scenario when one intermediate relay UE is connected to multiple next-hop relay UEs. This is, by default, should be supported. Otherwise, the intermediate relay UE’s usage in PS scenarios is quite limited. |
| Kyocera | Yes |
| Qualcomm | Again, it should be possible that different RSCs are requested by different Remote UE and different Relay UE support different RSC, so all of the scenario should exist. And also if the intermediate relay UE is not required to be in CONENCTED state, then all of the scenario can be supported for free. If we want to exclude these scenarios, we have to define solutions to identify this scenarios and address how to exclude these scenarios. Would like to revisit whether these scenario can be supported after determining control plane solution. |
| Xiaomi | OK to limit the scope |
| Sharp | Agree. And we suggest that, from the AS layer perspective, RAN2 does not consider multiple RSCs for the discussion. |
| CATT | Agree |
| vivo | We agree that all other cases can be deprioritized except Figure 2.5C-1, which we need to further clarify about the L2 ID issue. |
| Huawei, HiSilicon | Agree with the proposal |
| ZTE | The truth is, the intermediate relay should have only one parent relay and have only a single PC5 unicast link with its parent relay no matter in which cases. A general principle is preferred. We do not need to list all the scenarios not supported. |
| Lenovo | Agree |
| Spreadtrum | Agree |

# Conclusion and recommendation

In conclusion, Rapporteur recommends agreeing the following proposals:

[TDB]

# Annex A: RAN2 Agreements on Multi-hop Relay

## RAN2#127

- From RAN2 perspective, models A and B can both be supported.

- Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying.

- Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying

- Reuse existing resource pools defined for discovery message transmission and reception

- Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE

- Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18.

- If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

- If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

- FFS discovery conditions for the intermediate relay UEs.

- The following connections are assumed as a baseline to be needed:

  - From last Relay UE perspective:

    - A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

    - A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

  - From intermediate relay UE perspective (including first relay UE):

    - A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

    - An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

    - FFS what RRC states are supported for the intermediate relay UE

  - From U2N Remote UE perspective:

    - A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

    - An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

- The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18)

- The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).