3GPP TSG-RAN WG2 Meeting #129-bis	Tdoc R2-25xxxxx

Wuhan, China, April 7 – 11, 2025

Agenda:	x.x.x

Source:	Ericsson

Title:	Comments on MIMO Running CR for TS 38.331

Document for:	Discussion, Decision

# 1	Introduction

This document collects comments for the following e-mail discussion:

[Post129][207][MIMO_Ph5] Running CR for TS 38.331 (Ericsson)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

Companies are invited to provide contact details on the table below.

| Company | Name | E-mail |
| --- | --- | --- |
| OPPO | Yumin Wu | wuyumin@oppo.com |
| CATT | Da Wang | wangda@catt.cn |
| Sharp | Chongming Zhang | Chongming.zhang@cn.sharp-world.com |
| CMCC | Changhao Han | hanchanghao@chinamobile.com |
| Samsung | Shiyang Leng | shiyang.leng@samsung.com |
| Nokia | Andrew Lappalainen | andrew.lappalainen@nokia.com |

# 2	Discussion

The running CR implements the latest stable parameters from the list provided by RAN1 (R1-2501645). For reference, the parameter list is included in the draft discussion folder with green highlight for the parameters implemented and the column “RAN2 ASN.1 name” filled in. The additions compared to the previous version (R2-2408909) are with user “RAN2#129-bis”.

Please do not make changes/comments directly on the running CR - companies are invited to provide suggested changes/comments on the table below. To make it easier to track and reply to the comments, please label each comment i.e. [Issue 1], [Issue 2], and so on.

| Company | Comments | Rapporteur response |
| --- | --- | --- |
| OPPO | Some parameters (e.g. startingBitOfFormat2-3 and plOffset) are missing in the running CR. |  |
| CATT [Issue 1] | For the description of prachAssociationDCI-1-0, according to the below RAN1 agreement, one condition is missing, i.e., “ a UE provided with SSB-MTC-AddtionalPCI“. Agreement For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH: - The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH; - The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH: - In this case, the PRACH configuration associated with addition PCI is used. - This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured. |  |
| CATT [Issue 2] | Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, except for Parameter Combination 8 from Rel-17 FeType-II PS, all legacy Parameter Combinations from Rel-16 eType-II (regular), Rel-18 Type-II Doppler (regular), and Rel-17 FeType-II PS are supported. According to the above RAN1 agreement, the following value of paramCombination-r19 should be INTEGER (1..7) typeII-FePortSelection-r19 SEQUENCE { typeII-FePortSelectionRI-Restriction-r19 BIT STRING (SIZE (4)), numberOfPMI-SubbandsPerCQI-Subband-r19 INTEGER(1..2), paramCombination-r19 INTEGER (1..8), valueOfN-r19 ENUMERATED {n2, n4} OPTIONAL, -- Need R }, |  |
| CATT [Issue 3] | typeII-DopplerPortSelection-r19 SEQUENCE { typeII-PortSelectionRI-Restriction-r19 BIT STRING (SIZE (4)), numberOfPMI-SubbandsPerCQI-Subband-r19 INTEGER(1..2) } additionalOneSlotOffsetDoppler Configures 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet. This field is only configured for codebook typeII-Doppler-r19 and typeII-DopplerPortSelection-r19. In Rel-19， there is no enhancement of doppler port selection. Thus, the above parameter typeII-DopplerPortSelection should be deleted. |  |
| CATT [Issue 4] | Regarding to the description of additionalOneSlotOffset, based on the following RAN1 agreement, this field is also only configured for codebook typeII-FePortSelection-r19. Thus, typeII-FePortSelection-r19 should be added in the end of description, i.e., “This field is only configured for codebook typeI-SinglePanel-r19, typeI-MultiPanel-r19, etypeII-r19 and typeII-FePortSelection-r19”. Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, for AP-CSI-RS where the K NZP CSI-RS resources are located in two consecutive slots, • Except for codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the legacy resource-set-level slot offset configuration should be assumed or not • For codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the resource group slot offset should be assumed or not |  |
| CATT [Issue 5] | SRS-ResourceSet ::= SEQUENCE { ...omit... [[ associatedCSI-RS-Set-r19 NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R srs-TwoSeparatePowerControlAdjustmentStates-r19 ENUMERATED {enabled} OPTIONAL, -- Need R srs-PortGrouping-r19 ENUMERATED {enabled} OPTIONAL, -- Need R tpcOfSRS-ClosedLoopIndexInDCI-1-1-r19 ENUMERATED {enabled} OPTIONAL, -- Need R srsClosedLoopIndexIndicatorInDCI-1-1-r19 ENUMERATED {enabled} OPTIONAL -- Need R fourPortSRS-3Tx-r19 ENUMERATED {enabled} OPTIONAL -- Need R ]] } The highlight two parameters are not per SRS-ResourceSet configuration. Suggest to configure these two parameters in SRS-Config. |  |
| CATT [Issue 6] | There are no description of tpcOfSRS-ClosedLoopIndexInDCI-1-1-r19 and srsClosedLoopIndexIndicatorInDCI-1-1-r19. Suggest to add the descriptions. |  |
| CATT [Issue 7] | CSI-ReportCJTC-r19 ::= SEQUENCE { valueOfAD-r19 ENUMERATED {dot5, one}, valueOfMD-r19 ENUMERATED {n32, n64, n128, n256}, valueOfAFO-r19 ENUMERATED {zeroDot1, zeroDot2}, valueOfMFO-r19 ENUMERATED {n16, n32, n256 }, valueOfMPhi-r19 ENUMERATED {n16, n32}, linkedCJTCReport CSI-ReportConfigID, OPTIONAL -- Need R subbandSize ENUMERATED {n1, n2, n4, n8, n16, wideband}, } The above 6 highlight parameters shoule be optional, since which parameter is configured depends on the configuration of reportQuantityCJTC-r19. |  |
| ZTE[Issue 1] | The field description of the “pathlossOffsetPRACH-DCI-1-0-r19” has not been added yet |  |
| ZTE[Issue 2] | The paramCombination-r19/ paramCombination-Doppler-r19 should also be added to the field descriptipon of the “ paramCombination, paramCombination-CJT-r18, paramCombination-CJT-L-r18, paramCombination-CJT-PS-r18, paramCombination-CJT-PS-alpha, paramCombinationDoppler-r18, paramCombinationDoppler-PS-r18” and add the referred chapters in the 38.214 |  |
| ZTE[Issue 3] | csi-CRI-ValueOfM: In the field description, the restriction for the type 1 was not included “1,2,…,min(4,Ks) for Type-I, where Ks={2,3,4,…,8} is the number of CSI-RS resources”, we think it’s useful as a restriction for the network configuration. |  |
| ZTE[Issue 4] | srs-PortGrouping: If configured, it indicates that SRS port grouping is enabled. The below restriction is missed, Applicable only for reportQuantity = ‘cri-RI-CQI’ and when SRS for AS is xT6R or xT8R |  |
| ZTE[Issue 5] | For the bwp ID, it depends on whether the new beams configuration also include the BWP ID (as the legacy CSI-Report Config, the corresponding bwp-id was indicated in the CSI-ResourceConfig) For the servCellIndex, now the cross carrier scheduling scheme is still not so clear, thus, we can add some Editor’s note to these 2 elements. resourceForSecondChannelOfModeB-r19 SEQUENCE { configuredGrantConfigIndex-r19 ConfiguredGrantConfigIndex-r16, bwp-Id-r19 BWP-Id, servCellIndex-r19 ServCellIndex } |  |
| ZTE[Issue 6] | additionalOneSlotOffset Configures 1-slot offset (per NZP-CSI-RS Resource) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet. This field is only configured for codebook typeI-SinglePanel-r19, typeI-MultiPanel-r19 and etypeII-r19. We think the this field can also be configured for codebook typeII-FePortSelection-r19 |  |
| Sharp[Issue 1] | Considering csi-ReportUE-IBM is included in the IE CSI-ReportConfig, the existing description may need update. #### – CSI-ReportConfig The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1. |  |
| Sharp[Issue 2] | Regarding to the choice of periodicityAndOffset, the value range has not been defined yet from my understanding, maybe a note could be added for future updating. firstPUCCHResourceConfig-r19 SEQUENCE { periodicityAndOffset CHOICE { sym2 NULL, sym6or7 NULL, sl1 NULL, -- Recurs in every slot sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639) }, |  |
| CMCC[Issue 1] | Agreement RAN2#128 Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. The parameter plOffset is not included in this running CR, though it is included in the agreements of RAN2#128. |  |
| CMCC[Issue 2] | The paramCombination-r19/ paramCombination-Doppler-r19 should be added in the paramCombination field descriptipon. |  |
| CMCC[Issue 3] | SRS-ResourceSet ::= SEQUENCE { ...omit... [[ associatedCSI-RS-Set-r19 NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R srs-TwoSeparatePowerControlAdjustmentStates-r19 ENUMERATED {enabled} OPTIONAL, -- Need R srs-PortGrouping-r19 ENUMERATED {enabled} OPTIONAL, -- Need R tpcOfSRS-ClosedLoopIndexInDCI-1-1-r19 ENUMERATED {enabled} OPTIONAL, -- Need R srsClosedLoopIndexIndicatorInDCI-1-1-r19 ENUMERATED {enabled} OPTIONAL -- Need R fourPortSRS-3Tx-r19 ENUMERATED {enabled} OPTIONAL -- Need R ]] } We think these three parameters should be configured per SRS-Config. |  |
| CMCC[Issue 4] | For the new beam, there is also a BWP-ID of the DL CSI-RS indicated in the CSI-ResourceConfig. For this bwp-Id for Second Channel of ModeB, we think that it should be expressed as UL-bwp-Id-r19. resourceForSecondChannelOfModeB-r19 SEQUENCE { configuredGrantConfigIndex-r19 ConfiguredGrantConfigIndex-r16, UL-bwp-Id-r19 BWP-Id, servCellIndex-r19 ServCellIndex } |  |
| Samsung Issue1 | pathlossOffsetPRACH-DCI-1-0-r19 FD missing, according to RRC list “To indicate the presence of 1-bit DCI field in DCI format 1_0 for indicating the PL offset for PDCCH-order PRACH transmission” Agreement in RAN1#118bis For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: • This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. • When one joint/UL TCI state is indicated in Rel-17 unified TCI, o the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation o the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. • FFS: Whether the bit field can be used to indicate other information • FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI - It should be conditionally present: this field is optional present if at least one joint TCI state in dl-OrJointTCI-StateList or at least one UL TCI state in ul-TCI-StateList is configured with PL offset. |  |
| Samsung Issue2 | CodebookConfig-r19 ::= SEQUENCE { codebookType CHOICE { type1 SEQUENCE { subType CHOICE { typeI-SinglePanel-r19 SEQUENCE { codebookMode-r19 ENUMERATED {modeA,modeB}, typeI-SinglePanel-ri-Restriction-r19 BIT STRING (SIZE (8)) }, typeI-MultiPanel-r19 SEQUENCE { ri-Restriction-r19 BIT STRING (SIZE (4)) } }, }, type2 SEQUENCE { subType CHOICE { seems this subtype is not needed, we can directly use CHOICE as below? type1 CHOICE { typeI-SinglePanel-r19 typeI-MultiPanel-r19 }, |  |
| Samsung Issue3 | CSI-ReportUE-IBM-r19 ::= SEQUENCE { eventType-r19 ENUMERATED {event1, event2, event7}, valueOfQ-r19 INTEGER (1..8) OPTIONAL, -- Need R resourceForSecondChannelOfModeB-r19 SEQUENCE { configuredGrantConfigIndex-r19 ConfiguredGrantConfigIndex-r16, This field should be optional. In configuredGrantConfig, this index is mandatory present for a CG in configuredGrantConfigToAddModList-r16, otherwise the field is absent (i.e., there is only one CG per BWP config in Rel15 with no. Network can only configure CG in one BWP using either configuredGrantConfig or configuredGrantConfigToAddModList-r16. Also, a restriction should be added that this index can only refer to a type-1 CG, as RAN1 agreed. |  |
| Samsung Issue4 | firstPUCCHResourceConfig-r19 SEQUENCE { periodicityAndOffset CHOICE { sym2 NULL, sym6or7 NULL, sl1 NULL, -- Recurs in every slot sl2 INTEGER (0..1), this is copied from existing periodicityAndOffset since Rel-15, but just wonder why we need this originally? |  |
| Samsung Issue5 | firstPUCCHResourceConfig Indicates the periodic PUCCH resource for first PUCCH for both mode-A and mode-B. There is only one PUCCH transmission, it can be rephrased, otherwise may cause confusing. Suggest “for the first step of PUCCH transmission for both mode-A and mode-B UE-initated beam reporting” resourceForSecondChannelOfModeB Indicates Type-1 CG PUSCH resource for second channel in mode-B. Similarly, suggest to replace by “for the second step of PUSCH transmission in mode-B UE-initiated beam reporting.” |  |
| Samsung Issue6 | associatedCSI-RS-Set-r19 NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R As RAN1 indicated this is only for SRS resource set used for noncodebook, do we need -- Cond NonCodebook ? then the description in FD can be removed? associatedCSI-RS-Set ID of CSI-RS resource set associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause x.y). |  |
| Samsung Issue7 | srs-TwoSeparatePowerControlAdjustmentStates-r19 This is per BWP as RAN1 indicated, not per SRS resource set, should be included in SRS-Config. |  |
| Samsung Issue8 | tpcOfSRS-ClosedLoopIndexInDCI-1-1-r19 This is per BWP as RAN1 indicated, not per SRS resource set, should be included in SRS-Config. |  |
| Samsung Issue9 | srsClosedLoopIndexIndicatorInDCI-1-1-r19 This is per BWP as RAN1 indicated, not per SRS resource set, should be included in SRS-Config. Missing FD Also, according to RAN1 agreement Agreement in RAN1#119 The working assumption part of the following previous agreement is confirmed. Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured. => srsClosedLoopIndexIndicatorInDCI-1-1-r19 can be configured only if srs-TwoSeparatePowerControlAdjustmentStates-r19 is configured |  |
| Samsung Issue10 | p0AlphaSetforPUSCH, p0AlphaSetforPUCCH, p0AlphaSetforSRS Configures power control parameters for PUSCH, PUCCH and SRS (see TS 38.213 [13], clause 7.2). When the field alpha is absent in p0AlphaSetforPUSCH, the UE applies the value 1 for PUSCH power control. When the field alpha is absent in p0AlphaSetforSRS, the UE applies the value 1 for SRS power control. In p0AlphaSetForPUCCH, the field alpha is absent (not used). If srs-TwoSeparatePowerControlAdjustmentStates is configured and srs-PowerControlAdjustmentStates is set to separateClosedLoop, the field closedLoopIndex in p0AlphaSetforSRS indicates one of the separate SRS power control adjustment states i.e. value of i0 and i1 refer to the first and the second power control adjustment state separate from PUSH, respectively. Typo, => PUSCH |  |
| Nokia [Issue 1] | portSubsetIndicator is defined as below for R18 and R19. In R18, the network cannot indicate portSubsetIndicator simultaneously with a2-parameters. Presumably, the same restriction applies for R19. However, the field description for portSubsetIndicator only states that “The network does not configure portSubsetIndicator and portSubsetIndicator-v19xy simultaneously.”, which does not prevent indicating R18 a2-parameters simultaneously with portSubsetIndicator-v19xy. Therefore, wouldn’t a further restriction be needed to state that “The network does not configure a2-parameters and portSubsetIndicator-v19xy simultaneously?” CSI-ReportSubConfig-r18 ::= SEQUENCE { reportSubConfigId-r18 CSI-ReportSubConfigId-r18, reportSubConfigParams-r18 CHOICE { a1-parameters SEQUENCE { codebookSubConfig-r18 CodebookConfig OPTIONAL, -- Need R portSubsetIndicator-r18 CHOICE { p2 BIT STRING (SIZE (2)), p4 BIT STRING (SIZE (4)), p8 BIT STRING (SIZE (8)), p12 BIT STRING (SIZE (12)), p16 BIT STRING (SIZE (16)), p24 BIT STRING (SIZE (24)), p32 BIT STRING (SIZE (32)) } OPTIONAL, -- Need R non-PMI-PortIndication-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks OPTIONAL -- Need R }, a2-parameters SEQUENCE { nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 } } OPTIONAL, -- Need R powerOffset-r18 INTEGER(0..23) OPTIONAL -- Need R } CSI-ReportSubConfig-r19 ::= SEQUENCE { portSubsetIndicator-v19xy CHOICE { p48 BIT STRING (SIZE (48)), p64 BIT STRING (SIZE (64)), p128 BIT STRING (SIZE (128)) } OPTIONAL -- Need R } portSubsetIndicator, portSubsetIndicator-v19xy Indicates the (sub)set of CSI-RS antenna ports used for CSI calculation of the sub-configuration. In the bit string, each bit corresponds to an antenna port. When a bit is set to 1, the corresponding port is enabled for CSI calculation corresponding to the sub-configuration. When the bit is set to zero, the corresponding port is not enabled for CSI calcualton corresponding to the sub-configuration. The size of the bit string equals P bits, where P=2/4/8/12/16/24/32/48/64/128 represents the number of ports of the NZP CSI-RS resource(s) in the resource set for channel measurement associated with the CSI-ReportConfig. The network does not configure portSubsetIndicator and portSubsetIndicator-v19xy simultaneously. |  |
| Samsung Issue11 | csi-CRI-ValueOfM Number of CRIs. For Type-2 codebook configuration, the network only includes value 1 or 2. Based on RAN1 RRC list “1,2,…,min(4,Ks) for Type-I, where Ks={2,3,4,…,8} is the number of CSI-RS resources, 1,2 for Type-II”, we suggest to revise the sentence, e.g., For codebook type typeI-SinglePanel, NW does not configure a value larger than the number of NZP CSI RS resources per resource set; for codebook type typeII-r16, NW only configures 1 or 2. |  |
| Samsung Issue12 | linkedCJTCReport CSI-ReportConfigID, OPTIONAL -- Need R need suffix -r19? |  |
| Samsung Issue13 | subbandSize ENUMERATED {n1, n2, n4, n8, n16, wideband}, subbandSize Supported sub-band size(s) PRB for SB phase offset reporting. to distinguish from the existing parameter “subbandSize”, need to rename and add suffix, suggest to use “subbandSizeCJTC-19”. |  |
| Samsung Issue14 | csi-ReportCJTC Configures parameters used for CJT with non-ideal synchronization and backhaul. Seems no need to capture this kind of motivation. We can simply say “for CJT calibration.” |  |
| Samsung Issue15 | linkedCJTCReport Linked CJTC report when codebook type is set to eTypeII-CJT. the codebook type is typeII-CJT-r18 |  |
| Samsung Issue16 | valueOfMD MD Uniform-range quantization for the range of delay offset as specified in TS 38.214 [19], clause x.y.. valueOfMFO Uniform-range quantization for the range of frequency offset as specified in TS 38.214 [19], clause x.y.. valueOfMPhi Uniform-range quantization for the range of phase offset as specified in TS 38.214 [19], clause x.y.. For easy understanding, suggest to rephrase to “Indicate the value of MD/MFO/MPhi, the number of uniform quantization samples for the range of delay/frequency/phase offset, as specified in …” |  |
| Samsung Issue17 | additionalOneSlotOffset Configures 1-slot offset (per NZP-CSI-RS Resource) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet. This field is only configured for codebook typeI-SinglePanel-r19, typeI-MultiPanel-r19 and etypeII-r19. Also for typeII-FePortSelection-r19 |  |
| Samsung Issue18 | additionalOneSlotOffsetDoppler ENUMERATED{enabled} OPTIONAL -- Need R According to RAN1 list “For Rel-19 Type-II based on Rel-18 Type-II Doppler: 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet ”, this is per resource group. So we need 4/8/12 bits (i.e., bitmap), one bit per group, NW may configure different values for each group. |  |
| Samsung Issue19 | srs-PortGrouping If configured, it indicates that SRS port grouping is enabled. Accoring to RAN1 list “Applicable only for reportQuantity = ‘cri-RI-CQI’ and when SRS for AS is xT6R or xT8R”, this should be captured in FD, after translation to RRC language, e.g., This field can be configured only if reportQuantity is set to cri-RI-CQI and the usage of the SRS resource set is set to antennaSwitching. |  |
| Samsung Issue20 | fourPortSRS-3Tx Indicates whether port 1003 is disabled for both PUSCH and SRS. This field is only configured if nrofSRS-Ports is set to ports4. Not sure “for both PUSCH and SRS” is correct, what RAN1 provided in the RRC list is “1-bit RRC parameter to indicate that port 1003 is disabled for all the 4 -port resources in an SRS resource set”. So far, RAN1 only has explicit/clear agreement on SRS that 4-port SRS resource is reused and the last port (1003) is disabled for 3Tx, there is no RAN1 agreement on PUSCH since 3 antenna port PUSCH transmission can be directly done using UL codebook for 3TX PUSCH. This field is configured only for 3Tx, not always configured for 4-port SRS. This field is applied for all SRS resources in a SRS resource sets. 3Tx only supports same number of ports for all SRS resources in the SRS resource set. Suggest to revise as follows: Indicates whether port 1003 is disabled for all SRS resources in the SRS resource set. This field can be only configured if nrofSRS-Ports for each SRS resources in the SRS resource set is set to ports4. Alternatively, the last sentence can be captured by a presence condition. |  |