3GPP TSG RAN WG2 Meeting #130                	                	                          R2-250xxxx

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.x

Source:	Xiaomi, Ericsson

Title:	Report of [POST129][037][AI PHY] UE candidate data collection (Xiaomi/Ericsson)

Document for:	Discussion and Decision

# Introduction

This report provides a summary for the following post-meeting email discussion:

- [POST130][037][AI PHY] UE candidate data collection (Xiaomi/Ericsson)

Intended outcome: Discuss how to capture ‘The UE doesn’t need to measure the candidate data collection configuration(s)’ in RAN2 spec, and signalling details (including contents). Provide TP(s) for candidate UE side data collection configuration.

Deadline:  June 20th

Deadline for providing comments is Jun 20th, 2025, 10:00UTC.

Rapporteurs will provide proposals for RAN2#131 and a draft updated TP based on the outcome of this post email discussion after the deadline.

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |

# Discussion

During RAN2 #130 meeting, RAN2 discussed how to provide candidate UE data collection configurations and reached following agreements:

3 The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. 4 For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

In this email discussion, we further discuss the details of how to signal candidate UE data collection configuration(s) and how to capture ‘The UE doesn’t need to measure the candidate data collection configuration(s).’ in RAN2 specification. Three example TPs (merged in one document for easy comparison) are provided for different possible signaling solutions to facilitate the email discussion and a draft updated TP will be provided by the Rapporteurs after the conclusion of the email discussion, based on the outcome.

It is agreed that the agreement ‘The UE doesn’t need to measure the candidate data collection configuration(s)’ will be captured in RAN2 specification. Since all options listed as examples above (CSI-ReportConfig or simplified signaling) are intended to use RRC signaling to provide candidate UE data collection configuration(s) towards UE, it is straightforward to capture the corresponding UE behavior upon receiving candidate UE data collection configuration(s) in RRC specification (i.e., TS 38.331).

Observation 1: The agreement ‘The UE doesn’t need to measure the candidate data collection configuration(s)’ will be captured in TS 38.331.

## Container of Candidate Configuration

It is possible for the UE to support multiple configurations for UE-side data collection and it was agreed that the network can provide a list of (i.e., multiple) candidate configurations. Therefore, it is reasonable that the UE can also indicate multiple preferred configurations for network’s consideration.

##### Q1.  Do you agree that multiple preferred configurations can be indicated by the UE via UAI?

| Company | Yes/No | Comment |
| --- | --- | --- |

##### Q2. Do you agree that certain preferred configuration can be referred by the UE via an identifier associated to a candidate configuration?

| Company | Yes/No | Comment |
| --- | --- | --- |

The next question is which RRC parameter is used to contain the candidate configuration.

During online discussion, three main solutions were proposed to contain/signal candidate UE data collection configurations:

Solution 1: OtherConfig contains a list of candidate configurations as a list with a new IE, where the new IE (i.e., each candidate configuration) contains at least the following set of parameters as agreed in RAN2#130, i.e., identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs [4]. An example of DataCollectionPreferenceConfig-r19 is shown as below:

DataCollectionPreferenceConfig-r19 :: = SEQUENCE { -- Solution 1 dataCollectionCandidateConfigList-r19 SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfig-r19 OPTIONAL, -- Need N }

Solution 2: OtherConfig contains a list of candidate configurations as a list of CSI-ReportConfigId, where the candidate configurations are provided by the gNB within csi-ReportConfigToAddModList under CSI-ReportConfig [5]. An example of DataCollectionPreferenceConfig-r19 is shown as below:

DataCollectionPreferenceConfig-r19 :: = SEQUENCE { -- Solution 2 dataCollectionCandidateConfigList-r19 SEQUENCE (SIZE (1.. maxCandidateConfig-r19)) OF CSI-ReportConfigId OPTIONAL, -- Need R }

Solution 3: OtherConfig contains a list of candidate configurations as a new list of CSI-ReportConfig, i.e., creating a list of CSI-ReportConfig in otherConfig, which is independent from CSI-ReportConfig under CSI-MeasConfig. An example of DataCollectionPreferenceConfig-r19 is shown as below:

DataCollectionPreferenceConfig-r19 :: = SEQUENCE { -- Solution 3 dataCollectionCandidateConfigToAddModList SEQUENCE (SIZE (1..maxCandidateConfig -r19)) OF CSI-ReportConfig OPTIONAL, -- Need N }

Following agreements were further reached in RAN1 for the measured UE data collection configuration:

RAN1#120 Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B FFS: whether/how to support 'aperiodic' CSI RS RAN1 #121 Agreement For data collection for UE-sided model, in CSI-report configuration, reportQuantity is set to “none-BM-r19” RAN1 #121 Agreement For UE-sided model, regarding a CSI-ReportConfig for data collection, - Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured

It is observed that RAN1 has agreed, for UE measured UE-side data collection configuration provided in CSI-ReportConfig, considering UE needs to measure configurations but does not need to report, reportQuantity of such CSI-ReportConfig is set to ‘none-BM-r19’ and CPU occupation is 1.

Observation 2: RAN1 agreed a new reportQuantity ‘none-BM-r19’ for UE-side data collection that UE needs to measure but not need to report. CPU occupation for such measured UE-side data collection is 1.

As RAN2 agreed a new UE behaviour, i.e., ‘UE doesn’t need to measure the candidate data collection configuration(s)’, and that this needs to be captured in RAN2, the solution for signaling candidate data collection configuration(s) needs to fulfil these requirements and avoid RAN1 impact.

In order to clarify that no measurements need to be performed by the UE for the candidate configurations, Rapporteurs included the following note in Section 5.3.5.9 in all the TPs (we used the endorsed RRC running CR R2-2504349, except the NOTE highlighted in yellow):

Subclause 5.3.5.9 The UE shall: 1> if the received OtherConfig includes the DataCollectionPreferenceConfig: 2> if DataCollectionPreferenceConfig is set to setup: 3> consider itself to be configured to provide its preference on being configured with radio measurement resources for UE data collection configuration in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide its preference on being configured with radio measurement resources for UE data collection configuration. NOTE: UE does not need to measure radio measurement resources provided in DataCollectionPreferenceConfig.

In solution 1/3, proponent companies understand that, since all candidate CSI-RS resource configurations are provided in OtherConfig, according to the NOTE, it is clear that UE does not need to perform measurement towards CSI-RS resources configured in DataCollectionPreferenceConfig.

In solution 2, proponent companies understand that, if the inference configurations provided in CSI-ReportConfig (Option A) is not applicable (for periodic, semi-persistent or aperiodic) or not activated (for semi-persistent or aperiodic) and network has not released the corresponding inference configuration (included in csi-ReportConfigToAddModList), UE also does not need to perform measurement towards such CSI-ReportConfig.

##### Q3. Do you agree that the above NOTE in subclause 5.3.5.9 in TS 38.331 is sufficient to avoid RAN1 impacts for Solution 1/2/3? If the answer to the above question is No for certain solution, please explain what other impacts or RAN1 impacts foreseen.

| Company | Solution 1 Yes/No | Solution 2 Yes/No | Solution 3 Yes/No | Comment |
| --- | --- | --- | --- | --- |

Finally, if you agree with the intention of the NOTE, Rapporteurs would like to ask if companies see the need to capture in the procedural text the NOTE highlighted above or somewhere else (e.g. field description), please comment on that.

##### Q4. Do you agree to capture the above NOTE in subclause 5.3.5.9 in TS 38.331? If no, please comment in case you agree with the intention of the NOTE, but you prefer to include this information somewhere else (e.g. in the field description).

| Company | Yes/No | Comment (where and subclause to capture) |
| --- | --- | --- |

## Content

Following candidate configuration contents were agreed in RAN2 #130 meeting:

For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements

In the following, we discuss how to represent the above agreed content for each of the Solutions 1/2/3, and whether additional parameters are needed.

##### Q5-1. For Solution 1, besides the above parameters agreed in RAN2#130 (which are already captured in the proposed TP), which other parameters, if any, need to be included in candidate UE data collection configuration? Please explain your reasons.

| Company | Additional RRC parameters as candidate configuration in Solution 1? (Yes/No) | Comment (please list the RRC parameter and explain why this is needed as part of candidate data collection configuration) |
| --- | --- | --- |

##### Q5-2. For Solution 2 and 3, do you agree that the above parameters agreed in RAN2#130 would be already included as part of the CSI-ReportConfig (i.e., the above parameters would be already introduced in the ASN.1 in the CSI-ReportConfig to support the UE side data collection configuration)?

| Company | Yes/No | Comment |
| --- | --- | --- |

##### Q5-3. For Solution 2 and 3, besides the above parameters agreed in RAN2#130, which other parameters, if any, need to be included within the CSI-ReportConfig as candidate UE data collection configuration? Please explain your reasons.

| Company | Additional RRC parameters as candidate configuration in Solution 2/3? (Yes/No) | Comment (please list the RRC parameter and please explain why this is needed as part of candidate data collection configuration) |
| --- | --- | --- |

As discussed in Q1, network may provide an ID for each candidate configuration (either the new DataCollectionCandidateConfigId for Solution 1 or a CSI-ReportConfigId for Solutions 2 and 3). UE can directly report these ID(s) representing its preferred configuration(s). Besides the preferred candidate configuration ID(s), is there any other information needed that the UE should include in UAI along with the preferred configuration(s)?

##### Q6. What information does UE include in its preferred configuration via UAI (except configuration ID in Q2 if agreed)?

| Company | Parameter (besides configuration ID if agreed in Q2) in Solution 1, 2, 3 | Comment (please explain why this is needed as part of preferred data collection configuration) |
| --- | --- | --- |

##### Q7. Do you think prohibit timer should be considered for UE indicating its preferred data collection configuration?

| Company | Yes/No | Comment |
| --- | --- | --- |

##### Q8. Among Solution 1/2/3, which solution is acceptable/not acceptable?

| Company | Acceptable (Solution 1/2/3) | Not acceptable (Solution 1/2/3) |
| --- | --- | --- |

# Conclusion

# Reference

[1] R2-2504309	Remaining open issues: LCM for UE-sided model for BM use case	InterDigital

[2] R2-2504353	Open Issues on LCM for UE-sided Models for Beam Management and CSI Prediction 	Qualcomm Incorporated

[3] R2-2503775	Further Discussion on LCM of UE-sided Model for AI BM	MediaTek Inc.

[4] R2-2503449	Discussion on open issues of AI/ML air LCM	Xiaomi

[5] R2-2504414	On Enhancements for NW Involvement in UE-side DC for BM	Nokia, T-Mobile USA Inc., Ericsson, BT plc, Verizon, Deutsche Telekom AG