3GPP TSG-RAN WG2 Meeting #131	R2-250xxxx

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	8.11.1

Source: 	Huawei, HiSilicon (Rapporteur)

Title: 	Summary of [Post130][216][SBFD] Running CR for 38.331 (Huawei)

Document for:	Discussion and decision

# 1 Introduction

This is the summary of the following email discussion:

- [Post130][216][SBFD] Running CR for 38.331 (Huawei)

Intended outcome: Updated and reviewed the CR for endorsement, update the open issue list if needed, can also discuss open issues to form proposals to the next meeting

Deadline:  Long

The purpose of this email discussion is to identify remaining open issues related to RRC impacts and discuss the potential solutions in order to form proposals to the next meeting. Suggestions on corresponding RRC specification implementation are welcome. Companies are encouraged to provide inputs on the open issues and solutions, particularly those critical to the completion of this WI.

# 2 Open issues related to random access RRC signaling

## RRC-1 RRC signalling design for the NT indicates RO types to the UE

Issue description:

It is agreed in previous RAN2 meetings that for determining CBRA RO type (SBFD ROs or non-SBFD ROs) used in the RACH operation, the NT can indicate RO type to the UE directly through RRC signalling. When this signalling is absent, the UE can use RSRP measurement based method or use its implementation to determine the RO type (when the RSRP threshold signalling is also absent).

Though the current RRC running CR uses one bit for this RO type signalling, there is view that the detailed signalling design hasn't been agreed during the online meeting and different signalling design should be discussed.

According to [9] P2.1, P2.2, the NW can include a value  between 0 and 1 based on the NW load (FFS the granularity of the value generated (how many bits)). Upon receiving the indicated value, the UE generates a uniformly distributed random number between 0 and 1(using pseudo random number generator algorithm), and if the generated value is less than the value indicated by the NW, the UE uses the SBFD RO for random access. [12] P1 and [13] P6 proposed the similar approach that, the network indicates the RO type with not 1 bit but e.g. 2 bits representing four different values: {100%, 80%, 50%, 0%}. Rapporteur assumes the UE would use this indicated values in a way similar to what is proposed in [9].

Proposed resolution:

As this issue is about detailed RRC signalling design, Rapporteur suggests to down-select from the two options below. One option is to use the multiple values (e.g., 2 bits for four values) to assist the UE to select RO type, another option is to keep the signalling design in the current RRC running CR (i.e.,  1 bit indicating RO type directly) considering the UE can use RSRP  measurement based method to determine RO type. That is, when the NT chooses not to sent this RO type signalling, the RO type will be determined based on the UE RSRP measurement and the signalled RSRP threshold value (plus the RSRP threshold usage parameter "above", "below") can influence the "load distribution" regarding RO type selection.

Option 1:  Use the multiple values (e.g., 2 bits for four values) to assist the UE to select RO type.

Option 2:  Use 1 bit indicating RO type directly (as in the current RRC running CR).

Company inputs:

Question: Which option above would your company support?

| Company | Option | Comments |
| --- | --- | --- |
| Nokia | Option 1 | If we just indicate via 1-bit, then it would force all the SBFD UEs to use the indicated RO type (SBFD RO or legacy RO), which is not what we want to achieve. Meaning, what we want to achieve is that the network indicate via broadcast signalling that a portion of the SBFD UEs to use a type of RO based on the network load condition. |

[Summary]

[Proposal]

## 2.2 RRC-2 RRC signalling design for the NT indicates RSRP threshold on RO type selection

Issue description:

[18] P3 proposed that a different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs to limit inter-UE CLI and achieve a better load balancing between additional RO type and legacy RO type. Rapporteur understands this is to indicate more than one RSRP thresholds, and they are specific to an SSB or a group of SSBs.

Proposed resolution:

As agreed in RAN2, the NT can change the RSRP threshold value according to the cell situation. The RSRP measurement based RO type selection (when the direct RO type signalling is absent) can also achieve load balancing between additional RO type and legacy RO type. Based on those considerations and on the fact that RAN2 only agreed on one single RSRP threshold value, rapporteur proposes not to pursue this further optimization.

Proposal:  Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs.

Company inputs:

Question: Would your company agree with the above proposal regarding configuring more than one RSRP threshold values?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree with Rapp proposal | Additionally, if RA fails on the indicated RO type, RAN2 agreed on fall back to the other RO type. |

[Summary]

[Proposal]

## 2.3 RRC-3 RRC signalling design for SBFD RACH configuration

Issue description:

[9] P1 proposed that, for SBFD RACH configuration Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For SBFD RACH configuration Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration..

Proposed resolution:

Rapporteur considers the benefits of saving signalling overhead for the additional RACH configuration would be secondary and it is not critical w.r.t. the completion of this WI. On the other hand, extra work is needed to change the spec, e.g. to change related fields type from mandatory to optional and to mandate UE behaviour e.g. to read parameters from the legacy RACH configuration when the fields for additional RACH configuration are absent (changed from mandatory to optional). Thus rapporteur proposes not to pursue this optimization.

Proposal:  Not to pursue the optimization of parameter signalling of SBFD RACH configuration as proposed in R2-2503840.

Company inputs:

Question: Would your company agree with the above proposal regarding the optimization of parameter signalling of SBFD RACH configuration as proposed in R2-2503840?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Disagree | Since Option 2 necessitates support for two RACH configurations, it introduces some overhead, especially when redundant information is present in these configurations. Hence to reduce the signaling overhead, if the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters it would not be needed to include those parameters in the additional RACH configuration. In such a scenario, inclusion in the legacy RACH configuration would be enough. |

[Summary]

[Proposal]

# 3 RRC-4 SBFD frequency domain configuration

Issue description:

During the RRC running CR review, one company proposes to create SBD frequency domain configuration IE such that RAN3 spec can easily refer to it as changing below:

–	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]],

[[

sbfd-Subband-Allocation-r19        SBFD-Subband-Allocation-r19

]]

}

SBFD-Subband-Allocation ::= SEQUENCE {

ul-subbandlocationAndBandwidth-r19          INTEGER (0..37949)    OPTIONAL,   -- Need R

firstDL-subbandlocationAndBandwidth-r19     INTEGER (0..37949)    OPTIONAL,   -- Need R

secondDL-subbandlocationAndBandwidth-r19    INTEGER (0..37949)    OPTIONAL,   -- Need R

…

}                                                                OPTIONAL ---Need R

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

Proposed resolution:

Rapporteur understands this is small change on the RRC field/IE design and it still follows the instruction from RAN1 parameter list while might be beneficial for RAN3 spec. Rapporteur would like to check companies' view on below proposal as this issue is out of RAN2 scope.

Proposal: Adopt the above change in the frequency domain configuration signalling design.

Company inputs:

Question: Would your company agree with the above proposal regarding SBFD frequency domain configuration signalling design?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree |  |

# 4 RRC-5 Coexistence of SBFD and Rel-19 NES

Issue description:

[4] proposes that the coexistence of SBFD and Rel-19 NES can be supported. Rapporteur understands that it could be difficult to discuss the coexistence of SBFD and another Rel-19 feature as both may not be fully completed just yet and it is preferrable to discuss the coexistence related issues in e.g. a future release TEI. Rapporteur also understands this discussion does not concern the coexistence case, if any, that does not require spec changes.

Proposed resolution:

Accordingly, rapporteur asks for companies' view on the below proposal and if majority agrees, this issue can be closed. If no majority agreement, RAN2 needs to discuss whether the coexistence of SBFD and Rel-19 NES should be supported and needs to ask RAN1 questions related to coexistence configurations etc.

Proposal:  RAN2 postpones the discussion on the coexistence issue of SBFD and Rel-19 NES, and similarly on the coexistence issues of SBFD and other Rel-19 features that require spec changes.

Company inputs:

Question: Would your company agree with the above proposal regarding SBFD coexisting with Rel-19 NES and other Rel-19 features.

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree with Rapp |  |

[Summary]

[Proposal]

# 5 Residual RRC issues

Companies can list any residual RRC issues here. If justified, rapporteur can assign one open issue number and invite other companies' input on it:

| Company | Issue description and proposed solutions | Rapporteur response |
| --- | --- | --- |

# 6 Conclusion:

[Proposal 1] .

# 7 Reference

[1]	R2-2503379,	Impacts on the random access by the evolution of duplex operation, Huawei, HiSilicon

[2]	R2-2503386,	Random Access for SBFD,	NEC

[3]	R2-2503423,	Random Access in SBFD symbols,	 CATT

[4]	R2-2503434,	Discussion on RACH in SBFD,	 Xiaomi

[5]	R2-2503477,	Remaining issues on Random Access procedure for SBFD,	LG Electronics Inc.

[6]	R2-2503513,	Open Issues on SBFD Random Access, Sharp

[7]	R2-2503606,	Random access in SBFD, Samsung

[8]	R2-2503646,	Remaining issues of SBFD RACH procedure,	 OPPO

[9]	R2-2503840,	Random Access Operation of SBFD, Nokia Corporation

[10]	R2-2503874,	Discussion on random access procedure in SBFD, ZTE Corporation

[11]	R2-2504039,	Remaining issues on RACH procedure in SBFD,	vivo

[12]	R2-2504059,	Remaining issues for Random Access in SBFD Operation,	Sony

[13]	R2-2504169,	Remaining issues for RACH in SBFD,	 Apple

[14]	R2-2504223,	Views on random access for SBFD, Qualcomm Incorporated

[15]	R2-2504255,	SBFD RA remaining aspects,	Ericsson

[16]	R2-2504327,	Discussion on RACH aspect in SBFD,	 InterDigital, Inc.

[17]	R2-2504397,	Discussion on random access in SBFD, CMCC

[18]	R2-2504602,	Random Access in SBFD,	Lenovo

[19]	R2-2503424,	Discussion on other aspects of SBFD,	CATT

[20]	R2-2503441,	Other aspects of SBFD,	Xiaomi

[21]	R2-2503647,	Discussion on the SBFD related issues, OPPO

[22]	R2-2503875,	Discussion on L3 measurements in SBFD,  ZTE Corporation

[23]	R2-2503896,	Other aspects of SBFD,	Nokia

[24]	R2-2504040,	SBFD other aspects,	vivo

[25]	R2-2504225,	Other aspects of SBFD,	Qualcomm Incorporated

[26]	R2-2504256,	CSI-RS measurements in SBFD, Ericsson

[27]	R2-2504328,	Discussion on resource configuration aspect in SBFD, InterDigital, Inc.

[28]	R2-2504447,	Other Aspects of SBFD,	 Samsung

[29]	R2-2504640,	Other aspects about the evolution of duplex operation, Huawei, HiSilicon