3GPP TSG-RAN WG2 Meeting #131	R2-250xxxx

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	8.11.1

Source: 	Huawei, HiSilicon (Rapporteur)

Title: 	Summary of [Post130][216][SBFD] Running CR for 38.331 (Huawei)

Document for:	Discussion and decision

# 1 Introduction

This is the summary of the following email discussion:

- [Post130][216][SBFD] Running CR for 38.331 (Huawei)

Intended outcome: Updated and reviewed the CR for endorsement, update the open issue list if needed, can also discuss open issues to form proposals to the next meeting

Deadline:  Long

The purpose of this email discussion is to identify remaining open issues related to RRC impacts and discuss the potential solutions in order to form proposals to the next meeting. Suggestions on corresponding RRC specification implementation are welcome. Companies are encouraged to provide inputs on the open issues and solutions, particularly those critical to the completion of this WI.

# 2 Open issues related to random access RRC signaling

## RRC-1 RRC signalling design for the NT indicates RO types to the UE

Issue description:

It is agreed in previous RAN2 meetings that for determining CBRA RO type (SBFD ROs or non-SBFD ROs) used in the RACH operation, the NT can indicate RO type to the UE directly through RRC signalling. When this signalling is absent, the UE can use RSRP measurement based method or use its implementation to determine the RO type (when the RSRP threshold signalling is also absent).

Though the current RRC running CR uses one bit for this RO type signalling, there is view that the detailed signalling design hasn't been agreed during the online meeting and different signalling design should be discussed.

According to [9] P2.1, P2.2, the NW can include a value  between 0 and 1 based on the NW load (FFS the granularity of the value generated (how many bits)). Upon receiving the indicated value, the UE generates a uniformly distributed random number between 0 and 1(using pseudo random number generator algorithm), and if the generated value is less than the value indicated by the NW, the UE uses the SBFD RO for random access. [12] P1 and [13] P6 proposed the similar approach that, the network indicates the RO type with not 1 bit but e.g. 2 bits representing four different values: {100%, 80%, 50%, 0%}. Rapporteur assumes the UE would use this indicated values in a way similar to what is proposed in [9].

Proposed resolution:

As this issue is about detailed RRC signalling design, Rapporteur suggests to down-select from the two options below. One option is to use the multiple values (e.g., 2 bits for four values) to assist the UE to select RO type, another option is to keep the signalling design in the current RRC running CR (i.e.,  1 bit indicating RO type directly) considering the UE can use RSRP  measurement based method to determine RO type. That is, when the NT chooses not to sent this RO type signalling, the RO type will be determined based on the UE RSRP measurement and the signalled RSRP threshold value (plus the RSRP threshold usage parameter "above", "below") can influence the "load distribution" regarding RO type selection.

Option 1:  Use the multiple values (e.g., 2 bits for four values) to assist the UE to select RO type.

Option 2:  Use 1 bit indicating RO type directly (as in the current RRC running CR).

Company inputs:

Question: Which option above would your company support?

| Company | Option | Comments |
| --- | --- | --- |
| Nokia | Option 1 | If we just indicate via 1-bit, then it would force all the SBFD UEs to use the indicated RO type (SBFD RO or legacy RO), which is not what we want to achieve. Meaning, what we want to achieve is that the network indicate via broadcast signalling that a portion of the SBFD UEs to use a type of RO based on the network load condition. |
| Sony | Option 1 | Same view as Nokia, for load balancing reasons, we need more than 1-bit. |
| Samsung | Option 2 | Option 1 is further optimization and not essential considering RSRP based method can be used to serve the similar purpose. Considering the time budget, this issue is not critical for WI completion. |
| CATT | Option 2 | Share the same view as Samsung |
| IDC | Option 2 | We have the same view as Samsung’s comment. The option 1 is small signalling optimization of the option 2 and we do not have strong motivation for this. |
| ZTE | Option 2 | Based on NW indication + measured RSRP + UE implementation to choose RO type is enough. No need to optimize the NW indication. |
| NEC | Option 2 | Benefits/gains of Option 1 are unclear. We prefer the simpler Option 2. |
| Apple | Option 1 | The 1 bit indicator comes first when UE determines the RO type (RSRP threshold is used when the flag is not present). Therefore, we don’t think it works well for load balancing purpose. Once NW set it to non-SBFD, all Rel-19 SBFD-aware UE would quickly switch to legacy RO(s) leaving SBFD RACH resources not used at all. To avoid such situation, network needs to frequently change the indicator and let SBFD-aware UE to re-occupy the SBFD RACH resource. Therefore, we prefer to have a distribution factor to direct a subset of Rel-19 SBFD aware UE to select non-SBFD RO(s). Or we can remove the NW indicator for good and rely on RSRP threshold to do the load balancing. |
| LGE | No strong view | We understand the Nokia’s concern. However, if the distribution factor is adopted, additional MAC operation is needed in order to select the RO type randomly with the indicated distribution factor and the UE complexity is expected due to this additional step. Given that there is only one meeting left, it may be safer use only one bit indication for simplicity. |
| CMCC | Option 2 | We agree with Samsung and ZTE. |
| Qualcomm | Option 1 |  |
| Ericsson | Option 2 | The option 1 was raised during the online session before, which is not the detailed RRC signaling detail. We think RAN2 has already concluded option 2 in the final agreement. We see no need to re-open this issue. We prefer simpler Option 2, issue was already well discussed in RAN2 session and need not be rediscussed. |
| Huawei, HiSilicon | Option 2 | First the "load" on SBFD resources can be balanced using RARP based method if it is really needed, second this "load balancing" issue would not be critical for the first release of SBFD function. |

[Summary] Option 1: 4/13, Option 2: 8/13, No strong view: 1/13. The one company having no strong view still indicated in favour of Option 2/can agree with majority. One company supporting Option 1 also considered it as feasible to remove the NW indicator for good and rely on RSRP threshold to do the load balancing. Rapporteurs understands the NT can choose to do that already based on current design as the 1-bit RO type indicator is optional. As stated this is to target down-selecting one option for the next (and the last) meeting, rapporteur propose to support Option 2.

[Proposal for RRC-1] For the NT indicating RO type, use 1 bit signalling (as in the current RRC running CR). [8/13]

## 2.2 RRC-2 RRC signalling design for the NT indicates RSRP threshold on RO type selection

Issue description:

[18] P3 proposed that a different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs to limit inter-UE CLI and achieve a better load balancing between additional RO type and legacy RO type. Rapporteur understands this is to indicate more than one RSRP thresholds, and they are specific to an SSB or a group of SSBs.

Proposed resolution:

As agreed in RAN2, the NT can change the RSRP threshold value according to the cell situation. The RSRP measurement based RO type selection (when the direct RO type signalling is absent) can also achieve load balancing between additional RO type and legacy RO type. Based on those considerations and on the fact that RAN2 only agreed on one single RSRP threshold value, rapporteur proposes not to pursue this further optimization.

Proposal:  Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs.

Company inputs:

Question: Would your company agree with the above proposal regarding configuring more than one RSRP threshold values?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree with Rapp proposal | Additionally, if RA fails on the indicated RO type, RAN2 agreed on fall back to the other RO type. |
| Sony | Agree with Rapp proposal. | In our understanding, SBFD will be used either for coverage enhancement or to improve capacity/reduce latency in a cell. In this usage scenario, the NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are ‘below’ or ‘above’ the configured threshold in cell. So, in this case, SBFD aware UE must always have to initiate RA based on the network indication where the network indication based on RSRP threshold (below or above). However, if needed, the network can change the RSRP threshold semi-statically (from below to above or vice-versa in a cell). Hence, in our view, no need for further enhancement. |
| Samsung | Agree with Rapp proposal |  |
| CATT | Agree with Rapp proposal |  |
| IDC | Agree with Rapp proposal |  |
| ZTE | Agree with Rapp proposal |  |
| NEC | Agree with Rapp proposal | Share the same view/consideration with the rapporteur. |
| Apple | Agree with Rapp proposal |  |
| LGE | Agree with Rapp proposal |  |
| CMCC | Agree with Rapp proposal |  |
| Qualcomm | Agree with Rapp proposal |  |
| Ericsson | Agree with Rapp proposal | We do not understand why new issue (pure for optimization) brought up at this stage, should focus on issues brought up in review of RRC CR. |
| Huawei, HiSilicon | Agree |  |

[Summary] All participating companies agree with Rapp proposal.

[Proposal for RRC-2] Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs. [13/13]

## 2.3 RRC-3 RRC signalling design for SBFD RACH configuration

Issue description:

[9] P1 proposed that, for SBFD RACH configuration Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For SBFD RACH configuration Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration..

Proposed resolution:

Rapporteur considers the benefits of saving signalling overhead for the additional RACH configuration would be secondary and it is not critical w.r.t. the completion of this WI. On the other hand, extra work is needed to change the spec, e.g. to change related fields type from mandatory to optional and to mandate UE behaviour e.g. to read parameters from the legacy RACH configuration when the fields for additional RACH configuration are absent (changed from mandatory to optional). Thus rapporteur proposes not to pursue this optimization.

Proposal:  Not to pursue the optimization of parameter signalling of SBFD RACH configuration as proposed in R2-2503840.

Company inputs:

Question: Would your company agree with the above proposal regarding the optimization of parameter signalling of SBFD RACH configuration as proposed in R2-2503840?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Disagree | Since Option 2 necessitates support for two RACH configurations, it introduces some overhead, especially when redundant information is present in these configurations. Hence to reduce the signaling overhead, if the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters it would not be needed to include those parameters in the additional RACH configuration. In such a scenario, inclusion in the legacy RACH configuration would be enough. |
| Samsung | Agree with proposal |  |
| CATT | Agree with proposal | Agree with the analysis by rapporteur. |
| IDC | Agree with proposal |  |
| ZTE | Agree with proposal |  |
| NEC | Agree with proposal | Share the same view/consideration with the rapporteur. |
| Apple | Agree with proposal |  |
| LGE | Agree with Rapp proposal | Based on current specification, there are more than one RACH-ConfigCommon IE considering the feature combination due to RACH partitioning framework defined in Rel-17. Note that the RACH partitioning framework is defined in Rel-17, but there was discussion on which RA parameter should be used considering various RACH configurations until RAN#130 meeting [R2-2504456]. Therefore, it would be safer to configure the needed RA parameters in the separated RACH configuration. If we consider this optimization, there might be more issues to determine which RA parameter should be used considering various cases of RACH configurations on different feature/feature combinations. |
| CMCC | Agree with proposal |  |
| Qualcomm | Agree with proposal |  |
| Ericsson | Agree with proposal |  |
| Huawei, HiSilicon | Agree |  |

[Summary] No companies other than the proponent support this optimization and there is clear majority to support the Rapp proposal.

[Proposal for RRC-3] Not to pursue the further optimization of parameter signalling of SBFD RACH configuration. [11/12]

# 3 RRC-4 SBFD frequency domain configuration

Issue description:

During the RRC running CR review, one company proposes to create SBD frequency domain configuration IE such that RAN3 spec can easily refer to it as changing below:

–	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

…,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]],

[[

sbfd-Subband-Allocation-r19        SBFD-Subband-Allocation-r19

]]

}

SBFD-Subband-Allocation ::= SEQUENCE {

ul-subbandlocationAndBandwidth-r19          INTEGER (0..37949)    OPTIONAL,   -- Need R

firstDL-subbandlocationAndBandwidth-r19     INTEGER (0..37949)    OPTIONAL,   -- Need R

secondDL-subbandlocationAndBandwidth-r19    INTEGER (0..37949)    OPTIONAL,   -- Need R

…

}                                                                OPTIONAL ---Need R

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

Proposed resolution:

Rapporteur understands this is small change on the RRC field/IE design and it still follows the instruction from RAN1 parameter list while might be beneficial for RAN3 spec. Rapporteur would like to check companies’ view on below proposal as this issue is out of RAN2 scope.

Proposal: Adopt the above change in the frequency domain configuration signalling design.

Company inputs:

Question: Would your company agree with the above proposal regarding SBFD frequency domain configuration signalling design?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree |  |
| Sony | Agree with Rapp proposal. |  |
| Samsung | Agree |  |
| CATT | Agree with Rapp proposal. |  |
| IDC | Agree |  |
| ZTE | Agree with Rapp proposal. |  |
| NEC | Agree |  |
| Apple | Agree |  |
| LGE | Agree with Comment | Note that the definition of “SBFD-Subband-Allocation” is not Optional, but the signalling of “sbfd-Subband-Allocation” is optional, i.e., red-colored editorial modification is needed SCS-SpecificCarrier ::= SEQUENCE { offsetToCarrier INTEGER (0..2199), subcarrierSpacing SubcarrierSpacing, carrierBandwidth INTEGER (1..maxNrofPhysicalResourceBlocks), …, [[ txDirectCurrentLocation INTEGER (0..4095) OPTIONAL -- Need S ]], [[ ul-subbandlocationAndBandwidth-r19 INTEGER (0..37949) OPTIONAL, -- Need R firstDLsubbandlocationAndBandwidth-r19 INTEGER (0..37949) OPTIONAL, -- Need R secondDLsubbandlocationAndBandwidth-r19 INTEGER (0..37949) OPTIONAL -- Need R sbfd-Subband-Allocation-r19 SBFD-Subband-Allocation-r19 OPTIONAL -- Need R ]] } SBFD-Subband-Allocation ::= SEQUENCE { ul-subbandlocationAndBandwidth-r19 INTEGER (0..37949) OPTIONAL, -- Need R firstDL-subbandlocationAndBandwidth-r19 INTEGER (0..37949) OPTIONAL, -- Need R secondDL-subbandlocationAndBandwidth-r19 INTEGER (0..37949) OPTIONAL, -- Need R … } OPTIONAL ---Need R -- TAG-SCS-SPECIFICCARRIER-STOP -- ASN1STOP. |
| CMCC | Agree | We share the same view with the comment provided by LGE. |
| Qualcomm | Agree |  |
| Ericsson | Agree |  |

[Summary] All participating companies support the proposed frequency domain configuration signalling design. It will be implemented in the RRC running CR and no proposal is needed.

# 4 RRC-5 Coexistence of SBFD and Rel-19 NES

Issue description:

[4] proposes that the coexistence of SBFD and Rel-19 NES can be supported. Rapporteur understands that it could be difficult to discuss the coexistence of SBFD and another Rel-19 feature as both may not be fully completed just yet and it is preferrable to discuss the coexistence related issues in e.g. a future release TEI. Rapporteur also understands this discussion does not concern the coexistence case, if any, that does not require spec changes.

Proposed resolution:

Accordingly, rapporteur asks for companies' view on the below proposal and if majority agrees, this issue can be closed. If no majority agreement, RAN2 needs to discuss whether the coexistence of SBFD and Rel-19 NES should be supported and needs to ask RAN1 questions related to coexistence configurations etc.

Proposal:  RAN2 postpones the discussion on the coexistence issue of SBFD and Rel-19 NES, and similarly on the coexistence issues of SBFD and other Rel-19 features that require spec changes.

Company inputs:

Question: Would your company agree with the above proposal regarding SBFD coexisting with Rel-19 NES and other Rel-19 features.

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree with Rapp |  |
| Sony | Agree |  |
| Samsung | Agree |  |
| CATT | Agree |  |
| IDC | Agree |  |
| ZTE | Agree |  |
| NEC | Agree |  |
| Apple | Agree | Coexistence among features could be discussed in TEI phase. |
| LGE | Agree | In NES session of RAN2#130 meeting, the same issue is already discussed and following is agreed: - RAN2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence. Therefore, there is no need to further discuss on this issue. |
| CMCC | Agree |  |
| Qualcomm | Agree |  |
| Ericsson | Agree | We think it is important that RAN2 shall focus on only critical issues to complete the WI on time. Any issues for optimization should be left for future discussions (in TEI or future releases). |

[Summary] All participating companies agree with Rapp proposal. As LGE commented, there was already 130 meeting agreement on this issue, hence no need for further discussion nor proposal.

# 5 RRC-6 Whether to support SBFD for LTM

Issue description:

RAN2 agreed that Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. It is necessary to discuss Whether to support SBFD for LTM.

Proposed resolution:

As the second try on this topic (first in [Post129bis][212][SBFD] Running CR for 38.331 (Huawei)), Rapporteur thinks companies can discuss here and try to down-select from the below options, please clarify the impact on RRC spec (as well as on MAC spec):

Option 1: Not to support both "early UL synchronization with an LTM candidate cell in SBFD symbols” and ”RACH-based LTM in SBFD symbols".

Option 2:  Only support ”RACH-based LTM in SBFD symbols".

Option 3:  Support both "early UL synchronization with an LTM candidate cell in SBFD symbols” and ”RACH-based LTM in SBFD symbols".

Company inputs:

Question: Which option your company would support regarding LTM in SBFD symbols?

| Company | Option | Comments |
| --- | --- | --- |
| ZTE | Option 2 | Motivation: If CFRA parameter is included in the LTM cell switch command MAC CE, UE should only read and use the CFRA parameter in the MAC CE. If RO type indication is not supported in the MAC CE, it means even if the target cell supports UE to initiate RA using SBFD RO, the UE cannot use the SBFD RO. There is no need and no reason to have such restriction. MAC impact: add RO type indication in Rel-18 LTM cell switch command MAC CE; No impact on RRC spec; No impact on RAN3’s specification/work |
| Samsung | Option 1 | We see no considerable benefit by using SBFD RO in LTM cell swich. We also see there is RAN3 impact, since it is more reasonable for target cell to decide which RO type the UE should use in LTM cell switch, and hence, RAN3 signalling for the RO type is required for such purpose. |
| NEC | Option 2 | We support RACH-based LTM since there is benefit in latency reduction. |
| Apple | Option 2 | For early UL sync, the spec effort would be more as RACH for UL sync is a separate config. For RACH-based LTM in SBFD symbols, we could further discuss whether the NW indication on RO type is carried by RRC (no further change needed as RACH-ConfigDedicated already has RO type indicator) or LTM Command MAC CE (MAC should be changed). From our perspective, using LTM Command MAC CE to indicate RO type can be supported, at least for intra-CU scenario. |
| LGE | Option 2, slightly | First, as we discussed in last meeting, early UL sync procedure is not delay-sensitive and supporting SBFD RO for early UL sync procedure causes more specification changes. Therefore, we do not want to re-discuss on this issue. For the RACH-based LTM, it may be beneficial to allow the usage of SBFD RO for RACH-based LTM, given that CFRA for L3 handover already supports the SBFD RO. |
| IDC | Option 2 | The benefit of RACH-based LTM using SBFD symbols is clear (i.e., latency reduction). Moreover, to support this, there may be negligible spec impact on RAN2 which seems not a problem. |
| CMCC | Option 2 | We support RACH-based LTM in SBFD symbols since it helps to reduce the latency. |
| Qualcomm | Option 2 |  |
| CATT | Option 2 |  |
| Ericsson | Option 2 | We agree with Apple and other companies, there is limited additional spec change for RAN2 |
| Huawei, HiSilicon | Option 2 |  |

[Summary] There is clear majority to support Option 2 [10/11]. Rapporteur understands the (only) spec impact is adding RO type indication in Rel-18 LTM cell switch command MAC CE which is proposed here to help MAC spec implementation. Companies could discuss the spec impact for supporting RACH-based LTM cell change in SBFD symbols in their contributions.

[Proposal for RRC-6] (Only) support RACH-based LTM cell change in SBFD symbols [10/11]. Add RO type indication in LTM cell switch command MAC CE.

# 6 RRC-7 Whether to support SBFD with NR-DC

Issue description:

RAN1 has discussed supporting SBFD with NR-DC and its implication on RAN1 spec.

Proposed resolution:

Rapporteur understands that RAN2 still needs to discuss whether to support the simultaneous configuration of SBFD and NR-DC and its impact on RAN2 specs. We can decide via majority to down-select from the below options, please clarify the impacts on RAN2 specs:

Option 1: Not to support the simultaneous configuration of SBFD and NR-DC.

Option 2:  Support the simultaneous configuration of SBFD and NR-DC.

Company inputs:

Question: Which option your company would support regarding the simultaneous configuration of SBFD and NR-DC?

| Company | Option | Comments |
| --- | --- | --- |
| ZTE | Option 2 | Allowing SBFD+NR DC makes the feature more useful in different deployment. Inter band DC causes less inference than CA. Since CA with SBFD is verified and supported by RAN1 and RAN4, DC will not have inference problem to support SBFD. RAN1’s reply LS clearly states that it is feasible: Agreement - From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. - Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly. The answer proves that the SBFD+NR DC does not exceed the WID scope. And, RAN1 has already figured out the specific RAN1 spec impact, so the work load is not a big problem for RAN2 to consider. So, if it is supported, RAN2 can discuss and conclude the RAN2 spec impact during the India meeting. |
| Apple | Option 2 is slightly preferred with comments | I think the potential difficulty to support SBFD over DC is the frequent coordination between MN and SN if UE does not support simultaneousRxTxInterBandCA over a BC. Then, as long as we limit the use case as RAN1 indicated in the LS (UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier), the impact to Xn coordination would be very limited (a notification about SBFD configuration in one node to the other node is needed). |
| IDC | Option 2 | From our view, RAN1 had discussed the feasibility of SBFD with NR-DC according to the RAN2 LS. The conclusion of RAN1 is possible to support SBFD with NR-DC with some conditions/limitations. Based on the RAN1’s response LS and agreements, RAN2 also should support SBFD with NR-DC with conditions and discuss RAN2 spec impact and how to capture it. If the RAN2 goes to option 1, then there may be spec misalignment and different understanding between RAN1 and RAN2. |
| CMCC | Option 2 | We share the same view with Apple. |
| Qualcomm | Option 2 with comments | As pointed by RAN1 reply LS, RAN4 spec TS38.101-1 will be impacted and need to be updated. However, it seems that TS38.101-1 is not in the current WID, and WID may be updated as well in RAN plenary. |
| CATT | Option 2 with comments | From RAN2’s perspective, option 2 can be supported. However the impact on RAN4 should be discussed by RAN4 at first and the final decision replies on the reply LS from RAN4. |
| Ericsson | Not up to RAN2 alone | As pointed out in RAN1 LS, there might be impact on RAN4 spec that is currently not listed in the WID. Whether this RAN4 spec impact is critical or not critical for simultaneous configuration of SBFD and DC is for RAN4 to decide. Technically, we can discuss signalling impacts in RAN2 India meeting if time allows. But the decision to support simultaneous SBFD and DC depends on the potential RAN4 spec impact. We also think restrictions on simultaneous configuration of SBFD and CA/DC should be captured (at least) in RAN Stage 2 38.300. This should not be an issue to not complete the WI. |

[Summary] There is clear majority among participating companies to support Option 2 " Support the simultaneous configuration of SBFD and NR-DC " at least from RAN2 perspective. On the other hand, it is up to RAN4 to decide whether the simultaneous configuration of SBFD and NR-DC is eventually supported in Rel-19. Also this is not an issue regarding completing the WI from RAN2 perspective. Rapporteur proposes that RAN2 to capture this understanding and send reply LS to RAN4 (CC RAN1). Companies can discuss this issue (any impacts on RAN2 specs) as well as the content of the reply LS in their contributions .

[Proposal for RRC-7] RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. Send reply LS to RAN4 (CC RAN1) with this understanding.

# 7 Residual RRC issues

Companies can list any residual RRC issues here. If justified, rapporteur can assign one open issue number and invite other companies' input on it:

| Company | Issue description and proposed solutions | Rapporteur response |
| --- | --- | --- |
| IDC01 | In RAN2@129bis meeting, RAN2 agreed that Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. In the last meeting, RAN2 discussed whether to support SBFD with RACH-based LTM cell switch using MAC CE. However, RAN2 has not been reached a conclusion and RAN2 agreement remains FFS. Hence, for clarifying the FFS agreement, RAN2 should discuss as the residual issue whether to support SBFD with RACH-based LTM cell switch or not. | Discussed |
| IDC02 | In the last RAN1#121 meeting, RAN1 agreed the SBFD with NR-DC and response LS to RAN2 (R1-2504858) was approved. Agreement - From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. - Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly. From our view, RAN1 already agreed to support SBFD with NR-DC operation (with some limitations), so RAN2 should discuss how to support SBFD with NR-DC and further RAN2 spec impact based on the RAN1 agreement. | Discussed |
| Apple | To repeat one question we raised in MAC running CR review here, since it is relevant to RRC configuration. If the SBFD RO(s) are configured for feature specific RA, but not for legacy RA, UE may go to dead end as it cannot select any RA resource. Probably it is better to make a restriction that the legacy RA resource on SBFD RO is always there if feature specific RA resource on SBFD RO is configured. I got some comment saying such principle was agreed for RedCap but not captured in spec. We are fine to make some general agreement such that “legacy RA resource on SBFD RO is always configured if feature specific RA resource on SBFD RO is configured, by proper network configuration.”. | The proponent company can discuss this issue in their meeting contribution. Rapporteurs understands this issue may not be critical. |

# 8 Conclusion:

[Proposals for easy agreement]:

[Proposal for RRC-2] Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs. [13/13]

[Proposal for RRC-3] Not to pursue the further optimization of parameter signalling of SBFD RACH configuration. [11/12]

[Proposal for RRC-6] (Only) support RACH-based LTM cell change in SBFD symbols [10/11]. Add RO type indication in LTM cell switch command MAC CE.

[Proposals for discussion]:

[Proposal for RRC-1] For the NT indicating RO type, use 1 bit signalling (as in the current RRC running CR) [8/13].

[Proposal for RRC-7] RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. Send reply LS to RAN4 (CC RAN1) with this understanding.

# 9 Reference

[1]	R2-2503379,	Impacts on the random access by the evolution of duplex operation, Huawei, HiSilicon

[2]	R2-2503386,	Random Access for SBFD,	NEC

[3]	R2-2503423,	Random Access in SBFD symbols,	 CATT

[4]	R2-2503434,	Discussion on RACH in SBFD,	 Xiaomi

[5]	R2-2503477,	Remaining issues on Random Access procedure for SBFD,	LG Electronics Inc.

[6]	R2-2503513,	Open Issues on SBFD Random Access, Sharp

[7]	R2-2503606,	Random access in SBFD, Samsung

[8]	R2-2503646,	Remaining issues of SBFD RACH procedure,	 OPPO

[9]	R2-2503840,	Random Access Operation of SBFD, Nokia Corporation

[10]	R2-2503874,	Discussion on random access procedure in SBFD, ZTE Corporation

[11]	R2-2504039,	Remaining issues on RACH procedure in SBFD,	vivo

[12]	R2-2504059,	Remaining issues for Random Access in SBFD Operation,	Sony

[13]	R2-2504169,	Remaining issues for RACH in SBFD,	 Apple

[14]	R2-2504223,	Views on random access for SBFD, Qualcomm Incorporated

[15]	R2-2504255,	SBFD RA remaining aspects,	Ericsson

[16]	R2-2504327,	Discussion on RACH aspect in SBFD,	 InterDigital, Inc.

[17]	R2-2504397,	Discussion on random access in SBFD, CMCC

[18]	R2-2504602,	Random Access in SBFD,	Lenovo

[19]	R2-2503424,	Discussion on other aspects of SBFD,	CATT

[20]	R2-2503441,	Other aspects of SBFD,	Xiaomi

[21]	R2-2503647,	Discussion on the SBFD related issues, OPPO

[22]	R2-2503875,	Discussion on L3 measurements in SBFD,  ZTE Corporation

[23]	R2-2503896,	Other aspects of SBFD,	Nokia

[24]	R2-2504040,	SBFD other aspects,	vivo

[25]	R2-2504225,	Other aspects of SBFD,	Qualcomm Incorporated

[26]	R2-2504256,	CSI-RS measurements in SBFD, Ericsson

[27]	R2-2504328,	Discussion on resource configuration aspect in SBFD, InterDigital, Inc.

[28]	R2-2504447,	Other Aspects of SBFD,	 Samsung

[29]	R2-2504640,	Other aspects about the evolution of duplex operation, Huawei, HiSilicon