3GPP TSG RAN Meeting #108										RP-25xxxx

Prague, Czech Republic, June 9-13, 2025

Status Report to TSG

Agenda item:			9.2.4

| WI / SI Name | Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_AIML_Mob |  |  |  |  |
| Unique ID | 1020084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242393 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 09/2025 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 90% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Zhongda Du |
|  | Company | OPPO |
|  | Email | duzhongda@oppo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#129bis agreements:

R2-2501822	Text proposal of TR 38.744	OPPO	pCR	Rel-19	38.744	0.0.7	FS_NR_AIML_Mob

The TR is endorsed and will be further revised post meeting

Agreements

- For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that it is generalizable for the studied cases over cell configurations with different deployment scenarios

- For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline.

- For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario than the model trained in UMa scenario when tested in UMi scenario

- No more generalization work is expected

Agreements on measurement event prediction

1.  Option 2 and Option3 outperform legacy solutions in terms of HO failure number per UE per second

2.	Capture that most companies show in their simulation results that option 3 outperforms option 2 when prediction accuracy is good enough.  There are some companies that show the opposite and indicate why.

3	For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event.

4	From the small number of companies that provided results for direct prediction: Both direct event prediction and indirect prediction methodologies demonstrate a reduction in HOF number per UE per second when compared to the legacy approach at the system level performance.

5	No further simulation results are expected for future meetings

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

- Agreements

For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

RAN2#130 agreements:

R2-2503541	Text proposal of TR 38.744	OPPO	pCR	Rel-19	38.744	0.0.8	FS_NR_AIML_Mob

The TP is endorsed and will be further updated with RAN2#130 agreements

Agreements

1	The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts.     Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s).

2	The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity. - The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case. - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured) - Upon sending the availability indication, UE indicates: - Data is available - Reason for the triggering of the availability indication (full buffer. Threshold) - Low power indication - The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) - The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - The UE releases the collected data upon transitioning to IDLE/INACTIVE - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

Agreements

- RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case

- For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement

- FFS on UE awareness and preference for NW sided model

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

2.2.2	Remaining Open issues

Open issues on spec impact:

1, UE capability procedure

2, LCM procedures related to RRM measurement prediction (L3 cell level sub case 1 and 3), measurement event prediction and L3 beam level prediction

3, Data collection for UE sided model

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#114bis agreements:

RAN4 requirements for RRM Measurement prediction

Issue 2-1-4: Need of Relative RSRP accuracy

Agreement:

RAN4 to consider both Absolute RSRP and Relative RSRP accuracy as the candidate metrics for RRM prediction use case.

Issue 2-1-1: Relative RSRP accuracy definition

Agreement:

- For intra-frequency L3-RSRP,

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - FFS if this definition will be needed.

- For inter-frequency L3-RSRP,

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].

    - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

    - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

- FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

Issue 2-1-6: Impacts on beam/cell level measurements

Agreement:

- For Rel. 19 study:

- RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

Issue 2-1-8: Inter-frequency prediction scenario

Agreement:

- RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects:

  - the side condition of frequency prediction (e.g., EPRE difference)

  - the impact of correlation coefficient between measured and predicted frequency layers

  - impact of cluster approach, e.g.,

    - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

    - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1

Agreement:

During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase.

- At least capture the definition of the two Alternatives in the TR.

RAN4 requirements for measurement event prediction

Issue 3-1-1: Prioritization of cases for event prediction

Agreement:

- RAN4 should discuss both indirect and direct cases. If any prioritization will be agreed in RAN2, RAN4 to follow RAN2 prioritization

Issue 3-1-2: Performance metric for event prediction use case

Agreement:

- Potential RRM requirement impact for event prediction use case - for indirect case:

- RAN4 to study the requirement for the predicted event triggered reporting

  - RAN4 to study the requirement for [delay and accuracy]

  - RAN4 to discuss the metric for the event prediction

- Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time],

  - Candidate options:

    - Option 1: RAN4 will define requirement for the predicted RSRPs

    - Option 2: RAN4 will not define requirement for the predicted RSRPs

Testability and interoperability

Issue 4-1-2: Prediction consistency in time domain

Agreement:

- To ensure prediction consistency in time domain, RAN4 to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories.

  - Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation

  - RAN4 to prioritize discussion on FR1.

Issue 4-1-3: Multiple cells in the testing setup

Agreement:

- RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction.

- FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

RAN4#115 agreements:

RAN4 requirements for RRM Measurement prediction

Issue 2-1: Relative RSRP accuracy definition

Agreement:

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 1 and cell 2 are on the same frequency

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

- For inter-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2),

    - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

    - the reported RSRP of cell2 can be measured or predicted.

    - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

Issue 2-5: Impacts on beam/cell level measurements

Agreement:

Consider L3 beam-level (Point E) measurement prediction depending upon the progress in RAN2.

Note: RAN4 didn’t study impact of L3 beam level measurements. However, it doesn’t preclude the discussion on L3 beam level measurements in the work item phase.

Issue 2-7: Inter-frequency prediction scenario

Agreement:

RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

RAN4 requirements for measurement event prediction

Issue 3-1: Performance metric for event prediction use case

Agreement:

In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

Issue 1-2-3: Testing Setup for intra-frequency prediction

Agreement:

- Clarify the agreement in last meeting:

  - For intra-frequency intra-cell RRM measurement prediction/event prediction, RAN4 to consider two cells in the test: serving cell and another/target cell.

    - Note: The measurement and prediction are performed on the same cell.

R4-2508419	TP on RAN4 aspects for TR 38.744

Type: pCR		For: Approval
					38.744 v0.0.4	  CR-  rev  Cat:  (Rel-19)

					Source: OPPO, Nokia

Agreement: To add the SI SR that

This TP is approved under the assumption that the updated TR (with this TP implemented) is only for information in the June RAN plenary.

In the August meeting, RAN4 will further discuss other open issues not captured in the TP.

Decision:		Approved.

2.4.2	Remaining Open issues

Testing setup in FR1/FR2, and prediction consistency in time domain.

Interoperability and generalization issue.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#129bis tdocs

R2-2501789		Way forward on study of beam-level prediciton	vivo, Ericsson, ZTE, Huawei, HiSilicon, CATT, NTT DOCOMO, CMCC, Samsung, Interdigital

R2-2501790		Simulation results of model generalization on cell configuration	vivo

R2-2501791		Simulation results for measurement event prediction and SLS	vivo

R2-2501792		Discussion on LCM and spec impact	vivo

R2-2501821		Summary of [POST129][021][AI Mob] TR update (OPPO)	OPPO.

R2-2501822		Text proposal of TR 38.744	OPPO

R2-2501823		Simulation results of measurement event prediction	OPPO

R2-2501824		Discussion on model generalization of RRM measurement prediction	OPPO

R2-2501825		Discussion on  spec impact of AI mobility	OPPO

R2-2501862		RRM measurement prediction specification impact discussion	NEC

R2-2501863		Simulation assumptions, metrics and specification impact for measurement event prediction	NEC

R2-2501905		Discussion on measurement event prediction using large language model	BJTU

R2-2501926		Simulation results of Model Generalization	CATT, Turkcell

R2-2501927		Simulation results of measurement event prediction and handover performance	CATT, Turkcell

R2-2501928		Specification impact for AIML mobility	CATT, Turkcell

R2-2501935		Discussion on generalization performance over cell configuration	Xiaomi

R2-2501936		Discussion on event prediction simulation results	Xiaomi

R2-2501937		Discussion on LCM and spec impact for AI/ML mobility	Xiaomi

R2-2501990		LCM and Spec Impact for AI/ML Mobility	Lenovo

R2-2502001		Discussion on Generalization Issues for AI/ML Mobility	Samsung

R2-2502003		Discussion on AIML mobility LCM procedure	NEC

R2-2502106		Discussion and preliminary results for direct and indirect event prediction	MediaTek Inc.

R2-2502107		Discussion on the simulation results illustration for AI mobility	MediaTek Inc.

R2-2502112		Generalization performance evaluation	MediaTek Inc.

R2-2502113		Discussion on LCM and Specificaton Impact for AI Mobility	MediaTek Inc.

R2-2502138		Simulation results for measurement event prediction and system level performance	Samsung

R2-2502139		Discussion on LCM and spec impact for AI/ML mobility	Samsung

R2-2502176		On generalization across cell configurations	Apple

R2-2502177		On LCM for AI/ML mobility	Apple

R2-2502178		SLS evaluation results for RRM measurement prediction	Apple

R2-2502179		On standards impacts other than LCM	Apple

R2-2502230		Evaluations on measurement event prediction	NTT DOCOMO, INC.

R2-2502231		Discussions on LCM and specification impact for AI/ML mobility	NTT DOCOMO, INC.

R2-2502253		Generalization of AIML models for RRM measurement prediction	Interdigital Inc.

R2-2502254		Measurement event predictions	Interdigital Inc.

R2-2502255		LCM and spec impact for AI/ML mobility	Interdigital Inc.

R2-2502281		Simulation Results for Model Generalization	Qualcomm Incorporated

R2-2502282		Simulation Results for Measurement Event Prediction	Qualcomm Incorporated

R2-2502283		LCM and specifications impact for AI/ML for mobility	Qualcomm Incorporated

R2-2502437		Discussion on LCM and spec impact for AIML mobility	Spreadtrum, UNISOC

R2-2502460		Discussion on Specification-related Aspects for AI/ML-based Mobility	Sharp

R2-2502462		Discussion on specification impacts of AIML aided mobility	Huawei, HiSilicon

R2-2502463		RRM measurement prediction model generalization evaluation for different cell configurations	Huawei, HiSilicon

R2-2502464		Further simulation results for measurement event prediction in FR2	Huawei, HiSilicon

R2-2502500		Data collection for event prediction	Sony

R2-2502627		Generalization of the AI/ML models for RRM prediction	Ericsson

R2-2502628		SLS Results for Event Predictions	Ericsson

R2-2502650		Discussion on LCM for AI for Mobility	Ericsson

R2-2502711		Discussion on LCM for RRM measurement prediction with UE sided model	CMCC

R2-2502712		Discussion on LCM for RRM measurement prediction with NW sided model	CMCC

R2-2502733		Discussion on LCM for measurement event prediction	CMCC

R2-2502822		Discussion on LCM for AIML Mobility	ASUSTeK

R2-2502879		Evaluation results for RRM measurement prediction generalization	Nokia

R2-2502880		On the measurement event prediction	Nokia

R2-2502881		Considerations on LCM for AIML Mobility	Nokia

R2-2502925		Discussion on LCM and spec impact	ETRI

R2-2502949		Discussion on specification impact for event prediction	KDDI Corporation

R2-2502968		Discussion on measurement event prediction	ZTE Corporation

R2-2502969		Discussion on model generalization  of RRM measurement prediction	ZTE Corporation

R2-2502970		Discussion on spec impact for AI mob	ZTE Corporation

RAN2#130 tdoc

R2-2503371		Discussion on functionality management	vivo

R2-2503372		Configuration and report of inference input to network sided model	vivo

R2-2503373		Discussion on data collection	vivo

R2-2503465		Discussion on AIML mobility Performance monitoring	NEC

R2-2503466		Discussion on AIML mobility Configuration and Report of Inference input to network sided model	NEC

R2-2503514			AI/ML for mobility Configuration and Functionality management	Qualcomm Incorporated

R2-2503515		Configuration and reporting for inference for network-side model	Qualcomm Incorporated

R2-2503516		Performance monitoring of AI/ML for Mobility	Qualcomm Incorporated

R2-2503534		Discussions on functionality management	NTT DOCOMO, INC.

R2-2503535		Discussions on data collection	NTT DOCOMO, INC.

R2-2503536		Discussions on performance monitoring	NTT DOCOMO, INC.

R2-2503541		Text proposal of TR 38.744	OPPO

R2-2503542		Draft text porposal to capture simulation results	OPPO

R2-2503543		Discussion on functionality management for UE sided model	OPPO

R2-2503544		Discussion on performance monitoring	OPPO

R2-2503545		Discussion on data collection	OPPO

R2-2503572		Discussion on AIML mobility functionality management	NEC

R2-2503592		Functionality management of AIML mobility	CATT, Turkcell

R2-2503593		Discussion on data collection for AIML mobility	CATT, Turkcell

R2-2503594		Discussion on performance monitoring	CATT, Turkcell

R2-2503638		Discussion on inference solution and functionality management for UE sided model	Xiaomi

R2-2503639		Discussion on  configuration and report for NW sided model	Xiaomi

R2-2503640		Discussion on data collection	Xiaomi

R2-2503656		Discussion on Inference Reporting for AI/ML-based Mobility	Sharp

R2-2503686		Functionality Management for AI/ML Mobility	Lenovo

R2-2503687		Configuration and Report of Inference Input to NW-sided model	Lenovo

R2-2503688		Support of Model Monitoring for AI/ML Mobility	Lenovo

R2-2503693		Functionality management for UE sided model for RRM measurement prediction	Interdigital Inc.

R2-2503694		Data collection for RRM measurement prediction	Interdigital Inc.

R2-2503695		Performance Monitoring for AI/ML Mobility	Interdigital Inc.

R2-2503705		On functionality of UE-sided AI/ML mobility	Apple

R2-2503706		On NW-sided AI/ML mobility	Apple

R2-2503707		On data collection for AI/ML mobility	Apple

R2-2503778		Report of [POST129bis][020][AI Mob] Sim. Results Figures (Mediatek)	MediaTek Inc.

R2-2503780		Support of Spatial Domain Prediction for AI Mobility	MediaTek Inc., vivo, CATT, Qualcomm Incorporated, Nokia, Huawei, HiSilicon, Ericsson, Samsung

R2-2503782		Discussion on Functionality Management for AI Mobility	MediaTek Inc.

R2-2503783		Discussion on Performance Monitoring for AI Mobility	MediaTek Inc.

R2-2503964		Discussion on Functionality management for AI mobility	Spreadtrum, UNISOC

R2-2503965		Discussion on Performance monitoring for AI mobility	Spreadtrum, UNISOC

R2-2503983		Discussion on functionality management for UE-side model	Samsung

R2-2503984		Discussion on configuration and report of inference input for NW-side model	Samsung

R2-2504108		Discussion on functionality management for UE sided model	Huawei, HiSilicon

R2-2504109		Discussion on NW-sided model support	Huawei, HiSilicon

R2-2504110		Discussion on performance monitoring	Huawei, HiSilicon

R2-2504131		Inference and report configuration of UE sided model	Ericsson

R2-2504132		Data collection for network sided and UE sided models	Ericsson

R2-2504284		Functionality management for AI mobility	Nokia, Nokia Shanghai Bell

R2-2504285		Discussion on performance monitoring	Nokia

R2-2504286		Considerations on data collection	Nokia

R2-2504297		Discussion on performance monitoring	Ericsson

R2-2504361		Discussion on Performance Monitoring for AI/ML Mobility	Samsung

R2-2504380		Discussion on data collection for AI Mob	CMCC

R2-2504381		Discussion on performance monitoring for AI Mob	CMCC

R2-2504441		Discussion on functionality management for UE sided model	CMCC

R2-2504459		Discussion on functionality management for AI mob	ZTE Corporation

R2-2504460		Discussion on configuration and report of inference input to network sided model	ZTE Corporation

R2-2504461		Discussion on data collection for AI mob	ZTE Corporation

R2-2504492		Discussion on functionality management for AIML Mobility	ASUSTeK

R2-2504552		Discussion on measurement report and configuration for UE sided model	KDDI Corporation

R2-2504575		Performance monitoring	TURKCELL

RAN4#114bis tdocs

R4-2503300		Discussion on study of testability and interoperability for AIML mobility	Xiaomi

R4-2503308		Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility	Xiaomi

R4-2503309		Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility	Xiaomi

R4-2503350		Discussion on general aspects for AI mobility	OPPO

R4-2503351		Study of impacts on RAN4 requirements for RRM measurement prediction	OPPO

R4-2503352		Study of impacts on RAN4 requirements for measurement event prediction	OPPO

R4-2503353		Study of testability and interoperability for AI mobility	OPPO

R4-2503411		Discussion on impacts of AIML RRM measurement prediction on RRM requirements	CATT

R4-2503412		Discussion on impacts of AIML measurement event prediction on RRM requirements	CATT

R4-2503413		Discussion on testability and interoperability issues for AIML mobility	CATT

R4-2503520		On Testability and Interoperability Issues for AIML Mobility in NR	Apple

R4-2503521		General aspects on AI/ML for mobility in NR	Apple

R4-2503522		Study of RAN4 impacts for RRM measurement prediction for AIML Mobility in NR	Apple

R4-2503523		Study of RAN4 impacts for measurement event prediction for AIML Mobility in NR	Apple

R4-2503582		Discussion of RAN4 impacts for RRM measurement prediction	LG Electronics Inc.

R4-2503607		Discussion on RAN4 impacts for RRM measurement prediction	MediaTek Inc.

R4-2503608		Discussion on RAN4 impacts for measurement event prediction	MediaTek Inc.

R4-2503609		Discussion on testability and interoperability of AI mobility	MediaTek Inc.

R4-2503620		Topic summary for [114bis][210] FS_NR_AIML_Mob	Moderator (Nokia)

R4-2503645		TP on RAN4 aspects for TR 38.744	OPPO

R4-2503705		Study of impacts on RAN4 requirements for AIML mobility in NR	Tejas Network Limited

R4-2503774		Discussion on general part for AI/ML for mobility	CMCC

R4-2503775		Discussion on RAN4 impacts for RRM measurement prediction	CMCC

R4-2503776		Discussion on RAN4 impacts for measurement event prediction	CMCC

R4-2503777		Discussion on testability and interoperability for AI/ML for mobility	CMCC

R4-2503787		Discussions on RRM measurement impacts of AIML for mobility	NTT DOCOMO, INC.

R4-2503850		Discussion on General Aspects of AIML Mobility	ZTE Corporation, Sanechips

R4-2503851		Discussion impacts on requirements of AI/ML mobility	ZTE Corporation, Sanechips

R4-2503852		Discussion on impacts for measurement event prediction	ZTE Corporation, Sanechips

R4-2503853		Discussion on the interoperability and testability aspects	ZTE Corporation, Sanechips

R4-2503886		Discussion on genereal aspects in AIML mobility	Huawei, HiSilicon

R4-2503887		Discussion on impacts for RRM measurement prediction	Huawei, HiSilicon

R4-2503888		Discussion on impacts for measurement event prediction	Huawei, HiSilicon

R4-2503889		Discussion on testability and interoperability issues in AIML mobility	Huawei, HiSilicon

R4-2503928		General discussion on AI/ML for mobility	Ericsson

R4-2503929		On requirements for RRM measurement prediction for AI/ML based mobility	Ericsson

R4-2503930		On requirements for Event prediction for AI/ML based mobility	Ericsson

R4-2503931		On testing of AI/ML based mobility	Ericsson

R4-2504046		Discussion on general aspects for AI mobility	vivo

R4-2504047		Discussion on RAN4 impacts for RRM measurement prediction	vivo

R4-2504048		Discussion on RAN4 impacts for measurement event prediction	vivo

R4-2504049		Discussion on testability and interoperability for AI mobility	vivo

R4-2504232		Study of RAN4 impacts for RRM measurement prediction	Nokia

R4-2504233		Study of RAN4 impacts for measurement event prediction	Nokia

R4-2504234		Study of testability and interoperability on AI/ML Mobility	Nokia

R4-2504286		General discussion on AI mobility regarding testability and interoperability	Samsung

R4-2504287		General discussion on AI mobility regarding RAN4 requirements impact	Samsung

R4-2504288		Discussion on AI mobility general and scope	Samsung

R4-2504524		Impact of AI based mobility on RRM measurement prediction	Qualcomm Incorporated

R4-2504525		Impact of AI based mobility on RRM event prediction	Qualcomm Incorporated

R4-2504526		Testability and data consistency	Qualcomm Incorporated

R4-2504901		WF on RRM requirements for FS_NR_AIML_Mob	Nokia

R4-2504954		TP on RAN4 aspects for TR 38.744	OPPO

R4-2504963		TP on skeleton of RAN4 aspects for TR 38.744	OPPO

RAN4#115 tdocs

R4-2505564		Topic summary for [115][215] FS_NR_AIML_Mob_Part1	Moderator (Nokia)

R4-2505565		Topic summary for [115][216] FS_NR_AIML_Mob_Part2	Moderator (OPPO)

R4-2505694		Discussion on general aspects for AI mobility	OPPO

R4-2505695		TP on RAN4 aspects for TR 38.744	OPPO, Nokia

R4-2505696		Study of impacts on RAN4 requirements for RRM measurement prediction	OPPO

R4-2505697		Study of impacts on RAN4 requirements for measurement event prediction	OPPO

R4-2505698		Study of testability and interoperability for AI mobility	OPPO

R4-2505766		Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility	Xiaomi

R4-2505767		Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility	Xiaomi

R4-2505790		Discussion on study of testability and interoperability for AIML mobility	Xiaomi

R4-2505948		On Testability and Interoperability Issues for AIML Mobility in NR	Apple

R4-2505949		General aspects on AI/ML for mobility in NR	Apple

R4-2505950		Study of RAN4 impacts for RRM measurement prediction for AIML Mobility in NR	Apple

R4-2505951		Study of RAN4 impacts for measurement event prediction for AIML Mobility in NR	Apple

R4-2505985		Discussion on impacts of AIML RRM measurement prediction on RRM requirements	CATT

R4-2505986		Discussion on impacts of AIML measurement event prediction on RRM requirements	CATT

R4-2505987		Discussion on testability and interoperability issues for AIML mobility	CATT

R4-2506171		Study of RAN4 impacts for RRM measurement prediction for AIML mobility in NR	Tejas Network Limited

R4-2506306		Discussion of RAN4 impacts for RRM measurement prediction	LG Electronics Inc.

R4-2506507		Discussion on RAN4 impacts for RRM measurement prediction	MediaTek Inc.

R4-2506508		Discussion on RAN4 impacts for measurement event prediction	MediaTek Inc.

R4-2506509		Discussion on testability and interoperability of AI mobility	MediaTek Inc.

R4-2506547		Discussion on general aspects for AI mobility	vivo

R4-2506548		Discussion on RAN4 impacts for RRM measurement prediction	vivo

R4-2506549		Discussion on RAN4 impacts for measurement event prediction	vivo

R4-2506550		Discussion on testability and interoperability for AI mobility	vivo

R4-2506562		Discussion on the interoperability and testability aspects	ZTE Corporation, Sanechips

R4-2506563		Discussion impacts on requirements of AI/ML mobility	ZTE Corporation, Sanechips

R4-2506564		Discussion on General Aspects of AIML Mobility	ZTE Corporation, Sanechips

R4-2506565		Discussion on impacts for measurement event prediction	ZTE Corporation, Sanechips

R4-2506610		Discussion on general part for AI/ML for mobility	CMCC

R4-2506611		Discussion on RAN4 impacts for RRM measurement prediction	CMCC

R4-2506612		Discussion on RAN4 impacts for measurement event prediction	CMCC

R4-2506613		Discussion on testability and interoperability for AI/ML for mobility	CMCC

R4-2506735		Discussion on genereal aspects in AIML mobility	Huawei, HiSilicon

R4-2506736		Discussion on impacts for RRM measurement prediction	Huawei, HiSilicon

R4-2506737		Discussion on impacts for measurement event prediction	Huawei, HiSilicon

R4-2506738		Discussion on testability and interoperability issues in AIML mobility	Huawei, HiSilicon

R4-2507123		Discussions on requirements for RRM measurement prediction on AIML mobility	NTT DOCOMO, INC.

R4-2507124		Discussions on requirements for measurement event prediction on AIML mobility	NTT DOCOMO, INC.

R4-2507255		General discussion on AI mobility regarding testability and interoperability	Samsung

R4-2507432		On RAN4 Impacts for RRM measurement prediction in AIML Mobility	Nokia

R4-2507433		On RAN4 Impacts for Measurement Event Prediction in AIML Mobility	Nokia

R4-2507534		On General aspects in AIML Mobility	Nokia

R4-2507535		Testability and Interoperability Issues for AIML Mobility	Nokia

R4-2507665		TP for TR38.744	Ericsson

R4-2507666		General discussion on AI/ML for mobility	Ericsson

R4-2507667		On requirements for RRM measurement prediction for AI/ML based mobility	Ericsson

R4-2507668		On requirements for Event prediction for AIML based mobility	Ericsson

R4-2507669		On testing of AI/ML based mobility	Ericsson

R4-2507801		Impact of AI based mobility on RRM measurement prediction	Qualcomm Incorporated

R4-2507802		Impact of AI based mobility on RRM event prediction	Qualcomm Incorporated

R4-2507803		Testability and data consistency	Qualcomm Incorporated

R4-2508267		Ad-hoc minutes for FS_NR_AIML_Mob	Nokia

R4-2508268		WF on FS_NR_AIML_Mob_Part1	Nokia

R4-2508269		WF on FS_NR_AIML_Mob_Part2	OPPO

R4-2508419		TP on RAN4 aspects for TR 38.744	OPPO, Nokia