| 3GPP TS 38.391 V0.1.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Ambient IoT Medium Access Control Protocol specification (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	General	7

4.1	Introduction	7

4.2	A-IoT MAC architecture	7

4.3	Services	7

4.3.1	Services provided to upper layers	7

4.3.2	Services expected from physical layer	7

4.4	Functions	7

5	A-IoT MAC procedures	8

5.1	General	8

5.2	A-IoT paging	8

5.3	A-IoT access procedure	9

5.3.1	Contention-Based Random Access procedure	9

5.3.1.1	Selection of access occasion for D2R transmission of Access Random ID message	9

5.3.1.2	Transmission of Access Random ID message	10

5.3.1.3	Reception of Random ID Response message	10

5.3.2	Contention-Free Access procedure	10

5.4	A-IoT upper layer data procedure	11

5.4.1	General	11

5.4.2	D2R message transmission	11

5.4.3	R2D message reception	11

5.4.4	D2R segmentation	12

5.5	Failure detection	12

5.5.1	General	12

5.5.2	Detection of data transmission failure	13

5.5.3	Detection of CBRA failure	13

6	Protocol Data Units, formats and parameters	13

6.1	Protocol Data Units	13

6.1.1	General	13

6.2	A-IoT MAC messages	14

6.2.1	R2D messages	14

6.2.1.1	A-IoT Paging message	14

6.2.1.2	Access Trigger message	16

6.2.1.3	Random ID Response message (Msg2 in CBRA)	16

6.2.1.4	R2D Upper Layer Data Transfer message	17

6.2.1.5	NACK Feedback message	19

6.2.1.6	D2R Scheduling Info field description	19

6.2.2	D2R messages	23

6.2.2.1	Access Random ID message (Msg1 in CBRA)	23

6.2.2.2	D2R Upper Layer Data Transfer message	23

Annex <X> (informative): Change history	25

Annex	25

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the Medium Access Control (MAC) protocol of Ambient IoT.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.291: "Ambient IoT Physical layer".

[3]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[4]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[5]	3GPP TS 23.003: "Numbering, addressing and identification".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Device: A device that supports A-IoT radio interface towards reader, as defined in TS 38.300 [3].

Reader: A reader providing A-IoT protocol terminations towards the A-IoT device, as defined in TS 38.300 [3].

Access occasion: A time-frequency resource for device(s) to transmit Msg1 (i.e., the Access Random ID message) during a CBRA procedure.

AS ID: The AS layer identifier to address the specific device for R2D reception and D2R scheduling.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-IoT	Ambient IoT

CBRA	Contention-Based Random Access

CFA	Contention-Free Access

D2R	Device to reader

IoT	Internet of Things

PDRCH	Physical D2R channel

PRDCH	Physical R2D channel

R2D	Reader to device

TrCH	Transport Channel

# 4	General

## 4.1	Introduction

This clause describes the A-IoT MAC architecture and the A-IoT MAC entity of the device from a functional point of view.

## 4.2	A-IoT MAC architecture

Figure 4.2-1 illustrates a model of the A-IoT MAC entity; and it does not restrict implementations.

The A-IoT MAC entity of the device handles the data received from R2D transport channel or to be transmitted via D2R transport channel, as specified in TS 38.291 [2].

Figure 4.2-1: A-IoT MAC structure overview

## 4.3	Services

### 4.3.1	Services provided to upper layers

The A-IoT MAC layer provides the following services to upper layers:

-	data transfer;

### 4.3.2	Services expected from physical layer

The A-IoT MAC layer expects the following services from the physical layer:

-	data transfer;

## 4.4	Functions

The A-IoT MAC layer supports the following A-IoT MAC functions:

-	constructing MAC PDUs to be mapped onto transport blocks (TB) to be delivered to the physical layer on D2R transport channel;

-	receiving MAC PDUs from transport blocks (TB) delivered from the physical layer on R2D transport channel;

-	message type determination;

-	paging;

-	radio resource selection;

-	access;

-	transfer of upper layer data;

-	D2R segmentation;

-	failure detection.

# 5	A-IoT MAC procedures

## 5.1	General

The clause describes the A-IoT MAC procedures.

When the device is powered on, the device starts monitoring PRDCH for an R2D message, as specified in TS 38.291 [2], in order to perform the corresponding A-IoT MAC procedures.

## 5.2	A-IoT paging

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include the Paging ID field to select a specific device or a group of devices, or may not include Paging ID field to select all devices.

The device always monitors for the A-IoT Paging message, and determines whether the device is selected to initiate the access procedure.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

1>	if the Access Type field in the A-IoT Paging message indicates CBRA:

2>	if the device has no stored Transaction ID; or

2>	if the value of the Transaction ID field is different from the stored Transaction ID; or

2>	if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5:

3>	release the stored AS ID if any;

3>	store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID;

3>	if the Paging ID Presence Indication field indicates Paging ID field is absent:

4>	consider the device is selected and indicate to the upper layers;

3>	else:

4>	forward the value of the Paging ID field to the upper layers;

4>	if the upper layers indicate that the Paging ID is matched:

5>	consider the device is selected;

3>	if the device is selected:

4>	initiate Contention-Based Random Access procedure as specified in clause 5.3.1;

1>	else (i.e., the Access Type field in the A-IoT Paging message indicates CFA):

2>	release the stored AS ID if any;

2>	release the stored Transaction ID, if any

2>	forward the value of the Paging ID field to the upper layers;

2>	if the upper layers indicate that this Paging ID is matched:

3>	consider the device is selected;

3>	initiate Contention-Free Access procedure as specified in clause 5.3.2.

## 5.3	A-IoT access procedure

### 5.3.1	Contention-Based Random Access procedure

#### 5.3.1.1	Selection of access occasion for D2R transmission of Access Random ID message

If Contention-Based Random Access (CBRA) procedure is initiated due to a reception of A-IoT Paging message according to clause 5.2, the device shall randomly select an access occasion for D2R transmission of Access Random ID message based on A-IoT Paging message or Access Trigger message. If needed, the device monitors for Access Trigger message until it has received a A-IoT Paging message.

The A-IoT MAC entity shall:

1>	apply the D2R Scheduling Info received in the A-IoT Paging message;

1>	generate a random number 'i' in the range: 0 ≤ i ≤ n-1, where n is the number of access occasions configured in A-IoT Paging message;

- select an access occasion corresponding to the random number i;

The access occasion can be selected according to a counter-based count-down behavior, which starts with the A-IoT Paging message, and continues with subsequent Access trigger message(s), until Access Random ID message is transmitted or next A-IoT Paging message is received. For this, the A-IoT MAC entity should:

1>	set the ACCESS_OCCASION_COUNTER to 'i';

1>	if ACCESS_OCCASION_COUNTER < m, where m equals to X*NSFS (where X and NSFS are defined in clause 6.2.1.6):

2>	select the (ACCESS_OCCASION_COUNTER+1)th access occasion from the m access occasion(s) triggered by the A-IoT Paging message;

2>	initiate the transmission of Access Random ID message, as specified in clause 5.3.1.2;

1>	else (i.e. ACCESS_OCCASION_COUNTER >= m):

2>	perform the following procedure upon reception of each Access Trigger message until Access Random ID message is transmitted:

3>	decrement ACCESS_OCCASION_COUNTER by m;

3>	if ACCESS_OCCASION_COUNTER < m:

4>	select the (ACCESS_OCCASION_COUNTER+1)th access occasion from the m access occasion(s) triggered by this Access Trigger message;

4>	initiate the transmission of Access Random ID message, as specified in clause 5.3.1.2;

3>	else (i.e., ACCESS_OCCASION_COUNTER >= m):

4>	monitor for next Access Trigger message;

NOTE:	The count-down behaviour defined above does not preclude other device implementation alternatives of random selection of access occasion.

#### 5.3.1.2	Transmission of Access Random ID message

The A-IoT MAC entity shall:

1>	generate a 16-bit random number 'j' in the range: 0 ≤ j < 216;

1>	set the Random ID field to the ‘j’ in the Access Random ID message;

1>	instruct the physical layer to transmit the Access Random ID message using the selected access occasion as specified in clause 5.3.1.1, and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6.

#### 5.3.1.3	Reception of Random ID Response message

Once the Access Random ID message is transmitted, the device monitors for Random ID Response message until it has received  K Access Trigger message (K is configured in the A-IoT Paging message or one A-IoT Paging message (i.e., the device does not process the Random ID Response message after that).

Upon reception of Random ID Response message, the A-IoT MAC entity shall:

1>	if the device has no stored AS ID (i.e., initial reception of Random ID Response message):

2>	for each ID entry in Random ID Response message:

3>	if the value indicated by Echoed Random ID field is identical to the value of the Random ID field in the transmitted Access Random ID message; and

3>	if present (i.e., Frequency Index Present Indication is set to 1), the Frequency Index field matches the value of the small frequency shift factor used for the transmission of Access Random ID message:

4>	consider this CBRA procedure is successful;

4>	if the Assigned AS ID field corresponding to the Echoed Random ID field is included (i.e., AS ID Present Indication field is set to 1):

5>	set AS ID to the value indicated by the Assigned AS ID field and store the AS ID;

4>	else:

5>	set AS ID to the value indicated by the Echoed Random ID field and store the AS ID;

4>	initiate the D2R message transmission as specified in clause 5.4.1, upon which the procedure of processing this Random ID Response message ends;

1>	else:

2>	for each ID entry in the Random ID Response message:

3>	if the Assigned AS ID field corresponding to the Echoed Random ID field is included, and the value indicated by Assigned AS ID field is identical to the stored AS ID; or

3>	if the Assigned AS ID field corresponding to the Echoed Random ID field is not included, and the value indicated by Echoed Random ID field is identical to the stored AS ID:

4>	initiate the D2R message transmission as specified in clause 5.4.1, upon which the procedure of processing this Random ID Response message ends.

### 5.3.2	Contention-Free Access procedure

If Contention-Free Access (CFA) procedure is initiated according to clause 5.2, the A-IoT MAC entity shall:

1>	initiate the D2R message transmission as specified in clause 5.4.1.

## 5.4	A-IoT upper layer data procedure

### 5.4.1	General

The purpose of this procedure is for a device to transmit or receive upper layer data. The segmentation can be triggered for D2R message transmission.

### 5.4.2	D2R message transmission

Upon initiation of the procedure corresponding to the A-IoT access procedure or reception of a R2D Upper Layer Data Transfer message which contains either an upper layer data or a Received Data Size field set to 0, the A-IoT MAC entity shall:

1>	apply the D2R Scheduling Info, received in the A-IoT Paging message with Access Type set to CFA or in the Random ID Response message or in the R2D Upper Layer Data Transfer message containing either the Data SDU field or the Received Data Size field set to 0;

1>	if upper layer data is available to be transmitted:

2>	if the size of the resulting MAC PDU including the total upper layer data is smaller than or equal to the resource size given by the D2R TBS in the D2R Scheduling Info:

3>	generate the D2R Upper Layer Data Transfer message, as follows:

4>	set the More Data Indication field to value 0;

4>	include SDU Length field and Data SDU field;

4>	if the size of the resulting MAC PDU including the total upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

5>	include the MAC Padding field;

3>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6;

2>	else (the size of the resulting MAC PDU including the total upper layer data is larger than the resource size given by the D2R TBS in the D2R Scheduling Info):

3>	the upper layer data SDU is to be segmented according to clause 5.4.4;

1>	else (i.e., upper layer data is not available to be transmitted):

2>	generate the D2R Upper Layer Data Transfer message, as follows:

3>	set the More Data Indication field to 0;

3>	set the SDU Length field to 0;

3>	if the size of the resulting MAC PDU including no upper layer data is smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

4>	include the MAC Padding field;

2>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer, as specified in clause 6.2.1.6.

NOTE:	It is up to reader’s implementation to avoid segmentation for the D2R Upper Layer Data Transfer message for inventory response.

### 5.4.3	R2D message reception

Once a D2R Upper Layer Data Transfer message has been transmitted, the device monitors for R2D Upper Layer Data Transfer message.

Upon a R2D Upper Layer Data Transfer message is received, the A-IoT MAC entity shall:

1>	if the device has a stored AS ID and the R2D Upper Layer Data Transfer message is addressed to the device (i.e., the value of AS ID field is identical to the stored AS ID):

2>	if the Choice Indication field indicates that the Data SDU field is included:

3>	forward the upper layer data SDU in the Data SDU field to upper layers;

3>	initiate the following D2R message transmission, as specified in clause 5.4.2;

2>	else if the Choice Indication field indicates that the Received Data Size field is included:

3>	if the Received Data Size field is set to 0:

4>	perform the D2R message transmission procedure as specified in clause 5.4.2

3>		else:

4>	perform the D2R segmentation procedure using this information as specified in clause 5.4.4;

1>	else if the device has no stored AS ID, and if CFA procedure has been performed in the current procedure:

2>	if the Choice Indication field indicates that the Data SDU field is included:

3>	set AS ID to the value indicated by the AS ID field and store the AS ID;

3>	forward the upper layer data SDU in the Data SDU field to upper layers;

3>	initiate the following D2R message transmission, as specified in clause 5.4.2.

### 5.4.4	D2R segmentation

Upon initiation of this D2R segmentation procedure according to clause 5.4.2, or upon reception of R2D Upper Layer Data Transfer message containing the Received Data Size field not set to 0, as specified in clause 5.4.3, after this segmentation procedure is initiated, the A-IoT MAC entity shall:

1>	apply the received D2R Scheduling Info, received in the R2D Upper Layer Data Transfer message containing the Received Data Size field;

1>	generate the D2R Upper Layer Data Transfer message for this segment according to resource size given by the D2R TBS in the D2R Scheduling Info, as follows:

2>	include the SDU Length field and set the Data SDU field to include the segment which starts from the (x+1)th byte of the original upper layer data SDU, where x=0 if the Received Data Size field is not included, otherwise x equals to the value indicated by the Received Data Size field;

2>	if the segment is the last segment of the original upper layer data SDU:

3>	set More Data Indication field to value 0;

3>	if the size of the resulting MAC PDU including the segment is expected to be smaller than the resource size given by the D2R TBS in the D2R Scheduling Info:

4>	include the MAC Padding field;

2>	else:

3>	set More Data Indication field to value 1;

1>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message and indicate the L1 parameters to the physical layer as specified in clause 6.2.1.6.

## 5.5	Failure detection

### 5.5.1	General

The purpose of this procedure is to determine the failure cases.

### 5.5.2	Detection of data transmission failure

Once the device transmitted the first D2R Upper Layer Data Transfer message after A-IoT access procedure, the A-IoT MAC entity monitors for NACK Feedback message until the device receives a A-IoT Paging message or R2D Upper Layer Data Transfer message addressed to the device (i.e., the device does not process NACK Feedback message after that).

Upon reception of NACK Feedback message, the A-IoT MAC entity shall:

1>	for each AS ID entry in the NACK Feedback message:

2>	if the value indicated by the AS ID field is identical to the stored AS ID:

3>	release the stored AS ID;

3>	consider that the current procedure associated with the stored Transaction ID failed, upon which this procedure of processing NACK Feedback message ends.

### 5.5.3	Detection of CBRA failure

The A-IoT MAC entity shall:

1>	if CBRA procedure has been initiated as specified in clause 5.3.1:

2>	upon reception of A-IoT paging message:

3>	if CBRA procedure has not been considered as successful as specified in clause 5.3.1.3:

4>	consider that the current procedure associated with the stored Transaction ID failed.

# 6	Protocol Data Units, formats and parameters

## 6.1	Protocol Data Units

### 6.1.1	General

An A-IoT MAC Protocol Data Unit (PDU) is the data unit format in which the A-IoT MAC message is encapsulated for transmission through the lower layer of the A-IoT protocol stack. An A-IoT MAC PDU is a bit string that is byte aligned (i.e., multiple of 8 bits) in length, except the R2D Access Trigger message. The contents of each A-IoT MAC message are specified in clause 6.2 using tables to specify the fields in the message. In the tables, the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within a MAC PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

An A-IoT MAC SDU is a bit string that is byte aligned (i.e., multiple of 8 bits) in length. A MAC SDU is included into a MAC PDU from the first bit onward.

A-IoT MAC Padding is placed at the end of the A-IoT MAC PDU if present. Presence and length of padding is determined based on TBS corresponding to the A-IoT MAC PDU.

The R2D message type represents the set of A-IoT MAC messages that are sent from the reader to the device on the R2D transport channel. The R2D message names and the values of R2D message type are specified in Table 6.1-1.

Table 6.1-1: R2D Message Type

| R2D Message Type value | R2D message name |
| --- | --- |
| 000 | Reserved |
| 001 | A-IoT Paging message |
| 010 | Access Trigger message |
| 011 | Random ID Response message |
| 100 | R2D Upper Layer Data Transfer message |
| 101 | NACK Feedback message |
| 110 | Reserved |
| 111 | Reserved |

The D2R message type is the set of A-IoT MAC messages that are sent from the device to the reader on the D2R transport channel. The D2R message names and the values of D2R message type are listed in Table 6.1-2.

Table 6.1-2: D2R Message type

| D2R Message Type value | D2R message name |
| --- | --- |
| N/A | Access Random ID message |
| 00 | D2R Upper Layer Data Transfer message |
| 01 | Reserved |
| 10 | Reserved |
| 11 | Reserved |

.

## 6.2	A-IoT MAC messages

### 6.2.1	R2D messages

#### 6.2.1.1	A-IoT Paging message

Figure 6.2.1.1-1 and 6.2.1.1-2 show the formats of the A-IoT Paging message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R2D TBS Info: This field indicates the TBS of this R2D message. The length of the field is 7 bits.

-	Access Type (AT): This field indicates CBRA (when set to 1) or CFA (when set to 0). The length of the field is 1 bit.

For CBRA, the following fields are further included:

-	Transaction ID: This field associates an inventory procedure or command procedure as specified in TS 38.300 [3]. The length of the field is 6 bits.

-	Paging ID Presence Indication (PIPI): This field indicates whether Paging ID and Length of Paging ID are present (when set to 1) or absent (when set to 0). The length of the field is 1 bit.

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bit when Paging ID field is present. The length of the field is 8 bits.

-	Paging ID: This field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]).

-	Number of Access Occasions: This field indicates the number of access occasions. The length of the field is 4 bits. The value 0 (i.e., 0000) indicates the number of access occasions is 20. The value 1 (i.e., 0001) indicates the number of access occasions is 21. The value 2 (i.e., 0010) indicates the number of access occasions is 22. And so on. The maximum number of access occasions is 215 when this field is set to 15 (i.e., 1111).

D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 25 bits.

-	K: This field indicates that the value K is 1 (when set to 0) or 4 (when set to 1) used for determining monitor window for Random ID Response message. The length of the field is 1 bit.

-	Fill Bits: This field is of variable size and is optionally present. It can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the device shall ignore the values of this field.

For CFA, the following fields are further included:

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bit. The length of the field is 8 bits.

-	Paging ID: This field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]).

D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 24 bits.

-	Fill Bits: This field is of variable size, and can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the device shall ignore the values of this field.

Figure 6.2.1.1-1: MAC PDU of A-IoT Paging message indicating CBRA

Figure 6.2.1.1-2: MAC PDU of A-IoT Paging message indicating CFA

#### 6.2.1.2	Access Trigger message

Figure 6.2.1.2-1 shows the format of the Access Trigger message.

The field in this message is defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

Figure 6.2.1.2-1: MAC PDU of Access Trigger message

#### 6.2.1.3	Random ID Response message (Msg2 in CBRA)

Figure 6.2.1.3-1 shows the format of the Random ID Response message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R2D TBS Info: This field indicates the TBS of this R2D message. The length of the field is 7 bits.

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 24 bits.

-	Frequency Index Present Indication (FIPI): This field indicates whether the Frequency Index field is present (when set to 1) in each ID entry or not (when set to 0). This field applies to all ID entry(ies). The length of the field is 1 bit.

-	This message includes an ID entry list, which consists of at most 8 ID entries with the following fields included in each ID entry:

-	Frequency Index: This field indicates the small frequency shift factor value of the access occasion associated to the Echoed Random ID. For instance, the first codepoint (i.e., 000) presents the first value in {1, 2, 4, 8, 16, 32, 64, 128}, the second codepoint (i.e., 001) presents the second value in {1, 2, 4, 8, 16, 32, 64, 128}, and so on. The length of the field is 3 bits.

-	Echoed Random ID: The length of the field is 16 bits.

-	AS ID Presence Indication (AIPI): This field indicates whether a AS ID is assigned (when set to 1) for the corresponding Echoed Random ID or not (when set to 0). The length of the field is 1 bit.

-	Assigned AS ID: This field provides the value of assigned AS ID which is 16 bits, when AS ID Present Indication field is set to 1.

-	Fill Bits: This field is of variable size and is optionally present. It can be used to pad for byte alignment (1-7 bits).

Figure 6.2.1.3-1: MAC PDU of Random ID Response message

#### 6.2.1.4	R2D Upper Layer Data Transfer message

Figure 6.2.1.4-1 and Figure 6.2.1.4-2 show the formats of the R2D Upper Layer Data Transfer message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R2D TBS: This field indicates the TBS of this R2D message. The length of the field is 7 bits.

-	AS ID: This field provides/indicates the value of AS ID. The length of the field is 16 bits.

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 19 bits.

-	Choice Indication (CI): This field indicates either Data SDU field is included (when set to 1) or Received Data Size field is included (when set to 0). The length of the field is 1 bit.

When Data SDU field is included:

-	R1-R2: The 2 bits are set to 0, and the device ignores the value.

-	Data SDU: This field contains the upper layer data. This field is of variable size.

When Received Data Size field is included:

-	R1-R3: The 3 bits are set to 0, and the device ignores the value.

-	Received Data Size: This field is to indicate the number of bytes successfully received by the reader. This field is 7 bits.

Figure 6.2.1.4-1: MAC PDU of R2D Upper Layer Data Transfer message containing Data SDU

Figure 6.2.1.4-2: MAC PDU of R2D Upper Layer Data Transfer message containing Received Data Size

#### 6.2.1.5	NACK Feedback message

Figure 6.2.1.5-1 shows the format of the NACK Feedback message.

The field in this message is defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R2D TBS: This field indicates the TBS of this R2D message. The length of the field is 7 bits.

-	R1-R6: The 6 bits are set to 0, and the device ignores the value.

-	This message include a AS ID entry list which consists of one or multiple AS ID entries with the following field included in each AS ID entry:

-	AS ID: This field indicates transmission failure for the device identified by this AS ID. The length of the field is 16 bits.

Figure 6.2.1.5-1: MAC PDU of NACK Feedback message

#### 6.2.1.6	D2R Scheduling Info field description

This clause defines the child fields contained in D2R Scheduling Info field. See the Table 6.2.1.6-1.

For the child fields except Frequency Resource Indication, the set of valid values is given in the table and configured in the form of an enumeration type. A field with L bits can provide 2L codepoints. For instance, if L=2, the first codepoint (i.e., 00) represents the first value within the value range. The second codepoint (i.e., 01) represents the second value within the value range. And so on. If the number V of valid values in the value range is less than 2L, the codepoints after the (V+1)th codepoint are not to be used in this release.

The Time Resource Indication field is only present in the D2R Scheduling Info field contained in A-IoT Paging message indicating CBRA. The D2R TBS field is absent in the D2R Scheduling Info field contained in A-IoT Paging message indicating CBRA, and present in the D2R Scheduling Info field contained in A-IoT Paging message indicating CFA, Random ID Response message, and R2D Upper Layer Data Transfer message. All other fields are present in the D2R Scheduling Info field contained in A-IoT Paging message, Random ID Response message, and R2D Upper Layer Data Transfer message.

After applying the D2R Scheduling Info field, the MAC entity derives the parameters (listed in the last column in Table 6.2.1.6-1) and indicates them to the physical layer. The MAC entity also derives some configurations to be used in MAC, e.g., X, NSFS, D2R TBS.

Table 6.2.1.6-1: Child fields of D2R Scheduling Info field

| Field name | Length | Value range | Description | Indicated L1 parameter in TS 38.291 [2] |
| --- | --- | --- | --- | --- |
| Time Resource Indication | 1 bit | {1, 2} | The number of time domain resource of access occasions triggered by A-IoT Paging message or one Access Trigger message, i.e., X. | N/A |
| Bit Duration | 3 bits | {2, 1, 1/2, 1/4, 1/8, 1/16, 1/32, 1/96} ×τ, where τ=2×106/15000 | The duration in microseconds of each D2R bit. | TbitD2R |
| Frequency Resource IndicationBroadcast | 8 bits | An 8-bit bitmap. The values of small frequency shift factor are {1, 2, 4, 8, 16, 32, 64, 128}. In the bitmap, the first/leftmost bit of the bitmap corresponds to the first value of small frequency shift factor, the second bit corresponds to the second value of small frequency shift factor, and so on. For each bit, value 0 indicates that the corresponding value is not allowed, while value 1 indicates that the corresponding value can be used. | This field indicates: - the set of NSFS potential small frequency shift factors when present in A-IoT Paging message for CBRA. Each small frequency shift factor corresponding to X access occasion(s). NSFS is the number of frequency domain resource of access occasions triggered by A-IoT Paging message or one Access Trigger message, i.e., the number of bits set to value 1. Or - one value of small frequency shift factor when present in A-IoT Paging message for CFA. Or - one or multiple values of small frequency shift factor when present in Random ID Response message. A device determines its small frequency shift factor value for the following D2R transmission based on its order of Echoed Random ID field in the Random ID Response message. Regarding different Bit Duration, only the following values can be indicated to 1 in the bitmap: - {1, 2, 4, 8, 16, 32, 64, 128}, when Bit Duration is configured to 2τμs; - {1, 2, 4, 8, 16, 32, 64}, when Bit Duration is configured to τμs; - {1, 2, 4, 8, 16, 32}, when Bit Duration is configured to τ/2 μs; - {1, 2, 4, 8, 16}, when Bit Duration is configured to τ/4 μs; - {1, 2, 4, 8}, when Bit Duration is configured to τ/8 μs; - {1, 2, 4}, when Bit Duration is configured to τ/16 μs; - {1, 2}, when Bit Duration is configured to τ/32 μs; - {1}, when Bit Duration is configured to τ/96 μs. | RSFS associated to the selected access occasion or configured resource for D2R transmission |
| Frequency Resource IndicationUnicast | 3 bits | {1, 2, 4, 8, 16, 32, 64, 128} | This field indicates a value of small frequency shift factor when present in R2D Upper Layer Data Transfer message. | RSFS associated to the configured resource for D2R transmission |
| Block Repetition number | 1 bit | {1, 2} | The block repetition number. | Rblock |
| Channel Coding Indicator | 1 bit | {FEC, no FEC} | The channel coding indicator. | Rcode |
| Interval Bits | 2 bits | {S*48, S*96, S*168, S*240}, S is a scale factor, and equals to: - 1, when Bit Duration is configured to 2τ μs; - 2, when Bit Duration is configured to τ μs; - 4, when Bit Duration is configured to τ/2 μs; - 8, when Bit Duration is configured to τ/4 μs; - 16, when Bit Duration is configured to τ/8 μs; - 32, when Bit Duration is configured to τ/16 μs; - 64, when Bit Duration is configured to τ/32 μs; - 192, when Bit Duration is configured to τ/96 μs. | The interval in bits for D2R midamble insertion. | Ibit |
| Sequence Length Indicator | 1 bit | {short, long} | Sequence length indicator for D2R preamble/midamble. | Lamble |
| Additional Midamble Indicator | 1 bit | {absent, present} | Additional D2R midamble insertion indicator. | Iadd |
| D2R TBS | 7 bits | {1, 2, …, 124, 125}, i.e. integers from 1 to 125. | The D2R transport block size in bytes. | NTBSD2R |

### 6.2.2	D2R messages

#### 6.2.2.1	Access Random ID message (Msg1 in CBRA)

Figure 6.2.2.1-1 shows the format of the Access Random ID message.

The field in this message is defined as follows:

-	Random ID: This field includes a 16-bit random number.

Figure 6.2.2.1-1: MAC PDU of Access Random ID message

#### 6.2.2.2	D2R Upper Layer Data Transfer message

Figure 6.2.2.2-1 shows the format of the D2R Upper Layer Data Transfer message.

The fields in this message are defined as follows:

-	D2R Message Type: This field indicates the message type. See the Table 6.1-2. The length of the field is 2 bits.

-	R1-R6: The 6 bits are set to 0.

-	More Data Indication (MDI): This field indicates whether there are more upper layer data to be sent from the device (when set to 1) or not (when set to 0). This length of this field is 1 bit.

-	SDU Length: This field indicates the length of the Data SDU field in the unit of byte. The length of this field is 7 bits.

-	Data SDU: This field is of variable length and includes the upper layer data.

-	MAC Padding: This field includes padding bits. This field is optional.

Figure 6.2.2.2-1: MAC PDU of D2R Upper Layer Data Transfer message

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-03-28 | RAN2#129bis | R2-2502258 | - | - | - | Skeleton of A-IoT MAC Protocol | 0.01 |
| 2025-05-09 | RAN2#130 | R2-2503997 | - | - | - | To capture the RAN2 agreements achieved before RAN2 #130 | 0.0.2 |
| 2025-08-15 | RAN2#131 | R2-2505520 | - | - | - | Updates based on the agreements achieved in RAN2#130 | 0.0.3 |
| 2025-09-05 | RAN2#131 | xxx | - | - | - | Updates based on the agreements achieved in RAN2#131 | 0.1.0 |

# Annex

No MAC impact

Captured

Agreements

- RAN2 thinks it is feasible from a signalling perspective to add the 128 bits.  However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible.

- Indicate to SA3 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

- RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not.

- Reply to SA3

- -     RAN2 thinks it is feasible from a signalling perspective to add the 128 bits.  However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible.

- -     Indicate to SA3 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

-	Indicate space pressure from all the WG

Agreements

1	RAN2 confirms how to generate transaction ID is left to reader (no spec impact)

2	8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion.

3	RAN2 confirms the pervious RAN2 baseline that transaction ID is not included in paging message for CFRA.  Clarify that CBRA can be used by reader for single device.

4	No entry number is included in either Msg2 or NACK feedback message.  RAN2 understands that device can decode the entries one by one till message end, other implementations are not precluded (we will not capture this in the spec).

5	R2D TBS information is not included in the Access Trigger message.

6	Add a 7-bit R2D TBS field (in unit of byte) after R2D message type indication in variable-length R2D messages (i.e., Paging message, Random ID Response message, R2D Upper Layer Data Transfer message, NACK Feedback message).

7	6 bits for Transaction ID length.

8	Explicit AS ID release message is not needed.

9	For forward compatibility:

-	Paging message can be extended to add more fields at the end of the message in further releases, and Rel-19 devices ignore the extension parts added in future release instead of dropping the whole message, without extension indication. Future extension using the same message type is not supported for R2D messages other than paging message.

-	No version bit will be introduced

-	Remove the R-field in paging message from the running CR

-	Use 3-bit R2D message type.

10	A 2 bits D2R message type is introduced in this release.  For Rel-19 only one message type exists for D2R message.  RN16 doesn’t include message type as already agreed.

11	Access Trigger message is 3 bits and no padding bits are added (i.e. not byte aligned)

Agreements

1	Keep current agreement.   The reader should provide enough access trigger to cover at least signalled AOs in current round, unless the reader choses to start the subsequent paging round.    Capture in stage 2 and rapporteur will work in the wording.

Agreements

- RAN2 confirms that R2D trigger message does not include slot number/count down number in this release

- RAN2 acknowledges the problem, and it is up to reader implementation to avoid the mismatch between reader and device due to the miss of the A-IoT paging message with updated configurations in this release.  RAN2 will not add any explicit information in this release to address this problem

Agreements

- 3-bit frequency index is optionally included with each echoed random ID in MSG2.  We have 1 bit in MSG2 to indicate presence/absence of the frequency information for all included RN16s.

- The boundary is the reception of either the kth Access trigger message or the subsequent paging message. Reader implementation to send MSG2 immediately (before k) is allowed.    K can be configured to be either 1 or 4 in paging message.

Agreements on no data available due to delay in NAS

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

- The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all.   If this case exists, RAN2 will discuss this issue.

Agreements

To ensure byte alignment for the variable size R2D message:

- Paging and Msg2 (Variable bit length): add one “fill field” in the end of the message (1~7bits).

- NACK feedback (AS ID entry(ies) self-aligns, message type is 3-bit fixed): add R-bit(s) after message type field.

- R2D upper layer data message (data SDU self-aligns, other fields are of fixed bits): add R bits after CI field to differentiate R-bit number when data SDU or received data size is present. Confirm MAC padding field and SDU length field are not needed.

- R bit is set to zero in this release and ignored by the receiver.

- What’s included in the fill field is not specified, but device ignores the fill field.

- When a single D2R resource is signaled in R2D upper layer data transfer message , use 3-bit field to represent “Frequence Resource Indication” instead of the 8 bit bitmap.

Upon reception of NACK message addressed to the device, its AS ID is released