3GPP TSG RAN WG2 Meeting #131                	                	                          R2-250xxxx

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Xiaomi

Title:	Comment for [POST131][043][AI PHY] UE capabilities (Xiaomi)

Document for:	Discussion and Decision

# Introduction

- [POST131][043][AI PHY] UE capabilities (Xiaomi)

Intended outcome: align on remaining open issues and agreable proposals

Deadline:  Sept. 4th

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei, HiSilicon | Seau Sian Lim | seau.sian.lim@huawei.com |
| Lenovo | Congchi Zhang | zhangcc16@lenovo.com |
| Nokia | Sakira Hassan | sakira.hassan@nokia.com |

# Comment

Companies are invited to provide comments if there’s a change that is not agreeable. Other comments are welcomed.

| Capability IE/FG | Specification (306/331) | Comment | Proposed Change | Status updated by Rapp |
| --- | --- | --- | --- | --- |
| applicabilityReportingCSI | 306 | Combine the “support of updates via UAI” into the first sentence. | “Indicates whether the UE supports reporting of applicability status (via RRCReconfigurationComplete or via UEAssistanceInformation message) and its updates (via via UEAssistanceInformation message) based on inference configuration provided via CSI-ReportConfig, as specified in TS 38.331 [9].” | Resolved |
| applicabilityReportingOther-r19 | 306 | As above | As above | Resolved |
| applicabilityReportingOther-r19 | 306 | Editorial | “support of at least” | Resolved |
| dataThresholdAvailabilityIndication-r19 | 306 | Editorial | dataThresholdAvailabilityIndication-r19 Indicates whether the UE supports triggering data availability indication when logged data for data collection for NW-side model reaches a configured threshold. A UE supporting this feature shall also indicate support of loggedDataCollection-r19. | Resolved |
| eventBasedLoggedDataCollection-r19 | 306 | Editorial change to simplify the wording and reuse what’s being currently used in RRC specifications. | “event-triggered logging for data collection of NW-side model” “event-triggered logging of measurements for network data collection” | Resolved |
| loggedDataCollection-r19 | 306 | Same as above. Additionally, since logging can be either event based (which is a separate capability) or periodical, the text can be further simplified. | “Indicates whether the UE supports logged measurement of data collection for NW-side model, as specified in TS 38.331 [9].” “Indicates whether the UE supports periodical logging of measurements for network data collection” With that change the next sentence can also be modified accordingly: “The UE also supports periodical logging for data collection for NW-side model and providing full buffer indication and/or low power indication. “ “This capability also indicates that a UE supports providing full buffer indication and/or low power indication.” “Or” should be removed as the UE is expected to support both of these, not only one. | Resolved |
| NW-side data collection | 38.822 | In general, to align the wording with RRC, “NW-side data collection” should not be used. | Modify the naming and the descriptions to speak of “Network data collection”, instead of “NW-side data collection” or “data collection for NW-side model” | Resolved |
| applicabilityReportingCSI-r19 Indicates whether the UE supports applicability reporting based on inference configuration provided via CSI-ReportConfig, as specified in TS 38.331 [9]. The UE also supports providing updates of applicability reporting via UAI. A UE supporting this feature shall also indicate support at least one of aiml-BM-Case1-r19, aiml-BM-Case2-r19 and aiml-CSI-Prediction-r19. | 38.306 | On the CY condition in the field description, we think the way it is written is more for pre-requisite rather than for conditional mandatory. Hence it should be written in a CY manner. I.e. “This feature is mandatory if the UE supports aiml-BM-Case1-r19, aiml-BM-Case2-r19 and/or aiml-CSI-Prediction-r19’” | Propose the following for alignment: “This feature is mandatory if the UE supports aiml-BM-Case1-r19, aiml-BM-Case2-r19 and/or aiml-CSI-Prediction-r19’” | Resolved |
| applicabilityReportingOther-r19 Indicates whether the UE supports applicability reporting based on inference configuration provided via OtherConfig, as specified in TS 38.331 [9]. The UE also supports providing updates of applicability reporting via UAI. A UE supporting this feature shall also indicate support at least one of aiml-BM-Case1-r19, aiml-BM-Case2-r19 and aiml-CSI-Prediction-r19. | 38.306 | Same comment as above. It should be written in a CY manner. I.e. “This feature is mandatory if the UE supports aiml-BM-Case1-r19, aiml-BM-Case2-r19 and/or aiml-CSI-Prediction-r19’” | Propose the following for alignment: “This feature is mandatory if the UE supports aiml-BM-Case1-r19, aiml-BM-Case2-r19 and/or aiml-CSI-Prediction-r19’” | Resolved |
| dataThresholdAvailabilityIndication-r19 Indicates whether the UE supports triggering data availability indication when logged data for data collection for NW-side model reaches a threshold. A UE supporting this feature shall also indicate support of loggedDataCollection-r19. | 38.306 | For "data collection for NW-side model", I suggest to align it with 38.331 CR, i.e. In the latest RRC CR R2-2506401, it is using "network data collection". | Use "network data collection" instead of "data collection for NW-side model" across 38.306 CR and 38.331 CR for UE capabilities. | Resolved |
| Cover page related to Other specs | 38.331 | It should indicate the 38.306 with the corresponding CR | Add TS38.306 CRxxxx | Resolved |
| Cover page related to Other specs | 38.331 | [Lenovo] Suppose there is no need to include 38.331 again in “Other core specification” | Remove “TS38.331…” in Other core specifications. | Resolved |
| AIML-Parameters | 38.331 | [Lenovo] Editorial | In the description, “AIML-Parameters” should be italics “AIML-Parameters” | Resolved |
| applicabilityReportingOther-r19 | 38.306 | [Lenovo] “inference configuration provided via OtherConfig” Shouldn’t it be inference related parameters configuration instead? For Option B. | “inference related parameters configuration provided via OtherConfig” | Resolved |
| Others (related to 58-1-7) |  | [Apple] We are not sure whether/how to capture below agreement: 9 RAN2 will not introduce separate CSI resource capability for logged NW-side data collection. Legacy capability will be used for logged NW-side data collection. Check with RAN1 on whether this assumption is ok. | Maybe it is in another UE capability discussion. But we assume something like below can be captured: In 58-1-7, add “A UE supporting this feature and loggedDataCollection-r19 shall also support logging of L1 RSRP and beam ID for network data collection.” | rapporteur suggests to wait for next meeting if there’s any feedback from RAN1 on NW-side data collection, as rapporteur observes the ‘check with RAN1’ part has been included in the NW-side data collection LS to RAN1. |
| applicabilityReportingOther-r19 Indicates whether the UE supports applicability reporting based on inference configuration provided via OtherConfig, as specified in TS 38.331 [9]. The UE also supports providing updates of applicability reporting via UAI. A UE supporting this feature shall also indicate support at least one of aiml-BM-Case1-r19, aiml-BM-Case2-r19 and aiml-CSI-Prediction-r19. | 38.306 | [Nokia] Agree with Lenovo. For option B, it is ‘inference related parameters’ not ‘inference configuration’. We also agreed this in the last RAN2#131 meeting. 1 RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. 2 For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters according to RAN1#121 agreement: associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, nroftimeinstance-r19, applicabilityConfigId-r19. 3 For Option B for BM Case 1/2, inference related parameter set is configured per serving cell. | In the description field, replace ‘inference configuration’ with ‘inference related parameters’. The description in the Annex should also be updated. | Resolved |
| dataThresholdAvailabilityIndication-r19 Indicates whether the UE supports triggering data availability indication via UEAssistanceInformation message when logged data for network data collection reaches a configured threshold. A UE supporting this feature shall also indicate support of loggedDataCollection-r19. | 38.306 | [Nokia] On the ‘No’ condition of the ‘M’ field, we think this should be mandatory that is ‘CY’. Because it is mandate if the UE indicates the support of eventBasedLoggedDataCollection-r19 . We agreed in RAN2#131 ‘Event-based logging is an optional per UE capability separate from ‘loggedDataCollection-r19’. UE supporting this feature shall also indicate the support of ‘loggedDataCollection-r19’. If UE supports event-based logging it shall support data threshold-based data availability indication.’ | In the description, to add ‘It is mandate to indicate, if UE indicates the support of eventBasedLoggedDataCollection-r19.’ In the ‘M’ field, to replace ‘No’ with ‘CY’. | Rapporteur understands that the agreement is to consider whenever event-based logging is supported, data threshold-based indication is also supported, i.e., both capabilities are supported simultaneously. Rapporteur thinks current implementation aligns with the agreement. |
| UE-NR-Capability-v1900 | 38.331 | [Nokia] nonCriticalExtension is missing | Add ‘nonCriticalExtension SEQUENCE {} OPTIONAL’ in ASN.1 | Resolved |
| Annex: RAN2 UE capability feature list | 38.306, 38.822 | [Nokia] Based on the previous comment and reasoning, Mandatory/optional field for ‘dataThresholdAvailabilityIndication-r19’ in the Table 8.2.x-1 to be updated with ‘Conditional mandory with capability signaling’ | In the Mandatory/optional field of dataThresholdAvailabilityIndication-r19 to be replace with ‘Conditional mandatory with capability signalling’ | see reply earlier |
| Cover page related to Clauses affected | 38.306 | [Nokia] The terminology AI/ML needs to be added in Clause 3.3 | Add ‘AI/ML Artificial Intelligence/Machine Learning’ | Resolved |
| General comment on the terminology of ‘Network data collection’ | 38.306, 38.331, 38.822 | [Nokia] The interpretation of ‘Network data collection’ is misleading as it could mean data collection from the network only. If we use ‘data collection for network-side models’, then the purpose is clear. That is the data is being collected for the network side models. | Suggest to use ‘data collection for network-side models’ across all the specifications. | Rapporteur observes ‘network data collection’ is used in 331. To align the same understanding, rapporteur thinks the same wording would be better to avoid any confusion. |