3GPP TSG-RAN WG2 Meeting #131bis	R2-250xxxx

# Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		8.x.x

Source:	Xiaomi

Title:	Open issues of Rel-19 AIML air UE capabilities

Document for:		Discussion and Decision

# Introduction

As part of email discussion “[POST131][043][AI PHY] UE capabilities (Xiaomi)”, this document is to collect open issues on Rel-19 AIML air UE capabilities.

Phase 1: Companies are invited to provide input on open issue list no later than 19th Sep.

Phase 2: Rapporteur will further provide the resolution to each open issue afterwards. Companies further input on the open issue list and resolution are expected before 1st Oct.

| Company | Name | Email |
| --- | --- | --- |
| Samsung | Beom | s90.jeong@samsung.com |
| Lenovo | Congchi Zhang | zhangcc16@lenovo.com |
| Nokia | Sakira Hassan | sakira.hassan@nokia.com |
| Huawei | Seau Sian Lim | seau.sian.lim@huawei.com |

# Discussion

During previous discussion, rapporteur observes there’s only one open issue left:

Open issue UECap-1: whether UE can support other memory sizes and indicate to network via optional capability signaling

Issue Description:

This was discussed during the email discussion, and there are 5 companies think that UE can optionally indicate other AS layer memory sizes to network due to the following reasons:

-	network flexibility for higher buffer threshold configuration

-	common AS layer buffer might need larger memory size to support other use cases as well

4/9 companies don’t agree to indicate other memory size to network due to the following reasons:

-	for Rel-19 data collection, no need for additional memory

-	more memory size is up to UE implementation

-	network can retrieve data based on UE buffer full indication

Status in running CR:

Not captured

Based on offline and online discussion, the supporting of additional memory sizes mainly motivated by supporting more flexible buffer-threshold based configuration. It seems reasonable to consider dataThresholdAvailabiltyIndication-r19 as the prerequisite of this new capability (if introduced). If dataThresholdAvailabiltyIndication-r19 is not supported, UE does not need to support additional memory size.

Therefore, rapporteur proposes with the following way-forward, where the candidate values consider memory size supported in qoe-AdditionalMemoryMeasReport-r18 as baseline (except introducing a smaller value of 64kB):

Proposal 1: Define aiml-AdditionalMemoryReport-r19 to indicate the additional AS layer memory size the UE support for logged measurement of network-side data collection in addition to 64kB.

| aiml-AdditionalMemoryReport-r19 Indicates whether the UE supports the additional AS layer memory size the UE support for logged measurement of network-side data collection in addition to 64kB. Value kB64 means the UE supports at least 64 kilobytes for this purpose, and so on. A UE supporting this feature shall also indicate support of dataThresholdAvailabiltyIndication-r19. | UE | No | No | No |
| --- | --- | --- | --- | --- |

AIML-Parameters information element

-- ASN1START

-- TAG-AIML-PARAMETERS-START

AIML-Parameters-r19 ::=            SEQUENCE {

applicabilityReportingCSI-r19                  ENUMERATED {supported}                                      OPTIONAL,

applicabilityReportingOther-r19                ENUMERATED {supported}                                      OPTIONAL,

loggedDataCollection-r19                       ENUMERATED {supported}                                      OPTIONAL,

eventBasedLoggedDataCollection-r19             ENUMERATED {supported}                                      OPTIONAL,

dataThresholdAvailabilityIndication-r19        ENUMERATED {supported}                                      OPTIONAL,

aiml-AdditionalMemoryReport-r19                ENUMERATED {kB64, kB128, kB256, kB512, kB1024}

OPTIONAL

}

-- TAG-AIML-PARAMETERS-STOP

-- ASN1STOP

Q1: Do companies agree with above proposal and the relative TPs for this new introduced capability? Comment to the TP is also welcomed.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Samsung | Yes but | We assume additional memory is also useful for NW to select the UE with large memory to collect much data, rather configuring multiple UEs. So, we agree with this proposal but some updates are suggested 1. The proposed values (64KB, 128KB, …1024KB) are in addition to 64KB. It means UE’s total memory would be like 64+64=128KB, 128+64= 192KB, 256+64=320KB,…. In our understanding, common UE implementation usually supports the memory size as power of 2. In that sense, these sizes (128KB, 192KB, 320KB…) are not preferred. So, we suggest the memory as power of 2. It could be achieved with the following update: aiml-AdditionalMemoryReport-r19 Indicates whether the UE supports the additional AS layer memory size the UE supports for logged measurement of network-side data collection in addition tolarger than 64kB. Value kB64128 means the UE supports at least 64128 kilobytes for this purpose, and so on.. aiml-AdditionalMemoryReport-r19 ENUMERATED {kB64, kB128, kB256, kB512, kB1024} OPTIONAL } 2. We are not sure if this capability should be dependant to dataThresholdAvailabiltyIndication. We think this capability is still useful for NW’s UE selection with large buffer and other potential use-cases |
| Nokia | Yes with comments | - The use of the word ‘whether’ in the description is not clear - Supporting Samsung’s concern. That it is a bit misleading whether this capability to indicate additional memory along with what we are already supporting that is 64KB as default. It might be good to clarify that if UE is indicating this UE capability, then UE has the total memory of 64kB + 64kB = 128kB. - Our suggestion is to use similar text as in qoe-AdditionaMemeoryMeasReport-r18 Proposed changes: aiml-AdditionalMemoryReport-r19 Indicates whether the UE supports the additional the minimum AS layer memory size the UE supports for logged measurement of network-side data collection in addition to 64kB the “AS layer memory size for logged measurement of network-side data collection”. Value kB64 kB128 means the UE supports at least 64 128 kilobytes for this purpose, and so on. A UE supporting this feature shall also indicate support of dataThresholdAvailabiltyIndication-r19.. AIML-Parameters information element -- ASN1START -- TAG-AIML-PARAMETERS-START AIML-Parameters-r19 ::= SEQUENCE { applicabilityReportingCSI-r19 ENUMERATED {supported} OPTIONAL, applicabilityReportingOther-r19 ENUMERATED {supported} OPTIONAL, loggedDataCollection-r19 ENUMERATED {supported} OPTIONAL, eventBasedLoggedDataCollection-r19 ENUMERATED {supported} OPTIONAL, dataThresholdAvailabilityIndication-r19 ENUMERATED {supported} OPTIONAL, aiml-AdditionalMemoryReport-r19 ENUMERATED {kB64, kB128, kB256, kB512, kB1024} OPTIONAL } -- TAG-AIML-PARAMETERS-STOP -- ASN1STOP. |
| Huawei | Yes with comments | We agree with Samsung that there is no need to link this with dataThresholdAvailabiltyIndication-r19. Besides data threshold setting, it can also be used by the NW separately to select the UE for data collection. Furthermore, it can actually put additional requirement on the UE and so from UE perspective it is easier not to link and give the UE vendors implementation freedom in this case. Hence we would prefer to remove the following: A UE supporting this feature shall also indicate support of dataThresholdAvailabiltyIndication-r19. As on the exact values, if it is in additional to the 64 and to keep to power of 2, the values should be “64,192,448,960”. Alternatively, if we go with total memory as suggested by Samsung, it would be “128,256,512,1025” and the text will need to be updated as per Samsung’s suggestion. |
| Qualcomm | No | We do not agree to support indication of additional memory. Additional memory is up to UE implementation without signalling. |

Besides the above open issue, companies are invited to describe any identified open issues in the table below.

| Company | Description of open issues and potential resolution | Rapporteur comment |
| --- | --- | --- |
| Samsung | Given we agreed to report applicability in RRCResumeComplete message, the following update is needed: Definitions for parameters. applicabilityReportingCSI-r19 Indicates whether the UE supports applicability reporting and/or its updates (via RRCReconfigurationComplete or via UEAssistanceInformation message or via RRCResumeComplete) based on inference configuration provided via CSI-ReportConfig, as specified in TS 38.331 [9]. It is mandatory if UE supports at least one of aiml-BM-Case1-r19, aiml-BM-Case2-r19 and aiml-CSI-Prediction-r19.. applicabilityReportingOther-r19 Indicates whether the UE supports applicability reporting and/or its updates (via RRCReconfigurationComplete or via UEAssistanceInformation message or via RRCResumeComplete message) based on inference related configuration provided via OtherConfig, as specified in TS 38.331 [9]. It is mandatory if UE supports at least one of aiml-BM-Case1-r19 and aiml-BM-Case2-r19.. | As Nokia suggested below, rapporteur suggests to remove the example signalling since the field description also refers to 331 spec. |
| Lenovo | [Terminology Alignment] 38.331 To align with stage 2 CR. Suggest “AI/ML-based” beam management and “AI/ML-based” CSI prediction. | Ok with the change |
| Lenovo | [Terminology Alignment] 38.306 Mixed use of “memory” and “buffer” As raised also over email, we have been using "buffer" during our WI discussion. On the other hand, when it comes to spec terminology w.r.t logging, we notice the term "memory" is actually used in MDT/QoE description. Suggest “full buffer indication”  “full memory indication” It is also relevant to 38.331, 38.300. Depending on relevant rapporteurs, we can either decide here (since term “memory” and “buffer” differs from capability point of view) or in other discussion. | To be updated after checking all relevant specs. |
| Nokia | As Samsung brought to attention that we have additional agreements on reporting of applicability reporting with RRCResumeComplete message along with UEAssistanceInformation and RRCReconfigurationComplete, it would be better to remove the specific messaging details from the capability description, as these detailed information are already available in TS 38.331. Proposed changes: Definitions for parameters. applicabilityReportingCSI-r19 Indicates whether the UE supports applicability reporting and/or its updates (via RRCReconfigurationComplete or via UEAssistanceInformation message) based on inference configuration provided via CSI-ReportConfig, as specified in TS 38.331 [9]. It is mandatory if UE supports at least one of aiml-BM-Case1-r19, aiml-BM-Case2-r19 and aiml-CSI-Prediction-r19.. applicabilityReportingOther-r19 Indicates whether the UE supports applicability reporting and/or its updates (via RRCReconfigurationComplete or via UEAssistanceInformation message) based on inference related configuration provided via OtherConfig, as specified in TS 38.331 [9]. It is mandatory if UE supports at least one of aiml-BM-Case1-r19 and aiml-BM-Case2-r19.. | Agreed. |
| Huawei | Since the detailed messages for applicability reporting is already clear in TS38.331, we agree with Nokia that we can just remove ‘(via RRCReconfigurationComplete or via UEAssistanceInformation message)’. Furthermore, RRCResumeComplete cannot be added to applicabilityReportingOther-r19 | Agreed. |
| Huawei | One editorial comment, otherwise the current description seems a bit incomplete. loggedDataCollection-r19 Indicates whether the UE supports logged measurements for network-side data collection, as specified in TS 38.331 [9]. The UE supporting this feature also supports periodical logging and providing full buffer indication and low power indication via UEAssistanceInformation message. The minimum memory size for storing of logged measurement of network-side data collection is 64kB.; UE; No; No; No. | Ok with the change. |

# Conclusion

In this contribution, we collect open issues of Rel-19 AIML air UE capabilities. Based on above discussion, following open issues are identified: