# 3GPP TSG RAN WG2 Meeting #131      	                                                 R2-250xxxx
Bengaluru, Indian, August 25 – 29, 2025

Agenda item:	8.5.1

Source:	Apple (Rapporteur)

Title:	Summary report of [POST131][109][NES] 38.304 CR (Apple)

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1	Introduction

This is a summary document on collection of comments to TS 38.304 running CR after RAN2#131:

[[POST131][109][NES] (Apple)

Scope: Update NES 38.304 CR (including this meeting agreements also).

Intended outcome: 38.304 CR in R2-2506220 to be agreed.

Deadline: Short email discussion

## Contact information:

| Company | Delegate Name | Email |
| --- | --- | --- |
| Apple | Peng Cheng | pcheng24@apple.com |

# 2	Updates after RAN2#131

In RAN2#131, the following running CR was endorsed:

R2-2505496	Running 38.304 CR for network energy saving	Apple (Rapporteur)	CR	Rel-19	38.304	18.4.0	0442	-	B	Netw_Energy_NR_enh-Core

Endorsed and considered as baseline for further discussion.

After online and offline discussion of RAN2#131, CR rapporteur made the following updates (compared with R2-2505496) summarized in Table.1.

| Description of the change | Reason for the change |
| --- | --- |
| Section 7.1: For a UE in RRC_IDLE or RRC_INACTIVE state and supporting paging adaptation, if pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in SIB1, the UE determines the value of Ns from pagingAdaptation-NS, N and PF_offset from the parameter pagingAdaptationNAndPagingFrameOffset as defined in TS 38.331 [3], and only monitors the PO(s) derived from these paging parameters. The parameter firstPDCCH-MonitoringOccasionOfPO-r19 for paging adaptation is signalled in SIB1 for paging in the BWP configured by initialDownlinkBWP. For paging in a DL BWP other than the BWP configured by initialDownlinkBWP, the parameter firstPDCCH-MonitoringOccasionOfPO-r19 for paging adaptation is signaled in the corresponding BWP configuration. | Capture the following RAN2#131 agreement: - => firstPDCCHMonitoringOccasionOfPO-r19 is introduced both PCCH-Config and PDCCH-ConfigCommon. |
| Section 7.2.1: The time location of PEI-O for UE's PO is determined by a reference point and an offset: - The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by pei-FrameOffset (or pei-FrameOffset-r19) in SIB1; - The offset is a symbol-level offset from the reference point to the start of the first PDCCH MO of this PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O (or firstPDCCH-MonitoringOccasionOfPEI-O-r19) in SIB1. If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the PEI-O is provided by (SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4a in TS 38.213[4], T, Ns, and N are determined in clause 7.1. The PDCCH MOs for PEI are determined as specified in TS 38.213 [4] according to pei-SearchSpace, pei-FrameOffset (or pei-FrameOffset-r19), firstPDCCH-MonitoringOccasionOfPEI-O (or firstPDCCH-MonitoringOccasionOfPEI-O-r19) and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pei-SearchSpace, the PDCCH MOs for PEI are same as for RMSI as defined in clause 13 in TS 38.213 [4]. UE determines first PDCCH MO for PEI-O based on pei-FrameOffset (or pei-FrameOffset-r19), and firstPDCCH-MonitoringOccasionOfPEI-O (or firstPDCCH-MonitoringOccasionOfPEI-O-r19), as for the case with SearchSpaceId > 0 configured. | Capture the following RAN2#131 agreement: Proposal 4 (7/13) The following Rel-19 PEI configurations with the same value range (except pei-FrameOffset-r19) as Rel-17 PEI configuration need to be introduced. o po-NumPerPEI-r19 o payloadSizeDCI-2-7-r19 o pei-FrameOffset-r19: extend to 32 radio frame => All proposals above (proposal 1 - 5, and 8 - 15) are agreed. |
| Section 7.2.1: For a UE supporting paging adaptation and PEI, if pagingAdaptationPEI-Config is signaled in system information, the UE in RRC_IDLE or RRC_INACTIVE state can monitor the PEI occasion according to pagingAdaptationPEI-Config, pei-FrameOffset-r19 and firstPDCCH-MonitoringOccasionOfPEI-O-r19 (if configured). | Capture the following RAN2#131 agreement: Proposal 15: When pagingAdaptationPEI-Config is signaled in system information, pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 should be configured. => All proposals above (proposal 1 - 5, and 8 - 15) are agreed. |

Table.1 Summary of updates after RAN2#131

# 3	Collection of comments on running CR after RAN2#131

Please provide your comments in below table, and Rapporteur will response. Please do not insert any comments in running CR directly, which is hard for Rapporteur to follow all comments.

| Company +issue # (e.g. Apple 001) | Detailed issue and proposed change | Rapporteur response |
| --- | --- | --- |

| Nokia001 | Parameter having actual effect should not be listed in parenthesis. Anything in paranthesis is just informative and does not have any impact to UE implementation as such. So one should not have pei-frameOffset-r19 in parenthesis but just listed similarly as other parameters. | Thanks for raising the issue. However, existing 38.304 has used pei-frameOffset from Rel-17: The time location of PEI-O for UE's PO is determined by a reference point and an offset: - The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by pei-FrameOffset in SIB1; - The offset is a symbol-level offset from the reference point to the start of the first PDCCH MO of this PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O in SIB1. [vivo] Share same feeling as Nokia. One suggestion is to revise as below: The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by pei-FrameOffset in SIB1, or by pei-FrameOffset-r19 in SIB1 for UE supporting pagingAdaptation-r19; Similar changes can be adopted for other places. [Rapporteur update] I had an offline discussion with Nokia and vivo. And the conclusion is that latest RRC running CR has included when the UE uses Rel-17 pei-FrameOffset vs Rel-19 pei-FrameOffset in field description of pei-FrameOffset-r19: pagingAdaptPEI-Config The PEI related configuration for paging adaptation. If the UE supports paging adaptation for PEI, it ignores pei-Config-r17, if configured. As it is clear in RRC, we don’t need to distinguish in 38.304 whether UE uses r17 or r19 version. So, I remove the parentheses in 38.304, including firstPDCCH-MonitoringOccasionOfPEI-O (with same logic). It is reflected in v03. |
| --- | --- | --- |
| ERI 001 | The cover page should include the CR numbers for the mega capability CRs, i.e., 38.306 and 38.331. |  |
| ERI 002 | The names of the parameters odsib1-CellReselectionPriority and odsib1-CellReselectionSubPriority should be replaced with od-sib1-CellReselectionPriority and od-sib1-CellReselectionSubPriority to align with the 38.331 CR. |  |
| ERI 003 | Please check for similar others, such as intraFreqODSIB1-ExcludedCellList, interFreqODSIB1-ExcludedCellList, firstPDCCH-MonitoringOccasionOfPO, pagingAdaptation-NS, pagingAdaptationNAndPagingFrameOffset to make sure that parameter names are aligned. |  |
| ERI 004 | The following TP needs editorial revision: If a UE supporting OD-SIB1 is barred in a cell due to no available SIB1 request configuration as defined in section 5.2.2.3.1 of TS 38.331 [3], it considers the cell is no longer barred once SIB1 request configuration of the cell is acquired. |  |
| ERI 005 | No need to use the release extension with the parameter names: e.g., firstPDCCH-MonitoringOccasionOfPO-r19 |  |
| ERI 006 | Is the following text needed in section 7.1: For paging in a DL BWP other than the BWP configured by initialDownlinkBWP, the parameter firstPDCCH-MonitoringOccasionOfPO-r19 for paging adaptation is signaled in the corresponding BWP configuration. Is it correct at all since pagingAdaptFirstPDCCH-MonitoringOccasionOfPO is optional? |  |

# 3	Conclusion

TBD