3GPP TSG RAN WG2 Meeting #131bis      	                                                 R2-250xxxx
Prague, Czech Republic, 13th – 17th October 2025

Agenda item:	8.4.1

Source:	Huawei, HiSilicon (Rapporteur)

Title:	Open issues on Rel-19 LPWUS 38.306 CR

WID/SID:	NR_LPWUS-Core – Release 19

Document for:	Discussion and Decision

# 1	Introduction

Based on the guidelines from the chair, this document is to collect open issues on Rel-19 LPWUS 38.306 CR (R2-2505670).

[Post131][215][LPWUS] Draft CR for UE capability (Huawei)

Intended outcome: Endorse the draft CR for TS 38.306

Deadline:  Sept. 4th

1.	Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

2.	Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

Companies are invited to provide input no later than Friday September 26 18:00 UTC.

## Contact information:

| Company | Delegate Name | Email |
| --- | --- | --- |

# 2	Discussion

Rapporteur identified one open issue. Companies are invited to describe any identified open issues in the table below.

| Company+issue # | Description of open issues and potential resolution | Rapporteur comment |
| --- | --- | --- |
| HW-001 | The UE capabilities for IDLE/INACTIVE LP-WUS are included in Rel-19 paging container (i.e., UE-RadioPagingInfo-r19) using OCTET STRING, as captured in UE capability mega CR. It is the understanding that gNB is not required to decode the content of Rel-19 paging container. However, to assign proper dedicated cellReselectionPriorities for LP-WUS to the UE by RRC release message, the gNB needs to know whether the UE supports IDLE/INACTIVE LP-WUS and at least which band(s) is/are supported by the UE for IDLE/INACTIVE LP-WUS. Potential resolution: The UE capabilities for LP-WUS operation in IDLE/INACTIVE should also be added outside the Rel-19 paging container in UE capability information message. For the contents, there are following two options: - Option 1: only indicating the supported band list for LP-WUS regardless of the receiver type and other capabilities; - Option 2: indicating the supported band list for LP-WUS and the receiver type. Option 2 is better and suggested to avoid the case that the UE supporting only OOK-based LP-WUS is redirected to a cell with only OFDM-based receiver configured. |  |

# 3	Conclusion

In this contribution, we collect open issues of Rel-19 LPWUS 38.306 CR. Based on above discussion, following open issues are identified:

TBD