3GPP TSG RAN WG2 Meeting #131bis                	                	                          R2-250xxxx

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.20.1

Source:	CATT

Title:	Report of [Post131][225][NR_Others] On Rx BSF optimization (CATT)

Document for:	Discussion and Decision

Introduction

- [Post131][225][NR_Others] On Rx BSF optimization (CATT)

Intended outcome: Discussion summary on the UAI issue, and endorse the CR for TS38.331

Deadline:  Long

Rapporteur will provide RRC CR for RAN2#131bis based on the outcome of this post email discussion.

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| vivo | Dongdong Wei | weidongdong@vivo.com |
| Huawei, HiSilicon | Lili Zheng | zhenglili4@huawei.com |

Discussion

In RAN4 LS [1], the updated RAN4 agreements on Rx BSF are provided as below. And RAN2 work is to introduce a new UAI indicating UE preference to quit FBS. Regarding the UAI details, some issues have been listed and companies are invited to provide views.

# 1. Overall Description: RAN4 would like to update RAN2 on the Rx BSF optimization for NR RRM Phase 5. RAN4 had discussion on the Rx BSF optimization and the following agreements have been made for quitting condition of FBS: RAN4#115 Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1.. RAN4 further discussed the UAI and updated the agreement as follows: - Introduce a new UAI indicating UE preference to quit FBS: - RAN4 will not discuss the UE behaviour after reporting UAI For the value of T1, RAN4 agreed to define a fixed a value of 90 seconds in RAN4 spec, which has no RAN2 impact in RAN4 understanding. RAN4#116 Agreement: A fixed value of 90 seconds.. # 2. Actions: To RAN WG2 group: ACTION: RAN4 kindly requests RAN2 to take the above agreements into consideration for future work and define the new UAI in RAN2 specification.

UAI Content

The UAI is used to indicate UE preference to quit FBS, then whether the following UAI content is agreeable:

quitFBS-PreferenceFR2-r19           ENUMERATED {true}                               OPTIONAL

Q1. Do you agree the new UAI only indicates UE preference to quit FBS?

| Company | Yes/No | Comment |
| --- | --- | --- |
| vivo | Yes | Follow RAN4 agreements. |
| Huawei, HiSilicon | See comments | We prefer to include two values for this UAI, i.e., UE can report it wants to quit BSF or it wants to stay in BSF. UE cannot fall back to normal measurement during T1 and T1 is set to a quite long value (90s) by RAN4, we think it is possible that the UE preference could change during this 90s. Also, in the R18 multi-Rx, we allow UE to report either “preference on operating in multi-Rx” or “preference on not operating in multi-Rx”. |

Summary

Prohibit timer

Prohibit timer can prevent UE from frequently reporting UAI information. For example, T346m has been introduced for multi-Rx preference reporting. And the configuration of timer length is as below, i.e., timer length in seconds.

MultiRx-PreferenceReportingConfigFR2-r18 ::= SEQUENCE {

multiRx-PreferenceReportingConfigFR2ProhibitTimer-r18  ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

multiRx-PreferenceReportingConfigFR2ProhibitTimer Prohibit timer for multi-Rx operation preference reporting for FR2. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

Considering RAN4 has agreed that:

UE is allowed to fall back to normal measurement when it has performed fast measurement for 90s from the time point when UE enters the FBS mode after meeting the FBS entry condition.

it seems unnecessary to introduce a prohibit timer for this new UAI, as the time duration of FBS operation is only 90s, and after receiving this quitting-preference UAI during this 90s period, network can lower down the activation threshold so that UE can quit FBS operation.

Q2. Do you agree NOT to introduce a prohibit timer for this new UAI?

| Company | Yes/No | Comment |
| --- | --- | --- |
| vivo | Yes | No need to introduce a duplicated timer in RAN2. |
| Huawei, HiSilicon | No | Even if the UE reports the UAI during T1 (90s), the UE cannot exit BSF before T1 expires, and it is up to NW implementation to de-configure BSF configuration. It is likely that the UE could trigger another report during this 90s, and a prohibit timer would be useful to prevent frequent reporting. |

Summary

UAI configuration

Q3. If the answer to Q2 is Yes, whether the following configuration to enable the new UAI is agreeable:

OtherConfig-v19xy ::=                   SEQUENCE {

quitFBS-PreferenceReportingConfigFR2-r19 ENUMERATED {true}                                             OPTIONAL -- Need R

}

| Company | Yes/No | Comment |
| --- | --- | --- |
| vivo | Yes |  |
| Huawei, HiSilicon |  | If a prohibit timer is introduced, SetupRelease structure is better. |

UAI procedure

Q4. If the answers to the questions above are all Yes, whether the following UAI related procedural text in RRC is agreeable:

#### 5.7.4.2 Initiation <Unnecessary part omitted> 1> if configured to provide its preference for quitting FBS operation: 2> if the UE has a preference on quitting FBS operation for FR2: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the UE preference for quitting FBS operation for FR2. #### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message <Unnecessary part omitted> 1> if transmission of the UEAssistanceInformation message is initiated to provide preference on quitting FBS operation for FR2 according to 5.7.4.2: 2> set quitFBS-PreferenceFR2 to true:

| Company | Yes/No | Comment |
| --- | --- | --- |
| vivo | Yes | Besides these modifications, we prefer also adding this purpose (i.e. preference on quitting FBS operation) into section 5.7.4.1, e.g. - configured grant assistance information for NR sidelink positioning, or - its preference on quitting FBS operation for FR2. |
| Huawei，HiSilicon | Yes | To be consistent: 1> if configured to provide its preference for quitting FBS operation for FR2: 2> if the UE has a preference on quitting FBS operation for FR2: Besides, the UE behaviour of releasing this configuration should also be captured (e.g. during re-establishment, during resume). |

Summary:

Any other issues on the new UAI

Q5. For any other issues companies think RAN2 needs to discuss about this new UAI, please add them in the following table.

| Company | Comment |
| --- | --- |

Summary

RRC CR

Q6. Regarding the following RRC CR, companies are invited to leave further comments in the following table.

R2-2505205	Introduction of Rx BSF optimization for NR RRM Ph5	CATT, Ericsson, Apple, ZTE Corporation	draftCR	Rel-19	38.331	18.6.0	B	NR_RRM_Ph5-Core

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | There is no need to have a separate clause for criterion for L3 fast beam sweeping operation, it can be added in the field description of the new IE, the feature is mainly described in RAN4 spec anyway. |

Summary

Conclusion

References

[1] R4-2512333, LS on Rx BSF optimization for NR RRM Phase 5