3GPP TSG-RAN WG2 #131	R2-2506560

# Bengaluru, India, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2121 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NTN Phase 3 enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-09-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of Release 19 NR NTN phase 3 enhancements |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures the agreements of Solutions for NR to support non-terrestrial networks (NTN) in Release 19. Specifically, support for UE to report capability/request of Msg4 PDSCH repetition according to the following agreements from RAN2#131: - LCID in table 6.2.1-2C when LX field is set to 1 is used for UE to report capability/request of Msg4 PDSCH repetition. - RAN2 assumes combination of below features are possible and will define a signalling to support any combination of below features using LCID values in Table 6.2.1-2c: - Msg4 PDSCH repetition - Msg4 HARQ-ACK PUCCH repetition - (e)RedCap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Release 19 NR NTN phase 3 enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.300 CR 1023 TS 38.304 CR 0441 r1 TS 38.306 CR 1321 r1 TS 38.331 CR 5403 r1 TS 38.331 CR 5481 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | N/A |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First change begins >>>>>>>>>>>>>>>>>>>>

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

<<<<<<<<<<<<<<<<<<<< Unchanged text is omitted >>>>>>>>>>>>>>>>>>>>

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 | (216 + 328) | CCCH of size 48 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 9 | (216 + 329) | CCCH of size 64 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 10 | (216 + 330) | CCCH of size 48 bits for PDSCH repetition of Msg4 of a RedCap UE |
| 11 | (216 + 331) | CCCH of size 64 bits for PDSCH repetition of Msg4 of a RedCap UE |
| 12 | (216 + 332) | CCCH of size 48 bits for PDSCH repetition of Msg4 of an eRedCap UE |
| 13 | (216 + 333) | CCCH of size 64 bits for PDSCH repetition of Msg4 of an eRedCap UE |
| 14 | (216 + 334) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 15 | (216 + 335) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 16 | (216 + 336) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and PDSCH repetition of Msg4 of a RedCap UE |
| 17 | (216 + 337) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and PDSCH repetition of Msg4 of a RedCap UE |
| 18 | (216 + 338) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and PDSCH repetition of Msg4 of an eRedCap UE |
| 19 | (216 + 339) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and PDSCH repetition of Msg4 of an eRedCap UE |
| 20 to 63 | (216 + 340) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>