3GPP TSG-RAN WG2 Meeting #131	Draft R2-2506284

Bengaluru, India, 25 – 29 August 2025

Title:	[DRAFT] LS on power ramping and RRC configuration for CB-Msg3-EDT

Response to:	-

Release:	Release 19

Work Item:	IoT_NTN_Ph3-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Ping Yuan

E-mail Address:	Ping.1.yuan@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

During RAN2#131 meeting, RAN2 discussed CB-Msg3-EDT and achieved the following agreements regarding power ramping on the CB-Msg3 (re-)transmission:

Agreements regarding CB-Msg3-EDT power ramping: - We inform RAN1 that RAN2 agreed to support power ramping for both NB-IoT and eMTC copying RAN2 agreements asking RAN1 to check the specification impact for RAN1 specs - RAN2 to define two new RRC parameters for CB-Msg3 power ramping (i.e., powerRampingStep and cb-Msg3-InitialReceivedTargetPower), with the same value ranges as those defined for legacy Msg3 power ramping. - The UE applies power ramping when the CB-Msg3ResponseTimer has expired and the UE proceeds to the next CB-msg3 transmission, by reusing the CB_MSG3_TRANSMISSION_COUNTER_CE as defined in the MAC running CR. - The CB-Msg3 received target power can be calculated as below: the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

Besides, regarding CB-Msg3 TBS configuration, RAN2 agreed on the following, applicable to both NB-IoT and eMTC:

Agreements regarding TBS configuration for CB-Msg3: - We don’t support multiple TBSs per CE level for CB-Msg3 EDT. - We introduce one additional value smaller than the smallest possible TBS size at the moment (328bits), i.e. 144bits.

Furthermore, for eMTC, RAN2 agreed to define a set of MPDCCH narrowbands for MPDCCH monitoring for CB-Msg4:

Agreements regarding MPDCCH narrowband configuration: - We configure 2 MPDCCH narrowbands as a set (inform RAN1)

In addition, RAN2 made the following confirmation regarding RAN1’s questions indicated in the LS R1-2504959: :

RAN1 question: 	RAN1 wonders whether RAN2 intends to support multi-PRB allocation or sub-PRB allocation or both

- RAN2 confirms that both multi-PRB allocation and sub-PRB allocation are supported for CB-Msg3 (inform RAN1).

RAN1 question: To be confirmed by RAN2 whether to support both singleTone and multitone, or singleTone only for HL parameter npusch-MCS-r16.

- RAN2 confirms that both single-tone and multi-tone are supported for CB-Msg3, and intends to reuse the parameter npusch-MCS-r16 for CB-Msg3 (inform RAN1).

RAN2 kindly invites RAN1 to take the above agreements into account and:

- Check the specification impact in RAN1 to support power ramping for CB-Msg3 (re-)transmissions, and update the corresponding specification if needed.

- Check whether RAN1 has any concern regarding the additional TBS size configured for CB-Msg3, i.e., 144 bits.

- Check the specification impact in RAN1 to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding specification if needed.

- Check the specification impact in RAN1 to support both multi-PRB allocation and sub-PRB allocation for CB-Msg3 in eMTC, as well as both single-tone and multi-tone transmissions for CB-Msg3 in NB-IoT, and update the corresponding specification if needed.

2. Actions:

To RAN1:

ACTION: 	RAN2 respectfully asks RAN1 to take the above information into account, update the specification if needed, and provide feedback if any.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#131bis	from 2025-10-13	to 2025-10-17		Prague, CZ

RAN2#132	from 2025-11-17	to 2025-11-21		Dallas, USA