3GPP TSG RAN meeting 109										      RP-250XXX

Beijing, China, Sep 15-18, 2025

## Status Report to TSG

Agenda item:			9.6.2.1

| WI / SI Name | NR mobility enhancements Phase 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_Mob_Ph4 |  |  |  |  |
| Unique ID | 1020091 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250339 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 10% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Naveen Palle, Pei Lin |
|  | Company | Apple Inc., China Telecom |
|  | Email | naveen.palle@apple.com, linp@chinatelecom.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#122, the following agreements were made.

Agreement

Following TP to TS38.214 Section 5.2.4a is endorsed in principle.

- Reason for change: Current texts in the referred paragraph in TS 38.214 create some ambiguity. First, “corresponding NZP CSI-RS resources” is unclear. It presents unclear meaning of what the NZP CSI-RS resources are corresponding to. In our views, we should instead describe that NZP CSI-RS resources are corresponding to a [ltm-eCSI-ReportConfig]. Second, “start” is redundant here and never exists in the agreement.

- Summary of change: Describe UE measures NZP CSI-RS resources corresponding to a [ltm-eCSI-ReportConfig] before receiving LTM CSC MAC CE. Remove the redundant “start” wording.

- Consequences if not approved: Unclear UE behavior on LTM RS measurement before receiving LTM CSC MAC CE.

Text Proposal 1 for TS 38.214 Clause 5.2.4a < Unchanged parts are omitted > If a valid CSI is not available, the UE shall transmit a CSI report which contains a CQI corresponding to the lowest CQI index. [Depending on the UE capability] the UE may start measureing corresponding NZP CSI-RS resources corresponding to a [ltm-eCSI-ReportConfig] before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321].

Agreement

Following TP to TS38.213 Section 21 is endorsed as editorial change.

- Reason for change: RAN1 agreed that CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA. The agreement has been captured in above texts in TS 38.213. However, in current texts, it is said that the TRS and the CSI-RS resources to provide QCL 'typeA' and 'typeD' properties, which is incorrect. There should be only one CSI-RS resource for indicating QCL 'typeD' property. Hence, we propose to adopt the Text Proposal 2 as below.

- Summary of change: Remove unnecessary “s”.

- Consequences if not approved: Incorrect UE and/or NW behavior.

Text Proposal 2 for TS 38.213 Clause 21 < Unchanged parts are omitted > A UE can be provided by an LTM Cell Switch Command MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI state ID and/or an UL TCI state ID indicating a CandidateTCI-State and/or CandidateTCI-UL-State from ltm-DL-OrJointTCI-StateToAddModList and/or ltm-UL-TCI-StateToAddModList [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE may assume that DM-RS antenna ports for PDCCH receptions and for PDSCH receptions are quasi co-located either with the SS/PBCH block or the TRS in the TCI state with respect to quasi co-location 'typeA' and 'typeD' properties, when applicable, or with the TRS and the CSI-RS resources in the CSI-RS resource set configured with repetition in the TCI state with respect to quasi co-location 'typeA' and 'typeD' properties, respectively, when applicable. < Unchanged parts are omitted >

Agreement

Following TP to TS38.214 Section 5.2.4a is endorsed in principle.

| Reason for change: | For LTM CSI reporting, both channel measurements and interference measurements would be done by UE. For interference measurement, UE could either measure NZP CSI-RS or CSI-IM depending on the configuration. In TS 38.214, the measurement on CSI-IM resources is missing. |
| --- | --- |
| Summary of change: | For LTM CSI reporting, UE should measure CSI-IM resources if configured. |
| Consequences if not approved: | UE would not measure CSI-IM resources for interference measurement. |

-------------------- Start of text proposal to TS 38.214 v19.0.0 ---------------------------------------

5.2.4a CSI Reporting for LTM

<<< UNCHANGED PART OMITTED >>>

After a UE receives an LTM Cell Switch Command MAC CE [10, TS 38.321] providing a candidate cell (given by Target Configuration ID field), and a [ltm-eCSI-ReportConfig] is configured for the candidate cell, the UE can measure corresponding NZP CSI-RS resources and CSI-IM resources if configured, and shall transmit a CSI report to the candidate cell.

<<< UNCHANGED PART OMITTED >>>

---------------------- End of text proposal to TS 38.214 v19.0.0 ---------------------------------------

Agreement

Following TP to TS38.214 Section 5.2.4a is endorsed in principle.

5.2.4a CSI Reporting for LTM <omitted text> A UE configured with LTM-Config can be provided configurations for CSI acquisition, by up to one Reporting Setting, [ltm-eCSI-ReportConfig], for a candidate cell. Each Reporting Setting [ltm-eCSI-ReportConfig] is associated with either one or two Resource Settings - When one Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) is configured, it provides a list of NZP CSI-RS resources for both channel and interference measurements. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting. - When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) provides a list of NZP CSI-RS resources for channel measurement, and the second Resource Setting (given by higher layer parameter [ltm-ResourceForInterferenceMeasurements]), provides a list of [CSI-IM resources] for interference measurement. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Settings. The UE shall expect the following configuration provided in each [ltm-eCSI-ReportConfig]: - For the frequency granularity of the CSI report, the CQI format indicator is Wideband CQI. - For the frequency granularity of the CSI report, the PMI format indicator is Wideband PMI. - The codebook type is typeI-SinglePanel. - The reportQuantity is set to ‘cri-RI-PMI-CQI’. <omitted text>

Agreement

- Introduce a new RRC IE for LTM codebook configuration

CodebookConfig-LTM-r19 ::=         CHOICE {

twoToThirtyTwoPorts                 CodebookConfig,

moreThanThirtyTwoPorts           CodebookConfig-r19

...

}

- Add note in RRC parameter list to inform RAN2 to add ‘typeI and single panel’ restriciton.

Agreement

- Add the optional field ‘repetition’ to LTM-NZP-CSI-RS-ResourceSet

  - When LTM-NZP-CSI-RS-ResourceSet is used for beam-management, ‘repetition’ should be set to ‘off’ and when it is used for early CSI acquisition, the field should be absent.

Agreement

- Adapt LTM-CSI-ReportConfig to include cqi-Table for CQI reporting

- Clarify in RAN1 specification that the LTM-CandidateId-r18 in ltm-CandidateIdList-r19 of LTM-NZP-CSI-RS-ResourceSet-r19 should be same as that of LTM-Candidate-r18 under which CSI report configuration for CSI acquisition is configured.

Agreement

Semi-persistent CSI-IM measurement resource is supported for the CSI acquisition for candidate cells.

Agreement

For a UE capable of CSI acquisition of performing early CSI measurement operations before and after LTM Cell Switch Command MAC CE:

- For candidate cell(s), a periodic CSI-RS resource and port are counted as ‘active’ in a duration starting when the periodic CSI-RS resource is configured by higher layer signaling until the reception of the LTM cell switch command MAC CE or a RRC reconfiguration message, which comes first.

- For candidate cell(s), a SP CSI-RS resource and port are counted as ‘active’ in a duration starting from the end of when the activation command is applied until the reception of the LTM cell switch command MAC CE or a deactivation command MAC CE, which comes first.

Agreement

For a UE capable of CSI acquisition of performing early CSI measurement operations only after LTM Cell Switch Command MAC CE:

- For candidate cell(s), periodic and SP-CSI-RS resources and ports configured for early CSI acquisition in an LTM Candidate cell are not counted as active before reception of LTM Cell Switch Command MAC CE.

#### 2.1.2	Remaining Open issues

- None

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2-131:

Agreements on stage-3 details

- WI is completed from RAN2 point of view.

- Define a per UE capability for the support of all the LTM events

- No need to define a separate UE capability for the support of the following functions in L1 event triggered measurement reporting,

- -	The new MR MAC CE.

- -	The Truncated MR MAC CE.

- -	The event triggered periodic MR MAC CE reporting

- -	The reportOnLeave for LTM

- -	The current beam reporting in the MR MAC CE

- Discuss whether to define a per UE capability for reporting the beam not satisfying the event condition in the MR MAC CE.

- When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, and at least one of cltm-EarlyTA-Indication-r19 and ue-TA-Measurement-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. This revises the previous agreement.

- No UE capability for reporting the beam not satisfying the event condition in the MR MAC CE.

- A per UE capability for inter-CU LTM recovery.

- “cltm-ExecutionConditionL3-r19”and “cltm-ExecutionConditionL1-r19” are defined as per UE capabilities without FRx or xDD differentiation. Whether this capability is actually supported on given band depends on whether the L1/L3 measurement is supported on the corresponding band.

- No need to differentiate between intra-F CLTM and inter-F CLTM.

- Common SP CSI-RS Resource Set Activation/Deactivation MAC CE is used for both RRM measurement and early CSI acquisition purpose, as captured in the current MAC running CR.

- If a CSI-RS resource set configured for early CSI acquisition contains CSI-RSs from multiple candidates, the UE is expected to measure the RSs of target cell after reception of LTM CSC.

- SP CSI-RS Resource Set Activation/Deactivation MAC CE could be also used to activate and deactivate the configured semi-persistent CSI-IM resource sets of a LTM candidate cell, as captured in the current MAC running CR

- IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id.

- SP-CSI MAC CE includes up to two CSI-RS resource configuration ids.

- Detailed change is left to MAC CR rapporteur.

- MAC indicates the selected RS index to the lower layer for RACH-less CLTM, as captured in the current MAC running CR.

- RAN2 confirms when multiple MR(s) associated with an LTM report configuration is triggered, only one L1 measurement report MAC CE will be generated, as captured in the current MAC running CR.

- RAN2 assumes CG resource associated with CSI-RS is not supported.

- No impact on event triggered L1 MR procedure when beam failure is detected.

- RAN2 will keep the current definition of Type II in MAC, i.e. any beam.

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, UE always fall back to RACH-based LTM regardless whether UE based TA is valid or not.

- Stop the running ltm-Candidate-TimeAlignmentTimer for the corresponding candidate cell and release the stored TA value for the candidate cell if the corresponding CLTM candidate configuration is released.

- If RACH-less CLTM fallback to RACH-based CLTM due to PTAG expiration, RAN2 assumes it’s up to UE implementation to obtain the MAC PDU for 1st UL transmission. Captured as a Note in the MAC spec.

- For MAC-31, wait for RAN1. it is not essential issue, and does not impact the WI completion.

- In case of MCG failure recovery, network reconfigure the UE in case it wishes the UE to restart the evaluation of the LTM cell switch execution condtion.

- UE understands whether to send an MR or trigger a conditional LTM cell switch procedure based on the presence of the reporting content configuration. The related ASN.1 field is changed from mandatory to optional.

- The UE shall not perform secondary key change when intra-MN LTM is performed (take TP in R2-2505583 as baseline).

- The reference configuration can be signalled to UE via the ltm-Config and ltm-ConfigNRDC (current restrictions in RRC are removed).

- RRC-9, RRC-10, RRC-11, RRC-12 is handled during the post email discussion.

- RRC-5, RRC-7, and RRC-8 are not pursued.

Inter-CU details

- SP CSI-RS resource for target cell is deactivated after CSI reporting in the first UL transmission. How to capture it in MAC is up to MAC CR rapporteur.

- For inter-CU LTM failure recovery, even if the selected cell is the target cell, there is no security issue, and no specification change is needed in RRC CR for the case.

- After L3 HO or CHO failure, the UE cannot perform inter-CU LTM fast recovery.

- RAN2 to confirm following principles for LTM:

- 	MCG LTM configuration is always provided within ltm-Config in an MN RRC message;

- 	Intra-SN or inter-SN SCG LTM with MCG configuration is provided within ltm-ConfigNRDC in an MN RRC message;

- 	Intra-SN SCG LTM without MCG configuration is provided within ltm-Config in an SN RRC message;

- 	For one UE, if inter-SN and intra-SN candidate cells are configured simultaneously, SCG LTM candidate configurations can only be provided within the ltm-ConfigNR-DC in MN RRC message(s).

Event Triggered details

- When mTRP is configured in the serving cell the UE uses the best beam of the two “current beams” for LTM event evaluation, and it’s up to UE implementation to how to derive the best beam.

- No additional indication in MR MAC CE is needed.

- The coexistence between event-triggered L1 measurement reporting and mTRP for the source cell is supported in Release 19 LTM.

- “best beam of the two current beams” is changed to “best beam of the current beams”

- TP for stage-2 and MAC are baseline.

- For LTM2, network always configures reportCurrentBeam in the LTM-EventTriggeredReportContent.

- Serving beam measured result is only included at the end of a (truncated) MR MAC CE for LTM2.

- Confirm the following working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

CLTM details

- For C-LTM, if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released.

- It’s up to NW implementation to avoid the latency caused by the PDCP SN gap due to CLTM fast recovery.

- It will be added as a note into stage-2 spec. How to capture it is left to stage-2 spec CR rapporteur.

- If UE receives the LTM CS command triggered by NW, UE shall follow the R18 behaviour to initiate LTM CS in the target cell regardless of whether the UE has the available C-TA value.

- RAN2 understands RAN3 handles the remaining TA information exchange from the source to the target.

- No need to define a separate UE capability for CLTM fast recovery (i.e., reuse the UE capability for Rel-18 intra-CU LTM fast recovery).

- CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported.

#### 2.2.2	Remaining Open issues

- None

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#129:

- BL CR to 37.483 (R3-255057), BL CR to 38.420 (R3-255058), BL CR to 38.470 (R3-255059), BL CR to 37.340 (R3-255060), BL CR to 38.300 (R3-255061), BL CR to 38.401 (R3-255062), BLCR to 38.423 for DC (R3-255064), BLCR to 38.473 (R3-255065) and BL CR to 38.423 (R3-255715) are endorsed.

For inter-CU LTM:

- Confirm to have the explicit Request for CSI-RS Resource Configuration indicator in the Handover Request message.

- The candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE (e.g., refers to ltm-CSI-ReportConfig-r19) during the preparation phase.

- For deactivation of SP CSI-RS of candidate cell(s) after the UE’s successful cell switch, the CSI-RS Coordination procedure is re-used, triggered by the previous serving gNB-CU after successful cell switch is confirmed, and toward the relevant candidate gNB-DU(s) and gNB-CU(s).

- Agree to reuse LTM Configuration Update message to transfer Rel-19 set ID per candidate cell to the candidate CUs.

- Once the UE XnAP association is setup, the source gNB includes the target NG-RAN node UE XnAP ID in the handover request message for any follow-up preparation.

- The old source gNB can deliver the old target UE XnAP ID(s) to the new serving gNB via Cell Switch Notification and LTM Configuration Update message.

- The source gNB sends the Data Forwarding Information as per-PDU session level to the candidate gNBs in the LTM CONFIGURATION UPDATE message.

- Include the Tag ID Pointer and RACH resource request ID in TA Information Transfer in XnAP.

- Include the TA values in Cell Switch Notification message.

- For Rel-19 Set ID assignment, source gNB-CU sends the Rel-19 Set ID list per-node to candidate gNB-CUs in Handover Request message.

- To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

- For CSI-RS configuration in F1AP and XnAP, including the CSI-RS resource configuration and the CSI-RS resource type in the UE Context Setup Response, UE Context Modification Response and Handover Request Acknowledge message.

- For TCI State List configuration over the XnAP, adopt the similar solution as TEI18.

- Include the explicit request for CSI-RS Resource Configuration for CSI acquisition indicator in the Handover Request message.

- The candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE (e.g., refers to ltm-CSI-ReportConfig-r19) in the LTM Configuration Update Acknowledge and the UE Context Modification Response message.

- For CSI-RS coordination procedure, include the mandatory CSI Resource Config ID (i.e. LTM-CSI-ResourceConfigId) in the both request and response message in F1AP and XnAP.

- The CSI-RS-based beam measurement results should be sent from the CU to the DU in the F1AP CU-DU Mobility Initiation Request message.

- TP for BLCR of TS38.300 (R3-255857), TP for BLCR of TS38.401 (R3-255871), TP for BLCR of TS38.473 (R3-255962) and TP for BLCR of TS38.423(R3-255961) are agreed.

For inter-CU LTM in DC:

- Turn the WA into agreement: Different candidate PSCells in the same SN can have different Rel-19 set IDs.

- Do not support LTM modification on the already prepared LTM candidate PScells in DC scenario in Rel-19.

- Support both Source SN-initiated and Candidate SN-initiated LTM Cancel.

- For security update for inter-CU SCG LTM, the list of {KSN, sk-counter} is provided on the granularity of Rel-19 set ID.

- The MN provides a list of R19 set IDs to the candidate SN via the SN Addition Request message. MN sends R19 set ID for the candidate cell along with a list of {KSN, sk-counter} in the SN Modification Request message.

- The MN sends the R19 set ID for each candidate PSCell to the source SN and other candidate SNs via the SN Modification Request message.

- The candidate PSCells to be cancelled is included in the SN Modification Required message.

- The MN may inform the source SN about the cancellation of the prepared candidate PSCells at a candidate SN using SN modification request message.

- To support inter- or intra-CU MCG LTM with SCG, introduce a new XnAP IE to indicate to the candidate SN that the SN Addition/Modification preparation procedure is triggered as part of a MCG LTM procedure.

- To support inter- or intra-CU MCG LTM with SCG, introduce a new F1AP IE to indicate to the DU of a candidate SN that the UE Context Setup/Modification procedure is triggered as part of a MCG LTM procedure.

- Remove the FFS: FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

- SN Change Confirm message is used to notify the list of data forwarding addresses to the source SN.

- The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Addition Request Acknowledge message from the candidate SN to the MN, no RAN3 impact is foreseen.

- TP for BLCR of TS37.340 (R3-255904), and TP for BLCR of TS38.423 for DC (R3-255903) are agreed.

For intra-CU Conditional LTM:

- To convert the working assumptions into agreement:

  - To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

  - To introduce a new IE with a list of candidate cells for L1 execution condition.

- In case of L3 measurement report-triggered early RACH, to reuse the F1AP CU-DU Mobility Initiation procedure to notify the source DU to initiate early RACH procedure to the candidate cells.

- To remove the TAT value IE in the UE CONTEXT SETUP RESPONSE message in the BLCR.

- The source DU sends the TA values and the remaining time of the TA timers, and TAG ID of candidate cells and target cell to the CU by the UE CONTEXT MODIFICATION RESPONSE message after receiving the successful cell change notification from the CU.

- The CU reuses the UE Context Modification procedure to transfer the TA values and the remaining time of the TATs, and TAG ID to the target DU.

- TP for BLCR of TS38.473 (R3-255963) and TPs for BLCR of TS38.401 (R3-255936) are agreed.

#### 2.3.2	Remaining Open issues

- None

## 2.4	RAN4

#### 2.4.1	Agreements

1) Event triggered L1 measurement reporting

Issue 2-1: applicability of L1-RSRP reporting delay

Agreement:

- Capture the following conditions in test case but not in core requirements.

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

Issue 2-2: whether to introduce/update requirements for reporting criteria per measurement category

Agreement:

- No consensus on the necessity to introduce/update requirements for reporting criteria per measurement category.

- The issue does not impact the WI core part completion, and interested companies can further discuss this issue in maintenance phase based on contribution driven.

Issue 2-3: Impact on Cell switch and TCI activation requirements

Agreement:

- In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting and on leave reporting, and the UE has sent an event-triggered L1 report for the target cell and has not sent on leave report before reception of the cell switch command.

- Not to extend above agreements to FR2.

2) CSI-RS based L1 measurement

Issue 3-1: whether skipping step 2 for a serving cell when included in LTM candidate cell

Agreement:

- Define UE capability of skipping SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement, which include two Components:

1) Indicates support for skipping SSB-based L1-RSRP during neighboring cell CSI-RS-based L1-RSRP measurement.

2) Indicates support for skipping SSB-based L1-RSRP during both neighboring cell and serving cell CSI-RS-based L1-RSRP measurement.

- Not define additional/new RAN4 requirement for #2)

- Legacy requirement for serving cell operation such as RLM, BFD, CBD applies.

Issue 3-2: Measurement restriction

Agreement:

- In FR2-1, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms. The separation of the two windows is at least 4ms.

Issue 3-3: how to handle the case when CSI-RS based L1 RSRP measurement on neighbour cell when colliding with serving cell CSI-RS for L1-RSRP measurement?

Agreement:

- If the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, RAN4 to introduce measurement restriction as legacy.

Issue 3-4: how to handle the case when the CSI-RS based L1-RSRP measurement on neighbour cell in FR2-1 overlap with CSI-RS configured for RLM/BFD/CBD?

Agreement:

- For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when it is overlapped with CSI-RS configured for RLM/BFD/CBD.

Issue 3-5: Early CSI acquisition

Agreement:

- No consensus on defining new RAN4 requirements for early CSI acquisition.

  - Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached.

- It is RAN4 common understanding that for baseline early CSI acquisition,

  - If fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell.

  - Otherwise, UE shall start CSI acquisition at least after Tprocessing.

- No need to capture above agreement in RAN4 spec.

Issue 3-7: whether send LS to RAN1/2 on intra-frequency/inter-frequency definition for L1 measurement?

Agreement:

- Send LS to RAN1/2 about RAN4 agreement on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

3) CLTM

Issue 1-1-2: TCSI-RS_SFN_intra

Agreement:

- Remove the TCSI-RS_SFN_intra and reuse the applicable condition in Rel-18 LTM. Discuss whether additional condition is needed in CR directly

Issue 1-1-4: Whether to include unknown cell case

Agreement:

- Only cover the known cell case.

Issue 1-1-5: Side condition for known cell

Agreement:

- Follow the related agreements on CSI-RS based L1 measurement in part1.

Issue 1-2-1: Whether Tfirst-RS and TRS-proc are included in Tinterrupt

Agreement:

- Tfirst-RS and TRS-proc are included in Tinterrupt.

Issue 1-2-2: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

Candidate solutions:

- Proposal 1:

1> if the target TCI state is in the serving cell active TCI state list

2> Tfirst-RS = 0 and TRS-proc= 0

1> else if the UE is configured with CLTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and starting to execute cell switch towards the target cell is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or

3> if the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the CLTM candidate cell TCI state activation MAC-CE is received

4> Tfirst-RS = 0 and TRS-proc= 0

1> else if the target cell is an FR1 cell, and the UE is not configured with CLTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if not more than TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or

NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.

3> if not more than TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell

4> Tfirst-RS = 0 and TRS-proc= 0

2> else if the time between the latest PDCCH ordered RACH preamble transmission on the target cell and starting to execute cell switch towards the target cell is not more than 160 ms.

3> Tfirst-RS = 0 and TRS-proc= 0

1> else:

2> Tfirst-RS is the time to the first SSB transmission on the target cell after TLTM-processing.

- Proposal 2: If UE has received a PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TA timer is running.

Agreement:

- Proposal 1 is agreed.

- Proposal 2 can be discussed during maintenance phase without impact on core part completion.

Issue 1-2-3: CLTM requirements for candidate cells without active TCI states in FR2

Agreement:

- Capture the following in the WF.

    - As starting point: L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply.

- FFS whether and how to capture in the CR

4) performance

Issue 4-1-2: test configuration for event-triggered L1 reporting

Agreement:

- On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

Issue 4-2-1: CSI-RS based L1 measurement accuracy

Agreement:

- CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met:

  - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

  - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

- New requirements are defined in the following sections

| Accuracy requirements | Detail |
| --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Relative Accuracy |

Issue 4-3-1: CLTM test scope – frequency range

Agreement:

- CLTM test frequency range:

  - FR1 to FR1

  - FR2 to FR2

- Other options are not precluded.

#### 2.4.2	Remaining Open issues

Event triggered L1 measurement reporting

Test cases for event triggered L1 measurement reporting.

CSI-RS L1 RSRP measurement

Test cases for CSI-RS L1 RSRP measurement.

Conditional LTM

Test cases for CLTM.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RP-251187	Status Report for Work Item: NR mobility enhancements Phase 4 (rapporteur: Apple)

RP-250339	Revised Work Item: NR mobility enhancements Phase 4

RAN1#122:

R1-2505886	FL Summary #2 of NR Mobility enhancement Phase 4	Moderator (Apple)

R1-2505885	FL Summary #1 of NR Mobility enhancement Phase 4	Moderator (Apple)

R1-2505160	Remaining issues on measurements related enhancements for LTM	Spreadtrum, UNISOC

R1-2505244	Maintenance on measurement related enhancements for LTM	Nokia

R1-2505271	Maintenance on measurements related enhancements for LTM	ZTE Corporation, Sanechips

R1-2505333	Maintenance on measurements related enhancements for LTM	CATT

R1-2505386	Maintenance on measurements related enhancements for LTM	vivo

R1-2505550	Remaining issues on Rel-19 LTM including CSI-RS based measurement/reporting and early CSI acquisition	Samsung

R1-2505623	Maintenance on NR mobility enhancements Phase 4	Ericsson

R1-2505665	Discussion on NR mobility enhancement Phase 4	Ofinno

R1-2505740	Remaining Issues of measurement enhancement for LTM	OPPO

R1-2505811	Maintenance on the measurements for LTM	Lenovo

R1-2505848	Remaining issues on measurements related enhancements for LTM	LG Electronics

R1-2506071	Maintenance on measurements related enhancements for LTM	Sharp

R1-2505231	Maintenance on measurements related enhancements for LTM	Huawei, HiSilicon

RAN2#131:

R2-2505012	LS on frequency location of CSI-RS resources for CSI acquisition in LTM (R1-2504828; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN4	Cc:RAN2

R2-2505065	Reply LS on security handling for inter-CU LTM in non-DC cases (S3-252398; contact: Huawei)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN3	Cc:RAN2

R2-2505453	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	-	B	NR_Mob_Ph4-Core

R2-2506195	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	1	B	NR_Mob_Ph4-Core	R2-2505453

R2-2506415	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	2	B	NR_Mob_Ph4-Core	R2-2505453

R2-2505291	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	CR	Rel-19	37.340	18.6.0	0419	-	B	NR_Mob_Ph4-Core

R2-2505164	Report of [POST130][120][MOB]	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506217	Report of [AT131][107][MOB](CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505162	Draft 306 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	18.6.0	NR_Mob_Ph4-Core

R2-2505163	Draft 331 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	18.6.0	NR_Mob_Ph4-Core

R2-2505398	Discussion summary and list of MAC open issue for Mob Ph4	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505397	Introduction of NR mobility enhancements Phase 4 in MAC	vivo (Rapporteur)	CR	Rel-19	38.321	18.6.0	2098	-	B	NR_Mob_Ph4-Core

R2-2505815	Introduction of RRC changes for mobility enhancements phase 4	Ericsson	CR	Rel-19	38.331	18.6.0	5443	-	B	NR_Mob_Ph4-Core

R2-2505816	List of open issues for mobility ph4	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506213	Discussion report on [AT131][103][MOB] MAC open issues (vivo)	Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506212	List of open issues for mobility phase4	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505117	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505717	Final View on Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505893	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505399	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506139	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505158	UE capability for fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505158	UE capability for fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505165	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505311	Remaining issues on inter-CU LTM	ETRI	discussion	Rel-19

R2-2505399	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505455	Remaining issues of Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505517	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505546	Discussion on inter-CU LTM	Qualcomm Incorporated	discussion

R2-2505583	Remaining issues on inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505657	LTM CG Resource consumption for the target cells	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2505730	Remaining issues of Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505786	Discussion on open issues of inter-CU LTM	Ofinno	discussion	Rel-19

R2-2505869	How to include the NCC value in the Enhanced Cell Switch Command MAC CE	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505870	On remaining open issues for Inter-CU LTM and DC-LTM	Nokia	discussion

R2-2505893	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506022	Remaining issues and solutions on inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506113	Discussion on SP CSI-RS for target cell	NTT DOCOMO, INC.	discussion	Rel-19

R2-2506139	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506158	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505719	Report from [POST130][117][MOB] (Nokia)	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506214	Text Proposal for multi-TRP in Rel-19 LTM		Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506149	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion

R2-2505518	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506140	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: A per UE capability is introduced to report the beam not satisfying the event condition in the MR MAC CE.

R2-2505117	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505135	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505166	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505180	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2505348	Discussions on L1 event triggered measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505400	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505456	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505482	Remaining issues of LTM measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505518	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505544	Discussion on L1 event-triggered measurement reporting	Qualcomm Incorporated	discussion

R2-2505548	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2505717	Final View on Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505719	Report from [POST130][117][MOB] (Nokia)	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505731	Remaining issues of L1 event triggered measurement reporting 	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505764	Discussion on triggering of MR MAC CE for leaving beam 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2505787	Remaining issues for L1 event triggered measurement report	Ofinno	discussion	Rel-19

R2-2505868	Handling of SP CSI-RS resources of target cell after LTM cell switch	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505926	Leftover for L1 measurement and report	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2505966	Remaining issues of L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505974	Remaining issues of event-triggered L1 measurement reporting for LTM 	Kyocera 	discussion	Rel-19

R2-2506023	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506065	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506140	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506149	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505360	Issue in Fast LTM Recovery after Conditional LTM Failure	Fujitsu, NTT DOCOMO, InterDigital	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505167	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505483	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505312	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505945	Discussion on open issue of conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505894	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505098	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2505104	Discussion on remaining open issues of conditional intra-CU LTM	Transsion Holdings	discussion

R2-2505160	Remaining issues in conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505167	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505312	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

R2-2505349	Discussion on MAC open issues for Conditional intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505360	Issue in Fast LTM Recovery after Conditional LTM Failure	Fujitsu, NTT DOCOMO, InterDigital	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505401	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505483	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505519	Open issues for conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505545	Discussion on conditional intra-CU LTM	Qualcomm Incorporated	discussion

R2-2505584	Remaining issues on CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505620	Remaining Open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505641	Discussion on early TA for conditional LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2505696	Remaining issues for CLTM	Lenovo	discussion	Rel-19

R2-2505729	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505733	Introduction of Hybrid CHO/LTM Handover Mechanism	Jio Platforms	discussion	Rel-19

R2-2505788	Discussion on remaining issues of CLTM	Ofinno	discussion	Rel-19

R2-2505867	L2 Reset Coordination for Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505894	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505945	Discussion on open issue of conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506010	Remaining issues of Conditional intra-CU LTM 	Kyocera 	discussion

R2-2506024	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506032	Discussion on remaining issue for C-LTM	ASUSTeK	discussion	Rel-19	38.321	NR_Mob_Ph4-Core

R2-2506066	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506132	Considerations on conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2506141	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

RAN3 #129

- R3-255011 LS on RAN2 agreements for SP CSI-RS activation/deactivation	RAN2(CATT)

- R3-255027 Reply LS on security handling for inter-CU LTM in non-DC cases	SA3(Huawei)

- R3-255057 (BL CR to 37.483) Introducing Rel-19 Mobility enhancement	LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC, Ofinno, Jio Platforms (JPL)

- R3-255058 (BL CR to 38.420) Support for Inter-CU LTM	ZTE Corporation, China Telecom, Samsung, Nokia, CATT, NEC, LG Electronics, Ericsson, Huawei, Lenovo

- R3-255059 (BL CR to 38.470) Support for Inter-CU LTM procedure	Samsung, Huawei, LG Electronics, Ericsson, China Telecom, Nokia, ZTE Corporation, CATT

- R3-255060 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC	CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm

- R3-255061 (BL CR to 38.300) Support for Inter-CU LTM	Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

- R3-255062 (BL CR to 38.401) on conditional intra-CU LTM and intra-CU LTM	China Telecom, ZTE Corporation, NEC, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics, Qualcomm, Jio Platforms

- R3-255063 (BL CR to 38.423) Xn support for inter-CU LTM	Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

- R3-255064 (BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC	Lenovo, Ericsson, CATT, Huawei, Ofinno, Nokia, NEC, LGE, China Telecom, Google, ZTE, Samsung

- R3-255065 (BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM	Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated, Jio Platforms

- R3-255138 TP (BL CR TS 38.300, TS 38.473, TS 38.423) Remaining issues on Inter-CU LTM procedure	Nokia

- R3-255139 Discussion on Inter-CU LTM with Dual Connectivity	Nokia

- R3-255140 Discussion on Conditional LTM in split architecture	Nokia

- R3-255149 (TP to 38.423, 38.473) Inter-CU LTM	ZTE Corporation

- R3-255150 (TP to BL CR for TS 38.300, 38.473, 38.423) Rel-19 Set ID assignment	ZTE Corporation

- R3-255151 (TP to BL CR for TS 38.401, 38.473) TAT transfer for Conditional LTM	ZTE Corporation

- R3-255197 (TP to BL CR for TS 38.423 and 38.473 on Inter-CU LTM) Remaining Rel-19 inter-CU LTM issues	NEC

- R3-255198 (TP to BL CR for TS 38.423 and 37.340 on Inter-CU LTM with DC) Remaining issues of Rel-19 inter-CU LTM in DC scenario	NEC

- R3-255199 (TP to BL CR for 38.473 on conditional intra-CU LTM) Remaining issues of Rel-19 intra-CU Conditional LTM	NEC

- R3-255268 Completion of Inter-CU LTM	Ericsson

- R3-255269 (TP for LTM BL CR for TS 38.423, TS 38.473, TS 38.300, TS 38.401) – Support for inter-CU LTM	Ericsson

- R3-255270 (TP to CLTM BL CR for TS 38.473, TS 38.401) – Completion of Intra-CU Conditional LTM	Ericsson

- R3-255281 Remaining Issues on Data Forwarding for SN initiated Inter-SN LTM	Ofinno, LLC

- R3-255282 (TP for TS 38.423) Cell Switch Notification for LTM DC Scenario	Ofinno, LLC

- R3-255283 Open Issues on Access Success for Inter-SN SCG LTM	Ofinno, LLC

- R3-255301 Signalling enhancements for Inter-CU LTM handover	Qualcomm India Pvt Ltd

- R3-255302 Signalling enhancements for Intra-CU Conditional LTM	Qualcomm India Pvt Ltd

- R3-255374 (TP for TS38.401) On support of inter-CU LTM	China Telecom

- R3-255375 Discussion on inter-CU LTM in DC scenario	China Telecom

- R3-255376 (TP to TS38.401) On support of intra-CU Conditional LTM	China Telecom

- R3-255403 [TP to BLCR for TS 38.401] Inter-CU LTM	Lenovo

- R3-255404 [TP to BLCR for TS 38.423] Inter-CU LTM in DC	Lenovo

- R3-255405 [TP to BLCR for TS 38.423] Inter-CU LTM in DC - text update	Lenovo

- R3-255418 (TP to BL CR 38.423) Clarification on inter-CU LTM and LTM with SCG in NR-DC	Google

- R3-255419 (TP to BL CR 37.340 and 38.473) Clarification on inter-CU LTM and LTM with SCG in NR-DCs	Google

- R3-255421 Discussion on inter-CU LTM	NTT DOCOMO INC..

- R3-255424 (TP for LTM BLCR for TS38.300): Inter-CU LTM	Huawei

- R3-255425 (TP for LTM BLCR for TS38.473): Inter-CU LTM	Huawei

- R3-255426 [DRAFT] Reply LS on security handling for inter-CU LTM in non-DC cases	Huawei

- R3-255427 (TP for LTM BLCR for TS38.473, TS38.401): Intra-CU conditional LTM	Huawei

- R3-255440 (TP for LTM CR for TS38.423): LTM Resource Lifecycle Management in inter-CU LTM	Jio Platforms

- R3-255532 PRACH resources for RACH-less LTM	Ericsson, Jio Platforms, Lenovo, NTT DoCoMo

- R3-255533 (TP to BL CR for TS 38.423) – PRACH Resources for RACH-less LTM	Ericsson, Jio Platforms, Lenovo, NTT DoCoMo

- R3-255534 (TP to BL CR for TS 38.473) – PRACH Resources for RACH-less LTM	Ericsson, Jio Platforms, Lenovo, NTT DoCoMo

- R3-255550 gNB-DU initiated LTM resource reconfiguration	Rakuten Mobile Inc, Qualcomm Inc, NTT DOCOMO INC

- R3-255601 Inter-CU LTM Robustness Enhancements	Jio Platforms

- R3-255604 Discussion for general issues in Inter-CU LTM	CATT

- R3-255605 (TP to BL CR for TS37.340) Discussion for Inter-CU LTM in DC	CATT

- R3-255606 (TP to 38.473) Discussion for C-LTM	CATT

- R3-255614 Discussions on finalizing the essential aspects of Inter-CU LTM	LG Electronics Inc.

- R3-255615 (TP for NR_Mob_Ph4 TS 38.423) Inter-CU LTM	LG Electronics Inc.

- R3-255616 (TPs for NR_Mob_Ph4 TS 38.473 and TS 38.401) Discussions on the remaining aspects of Conditional Intra-CU LTM	LG Electronics Inc.

- R3-255625 Fetching reference configuration from candidate gNB in inter-CU LTM	Huawei, Google, Nokia, Jio Platforms, CATT, CMCC, NTT Docomo, Lenovo, China Telecom, Samsung

- R3-255626 (TP for LTM BLCR for TS38.423): Fetching reference configuration from candidate gNB in inter-CU LTM	Huawei, Google, Nokia, Jio Platforms, CATT, CMCC, NTT Docomo, Lenovo, China Telecom, Samsung

- R3-255627 (TP for LTM BLCR for TS38.300): Fetching reference configuration from candidate gNB in inter-CU LTM	Huawei, Google, Nokia, Jio Platforms, CATT, CMCC, NTT Docomo, Lenovo, China Telecom, Samsung

- R3-255628 Clarification on the single UE XnAP association in inter-CU LTM	Huawei, NEC, LG Electronics

- R3-255629 (TP for LTM BLCR for TS38.300): Clarification on the single Xn UE association in inter-CU LTM	Huawei, NEC, LG Electronics

- R3-255630 (TP for LTM BLCR for TS38.423): Clarification on the single Xn UE association in inter-CU LTM	Huawei, NEC, LG Electronics

- R3-255659 (TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM	Samsung

- R3-255660 Additional Discussion on inter-gNB-CU LTM	Samsung

- R3-255685 Remaining issues on Intra-CU Conditional LTM	Samsung

- R3-255715 (BL CR to 38.423) Xn support for inter-CU LTM	Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

- R3-255724 Support for Semi-persistent CSI-RS transmission	Ericsson, Jio Platforms, Verizon Wireless, ZTE, Ofinno

- R3-255725 (TP to BL CR for TS 38.423) Support for Semi-persistent CSI-RS transmission (Option 1)	Ericsson, Jio Platforms, Verizon Wireless, ZTE

- R3-255726 (TP to BL CR for TS 38.473) Support for Semi-persistent CSI-RS transmission (Option 1)	Ericsson, Jio Platforms, Verizon Wireless, ZTE

- R3-255727 (TP to BL CR for TS 38.423) Support for Semi-persistent CSI-RS transmission (Option 2)	Ericsson, Jio Platforms, Verizon Wireless, ZTE

- R3-255728 (TP to BL CR for TS 38.473) Support for Semi-persistent CSI-RS transmission (Option 2)	Ericsson, Jio Platforms, Verizon Wireless, ZTE

- R3-255772 Summary of offline discussion on inter-CU LTM	China Telecom

- R3-255781 CB:#MobilityEnh_LTM	China Telecom

- R3-255782 CB:#MobilityEnh_LTMNR-DC	Lenovo

- R3-255783 CB:#MobilityEnh_CLTM	Nokia

- R3-255835 (TP for LTM BLCR for TS38.473): Inter-CU LTM	Huawei

- R3-255836 TP for LTM BLCR for TS38.473): Intra-CU conditional LTM	Huawei

- R3-255839 (TP to BL CR for TS 38.401, 38.473) TAT transfer for Conditional LTM	ZTE Corporation

- R3-255842 (TP for LTM BL CR for TS 38.423) - Support for inter-CU LTM	Ericsson

- R3-255857 TP (BL CR TS 38.300) Remaining issues on Inter-CU LTM procedure	Nokia

- R3-255871 (TP for TS38.401) On support of inter-CU LTM	China Telecom

- R3-255903 (TP to BLCR for TS 38.423) Inter-CU LTM in DC	LG Electronics Inc.

- R3-255904 (TP to BL CR for TS37.340) Discussion for Inter-CU LTM in DC	CATT

- R3-255936 (TP to BL CR for TS 38.401, 38.473) TAT transfer for Conditional LTM	ZTE Corporation

- R3-255961 (TP for LTM BL CR for TS 38.423) - Support for inter-CU LTM	Ericsson

- R3-255962 (TP for LTM BLCR for TS38.473): Inter-CU LTM	Huawei

- R3-255963 (TP for LTM BLCR for TS38.473): Intra-CU conditional LTM	Huawei

- R3-256003 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC	CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm

- R3-256004 (BL CR to 38.300) Support for Inter-CU LTM	Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

- R3-256005 (BL CR to 38.401) on conditional intra-CU LTM and intra-CU LTM	China Telecom, ZTE Corporation, NEC, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics, Qualcomm, Jio Platforms

- R3-256006 (BL CR to 38.423) Xn support for inter-CU LTM	Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

- R3-256007 (BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM	Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated, Jio Platforms

- R3-256023, (BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC Lenovo, Ericsson, CATT, Huawei, Ofinno, Nokia, NEC, LGE, China Telecom, Google, ZTE, Samsung

RAN4 #116

R4-2509066	Topic summary for [116][223] NR_Mob_Ph4_Part1

R4-2509067	Topic summary for [116][224] NR_Mob_Ph4_Part2

R4-2509160	On LTM L1 event-triggered reporting

R4-2509161	DraftCR for LTM L1 event-triggered reporting

R4-2509232	Discussion on performance part for Rel-19 LTM

R4-2509288	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements

R4-2509289	Discussion on CSI-RS based L1 measurement  for Rel-19 LTM enhancements

R4-2509290	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

R4-2509291	Discussion on Rel-19 NR mobility Phase4 RRM performance requirements

R4-2509455	BigCR for R19 mobility RRM

R4-2509456	Discussion on event triggered L1 measurement reporting

R4-2509457	Discussion on RRM performance of NR mobility enhancements Phase 4

R4-2509461	R19 mobility CR for measurement restriction in serving cell

R4-2509466	(NR_Mob_Ph4-Core) On remaining issue for CSI-RS based L1 measurement

R4-2509467	(NR_Mob_Ph4-Core) draft CR on CSI-RS based L1 measurement requirement

R4-2509592	RRM Core requirements on event triggered L1 measurement reporting

R4-2509593	RRM Core requirements on CSI-RS based L1 measurement

R4-2509594	RRM Core requirements on conditional Intra-CU LTM

R4-2509595	DraftCR to 38.133 on conditional Intra-CU LTM

R4-2509596	RRM performance requirements on NR mobility enhancements Phase 4

R4-2509698	On event triggered L1 measurement reporting

R4-2509699	DraftCR on capabilities for Support of Event Triggering and Reporting Criteria

R4-2509700	On CSI-RS based L1 measurements

R4-2509701	On RRM performance requirements of R19 mobility

R4-2509765	Discussion on event triggered L1 measurement reporting for mobility

R4-2509766	Discussion on CSI-RS based L1 for mobility

R4-2509928	Discussion on Conditional Intra-CU LTM

R4-2509929	DraftCR for Conditional Intra-CU LTM

R4-2510099	Discussion on event triggered L1 report for LTM

R4-2510100	Discussion on CSI-RS L1 measurement for LTM

R4-2510101	Draft CR on introduction of CSI-RS L1-RSRP measurement

R4-2510102	Discussion on Conditional LTM

R4-2510103	Discussion on RRM performance requirements for R19 mobility

R4-2510145	Discussion on event triggered L1 measurement reporting

R4-2510146	Discussion on CSI-RS based L1 measurement

R4-2510147	Discussion on conditional intra-CU LTM

R4-2510148	Discussion on RRM performance requirements for NR mobility enhancements

R4-2510175	DraftCR on event-triggered reporting for CSI-RS based L1 measurement

R4-2510328	On RRM requirements for LTM CSI-RS based L1 measurement

R4-2510329	On RRM requirements for LTM event-triggered L1-RSRP reporting

R4-2510330	Updates on UE capability for mobility enhancement phase 4 WI

R4-2510331	Draft CR on measurement reporting requirements for CSI-RS based LTM

R4-2510332	On performance requirements for mobility enh phase 4

R4-2510447	Discussion on Conditional Intra-CU LTM

R4-2510448	Discussion on CSI-RS based L1-RSRP measurement

R4-2510449	Discussion on Event triggered L1 measurement reporting

R4-2510450	Discussion on performance requirements for LTM

R4-2510454	draftCR on measurement restriction for CSI-RS based L1-RSRP

R4-2510494	Further discussion on Conditional Intra-CU LTM

R4-2510572	Discussion on Event triggered L1 measurement reporting

R4-2510573	Discussion on CSI-RS based L1 measurement

R4-2510574	Draft CR on scheduling availability of UE during CSI-RS based L1-RSRP measurement

R4-2510575	Discussion on conditional Intra-CU LTM

R4-2510576	Discussion on performance requirements of mobility enhancement phase 4

R4-2510781	Discussions on event-triggered reporting for LTM

R4-2510782	Draft CR for event-triggered reporting on LTM

R4-2510783	Discussions on CSI-RS based L1 measurements

R4-2510784	Draft CR for CSI-RS ased L1 measurements

R4-2510785	Discussions on performance requirements for Mobility enhancements Ph4

R4-2510802	On Intra-CU conditional LTM

R4-2510803	Draft CR to 38.133 on intra-CU conditional LTM

R4-2511245	Discussion on CSI-RS measurements core part

R4-2511246	DraftCR to 38.133 on LTM CSI-RS applicability

R4-2511379	Discussions on event-triggered reporting for LTM

R4-2511506	Event triggered L1 measurement reporting for LTM

R4-2511507	CSI-RS based L1 measurement for LTM

R4-2511508	Conditional LTM

R4-2512140	Ad-hoc minutes for NR_Mob_Ph4

R4-2512141	WF on RRM requirements for NR_Mob_Ph4_Part1

R4-2512142	WF on RRM requirements for NR_Mob_Ph4_Part2

R4-2512158	LS on definition of CSI-RS based L1 intra/inter-frequency measurement

R4-2512184	(NR_Mob_Ph4-Core) draft CR on CSI-RS based L1 measurement requirement

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template